3GPP TSG RAN WG1 #118bis                              		                                       R1-2408994

Hefei, China, October 14th – 18th, 2024

Source:	Moderator (OPPO)

Title:	Summary on Rel-18 STxMP

Agenda Item:	8.1

Document for:	Discussion and Decision

# Introduction

This document summarizes draft CRs on Rel-18 STxMP proposed in company contributions of AI 8.1.

# Draft CRs

## Clarify the precoding behavior of SDM/SFN in 38.211

ZTE/Sanechips proposed to add some text to describe the precoding behavior of SDM and SFN scheme in 38.211.

- The draft CR is in R1-2407782

- along with the discussion paper R1-2407781.

Proposal 1: Adopt the text proposal for TS 38.211 in R1-2407782

| Company | Comments |
| --- | --- |
| Mod | This issue has been discussed for a few meetings. Some companies thought this change is not needed. Please share your views. |
| Samsung | Support. Otherwise, it is not clear how to apply precoding matrix for each of SDM or SFN. |
| ZTE | Support. It is critical to reach the mutual understanding accurately between UE and gNB in terms of precoding matrix for SDM and SFN STxMP PUSCH transmission. |
| Nokia | We do not think it is needed, 38.214 would have the necessary description already. |
| Google | We think this issue should be closed in this meeting. The CR seems to miss the CG-PUSCH. |
| QC | As discussed in the previous meetings, the TP is redundant. It would be good to have a conclusion on this to avoid repeated discussions. We can be fine either way (can accept majority view). |
| CATT | Support and it is a valid case. |
| Spreadtrum | Based on previous meetings’ discussions, it seems that this CR is not necessary. We are fine with majority view. |
| vivo | Not needed |
| Mod | A few companies (Nokia, QC, vivo, Spreadtrum) suggested this CR is not needed. |
| Huawei, HiSilicon | It is not needed. This issue has been discussed from back in Rel-18. In the case of SFN/SDM, it is clear that the precoding in 6.1.3.5 of 38.211 should be viewed as per panel procedure. |

## Clarify differential L1-RSRP reporting format is applied to Rel-18 group-based beam report

Samsung proposed draft CR for 38.214 in R1-2408622 to clarify that differential L1-RSRP reporting format is also applied to rel-18 group-based beam report.

Mod: My understanding is that this change is needed. The agreed rel-18 group-based beam report was really an extension of rel-17 group-based beam reporting. The rel-18 group-based beam report is adding simultaneous UL transmission on the basis of rel-17 group-based beam report.

Proposal 2: Adopt the text proposal in R1-2408622 for alignment CR for TS 38.214, with the following update:

- The “groupBasedBeamReporting-r18” is revised to “groupBasedBeamReporting-v18”

| Company | Comments |
| --- | --- |
| Mod | Please share your view. |
| Samsung | Support. |
| ZTE | Support. Especially, it is worth to be noticed that the RRC name of this higher layer parameter is specified as groupBasedBeamReporting-v18, rather than groupBasedBeamReporting-r18, in both TS 38.331 and TS 38.214, which should be corrected for alignment. Besides, note that the similar wording “groupBasedBeamReporting-r17 or groupBasedBeamReporting-v18” was already described in many parts in TS 38.214, we just wonder whether it is proper to take this TP as an alignment CR? |
| Nokia | A clarification is needed, perhaps more as an alignment CR. |
| Google | OK. We also think this is an alignment CR |
| Lenovo | Support to take it as an alignment CR. |
| CATT | Ok to support. |
| Spreadtrum | OK |
| vivo | Support the CR |
| Mod | Given the comments, I update the proposal to alignment CR |
| Huawei, Hisilicon | OK in principle and agree with ZTE observation. The correct parameter name is groupBasedBeamReporting-v18. Also, all instances of “groupBasedBeamReporting-r17” in 38.214 should in fact be changed to “groupBasedBeamReporting-v17”. |
| LG | Support |

## Correction on SRI/TPMI for mTRP PUSCH TDM repetition and SDM/SFN

Ericsson proposed draft CR for 38.214 in R1-2408962 to correct the text description on SRI/TPMI for TDM repetition, SDM and SFN schemes for both codebook based and non-codebook based PUSCH for the following reasons:

- A non-existent second ‘Precoding information and number of layers’ field in DCI is used by the UE for its second TPMI for a second SRS resource set, and the Second SRS resource indicator from DCI is not used.

- Which DCI field corresponds to the first or second TPMI/SRI is not stated.

- “Two TPMI(s)” in the text incorrectly implies there can be one or two TPMIs, since two are always provided when two SRS resource sets are configured.

- “Two SRI(s)” in the text incorrectly implies there be one or two SRI(s), when either 0 or 2 are used, since SRI’s use is optional, and since the number of SRS resources must be the same in each SRS resource set.

- For STxMP, the second TPMI or SRI is reserved for SRSI=00 or 01, which may be understood as reserved in DCI and not provided, which is incorrect.

Proposal 3: Adopt the text proposal for TS 38.214 in R1-2408962

| Company | Comments |
| --- | --- |
| Mod | Please share your views on the TP |
| Samsung | We are generally fine. We have two questions. - There are 3 parts adding “and Second Precoding information field, respectively,”, and 1 part adding “and Second Precoding information, respectively,”. We can align these, either putting “field” or not. - For changing “the second SRI is reserved” into “the second SRI not used”, we think that the meaning is same. “the second SRI is reserved” means that there is the second SRI field in DCI, but the bits in that field do not have any meaning, which is same as “the second SRI not used”. Hence, we think that it is okay to keep current specification in this part. |
| ZTE | This TP seems not needed, we think the current wording in TS 38.214 is clear enough without any ambiguity. |
| Nokia | Some form of clarification is possible, but not to the level proposed in the CR. We prefer to discuss this more during the meeting! |
| Google | We think this needs more discussion. Some changes seem to be for R17 mTRP PUSCH instead of R18 STxMP. |
| Lenovo | We are fine to discuss this CR. We have the following comments: The correction for the case that ‘neither multipanelSchemeSDM nor multipanelSchemeSFN is configured and two SRS resource sets are configured’ seems contradictory in the corresponding paragraph. The beginning says that ‘zero or two Precoding information and number of layers fields will be indicated in the DCI’, but the later description states that ‘For one or two TPMI(s) applied to the PUSCH repetition(s),’. Secondly, we agree with Samsung that the change “the second SRI is reserved” into “the second SRI not used” is not needed. |
| QC | Fine with the TP in general. For changing “reserved” to “not used”, we tend to agree Samsung that “reserved” means the field is present but not used. Therefore, this change seems unnecessary. |
| CATT | Generally OK but raise the same comments as Samsung and QC mentioned. Such modifications seems not necessary. |
| vivo | Seems not needed, open for discussion |
| Ericsson | Thanks for the comments. - Regarding the need for some change: In our understanding, some text is clearly incorrect. There is a ‘second precoding information’ field in DCI, but not a ‘second precoding information and number of layers field’, as there is no number of layers indicated in the second precoding information field. Therefore, the text ‘two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]’ is incorrect. Similarly, the wording ‘SRI(s)’ and ‘TPMI(s)‘ implies there can be one or more, when only two can be present. - @Samsung: Agree the proposal should be improved a bit with respect to consistent use of ‘field’. - @Samsung & @Lenovo: Regarding ‘reserved’: for me the current text is a little confusing, even if we interpret that the field is provided. The gNB should not have to set e.g. the TPMI to one of the ‘reserved’ values from the tables in 38.212, although that is one possible interpretation of ‘the second SRI and second TPMI are reserved,’. Also codepoint ‘11” in the next bullet is ‘reserved’, which means that state will not be set, which is different from the behavior for codepoints 00 or 01, where ‘reserved’ means to not use the value provided in the DCI. - @Lenovo: In my understanding, non-codebook is supported for M-TRP PUSCH repetition, which motivates ‘zero or two’. Two TPMIs are always provided in DCI for codebook based operation if two SRS resource sets are configured. However, one or two TPMI(s) may be applied, depending on what value of SRSI is signaled. Does that clarify? - As Google points out, the proposal addresses both Rel-17 M-TRP PUSCH repetition and Rel-18 STxMP. We have a Rel-17 CR (R1-2408963) that attempts to ensure the align the Rel-18 and Rel-17 corrections. |
| Mod | Majority companies are ok to discuss the issue, though a couple of companies think the change is not needed. @Ericsson, you might provide updated version with improved wording. |
| Huawei, Hisilicon | Generally supportive but we also think changing “reserved” to “not used” may not be a good idea. Our understanding is that, the second SRI field still exists (non-zero number of bits still allocated to it in the DCI) but UE skip those bits. Technically, this can be interpreted as “not used” but the term “reserved” and not “not used” has been used elsewhere in the spec for similar situations. |

## Editorial change: RRC parameter alignment.

R1-2408081 proposed RRC parameter alignment for TS 38.213:

- Replace enableSTx2PofmDCI to sTx-2Panel

- Replace multipanelSFN-Scheme to multipanelSchemeSFN

- Replace multipanelScheme to multipanelSchemeSDM or multipanelSchemeSFN

Proposal 4: Adopt the following TP for alignment CR:

- R1-2408081 (for TS 38.213) with excluding the suggested changes in Clauses 7.7.1 and 9.2.6

| Company | Comments |
| --- | --- |
| Mod | Please share your view on this editorial change |
| Samsung | Fine with editorial change. |
| ZTE | Support |
| Nokia | Support |
| Google | OK |
| Lenovo | Fine |
| CATT | OK. |
| vivo | Ok |
| Huawei, HiSilicon | OK |
| Huawei, HiSilicon2 | Just noticed that multipanelSFN-Scheme is correct and should not be changed to multipanelSFNScheme. multipanelSFN-Scheme is the PUCCH related RRC parameter while multipanelSFNScheme is PUSCH related parameter. |
| OPPO | @HW, thanks for spotting that. You are right. The changes on PUCCH shall be excluded. The proposal is updated accordingly. |
| LG | Ok |

# Proposals for Online Discussion

Proposal 4: Adopt the following TP for alignment CR:

- R1-2408081 (for TS 38.213) with excluding the suggested changes in Clauses 7.7.1 and 9.2.6

Proposal 2: Adopt the text proposal in R1-2408622 for alignment CR for TS 38.214, with the following update:

- The “groupBasedBeamReporting-r18” is revised to “groupBasedBeamReporting-v18”

Proposal 1: Adopt the text proposal for TS 38.211 in R1-2407782

Concerns: Nokia, QC, vivo, Spreadtrum, Huawei/HiSilicon