3GPP TSG RAN WG1 #118bis	R1-2409004

Hefei, China, October 14 – 18, 2024	(revised from R1-2408945)

Source:	Inmarsat, Viasat

Title:	Operator views on DL Coverage Enhancements for NR NTN

Agenda Item:	9.11.1

Document for:	Discussion and Decision

# Introduction

The following Observations and Agreements have been captured in Maastricht during RAN#118 [1]:

Observation

Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints:

- With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used.

- With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used.

- Note: the overhead of SIB19 was included in some of the results

- Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation

Agreement

As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider:

- Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

  - Default value[s] with extended periodicity assumed by NTN UE for initial access can be:

    - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms}

- Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

- Link-level enhancements for the following channels:

- PDCCH

- PDSCH with Msg 4

- PDSCH with SIB1/SIB19.

  - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact.

- Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

This document presents some key considerations from an NTN operator’s perspective on DL Coverage Enhancements, and in particular on the system-level considerations, such as SSB periodicity relaxation and improved support for Multi-Tiered Beam operation.

# Discussion

Summary

This paper discusses enhancements targeted by DL Coverage Enhancements, in particular the System-level enhancements, such as SSB Periodicity Relaxation and Multi-Tiered Beam operation.  Both those mechanisms can be of critical importance to an NR NTN system, especially in the initial phases of deployment.

Both of these mechanisms are very important to satellite systems that are either power-limited (in terms of total payload power aka aggregate power/EIRP) or hardware-limited (physical limit in the number of beamformer ports), or both. However, it is also important to understand that these two mechanisms are not necessarily a replacement for each other, and may likely be both employed, for different purposes, as they have different advantage and applications.

These two mechanisms are also not mutually-exclusive, and can be combined by an NTN operator as part of their deployment planning, either to work around deployment constraints, or to minimize impact on user experience, including device battery drain, which can be critical in emergency situations.

Thus, we strongly urge to continue the work on specifying both SSB periodicity relaxation as well as enhancement to multi-tiered beams operation.

Proposal 1: Specify both SSB periodicity relaxation and Multi-Tiered Beam operation with Wide Beam for DL SSB and common channels, and Narrow Beams for UL channels (including and especially PRACH) and DL dedicated channels.

SSB Periodicity Relaxation

SSB periodicity relaxation is a critical enhancement in order to make sure NR NTN can be deployed with cost-effective space segment (that is by its nature, power and hardware limited).  The benefits have been clearly verified and understood by all companies, as it has been shown that the SSB and CORESET periodicity relaxation allows to cover all possible cell positions in the presence of space segment power and hardware limitations.

Observation 1: The study phase has clearly shown that SSB periodicity relaxation of at least up to 160 ms can address the problem of covering all possible cell positions with a subset of total maximum simultaneously active beams.

In addition to enabling support of the complete coverage footprint, it has also been clearly shown by the study phase that these enhancements can significantly reduce the DL control channel overhead, even in scenarios where coverage can be fulfilled.

In practice, this translates to a concrete overall increase of the total usable (and thus sellable) capacity, both in DL and UL.

Observation 2: It has also been shown that SSB and CORESET periodicity relaxation can significantly reduce the DL control channel overhead, overall increasing the total sellable capacity (both DL and UL).

To realize the benefits, however, it is important that periodicities of 160 ms are enabled, with 80 ms as an absolute minimum.  These are also expected to be the schemes with the least specification impact, as they are currently nominally already supported in the specification.

Furthermore, RAN2 has studied the issue and identified no impact, as long as periodicity is relaxed no further than 160 ms.

The following agreement was reached in RAN2 during RAN2#127:

From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

Thus, we propose the following for agreement:

Proposal 2: For SSB periodicity relaxation, specify periodicity relaxation up to 160 ms, or at least up to 80 ms as a minimum.

It is understood, as it has been already highlighted by a number of companies, that SSB periodicity relaxation is likely to have an impact on cell search performance, specifically on cell search time.  This is because, although not formally specified, as we understand, the majority of current UE implementations expect an SSB periodicity of 20 ms. A relaxation of periodicity to up 160 ms would constitute an 8-fold increase in the interval between SSB occasions.

For a 20 ms periodicity, a UE has an opportunity to detect a cell at least every 20 ms.  If the detection is missed, the UE has to wait for the next occasion to verify that no cell is present, before moving on.  For a 160 ms periodicity, the UE has to dwell 8 times longer in each raster point, or possibly up to 16 times longer in case it waits for a further instance of the SSB before continuing the cell search.

However, in practice, this impact affects only the initial cell search, which is assumed to be unassisted and generally blind. As for handover and re-selection, SSB measurement windows are configured by the network.

Observation 3: It is understood that SSB periodicity relaxation up to 160 ms impacts only unassisted initial cell search.

This is a very significant impact on the cell search, and it needs to be addressed, however, the alternative to avoiding a relaxation in SSB periodicity is likely only to turning off coverage in certain cells.  This can have severe impact, as an operator will likely sacrifice coverage in areas which are likely to yield a lower revenue.  However, these areas are also those where a single, stranded user in distress is more likely to be in need of emergency communication via NTN.

Being able to provide coverage at reduced SSB periodicity can be the difference in providing life-saving coverage in low demand areas.  Instead, the impact to cell search should be address via other means, such as cell search assistance information or configuration.

Observation 4 : It is understood by operators that SSB periodicity relaxation has impact on cell search time, however, this is an important trade-off between providing life-saving coverage with a longer cell search time in unserved, low-demand areas, vs not providing coverage at all.

It is also assumed that lower SSB periodicity may not be required in all coverage areas.  In areas where this functionality is not required, such as areas nearing Terrestrial coverage or areas with higher user densities, relaxation with higher periodicities or no relaxation at all might be adopted.

Observation 5: SSB periodicity relaxation may not be required in all cell coverage areas throughout an NTN deployment.

Therefore, SSB periodicity relaxation should not be limited to a single value, but rather, configurable and pre-agreed

Proposal 3: Multiple SSB periodicities shall be supported.

It is also not unreasonable to assume that an NTN operator could arrange to pre-configure the UE with assistance information about the supported SSB periodicities.

Observation 6: It is not unreasonable to assume that a UE could be pre-configured to be aware of the maximum SSB periodicity of a given NTN network or cell coverage area.

Proposal 4: Maximum SSB periodicity supported shall be assumed to be pre-configurable in advance based on deployment/constellation on a per-coverage area or raster point basis via assistance information to the UE. How this is configured can be left to implementation and is outside of the scope of RAN1.

It is also important to note that the need for SSB periodicity relaxation may progressively be reduced and eventually go away, as constellations evolve in generations, with higher density of satellites (thus reducing the individual satellite coverage area), higher power satellites or beamforming arrays with larger number of ports.

Observation 7: The need for SSB periodicity relaxation will be reduced as NTN constellations evolve with a higher number of satellites, and with satellites that are more powerful and have beamforming arrays with a larger number of physical beamformer ports.  However, it is of critical importance.

Multi-Tiered Beam Operation

Multi-tiered beam operation has also been discussed in the context of DL Coverage Enhancements and it is a critical enhancement, in conjunction with SSB periodicity relaxation.

Proposal 5: Specify multi-tiered beam operation in addition to SSB periodicity relaxation. The aim is to allow Wide Beam for DL SSB and common channels, and Narrow Beams for UL channels (including and especially PRACH) and DL dedicated channels.

This mode of operation is already widely used by existing satellite systems, and it is a proven method of improving space segment utilization, power efficiency, spectrum usage efficiency and equalizing beam power delivery across the coverage area.

One issue in particular in NGSO systems is that there is a big relative difference in the slant range between beams close to satellite nadir, compared to beams at the edge of coverage, especially for low elevation passes.  With a constant EIRP and receive gain profile, this can cause a significant imbalance of cell signal power and gain from the perspective of a UE on the ground.

In addition, the lower elevation angle causes edge-of-coverage cells to be larger on the ground, and as noted also in prior TDocs, such as R1-2405838 [2], result in increasing coverage gaps towards the edge of coverage, in case the adjacent beam spacing (ABS) is not compensated for.

To counter this issue, NTN satellite systems are likely to target delivering a constant DL Power Flux Density (PFD) and UL receive gain contour on the ground, including compensation of the ABS.  However, the longer slant range translates into a significantly higher power demand from the payload for edge-of-coverage beams compared to center coverage beams.

Observation 8: NTN satellite systems are likely to target a constant Power Flux Density (PFD) and receive gain contour on the ground.

Observation 9: The increase in slant range a lower elevations causes satellite beams to require higher power for delivery and tends to extend the shape of the satellite beams, thus increasing cell size and, without correction of the beam spacing, resulting in coverage gaps towards the edge of coverage.

To counter the increased power demand from the space array, NTN systems may thus also opt to re-balance the beam footprints, that is, adjust the adjacent beam spacing (ABS) on the UV-plane, such that the beam projections on the ground are reasonably constant and uniform.

However, given that the minimum beam resolution is limited by the antenna aperture size and array architecture, the only possible option is to increase the footprint of the beams at nadir by reducing gain, to better align it to that of the beams at the edge of coverage, and correspondingly decrease the beam spacing at the edge of coverage in order to reduce the edge cell size.

Observation 10: NTN systems are likely to target reduction of beams size at the edge of coverage trading off a larger beam size towards nadir. This requires the use combination of wider and narrow beams.

This approach also allows to save DL power in the beams close to nadir, allowing an additional degree of freedom in the DL power allocation, that for example can be used for SSB DL power boosting at wider beams towards the edge of coverage.  This however does not improve the UL receive gain, which is again ultimately limited by antenna aperture size, therefore, multiple narrower beams would need to be allocated with a tighter ABS would need to be allocated to cover the wider coverage area at the edge. This is a very critical problem for UL link budget, however, which is already quite limited, and in particular with PRACH UL reception, as PRACH is a critical signal that the network has to receive, therefore, any UL link budget loss would be unacceptable.  In order to avoid this issue, PRACH and other UL signals would need to be mapped to narrow beams.

Observation 11: UL link budget is very challenging in NR NTN. DL coverage extension cannot come at the expense of UL coverage, in particular for critical signals such as PRACH.  PRACH on the wider lower-gain beams would lead to an unacceptable PRACH link budget degradation.

This however, would require UL channels, including PRACH, to be mapped to narrow beams, with DL channels mapped to wider beams.  Currently, the NR specification does allow some degree of multi-tiered beam operation. In particular, it is technically already possible to map SSB/control channels and data channels to beams of different resolution.  For example use wider SSB beams and narrower, higher gain beams for data, however, this granularity is not supported for PRACH.  Due to the direct implicit association between UL PRACH resources and SSB instances, the PRACH resources are currently associated with the SSB, which means that it is not possible to selectively associate PRACH resources to a smaller beam with a higher resolution than the SSB beam.  Or, rather, it is possible, but there is no mechanism of uniquely associating those PRACH resources with a unique spatial beam position.

In practice, this means that in many cases it is not possible to identify in which equivalent narrow beam coverage position a UE might be located, purely by its initial RACH preamble transmission.

Observation 12: It is currently not possible to establish a PRACH resource mapping to UL narrow beam position, in combination with a wider DL SSB beam.

To address this problem, one possible solution is to specify a mechanism for association between a specific set of PRACH resources and a given narrow beam.  This association could be either implicit (e.g. with a resource location understood in advance by the UE, mapped implicitly to a corresponding DL resource), or explicit (e.g. signaled).

This would allow the UE to pick the correct PRACH resources for initial RA, and the network to identify in which PRACH narrow beam the UE is (or likely to be) located.

Proposal 6: For multi-tiered beam operation, specify mechanism for association of PRACH resources with UL wide or narrow beam(s), implicitly or explicitly.

Some proposals have suggested to utilize Non-Cell-Defining SSB (NCD-SSB) as a reference for this new PRACH mapping, however, there is a concern with the additional resource overhead associated with this, which would negate many of the advantages of using wider beams for CD-SSB in the first place.

Observation 13: The association of new beam-specific PRACH resources to NCD-SSB (if transmitted) would create unacceptable resource overhead which would likely negate the advantages of wide beam and SSB periodicity reduction.

Proposal 7: The specification of new beam-specific PRACH resources should avoid impacting the DL resource overhead.

Another issue that arises from this is the ability to maintain backwards compatibility with existing UE that do not support multi-tiered beam operation. This could still be supported by defining the beam-associated PRACH resources as an additional set of logical resources compared to the common RACH resources.

Proposal 8: For multi-tiered beam operation, consider specifying an additional set of PRACH resources associated to narrow beams, in order to maintain legacy compatibility.

# Conclusions

Proposal 1: Specify both SSB periodicity relaxation and Multi-Tiered Beam operation with Wide Beam for DL SSB and common channels, and Narrow Beams for UL channels (including and especially PRACH) and DL dedicated channels.

Observation 1: The study phase has clearly shown that SSB periodicity relaxation of at least up to 160 ms can address the problem of covering all possible cell positions with a subset of total maximum simultaneously active beams.

Observation 2: It has also been shown that SSB and CORESET periodicity relaxation can significantly reduce the DL control channel overhead, overall increasing the total sellable capacity (both DL and UL).

Proposal 2: For SSB periodicity relaxation, specify periodicity relaxation up to 160 ms, or at least up to 80 ms as a minimum.

Observation 3: It is understood that SSB periodicity relaxation up to 160 ms impacts only unassisted initial cell search.

Observation 4 : It is understood by operators that SSB periodicity relaxation has impact on cell search time, however, this is an important trade-off between providing life-saving coverage with a longer cell search time in unserved, low-demand areas, vs not providing coverage at all.

Observation 5: SSB periodicity relaxation may not be required in all cell coverage areas throughout an NTN deployment.

Proposal 3: Multiple SSB periodicities shall be supported.

Observation 6: It is not unreasonable to assume that a UE could be pre-configured to be aware of the maximum SSB periodicity of a given NTN network or cell coverage area.

Proposal 4: Maximum SSB periodicity supported shall be assumed to be pre-configurable in advance based on deployment/constellation on a per-coverage area or raster point basis via assistance information to the UE. How this is configured can be left to implementation and is outside of the scope of RAN1.

Proposal 5: Specify multi-tiered beam operation in addition to SSB periodicity relaxation. The aim is to allow Wide Beam for DL SSB and common channels, and Narrow Beams for UL channels (including and especially PRACH) and DL dedicated channels.

Observation 8: NTN satellite systems are likely to target a constant Power Flux Density (PFD) and receive gain contour on the ground.

Observation 9: The increase in slant range a lower elevations causes satellite beams to require higher power for delivery and tends to extend the shape of the satellite beams, thus increasing cell size and, without correction of the beam spacing, resulting in coverage gaps towards the edge of coverage.

Observation 10: NTN systems are likely to target reduction of beams size at the edge of coverage trading off a larger beam size towards nadir. This requires the use combination of wider and narrow beams.

Observation 11: UL link budget is very challenging in NR NTN. DL coverage extension cannot come at the expense of UL coverage, in particular for critical signals such as PRACH.  PRACH on the wider lower-gain beams would lead to an unacceptable PRACH link budget degradation.

Observation 12: It is currently not possible to establish a PRACH resource mapping to UL narrow beam position, in combination with a wider DL SSB beam.

Proposal 6: For multi-tiered beam operation, specify mechanism for association of PRACH resources with UL wide or narrow beam(s), implicitly or explicitly.

Observation 13: The association of new beam-specific PRACH resources to NCD-SSB (if transmitted) would create unacceptable resource overhead which would likely negate the advantages of wide beam and SSB periodicity reduction.

Proposal 7: The specification of new beam-specific PRACH resources should avoid impacting the DL resource overhead.

Proposal 8: For multi-tiered beam operation, consider specifying an additional set of PRACH resources associated to narrow beams, in order to maintain legacy compatibility.

# Reference

- R1-2407482 - Session notes for 9.11 (Non-Terrestrial Networks for NR Phase 3 and Internet of Things Phase 3)   Ad-Hoc Chair (Huawei)

- R1-2405838 - Discussion on downlink coverage enhancements for NR NTN		Huawei, HiSilicon