3GPP TSG RAN WG1 meeting #118	bis                                   R1-2409005

Hefei, China, October 14th – 18th, 2024

Source: 	ZTE Corporation, Sanechips

Title:	Discussion on general aspects of physical layer design for Ambient IoT

Agenda Item:	9.4.2.1

Document for:	Discussion and decision

- Introduction

In RAN1 #118 meeting, agreements on general aspects of physical layer design for Ambient IoT were reached as followings [1].

Agreement The following table is a starting point for the study of M values and the associated minimum Btx,R2D value - Reader can use any R2D transmission bandwidth >= minimum Btx,R2D - FFS: Impacts, if any, of CP handling solutions, and other impacts - Note: depending on further study, the maximum value of M may be less than 32 M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 2. 32; 3. Agreement Small frequency shifts for D2R are studied for OOK and BPSK: - For applying with Manchester line codes - Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. - Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift. - Companies to report how they perform multiplying for option 2 - For applying with Miller line codes, according to Figure 6-13 of UHF RFID standard. - For FM0, small frequency shift is not defined - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift. - Potential purposes include: - FDMA of D2R, if supported - CW interference avoidance, if supported Note: small frequency shifts for D2R are studied for the same potential purposes for relevant identified BFSK variant(s) Agreement For D2R FEC study, the LTE convolutional code polynomials are a reference. Other designs can be studied subject to: - Constraint length K = 8 or K = 7 or K = 6 or K = 4 for further study - Mother code-rate R = 1/6, 1/4, 1/3, 1/2 for further study - FFS other details, e.g. final code rate by puncturing, shift-register initialization/termination. Agreement For D2R line codes, the study assumes the following codewords corresponding to an information bit 0 or bit 1, before considering potential small frequency-shifting: - For FM0: - According to Figures 6-8 and 6-9 of UHF RFID standard - For Miller: - According to Figure 6-12 of UHF RFID standard. Agreement - OOK and Binary PSK for baseband modulation are feasible for D2R for all devices. - The variant of Binary FSK to study further for all devices is identified as MSK (and not GMSK) - FFS: whether MSK is feasible for all devices

In this contribution, based on the above agreements and the discussion in last RAN1 meeting, the Ambient IoT physical layer design including waveform, modulation, coding, multiple access and bandwidths is further discussed.

- Discussion on A-IoT R2D

  - R2D waveform

In the last RAN1 meeting [2], a number of candidate alternatives of Method Type 1 and Method Type 2 were agreed for R2D CP handling for OFDM based OOK waveform.

Agreement in RAN1#117 Study the following regarding CP location/length determination for Method Type 1: - Alt 1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt 2: duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Companies are encouraged to clarify the CP removal method used and implementation aspects for the device - Evaluations are encouraged to be performed for a small value of M, e.g. 4 and a large value of M, e.g. 24, at least by comparison to the case where the CP length of each OFDM symbol is known by device - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement in RAN1#117 Study the following options regarding subcarrier orthogonality for Method Type 2: - Alt 1: Method Type 2 retains subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol) - Alt 1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same - FFS: whether this alternative applies if CP length is longer than the chip duration - Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP - Other potential methods are not precluded - Alt 2: Method Type 2 does not retain subcarrier orthogonality - Proponents to bring further details to RAN1#118 - Evaluations and discussions are encouraged to be performed for a small value of M, e.g. M = 4 and a large value of M, e.g. M = 24. - Companies should report the values of SFO, and SFO detection methods used in evaluations

For Method Type 1, two alternatives, i.e. Alt 1 and Alt 2, are agreed to be studied. It is proposed to remove equivalent CP length or determine CP location/length at device side, instead of reader side. Herein, whether device can determine the CP location correctly or not is evaluated.

For Method Type 2, three alternatives, i.e. Alt 1-1, Alt 1-2 and Alt 2, are agreed to be studied. For Alt 1-1 and Alt 1-2 of Method Type 2, subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol) needs to retain. While Alt 2 doesn’t need to retain subcarrier orthogonality.

In this section, different CP handling resolutions are discussed and evaluation results are provided. .

- Baseline: R2D transmission without CP is assumed as baseline.

- Method Type 1,

  - Alt 1: As shown in Figure 1(a), the CP may introduce a false transition edge with Method Type 1. While Device assumes same CP length (i.e. 9 sampling points) for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols. The CP is removed according to the location of the last transition edge, the assumed chip length and CP length.

  - Alt 2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration. According to our simulation implementation, this method is hard for device to determine an accurate CP location/length. Because of the inferior detection performance, the BLER performance for Alt 2 of Method Type 1 is not provided.

- Method Type 2,

  - Alt 1-1: The last OOK chip(s) is the same as the first OOK chip(s) in an OFDM symbol.

  - Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP.

    - In the evaluation of Alt 1-2, it is assumed the whole R2D data transmission is started from the last chip in the first OFDM symbol. For example, for M=4, R2D data using 1/2 Manchester code starts from the fourth chip in the 1st OFDM symbol as shown in Figure 1(b). Hence, for every two adjacent OFDM symbols, two chips of a Manchester codeword, which have different power levels, are separately located before and after the CP. In this case, the CP concatenated with either the first or the last chip of the Manchester codeword doesn’t introduce false transition edge. To this end, it ensures that the transition edge during a codeword occurs only at the start or end of the CP.

(a) Alt 1 of Method Type 1

(b) Alt 1-2 of Method Type 2

Figure 1 Alt 1 of Method Type 1 vs Alt 1-2 of Method Type 2

  - Alt 2: This method does not retain subcarrier orthogonality. In the evaluation, the CP is copied from the first of an OFDM symbol. In addition, a scheme of no CP, i.e. CP duration divided into each chip equally in an OFDM symbol, also belongs to this method type of Alt 2, which is the baseline method.

    - CP handling resolutions for larger M values

Before the evaluations of the above CP handling schemes, the issues of R2D CP handling methods which were discussed as followings in the last meeting [3] are resolved in this section.

Proposed Observation 2.1.1b(II): For R2D CP handling Method 1, at least for Alt 1, device needs to be aware of the boundary of OFDM symbol (i.e. beginning of the OFDM symbol) to determine CP location How device is aware of or determines the boundary would be considered under normative details (if any) Proposed Observation 2.1.1c(II): For R2D CP handling Method 2, device does not need to be aware of the boundary of OFDM symbol (i.e. beginning of the OFDM symbol) to determine CP location Proposal 2.1.1d(II): For R2D CP handling Method 1, device can use Alt 1 and/or Alt 2 (if both alternatives are supported) which is up to device implementation.

According to these proposals, the R2D CP handling Method 1 and Method 2 are distinguished by whether device needs to be aware of the boundary of OFDM symbols to determine CP location. To be more specific, it is assumed for R2D CP handling Method Type 1, device may need to be aware of OFDM symbol boundary in order to perform accurate CP removal, while for R2D CP handling Method Type 2, device does not need to be aware of OFDM symbol boundary. However, in order to ensure fixed chip length for detection, especially for cases of M≥24, device being aware of OFDM symbol boundary is preferred. Therefore, based on our understanding, whether and how device will be aware of OFDM symbol boundary depends on R2D design and device implementation. It is premature to restrict device behavior as being aware or unaware of OFDM symbol boundary.

For both R2D CP handling Method Type 1 and Method Type 2, whether and how device is aware of OFDM symbol boundary depends on R2D design and device implementation.

It is noted that in the case of larger M configuration (i.e. M≥24), the CP covers more than one chip which will introduce additional transition edge(s) during the CP duration. In our views, two methods can be considered to resolve this issue, i.e., CP removal and chip length adjustment. Taking the case of M=24 as an example, the performance impact of inaccurate CP removal for Method 1 and Method 2 and chip length adjustment are analyzed as below.

- Performance impact of inaccurate CP removal

As shown in Figure 2 (a) and (b), the CP handling results using Alt 1 of Method 1 and Alt 1-2 of Method 2 are illustrated based on the above CP handling schemes. We can see that CP length is almost twice of chip length under the case of M=24. For Alt 1-2 of Method Type 2, though the start position of CP removal is advanced or delayed because of the inaccurate edge detection, there is no additional transition edge introduced by the remaining CP chip. The reason is that Alt 1-2 of Method Type 2 always puts a codeword across CP so that it can ensure no additional edge introduced by CP removal as long as edge detection error does not exceed the length of chip. However, for Alt 1 of Method Type 1, the cases of additional transition edge introduced by CP removal cannot be completely eliminated.

For Alt 1-2 of Method Type 2, a codeword is always set across CP so that it can ensure no additional edge introduced by CP removal as long as the edge detection error does not exceed the length of chip. However, for Alt 1 of Method Type 1, the additional transition edge introduced by CP removal cannot be completely eliminated.

| - Alt 1-2 of Method Type 2 | - Alt 1 of Method Type 1 |
| --- | --- |

Figure 2 CP removal results based on Alt 1 of Method 1 and Alt 1-2 of Method 2

- Chip length adjustment

Alternatively, another solution to ensure no transition edge occurs during the CP by adjusting the length of last chips in an OFDM symbol as the length of CP can be used. For example, under the case of M=24, the length of CP (i.e. 10 or 9 sampling points) is twice of the chip length (i.e. 5 or 6 sampling points). As shown in Figure 3, based on the total of sampling points=128 for M=24 in an OFDM symbol, the length of the 1st ~ 23th chips is 5 sampling points and the length of the 24th chip is 13.

Figure 3 {5 points for 1st ~ 23th chips and 13 points for 24th chip} for Alt 1-2 of Method Type 2

According to the above analysis, Alt 1-2 of Method Type2 is proposed to be used for CP handling. Under the case of M≥24, adjusting the length of last chips in an OFDM symbol as the length of CP is proposed to ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP.

Under the case of M≥24, adjusting the length of last chip in an OFDM symbol as the length of CP is proposed to ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP.

    - CP handling evaluations

According to the above CP methods, simulation results are shown in Figure 4. According to the BLER performance of Method Type 2 with that of Method Type 1 in Figure 4, it is observed that, for the cases of different M configurations, baseline achieves the best performance because of no CP transmission and CP handling. Moreover, it is observed that, for the cases of different M configurations, Alt 1-2 of Method Type 2 has the best BLER performance among all of simulated CP handling methods except for baseline. We can also observe that Alt 1 of Method Type 1 has inferior BLER performance among all of simulated CP handling methods.

According to the simulation results, it is observed that, for the cases of different M configurations, Alt 1-2 of Method Type 2 has the best BLER performance among all of simulated CP handling methods except for baseline.

According to the simulation results, it is observed that, for the cases of different M configurations, Alt 1 of Method Type 1 has inferior BLER performance among all of simulated CP handling methods.

Figure 4 BLER performance based on different CP handling methods

Analysis on BLER performance for different CP handling methods are as followings.

- Baseline: There is no CP transmission so that the BLER performance is the best.

- Method Type 1, Alt 1: Because of CP is removed based on an equivalent value (i.e. 9 sampling points), both the CP location and CP length are not accurate if the last transition edge is not accurately determined. Moreover, for the case of larger M configuration (e.g. M=24 as shown in Figure 2 (b)), the additional transition introduced by CP can result into large BLER performance degradation

For Alt 1 of Method Type 1, the additional transition introduced by CP can result into large BLER performance degradation.

- Method Type 1, Alt 2: For different M configurations, it is hard to determine an accurate CP location/length based on the transition edge and chip length during a CP. For example, from M=4 to M =24, the CP length increases from 30% of chip length to almost twice of chip length. It is easy to result into false transition edge. Therefore, the BLER performance degrades with the increase of M value.

For Alt 2 of Method Type 1, it is hard to determine an accurate CP location/length based on the transition edge and chip length during a CP.

- Method Type 2, Alt 1-1: There is a large overhead because of setting the first OOK chip and the last OOK chip(s) to be the same in each OFDM symbol in order to ensure no additional transition introduced by CP, i.e. the overhead of the last OOK chip(s) should be at least equal to the length of CP.

For Alt 1-1 of Method Type 2, there is a large overhead because of setting the first OOK chip(s) and the last OOK chip(s) to be the same in each OFDM symbol in order to ensure no additional transition introduced by CP.

- Method Type 2, Alt 1-2: This method has no additional overhead, i.e. the first odd number of chip(s) highlighted in green in Figure 1(b) in the first OFDM symbol can be reserved for preamble transmission. Moreover, for the case of larger M configuration (i.e. M=24), the CP covers more than one chip as shown in Figure 2 (a), CP removal can be used to detect the transition at the start or the end of CP. On the other hand, chip length adjustment can, alternatively, solve the additional transition easily as shown in Figure 3. Therefore, Alt 1-2 of Method Type 2 is proposed to be an efficient CP handling method for R2D transmission.

For Alt 1-2 of Method Type 2, there is no additional overhead (i.e. the first odd number of chip(s) in the first OFDM symbol can be used for preamble transmission) to ensure a transition edge occurs only at the start or only at the end of the CP.

- Method Type 2, Alt 2: For this method, the interference to NR legacy UE and BS is larger compared to other alternatives because this method does not retain subcarrier orthogonality. Moreover, the performance is inferior to other schemes.

For Alt 2 of Method Type 2, the interference to NR is larger and BLER performance is inferior compared to other alternatives.

According to the above simulation results and analysis, Alt 1-2 of Method Type2 is proposed to be used for CP handling.

Alt 1-2 of Method Type2 is proposed to be used for CP handling.

  - R2D modulation and bandwidth

In this section, modulation and the transmission bandwidth are discussed.

- M and the associated minimum Btx,R2D

In the last RAN1 meeting [1], the M values and the associated minimum Btx,R2D value were agreed. For the determination of value(s) of M and the corresponding Btx,R2D, it relies on the data rate, i.e. data rate=M*R/TOFDM_symb, and the detection performance, i.e. CNR at the target BLER. Where R is the code rate, TOFDM_symb is the length of OFDM symbol and the target BLER is set to 10%.

According to our simulation results for different pairs of M and Btx,R2D values in [4], it is observed that some pairs of M values and the associated minimum Btx,R2D value with higher data rate have better BLER performance that that with lower data rate. As shown in Figure 5, M=32 has a higher data rate than the case of M=24, but the BLER performance of {M=32, minimum Btx,R2D #3PRBs} is better than that of {M=24, minimum Btx,R2D #2PRBs}. This trend leads to a decrease in the SNR value at the target BLER of 10% as the data rate increases. However, as systems are pushed towards higher data rates, it is common sense that to achieve a higher data rate, a higher SNR value is needed. So it is crucial to ensure stable link performance under these conditions by overcoming the decrease in SNR that accompanies an increase in data rate. Therefore, we propose that two options can be used to reform the performance trend among different pairs of M and Btx,R2D values.

- Option 1: change the minimum Btx,R2D value for M=24 as 540kHz;

- Option 2: prudent resource allocation for the pairs of M and Btx,R2D values by implementation.

Figure 5 BLER performance for different pairs of {M, Btx,R2D}

According to our simulation results for different pairs of M and Btx,R2D values in [3], it is observed that some pairs of M values and the associated minimum Btx,R2D value with higher data rate have better BLER performance that that with lower data rate.

Two options are proposed to reform the performance trend among different pairs of M and Btx,R2D values.

- Option 1: change the minimum Btx,R2D value for M=24 as 540kHz;

- Option 2: prudent resource allocation for the pairs of M and Btx,R2D values by implementation.

- 1SB and 2SB

For the discussion on double sideband or single sideband modulation for R2D transmission [3], the OOK1 or OOK 4 based R2D signal is generated in frequency domain then converted to time domain. Therefore, it is proposed that it is unnecessary to discuss 1SB and 2SB for R2D.

Proposal 2.2.2a(II): R2D transmission can be either double sideband or single sideband, up to Reader implementation. The TR records this statement, and no further study is needed in RAN1.

It is unnecessary to discuss 1SB and 2SB for R2D.

- Transmission bandwidth value Btx,R2D

According to the agreements in RAN1#116bis meeting [5], for R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is ≤ [12] PRBs and is down-selected among the following three alternatives.

Agreement in RAN1#116bis meeting For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is ≤ [12] PRBs and is down-selected among: - Alt 1: Including 180 kHz, 360 kHz, and FFS other values - Alt 2: Integer multiple(s) of 180 kHz (FFS: what integer(s)) - Alt 3: Integer multiple(s) of the subcarrier spacing (FFS: what integer(s))

In our views, for OFDM-based OOK waveform, the BLER performance increases with the increase of frequency resource allocation for each chip. Therefore, at least two candidate values of Btx,R2D should be supported and the value of transmission bandwidth (Btx,R2D) should be based on the values of M to ensure an acceptable performance . For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Alt 1: Btx,R2D including 180 kHz, 360 kHz, and 540kHz if M=32 is supported according to the simulation results in [3], should be proposed.

For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Alt 1: Btx,R2D including 180 kHz, 360 kHz, and 540kHz if M=32 is supported, should be proposed.

- For R2D OFDM-based waveform, at least two candidate values of Btx,R2D should be supported and the value of transmission bandwidth (Btx,R2D) should be based on the values of M to ensure an acceptable performance and data rate.

  - R2D coding

In RF ID system, line code is used for clock recovery and symbol boundary identification. Both Manchester and PIE are used in reader to device communication, and each of them has the pros and cons. For A-IoT R2D transmission, the mapping rule from bit to line-code codeword can be defined according to the coding scheme and code rate.

In the last RAN1 meeting [3], codewords of Manchester code and PIE with different code rates were discussed as followings.

Proposal 2.3a(II): For R2D line coding - Manchester coding is the baseline. - The study assumes the following bit to chip mapping for PIE: bit {0} → chips{10}, bit{1}→chips{1110}

For Manchester code, it contains state change (0->1, or 1->0) in each bit, which provides better detection performance. For PIE, it is beneficial for energy harvesting during R2D data transmission due to the longer high-power duration of bit-1. However, all the three A-IoT devices are equipped with energy storage. Moreover, whether the antenna for energy harvesting and R2D detection is shared requires further discussion. And due to the different symbol length for the encoded bit-0 and bit-1, the R2D data transmission is uncertain with PIE encoding.  To further investigate the R2D performance, Manchester and PIE are simulated and the performance is shown in Figure 6. The received data is detected by identifying the ascending and descending edge and the simulation assumptions are shown in Table A.1 in Annex.

According to the simulation results in Figure 6, it is observed that under AWGN and TDL-A channel model, the performance of Manchester code is better than that of PIE, i.e. almost 4~6 dB performance gains. The reason is that the Manchester code has state transition in the middle of symbol (i.e. self-clock synchronization and better minimum code distance) and has shorter transmission length (smaller average codeword length than PIE).

Based on the analysis and evaluation results, it is proposed to consider Manchester encoding for R2D transmission.

It is proposed that Manchester code is supported by R2D.

| (a) AWGN | (b) TDL-A |
| --- | --- |

Figure 6 Performance of Manchester and PIE code

Given the low complexity and low power consumption of A-IoT device, R2D with no forward error-correction code (FEC) is studied as baseline.

FEC is not applied to R2D transmission.

According to the RAN design target, the user experienced data rate is 1~5kpbs. In this case, repetition operations should be used to achieve the target data rate.

In the case of ultra-low data rate, repetition operation should be used to achieve the target data rate.

In RAN1#117 meeting [2], three repetition types were agreed to be considered for R2D study purposes.

Agreement in RAN1#117 Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times

At device side, signals are detected in time domain so that the time diversity gain is the most important to improve the R2D detection performance. By repeating the TB multiple times consecutively, each transmission encounters potentially independent fading conditions due to the time-varying nature of the wireless channel. This can result in a higher probability that at least one of the repetitions will be received with sufficient quality to correctly decode the information, even if other repetitions were impaired by deep fading or severe multipath. But for chip level and bit level repetition type, each repetition may experience similar channel conditions, reducing the benefit of time diversity gain. For block level repetition type, it is easier to guarantee different channel within the time frame of repeating block so that the detection performance is largely improved, especially for the cases of large TBS. Therefore, block level repetition is proposed for R2D study purpose.

For block level repetition type, it is easier to guarantee different channel within the time frame of repeating block so that the detection performance is largely improved under the case of larger transmission block.

Block level repetition should be considered for R2D study purposes.

  - R2D multiple access

According to the discussion in the last RAN1 meeting [3], the proposal of A-IoT DL resource scheduling schemes is as following:

Conclusion 2.6a(I): Due to the agreements in RAN1 and RAN2 related to support of slotted-ALOHA, time-domain multiple access of R2D transmissions is already supported. Proposal 2.6b(II): Readers are each treated as a separate A-IoT system from the RAN1 perspective. If FDMA of R2D is implemented by planning of the multiple systems together with the devices is an implementation matter. - Capture this in the TR. Proposal 2.6c(II): Regarding potential FDMA for R2D among different devices by one reader, i.e. within an A-IoT system: - Further study the implications of feasible reader operation if devices with RF envelope detectors are assumed to support FDMA. - FFS: For devices with IF envelope and ZIF detectors, discuss whether potential support of FDMA is compatible with the SID stipulation of “a harmonized air interface design with minimized differences (where necessary)”.

In our views, time domain scheduling by reader is relatively easy to implement for devices and can be considered as a baseline.

FDMA allows multiple users to share a communication channel by allocating distinct frequency bands to each user.

- For devices with RF envelope detectors, the bandwidth of matching network of A-IoT device can be up to tens of MHz so that it is challenging for these A-IoT devices to distinguish different frequency-domain resources if FDMA is used among different devices by one reader. Considering the available bandwidth for FDD spectrum, FDMA is challenging for RF envelope detectors.

- For devices with IF envelope and ZIF detectors, the bandwidth of matching network of A-IoT device can be narrowed to hundreds of kHz so that it is capable for these A-IoT devices to distinguish different frequency-domain resources if FDMA is used among different devices by one reader. Therefore, it is feasible to support FDMA for devices with IF envelope and ZIF detectors.

In summary, assuming devices support FDMA for R2D implicates trade-offs between spectral efficiency and device complexity. Careful system design is required to balance these factors and leverage the inherent strengths of detectors of different device types while implementing an efficient FDMA mechanism. Therefore, FDMA among different devices by reader is only feasible for devices with IF envelope and ZIF detectors. In our views, it is proposed that FDMA should not be studied in Rel-19 for A-IoT R2D.

FDMA among different devices by one reader is feasible for devices with IF envelope and ZIF detectors.

For A-IoT R2D, FDMA among different devices by one reader is not supported in Rel-19.

  - R2D time-domain definition

- Subcarrier spacing

According to the agreements in RAN1#116bis meeting [5], R2D study includes subcarrier spacing of 15 kHz, from the reader perspective. The inclusion of subcarrier spacing of 30 kHz is FFS. In our views, the inclusion of subcarrier spacing of 30 kHz depends on the operating band for A-IoT R2D transmission.

The inclusion of subcarrier spacing of 30 kHz depends on the operating band for A-IoT R2D transmission.

Agreement in RAN1#116bis meeting R2D study includes subcarrier spacing of 15 kHz, from the reader perspective, for OFDM-based waveform. - Inclusion in the study of subcarrier spacing of 30 kHz is FFS.

- Time unit(s)

According to the discussion in the last RAN1 meeting [3], proposals for the definition of time unit(s) are shown as followings.

Proposal 2.7.2a(II): In R2D, a chip: - Corresponds to one modulated symbol, e.g. according to agreed OOK modulation. - Chip duration = (1/M) × {OFDM symbol duration excluding CP part} OR {OFDM symbol duration including CP part} according to Proposal 2.1.1e. Proposal 2.7.2b(II): The smallest time unit of resource allocation in R2D is [at least] corresponding to: - Option 1: All the chips of one modulated symbol. - Option 2: One chip of a modulated symbol.

In our views, from reader perspective, the smallest time unit of DL Tc reuses the definition in NR. A chip, i.e. a line-code chip corresponding to one modulated symbol, i.e. according to OOK modulation, can be defined as the smallest unit of resource element. Also, similar to NR, for the definition of the smallest time unit of resource allocation in R2D, the duration of OFDM symbol is more appropriate than a chip.

The smallest time unit of DL Tc reuses the definition in NR from reader perspective.

The duration of OFDM symbol is proposed as the smallest time unit of resource allocation in R2D.

For the definition of chip duration, if the Nf,ref (i.e. the number of IFFT points of an OFDM symbol) can not be evenly divided by M, the chip duration defined by (1/M) × {OFDM symbol duration} is not correct. To be more specific, if the number of sampling points is the ceiling of (1/M) × {OFDM symbol's sample points excluding/including CP part}, the total of sampling points of M chips may be larger than the number of OFDM symbol's sample points excluding/including CP part. It will lead to non-orthogonality and misalignment issues. Therefore, for the definition of the smallest time unit of resource allocation, the duration of OFDM symbol or slot is more appropriate than a chip.

If the number of sampling points is the ceiling of (1/M) × {OFDM symbol's sample points excluding/including CP part}, the total of sampling points of M chips may be larger than the number of OFDM symbol's sample points excluding/including CP part. It will lead to non-orthogonality and misalignment issues.

Meanwhile, to align the understanding of chip duration at both reader and device sides, more specific chip duration under the case of different M values can be defined. For R2D chip duration definition, the following rules are listed:

- the total length of M chip(s) should be equal to the duration of OFDM symbol, i.e. the smallest time unit of resource allocation;

- under the case of larger M values (e.g. M≥24), considering CP handling, dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length.

- For example, under the case of SCS =15kHz and M=24, chip duration is 204824∙Ts=2.8 us or 204824∙Ts=2.77 us. Because CP length is almost twice of chip length, additional transition edge may be occurred during the CP duration. This rule is applied so that the chip duration is modified as each of the first 23 chips’ duration is 204824∙Ts=2.77 us and the last chip duration is (2048−23×204824)∙Ts=3 us.

- for other cases,

- if the number of IFFT points of an OFDM symbol can be evenly divided by M, chip duration = (1/M) × {OFDM symbol duration excluding CP part};

- For example, under the case of SCS =15kHz and M=2, the total length of 2 chips is equal to the duration of OFDM symbol, i.e. 66.67us and each chip duration is 1/2×66.67 =33.33 us because the number of IFFT points of an OFDM symbol, i.e. 2048, can be evenly divided by 2.

- if the number of IFFT points of an OFDM symbol cannot be evenly divided by M, chip duration = Nf,refM∙Ts or Nf,refM∙Ts, where Ts/Tc=64;

- For example, under the case of SCS =15kHz and M=12, chip duration is 204812∙Ts=5.57 us or 204812∙Ts=5.53 us because the number of IFFT points of an OFDM symbol cannot be evenly divided by 12.

- In R2D, the smallest time unit of resource allocation is a duration of OFDM symbol and the following rules can be used for R2D chip duration definition:

- the total length of M chip(s) should be equal to the duration of OFDM symbol, i.e. the smallest time unit of resource allocation;

- for the case of larger M values (e.g. M≥24), dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length;

- for other cases,

  - if the number of IFFT points of an OFDM symbol can be evenly divided by M, chip duration = (1/M) × {OFDM symbol duration excluding CP part};

  - if the number of IFFT points of an OFDM symbol can not be evenly divided by M, chip duration = Nf,refM∙Ts or Nf,refM∙Ts, where Ts/Tc=64.

- Discussion on A-IoT D2R

  - D2R waveform

In the last RAN1 meeting [3], discussion on D2R waveform is shown as following.

Proposal 3.1a(I): The D2R baseband waveform is single-carrier waveform and it can be used by all devices 1/2a/2b, i.e. it is non-OFDM based.

In our view, for A-IoT D2R, the baseband waveform design is more related to device types (i.e. device capabilities). Device 1 and device 2a only support UL transmission backscattered on CW provided externally and device 2b supports UL transmission generated internally. Moreover, CW based on single-tone waveform and multiple single-tone waveform were discussed in section 9.4.2.4. In our views, at least single-tone waveform can be used by all devices 1/2a/2b. Therefore, it is proposed that the single-tone based D2R waveforms can be used by all devices 1/2a/2b. While for multi-tone, whether it can be supported by device 2b needs further study.

Single-tone based D2R waveforms can be used by all devices 1/2a/2b. Multi-tone based D2R waveform can be used by at least device 1/2a.

  - D2R modulation

In the last RAN1 meeting [1], OOK and BPSK were agreed as feasible baseband modulation schemes for D2R for all devices and the variant of BFSK is identified as MSK and FFS: whether MSK is feasible for all devices.

In our views, to support FSK modulation, frequency shift is required by A-IoT.

- For small frequency shift by A-IoT, it is similar with line code-based subcarrier modulation. However, due to the low synchronization accuracy of A-IoT device, the spectrum efficiency should be considered. It is noted that the inaccuracy of frequency shift implemented by A-IoT device depends on the clock error (i.e., SFO), counting error (e.g., clock recovery during D2R preamble detection), frequency division error. The resultant frequency shift error can be far larger than the SFO.

- For large frequency shift with tens of MHz, it requires mixer, oscillator, and PLL/FLL. Device type 2a may support these components. However, these component requirements exceed device type 1’s capability. Therefore, we propose to clarify FSK modulation generation method and study the feasibility including spectral or resource efficiency, power consumption and complexity feasibility for the A-IoT devices.

Moreover, for MSK, because it is characterized by its continuous and smooth frequency spectrum and a requirement for phase continuity, it needs complex modulator and filter to ensure minimal frequency deviation and a constant envelope. Therefore, it is hard for all devices to support MSK because of its higher generation complexity. Moreover, the benefits and necessity to support MSK in addition to OOK and BPSK modulation should be clarified.

The benefits and necessity of supporting MSK for all devices should be clarified.

According to the discussion in the last RAN1 meeting [4], the proposal for D2R sideband is as following. For 1SB based D2R modulation, device (at least device 1/2a) needs to suppress the image frequency. In our views, 2SB is proposed for the study of D2R by considering the device capability.

Proposal 3.2.2a(I): 2SB modulation is supported for D2R transmission for all devices. - FFS if 1SB can be supported by all, or any, devices, taking account of other issue such as how to achieve small frequency shift.

  - D2R coding

    - Line coding

In the last RAN1 meeting [1], the mapping(s) from bit(s) to line-code codewords for D2R Manchester, FM0 and Miller were agreed.

For A-IoT D2R transmission, it is essential to account for the effect of sampling frequency offset (SFO) because SFO can result in uncertain symbol duration. It is easier to determine the symbol boundary by using Manchester, FM0 and Miller in the D2R transmission. Therefore, Manchester, FM0 and Miller can be studied as the starting point for A-IoT D2R transmission.

For UL transmission, sampling frequency offset (SFO) can result in uncertain symbol duration which should be considered in coding scheme design.

The simulation results of Manchester, FM0, Miller under AWGN and TDL-A channel are provided in Figure 7. The Bahl-Cocke-Jelinek-Raviv (BCJR) decoding algorithm [6] with soft input soft output (SISO) is used for simulation. The total coding rate is 1/2 and information block length is 96 for all cases. It can be observed that Manchester code has the best performance, followed by FM0, while Miller code has the poorest performance.

The performance of line code is: Manchester code > FM0 > Miller code.

| - AWGN | (b) TDL-A |
| --- | --- |

Figure 7 BLER performance of Manchester/FM0/Miller

Simulation assumptions:

|  | Configurations | Notes |
| --- | --- | --- |
| Information block length | 96 bits | BLER is computed per information block. |
| Modulation | BPSK |  |
| FM0 | k=1, n=2 | BCJR decoder (SISO) |
| Miller-M | k=1, n=2, M=1 | BCJR decoder (SISO) |

    - Small frequency shift

In last RAN1 meeting [1], the options of small frequency shift are agreed as below, wherein it also implies that small frequency shift can be potentially used for FDMA.

Agreement Small frequency shifts for D2R are studied for OOK and BPSK: - For applying with Manchester line codes - Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. - Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift. - Companies to report how they perform multiplying for option 2 - For applying with Miller line codes, according to Figure 6-13 of UHF RFID standard. - For FM0, small frequency shift is not defined - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift. - Potential purposes include: - FDMA of D2R, if supported - CW interference avoidance, if supported - Note: small frequency shifts for D2R are studied for the same potential purposes for relevant identified BFSK variant(s)

In this section, analysis on schemes that achieve small frequency shift (SFS), i.e., Manchester code option 1, Manchester code option2 and Miller code, is provided.

For Manchester code option1, each source bit is encoded as Manchester codeword and then the codeword is repeated to M times during the time duration of one source bit, as shown in Figure 8(a).

For Manchester code option2, each source bit is firstly encoding as Manchester codeword and then multiplied with a square wave, wherein the chip length of the Manchester codeword is M times that of a square wave. More specifically, the multiplying is performed as XOR operation for OOK modulation and the multiplication for BPSK modulation. Figure 8(b) illustrates the waveform of Manchester code option2. It is pointed out that, different from the Manchester code option1 by repetition, the multiplying operation in Manchester code option2 introduces flipping in the middle of one source bit duration. That is, the waveform of the second half of one source bit duration of Manchester code option2 is reverse to that of Manchester code option1.

For Miller code, each source bit is firstly encoding as Miller codeword and then multiplied with a square wave according to UHF RFID standard, as shown in Figure 9.

- SFS: Manchester code option 1

- SFS: Manchester code option 2

Figure 8 Small frequency shifts with Manchester code

Figure 9 Small frequency shifts with Miller code

In the following, we evaluate the performance of the three small frequency shift schemes. The simulation results for TBS20 over TDL-A channel with the consideration of different device number (i.e., 1/2/3/4), SFO (i.e., 0/1E4~1E5) and TO (i.e., 0/2, where the detailed explanation of TO can be found in section 3.4.3) are shown in Figure10. In the simulation, the M values of 2/4/8/16 are chosen as the candidates for different devices, so as to maintain the same occupied frequency resource among three schemes. Consequently, the filter bandwidths are also the same. It can be observed from Figure 10 that the performance of Miller code is worse than that of Manchester code even without SFO and TO, thus the Miller code should be deprioritized. As for the Manchester code, option1 has comparable performance with option2 when there is no SFO and TO. But Manchester code option1 is more susceptible to SFO and TO and has worse performance if SFO and TO are considered.

The performance of small frequency shift by Miller code is worse than that of Manchester code.

Compared with Manchester code option2, Manchester code option1 is more susceptible to SFO (and TO) and has worse performance.

- SFO = 0, TO = 0

- SFO = 1E4~1E5, TO = 0

- SFO=1E4~1E5, TO = 2

Figure 10 Simulation results for three schemes of small frequency shifts

According to the above analysis on the performance above, it can be seen that Manchester code has better performance than Miller code in the case of with/without small frequency shifts. Therefore, Manchester code Manchester should be supported for D2R and Miller should be deprioritized.

Manchester should be supported for D2R study. Miller code should be deprioritized.

Manchester code option 2 should be prioritized in D2R small frequency shift study.

    - FEC

In the last RAN1 meeting [1], D2R FEC of convolutional codes including polynomials, constraint length and mother code-rate were agreed as study and other details were FFS. The concatenation of CC and line code, constraint length and mother code rate of CC are discussed as followings.

3.3.2.1 Concatenation of CC and line code

Convolutional coding structure with a constraint length of 7(64 states) is shown in Figure 11, and 4 polynomials are g0D=1+D2+D3+D5+D6, g1D=1+D1+D2+D3+D6, g2D=1+D1+D2+D4+D6 and  g3D=1+D2+D5+D6. It includes 6 registers and 15 XOR gates. For an input bit , an output coded bits with 4 bits(,,  and ) can be obtained and the code rate is 1/4. The first 3 polynomials (g0D, g1D, g2D) in LTE are reused, as described in reference [1].

Figure 11 Convolutional coding with constraint length of 7 and mother code rate of 1/4

In our views, CC can improve the decoding performance with minimal complexity for device hardware, e.g. only several registers and XOR gates at device side as shown in Figure 11. Therefore, CC and CC combined with line codes can be considered because of the low encoding complexity and better coding gain.

Both CC and CC combined with line codes have low complexity for A-IoT D2R transmission.

We give the simulation results for A-IoT D2R transmission by using FM0, Miller and TBCC under AWGN channel. Figure 12 demonstrates BLER performances of FEC-encoded and Miller/FM0/Manchester-encoded BPSK sequences in AWGN channel. Total coding rate is 1/4 and information block length is 96 for all cases.

For A-IoT D2R, the receiver can accommodate decoding algorithms of a certain level of complexity. Therefore, the BCJR soft decoding algorithm can be employed for Miller / FM0 codes, as well as soft-input Viterbi decoding for tail-biting convolutional code.

It can be seen from Figure 12 that the concatenation of convolutional code and line code achieves performance gain of 2~5 dB, compared with Manchester code, Miller code, and FM0 code at the same coding rate.

Figure 12 BLER performance of Manchester/FM0/Miller/TBCC

Simulation assumptions

|  | Configurations | Notes |
| --- | --- | --- |
| Information block length | 96 bits | BLER is computed per information block. |
| CRC length | 16 bits |  |
| Modulation | BPSK |  |
| FM0 | k=1, n=2 | BCJR decoder (SISO) |
| Miller-M | k=1, n=2, M=2 | BCJR decoder (SISO) |
| TBCC | k=1, n=2; CC coding polynomials with g0=1338, and g1=1718; constrain length = 7 | Soft input Viterbi decoder |
| Repetition | The Repetition denotes twice of repetition for each BPSK symbol. Decoder performs soft combinations. |  |

The concatenation of convolutional code and Manchester code has best performance.

CC should be supported for all devices for D2R study.

For D2R transmission, concatenation of convolutional code and Manchester code can be considered.

3.3.2.2 Constraint length of CC

In the last RAN1 meeting [1], for D2R CC, constraint length K = 8 or K = 7 or K = 6 or K = 4 for further study were agreed.

As shown in Figure 13 and Figure 14, it compares the performance of TBCC under different constraint lengths, with TBS=96. It can be observed that an increase in the constraint length correlates with enhanced decoding performance. As shown in Figure 13 for code rate of 1/2, a constraint length of 8 is approximately 1.2 dB better than that of 4, a constraint length of 7 is about 0.4 dB better than that of 6, and a constraint length of 8 is 0.4 dB better than that of 7. As shown in Figure 14 for code rate of 1/4, a constraint length of 8 is approximately 1.5 dB better than that of 4, a constraint length of 7 is about 0.4 dB better than that of 6, and a constraint length of 8 is roughly 0.2 dB better than that of 7.

Figure 13 Performance of CC with different constraint lengths for code rate 1/2

Figure 14 Performance of CC with different constraint lengths for code rate 1/4

Simulation assumptions

|  | Configurations | Notes |
| --- | --- | --- |
| Information block length | 96 bits | BLER is computed per information block. |
| CRC length | 16 bits |  |
| Modulation | BPSK |  |
| TBCC | k=1, n=2 and 4; mother code rate = 1/2 and 1/4; constrain length = 4, 6, 7 and 8 | Soft input Viterbi decoder |

For code rate of 1/2, a constraint length of 8 is approximately 1.2 dB better than that of 4, a constraint length of 7 is about 0.4 dB better than that of 6, and a constraint length of 8 is 0.4 dB better than that of 7. For code rate of 1/4, a constraint length of 8 is approximately 1.5 dB better than that of 4, a constraint length of 7 is about 0.4 dB better than that of 6, and a constraint length of 8 is roughly 0.2 dB better than that of 7.

Convolutional code with larger constraint length can achieve better performance.

From the perspective of the encoding side, increasing the constraint length of CC coding simply requires adding a few more registers and XOR gates. For example, compared to a constraint length of 7, a constraint length of 8 in CC encoding requires an additional 1 register and several XOR gates for CC with mother code rate of 1/3 as shown in Figure 11 and Figure 15.

Compared to a constraint length of 7, a constraint length of 8 in CC encoding requires an additional 1 register and several XOR gates.

Constraint length K = 8 is proposed for D2R CC study.

3.3.2.3 Code rate of CC

In the last RAN1 meeting [1], for D2R CC, mother code-rate R = 1/6, 1/4, 1/3, 1/2 for further study were agreed. The code rate of CC is analyzed as followings.

Reducing the code rate also can improve the CC coding performance. To achieve this, it is only necessary to add a few parity streams(polynomials). For the encoding side, adding more polynomials will increase a few of XOR gates, and the increase in the number of XOR gates is linearly related to the number of polynomials added. For example, for the case of CL=7, reducing the code rate from 1/3 to 1/4 only requires 3 more XOR gates and CC with mother code rate of 1/4 and CL=7 had been evaluated as a better BLER performance in [4]. Moreover, for the case of CL=8, reducing the code rate from 1/3 to 1/4 requires 6 more XOR gates as shown in Figure 15, respectively. It is easy for devices supporting CC with lower code rate (i.e. R=1/4) to generate signal with higher code rate (i.e. R=1/3 or 1/2) by ignoring one of parity-bit branches. For the decoding side, adding more polynomials merely involves additional computation for branch metrics, which does not significantly increase complexity. Therefore, it is possible to consider a lower mother code rate than 1/3 to achieve better coding gain.

Figure 15 A convolutional coding with constraint length of 8 and mother code rate of 1/4

Reducing the mother code rate does not significantly increase the complexity of CC encoding and decoding.

As shown in Figure 16, BLER performances of TBCC with low code rates of mother code rate of 1/2, 1/3 and 1/4 with constraint length of 8 are provided. The low coding rates are 1/4, 1/5, 1/6, and 1/12, respectively. It can be observed that using a 1/4 mother code rate has better performance and using lower coding rate, i.e. 1/6 and 1/12 has better performance comparing to the cases of using mother code rate.

Figure 16 BLER performance of TBCC under cases of mother code rate of 1/2, 1/3 and 1/4

It is proposed that lower mother code rate, i.e. 1/4, can be adopted for CC coding to achieve more coding gain for A-IoT D2R.

Simulation assumptions

|  | Configurations | Notes |
| --- | --- | --- |
| Information block length | 96 bits | BLER is computed per information block. |
| CRC length | 16 bits |  |
| Modulation | BPSK |  |
| TBCC | k=1, n=2, 3 and 4; polynomials with g0=2558 , g1=3638 , g2=2278 and g3=3378; mother code rate = 1/2, 1/3 and 1/4; constrain length = 8 | Soft input Viterbi decoder For the mother code rate 1/n and CC coding rate of R=1/v, the parity streams are , . |

In LTE/NR, the MCS table with different modulation order and code rate is used for network dynamically adapt the scheduling resource according the varied channel condition. For Ambient IoT design, similar principle should be also considered. Under good channel states, coding schemes with higher code rate, i.e. 1/2, 1/3, 1/4 should be prioritized in scheduling to maximize spectrum efficiency and throughput. Conversely, in scenarios where channel conditions are worse, lower code rate, such as 1/6, 1/12 can be also considered to guarantee the D2R performance. Therefore, it is proposed that multiple code rates, i.e. 1/2, 1/3, 1/4, 1/6 and 1/12, can be adopted for CC coding to flexibly respond to varying channel states, balancing between high resource utilization and robustness.

Moreover, in the case of code rate higher than mother code rate, a simple rate matching solution is to omit one or two parity-bit branches of convolutional code. Meanwhile, to achieve lower code rate, repetition can be considered.

It is proposed that multiple code rates, i.e. 1/2, 1/3, 1/4, 1/6 and 1/12, can be adopted for CC coding to for A-IoT D2R.

To support code rate (i.e. 1/2, 1/3 ) higher than mother code rate (1/4), one or two parity-bit branches of convolutional code can be punctured for rate matching; to achieve lower code rate(i.e. 1/6, 1/12) than mother code rate(1/4), repetition can be considered.

    - Repetition

For A-IoT devices that require minimal circuitry for implementation, the operation of repetition at the physical layer is a simple and effective method to improve coverage. Repetition can be performed on a per-block basis, or it can be done on a per-bit basis. In RAN1#117 meeting [2], at least block-level and bit-level repetition type 1 and type 2 for D2R study purposes were agreed.

Agreement For D2R, study at least block-level and bit-level repetition type 1 and type 2.

Three types of repetition methods are shown as followings and the corresponding diagrams are shown in Figure 17.

- Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times

- Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times

- Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times

- Block level

- Bit level type 1

- Bit level type 2

Figure 17 Diagram of different repetition types for D2R

As shown in Figure 18, it illustrates a performance comparison between using repetition (two repetition schemes of Manchester with code rate of 1/2) and non-repetition (Manchester with a code rate of 1/2) techniques. It can be observed that, for block level and bit level type1 repetition, the performance gain is approximate 3 dB for each two times repetition under the cases of AWGN and TDL-A channel.

| (a) AWGN | (b) TDL-A |
| --- | --- |

Figure 18 Performance of Manchester with/without repetition

As shown in Figure 19, it illustrates a performance comparison between using repetition (three repetition schemes of CC coding with code rate of 1/4) and non-repetition (CC coding scheme with a code rate of 1/4) techniques. It can be observed that the performance gain of block level repetition and bit level type 2 repetition is approximate 3 dB and the performance of bit level type 1 repetition is degraded under the cases of AWGN and TDL-A channel. The performance degradation of bit level type 1 repetition may be due to the following reasons: 1) Since CC coding is not a systematic code, it cannot perform LLR combining for information repetition. 2) The longer the information length, the worse the block error rate of CC coding. Bit level type 1 repetition is equivalent to a longer information length for CC coding, so its performance is slightly inferior to the original code rate.

| (a) AWGN | (b) TDL-A |
| --- | --- |

Figure 19 BLER performance of TBCC with/without repetition

For both D2R Manchester and CC coding, block-level repetition and bit-level type 2 repetition can provide 3 dB performance gains.

Simulation assumptions

|  | Configurations | Notes |
| --- | --- | --- |
| Information block length | 96 bits | BLER is computed per information block. |
| CRC length | 16 bits |  |
| Modulation | BPSK (TBCC) OOK (Manchester) |  |
| Manchester | R=1/2 |  |
| TBCC | k=1, n=4; polynomials with g0=1338, g1=1718, g0=1658 , and g1=1178; mother code rate = 1/4; constrain length = 7. | Soft input Viterbi decoder |
| Repetition | The Repetition denotes twice of repetition for each BPSK symbol. Decoder performs soft combinations. |  |

For block-based repetition, A-IoT devices need to buffer the entire coded bits or repeatedly encode (FEC and line coding), which increases complexity.

Here, we consider a repetition method in the context of convolutional coding, as shown in Figure 20 where the repetition is 1 time. The k-th bit of the information sequence is input into the CC encoder, and output 4 coded bits(,, and ) , which are then repeated on a block basis. For example, the input information sequence with a length of K bits is , and the sequence after CC encoding and repetition operation is . Since the encoding, repetition and data transmission are carried out simultaneously, A-IoT devices do not require buffering coded bits, resulting in simpler hardware implementation.

Repeat on a block (n coded bits of the convolutional coding, n is the number of polynomials) basis, A-IoT devices do not require buffering coded bits, resulting in simpler hardware.

Figure 20 Repetition after TBCC coding

It is proposed that n coded bits of the convolutional coding are repeated for each input information bit, n is the number of polynomials, such as the output sequence after repetition is , where ,, and  are the coded bits for the k-th bit of the information sequence.

Another coverage enhancement solution is spreading code. Based on our evaluation results in section 3.4, it has similar performance with repetition and can provide benefits in D2R multiplexing.

  - D2R multiple access

In RAN1#116bis, the agreements about time/frequency/code-domain multiple access for D2R transmissions as below [5].

Agreement in RAN1#116bis Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement in RAN1#116bis Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement in RAN1#116bis Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

In this section, analysis on TDMA, FDMA and CDMA schemes are as followings.

    - TDMA

In our views, TDMA is relatively easy to be implemented by devices and can be considered as the baseline. However, due to the increase of coverage, the system capacity also needs to be expanded correspondingly to meet business needs, especially in delay sensitive scenarios. For instance, ensuring timely completion of tasks like completing the labeling and inventory of all goods transported by vehicles passing through the checkpoints. Therefore, the TDMA method may not meet the requirements and further enhancement is needed.

    - FDMA

For A-IoT devices that support FDMA by frequency shifting, spectral leakage can occur due to two factors: 1) subcarrier modulation and 2) device clock accuracy. Herein, the spectral analysis is shown in Figure 21 for the waveform by using line code. It is assumed that the target D2R link frequency is f1=160kHz, but the actual D2R link frequency is f2=128kHz considering 20% frequency inaccuracy. It is observed that it produces harmonic components. That is to say, the power of the main frequency component(s) is dispersed to the harmonic components and the corresponding signal waveform is distorted. Therefore, the impacts of spectral leakage and inaccurate frequency division should be considered for FDM-ed devices.

Figure 21 Spectral analysis on Miller code

For FDMA for A-IoT D2R, due to the harmonics and mirror image interference, and device clock accuracy, it is necessary to reserve a large guard band in frequency domain which leads to a low frequency efficiency.

Moreover, in the case of long D2R transmission supporting FDMA, complexity of implementation and frequency planning is increased. On one hand, implementing FDMA at the device side requires sophisticated filtering and modulation capabilities to ensure that signals are properly separated in the frequency domain. On the other hand, efficient use of FDM requires careful frequency planning to avoid interference with other users or systems operating in the same band. Moreover, FDMA may be less flexible than TDMA or CDMA when dealing with varying data rates or dynamic channel conditions because adjusting bandwidth allocation for different users or data streams in real-time can be challenging.

In the case of long D2R transmission supporting FDMA, complexity of implementation and frequency planning is increased a lot and FDMA may be less flexible than TDMA or CDMA when dealing with varying data rates or dynamic channel conditions because adjusting bandwidth allocation for different users or data streams in real-time can be challenging.

In addition, the performance of FDMA achieved by small frequency shift is provided in section 3.3.2. It can be observed that the number of supported devices, SFO, time offset among different devices have impact on the FDMA performance, which should be carefully studied.

Study the impact of the number of supported devices, SFO, time offset among different devices on the FDMA performance.

    - CDM

When CDM is applied to uplink, the main complexity lies in decoding at the receiver side. While the device implementation is relatively simple, with minimal impact on the device.

In addition, in order to enhance uplink coverage, time-domain extension is likely to be applied. In the scenario of coverage enhancement, CDM doesn’t introduce additional resource overhead and complexity for device and can increase uplink capacity compared to employing time-domain repetition. In the worst cases, the performance of CDM is at least better than performance for the case without CDM when two devices collided in time domain. Therefore, CDM can also be considered for uplink multiple access. In RAN1#118 meeting [3], the following proposal about CDM was discussed.

Proposal 3.6b(I): For considering feasibility and necessity of code-domain multiple access of D2R transmissions for all devices, [study OR list] at least the following aspects: - How CDMA is used for D2R transmissions carrying information in the same time-frequency resource - The impact of SFO. - The impact of timing offset between devices - Note: The timing offset can be caused by the different processing time and sampling frequency offset between devices. - The number of codes with required correlation properties in a set - Note: The corresponding code length should also be reported. - The potential gain of D2R transmission efficiency by CDMA comparing to only TDMA - Which messages of RAN2’s defined procedures CDMA could be applicable to - Impact on latency vs. the latency target due to e.g. lengths of spreading sequences - Impact of CFO for Device 2b.

In this sub-clause, the issues of CDM listed above are analyzed.

- Issue 1: time offset among devices

It is acknowledged that SFO has the most significant impacts on A-IoT D2R reception in the case of CDM. Due to the significant SFO and potential difference processing time, the time and frequency domains among different devices become asynchronous. An example of timing difference(TO) for D2R transmission is shown in Figure 22.

Figure 22 Time difference for D2R transmission

But the negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission.

The negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission.

- Issue 2: near-far effect without power control

For CDM, SIC receiver is useful to perform interference cancellation and signal recovery for a target device among UL transmissions sent by multiple devices. Because it is hard to implement power control by A-IoT device, the received power among multiple devices may be unequal which leads to near-far effect. In fact, during the SIC procedure, the greater the disparity in received power among multiple devices, the better performance will be achieved with CDM scheme. The reason is that SIC receiver leverages unequal power distribution to sequentially detect each device’s signal.

Even though there will be near-far effect without power control by A-IoT device, the greater the disparity in received power among multiple devices, the better performance will be obtained by SIC receiver with CDM-based multiple access scheme.

    - Evaluation results

- In this section, the performance of CDM and FDMA based random access are provided and compared.

- CDM

For CDM based A-IoT D2R multiple-access transmission, an A-IoT device can randomly select a spreading code from a predefined codebook. Subsequently, it maps its information bits to CDM-ed bits. The reader then utilizes the same codebook to detect the received bits and identify the A-IoT device.

Taking the Hadamard codes as an example of the codebook, the A-IoT device can map the information bits to CDM-ed bits according to the following steps:

- Step1.1: encode information bits into binary codeword bits according to line code, e.g., Manchester code.

- Step1.2: select a spreading code from the Hadamard codes. With the chosen spreading code, the A-IoT device proceeds to map each binary codeword bit into CDM-ed bits. For example, bit-0 is mapped into [0 0 1 1] and bit-1 is mapped into [1 1 0 0] if Hadamard code of [1 1 0 0] is selected.

- Step1.3: modulate the CDM-ed bits according to the modulation and waveform scheme and then transmit the modulated bits to the reader.

The reader detects the data and identifies the A-IoT device from the received signal according to the following steps:

- Step2.1: estimate the channel using the preamble and pre-process the received signal to restore the orthogonality of the spreading codes. The pre-processing can be represented as , wherein  is the estimated channel, y is the received signal,  is the signal after pre-processing;

- Step2.2: detect the data through conventional methods such as decorrelation algorithm.

- FDM

It is assumed that the FDM based A-IoT D2R multiple-access transmission is achieved by line code (such as Manchester code) with different M values to shift the transmitted signal at different frequencies. The candidate M values should be designed carefully to guarantee enough guard band and avoid interference among difference devices.

Taking 2, 4, 8 and 16 as the available M values as example, the A-IoT device can map the information bits to FDMA-ed bits according to the following steps, wherein the FDMA is achieved via the Manchester code option2 in section 3.3.2, which has the best performance among all three small frequency shift schemes.

- Step3.1: encode information bits into binary codeword bits according to line code, e.g., Manchester code.

- Step3.2: select a M value from 2/4/8/16, then map each binary codeword bit into FDM-ed signals using a square-wave with M times of the chip rate in step 3.1.

- Step3.3: modulate the FDMA-ed signals according to the modulation and waveform scheme and then transmit to the reader.

The reader detects the data and identifies the A-IoT device from the received signal according to the following steps:

- Step4.1: transform the received time-domain signal into frequency-domain, then utilize four bandpass filters to extract the potential signals, wherein the central frequency and pass band width of the four bandpass filters is related to the M values.

- Setp4.2: transform the potential signals into time-domain signals, and then attempt to detect if useful bits are carried through conventional methods such as correlation detection.

The difference between CDM and FDM based A-IoT UL multiple-access transmission is that A-IoT device selects a spreading code for CDM based multi-access, while device selects a M value for FDM based multi-access. Consequently, at the reader side, it performs time-domain detection for CDM based MA, while for FDM based MA, reader first filters out the detected signal and then perform time-domain based detection.

The differences of CDM and FDM based multiple access are as below:

- For CDM based multiple access, length-L codebook is applied at A-IOT device side, the reader directly performs time domain-based detection.

- For FDM based multiple access, line code with different M values are used, the reader first filters out the detected signal and then perform time-domain based detection.

Furthermore, the impact of SFO and TO for CDM and FDM based multiple access should also be considered. The modelling and corresponding processing of SFO and TO are detailed as follows:

For an A-IoT device, the SFO is modeled as the variance of chip length in a corresponding range. More specifically, SFO=1E3~1E4 ppm is modeled as the variance of chip length in a range of (0.1%~1%). For each A-IoT device, the SFO is randomly selected among the specific SFO range and remains constant for all chips in one D2R transmission frame.

The TO measures the timing difference between the real D2R data transmission time and target D2R data transmission time, as shown in Figure 22. In the evaluation, it is modeled as the arrival delay of the received D2R data, which is padded with noise. Considering the D2R timing definition and the resource allocation efficiency, the evaluated TO is up to 2 chip lengths, wherein the chip length is 25us. For each A-IoT device, the delay randomly determined within the given TO range in every transmission frame.

As for the processing of SFO and TO, the receiver can estimate the SFO and TO of each transmission frame by detecting its preamble and postamble, which is detailed in [7]. Then the impact of SFO and TO on the D2R transmission frame can be eliminated, and the data detection can be performed according to steps 2.1-2.2 for CDM receiver and steps 4.1-4.2 for FDMA receiver mentioned above.

3.4.4.1 Manchester code+4-lengeth Hadamard codes

In the simulation, both TBS96 with CRC-16 and TBS20 with no CRC are considered. The Manchester code of rate 1/2 is used as the line code and the 4-length Hadamard codes are used as the codebook for CDM scheme in the simulation, thus the total coding rate is 1/8. While the Manchester code of code rate 1/8 is used as the line code and the available M value of 2/4/8/16 is considered for FDMA scheme in the simulation. Besides, for the simulation of single device, the repetition scheme with Manchester code of code rate 1/8 and M =1 is evaluated to compare with the performance of CDM scheme with spreading code. Furthermore, the evaluated SFO range includes 0ppm, 1E3~1E4ppm and 1E4~1E5ppm, and the simulated of TO includes up to 0 and 1 chip lengths, wherein the chip length is considered as 25us. Last but not least, the same bandwidth for each device is assumed for the three schemes to guarantee the same data rate and spectrum efficiency for each device. However, the total bandwidth for FDM would be larger than that of CDM and TDM counterpart.

Figure 23 shows a waveform diagram for the CDM-ed and FDMA-ed signals with the same time-domain resources. Besides, the waveform for the bit level repetition scheme is also presented. It can be observed that, the waveform of CDM-ed scheme with codebook “1010” is equivalent to the line-coded waveform multiplied by a square-wave at M=1, and so is the waveform of repetition scheme.

Figure 23: waveforms for the CDM/FDMA/Repetition schemes.

The waveform of CDM-ed scheme is equivalent to the line-coded waveform multiplied by a square-wave at M=1 and the waveform of bit level repetition.

The time domain resource of FDM-ed scheme and CDM-ed scheme is the same but FDM-ed scheme requires more frequency domain resource that CDM-ed scheme.

As for the CDM/FDM-based multiple access, up to four devices are considered in the preliminary evaluation. When these multiple devices transmit data simultaneously, orthogonal spreading codes selected by A-IOT devices for CDM-based multiple access and different M values selected for FDM-based multiple access are evaluated. Besides, for the multiple devices simulations, the transmitting power of each device is varied to reflect the target SNR with a fluctuation within a 3dB range. This variance is intended to emulate the near-far effect experienced in scenarios with multiple devices

Simulation assumptions:

|  | Configurations | Notes |
| --- | --- | --- |
| Information block length | 96 bits with 16-CRC, 20 bits without CRC | BLER is computed per information block. |
| Manchester code | code rate 1/2: 0->01, 1->10 code rate 1/8: 0->01010101, 1->10101010 |  |
| CDM | 4-length Hadamard codes |  |
| FDMA | Manchester code with M=2/4/8/16 |  |
| Modulation | OOK |  |
| Waveform | single carrier |  |
| Chip length | 25us |  |
| Sampling frequency | 1.92MHz at device side | SFO may occur at device side |
| SFO | 0, 1E3~1E4 ppm, 1E4~1E5 ppm | SFO=1E3~1E4ppm or 1E4~1E5ppm is modelled as the variance of chip length in a range of ±(0.1% ~1%) or ±(1%~10%) |
| TO | up to 0, 1, 2 chip lengths | TO=1(2) is modelled as the random timing offset in a range of 0~25(50)us |
| Channel model | TDL-A |  |
| Device number | 1, 2, 3, 4 |  |
| Tx power variance at device side | Target SNR + [-3dB, 3dB] | To simulate the near-far effect of multiple devices |

Figure 24 Performance for TBS20+CRC0 under SFO=0 ppm and TO = 0/1 over TDL-A channel

Figure 25 Performance for TBS20+CRC0 under SFO=1E3~1E4 ppm and TO = 0/1 over TDL-A channel

Figure 26 Performance for TBS20+CRC0 under SFO=1E4~1E5 ppm and TO = 0/1 over TDL-A channel

The detection performance of relatively small information block (i.e., 20) over TDL-A channel are shown in Figure 24 ~ Figure 26.

Considering the simulation scenario of single device, the CDM scheme can maintain the similar performance compared to the repetition scheme, thus the 1/4 spreading coding can work as effective as the 1/4 repetition coding.

For the comparisons of CDM scheme and FDMA scheme, when the evaluated TO is 0, the CDM-ed MA has better BLER performance than FDM-ed MA under different SFO assumptions and device numbers, with a maximum performance gain of about 1.5dB. When up to one-chip TO exists, the CDM-ed MA still has comparable performance with FDM-ed MA under different SFO assumptions and device numbers. In our views, at A-IoT study stage, performance gains of FDM based multiple access should be evaluated under different scenarios.

Under the case of single user, comparing to TDM based multiple access, FDM based multiple access has 3dB performance loss.

The performance differences of CDM and FDM based multiple access are as below:

- If TO=0, CDM-ed MA has better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3).

- If TO exists, CDM-ed MA has comparable performance with FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3).

At A-IoT study stage, performance gains of FDM based multiple access should be evaluated under different scenarios.

3.4.4.2 Convolutional code+4-lengeth Hadamard codes

Besides, the convolutional codes (CC) mentioned above can also be adopted here to improve the performance by utilizing its error correcting ability. In the following simulations, the CC with 1/3 code rate and constraint length 7 is used before the Manchester encoding. The simulations also evaluate each A-IoT device to backscatter its data by both OOK modulation and BPSK modulation.

Take SFO being 1E4~1E5 ppm as an example, the performance of relatively small information block (i.e., 20) with CC and OOK modulation over TDL-A channel is shown in Figure 27, wherein the evaluated device number is 2, 3 and 4, and the TO is up to 0, 1 and 2 chip lengths. It can be figured out that with the aid of CC, the performance of MA is improved, where CDM-ed MA provides at least 0.5dB gain than that of FDM-ed MA at 10% BLER even under the assumption of SFO being 1E4~1E5 ppm, TO being 2 chip lengths and device number being 4.

Figure 27 Performance for TBS20+CRC0 with CC and OOK under SFO=1E4~1E5 ppm, TO = 0/1/2 and device number being 2/3/4 over TDL-A channel

The BLER performance of multiple access is improved due to CC.

When CC +OOK is used for D2R transmission, the BLER performance of CDM-ed MA is slightly better than that of FDM-ed MA.

With the consideration of CC and BPSK modulation, the performances of relatively small information block (i.e., 20) over TDL-A channel under the assumption of different SFO (i.e., 0 and 1E4~1E5 ppm), TO (i.e., up to 0/1/2 chip lengths) and device number (i.e., 1/2/3/4) are shown in Figure 28~Figure 30. It can be figured out that with the aid of both CC and BPSK modulation, the performance improvement of CDM-ed MA is larger than that of FDM-ed MA. More specifically, the CDM-ed MA has the performance gain of about 2~4dB at 10% BLER.

Figure 28 Performance for TBS20+CRC0 with CC and BPSK under TO = 0 and different SFO and device number over TDL-A channel

Figure 29 Performance for TBS20+CRC0 with CC and BPSK under TO = 1 and different SFO over TDL-A channel

Figure 30 Performance for TBS20+CRC0 with CC and BPSK under TO = 2 and different SFO and device number over TDL-A channel

Besides, the performance of larger TBS (i.e., TBS96+CRC-16) with CC and BPSK modulation over TDL-A channel is shown in Figure 31~Figure 33, wherein SFO being 0 and 1E4~1E5 ppm, TO being up to 0/1/2 chip lengths and simultaneous device number being 1/2/3/4 are evaluated respectively. It can be pointed out that the performance of CDM-ed MA is still better that that of FDM-ed MA for larger TBS transmission, wherein the performance gains at 10% BLER is about 1~2dB under different SFO, TO and device numbers.

Figure 31 Performance for TBS96+CRC-16 with CC and BPSK under TO = 0 and different SFO and device number over TDL-A channel

Figure 32 Performance for TBS96+CRC-16 with CC and BPSK under TO = 1 and different SFO and device number over TDL-A channel

Figure 33 Performance for TBS96+CRC-16 with CC and BPSK under TO = 2 and different SFO and device number over TDL-A channel

When CC+BPSK is used for D2R transmission, the BLER performance gain of CDM over FDM is 2~4dB for TBS20 and 1~2dB for TBS96+CRC-16.

3.4.4.3 Manchester code+64-lengeth Hadamard codes

Furthermore, the spreading codes can be optimized to improve the performance of CDM-ed MA under larger SFO, TO and simultaneous device number. One promising direction is to extend the length of spreading codes to achieve better self-correlation and lower cross-correlation property especially in the presence of SFO and TO. Considering this, the performance of CDM-ed MA with longer spreading codes is evaluated and compared with that of FDM-ed MA in the following. To be more specific, the Hadamard codes with length 64 is used in the simulation, and each chip length is set as 1.5625us to maintain the same time resource as FDM-ed MA, wherein the chip number of CDM-ed MA after 1/2 Manchester coding and 1/64 spreading coding is (TBS+CRC)*2*64, while the chip number of FDM-ed MA after 1/8 Manchester coding is (TBS+CRC)*8 and each chip length is 25us as mentioned above.

The performances for TBS20+CRC0 under the assumption of different SFO (i.e., 0 and 1E4~1E5 ppm), TO (i.e., up to 0/1/2 chip lengths) and device number (i.e., 3/4) are shown in Figure 34. Note that, no FEC is used and the modulation scheme is OOK in the simulation of Figure 34. It can be observed that with the aid of longer spreading codes, the performance of CDM-ed MA is significantly improved. Besides, the performance of CDM-ed MA is better that that of FDM-ed MA, and the performance gain is 1~2.5dB at 10% BLER.

Figure 34 performance for TBS20+CRC0 under different SFO, TO and device number over TDL-A channel

3.4.4.4 Convolutional code+16/64-lengeth Hadamard codes

Figure 35 shows the performances of longer spreading codes-aided CDM-ed MA when CC and BPSK modulation is used for D2R transmission, and also provides comparison with FDM-ed MA. As shown in Figure 28~Figure 30, CC and BPSK modulation can improved the performance of CDM-ed MA scheme by error correction ability and optimized orthogonality. Thus, in Figure 35, the performances of CDM-ed MA with spreading codes length being 16 and 64 are both provided, wherein each chip length is set as 6.25us when the Hadamard codes with length 16 is used so as to maintain the same time resource. It can be observed from the evaluation results that, with the aid of longer spreading codes, the performance of CDM-ed MA is also significantly improved for transmission with CC and BPSK modulation. Besides, the performances of CDM-ed MA with spreading code length being either 16 or 64 are better that that of FDM-ed MA, and the performance gain is 1~3dB at 10% BLER.

With the aid of longer spreading codes, the performance gain at 10% BLER of CDM-ed MA compared with those of FDM-ed MA is 1~2.5dB for TBS20 transmission with OOK and no FEC, and is 1~3dB for TBS20 transmission with CC and BPSK.

Figure 35 performance for TBS20+CRC0 with CC and BPSK under different SFO, TO and device number over TDL-A channel

3.4.4.5 Device number=6

Inspired by the performance gain of longer spreading codes-aided CDM-ed MA compared to FDM-ed MA, the performance of more simultaneous devices can be further evaluated. We increase the device number to 6, and the corresponding simulation setting is detailed as follows.

For FDM-ed MA, the 1/2 Manchester code is used wherein the chip length is set as 100us to maintain the same time resource as above simulations, that is, the duration of one source bit is 200us as illustrated in Figure 23. Under this longer chip length, larger M value is feasible. We choose M to be 2/4/8/16/32/64 so that the device number can be up to 6.

For CDM-ed MA, the 1/2 Manchester code is also used. Then the Hadamard codes with length 64 is used and the chip length is set as 1.5625us so that the duration of one source bit is also 200us (=1.5625*64*2). Optionally, the Hadamard codes with length 16 can also be used and the chip length is set as 6.25us so that the duration of one source bit is also 200us (=6.25*16*2). Each device then randomly chooses one spreading code from the Hadamard codes pool.

The performances for TBS20+CRC0 under the assumption of different SFO (i.e., 0 and 1E4~1E5 ppm) and TO (i.e., up to 0/1/2 chip lengths) are shown in Figure 36~37, wherein OOK modulation and no FEC is used in Figure 36, while CC and BPSK modulation is used in Figure 37. It can be pointed out from Figure 36 that the CDM-ed MA scheme with spreading code length being 64 has better performance than FDM-ed MA scheme when the device number increases to 6. More specifically, the performance gains at 10% BLER of CDM-ed MA compared with those of FDM-ed MA are 1~12dB. In Figure 37, the CDM-ed MA scheme with spreading code length being 16 has better performance than FDM-ed MA scheme, let alone the CDM-ed MA scheme with spreading code length being 64. The performance gains at 10% BLER of CDM-ed MA compared with those of FDM-ed MA are 1~7dB.

Besides, when increasing the device number to 6, the FDM-ed MA scheme shows significant performance degradation in the presence of SFO, which is due to the more severe main lobe aliasing among the spectrums of the more M values. Therefore, careful design of candidate M values and filtering bands is necessary for FDM-ed MA for different device numbers and chip lengths, resulting in poor generalization design across different device types and transmission scenarios.

Figure 36 performance for 6 devices with OOK and no FEC under different SFO, TO over TDL-A channel

Figure 37 performance for 6 devices with CC and BPSK under different SFO, TO over TDL-A channel

When increasing the device number to 6, the CDM-ed MA is feasible to maintain the performance by using longer spreading codes, while the FDM-ed MA shows the performance degradation in the presence of SFO.

Note that for the above simulations, the following assumption update and corresponding observations can be drawn:

- In initial access phase, the collision probability between devices will be pivoting in final performance for both FDM and CDM and it's difficult if not impossible to maintain the same level of collision probability for both schemes considering the flexibility of code domain occasions that can be provided within a feasible resource. The performance of FDM even under reasonable SFO setting is highly doubtable.

Based on the discussion and evaluation above, we have the following proposal.

For A-IoT D2R transmission, both TDMA and CDM based multiple access can be considered.

  - D2R time-domain definition

According to the discussions in the last RAN1 meeting [3], proposal on D2R time-domain definition is as following. In our views, the chip for D2R corresponds to one BPSK or OOK modulated symbol. The smallest time unit of resource allocation for D2R can the duration of a codeword, i.e. the product of the number of bits of a codeword and the chip duration.

Proposal 3.7a(II): In D2R, a chip - Corresponds to one modulated symbol - Chip duration is: - Option 1: a reference chip length corresponding to Btx,D2Rfrequency−shifting factor - The reference chip length is calculated by: 2/(Btx,D2R) for 2SB, or 1/(Btx,D2R) for 1SB, if supported - Option 2: One of a pre-defined set of pulse time durations.

The smallest time unit of resource allocation for D2R can be the duration of a codeword, i.e. the product of the number of bits of a codeword and the chip duration.

For the chip duration of D2R, because D2R transmission is based on non-OFDM waveform, the chip duration is related to the bandwidth under the case of D2R transmission without frequency shift, i.e. a chip duration=1/(Btx,D2R/2) for double sideband modulation.

For D2R, the chip duration is related to the bandwidth under the case of D2R transmission without frequency shift, i.e. a chip duration=1/(Btx,D2R/2) for double sideband modulation.

  - D2R bandwidths

According to the discussion in the last RAN1 meeting [3], the proposal for A-IoT D2R bandwidth is shown as followings.

Proposal 3.8.1a(I) For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave: - The bandwidth counts the main lobes on the two sides of one/each single-tone of a carrier-wave for DSB modulation - NOTE: Carrier-wave is internal or external to device as appropriate. - The bandwidth equals 2 / (Chip_length × Frequency-shift factor) for DSB modulation - Frequency-shift factor equals the repetition number of line code for small frequency shifting by line coding - FFS the value of frequency-shift factor for small frequency shifting by square-wave - FFS the SSB modulation case Proposal 3.8.1b(I): For the study of FDMA, Bocc,D2R of the D2R transmission associated with one/each single-tone of a carrier wave: - Bocc,D2R includes ≥99% power of the D2R transmission, with harmonics being taken into account - The guard band Bguard,D2R would be necessary due to SFO - The guard band Bguard,D2R would be necessary due to CFO for Device 2b - The guard band Bguard,D2R is around the main lobes on the two sides of one/each single-tone of a carrier-wave for DSB modulation - Bocc,D2R does not count the unoccupied bandwidth between the two main lobes - FFS the SSB modulation case Proposal 3.8.1c(I) For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier wave, it can be: - Alt 1: An integer number of PRBs - Alt 2: An integer multiple of SCS

For A-IoT D2R transmission based on single-tone and multi-tone waveform, the Btx,D2R is associated with the data rate. In RFID 18000-C specification, the data rate is in a range of 20 to 640kbps, while A-IoT D2R can be designed to support a same or larger range of data rate than RFID. Therefore, the potential values of Btx,D2R and chip duration corresponding to different data rate for the case of double sideband modulation without frequency shift are shown in Table 1. It shows that A-IoT D2R transmission can support the data rate in the range of 3.75 to 960kbps .

It is observed from Table 1 that A-IoT D2R transmission can support the data rate in the range of 3.75 to 960kbps which covers the data rate supported by RFID reader-to-tag transmission.

Combinations of Btx,D2R, chip duration and data rate for D2R in Table 1 can be used for D2R study.

Table 1 Combinations of Btx,D2R, chip duration and data rate for D2R without frequency shift

| Btx,D2R (kHz) | Chip duration (us) | Data rate w.o. coding (kbps) | Data rate w.i. 1/2 coding (kbps) | Btx,D2R (kHz) | Chip duration (us) | Data rate w.o. coding (kbps) | Data rate w.i. 1/2 coding (kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3840 | 0.521 | 1920 | 960 | 300 | 6.667 | 150 | 75 |
| 2880 | 0.694 | 1440 | 720 | 240 | 8.333 | 120 | 60 |
| 1920 | 1.042 | 960 | 480 | 180 | 11.11 | 90 | 45 |
| 1440 | 1.389 | 720 | 360 | 120 | 16.67 | 60 | 30 |
| 1200 | 1.667 | 600 | 300 | 90 | 22.22 | 45 | 22.5 |
| 960 | 2.083 | 480 | 240 | 75 | 26.67 | 37.5 | 18.75 |
| 720 | 2.778 | 360 | 180 | 60 | 33.33 | 30 | 15 |
| 600 | 3.333 | 300 | 150 | 45 | 44.44 | 22.5 | 11.25 |
| 480 | 4.167 | 240 | 120 | 30 | 66.67 | 15 | 7.5 |
| 360 | 5.556 | 180 | 90 | 15 | 133.3 | 7.5 | 3.75 |

- Discussion on R2D and D2R CRC

According to the agreements in RAN1#116bis meeting [5], other 6 bits and 16 bits CRC with different polynomials than from TS 38.212 is FFS.

Agreement in RAN1#116bis Study - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PRDCH - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PDRCH - FFS: details when different CRC lengths or no CRC may be used - FFS: other 6 bits and 16 bits CRC with different polynomials than from TS 38.212

In our views, we suppose same CRC design for both A-IoT D2R and R2D study. For CRC of A-IoT D2R study, the CRC polynomial(s) and the association between down-selected CRC(s) and message size, considering at least false-alarm rate target are discussed in this section.

It is proposed that same CRC design for both A-IoT D2R and R2D study.

CRC can detect errors in transmitted data, which can ensure data integrity during transmission. 6 bits and 16 bits CRC polynomials in TS 38.212 are shown as following.

6 bits and 16 bits CRCs in TS 38.212 - for a CRC length ; - for a CRC length .

These two CRC polynomials can only be implemented by two circuits. To be more specific, one set of circuits (6 registers and 2 XOR gates) is used for CRC-6, and another separate set of circuits (16 registers and 3 XOR gates) is used for CRC-16. That is, a total of 22 registers and 5 XOR gates are required for CRC-6 and CRC-16 in TS 38.212.

However, if two CRC polynomials are nested, it allows for the implementation of a single circuit. The following outlines the proposed polynomials for both CRC-6 and CRC-16. Specifically, the CRC-6 polynomial remains consistent with that of CRC-6 in 38.212, and the new CRC-16 incorporates CRC-6 within its structure, indicating that CRC-6 is nested within the CRC-16 polynomial.

Proposed 6 bits and 16 bits CRCs for A-IoT - for a CRC length ; - for a CRC length .

Figure 38 shows an example of implementing of CRC-6 and CRC-16 using a unified circuit. In this case, CRC-16 is calculated using the entire circuit, while CRC-6 is implemented using the part of the circuit outlined in blue. Therefore, it requires only 16 registers and 4 XOR gates for CRC-6 and CRC-16. The A-IoT implementation complexity and power consumption can be reduced with less registers and XOR gates.

Figure 38 An example of unified circuit for CRC-6 and CRC-16

When contrasting our approach with the use of CRC-6 and CRC-16 polynomials from NR, the complexity analysis is shown in the Table 2. It is observed that the complexity of the proposed design is roughly 75% that of the CRC-6 and CRC-16 in NR. For A-IoT devices with extremely stringent requirements for complexity and cost, it is worthy to consider the proposed CRC polynomials to reduce complexity, cost and power consumption.

Table 2 Complexity comparisons between proposed CRC design and the CRCs in NR

| - | Proposed CRC-6 and CRC-16 | CRC-6 and CRC-16 in TS 38.212 | Ratio |
| --- | --- | --- | --- |
| Registers | 16 | 22 | 72.73% |
| XOR | 4 | 5 | 80% |

The nested design of CRCs can reduce the complexity for A-IoT by approximately one-quarter.

The False Alarm Rate (FAR) of the CRC-6/CRC-16 are simulated. Specifically, Figure 39 presents the simulation results for a TBS of 128, while Figure 40 shows the results for a TBS of 1000. The results demonstrate that the FAR performance of new CRC-16 polynomial is equivalent to that of the CRC-16 polynomial in NR, indicating that the new design is effective for both short and long TBS.

Figure 39 FAR of CRC-16 for TBS 128

Figure 40 FAR of CRC-16 for TBS 1000

FAR performance of new designed CRC-16 polynomial is equivalent to that of the CRC-16 polynomial in NR for both short and long TBS.

Nested polynomials for 6-bit CRC and 16-bit CRC can be considered in A-IoT.

It is proposed that the following 6-bit and 16-bit CRCs are used in A-IoT:

-	 for a CRC length ;

-	 for a CRC length .

In the last RAN1 meeting [3], CRC attachment for R2D and D2R transmission was discussed as followings.

Proposal 4.1a(I): For PRDCH/PDRCH transmissions with CRC, the used CRC length depends on the number of bits Z before CRC, i.e. CRC-6 for Z<=X bits, while CRC-16 for Z > X bits - Option 1: X = 16 - Option 2: X = 24 Note: This does not preclude PRDCH/PDRCH transmissions also without CRC. Proposal 4.1b(I): For further study of possibly using no CRC in some cases: - Companies to identify potentially applicable maximum number of bits Z=Y < X - Companies to identify potentially applicable message(s)/channel type(s)

The overhead calculation formula for CRC is as follows:

It can be seen that the overhead of CRC decreases as the TBS increases. Additionally, a larger number of CRC parity bits will also increase the overhead of CRC. As shown in Figure 41, the overhead of CRC-6 and CRC-16 is presented. It can be seen that CRC-16 has higher overhead than CRC-6. Moreover, when the TBS is below approximately 56 bits, CRC-16 has overhead of more than 30%. At TBS of 16, CRC-16 has overhead of 100%. On the other hand, for CRC-6, its maximum overhead is only equal to 40%.

The CRC-16 has higher overhead than CRC-6. Maximum overhead is only 40% for CRC-6, while it is 100% for CRC-16.

Figure 41  Overheads of CRC-6 and CRC-16

Considering the overhead mentioned above, CRC-16 may not be used for very short transport blocks. For long transmission blocks, the FAR performance of CRC-6 may be high. Figure 42 shows the FAR simulation of CRC-6 , wherein the TBSs are equal to {40, 48, 56, 57, 58, 59, 64, 72, 80}. It can be observed that when TBS is less than or equal to 57, different sizes of transport blocks have almost identical FAR under the same BLER (such as BLER=0.1). However, when TBS exceeds 57, the FAR is gradually deteriorate.

For CRC-6, when TBS is less than or equal to 57, different sizes of transport blocks have almost identical FAR performance for the same BLER.

For CRC-6, when TBS exceeds 57, the FAR is gradually deteriorate.

Figure 42  FAR of CRC-6 for different TBSs

Considering the impact of transmission overhead and FAR performance, it is proposed that the used CRC length depends on the number of bits TBS before CRC, i.e. CRC-6 for TBS<=X bits, while CRC-16 for TBS > X bits, wherein X may be equal to 56 or 57.

The used CRC length depends on the number of bits TBS before CRC, i.e. CRC-6 for TBS<=X bits, while CRC-16 for TBS > X bits, wherein X is equal to 56.

When TBS is too small, the overhead of CRC-6 becomes quite significant, such as exceeding 40%. For instance, the overhead of CRC-6 is 6/15 = 40% for TBS=15. Therefore, it is recommended not to use CRC when TBS is less than or equal to 14.

It is proposed that no CRC is used for TBS<=Z bits, wherein Z is equal to 14.

From Figure 39, it can be observed that when the TBS exceeds 57 bits, CRC-16 still incurs a relatively high overhead, such as reaching approximately 20%. Moreover, such a significant overhead is highly detrimental to A-IoT. Additionally, the energy consumption for CRC processing (encoding or decoding) mainly comes from two aspects: 1) the number of registers, where the switching between on and off states of registers may consume a large amount of energy; 2) the XOR operations. Therefore, the energy consumption of CRC-6 (6 registers) will be significantly lower than that of CRC-16 (16 registers) .

For CRC-16, when TBS is less than or equal to 57, it still incurs a relatively high overhead, such as approximately 20% overhead.

The energy consumption of CRC-6 may be significantly lower than that of CRC-16.

Therefore, when the TBS is greater than X (X=56), CRC-6 may be still used. This means that the transport block needs to be divided into multiple sub-blocks, and then each sub-block is encoded with CRC-6 separately. Figure 43 shows the overhead of CRC-6 which adopts the maximum sub-block length of 56 (when TBS is greater than 56, it is divided into C sub-blocks, wherein C=ceil(TBS/56)), and the overhead of CRC-16 adopts the maximum sub-block length of 1024 (i.e., only one sub-block). It can be seen that when TBS is less than or equal to 112, CRC-6 has a lower overhead; when it is larger than 112, CRC-16 has a lower overhead.

When TBS is less than or equal to 112, CRC-6 has a lower overhead; when it is larger than 112, CRC-16 has a lower overhead.

Figure 43 Overheads comparison between CRC-6 and CRC-16

The FAR performance for TBS around 112 is shown in Figure 44, wherein CRC-6 is used for simulation and CRC information length is equal to TBS/2. It can be observed that when TBS is less than or equal to 114, the FAR performance is almost the same; and when it is larger than 114, the FAR performance gradually deteriorate.

For CRC-6 with Kcrc ≤ 57, when TBS is less than or equal to 114, the FAR performance is almost the same; when it is greater than 114, the FAR performance gradually deteriorates.

When TBS is less than or equal to 112 and larger than 56, the transport block needs to be divided into C sub-blocks (maximum block length of 56), wherein C=ceil(TBS/56), and then each sub-block is encoded with CRC-6 separately.

Figure 44 FAR performances of CRC-6 for different TBS with block segmentation

Figure 45 shows the payload sizes comparison between CRC-6 and CRC-16, wherein Manchester code is used for evaluation. In Figure 45, the horizontal axis represents the coded block length of Manchester code, and the vertical axis represents the number of information bits that can be carried in payload. It divides the transport block into two sub-blocks(wherein the maximum sub-block length is 56) for CRC-6, while it doesn’t divide for CRC-16. Due to the lower overhead of CRC-6, it is equivalent to transmitting more information bits in payload.

Figure 45  Payload sizes comparison between CRC-6 and CRC-16

- Conclusion

In this contribution, we discuss the Ambient IoT physical layer design and have the following observations and proposals.

Observation 1:	For both R2D CP handling Method Type 1 and Method Type 2, whether and how device is aware of OFDM symbol boundary depends on R2D design and device implementation.

Observation 2:	For Alt 1-2 of Method Type 2, a codeword is always set across CP so that it can ensure no additional edge introduced by CP removal as long as the edge detection error does not exceed the length of chip. However, for Alt 1 of Method Type 1, the additional transition edge introduced by CP removal cannot be completely eliminated.

Observation 3:	According to the simulation results, it is observed that, for the cases of different M configurations, Alt 1-2 of Method Type 2 has the best BLER performance among all of simulated CP handling methods except for baseline.

Observation 4:	According to the simulation results, it is observed that, for the cases of different M configurations, Alt 1 of Method Type 1 has inferior BLER performance among all of simulated CP handling methods.

Observation 5:	For Alt 1 of Method Type 1, the additional transition introduced by CP can result into large BLER performance degradation.

Observation 6:	For Alt 2 of Method Type 1, it is hard to determine an accurate CP location/length based on the transition edge and chip length during a CP.

Observation 7:	For Alt 1-1 of Method Type 2, there is a large overhead because of setting the first OOK chip(s) and the last OOK chip(s) to be the same in each OFDM symbol in order to ensure no additional transition introduced by CP.

Observation 8:	For Alt 1-2 of Method Type 2, there is no additional overhead (i.e. the first odd number of chip(s) in the first OFDM symbol can be used for preamble transmission) to ensure a transition edge occurs only at the start or only at the end of the CP.

Observation 9:	For Alt 2 of Method Type 2, the interference to NR is larger and BLER performance is inferior compared to other alternatives.

Observation 10:	According to our simulation results for different pairs of M and Btx,R2D values in [3], it is observed that some pairs of M values and the associated minimum Btx,R2D value with higher data rate have better BLER performance that that with lower data rate.

Observation 11:	In the case of ultra-low data rate, repetition operation should be used to achieve the target data rate.

Observation 12:	For block level repetition type, it is easier to guarantee different channel within the time frame of repeating block so that the detection performance is largely improved under the case of larger transmission block.

Observation 13:	FDMA among different devices by one reader is feasible for devices with IF envelope and ZIF detectors.

Observation 14:	If the number of sampling points is the ceiling of (1/M) × {OFDM symbol's sample points excluding/including CP part}, the total of sampling points of M chips may be larger than the number of OFDM symbol's sample points excluding/including CP part. It will lead to non-orthogonality and misalignment issues.

Observation 15:	For UL transmission, sampling frequency offset (SFO) can result in uncertain symbol duration which should be considered in coding scheme design.

Observation 16:	The performance of line code is: Manchester code > FM0 > Miller code.

Observation 17:	The performance of small frequency shift by Miller code is worse than that of Manchester code.

Observation 18:	Compared with Manchester code option2, Manchester code option1 is more susceptible to SFO (and TO) and has worse performance.

Observation 19:	Both CC and CC combined with line codes have low complexity for A-IoT D2R transmission.

Observation 20:	The concatenation of convolutional code and Manchester code has best performance.

Observation 21:	For code rate of 1/2, a constraint length of 8 is approximately 1.2 dB better than that of 4, a constraint length of 7 is about 0.4 dB better than that of 6, and a constraint length of 8 is 0.4 dB better than that of 7. For code rate of 1/4, a constraint length of 8 is approximately 1.5 dB better than that of 4, a constraint length of 7 is about 0.4 dB better than that of 6, and a constraint length of 8 is roughly 0.2 dB better than that of 7.

Observation 22:	Convolutional code with larger constraint length can achieve better performance.

Observation 23:	Compared to a constraint length of 7, a constraint length of 8 in CC encoding requires an additional 1 register and several XOR gates.

Observation 24:	Reducing the mother code rate does not significantly increase the complexity of CC encoding and decoding.

Observation 25:	For both D2R Manchester and CC coding, block-level repetition and bit-level type 2 repetition can provide 3 dB performance gains.

Observation 26:	Repeat on a block (n coded bits of the convolutional coding, n is the number of polynomials) basis, A-IoT devices do not require buffering coded bits, resulting in simpler hardware.

Observation 27:	For FDMA for A-IoT D2R, due to the harmonics and mirror image interference, and device clock accuracy, it is necessary to reserve a large guard band in frequency domain which leads to a low frequency efficiency.

Observation 28:	In the case of long D2R transmission supporting FDMA, complexity of implementation and frequency planning is increased a lot and FDMA may be less flexible than TDMA or CDMA when dealing with varying data rates or dynamic channel conditions because adjusting bandwidth allocation for different users or data streams in real-time can be challenging.

Observation 29:	The negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission.

Observation 30:	Even though there will be near-far effect without power control by A-IoT device, the greater the disparity in received power among multiple devices, the better performance will be obtained by SIC receiver with CDM-based multiple access scheme.

Observation 31:	The differences of CDM and FDM based multiple access are as below:

	For CDM based multiple access, length-L codebook is applied at A-IOT device side, the reader directly performs time domain-based detection.

	For FDM based multiple access, line code with different M values are used, the reader first filters out the detected signal and then perform time-domain based detection.

Observation 32:	The waveform of CDM-ed scheme is equivalent to the line-coded waveform multiplied by a square-wave at M=1 and the waveform of bit level repetition.

Observation 33:	The time domain resource of FDM-ed scheme and CDM-ed scheme is the same but FDM-ed scheme requires more frequency domain resource that CDM-ed scheme.

Observation 34:	Under the case of single user, comparing to TDM based multiple access, FDM based multiple access has 3dB performance loss.

Observation 35:	The performance differences of CDM and FDM based multiple access are as below:

	If TO=0, CDM-ed MA has better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3).

	If TO exists, CDM-ed MA has comparable performance with FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3).

Observation 36:	The BLER performance of multiple access is improved due to CC.

Observation 37:	When CC +OOK is used for D2R transmission, the BLER performance of CDM-ed MA is slightly better than that of FDM-ed MA.

Observation 38:	When CC+BPSK is used for D2R transmission, the BLER performance gain of CDM over FDM is 2~4dB for TBS20 and 1~2dB for TBS96+CRC-16.

Observation 39:	With the aid of longer spreading codes, the performance gain at 10% BLER of CDM-ed MA compared with those of FDM-ed MA is 1~2.5dB for TBS20 transmission with OOK and no FEC, and is 1~3dB for TBS20 transmission with CC and BPSK.

Observation 40:	When increasing the device number to 6, the CDM-ed MA is feasible to maintain the performance by using longer spreading codes, while the FDM-ed MA shows the performance degradation in the presence of SFO.

Observation 41:	It is observed from Table 1 that A-IoT D2R transmission can support the data rate in the range of 3.75 to 960kbps which covers the data rate supported by RFID reader-to-tag transmission.

Observation 42:	The nested design of CRCs can reduce the complexity for A-IoT by approximately one-quarter.

Observation 43:	FAR performance of new designed CRC-16 polynomial is equivalent to that of the CRC-16 polynomial in NR for both short and long TBS.

Observation 44:	The CRC-16 has higher overhead than CRC-6. Maximum overhead is only 40% for CRC-6, while it is 100% for CRC-16.

Observation 45:	For CRC-6, when TBS is less than or equal to 57, different sizes of transport blocks have almost identical FAR performance for the same BLER.

Observation 46:	For CRC-6, when TBS exceeds 57, the FAR is gradually deteriorate.

Observation 47:	For CRC-16, when TBS is less than or equal to 57, it still incurs a relatively high overhead, such as approximately 20% overhead.

Observation 48:	The energy consumption of CRC-6 may be significantly lower than that of CRC-16.

Observation 49:	When TBS is less than or equal to 112, CRC-6 has a lower overhead; when it is larger than 112, CRC-16 has a lower overhead.

Observation 50:	For CRC-6 with Kcrc ≤ 57, when TBS is less than or equal to 114, the FAR performance is almost the same; when it is greater than 114, the FAR performance gradually deteriorates.

Proposal 1:	Under the case of M≥24, adjusting the length of last chip in an OFDM symbol as the length of CP is proposed to ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP.

Proposal 2:	Alt 1-2 of Method Type2 is proposed to be used for CP handling.

Proposal 3:	Two options are proposed to reform the performance trend among different pairs of M and Btx,R2D values.

	Option 1: change the minimum Btx,R2D value for M=24 as 540kHz;

	Option 2: prudent resource allocation for the pairs of M and Btx,R2D values by implementation.

Proposal 4:	It is unnecessary to discuss 1SB and 2SB for R2D.

Proposal 5:	For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Alt 1: Btx,R2D including 180 kHz, 360 kHz, and 540kHz if M=32 is supported, should be proposed.

	For R2D OFDM-based waveform, at least two candidate values of Btx,R2D should be supported and the value of transmission bandwidth (Btx,R2D) should be based on the values of M to ensure an acceptable performance and data rate.

Proposal 6:	It is proposed that Manchester code is supported by R2D.

Proposal 7:	FEC is not applied to R2D transmission.

Proposal 8:	Block level repetition should be considered for R2D study purposes.

Proposal 9:	For A-IoT R2D, FDMA among different devices by one reader is not supported in Rel-19.

Proposal 10:	The inclusion of subcarrier spacing of 30 kHz depends on the operating band for A-IoT R2D transmission.

Proposal 11:	The smallest time unit of DL Tc reuses the definition in NR from reader perspective.

Proposal 12:	The duration of OFDM symbol is proposed as the smallest time unit of resource allocation in R2D.

Proposal 13:	In R2D, the smallest time unit of resource allocation is a duration of OFDM symbol and the following rules can be used for R2D chip duration definition:

	the total length of M chip(s) should be equal to the duration of OFDM symbol, i.e. the smallest time unit of resource allocation;

	for the case of larger M values (e.g. M≥24), dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length;

	for other cases,

  - if the number of IFFT points of an OFDM symbol can be evenly divided by M, chip duration = (1/M) × {OFDM symbol duration excluding CP part};

  - if the number of IFFT points of an OFDM symbol can not be evenly divided by M,   chip duration = Nf,refM∙Ts or Nf,refM∙Ts, where Ts/Tc=64.

Proposal 14:	Single-tone based D2R waveforms can be used by all devices 1/2a/2b. Multi-tone based D2R waveform can be used by at least device 1/2a.

Proposal 15:	The benefits and necessity of supporting MSK for all devices should be clarified.

Proposal 16:	Manchester should be supported for D2R study. Miller code should be deprioritized.

Proposal 17:	Manchester code option 2 should be prioritized in D2R small frequency shift study.

Proposal 18:	CC should be supported for all devices for D2R study.

Proposal 19:	For D2R transmission, concatenation of convolutional code and Manchester code can be considered.

Proposal 20:	Constraint length K = 8 is proposed for D2R CC study.

Proposal 21:	It is proposed that lower mother code rate, i.e. 1/4, can be adopted for CC coding to achieve more coding gain for A-IoT D2R.

Proposal 22:	It is proposed that multiple code rates, i.e. 1/2, 1/3, 1/4, 1/6 and 1/12, can be adopted for CC coding to for A-IoT D2R.

Proposal 23:	To support code rate (i.e. 1/2, 1/3 ) higher than mother code rate (1/4), one or two parity-bit branches of convolutional code can be punctured for rate matching; To achieve lower code rate(i.e. 1/6, 1/12) than mother code rate(1/4), repetition can be considered.

Proposal 24:	It is proposed that n coded bits of the convolutional coding are repeated for each input information bit, n is the number of polynomials, such as the output sequence after repetition is , where ,, and  are the coded bits for the k-th bit of the information sequence.

Proposal 25:	Study the impact of the number of supported devices, SFO, time offset among different devices on the FDMA performance.

Proposal 26:	At A-IoT study stage, performance gains of FDM based multiple access should be evaluated under different scenarios.

Proposal 27:	For A-IoT D2R transmission, both TDMA and CDM based multiple access can be considered.

Proposal 28:	The smallest time unit of resource allocation for D2R can be the duration of a codeword, i.e. the product of the number of bits of a codeword and the chip duration.

Proposal 29:	For D2R, the chip duration is related to the bandwidth under the case of D2R transmission without frequency shift, i.e. a chip duration=1/(Btx,D2R/2) for double sideband modulation.

Proposal 30:	Combinations of Btx,D2R, chip duration and data rate for D2R in Table 1 can be used for D2R study.

Proposal 31:	It is proposed that same CRC design for both A-IoT D2R and R2D study.

Proposal 32:	Nested polynomials for 6-bit CRC and 16-bit CRC can be considered in A-IoT.

Proposal 33:	It is proposed that the following 6-bit and 16-bit CRCs are used in A-IoT:

-	 for a CRC length ;

-	 for a CRC length .

Proposal 34:	The used CRC length depends on the number of bits TBS before CRC, i.e. CRC-6 for TBS<=X bits, while CRC-16 for TBS > X bits, wherein X is equal to 56.

Proposal 35:	It is proposed that no CRC is used for TBS<=Z bits, wherein Z is equal to 14.

Proposal 36:	When TBS is less than or equal to 112 and larger than 56, the transport block needs to be divided into C sub-blocks (maximum block length of 56), wherein C=ceil(TBS/56), and then each sub-block is encoded with CRC-6 separately.

- References

Draft_Minutes_report_RAN1#118_v030.

Chair notes RAN1 #117 eom4.

R1-2407249	Feature Lead Summary #1 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

R1-2406405	Discussion on general aspects of physical layer design for Ambient IoT	ZTE Corporation, Sanechips.

Draft Report of 3GPP TSG RAN WG1 #116bis v0.3.0.

Lv X, Li J.New Miller Codes for Run-Length Control in Visible Light Communications[J].IEEE Transactions on Wireless Communications, 2017, PP(99):1-1.DOI:10.1109/TWC.2017.2785398.

R1-2408068 “Discussion on frame structure and physical layer procedure for Ambient IoT”(ZTE Corporation, Sanechips)

- Annex

Table A.1: Simulation assumptions

| Parameters |  | Assumptions |
| --- | --- | --- |
| R2D/D2R common parameters |  |  |
| Carrier frequency |  | 900MHz |
| SCS |  | 15 kHz |
| Block structure |  | 20 info. bits + 6bits CRC 96 info. Bits + 16 bits CRC 400 info. Bits + 16 bits CRC |
| Channel model |  | TDL-A, TDL-D |
| Delay spread |  | 30 ns |
| Device velocity |  | 3 km/h |
| Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| BS | Number of antenna elements | 2 |
|  | Number of TXRUs | 2 |
| Intermediate UE | Number of antenna elements | 1 |
|  | Number of TXRUs | 1 |
| BLER target |  | 10%, 1% |
| Sampling frequency |  | 1.92MHz |
| R2D specific parameters |  |  |
| Transmission bandwidth |  | 180/360/540/720 kHz |
| RF-ED bandwidth |  | 180*n kHz |
| BB LPF |  | 3-order Butterworth filter with cutoff frequency at 180/360/540/720 kHz |
| Waveform |  | OOK waveform generated by OFDM modulator and N=128, N'=128, X=12 |
| Modulation |  | OOK-4 with M=1, 2, 4, 6, 8, 12, 16, 24, 32 chips per OFDM symbol |
| Line code |  | 1/2 Manchester: 0-->{01}, 1-->{10} PIE: 0-->{10}, 1-->{1110} |
| FEC |  | No FEC |
| ADC bit width |  | - bit for device 1; 4-bit for device 2 |
| D2R specific parameters |  |  |
| Transmission bandwidth (w.r.t. D2R data rate) |  | 15 kHz as baseline |
| Waveform (CW) |  | unmodulated single tone, multi-tone |
| Modulation |  | OOK, BPSK |
| Line code |  | Miller, FM0, Manchester |
| FEC |  | CC, No FEC |
| ADC bit width |  | 11-bit |