3GPP TSG-RAN WG1 Meeting #118bis	R1-2409024

Hefei, China, 14th – 18th October 2024

Agenda Item:	5

Source:	Ericsson

Title:	On RAN2 LS on data block sizes for Ambient IoT

Document for:	Discussion, Decision

# 1	Introduction

According to the LS from RAN2 [1], RAN2 would like RAN1 to provide the minimum and maximum TBS and corresponding limitations/conditions.

1. Overall Description: RAN2 have discussed the role of the MAC in handling upper-layer data blocks (MAC SDUs) and processing them into transport blocks (MAC PDUs) and the need of segmentation in RAN2. RAN2 expect to follow RAN1 conclusion and define MAC PDU sizes to align with the capacity of the physical layer/TB sizes. To take an informed decision about the need for the segmentation, RAN2 would like to know the TB sizes (in both D2R and R2D directions) RAN1 intend to specify/use. TR 38.848 and TS 22.369 indicate that a maximum message size of “approximately” 1000 bits is expected. RAN2 would like to understand what maximum size of application data could be expected in reality and if upper layers need to support segmentation to adapt their data to the maximum and typical TB sizes (both D2R and R2D directions). 2. Actions: To RAN1: RAN2 respectfully ask RAN1 to indicate what maximum and minimum TB sizes are expected to be specified in PHY, in both D2R and R2D directions and the limitations/conditions under which TBs of different sizes can be transmitted.

In this contribution, we provide some link-level evaluation results as input to the LS reply to RAN2.

# 2	Discussion

If it is concluded that it would be too challenging to transmit the entire message in a transport block when the R2D/D2R distance is up to at least 10 m with a targeted data rate of 7 kbps with a 10% BLER target, which may especially be the case if transmissions are done without much redundancy, segmentation into multiple smaller TBs can be considered.

The required maximum TB size will depend on whether messages can be segmented into multiple smaller TBs or not.

  - R2D

Evaluation results are presented for R2D message sizes of 20, 96, 400, and 1000 bits in Figure 2, Table 3, and Table 4. We simulated OOK-1, TDL-A, 2 reader Tx antennas, and multiple SFO values from 0 to 10^5 for Device 2 with 20 MHz Rx bandwidth and 4-bit ADC.

Figure 2: R2D link level results with SFO

Table 3: Link level results for R2D

|  | R2D message size (bits) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 |  | 96 |  |  | 400 |  | 1000 |
|  | SFO ≤10^3 ppm | SFO ≥10^4 ppm | SFO ≤100 ppm | SFO 10^3 ppm | SFO ≥10^4 ppm | SFO ≤10 ppm | SFO ≥100 ppm | No SFO or any SFO |
| SNR of 10% BLER (dB) | 20 | BLER floor above 10% | 22.5 | 27 | BLER floor above 10% | 37 | BLER floor above 10% | BLER floor above 10% |
| SNR of 1% BLER (dB) | 32 |  | 35 | BLER floor above 1% |  | BLER floor above 1% |  |  |

Table 4: Link budget calculation for R2D

| Message size (bits) | 20 | 96 | 400 |
| --- | --- | --- | --- |
| Residual SFO (ppm) | ≤10^3 | ≤100 | ≤10 |
| SNR of 10% BLER (dB) | 20 | 22.5 | 37 |
| R2D distance (m) | 2117 | 1627 | 354 |

R2D transmission with message size of 1000 bits (without segmentation) cannot meet a 7-kbps data rate target with 10% BLER with even without SFO.

R2D transmission with message sizes of 20, 96, and 400 bits (without segmentation) can meet a 10-m coverage requirement and a 7-kbps data rate target with 10% BLER if SFO is no larger than 10^3 ppm, 10^2 ppm, and 10 ppm, respectively.

  - D2R

Evaluation results are presented for D2R message sizes of 20, 96, and 400 bits in Figure 3 and Table 5. We simulated 5kbps data rate and multiple SFO values from 0 to 10^4 for Device 2b. Other assumptions include non-coherent OOK detection, Manchester encoding, TDL_A (30ns delay spread), sampling frequency 1.92MHz, no FEC, no CFO, and 2 reader Rx antennas.

Figure 3: D2R link level results with SFO

Table 5: Link level results for D2R

|  | D2R message size (bits) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 |  |  |  | 96 |  |  |  | 400 |  |
|  | SFO ≤2000 ppm | SFO 5000 ppm | SFO 8000 ppm | SFO 10000ppm | SFO ≤500 ppm | SFO 1000 ppm | SFO 2000 ppm | SFO ≥5000 ppm | SFO ≤100 ppm | SFO ≥500 ppm |
| SNR of 10% BLER (dB) | 11dB | 12dB | 13.5dB | 15.8dB | 13.9 dB | 14.6dB | 19.2dB | BLER floor above 10% | 16 dB | BLER floor above 10% |
| D2R distance (m) with 10% BLER | 375 | 338 | 289 | 226 | 277 | 257 | 158 |  | 222 |  |
| SNR of 1% BLER (dB) | 16dB | 17dB | 19dB | 22dB | 20.2dB | 21.2dB | 25.8dB |  | 24dB |  |
| D2R distance (m) with 1% BLER | 222 | 200 | 162 | 118 | 142 | 128 | 79 |  | 95 |  |

D2R transmission with message size of 400 bits (without segmentation) cannot meet a 5-kbps data rate target with 10% BLER with SFO ≥500 ppm.

D2R transmission with message size of 96 bits (without segmentation) cannot meet a 5-kbps data rate target with 10% BLER with SFO ≥5000 ppm.

D2R transmission with message sizes of 20, 96, and 400 bits (without segmentation) can meet 10-50 m coverage requirement and a 5-kbps data rate target with 1% BLER if SFO is no larger than 10^5 ppm, 2000 ppm, and 100 ppm, respectively.

# 3	Conclusion

In the previous sections, we made the following observations, which can be captured in a reply to RAN2.

Observation 1	The required maximum TB size will depend on whether messages can be segmented into multiple smaller TBs or not.

Observation 2	R2D transmission with message size of 1000 bits (without segmentation) cannot meet a 7-kbps data rate target with 10% BLER with even without SFO.

Observation 3	R2D transmission with message sizes of 20, 96, and 400 bits (without segmentation) can meet a 10-m coverage requirement and a 7-kbps data rate target with 10% BLER if SFO is no larger than 10^3 ppm, 10^2 ppm, and 10 ppm, respectively.

Observation 4	D2R transmission with message size of 400 bits (without segmentation) cannot meet a 5-kbps data rate target with 10% BLER with SFO ≥500 ppm.

Observation 5	D2R transmission with message size of 96 bits (without segmentation) cannot meet a 5-kbps data rate target with 10% BLER with SFO ≥5000 ppm.

Observation 6	D2R transmission with message sizes of 20, 96, and 400 bits (without segmentation) can meet 10-50 m coverage requirement and a 5-kbps data rate target with 1% BLER if SFO is no larger than 10^5 ppm, 2000 ppm, and 100 ppm, respectively.

# References

R1-2407593, “LS on data block sizes for Ambient IoT”, MediaTek, RAN1#118bis, October 2024.