3GPP TSG-RAN WG1 Meeting #118bis	R1-2409092

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.11.3

Source:	Huawei, HiSilicon

Title:	Discussion on uplink capacity/throughput enhancement for FR1-NTN

Document for:	Discussion and Decision

Introduction

In RAN#118, the follows agreements were made.

Performance of OCC Techniques: Conclusion Multiplexing of 8 UEs with PUSCH OCC is not discussed in RAN1 until the work for multiplexing of less than 8 UEs has been completed. Mapping of OCC schemes: Agreement At least one of the OCC techniques when PUSCH repetitions are used will be specified: - Inter-slot time-domain OCC with OCC length 2 - Inter-slot time-domain OCC with OCC length 2 and 4 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4 - Note: combination of techniques is not precluded - PUSCH repetition Type B is not considered

In this contribution, we provide our views on enhancing uplink capacity/throughput via employing orthogonal cover code (OCC) on PUSCH, as well as the specification aspects related to OCC techniques.

Uplink Capacity/Throughput enhancement

As defined in TS38.211, a DFT-s-OFDM PUSCH transmission includes the steps of scrambling, modulation, layer mapping, transform precoding, precoding and resource mapping. As OCC can be applied across slots and within an OFDM symbol, we separately analyse these OCC schemes in this section.

  - Inter-slot time-domain OCC

For inter-slot time-domain OCC with PUSCH repetition type A, elements of OCC sequence can be multiplied on each repetition in a slot for a PUSCH configured with repetition type A. Specifically, the group of complex-valued symbols AFTER transform precoding per repetition shall be block-wise spread with the orthogonal sequence wim according to the following equation.

zmMRBPUSCHNscRBNsymbsh+n=wimyn                                                                                    n=0,… ,MRBPUSCHNscRBNsymbsh-1                                                               m=0,… ,NSFPUSCH-1                                  (1)

where Nsymbsh is the number of DFT-s-OFDM symbols per repetition according PUSCH resource allocation in time domain. The time span of an OCC sequence is of occ-length slots. An example of the OCC across 2 slots and occ-length of NSFPUSCH = 2 is shown in Figure 1.

Figure 1 An example of inter-slot OCC with PUSCH repetition type A under occ-length=2

Observation 1: The existing resource mapping can be reused for inter-slot time-domain OCC with PUSCH repetition type A.

  - Intra-symbol pre-DFT-s OCC

In order to alleviate the negative effect due to timing and phase drift across symbols or slots, OCC can also be applied in PUSCH within an DFT-s-OFDM symbol, similar as that used in PUCCH format 4 defined in clause 6.3.2.6.3 of TS38.211. Specifically, the block of complex-valued symbols d(0), …,d(Msymb-1) for a PUSCH BEFORE transform precoding is block-wise spread according to the following equation.

xlMscPUSCH+k=wnkNSFPUSCHMscPUSCHdlMscPUSCHNSFPUSCH+k mod MscPUSCHNSFPUSCHk=0,1,…, MscPUSCH-1                                l=0,1,…,NSFPUSCHMsymbMscPUSCH-1                                            (2)

where MscPUSCH=MRBPUSCHNscRB, and NSFPUSCH is the occ-length. The time span of an OCC sequence is one DFT-s-OFDM symbol. An example of OCC scheme within an OFDM symbol with MRBPUSCH=1, and the length of OCC NSFPUSCH = 2 is shown in Figure 2.

Figure 2 An example of intra-symbol OCC with occ-length=2.

Under intra-symbol OCC, the number of subcarriers available to TB mapping is reduced to 1 over occ-length of the total allocated number. In order to keep same data rate as non-OCC scheme, gNB could either increase the MCS for a TB, or allocate more slots for a TB with TBoMS and reduce repetition number. As the OCC is spread within a DFT-s-OFDM symbol, the time domain resource mapping for PUSCH repetition type A and TBoMS can be reused.

Observation 2: Intra-symbol pre-DFT-s OCC can be applied to PUSCH with repetition type A and TBoMS. The legacy time domain resource mapping can be reused.

  - Specification aspects

RV cycling

For the transmission of PUSCH with dynamic grant or uplink grant received in a MAC RAR, the sequence of redundancy versions (RVs) is determined in clause 6.1.2.1 of TS38.214. For the transmission of PUSCH with configured grant, the sequence of RVs is determined in clause 6.1.2.3 of TS38.214.

For inter-slot OCC with PUSCH repetition type A, the RV cycling period should be increased with the OCC length in order to ensure the repetitions multiplied with same OCC sequence having same RV.

For intra-symbol OCC, legacy RV cycling can be reused.

Observation 3: RV cycling period should be aligned with the span of an OCC sequence for inter-slot time domain OCC with PUSCH repetition type A. Legacy RV cycling can be reused for intra-symbol OCC.

TBS calculation

The determination of TBS is given in clause 6.1.4.2 of TS38.214 and is shown as the following:

……. The UE shall first determine the number of REs (NRE) within the slot: - A UE first determines the number of REs allocated for PUSCH within a PRB by - , where is the number of subcarriers in the frequency domain in a physical resource block, Nsymbslotis the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH or Clause 6.1.2.3 for configured PUSCH, is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1, 0_2 or 0_3 or as described for DCI format 0_0 in Clause 6.2.2, and is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the is not configured (a value from 6, 12, or 18), the is assumed to be 0. For Msg3 or MsgA PUSCH transmission the is always set to 0. In case of PUSCH repetition Type B, is determined assuming a nominal repetition with the duration of L symbols without segmentation. - A UE determines the total number of REs allocated for PUSCH as follows - For TB processing over multiple slots, NRE=N*min156,NRE'⋅nPRB where is the total number of allocated PRBs for the UE and N is the number of slots used for TBS determination indicated by numberOfSlotsTBoMS. - Otherwise, NRE=min156,NRE'⋅nPRB. - Next, proceed with steps 2-4 as defined in Clause 5.1.3.2 - For a PUSCH scheduled by fallbackRAR UL grant, UE assumes the TB size determined by the UL grant in the fallbackRAR shall be the same as the TB size used in the corresponding MsgA PUSCH transmission. ……

For inter-slot OCC with repetition type A, the TBS determination procedure in clause 6.1.4.2 of TS38.214 can be reused because the number of REs available for TB mapping is not changed.

Since OCC within a DFT-s-OFDM symbol (for intra-symbol pre-DFT-s OCC) is equivalent to repeating the complex-valued symbols before transform precoding by the length of OCC NSFPUSCH, the number of allocated subcarriers for PUSCH should be divided by the length of OCC. Hence, the number of REs available to a PUSCH repetition in a slot (NRE), as defined in clause 6.1.4.2 of TS38.214, should also scaled down by the length of OCC when determining the TB size.

Observation 4: TBS determination procedure in clause 6.1.4.2 of TS38.214 can be reused for inter-slot time domain OCC with repetition type A. The number of available RE per slot (NRE) for TB size determination should be scaled down by occ-length for intra-symbol pre-DFT-s OCC.

UCI multiplex

Rate matching for UCI multiplexed on PUSCH is given in clause 6.3.2.4 of TS38.212, where the number of modulation symbols for HARQ-ACK transmission Q'ACK, the number of symbols for CSI part 1 transmission Q'CSI-1, and the number of symbols for CSI part 2 transmission Q'CSI-2 are respectively calculated. And the procedures to multiplex UCI on PUSCH so as to form a codeword is given in clause 6.2.7 of TS38.212.

In addition, according to clause 9 of TS38.213, if a UE transmits a PUSCH over one or more slots or multiple PUSCHs over one or more slots that are scheduled by a DCI format, and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in the one or more slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots.

For inter-slot time-domain OCC with repetition type A, since the number of REs that can be used for transmission of UCI in each OFDM symbol remains unchanged, the calculation of Q'ACK, Q'CSI-1 and Q'CSI-2, as well as the legacy procedures of multiplexing UCI on PUSCH, can be reused. However, as OCC sequence is applied on multiple slots/repetitions, the UCI should be also multiplexed on the slots/repetitions which are multiplied with the same OCC sequence in order to keep the orthogonality of OCC sequence.

gNB should consider the impact on timeline for HARQ/CSI feedback due to the time span of an OCC sequence, when the UCI is multiplexed with PUSCH spread by OCC. In addition, according to clause 9.2.6 of TS38.213, when PUCCH repetitions are overlapped with the repeated PUSCH, the overlapped PUSCH slot will be dropped, as shown in the Figure 3. For inter slot time-domain OCC with PUSCH repetition type A, the repeated PUSCHs are not necessary to be dropped as the UCI is spread across repetitions by OCC anyway.

Figure 3 An example of PUCCH and PUSCH overlapped

As shown in the following Figure 4, if the PUCCH is overlapped with the PUSCH in the first PUSCH spreading slot, the UCI can be multiplexed into PUSCH before PUSCH spreading operation is applied. If the PUCCH is overlapped with the PUSCH in the slots other than the first PUSCH slot, the PUCCH can be dropped if the timeline conditions are not satisfied. The reason is: the UCI multiplexing should be done before spreading operation applied. If the PUSCH spreading transmission has started without considering the PUCCH carrying UCI information, the later appearing UCI on PUCCH should be dropped. Otherwise, the information in different PUSCH spreading slots will be different.

Figure 4 An example of PUCCH and PUSCH overlapped

As for the intra-symbol pre-DFT-s OCC, the UCI multiplexed with a PUSCH in a slot should also be block-wise spread together with UL-SCH before transform precoding such that the number of REs l=0Nsymb,allPUSCH-1MscUCI(l) that can be used for transmission of UCI needs to be scaled down by the length of OCC. The legacy procedures of multiplexing UCI on PUSCH can be reused.

Observation 5:

- For inter-slot time domain OCC with PUSCH repetition type A, UCI should be multiplexed on every slot of the time span of an OCC sequence.

- For intra-symbol pre-DFT-s OCC, existing RE mapping for UCI multiplexing can be reused.

Observation 6: When multiplexing UCI on PUSCH, the calculation of the number of symbols for HARQ-ACK transmission, CSI part 1 transmission and CSI part 2 transmission can be reused for inter-slot time domain OCC with repetition type A. Whereas the number of REs that can be used for transmission of UCI should be scaled by occ-length for intra-symbol pre-DFT-s OCC.

Frequency hopping

As the phase continuity may not be guaranteed after frequency hopping, the time interval of inter slot frequency hopping should be extended to every OCC-length slots for inter-slot time-domain OCC with PUSCH repetition type A. And intra-slot frequency hopping should be avoided for inter-slot time domain OCC with PUSCH repetition type A.

As the OCC spreading is within an DFT-s-OFDM symbol for intra-symbol pre-DFT-s OCC, both intra-slot and inter-slot frequency hopping can be used without change.

Observation 7:

- For inter-slot time-domain OCC with PUSCH repetition type A, the time interval of inter-slot frequency hopping should be extended to a number of slots aligned with the time span of an OCC sequence, and intra-slot frequency hopping should be avoided.

- For intra symbol pre-DFT-s OCC, both intra-slot and inter-slot frequency hopping can be used without change.

Power control

In TS38.213, the power control mechanism for PUSCH is given as the following:

If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as

[dBm]

where,

……

- ∆TF,b,f,ci=10log102BPRE⋅Ks-1⋅βoffsetPUSCH for Ks=1.25 and ∆TF,b,f,ci=0 for Ks=0 where Ks is provided by deltaMCS for each UL BWP b of each carrier f and serving cell c. If the PUSCH transmission is over more than one layer [6, TS 38.214], ∆TF,b,f,ci=0. BPRE and βoffsetPUSCH, for active UL BWP b of each carrier f and each serving cell c, are computed as below

- BPRE=r=0C-1KrNRE for PUSCH with UL-SCH data and BPRE=Qm⋅RβoffsetPUSCH for CSI transmission in a PUSCH without UL-SCH data, where

- C is a number of transmitted code blocks, Kr is a size for code block r, and NRE is a number of resource elements determined as NRE=N⋅MRB,b,f,cPUSCH(i)⋅j=0Nsymb,b,f,cPUSCHi-1Nsc,dataRB(i,j), where N≥1 is provided by numberOfSlotsTBoMS as described in [6, TS 38.214] and N=1 if numberOfSlotsTBoMS is not provided, Nsymb,b,f,cPUSCHi is a number of symbols for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, Nsc,dataRB(i,j) is a number of subcarriers excluding DM-RS subcarriers and phase-tracking RS samples [4, TS 38.211] in PUSCH symbol j and assuming no segmentation for a nominal repetition in case the PUSCH transmission is with repetition Type B, 0≤j<Nsymb,b,f,cPUSCHi, and c, Kr are defined in [5, TS 38.212]

……

The potential impact on the PUSCH power control by OCC is the calculation of BPRE. For a given TB to be transmitted, the BPRE value is impacted by the TB size (r=0C-1Kr) and number of resource elements (NRE) available for TB mapping. The impact on TB size has been discussed in section 2.3.2. For inter-slot time domain OCC with PUSCH repetition type A, existing definition of NRE can be reused. For intra-symbol OCC, NRE depends on how the TDRA in DCI is interpreted.  If the TDRA is interpreted as time domain resource after  OCC, existing definition of NRE can be reused. If the TDRA is interpreted as time domain resource before OCC, then the value of NRE should be multiplied by NOCC since the actual PUSCH resource is NOCC times of the indicated one.

Observation 8: For inter-slot time-domain OCC with PUSCH repetition type A, the existing BPRE of the PUSCH power control can be reused. For intra-symbol OCC, the number of resource elements calculating BPRE for PUSCH power control should be the total resource elements after OCC spreading.

OCC indication and signaling

- In clause 6.1 of TS38.214, the scheduling of PUSCH transmission is shown as the following:

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

Under the configured grant Type 1, the PUSCH transmission is usually performed after reception of configuredGrantConfig. Thus, OCC-related parameters, such as occ-Index and occ-Length, can be included in the rrc-ConfiguredUplinkGrant within configuredGrantConfig. As for the configured grant Type 2, where the PUSCH transmission is activated by a DCI scrambled by CS-RNTI, the OCC-related parameters can be included in DCI, or configured within configuredGrantConfig/pusch-config. Similarly, for the PUSCH transmission scheduled by the UL grant in a DCI, the OCC-related parameters can be included in a DCI or configured by higher layer signaling.

Proposal 1: For PUSCH transmission corresponding to the configured grant type 1, OCC-related parameters, such as occ-Index and occ-Length, can be configured within configuredGrantConfig.

Proposal 2: For PUSCH transmission corresponding to the configured grant type 2 or dynamic granted based PUSCH, OCC-related parameters, such as occ-Index can be included in the activation/scheduling DCI, and occ-Length can be configured within configuredGrantConfig/pusch-config.

UE multiplexing

One of the justifications to introduce OCC on PUSCH in NTN scenario is due to large number of PUSCH repetitions used by UE to meet the link budget requirement. The system capacity can be improved if Rel-19 UE can be multiplexed onto the time/domain resource already allocated non-Rel-19 UE. For inter-slot time-domain OCC with PUSCH repetition type A, the Rel-19 UE can be multiplexed on the non-Rel-19 UE with PUSCH repetition type A, as long as the PUSCH repetitions are aligned in time domain and RV cycling of non-Rel-19 is switched off, e.g., by configuring repK-RV in configuredGrantConfig as {0,0,0,0} for non-Rel-19 UE. For intra-symbol pre-DFT-s OCC, it is not able to multiplex Rel-19 UE on the PUSCH repetitions of non-Rel-19 UE due to totally different resource mapping.

Observation 9: It is beneficial from system capacity perspective to support multiplexing of Rel-19 PUSCH repetitions with OCC on the time/frequency resource allocated to non-Rel-19 PUSCH repetitions (i.e., backward compatibility with non-Rel-19 UE). For inter-slot OCC with PUSCH repetition type A, it is possible to multiplex Rel-19 PUSCH with OCC on non-Rel-19 Configured Grant PUSCH with repetitions if the PUSCH repetitions are aligned and RV sequence is configured as all 0.

With respect to the combination of multiple OCC schemes, in addition to the fact that inter-slot OCC can satisfy the KPI requirements as shown in the simulation results in the following section, the combination will also significantly increase the implementation complexity and standard efforts. Thus, combination of multiple OCC scheme should not be supported in Rel-19.

Proposal 3: Only support multiplexing 2 or 4 UEs without combination of OCC schemes in Rel-19 NR-NTN.

OCC sequence design

To multiplex 2 UEs with inter slot OCC with PUSCH repetition type A, the minimum length of an OCC sequence is 2. In order to multiplex with non-Rel-19 PUSCH with repetition as discussed in section 2.3.7, there should be a row with all “1” elements. Then, two OCC sequences can be defined from the rows of matrix W2, which is equivalent to Walsh or DFT matrix of size 2.

W2=111-1                                                                        (3)

Proposal 4: For OCC length 2, support to define the OCC sequences from the rows of matrix W2=111-1.

To multiplex at most 4 UEs with inter slot OCC with PUSCH repetition type A, the minimum length of an OCC sequence is 4. For UEs configured with different OCC lengths, scheduling restrictions will occur if only UEs with the same OCC length can be multiplexed together. This issue can be solved if a row of matrix W2 is orthogonal to a subset of the rows in W4, from the rows of which the OCC sequences of length 4 are defined. It is observed that either Walsh or DFT matrix of size 4 satisfies such condition. For Walsh matrix of size 4 as shown in (4), the 1st row of W2 is orthogonal to the 2nd and 4th rows in W4, Walsh. It means UE spread with 1st row of W2 can be multiplexed with UEs spread with the 2nd or the 4th row of W4, Walsh. Similarly, UE spread with 1st row of W2 can only be multiplexed with UE spread with the 3rd row of W4, DFT as shown in (5). Alternatively, if we swap the 2nd and the 3rd column of DFT matrix as shown in (6), the flexibility of UE multiplexing can be improved.

W4, Walsh=111-1111-1111-1-1-1-11  	                                                     (4)

W4, DFT=11111j-1-j1-11-11-j-1j 		                                                  (5)

W4, permutated-DFT=11111-1j-j11-1-11-1-jj                                                                  (6)

The robustness to the phase rotation from residual CFO is another important perspective we should take into account when making the choice of OCC sequence. Although the inner product of OCC sequences is equal to 0, phase rotation introduced by residual frequency offset may destroy the orthogonality. For example, if we consider a phase rotation of 1j-1-j  on the four repeated DFT-s-OFDM symbols for inter slot OCC length-4 with PUSCH repetition type A assuming 250Hz CFO, the first row of W4, DFT rotated by linear phase 1j-1-j is exactly the same as the second row, and the inner product of the rotated first row and the second row is 4. Accordingly, the inner product of the rotated first row and the third row is 0, and the inner product of the rotated first row and the fourth row is 0. Therefore, three inner products are 4,0,0 with one maximum amplitude 4. Note that the value of inner product between the rotated uth row and the vth row falls within the range of [0, 4], for any u≠v. For the first row of W4, Walsh rotated by the same linear phase vector 1j-1-j, the inner product of the rotated first row and the second row is 0, the inner product of the rotated first row and the third row is 2+2j, and the inner product of the rotated first row and the fourth row is 2-2j. Therefore, three inner products are 0,2+2j,2-2j with amplitudes 0,22,22. The amplitudes of inner products for different spreading sequences using W4, permutated-DFT are (2,22,2), which shows that W4, permutated-DFT is more robust than the other two sequences due to lower or less peak values of inner products. Hence orthogonality of regular DFT sequences is more vulnerable to the phase shift than Walsh matrix, while permutated-DFT is better than Walsh matrix.

Proposal 5: The choice of OCC sequence of length 4 should take the following perspectives into account:

- Flexibility to multiplex UE with different OCC lengths;

- Orthogonality under residual CFO.

Proposal 6: For OCC length 4, support to define the OCC sequences from the rows of permutated DFT matrix W4=11111-1j-j11-1-11-1-jj.

Joint operation with TBoMS

TBoMS was introduced in Rel-17 to obtain the channel coding gain via transmitting a single TB across multiple slots. Although the performance gains of intra-symbol OCC can only be obtained when combined with TBoMS as shown in [1], TBoMS is not necessarily needed for inter-slot time domain OCC with PUSCH repetition type A. In addition, TBoMS is only an optional feature for coverage enhancement and relying on it will significantly compromise the flexibility and applicability of employing OCC schemes. Thus, TBoMS can be used when OCC is applied, while there is no need to bond it with OCC.

Observation 10: TBoMS is an optional feature and bonding with it will greatly limit the flexibility of employing OCC techniques.

Proposal 7: TBoMS can be used when OCC is applied, while there is no need to bond it with OCC.

  - Comparison of OCC schemes

In summary, the characteristics of OCC schemes and potential standard impacts are compared in the following table.

Table 1 Comparison of OCC schemes

| - OCC scheme | - Inter-slot OCC with PUSCH repetition type A | - Intra-symbol pre-DFT-s OCC |
| --- | --- | --- |
| - Time to perform Block spreading | - After transform precoding | - Before transform precoding |
| - Scheduling flexibility | - No restriction | - OCC length should divide allocated number of subcarriers MscPUSCH |
| - Combinations with TBoMS | - No need to bond with TBoMS | - Relying on TBoMS to obtain performance gains |
| - TBS determination | - No change | - Scale down by OCC length |
| - UCI multiplexing | - Multiplexed in all slots of a span of OCC sequence overlapping a PUCCH. - The UCI RE mapping within a slot can be reused | - Multiplexed in the slot overlap with a PUCCH (no change). - The UCI RE mapping within a slot can be reused. |
| - RV cycling | - Repetitions spread with same OCC sequence having same RV | - No change |
| - Frequency hopping | - The hopping interval of inter-slot FH should be extended to X slots, where X = OCC-length. - Intra-slot frequency hopping should be disabled | - No change |
| - Power control | - No change | - Depending on how to interpret TDRA |
| - OCC indication/configuration | - Configured within configuredGrantConfig/ pusch-config or included in DCI |  |
| - Back compatibility | - Backward compatible with non-Rel-19 UEs. | - Not backward compatible with non-Rel-19 UEs |
| - Performance | - For OCC2, similar as intra symbol with TBoMS. Better than intra symbol without TBoMS - For OCC4, satisfy the KPI requirement. Better than intra symbol without TBoMS. Slightly worse than intra symbol with TBoMS. | - For OCC2, similar as inter slot if with TBoMS. Otherwise, worse than inter slot. - For OCC4, satisfy the KPI requirement. Similar as and slightly better than inter slot if with TBoMS. Otherwise, worse than inter slot. |

By considering performance, usage scenario and standard effort of the two OCC schemes, we have following proposal.

Proposal 8: For FR1-NTN, support to only specify inter-slot time-domain OCC with PUSCH repetition type A.

Evaluation methodology and results

In our previous submission[1], we observed that inter-slot OCC and intra-symbol OCC w/o TBoMS share almost same BLER performance and aggregated throughput for both VoIP and low data rate with occ-length = 2. In this paper, we updated the simulation results for occ-length = 4 with multi-user MMSE receiver.

Since an increasing number of UEs will aggravate the inter-cell interference that is challenging to be modelled in link-level simulations, system-level simulation results are additionally provided in this section to evaluate the SINR distribution of the target UE under OCC scheme.

  - Link-level Simulation results for OCC-length=4

PUSCH for VoIP

Table 2 Comparison of KPIs under different OCC schemes for PUSCH of VoIP

| VoIP |  | baseline |  | OCC w/ TBoMS |  | OCC w/o TBoMS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SNR@2%BLER | Aggr. Thrpt. @Required SNR | SNR@2%BLER | Aggr. Thrpt. @Required SNR | SNR@2%BLER | Aggr. Thrpt. @Required SNR |
| 1T1R | Inter-slot OCC, Walsh | -5.6dB | 10.2kbps | NA | NA | -4.6dB | 39.9kbps |
|  | Intra-symbol OCC |  |  | -5.6dB | 40.7kbps | Not reach | 3kbps |
| 1T2R | Inter-slot OCC, Walsh | -9.6kB | 10.4kbps | NA | NA | -8.6dB | 41.5kbps |
|  | Intra-symbol OCC |  |  | -9.6dB | 41.6kbps | Not reach | 13.4kbps |

The performance of different OCC schemes and the corresponding aggregated throughput of four UEs are shown in Figure 5, where the vertical line denotes the required SNR (-5.66dB). It can be found that intra-symbol OCC w/o TBoMS has error floor at 30% BLER and cannot reach the requirement of 2%BLER of VoIP. Intra-symbol OCC w/ TBoMS and single UE w/o OCC share a similar performance, and slightly outperform inter-slot OCC with a gap of 1dB at SNR@2% BLER. It is also worth noting that under the antenna configuration of 1T1R, all of the schemes cannot achieve 2%BLER at the required SNR of -5.66dB. By adding one receiving antenna at satellite (i.e., 1T2R), both intra-symbol OCC w/ TBoMS and inter-slot OCC can satisfy the required SNR @2% BLER.

Observation 11: For PUSCH of VoIP with occ-length = 4, intra-symbol OCC w/o TBoMS cannot reach 2%BLER in all SNR range due to the error floor@30% BLER.

Observation 12: For PUSCH of VoIP with occ-length = 4, the SNR gap between intra-symbol OCC w/ TBoMS and inter-slot OCC is at most 1dB @2%BLER when 4 UE are multiplexed.

Observation 13: For PUSCH of VoIP with occ-length = 4, all schemes (single UE w/o OCC, intra-symbol OCC with TBoMS, inter-slot OCC) cannot achieve the 2% BLER at required SNR at cell edge of LEO600 with one receiving antenna. All of the schemes can achieve the 2% BLER at required SNR at cell edge of LEO600 with two receiving antennas.

As illustrated in Figure 5(b), when compared to the single w/o OCC scheme at the required SNR, the aggregated throughput increases from 10.2kbps to 39.9kbps under inter-slot OCC, 40.7kbps under intra-symbol OCC w/ TBoMS, and thus a 291% throughput gain, and a 299% throughput gain can be obtained, respectively. Under the antenna configuration of 1T2R, both intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a quadruple aggregated throughput as the single UE w/o OCC.

Observation 14: For PUSCH of VoIP with occ-length = 4, intra-symbol OCC with TBoMS and inter-slot OCC achieve a similar performance in terms of aggregated throughput.

- The BLER performance		(b) The aggregated throughput performance

Figure 5 The performance of different OCC schemes for PUSCH of VoIP with occ-length = 4.

In the real deployment, there are not always 4 PUSCHs to be multiplexed at the same time/frequency resource even if 4 UEs are paired together and configured with OCC-length=4. According to [2], the VoIP traffic model can be characterized by a simple 2-state voice activity model shown in Figure 6, where parameter “a” denotes the probability of transitioning from active state 1 to inactive state 0, and parameter “c” is the probability of transitioning from inactive state 0 to active state 1. A typical setting of a = 0.004 and c = 0.006 is widely used in the evaluation. Assuming 4 UEs initiating their VoIP sessions at same time (i.e., sharing the same CG-PUSCH T/F allocation) but with independent transition between active and inactive state (the transmission of CG-PUSCH is independent), the probability for different number of simultaneous CG-PUSCH transmissions, i.e., the UEs that are in active state simultaneously, within the simulation time of 120s is illustrated in Figure 7. It is observed that the probability of 4 UEs in active state simultaneously is lower than 10%.

Observation 15: For PUSCH of VoIP with occ-length = 4, the probability of 4 UEs that can be multiplexed at the same time is lower than 10%.

Figure 6 2-state voice activity model

Figure 7 Probability for different number of active UEs.

The BLER performances of different number of active UEs for intra-symbol OCC w/ TBoMS and inter-slot OCC under 1T1R are compared in Figure 8. It is noted that the performance gap between intra-symbol OCC w/ TBoMS and inter-slot OCC diminishes with the decreasing number of active UEs. When the number of active UEs is 1 and 2 (more than 60% in VoIP), intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a similar performance.

Figure 8 The BLER performance for different OCC schemes under occ length =4 with different number of active UEs for PUSCH of VoIP.

Observation 16: For PUSCH of VoIP with occ-length = 4, the performance gap between intra-symbol OCC w/ TBoMS and inter-slot OCC diminishes with the decreasing number of active UEs. When the number of active UEs is 1 or 2, intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a similar performance.

PUSCH for low data rate

Table 3 Comparison of KPIs under different OCC schemes for PUSCH of low data rate

| VoIP |  | baseline |  | OCC w/ TBoMS |  | OCC w/o TBoMS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SNR@10%BLER | Aggr. Thrpt. @Required SNR | SNR@10%BLER | Aggr. Thrpt. @Required SNR | SNR@10%BLER | Aggr. Thrpt. @Required SNR |
| 1T1R | Inter-slot OCC, Walsh | -6.5dB | 6.5kbps | NA | NA | -5.5dB | 25.3kbps |
|  | Intra-symbol OCC |  |  | -6.3dB | 25.8kbps | Not reach | 2kbps |
| 1T2R | Inter-slot OCC, Walsh | -9.7dB | 6.5kbps | NA | NA | -8.9dB | 26kbps |
|  | Intra-symbol OCC |  |  | -9.6dB | 26kbps | Not reach | 2.7kbps |

The performance of different OCC schemes and the corresponding aggregated throughput under occ-length = 4 is shown Figure 9. It can be observed that the target BLER can be satisfied at the required SNR for intra-symbol OCC w/ TBoMS and inter-slot OCC under the antenna configuration of both 1T1R and 1T2R. And similar as the PUSCH of VoIP, intra-symbol OCC w/o TBoMS cannot achieve 10% BLER in all SNR range due to the error floor even under the antenna configuration of 1T2R.

Observation 17: For PUSCH of low data rate with occ-length = 4, intra-symbol OCC w/o TBoMS cannot achieve 10%BLER at cell edge of LEO600.

Observation 18: For PUSCH of low data rate with occ-length = 4, both intra-symbol OCC w/ TBoMS and inter-slot OCC can satisfy the KPI requirement under the antenna configuration of 1T1R and 1T2R at cell edge of LEO600. The SNR gap @10%BLER is at most 0.8dB when 4 UEs are multiplexed.

When compared to the scheme of single UE w/o OCC, the aggregated throughput increases from 6.5kbps to 25.3kbps under inter-slot OCC, 25.8kbps under intra-symbol OCC w/ TBoMS, and thus a 289% throughput gain, and a 297% throughput gain can be obtained, respectively. When it comes to antenna configuration of 1T2R, similar as PUSCH for VoIP, both intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a quadruple aggregated throughput as the single UE w/o OCC.

Observation 19: For PUSCH of low data rate with occ-length = 4, intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a similar performance in terms of aggregated throughput.

- The BLER performance		(b) The aggregated throughput performance

Figure 9 The performance of different OCC schemes for PUSCH with low data rate and with occ-length = 4.

Similar as VoIP service, there also exists the scenario where not all 4 UEs simultaneously transmit information even under occ-length = 4 for low data rate service. The BLER performance of different number of active UEs for intra-symbol OCC w/ TBoMS and inter-slot OCC is shown in Figure 10. It can be similarly observed that the performance gap between intra-symbol OCC w/ TBoMS and inter-slot OCC diminishes with the decreasing number of active UEs. When the number of active UEs is 1 and 2, intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a similar performance.

Figure 10 The BLER performance for different OCC schemes under occ-length = 4 with different number of active UEs for PUSCH of low data rate.

Observation 20: For PUSCH of low data rate with occ-length = 4, the performance gap between intra-symbol OCC w/ TBoMS and inter-slot OCC diminishes with the decreasing number of active UEs. When the number of active UEs is 1 or 2, intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a similar performance.

Simulation results for OCC sequences

The BLER performances of different OCC sequences under inter-slot OCC length-4 are compared for VoIP and low data rate services, respectively. It is observed in Figure 11 that permutated DFT sequence is better than Walsh sequence by SNR gain of 0.4dB @2%BLER for VoIP and SNR gain of 0.1dB @10%BLER for low data rate, and much better than DFT sequence by SNR gain of 4.7dB @2%BLER for PUSCH with VoIP and SNR gain of 1.0dB @10%BLER for PUSCH with low data rate. Hence permutated DFT shall be adopted for inter-slot OCC.

Observation 21: For inter-slot OCC length-4, permutated DFT sequence is better than Walsh sequence, and much better than DFT sequence.

- PUSCH with VoIP				(b) PUSCH with low data rate

Figure 11 The BLER performance of difference OCC sequences under inter-slot OCC length-4.

For both grant-based and grant-free services, due to practical scheduling restrictions and UE traffic, the number of practically paired UEs is not necessarily equal to the maximum multiplexing capability (i.e. configured OCC length). The BLER performances of difference OCC schemes are compared under OCC length - 4 of 1 to 4 paired UEs for PUSCH with VoIP. It is observed in Figure 12 that for 3 and 4 paired UEs, inter-slot OCC under permutated DFT sequence is worse than intra-symbol OCC by SNR loss of 0.3dB and 0.6dB @2%BLER, respectively. For 2 paired UEs and single UE, inter-slot OCC is approximate to intra-symbol OCC @2%BLER. It is worth noting that different from intra-symbol OCC with TBoMS, inter-slot OCC does not necessarily need TBoMS, hence making early decoding at base station possible which facilitates energy saving.

Observation 22: For PUSCH of VoIP, the performance gap between intra-symbol OCC and inter-slot OCC can be further reduced if permutated DFT sequence is employed for 4 and 3 paired UEs. For 2 and 1 paired UEs, inter-slot OCC and intra-symbol OCC achieve a similar performance.

Table 4 Comparison of SNR@2%BLER for different OCC schemes for PUSCH of VoIP

| # of active UEs | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Inter-slot OCC, Walsh | -5.6dB | -5.5dB | -5.1dB | -4.6dB |
| Inter-slot OCC, Perm.DFT | -5.6dB | -5.5dB | -5.3dB | -5.0dB |
| Intra-symbol OCC | -5.6dB | -5.6dB | -5.6dB | -5.6dB |

Figure 12 The BLER performances of different OCC schemes under OCC length - 4 of 1 to 4 paired UEs for PUSCH with VoIP.

  - System-level Simulation results

System-level simulation results are provided to evaluate the impact of OCC scheme on SINR distribution of the target UE, whose evaluate angle ranges from 30° to 90°. SINR of the target UE is assumed to be free from the intra-cell interference, i.e. not influenced by UEs that are multiplexed onto the same tine/domain resource under the OCC scheme, but is affected by the inter-cell interference from UEs that are multiplexed onto the same tine/domain resource w/o OCC scheme. Simulation methodology is similar as that used in TS38.821, and key simulation parameters are listed in Appendix B.

The SINR distribution of the target UE with 2 PRBs (PUSCH for VoIP) is illustrated in Figure 13. It can be observed in Figure 13 that the SINR of the target UE under the interference of 4 UEs in each beam ranges from -12.1dB to -4.0dB , and SINR@50% is –7dB, which is much smaller than the required SNR (-5.66dB).

Figure 13 SINR distribution for PUSCH of VoIP

The SINR distribution of the target UE with 1 PRB (PUSCH for low data rate) is demonstrated in Figure 14. Similar as the PUSCH for VoIP, it is noted that SINR distribution under PUSCH for low data rate is also within a low SINR regime compared to the SNR requirement (-2.7dB) as discussed in 3.1.2. Thus, a large number of repetitions is also required for PUSCH of low data rate.

FR1

Figure 14 SINR distribution for PUSCH of low data rate

Observation 23: The SINR range of OCC4 is -12.1dB ~-4.0dB and -12.1dB ~-2.0dB for VoIP and low data rate, respectively.

Conclusions

In this contribution, enhancements on PUSCH via OCC are discussed. The proposals and observations are summarized as below:

Observation 1: The existing resource mapping can be reused for inter-slot time-domain OCC with PUSCH repetition type A.

Observation 2: Intra-symbol pre-DFT-s OCC can be applied to PUSCH with repetition type A and TBoMS. The legacy time domain resource mapping can be reused.

Observation 3: RV cycling period should be aligned with the span of an OCC sequence for inter-slot time domain OCC with PUSCH repetition type A. Legacy RV cycling can be reused for intra-symbol OCC.

Observation 4: TBS determination procedure in clause 6.1.4.2 of TS38.214 can be reused for inter-slot time domain OCC with repetition type A. The number of available RE per slot (NRE) for TB size determination should be scaled down by occ-length for intra-symbol pre-DFT-s OCC.

Observation 5:

- For inter-slot time domain OCC with PUSCH repetition type A, UCI should be multiplexed on every slot of the time span of an OCC sequence.

- For intra-symbol pre-DFT-s OCC, existing RE mapping for UCI multiplexing can be reused.

Observation 6: When multiplexing UCI on PUSCH, the calculation of the number of symbols for HARQ-ACK transmission, CSI part 1 transmission and CSI part 2 transmission can be reused for inter-slot time domain OCC with repetition type A. Whereas the number of REs that can be used for transmission of UCI should be scaled by occ-length for intra-symbol pre-DFT-s OCC.

Observation 7:

- For inter-slot time-domain OCC with PUSCH repetition type A, the time interval of inter-slot frequency hopping should be extended to a number of slots aligned with the time span of an OCC sequence, and intra-slot frequency hopping should be avoided.

- For intra symbol pre-DFT-s OCC, both intra-slot and inter-slot frequency hopping can be used without change.

Observation 8: For inter-slot time-domain OCC with PUSCH repetition type A, the existing BPRE of the PUSCH power control can be reused. For intra-symbol OCC, the number of resource elements calculating BPRE for PUSCH power control should be the total resource elements after OCC spreading.

Observation 9: It is beneficial from system capacity perspective to support multiplexing of Rel-19 PUSCH repetitions with OCC on the time/frequency resource allocated to non-Rel-19 PUSCH repetitions (i.e., backward compatibility with non-Rel-19 UE). For inter-slot OCC with PUSCH repetition type A, it is possible to multiplex Rel-19 PUSCH with OCC on non-Rel-19 Configured Grant PUSCH with repetitions if the PUSCH repetitions are aligned and RV sequence is configured as all 0.

Observation 10: TBoMS is an optional feature and bonding with it will greatly limit the flexibility of employing OCC techniques.

Observation 11: For PUSCH of VoIP with occ-length = 4, intra-symbol OCC w/o TBoMS cannot reach 2%BLER in all SNR range due to the error floor@30% BLER.

Observation 12: For PUSCH of VoIP with occ-length = 4, the SNR gap between intra-symbol OCC w/ TBoMS and inter-slot OCC is at most 1dB @2%BLER when 4 UE are multiplexed.

Observation 13: For PUSCH of VoIP with occ-length = 4, all schemes (single UE w/o OCC, intra-symbol OCC with TBoMS, inter-slot OCC) cannot achieve the 2% BLER at required SNR at cell edge of LEO600 with one receiving antenna. All of the schemes can achieve the 2% BLER at required SNR at cell edge of LEO600 with two receiving antennas.

Observation 14: For PUSCH of VoIP with occ-length = 4, intra-symbol OCC with TBoMS and inter-slot OCC achieve a similar performance in terms of aggregated throughput.

Observation 15: For PUSCH of VoIP with occ-length = 4, the probability of 4 UEs that can be multiplexed at the same time is lower than 10%.

Observation 16: For PUSCH of VoIP with occ-length = 4, the performance gap between intra-symbol OCC w/ TBoMS and inter-slot OCC diminishes with the decreasing number of active UEs. When the number of active UEs is 1 or 2, intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a similar performance.

Observation 17: For PUSCH of low data rate with occ-length = 4, intra-symbol OCC w/o TBoMS cannot achieve 10%BLER at cell edge of LEO600.

Observation 18: For PUSCH of low data rate with occ-length = 4, both intra-symbol OCC w/ TBoMS and inter-slot OCC can satisfy the KPI requirement under the antenna configuration of 1T1R and 1T2R at cell edge of LEO600. The SNR gap @10%BLER is at most 0.8dB when 4 UEs are multiplexed.

Observation 19: For PUSCH of low data rate with occ-length = 4, intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a similar performance in terms of aggregated throughput.

Observation 20: For PUSCH of low data rate with occ-length = 4, the performance gap between intra-symbol OCC w/ TBoMS and inter-slot OCC diminishes with the decreasing number of active UEs. When the number of active UEs is 1 or 2, intra-symbol OCC w/ TBoMS and inter-slot OCC achieve a similar performance.

Observation 21: For inter-slot OCC length-4, permutated DFT sequence is better than Walsh sequence, and much better than DFT sequence.

Observation 22: For PUSCH of VoIP, the performance gap between intra-symbol OCC and inter-slot OCC can be further reduced if permutated DFT sequence is employed for 4 and 3 paired UEs. For 2 and 1 paired UEs, inter-slot OCC and intra-symbol OCC achieve a similar performance.

Observation 23: The SINR range of OCC4 is -12.1dB ~-4.0dB and -12.1dB ~-2.0dB for VoIP and low data rate, respectively.

Proposal 1: For PUSCH transmission corresponding to the configured grant type 1, OCC-related parameters, such as occ-Index and occ-Length, can be configured within configuredGrantConfig.

Proposal 2: For PUSCH transmission corresponding to the configured grant type 2 or dynamic granted based PUSCH, OCC-related parameters, such as occ-Index can be included in the activation/scheduling DCI, and occ-Length can be configured within configuredGrantConfig/pusch-config.

Proposal 3: Only support multiplexing 2 or 4 UEs without combination of OCC schemes in Rel-19 NR-NTN.

Proposal 4: For OCC length 2, support to define the OCC sequences from the rows of matrix W2=111-1.

Proposal 5: The choice of OCC sequence of length 4 should take the following perspectives into account:

- Flexibility to multiplex UE with different OCC lengths;

- Orthogonality under residual CFO.

Proposal 6: For OCC length 4, support to define the OCC sequences from the rows of permutated DFT matrix W4=11111-1j-j11-1-11-1-jj.

Proposal 7: TBoMS can be used when OCC is applied, while there is no need to bond it with OCC.

Proposal 8: For FR1-NTN, support to only specify inter-slot time-domain OCC with PUSCH repetition type A.

Proposal 9: For PUSCH of VoIP and PUSCH of low data rate, since the SINR distribution of the target UE under the inter-cell interference can hardly satisfy the SNR requirement, especially under occ-length =4, a larger number of repetitions is typically required.

- References

R1-2405550, “Discussion on uplink capacity/throughput enhancement for FR1-NTN,” Huawei， HiSilicon.

R1-070674, “LTE physical layer framework for performance verification”.

- Appendix A: Link level evaluation assumption

Table 5 Common simulation assumption

|  | Parameter | Value |
| --- | --- | --- |
| Scenario | Orbit | LEO600 |
|  | Elevation angle | 30 degree |
| Channel and impairments | carrier frequency | 2GHz |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |
|  | Frequency error | Random selection from [-0.1 ppm, +0.1 ppm] for UEs Variation of frequency error is negligible. |
|  | Timing error | Random selection from [-29Ts, +29Ts] for UEs Timing drift is negligible. |
| Transmitter | SCS | 15KHz |
|  | Waveform | DFT-s-OFDM |
|  | Frequency hopping | w/o frequency hopping |
|  | MIMO scheme | 1T1R, 1T2R |
|  | OCC sequence | Table 6.3.2.6.3-1/2 in TS38.211 for intra symbol OCC |
|  | Velocity of UE | 3km/h |
| Receiver | Receiver algorithm | MMSE |

Table 6 Simulation assumption for PUSCH with VoIP

|  | Parameter |  | Value |
| --- | --- | --- | --- |
| Transmitter | Number of PRBs |  | 2 |
|  | Repetition scheme |  | Total slots Nrep = 20 with repetition type A repetition |
|  | DMRS configuration |  | DMRS for PUSCH with transform precoding DMRS positions for single-symbol DMRS for PUSCH mapping type A defined in Table 6.4.1.1.3 with ld=14, l0=2 and pos1 in [38.211]. |
|  | DMRS port |  | 1000, 1001, 1002, 1003 |
|  | PUSCH |  | PUSCH mapping type A with 12 OS |
|  | Inter-slot OCC, intra-symbol OCC w/ TBoMS | TBS | 208bits |
|  |  | MCS | MCS 11 in MCS Table 6.1.4.1-2 in [TS38.214] |
|  | Intra-symbol OCC w/o TBoMS | TBS | 224bits (occ-length = 4) |
|  |  | MCS | MCS 23(occ-length = 4) in MCS Table 6.1.4.1-2 in [TS38.214] |
|  | OCC length |  | 4 |
|  | Number of UEs |  | 4 with same transmit power |
| KPI | BLER per UE |  | SNR @2% BLER |
|  | Aggregated throughput |  | Total throughout of 4 UEs multiplexed |

Table 7 Simulation assumption for PUSCH with low data rate

|  | Parameter |  | Value |
| --- | --- | --- | --- |
| Transmitter | Number of PRBs |  | 1 |
|  | Repetition scheme |  | Total slots Nrep = 16 with repetition type A repetition |
|  | DMRS configuration |  | DMRS for PUSCH with transform precoding DMRS positions for single-symbol DMRS for PUSCH mapping type A defined in Table 6.4.1.1.3 with ld=14, l0=2 and pos1 in [38.211]. |
|  | DMRS port |  | 1000, 1001, 1002, 1003 |
|  | PUSCH |  | PUSCH mapping type A with 12 OS |
|  | Inter-slot OCC, intra-symbol OCC w/ TBoMS | TBS | 104bits |
|  |  | MCS | MCS 5 in MCS Table 6.1.4.1-1 in [TS38.214] |
|  | Intra-symbol OCC w/O TBoMS | TBS | 96bits (occ-legnth = 4) |
|  |  | MCS | 17 (occ-legnth = 4) in MCS Table 6.1.4.1-1 in [TS38.214] |
|  | Number of UEs |  | 4 with same transmit power |
| KPI | BLER per UE |  | SNR @10% BLER |
|  | Aggregated throughput |  | Total throughout of 4 UEs multiplexed |

- Appendix B: System level evaluation assumption

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2GHz |
| Satellite altitude | 600km |
| Satellite Rx antenna gain | 30dBi |
| Beam diameter | 50km |
| Adjacent beam spacing on UV | 0.0668 |
| UE Tx transmit power | 23dBm |
| UE Tx antenna gain | -5.5dBi |
| Noise Figure | 5dB |
| Ambient Temperature | 290T |
| Rx G/T | 1.1dB/T |
| Shadowing loss | 3dB |
| Scintillation Loss | 2.2dB |
| Polarization loss | 3dB |
| Atmospheric loss | 0.1dB |
| Total number of beam footprints | 511 |
| Beams of interest | 19 |