3GPP TSG RAN WG1 #118-bis                                                                                R1- 24xxxxx

Hefei, China, October 14th – October 18th, 2024

Agenda item:	9.1.4.1

Source: 	Moderator (Qualcomm)

Title: 		Draft summary of Additional study on AI/ML for NR air interface: CSI compression

Document for:	Discussion and Decision

# Introduction

In RAN#102, a new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The WID includes study objectives related to AI/ML for CSI compression using a two-sided model.

…… Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - …… ……

In RAN#105, the WID was revised as follows

… Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. …

This document summarizes the issues regarding agenda item 9.1.4.1 (Additional study on AI/ML for NR air interface: CSI compression) in RAN#118-bis.

## Contact Information

Please provide / update your contact information.

| Company | Name | E-mail |
| --- | --- | --- |
| Moderator (Qualcomm) | Taesang Yoo | taesangy@qti.qualcomm.com |
| Lenovo | Vahid Pourahmadi | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| Indian Institute of Technology Madras | Anil Kumar Yerrapragada | anilkumar@5gtbiitm.in |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| vivo | Peng SUN | sunpeng@vivo.com |
| SK telecom | Yunesung Kim | yunesung.kim@sktelecom.com |
| Mavenir | Ali Fatih Demir | ali.demir@mavenir.com |
| Panasonic | Tetsuya Yamamoto Hidetoshi Suzuki | yamamoto.tetsuya001@jp.panasonic.com suzuki.hidetoshi@ jp.panasonic.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| NTT DOCOMO | Xin Wang Haruhi Echigo | wangx@docomolabs-beijing.com.cn haruhi.echigo.fw@nttdocomo.com |
| ETRI | Anseok Lee | alee@etri.re.kr |
| Ericsson | Jingya Li Siva Muruganathan Jianwei Zhang | Jingya.li@ericsson.com Siva.muruganathan@ericsson.com Jianwei.zhang@ericsson.com |
| ZTE | Lun Li Xingguang Wei | li.lun1@zte.com.cn wei.xingguang@zte.com.cn |
| IIT Kanpur | Shyam Vijay Gadhai Abhishek Kumar Singh | svgadhai@iitk.ac.in abhishekks@iitk.ac.in |
| Spreadtrum | Mimi Chen | Mimi.chen@unisoc.com |
| LG | Jaehoon Chung | jhoon.chung@lge.com |
| CATT | Qianrui Li | liqianrui@catt.cn |
| Intel | Victor Sergeev | Victor.sergeev@intel.com |
| Xiaomi | Liuzhengxuan Liumin | liuzhengxuan@xiaomi.com liumin10@xiaomi.com |
| AT&T | Isfar Tariq Salam Akoum | Isfar.tariq@att.com Salam.akoum@att.com |
| CMCC | Yuhua Cao Yi Zheng Dan Song | caoyuhua@chinamobile.com zhengyi@chinamobile.com songdan@chinamobile.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| MediaTek | Pedram Kheirkhah Sangdeh | Pedram.kheirkhah@mediatek.com |
| Futurewei | Baoling S Sheen | bsheen@futurewei.com |
| TCL | Yunsheng Kuang | yunsheng.kuang@tcl.com |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| CEWiT(TSDSI) | Shiv Shankar Dhivagar Baskaran | shivshankar@cewit.org.in dhivagar.b@cewit.org.in |
| NEC | Zhen He Pravjyot Deogun | he_zhen@nec.cn pravjyot.deogun@EMEA.NEC.COM |
| Qualcomm | Chenxi Hao | chenxih@qti.qualcomm.com |
| Sony | Hiroki Matsuda Sam Atungsiri | hiroki.matsuda@sony.com sam.atungsiri@sony.com |
| Apple | Huaning Niu | Huaning_niu@apple.com |
| Tejas Networks | Pavan Kalyan | pavankalyand@tejasnetworks.com |

# Temporal domain aspects of CSI compression

## Summary of company proposals

### Case 1/2/5

Apple Inc.

Proposal 2: For case 2 and case 4 of time-frequency-spatial domain CSI compression, the following potential specification impact are proposed:

- Enable semi-persistent CSI reporting

- Enable DCI based reset state

- Considering UCI retransmission in case of large amount of UCI drop or loss, to avoid the state at UE and gNB out of sync.

- For RI, consider longer term RI update across different CSI reports.

Proposal 6: For time-frequency-spatial domain CSI compression, flexible CSI report configuration to support different cases should be studied.

CATT

Proposal 1: For Case 2, specification support on dealing with non-ideal UCI feedback (i.e., UCI loss) is not needed.

Ericsson

Proposal 20	Further study methods to reduce the AI computational complexity for AI CSI compression Case 2.

ETRI

Proposal 1: For AI/ML-based CSI compression using two-sided model, when UE and/or NW uses past CSI information, reuse the current specification on CSI-RS transmissions as much as possible.

Proposal 2: For AI/ML-based CSI compression using two-sided model, when NW uses past CSI information, study method to detect and mitigate inconsistency of the availability of past CSI information between the UE and the NW.

Fujitsu

Proposal 2: For Case 2, RAN1 to study how to address layer disorder issue for training data collection, model inference and performance monitoring.

Proposal 3: For CSI compression using two-sided AI/ML models, support both the following alternatives of precoding matrix for output-CSI-UE and input-CSI-NW:

o	Alt 1: The precoding matrix in spatial-frequency/spatial-frequency-time domain for Case 2/Case 3

o	Alt 2: The precoding matrix represented using angular-delay/angular-delay-doppler domain projection for Case 2/Case 3.

Google

Proposal 1: For case 2 and case 3, support the UE to report W1 and compressed W2 for a configured rank

•The compressed W2 is calculated based on the AI/ML

•NW can configure the UE to report RI and CQI based on precoded CSI-RS resources after receiving the W1 and compressed W2

Huawei, HiSilicon

Proposal 8: For the additional potential spec impact of temporal domain CSI compression Case 2 on top of Rel-18 SF domain CSI compression, consider methods to handle the misalignment of the accumulated CSI between NW part model and UE part model due to UCI missing.

InterDigital, Inc.

Proposal 1: Study the specification impact of adapting the length of the historical CSI buffer to channel conditions, such as UE speed.

Proposal 4: TSF compression performance should be evaluated under multiple observation window lengths.

Proposal 5: For further performance improvement of AI-TSF compression Case2 with missing UCI, study different missing UCI mitigation solutions e.g., partial buffer reset, or retransmission of the missed UCI.

Lenovo

Proposal 16: 	Prioritize Case 2 and Case 3 for temporal domain aspects of AI/ML-based CSI compression using two-sided model.

Proposal 17: 	Support procedures/signalling enabling CSI-compression models having both Scaler and vector Quantizers for generation of the CSI-feedback bits.

LG Electronics

Proposal #1: Regarding temporal/spatial/frequency (TSF)-domain CSI compression, study methods/mechanisms to manage the similarity/synchronization of accumulated past CSI at UE-side and/or NW-side.

Proposal #2: Regarding TSF-domain CSI compression, discuss the format of past CSI information and how to report it at least for performance monitoring perspective.

Proposal #4: Regarding non-ideal UCI feedback on TSF-domain CSI compression,

- Consider two-step performance monitoring to check that the performance degradation of the AI/ML model is originated from whether the accumulated past CSI has a problem or the AI/ML model is not suitable for the deployed environment

- Consider to report past CSI information via NW-triggered signaling when UCI missing or UCI dropping

MediaTek Inc

Proposal 1. Clarify the boundary of AI/ML model, i.e., in which step it starts or ends.  Temporal-spatial-frequency (TSF) domain compression.

Proposal 4. Evaluate the feedback error tolerance of eType II and compare it with that of AI/ML model.

NEC

Proposal 7: At least for Case 2, CSI buffer reset should be supported to address misalignment of historical CSI used at UE side and NW side. And the definition, determination or indication of the reset value need to be further studied.

Proposal 8: At least for Case 2, further study effective availability of historical CSI information over time.

Nokia

Proposal 1: For temporal domain aspects Case 2 and Case 3, it is encouraged to investigate schemes to exploit temporal correlation at the latent domain with consideration of quantizer/dequantizer with memory, such that the independently developed SF compression (encoder) – reconstruction (decoder) model structures (Case 0) shall be rendered directly usable for Case 2 and Case 3 without modification of its structure nor re-training.

Samsung

Proposal#1: For the Case 2 of temporal aspects of AI/ML-based CSI compression using two-sided model, consider at least the following two options for the past CSI information

- Case 2-1: Past CSI information generated by the UE-part and/or network-part of two-sided model

- Case 2-2: Information on SD/FD basis vectors as past CSI information with angle-delay domain compression.

Proposal#2: For the Case 2 of temporal aspects of AI/ML-based CSI compression using two-sided model:

- when past CSI information corresponds to SD/FD basis and AI/ML CSI compression is in the angle-delay domain, consider SD/FD basis reporting per N CSI reporting occasions, i.e., N times longer periodicity.

- FFS on the values of N.

Proposal#5: For the Case 2 of temporal aspects of AI/ML-based CSI compression using two-sided model, for the evaluation of performance impact resulting from CSI dropping

- Modelling for CSI dropping probabilities

  - x% Group 2 of Part II CSI dropping probability

  - y% Group 1 of Part II CSI dropping probability

  - z% Group 0 of Part II CSI dropping probability

where ≥y≥z .

Note: Companies to report the partitioning of part I and part II CSI and the values for x, y, and z.

Proposal#6: Study the impact of input pre-processing (dimensionality reduction) on performance, model complexity and model generalizability.

Proposal#7: In Angle-delay (W2)-domain CSI compression, study the impact of the number of SD/FD basis vectors for performance-complexity tradeoff.

Proposal#8: For the cases of CSI compression with temporal aspects, consider the following for the network’s ground-truth CSI collection

- For cases that require multiple time-domain samples for inference, cases 2/3/4/5, high resolution codebook quantization including temporal aspects, e.g., Rel-18 eType II-like method with new parameters.

- For cases with CSI prediction, e.g., cases 3/4, high resolution codebook quantization for explicit channel matrices, e.g., codebook to report the left and right eigenvectors of a channel matrix H=UΛVH

- Specification impact on measurement and reporting for ground-truth CSI

Spreadtrum

Proposal 4: Consider to report historical CSI information via NW-triggered signaling when UCI missing or UCI dropping.

Tejas Networks

Proposal 1: For the EVM of temporal domain CSI compression, consider the following assumptions for the CSI generation part and CSI reconstruction part, respectively:

- CSI generation part at t=T:

- Model input: Pre-processed channel matrix is given as a input to the CsiNet encoder.

- First layer of encoder is Convolutional layer with real and imaginary parts of channel as its input.  This layer uses 3×3 kernel size to generate two feature maps. Following the convolutional layer, we reshape the feature maps into a vector and use a fully connected layer to generate the codeword s, which is a real-valued vector of size M.

- Model output: Encoder transforms channel matrix into a vector of size M×1.

- CSI reconstruction part at t=T+∂ (where ∂ is an uplink latency)

- Model input: Codeword of size M×1 is given as a input to the CsiNet decoder.

- Model output: First layer of decoder is dense fully connected layer which gives the initial estimate of the channel followed by several RefineNet units gives the CSI.

FFS: Study the effect of quantization

Proposal 2: Consider a missing rate (e.g., 10%) for each individual CSI report occasion to model UCI loss in case 1 and case 5.

Proposal 3: For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model, for case2:

- Scenario A provides the baseline for comparison, offering the best-case performance under ideal conditions.

- Scenario B highlights the need for NW-side mitigation strategies, but its effectiveness is limited due to UE-side mismatches.

- Scenario C offers the most comprehensive mitigation, enabling significant performance recovery through coordinated actions between NW and UE.

Vivo

Proposal 6:	As the model structure for TSF case2 and case3 is closely related to and can be generated based on the model structure for TSF case0, it is recommended to start the discussion of standardization of model structure for CSI compression with TSF case0 (i.e., SF compression).

Xiaomi

Proposal 1: To align the missed historical CSI information when UCI loss occurs, the latest output historic CSI of encoder and decoder before missing UCI could be respectively adopted as the input of encoder and decoder in the current inference instance.

ZTE

- To reduce the efforts for specification impact analysis and standardization, conduct comparison between Case 2 and Case 3 for potential down selection.

### Case 3/4

Huawei, HiSilicon

Proposal 9: For the additional potential spec impact of temporal domain CSI compression Case 3 on top of Rel-18 SF domain CSI compression,

- for separate prediction and compression, potential spec impact may be needed for data collection of the CSI compression model and monitoring of the CSI prediction model.

- for joint prediction and compression, potential spec impact may be needed for data collection, inference, and monitoring.

Tejas Networks

Proposal 4: In the results template for capturing the evaluation of temporal domain aspects Case 3/4 of AI/ML based CSI compression, it is clarified that the upper bound is calculated based on ideal CSI prediction and without CSI compression.

Proposal 5: For temporal domain aspects Case 3 and 4, study the impact on LCM aspects (e.g., data collection, training, monitoring, and model control) of separate prediction and compression vs. joint prediction and compression.

ZTE

- To reduce the efforts for specification impact analysis and standardization, conduct comparison between Case 2 and Case 3 for potential down selection.

Vivo

Proposal 6:	As the model structure for TSF case2 and case3 is closely related to and can be generated based on the model structure for TSF case0, it is recommended to start the discussion of standardization of model structure for CSI compression with TSF case0 (i.e., SF compression).

Google

Proposal 1: For case 2 and case 3, support the UE to report W1 and compressed W2 for a configured rank

- The compressed W2 is calculated based on the AI/ML

- NW can configure the UE to report RI and CQI based on precoded CSI-RS resources after receiving the W1 and compressed W2

CATT

Proposal 15: In CSI compression using two-sided model use case, for L1 signalling based reporting of ground-truth CSI/target CSI for NW-side data collection, study whether multiple CSI in the same report is supported for temporal domain aspects Case 2/3/4.

Proposal 22: In CSI compression using two-sided model use case, for temporal domain aspects Case 3 and Case 4 with separate prediction and compression adopted, support monitoring the performance of the model for prediction and the performance of the model for compression separately.

Ericsson

Proposal 19	Further study methods to reduce the AI computational complexity for AI CSI compression Case 3.

Fujitsu

Proposal 3: For CSI compression using two-sided AI/ML models, support both the following alternatives of precoding matrix for output-CSI-UE and input-CSI-NW:

- Alt 1: The precoding matrix in spatial-frequency/spatial-frequency-time domain for Case 2/Case 3

- Alt 2: The precoding matrix represented using angular-delay/angular-delay-doppler domain projection for Case 2/Case 3.

OPPO

Proposal 6: Regarding the standardization of model structure in Option 3, suggest to prioritize the work on Case 0. Case 2 and Case 3 can be further considered after model structure in Case 0 has been specified.

LG

Proposal #3: Regarding TSF-domain CSI compression Case 3 and 4, consider performance monitoring method on joint CSI compression and prediction by adapting the operation on the AI/ML model between CSI compression and prediction.

Lenovo

Proposal 16:	Prioritize Case 2 and Case 3 for temporal domain aspects of AI/ML-based CSI compression using two-sided model.

InterDigital

Proposal 4: TSF compression performance should be evaluated under multiple observation window lengths.

Apple

Proposal 2: For case 2 and case 4 of time-frequency-spatial domain CSI compression, the following potential specification impact are proposed:

•	Enable semi-persistent CSI reporting

•	Enable DCI based reset state

•	Considering UCI retransmission in case of large amount of UCI drop or loss, to avoid the state at UE and gNB out of sync.

•	For RI, consider longer term RI update across different CSI reports.

Proposal 4: For performance monitoring of case 4 of time-frequency-spatial domain CSI compression, CSI measurement in prediction window is the target CSI for NW side or UE side performance monitoring. The intermediate KPI or eventual KPI includes both compression and prediction performance.

Proposal 5: For case 3 of time-frequency-spatial domain CSI compression, CSI measurement in prediction window is the target CSI for NW side or UE side performance monitoring. The intermediate KPI or eventual KPI includes both compression and prediction performance.

Proposal 6: For time-frequency-spatial domain CSI compression, flexible CSI report configuration to support different cases should be studied.

Nokia

Proposal 1: For temporal domain aspects Case 2 and Case 3, it is encouraged to investigate schemes to exploit temporal correlation at the latent domain with consideration of quantizer/dequantizer with memory, such that the independently developed SF compression (encoder) – reconstruction (decoder) model structures (Case 0) shall be rendered directly usable for Case 2 and Case 3 without modification of its structure nor re-training.

ETRI

Proposal 3: For AI/ML-based CSI compression using two-sided model, when the target CSI is Future slot(s), study following aspects, for performance monitoring operations:

- Method to align whether prediction and compression occur in separate steps or simultaneously between UE and NW

- Either UE-side or NW-side performance monitoring.

Samsung

Proposal#4: Among the identified six categories for AI/ML-based CSI compression using two-sided model, for Case 3, consider N_4≥1 prediction instances (Doppler time intervals).

Option1: AI/ML-based CSI compression in spatial-frequency-time domain

Option 2: The AI/ML-based CSI compression in angle-delay-time domain

Option 3: The AI/ML-based CSI compression in angle-delay-Doppler domain

MediaTek

Proposal 1.	Clarify the boundary of AI/ML model, i.e., in which step it starts or ends.  Temporal-spatial-frequency (TSF) domain compression.

Proposal 3.	Deprioritize  case 3 which may use one AI/ML model for doing both CSI compression and CSI prediction in this agenda item.

## Discussion

### UCI loss

Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, study the performance impact resulting from non-ideal UCI feedback. - Scenario A: no UCI loss - Note: Corresponds to an upper bound or re-aligning missing historical CSI information - Scenario B: UCI loss, known at NW and unknown at UE, with mitigation at NW - Note: Corresponds to implementation-based mitigation at NW but no signaling to UE. - Scenario C: UCI loss, known at NW and UE, with mitigation at NW and UE - Note: Corresponds to reset of historical CSI information at both UE and NW or any other mitigation approach enabled by signaling. - UCI loss modeling - 10% UCI loss probability on all UCI reports - Other options are not precluded, e.g., No UCI loss for the first UCI report of each observation window, and 10% UCI loss probability for the subsequent reports of each observation window, as a shortcut to simplify the evaluation work. - Other values for UCI loss probability are not precluded. - Note: The same UCI loss modeling shall be applied to the benchmark for fair comparison. - FFS: partial PMI-related UCI loss

##### Discussion 21a:

Some companies report large SCGS drop in case of UCI loss particularly when no special handling is done, motivating the need for a mitigation method at least at network (Scenario B), whereas some other companies report limited SCGS losses for scenario B and C when the missing report is replaced with the latest one.

Further evaluations are needed to conclude the impact of UCI loss over the benchmarks and determine whether specification is not needed (i.e., Scenario B performs well) or specification is needed for UCI loss handling (i.e., Scenario B shows nontrivial loss but Scenario C performs well).

For the results template used to collect evaluation results for temporal domain compression Case 2, please report the following information in the row “UCI loss modeling and handling”:

- The scenario, i.e., scenario B or C in the result.

- Action taken by NW or UE for the UCI loss.

- UCI loss probability

Note that the same UCI loss modeling shall be applied to the benchmark for fair comparison. If UCI loss was modelled for Case 2 but not for the benchmark, please use two separate columns.

| Company | Comments |
| --- | --- |
| Ericsson | Support |

### Separate vs. joint prediction and compression for Case 3

For temporal domain aspects Case 3, the UE may perform prediction as a separate step or jointly with compression. Companies noted that whether the prediction and compression is performed jointly or separately may affect LCM aspects such as data collection, training, monitoring, and model control. So, it is suggested to study this further.

From the evaluation results on Case 3, majority of companies considered separate prediction and compression (SPC) over joint prediction and compression (JPC). Additionally, some companies are proposing the prioritization of SPC.

| Case-scheme | Companies evaluation | Support |
| --- | --- | --- |
| Case3-SPC | [oppo, vivo, QC, Fujitsu, ZTE, DOCOMO, CATT, Ericsson, Samsung, CMCC, Xiaomi, InterDigital] | [QC, MediaTek] |
| Case3-JPC | [CMCC] |  |

##### Proposal 22a: SPC vs JPC (same as proposal 2a of the last meeting)

For temporal domain aspects Case 3, take separate prediction and compression (SPC) as a baseline, and study its LCM aspects and specification impacts. FFS: Joint prediction and compression (JPC), its LCM aspects and specification impacts.

For SPC, study the following options for training data collection

- Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s).

- Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s).

- Note: During inference, the input to the CSI generation part is derived based on the predicted CSI.

For SPC, study the following options for the monitoring target

- Option 1: The monitoring target is derived based on the predicted CSI of the future slot(s).

  - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately.

- Option 2: The monitoring target is derived based on the measured CSI of the future slot(s)

  - Note: This corresponds to end-to-end monitoring of CSI prediction and compression.

For SPC, study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts.

| Support / Can accept | Sony |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | OK for the study of the two options. Between Option 1 and Option 2, we prefer Option 1. On one hand, predicted CSI may have different data distribution to the actually measured CSI, so using predicted CSI as label may cause generalization issue for the training data sample of the compression model and cause bias for the monitoring accuracy. On the other hand, if measured CSI is used as labels, there is no difference from Case 0 from spec perspective. |
| Ericsson | Too early to discuss the LCM details for this use case. We need to solve the performance gain vs. complexity trade-off for this use case first. |
| Sony | We support study of the two options |
| LG | Fine with this direction to study |
| ZTE | Agree with Ericsson. We also think it is too early to discuss the LCM details since we need to address the performance issue vs complexity issue first. |
| CATT | Ok |
| Tejas | Support |
| Fujitsu | Regarding training data collection, since the CSI prediction and CSI compression are separate, Option 2 is preferred for training data collection, which utilize the measured CSI as label. Regarding performance monitoring, if the performance of CSI prediction and CSI compression are monitored separately, then for CSI compression, the measured CSI could also be used. In addition, for training data collection, two options are listed, one use predicted CSI, and the other one use measured CSI. For performance monitoring, it should be aligned with training data collection on whether predicted CSI or measured CSI is used. Hence, we suggest the following for performance monitoring: For SPC, study the following options for the monitoring target - Option 1: Separate monitoring for CSI compression and CSI prediction. The monitoring target is derived based on the predicted CSI of the future slot(s). - FFS: whether the monitoring target for CSI compression is derived based on measured CSI or predicted CSI. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. - Option 2: End-to-end monitoring of CSI prediction and compression. The monitoring target is derived based on the measured CSI of the future slot(s) Note: This corresponds to end-to-end monitoring of CSI prediction and compression. |

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |
| Sony | For the case of SPC, RAN1 should assume the use of the output of CSI prediction from AI 9.1.3 as baseline. |

# Localized models

## Summary of company proposals

Tejas Networks

Proposal 6:

In case of N different local regions and train N different localized models for each region, average performance should be considered over the N local regions.

NVIDIA

Proposal 1: Site-specific AI/ML models for CSI compression should be considered to improve performance gain.

MediaTek

Clarify the boundary of AI/ML model, i.e., in which step it starts or ends.  Temporal-spatial-frequency (TSF) domain compression.

## Discussion

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |

# Inter-vendor training collaboration

## Summary of company proposals

From the submitted contributions, proposals related to inter-vendor training collaboration are summarized below.

Futurewei

Proposal 1: For issues identified for Direction A related to on-device operation and UE side offline engineering, consider the analysis captured in Table 2.1.1-1 and the corresponding notes.

Table 2.1.1-1: Direction A issue applicability for each inter-vendor collaboration option

| Inter-vendor collaboration option | Issue 1 | Issue 2 | Issue 3 | Issue 4 |
| --- | --- | --- | --- | --- |
| 3a-1 | If the exchanged model or parameters is/are site, scenario or configuration-specific, NW also needs to share such information. | Not directly through the model parameters (unless other information is also shared). | Yes, but less concern than sharing dataset(s). | Yes (see note 1) |
| 3a-2 | May need to share reference performance value. | Not directly through the model parameters (unless other information is also shared). | Yes, but less concern than sharing dataset(s). | Yes (see note 2) |
| 3a-3 | May need to share reference performance value. | Not directly through the model parameters (unless other information is also shared). | Yes, parameter exchange overhead is more than 3a-1 and 3a-2, but in general the concern is less that sharing dataset(s). | Yes (see note 3) |
| 5a-1 | UE-side is not able to perform end-to-end testing unless the CSI reconstruction part is also available. | Not directly through the model (unless other information is also shared). | Yes, but less concern. | Yes (see note 4) (same as 3a-1) |
| 5a-2 | No | Not directly through the model (unless other information is also shared). | Yes, but less concern. | Yes (see note 5) (same as 3a-2) |
| 5a-3 | No | Not directly through the models (unless other information is also shared). | Yes, but less concern (more model exchange overhead than 5a-1 and 5a-2) | Yes (see note 6) (same as 3a-3) |
| 4-1 | UE-side is not able to perform end-to-end testing unless some samples for reconstructed CSI are also available. | No (at least not directly) | Yes | Yes (see note 7) |
| 4-2 | No | No (at least not directly) | Yes | Yes (see note 8) |
| 4-3 | No | No (at least not directly) | Yes (more data exchange overhead than 4-1 and 4-2) | Yes (see note 9) but less serious than 4-1 and 4-2 (see note 9) |

Proposal 2: For issues identified for Direction B related to on-device operation and UE side offline engineering, consider the analysis captured in Table 2.1.2-1 and the corresponding notes.

Table 2.1.2-1: Direction B issue applicability for inter-vendor collaboration option 3b

| Inter-vendor collaboration option | Issue 3 | Issue 5 | Issue 6 | Issue 7 |
| --- | --- | --- | --- | --- |
| 3b | Yes (similar as 3a) | Maybe (see Note 10) | Yes (see note 11) | Maybe (see Note 12) |

Note 10: In Option 3b, the parameters exchanged from the NW-side to UE-side is CSI generation part.

- Whether it is feasible to use a common encoder across UEs

  - As Option 3b is for UE-side to directly use the trained CSI generation part (trained at the NW-side) with on-device operation, if NW-side is able to collect samples from various UEs (or UE types/groups), then NW-side may be able to train a common encoder across UEs (or UE types/groups) while some ML techniques may need to be leveraged.

- Whether it is feasible for NW-side to train multiple encoders for different UEs

  - This depends on how the training samples were collected at NW-side. If NW-side is able to collect samples from various UEs (or UE types/groups), then it is feasible for NW-side to train more than one encoder, one for each UE type/group.

Note 11: similar as Option 3a, if mismatch between NW side data distribution and UE side inference data distribution occurs, E2E performance will be impacted. In such case, NW-side needs to fine tune the model(s) with new samples then deliver the new CSI generation part to UE-side.

Note 12: If the goal is to train multiple encoders at NW-side, one for each UE type/group, or one for each scenario/configuration then some information related to UE type/group and scenario/configuration may need to be shared between UE vendors and NW vendor(s).

Proposal 3: For issues identified for Direction C related to on-device operation and UE side offline engineering, consider the analysis captured in Table 2.1.3-1 and the corresponding notes.

Table 2.1.3-1: Direction C issue applicability for inter-vendor collaboration option 1

| Inter-vendor collaboration option | Issue 8 | Issue 9 | Issue 10 |
| --- | --- | --- | --- |
| 1 | See Note 13 | Yes (see note 14) | See Note 15 |

Note 13: using field data for training the reference model may involve more issues and effort. Therefore, more discussions are needed among companies. Considering this, using 3GPP’s statistical channel model to generate data for training the reference model is a more practical option. However, 3GPP’s statistical channel model may not match the real channel samples collected in the field; thus, the performance of the reference model may be questionable. This issue is tangled together with Issue 9.

Note 15: If companies agree to specify/standardize multiple reference models, e.g., for different scenarios/configurations, UE types/groups, or other conditions, then those scenarios/configurations and other related information may need to be specified as well.

Proposal 4: Among the (sub) options to alleviate/resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, consider using standardized signalling in upper layers (case z4) as one of the delivery options to save the air-interface overhead at least for Option 3a/4/5a.

Proposal 5: For AI/ML-based CSI compression using two-sided model, further study the following potential specification impact related to quantization of CSI feedback, at least for Option 3a/3b/4/5a in alleviating/resolving the issues related to inter-vendor training collaboration:

- Vector quantization:

  - Exchange of vector quantization codebook(s).

  - Segmentation information (if segmentation is used) of the CSI output.

- Scalar quantization:

  - Configuration of quantization granularity and the corresponding range values.

  - Exchange of scalar quantization dictionary.

Huawei, HiSilicon

Proposal 1: For additional information of Direction A (Option 3a/5a/4), study the performance target including at least the following aspects:

- End-to-end performance metric and/or encoder only performance metric.

- The type of performance metric.

- Testing dataset used for metric calculation to test the performance target.

  - Note: the performance target is only applicable for the testing dataset shared from NW.

Proposal 2: For additional information of Direction A (Option 3a/5a), the dataset of at least Target CSI should be also shared from NW side to UE side for Option 3a/5a to ensure:

- Aligned dataset for training NW part CSI reconstruction model and UE part CSI generation model.

- Aligned dataset for metric calculation to test the performance target at UE side.

Proposal 3: Regarding the data distribution mismatch issue of Direction A/B, the necessity and feasibility to resolve it may need careful justification, considering:

- The mismatch issue may not exist considering NW side collect sufficiently diverse training dataset which could be generalized to the new UE side data.

- The mismatch issue (if exists) can be alleviated to a large extent with timely dataset update by NW, where the NW side would perform NW side data collection from the new UEs and update the delivered dataset/model/parameters to represent the new UE side data.

- The mismatch issue (if exists) cannot be solely resolved by the UE side training, as the NW part model is still not generalized with the new UE side data.

Proposal 4: For the dataset sharing for Option 3a/4/5a over air-interface, study the solution to relieve the overhead.

- E.g., NW splits the overall dataset into many subsets each with a limited number of data samples (e.g., with an overhead affordable by the RRC signaling). The subsets can be separately sent to different UEs, and all subsets are associated with a common dataset ID for the UE side re-combination.

Observation 3: For the overhead analysis of Direction A, the overhead between Option 3a and Option 4 may be comparable.

Observation 4: For the overhead analysis of Direction A/B, the overall consumed air-interface overhead subject to Direction A (Option 3a/4/5a) is much less than Direction B, since parameter delivery of Direction B needs to be repeatedly performed to each UE for per trained model.

Proposal 5: For the further study of Direction A/B, consider Option 3a-3/4-3/5a-3 with lower priority due to the price of larger overhead and higher risk of proprietary disclosure than Option 3a-1/3a-2/4-1/4-2/5a-1/5a-2 with no clear benefit on performance.

Observation 7: For Direction C, the performance is negatively impacted due to mismatch between the synthetic dataset used for reference model(s) training and the field data for inference.

- The mismatch issue persistently exists for long time period till the reference model(s) are updated based on the evolution of specifications.

Proposal 6: For studying the standardized model structure of inter-vendor collaboration Option 3, strive to perform down selection to the factors (e.g., model input type, rank>1 options, generalization aspects, scalability approaches, quantization/dequantization method, etc.) so that a single model structure along with pre/post processing approach is aligned before starting the evaluation.

Proposal 7: For studying the standardized dataset format of inter-vendor collaboration Option 4, consider at least the following aspects:

- Data sample format related aspects, e.g., type of ground-truth CSI, the dimension of input/output, CSI feedback related information.

- Dataset construction related aspects, e.g., number of data samples, dataset ID, dataset split/segmentation, association between ground-truth CSI and CSI feedback subject to a data sample, observation window/prediction window information.

- Scalability information.

- Target performance information.

Spreadtrum

Observation 4: For options 4, in addition to the dataset, NW needs to transfer additional information to ensure that UE-trained generation model match performance requirements.

Proposal 1: Deprioritize option 5a for inter-vendor training collaboration.

Proposal 2: Options that only transfer CSI generation part to UE can be considered with high priority.

Observation 5: Compared to Option 3a, options 3b may have limited flexibility and performance.

Tejas Network

Observation 4:

For Down-Selection of Sub-Options for CSI Compression:

- For performance, Option 4-1 offers the most robust results with minimal data mismatch issues, while 3a/5a-3 balances performance and model sharing.

- For proprietary protection, 3a/5a-1 and 4-1 are less risky as they transfer either limited or well-defined datasets.

- Regarding overhead, Options 3a/5a-1 and 3b/5b offer the most efficient model transfer without significant impact.

Proposal 7: For Down-Selection of Sub-options, for CSI Compression prioritize 3a/5a-1 and

4-1 for further study, considering their balance between performance, proprietary protection, and manageable overhead.

ZTE

- Observation 1: Additional dataset for model retraining is essential to facilitate the UE-side offline engineering, which may be directly collected by the UE, collected and then transferred by the NW, or a combination of both.

- Proposal 4: For Option 3a/5a of direction A, at least performance target should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing.

- Proposal 5: For Option 4 of direction A, performance target and model backbone should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing.

- Observation 2: For options 3a/5a-2, 3a/5a-3, 4 of direction A, there is an inherent risk of proprietary information disclosure arising from the exchange of dataset or reference model/parameters related to the decoder from the NW-side to UE-side.

Observation 3: Among different options of direction A, option 4 faces serious overhead concerns across all its sub-options, while options 3a and 5a incur much lower overhead due to the nature of their parameter exchange mechanisms.

- Proposal 6: For the proprietary design of direction A, the underlying NW-side and UE-side additional conditions of the dataset for encoder training at the UE side should be aligned with that of the dataset for decoder training at the NW side.

- Proposal 7: For direction A, there is only marginal performance impact due to mismatch between NW side data distribution and UE side data distribution, if the NW-side and UE-side additional conditions used for encoder or decoder training can be communicated to the counterpart in advance.

- Proposal 8: For direction B, the number of parameters associated with the parameter exchange is estimated to range between 1 million to 13 million, which is acceptable for transmission via over-the-air interface signaling.

- Proposal 9: For direction B, it is infeasible to implement a universal encoder across various UEs due to the inherent diversity in model structure preferences, data pre-processing requirements, hardware capabilities, power constraints, and quantization methodologies among different UE or chipset vendors.

- Proposal 10: For direction B, it is feasible for the network side to train multiple encoders tailored to different UEs, allowing for the maintenance of complexity at a manageable level while preserving the performance advantages of localized models.

- Proposal 11: For direction B, data categorization using associated IDs and continuous monitoring can be considered for addressing the data distribution mismatch issues.

Observation 5: Direction B may carry risks of disclosing proprietary information due to the sharing of model parameters and device-specific design decisions.

- Proposal 12: For direction C, synthetic data generated under 3GPP’s statistical channel model can be a starting point for reference model training.

- Proposal 13: Whether there is any performance impact arising from the data distribution mismatch depends on the generalization capability of the standardized reference model and thus further evaluations are needed.

- Proposal 14: For direction C, model retraining based on data collected in real world can be performed to bridge the gap between synthetic data and field data.

- Proposal 15: Issues 8 and 9 are also applicable to directions A and B due to the standardization requirement of reference model structure

- Proposal 16: For Option 1, prioritize to study the case of standardized CSI generation part.

Observation 9: Performance of Option 3a may be guaranteed via offline engineering with exchanged additional information. However, the problem of proprietary information disclosure would hinder the interoperability issue between UE side and NW side.

Observation 10: Though the interoperability issue of Option 3a-3 may be resolved, the end-to-end performance is limited to the whole specified two-sided reference model structure, which may have lower performance upper-bound compared with Option 3a-1 and Option 3a-2.

- Proposal 17: For Option 3, further study and evaluate the actual end-to-end performance of different sub-options when either side and/or both sides perform offline secondary development before actual system use.

Observation 12: The interoperability issue is questionable when model backbones are misaligned between UE side and NW side.

- Proposal 18: For Option 3/4/5, prioritize to study NW-first training scheme and exchanging the parameters/model/dataset from NW side to UE side.

- Proposal 19: For Option 3/4/5, prioritize to study over-the-air delivery scheme.

- Proposal 20: For Option 3a/4/5a, further study the post-development operations on the AI/ML model during the actual deployment to guarantee the interoperability issue and performance when offline engineering is performed by UE vendors and/or NW vendors.

- Proposal 21: For comprehensive analysis on AI/ML framework for two-sided model, further study a complete and unified solution for model identification, multi-vendor collaboration, and model pairing.

Vivo

- For options 3a/5a/4 in direction A, the additional information including performance targets and dataset or information related to collecting dataset should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing.

- Clarify what information related to collecting dataset should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing

- With consideration for future use case extensions in CSI compression, the issue of aligning model structure to achieve good performance in separate training could not be thoroughly addressed without specifying reference model structures.

- For option 4, the additional information include the information related of model structure should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing.

- Sharing information related to collecting dataset for all the cases in direction A may have the risk of NW side proprietary information disclosure.

- Sharing unspecified model structure in direction A may involve NW side proprietary information disclosure.

- There could be proprietary concern on sharing weights of CSI reconstruction part (i.e., option 3a-2/3) in direction A, as other vendors could benefit from disclosed information.

- There would be little proprietary concern on sharing weights of CSI generation part (i.e., option 3a-1), as other vendors can hardly benefit from disclosed information.

Observation 8: There would be little overhead concern on direction A with CSI generation model transfer. Overhead concern for direction A mainly lies in training dataset delivery.

Observation 17: For direction B, it is feasible to address the issue of data distribution mismatch between NW side training and UE side inference via the following steps:

- NW data drift detection based on identifying cause of performance degradation;

- NW side data collection;

- NW side model updating in an end-to-end manner;

- UE side update model for inference based on transferred encoder parameter from NW side.

- For direction C, the discussion of the reference model can be started with data generated from 3GPP’s statistical channel model with different dataset distribution to evaluate the performance.

Observation 22: From initial results of field test, the generalization performance of AI/ML model trained by UMa simulation data to field data has some degradation compared with model trained with field data itself but still shows robustness in its performance at least for dataset generated with Uma channel model.

- As there are too many open questions for direction C for now, it is better to wait till more agreements are made and then discuss issue 10 accordingly.

- The reference model structure may be aligned through the following procedures

- Step 0: Determine the model backbone and the hyper-parameters that need to be aligned for the backbones.

- Step 1: Align evaluation assumptions.

- Step 2: Align the hyperparameters of the model.

- Step 3: Consider additional processing for scalability.

- As the model structure for TSF case2 and case3 is closely related to and can be generated based on the model structure for TSF case0, it is recommended to start the discussion of standardization of model structure for CSI compression with TSF case0 (i.e., SF compression).

- For studying standardization of encoder model structure in RAN1, RAN1 considers the following candidate backbones for case 0:

- Transformer

- CNN

- FFS on applicability of candidate backbones for Direction A/B/C.

- For convolution layer in CNN, the following hyper-parameter needs to be specified for standardized model structure:

- Activation function;

- Hyper-parameters in each convolution operation, including kernel size, stride, and padding.

- For CNNs, RAN1 can consider the structure graph agreed in RAN4 as a starting point for discussion.

- For model with Transformer backbone, the following hyper-parameter needs to be specified for standardized model structure:

- Number of Transformer blocks;

- Embedding length;

- Hyper-parameters in the self-attention blocks, including dimension of each head, and number of attenuation heads in one transformer block;

- Hyper-parameters in the feed-forward blocks, including dimension of feedforward latent space, and activation function.

- Consider the agreed evaluation assumptions for comparing different combinations of hyper-parameters in RAN4 as the starting point for discussing specified model structure in RAN1. RAN1 can further discuss the additional necessary evaluation assumptions not covered by RAN4 but necessary for RAN1 study.

- RAN1 can consider the CNN structure presented in the following table (cited from RAN4 agreements) to start the discussion of specifying model structure.

- RAN1 can consider the Transformer structure presented in the following tables to start the discussion of specifying model structure.

- RAN1 can firstly focus on the discussion of model structure based on a set of parameters for evaluations and then based on the aligned model structure to further consider scalability over number of Tx ports, payload sizes and bandwidths.

CMCC

- Proposal 1: The progress on reference model in RAN4 could be reused for the standardization of reference model structure in Option 3, no matter the standardized model itself or the method on how to standardize an AI model.

- Proposal 2: It should be assumed the CSI generation or reconstruction part that exchanged parameters belongs to is aligned with the CSI generation or reconstruction part that will be standardized in Option 3:

- For Option 3a-1, at least the reference model structure of CSI generation part should be standardized

- For Option 3a-2, at least the reference model structure of CSI reconstruction part should be standardized

- For Option 3a-3, the reference model structure of CSI generation part and reconstruction part should be standardized

- For Option 3b, at least the reference model structure of CSI generation part should be standardized

- Proposal 3: There might be following possible alternatives for model deployment procedure of Option 3a-1:

- Alt 1: The applied CSI generation part consists of reference model structure via pre-deployment and model parameters through offline engineering.

- Alt 2: The applied CSI generation part consists of model structure and parameters through offline engineering.

- Proposal 4: There might be the following possible alternatives for model deployment procedure of Option 3a-2:

- Alt 1: The applied CSI generation part consists of reference model structure via pre-deployment and model parameters through offline engineering.

  - The reference model structure of both CSI generation part and reconstruction part should be standardized.

- Alt 2: The applied CSI generation part consists of model structure and parameters through offline engineering.

- Proposal 5: There might be the following possible alternatives for model deployment procedure of Option 3a-3:

- Alt 1: The applied CSI generation part consists of reference model structure via pre-deployment and model parameters through offline engineering.

  - The standardized reference model structure of CSI generation part is applied for actual model deployment.

- Alt 2: The applied CSI generation part consists of model structure and parameters through offline engineering.

  - The standardized reference model structure of CSI generation part is only used for exchanged model parameters alignment.

- Proposal 6: It is proposed to discuss the inter-vendor collaboration complexity, performance and feasibility of Option 4-1, 4-2 and 4-3 together.

- Proposal 7: It should be assumed the CSI generation or reconstruction part exchanged is aligned with the CSI generation or reconstruction part of which the model structure aligned based on offline inter-vendor collaboration in Option 5:

- For Option 5a-1, at least the model structure of CSI generation part should be aligned offline

- For Option 5a-2, at least the model structure of CSI reconstruction part should be aligned offline

- For Option 5a-3, the model structure of CSI generation part and reconstruction part should be aligned offline

- For Option 5b, at least the model structure of CSI generation part should be aligned offline

- Proposal 8: It is proposed to discuss the inter-vendor collaboration complexity, performance and feasibility of Option 5a-1, 5a-2, and 5a-3 together.

- Proposal 9: At least Option 3a, 3b and 5a should be prioritized for further study.

- Observation 22: [Issue 3] For Direction A, for option 3a, 4 and 5a, it is necessary to perform model/ dataset/ parameter exchange over the air, which will bring extra overhead issues.

- Observation 23: [Issue 4] For Direction A, there will be performance impact due to mismatch between NW side data distribution and UE side data distribution, especially for option 3a-1, 4-1, 5a-1.

- Observation 25: [Issue 5] For Direction B, it is feasible to use a common encoder across UEs, and it is feasible for NW-side to train multiple encoders for different UEs.

- Observation 26: [Issue 6] For Direction B, there is not much performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if it exists, it can be solved by NW-side implementation.

- Observation 29: [Issue 9] For Direction C, there is performance impact due to mismatch between the distribution of the dataset used for reference model(s) training and UE-side data distribution/NW-side data distribution; and there is performance impact due to mismatch between UE-side data distribution and NW-side data distribution.

- Observation 30: [Issue 10] For Direction C, if NW-side and/or UE-side will train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder/decoder, additional information should be shared to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing; if the reference encoder/decoder are actually deployed at UE-side/ NW-side, additional information is not needed to be specified.

- Observation 31: [Issue 10] For Direction C, if additional information is needed to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing, it includes:

- Dataset or information related to collecting dataset

- Performance target

- Proposal 10: Direction A and B should be prioritized for further study.

TCL

Proposal 5: Option 1 and 2 should not be standardized, at least they are out of the scope of R19.

Proposal 6: RAN 1 should down select among option 3, 4 and 5 considering if unified model format or structure is shared between the NW and UE side model respectively.

- For option 4, there may be no need for offline-engineering.

- Option 3 and 5 are preferred if model structure or format is exchanged from NW to UE.

Xiaomi

Proposal 2: Parameter or model could be exchanged via standardized signalling, and the following exchange method, format of parameter or model could be considered:

- Model delivery/transfer Case z4 could be used to exchange parameter or model.

- The format could be further studied and defined in 3GPP range if two-sided AI/ML model based CSI compression is studied as a normative work.

Proposal 3: Dataset could be exchanged via standardized signalling, and the following definition on format, type and contents of dataset could be considered:

- The format of dataset could be codebook-based quantization (e.g., e-type II like)

- The type of dataset could be eigenvector of raw channel.

- The contents of dataset are target CSI, CSI feedback and/or CSI feedback and reconstructed target CSI.

Proposal 4: In order to make the two-sided model be compatible, it is necessary to study the model pairing for Option 3a/3b/4/5a of inter-vendor training collaboration. The pairing information could be assigned by gNB during model/parameter/dataset exchange.

Proposal 5: It is necessary to report UE capability on training model, so that gNB could adopt different options for inter-vendor training collaboration.

Proposal 6: For Option 3a/4/5a, it is necessary to report the model related aspects, such as scalability, rank and layer handling so that gNB could make suitable parameter configuration for CSI feedback. While it is not necessary to report such model related aspects for Option 3b.

Proposal 7: For Option 3a/3b/4/5a, a quantization approach could be standardized reduce the processing complexity of dequantization at gNB or signalling overhead.

Observation 5: The following information could be shared to UE as additional information shared to enable UE-side encoder training, validation, and testing:

- Associated ID

- The quantization approaches of CSI feedback

- The data format and/or data type of input/output of encoder

- Whether to train a generalized or scalable encoder

- Training/validation/testing dataset size

- Performance target

Proposal 10:  Associated ID is introduced to address the issue of NW’s proprietary information disclosure.

Observation 6：The overhead of model parameter or model transmission is affordable for sub-option 3a/5a, while the overhead of training dataset is too large to be affordable for sub-option 4.

Proposal 11: For training dataset exchange, Rel-16 eType II codebook or high-resolution eType II codebook could be used to transfer dataset for address the overhead issue.

Proposal 12: For different inter-vendor training option, encoder could be retained or updated by using training type 1/type 2/ type 3 to address the issue on mismatch between NW side data distribution and UE side data distribution.

Proposal 13：It is feasible to train multiple encoders for different UEs.

Proposal 14：Suggest the collected dataset based on 3GPP’s statistical channel model for reference model(s) training.

Proposal 15: In order to address the issue, the standardized reference model or model parameters with specified CSI generation part and/or CSI reconstruction part can be retrained or updated by using the UE-side data and NW-side data.

Google

Proposal 13: Consider to prioritize the option 3 for inter-vendor training collaboration for further study.

- Whether the UE directly uses the received parameter/model or not is up to UE implementation

CATT

Proposal 2: Regarding direction A, for Option 3/4/5, prioritize sub-option 1 over sub-option 2/3.

Proposal 3: Regarding direction A, B and C in inter-vendor collaboration, consider following comparisons of issues:

- For Issue 1, additional information of dataset or information related to collecting dataset is necessary. Backbone/structure information and NW-side parameter quantization resolution can also be considered as potential additional information. Whether performance target from NW-side to UE-side can be considered as additional information should be further studied.

- For Issue 2, there is no risk of NW’s proprietary information disclosure for Option 3a-1/5a-1.

- For Issue 3, if OTA signaling is considered, Option 4 has more overhead concerns. Option 3b has less overhead concerns than Option 3a/5a.

- For Issue 4, if dataset or information related to collecting dataset is not provided from NW-side to UE-side as additional information, there is potential performance impact due to mismatch between NW side data distribution and UE side data distribution for Option 3a-1/5a-1. This can be addressed if additional information of dataset or information related to collecting dataset is exchanged.

- For Issue 5, it is not feasible to either use a common encoder across UEs or train UE-specific encoders for different UEs at NW-side. To address Issue 5, NW-side can maintain a common encoder per UE capability group and train capability group-specific encoder at NW-side.

- For Issue 6, there is performance impact due to mismatch between NW side data distribution and UE side inference data distribution. Such mismatch is a consistency issue between training and inference phase which is common for direction A, B and C. Performance monitoring based method can be considered to address this issue.

- For Issue 7, there is no risk of either NW’s or UE’s proprietary information disclosure for Option 3b.

- For Issue 8, field data based reference model training is likely to provide better post-deployment performance than synthetic data based reference model training using 3GPP’s statistical channel model.

- For Issue 9, there is potential performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution. Potential method to address this issue is to collect field data for reference model training.

Proposal 4: For Option 1, to avoid duplicate work between RAN1 and RAN4, study the aspects different from RAN4 option 3/RAN 4 option 4, and not to study the feasibility and methodology(s) on fully standardized reference model in RAN1.

Proposal 5: If Option 3 is supported, prioritize the solution with model structure of CSI generation part is standardized and model/parameter exchanged from the NW-side to UE-side for CSI generation part.

Proposal 6: If Option 4 is supported, prioritize the solution with dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback).

Proposal 7: Regarding inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, deprioritize the solutions with UE-side/NW-side servers involved.

Proposal 8: Regarding inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, for parameter/dataset/model exchange between UE-side and NW-side(Option 3/4/5), prioritize the solutions with over-the-air signaling standardized, and RRC signaling can be considered as a starting point.

Proposal 9: Down-select options for inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, and the follow-up study on specification impacts can focus on the selected option(s).

Proposal 10: Regarding inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, consider the following comparisons of the options:

Ericsson

RAN1 conclude that the reference model(s) (inter vendor collaboration option 1) and/or reference model structure(s) (inter vendor collaboration option 3a and 3b), if supported, is/are designed and specified using 3GPP channel model based synthetic data.

Observation 1: By defining the reference model for one part of the two-sided model, information about the reference data set, and the nominal/minimum performance level requirement, the second side can train its nominal model by choosing its desired implementation and model structure independently and yet interoperability can be maintained in a multi-vendor eco-system.

Option 1 with one part of the two-sided model fully standardized enables the other side to train its model independently considering its implementation capability and yet interoperability can be maintained in a multi-vendor eco-system.

For Direction C (inter vendor collaboration option 1), UE side and NW side can independently further improve their actual UE-part model and NW-part model using dataset collected in the field, if needed.

For Direction B (inter vendor collaboration option 3b), there is overhead concern for the NW-side if NW needs to train/store/transfer different encoder parameters for different UE/chipset types.

For Direction B (inter vendor collaboration option 3b), there may be overhead concern for the UE-side since a UE needs to support switching/loading a different encoder parameter set for its encoder inference when connecting to a different NW vendor.

For Direction B (inter vendor collaboration option 3b), to mitigate inter-vendor collaboration complexity and improve feasibility for UE to use received parameters directly for inference, at least the model parameter quantization and input data pre-processing must be standardized together with the CSI generation model structure. RAN1 should conclude on the required additional information that needs to be standardized to enable option 3b.

For Directions A and B, discuss and conclude on how to model the mismatch between NW-side data distribution and UE side inference data distribution for performance impact study.

For Direction B (inter vendor collaboration option 3b), study and conclude on whether it can bring additional performance gain compared with Direction C (inter vendor collaboration option 1), assuming a common encoder across UEs with standardized model structure, standardized model parameter quantization and standardized input data pre-processing.

For inter-vendor training collaboration, UE-side first training cases for RAN1 Option 3/4/5, where the exchange of parameter/dataset/model originating from the UE-side and ending at the NW-side is not supported as it necessitates multiple models to operate in parallel at the NW-side.

For Option 3a-2/3, it is not feasible for the NW-side to provide actual reconstructed model parameters to the UE-side, since it risks the NW-side to disclose its proprietary implementation related information.

Option 3a-2/3 for inter-vendor training collaboration is not supported.

For option 3a-1, conclude that in addition to CSI generation model parameters, the following additional information are necessary to be shared from the NW-side to the UE-side

  - Information about any regularization or normalization, if any, used during training to enforce a latent space distribution

  - CSI generation model input pre-processing steps

  - Training dataset or information related to collecting training dataset

  - Performance target and encoder/reconstruction loss function

  - Testing dataset or information related to collecting testing dataset

For Option 3a, Over-the-air delivery method for exchanging information from the NW-side to UE-side is not supported.

For option 4, conclude that the UE-side training should be assisted by additional information about the NW side training/testing procedure:

  - Any data preprocessing steps applied to the input of the encoder.

  - Training objectives:

    - Loss function that aligns with the decoder’s requirements

    - If the NW side expects that the training and the corresponding latent space should capture certain features or distribution.

    - Evaluation metric including how well the latent space aligns with the expected input to the decoder, and how the reconstruction accuracy should be.

  - Postprocessing steps applied to the output of the encoder, e.g. if the encoder output is regularized or constrained to match this distribution.

  - Potentially, the following could also help improve the performance:

    - Model architecture, optimization strategies, regularization (e.g. drop out or added noise)

  - Validation/testing data to assess compatibility.

  - Performance targets include e.g. reconstruction accuracy, intermediate KPI like SGCS/NMSE/loss-value, and latent space validation.

Option 4-2 and 4-3 for inter-vendor training collaboration is not supported.

For Option 4-1, study the feasibility of ensure UE-side trained encoder to be compatible with the NW-side decoder without NW-side providing decoder output related information.

For Option 4, over-the-air delivery method for exchanging information from the NW-side to UE-side is not supported.

For Direction B (inter vendor collaboration option 3b) and inter vendor collaboration option 3a in Direction A, the following additional issues shall be studied:

  - [Issue 11] Whether to consider 3GPP’s statistical channel model or field data for the design of reference model structure(s). In case of the latter, how does RAN1 collect field data and agree on them?

  - [Issue 12] Is there performance impact due to mismatch between the distribution of the dataset used for designing the reference model structure(s), UE-side data distribution, and NW-side data distribution, and if so, how to address it?

- Standardize the phase normalization of a precoding vector used as input to an encoder for the CSI compression using two-sided model use case.

- For inter vendor collaboration option 3a in Direction A, Direction B (inter vendor collaboration option 3b) and Direction C (inter vendor collaboration option 1), the following additional issue shall be studied:

  - [Issue 13] How to represent a standardized reference model (structure + parameters) and/or a standardized reference model structure in 3GPP specifications?

Fujistu

Proposal 14:

- To enable UE-sided offline engineering and alleviate inter-vendor training collaboration, the following information could be shared from NW side to UE side:

  - Model input/output related information

    - Input related information, such as pre-processing (normalization/phase correction/domain-transform/scalability)

    - Output related information, such as post-processing (domain-transform/scalability)

  - Quantization related information

    - Quantization type

    - Quantization granularity (e.g., vector length, quantization bit)

    - Quantization table for trainable vector quantization if applied

  - Dataset related information

    - Scenario/configuration information

    - Cell/site information

Proposal 15:

- To enable UE-sided offline engineering, the minimum performance requirement should be shared to UE side for retrained/redeveloped AI/ML models.

  - For sub-option 3a-1/4-1/5a-1, NMSE and cross entropy loss could be considered as the performance metric.

  - For sub-option 3a-2/3a-3/4-2/4-3/5a-2/5a-3, SGCS could be considered as the performance metric.

Proposal 16:

- Options 3a-1/4-1/5a-1 and Options 3a-3/4-3/5a-3 should be deprioritized compared to Options 3a-2/4-2/5a-2.

Observation 9:

- For Option-4, the dataset for training can be divided into multiple parts and can be sent in multiple times based on traffic conditions. Offline delivery could be used for dataset exchange.

Observation 10:

- For sub-option 3a/4/5a, the retraining/redevelopment of UE-sided encoder could reduce the performance loss due to dataset distribution mismatch, to some extent. But the reduction of performance loss is limited by NW-sided decoder.

Observation 11:

- For sub-option 3b, the parameters trained/finetuned with the dataset from UE vendor/side could reduce the performance loss due to dataset distribution mismatch; If NW-sided decoder could also be retrained/finetuned with the dataset from UE vender/side, the dataset mismatch issue could be solved in principle.

OPPO

Proposal 1: Suggest to distinguish the reference model in RAN1 to in RAN4

- Higher requirement on model performance for reference model in RAN1

- RAN1 cannot directly use the agreement on reference model in RAN4

Proposal 2: Regarding [issue 8] in Direction C, RAN1 can refer to RAN4’s discussion on non-specified public dataset:

- 3GPP’s statistical channel model can be a starting point

Proposal 3: Regarding [issue 9] in Direction C, UE-side/NW-side fine-tuning can be conducted to alleviate the dataset mismatch issue.

Proposal 4: Regarding [issue 10] in Direction C, additional information at least includes:

- From UE to NW: target label

- From NW to UE: output of actual CSI reconstruction part and performance target

Proposal 5: Deprioritize Direction C (Option 1) in Rel-19.

Proposal 6: Regarding the standardization of model structure in Option 3, suggest to prioritize the work on Case 0. Case 2 and Case 3 can be further considered after model structure in Case 0 has been specified.

Proposal 7: Regarding [issue 1] for Direction A (sub-option 3a-1/2/3), additional information can include:

- Performance target

- Dataset and dataset related information

Other necessary additional information is not precluded

Proposal 8: Regarding proprietary issue ([issue 2] for Direction A, [issue 7] for Direction B),

- No NW’s proprietary concern for sub-option 3a-1 in Direction A and Direction B (option 3b)

- No UE’s proprietary concern for Direction B (option 3b)

- Knowledge distillation can be used to resolve the NW’s proprietary disclosure for sub-option 3a-2/3 in Direction A

Proposal 9: Regarding [issue 3] for Direction A and Direction B, overhead concern is more critical for over-the-air exchange compared to offline exchange, mainly from two aspects:

- Parameters exchange: can be alleviated by proper model compression methods

  - E.g., knowledge distillation, parameter quantization, pruning

- Dataset exchange: can be alleviated by target CSI label quantization methods

  - E.g., float16, eType II-like codebook

Proposal 10: Regarding [issue 4] for Direction A, [issue 6] for Direction B, performance impacts due to data distribution mismatch can be addressed by:

- UE reports UE-side data to NW, and NW performs encoder fine-tuning for sub-option 3a-1 in Direction A and Direction B (option 3b)

- UE-side encoder fine-tuning for sub-option 3a-2/3 in Direction A

Proposal 11: Regarding [issue 5] for Direction B, the feasibility should be considered from following two aspects:

- Performance aspect: feasible to train multiple encoders for different UEs

- UE capability aspects: feasible if supporting standardizing multiple encoder structures

Proposal 12: RAN1 further study how to standardize data / dataset format in Option 4.

Proposal 13: Regarding [Issue 1] for Direction A (Option 4), additional information can include:

- Performance target

- Dataset related information

Other necessary additional information is not precluded

Proposal 14: Regarding [Issue 2] for Direction A (Option 4), no NW’s proprietary information disclosure concern.

Proposal 15: Regarding [Issue 3] for Direction A (Option 4), overhead can be alleviated by target CSI label quantization methods, e.g., float16, eType II-like codebook.

Proposal 16: Regarding [issue 4] for Direction A (Option 4), performance impacts due to data distribution mismatch can be addressed by:

- UE reports UE-side data to NW, and NW performs inference, and then exchange the CSI feedback information corresponding to UE-side data to UE for sub-option 4-1

- UE-side mix training considering both NW-side and UE-side data for sub-option 4-2/3

Proposal 17: Deprioritize Option 5 in Rel-19.

NEC

Proposal 1: RAN1 to prioritize studying Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4).

Proposal 2: For Direction A, RAN1 to prioritize the following sub-options:

- Option 3a-2/5a-2: Model/Parameters exchanged from the NW-side to UE-side is CSI reconstruction part.

- Option 3a-3/5a-3: Model/Parameters exchanged from the NW-side to UE-side are both CSI generation part and CSI reconstruction part.

- Option 4-2: Dataset exchanged from the NW-side to UE-side consists of (CSI feedback, reconstructed target CSI).

- Option 4-3: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback, reconstructed target CSI).

Intel

Proposal 1:

- Regarding Issue 1 for Direction A, in case of options 3a/5a-1/2/3, consider sharing the following additional information for model training/validation at the stage of UE offline engineering

  - (Target CSI) dataset

  - (Target CSI) dataset characteristics (e.g., channel model, propagation conditions, LoS/NLoS probability, gNB antenna parameters, etc. as applicable)

Proposal 2:

- Option 4-2 is deprioritized for further discussion in RAN1

Proposal 3:

- Regarding Issue 1 for Direction A, consider sharing the following additional information for model verification/testing at the stage of UE offline engineering

  - For options 3a/5a-1 and 4-1/2/3, test CSI reconstruction part, test dataset with the corresponding minimum performance

  - For options 3a/5a-2/3, test dataset with the corresponding minimum performance

Proposal 4:

- Regarding Issue 3 for Direction A, do not support sharing of model/parameters/dataset via air interface

Proposal 5:

- Regarding Issue 4 for Direction A, misalignment of UE-side or NW-side data distribution shall be minimized at the stage of data collection

  - Model refinement at the stage of offline engineering at the NW-side and at the UE-side can be considered if the misalignment is still significant

Observation 5:

- Regarding Issue 5 for Direction B,

  - It is feasible to use a common encoder across UEs if indicated UE capabilities allow,

  - It is feasible for NW-side to train multiple encoders for different UEs assuming the same decoder

- Regarding Issue 6 for Direction B,

  - Training of different encoders across different UEs with a single decoder can be considered to avoid misalignment of UE-side data distributions

Proposal 6:

- Regarding Issue 8 for Direction C, RAN1 assumes 3GPP statistical channel model for dataset generation used for model training in Rel-19

Proposal 7:

- Regarding Issue 9 for Direction C, at least the following solutions can be considered to minimize misalignment of antenna array configuration

  - Realistic assumptions for data generation and/or diversified dataset with various assumptions on antenna configuration

  - Antenna configuration-specific pre-processing (e.g., transform to angular domain)

  - Model refinement/retuning at the UE and/or NW sides respectively with additional datasets based on field data

LGE

Proposal #7: On inter-vendor training collaboration via dataset exchange (i.e., Option 4), consider assistant information regarding relationship between target CSI and reconstructed target CSI to reduce signaling overhead.

Proposal #8: Study on model complexity reduction method, e.g., knowledge distillation, to further reduce the CSI training/signaling complexity for Type 3 training collaboration.

Proposal #9: Support Direction A and prioritize Option 4 for addressing inter-vendor training collaboration.

Panasonic

Proposal 1: For Direction C, RAN4 proposed model structure and/or model parameters for the feasibility study of testing options could be starting point.

Proposal 2: Direction C can be used to define some of “minimum performance”. Further extension on top of Direction C (e.g., the combination with Direction A or B) can be considered.

Proposal 3: RAN1 consider the combination of Direction A and Direction C in Rel.19.

Nvidia

Proposal 5: RAN1 to conclude that it is feasible to resolve issues related to inter-vendor training collaboration for AI/ML-based CSI compression.

Lenovo

Proposal 5: RAN1 to conclude that it is feasible to resolve issues related to inter-vendor training collaboration for AI/ML-based CSI compression.

UE can use information received for decoder model/parameters, to construct a “local decoder model”, then fix (freeze) the weights of that model and use that to train the encoder model.

UE can use the dataset of (CSI feedback, reconstructed target CSI) to first construct a “local decoder model”, then fix (freeze) the weights of that model and use that to train the encoder model.

It is important to have samples of the form (CSI feedback, reconstructed target CSI) and not (CSI feedback, target CSI) so the trained “local” decoder model can have a good match with the decoder model already trained at the gNB side.

Proposal 6: For Direction A, prioritize schemes based on exchange of information regarding the decoder model over options exchanging information of encoder model. It is since, both methods have almost similar 1) signaling overhead, 2) specification complexity, and 3) offline training complexity, while sharing of the decoder model enables the UE to evaluate the performance of the trained encoder model.

- Proposal 7: For different sub-options of Direction A, a) prioritize Option 3-3, 4-3, and 5-3 if the final model intend to support UE-sided root-cause procedures, b) deprioritize Option 3-3, 4-3, and 5-3 in favor of Option 3-2, 4-2, and 5-2, if the final model does not intend to support UE-sided root-cause procedures.

- Proposal 8: Prioritize options based on exchange of information regarding the decoder model, i.e., (CSI feedback, reconstructed target CSI) samples (Option 4-2), over exchange of information regarding the encoder model, i.e., (target CSI,  CSI feedback) samples (Option 4-1) .

- Proposal 9: Despite the higher inter-vendor collaboration complexity, prioritize schemes based on exchange of complete model or options based on dataset exchange over options based on exchange of model parameters only.

- Proposal 10: Despite potentially lower complexity, direct use of received parameters (Direction B) (instead of Direction A) may result in UE encoder with not acceptable performance due to mismatch between NW side data distribution and UE side inference data distribution. Further study is needed in this regard.

- Proposal 11: Until further investigation, give higher priority to options based on Direction A over options based on Direction B.

- Proposal 12: Based on the above observations and proposals, we suggest to prioritize 1) Direction A (with offline engineering) and 2) are based on transfer of the decoder models, i.e., Options 3a-2, 5a-2 and 4-2. Among these options, to enable possibilities for future enhancement, we propose to further prioritize options 5a-2 and 4-2 (highlighted in yellow in the table). In cases where UE-sided root cause procedure is also implemented, we propose to prioritize options 5a-3 and 4-3 (highlighted in orange in the table).

Apple

Observation 1: option 3a-1 and 5a-1 which provide CSI generation reference model, cannot handle UE side additional condition.

Observation 2: option 3a-2/3a-3 which provide CSI reconstruction reference model, can handle UE side additional condition.

Observation 3: option 4-1 which provides [target CSI, CSI-feedback] dataset, when used directly to train encoder, cannot handle UE side additional condition.

Observation 4: option 4-1 which provides [target CSI, CSI-feedback] dataset, when used to train reference decoder first, then retrain encoder using new dataset, has lower performance loss.

Observation 5: To handle UE side additional condition, reference decoder is required. The more accurate of the reference decoder to the actual decoder, the better E2E performance with encoder training.

Observation 6: Inter-vendor training collaboration option 3b and 5b requires advanced on device compiling capability and is challenging to handle the UE side additional condition.

Observation 7: Comparing to legacy codebook, AI model such as transformer-based auto-encoder lacks explainability, making it difficult to ensure Option 1 (fully standardized model) has good performance, and difficult to agree upon one particular model/parameter for RAN1 specification.

Proposal 1: Support option 3a-2, 3a-3, option 5a-2, 5a-3, and option 4-2 and option 4-3 which provide information related to CSI reconstruction model. Reference CSI reconstruction model can ensure E2E performance target, handle UE side additional condition and enable extendibility of future releases.

Nokia

Proposal 1: For temporal domain aspects Case 2 and Case 3, it is encouraged to investigate schemes to exploit temporal correlation at the latent domain with consideration of quantizer/dequantizer with memory, such that the independently developed SF compression (encoder) – reconstruction (decoder) model structures (Case 0) shall be rendered directly usable for Case 2 and Case 3 without modification of its structure nor re-training.

Proposal 2: 3GPP is recommended to consider SQ with priority for the quantization scheme to represent the latent space, unless otherwise there emerges strong analytical or numerical evidence to support the contrary view.

Observation 6: Unlike Directions A and C, Direction B does not allow UE side offline engineering, making it very challenging to address [Issue 6], without compromising [Issue 7]. It is also important to highlight that Issue 6 is not valid for Option 4-1.

Observation 7: Unlike Directions A and C, Direction B alone cannot easily provide a solution to scalability/extendibility problems, i.e., how to handle new UEs and chipsets after the initial decoder deployment, without compromising proprietary information disclosure issue.

Proposal 3: As regards Direction B, 3GPP needs to investigate how to address potential scalability/extendibility problems, i.e., how to handle new UEs and chipsets after the initial decoder deployment, without compromising proprietary information disclosure issue.

Proposal 4: Propose 3GPP to prioritize specification of option 4 in Direction A, as it can be done with relatively manageable effort and its expected E2E performance seems to be on par with other options in Direction A.

ETRI

Proposal 4: For AI/ML-based CSI compression using two-sided model, for the study of inter-vendor training collaborations, prioritize both Direction A and B among 3 directions.

Proposal 5: For AI/ML-based CSI compression using two-sided model, regarding inter-vendor training options and their sub-options of Direction A, prioritize Option 3a-1, 4-1, and 5a-1.

Proposal 6: For AI/ML-based CSI compression using two-sided model, study functionality/model identification procedure for supporting both Direction A and B.

Samsung

Proposal#13 For Issue 9 of Direction C, consider the following steps for evaluation

- Step 1: The reference model with respective UE part (Enc*) and network part (Dec*) is trained based on Dataset#1

- Step2: The UE and network side train their respective actual models to be compatible with the reference models

- UE side trains the actual UE-part (Enc) to be compatible to NW-part (Dec*) of the reference model with training Dataset#2

- Network side trains the actual NW-part (Dec) to be compatible to the UE-part (Enc*) of the reference model with training Dataset#3

- Step3: The actual UE-part and NW-part models are tested on test Dataset#2 or Dataset#3

Consider the impact of the availability of Dataset#1 for Step#2. Training datasets to be constructed by mixing Dataset#1 with respective Dataset#2 and Dataset#3.

Dataset#2 and Dataset#3 can be proprietary datasets such as field dataset.

Consider baseline models with both UE-part and network-part trained on Dataset#2 and Dataset#3 without, respectively, are considered for evaluation

Observation#11: For separate training/fine-tuning of UE-part and NW-part of proprietary models with UE-side and network-side training/fine-tuning datasets to be compatible with fully standardized reference two-sided model for CSI compression (Direction C)

- When the model input is precoding vectors in the spatial-frequency (W) domain, up to 30.5% SGCS degradation is observed as compared to baseline models trained on UE/NW-side training datasets.

- When the model input is precoding vectors in the angle-frequency (W2) domain, a minor 1.5% SGCS degradation is observed as compared to baseline models trained on UE/NW-side training datasets.

Observation#12: It is feasible to separately train/fine-tune UE-part and NW-part of proprietary models to be compatible with fully standardized reference two-sided model with UE-side and network-side training/finetuning datasets, respectively, at least for the case wherein the model input is precoding vectors in the angular-delay (W2) domain.

Observation#13: For separate training/fine-tuning of UE-part and NW-part of proprietary models with UE-side and network-side training/fine-tuning datasets to be compatible with fully standardized reference two-sided model for CSI compression, when the UE-side and NW-side training/fine-tuning dataset includes the dataset for reference model training

- When the model input is precoding vectors in the spatial-frequency (W) domain, up to 1.2% SGCS minor degradation is observed as compared to baseline models trained on UE/NW-side training datasets.

- When the model input is precoding vectors in the angle-frequency (W2) domain, a minor 0.4% SGCS minor degradation is observed as compared to baseline models trained on UE/NW-side training datasets.

Observation#14: For separate training/fine-tuning of UE-part and NW-part of proprietary models to be compatible with fully standardized reference two-sided models (Direction C), including the training dataset for the reference model in the training/fine-tuning dataset improves performance for both cases wherein model input is in the spatial-frequency (W) domain and the angle-frequency (W2) domain.

Proposal#14: Considers at least the following candidate backbone for standardization of model structure:

- CNN

- FFS: on applicability of candidate backbones for Directions A/B/C.

- FFS: on number of layers, layer sizes, input/output dimensions, model and input quantization.

MTK

- Proposal 2: Discuss direct usage of reference two-sided AI/ML model on its one or both parts without additional engineering efforts.

- Proposal 5: Prioritize transferring AI/ML model rather than transferring data through Direction A.

- Proposal 6: Discuss how to consider UE’s computational constraints when encoder is provided by NW.

- Proposal 7: Discuss whether re-engineering on reference AI/ML models are allowed right before their deployment, and if yes, what are the acceptable re-engineering efforts and constraints to do that

Docomo

Proposal 1

- Capture the following views to address the remaining issues of Direction C

  - [Issue 8: Training data] Multiple models/layers can be trained by variant 3GPP models and switched during the deployment. Field data is not mandatory but can be used with Option 1 and 4 combinations to optimize the performance further.

  - [Issue 9: Data mismatch] The performance issue can be solved in multiple ways, such as layer switching or combined with other Options.

  - [Issue 10: Information for training/validation/testing] It is up to RAN4 since the reference model is specified.

Proposal 2

- Capture the following views to address the remaining issues of Direction A

  - [Issue 1: Information for training/validation/testing] Validation/test datasets should be exchanged to validate/test the models offline before deployment.

  - [Issue 2: NW’s proprietary] For Option 3a/4, the concerns can be addressed based on Option 1 as discussed. For Option 5a, there are high risks.

  - [Issue 3: Overhead] The model transfer/delivery overhead is acceptable. However, the overhead for dataset delivery depends on the required dataset size and delivery frequency for different approaches.

  - [Issue 4: Data mismatch] Not a severe issue for Direction A.

Proposal 3

- Study Option 3a/4 by taking Option 1 as a baseline scheme to alleviate the concerns of Issue 1- Issue 3.

Proposal 4

- Deprioritize the study of Option 5a in RAN1.

Proposal 5

- Focus the study on Issue 5 for Direction B (Option 3b).

CAICT

Proposal 1: Dataset exchange between NW and UE OTT server for option 3a/5a should be considered for UE side offline engineering.

Proposal 2: Option 3a/4 is proposed to continue study for UE side offline engineering.

Proposal 3: Option 3b is proposed to continue study for UE side inference directly with on-device operation.

Proposal 4: Fully standardized reference model(s) should be considered for RAN4 testing and flexible adaption mechanism should be considered from RAN1 to ensure the performance in real deployment.

Qualcomm Incorporated

Proposal 1: Prioritize direction A with dataset sharing (4-1, 4-2) or decoder sharing (3a-2, 3a-3, 5a-2, 5a-3) for addressing inter-vendor collaboration complexity in Rel-19 two-sided CSF study.

Proposal 2: Specification of models or synthetic data as additional information in Direction C is not sufficient, indication of trained model to the other side to facilitate offline engineering is needed.

Proposal 3: Specification of model LCM aspects should accommodate models that are designed via proprietary structure and proprietary signallings.

Proposal 4: Over-the-air delivery/transfer of the training dataset, parameter, or reference model to support UE-side model training is not a suitable or scalable solution. Please refer to the issues mentioned in Observation 8:.

Proposal 5: To minimize the inter-vendor collaboration for transferring the training dataset, parameter, or reference model to support UE-side model training while reducing the Uu overhead, the RAN is requested to discuss a solution, where

- The UE can provide required information for sharing the training dataset, parameter, or reference model with the UE-side to the serving gNB, thereafter

- Serving gNB can provide OAM or NF with the required information for sharing the training dataset, parameter, or reference model to the UE-side, thereafter

- OAM or NF then delivers/transfers the training dataset, parameter, or reference model with the UE-side using the information provided from the serving gNB.

Proposal 6: To minimize the inter-vendor collaboration while reducing the Uu overhead, RAN2 is requested to discuss how a standardized solution for offline delivery/transfer of training dataset, parameter, or reference model from the network entity (e.g., RAN/OAM/NF) to the UE-side training server can be supported.

IIT Kanpur

Proposal 1: Prioritize sub-option 1 for Options 3, 4, and 5—specifically, 3a-1 and 5a-1—over the other sub-options.

Proposal 2: For Options 3a and 5a, the NW could share performance targets and/or a portion of the dataset to facilitate UE-side offline engineering and re-training, if necessary. Prioritize a specification impact analysis study for this scenario.

Proposal 3: Prioritize Directions A and B over Direction C, as they provide greater performance potential while requiring less standardization effort.

## Discussion

### Direction A

| Direction A |  |
| --- | --- |
| Issue 1 Additional information | Performance requirement / guidance / target - Huawei, QC, ZTE, vivo, CATT, Ericsson, Fujistu, OPPO, Intel Dataset (in addition to model / parameter exchange) - Huawei, QC, CATT, Ericsson, OPPO, Intel, DCM Model backbone (in addition to dataset exchange) - Spreadtrum, ZTE, vivo, CATT, Ericsson Additional information related to dataset (ID, dataset size) - Vivo, QC, Xiaomi, Ericsson, Fujitsu, OPPO, Intel Quantization - Xiaomi, QC, Fujitsu Preprocessing, post-processing, model scalability - Xiaomi, Ericsson, Fujitsu |
| Issue 2 Proprietary information disclosure | Concern with model / parameter sharing - Ericsson (decoder), Huawei, ZTE, vivo (unspecified model structure, decoder), CATT (decoder), DCM (can be addressed combined with option 1) Concern with dataset sharing - Ericsson (4-2), ZTE (4-1, 4-2) |
| Issue 3 Overhead concern | Parameter sharing less concern than dataset (Futurewei, ZTE, vivo, Xiaomi, CATT, DCM) Comparable for parameter sharing vs. dataset sharing (Huawei) Address the dataset sharing overhead: - multiple pieces (Huawei, Fujitsu) - standardized offline signalling (Futurewei, QC, OPPO, Intel) - reduced by eT2 format - depend on dataset size, delivery frequency (DCM) yes for all sub-options, especially over-the-air (CMCC) Signalling: - OTA Signaling: Huawei, ZTE, CATT - Standardized offline signalling: Futurewei, Xiaomi, QC, Ericsson, Nokia, Intel, OPPO |
| Issue 4 Performance impact due to NW / UE data distribution mismatch | Can be addressed by data collection w/ associated ID (ZTE, CATT, OPPO, Intel) Underlying NW / UE side additional should be aligned (ZTE) How to model NW vs. UE side data distribution mismatch (Ericsson) Specify pre-processing (Ericsson) Timely update by NW (Huawei) Cannot be solely resolved by UE side training, NW model is not generalized to such new UE data (Huawei, Fujistu) Issue only for encode sharing only (CMCC) Addressed by encoder fine-tuning in decoder sharing (QC, OPPO) |

Based on the proposals, a good number of companies elaborate on the additional information that need to be shared from NW side to UE side (issue 1). Regarding this, one aspect is to ensure that a good performance can be achieved via UE offline engineering. To this end, performance requirement may need to be shared, target CSI may be needed as ground-truth so as to assess the performance. Besides, some companies mention model structure misalignment may result in performance degradation, thus sharing model backbone information is beneficial. From this angle, the combination of option 3 and 4 may be a possible way forward for direction A.

Regarding concern on proprietary information disclosure, 5 infra vendors mention the concern of sharing decoder model / parameters. There is less concern of sharing input/output of encoder or decoder.

The main concern on direction A lies in the overhead. A good number of companies mention it may not be feasible to exchange the dataset / parameter / model over-the-air, while two companies prefer over-the-air signaling. From the Feature Lead perspective, RAN1 may focus on conclusion for RAN1 aspect of the over-the-air option, and non-RAN1 aspects of the over-the-air option, as well as other signalling options that may address the concern, can be further discussed by other working group, e.g., RAN2.

For performance impact due to NW / UE side, one company proposes to standardize the pre-processing (phase normalization across frequency) to avoid this issue. Some other companies think this can be addressed by aligning NW side additional condition or association ID. However, the main issue lies in that, considering that NW side decoder is trained using NW data collection of variety of UEs, whether UE offline engineering can train a UE-vendor specific encoder using UE side data of its own UEs against the NW side decoder. This is applied for all UE vendors in initial deployment, and also applied to new UE / chipset vendors whose data has not yet been collected via NW side data collection. Besides, there are also some companies mentioning that the mismatch can be addressed via UE side offline engineering with dataset / decoder sharing (one company shows evaluation). Some other companies mention this can be addressed by timely NW side data collection especially for new UE / chipset vendors. To better study this issue, it may be good to first clarify UE side offline engineering procedures.

##### Proposal 42a: Direction A

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Issue 1: additional information includes

  - Performance requirements (3a/5a/4)

  - Assistant information of the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4)

  - Dataset containing the target CSI (3a/5a)

  - Information related to collecting UE-side training dataset (3a/5a/4)

  - Model backbone information (4)

  - Note: other additional information is not precluded

- Issue 2: There is no proprietary information concern of disclosing encoder input/output from NW side to UE side (4-1).

  - Further discuss whether there is concern for other sub-options under Direction A.

- Issue 3: Exchanging dataset / model / parameter over-the-air may incur large overhead if over-the-air signalling is used. Options to address this issue is up to RAN2 discussion.

- Issue 4: Performance impact due to NW / UE side data distribution mismatch can be at least addressed by

  - For decoder sharing and dataset sharing, UE side offline engineering based on UE side data.

  - NW side timely data collection and decoder (information) update

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | Our comments are as following: For issue 1, if UE needs to train decoder first, model backbone information for Option4 may not be enough, e.g., UE may need more detailed model structure information to train a model that is closely approximating NW decoder model. - Issue 1: additional information includes - Performance requirements (3a/5a/4) - Assistant information of the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4) - Dataset containing the target CSI (3a/5a) - Information related to collecting UE-side training dataset (3a/5a/4) - Model (decoder) backbone/structure information (4) - Note: other additional information is not precluded For issue 2, if we go with the assumption that UE needs to train a decoder as close to NW side as possible, only sharing encoder input/output for 4-1 is not enough. UE would also need information on decoder structure/backbone. Such information would have proprietary information disclosure. - Issue 2: There is proprietary information disclosure concern for 3-2, 3-3, 4-2 and 4-3. Further study whether there is proprietary information disclosure concern if UE needs to train nominal NW decoder first to tune its encoder based on UE side data of disclosing encoder input/output from NW side to UE side (4-1). - Further discuss whether there is concern for other sub-options under Direction A. For issue 3, we don’t think exchanging model parameter would have “large overhead”. For typical model structure at UE, the size would be small. - Issue 3: Exchanging dataset / model / parameter over-the-air may incur large overhead if over-the-air signalling is used. Options to address this issue is up to RAN2 discussion. For issue 4, for dataset sharing, it seems the understanding should be clarified first on whether decoder backbone/structure information needs to be shared. For example, if decoder backbone/structure information is not shared, our understanding is that UE side offline engineering based on UE side data can not be addressed. - Issue 4: Performance impact due to NW / UE side data distribution mismatch can be at least addressed by - For decoder sharing and dataset sharing, UE side offline engineering based on UE side data. - NW side timely data collection and decoder (information) update |
| OPPO | [Issue 1]: we want to clarify that the listed additional information is not mandatory. For some sub-options, only part of them should be exchanged. [Issue 2]: besides option 4-1, we think there’s also no proprietary information disclosure concern for 3a/5a-1 and option 4-2/3. [Issue 3]: the OTA overhead may depend on the standardized model structure and dataset size. We should identify the typical overhead firstly before approving this proposal. Also, may be some AI-related methods can be used to reduce the overhead. |
| Huawei, HiSilicon | For Issue 1: 1) not clear what information is needed for UE side training dataset – UE can obtain its own dataset by implementation if needed. 2) assistance information can include model ID or pairing ID – that is sufficient for UE side data categorization, so there seems no need to add “associated ID” – by definition, associated ID is used for “data collection related configuration(s)”, and is irrelevant with dataset/model delivery procedure. 3) For model backbone, it is our understanding that as long as the target performance is delivered to UE side, the UE side will select appropriate model backbone without knowing what backbone is used at the NW side, e.g., TF can pair with CNN. So, backbone information may not be needed. For Issue 3: Over-the-air overhead is the concern only when the model/dataset are transmitted to a single UE. Realistically the NW will not choose a single UE to complete the delivery procedure. For Issue 4: We can generically say the mismatch can be handled by UE side data collection or NW side update of the model/dataset. For UE side data collection, it may not be limited to -2/-3 series. For issues listed for inter-vendor collaboration Direction A, conclude the following - Issue 1: additional information includes at least - Performance requirements (3a/5a/4) - Assistant information of the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4) - Dataset containing the target CSI (3a/5a) - Information related to collecting UE-side training dataset (3a/5a/4) - FFS Model backbone information (4) - Note: other additional information is not precluded - Issue 2: There is no proprietary information concern of disclosing encoder input/output from NW side to UE side (4-1). - Further discuss whether there is concern for other sub-options under Direction A. - Issue 3: Exchanging dataset / model / parameter over-the-air may incur large overhead if over-the-air signalling is used if the whole dataset/model/parameters are delivered to a single UE. Options to address this issue is up to RAN2 discussion. - Issue 4: Performance impact due to NW / UE side data distribution mismatch can be at least addressed by - For decoder sharing and dataset sharing, UE side offline engineering based on UE side data. - NW side timely data collection and the subsequent decoder (information) update of delivered dataset/model/parameter to UE side. |
| Lenovo | As we have shown, if we want to perform root-cause analysis, it is beneficial to send information regarding the NW-side decoder model to the UE, so for issue-1 we suggest to add: - Issue 1: additional information includes - If not already exchanged, information regarding the NW-side decoder model For issue 4, if we suggest to add the second bullet, as if we want to update the NW-side model it is then increase the vendor collaboration complexity. - Issue 4: Performance impact due to NW / UE side data distribution mismatch can be at least addressed by - For decoder sharing and dataset sharing, UE side offline engineering based on UE side data. - NW side timely data collection and decoder (information) update |
| Panasonic | For Issue 2, we share the similar view to vivo. If only dataset is delivered to UE instead of model structure and/or parameters, there is more uncertainty on the reference model expression. Therefore, Option 4-1 may need the assumption that the model structure is aligned based on offline inter-vendor collaboration. In this case, there might be concern on proprietary information disclosure. |
| Samsung | It is too early to make meaningful conclusion on Issue 4 especially in comparison to Direction B/C. For now, companies do not even have common modelling on the network-side and UE-side data distribution mismatch. We propose to align the assumption on this mismatch among companies. |
| Qualcomm | Regarding model backbone mismatch, we share similar view as Huawei that as long as performance requirement is provided, UE side should optimize the model design to satisfy the requirement either using the shared backbone or not using the shared backbone. The model backbone shared (if can be shared or specified) may help the process, but there should be less performance concern considering performance requirement is given. Regarding issue 4, we think it should be mentioned under which case UE can develop a UE side nominal decoder comparable to the NW side decoder. Without that, UE side encoder design will not be compatible with the NW side decoder. This is shown in our tdoc, Table 1, row 6-8 where encoder sharing only cannot compensate the loss due to NW / UE data distribution mismatch. So, we think we should keep “decoder or dataset sharing”. Besides, only relying on NW side timely data collection and sharing decoder again is not a scalable solution. |
| Ericsson | For issue 1, UE-side needs dataset to verify if the trained actual encoder is compatible with the actual decoder at the NW-side and fulfils the performance requirements. Hence, we propose to add the following bullet. - Dataset or information related to collecting dataset for assessing whether the trained actual encoder is compatible with the actual decoder at the NW-side and fulfils the performance requirements (3a/5a/4) Could the FL explain what does the following bullet mean? - Information related to collecting UE-side training dataset (3a/5a/4) For issue 2, companies have proprietary information disclosure concerns of NW-side sharing decoder related information to the UE-side. Based on the definition, inter-vendor training collaboration sub-options 3a-2, 4-2, 5a-2, 3a-3, 4-3 and 5a-3 belong to this category. For the rest sub-options under direction A (3a-1, 4-1, and 5a-1), it is unclear if the feasibility issue can be fully resolved without NW-side providing decoder related information, hence, requires further study. Therefore, we suggest reformulating the text as the following: - Issue 2: There is no proprietary information concern of disclosing encoder input/output from NW side to UE side (4-1). There is proprietary information disclosure concern for 3a-2, 3a-3, 4-2, 4-3, 5a-2 and 5a-3. - Further discuss whether there is concern for other sub-options under Direction A. For issue 4, more study is need, it is too early to draw the conclusion based on the proposal text now. There is lack of common understanding on what are the main factors that may cause this dataset distribution mismatch, whether it is an issue or not. In addition, there are potential feasibility issues for all listed solutions. - Issue 4: - Study how to model the dataset distribution mismatch for performance impact evaluations. - Further study performance degradation due to NW / UE side data distribution mismatch, considering the feasibility of UE-side obtaining decoder related information from NW-side, the feasibility of NW-side collecting new UE-type specific dataset for its decoder update, and the compatibility of the updated decoder with the already deployed encoder at legacy UEs. Issues identified for Direction C, also applicable for some options in Direction A: The following issues, which are similar to Issue 8 and Issue 9 for Direction C, should be added for sub-options 3a-1, 3a-2, 3a-3 in direction A, where standardized model structure is required: Issue 11 (for sub-options 3a-1, 3a-2, 3a-3 in Direction A): Whether to consider 3GPP’s statistical channel model or field data for the design of reference model structure(s). In case of the latter, how does RAN1 collect field data and agree on them? Issue 12 (for sub-options 3a-1, 3a-2, 3a-3 in Direction A): Is there performance impact due to mismatch between the distribution of the dataset used for designing the reference model structure(s), UE-side data distribution, and NW-side data distribution, and if so, how to address it? For inter-vender training options that required standardized model/model-structure/model-format, it is also a feasibility question on how to represent the model/model-structure/model-format in the standard. Hence, we propose to study the following issue: Issues 13 for Option 3a/5a in Direction A, Direction B (Option 3b), and Direction C (Option 1): How to represent a standardized reference model (structure + parameters), a standardized reference model structure, and/or reference model format in 3GPP specifications? |
| Xiaomi | Issue 1: At least for Option 3a/5a, quantization approach needs to indicate to UE as an additional information. Issue 2: In addition Option4-1, there is no proprietary information concern of disclosing encoder input/output from NW side to UE side for Option3a-1/5a-1. Issue 3: The overhead on model/ parameter could be affordable to transfer via over-the-air considering the size of model/parameter. Issue 4: For the second sub-bullet, assume the updated decoder addresses the issue on data distribution mismatch between NW and the new UE. It is not clear whether the updated decoder could have impact on the performance of original UEs. |
| NTT DOCOMO | For Issue 1, we would like to clarify that the listed items are only candidates and whether to introduce them needs FFS. For Issue 3, RAN1 should first study the frequency of the model/dataset exchange and how much data is required for each option to conclude whether the overhead is an issue. |
| LG | Issue2, All the options on 4 (4-2, 4-3) are no proprietary issue to design the model Issue3, It depends on the model structure and the composition of the parameter. So, it is early to draw this kind of conclusion |
| ZTE | Issue 1: The third and fourth subbullet seem conflict with each other. If the dataset containing the target CSI is shared from the NW side for encoder development at the UE side, does the information related to collecting UE-side training dataset still necessary? Besides, the content of the assistant information in the second subbullet is not clear since the association ID related to the CSI compression use case has not been fully discussed. Issue 2: Whether there is proprietary information concern depends on the additional information discussion in Issue 1. There may be proprietary information disclosure risks if information related to collecting UE-side training dataset or model backbone information is shared from the NW side as additional information. Issue 4: We cannot conclude this without any performance evaluation results. The impacting factors and performance impact of data distribution mismatch are not clear. For the listed method, without any alignment mechanism on the NW-side additional conditions (e.g., TxRU mappings, antenna height, antenna down tilt) and UE-side additional conditions (e.g., UE speed, RF characteristics), it’s hard to say the dataset sharing or UE-side data collection can address the data distribution mismatch issue. Besides, the feasibility of decoder sharing or frequently decoder update is questionable. |
| CATT | [Issue 1]: we tend to agree with Oppo that listed additional information is not mandatory, for some cases only a part of it will be exchanged. One example is the performance requirements/targets, if UE directly trains the actual encoder by mimicking the received encoder, the performance requirements/targets may not be needed. [Issue 2]: we agree with vivo, if nominal decoder is trained at UE side and backbone information is exchanged, there is potential risk of NW proprietary information disclosure. |
| Fujitsu | For issue 1, we think the information related to pre-processing/post-processing and quantization should be shared. Otherwise, it is required to exhaustively attempt all possibility for UE-sided encoder training. So, we have the following suggestion: - Issue 1: additional information includes - Performance requirements (3a/5a/4) - Assistant information of the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4) - Dataset containing the target CSI (3a/5a) - Information related to collecting UE-side training dataset (3a/5a/4) - Model backbone information (4) - Information related to pre-processing and post-processing - Information related to quantization - Note: other additional information is not precluded For issue 3, it depends on the size of exchanged model/parameter/dataset and the method of overhead reduction. Further study may be needed. For issue 4, we think it is not sufficient to draw the conclusion at this stage. UE side offline engineering with UE-sided dataset could only partially address the distribution mismatch issue and the mitigation of performance degradation resulting from this issue should be limited by NW-sided decoder. NW side decoder update may face the extendibility/scalability problem. |

##### Proposal 42b: Direction A

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Issue 1: additional information includes

  - Performance requirements (3a/5a/4)

  - Assistant information of the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4)

  - Dataset containing the target CSI (3a/5a)

  - Information related to collecting UE-side training dataset (3a/5a/4)

  - FFS Model backbone information (4)

  - Note: other additional information is not precluded

- Issue 2: There is no proprietary information concern of disclosing encoder input/output from NW side to UE side (4-1).

  - Further discuss whether there is concern for other sub-options under Direction A.

- Issue 3: Exchanging dataset / model / parameter over-the-air may incur large overhead if over-the-air signalling is used. Options to address this issue is up to RAN2 discussion.

- Issue 4: Performance impact due to NW / UE side data distribution mismatch can be at least addressed by

  - For decoder sharing and dataset sharing, UE side offline engineering based on UE side data.

  - NW side timely data collection and decoder (information) update

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

##### Proposal 42c: Direction A

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Answer to Issue 1: additional information includes

  - Performance requirements (3a/5a/4)

  - Assistant information of the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4)

  - Dataset containing the target CSI (3a/5a)

  - Information related to collecting UE-side training dataset (3a/5a/4)

  - FFS Model backbone information (4)

  - Note: other additional information is not precluded

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | As commented online yesterday, we would like to add the following: For issues listed for inter-vendor collaboration Direction A, conclude the following - Answer to Issue 1: additional information may includes - Performance requirements (3a/5a/4) - Assistant information of the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4) - Dataset containing the target CSI (3a/5a) - Information related to collecting UE-side training dataset (3a/5a/4) - FFS Model backbone/structure information (4) - Note: other additional information is not precluded |
| ETRI | Regarding the Issue 1, we are little unclear on the assistant information of exchanged dataset/model/parameter. Is it for identification of dataset / model / parameter, or does it have some other motivation? |

##### Proposal 42d: Direction A

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Answer to Issue 1: additional information may include

  - Performance requirements (3a/5a/4)

  - Assistant information of associated with the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4)

  - Dataset containing the target CSI (3a/5a)

  - Information related to collecting UE-side training dataset (3a/5a/4)

  - Model backbone information (4)

  - Note: other additional information is not precluded

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

### Direction B

| Direction B |  |
| --- | --- |
| Issue 3 Overhead concern | Yes: - Futurewei, Huawei (repeatedly for each UE), Ericsson (for storage / switching of multiple encoders) No: - ZTE, QC (for model deployment, it’s common to all directions), CATT |
| Issue 5 Feasibility of UE common / specific encoder | Common encoder is not feasible - UE specific side condition (ZTE) specific encoder is not feasible (QC, Nokia) - Model management / scheduling / switching complexity (QC) - Further study performance vs. Direction C considering common encoder (Ericsson) Specific encoder is feasible - ZTE, CMCC, Xiaomi, CATT (group-specific) Important issue for Direction B as it highly depends on UE implementations (DCM) |
| Issue 6 Performance impact due to NW / UE data distribution mismatch | NW needs to fine-tune / update to include UE’s dataset Limited performance Can be addressed by data collection w/ associated ID, model update (ZTE, vivo) Addressed by continuous monitoring (ZTE, CATT) Addressed by NW implementation (CMCC) How to model NW vs. UE side data distribution mismatch (Ericsson) Alleviated by multiple encoder training (Intel) Specify pre-processing (Ericson) How to handle new UE after initial decoder deployment without proprietary information |
| Issue 7 Proprietary information disclosure | Maybe: - For UE specific encoder, information needs to be disclosed to NW side via offline collaboration (Tejas, ZTE, Nokia) No: - CATT, OPPO |
| Additional information | Model parameter quantization, input data pre-processing (Ericsson) |

Based on the proposals regarding direction B, the overhead concern is controversial. Some companies raise the concern because of the parameter needs to be transferred to each UE device repeatedly, while some companies think this is for model deployment rather than model training, thus such overhead is common to both Direction A and B.

The main issues of Direction B is regarding the feasibility of developing UE-specific or UE-common encoders. This issue is also related to performance and/or proprietary information disclosure. More specifically, some companies mention that UE-common encoder is infeasible as each UE / chipset vendor may have their own implementation choice and wish to optimize the model based on the HW/FW implementation. Some companies mention UE-specific encoder is infeasible because it relies on disclosing UE vendor’s proprietary information and more offline inter-vendor collaboration is needed. Also, if encoder is not optimized in a UE / chipset specific manner, its performance in the field may be limited.

##### Proposal 43a: Direction B:

For issues listed for inter-vendor collaboration direction B, conclude the following

- Issue 3: Overhead in Direction B is not a big concern.

- Issue 5:

  - Further study the feasibility of UE-common encoder considering different implementation preferences and optimizations across different UE / chipset vendors and implementations.

  - Further study feasibility of UE-specific or UE-group-specific encoder considering feasibility of disclosing UE-side proprietary information, and feasibility of training and managing multiple encoders at NW-side.

- Issue 5 & 6: Further study performance due to NW / UE data mismatch considering feasibility of training UE-specific / UE-group-specific encoder

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | For issue 5 and 6, we have shown in our evaluations (Section 2.2, issue 6) that mixed training can effectively resolve NW/UE data mismatch and also mismatch between different UEs. Such mixed training can be effectively achieved with NW timely data collection. We can use similar conclusion as issue 4. And for issue 5 itself, we don’t think 3GPP is the right place to study implementation preferences. Thus at least from 3GPP perspective, we can conclude if common encoders are used, whether there are any issues. - Issue 5: - Further study the feasibility of UE-common encoder considering different implementation preferences and optimizations across different UE / chipset vendors and implementations. - Further study feasibility of UE-specific or UE-group-specific encoder considering feasibility of disclosing UE-side proprietary information, and feasibility of training and managing multiple encoders at NW-side. - Issue 5 & 6: Performance impact due to NW / UE side data distribution mismatch can be at least addressed by NW side timely data collection and UE-side encoder update/NW-side decoder update if common encoders are used across UEs. |
| OPPO | [Issue 3]: encoder parameter exchange from NW to UE raises some overhead. Whether overhead is a big concern should be further discussed. [Issue 5]: we agree to further study the feasibility of UE-common encoder and UE-specific or UE-group-specific encoder. But regarding the UE/UE chipset-implementation preferences, we also share similar view to vivo. Hence, it is better to remove the later part of each sub-bullet as: - Further study the feasibility of UE-common encoder. - Further study feasibility of UE-specific or UE-group-specific encoder. |
| Huawei, HiSilicon | For Issue 3, the overhead depends on the structure of the model – from TR38.843, the evaluated TF structure could be as large as tens of MBytes, which we think is not ignorable. Thus, we can first discuss the structure before conclude whether there is overhead concern. For 2nd and 3rd bullets, general OK for the direction of study. We can make separate proposal since they are subject to “further study” instead of a “conclusion” as mentioned in the main text. - Issue 3: Overhead in Direction B depends on later discussion on the potential specified structure is not a big concern.. - Issue 5: - Further study the feasibility of UE-common encoder considering different implementation preferences and optimizations across different UE / chipset vendors and implementations. - Further study feasibility of UE-specific or UE-group-specific encoder considering feasibility of disclosing UE-side proprietary information, and feasibility of training and managing multiple encoders at NW-side. - Issue 5 & 6: Further study performance due to NW / UE data mismatch considering feasibility of training UE-specific / UE-group-specific encoder |
| Lenovo | As in direction B, parameters are needed to be transmitted over the air, Issue 3 still needs to be further studied based on the size of the encoder and how often the transfer needs to be take place - Issue 3: Further study the overhead in Direction B based on the size of the encoder and how often the parameters needs to be exchanged. Issue 6: As simulation results shows, there could be some loss due to mismatch between NW side data distribution and UE side inference data distribution. So, we believe we can first conclude regarding this issue and also we can further study if there are schemes to solve that or not. - Issue 5 & 6: Performance degradation occurs due to mismatch between NW side data distribution and UE side inference data distribution. - Further study performance due to NW / UE data mismatch considering feasibility of training UE-specific / UE-group-specific encoder |
| Panasonic | For Issue 3, we share the view to the above companies. We think further study is needed. |
| ETRI | [Issue3] We see that the potential overhead issue can be arise due to model/parameter exchange via OTA and needs to be carefully considered in Direction B. [Issue5] We see that using a common encoder across various UEs is feasible if AI/ML model structures are standardized, enabling inference operations across multiple UEs. |
| Ericsson | For issue 3, there can be overhead concern for the NW-side, since each gNB needs store the encoder parameters and transfer the model parameters to each UE over-the-air. Such overhead is not common for Direction A and Direction B, since model deployment for Direction A can be done by using other methods (e.g., over-the-top) than over-the-air signalling. - Issue 3: Overhead in Direction B is not a big can be a concern, since each gNB needs to store and transfer encoder parameters to each UE over-the-air. Further study is needed. Issues identified for Direction C, also applicable for Direction B: The following issues, which are similar to Issue 8 and Issue 9 for Direction C, should be added for Direction B, where standardized model structure is required: Issue 11: Whether to consider 3GPP’s statistical channel model or field data for the design of reference model structure(s). In case of the latter, how does RAN1 collect field data and agree on them? Issue 12: Is there performance impact due to mismatch between the distribution of the dataset used for designing the reference model structure(s), UE-side data distribution, and NW-side data distribution, and if so, how to address it? For inter-vender training options that required standardized model/model-structure/model-format, it is also a feasibility question on how to represent the model/model-structure/model-format in the standard. Hence, we propose to study the following issue: Issues 13 for Option 3a/5a in Direction A, Direction B (Option 3b), and Direction C (Option 1): How to represent a standardized reference model (structure + parameters), a standardized reference model structure, and/or reference model format in 3GPP specifications? |
| Xiaomi | We are fine with the discussion on this direction. UE-specific encoder will significantly increase burden of model storage and model management at NW-side considering there are too many different UE-vendors/chip-vendors. The motivation to train UE-specific encoder needs to clarify. |
| NTT DOCOMO | For Issue 3, the overhead of Option 3b is comparable with Option 3a. It should be further studied following the discussions in Direction A. |
| LG | Issue3, similar view as mentioned in advance on Option 3b and it is not much difference compared to Option 3a |
| Futurewei | [Issue 3] Transferring model parameters will incur some overhead concern, however, this issue should consider some relative measure instead; thus, we suggest comparing the overhead incurred due to parameter exchange vs. other options or sub-options, e.g., dataset exchange. In this case, the overhead for exchanging parameters may be lower compared to exchanging datasets. |
| ZTE | For Issue 5, there can be other issues that should be considered for the feasibility study, not only the UE implementation consideration. For example, the field performance of UE-common encoder may be degraded since the encoder has to be compatible with multiple UEs and there is no UE-specific design. |
| CATT | Ok with the proposal |
| Fujitsu | For issue 3, We agree with Lenovo that the size of encoder and how often the transfer needs should be studied before drawing the conclusion. For issue 5 and issue 6, generally fine with this direction. |

##### Proposal 43b: Direction B

For issues listed for inter-vendor collaboration direction B, conclude the following

- Issue 3: Overhead in Direction B is not a big concern.  Further study the overhead in Direction B based on the size of the encoder, the number of encoders, and how often the parameters need to be exchanged.

- Issue 5 & 7:

  - Further study the feasibility and performance of UE-common encoder considering different implementation preferences and optimizations across different UE / chipset vendors and implementations.

  - Further study feasibility of UE-specific or UE-group-specific encoder considering feasibility of disclosing UE-side proprietary information, and feasibility of training and managing multiple encoders at NW-side.

- Issue 5 & 6: Further study performance due to NW / UE data mismatch considering feasibility of training UE-specific / UE-group-specific encoder

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | As we commented in the first round, we don’t know how to study implementation preferences. The essential issue that needs to be studied is if common encoder is used, whether the performance is good if there are different UE data distribution. ##### Proposal 43b: Direction B For issues listed for inter-vendor collaboration direction B, conclude the following - Issue 3: Overhead in Direction B is not a big concern. Further study the overhead in Direction B based on the size of the encoder, the number of encoders, and how often the parameters need to be exchanged. - Issue 5 & 6 & 7: - Further study the feasibility and performance of UE-common encoder considering different implementation preferences and optimizations across different UE / chipset vendors and implementations. - Further study feasibility of UE-specific or UE-group-specific encoder considering feasibility of disclosing UE-side proprietary information, and feasibility of training and managing multiple encoders at NW-side. - Issue 5 & 6: Further study performance due to NW / UE data mismatch considering feasibility of training UE-specific / UE-group-specific encoder |

##### Proposal 43c: Direction B

For issues listed for inter-vendor collaboration direction B, conclude the following

- Issue 3: Overhead in Direction B is not a big concern.  Further study the overhead in Direction B based on the size of the encoder (i.e., number of parameters and quantization level), the number of encoders, and how often the parameters need to be transferred.

- Issue 5 & 7:

  - Further study the feasibility and performance of UE-common encoder considering different implementation preferences and optimizations across different UE / chipset vendors and implementations.

  - Further study feasibility of UE-specific or UE-group-specific encoder considering feasibility of disclosing UE-side proprietary information, and feasibility of training and managing multiple encoders at NW-side.

- Issue 5 & 6: Further study performance due to NW / UE data mismatch considering feasibility of training UE-common / UE-specific encoders

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

### Direction C

| Direction C |  |
| --- | --- |
| Issue 8 Feasibility of using field data in RAN1 training | May not feasible, using synthetic data as starting point Field data can be used by combination of option 1 and 4. (DCM) |
| Issue 9 Performance impact due to mismatch among synthetic data, NW data, UE data | Further evaluation needed, bridged by training using field data (ZTE) Degradation shown by vivo, QC using field data Collect data for ref model training (CATT) Can be used for nominal / minimum performance requirement (Ericsson, Panasonic) Study how to minimize misalignment of antenna array configuration (Intel) Improve by offline engineering (Ericsson, QC (field data), OPPO) Two side independent training may be worse than one-side training (QC) Specify pre-processing of phase normalization across subbands (Ericsson) Minor loss 1.5% if input CSI is angle-frequency domain, 30% loss if input CSI is spatial-frequency. Even minor loss if UE/NW training includes synthetic data (Samsung) Solved by multiple ways, e.g., layer switching or combined with other options (DCM) |
| Issue 10 Additional information | Specify conditions used to train the ref model(s) (Futurewei, OPPO) Performance target (CMCC, OPPO) Dataset or information related to dataset collection (CMCC) Output of decoder (OPPO) |
| Other | Distinguish ref model in RAN1 / RAN4 (OPPO, QC) |

Based on the discussion and proposals for direction C, almost all companies think it is infeasible to use field data to develop a model in RAN1. Regarding the evaluation of performance impact due to mismatch between field data and synthetic data, two companies performed evaluation using actual field data. They observe performance degradation if the model is trained on synthetic data.

Between these two companies, one company further evaluated the performance of performing offline engineering at one side and two sides independently (transparent to each other). It is observed that one sided engineering based on field data can improve the performance compared to running inference using specified model directly. However, performance degradation is observed compared to direction A. It is also observed that offline engineering at both sides independently without coordination can hurt the performance, giving worse performance than offline engineering at one side only.

On the other hand, two companies model synthetic data and field data using two different synthetic channel models. They both observe performance improvement if offline engineering can be done at least at one side. If two sides performance offline engineering independently, one company even observes no performance loss than joint training. However, these studies are based on synthetic data, not actual field data, so the conclusion will be dependent on how different or similar the two synthetic datasets are.

Regarding additional information, some companies mention synthetic data may need to be specified to facilitate offline engineering at two sides. Some companies mention performance requirement is also needed similar to direction A.

##### Proposal 44a: Direction C

For issues listed for inter-vendor collaboration direction C, conclude the following

- Issue 8: It is not feasible for RAN1 to use field data to generate standardized model. RAN1 standardized model is trained using synthetic data as a starting point.

- Issue 9:

  - Performance degradation occurs in the field if the standardized model trained based on synthetic data is used for inference without offline engineering based on field data.

  - Offline engineering based on field data at one side (e.g., training/finetuning at one side based on field data) can partially mitigate the performance degradation. However, its performance is still worse than Direction A.

  - Offline engineering at both sides independently without inter-vendor collaboration may give worse performance than offline engineering at one side.

  - Further study performance degradation due to mismatch between NW side data and UE side data

- Issue 10: Additional information includes

  - Performance requirements

  - Assistant information for dataset alignment (e.g., association ID or model ID or pairing ID)

  - Information related to collecting UE-side training dataset

  - Specification of synthetic data unless both reference decoder and reference encoder are specified

  - Note: other additional information is not precluded

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| OPPO | [Issue 8]: it seems a big work to obtain the standardized model and parameters without a common dataset. |
| Huawei, HiSilicon | For Issue 9, we can make a simple conclusion that the offline engineering, with or without using field data, cannot provide decent performance (compared with other directions). This is because the performance is upper bounded by the pair of specified models. If both sides use different datasets from field, the pairing cannot be ensured. For Issue 10, we think the spirit of Direction C is not to have any interaction between NW side and UE side, otherwise why not provide more information such as model or dataset? - Issue 8: It is not feasible for RAN1 to use field data to generate standardized model. RAN1 standardized model is trained using synthetic data as a starting point. - Issue 9: - Performance degradation occurs in the field if the standardized model trained based on synthetic data is used for inference without offline engineering based on field data. - Offline engineering based on field data at one side (e.g., training/finetuning at one side based on field data) can partially mitigate the performance degradation. However, its performance is still worse than Direction A. - Offline engineering (with or without using field data) at both sides or either side independently without inter-vendor collaboration may not ensure the performance compared with other Directions may give worse performance than offline engineering at one side. - Further study performance degradation due to mismatch between NW side data and UE side data - Issue 10: Additional information includes - Performance requirements - Assistant information for dataset alignment (e.g., association ID or model ID or pairing ID) - Information related to collecting UE-side training dataset - Specification of synthetic data unless both reference decoder and reference encoder are specified - Note: other additional information is not precluded |
| Panasonic | OK in general. For issue 8, future discussion should not be constrained. Therefore, "it is not feasible for RAN1 to use field data to generate standardized model at least for now. |
| Samsung | In our reading, it is too early for RAN1 to make conclusions on Issue 9 and consequently Issue 10. For example, in our contribution, the performance of independent training of the UE/NW parts model w.r.t. reference model is dependent on the input type/domain (angle-delay better than spatial-frequency domain), the datasets used to train the reference and actual models, etc. We propose to align the evaluation assumptions among companies in order to conclude on these issues. We may start with the training and test datasets consideration for reference and actual models training. |
| ETRI | Direction C is based on a standardized reference model and parameter, eliminating the need for offline engineering that’s required in Direction A. However, adding offline engineering to Direction C would significantly increase the overhead and complexity involved in intervendor training collaboration efforts, in our view. |
| Qualcomm | Regarding issue 10, if without performance requirement and additional information on dataset / model, NW and UE may use mismatched dataset with different NW side condition or optimize their model w.r.t. different specified models (if there are multiple models specified). |
| Ericsson | For issue 9, the first three sub-bullets are conclusions drawn based on results provided by only one or two companies. Hence, they should not be captured as a general conclusion, and more study is needed. - Issue 9: - Performance degradation occurs in the field if the standardized model trained based on synthetic data is used for inference without offline engineering based on field data. - Offline engineering based on field data at one side (e.g., training/finetuning at one side based on field data) can partially mitigate the performance degradation. However, its performance is still worse than Direction A. - Offline engineering at both sides independently without inter-vendor collaboration may give worse performance than offline engineering at one side. - Study how to model the distribution mismatch between reference model(s) training dataset, NW-side dataset, and UE-side dataset. - Further study performance degradation due to mismatch between between reference model(s) training dataset, NW side data and UE side data For inter-vender training options that required standardized model/model-structure/model-format, it is also a feasibility question on how to represent the model/model-structure/model-format in the standard. Hence, we propose to study the following issue: Issues 13 for Option 3a/5a in Direction A, Direction B (Option 3b), and Direction C (Option 1): How to represent a standardized reference model (structure + parameters), a standardized reference model structure, and/or reference model format in 3GPP specifications? |
| Xiaomi | Issue 10: As observed in issue 9, offline engineering based on field data at one side could partially mitigate the performance degradation. Performance loss could be further reduced if the retained model also adopts the synthetic data which is used to train reference model. Since the synthetic data could be transferred to other side, it is not necessary to specify the synthetic data. |
| NTT DOCOMO | About the first sentence of Issue 8, we think the 3GPP model is more practical and can be used as a starting point, but the field data is not totally infeasible. We prefer the following wording, It is more practical for RAN1 to use the 3GPP model to generate a standardized model than the field data. For the 2nd and 3rd sub-bullets of Issue 9, RAN1 has not collected enough results to draw a conclusion. It requires further study. For Issue 10, the same clarification as Direction A is needed. The listed ones are candidates for FFS. This does not mean making them mandatory. |
| Sony | We share OPPO’s view |
| Futurewei | For [issue 9], we are ok with the first and last bullets, but we think it’s too early to conclude the 2nd and 3rd bullets. For [issue 10], the purpose or benefit of Direction C is to eliminate or minimize inter-vendor training collaboration. If vendors need to exchange or negotiate additional information for offline engineering purpose, then the benefit of adopting this Direction may no longer exist. |
| ZTE | Further evaluations are needed before having this conclusion. Whether there is any performance impact due to data distribution mismatch depends on the generalization capability of the standardized reference model and the similarity between the synthetic data for training and field data for inference. Therefore, further evaluations are necessary before drawing any hasty conclusions. Currently there were only few companies conducted such evaluations and diverging results were observed. |
| CATT | [Issue 8]: it is fine to just mention “RAN1 standardized model is trained using synthetic data as a starting point.” |

##### Proposal 44b: Direction C

For issues listed for inter-vendor collaboration direction C, conclude the following

- Answer to Issue 8: It is not feasible for RAN1 to use field data to generate standardized model. RAN1 standardized model is trained using synthetic data as a starting point.

- Answer to Issue 9:

  - Performance degradation occurs in the field if the standardized model trained based on synthetic data is used for inference without offline engineering based on field data.

  - Further study the performance with offline engineering

  - Offline engineering based on field data at one side (e.g., training/finetuning at one side based on field data) can partially mitigate the performance degradation. However, its performance is still worse than Direction A.

  - Offline engineering at both sides independently without inter-vendor collaboration may give worse performance than offline engineering at one side.

  - Further study performance degradation due to mismatch between NW side data and UE side data

- Answer to Issue 10: Additional information includes

  - Performance requirements

  - Assistant information for dataset alignment (e.g., association ID or model ID or pairing ID)

  - Information related to collecting UE-side training dataset

  - Specification of synthetic data unless both reference decoder and reference encoder are specified

  - Note: other additional information is not precluded

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

##### Proposal 44c: Direction C

For issues listed for inter-vendor collaboration direction C, conclude the following

- Answer to Issue 8: It is not feasible for RAN1 to use field data to generate standardized model. RAN1 standardized model is trained using synthetic data as a starting point.  rewording to be done

- Answer to Issue 9:

  - Performance degradation occurs in the field if the standardized model trained based on synthetic data is used for inference without offline engineering based on field data.

  - Further study the performance with offline engineering

  - Offline engineering based on field data at one side (e.g., training/finetuning at one side based on field data) can partially mitigate the performance degradation. However, its performance is still worse than Direction A.

  - Offline engineering at both sides independently without inter-vendor collaboration may give worse performance than offline engineering at one side.

  - Further study performance degradation due to mismatch between NW side data and UE side data

- Answer to Issue 10: Additional information includes

  - Performance requirements

  - Assistant information for dataset alignment (e.g., association ID or model ID or pairing ID)

  - Information related to collecting UE-side training dataset

  - Specification of synthetic data unless both reference decoder and reference encoder are specified

  - Note: other additional information is not precluded

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

##### Proposal 44d: Direction C

For issues listed for inter-vendor collaboration direction C, conclude the following

- Answer to Issue 8: RAN1 will consider 3GPP’s statistical channel model.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

### Way forward on Directions

Based on the proposals on prioritization of directions, FL observes that majority interest is lying on Direction A. There are 4 companies support direction C, given that offline-engineering on top of the specified model may improve the performance. Among them, one company mention combination of direction A and C can be considered with the understanding that direction C is used as minimum performance requirement and vendors can improve the performance in the field via exchanging dataset / model / parameter along direction C. Besides, there are 3 companies interested in direction B.

Prioritization (based on companies’ proposals)

- Direction A (vivo, QC, CMCC, TCL, Google, NEC, LGE, Lenovo, Nokia, ETRI, DCM (combined with option 1), CAICT, Panasonic (combined with direction C))

  - Sharing encoder (Spreadtrum, Tejas, ZTE, Google, CATT, ETRI, DCM)

  - Sharing decoder (QC, Lenovo, Apple, Fujistu, NEC, DCM)

  - Dataset 4-1 (Tejas, ZTE, QC, CATT, LGE, Nokia, ETRI, DCM)

  - Dataset 4-2 (QC, Lenovo, Apple, Fujistu, NEC, LGE, Nokia, DCM)

- Direction B (CMCC, vivo, ETRI)

- Direction C (Ericsson, Panasonic, Samsung, DCM (no offline engineering), ZTE?)

##### Proposal 45a: way forward on Directions

For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, focus on the following

- Consider a fully specified reference model for minimum performance requirement (Direction C)

  - It is clarified that, in RAN1, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side.

    - It is up to the UE-side (NW-side) whether to directly implement the fully specified CSI generation (reconstruction) part.

  - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3)

  - FFS whether the reference model is the same as RAN4 specified model

  - FFS feasibility and performance of using fully specified reference models or their compatible models for deployment at NW and UE

    - Including feasibility and performance of training/finetuning the models based on field data and the corresponding need of inter-vendor collaboration.

- Consider inter-vendor collaboration with dataset exchanged from NW-side to UE-side (Direction A, with Direction B as a special case)

  - FFS down-selection among the following,

    - 4-1: dataset is {target CSI, CSI feedback},

    - 4-2: dataset is {CSI feedback, reconstructed CSI}

    - 3a: dataset is {target CSI}, in addition to encoder parameter exchange

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | We would like to revise the way forward as following: ##### Proposal 45a: way forward on Directions For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, focus on the following - Consider a fully specified reference model for minimum performance requirement (Direction C) - It is clarified that, in RAN1, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side. - It is up to the UE-side (NW-side) whether to directly implement the fully specified CSI generation (reconstruction) part. - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3) - FFS whether the reference model is the same as RAN4 specified model - FFS feasibility and performance of using fully specified reference models or their compatible models for deployment at NW and UE - Including feasibility and performance of training/finetuning the models based on field data and the corresponding need of inter-vendor collaboration. - Consider inter-vendor collaboration with dataset/model exchanged from NW-side to UE-side (Direction A, with Direction B as a special case) - FFS down-selection among the following for direction A, - 4-1: dataset is {target CSI, CSI feedback}, - 4-2: dataset is {CSI feedback, reconstructed CSI} - 3a: dataset is {target CSI}, in addition to encoder parameter exchange |
| OPPO | We are confused about the current way forward version. Regarding the first sub-bullet (Direction C), it means we should go with direction C firstly? But as per our understanding, the effort to fully specify the model and parameters is very large, the purpose of doing this is only to satisfy the minimum performance requirement? Regarding the second sub-bullet, it seems only option 4 is included by saying “with dataset exchanged from NW-side to UE-side”. Reading this proposal, we don’t know how FL will treat options/sub-options with encoder and/or decoder parameter exchange. |
| Huawei, HiSilicon | One clarification question: what does it mean to say “Direction B as a special case” of “dataset exchange”? - Consider inter-vendor collaboration with dataset exchanged from NW-side to UE-side (Direction A, with Direction B as a special case) In addition, for “3a: dataset is {target CSI}, in addition to encoder parameter exchange”, does it mean 3a-1: dataset is {target CSI}, in addition to encoder parameter exchange? |
| Lenovo | We are generally fine with this direction. We are not sure about the FFS on second main bullet and if you mean to remove other options like 3a-1, 3a-2 and … . So we propose to remove the examples. Although Direction A and Direction B are similar, due to their online and offline flavour they result in different specifications impact. So we suggest to make them separate. - Consider inter-vendor collaboration with dataset/model exchanged from NW-side to UE-side (Direction A, with Direction B as a special case) - FFS down-selection among different suboptions. the following, - 4-1: dataset is {target CSI, CSI feedback}, - 4-2: dataset is {CSI feedback, reconstructed CSI} - 3a: dataset is {target CSI}, in addition to encoder parameter exchange |
| Panasonic | We are fine with this direction. |
| Samsung | Ok with the way forward. However, we would like to add the following study - Consider inter-vendor collaboration with dataset exchanged from NW-side to UE-side (Direction A, with Direction B as a special case) - FFS down-selection among the following, - 4-1: dataset is {target CSI, CSI feedback}, - 4-2: dataset is {CSI feedback, reconstructed CSI} - 3a: dataset is {target CSI}, in addition to encoder parameter exchange - FFS feasibility and performance of using exchanged models or their compatible models for deployment at NW and UE - Including feasibility and performance impact of training/finetuning the models based on UE-side or NW-side data in addition to the exchanged reference model/dataset. |
| ETRI | Regarding the first bullet point, we’re unclear about “for minimum performance requirement”. Is it accurate to assume that the goal of Direction C is to ensure a minimum performance level for AI/ML-based CSI feedback? We believe that this objective isn’t necessary for AI/ML-based CSI feedback, as we already have legacy CSI feedback mechanisms that can provide the minimum required performance. For the second bullet, we agree on the modification of Lenovo. |
| Ericsson | Could FL explain the intention for this proposal? How it is related to the discussion about studying the issues identified for each direction and checking the feasibility of each inter-vendor training sub-option? It seems that some option/sub-options (e.g., option 5 and sub-option 4-3) have been removed from this proposal? Can we confirm this? What does it mean by “Direction A, with Direction B as a special case”? Is this a new direction? Do we need to re-discuss and identify issues for this one? |
| NTT DOCOMO | We are fine with the proposal. A reference model is necessary to solve the concerns of Direction A/B. |
| Sony | We are generally fine with this direction |
| Futurewei | We are ok with the edits from Lenovo. |
| ZTE | Too early to have this conclusion. There are still on-going discussions about the feasibility of different directions and many issues have not been determined yet, such as the encoder training procedures for direction A, the content and overhead of additional information sharing from NW to UE, the impact of data distribution mismatch, etc. Therefore, we suggest to postpone the down selection of different directions as implied in this proposal. |
| CATT | We are interested in Direction B too. Generally fine with this way forward. |
| Fujitsu | Generally fine with the proposal from FL. |

##### Proposal 45b: way forward on Directions

For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression,

- Conclude that the following two Directions are needed for the normative work.

  - Direction A, B or both

  - Direction C

- For Direction A:

  - Further study the following options with potential down-selection

    - 4-1: dataset is {target CSI, CSI feedback},

    - 4-2: dataset is {CSI feedback, reconstructed CSI}

    - 3a-1: dataset is {target CSI}, in addition to encoder parameter exchange

- For Direction C:

  - The fully specified reference model is at least for minimum performance requirement.

    - Study whether the fully specified reference models or their compatible models may be used for deployment at NW and UE

      - Including performance evaluation of training/finetuning the models based on field data and the corresponding need of inter-vendor collaboration.

  - It is clarified that, in RAN1, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side.

    - It is up to the UE-side (NW-side) whether to directly implement the fully specified CSI generation (reconstruction) part.

  - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3)

  - FFS whether the reference model is the same as RAN4 specified model

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | ##### We would like to update as following. ##### Proposal 45b: way forward on Directions For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - Focusing on Conclude that the following two Directions for Rel-19 study are needed for the normative work. - Direction A, B or both - Direction C - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 4-2: dataset is {CSI feedback, reconstructed CSI} - 3a-1: dataset is {target CSI}, in addition to encoder parameter exchange, a dataset with {target CSI} may also be transmitted. - For Direction C: - The fully specified reference model is at least for minimum performance requirement. - Study whether the fully specified reference models or their compatible models may be used for deployment at NW and UE - Including performance evaluation of training/finetuning the models based on field data and the corresponding need of inter-vendor collaboration. - It is clarified that, in RAN1, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side. - Study whether the fully specified reference models and their compatible models may be used for deployment at NW and UE - Including performance evaluation of training/finetuning the models based on field data and the corresponding need of inter-vendor collaboration. - It is up to UE/gNB’s implementation on use of fully specified CSI generation (reconstruction) part as long as above compatibility purpose is satisfied the UE-side (NW-side) whether to directly implement the fully specified CSI generation (reconstruction) part. - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3) - FFS whether the reference model is the same as RAN4 specified model |
| ETRI | For the first bullet, can FL explain about “Direction A, B or both”?. Does it mean A or B, or both of them? By the way, both directions are needed to be considered for alleviating intervendor training collaboration at this stage, in our view. For the second bullet (Direction A), the dataset for option 3a-1 is an additional and optional information for performance validation or alleviation of performance from dataset misalignment, to our understanding. Based on this, proposed modification based on vivo’s is below: - 3a-1: dataset is {target CSI}, in addition to encoder parameter exchange, a dataset with {target CSI} may or may not also be transmitted. |

##### Proposal 45c: way forward on Directions

For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression,

- Conclude that the following two Directions are needed for the normative work.

  - Direction A, B or both

  - Direction C

- For Direction A:

  - Further study the following options with potential down-selection

    - 4-1: dataset is {target CSI, CSI feedback},

    - 4-2: dataset is {CSI feedback, reconstructed CSI}

    - 3a-1: encoder parameter exchange, with {target CSI}

    - Note: Target CSI is needed at UE-side but may not need to be exchanged if UE-side has other means of obtaining it.

    - Note: Additional information beyond what’s listed above may needs to be exchanged if necessary for the UE-side encoder development/training.

- For Direction C:

  - The fully specified reference model is at least for minimum performance requirement.

  - It is clarified that, in RAN1, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side.

    - It is up to the UE-side (NW-side) whether to directly implement the fully specified CSI generation (reconstruction) part as long as above compatibility purpose is satisfied.

  - Study whether the fully specified reference models or their compatible models may be used provide reasonable performance for deployment at NW and UE

    - Including performance evaluation of training/finetuning the models based on field data (either by modeling data distribution mismatch and/or by using actual field data)

      - Training/finetuning done at either or both sides without inter-vendor collaboration. The results may be compared to the performance of inter-vendor collaboration.

      - Note: companies may evaluate training/finetuning without inter-vendor collaboration (Direction C) and/or with inter-vendor collaboration (Direction A/B)

    - Including performance evaluation of NW-side and UE-side data distribution mismatch

  - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3)

  - FFS whether the reference model is the same as RAN4 specified model

  - FFS whether a single or multiple reference (and how many) models need to be specified

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

### Method of exchange

For Direction A, companies’ preferences are

- OTA Signaling: Huawei, ZTE, CATT

- Standardized offline signalling: Futurewei, Xiaomi, QC, Ericsson, Nokia, Intel, OPPO

##### Proposal 46a: method of exchange for Direction A

Study solutions for standardized signaling for dataset / parameter / model exchange from the NW-side to the UE-side server in inter-vendor training collaboration Options 3a/4/5a.

From RAN1 perspective, solutions include

- Over the air-interface, delivered to the UE-side server via UE(s)

- Offline delivery

- Other options within/beyond RAN1 scope are not precluded.

Aspects to consider includes, at least,

- Feasibility

- Overhead of exchange

- Inter-vendor collaboration complexity

Note: Assessment of any of the above solutions requires studies by RAN2 and/or other working groups.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

### Model structure specification

There are 3 companies (Huawei, vivo, Samsung) discussing model structure and design aspects.

- Model design aspects, scalability, e.g., model input type, rank>1 options, generalization aspects, scalability approaches, quantization/dequantization method, etc (Huawei)

- General procedures for model / model structure standardization (vivo)

- Candidate backbone, e.g., TF, CNN (vivo, Samsung)

- Determine hyper-parameters, input/output dimension, #layers, size, quantization precision (vivo, Samsung)

We have the following agreement from the last meeting.

Agreement For studying the standardization of model structure RAN1, RAN1 assumes at least the following: - Precoding matrix as an input (as opposed to raw channel matrix) - Per-layer processing, with common structure across ranks and layers (corresponding to Option 2-1, 2-2, 3-1, or 3-2 for handling rank ≥ 1) - For temporal domain aspects Case 2 and Case 3, strive to reuse the model structure of Case 0 where appropriate, with additional layers or operations either at the input/output domain or at the latent domain. - For Case 0, use precoding matrix, e.g., eigen vector in spatial-frequency domain, and/or angular, delay domain representation such as eType-II W2. - For Case 2 and Case 3, use precoding matrix, e.g., spatial-frequency domain for each CSI observation instance, and/or angular, delay, and Doppler domain representation such as eType-II W2. - Consider scalability approaches over numbers of Tx ports, CSI feedback payload sizes, and bandwidths.

According to the agreement, RAN1 may strive to reuse the model structure of Case 0 where appropriate, with additional layers or operations either at the input/output domain or at the latent domain.

For evaluations of Case 0, companies used the following backbone structures in their evaluations:

- Spatial-frequency domain input

  - Transformer [China Telecom, Intel, Lenovo, Qualcomm, ETRI, Huawei, vivo, ZTE, Xiaomi, Fujitsu, OPPO, MediaTek, NTT DOCOMO, Nokia, Apple, CATT, CMCC

  - CNN [BJTU, InterDigital]

- Angular-delay domain input

  - ResNet [Samsung, Ericsson]

  - Bi-LSTM [Samsung]

- Angular-frequency domain input

  - Transformer [InterDigital]

For Case 2, for additional layers or operations on top of case 0, companies used the following approaches

- Spatial-frequency domain input

  - Input/output adaptation

    - Conv-LSTM [Fujitsu, ZTE, Apple, CATT, CMCC, Xiaomi, ETRI, Nokia]

    - LSTM [Huawei, IIT Kanpur]

  - Latent adaptation

    - Reusing Case 0 structure and only changing quantization operation [Qualcomm, Nokia]

- Angular-delay domain input

  - Pre-processing [Samsung, Ericsson]

- Angular-frequency domain input

  - Pre-processing [InterDigital]

For Case 3, for additional layers or operations on top of case 0, companies used the following approaches

- Spatial-frequency domain input

  - Input/output adaptation

    - Scale NN input/output by number of TD samples [Vivo, QC, Ericsson, MediaTek, ZTE]

  - Latent adaptation

    - Combine after each TD sample goes through the encoder [CATT]

- Angular-delay-Doppler domain input

  - Pre-processing [Samsung]

##### Proposal 47a: model structure specification

For model structure specification,

- For temporal domain Case 0,

  - In case of spatial-frequency domain input, adopt Transformer as the backbone structure.

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 2,

  - In case of spatial-frequency domain input, reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation on top of Case 0 model structure

      - Conv-LSTM

      - LSTM

    - Option 2: Latent adaptation on top of Case 0 model structure

      - Reusing Case 0 structure and only changing quantization operation

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 3,

  - In case of spatial-frequency domain input, reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation

    - Option 2: Latent adaptation

  - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | First, the model structure is for study. Secondly, for “reuse the model structure of Case 0 with an adaptation”, if the input/output dimension to the TF part changes, the model structure of the TF part in Case 2/3 may also be different from Case 0. It may be too strong to say “reuse”. To be safe, we can replace it with “take into account”. For studying the standardized model structure specification, - For temporal domain Case 0, - In case of spatial-frequency domain input, adopt Transformer as the backbone structure. - Further study angular-frequency and angular-delay domain input - For temporal domain Case 2, - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation with additional layerson top of Case 0 model structure - Conv-LSTM - LSTM - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and only changing quantization operation - Further study angular-frequency and angular-delay domain input - For temporal domain Case 3, - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation - Option 2: Latent adaptation - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input. |
| Panasonic | In Proposal 45a, whether the reference model is the same as RAN4 specified model is FFS. In our understanding, RAN4 discussed the detailed model diagrams based on CNN. In our view, RAN4 proposed model structure for the feasibility study of testing options could be starting point. We are OK if the proposal is for study not for specification. |
| Samsung | Is the above applicable to all directions Direction A (3a)/B/C? In our view, the considerations for the different directions can be differentiated, e.g., considering Direction B requires simpler and widely deployable model. Moreover, we showed W2-domain compression has benefits both in terms their usage in Direction C as well as generalization performance. It is useful to capture at least CNN model structure is applicable for this domain. We support HW’s update to formulate the proposal in to further study. |
| ETRI | In our view, companies consider CNN as another type of backbone network and it could be another option of backbone network. |
| Futurewei | Our understanding is that the proposal is for studying reference model architecture, not for finalizing the model backbone at this time. If this is the case, we suggest clarifying the proposal to reflect the “study” aspect like what was suggested by Huawei “For studying the standardized model structure”. For studying purpose, we agree with Panasonic’s view that the model architecture proposed in RAN4 may be considered as an option as well. |
| Fujitsu | We have the following comment and suggestion for temporal domain Case2. - For temporal domain Case 2, - In case of spatial-frequency domain input, reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation on top of Case 0 model structure - Conv-LSTM - LSTM - Accumulated past CSI with conv-LSMT/LSTM - Concatenated past CSI with one or multiple slots - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and only changing quantization operation and/or feedback payload over different slots - Further study angular-frequency and angular-delay domain input |
| vivo | For temporal domain Case 2, we have additional inputs as follows: - For temporal domain Case 2, - In case of spatial-frequency domain input, reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation on top of Case 0 model structure - Conv-LSTM - LSTM - Adding (processed) previous latent feature to the input of current slot processing - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and only changing quantization operation - Reusing Case 0 transformer structure and using attention with longer range - Further study angular-frequency and angular-delay domain input Particularly, we explain our proposal with more details: For option1, we propose to add the (processed) latent feature of previous spatial-frequency domain CSI (i.e., the feature just before quantization) to the (embedded) input of next spatial-frequency domain CSI (encoder part is illustrated in the following figure for understanding, and the decoder part can be handled in a similar way). The design does not rely on additional blocks/layers such as LSTM or Conv-LSTM to leverage temporal domain correlation, which simplifies the structure. For option2, we propose to reuse Case 0 transformer structure and consider attention with longer CSI context in each Transformer block to leverage temporal domain correlation. An illustration can be referred to the following figure (encoder part is illustrated, and the decoder can be handled in a similar manner). |

##### Proposal 47b: model structure specification

For studying the standardized model structure specification,

- For temporal domain Case 0,

  - In case of spatial-frequency domain input, adopt Transformer as the backbone structure.

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 2,

  - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation with additional layerson top of Case 0 model structure

      - Conv-LSTM

      - LSTM

    - Option 2: Latent adaptation on top of Case 0 model structure

      - Reusing Case 0 structure and only changing quantization operation and/or feedback payload over different slots

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 3,

  - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation

    - Option 2: Latent adaptation

  - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | ##### Proposal 47b: model structure specification For studying the standardized model structure specification, - For temporal domain Case 0, - In case of spatial-frequency domain input, adopt Transformer as the backbone structure. - Further study angular-frequency and angular-delay domain input - For temporal domain Case 2, - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation with additional layerson top of Case 0 model structure - Conv-LSTM - LSTM - Adding (processed) previous latent feature to the input of current slot processing - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and only changing quantization operation and/or feedback payload over different slots - Reusing Case 0 transformer structure and using attention with longer range - Further study angular-frequency and angular-delay domain input - For temporal domain Case 3, - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation - Option 2: Latent adaptation - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input. |

### Evaluation studies for Directions

##### Proposal 41a: Further clarification of UE side offline engineering in direction A

For Direction A, it is clarified that

- For decoder model / parameter exchange from NW-side to UE-side (3a-2, 5a-2), UE-side develops its actual encoder against the exchanged decoder based on UE-side data collection.

- For encoder model / parameter exchange from NW-side to UE-side (3a-1, 5a-1),

  - Step 1: UE-side develops a nominal decoder against the exchanged encoder.

    - If dataset is also exchanged from NW-side to UE-side as additional information, the nominal decoder is developed using the dataset.

    - Otherwise, the reference decoder is developed based on UE-side data collection.

  - Step 2: UE-side develops actual encoder against the nominal decoder based on UE-side data collection

- For dataset exchange from NW-side to UE-side ({target CSI, CSI feedback} of 4-1, {CSI feedback, reconstructed CSI} of 4-2),

  - Step 1: UE-side develops a nominal decoder using the exchanged dataset.

  - Step 2: UE-side develops an actual encoder against the nominal decoder based on UE-side data collection.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | For model (encoder) parameter sharing and dataset sharing ({target CSI, CSI feedback}), there is another way of training. UE can directly train its encoder rather than first train a decoder. Without information of decoder backbone, the trained decoder at UE side would be dramatically different from NW side, which would result in performance degradation when UE train its encoder. |
| OPPO | We share the same view to vivo. Nominal decoder is not necessary for UE to train its UE-side actual encoder. Both two ways should be considered. |
| Huawei, HiSilicon | 1) The methods for UE side to train UE part model is UE vendor implementation. So the proposal should add “example” in the main bullet and a note that “other training examples are not precluded”. 2) For 3a-2/5a-2, UE can either use UE side data or NW shared dataset of Target CSI (which we believe is more reasonable). 3) For 3a-1/5a-1, UE can either first train a nominal decoder, or directly train the actual encoder by mimicking the delivered encoder (as also mentioned by vivo, OPPO). 4) For 4-1/4-2, similarly, UE can either first train a nominal decoder, or directly train the actual encoder based on the delivered dataset, e.g., using CSI feedback as the label, and using Target CSI/ reconstructed CSI as input. For Direction A, as an example of UE side training implementation, it is clarified that - For decoder model / parameter exchange from NW-side to UE-side (3a-2, 5a-2), UE-side can develops its actual encoder against the exchanged decoder based on UE-side data collection or dataset delivered from NW side. - For encoder model / parameter exchange from NW-side to UE-side (3a-1, 5a-1), - Alt. 1: UE first trains a nominal decoder - Step 1: UE-side can develops a nominal decoder against the exchanged encoder. - If dataset is also exchanged from NW-side to UE-side as additional information, the nominal decoder is developed using the dataset. - Otherwise, the reference decoder is developed based on UE-side data collection. - Step 2: UE-side develops actual encoder against the nominal decoder based on UE-side data collection - Alt. 2: UE directly trains the actual encoder. UE can develop the encoder and train it by mimicking the encoder delivered by NW. - For dataset exchange from NW-side to UE-side ({target CSI, CSI feedback} of 4-1, {CSI feedback, reconstructed CSI} of 4-2), - Alt. 1: UE first trains a nominal decoder - Step 1: UE-side can develops a nominal decoder using the exchanged dataset. - Step 2: UE-side can develops an actual encoder against the nominal decoder based on UE-side data collection or dataset delivered from NW side. - Alt. 2: UE directly trains the actual encoder. UE can develop the encoder and train it by using the delivered CSI feedback as label. - Other examples for UE side training is not precluded. |
| Lenovo | We also believe there are methods based on direct training of encoder model based on information received for the encoder model. - For encoder model / parameter exchange from NW-side to UE-side (3a-1, 5a-1), - Step 1: UE-side develops a nominal decoder against the exchanged encoder. - If dataset is also exchanged from NW-side to UE-side as additional information, the nominal decoder is developed using the dataset. - Otherwise, the reference decoder is developed based on UE-side data collection. - Step 2: UE-side develops actual encoder against the nominal decoder based on UE-side data collection , UE-side develops its actual encoder against the exchanged encoder model based on UE-side data collection. |
| Panasonic | We are fine with the proposed clarification. |
| Samsung | We are also fine to start the discussion with at least the above training considerations. |
| ETRI | In our view, the trianing method can be different vendor to vendor and no need to discussed in RAN1. Moreover, we don’t see much reason that mandates the nominal decoder for the exchanged encoder (for case 3a/5a-1) for training actual encoder. For example, a UE-side can train their actual encoder directly using the exchanged encoder (i.e., exchanged encoder can generates labels for the UE-side’s input). |
| Qualcomm | We don’t think UE side will train encoder directly without building a nominal decoder. - If UE trains the encoder using NW side shared data, this will upper-bound UE side training performance, as the outcome will be just mimicking the NW side encoder. If UE trains the encoder using UE side data, UE has no idea how it is compatible with the NW side decoder and cannot assess whether meet the performance requirement. |
| Ericsson | Some sub-options belong to Direction A are missing in this proposal. Does this proposal imply that options 3a-3, 4-3 and 5a-3 are not supported in 3GPP? Regarding the UE-side offline engineering procedures described for options 3a-1, 5a-1 and 4-1, agree with Vivo and Oppo that there can be different ways for the UE- sided to develop an actual encoder. Since these offline engineering procedures will not be specified, can we ensure that the UE-side follows the clarified procedures? If not, the feasibility study cannot rely on that UE-side implementation is based on the clarified procedures. Regardless of what procedures are used at the UE-side for training an actual encoder, methods are needed to enable that the trained actual encoder is compatible with the actual decoder used at the NW, i.e., the end-to-end paired model performance shall meet a minimum performance requirement. Hence, it is more important to clarify how this minimum end-to-end performance can be achieved for options 3a-1, 5a-1 and 4-1. |
| Xiaomi | We are fine with the discussion on this direction. |
| NTT DOCOMO | We agree with OPPO/vivo/HW’s comments that developing a decoder is only some of the approaches. We either list all options or remove it. |
| Sony | We support the clarification |
| LG | Fine with the direction, but it seems that the steps for second and third bullet is a kind of example to construct UE-side model |
| Futurewei | We share similar view as ETRI that how to train the actual encoder depends on UE-side implementation and there is no need to mandate the procedures. The steps mentioned in the proposal can serve as an example. |
| ZTE | - Whether the dataset for UE-side offline engineering is collected by the UE or transferred by the NW is not determined yet, depending on whether/how the underlying NW/UE-side additional condition associated with the dataset is aligned between both sides. Therefore, for the first subbullet, it should be open for both UE-side data collection and dataset transferred from NW. For the second subbullet, there can be another possible way for encoder development, that is, the UE directly train a new encoder by mimicking the input/output mapping relationship of the transferred encoder, in which case the performance target is subject to the loss of the encoder output. Similar training method applies to the dataset exchange option in the third subbullet. |
| CATT | We are ok with the clarification proposed by Huawei. |
| Fujitsu | Generally agree with the view from vivo and Huawei. |

##### Proposal 41b: evaluation study for Direction A

For the evaluation studies of Direction A, it is clarified that

- For decoder model / parameter exchange from NW-side to UE-side (3a-2, 5a-2), UE-side develops its actual encoder against the exchanged decoder based on UE-side data collection.

  - Alt 1a: The actual encoder is developed/trained based on UE-side data collection.

  - Alt 1b: Dataset is also exchanged from NW-side to UE-side as additional information, and the actual encoder is developed/trained based on the exchanged dataset.

- For encoder model / parameter exchange from NW-side to UE-side (3a-1, 5a-1),

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder against the exchanged encoder.

      - Alt 1a: Otherwise, The reference nominal decoder is developed based on UE-side data collection.

      - Alt 1b, 1c: If Dataset ({target CSI}) is also exchanged from NW-side to UE-side as additional information, and the nominal decoder is developed using the exchanged dataset and the exchanged encoder.

    - Step 2: UE-side develops actual encoder against the nominal decoder based on UE-side data collection

      - Alt 1a, 1c: The actual encoder is developed/trained based on UE-side data collection.

      - Alt 1b: The actual encoder is developed/trained based on the exchange dataset from the NW-side.

  - Alt. 2: UE-side directly develops and trains the actual encoder by mimicking the encoder exchanged by NW.

    - Alt 2a: The actual encoder is developed based on UE-side data collection.

    - Alt 2b: Dataset ({target CSI}) is also exchanged from NW-side to UE-side as additional information, and the actual encoder is developed using the dataset.

- For dataset exchange from NW-side to UE-side ({target CSI, CSI feedback} of 4-1, {CSI feedback, reconstructed CSI} of 4-2),

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder using the exchanged dataset.

    - Step 2: UE-side develops an actual encoder against the nominal decoder based on UE-side data collection.

      - Alt 1c: The actual encoder is developed/trained based on UE-side data collection.

      - Alt 1b: The actual encoder is developed/trained based on the exchange dataset from the NW-side.

  - Alt. 2: UE-side directly develops and trains the actual encoder.

    - Alt 2b: The actual encoder is developed using the dataset.

- Other alternatives are not precluded.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | ##### Proposal 41b: evaluation study for Direction A For the evaluation studies of Direction A, it is clarified that - For decoder model / parameter exchange from NW-side to UE-side (3a-2, 5a-2), UE-side develops its actual encoder against the exchanged decoder based on UE-side data collection. - Alt 1a: The actual encoder is developed/trained based on UE-side data collection. - Alt 1b: Dataset is also exchanged from NW-side to UE-side as additional information, and the actual encoder is developed/trained based on the exchanged dataset. - For encoder model / parameter exchange from NW-side to UE-side (3a-1, 5a-1), - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder against the exchanged encoder. - Alt 1a: Otherwise, The reference nominal decoder is developed based on UE-side data collection. - Alt 1b, 1c: If Dataset ({target CSI}) is also exchanged from NW-side to UE-side as additional information, and the nominal decoder is developed using the exchanged dataset and the exchanged encoder. - Step 2: UE-side develops actual encoder against the nominal decoder based on UE-side data collection - The actual encoder is developed/trained based on UE-side data collection and/or exchanged dataset from NW-side. - Alt 1b: The actual encoder is developed/trained based on the exchange dataset from the NW-side. - Alt. 2: UE-side directly develops and trains the actual encoder by mimicking the encoder exchanged by NW. - Alt 2a: The actual encoder is developed based on UE-side data collection. - Alt 2b: Dataset ({target CSI}) is also exchanged from NW-side to UE-side as additional information, and the actual encoder is developed using the dataset. - For dataset exchange from NW-side to UE-side ({target CSI, CSI feedback} of 4-1, {CSI feedback, reconstructed CSI} of 4-2), - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder using the exchanged dataset. - Step 2: UE-side develops an actual encoder against the nominal decoder based on UE-side data collection. - Alt 1c: The actual encoder is developed/trained based on UE-side data collection. - Alt 1b: The actual encoder is developed/trained based on the exchange dataset from the NW-side. - Alt. 2: UE-side directly develops and trains the actual encoder. - Alt 2b: The actual encoder is developed using the dataset. - Other alternatives are not precluded. |

##### Proposal 48a: evaluation study for Direction C

- For the evaluation study of offline engineering for Direction C:

  - Case 1: The AI/ML model is trained based on training dataset from one Scenario#B/Configuration#B, and then the AI/ML model performs inference/test on a dataset from the same Scenario#B/Configuration#B.

    - Note: This serves as the upper-bound performance.

  - Case 2: The AI/ML model is trained based on training dataset from a different data set than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on Scenario#B/Configuration#B.

    - Note: This represents the performance of Direction C when not using field data.

  - Case 2A: The AI/ML model is trained based on training dataset from a different data set than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model is updated based on training dataset from one Scenario#B/Configuration#B, and then the AI/ML model performs inference/test on a dataset from the same Scenario#B/Configuration#B

    - Note: This represents the performance of Direction C when the AI/ML models are updated based on field data.

    - For the AI/ML model update, NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately, as agreed in note 1-1, 1-2, and 1-3 as agreed in RAN1#118.

  - Note: Scenario#A/Configuration#A represents the synthetic data used for fully specified model development, Scenario#B/Configuration#B represents the field data.

    - Examples for scenario A and B: TxRU mapping, UMi vs. UMa, UMa vs. InH, synthetic vs. actual field data (if available)

  - Note: Case 1 and Case 2 have been studied during Rel-18 generalization study and conclusions have been captured in the TR.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

##### Proposal 49a: evaluation study for UE-side and NW-side data distribution mismatch (for issues 4,6,9)

For discussion on performance degradation due to UE-side / NW-side data distribution mismatch (issue 4, 6, and 9), consider

- NW-side data (dataset A) and UE-side data (dataset B) are mismatched in terms of UE-side additional condition (e.g., SVD phase normalization method, UE antenna virtualization, antenna imbalance, antenna spacing / layout, etc)

  - Note: dataset A may be formed by datasets from multiple UEs including dataset B or formed by dataset of other UEs excluding dataset B

  - Note: if dataset A includes dataset B, this means NW timely collect data from the UE

- Case 1: An encoder-decoder pair is trained on dataset B. This serves as an upper-bound.

- Case 2 (for direction A and C): NW-side trains a decoder on dataset A, and UE-side trains an encoder based on either dataset A, dataset B, or both (where the choice depends on the inter-vendor collaboration options and training approaches)

- Case 3 (for direction B): NW-side train an encoder-decoder on dataset A and transfers the encoder parameters to UE, where the inference is performed based on dataset B.

- Study issue 4, 6, and 9 by evaluating the inference performance for Case 2 / 3 on dataset B, comparing to the inference performance for case 1.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| vivo | We would like to point out two additional aspects that need to be studied - Whether NW side decoder model update without changing UE side model can solve the problem. - Direction A and C evaluations are put under the same bullet which would creates confusion. ##### Proposal 49a: evaluation study for UE-side and NW-side data distribution mismatch (for issues 4,6,9) For discussion on performance degradation due to UE-side / NW-side data distribution mismatch (issue 4, 6, and 9), consider - NW-side data (dataset A) and UE-side data (dataset B) are mismatched in terms of UE-side additional condition (e.g., SVD phase normalization method, UE antenna virtualization, antenna imbalance, antenna spacing / layout, etc) - Note: dataset A may be formed by datasets from multiple UEs including dataset B or formed by dataset of other UEs excluding dataset B - Note: if dataset A includes dataset B, this means NW timely collect data from the UE - Case 1: An encoder-decoder pair is trained on dataset B. This serves as an upper-bound. - Case 2a (for direction A and C): NW-side trains a decoder on dataset A, and UE-side trains an encoder based on either dataset A, dataset B, or both (where the choice depends on the inter-vendor collaboration options and training approaches) - Case 3 (for direction B): NW-side train an encoder-decoder on dataset A or dataset A+B and transfers the encoder parameters to UE, where the inference is performed based on dataset B (which can be subset of dataset A). - Case 2c (for direction C): NW-side trains a decoder on dataset A, and UE-side trains an encoder based on dataset B - Study issue 4, 6, and 9 by evaluating the inference performance for Case 2 / 3 on dataset B, comparing to the inference performance for case 1. - Evaluation study can consider the case that only decoder is updated without changing UE side deployed model. |

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |

# Data collection & consistency between training and inference

## Summary of company proposals

From the submitted contributions, proposals related to data collection are summarized below.

Futurewei

Proposal 6: In AI/ML-based CSI compression using two-sided model, at least for NW-side ground-truth data collection for model training, consider adopting Rel-16 eType II CB based quantization with new parameters to achieve better performance.

Huawei

Proposal 10: For the NW side data collection, confirm the necessity and feasibility of UE report of the ground-truth CSI.

- For the data sample type, prioritize precoding matrix over channel matrix.

- For the data sample format, prioritize Rel-16 eType II CB based quantization with new parameters, and take the following new parameters (captured in the Rel-18 observation) as candidates for discussion.

  - L= 8, 10, 12; pv = 0.8, 0.9, 0.95; reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits.

- For the number/index(es) of layers for the collected ground-truth CSI, it can be indicated by NW.

- To alleviate UE complexity, it can be considered to limit the number of subband for each CSI report.

Tejas Network

Proposal 8: For the NW side data collection prioritize UE reporting precoding matrix than UE reporting channel matrix as a ground truth.

ZTE

- Proposal 22: For network side data collection, support to further study

- Enhanced Rel-16 eTypeII codebook design to achieve high-resolution CSI for model training and performance monitoring

- Proposal 23: To enable high-quality data collection from UE to network, at least support

- UE reports data quality related information to NW, e.g., SINR, CQI, positioning information

- NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

CMCC

Proposal 12: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection for performance monitoring and model training, R16 eType-II codebook and Rel-18 Doppler codebook can be used as a starting point.

Proposal 13: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection, the basic codebook structure could be reused, along with the basic concept of spatial domain, frequency domain and Doppler domain basis.

Proposal 14: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection, the exact supported values of codebook parameters can be studied to make sure high resolution data report.

Xiaomi

Proposal 16: For NW side data collection, the following aspects could be considered:

- Ground truth CSI is reported by using codebook, e.g., Rel-16 eType II codebook or eType II codebook with enhanced parameters.

- Cell-specific CSI-RS resource is configurated by NW-side to collect training dataset.

- Associated information, such as TDCP, is reported to capture the UE side condition at least for Case 3.

- Ground truth CSI could be aperiodically /semi-persistently or periodically reported by using L1 or RRC signalling

Proposal 17: For UE side data collection, the following aspects could be considered:

- NW configuration or UE request could be considered to collect dataset at UE side.

- The associated ID which is associated with NW side additional condition.

- The configuration of temporal aspects for Rel-18 Type II Doppler codebook could be as a starting point for Case 2/3.

Google

Proposal 7: Support to configure the number of layers for the report for NW side data collection for performance monitoring.

Proposal 8: Support to report singular values for the ground-truth CSI.

Proposal 9: Support to report CQI/RI in addition to the ground-truth CSI.

Proposal 10: Reuse the existing CPU framework to handle the UE complexity for the measurement and report for NW side data collection.

Proposal 11: Support to maintain the same understanding between the NW and UE on when to perform the measurement for UE side data collection based on the following options:

- Option 1: The measurement for UE side data collection is configured by the NW

- Option 2: UE request CSI-RS for data collection

CATT

Proposal 11: In CSI compression using two-sided model use case, for NW-side data collection for model training, focus on CSI-RS measurement based data collection.

Proposal 12: In CSI compression using two-sided model use case, both L1 signalling based reporting and RRC signalling based reporting are supported for ground-truth CSI for NW-side data collection for model training.

Proposal 13: In CSI compression using two-sided model use case, L1 signalling based reporting of target CSI is supported for NW-side data collection for performance monitoring.

Proposal 14: In CSI compression using two-sided model use case, for L1 signalling based reporting of ground-truth CSI/target CSI for NW-side data collection, legacy CSI feedback framework can be reused for Case 0.

Proposal 15: In CSI compression using two-sided model use case, for L1 signalling based reporting of ground-truth CSI/target CSI for NW-side data collection, study whether multiple CSI in the same report is supported for temporal domain aspects Case 2/3/4.

Proposal 16: In CSI compression using two-sided model use case, for NW-side data collection for model training, collecting ground-truth data in type of precoding matrix is supported.

Proposal 17: In CSI compression using two-sided model use case, for NW-side data collection for model training, NW determines the number of layers for ground-truth CSI data collection.

Fujistu

Proposal 2:

- For Case 2, RAN1 to study how to address layer disorder issue for training data collection, model inference and performance monitoring.

OPPO

Proposal 18: Regarding the CSI-RS configuration for UE-side and NW-side data collection, at least the follow aspects should be considered:

- Data collection specific CSI-RS or not

- Cell-specific or UE specific CSI-RS

- Trade-off between performance and overhead

Proposal 19: Regarding the data collection for CSI compression, cell/site/scenario related “condition information” and “addition condition information” should be considered during the data collection stage

- Condition information including CSI-related information such as the CSI type, e.g. raw channel or precoding matrix, and the CSI configurations, e.g. number of antenna ports, number of sub-bands, ranks.

- Additional condition information including cell/site/scenario related information such as cell/site/scenario ID, indoor/outdoor indication, LoS/NLoS flag and UE ID.

NEC

Proposal 10: For defining the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB, down select from the following options:

- Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use.

- Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use.

- Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID.

Panasonic

Observation 20: Data collection for model training and non-real time (slow) monitoring is not required to be real-time and then latency requirement can be relaxed.

Observation 21: Ground-truth CSI reporting could be realized through U-plane at least for data collection for model training and non-real time (slow) monitoring.

Observation 22: Assuming fast monitoring is 100s of ms order, U-plane, RRC or MAC-CE can be sufficient.

Observation 23: On data sample type / format for ground-truth CSI reporting, high resolution codebook-based format e.g., legacy codebook (e.g., eType II codebook) with potential enhancements such as extend more configurations in some parameters, should be studied.

Observation 24: For NW-side data collection, at least time stamps / situation of measurement, cell ID and UE location should be considered as the UE-side additional condition.

Observation 25: For NW-side data collection, the necessity and feasibility of UE reporting Rx filter assumption to network should be studied. Instead of informing actual configuration, UE-side associated ID is necessary.

Observation 26: For UE-side data collection for UE side training, in order to identify the scenario / configuration, how to share the NW-side additional condition should be studied. Instead of informing actual configuration, some kind of configuration ID and /or change timing of NW-side additional condition is necessary.

Lenovo

Proposal 1: Support procedures/signaling enabling UE/NW to associate the data/samples with the conditions/additional conditions under which the data/samples has been collected.

Proposal 2: Support procedures/signaling enabling UE/NW for transmission of subset of samples among the set of measured/collected samples from the environment.

Proposal 3: For transmission of ground-truth CSI samples, consider the performance of transmitting more samples, instead of fewer samples with higher resolution per sample (e.g., more samples with current parameter configurations for Rel-16 Type II, instead of less samples with a new parameter configuration for Rel-16 Type II), especially for cases that the overhead is more important, e.g., ground-truth data transfer for model monitoring or model update.

Support definition of pairing information based on the conditions/additional conditions assigned to the samples of the datasets used for training of the model.

ETRI

Proposal 7: For dataset delivery for training collaboration type 3, for CSI compression sub-use case using two-sided model, consider the limited number of dataset samples to assess the feasibility of incorporating standardized signaling.

Samsung

Proposal#8: For the cases of CSI compression with temporal aspects, consider the following for the network’s ground-truth CSI collection

- For cases that require multiple time-domain samples for inference, cases 2/3/4/5, high resolution codebook quantization including temporal aspects, e.g., Rel-18 eType II-like method with new parameters.

- For cases with CSI prediction, e.g., cases 3/4, high resolution codebook quantization for explicit channel matrices, e.g., codebook to report the left and right eigenvectors of a channel matrix H=UΛVH

- Specification impact on measurement and reporting for ground-truth CSI

Proposal#9: For NW-first training with Option 4 (standardized data / dataset format + dataset exchange between NW-side and UE-side) study the necessity and specification impact of indication on the network-side additional condition.

For two-sided models development, NW-part of two-sided model associated with a dataset can be considered as NW-side additional condition.

MTK

For NW-side AI/ML model training, NW can rely on UL CSI samples collected from SRS sent by UEs. Use of DL CSI can be limited to finetuning purposes.

Qualcomm Incorporated

Proposal 7 :For two-sided CSF use case, confirm the necessity of both NW side data collection and UE side data collection.

Proposal 8: For two-sided CSF use case, consider using pairing ID to keep the consistency between training and inference. Study the specification impact of providing pairing ID in data collection RS request, RS configuration, data collection and inference configuration

- RAN1 should discuss the relationship between association ID(s) and pairing ID(s), and further study whether association ID(s) is needed for a given pairing ID.

## Discussion

Companies’ views can be summarized as follows. For NW side data collection, there are extensive discussions on whether high-resolution eT2 like data format is needed. Besides, some companies propose to include data quality and association ID representing UE side data collection. Configurations regarding rank and subband is also mentioned by some companies. For UE side data collection, there are proposals on triggering mechanism and whether ID is needed to capture NW side data collection. From FL perspective, it is more important to confirm the necessity of both NW side and UE side data collection considering that there will be offline engineering at two sides. The details can be further studied. Thus, we can resume from the FL proposals in last meeting.

Another aspect lies in pairing information that can be used in inter-vendor collaboration and inference configuration. In beam management and other agenda, association ID is used to label the collected data, and consistency between training and inference can be achieved with such ID for a single-sided model. For two-sided CSF, RAN1 needs to discuss whether such ID is needed and can be used in dataset / model / parameter exchange in inter-vendor collaboration and inference configuration. If not feasible, whether other IDs, such as model ID and pairing ID can be used to achieve model pairing and inference configuration.

NW side data collection:

- Data format

  - High resolution eT2 or doppler eT2 (6)

    - Futurewei, Huawei, ZTE, CMCC, Xiaomi, Panasonic, Samsung

  - Existing eT2, more data less quality (1, Lenovo)

- Data sample type:

  - Precoder over channel matrix (Huawei, Tejas Network, CATT)

- Configurations

  - Rank / layer configured by NW (Huawei, Google, CATT)

  - Limitation on number of subbands (Huawei)

  - Threshold for data quality (ZTE)

- Additional information of UE reporting

  - Data quality (ZTE)

  - Associated information, e.g., TDCP, for case 3 (Xiaomi), as UE condition

  - Singular values (Google)

  - CQI / RI (Google)

- Signalling to carry the report

  - L1 or RRC signaling (Xiaomi, CATT)

  - Non-real time, U-plane, RRC, MACCE (Panasonic)

- Measurement

  - Reuse CSI framework, CSI-RS based measurement (CATT)

  - Whether to use data collection specific RS (OPPO)

- Other aspects

  - Whether multiple CSIs in the same report for temporal case 2/3/4 (CATT).

  - How to address layer disorder issue for case 2 (Fujistu)

  - UE/NW transmission of subset of samples (Lenovo)

  - Using UL CSI samples (MTK)

  - Confirm necessity of both NW and UE side data collection (QC)

  - Use CPU framework for processing criteria (Google)

UE side data collection

- Mechanism

  - Based on NW configuration or UE request (Xiaomi, Google)

- Configuration

  - Association ID for NW condition (Xiaomi)

Pairing information to keep consistency between training and inference

- Association ID for NW condition (Xiaomi)

- Condition (e.g., CSI type, CSI configurations) and additional condition (scenario, site) should be considered (OPPO)

- Downselection among decoder ID, encoder ID, pairing ID (NEC)

- UE side additional condition includes cell ID, UE location, time stamps / situation of measurement (Panasonic)

- NW condition in form of ID and / or change timing of NW additional condition (Panasonic)

- Procedures / signalling to enable UE/NW associate conditions/ additional conditions (Lenovo)

- Use dataset information for pairing (Lenovo)

- NW-part of two-sided model associated with a dataset can be considered as NW-side additional condition (Samsung)

- Use Pairing ID, study difference between pairing ID and association ID (QC)

##### Proposal 51a: NW-side data collection (continue from proposal 31c of the last meeting)

Confirm necessity and feasibility of ground-truth reporting for NW-side data collection for training. Consider following spec impacts

- Data format: codebook-based Rel-16 eType2 or Rel-18 eType2.

  - FFS if enhancement is needed.

  - FFS number of samples in the report.

  - FFS whether channel or precoder is needed for temporal Cases 3

- Configuration of rank/layer, number of subbands

- Report CSI quality, FFS the definition of CSI quality and corresponding parameters.

- FFS: Report associated information that captures UE side additional condition

- FFS if enhancements in CSI-RS configuration is needed.

- Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI

- Mechanism for ground-truth reporting. Mechanisms outside L1-signlaing (e.g., RRC, U-plane, etc.) can be studied by other working groups.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | 1) New parameter is need for eT2 CB in our view to provide good enough performance. If even the label cannot outperform eT2, the recovery CSI cannot outperform eT2 most likely. We may further study the methods to alleviate UE complexity, e.g., limit the subband number for per report, etc. 2) For Case 2/3, UE may need to report multiple instances of Target CSI (i.e., input CSI) in one report. But it is not clear why it needs to be indicated with the “association between input and output CSI”? 3) For data collection signalling, it is not clear how UP signalling is workable as it may not be accessed by RAN. We do not need to mention the examples in the round bracket. Confirm necessity and feasibility of ground-truth reporting for NW-side data collection for training. Consider following spec impacts - Data format: codebook-based Rel-16 eType2 with new parameters or Rel-18 eType2 with new parameters. - FFS if other enhancement is needed. - FFS number of samples in the report. - FFS whether channel or precoder is needed for temporal Cases 3 - FFS the enhanced values of parameters, and methods to alleviate UE complexity - Configuration of rank/layer, number of subbands - Report CSI quality, FFS the definition of CSI quality and corresponding parameters. - FFS: Report associated information that captures UE side additional condition - FFS if enhancements in CSI-RS configuration is needed. - FFS Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI - Mechanism for ground-truth reporting. Mechanisms outside L1-signlaing (e.g., RRC, U-plane, etc.) can be studied by other working groups. |
| Lenovo | We believe we can extend the third bullet to cover other information such as how often a particular sample is measured, - Report additional information regarding the samples (e.g., CSI quality), FFS the definition of each information, how to configure the UE to send them and CSI quality and corresponding parameters. |
| Panasonic | We are fine with the proposal. |
| Samsung | Ok |
| Ericsson | - Data format: codebook-based Rel-16 eType2 with new parameters or Rel-18 eType2. - FFS if enhancement is needed (e.g., when considering Use Case 2 and/or Use Case 3). - FFS number of samples in the report. - FFS whether channel or precoder is needed for temporal Cases 3 |
| Xiaomi | For the fourth bullet, the FFS could be removed. The configuration/reporting of temporal aspects for temporal Case 2 and Case 3 depends on the UE’s additional condition. For example, for different UE’s speed, the configuration of temporal aspects will be different. Hence, it is necessary to report associated information that captures UE side additional condition. |
| New H3C | OK |
| NTT DOCOMO | Fine with the proposals. |
| LG | Generally fine, but the codebook based solution also considers the extension or introduction of PC for the ground truth CSI |
| Futurewei | For the first bullet on data format, as Rel-18 study showed enhanced parameters can achieve better performance than existing parameters with some additional cost; thus, we suggest the following wordings: - Data format: codebook-based Rel-16 eType2 or Rel-18 eType2, with enhanced parameters. - FFS if enhancement is needed. - FFS mechanism(s) to reduce the overhead/complexity incurred due to implementing enhanced parameters. - FFS number of samples in the report. - FFS whether channel or precoder is needed for temporal Cases 3 |
| ZTE | We share the similar view as HW. In R18 study, some companies provided the simulation results of “New parameter is for eT2 CB”, which shows better performance than current paraCombinations. So, we think it is crucial to further study the data format with new parameters to guarantee the training performance. Confirm necessity and feasibility of ground-truth reporting for NW-side data collection for training. Consider following spec impacts - Data format: codebook-based Rel-16 eType2 with new parameters or Rel-18 eType2 with new parameters. In addition, we are not clear how U-plane signalling is workable and it may not be necessary to be mentioned in RAN1 working group. So, we suggest removing it in the bracket as Mechanism for ground-truth reporting. Mechanisms outside L1-signlaing (e.g., RRC, U-plane, etc.) can be studied by other working groups. |
| Tejas | Support |

##### Proposal 52a: UE-side data collection (continue from proposal 32c of the last meeting)

Confirm the necessity and feasibility of UE-side data collection for training. Consider following spec impacts

- NW configuration or UE request for UE side data collection

- Configuration of temporal aspects for temporal case 2/3, e.g., association between input and output CSI

- FFS: Configuration of associated ID, pairing ID, dataset ID, or model ID, that captures NW side additional condition

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | As we have been focusing on NW-side training and NW-first training, why do we still need a specified manner to enable UE to collect raw data? Regardless of option 3a/4/5a which may need dataset delivery, UE side can receive the collected/post trained dataset delivered by NW side. If UE side may want to generate some new data, it can obtain it with implementation manner. |
| Lenovo | As we are in the data collection phase, we are not sure about the pairing id, model ID in the last bullet, so we suggest: - FFS: Configuration of associated ID, pairing ID, dataset ID, or model ID, that captures NW side additional condition |
| Panasonic | We are OK with the proposal. |
| Samsung | Ok. |
| Ericsson | UE-side data collection for training can refer to different aspects depending on the inter-vendor training options: - Dataset exchanged from NW-side server to UE-side server. - UE-specific dataset collected by the UE-side server from its UEs. For 1) the necessity depends on which inter-vendor training collaboration option is selected, which is still under discussion. For 2), we should strive to reuse the methods selected for one-sided UE-sided training data collection, if applicable, as a starting point. One-sided UE-sided training data collection is still under discussion in RAN2. |
| Xiaomi | OK |
| New H3C | OK |
| Tejas | Fine with the proposal. |

##### Proposal 53a: pairing information

Regarding pairing information, further study the difference between association ID, model ID, dataset ID, and pairing ID considering following aspects

- Whether and which ID is provided from NW-side to UE-side along with inter-vendor collaboration model/parameter/dataset exchange

- Whether and which ID is provided from NW to UE for UE-side data collection

- Whether and which ID is used for inference configuration

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Lenovo | We suggest also to also add: - Whether/how one ID (e.g., the pairing ID) can be defined based on other IDs (e.g., association ID) and other configurations. |
| Samsung | We suggest associated ID to be signalled along reference dataset and models in a similar manner as data collection for beam management. |
| Ericsson | What is the relationship between this proposal on pairing information, and the discussion under the other aspects agenda item on model identification for two-sided models? |
| ZTE | Per our understanding, we can defer the discussion on this proposal until the necessity and feasibility of these kinds of IDs are identified and concluded first. |
| Tejas | Support |

# Monitoring

## Summary of company proposals

From the submitted contributions, proposals related to monitoring are summarized below.

Futurewei

Proposal 7: In AI/ML-based CSI compression using two-sided model, for NW-side monitoring based on the target CSI reported by the UE, consider adopting Rel-16 eType II CB based quantization with new parameters to achieve better monitoring performance.

Proposal 8: In AI/ML-based CSI compression using two-sided model, for UE-side monitoring, if the CSI reconstruction model or a reference CSI reconstruction model is available at UE, support at least the following:

- Based on the output of the CSI reconstruction model at the UE (Case 2-1)

Proposal 9: In AI/ML-based CSI compression using two-sided model, for UE-side monitoring, further study the LCM complexity associated with using a proxy reconstruction model first before discussing whether to support the use of proxy reconstruction model at UE side for performance monitoring.

Huawei

Proposal 11: For NW-side monitoring, consider ground-truth CSI based monitoring in Rel-19.

- eT2-like high-resolution codebook for reporting format with higher priority.

- SGCS for the type of intermediate KPI with higher priority.

- Further discuss the reporting mode, e.g., per sample reporting and reporting of a number of monitored samples.

Proposal 12: There is no strong motivation for specifying the UE side proxy model (Case 2-1/2-2) for monitoring.

Proposal 13: For UE-side monitoring, consider precoded RS (transmitted from NW based on the output of the CSI reconstruction model) based monitoring in Rel-19.

- Further study the type of monitoring KPI calculated from the precoded RS.

Spreadtrum

Proposal 5: For performance monitoring, support the following monitoring options.

- NW side monitoring based on the ground-truth CSI reported by UE.

- UE side monitoring based on the recovery CSI indicated by NW.

Tejas Network

Proposal 9: For NW side monitoring consider the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) for better monitoring accuracy.

Proposal 10: For UE side monitoring consider the Case 2-1 for better monitoring accuracy

  - Based on the output of the CSI reconstruction model at the UE (Case 2-1)

    - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side.

ZTE

In CSI compression using two-sided model use case, deprioritize the study on UE-side monitoring in Rel-19 study phase.

In CSI compression using two-sided model use case, further study a set of appropriate monitoring methodology and reasonable monitoring metrics for the given KPI under rank>1.

CMCC

Proposal 11: For performance monitoring, the following two options can be prioritized:

- NW-side monitoring based on the ground-truth CSI report.

- UE-side monitoring based on the recovery CSI indication.

TCL

Observation 4: The CSI transferring for model monitoring in AI/ML based CSI compression introduces considerable overhead, RAN 1 should strive for an efficient signaling and overhead reduction mechanism for the CSI transferring for model monitoring purpose.

Observation 5: The reliability of the overhead reduction scheme for CSI transferring for model monitoring in AI/ML based CSI compression should be guaranteed.

Xiaomi

Proposal 8: Considering the accuracy of monitoring and signalling overhead, identifying the cause of performance degradation based on UE-side performance monitoring is studied with low priority.

Proposal 9:  In order to identify the cause of the performance degradation, both target CSI and CSI feedback measured by UE are reported for NW-side performance monitoring.

Google

Proposal 4: For CSI report based performance monitoring, the following spec impact should be considered:

- New report quantity PMI only should be introduced, where UE reports the PMI based on a configured rank

  - Support a further enhancement to report subband L1-SINR in addition to the PMI to facilitate the precoder and MCS selection for PDSCH

- With regard to measurement accuracy in low SINR case, support the UE to report a state of CSI indicating the CSI is invalid for performance monitoring

- With regard to joint ML based CSI compression and prediction, support to configure whether the UE should perform the CSI predication based on ML or non-ML and the CSI quantization based on ML or non-ML for separate performance monitoring for ML based CSI prediction and ML based CSI compression.

Proposal 5: For SRS based performance monitoring, the following spec impact should be considered:

- Support to configure the SRS linked with a CSI-RS report configuration for ML based CSI, where the UE uses the same ports including antenna virtualization scheme to transmit the SRS and to receive the CSI-RS for ML based CSI

- Support burst based SRS with frequency hopping to facilitate the performance monitoring for wideband channel for coverage-limited UE

Proposal 6: For UE-side monitoring, the following spec impact should be considered:

- Introduce the hypothetical BLER as the metric for performance calculation

- Configuration of precoded CSI-RS for hypothetical BLER calculation

CATT

Proposal 18:  In CSI compression using two-sided model use case, for NW-side monitoring Case 1, further study the signalling and procedures for reporting target CSI, with the following two options considered:

- Option 1: The target CSI is reported separately from its associated CSI report;

- Option 2: The target CSI is reported together with its associated CSI report.

Proposal 19: In CSI compression using two-sided model use case, performance monitoring at UE-side based on reference model or proxy model can be deprioritized.

Proposal 20: In CSI compression using two-sided model use case, if eventual KPI is adopted as monitoring metric, how to exclude the impacts of other factors other than AI/ML model performance should be studied

Proposal 21: In CSI compression using two-sided model use case, support UE-side monitoring based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model.

Proposal 22: In CSI compression using two-sided model use case, for temporal domain aspects Case 3 and Case 4 with separate prediction and compression adopted, support monitoring the performance of the model for prediction and the performance of the model for compression separately.

Proposal 23: In CSI compression using two-sided model use case, for option 1/3/4/5, when the performance is degraded and it has been identified that the root-cause is not UE-side issues, study whether and how to identify if the root-cause of performance degradation is data drift or NW-side issues.

Proposal 24: In CSI compression using two-sided model use case, if NW-side performance monitoring based on intermediate KPI is adopted, NW-side monitoring for identifying whether performance degradation is caused by UE-side issues can be considered, based on target CSI reported by the UE.

Proposal 25: In CSI compression using two-sided model use case, further study whether and how to support UE-side monitoring for identifying if performance degradation is caused by UE-side issues or not.

Ericsson

Proposal 16: Conclude that it is necessary to specify UE reporting high resolution target CSI to enable NW-side intermediate KPIs based performance monitoring and performance degradation error cause detection for two-sided CSI-compression use case.

Proposal 17: In CSI compression using two-sided model use case, capture in TR that ground-truth CSI report based on enhancements of the eType-II format with new parameters shall be defined to ensure high-accuracy model performance monitoring and error cause detection at the NW-side. Potential specification impact include:

  - Define the target-CSI format (e.g., Rel16 eType II CB with new parameters) for NW-side data collection (can reuse the ground truth defined for model training data collection)

  - Mechanisms (e.g., RRC-message based methods) to support UE reporting the target CSI together with the encoder output for NW-side data collection for performance monitoring.

  - Signaling and configuration for event triggered and periodical data collection at the NW-side.

Proposal 18: For CSI compression using two-sided model use case, for any UE-sided performance monitoring method (if its feasibility and performance are justified), to enable the testability of the quality of the UE reported monitoring metrics, at least the following spec impact are identified:

  - The format of the monitoring metrics

  - Singaling and mechanisms for UE reporting monitoring metrics

  - RAN4 performance testing of the reported monitoring metrics

  - Data collection at the NW-side based on UE reporting monitoring data samples including target CSI, encoder output and monitoring metrics (to enable NW-side test the quality of the reported monitoring metric in the field).

Fujistu

Proposal 8:

- For CSI compression using two-sided AI/ML models, the feasibility, reliability, and generalization capability of the UE-side AI/ML model performance monitoring using proxy model(s) should be evaluated and concluded before any further discussion on the related specification impacts.

Proposal 9:

- For the NW-side AI/ML model performance monitoring for CSI compression, RAN1 to prioritize the study of using the codebook-based quantization method to obtain the ground-truth CSI. Besides, adding new parameter values to legacy codebook for higher resolution ground-truth CSI should be studied.

Proposal 10:

- For CSI compression using two-sided AI/ML models, RAN1 to study the signaling and configuration for NW-side AI/ML model performance monitoring.

Proposal 11:

- For CSI compression using two-sided AI/ML models, regarding the NW-side AI/ML model performance monitoring using an existing CSI feedback scheme as a reference, RAN1 to study the potential specification impacts for the following three options:

  - Option-1: UE selects and reports PMI to the NW.

  - Option-2: UE computes and reports the intermediate KPI for the reference scheme, e.g., the SGCS of the recovered CSI from PMI and the ground-truth CSI.

  - Option-3: NW selects the PMI based on the ground-truth CSI reported by a UE.

Proposal 12:

- For CSI compression using two-sided AI/ML models, RAN1 to study the procedures and signalling needed for the follow-up actions after the AI/ML model performance monitoring, including falling back to legacy codebook-based CSI reporting from AI/ML-based methods.

Proposal 13:

- For the performance monitoring of CSI compression using two-sided AI/ML models, RAN1 to study the potential specification impacts on monitoring the performance of an inactive AI/ML model, taking at least the following cases into consideration:

  - Initial activation of an AI/ML model.

  - Re-activation of an AI/ML model.

NEC

Proposal 3: Support NW-side monitoring based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook.

Proposal 4: For NW-side monitoring, the AI CSI and associated target CSI can be reported in the same reporting instance, or two separate reports.

Proposal 5: For NW-side monitoring, RAN1 to discuss and evaluate whether ground truth reporting from UE can be transmitted over L1 or higher layer signaling (e.g. RRC).

Proposal 6: Support UE-side monitoring, the following can be considered:

- Based on the output of the CSI reconstruction model at the UE. Where the CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side.

- Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI.

Panasonic

Observation 30: Further study Direction 1 and Direction 3 with proxy model framework.

- Direction 1: Network-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE side.

- Direction 3: UE-side monitoring based on the output of the CSI reconstruction model at the UE side.

  - The CSI reconstruction part for performance monitoring at the UE is a proxy model, which is different from the actual CSI reconstruction part at the network.

Nvidia

Proposal 6: RAN1 to study post-deployment performance monitoring mechanisms to detect performance degradation and non-compliance to guarantee satisfactory performance of AI/ML-based CSI compression in the field.

Lenovo

Proposal 1: In options 3a-1, 3b, 4-1, 5a-1, 5b, study mechanisms to ensure the applicability of the information received for the reference encoder model encoder model for the current input statistics.

Proposal 2: Study mechanism to determine the main contributor(s) of the lower performance of the model at least for the issues related to a) the “deployed” encoder and/or decoder model, b) the communication link, or c) data-drift of the input data. We note that, in general, the “deployed” encoder/decoder model could have different performance that the “trained” (reference) encoder or decoder model.

Proposal 3: Study mechanism for root-cause determination based on exchange of some test data-set between the NW and the UE.

Proposal 4: Study mechanism for root-cause determination based on exchange of information regarding the NW-side trained encoder and/or decoder model.

InterDigital

Proposal 2:	Study further the following aspects for model monitoring in Rel-19:

- Details of reporting mechanism for the monitoring metrics with both time/event-trigger based

- Appropriate UE-side monitoring metric which reflects AI/ML model performance accurately

- UE-side monitoring based on precoded RS (CSI-RS, DM-RS)

- Reporting contents/structure of UE-side monitoring metric and its associated feedback overhead

- NW-side monitoring with lower signaling overhead

Proposal 3: Mechanisms for identifying the cause of performance degradation include:

- UE assistance information such as UE-side model monitoring metrics and UE-side out-of-distribution metrics

- Reporting the UE-assessed error cause (e.g., data drift, UE-side, or undetermined)

- Mitigation mechanisms, including fallback to legacy CSI reporting, model switching

Apple

Observation 11: NW side performance monitoring has issue of high feedback overhead and additional UE complexity and power consumption for extended parameter sets.

Observation 12: For UE side performance monitoring using proxy model, since inter-vendor collaboration option 3a-2, 3a-3, option 5a-2, 5a-3, and option 4-2 and option 4-3 provide information related to CSI reconstruction model, the proxy model can be trained based on this information. However, NW may need to perform performance monitoring on the proxy model.

Observation 13: For UE side performance, NW implicitly transmit output CSI using precoded CSI-RS to the UE provide a simple and low overhead solution.

Proposal 7: For CSI compression using two-sided model, for UE side performance, further study RLF/BFD like mechanism for UE initiated report.

Proposal 8: gNB can use UL SLS measurement to detect data drift due to channel statistics change.

Proposal 9: gNB can trigger root cause detection data collection when performance monitoring detects degradation, and data drift due to channel statistics is not observed. The dataset collection for root cause detection can reuse the immediate MDT with logged data, following current RAN2 agreement on data collection for training.

Proposal 10: Further study root cause detection due to data distribution mismatch due to UE side additional condition.

Proposal 3: For performance monitoring of case 2 of time-frequency-spatial domain CSI compression, options discussed for frequency-spatial domain CSI compression can be used for NW side and UE side performance monitoring.

Proposal 4: For performance monitoring of case 4 of time-frequency-spatial domain CSI compression, CSI measurement in prediction window is the target CSI for NW side or UE side performance monitoring. The intermediate KPI or eventual KPI includes both compression and prediction performance.

Proposal 5: For case 3 of time-frequency-spatial domain CSI compression, CSI measurement in prediction window is the target CSI for NW side or UE side performance monitoring. The intermediate KPI or eventual KPI includes both compression and prediction performance.

ETRI

Proposal 8: For the performance monitoring of CSI compression sub-use case using two-sided model, conclude that Case 2-1 is feasible with considerations of complexity, latency, and accuracy.

Proposal 3: For AI/ML-based CSI compression using two-sided model, when the target CSI is Future slot(s), study following aspects, for performance monitoring operations:

Method to align whether prediction and compression occur in separate steps or simultaneously between UE and NW

Either UE-side or NW-side performance monitoring.

Samsung

Proposal#10: For performance monitoring, consider causes of performance loss that particularly affect the AI/ML based approach.

- Consider KPI for monitoring such as KPImonitoring=1exp⁡(1)exp⁡(SGCSAIML-SGCSbaseline), where SGCSAIML and SGCSbaseline  are the SGCS for AI/ML-based CSI and baseline CSI, e.g., eType-II, respectively.

- To evaluate the monitoring options, apply the evaluation mechanism agreed in Rel-18 by replacing intermediate KPI by KPImonitoring .

- FFS: baseline CSI for a payload size.

Proposal#11: For NW-side monitoring of two-sided models, when the input CSI is the (W2) domain, consider the same SD, FD and/or DD basis vectors for the CSI report for inference and monitoring purposes.

Proposal#12: To assess the accuracy of NW-side monitoring of two-sided models, when the input CSI is in the (W2) domain, KPIGenie  is calculated with ground-truth CSI of Float32 representation of the W2 matrices.

MTK

- Proposal 8: Discuss NW-side AI/ML model I/O-based monitoring using uplink CSI samples collected from SRS.

- Proposal 9: Deprioritize coarse granularity of associate ID, i.e., assigning one associate ID to multiple cells.

DCM

Proposal 5

- For NW-side monitoring, support the scheme based on the eT2-like CSI report from the UE.

- For UE-side monitoring, further study the reliability of direct KPI estimation for the down-selection among direct KPI estimation, precoded RS, and eT2-like indication.

Qualcomm Incorporated

Proposal 9: Using existing parameter-combinations of eType II codebook is sufficient for ground-truth reporting of NW side monitoring.

Proposal 10: RAN1 to perform evaluation study for monitoring decision accuracy in terms of false alarms and miss-detection. AvgSGCSmeasured averaged over N samples may be used as a monitoring metric for the monitoring decision, where monitoring decision is made by comparing the metric against a threshold.

Proposal 11: Consider two-phase mechanism for performance monitoring and identification of root cause

- Phase 1: identifying whether there is performance degradation

- Phase 2: identifying whether the performance degradation is due to UE side CSI generation part, NW side CSI reconstruction part, or data drift, etc.

Proposal 12: Consider following model performance monitoring mechanisms

- NW side monitoring mechanism with ground-truth reporting for monitoring of performance degradation and monitoring root cause identification

    - Further study method of handling incurred overhead/ latency

- UE side monitoring mechanism

  - Phase 1 monitoring of performance degradation via SGCS estimator

  - Phase 2 monitoring of root cause identification with ground-truth reporting:

    - Further study triggering event of ground-truth reporting based on SGCS generated via SGCS estimator.

## Discussion

Companies’ proposals are summarized as below. For NW side monitoring, majority companies propose to use high-resolution eT2 for ground-truth reporting, while some others think existing eT2 is sufficient. Besides, there are concerns raised regarding overhead and latency incurred in NW side ground-truth reporting. Reporting mechanisms, configuration and signalling are also discussed.

For UE side monitoring, the views on detailed scheme is still diverse. Some companies mention that precoded RS is feasible if the monitoring metric issue can be addressed. There are multiple factors that may impact the performance of precoded RS vs. original ground-truth CSI, e.g., channel variation, channel estimation, etc. Some other companies propose that UE side reference decoder output can be used for UE side monitoring, and 2 companies show interest in SGCS / monitoring output estimator. But the concern lies in additional LCM burden for the proxy / estimator model. One company propose that, in general for all UE side monitoring schemes, RAN4 testing and data collection by NW to perform NW side testing of the proxy / estimator can be considered.

Moreover, one company raises a concern of monitoring metric. SGCS values have a wide variation across UEs and channel realizations, so high accuracy in monitoring metric may not be important in determining monitoring decisions. Thus, it is proposed to use false alarm detection and misdetection rate as metrics.

Regarding root cause detection, a good number of companies propose that ground-truth reporting can help resolve the issue. Two companies propose that root cause identification should be performed after performance degradation is identified in general.

Based on the proposals, FL thinks we can resume from the FL proposals in last meeting.

NW side monitoring

- Support high-resolution eT2 (9, 5 companies do not have strong view between existing eT2 or high-resolution eT2)

  - Futurewei, Huawei, Ericsson, Fujistu, Tejas Network, CMCC, NEC, Panasonic, DCM

- Existing eT2 is sufficient (7, 5 companies do not have strong view between existing eT2 or high-resolution eT2)

  - QC, MTK, Tejas Network, CMCC, NEC, Panasonic, DCM

- Reporting aspects

  - per-sample report or reporting of a number of samples (Huawei)

  - report quality of the ground-truth CSI whether it is invalid for monitoring (Google)

- Signalling / configuration / mechanism

  - configure whether UE perform CSI prediction to report ground-truth (Google)

  - new report quantity, PMI only (Google)

  - Mechanism to support UE reporting ground-truth (Ericsson, CATT, Fujistu, NEC)

    - together with CSI feedback (Ericsson, CATT, NEC)

  - Signaling and configuration for event triggered and periodic data collection at NW side (Ericsson)

  - L1 or RRC signaling (Ericsson, NEC)

- Supporting SRS-based performance monitoring (Google, MTK)

  - Association between SNR and a CSI report (Google)

  - Burst SRS with frequency hopping for wideband channel for coverage-limited UE (Google)

UE side monitoring

- Methods

  - Based on Reference decoder output or actual decoder output if available at UE (5)

    - Futurewei, Tejas Network, NEC, Panasonic, ETRI

  - Based on SGCS estimator or output estimator (2)

    - NEC, QC

  - Based on precoded RS (5)

    - Huawei, Google, CATT (hypo BLER as metric), Apple, InterDigital

  - Based on indication of reconstructed CSI (2)

    - Spreadtrum, CMCC

  - No support of UE side proxy model (Futurewei, Huawei, CATT)

- Enable testability (RAN4 and with monitoring mechanism) of the quality of UE reported monitoring metrics (Ericsson)

  - Via data collection at NW side includes ground-truth, monitoring metrics, CSI feedback

- Mechanisms / reporting (Ericsson, InterDigital, Apple)

  - Details of reporting mechanisms with time/event-triggered (InterDigital)

  - Reporting content/structure of UE side monitoring and associated overhead (InterDigital)

  - RLF/BFD like mechanism (Apple)

- Deprioritize UE side monitoring (ZTE)

Root cause detection

- Achieved via NW side monitoring with ground-truth reporting (Xiaomi, CATT, Ericsson, Lenovo, Apple)

  - Ground-truth reporting via MDT following current RAN2 agreement (Apple)

  - Two-phase monitoring procedure, ground-truth triggering based on SGCS reported by UE in performance monitoring phase (QC)

  - Triggering root cause detection after monitoring performance degradation (Apple, QC)

- NW can use SRS to detect data drift (Apple)

- Study UE side root cause detection (InterDigtial)

  - UE assistance information such as monitoring metrics and UE side OOD metrics

  - Reporting UE-assessed error cause

  - Mitigation mechanisms, including fallback / switching

- Study whether and how to identify root cause of either decoder or data drift (CATT)

- Further study root cause identification due to mismatch of UE side additional condition (Apple)

Other aspects

- New monitoring metrics

  - False alarm rate of Average SGCS over N samples vs. threshold (QC)

  - Exponential of SGCS accuracy and float32 is considered for KPI_genie (Samsung)

  - Appropriate monitoring metric reflect model performance accurately (InterDigital)

- Overhead concern and potential reduction mechanism for performance monitoring (TCL, InterDigital, Apple, QC)

- Consider methodology and metric for rank > 1 (ZTE)

- Separate monitoring for prediction and compression (CATT)

- Follow-up actions, e.g., fallback to codebook-based scheme (Fujistu)

- Monitoring inactive models (Fujistu)

##### Proposal 61a: monitoring options down-selection

FL note: This is down-selection among the options agreed in RAN1 #117.

Further study following monitoring options in Rel-19, including the necessity and feasibility,

- NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1)

    - Further study whether eType2-like high-resolution codebook is needed.

- UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | For UE side monitoring, fine with it. For NW side monitoring, our understanding is that eT2-like high-resolution codebook with new parameters is needed. Let’s consider an extreme example, where we adopt legacy eT2 as the label fed back by UE, and assuming a benchmark of legacy eT2 as the non-AI solution, then from monitoring results at NW, benchmark performance is always 100%, and there is never a need to enable the AI solution, which is unreasonable. |
| Lenovo | In some cases, if we consider inter-vendor collaboration scheme where some decoder information is exchanged, the UE already have access to information regarding the reference decoder which is in line with UE-sided monitoring Based on Reference decoder. So, we suggest to keep this option. - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability: - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on Reference decoder output or actual decoder output if available at UE |
| Samsung | Ok. |
| ETRI | We have a similar view with Lenovo’s. |
| Ericsson | For the NW-sided monitoring, Rel-18 study results showed that eT2-like high-resolution codebook with new parameters is needed. We do not support the selected UE-sided monitoring approach. With this approach, the NW needs to first use its decoder to generate the reconstructed CSI, and then derive a precoding matrix to precode a RS, and then, transmit the RS to the UE. By measuring on the precoded RS, the UE may derive some link-level KPIs like CQI, BLER, etc., but the UE cannot derive intermediate KPI. This method cannot support a UE to obtain accurate information of the model performance, yet it introduces additional latency, RS signalling overhead, and processing complexity at both UE and NW side. In addition, this method does not scale if considering multiple UEs in the network and many monitoring samples are required per UE to derive a monitoring result. |
| New H3C | OK |
| ZTE | We are OK to further study NW-side monitoring methods with eType2-like higher-resolution codebook since the accuracy of actual labels is quite important. In addition, according to the simulation results captured in TR, some companies showed that NW-side monitoring can achieve more accurate monitoring results based on CSI labels with eType2-like higher-resolution codebook of new parameters. For UE-side monitoring based on precoded CSI-RS, we think it is challenging as follows: - Additional latency is needed for precoded RS transmission. - Time span of monitoring process is increased, where two-step procedure is needed to finish performance monitoring. - Monitoring results may not adapt to the channel variation/aging Therefore, we suggest focusing on the NW-side monitoring first. |
| Tejas | We are ok with NW side monitoring. |
| Fujitsu | For NW side monitoring, agree with Huawei that eT2-like codebook with high-resolution parameter combination is needed. For UE-side monitoring based on pre-coded RS, monitoring accuracy may be affected by CSI aging for the scenario of high UE speed. |
| QC | Disagree with HW and Ericsson. The need of higher resolution eT2 toward monitoring decision has not been proven yet. What was studied in Rel-18 is that higher resolution eT2 helps the accuracy of the KPI (such as the accuracy of the SGCS estimate), but whether more accurate KPI estimation (at the expense of higher overhead and UE capability requirement) actually helps improving monitoring decision is not clear yet and should be proven via evaluations. In fact, some evaluations by companies suggest that a coarse accuracy is sufficient for monitoring decisions. |

##### Proposal 62a: monitoring decision performance evaluation

Evaluate the monitoring decision performance, including accuracy, overhead, latency, complexity, and generalization ability for NW-side and UE-side monitoring approaches.

- Use false alarm probability and missed detection probability as KPIs for the monitoring decision accuracy as a starting point.

  - KPIFA =Prob(KPIActual < thr), where the AI/ML model#A is trained based on training dataset from Scenario#A/Configuration#A, and the evaluation is performed on the model#A and a dataset from the same Scenario#A/Configuration#A.

  - KPIMD =Prob(KPIActual >= thr), where the AI/ML model#A is trained based on training dataset from Scenario#A/Configuration#A, and the evaluation is performed on either a different model than model#A (representing an erroneous model usage) or a different dataset than Scenario#A/Configuration#A (representing a data distribution drift).

  - KPIActual is obtained based on N samples.

    - In case of SGCS as KPI, KPIActual can be derived by averaging the SGCS estimate over N samples.

    - N should be large enough to average out channel variations and allow accurate monitoring decisions

  - To also consider rank>1 with an appropriate metric for the given KPI.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | 1) We have a different understanding on the definition of miss detection and false alarm. For the definition of monitoring accuracy, we have the formula in the TR, i.e., the gap between KPIActual (the actual intermediate KPI) and KPIGenie (the genie-aided intermediate KPI). KPIDiff = f ( KPIActual , KPIGenie ) can take the following forms: - Option 1 (baseline for calibration): Gap between KPIActual and KPIGenie, i.e. KPIDiff = (KPIActual - KPIGenie); Monitoring accuracy is the percentage of samples for which ¦ KPIDiff¦ < KPIth 1, where KPIth 1 is a threshold of the intermediate KPI gap which can take the following values: 0.02, 0.05 and 0.1. - Option 2 (optional and up to companies to report): Binary state where KPIActual and KPIGenie, have different relationships to their threshold(s), i.e., KPIDiff = (KPIActual > KPIth 2, KPIGenie < KPIth 3) OR (KPIActual < KPIth 2, KPIGenie > KPIth 3), where KPIth 2 is considered to be the same as KPIth 3. Monitoring accuracy is the percentage of samples for which KPIDiff = 0.. For false alarm and miss detection, they should not be defined as the absolute value of the actual KPI, but rather the bias between actual KPI and genie-aided KPI. E.g., under Option 1, if KPIDiff > KPIth 1, where KPIDiff is positive value, it means miss detection; if KPIDiff < -KPIth 1, where KPIDiff is negative value, it means false alarm. 2) Per sample based monitoring can reflect the reliability of the model, and can support fast feedback wot NW, which can compare the KPI with legacy for a quick error identification. Therefore, we think both monitoring methods are valid. - KPIActual is obtained based on both 1 sample and N samples. - Monitoring KPI based on 1 sample is calculated same as R18 EVM. - For N samples, In case of SGCS as KPI, KPIActual can be derived by averaging the SGCS estimate over N samples. - N should be large enough to average out channel variations and allow accurate monitoring decisions |
| Mod | To HW: My understanding is that KPIGenie is only for evaluation purposes. In reality, NW does not know the value of KPIGenie (unless RAN1 defines a floating point feedback of target CSI), so it is NOT possible for NW to make monitoring decisions based on KPIGenie. May I ask HW’s understanding on this? |
| Ericsson | Average/mean SGCS over N samples is likely not a good monitoring metric to use for deriving false alarm rate and missed detection rate. As already shown in RAN4 results calibration study, different AI/ML models may have similar average/mean SGCS values but their SGCS CDF curves can be quite different. A more reliable metric to use for KPIActual is the distance of the SGCS distribution based on monitoring dataset and the SGCS distribution based on verification data samples used in the model training stage. In such case, “thr” is the distribution distance threshold. It should be pointed out that sufficient number of samples are needed to represent the statistic information; hence, N needs to be sufficiently large. This applies for both NW-sided and UE-sided model performance monitoring approaches. In addition, the threshold value needs to be carefully optimized to achieve good monitoring performance in terms of false alarm rate and missed detection rate. And this threshold value is likely to be model dependent. |
| ZTE | To our understanding, there is no need to further make a definition or formula in TR since the Option 2 for performance monitoring captured in TR can already reflect FA and MD. For example, KPIDiff = (KPIActual > KPIth 2, KPIGenie < KPIth 3) can be used to evaluate MD and KPIDiff = (KPIActual < KPIth 2, KPIGenie > KPIth 3) can be applied to FA. In addition, absolute KPI value can also reflect the actual SGCS performance unlike a gap value for Option 1, which cannot be observed whether the current model performance is failed, e.g., SGCS is very low. KPIDiff = f ( KPIActual , KPIGenie ) can take the following forms: - Option 1 (baseline for calibration): Gap between KPIActual and KPIGenie, i.e. KPIDiff = (KPIActual - KPIGenie); Monitoring accuracy is the percentage of samples for which ¦ KPIDiff¦ < KPIth 1, where KPIth 1 is a threshold of the intermediate KPI gap which can take the following values: 0.02, 0.05 and 0.1. - Option 2 (optional and up to companies to report): Binary state where KPIActual and KPIGenie, have different relationships to their threshold(s), i.e., KPIDiff = (KPIActual > KPIth 2, KPIGenie < KPIth 3) OR (KPIActual < KPIth 2, KPIGenie > KPIth 3), where KPIth 2 is considered to be the same as KPIth 3. Monitoring accuracy is the percentage of samples for which KPIDiff = 0.. In addition, we also agree to further study “KPIActual is obtained based on both 1 sample and N samples”, since both methods can be applicable. |
| Mod | To HW and ZTE: Just want to re-emphasize that this is for the “monitoring decision”, not for “monitoring accuracy”. So, the false alarm and miss detection refer to those for the monitoring decision. Please note that KPIGenie is only for evaluation purposes. In reality, NW does not know the value of KPIGenie (unless RAN1 defines a floating point feedback of target CSI), so it is NOT possible for NW to make monitoring decisions based on KPIGenie. This is one of the limitations of the monitoring accuracy metric used in the Rel-18 evaluation. To Ericsson: Thanks for raising the point. I agree that the averaging is for simplicity of the study. While we can generally consider distance in distribution, it’s not clear what metric we could use. Please feel free to propose a better metric. |

##### Proposal 63a: root cause (continue from 42b of the last meeting)

For NW-first training, in inter-vendor training collaboration options 3, 4, and 5, conclude that root cause identification can be achieved at least by ground-truth CSI reporting

- e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support in general. “NW first training” may cause confusion, and is crossed out. For NW-first training, in inter-vendor training collaboration options 3, 4, and 5, |
| Lenovo | In some cases, if we consider inter-vendor collaboration scheme where some decoder information is exchanged, the UE already can perfume some root-cause analysis without the overhead of ground-truth CSI reporting. For NW-first training, in inter-vendor training collaboration options 3, 4, and 5, conclude that root cause identification can be achieved at least by ground-truth CSI reporting using some NW side or UE side analysis: - FFS on the required information of each case. |
| Panasonic | We are OK with the proposal. |
| Ericsson | It is unclear how root cause identification (e.g., NW side, UE side, data drift) can be done based on the simple example text in the proposal. More discussion and clarification are needed. |
| Xiaomi | For the last bullet, we think triggering of root cause detection may be after detecting performance degradation compared with performance guidance. |
| Futurewei | We are ok with the proposal. |
| ZTE | OK with the direction. |

# Inference aspects (CQI / quantization / CSI framework / config)

## Summary of company proposals

From the submitted contributions, proposals related to inference aspects (pairing, CQI, quantization, etc.) are summarized below.

Futurewei

Proposal 10: In AI/ML-based CSI compression using two-sided model, for CQI determination, if the CSI reconstruction model or a reference CSI reconstruction model is available at UE, adopt Option 2a to determine CQI at UE.

Huawei

Proposal 8: For the additional potential spec impact of temporal domain CSI compression Case 2 on top of Rel-18 SF domain CSI compression, consider methods to handle the misalignment of the accumulated CSI between NW part model and UE part model due to UCI missing.

Proposal 9: For the additional potential spec impact of temporal domain CSI compression Case 3 on top of Rel-18 SF domain CSI compression,

- for separate prediction and compression, potential spec impact may be needed for data collection of the CSI compression model and monitoring of the CSI prediction model.

- for joint prediction and compression, potential spec impact may be needed for data collection, inference, and monitoring.

Proposal 14: For quantization methods of the CSI report, further study potential specification impact on quantization alignment using standardized quantization scheme.

- For vector quantization,

  - Configuration/reporting/updating of the quantization dictionary.

  - Segmentation of the CSI generation model output to map with short VQ vector.

- For scalar quantization,

  - The configuration of the quantization granularity/range.

Proposal 15: For the study of CQI determination in inference, consider Option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation) as a starting point.

Proposal 16: For CSI report in inference, on top of the legacy CSI reporting principles, the following AI/ML specific aspects may be additionally studied:

- The CSI priority rules, e.g., priority rules by considering the AI/ML specific reporting type, priority rules within the bit sequence of per AI/ML specific inference CSI report.

- The CSI processing unit (CPU), e.g., the required CPU value may consider difference of UE part model complexity.

- The CSI mapping, e.g., factors representing the part 2 size in CSI part 1, mapping of the CSI generation part output in CSI part 2, etc.

Proposal 17: For Rank>1 options in inference, further study Option 3-1 (layer common and rank common) Option 3-2 (layer common and rank specific) and Option 2-1 (layer specific and rank common) with higher priority.

- For Option 3-1, it is straightforward that the LCM of activation/deactivation/monitoring is applied to this single model.

- For Option 3-2 and Option 2-1, need to further discuss whether to consider the multiple models as separate models or a single model from LCM perspective.

Spreadtrum

Proposal 3: For the study of CQI determination in inference, support Option 1b (CQI is calculated based on target CSI with realistic channel measurement and potential adjustment).

Proposal 4: Consider to report historical CSI information via NW-triggered signaling when UCI missing or UCI dropping.

ZTE

For CQI determination, at least prioritize the specification impact discussions on Option 1a, Option 1b.

TCL

Proposal 1: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects:

- AI/ML-specific CSI-RS resource configuration for CSI compression

- AI/ML-specific fields in CSI-ReportConfig IE

- Dedicated report quantities and report configurations for AI

Proposal 2: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules.

Proposal 3: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting.

Proposal 4: The study of how to describe the capabilities of a UE to implement AI/ML models for inference on CSI compression and calculations should be undertaken.

Google

Proposal 1: For case 2 and case 3, support the UE to report W1 and compressed W2 for a configured rank

- The compressed W2 is calculated based on the AI/ML

- NW can configure the UE to report RI and CQI based on precoded CSI-RS resources after receiving the W1 and compressed W2

Proposal 2: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report.

Proposal 3: Support the CPU occupancy rule for ML based CSI based on two types processing unit

- Type1 CPU: a measurement processing unit (MPU) used for channel estimation and pre-processing

- Type2 CPU: an inference processing unit (IPU) used for inference for ML based CSI

CATT

Proposal 26: In CSI compression using two-sided model use case, quantization alignment between UE-side and NW-side in a 3GPP non-transparent manner is supported.

Proposal 27: In CSI compression using two-sided model use case, legacy CSI reporting principles are reused as much as possible.

Proposal 28: In CSI compression using two-sided model use case, if CQI in CSI report is configured, for CQI determination in CSI report, one of the sub options of Option 1 is adopted:

- Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

  - Option 1a: CQI is calculated based on target CSI with realistic channel measurement

  - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

  - Option 1c: CQI is calculated based on legacy codebook.

- Proposal 29: For CQI reporting in CSI compression using two-sided model use case, the same quantization scheme as that in Rel-17 for codebook based CSI feedback is considered.

Fujistu

Proposal 3:

- For CSI compression using two-sided AI/ML models, support both the following alternatives of precoding matrix for output-CSI-UE and input-CSI-NW:

  - Alt 1: The precoding matrix in spatial-frequency/spatial-frequency-time domain for Case 2/Case 3

  - Alt 2: The precoding matrix represented using angular-delay/angular-delay-doppler domain projection for Case 2/Case 3.

Proposal 4:

- For CSI compression using two-sided AI/ML models, RAN1 to further study the configurations and CSI reporting formats required for various AI/ML model settings. To reduce the normative workload, the following could be down selected:

  - AI/ML-model-setting-specific CSI configurations and CSI reporting formats.

  - A unified configuration and CSI reporting format adapting to various possibilities, including at least

    - layer specific and rank common.

    - layer specific and rank specific.

    - layer common and rank common.

    - layer common and rank specific.

Proposal 5:

- For CSI compression using two-sided AI/ML models, deprioritize Option 2 proposed in RAN1 #112 for CQI determination.

  - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation.

Proposal 6:

- In the CSI reports generated by AI/ML, include the information for the order of the spatial layers of the reported precoding matrix related information, if the reported rank is larger than 1.

- Furthermore, the layer index/order related information should be in the Part I CSI.

Proposal 7:

- In the CSI reports generated by AI/ML, the criteria of the Part II CSI priority can be set by the order of the spatial layers of the precoding matrix related information, if the reported rank is larger than 1.

- Furthermore, the order of the spatial layers in the first bullet can be

  - Alt 1: the same as the order of the reported layers of the precoding matrix related information.

  - Alt 2: the descending order of the singular value corresponding to the precoding vectors.

NEC

Proposal 7: At least for Case 2, CSI buffer reset should be supported to address misalignment of historical CSI used at UE side and NW side. And the definition, determination or indication of the reset value need to be further studied.

Proposal 8: At least for Case 2, further study effective availability of historical CSI information over time.

Proposal 9: If the CSI reconstruction part/model at UE side is available, support Option 2a for CQI determination.

LGE

Proposal #1: Regarding temporal/spatial/frequency (TSF)-domain CSI compression, study methods/mechanisms to manage the similarity/synchronization of accumulated past CSI at UE-side and/or NW-side.

Proposal #2: Regarding TSF-domain CSI compression, discuss the format of past CSI information and how to report it at least for performance monitoring perspective.

Proposal #4: Regarding non-ideal UCI feedback on TSF-domain CSI compression,

- Consider two-step performance monitoring to check that the performance degradation of the AI/ML model is originated from whether the accumulated past CSI has a problem or the AI/ML model is not suitable for the deployed environment

- Consider to report past CSI information via NW-triggered signaling when UCI missing or UCI dropping

Proposal #5: Consider the method on the rank adaptation based on the availability check of layer(s) for a given RI.

Proposal #6: For CQI determination in CSI compression using two-sided model, consider to prioritize Option 1. If Option 2 is supported, further consider

- Option 2a: Utilizing AI/ML model complexity reduction method to reduce the signaling overhead to deliver the CSI reconstruction part at NW-side.

Lenovo

Support procedures/signalling enabling CSI-compression models having both Scaler and vector Quantizers for generation of the CSI-feedback bits.

InterDigital

Proposal 1: Study the specification impact of adapting the length of the historical CSI buffer to channel conditions, such as UE speed.

Apple

Proposal 2: For case 2 and case 4 of time-frequency-spatial domain CSI compression, the following potential specification impact are proposed:

- Enable semi-persistent CSI reporting

- Enable DCI based reset state

- Considering UCI retransmission in case of large amount of UCI drop or loss, to avoid the state at UE and gNB out of sync.

- For RI, consider longer term RI update across different CSI reports.

Proposal 6: For time-frequency-spatial domain CSI compression, flexible CSI report configuration to support different cases should be studied.

ETRI

Proposal 1: For AI/ML-based CSI compression using two-sided model, when UE and/or NW uses past CSI information, reuse the current specification on CSI-RS transmissions as much as possible.

Proposal 2: For AI/ML-based CSI compression using two-sided model, when NW uses past CSI information, study method to detect and mitigate inconsistency of the availability of past CSI information between the UE and the NW.

Samsung

Proposal#1: For the Case 2 of temporal aspects of AI/ML-based CSI compression using two-sided model, consider at least the following two options for the past CSI information

- Case 2-1: Past CSI information generated by the UE-part and/or network-part of two-sided model

- Case 2-2: Information on SD/FD basis vectors as past CSI information with angle-delay domain compression.

Proposal#2: For the Case 2 of temporal aspects of AI/ML-based CSI compression using two-sided model:

when past CSI information corresponds to SD/FD basis and AI/ML CSI compression is in the angle-delay domain, consider SD/FD basis reporting per N CSI reporting occasions, i.e., N times longer periodicity.

FFS on the values of N.

Proposal#4: Among the identified six categories for AI/ML-based CSI compression using two-sided model, for Case 3, consider N4≥1 prediction instances (Doppler time intervals).

- Option1: AI/ML-based CSI compression in spatial-frequency-time domain

- Option 2: The AI/ML-based CSI compression in angle-delay-time domain

- Option 3: The AI/ML-based CSI compression in angle-delay-Doppler domain

Proposal#5: For the Case 2 of temporal aspects of AI/ML-based CSI compression using two-sided model, for the evaluation of performance impact resulting from CSI dropping

- Modelling for CSI dropping probabilities

  - x% Group 2 of Part II CSI dropping probability

  - y% Group 1 of Part II CSI dropping probability

  - z% Group 0 of Part II CSI dropping probability

  - where ≥y≥z .

- Note: Companies to report the partitioning of part I and part II CSI and the values for x, y, and z.

Proposal#6: Study the impact of input pre-processing (dimensionality reduction) on performance, model complexity and model generalizability.

Proposal#7: In Angle-delay (W2)-domain CSI compression, study the impact of the number of SD/FD basis vectors for performance-complexity tradeoff.

MTK

- Proposal 1: Clarify the boundary of AI/ML model, i.e., in which step it starts or ends.  Temporal-spatial-frequency (TSF) domain compression.

- Proposal 3: Deprioritize  case 3 which may use one AI/ML model for doing both CSI compression and CSI prediction in this agenda item.

- Proposal 4: Evaluate the feedback error tolerance of eType II and compare it with that of AI/ML model.

Qualcomm Incorporated

- Proposal 13: Conclude that CQI calculation option 2a (where UE runs CSI reconstruction model and use its output for CQI calculation) can be employed with the consideration of potentially higher timeline and higher cost of processing unit and memory.

- Proposal 14: Further study CQI calculation option 1b (CQI is calculated based on target CSI with realistic channel measurement and potential adjustment) considering adjustment measurement at UE side based on intermediate KPI or intermediate output of the CSI generation model

- Proposal 15: Consider layer-common and rank common (Option 3-1) structure for CSI generation model and/or CSI reconstruction model for specified structures. Layer-common (Option 3-1) or layer-specific (Option 2-1) parameters can be upto vendor’s implementation choice.

- Note: The standardized model structure is used to address inter-vendor collaboration complexity. The specification should be flexible to allow actual model for inference designed using all options 1-1, 1-2, 2-1, 2-2, 3-1 and 3-2.

- Proposal 16: Regarding quantization alignment for two-sided CSF use case, study proprietary quantization codebook with standardized exchange signaling from NW side to UE side for inter-vendor collaboration direction A and B.

- Note: the exchange can be via standardized signalling or proprietary signalling

## Discussion

The summary of companies proposals are presented below.

CQI

- Option 1a: based on target CSI (5)

  - Huawei, ZTE, CATT, Fujistu, LGE

- Option 1b: based on target CSI with bias (7)

  - Huawei, Spreadtrum, ZTE, CATT, Fujistu, LGE, QC

- Option 1c: based on legacy codebook (4)

  - Huawei, CATT, Fujistu, LGE

- Option 2a: using ref decoder if available (3)

  - Futurewei, NEC, QC

- Option 2b: based on precoded CSI-RS (1)

  - Google

Temporal use cases

- Handling UCI missing or rank / bases variation (8, Huawei, ETRI, NEC, Apple, LGE, InterDigital, Google, Samsung)

  - Buffer reset and availability of historical CSI in time (NEC, Apple)

  - Past CSI retransmission (LGE (via two-step approach based on monitoring), Apple)

  - Adaptation of length of historical CSI buffer per channel condition (InterDigital)

  - Longer term RI (Apple), or configured RI (Google)

  - Reporting SD/FD bases per N reporting occasions (Samsung)

- For case 3, separate prediction + compression needs spec impacts on data collection for CSF and monitoring for CSI prediction; joint prediction + compression needs spec impacts on data collection, monitoring, inference (Huawei)

- Past CSI generated by models or past CSI is SD/FD bases (Samsung)

- Modeling of UCI dropping (Samsung)

- Deprioritize case 3 with joint prediction and compression (MTK)

Quantization

- Spec impact on configuration / reporting / updating of dictionary, segmentation (Huawei)

- Align quantization via non-transparent manner (CATT)

- AI/ML mode supporting both VQ and SQ (Lenovo)

- Proprietary quantization and exchanged along with dataset / model / parameter exchange in direction A (QC)

CSI principles

- Priority across multiple reports (Huawei, TCL, Google)

  - AI/ML has higher priority than other report (Google)

- CPU rule (Huawei, Google)

  - MPU for non-AI measurement and pre-processing, IPU for AI/ML inference (Google)

- UCI mapping and priority (Huawei, Fujistu)

  - Downslection among layer common/specific and/or rank common/specific CSI (Fujistu)

  - Report layer ordering, omission based on ordering (Fujistu)

- AI/ML specific RS configuration, report configuration, report quantity (TCL)

- How to describe capability of UE implementing AI/ML model for inference (TCL)

- Flexible report configuration for various use cases (Apple)

Model design for high rank

- Support layer-common rank-common (Huawei, QC)

- Support layer-common rank-specific (Huawei, FFS LCM)

- Support layer-specific rank-common (Huawei, FFS LCM, QC)

CQI options are discussed by 11 companies. Majority view lies in option 1 where CQI is based on target CSI with or without adjustment or precoded CSI-RS. There are also some companies proposing that CQI option 2a (based on reference decoder at UE side) can be considered if UE is able to run a reference decoder or actual decoder.

The topic that receives second most attention is how to handle UCI missing or rank/bases variation for temporal use cses. Companies propose to use CSI buffer reset, past CSI retransmission, and constant rank / bases for multiple CSI report instances.

Model design aspects regarding high rank support is discussed by 2 companies, layer-common rank-common, layer-common rank-specific, layer-specific rank-common are favored.

Quantization alignment methods is discussed by 4 companies, 3 of them are interested in standardized approach, while 1 company mention it can be achieved via model / parmater / dataset sharing in inter-vnedor collaboration options. The configuration and indication of codebook / dictionary are also discussed.

CSI priority rule, CSI processing criteria (CPU), UCI mapping are discussed by 5 companies. ML CSI has different (or higher) priority than other non-AI CSI. Different processing unit may be needed for measurement and inference. CSI in part 2 may be ordered by layer indices, the dropping may be based on index or singular values.

##### Discussion 71a:

FL does not think that the above aspects are critical for determining feasibility of the CSI compression use case. They can generally be studied in the later meetings. Therefore, FL wants to deprioritize discussion of the above aspects.

If companies disagree with the FL’s assessment and believe that certain aspects are critical for determining feasibility of the CSI compression, please state those aspects and why.

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |

# Complexity performance trade-off

## Summary of company proposals

Samsung

Proposal#7: In Angle-delay (W2)-domain CSI compression, study the impact of the number of SD/FD basis vectors for performance-complexity tradeoff.

Ericsson

Proposal 19	Further study methods to reduce the AI computational complexity for AI CSI compression Case 3.

Proposal 20	Further study methods to reduce the AI computational complexity for AI CSI compression Case 2.

Qualcomm

Proposal 17: Complexity reduction by representing the precoder in angle-delay domain needs further study, and the following aspects need to be considered

•	SGCS performance comparison with baseline model

•	Complexity reduction in model-size and/or FLOPs compared with baseline model

•	Complexity associated with generating the angle-delay domain representation

•	Overhead bits needed for signalling the SD/FD basis

Proposal 18: In addition to angle-delay domain representation, other complexity reduction techniques need to be considered when studying the ML CSF performance-complexity trade-off.

LG

Proposal #8: Study on model complexity method, e.g., knowledge distillation, to further reduce the CSI training/signaling complexity for Type 3 training collaboration.

## Discussion

Regarding complexity, it is noted by companies that

- Use of localized models may reduce complexity.

- Use of temporal aspects may reduce complexity.

- Use of transformed domain input (e.g., angular, delay, Doppler) may reduce complexity.

- Other ML model complexity reduction methods such as architecture optimization/pruning and Knowledge distillation

Below, the FL captured the SGCS gain vs. complexity (combined FLOPs for CSI generation part and the CSI reconstruction part) for Case 0 based on companies’ submissions in the Results Spreadsheet.

From the figure, we observe that larger NN models do not necessarily provide larger gains. For example, a model of size around 20MFlops seems to achieve most of the gain. This implies that some of the AI/ML models used in the evaluation study may not have been optimized in terms of complexity that that lower complexity models may achieve similar performance.

Also, it is important to note that the actual AI/ML model complexity depends on various platform-dependent choices for modem implementation. The values reported here should be considered as representative values and not as a precise complexity estimate for implementation of AI/ML-based CSI compression.

For lower complexity models utilizing angular-delay domain, it is not yet clear whether the lower complexity comes with suboptimal performance. Results comparing the performance of the angular-delay domain and the spatial-frequency domain approaches are lacking, because companies chose either approach but not both.

##### Proposal 81a: complexity-performance tradeoff

Study performance-complexity trade-off by comparing different AI/ML models and by comparing different precoder representation in (spatial-frequency and angle-delay) or (spatial-frequency-time and angle-delay-doppler) by considering the following aspects

- Performance comparison between different designs

- Complexity numbers (FLOP, measured latency or power consumption if available)

| Support / Can accept |  |
| --- | --- |
| Object / Have a concern |  |

| Company | Comments |
| --- | --- |
| Ericsson | Support to further study the performance-complexity trade-off, but we don’t have a common definition on measured latency nor on power consumption yet. We suggest also comparing the complexity numbers between AI and non-AI benchmark schemes (e.g., FLOPs ratio between AI scheme and the corresponding non-AI benchmark scheme) |
| Mod | To Ericsson: the latency or power consumption refers to the actual measurements. |

##### Proposal 81b: complexity-performance trade-off

Study performance-complexity trade-off by comparing different AI/ML models and by comparing different precoder representation in (spatial-frequency and angle-delay) or (spatial-frequency-time and angle-delay-doppler) by considering the following aspects

- Performance comparison between different AI/ML designs and benchmark schemes

- Complexity numbers (FLOP, calculated/projected latency or power consumption if available, measured latency or power consumption if available) of different AI/ML designs and benchmark schemes

| Company | Comments |
| --- | --- |

# Other topics (work plan, new use cases)

## Summary of company proposals

From the submitted contributions, proposals related to other aspects not covered in other sections (work plan, new use cases, etc.) are summarized below.

Huawei

## Discussion

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |

# Proposals for online sessions

## Proposals for Monday online session

##### Proposal 42b: Direction A

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Issue 1: additional information includes

  - Performance requirements (3a/5a/4)

  - Assistant information of the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4)

  - Dataset containing the target CSI (3a/5a)

  - Information related to collecting UE-side training dataset (3a/5a/4)

  - FFS Model backbone information (4)

  - Note: other additional information is not precluded

- Issue 2: There is no proprietary information concern of disclosing encoder input/output from NW side to UE side (4-1).

  - Further discuss whether there is concern for other sub-options under Direction A.

- Issue 3: Exchanging dataset / model / parameter over-the-air may incur large overhead if over-the-air signalling is used. Options to address this issue is up to RAN2 discussion.

- Issue 4: Performance impact due to NW / UE side data distribution mismatch can be at least addressed by

  - For decoder sharing and dataset sharing, UE side offline engineering based on UE side data.

  - NW side timely data collection and decoder (information) update

##### Proposal 45b: way forward on Directions

For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression,

- Conclude that the following two Directions are needed for the normative work.

  - Direction A, B or both

  - Direction C

- For Direction A:

  - Further study the following options with potential down-selection

    - 4-1: dataset is {target CSI, CSI feedback},

    - 4-2: dataset is {CSI feedback, reconstructed CSI}

    - 3a-1: dataset is {target CSI}, in addition to encoder parameter exchange

- For Direction C:

  - The fully specified reference model is at least for minimum performance requirement.

    - Study whether the fully specified reference models or their compatible models may be used for deployment at NW and UE

      - Including performance evaluation of training/finetuning the models based on field data and the corresponding need of inter-vendor collaboration.

  - It is clarified that, in RAN1, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side.

    - It is up to the UE-side (NW-side) whether to directly implement the fully specified CSI generation (reconstruction) part.

  - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3)

  - FFS whether the reference model is the same as RAN4 specified model

## Proposals for Tuesday online session

##### Proposal 42d: Direction A

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Answer to Issue 1: additional information may include

  - Performance requirements (3a/5a/4)

  - Assistant information of associated with the exchanged dataset / model / parameter (e.g., association ID, model ID or pairing ID) (3a/5a/4)

  - Dataset containing the target CSI (3a/5a)

  - Information related to collecting UE-side training dataset (3a/5a/4)

  - Model backbone information (4)

  - Note: other additional information is not precluded

##### Proposal 43b: Direction B

For issues listed for inter-vendor collaboration direction B, conclude the following

- Issue 3: Overhead in Direction B is not a big concern.  Further study the overhead in Direction B based on the size of the encoder, the number of encoders, and how often the parameters need to be exchanged.

- Issue 5 & 7:

  - Further study the feasibility and performance of UE-common encoder considering different implementation preferences and optimizations across different UE / chipset vendors and implementations.

  - Further study feasibility of UE-specific or UE-group-specific encoder considering feasibility of disclosing UE-side proprietary information, and feasibility of training and managing multiple encoders at NW-side.

- Issue 5 & 6: Further study performance due to NW / UE data mismatch considering feasibility of training UE-specific / UE-group-specific encoder

##### Proposal 44b: Direction C

For issues listed for inter-vendor collaboration direction C, conclude the following

- Answer to Issue 8: It is not feasible for RAN1 to use field data to generate standardized model. RAN1 standardized model is trained using synthetic data as a starting point.

- Answer to Issue 9:

  - Performance degradation occurs in the field if the standardized model trained based on synthetic data is used for inference without offline engineering based on field data.

  - Further study the performance with offline engineering

  - Offline engineering based on field data at one side (e.g., training/finetuning at one side based on field data) can partially mitigate the performance degradation. However, its performance is still worse than Direction A.

  - Offline engineering at both sides independently without inter-vendor collaboration may give worse performance than offline engineering at one side.

  - Further study performance degradation due to mismatch between NW side data and UE side data

- Answer to Issue 10: Additional information includes

  - Performance requirements

  - Assistant information for dataset alignment (e.g., association ID or model ID or pairing ID)

  - Information related to collecting UE-side training dataset

  - Specification of synthetic data unless both reference decoder and reference encoder are specified

  - Note: other additional information is not precluded

##### Proposal 45b: way forward on Directions

For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression,

- Conclude that the following two Directions are needed for the normative work.

  - Direction A, B or both

  - Direction C

- For Direction A:

  - Further study the following options with potential down-selection

    - 4-1: dataset is {target CSI, CSI feedback},

    - 4-2: dataset is {CSI feedback, reconstructed CSI}

    - 3a-1: dataset is {target CSI}, in addition to encoder parameter exchange

- For Direction C:

  - The fully specified reference model is at least for minimum performance requirement.

    - Study whether the fully specified reference models or their compatible models may be used for deployment at NW and UE

      - Including performance evaluation of training/finetuning the models based on field data and the corresponding need of inter-vendor collaboration.

  - It is clarified that, in RAN1, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side.

    - It is up to the UE-side (NW-side) whether to directly implement the fully specified CSI generation (reconstruction) part.

  - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3)

  - FFS whether the reference model is the same as RAN4 specified model

## Proposals for Wednesday online session 1

##### Proposal 43c: Direction B

For issues listed for inter-vendor collaboration direction B, conclude the following

- Issue 3: Overhead in Direction B is not a big concern.  Further study the overhead in Direction B based on the size of the encoder (i.e., number of parameters and quantization level), the number of encoders, and how often the parameters need to be transferred.

- Issue 5 & 7:

  - Further study the feasibility and performance of UE-common encoder considering different implementation preferences and optimizations across different UE / chipset vendors and implementations.

  - Further study feasibility of UE-specific or UE-group-specific encoder considering feasibility of disclosing UE-side proprietary information, and feasibility of training and managing multiple encoders at NW-side.

- Issue 5 & 6: Further study performance due to NW / UE data mismatch considering feasibility of training UE-common / UE-specific encoders

##### Proposal 44c: Direction C

For issues listed for inter-vendor collaboration direction C, conclude the following

- Answer to Issue 8: It is not feasible for RAN1 to use field data to generate standardized model. RAN1 standardized model is trained using synthetic data as a starting point.  rewording to be done

- Answer to Issue 9:

  - Performance degradation occurs in the field if the standardized model trained based on synthetic data is used for inference without offline engineering based on field data.

  - Further study the performance with offline engineering

  - Offline engineering based on field data at one side (e.g., training/finetuning at one side based on field data) can partially mitigate the performance degradation. However, its performance is still worse than Direction A.

  - Offline engineering at both sides independently without inter-vendor collaboration may give worse performance than offline engineering at one side.

  - Further study performance degradation due to mismatch between NW side data and UE side data

- Answer to Issue 10: Additional information includes

  - Performance requirements

  - Assistant information for dataset alignment (e.g., association ID or model ID or pairing ID)

  - Information related to collecting UE-side training dataset

  - Specification of synthetic data unless both reference decoder and reference encoder are specified

  - Note: other additional information is not precluded

##### Proposal 45d: way forward on Directions

For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression,

- Conclude that the following two Directions are needed for the normative work.

  - Direction A, B or both

  - Direction C

- For Direction A:

  - Further study the following options with potential down-selection

    - 4-1: dataset is {target CSI, CSI feedback},

    - 4-2: dataset is {CSI feedback, reconstructed CSI}

    - 3a-1: encoder parameter exchange, with {target CSI}

    - Note: Target CSI is needed at UE-side but may not need to be exchanged if UE-side has other means of obtaining it.

    - Note: Additional information beyond what’s listed above may needs to be exchanged if necessary for the UE-side encoder development/training.

- For Direction C:

  - The fully specified reference model is at least for minimum performance requirement.

  - It is clarified that, in RAN1, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side.

    - It is up to the UE-side (NW-side) whether to directly implement the fully specified CSI generation (reconstruction) part as long as above compatibility purpose is satisfied.

  - Study whether the fully specified reference models or their compatible models may be used provide reasonable performance for deployment at NW and UE

    - Including performance evaluation of training/finetuning the models based on field data (either by modeling data distribution mismatch and/or by using actual field data)

      - Training/finetuning done at either or both sides without inter-vendor collaboration. The results may be compared to the performance of inter-vendor collaboration.

      - Note: companies may evaluate training/finetuning without inter-vendor collaboration (Direction C) and/or with inter-vendor collaboration (Direction A/B)

    - Including performance evaluation of NW-side and UE-side data distribution mismatch

  - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3)

  - FFS whether the reference model is the same as RAN4 specified model

  - FFS whether a single or multiple reference (and how many) models need to be specified

##### Proposal 41b: evaluation study for Direction A

For the evaluation studies of Direction A, it is clarified that

- For decoder model / parameter exchange from NW-side to UE-side (3a-2, 5a-2), UE-side develops its actual encoder against the exchanged decoder based on UE-side data collection.

  - Alt 1a: The actual encoder is developed/trained based on UE-side data collection.

  - Alt 1b: Dataset is also exchanged from NW-side to UE-side as additional information, and the actual encoder is developed/trained based on the exchanged dataset.

- For encoder model / parameter exchange from NW-side to UE-side (3a-1, 5a-1),

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder against the exchanged encoder.

      - Alt 1a: Otherwise, The reference nominal decoder is developed based on UE-side data collection.

      - Alt 1b, 1c: If Dataset ({target CSI}) is also exchanged from NW-side to UE-side as additional information, and the nominal decoder is developed using the exchanged dataset and the exchanged encoder.

    - Step 2: UE-side develops actual encoder against the nominal decoder based on UE-side data collection

      - Alt 1a, 1c: The actual encoder is developed/trained based on UE-side data collection.

      - Alt 1b: The actual encoder is developed/trained based on the exchange dataset from the NW-side.

  - Alt. 2: UE-side directly develops and trains the actual encoder by mimicking the encoder exchanged by NW.

    - Alt 2a: The actual encoder is developed based on UE-side data collection.

    - Alt 2b: Dataset ({target CSI}) is also exchanged from NW-side to UE-side as additional information, and the actual encoder is developed using the dataset.

- For dataset exchange from NW-side to UE-side ({target CSI, CSI feedback} of 4-1, {CSI feedback, reconstructed CSI} of 4-2),

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder using the exchanged dataset.

    - Step 2: UE-side develops an actual encoder against the nominal decoder based on UE-side data collection.

      - Alt 1c: The actual encoder is developed/trained based on UE-side data collection.

      - Alt 1b: The actual encoder is developed/trained based on the exchange dataset from the NW-side.

  - Alt. 2: UE-side directly develops and trains the actual encoder.

    - Alt 2b: The actual encoder is developed using the dataset.

- Other alternatives are not precluded.

##### Proposal 48a: evaluation study for Direction C

- For the evaluation study of offline engineering for Direction C:

  - Case 1: The AI/ML model is trained based on training dataset from one Scenario#B/Configuration#B, and then the AI/ML model performs inference/test on a dataset from the same Scenario#B/Configuration#B.

    - Note: This serves as the upper-bound performance.

  - Case 2: The AI/ML model is trained based on training dataset from a different data set than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on Scenario#B/Configuration#B.

    - Note: This represents the performance of Direction C when not using field data.

  - Case 2A: The AI/ML model is trained based on training dataset from a different data set than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model is updated based on training dataset from one Scenario#B/Configuration#B, and then the AI/ML model performs inference/test on a dataset from the same Scenario#B/Configuration#B

    - Note: This represents the performance of Direction C when the AI/ML models are updated based on field data.

    - For the AI/ML model update, NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately, as agreed in note 1-1, 1-2, and 1-3 as agreed in RAN1#118.

  - Note: Scenario#A/Configuration#A represents the synthetic data used for fully specified model development, Scenario#B/Configuration#B represents the field data.

    - Examples for scenario A and B: TxRU mapping, UMi vs. UMa, UMa vs. InH, synthetic vs. actual field data (if available)

  - Note: Case 1 and Case 2 have been studied during Rel-18 generalization study and conclusions have been captured in the TR.

##### Proposal 49a: evaluation study for UE-side and NW-side data distribution mismatch (for issues 4,6,9)

For discussion on performance degradation due to UE-side / NW-side data distribution mismatch (issue 4, 6, and 9), consider

- NW-side data (dataset A) and UE-side data (dataset B) are mismatched in terms of UE-side additional condition (e.g., SVD phase normalization method, UE antenna virtualization, antenna imbalance, antenna spacing / layout, etc)

  - Note: dataset A may be formed by datasets from multiple UEs including dataset B or formed by dataset of other UEs excluding dataset B

  - Note: if dataset A includes dataset B, this means NW timely collect data from the UE

- Case 1: An encoder-decoder pair is trained on dataset B. This serves as an upper-bound.

- Case 2 (for direction A and C): NW-side trains a decoder on dataset A, and UE-side trains an encoder based on either dataset A, dataset B, or both (where the choice depends on the inter-vendor collaboration options and training approaches)

- Case 3 (for direction B): NW-side train an encoder-decoder on dataset A and transfers the encoder parameters to UE, where the inference is performed based on dataset B.

- Study issue 4, 6, and 9 by evaluating the inference performance for Case 2 / 3 on dataset B, comparing to the inference performance for case 1.

##### Proposal 47b: model structure specification

For studying the standardized model structure specification,

- For temporal domain Case 0,

  - In case of spatial-frequency domain input, adopt Transformer as the backbone structure.

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 2,

  - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation with additional layerson top of Case 0 model structure

      - Conv-LSTM

      - LSTM

    - Option 2: Latent adaptation on top of Case 0 model structure

      - Reusing Case 0 structure and only changing quantization operation and/or feedback payload over different slots

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 3,

  - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation

    - Option 2: Latent adaptation

  - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input.

## Proposals for Wednesday online session 2

##### Proposal 44d: Direction C

For issues listed for inter-vendor collaboration direction C, conclude the following

- Answer to Issue 8: RAN1 will consider 3GPP’s statistical channel model.

##### Proposal 41b: evaluation study for Direction A

For the evaluation studies of Direction A, it is clarified that

- For decoder model / parameter exchange from NW-side to UE-side (3a-2, 5a-2), UE-side develops its actual encoder against the exchanged decoder based on UE-side data collection.

  - Alt 1a: The actual encoder is developed/trained based on UE-side data collection.

  - Alt 1b: Dataset is also exchanged from NW-side to UE-side as additional information, and the actual encoder is developed/trained based on the exchanged dataset.

- For encoder model / parameter exchange from NW-side to UE-side (3a-1, 5a-1),

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder against the exchanged encoder.

      - Alt 1a: Otherwise, The reference nominal decoder is developed based on UE-side data collection.

      - Alt 1b, 1c: If Dataset ({target CSI}) is also exchanged from NW-side to UE-side as additional information, and the nominal decoder is developed using the exchanged dataset and the exchanged encoder.

    - Step 2: UE-side develops actual encoder against the nominal decoder based on UE-side data collection

      - Alt 1a, 1c: The actual encoder is developed/trained based on UE-side data collection.

      - Alt 1b: The actual encoder is developed/trained based on the exchange dataset from the NW-side.

  - Alt. 2: UE-side directly develops and trains the actual encoder by mimicking the encoder exchanged by NW.

    - Alt 2a: The actual encoder is developed based on UE-side data collection.

    - Alt 2b: Dataset ({target CSI}) is also exchanged from NW-side to UE-side as additional information, and the actual encoder is developed using the dataset.

- For dataset exchange from NW-side to UE-side ({target CSI, CSI feedback} of 4-1, {CSI feedback, reconstructed CSI} of 4-2),

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder using the exchanged dataset.

    - Step 2: UE-side develops an actual encoder against the nominal decoder based on UE-side data collection.

      - Alt 1c: The actual encoder is developed/trained based on UE-side data collection.

      - Alt 1b: The actual encoder is developed/trained based on the exchange dataset from the NW-side.

  - Alt. 2: UE-side directly develops and trains the actual encoder.

    - Alt 2b: The actual encoder is developed using the dataset.

- Other alternatives are not precluded.

##### Proposal 48a: evaluation study for Direction C

- For the evaluation study of offline engineering for Direction C:

  - Case 1: The AI/ML model is trained based on training dataset from one Scenario#B/Configuration#B, and then the AI/ML model performs inference/test on a dataset from the same Scenario#B/Configuration#B.

    - Note: This serves as the upper-bound performance.

  - Case 2: The AI/ML model is trained based on training dataset from a different data set than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on Scenario#B/Configuration#B.

    - Note: This represents the performance of Direction C when not using field data.

  - Case 2A: The AI/ML model is trained based on training dataset from a different data set than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model is updated based on training dataset from one Scenario#B/Configuration#B, and then the AI/ML model performs inference/test on a dataset from the same Scenario#B/Configuration#B

    - Note: This represents the performance of Direction C when the AI/ML models are updated based on field data.

    - For the AI/ML model update, NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately, as agreed in note 1-1, 1-2, and 1-3 as agreed in RAN1#118.

  - Note: Scenario#A/Configuration#A represents the synthetic data used for fully specified model development, Scenario#B/Configuration#B represents the field data.

    - Examples for scenario A and B: TxRU mapping, UMi vs. UMa, UMa vs. InH, synthetic vs. actual field data (if available)

  - Note: Case 1 and Case 2 have been studied during Rel-18 generalization study and conclusions have been captured in the TR.

##### Proposal 49a: evaluation study for UE-side and NW-side data distribution mismatch (for issues 4,6,9)

For discussion on performance degradation due to UE-side / NW-side data distribution mismatch (issue 4, 6, and 9), consider

- NW-side data (dataset A) and UE-side data (dataset B) are mismatched in terms of UE-side additional condition (e.g., SVD phase normalization method, UE antenna virtualization, antenna imbalance, antenna spacing / layout, etc)

  - Note: dataset A may be formed by datasets from multiple UEs including dataset B or formed by dataset of other UEs excluding dataset B

  - Note: if dataset A includes dataset B, this means NW timely collect data from the UE

- Case 1: An encoder-decoder pair is trained on dataset B. This serves as an upper-bound.

- Case 2 (for direction A and C): NW-side trains a decoder on dataset A, and UE-side trains an encoder based on either dataset A, dataset B, or both (where the choice depends on the inter-vendor collaboration options and training approaches)

- Case 3 (for direction B): NW-side train an encoder-decoder on dataset A and transfers the encoder parameters to UE, where the inference is performed based on dataset B.

- Study issue 4, 6, and 9 by evaluating the inference performance for Case 2 / 3 on dataset B, comparing to the inference performance for case 1.

##### Proposal 47b: model structure specification

For studying the standardized model structure specification,

- For temporal domain Case 0,

  - In case of spatial-frequency domain input, adopt Transformer as the backbone structure.

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 2,

  - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation with additional layerson top of Case 0 model structure

      - Conv-LSTM

      - LSTM

    - Option 2: Latent adaptation on top of Case 0 model structure

      - Reusing Case 0 structure and only changing quantization operation and/or feedback payload over different slots

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 3,

  - In case of spatial-frequency domain input, take into account reuse the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation

    - Option 2: Latent adaptation

  - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input.

# FL closing remark

# List of agreements

## Agreements from RAN1 #116

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study:

| Case | Target CSI slot(s) | Whether the UE uses past CSI information | Whether the network uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |
| 5 | Present slot | No | Yes |

Note 1: For the UE, the past CSI information may include past model inputs and/or any information derived from them. For the network, the past CSI information may include past CSI feedback instances and/or any information derived from them.

Note 2: For case 3 and case 4, the UE may perform prediction as a separate step or jointly with compression. Similarly, the network may perform prediction as a separate step or jointly with reconstruction. Companies to report which option is selected, the number of future slots, and whether the prediction is AI/ML-based or not.

Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well.

Note 4: Down-selection is not precluded.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following as baseline options for UE distribution:

- Option 1: 80% indoor, 20% outdoor

- Option 2: 100% outdoor

Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 10 km/h, 20 km/h, 30 km/h, 60 km/h, 120 km/h. Assumption on O2I car penetration loss and spatial consistency follow the R18 AI based CSI prediction.

Working Assumption

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following benchmark scheme for performance comparison:

- For cases without prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI compression study.

- For cases with prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI prediction study, with R18 MIMO eType II codebook for compressing the feedback.

Agreement

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, study the following aspects of the performance/complexity trade-off when comparing the localized model with a benchmark model that is not localized:

- Performance of the localized model that has similar or lower complexity as the benchmark model.

- Model complexity of the localized model that achieves similar or better performance as the benchmark model.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions:

- CSI-RS configuration

  - Periodic: 5 ms periodicity (baseline), 20 ms periodicity(encouraged)

  - Aperiodic (for cases with prediction): Optional, CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II)

- CSI reporting periodicity: {5, 10, 20} ms; other values are not precluded

- For cases with the use of past CSI information, to report observation window, including number/time distance of historic CSI/channel measurements.

- For cases with prediction, to report prediction window, including number/time distance of predicted CSI/channel.

Agreement

To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc.

Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

Agreement

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Note: While modelling the spatial correlation, strive to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. To report methods to generate training and testing dataset.

Agreement

- For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19,

  - adopt the CSI feedback overhead rate as reference, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead across time.

Note: The CSI feedback overhead of a single report is calculated as in R18 CSI compression study.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for cases with prediction of future CSI, in which prediction and compression are separated, to optionally evaluate a scheme with ideal prediction as an additional evaluation case for reference.

Note: The ideal prediction scheme should model realistic channel estimation.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, Case 4 and Case 5, study the performance impact resulting from non-ideal UCI feedback.

Agreement

For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider at least the following aspects when comparing different options:

- Inter-vendor collaboration complexity, e.g., whether bilateral collaboration is required between vendors.

- Performance.

- Interoperability and RAN4 / testing related aspects.

- Feasibility.

## Agreements from RAN1 #116-bis

Agreement

For the results template used to collect evaluation results for temporal domain compression Case 1/2/5, adopt Table 1 used in Rel-18 as starting point with the following additions:

- Temporal domain CSI setting

  - CSI feedback periodicity

  - CSI-RS periodicity

- Description of model input/output and Case

  - Compression case, e.g., Case 1/2/5

  - Usage of historical CSI at UE/NW side (e.g., number / time distance, eigen-vectors / raw channels, etc)

  - Methods to handle UCI loss (if applicable), e.g., CSI buffer reset, CSI retransmission, etc.

  - Methods to handle rank adaptation (if applicable)

- UE distribution (Option 1 or Option 2) and UE speed

- CSI feedback overhead rate: X/Y/Z bits per normalized time unit

  - Normalized time unit = 5ms and adopt same X/Y/Z values as in Table 1 of Rel-18

- Benchmark scheme

  - Rel-16 eT2 and compression Case 0 (i.e., Rel-18 AI/ML based CSI compression)

- Whether/how spatial consistency is modelled

- Whether/how UCI loss is modelled

  - The same UCI loss model shall be applied to the benchmark for fair comparison.

- Whether/how rank adaptation is modelled

- Modelling of channel estimation error

- Whether/how phase discontinuity is modelled (if applicable)

Agreement

For the results template used to collect evaluation results for temporal domain prediction and compression Case 3/4, adopt Table 1 used in Rel-18 as starting point with the following additions:

- Temporal domain CSI setting

  - CSI feedback periodicity

  - CSI-RS periodicity

- Description of model input/output and use case

  - Compression case, e.g., case 3 / 4

  - Observation window (usage of historical CSI at UE/NW side, e.g., number / time distance, eigen-vectors / raw channels, etc)

  - Prediction window (e.g., time distance between 1st prediction instance and last observation instance, number / time distance of predicted CSI)

  - Methods to handle UCI loss (if applicable)

- UE distribution (Option 1 or Option 2) and UE speed

- CSI feedback overhead rate: X/Y/Z bits per normalized time unit

  - Normalized time unit = 5ms and adopt same X/Y/Z values as in Table 1 of Rel-18

- SGCS values before (if applicable) and after compression

- Assumption on the prediction of future CSI

  - Separate step or jointly with compression

  - If separate, description of the AI or non-AI prediction algorithms: ideal prediction, AI-based prediction, non-AI-based prediction (e.g., nearest historical CSI and its location, learning window size / time correlation matrix size for auto-regression based prediction),

    - Note: the same prediction algorithm to be used for the benchmark scheme.

- Benchmark schemes

  - Description of feedback schemes, i.e., Rel-18 doppler eT2

- Whether/how spatial consistency is modelied

- Whether/how UCI loss is modelled

  - The same UCI loss model shall be applied to the benchmark for fair comparison.

- Modelling of channel estimation error

- Whether/how phase discontinuity is modelled (if applicable) Modelling of phase discontinuity

Conclusion

For multi-vendor results table, adopt Rel-18 Table 4 for joint training and Rel-18 Table 5 for separate training as starting point, with the same additions of above 2 agreements.

Conclusion

For model generalization results table, adopt Rel-18 Table 2 and Generalization Case 1 / 2 / 3 as starting point with same additions above. For generalization aspects, adopt the following

- Various UE speed

- UE distribution

- Various CSI-RS periodicity

Conclusion

For model scalability results table, adopt Rel-18 Table 3 and Generalization Case 1 / 2 / 3 as starting point with same additions above. For generalization aspects, adopt the following

- Various numbers of antenna ports

- Various frequency granularity

- Various payload size

Conclusion:

- Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors).

- It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4.

Observation

- Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed

- Option 1 and 2 may require high specification effort from RAN1 perspective.

Conclusion

- Deprioritize Option 2 for inter-vendor training collaboration.

  - Note: This deprioritization shall not affect the ongoing discussion in RAN4 on RAN4-Option3 and RAN4-Option4.

Agreement

- For Option 3, further define the two sub-options:

  - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 5, further define the two sub-options:

  - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 4, it is clarified that:

  - Dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing.

- Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavors (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Agreement

- For Option 3/4/5, focus further discussion on the following assumptions:

  - Option 3a/5a

    - The model(5a)/parameter(3a) exchange originates from the NW-side and ends at the UE-side.

    - Model(5a)/parameters(3a) exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both.

      - Option 3a-1/5a-1: Model/Parameters exchanged from the NW-side to UE-side is CSI generation part.

      - Option 3a-2/5a-2: Model/Parameters exchanged from the NW-side to UE-side is CSI reconstruction part.

      - Option 3a-3/5a-3: Model/Parameters exchanged from the NW-side to UE-side are both CSI generation part and CSI reconstruction part.

      - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance.

        - Performance target

        - Dataset or information related to collecting dataset

    - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc.

  - Option 3b

    - The method of exchanging is over the air-interface via model transfer/delivery Case z4.

    - The parameter exchange is from NW to UE.

    - Parameters exchanged from the NW-side to UE-side is CSI generation part.

  - Option 5b

    - The method of exchanging is over the air-interface via model transfer/delivery Case z4, assuming that the model structure is aligned based on offline inter-vendor collaboration.

    - The model exchange is from NW to UE.

    - Model exchanged from the NW-side to UE-side is CSI generation part.

  - Option 4:

    - The dataset exchange originates from the NW-side and ends at the UE-side.

    - Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI,  CSI feedback).

    - Option 4-2: Dataset exchanged from the NW-side to UE-side consists of (CSI feedback, reconstructed target CSI).

    - Option 4-3: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback, reconstructed target CSI).

    - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance.

      - Performance target

    - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc.

  - Note: For each option/sub-option of interest, companies to bring discussion on how inter-vendor collaboration complexity, interoperability, and feasibility may be addressed. Companies to strive to provide solution(s) that can address all the following aspects: inter-vendor collaboration complexity, performance, interoperability, and feasibility.

  - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavors (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Agreement

- For the results template used to collect evaluation results for AI/ML-based CSI compression using localized models, adopt Table 1 used in Rel-18 as starting point, capturing the generalized model result and the localized model result as separate columns, with the following additions for the localized model:

- Dataset description

  - Local region modelling: e.g., Option 1 or Option 2, and further details

  - Temporal modelling: e.g., how temporal variation is modelled in train and test sets

  - Dataset description for generalized model

Conclusion

In Rel-19 study of temporal domain aspects of AI/ML-based CSI compression using two-sided model, CSI prediction that is performed entirely at NW-side is deprioritized.

Agreement

- For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for the temporal domain prediction and compression Case 3 and Case 4, adopt the following evaluation assumptions as baseline:

  - Observation window (number/distance):

    - For periodic CSI-RS with 5ms periodicity: 12/5ms, 10/5ms, 8/5ms, 5/5ms, 4/5ms, unrestricted observation window

    - For periodic CSI-RS with 20ms periodicity: up to companies (encouraged)

    - For aperiodic CSI-RS: 12/2ms, 8/2ms, 4/2ms

    - Others can be additionally submitted

  - Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance):  4/5ms/5ms

    - Others can be additionally submitted, e.g. 4/1ms/5ms, 8/1ms/5ms, 4/5ms/10ms, 1/-/5ms

Agreement

For the results template used to collect evaluation results for temporal domain prediction and compression Case 4, adopt Table 1 used in Rel-18 as starting point with the following additions:

- Description of model input/output and use case

  - Methods to handle rank adaptation (if applicable)

## Agreements from RAN1 #117

Conclusion

Standardized signalling, if feasible and specified, can be used for parameter / model exchange in option 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity.

- Standardized signalling may be reused for exchanging CSI generation part, CSI reconstruction part, or both, etc, when necessary and feasible.

- Standarized signalling may be over-the-air, or other approaches.

Standardized signalling, if feasible and specified, can be used for dataset exchange in option 4 to alleviate/resolve the inter-vendor training collaboration complexity.

- Standardized signalling may be reused for dataset exchanging, when necessary and feasible.

- Standarized signalling may be over-the-air, or other approaches.

Note: feasibility will be discussed separately.

Agreement

- For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact.

- Exchange

  - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a)

  - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4)

  - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4)

    - Performance target (3a/5a/4)

    - Dataset or information related to collecting dataset (3a/5a)

    - Any other additional information

- Model pairing (3a/3b/4/5a)

- UE capability (3a/3b/4/5a)

- Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling (3a/3b/4/5a)

- Quantization of feedback (3a/3b/4/5a)

- Model structure details (3a/3b)

Note: Option 3a/4/5a and option 3b serve two different deployment time scales, UE capabilities, device-side optimizations, and training methods, and therefore may be complementary to each other, with potential specification of both.

- Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters.

Agreement

For option 1 / 3 / 4 / 5 and their sub-options, study mechanisms (e.g., post-deployment performance monitoring) for identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation to guarantee good performance in the field.

Agreement

For temporal domain aspects Case 3/4, change the small / medium / large payload region definition as follows:

Note: X, Y, Z, A, B, and C are feedback overhead rates in bits per time unit of 5ms. Note: For X, Y, and Z, α=[2] for rank=1/2 and α=[4] for rank=4 Note: For A, B, and C, β=[0.5] for rank=1 and β=[0.75] for rank=2/4

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression (Cases 1-5), in addition to FLOPs, also consider FLOPs per normalized time unit. Use 5msec as the normalized time unit.

Agreement

In the results template for capturing the evaluation of temporal domain aspects Case 3/4 of AI/ML based CSI compression, regarding the “upper bound”, capture both of the following:

- upper bound based on ideal CSI prediction and without CSI compression

- upper bound based on benchmark CSI prediction and without CSI compression

Agreement

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, regarding training,

- The k-th local model is trained on region #B_k (the k-th local region), 1<=k<=N.

- The generalized model is trained on Region #A that may be constructed via any of the following methods that is appropriate for the given generalized/local region modeling approach.

  - Region #A is the same as the union of regions #B_1, …, #B_N.

  - Region #A is a proper superset of the union of regions #B_1, …, #B_N.

  - Region #A is generated separately from regions #B_1, …, #B_N.

  - Note: companies to report which method was used.

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, regarding testing,

- The trained generalized model, local model, and the non-AI/ML benchmark are tested on the regions #B_1, …, #B_N.

- In case N>1, when reporting the results, companies may report the performance of the generalized model, the local models, and the non-AI/ML benchmark, by averaging the performance over the regions #B_1,…,B_N. Companies to report the value of N.

Agreement

For collecting evaluation results for temporal domain aspects of AI/ML-based CSI compression using localized models, use the same results template used to collect evaluation results for AI/ML-based CSI compression using localized models

- Adding the same temporal setting that is used for results template used to collect evaluation results for temporal domain compression Case 1/2/5.

| Temporal setting | Temporal domain aspect Case 1-5 |
| --- | --- |
|  | CSI-RS configuration: periodic or aperiodic For periodic: periodicity For aperiodic: # of resources K in the CSI-RS burst / time internal m in msec |
|  | CSI reporting periodicity |
|  | Usage of historical CSI at UE side: number / time distance |
|  | Usage of historical CSI at NW side: number / time distance |
|  | Prediction window: number / time distance between prediction instances / distance from the last observation instance to the 1st prediction instance (Only applicable to Case 3,4) |

Agreement

Further study following monitoring options in Rel-19, including the necessity and feasibility,

- NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1)

  - SRS-based monitoring

- UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on the output of the CSI reconstruction model at the UE (Case 2-1)

    - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side.

  - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2)

  - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI

  - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model

  - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook

Regarding monitoring metrics:

- Monitoring accuracy also includes generalization considerations, if applicable.

- Complexity also includes LCM complexity, if applicable.

- Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions.

- Testability of UE reported metrics

Discussion may include the following aspects:

- Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration

- Temporal domain aspects of CSI compression

- How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation

Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches.

Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

Agreement

For temporal domain aspects Case 3 and 4, study the impact on LCM aspects of separate prediction and compression, and joint prediction and compression.

Note: Observations of companies results till RAN1#117 are captured in FL summary R1-2405419.

## Agreements from RAN1 #118

Agreement

For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3.

Agreement

For studying the standardization of model structure RAN1, RAN1 assumes at least the following:

- Precoding matrix as an input (as opposed to raw channel matrix)

  - Per-layer processing, with common structure across ranks and layers (corresponding to Option 2-1, 2-2, 3-1, or 3-2 for handling rank ≥ 1)

- For temporal domain aspects Case 2 and Case 3, strive to reuse the model structure of Case 0 where appropriate, with additional layers or operations either at the input/output domain or at the latent domain.

- For Case 0, use precoding matrix, e.g., eigen vector in spatial-frequency domain, and/or angular, delay domain representation such as eType-II W2.

- For Case 2 and Case 3, use precoding matrix, e.g., spatial-frequency domain for each CSI observation instance, and/or angular, delay, and Doppler domain representation such as eType-II W2.

- Consider scalability approaches over numbers of Tx ports, CSI feedback payload sizes, and bandwidths.

Observation

Option 5 alone doesn’t fully address inter-vendor collaboration complexity, but Option 5 has potential performance benefit and flexibility compared to Option 3.

Agreement

Continue the study of following directions - on-device operation and UE side offline engineering

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4)

  - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues

    - [Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing?

    - [Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it?

    - [Issue 3] Is there an overhead concern, and if so, how to address it?

    - [Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it?

- Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues

  - [Issue 3] Is there an overhead concern, and if so, how to address it?

  - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs?

  - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it?

  - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it?

- Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues

  - [Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them?

  - [Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it?

  - [Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing?

  - Note:

    - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder.

    - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder.

    - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder.

- Note: UE-side data and NW-side data in “UE-side data distribution” and “NW-side data distribution” are field data.

- Note: Some issues identified in one direction may/may not be applicable for other Direction.

- Note: potential down selection among the 3 directions is not precluded

- Study of data distribution mismatch to consider the use of synthetic data and/or field data.

Agreement

Adopt all the observations in Section 2 of R1-2407499.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, study the performance impact resulting from non-ideal UCI feedback.

- Scenario A: no UCI loss

  - Note: Corresponds to an upper bound or re-aligning missing historical CSI information

- Scenario B: UCI loss, known at NW and unknown at UE, with mitigation at NW

  - Note: Corresponds to implementation-based mitigation at NW but no signaling to UE.

- Scenario C: UCI loss, known at NW and UE, with mitigation at NW and UE

  - Note: Corresponds to reset of historical CSI information at both UE and NW or any other mitigation approach enabled by signaling.

- UCI loss modeling

  - 10% UCI loss probability on all UCI reports

  - Other options are not precluded, e.g., No UCI loss for the first UCI report of each observation window, and 10% UCI loss probability for the subsequent reports of each observation window, as a shortcut to simplify the evaluation work.

  - Other values for UCI loss probability are not precluded.

- Note: The same UCI loss modeling shall be applied to the benchmark for fair comparison.

- FFS: partial PMI-related UCI loss

Agreement

According to the Rel-19 study objective,

- For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

RAN1 is recommending extending the study of the AI/ML CSI compression based on RAN1 understanding the study on CSI compression is not completed.

## Agreements from RAN1 #118-bis

Conclusion

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Answer to Issue 2: From RAN1 perspective, at least,

  - For option 4-1, there is no proprietary information concern of disclosing encoder input/output from NW side to UE side

  - For option 3a-1, there is no proprietary information concern of disclosing encoder parameters from NW side to UE side

  - Note: If some additional information is disclosed from network side to UE side, proprietary concerns for option 4-1 and 3a-1 can not be precluded by default.

# References

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” (Release 18), December 2023.

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #102, December 2023.

- R1-2401561, “Revised Final summary for Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #116, Feb. 2024.

- RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #103, March 2024.

- R1-2403505, “Final summary of Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #116-bis, Apr. 2024.

- R1-2405419, “Final summary of Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #117, May, 2024

- R1-2407347, “Final summary of Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #118, Aug. 2024

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #105, September 2024.