3GPP TSG RAN WG1 #118bis	R1-2409167

Hefei, China, 14th–18th October 2024

Agenda Item:	9.4

Source:	Moderator (MediaTek Inc.)

Title:	Moderator summary #1 for RAN1 reply to RAN2 LS on TB size for Ambient IoT

Document for:	Discussion and Decision

Introduction

This summary concerns the RAN1 reply to the RAN2 LS [1], where RAN2 asks for RAN1’s input regarding the following aspects:

- The maximum and minimum TB sizes expected to be specified in PHY, in both D2R and R2D directions

- The limitations/conditions under which TBs of different sizes can be transmitted:

- Overall Description: RAN2 have discussed the role of the MAC in handling upper-layer data blocks (MAC SDUs) and processing them into transport blocks (MAC PDUs) and the need of segmentation in RAN2. RAN2 expect to follow RAN1 conclusion and define MAC PDU sizes to align with the capacity of the physical layer/TB sizes. To take an informed decision about the need for the segmentation, RAN2 would like to know the TB sizes (in both D2R and R2D directions) RAN1 intend to specify/use. ⋯ - Actions: To RAN1: RAN2 respectfully ask RAN1 to indicate what maximum and minimum TB sizes are expected to be specified in PHY, in both D2R and R2D directions and the limitations/conditions under which TBs of different sizes can be transmitted. ⋯

Companies have provided their input in contributions [2] – [21] submitted to agenda item 5 and contributions [22] – [30] submitted to agenda item 9.4.2.3.

The issues that are in focus in the initial discussion round are tagged FL1

To avoid excessive email load on the RAN1 email reflector, please note that there is NO need to send an info email to the reflector just to inform that you have uploaded a new version of this document.

Companies are invited to enter the contact info in the table below.

FL1 Question 0a: Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| MTK | Junqiang Cheng | Junqiang.cheng@mediatek.com |
| Lenovo | Karthikeyan Ganesan | kganesan@lenovo.com |
| NTT DOCOMO | Mayuko Okano | mayuko.okano.ca@nttdocomo.com |
| Huawei | Matthew Webb | matthew.webb@huawei.com |
| Huawei | Yiling Wu | wuyiling@huawei.com |
| LG Electronics | Youngdae Lee | youngdae.lee@lge.com |
| Ericsson | Johan Bergman | johan.bergman@ericsson.com |
| Nokia |  | torsten.wildschek@nokia.com |

Discussions

Maximum TB size and limitations/conditions

Based on the submitted contributions in this meeting, companies provide their view regarding the maximum TB size in PHY for R2D and D2R directions, and the corresponding limitations/conditions for achieving the maximum TB size.

- Maximum TB size in R2D and D2R directions

  - 1000 bits: ZTE, Huawei, SPRD, Vivo, Xiaomi, Oppo, Apple, CMCC, China Telecom, MTK, InterDigital, Qualcomm, NEC (at least for R2D), Sharp, Google, Samsung (proof needed), LGE,  IIT-K, IIT-M, CATT (128 bytes).

  - SPRD also proposes 500 bits.

  - Some companies show concern on the performance/coverage achievability of a TB size up to 1000bis. For example, Ericsson observes R2D transmission with message size of 1000 bits (without segmentation) cannot meet a 7-kbps data rate target with 10% BLER even without SFO. Nokia thinks the maximum PRDCH payload size must be restricted to ≤400bits while considering the sampling offset of ≤100ppm. While several other companies, e.g., CMCC, MTK, Oppo, Apple, etc., show that for R2D direction, considering a higher device sensitivity defined by Budget-Alt1, also the ON-OFF edge detection and clock calibration at device side, 1000 bits is feasibility with the consideration of the performance/coverage achievability.

  - FUTUREWEI proposes three message sizes of 20, 96, and 400 bits are evaluated. It is expected that message sizes of at least several hundred bits can be supported.

- Limitations/conditions for determining maximum TB size

  - Performance achievability: Ericsson, Samsung, ZTE, CMCC, NTT DOCOMO, Vivo, Xiaomi, InterDigital, MTK, Apple, Oppo, Nokia, Lenovo.

  - Data rate: ZTE, NTT DOCOMO, Vivo, Xiaomi, MTK, Apple.

  - Device availability duration: Samsung, ZTE, NTT DOCOMO, , Vivo, Xiaomi, InterDigital, MTK, Lenovo, Apple.

  - Device buffer size: Xiaomi, Samsung, MTK

Based on the inputs, it seems that most companies think the maximum TB size in PHY is 1000bits, but it is only achievable under some specific conditions, e.g., higher data, and/or longer device availability duration, etc.

Therefore, Moderator wants to ask company’s view on the following questions.

FL1 Question 1a: Do you agree that the maximum TB size in PHY is 1000 bits for R2D and D2R directions? (If no, please elaborate in the comment field your view on the maximum TB size achievable in PHY for R2D and D2R directions).

| Company | Y/N | Comments |
| --- | --- | --- |
| Lenovo | N | The Max. TB size is affected by performance loss – SNR degradation for larger TBS, number of repetition needed to recover the loss, impact on SFO on larger TBS, device availability for larger TBS. Since we evaluated only 20, 96 and 400 bits and did not agree to evaluate 1000 bits in LLS, so we need to understand the SNR degradation from 96 to 1000 bits. From our LLS simulation, SNR degradation from 96 to 400 bits is ~4dB. The impact on SFO on larger TBS is more compared to smaller TBS. The larger TB size needs more repetition to reach 10% or 1% BLER which increases the sustainable operation time and moreover the impact on SFO is more for larger TB size. The device availability for this payload size has been found as: Observation 8: For 96 bits payload, the transmission time increases from 14.9ms to 89.6ms when the data rate decreases from 6kbps to 1kbps. - With duty-cycle and energy harvesting, the device can be sustainably operated in case of 6kbps with storage smaller of less than 0.5uF, while it requires at least 1.5uF storage to be sustainably operate in case of 1kbps. - With always ON condition, the device can be sustainably operated in case of 6kbps with storage smaller of less than 2uF, while it requires at least 10uF storage to be sustainably operate in case of 1kbps. Observation 9: For 400 bits payload, the transmission time increases from 69.3ms to 388ms when the data rate decreases from 6kbps to 1kbps. - With duty-cycle and energy harvesting, the device can be sustainably operated in case of 6kbps with storage smaller of less than 1.5uF, while it requires at least 6uF storage to be sustainably operate in case of 1kbps. - With always ON condition, the device can be sustainably operated in case of 6kbps with storage smaller of less than 6uF, while it requires very high storage capacity storage to be sustainably operate in case of 1kbps. Hence the max. TB size should be restricted to be around ~150 bits to meet the coverage target and operate sustainably. |
| DOCOMO | Need more discussion | First, achievable data rate and device sustainable operation time for each device type should be discussed. For example, for R2D, if M=16 is available as max M value and Manchester encoding is applied, 1000bits TB requires 8.9 ms or longer, e.g., more than 10ms with some overheads to receive the TB. For D2R, FEC is considered as baseline and it can be applied even if no line coding is applied, and hence the required time duration to transmit 1000bits TB would be comparable or longer than R2D with the similar data rate as R2D. In our view, device can sustain the duration corresponding to 1000bits TB Tx/Rx for device 1, however, it is questionable whether device 2 which consumes much larger power can sustain the duration especially for D2R transmission even with the highest peak rate. Therefore, we prefer to discuss such aspects first as well as concern on coverage before rushing to the conclusion. |
| Qualcomm | Y | We think it is possible in certain conditions, e.g., for a device with high data rate and long enough sustainable time. The max TBS to be defined in spec does not mean it can be always supported. |
| Moderator |  | To Lenovo: Based on the TDoc review, it seems most companies think when we say the maximum TB size of 1000 bits, it means 1000 bits is achievable in some certain conditions. It does not necessarily mean that 1000 bits should always be achievable/guaranteed. To ALL, please pay attention to Question 2a as well, which is highly related to Question 1a. |
| Huawei, HiSilicon | Y |  |
| Vivo1 |  | We think 1000bits is feasible only when some conditions are met, e.g, data rate is high, SNR is high, sufficient resources available for AIOT, device has sufficient power to finish the transmission/reception. For Question1a, we are open to consider 1000 bits but it should not give other WG impression that it is typical value assumed in RAN1. RAN1 need to mention in the reply LS that it may not always be guaranteed considering the factors such as radio conditions, resource allocation, and the energy required at the device for transmitting or receiving the maximum TB size of 1000 bits. |
| LG Electronics | Y | In the relay LS, RAN1 can mention that 1000bits can be supported in some conditions, but it is not always achievable/guaranteed. |
| Spreadtrum | Y |  |
| Ericsson | N | Our understanding is that RAN2 wants RAN1 input that will help RAN2 to decide whether there is a need for MAC-layer segmentation. This seems to require that aspects such as achievable bit rate and sustainable operation time should be considered. In our contribution, we have provided simulation results indicating that R2D/D2R TB sizes of hundreds of bits may only be achievable if the SFO is rather low, e.g., 100 ppm. |
| Apple | Y | 1000 bits TBS is expected to be supported mainly in high SNR conditions when the device is near the reader and a high data rate can be supported by the link |
| Nokia, NSB | N | We are not convinced that 1000 bit TBS is achievable for R2D even in good link conditions unless the SFO is low, according to our evaluations. |

FL1 Question 2a: Do you agree that the maximum TB size in PHY is only achievable in certain limitations/conditions, e.g., smaller timing drift, higher data rate, and/or longer device availability duration?

| Company | Y/N | Comments |
| --- | --- | --- |
| Lenovo | Y | What is the point of defining max TBS of 1000 bits and have restriction to achieve that, which means it cannot be used most of the time. Also, we need study how to apply such restriction to select the max. TBS which is unnecessary So lets limit the Max. TBS to a reasonable value of ~150 bits |
| DOCOMO | Y | We agree that listed aspects such as timing drift, higher data rate and availability duration of device can be the condition for availability of max TBS. |
| Qualcomm | Y |  |
| Moderator |  | To Lenovo: Please find Moderator’s reply to Question 1a on the understanding of maximum TB size. |
| Huawei, HiSilicon | This is not really the point | It’s a strange question, because the TBS to be used is (at least in legacy systems) instructed to PHY by MAC, based on scheduler implementation. The physical layer transmits what it is told, and the transmission may or may not succeed. Nevertheless, the max TBS is “available”, and the scheduling has to work out what is appropriate. How to do that is never specified. We don’t think it can be productive for RAN1 to devise A-IoT scheduling descriptions, and RAN2 should not expect such information. |
| Vivo1 | Y | In addition to the conditions listed above, we think sufficient resource should also be included in the e.g., part. For example, in topology2, if NW allocate a resource pattern with very a short time resource duration to IN UE reader, UE reader is not possible to schedule TBS with 1000 bits e.g., smaller timing drift, higher data rate, sufficient available resource and/or longer device availability duration? |
| LG Electronics | Y |  |
| Spreadtrum | Y |  |
| Ericsson | Y |  |
| Apple | Y |  |
| Nokia | Y |  |

Minimum TB size and limitations/conditions

Based on the submitted contributions in this meeting, companies provide their view regarding the minimum TB size in PHY for R2D and D2R directions, and the considerations for determining the minimum TB size.

- Minimum TB size in R2D and D2R directions

  - 1 bit: ZTE

    - ZTE also mentions actual TB size pending requirements like inventory/command signaling design, etc.

  - 4 bits: Xiaomi

    - 4 bit is from the minimum command size in RFID, i.e., QueryRep.

  - 8 bits: Qualcomm

    - Qualcomm also proposes a typical TB size of ~15 bytes.

  - 16 bits: Apple, CMCC (smaller value not precluded)

    - 16 bits is from the length of random ID agreed in RAN2

  - 16 or 20 bits: Vivo

    - Vivo thinks the minimum TB size is consistently available and feasible

  - 20 bits: China Telecom, FUTUREWEI

    - 20 bits is from the minimum message size agreed for evaluation in agenda 9.4.1.1, i.e., {20 bits, 96 bits, 400 bits})

  - [20, 60] bits: SPRD

    - The range of [20, 60] is calculated based on some PHY conditions like data rate and TTI.

  - 100 bits: MTK

    - The value is calculated based on some PHY conditions like data rate and device availability duration, and MTK thinks the minimum TB size is always achievable.

  - No restriction: Huawei (e.g., 16), Oppo, NTT DOCOMO.

Based on the inputs, companies’ view on the min TB size are quite divergent because companies may share different understandings on the definition of the minimum TB size.

Some companies think that the minimum TB size is defined as the minimum of the TB sizes determined by the PHY conditions (e.g., data rate and/or device availability duration). For example, based on different PHY conditions, SPRD calculates that the minimum TB size ranges from 20 to 60 bits [8], and MTK concludes it could be 100 bits [16]. In that sense, they both think for a message with size smaller than the minimum TB size, the segmentation could be avoided. ZTE mentions without further conclusion on some PHY conditions, e.g., R2D/D2R waveform and channel/line coding schemes, the minimum TB size can be as small as 1 bit, but the actual TB size depends on the requirements such as inventory/command signaling design.

While some other companies think that the minimum TB size is defined as the minimum message size delivered from higher-layer (e.g., no limitation on the possible minimum TB size in PHY), e.g., CMCC, NTT DOCOMO, Apple, Huawei, Xiaomi, Oppo, etc. For example, CMCC mentions the minimum TB size in PHY is 16 bits (smaller value not precluded) considering the random ID for 3-step CBRA is fixed as 16 bits by RAN2 [20]. Xiaomi mentions it could be 4 bits considering the minimum command size in legacy RFID is QueryRep with length of 4 bits [10].

Therefore, Moderator wants to ask company’s view on the following questions.

FL1 Question 3a: Which option you think should be used for defining the minimum TB size in PHY for both R2D and D2R directions?

- Option 1: The minimum of TB sizes determined by PHY conditions.

- Option 2: The minimum message size delivered from higher-layer.

| Company | Supported option | Comments |
| --- | --- | --- |
| Lenovo | 2 | The minimum message size delivered from higher layer lets say command etc., should not be segmented. |
| DOCOMO | Option 2 | We are open to discuss the min TBS condition from RAN1 perspective, but we don’t see any condition so far. |
| Qualcomm | Need more clarification | If following Option 2, is the TBS counted as 0bits in case there is only L1 control information in PRDCH? If following Option 1, does it mean it is min TBS considering the worst PHY conditions? |
| Moderator |  | To Qualcomm: For the 1st question, moderator understands the pre-condition for this LS is, there is something delivered from higher layer (please see the LS content copied below.) “RAN2 have discussed the role of the MAC in handling upper-layer data blocks (MAC SDUs) and processing them into transport blocks (MAC PDUs) and the need of segmentation in RAN2.” For the 2nd question, moderator feels it may be pending some related discussions. For example, when there is a block delivered from higher layer, whether PHY is prepared for this. Considering Solution1/Solution2 is under discussion in agenda 9.4.2.2, the PHY conditions may be different from the device perspective if different solutions applied. Of course, let’s see more comments. |
| Huawei, HiSilicon | No particular minimum from RAN1 perspective. | It’s not logical to say via Option 1 that the minimum TB size varies. It is a minimum, after all; there cannot be anything smaller than the minimum. Option 2 is probably relevant, but in fact there is no air interface constraint to any small values. RAN2 should already know that well enough, so we can simply indicate there is no particular minimum from RAN1’s perspective. |
| Vivo1 | Option2 |  |
| LG Electronics | Option 2 | There is no constraint to minimum TB size from RAN1 perspective. It can be up to RAN2. |
| Spreadtrum |  | We don’t find the strict constraint for determining the minimum TBS. |
| Ericsson | Option 2 |  |
| Apple |  | I don’t think we need to consider either of this direction. Simply we can say there is no constraint from RAN1 perspective on the minimum size. |
| Nokia | Comment | Agree with others that we should just state that the minimum is not constrained from RAN1 perspective. |

FL1 Question 4a: Which option you think should be used for specifying the value of the minimum TB size in PHY for R2D and D2R directions?

- Option 1: The minimum TB size in PHY for R2D and D2R directions could be one of {1, 4, 8, 16, 20~60, 100} bits pending further study, e.g., data rate and/or device availability duration, etc.

- Option 2: The minimum TB size in PHY for R2D and D2R directions is around 16 bits. Other values are not precluded and may be specified pending further study, e.g., data rate and/or device availability duration, etc.

| Company | Supported option | Comments |
| --- | --- | --- |
| Lenovo | 2 |  |
| DOCOMO |  | It depends on the discussion of Question 3a. |
| Qualcomm |  | Is Op1 just to indicate how to study “Other values are not precluded and may be specified pending further study” in Opt2? |
| Moderator |  | To Qualcomm: For option 1, there is a limitation on the possible candidates of min TBS based on company’s input in this meeting. While for option 2, any possible candidates of min TBS are not precluded pending further study, and the min TBS of 16 bits is given as an example considering the situation so far. |
| Huawei, HiSilicon | ? | What’s the difference from Q3a? Doesn’t it contradict to have this question? Option 1 is not reasonable, as transmitting small TBS is very easy. Option 2 is possible, but we don’t think there is any particular minimum that RAN1 cannot specify less than. All of the discussion around how to write a scheduler is not relevant. |
| Vivo1 |  | Depends on the discussion of Question 3a. We can also inform RAN2 that the minimum TBS size in RAN1 evaluation is 20 bits. |
| LG Electronics |  | It depends on the discussion of Question 3a. We think that the value of the minimum TB size can be up to RAN2. |
| Spreadtrum |  | Same to the previous question. |
| Ericsson | 2 |  |
| Apple |  | Same comment as previous question |
| Nokia |  | See reply to previous question |

FL1 Question 5a: Do you agree that from RAN1 understanding, it does not mean that the segmentation is always required for a block size larger than the minimum TB size in PHY?

| Company | Y/N | Comments |
| --- | --- | --- |
| Lenovo | N | Lets rephrase the sentence and say that the segmentation is needed when the block size from high layer larger than the Max. TBS supported in PHY |
| DOCOMO | Y | In our view, segmentation is required only when the message delivered from higher layer exceeds max TBS of PHY. |
| Qualcomm | N | Our understanding is that the higher layer should schedule a TBS considering the PHY conditions, which is after segmentation if needed. But it does not mean that segmentation is always required if TBS >min TBS |
| Moderator |  | Moderator agrees the relation between segmentation and the max TBS in PHY, i.e., segmentation is needed when the block delivered from higher layer has a size larger than the max TB size in PHY, which seems is the majority view based on TDoc review. The point for this question is to collect company’s view on the relation between segmentation and the min TBS in PHY, for which Moderator observes some different understandings among companies. For example, some companies may think when the block delivered from higher layer has a size larger than the min TBS in PHY, then segmentation may be needed. While some other companies may think NO. That is the min TBS here only provided a lower bound for the TBS would be used in PHY, whether segmentation is needed when the packet delivered from higher layer has a size larger than the min TBS, it actually depends on some “real-time” PHY conditions, e.g., data rate, device availability duration, etc. |
| Huawei, HiSilicon | Out of scope of LS reply | There is no question in the LS for this. Question is out of scope. |
| Vivo1 |  | We share a similar view as QUALCOMM, whether segmentation is needed is depends on the real-time conditions and decided by reader. After that, reader determines the TBS for R2D transmission or D2R scheduling. But the question seems to imply segmentation is decided after determining the TBS. The intention behind the question from RAN2 is whether segmentation should be supported, and we think even if a message with a few bits is delivered from higher layer, it does not mean that reader/device can or is mandated to transmit it in one shot, it still depends on the physical transmission conditions. Therefore, the key point from RAN1 perspective is that Reader should determine one proper TB size for each R2D reception or D2R transmission, taking into account various factors. From RAN1 understanding, it is reader to determine a proper TBS and determine whether segmentation is needed or not by considering various factors including radio conditions, coverage objectives, resource availability, resource utilization efficiency, and an estimation of the energy available at the device side |
| LG Electronics |  | We think that segmentation is related to the max TBS, not min TBS. Min TBS may be meaningful just for TBS indication in L1 control info, if any. In any case, this question is out of scope. Details about segmentation can be up to RAN2. |
| Spreadtrum |  | This question is out of scope. |
| Ericsson | Y | Agree with comments from Lenovo and DOCOMO |
| Apple |  | In our view, we should not discuss on what RAN2 may potentially need to do. |
| Nokia | Y |  |

Proposal for Online (Tue)

Proposal 2.1a [H]

From RAN1 perspective, the maximum TB size in PHY for R2D and D2R directions is 1000 bits.

- The achievability may depend on timing drift, data rate, available resource, and/or device availability duration, etc.

- RAN1 have no consensus on any details of the above.

Proposal 2.2a [H]

From RAN1 perspective, the minimum TB size in PHY for R2D and D2R directions is around 16 bits. Other value(s) is feasible and not precluded pending further study.

Note: There is no air interface constraint to any small value.

Proposal 2.3a [M]

It is RAN1 understanding that segmentation is required for a block delivered from higher-layer with a size larger than the maximum TB size in PHY.

Proposal 2.4a [L]

It is RAN1 understanding that segmentation is not always required for a block delivered from higher-layer with a size larger than the minimum TB size in PHY.

# References

| [1] | R1-2407593 | LS on data block sizes for Ambient IoT | RAN2 MediaTek Inc. |
| --- | --- | --- | --- |
| [2] | R1-2408063 | Discussion on data block sizes for Ambient IoT | ZTE Corporation, Sanechips |
| [3] | R1-2408064 | Draft reply to LS on data block sizes for Ambient IoT | ZTE Corporation, Sanechips |
| [4] | R1-2408770 | Draft reply LS on data block sizes for Ambient IoT | NTT DOCOMO, INC. |
| [5] | R1-2409024 | On RAN2 LS on data block sizes for Ambient IoT | Ericsson |
| [6] | R1-2407673 | Discussion on RAN2 LS on Clarification of data block sizes for Ambient IoT | Huawei, HiSilicon |
| [7] | R1-2407727 | Discussion on RAN2 LS on data block sizes for Ambient IoT | China Telecom |
| [8] | R1-2407791 | Discussion on LS on data block sizes for Ambient IoT | Spreadtrum Communications |
| [9] | R1-2407827 | Draft reply LS on data block sizes for Ambient IoT | Vivo |
| [10] | R1-2407942 | Discussion on RAN2 LS on data block sizes for Ambient IoT | Xiaomi |
| [11] | R1-2407943 | Draft reply LS on RAN2 LS on data block sizes for Ambient IoT | Xiaomi |
| [12] | R1-2408151 | Discussion on LS from RAN2 on data block sizes for Ambient IoT | OPPO |
| [13] | R1-2408152 | Draft reply LS to RAN2 on data block sizes for Ambient IoT | OPPO |
| [14] | R1-2408445 | Discussion on RAN2 LS on data block sizes for AIoT | Apple |
| [15] | R1-2408446 | Draft reply LS to RAN2 on data block sizes for AIoT | Apple |
| [16] | R1-2408696 | Discussion on RAN2 LS on data block sizes for Ambient IoT | MediaTek Inc. |
| [17] | R1-2408697 | Draft reply LS to RAN2 on data block sizes for Ambient IoT | MediaTek Inc. |
| [18] | R1-2408725 | Discussion on RAN2 LS on data block sizes for Ambient IoT | InterDigital, Inc. |
| [19] | R1-2408831 | Draft reply LS on data block sizes for Ambient IoT | Qualcomm Incorporated |
| [20] | R1-2407886 | Discussion on RAN2 LS on data block size for Ambient IOT | CMCC |
| [21] | R1-2407887 | [Draft] Reply LS on data block size for Ambient IoT | CMCC |
| [22] | R1-2407612 | Discussion on D2R and R2D Channel/Signal Aspects for Ambient IoT | FUTUREWEI |
| [23] | R1-2408649 | Considerations for downlink and uplink channel/signal aspect | Samsung |
| [24] | R1-2407647 | R2D and D2R channel/signal aspects for Ambient IoT | Nokia |
| [25] | R1-2407819 | Discussion on channel/signal aspects for Ambient IoT | Lenovo |
| [26] | R1-2408050 | DL and UL Physical Channels/signals design in support of Ambient IoT devices | CATT |
| [27] | R1-2408208 | Discussion on downlink and uplink channel for ambient IoT | NEC |
| [28] | R1-2408252 | Discussion on downlink and uplink channel/signal aspects | Sharp |
| [29] | R1-2408371 | Discussion on downlink and uplink transmission aspects | Google |
| [30] | R1-2408943 | Discussion on Downlink and Uplink channel signal aspects for AIoT | IIT Kanpur, Indian Institute of Tech (M) |