3GPP TSG-RAN WG1 Meeting #118bis	R1-2409197

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.6.3

Title:	FL summary #2 on LP-WUS operation in CONNECTED mode

Source:	Moderator (NTT DOCOMO)

Document for:	Discussion, Decision

1	Introduction

The core part of the Rel-19 LP-WUS/WUR WI [1] has the following objective:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

This document summarizes contributions [4] – [27] submitted to agenda item 9.6.3 (LP-WUS operation in CONNECTED mode).

2	Proposals for Online Sessions

2.1	Proposals for Monday Online

[Mon]Proposal 3.1-1a:

- For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

UE is configured with legacy C-DRX configurations as Rel-18

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

    - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

[Mon]Proposal 3.1-2:

Update following agreement in red. Send LS to RAN2 to ask whether to confirm it or not

Agreement

- For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

UE is configured with legacy C-DRX configurations as Rel-18

UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

    - FFS potential No restriction for LP-WUS configuration in relation with C-DRX configuration

LP-WUS triggers the start of a timer during which UE monitors PDCCH

    - FFS the timer is existing timer and/or new timer

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

    - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

    - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

Summary of companies view

- FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

  - No restriction: Xiami, CMCC, LGE, TCL, vivo

  - Some restriction: ZTE, ETRI, Apple

  - Up to RAN2/Wait for RAN2 progress: QC, CATT, Nokia

- FFS the timer is existing timer and/or new timer

  - New timer: Xiami, ZTE, CMCC, LGE, vivo

  - Exsting timer: ZTE

  - Up to RAN2/Wait for RAN2 progress: QC, CATT, Nokia, Apple, TCL, Lenovo, MTK

[Mon]Proposal 3.1-3:

Confirm following working assumption

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Summary of companies view

- Confirm WA: ZTE, CATT, CMCC, Nokia, SPRD, ETRI, LGE, TCL, Lenovo, vivo, MTK

- Keep as WA: QC, Apple, Pana,

2.2	Proposals for Wednesday Online

[Wed]Proposal 3.2-1c:

Select one of the following alternatives in RAN1#118bis

Alt 1

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Alt2

For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Different minimum time gaps correspond to different sleep states

Alt3

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

UE can indicate additional UAI to assist NW to decide/adapt the configured time offset, FFS details

A single time offset is configured for a UE at a given time

FFS: whether/what additional information should be indicated to the UE to determine the minimum time gap

Strong concern

- Alt1: vivo, Xiaomi, TCL

- Alt2: ZTE, Nokia, LGE, MTK, E///, CATT

- Alt3: Nordic, HW, SS

[Wed]Proposal 4-2a:

- For RRC connected mode, support at least the following case:

LP-WUS is not expected to be monitored during the time where MR is active.

FFS the case where LP-WUS is expected to be monitored during the time where MR is active.

[Wed]Proposal 5-1:

- LP-WUS works when UE is configured with DC in RRC CONNECTED mode

FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

FFS: The cell(s) where LP-WUS is monitored

[Wed]Proposal 4-1:

- When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

FFS the condition for the fallback

For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

For LP-WUS operation option 1-2, FFS whether UE fallbacks to legacy C-DRX behavior or PDCCH monitoring without legacy C-DRX

[Wed]Proposal 3.1-4a:

- For LP-WUS CONNECTED mode operation, Option 1-2 is specified as standalone feature from Option 1-1 (i.e., Option 1-2-2 in previous agreement)

[Wed]Proposal 3.1-2b:

- Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, following can be considered for down selection

AltOpt1: No restriction for LP-WUS configuration in relation with C-DRX configuration

AltOpt2: At least LP-WUS periodicity is no larger than C-DRX cycle, FFS other restrictions

3	LP-WUS Procedures to trigger PDCCH monitoring

3.1	High-level procedures

This issue has been discussed in RAN1#116bis and following agreement was made.

Agreement Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

In general, it can be said that,

- Option 1-1: small power saving gain, no latency reduction, small spec impact, majority companies support w/o different interpretations of the previous agreement

- Option 1-2: tradeoff between large power saving gain and large latency reduction, large spec impact

  - Companies have different understanding which options 1-2-1 or 1-2-2 have larger spec impact, especially on the relationship between LP-WUS monitoring and C-DRX related timers

- Option 1-3: small power saving gain, useful for traffic with jitter, small/medium spec impact

  - Companies may have different understanding on the relationship between LP-WUS monitoring and C-DRX related timers

In RAN1#118, hence, much time has been used to have common understanding on Option 1-2 and following agreement was made

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. UE is configured with legacy C-DRX configurations as Rel-18 UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration LP-WUS triggers the start of a timer during which UE monitors PDCCH FFS the timer is existing timer and/or new timer UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL No impact on RRM/RLM/BFD measurement requirements is assumed For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

Furthermore, in the last RAN1 meeting, following working assumption was also made thanks to the great effort/compromise of companies.

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

According to companies input, it would be worth making similar agreement for Option 1-1 to clarify the behaviour, and following proposal is made (where difference from Option 1-2 is shown in bule)

[Closed]Proposal 3.1-1:

For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

UE is configured with legacy C-DRX configurations as Rel-18

UE is expected to be configured with LP-WUS monitoring configuration

FFS periodicity and offset in relation to the C-DRX configuration

LP-WUS triggers the start of drx-onDurationTimer during which UE monitors PDCCH

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following

Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

| Company | Y/N | Comments |
| --- | --- | --- |
| xiaomi | Y | OK with the proposal. |
| Qualcomm | Y |  |
| ZTE, Sanechips |  | For the CSI/L1-RSRP reporting, both option 1-1 and option 1-2 need further clarification. One point is for the aperiodic CSI/L1 RSRP report, we also need some conclusion. Another point is for the periodic CSI report, current spec for DCI 2-6, just say ‘if the UE is configured to monitor DCI format 2_6 ’, and did not say it indicates wake-up. Therefore, regardless LP-WUS indicate wake-up or not, in my understanding, the UE is required to report CSI during legacy active time, which is aligned with current spec. So, we hope, together aperiodic CSI report, we should have a overall solution. And also, discuss more details for the CSI reporting issue for both option 1-1 and option 1-2. |
| CATT | Y |  |
| CMCC | Y | Fine with the proposal. |
| NOKIA1 | Y |  |
| Spreadtrum | Y |  |
| ETRI | Y |  |
| LGE | Y |  |
| Apple | Y |  |
| TCL | Y |  |
| Panasonic | Y |  |
| Lenovo | Y |  |
| vivo | Y | For the clarification of periodic CSI/L1-RSRP reporting, in our view, we propose the following: Clarify that for both LP-WUS operation option 1-1 and option 1-2 of LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with one of the following: - Configuration 1: - If the UE is not indicated to wake-up, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake-up, the periodic CSI/L1-RSRP is reported during the DRX active time. - Configuration 2: - If the UE is not indicated to wake-up, the periodic CSI/L1-RSRP is reported during the time duration indicated by drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start. - If the UE is indicated to wake-up, the periodic CSI/L1-RSRP is reported during the DRX active time. |
| MTK | Y |  |

[Closed]Proposal 3.1-1a:

For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

UE is configured with legacy C-DRX configurations as Rel-18

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following was agreed in Monday online Agreement For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer UE is configured with legacy C-DRX configurations as Rel-18 UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal No impact on RRM/RLM/BFD measurement requirements is assumed For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification |

In addition to the above proposal, companies provided further view, which can be discussed after the above proposal is stable.

- UE LP-WUS monitoring

  - Vivo

    - if the LP-WUS is not detected on a WUS MO by the UE, the UE is not required to trigger the starting of the drx-onDurationTimer

  - QC

    - For option 1-1 of LP-WUS CONNECTED mode operation, discuss and clarify whether a UE is expected or not expected to monitor LP-WUS in DRX Active Time.

- LP-WUS configuration

  - CMCC

    - a time offset between LP-WUS and the starting point of DRX ON is configured without the configuration of periodicity

  - ZTE

    - LP-WUS monitoring could be configured with a same periodicity with C-DRX and an offset

      - Discuss whether LP-WUS could trigger short DRX cycle

      - Discuss the offset is relative to C-DRX or others

  - OPPO

    - LP-WUS monitoring periodicity and offset can be introduced for triggering a PDCCH monitoring timer, in CONNECTED mode for both Options.

  - Samsung

    - The periodicity for LP-WUS monitoring occasions is same as the DRX cycle;

    - At least one LP-WUS monitoring occasion is configured before the drx-onDurationTimer, subject to the minimum delay requirement;

  - LGE

    - Specify LP-WUS MOs with a periodicity.

      - LP-WUS inside monitoring window triggers PDCCH monitoring by starting drx-onDurationTimer (Option 1-1).

      - LP-WUS outside monitoring window triggers PDCCH monitoring outside legacy DRX Active Time (Option 1-2).

  - E///

    - the periodicity of LP-WUS monitoring is same as the C-DRX cycle length.

    - wus-offset for Option 1-1: an offset indicating a start of a time window where the UE starts monitoring LP-WUS, prior to a slot where the drx-onDurationTimer is to be started.

  - QC

    - Consider MAC-CE for LP-WUS activation/deactivation

      - A UE can be configured with one or multiple LP-WUS configurations

      - The MAC-CE activates one (or none) of the LP-WUS configurations dynamically

Note that RAN2 made following agreement, i.e., LP-WUS MO is configured with a time offset before the start of drx-onDurationTimer

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

Regarding Option 1-2, the previous agreement includes some FFS, and companies provided view

- FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

  - dicussed in RAN2: QC

  - No relation: HW/HiSi, vivo, SPRD, Nokia, E///

  - the periodicity should be the same or integer multiple of C-DRX periodicity: ETRI

- FFS the timer is existing timer and/or new timer

  - dicussed in RAN2: HW/HiSi, ZTE, QC

  - new timer: vivo, IDC, Apple, Nokia, LGE, E///

  - drx-InactivityTimer: LGE

  - existing timer: TCL

- Wait for RAN2 progress: CATT

- Wait for Option 1-1 progress: Pana

Overall, a number of companies assume these FFS can be discussed in RAN2, while there is clear majority in RAN1. One potential way forward is that RAN1 make decision from RAN1 perspective and ask RAN2 whether to confirm it or not. Note that RAN2 made following agreement in the last RAN2 meeting.

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

[Closed]Proposal 3.1-2:

Update following agreement in red. Send LS to RAN2 to ask whether to confirm it or not

Agreement

For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

UE is configured with legacy C-DRX configurations as Rel-18

UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

FFS potential No restriction for LP-WUS configuration in relation with C-DRX configuration

LP-WUS triggers the start of a timer during which UE monitors PDCCH

FFS the timer is existing timer and/or new timer

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

| Company | Y/N | Comments |
| --- | --- | --- |
| xiaomi | Y | Generally OK with the proposal. But still one question about drx-onDurationTimer, whether the drx-onDurationTimer is started or not in Option 1-2? |
| Qualcomm |  | We think it is more straightforward to let RAN2 to fix these FFSs for Option 1-2. However, we are open to do so in RAN1 if it is majority view. And the proposal is fine for us. Note that for Option 1-1, some companies propose to configure time offset to the start of drx-onDurationTimer only (without periodicity). In Rel-16 DCP, monitoring occasions for SS set for DCI format 2_6 can be independent from C-DRX cycle and the start of drx-onDurationTimer. From the configured SS set monitoring occasions, some are defined as valid occasions that may carry the DCI format 2_6. In this sense, we think the same applicable to Option 1-1. I.e., the LP-WUS occasions for Option 1-1 can be configured in the same way as for Option 1-2. |
| ZTE, Sanechips |  | - If the timer is only new timer, that’ s to say option 1-2-1 is precluded. We prefer to support both at this moment. - It is to early to say No restriction for LP-WUS configuration in relation with C-DRX configuration. For example, what’s the motivation to support a LP-WUS periodicity larger than C-DRX, considering there could be periodic CSI-RS report in legacy active time. - As option 1-1, CSI report issue should be further discussed. |
| CATT |  | Regarding the FFS the timer is existing timer and/or new time, we also prefer the new timer due to the new timer can start on-demand based on the LP-WUS indication in Option 1-2. However, there are lots of procedures and behaviours associated with the new timer to be discussed in RAN2. For example, how to handle the legacy drx-onDurationTimer for the periodic CSI/L1-RSRP reporting should be further studied. Also, the criteria of stop PDCCH monitoring for achieving UE power saving is not defined and needs to be further discussed. DCI-based, the LP-WUS or MAC-CE can be applied to trigger the termination of the PDCCH monitoring. Thus, we suggest RAN1 wait more progress from RAN2 for Option1-2. |
| CMCC | Y | We are generally fine with this proposal. For Xiaomi’s concern, our understanding is that Option 1-2 does not impact both UE and gNB behaviour on starting drx-onDurationTimer. |
| NOKIA1 |  | Generally fine with the proposal, though we appreciate RAN2 can/should have the ultimate decision on the new timer (s). We also wonder, if given the potentially very different setup of LP-WUS with 1-2 compared to legacy cDRX, whether in addition to a new OnDuration timer, a new Inactivity timer if appropriate. |
| ETRI |  | Seems to agree with ZTE’s views. According to the previous agreement, Option 1-2 is only valid outside C-DRX active time. With this assumption, is there any reason to remove the restriction for LP-WUS configuration in relation with C-DRX configuration? |
| LGE | Y | Generally fine with the proposal. |
| Apple |  | For FFS on the timer, although we support introducing new timers to be triggered by LP-WUS, we also think it is more suitable to be discussed in RAN2. For the relation between LP-WUS configuration and C-DRX configuration, we have similar view as ZTE, e.g. at least LP-WUS periodicity should be smaller than C-DRX cycle. |
| TCL |  | Generally Fine with the proposal. However, regarding the new timer or existing timer, in our view, the new timer will increase the RAN2 work. Therefore the timer decision shall be left to RAN2. |
| Lenovo |  | At least the timer issue should be discussed by RAN2 instead of RAN1. |
| vivo | Y | When LP-WUS is detected, LP-WUS triggers the start of a new timer during which UE monitors PDCCH for Option 1-2, since the new timer is preferred for a cleaner design as the drx-onDurationTimer covers traffic arrival uncertainty instead of confirmed traffic arrivals. Besides, no need to restrict for LP-WUS configuration in relation with C-DRX configuration for option 1-2. |
| MTK |  | The timer FFS can be left to R2. |

- FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

  - No restriction: Xiami, CMCC, LGE, TCL, vivo

  - Some restriction: ZTE, ETRI, Apple

  - Up to RAN2/Wait for RAN2 progress: QC, CATT, Nokia

- FFS the timer is existing timer and/or new timer

  - New timer: Xiami, ZTE, CMCC, LGE, vivo

  - Exsting timer: ZTE

  - Up to RAN2/Wait for RAN2 progress: QC, CATT, Nokia, Apple, TCL, Lenovo, MTK

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following was agreed in Monday online Agreement Update following agreement in red. Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. UE is configured with legacy C-DRX configurations as Rel-18 UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration LP-WUS triggers the start of a timer during which UE monitors PDCCH FFS the timer is existing timer and/or new timer UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL No impact on RRM/RLM/BFD measurement requirements is assumed For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS For the second FFS in the above agreement, the final decision is left to RAN2. Regarding the FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration, additional proposal is made for further discussion. Regarding the periodic CSI/L1-RSRP reporting, this can be further discussed in Proposal 3.1-6 |

[Old]Proposal 3.1-2a:

Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, following can be considered for down selection

Alt1: No restriction for LP-WUS configuration in relation with C-DRX configuration

Alt2: At least LP-WUS periodicity is no larger than C-DRX cycle, FFS other restrictions

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal is made based on the comments from companies |
| CMCC | Y | Slightly prefer Alt1. In addition, from our understanding this can be implicitly ensured by configure relatively small periodicity for LP-WUS MO. |
| ZTE, Sanechips | Y | Based on option 1-2-1, alt2 is nature and there is no need to configure a large periodicity than C-DRX. Based on option 1-2-2, if periodic CSI report is needed. Also, alt2 can be nature. Compared with option 1-1, option2 provide the latency and the PSG. If only PSG is pursued, then option 1-1 would be enough. |
| Qualcomm |  | We suggest to change ‘Alt’ to ‘Opt’ and delete ‘for down selection’. Between the two ways, second one sounds reasonable, since LP-WUS periodicity larger than C-DRX cycle can be supported by Option 1-1. |
| NOKIA2 |  | Slight preference for Alt1, for the following reasons: (1) keeps the option open to use a sparse LP-WUS (periodicity larger than cDRX) in the event that LP-WUS resources are low and one LP-WUS is used to wake MR for a succession/set of consecutive ACTIVE periods. (2) Alt2 is a subset of Alt1 |
| ETRI | Y | Prefer Alt 1. Anyway Option 1-2 has a relationship with C-DRX since it is available outside of C-DRX active time. |
| CATT | Y | OK with the proposal, and we prefer Alt1. |

Alt1: CMCC, Nokia, ETRI, CATT

Alt2: ZTE, QC

[Wed]Proposal 3.1-2b:

Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, following can be considered for down selection

AltOpt1: No restriction for LP-WUS configuration in relation with C-DRX configuration

AltOpt2: At least LP-WUS periodicity is no larger than C-DRX cycle, FFS other restrictions

| Company | Y/N | Comments |
| --- | --- | --- |

In addition to the above proposal, companies provided further view, which can be discussed after the above proposal is stable.

- UE LP-WUS monitoring

  - Vivo

    - The UE starts LP-WUS monitoring and stops PDCCH monitoring based on one of the following

      - No PDCCH is detected during the new timer running and the new timer expires

      - Expiration of drx-InactivityTimer

      - Explicit indication received to deactivate PDCCH monitoring and activate LP-WUS monitoring, e.g. by PDCCH skipping command.

    - if the LP-WUS is not detected on a WUS MO by the UE, the UE is not required to wake up the MR for PDCCH monitoring

  - ZTE

    - based on RRC configuration, the UE determines whether to monitor PDCCH during legacy C-DRX active time.

      - i.e., based on RRC configuration, UE determine monitoring LP-WUS based on option 1-2-1 and option 1-2-2

      - RRM may perform measurement during legacy C-DRX active time.

  - Nokia

    - discuss the need for:

      - Short to Long DRX cycle changes.

      - A new LP_WUS specific PDCCH monitoring INACTIVITY timer

  - Samsung

    - RAN1 needs to further clarify which one or both of the following two scenarios is supported:

      - Scenario 1: Option 1-2 timer start within the drx-onDurationTimer;

      - Scenario 2: Option 1-2 timer start outside the drx-onDurationTimer.

        - For Scenario 2, RAN1 needs to further clarify whether RRM/RLM/BFD is performed during gap between the start of Option 1-2 timer and start of the drx-onDurationTimer.

  - LGE

    - Consider candidate starting points of potential DRX Active Time triggered by LP-WUS

  - E///

    - when the legacy DRX On duration timer is running, the UE should monitor LP-WUS and trigger any associated PDCCH monitoring

    - active time includes the time when the timer triggered by LP-WUS (i.e. wus-onDurationTimer) is running

  - QC

    - For option 1-2 of LP-WUS CONNECTED mode operation, discuss and clarify whether a UE is expected or not expected to monitor LP-WUS in DRX Active Time.

- LP-WUS configuration

  - CMCC

    - a time offset and periodicity can be configured similar as search space configuration

  - ZTE

    - LP-WUS monitoring may be configured with

      - An offset relative to legacy C-DRX

        - Another offset could be introduced between new active time and LP-WUS

      - An periodicity no larger than C-DRX periodicity

      - A LP-WUS monitoring window

  - OPPO

    - LP-WUS monitoring periodicity and offset can be introduced for triggering a PDCCH monitoring timer, in CONNECTED mode for both Options.

  - E///

    - wus-offset for Option 1-2: an offset indicating a time where the UE starts monitoring PDCCH, after a slot where the UE detects LP-WUS

  - QC

    - Consider MAC-CE for LP-WUS activation/deactivation

      - A UE can be configured with one or multiple LP-WUS configurations

      - The MAC-CE activates one (or none) of the LP-WUS configurations dynamically

Regarding the working assumption, a lot of companies proposed to confirm it, while it says this can be revisited considering RAN2 decisions, and hence moderator assumes this confirmation is not urgent. Let’s see whether this is accepable to all companies

- Confirm WA: HW/HiSi, SPRD, CMCC, IDC, Sony, Nokia, ETRI (Option 1-1), MTK, DCM (as separate UE capabilities)

[Closed]Proposal 3.1-3:

Confirm following working assumption

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

| Company | Y/N | Comments |
| --- | --- | --- |
| Xiaomi |  | We are not sure whether drx-onDurationTimer is started or not in Option 1-2. If it is started, then when drx-onDurationTimer is running, it is still legacy active time according to the active time definition in TS 38.321, but without PDCCH monitoring. And based on the current version of the proposal, LP WUS monitoring should be outside legacy active time, that is, LP WUS monitoring is not allowed when drx-onDurationTimer is running, which means gNB can not trigger PDCCH monitoring during the whole drx-onDurationTimer. And that is not desirable form our point of view. |
| Qualcomm |  | We think we can keep the working assumption as is – no need to rush to confirm it. |
| ZTE, Sanechips | Y | It could be confirmed. |
| CATT |  | We think the working assumption can be confirmed, and the related procedures of Option1-1 can first discussed in RAN1 and wait more progress from RAN2 for Option1-2. |
| CMCC | Y | In current discussion, - UE is allowed to monitor LP-WUS outside active time, whether UE is allowed to monitor LP-WUS during active time still need FFS. - UE behaviour on starting drx-onDurationTimer is not impacted by LP-WUS. - While drx-onDurationTimer is running, UE shall treat this period as C-DRX active time. UE behaviour on PDCCH monitoring under this period is not deteremined yet. To sum up, the current description on working assumption seems clear, and we are generally fine with this working assumption. |
| NOKIA1 | Y |  |
| Spreadtrum | Y |  |
| ETRI | Y |  |
| LGE |  | We are okay to confirm it. But the working assumption not the agreement was made for waiting RAN2 discussion with possibility of revisiting. So, It does not need to be urgent. |
| Apple |  | Agree with moderator it not urgent to confirm. |
| TCL | Y |  |
| Panasonic |  | Before confirming the working assumption, the behavior of Option 1-1 needs to be clarified/discussed first. |
| Lenovo | Y |  |
| vivo | Y |  |
| MTK | Y |  |

- Confirm WA: ZTE, CATT, CMCC, Nokia, SPRD, ETRI, LGE, TCL, Lenovo, vivo, MTK

- Keep as WA: QC, Apple, Pana,

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following was agreed in Monday online Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. |

Regarding the FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE in the above working assumption, following views are provided from companies. In general, more companies support the simultaneous configuration, while still divergent view on how to support it. As pointed out some companies, this can be achieved as Option 1-2-1 in previous agreement, i.e., Option 1-2 is add-on feature on top of Option 1-1. On the other hand, according to the company contributions, a number of companies assume that Option 1-2 is standalone feature from Option 1-1, and they can be specified/configured separately. Hence, before discussing details on how to support both Option 1-1 and Option 1-2 simultaneously, it would be beneficial to discuss whether to specify Option 1-2 as add-on or standalone feature.

- Support: SPRD, NEC, IDC, ETRI, DCM, QC

  - SPRD, ETRI: as Option 1-2-1

  - IDC: Introduce a mechanism to properly manage PDCCH monitoring based on Option 1-1 and Option 1-2

  - DCM: down select from one of the following

    - Alt1: Two options are specified with separate frameworks/configurations (e.g., separate LP-WUS MOs, separate PDCCH monitoring timers). If both frameworks are configured for a UE, the UE operate both simultaneously

    - Alt2: Two options are specified with a common framework/configuration (e.g., common LP-WUS MOs, common PDCCH monitoring timer). If the framework including both options is configured for a UE, the UE operate both simultaneously

  - QC: Discuss following two alternatives:

    - Alt.1: MAC-CE activates only one LP-WUS configuration

      - Active LP-WUS configuration is for enabling either Option 1-1 or Option 1-2

    - Alt.2: MAC-CE activates up to two LP-WUS configurations

      - One active LP-WUS configuration is for enabling Option 1-1

      - Another LP-WUS configuration can additionally activated for enabling Option 1-2

- Not support: HW/HiSi, Sharp

  - HW/HiSi: monitoring parameters of LP-WUS can be adjusted by MR signaling and/or pre-defined rules at least for different coverage requirement

- FFS: ZTE

[Old]Question 3.1-4:

- Which Alternative do you assume to specify Option 1-2, considering the support of both Option 1-1 and Option 1-2 simultaneously configured for the same UE?

  - Alt1: Option 1-2 is specified as add-on feature on top of Option 1-1 (i.e., Option 1-2-1 in previous agreement)

  - Alt2: Option 1-2 is specified as standalone feature from Option 1-1 (i.e., Option 1-2-2 in previous agreement)

| Company | Alt1 or Alt2 | Comments |
| --- | --- | --- |
| ZTE, Sanechips |  | Preferred a unified solution to support both options. We do not prefer to say option 1-2 is add-on feature which implies option 1-1 is the baseline. |
| NOKIA1 |  | Slight preference for ALT2, which offers the higher PSGs for little/no additional RAN4 effort (assuming CSI/L1-RSRP measurements procedures are largely untouched) |
| Spreadtrum |  | Similar views as ZTE |
| LGE |  | Open to discuss. If Option 1-1 and Option 1-2 are configured for a UE simultaneously, then how the UE performs needs to be considered. We think there are some issues if two options are configured simultaneously, for example, overlapping of PDCCH monitoring occasions of two options, performing CSI/L1-RSRP reporting etc. |
| Apple |  | we would like to further understand the meaning of Alt 2. If Option 1-2 is a standalone feature from Option 1-1, can Option 1-2 and Option1 -1 simultaneously configured to the same UE? If yes, we are wondering what is the difference between Alt 2 and Alt 1. |
| Lenovo |  | Prefer to Alt-2 to simplify the UE behaviour of the 2 cases. |
| vivo |  | In our view, Option 1-2 is totally a standalone method from option 1-1, which has different configuration parameters such as new timer for option 1-2 and no restriction of LP-WUS configuration for option 1-2 with CDRX configuration. |
| MTK |  | Alt2 |
| OPPO |  | Option 1-2 is a stand alone feature but can be configured in the same time with Option 1-1. |
| ETRI |  | For the clarification, if Option 1-2 is configured only to a UE, the UE cannot monitor PDCCH during legacy C-DRX active time? |
| CATT |  | Alt2. |

Alt1: ZTE, SPRD

Alt2: Nokia, Lenovo, vivo, MTK, OPPO, CATT

[Wed]Proposal 3.1-4a:

- For LP-WUS CONNECTED mode operation, Option 1-2 is specified as standalone feature from Option 1-1 (i.e., Option 1-2-2 in previous agreement)

| Company | Y/N | Comments |
| --- | --- | --- |

Regarding Option 1-3, which has not been agreed yet, a few companies propose to support it while some others propose to deprioritize it. Considering the situation, moderator cannot propose to support it now, while one potential way forward is to follow Option 1-2 approach, i.e., supporting companies try to coverge to a common understanding on Option 1-3 behavior and check whether companies can support it or not.

- Support: Xiaomi, ETRI

- Deprioritize: Nokia

[Open]Question 3.1-5:

Companies supporting Option 1-3 are encouraged to provide converged view on Option 1-3, using following template as starting point

For option 1-3 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

UE is configured with legacy C-DRX configurations as Rel-18

UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

LP-WUS triggers the start of a timer during which UE monitors PDCCH

FFS the timer is existing timer and/or new timer

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Open to discuss but it is hoped that the what benefits of option 1-3 are expected besides option1-1 and option 1-2. |
| NOKIA1 | If option 1-2-2 is adopted and the accepted interpretation of 1-2-2 is that the new OnDurations can be triggered anywhere (i.e. unrestricted by legacy cDRX), then for the following reasons: the flexibility 1-2-2 already offers the variety of legacy features to modify PDCCH monitoring during ACTIVE time (PDCCH skipping & SSSG switching Limited TUs We feel that Option 1-3 should be deprioritised. |
| LGE | The benefit of Option 1-3 is still vague comparing Option 1-2. |
| Apple | We think what needs to be discussed is that during the Active Time (including the legacy C-DRX Active Time and the Active Time triggered by LP-WUS), does the UE needs to monitor LP-WUS? The following two alternatives can be discussed: Alt 1: UE monitors LP-WUS within Active Time according to LP-WUS configuration Alt 2: UE can switch to LP-WUS monitoring within Active Time It needs to be clarified that during the time that the active timers are running but UE is not monitoring PDCCH (either through legacy features or new mechanisms), UE can monitor LP-WUS with the suitable configurations. |
| Lenovo | Revisit the issue after the stability of the Option 1-1 and Option 1-2. |
| ETRI | If majority prefer to deprioritize Option 1-3, we can live with that. |
| CATT | Deprioritize Option 1-3. |

Regarding the periodic CSI/L1-RSRP report, some companies (vivo, Xiaomi, IDC) proposed to clarify the UE behaviour and following proposal is made accordingly

[Old]Proposal 3.1-6:

- For LP-WUS CONNECTED mode operation, UE can be configured with one of the following for periodic CSI/L1-RSRP reporting:

  - Configuration 1:

    - If the UE is not indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - Configuration 2:

    - If the UE is not indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time duration indicated by drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start.

    - If the UE is indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

| Company | Y/N | Comments |
| --- | --- | --- |
| ZTE, Sanechips |  | This issue is not needed to be concluded currently. RAN2 would discuss related measurement behavior, if UE supports measurement in new active time, the above can not be applied. Additionally, according to the legacy spec, the most recent CSI measurement occasion possibly occurs outside active time. Current proposal only focus on DRX active time. We do not think it works perfectly. Therefore, we would suggest to postpone this issue until there is a clear understanding from RAN1 and RAN2. |
| NOKIA1 |  | We’d like some RAN1 discussion on this topic: For example, with configuration 1 what happens with configuration 1, if LP-WUS has not indicated wake up for a LONG while? with configuration 2, is it possible that too many measurements are reported back? |
| LGE |  | We are not sure these two configurations are necessary. Without these two configurations, two RRC parameters ps-TransmitPeriodicL1-RSRP, ps-TransmitOtherPeriodicCS can control CSI/L1-RSRP reporting when the UE is not indicated to wake-up. We can use current specification of C-DRX, why do we need new configurations? |
| TCL |  | The issues related to configuration 1 and configuration 2 may not related to RAN1 work and we are not sure why we consider these issues in RAN1. |
| vivo |  | In the last meeting, we have agreed on that “For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)”. To make this clearer, here our intention is to clarify the understanding among companies as summarized as the two configurations in the proposal 3.1-6. More discussion can be in this meeting. |
| MTK |  | It seems R2 issues. To my understanding, the spec impact will be TS 38.321. |
| Moderator |  | Companies are invited to provide views considering Monday online discussion |
| OPPO |  | On top of the Monday online, we consider the configuration of periodic CSI/L1-RSRP reporting should generally capture what we had for option 1-1. However, the red part of “Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up”, is unclear. This is due to the 1-2 may not start the drx-on timer by “wake-up”. The wake-up means previous ones and could be multiple. |

[Wed]Proposal 3.1-6a:

- For LP-WUS CONNECTED mode operation, UE can be configured with a parameter to enable/disable one of the following for periodic CSI/L1-RSRP reporting, respectively:

  - Configuration 1 If the parameter is NOT configured:

    - If the UE is not indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification.

  - Configuration 2 If the parameter is configured:

    - If the UE is not indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time duration indicated given by the configured drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start regardless if UE is indicated to wake-up or not.

    - If the UE is indicated to wake-up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time as in legacy specification.

| Company | Y/N | Comments |
| --- | --- | --- |

3.2	Timeline between LP-WUS reception/detection and PDCCH monitoring

Regarding the timeline LP-WUS reception/detection and PDCCH monitoring, following agreements were made in previous RAN1 meetings

Agreement For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - FFS whether UE can report supported minimum time gap from candidate values FFS: Whether the minimum time gap values can be more than one Agreement For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - FFS: exact value(s) of the minimum time gap - FFS: support of multiple minimum time gaps - FFS whether the reported value includes the duration for time/frequency synchronization of MR Agreement Select one of the following alternatives in RAN1#118bis Alt 1 For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. FFS: X FFS: definition of reported value Alt2 For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. FFS: X FFS: definition of reported value Different minimum time gaps correspond to different sleep states Companies are encouraged to details on how the reported values are to be used by the network

A number of companies provided their view on the FFSs in the agreements:

- Alt1 vs Alt2

  - Alt1: CATT, ZTE, OPPO, Pana, Nokia, ETRI, Samsung, LGE, MTK, DCM, E///, QC, TCL

  - Alt2: HW/HiSi, vivo, CMCC, Xiaomi, Sharp, IDC, Lenovo, Sony, Apple

    - larger reported value allows the UE to have larger UE power saving gain

    - Lenovo: UE shall go to the corresponding sleep state (e.g., deep sleep or light sleep) based on the configured time gap and/or synchronization performance

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

  - Yes: Lenovo, Pana (and optionally the time to change the carrier), LGE

  - No: HW/HiSi, vivo, CMCC

    - i.e., minimum time gap Xmin between LP-WUS reception and MR to start PDCCH monitoring is T1 + T2

      - T1 is the reported UE capability which includes LP-WUS processing time and MR transition time for ramp up

      - T2 includes the duration for time/frequency synchronization of MR

- Reference SCS

  - LP-WUS: vivo

Regarding the exact value(s) of the minimum time gap and reference SCS, these can be discussed after LP-WUS design becomes clear.

Regarding Alt1 vs Alt2, as mentioned above, this intends to discuss whether a UE reports one or multiple minimum time gaps. According to companies input, the main motivation seems that a UE reports different minimum time gaps for different sleep states, as captured in TR 38.869 below.

The MR sleep states considered for LP-WUS/WUR evaluation in RRC_CONNECTED are the same as for baseline: deep sleep state with a 20 ms transition time, light sleep state with a 6 ms transition time, or micro sleep without any transition time, as described TR 38.840. Ultra-deep sleep state is not considered for LP-WUS/WUR in RRC_CONNECTED state as a 400 ms transition time is too long to allow the MR to be ready for PDCCH monitoring from the ultra-deep sleep state considering the traffic requirements for NR.

Companies have divergent views. Some companies prefer Alt2 due to the above reason and different capability can be used to different traffic types (e.g., delay sensitive vs non-delay sensitive). On the other hand, some other companies prefer Alt1 similar to the agreement in idle/inactive mode. Also, gNB does not know how much power saving gain can be achieved even if longer time gap corresponding to deep sleep state is configured unless there is no clear relation between UE capability and sleep state. To move forward, it would be beneficial to discuss how to define “Different minimum time gaps correspond to different sleep states” in the spec, and see how the reported values are to be used by the network appropriately.

[Old]Question 3.2-1:

For RRC CONNECTED mode, if it is supported that UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting, how the following is defined in the spec to be used by network appropriately?

Different minimum time gaps correspond to different sleep states

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | - Report multiple values is meaningless to the NW, the NW only configure the offset based on the current NW load situation, UE’s power saving gain and some other potential factors. It is not related to how many values are reported, and the NW only refer to the minimum one since the NW knows larger offset always bring more UE power saving gain. - Different values based on different sleep states is meaningless. Different UE have different time to go to sleep, different UE has different implementation, we only need to define a minimum one based on one implementation to ensure the time needed such that the UE can normally wake-up and monitor the followed PDCCH. If UE report multiple values corresponding to multiple sleep states, the NW only refer to the minimum one to configure the offset based on the expected PSG, NW load situation and so on. - For different WUR type, the ramp up time is different, the processing time also is different, we can not assume the same minimum time gap for different WUR type. |
| NOKIA1 | Given the majority of the companies lean to Alt1 (13 versus 10?), we find this a strange question, attempting to force Alt2. As has been raised by several companies before, even if the network had multiple values, how would the know which value to choose when? If it had extra values, how would it know which value the UE preferred at any given instance? What would be the power saving gain of one value over another? Are proponents proposing extra signalling or rigid/restrictive implementation assumptions? |
| LGE | Similar view with ZTE. |
| Apple | Our understanding is that one value can be used outside Active Time and the other value can be used within Active Time. If the UE is switched to LP-WUS monitoring during Active Time, e.g. drx-InactivitityTimer is running, the UE is likely to enter a lighter sleep state than outside the Active Time, especially under Option 1-1. |
| TCL | In our view, the time gap between LP-WUS and PDCCH monitoring depends on the UE capability and not on the different sleep states. The candidate values for time gap shall consider the ramp up time between the LP-WUS and PDCCH, not the different sleeping states, as mentioned by ZTE. |
| Lenovo | Fine with the proposal, UE may need to report one or more suitable time gaps for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring (e.g., one for deep sleep state with a 20ms transition time, another for light sleep state with a 6ms transition time), if UE only report one time gaps, default as for deep sleep or light sleep. |
| vivo | We support Alt.2. Alt.2, which allows a UE to report multiple gap values for MR waking up, offers more network flexibility in balancing latency and power saving according to the traffic needs, especially when there are significant differences in power-saving gains among these multiple values. Depending on the UE implementation, the UE can report one or multiple values, and if multiple values are reported, network can control the UE MR sleep behaviour by configuring the proper time gap considering tradeoff between traffic need and UE power saving. For example, if two values are reported by the UE, the one with smaller gap value (corresponding to MR micro or ligh sleep) can be used by the network when UE is operating with relatively latency sensitive traffics (e.g. XR), the one with lager gap value (corresponding to MR deep sleep) can be used by the network when UE is operating with non-latency sensitive traffics (e.g. FTP download). |
| MTK | Alt 1. The reporting value is used for gNB to know whether UE will monitor LPWUS. gNB expects not monitoring LP-WUS when a scheduling offset is smaller than the reported value. For this purpose, single reporting value is enough. |
| OPPO | UE only need to report one value. No sleep state assumption should be associated. |
| CATT | Support Alt1. Similar views with ZTE, different UE has different implementation and it is meanless for different values based on different sleep states. |

[Old]Proposal 3.2-1b:

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

UE can indicate preferred time gap in UAI, which can be equal to or longer than the reported minimum gap by capability

FFS: whether/what additional information should be indicated to the UE to determine the minimum time gap

Strong concern

Alt1: vivo, Xiaomi, TCL

Alt2: ZTE, Nokia, LGE, MTK, E///, CATT

WF (Alt1+UAI): Nordic, HW, SS

[Wed]Proposal 3.2-1c:

Select one of the following alternatives in RAN1#118bis

Alt 1

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Alt2

For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Different minimum time gaps correspond to different sleep states

Alt3

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

UE can indicate additional UAI to assist NW to decide/adapt the time gap, FFS details

A single time offset is configured for a UE at a given time

FFS: whether/what additional information should be indicated to the UE to determine the minimum time gap

| Company | Y/N | Comments |
| --- | --- | --- |

Regarding whether the reported value includes the duration for time/frequency synchronization of MR, companies have divergent views. One of the motivations not to include the duration for time/frequency synchronization of MR would be that, time/frequency synchronization of MR is not related to the UE capability but the duration where sufficient number of sync sources, such as SSB and TRS, can be received after MR wake up. In this case, minimum time gap Xmin between LP-WUS reception and MR to start PDCCH monitoring can be expressed as T1 + T2, where

- T1 is the reported UE capability which includes LP-WUS processing time and MR transition time for ramp up

- T2 is the duration for time/frequency synchronization of MR

Another motivation is that T2 can be 0 since MR does not go to ultra-deep sleep state and hence MR can receive SSB for sync and RRM.

[Old]Proposal 3.2-2:

For RRC CONNECTED mode, the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is expressed as T1 + T2 [symbols/slots], where

T1 is given by the reported UE capability which includes LP-WUS processing time and MR transition time for ramp up

T2 is the duration for time/frequency synchronization of MR (FFS: T2=0)

| Company | Y/N | Comments |
| --- | --- | --- |
| ZTE, Sanechips |  | T2 is not needed. The reason includes - The assumption that LP-SS is not needed in connected mode is that sync performance can be maintained. Otherwise, we’d better consider to introduce LP-SS in connected mode. - Sync time is random which depends on the MR, and available RS. It is hard to say in several miliseconds, sync can be completed if there is a sync performance issue. |
| NOKIA1 | Y | Yes to the intention that the minimum time gap includes the t/f sync time. Note, we’d like RAN1 to discuss if the network supplies additional information to the UE before the UE reports this capability, so that the UE can make the best estimate given it’s implementation and potentially key network configuration features (eg cDRX cycle for option 1-1, shortest period between ACTIVE durations for option 1-2, maximum time between sync signals, etc). Suggestion: FFS What additional information should be indicated to the UE to determine the minimum time gap. |
| Spreadtrum | Y | In our view, the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is configured by network. And the value of the minimum gap should not less than the value reported by UE. |
| LGE |  | We agree that the duration of T/F sync for MR can need to be considered. However, T2 can be dependent on UE’s sleep state, and the network cannot know the UE’s sleep state. So, we think considering T2 is complicate. |
| TCL |  | We are fine with the explanation of T1. However, T2 is not required for UE in connected state. |
| Panasonic | Y |  |
| Lenovo |  | T2 is not needed. |
| MTK | Y |  |
| OPPO |  | We think the total value should be reported by capability. |
| CATT |  | T2 is included in the minimum time gap. |

[Wed]Proposal 3.2-2a:

- For RRC CONNECTED mode, the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is expressed as T1 + T2 [symbols/slots], where

T1 is given by the reported UE capability which includes LP-WUS processing time and MR transition time for ramp up

T2 is the duration for time/frequency synchronization of MR (FFS: T2=0)

FFS what additional information should be indicated to the UE to determine the minimum time gap

| Company | Y/N | Comments |
| --- | --- | --- |

3.3 LP-WUS monitoring occasions

For LP-WUS monitoring in connected mode, following agreements were made in previous RAN1 meetings:

Agreement LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode. - FFS whether to define a time window for MOs - It is at least supported that a UE can monitor MOs with a periodicity. - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured

- FFS whether to define a time window for MOs

  - Yes: E///, Xiaomi (including repetition), Lenovo, Apple, TCL

  - No: CATT, Nokia

- Frequency location of LP-WUS

  - QC

    - As baseline, LP-WUS on a serving cell is configured within active DL BWP of the serving cell

      - FFS: outside active DL BWP of the serving cell (as optional UE capability)

  - ZTE

    - The LP-WUS is applied for active DL BWP for PDCCH monitoring

      - Target all the configured searchspaces in the active DL BWP

      - LP-WUS could be configured outside the active BWP

Regarding whether to define a time window for MOs, most companies are OK to define it so as to include multiple LP-WUS MOs before existing/new timers.

[Old]Proposal 3.3-1:

Define LP-WUS monitoring window which includes one or multiple LP-WUS MOs with a periodicity and offset in RRC CONNECTED mode.

FFS periodicity and offset

FFS number of MOs in the window

| Company | Y/N | Comments |
| --- | --- | --- |
| ZTE, Sanechips | Y | We are OK to consider a window to provide the scheduling flexibility for the NW. |
| NOKIA1 |  | We are wary of the window concept, given that the LP-WUS is not a spectrally efficient signal. However we are open to discuss this proposal, assuming it allows the option to define 1 MO and does not assume continuous monitoring. Questions we have for proponents, (1) when does ACTIVE time actually start? Is it relative to the detected MO or relative to the end of the window? (2) within a window, are the LP-WUS transmissions effectively repetitions or can information content change? |
| LGE | Y | Fine with the proposal. Regarding Nokia’s concern, we think reminding Rel-16 could be helpful. The start of DRX Active time (drx-onDurationTimer) is fixed and there might be some duration a UE does not need to monitor LP-WUS and prepares C-DRX operation. And, regarding LP-WUS transmission, it can depend on how LP-WUS payload is constructed. |
| Apple |  | At least for Option 1-1, we think a window would be needed. |
| TCL | Y | Support the proposal. |
| Panasonic | Y | The concept of LP-WUS monitoring window could be useful to consider whether LP-WUS works with existing UE power saving features, e.g., C-DRX, if the periodicity and time window can be flexibly configured. |
| Lenovo |  | Is the proposal targeted for both Option 1-1 and Option 1-2? |
| MTK | Y |  |
| OPPO |  | We could consider any configuration that can support multiple MO in certain period of time. Does it have to be a time window? |
| CATT | N | Unlike DCP, the periodicity of LP-WUS can be configured deterministically. UE can monitor the LP-WUS MOs based the configured periodicity. Thus, it is not needed to define a time window for LP-WUS MOs to minimize the UE power consumption of LP-WUS detection |

[Wed]Proposal 3.3-1:

- Define LP-WUS monitoring window which includes one or multiple LP-WUS MOs with a periodicity and offset in RRC CONNECTED mode.

FFS periodicity and offset

FFS number of MOs in the window

| Company | Y/N | Comments |
| --- | --- | --- |

Regarding the QCL/TCI state for LP-WUS in CONNECTED mode, following agreements were made in previous RAN1 meetings:

Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS FFS applicable QCL type(s)

Following views are provide in the contributions:

- QCL source

  - SSB include both CD-SSB and NDC-SSB: ZTE

- FFS applicable QCL type(s)

  - Type A: CMCC, Xiaomi (LP-SS), Pana (LP-SS/SSB)

  - Type B: Pana (TRS)

  - Type C: Xiaomi (SSB), ZTE

  - Type D: vivo (FR2), Pana (LP-SS/SSB), Nokia

  - QCL Type(s) defined for CORESET/PDCCH: QC

- How to define/configure/indicate TCI-state of LP-WUS

  - can be discussed after the progress achieved on LP-WUS monitoring activation/deactivation: HW/HiSi

  - Reuse TCI-state framework w/ or w/o unified framework: vivo

  - Reuse TCI-state framework of CORESET: Nokia

  - Reuse TCI-state framework + default TCI state: LGE

  - Same TCI state as the CORESET used for PDCCH monitoring triggered by LP-WUS: CATT, OPPO, E///

  - Latest TCI state of CORESET + default TCI state: ETRI

  - As part of the configuration for LP-WUS: Samsung, QC

  - Multiple LP-WUS monitoring searchspace configuration to have different TCI-state configuration: ZTE

Regarding how to define/configure/indicate TCI-state of LP-WUS, companies view can be categorized as

- Reuse TCI-state framework to indicate TCI-state of LP-WUS

- TCI-state association between LP-WUS and CORESET

- Default TCI-state

[Old]Proposal 3.3-2:

For LP-WUS monitoring in RRC CONNECTED mode, consider following options on how to define/configure/indicate TCI-state of LP-WUS for down selection

Opt1: Reuse TCI-state framework to indicate TCI-state of LP-WUS

Opt2: TCI-state association between LP-WUS and CORESET

Opt3: Default TCI-state

Opt4: As part of the configuration for LP-WUS

| Company | Y/N | Comments |
| --- | --- | --- |
| ZTE, Sanechips |  | Option4 provide flexibility but multiple TCI-states should be considered. Ne TCI state is not enough. Not sure the difference between option1 and option4. Option3 is not flexible and can not adapt to different beam direction or situation. Option2 is confusing since there are several CORESETs in the BWP and which CORESET is associated is not clear. Therefore, hope option1 is further clarified. Option4 should support multiple TCI-states. |
| NOKIA1 |  | Similar view to ZTE, what is the difference between options 1 and 4? also, several companies raised the potential need for a default/fallback TCI state … should opt3 be treated separately? Is it really a down-selection or a stand-alone issue? |
| LGE |  | We would like to support combination of those options thus we cannot support down selection. As Nokia commented, default state does not need to be separate option. |
| TCL |  | We support option 1 and option 3. |
| OPPO |  | We also think those may be exclusive options to each other. We need detail scheme for applying TCI states. |
| CATT |  | Since the LP-WUS is used to trigger PDCCH monitoring and the TCI-state is configured per CORESET for PDCCH, the LP-WUS can be QCLed with the RS (reference signal) associated with the TCI state of the CORESET. One LP-WUS would be configured to be QCLed with the RS associated with one of the TCI states of the CORESETs when the UE is configured with more than one CORESET for PDCCH monitoring. |

[Wed]Proposal 3.3-2a:

- For LP-WUS monitoring in RRC CONNECTED mode, consider following options on how to define/configure/indicate TCI-state of LP-WUS for down selection

Opt1: Reuse TCI-state framework to indicate TCI-state of LP-WUS

Opt2: TCI-state association between LP-WUS and CORESET

Opt3: Default TCI-state

Opt4: As part of the configuration for LP-WUS

| Company | Y/N | Comments |
| --- | --- | --- |

4	Activation/deactivation of LP-WUS monitoring

This issue has been discussed in previous RAN1 meetings and following agreements were made:

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Many companies provided their view on these options

- Option 1: CMCC, CATT, Sony, Nokia, E///

- Option 2: HW/HiSi, SRRD, vivo, ZTE, Apple, Samsung, LGE

- Option 3: HW/HiSi, ZTE, OPPO, IDC, Lenovo, Apple, Samsung, LGE, TCL

  - Supported by Option 1-2

  - Autonomous fallback

  - Level of UE activity

- Option 4: HW/HiSi, vivo, LGE, TCL

  - UL transmissions such as SR, PRACH and CG PUSCH

Also, some companies provide views on the FFS in the last agreement:

- FFS whether to support UE assistance.

  - Yes: IDC, Nokia, OPPO (MR/LP-SS measurement report)

  - No: CATT

- FFS whether implicit/explicit indication from UE is necessary

  - Explicit UE feedback to indicate loss of LP-WUS: Nokia

  - L1/L2 dynamic report of UE status: QC (Option 1-2)

  - implicit indication (HARQ-ACK, scheduled UL): ETRI

  - Not support: vivo

Some companies also propose autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

- Autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

  - When UE does not receive LP-WUS for a long time: ZTE

  - When the channel/beam quality is not satisfactory for successful LP-WUS reception: QC (in addition to recovery from RLF/BFR during LP-WUS monitoring), Lenovo, ETRI

  - Without specified condition: vivo

    - For LP-WUS operation option 1-1, the fallback PDCCH monitoring behavior is according to the C-DRX configuration

    - For LP-WUS operation option 1-2, the fallback PDCCH monitoring behavior is always-on PDCCH monitoring

Beside, companies provide some fundamental assumptions on LP-WUS/PDCCH monitoring

- HW/HiSi

  - It is assumed in the design of LP-WUS in CONNECTED mode that MR and LP-WUR cannot receive DL signal simultaneously.

  - PDCCH monitoring of a UE is suspended if LP-WUS monitoring of the UE is activated, and vice versa. PDCCH monitoring of a UE is resumed if LP-WUS monitoring of the UE is deactivated. When a UE is in the state of deactivated LP-WUS monitoring, the UE may or may not monitor LP-WUS.

  - UE switches from PDCCH monitoring to LP-WUS monitoring after the timer triggered by LP-WUS expires with proper switching time, if legacy DRX related timers is not running.

- LGE

  - Consider the case where LP-WUS monitoring is in active state even during PDCCH monitoring is activated.

- DCM

  - For RRC CONNECTED mode, when UE is configured with LP-WUS by gNB RRC signaling, UE is expected to monitor LP-WUS according to the LP-WUS monitoring configuration

  - For RRC CONNECTED mode, when UE detects a LP-WUS indicating the UE to wake-up, UE monitors PDCCH, and UE is not required to monitor LP-WUS during the PDCCH monitoring

  - From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, PDCCH monitoring is resumed when LP-WUS monitoring is deactivated by additional indication/condition (if supported)

- E///

  - OOK-based LP-WUS in connected mode should only be used for triggering PDCCH monitoring for a limited set of PDCCH CCE aggregation levels. For example,

    - PDCCH aggregation levels 1,2,4 for PDCCH 1Rx reception

    - PDCCH aggregation levels 1,2 for PDCCH 2Rx reception

    - PDCCH aggregation level 1 for PDCCH 4Rx reception

Regarding additional indication/condition for activation/deactivation of LP-WUS monitoring, companies have different preference, and need further discussion is necessary. As supported by a number of companies, in Option 3, which has the most support, autonomous fallback can be considered as starting point for further discussion

[Old]Proposal 4-1:

When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

FFS the condition for the fallback

For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

For LP-WUS operation option 1-2, FFS whether UE fallbacks to legacy C-DRX behavior or PDCCH monitoring without legacy C-DRX

| Company | Option(s) | Comments |
| --- | --- | --- |
| Qualcomm | Y | We think there should be no specific condition for the fallback. For Option 1-2, if a UE has to fallback PDCCH monitoring without legacy C-DRX, the UE has to be reachable anytime, the power consumption is even higher than legacy C-DRX operation. We are OK with the FFS, but we do not think it is good solution. |
| ZTE, Sanechips |  | Support the FL proposal. |
| NOKIA1 | Y |  |
| LGE | Y |  |
| Apple |  | It is preferred that the fallback behavior is up to UE implementation and for option 1-2, fall back to legacy C-DRX is preferred. |
| TCL | Y |  |
| Panasonic | Y |  |
| OPPO | Y | We agree the proposal. There should be some timer mechanism at least. |
| ETRI | Y |  |
| CATT | N | The UE can monitor PDCCH and LP-WUS simultaneously. |

[Wed]Proposal 4-1:

- When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

FFS the condition for the fallback

For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

For LP-WUS operation option 1-2, FFS whether UE fallbacks to legacy C-DRX behavior or PDCCH monitoring without legacy C-DRX

| Company | Y/N | Comments |
| --- | --- | --- |

[Old]Proposal 4-2:

It is assumed in the design of LP-WUS in CONNECTED mode that MR and LP-WUR cannot receive DL signal simultaneously.

| Company | Option(s) | Comments |
| --- | --- | --- |
| Moderator |  | Additional proposal based on the offline discussion with some companies. It would be good if this fundamental assumption can be clarified for better understanding |
| CMCC | Y, and | From our understanding this is a strong implementation-related issue, and in order to ensure LP-WUS can be applicable for UEs in different capabilities, it is recommended that the gNB can guarantee MR and LP-WUR not perform simultaneously. |
| ZTE, Sanechips | Y | It should be nature. If the UE has stronger capability of receiving both DL signal simultaneously, it could be optional considered. As the baseline, this proposal makes sense. |
| Qualcomm |  | We prefer to keep a door open for simultaneous DL receptions using LP-WUR and MR as an optional case. Therefore, we suggest following: For RRC connected mode, support at least the following case: LP-WUS is not expected to be monitored during the time where MR is active. |
| OPPO | Y | Those could be assumption when we design the procedure of LP-WUS. |
| NOKIA2 |  | We would like to make this a subject to UE Implementation/Capability. We can envisage LP-WUS entry/exit scenarios, specifically those covered by Option 4 (see below), where the overall need/cost of handling temporary transitions are reduced by supporting simultaneous operation. Reminder of option 4: “Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.” |
| LGE | N | We understand Moderator’s intention, but simultaneous DL reception needs to be considered. We think it can be valuable when Option 1-1 and Option 1-2 are configured for a UE simultaneously. So, we prefer to keep it open for now. |
| CATT | N | The UE can monitor PDCCH and LP-WUS simultaneously. |

[Wed]Proposal 4-2a:

- For RRC connected mode, support at least the following case:

LP-WUS is not expected to be monitored during the time where MR is active.

FFS the case where LP-WUS is expected to be monitored during the time where MR is active.

| Company | Y/N | Comments |
| --- | --- | --- |

In addition, following views are provided related to activation/deactivation of LP-WUS monitoring, which can be discussed after high-level design of activation/deactivation of LP-WUS monitoring is decided.

- HW/HiSi

  - The impact due to miss-detection/falsely-detection of LP-WUS activation/deactivation signaling should be minimized/avoided for CONNECTED mode UE by e.g. explicit feedback signaling from UE to gNB

- IDC

  - Support a mechanism to report UE decision of LP-WUS monitoring deactivation potentially with reason for the decision.

- Samsung

  - For activation/deactivation of LP-WUS monitoring, prioritize Option 2 and Option 3, and support explicit confirmation message(s) from the UE to the gNB for handshake in Option 2.

5	LP-WUS payload

This issue has been discussed in previous RAN1 meetings, and following agreements were made:

Agreement For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits - FFS value X, which is no more than [8 or 16] Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following: - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI - Option 3: A codepoint value corresponding to [one or more] UEs - Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s) - Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s) - Combination of above options are not precluded. - FFS how to carry LP-WUS information, e.g, by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - FFS details of LP-WUS information to trigger PDCCH monitoring (e.g. whether above is applicable to one or more serving cells) Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs FFS details for extension of option 1 and/or 3 when UE is configured with CA

Many companies provided their view on the remaining issues

- Value X (to be further discussed in AI 9.6.1)

  - 8: Sony

  - 16: HW/HiSi, vivo, Lenovo

  - LP-WUS duration used for connected mode should be limited to at most 2 slots: E///

- Bitmap vs codepoint (to be discussed in AI 9.6.1)

  - Option 1 (bitmap): vivo, ZTE, Sharp

  - Option 3 (codepoint): OPPO, Sony, DCM

- Other contents

  - Scell dormancy

    - Yes: CATT, IDC

    - No: vivo, OPPO, Sharp

  - SSSG switching: Xiaomi

  - BWP switching: Xiaomi

Regarding the value X, since the above agreement was made in RAN1#116 meeting as a great compromise among companies, moderator assumes we don’t need to further down select the value in this agenda, but it can be further discussed in AI 9.6.1 considering performance, system overhead, etc.

Regarding Bitmap vs codepoint, this is highly related to LP-WUS structure and hence, those can be further discussed in AI 9.6.1.

Regarding the other contents than wake-up indication (PDCCH triggering), there is no clear majority view and they can be discussed after fundamental part of LP-WUS monitoring made progress, if necessary.

Regarding whether wake-up indication is applicable to one or more serving cells, following agreements were made in previous RAN1 meetings

Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable Option 1: one or more serving cells based on gNB indication/configuration Option 2: all activated serving cells Note: other options are not precluded Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC

Many companies provided their view on the remaining issues

- LP-WUS is supported for the case where UE is configured with DC in RRC CONNECTED mode

  - Yes: HW/HiSi, vivo, ZTE, Pana (NR-DC), DCM, QC

    - HW/HiSi: LP-WUS and the PDCCH monitoring that the LP-WUS can trigger are expected to be in the same cell group

  - Deprioritize: IDC

- FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

  - Option 1: SPRD, CMCC, ZTE, OPPO (same carrier), Lenovo, LGE, Sharp, MTK

  - Option 2: QC, vivo, CATT, Pana, Nokia, IDC, ETRI, Apple, DCM, E///

- The cell(s) where LP-WUS is monitored

  - CA

    - on a serving cell: vivo, Pana, TCL

    - on Pcell: ZTE, Sharp, E///

  - DC

    - on a serving cell per cell-group: vivo (subject to UE capability), QC, ZTE

    - on PCell only and indicate whether to wake-up PSCell in addition to Pcell: ZTE

    - onPCell or SCell in MCG to indicate PDCCH monitoring in both MCG and SCG: Pana

- Dual DRX

  - Samsung: FFS

  - QC

    - For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group

      - Opt.1: LP-WUS is configured to be monitored per DRX group

        - For dual DRX groups, LP-WUS monitoring is configured per DRX group

        - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group

      - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

  - CMCC

    - For the case that a MAC entity has two DRX cell groups, LP-WUS should be separately configured for each DRX cell group.

  - DCM

    - LP-WUS indication is applicable to all activated serving cells in the same DRX group(s), if configured

    - For dual DRX groups, further consider following options

      - Option 1: LP-WUS monitoring/indication is applied per DRX group

      - Option 2: LP-WUS monitoring on one of DRX group, LP-WUS indication to either or both DRX group(s)

- Special handling

  - HW/HiSi

    - For LP-WUS in FR1-FR2 CA, the trigger of PDCCH monitoring of FR2 cells can be separate from that of FR1 cells.

      - If PDCCH monitoring of an SCell is triggered, then PDCCH monitoring of PCell is also triggered, regardless their frequency range.

In addition to CA, majority companies assume LP-WUS is supported for the case where UE is configured with DC in RRC CONNECTED mode, while companies have divergent views on which cell(s) PDCCH monitoring triggered by a LP-WUS is applicable and also on which cell(s) LP-WUS is monitored. Following can be considered as the starting point

[Old]Proposal 5-1:

- LP-WUS works when UE is configured with DC in RRC CONNECTED mode

FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

FFS: The cell(s) where LP-WUS is monitored

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | LP-WUS in a cell of a cell-group triggers PDCCH monitoring of a cell of the other cell-group does not work without inter-gNB coordination. This way requires RAN3 work, which we believe is beyond the scope. Independent LP-WUS operation per cell-group is a traditional approach of RAN1 work (except for some particular topics, such as NR-DC power control across cells in different cell-groups, peak data rate split to cell groups in NR-DC, etc). We don’t quite understand what is the controversial point here. |
| ZTE, Sanechips |  | It is not expected that the LP-WUS can wake up another cell group. |
| LGE |  | If LP-WUS triggers PDCCH monitoring of cells in a cell group, it might not be an issue. Open to discuss. |
| Panasonic | Y | For FFS bullets, we think LP-WUS can be monitored in either PCell or SCell in PCG to indicate PDCCH monitoring in both PCG and SCG. |
| OPPO |  | We also hesitate to have cross group wake-up. |
| CATT | Y | OK with the proposal. |

[Wed]Proposal 5-1:

- LP-WUS works when UE is configured with DC in RRC CONNECTED mode

FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

FFS: The cell(s) where LP-WUS is monitored

| Company | Y/N | Comments |
| --- | --- | --- |

In addition, following views are provided related to how to inform LP-WUS information in addition to the LP-WUS payload, which can be discussed after high-level design of LP-WUS payload is decided in AI 9.6.1.

- IDC

  - A portion of LP-WUS information can be informed to UE implicitly using LP-WUS monitoring resource association (e.g., LO and MO) and structure of LP-WUS.

- LGE

  - To overcome limited maximum number of LP-WUS information bits, consider the combination of time/frequency resources and LP-WUS payload for UE identification.

6	Coexistence with existing UE power saving features

In addition to C-DRX discussed in Section 3, many companies provided their view on the coexistence with existing UE power saving features, roughly categorized as follows:

Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. Rel-16 DCP Rel-17 PDCCH skipping Rel-17 SSSG switching Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

- PDCCH skipping

  - HW/HiSi: After LP-WUS monitoring is activated, the UE should stop/suspend the Rel-17 PDCCH adaption mechanism, if configured

  - CMCC: LP-WUS procedure Option 1-1/Option 1-2 and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously

  - Vivo: if PDCCH skipping indication is used as LP-WUS activation signalling, enhanced PDCCH skipping indication can be used to indicate UE stop PDCCH monitoring on one or mulitiple serving cells and starts LP-WUS monitoring

  - OPPO: When PDCCH Skipping is configured, the PDCCH skipping indication can enable the LP-WUS monitoring occasions

  - QC: confirm legacy PDCCH skipping or SSSG switching can be configured simultaneously for further power saving.

  - LGE: Co-existence of Rel-17 PDCCH monitoring adaptation and LP-WUS can be revisited after the LP-WUS procedure about PDCCH monitoring are discussed

- SSSG switching

  - HW/HiSi: After LP-WUS monitoring is activated, the UE should stop/suspend the Rel-17 PDCCH adaption mechanism, if configured

  - Vivo: For a UE configured with both LP-WUS monitoring and Rel-17 SSSG switching, Rel-17 SSSG switching is applied during the time period when MR performs PDCCH monitoring. There should be no impact due to LP-WUS monitoring.

  - CMCC: LP-WUS procedure Option 1-1/Option 1-2 and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously

  - QC: confirm legacy PDCCH skipping or SSSG switching can be configured simultaneously for further power saving.

  - LGE: Co-existence of Rel-17 PDCCH monitoring adaptation and LP-WUS can be revisited after the LP-WUS procedure about PDCCH monitoring are discussed

- Cell DTX

  - HW/HiSi: LP-WUS monitoring outside legacy cell DTX active time according to the LP-WUS monitoring configuration can trigger the PDCCH monitoring

  - OPPO: LP-WUS is assumed to be disabled during the DTX time

  - ZTE: During Cell-DTX off period, UE does not need to monitor LP-WUS

  - LGE: Further discuss how LP-WUS (and LP-SS) can be handled or interacted with cell DTX configurations

  - TCL: LP-WUS can be configured together with Cell DTX

Most all companies assume LP-WUS works with {PDCCH skipping, SSSG switching, cell DTX}. How LP-WUS works with these features can be discussed further.

[Old]Proposal 6-1:

- For RRC CONNECTED mode, LP-WUS can be configured with PDCCH skipping

PDCCH skipping can indicate to stop PDCCH monitoring and start LP-WUS monitoring

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| ZTE, Sanechips |  | The LP-WUS is monitored outside legacy active time or new active time based on option 1-1 and option 1-2. I’am not sure why we need this optimization. Additionally, what is the relationship between this optimization and option 1-3? |
| NOKIA1 |  | Defer until decision on 1-3 made. |
| LGE |  | The sub-bullet is correlated to Option 1-3, so discussion of Option 1-3 should be the first. |
| Apple |  | Legacy PDCCH skipping does not start LP-WUS monitoring, then if the subbullet is agreed, it is a different feature from legacy PDCCH skipping. In that sense, we prefer to keep it a separate feature from legacy PDCCH skipping. |
| TCL | Y |  |
| Panasonic |  | The discussion should wait for this progress of LP-WUS monitoring procedures triggering PDCCH monitoring. |
| OPPO | Y | We are fine for the proposal. |
| ETRI | Y |  |

[Wed]Proposal 6-1a:

- For RRC CONNECTED mode, LP-WUS can be configured with PDCCH skipping

FFS: PDCCH skipping can indicate to stop PDCCH monitoring and start LP-WUS monitoring

| Company | Y/N | Comments |
| --- | --- | --- |

[Old]Proposal 6-2:

- For RRC CONNECTED mode, LP-WUS can be configured with SSSG switching

No interaction between SSSG swiching and LP-WUS monitoring is assumed

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| ZTE, Sanechips | Y | The LP-WUS is monitored outside legacy active time or new active time based on option 1-1 and option 1-2. SSSG switching is performed in active time. |
| NOKIA1 | Y |  |
| LGE | Y |  |
| Apple | OK |  |
| TCL | Y |  |
| Panasonic |  | The discussion should wait for this progress of LP-WUS monitoring procedures triggering PDCCH monitoring. |
| OPPO | Y |  |

[Wed]Proposal 6-2:

- For RRC CONNECTED mode, LP-WUS can be configured with SSSG switching

No interaction between SSSG swiching and LP-WUS monitoring is assumed

| Company | Y/N | Comments |
| --- | --- | --- |

[Old]Proposal 6-3:

- For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX

During Cell DTX inactive time, the UE is not expected to monitor LP-WUS

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| ZTE, Sanechips | Y | Support FL proposal since during cell DTX inactive time, we should avoid new scheduling to cause the NW power consumption. And the LP-WUS is mainly for the new scheduling. |
| NOKIA1 | Y |  |
| Apple |  | Support in principle. One point we would like to note is that for Option 1-1, it might be possible that UE C-DRX cycle is not fully overlapped with cell DTX (e.g. the start of C-DRX is ahead of start of cell DTX) and the LP-WUS may be always within cell DTX non-active period. We suggest add one FFS: FFS: whether/how to handle the case when LP-WUS monitoring occasions are all configured within cell DTX non-active period |
| TCL | Y |  |
| Panasonic |  | The discussion should wait for this progress of LP-WUS monitoring procedures triggering PDCCH monitoring. |
| OPPO | Y |  |

[Wed]Proposal 6-3a:

- For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX

During Cell DTX inactive time, the UE is not expected to monitor LP-WUS

FFS: whether/how to handle the case when LP-WUS MOs are all configured within cell DTX inactive time

| Company | Y/N | Comments |
| --- | --- | --- |

7	Conclusions

Following agreements were made in this meeting

Agreement

- For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

UE is configured with legacy C-DRX configurations as Rel-18

UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

    - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

      - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

Agreement

Update following agreement in red.

Agreement

- For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

UE is configured with legacy C-DRX configurations as Rel-18

UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

    - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

LP-WUS triggers the start of a timer during which UE monitors PDCCH

    - FFS the timer is existing timer and/or new timer

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

    - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

    - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

      - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

For the second FFS in the above agreement, the final decision is left to RAN2.

Agreement

Confirm following working assumption

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

To be updated

References

| [1] | RP-241824 | Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR) | vivo, NTT DOCOMO |
| --- | --- | --- | --- |
| [2] | R1-2400640 | Work plan for Low-power wake-up signal and receiver for NR | vivo, NTT DOCOMO |
| [3] | R1-2407517 | FL summary #4 on LP-WUS operation in CONNECTED mode | Moderator (NTT DOCOMO) |
| [4] | R1-2407666 | Procedures and functionalities of LP-WUS in CONNECTED mode | Huawei, HiSilicon |
| [5] | R1-2407716 | Discussion on LP-WUS operation in CONNECTED modes | Spreadtrum Communications |
| [6] | R1-2407871 | Discussion on LP-WUS operation in CONNECTED modes | vivo |
| [7] | R1-2407915 | Discussion on LP-WUS operation in CONNECTED mode | CMCC |
| [8] | R1-2407979 | Discussion on LP-WUS operation in Connected mode | Xiaomi |
| [9] | R1-2408057 | System design and procedure of LP-WUS operation for UE in CONNECTED Modes | CATT |
| [10] | R1-2408076 | Discussion on LP-WUS operation in CONNECTED mode | ZTE Corporation, Sanechips |
| [11] | R1-2408131 | Further consideration on LP-WUS operation in connected mode | OPPO |
| [12] | R1-2408219 | Discussion on LP-WUS operation in RRC CONNECTED mode | NEC |
| [13] | R1-2408255 | Discussion on LP-WUS operation in CONNECTED modes | Sharp |
| [14] | R1-2408282 | Discussion on RRC CONNECTED mode LP-WUS monitoring | InterDigital, Inc. |
| [15] | R1-2408322 | Discussion on LP-WUS operation in CONNECTED modes | Lenovo |
| [16] | R1-2408418 | LP-WUS operation in CONNECTED mode | Sony |
| [17] | R1-2408478 | LP-WUS operation in CONNECTED modes | Apple |
| [18] | R1-2408495 | Discussion on LP-WUS operation in CONNECTED mode | Panasonic |
| [19] | R1-2408539 | LP-WUS operation in CONNECTED mode | Nokia |
| [20] | R1-2408576 | Discussion on LP-WUS operation in CONNECTED modes | ETRI |
| [21] | R1-2408656 | Discussion on LP-WUS operation in CONNECTED modes | Samsung |
| [22] | R1-2408681 | Discussion on LP-WUS operation in CONNECTED modes | LG Electronics |
| [23] | R1-2408693 | LP-WUS operation in CONNECTED modes | MediaTek Inc. |
| [24] | R1-2408796 | Discussion on LP-WUS operation in CONNECTED mode | NTT DOCOMO, INC. |
| [25] | R1-2408822 | LP-WUS operation in CONNECTED mode | Ericsson |
| [26] | R1-2408860 | LP-WUR operation in connected mode | Qualcomm Incorporated |
| [27] | R1-2408921 | Discussion on LP-WUS procedures in Connected mode | TCL |

Summary of observations/proposals in contributions

R1-2407666	Huawei, HiSilicon

- LP-WUS configuration (periodicity and offset) can be decoupled from C-DRX configuration. - Whether a timer trigged by LP-WUS for UE monitoring PDCCH is existing timer and/or new timer is more related to RAN2 specification. - Option 1-2 can achieve the same effect as Option 1-1 if configured properly. - Option 1-1 cannot achieve the same effect as Option 1-2 since shorter C-DRX cycle introduces more measurements and may not always feasible in real deployment. - If there is false detection of LP-WUS, it is beneficial for UE power saving to let UE go back to LP-WUS monitoring when the timer triggered by LP-WUS expires. - If Active Time is not ended (by legacy DRX related timers) when the timer triggered by LP-WUS expires, it is better to keep UE monitoring PDCCH for potential follow-up scheduling. - It is assumed in the design of LP-WUS in CONNECTED mode that MR and LP-WUR cannot receive DL signal simultaneously. - For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting(Alt2), where X > 1. - If a LP-WUS triggering a PDCCH monitoring is received by the UE at symbol n, the UE is not expected to monitor a PDCCH before symbol n+Y where Y is a minimum time gap determined from the minimum value among the above multiple reported values. - A gNB can apply any one of the reported values to determine the minimum time gap for resource allocation of LP-WUS and its corresponding triggered PDCCH, where it is assumed that applying a larger reported value allows the UE to have larger UE power saving gain. - For LP-WUS in CONNECTED mode, if a UE receives LP-WUS targeting to itself, the UE is not required to start monitoring PDCCH before t0+A+B, where: - t0 is the time point corresponding to the end of LP-WUS - A is the time duration for ramp-up based on UE capability reporting - More than one minimum time gaps are supported and can be reported by UE capability - B is the time duration for time/frequency synchronization of MR - In CONNECTED mode, maximum number of LP-WUS information bits is up to 16 bits. - The impact due to miss-detection/falsely-detection of LP-WUS activation/deactivation signaling should be minimized/avoided for CONNECTED mode UE by e.g. explicit feedback signaling from UE to gNB. - In CONNECTED mode, if LP-WUS monitoring is enabled, at least the following options are supported which can be jointly used - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - It is noted that this option 3 has been supported by the agreement for UE behavior Option 1-2 in RAN1#118. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission. - Note: PDCCH monitoring of a UE is suspended if LP-WUS monitoring of the UE is activated, and vice versa. PDCCH monitoring of a UE is resumed if LP-WUS monitoring of the UE is deactivated. When a UE is in the state of deactivated LP-WUS monitoring, the UE may or may not monitor LP-WUS. - Confirm the WA, i.e. supporting both Option 1-1 and Option 1-2. - In CONNECTED mode, LP-WUS configuration has no relation with C-DRX configuration - continuous monitoring is supported by configuring proper value of monitoring occasion number and monitoring periodicity. - It is not supported that both Option 1-1 and Option 1-2 are simultaneously configured for the same UE. - UE switches from PDCCH monitoring to LP-WUS monitoring after the timer triggered by LP-WUS expires with proper switching time, if legacy DRX related timers is not running. - If DCP is configured, after LP-WUS monitoring is activated, the UE should not monitor DCP in MR. - After LP-WUS monitoring is activated, the UE should stop/suspend the Rel-17 PDCCH adaption mechanism, if configured. - LP-WUS monitoring outside legacy cell DTX active time according to the LP-WUS monitoring configuration can trigger the PDCCH monitoring. - For LP-WUS in FR1-FR2 CA, the trigger of PDCCH monitoring of FR2 cells can be separate from that of FR1 cells - If PDCCH monitoring of an SCell is triggered, then PDCCH monitoring of PCell is also triggered, regardless their frequency range. - For DC case, a LP-WUS and the PDCCH monitoring that the LP-WUS can trigger are expected to be in the same cell group - No specification impact specific to DC. - The TCI state determination can be discussed after the progress achieved on LP-WUS monitoring activation/deactivation - If LP-WUS monitoring is activated by PDCCH, UE can assume the same TCI state as the CORESET to receive the PDCCH activating LP-WUS monitoring - If LP-WUS monitoring is activated by timer, the TCI state can be the same as the CORESET to receive the last PDCCH before the timer expires. - In CONNECTED mode, monitoring parameters of LP-WUS can be adjusted by MR signaling and/or pre-defined rules at least for different coverage requirement.

R1-2407716	Spreadtrum Communications

Proposal 1: Confirm the following working assumption. Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE. Note: Above can be revisited considering RAN2 decisions.. Proposal 2: Both Option 1-1 and Option 1-2-1 simultaneously configured for the same UE should be supported. Proposal 3: When both Option 1-1 and Option 1-2-1 simultaneously configured for the same UE, if the drx-onDurationTimer is not started by the first LP-WUS, there are three kinds of UE behaviors during the drx-onDurationTimer. - Alt 1：UEs do not need to monitor second LP-WUS during the drx-onDurationTimer. - Alt 2: UEs need to monitor second LP-WUS during the drx-onDurationTimer. - Alt 3: The first LP-WUS indicates whether the UE needs to monitor second LP-WUS during the drx-onDurationTimer. Proposal 4: For Option 1-1, the periodicity of LP-WUS monitoring can be derived from DRX cycle Proposal 5: For Option 1-2, the periodicity and offset of LP-WUS can be different from those from C-DRX configuration. Proposal 6: Activation/deactivation of LP-WUS monitoring by gNB L1 signaling (e.g., DCI) can be considered. Proposal 7: For Option 1-2, wake-up indication by LP-WUS can be applied for a group of activated serving cells for triggering PDCCH monitoring.

R1-2407871	vivo

Proposal 1: Supporting following for Option 1-2. - When LP-WUS is detected, LP-WUS triggers the start of a new timer during which UE monitors PDCCH - The UE starts LP-WUS monitoring and stops PDCCH monitoring based on one of the following - No PDCCH is detected during the new timer running and the new timer expires - Expiration of drx-InactivityTimer - Explicit indication received to deactivate PDCCH monitoring and activate LP-WUS monitoring, e.g. by PDCCH skipping command. Proposal 2: Clarify that for both LP-WUS operation option 1-1 and option 1-2 of LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operaion is same as Rel-16 DCP, that is the UE can be configured with one of the following: - Configuration 1: - If the UE is not indicated to wake-up, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake-up, the periodic CSI/L1-RSRP is reported during the DRX active time. - Configuration 2: - If the UE is not indicated to wake-up, the periodic CSI/L1-RSRP is reported during the time duration indicated by drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start. - If the UE is indicated to wake-up, the periodic CSI/L1-RSRP is reported during the DRX active time. Proposal 3: For LP-WUS operation option 1-2, there is no restriction for LP-WUS monitoring configuration (periodicity and offset) in relation with C-DRX configuration. Proposal 4: For both LP-WUS operation option 1-1 and option 1-2, LP-WUS does not support SCell dormancy indication. Proposal 5: For both LP-WUS operation option 1-1 and option 1-2, a UE monitors LP-WUS only on one serving cell per MAC entity when UE is configured with CA in RRC CONNECTED mode. - The serving cell where to monitor LP-WUS is configured by the network. - This applies to UEs configured with single and dual DRX groups Proposal 6: For both LP-WUS operation option 1-1 and option 1-2, when UE is configured with CA in RRC CONNECTED mode, the LP-WUS information is applied to all activated serving/scheduling cells. - FFS for the case of dual DRX groups, LP-WUS received in the serving cell of primary DRX group indicates the triggering of PDCCH monitoring in primary DRX group only, or both DRX groups Proposal 7: For a UE configured with DC and for both LP-WUS operation option 1-1 and option 1-2, it can support LP-WUS monitoring on a serving cell per cell-group depending on UE capability. Proposal 8: For both LP-WUS operation option 1-1 and option 1-2 in or RRC CONNECTED mode, the reported value(s) of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring includes LP-WUS processing time and MR transition time for ramp up and does not include the duration for time/frequency synchronization of MR. Proposal 9: Clarify that the SCS for the reported value(s) for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is the respective SCS of the carrier where LP-WUS is monitored. Proposal 10: Support Alt.2 that for RRC CONNECTED mode for both LP-WUS operation option 1-1 and option 1-2, UE reports multiple values for each SCS from two candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - Different minimum time gaps correspond to different sleep states - The reported value(s) includes LP-WUS processing time and MR transition time for ramp up and does not include the duration for time/frequency synchronization of MR. - Network can configure a proper time gap according to UE reporting, and considering the trade-off between traffic latency requirement and the UE power saving benefit. Proposal 11: In RRC_CONNECTED mode, if the LP-WUS is not detected on a WUS monitoring occasion (MO) by the UE, - For LP-WUS operation option 1-1, the UE is not required to trigger the starting of the drx-onDurationTimer. - For LP-WUS operation option 1-2, the UE is not required to wake up the MR for PDCCH monitoring for option 1-2. Proposal 12: In RRC_CONNECTED mode, for both LP-WUS operation option 1-1 and option 1-2, the indication from UE to indicate whether it monitors LP-WUS or PDCCH is not needed. Proposal 13: After LP-WUS monitoring is enabled by RRC signalling, the specified conditions used for LP-WUS deactivation due to poor channel condition for LP-WUS monitoring is not needed for both LP-WUS operation option 1-1 and option 1-2. Proposal 14: UE is allowed to fallback to MR PDCCH monitoring by implementation (e.g. due to poor channel condition) without specified condition. - For LP-WUS operation option 1-1, the fallback PDCCH monitoring behavior is according to the C-DRX configuration - For LP-WUS operation option 1-2, the fallback PDCCH monitoring behavior is always-on PDCCH monitoring. Proposal 15: For LP-WUS operation option 1-1 and option 1-2 in RRC_CONNECTED mode, UL transmissions such as SR, PRACH and CG PUSCH can trigger MR PDCCH monitoring according to the legacy UE behavior. Proposal 16: For LP-WUS operation option 1-1 and option 1-2 in RRC_CONNECTED mode, UE switching from PDCCH monitoring to LP-WUS monitoring is supported by gNB L1/L2 signaling. Proposal 17: if PDCCH skipping indication is used as LP-WUS activation signalling, enhanced PDCCH skipping indication can be used to indicate UE stop PDCCH monitoring on one or mulitiple serving cells and starts LP-WUS monitoring. Proposal 18: For a UE configured with both LP-WUS monitoring and Rel-17 SSSG switching, Rel-17 SSSG switching is applied during the time period when MR performs PDCCH monitoring. There should be no impact due to LP-WUS monitoring. Proposal 19: A UE does not expect to monitor the LP-WUS during the non-active periods of cell DTX of a serving cell, if cell DTX is activated for the serving cell with the LP-WUS monitoring. Proposal 20: As in legacy, UE monitors PDCCH on a cell during the cell-DTX active period in case LP-WUS for the cell is detected. Proposal 21: LP-WUS monitoring occasions (MOs) are defined by a network-configured Periodicity and Offset. The configuration should allow multiple LP-WUS MOs within each periodicity. - For LP-WUS operation option 1-1, the periodicity equals to the C-DRX periodicity - For LP-WUS operation option 1-2, the periodicity is separately configured from C-DRX periodicity Proposal 22: For RRC CONNECTED mode, for LP-WUS monitoring in FR2, QCL Type D is applicable for LP-WUS QCLed with SSB/CSI-RS. - FFS QCL Type A, B and C for FR1 and FR2 Proposal 23: Re-use existing TCI state with or without unified framework to support the TCI state configuration/indication for a LP-WUS resource. Proposal 24: Support at least bitmap for RRC connected mode and the maximum number of LP-WUS information bits is up to 16 bits.

R1-2407915	CMCC

Proposal 1. Confirm the working assumption that both Option 1-1 and Option 1-2 can be supported for PDCCH monitoring triggered by LP-WUS with C-DRX configuration. Proposal 2. Support to introduce a new timer for Option 1-2. Proposal 3. LP-WUS procedure Option 1-1 and Rel-16 DCP cannot be configured or activated to one UE simultaneously. Proposal 4. LP-WUS procedure Option 1-1/Option 1-2 and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously. Proposal 5. Support Alt 2: UE report more than one minimum time gaps from multiple candidate values. - gNB can configure a reasonable value from the reported values. Proposal 6. Support the minimum time gap not include the duration for time/frequency sync of MR. - Also note that there is no need to define the specific composition of the candidate values. Proposal 7. Support LP-WUS QCLed with SSB and/or CSI-RS in QCL-Type A. Proposal 8. Support to not introduce additional activation/deactivation mechanism for LP-WUS monitoring. Proposal 9. Regarding the issue on LP-WUS monitoring configuration, support: - For Option 1-1, a time offset between LP-WUS and the starting point of DRX ON is configured without the configuration of periodicity. - For Option 1-2, a time offset and periodicity can be configured similar as search space configuration. Proposal 10. Support LP-WUS applicable to one or more serving cells in a (DRX) cell group under CA scenario. - For the case that a MAC entity has two DRX cell groups, LP-WUS should be separately configured for each DRX cell group.

R1-2407979	Xiaomi

Proposal 1: How to reduce unnecessary PDCCH monitoring within legacy C-DRX active time should be considered for both Option 1-1 and Option 1-2. Proposal 2: LP WUS applied within C-DRX legacy active time should be considered and down-selected, Case 1: LP WUS is used within C-DRX re-transmission timer running duration; Case 2: LP WUS is used within C-DRX active time and within PDCCH skipping duration; Case 3: LP WUS is used within C-DRX active time and outside PDCCH skipping duration. Proposal 3: From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration, Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. Support Option 1-3: LP-WUS monitoring within legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. Proposal 4: Support multiple repetitions of a LP WUS occasion. Proposal 5: LP WUS can be configured in the same or different carrier/band from where MR operates. Proposal 6: UE is only required to monitor LP WUS on one carrier. Proposal 7: LP WUS at least includes bits indicating whether to monitor PDCCH. SSSG switching, BWP switching indication can also be considered. Proposal 8: LP WUS indication can be per cell or per cell group when UE is configured with multiple serving cells. Proposal 9: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. X is no less than 3. Proposal 10: If UE reports multiple wake-up delay values for each SCS, network can indicate UE a suitable wake-up delay based on its knowledge of the UE’s traffic characteristics. Proposal 11: Semi-persistent channels’ impact on LP WUS wake-up delay should be considered. Proposal 12: Support CSI report skipped or relaxed during the MR’s non-active period and support CSI report for additional active time triggered by LP WUS. Proposal 13: LP WUS in connected state can be QCLed with LP -SS by QCL type A, and can be QCLed with SSB by QCL type C.

R1-2408057	CATT

Proposal 1: For RRC_CONNECTED mode, the LP-WUS could be configured for one or more UEs for the control signaling to indicate the UE wake up in the subsequent DRX ON. Proposal 2: UE can only report one minimum time gap and the value of minimum gap depends on the functionality of LP-WUS whether the LP-WUS indication includes the SCell dormancy indication. Proposal 3: The periodicity of LP-WUS MOs can be configured deterministically. Proposal 4: One LP-WUS is configured to be QCLed with the RS associated with each of the TCI states of the CORESETs in order to trigger the PDCCH monitoring at the associated CORESETs when LP-WUS is detected. Proposal 5：The related procedures of Option1-1 can first discussed in RAN1 and wait more progress from RAN2 for Option1-2. Proposal 6: NOT support UE assistance for PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling. Proposal 7: Support Option 1: No additional indication/condition is introduced for activation/deactivation of LP-WUS monitoring. Proposal 8: PDCCH monitoring triggered by a LP-WUS is applicable for all serving cells with SCell dormancy indication in LP-WUS.

R1-2408076	ZTE Corporation, Sanechips

Observations: Observation 1: Monitoring DCI for LP-WUS activation, compared with RRC activation, causes more power consumption. Observation 2: If LP-WUS is configured in BWP framework, the retuning capability is required and the feasibility for RF-ED architecture may need further studied. Observation 3: - LP-WUS indicating wake-up for all SCells in FR1 or FR2 belongs to option1 - LP-WUS indicating wake-up for all SCells in first DRX group or second DRX group belongs to option1 - Option2 does not differentiate any serving cells group based on DRX or frequency range Observation 4: LP-WUS wake-up all the activated serving cells [based on DRX group or FR1 or FR2 ] bring massive unnecessary power consumption. - LP-WUS can not bring PSG but more power consumption Observation 5: For LP-WUS indication method, bitmap method has obvious less system overhead than codepoint method especially for frequent traffic case. Observation 6: For LP-WUS indication method, bitmap method may occupies more MOs, but not obvious difference as codepoint method. Observation 7: For the CA support in LP-WUS indication, bitmap method is easier to be extended. Observation 8: If LP-WUS is monitored outside active time, no spec impact is foreseen for PDCCH skipping and LP-WUS coexistence. Observation 9: No much power saving is observed if LP-WUS is used for SSS switching and no issue is observed when LP-WUS coexist with SSSG switching. Proposals: Proposal 1: RRC activation/deactivation is the baseline - DCI deactivation and fallback mechanism based on that UE does not received LP-WUS for a long time can be considered. - The motivation for L2 activation/deactivation should be further clarified if L1 deactivation is introduced. - Option 4 is not considered for activation/deactivation of LP-WUS monitoring. Proposal 2: For option 1-1, LP-WUS monitoring could be configured with a same periodicity with C-DRX and an offset - Discuss whether LP-WUS could trigger short DRX cycle - Discuss the offset is relative to C-DRX or others Proposal 3: For LP-WUS monitoring with option 1-1, consider following two alternatives to achieve better scheduling flexibility - Alternative 1: multiple LP-WUS monitoring space with different offsets - Alternative 2: a monitoring window with configurable length is considered for LP-WUS monitoring Proposal 4: For option 1-2, based on RRC configuration, the UE determines whether to monitor PDCCH during legacy C-DRX active time. - i.e., based on RRC configuration, UE determine monitoring LP-WUS based on option 1-2-1 and option 1-2-2 - RRM may perform measurement during legacy C-DRX active time. Proposal 5: For option 1-2, LP-WUS monitoring may be configured with - An offset relative to legacy C-DRX - Another offset could be introduced between new active time and LP-WUS - An periodicity no larger than C-DRX periodicity - A LP-WUS monitoring window - New active time related configurations, e.g., timer or time duration, which could be up to RAN2. Proposal 6: For support option1-1 and option 1-2 simultaneously, consider the following directions for down-selection: - Direction 1: UE supports only one of option 1-1 and option 1-2 - Direction 2:UE can support both of option 1-1 and option 1-2, but gNB only configure one of them via RRC signalling - Direction 3: UE can support both of option 1-1 and option 1-2, and gNB can configure both of them via RRC signalling Proposal 7: The LP-WUS is applied for active DL BWP for PDCCH monitoring - LP-WUS could be configured outside the active BWP Proposal 8: Outside new or legacy active time, reusing legacy mechanism as DCP for Searchspace monitoring. Proposal 9: The LP-WUS is used to indicate wake-up for PDCCH monitoring - Target all the configured searchspaces in the active DL BWP Proposal 10: In connected mode, support multiple LP-WUS monitoring searchspace configuration to have different TCI-state configuration Proposal 11: The LP-WUS QCL source could be CD-SSB, NCD-SSB or CSI-RS. - At least type C QCL relationship can be supported. Proposal 12: LP-WUS is only transmitted in primary Cell. Proposal 13: Consider Option 1: one or more serving cells based on gNB indication/configuration Proposal 14: LP-WUS could be used to wake-up one or more activated SCells. - PCell is always wake-up after LP-WUS reception Proposal 15: Select one of the following for DC case Case 1: LP-WUS is only transmitted in PCell, and LP-WUS to indicate whether PSCell is to wake-up for PDCCH monitoring Case 2: LP-WUS is transmitted in both PCell and PSCell, LP-WUS to indicate wake-up for PDCCH monitoring based on its own PCell or PSCell. Proposal 16: Support Option 1: bitmap corresponding to [one or more] UEs as baseline Proposal 17: Alt 2 should not be supported for wake-up delay report. Proposal 18: For the minimum time gap, UE reports one value based on the following template SCS (kHz); Minimum Time Gap X (slots). OOK WUR; OFDM WUR. 15; One or two values; One or two values. 30; One or two values; One or two values. Proposal 19: During Cell-DTX off period, UE does not need to monitor LP-WUS.

R1-2408131	OPPO

Proposal 1: A UE can be indicated by LP-WUS codepoints, in which each value can be corresponding to one or more UEs in CONNECTED mode. UE in CONNECTED mode is UE-specifically configured with the LP-WUS resources and codepoints. Proposal 2: The LP-WUS monitoring periodicity and offset can be introduced for triggering a PDCCH monitoring timer, in CONNECTED mode for both Options. Proposal 3: The LP-WUS should be configured with following legacy schemes in CONNECTED mode: Rel-16 Cross-slot scheduling can be configured together with LP-WUS in CONNECTED mode without specification impact. When PDCCH Skipping/SSSG switching is configured, the PDCCH skipping indication can enable the LP-WUS monitoring occasions. For the Cell DTX, LP-WUS is assumed to be disabled during the DTX time. Proposal 4: When the LP-WUS is configured with CA in CONNECTED mode: One serving cell should be only indicated by the LP-WUS in the same carrier. Only wake-up bits are carried by LP-WUS, e.g., no Scell dormancy indication. Proposal 5: For the LP-WUS activation and deactivation by pre-configured condition(s), timer-based monitoring of LP-WUS is the baseline. Proposal 6: LP-WUS Monitoring Occasions can be associated with CORESETs for the TCI states determination. The activated TCI state of the CORESET is applied for the associated LP-WUS Monitoring Occasion. Proposal 7: RAN1 can assume that MR measurements are one of the UE assistant information for the LP-WUS activation and deactivation. RAN1 discuss the introduction LP-SS measurement in CONNECTED mode as another UE assistant information. Proposal 8: In RRC CONNECTED mode, support UE capability report one value among candidate values for minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. Proposal 9: RAN1 may specify that the monitoring occasion of LP-WUS and associated monitoring occasion of PDCCH should meet the reported minimum gap.

R1-2408219	NEC

Proposal 1: support LP-WUS in combination with DCP to further reduce power consumption without introducing too much overhead. Proposal 2: support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

R1-2408255	Sharp

Observation 1: DCI format 0_1/1_1 can also provide the function of dormancy indication. whether including dormancy indication in LP-WUS needs further study. Proposal 1: Does not support the simultaneous configuration of both options for one UE. Proposal 2: Support bitmap with each bit corresponding to one UE for UE in connected mode for non-CA case. Proposal 3: Support to provide only LP-WUS in the Pcell to carry indication for all SCells in CA cases. Proposal 4: Support Option 1, which can include indications for one or more serving cells seperately with an LP-WUS. Proposal 5: It is not necessary to include dormancy indication in LP-WUS. Proposal 6: Support Alternative 2, which allows UEs to report multiple values for each SCS.

R1-2408282	InterDigital, Inc.

Observation 1: Latency due to use of Option 1-1 can be reduced by using Option 1-2 to trigger PDCCH monitoring (e.g., starting a drx-onDurationTimer) outside C-DRX active time. Observation 2: For Option 1-2, using a new timer for UE to monitor PDCCH is beneficial over re-using an existing timer Observation 3: Supporting only Option 1-2 can cause issues due to limited coverage of LP-WUS. Observation 4: Identifying an option between Option 1-1 and Option 1-2 for triggering PDCCH monitoring is needed when Option 1-1 and Option 1-2 are simultaneously configured. Observation 5: LP-WUS monitoring configuration should be selected to match the conditions UE is experiencing. Observation 6: Higher power saving can be achieved by activating LP-WUS monitoring only when UE is in appropriate conditions. Observation 7: gNB is unable to select all the appropriate LP-WUS monitoring configuration and determine to activate/deactivate LP-WUS monitoring without UE assistance. Observation 8: Limiting PDCCH monitoring triggered by LP-WUS to a subset of serving cells can make LP-WUS monitoring not suitable for users that required fast access to additional bandwidth. Observation 9: A LP-WUS design that can control waking-up of each UE separately can provide higher power saving compared to a LP-WUS design in which multiple UEs share the same wake-up indication. Observation 10: Supporting SCell dormancy indication via LP-WUS is important to make LP-WUS monitoring can be used for all the use cases supported by DCP. Observation 11: Compact LP-WUS payload size is important to support coverage comparable with PUSCH for Msg3. Observation 12: LP-WUS payload can be reduced by implicitly indicating some of information of LP-WUS. Observation 13: Use of sleep states achieving higher power saving can cause increased latency while use of sleep states that supports lower latency can reduce power saving gains of LP-WUS monitoring. Observation 14: Configuration of periodic CSI/L1-RSRP reporting mechanism can be inefficient in case of long inactive durations. Proposal 1: Confirm the working assumption on supporting both Option 1-1 and Option 1-2. - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. Proposal 2: A new timer is introduced for supporting Option 1-2. Proposal 3: Support simultaneous configuration of Option 1-1 and Option 1-2. Proposal 4: Introduce a mechanism to properly manage PDCCH monitoring based on Option 1-1 and Option 1-2. Proposal 5: PDCCH monitoring triggered by LP-WUS is enabled/disabled or activated/deactivated with UE assistance. Proposal 6: For a LP-WUS monitoring enabled UE, support activating/deactivating LP-WUS monitoring based on following preconfigured conditions. - For activating LP-WUS monitoring: level of UE activity (e.g., timer and/or counter associated with UE activity), channel/signal quality, mobility. - For deactivating LP-WUS monitoring: level of UE activity (e.g., timer and/or counter associated with UE activity), channel/signal quality, mobility, LP-WUS reception quality. Proposal 7: For OFDM based LP-WUR, both NR-SS (existing SSB) and LP-SS can be used to determine activation/deactivation of LP-WUS monitoring. Proposal 8: Support a mechanism to report UE decision of LP-WUS monitoring deactivation potentially with reason for the decision. Proposal 9: When UE is configured with CA in RRC connected mode, PDCCH monitoring triggered by a LP-WUS is applicable to all active cells (Option 2). Proposal 10: Supporting DC with connected model LP-WUS monitoring is deprioritized in Rel-19. Proposal 11: Indications supported in DCP, i.e., per UE wake-up indication and SCell dormancy indication, should be supported in LP-WUS. Proposal 12: A subset of LP-WUS information can be informed to UE implicitly using LP-WUS monitoring resource (e.g., LO and MO) association and structure of LP-WUS. Proposal 13: For reporting time gaps between LP-WUS reception and MR to start PDCCH monitoring in connected mode LP-WUS monitoring, Alt 2 is supported. - Alt 2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. Proposal 14: For connected mode LP-WUS monitoring, introduce a mechanism to support CSI/L1-RSRP reporting triggered when inactivity periods exceed a preconfigured threshold.

R1-2408322	Lenovo

Proposal 1: UE may need to report one or more suitable time gaps for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring (e.g., one for deep sleep state with a 20ms transition time, another for light sleep state with a 6ms transition time). Proposal 2: The reported time gap value includes LP-WUS processing time, MR transition time for ramp up, and the duration for time/frequency synchronization of MR. Proposal 3: UE shall go to the corresponding sleep state (e.g., deep sleep or light sleep) based on the configured time gap and/or synchronization performance. Proposal 4: One or more periodical LP-WUS occasions are configured in WUS monitoring window before drx-onDurationTimer to trigger the PDCCH monitoring at least for Option 1-1. Proposal 5: For CONNECTED mode, LP-WUS monitoring can be activated/deactivated by timer (especially for switching between LP-WUS monitoring and MR PDCCH monitoring after activation). Proposal 6: When channel/beam quality is below a certain value for a certain time in LR, UE is expected to fallback to trigger PDCCH monitoring in MR. Proposal 7: gNB configures the cell lists/carrier lists for UE PDCCH monitoring if MR is configured with CA. Proposal 8: For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X=16 bits.

R1-2408418	Sony

Proposal 1: Confirm the working assumption to support Option 1-1 and Option 1-2. Proposal 2: Support LP-WUS in RRC connected mode with multiple time-gaps Proposal 5: At least for Option 1-2, LP-WUS transmission in RRC Connected mode indicates the UE on the subsequent action to be performed upon the reception of LP-WUS. Proposal 4: RAN1 strives for the LP-WUS common design including supporting sequence-based LP-WUS operation in both RRC CONNECTED and RRC idle/inactive mode. Proposal 6: The maximum number of LP-WUS information bits for RRC connected mode is up to 8 bits Proposal 3: The UE operating in RRC connected mode and configured to receive LP-WUS is expected to monitor a given LP-WUS resource. Proposal 7: Support both duty-cycled monitoring mode and continuous monitoring mode for LP-WUS in RRC-connected. Proposal 8: Enabling/Disabling of LP-WUS monitoring by RRC signalling is sufficient. No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring (known as Option 1).

R1-2408478	Apple

Observation: Duration when LP-WUS triggered PDCCH monitoring timer is running should be included in the Active Time. Proposal 1: For Option 1-2, detection of LP-WUS triggers a new timer lp-TriggeredTimer for PDCCH monitoring. Switching to LP-WUS during Active Time Proposal 2: Support UE switching to LP-WUS monitoring within Active Time, based on L1/L2 signaling or timer. - It can be further discussed which active timer(s) are considered. LP-WUS monitoring modes Proposal 3: Support the following four monitoring modes of LP-WUS monitoring, and NW configures the corresponding mode for the LP-WUS configuration: - Mode 1: LP-WUS monitoring occasions configured by RRC are always monitored. - Mode 2: LP-WUS monitoring occasions configured by RRC are monitored outside Active Time and not monitored within Active Time. - Mode 3: LP-WUS monitoring occasions configured by RRC are monitored outside Active Time and not monitored within Active Time until L1/L2 signaling indication. - Mode 4: LP-WUS monitoring occasions configured by RRC are not monitored by default and are monitored according to L1/L2 signaling within Active Time. LP-WUS monitoring occasions Proposal 4: If Option 1-1 is supported, a time window for LP-WUS monitoring can be configured, otherwise no explicit configuration of time window is needed. Minimum time gap Proposal 5: UE could report 2 values of minimum gap according to different sleep modes outside and within Active Time. LP-WUS operation with CA Proposal 6: Support LP-WUS triggerring all activated serving cells (Option 2), considering impact on UE complexity as well as NW signaling overhead.

R1-2408495	Panasonic

Proposal 1: Firstly, more concrete design of Option 1-1 should be progressed especially to the relation with legacy C-DRX active time other than drx-onDurationTimer. Then missing function may be added by Option 1-2. Proposal 2: The reported value in UE capability report should be defined as the summed value of LP-WUS processing delay in LR, ramping up time of MR, the duration for time/frequency synchronization of MR and optionally the time to change the carrier. Proposal 3: Alt1 should be supported as the baseline for the reported value for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability report. For the case of shorter reporting values assuming deep sleep and the carrier is different between LP-WUS reception and MR operation, additional time for switch the carriers can be reported, which can be seen as Alt2. Proposal 4: For supporting CA, LP-WUS can be supported in either PCell or SCell. Proposal 5: For DC with LTE-NR case, LP-WUS is not supported. Proposal 6: For DC with NR-NR case, LP-WUS can be supported and monitored in PCell or SCell in PCG to indicate PDCCH monitoring in both PCG and SCG. Proposal 7: The relation between LP-SS and LP-WUS is QCL type A+D. Proposal 8: The relation between SSB and LP-SS/LP-WUS is QCL type A. To support beam forming, QCL type A+D can be configurable. Proposal 9: The relation between TRS and LP-SS/LP-WUS is QCL type B when TRS is configured. To support beam forming, QCL type A+D can be configurable when SSB and LP-SS/LP-WUS is QCL type A+D. Proposal 10: The relation between CSI-RS other than TRS and LP-SS/LP-WUS is not required to be defined.

R1-2408539	Nokia

Observations: Observation 1: The RAN1 RRC IDLE-INACTIVE mode definition of LO supports - Potentially multiple Mos, depending on the number MOs for each beam. - A periodicity aligned to the iDRX cycle. - A start point is offset from a reference PO/PF. Observation 2: The RAN2 agreements for RRC Connected mode support: - For options 1-1 and 1-2, a new time offset between LP-WUS MO at the start of the ACTIVE time. - For option 1-2, a timer to limit PDCCH monitoring. Observation 3: Circular shifted sequences can be used to indicate either the superset or subset of PDCCH monitoring options. FFS: The specific PDCCH monitoring options. Observation 4: For option 1-2 the wake-ups can be more frequent, therefore having shortened on-Duration timer triggered by the LP-WUS is desirable. Observation 5: The onus of UE power management, should be on the UE and not the network, given the number of factors that are known only to the UE. Observation 6: Requiring the network to determine/compute the minimum time gaps: - Increases network complexity and adds additional signalling - Makes potentially limiting assumptions about the UE design and capabilities - Deviates from the principle applied to signal the similar value for the Idle/inactive mode. Observation 7: For RRC CONNECTED mode, legacy procedures can be used to vary the activation and dormancy states of SCells during the PDCCH monitoring phase between LP-WUS monitoring periods. Observation 8: For RRC CONNECTED mode, the key difference between an ACTIVATED SCell that is in a DORMANT state and one that is a NON-DORMANT state, is that the NON-DORMANT SCell supports both measurements and PDCCH monitoring whereas the DORMANT SCell supports only a minimal set of measurements. Observation 9: LP-WUS does not impose limitations on existing CA configuration. Observation 10: Minimising signalling to support the operation of LP-WUS can help reduce power and resource consumption as well as minimise processing delays. Observation 11: For CONNECTED mode, LP-WUR time and frequency synchronisation can be provided by the MR. Proposals: Proposal 1: Convert the working assumption regarding the support of Options 1-1 and 1-2, into an agreement. Proposal 2: For RRC Connected Mode, a time window concept for MO(s) is not supported. Proposal 3: RAN1 discuss the specific group of PDCCH occasions that the LP-WUS triggers for options 1-1 and 1-2. Options include: 1. LP-WUS triggers PDCCH monitoring in the next OnDuration only 2. LP-WUS triggers PDCCH monitoring in the next “N” OnDurations 3. LP-WUS triggers PDCCH monitoring at specific PDCCH locations within the next OnDuration FFS: Signaling of different options. Proposal 4: For connected mode option 1-2, legacy C-DRX configuration does not restrict LP-WUS configuration. Proposal 5: For connected mode option 1-2, a new On-duration timer is supported to limit the PDCCH monitoring time. Proposal 6: For connected mode LP-WUS option 1-2 operation, discuss the need for: - Short to Long DRX cycle changes. - A new LP_WUS specific PDCCH monitoring INACTIVITY timer Proposal 7: Option 1-3 is deprioritised. Proposal 8: The UE reports one value of minimum time gap (alt. 1), corresponding to the minimum time gap that can enable the UE to save power when using LP-WUS. Proposal 9: Consider a procedure or procedure enhancements, to enable accounting for the configuration and/or changes in the configuration, in the UE capability reporting for minimum time gap (eg the UE Assistance Information). Proposal 10: Consider the early indication of LP-WUS connected mode characteristics, that the UE can use to determine optimum minimum time gap. FFS: The list of characteristics, which may include: - Availability of synchronisation sources - LP-WUS Mode of operation (1-1 or 1-2) - The cDRX characteristics. - Carrier Mode scheduling configuration (if supported) Proposal 11: For the RRC CONNECTED mode, the UE reports one value from X candidate values for the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting, that includes the MR time to reacquire time/frequency synchronisation. FFS: The value X FFS: Characteristics that should be signalled to the UE early, for it to compute it’s preferred minimum time gap value. Proposal 12: When LP-WUS is used to trigger PDCCH monitoring with CA configured, PDCCH monitoring is started on only the cells where scheduling is supported and no implicit Scell status change. Proposal 13: RAN1 discuss the handling of SCell dormancy when LP-WUS is in operation. Proposal 14: The Network re-uses the TCI-state framework for the PDCCH Coreset associated with the LP-WUS, to determine the QCL relationship with LP-WUS. Proposal 15: The LP-WUS from the gNB is QCLed with the SSB/CSI-RS via at least Type-D. FFS: If Type-B/C QCL relationships are also applicable Proposal 16: The activation and deactivation of LP-WUS monitoring can be achieved without additional indication from the UE. FFS: RAN4 to determine the characteristics of the MR measurements required to ensure this “no indication” method is reliable. Proposal 17: LP-WUS monitoring can be supported whilst the MR is performing other actions. Proposal 18: From a RAN1 perspective, an option to support explicit UE feedback to indicate loss of LP-WUS in CONNECTED mode is supported. Proposal 19: Discussion of CONNECTED mode payload size and coding is deferred to agenda item 9.6.1 Proposal 20: For CONNECTED mode, LP-SS is not needed during normal LP-WUR operation with LP-WUR. Proposal 21: For CONNECTED mode, there is no change to how existing MR based measurements (RRM/RLM/BFD/CSI) are determined when LP-WUS is enabled.

R1-2408576	ETRI

Proposal 1: The working assumption for supporting Option 1-1 which has the same function as Rel-16 DCP should be confirmed. Proposal 2: It is proposed to support both Option 1-1 and Option 1-2 simultaneously when Option 1-2 is configured. Proposal 3: Regarding Option 1-3, it is proposed to support the followings: - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Option 1-3 should be configured together with Option 1-1. - This option is only applicable to limited cases where drx-onDurationTimer is longer than a specific threshold. Proposal 4: The LP-WUS occasions for additional options i.e., Option 1-2 or Option 1-3 should be configured separately from the configuration for Option 1-1. Proposal 5: It is proposed to focus on the above three options i.e., Option 1-1, the combination of Option 1-1 and Option 1-2, the combination of Option 1-1 and Option 1-3 for further study, and preclude other options. Proposal 6: It is proposed to focus only duty-cycled monitoring mode for LP-WUS in Rel-19. Proposal 7: For the UE capability report, it is proposed to support - Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - X = 2 - FFS: definition of reported value Proposal 8: Regarding periodicity of LP-WUS monitoring occasions, the periodicity should be the same or integer multiple of C-DRX periodicity. - If the periodicity of LP-WUS monitoring occasions is the same as the C-DRX periodicity, no sperate configuration is necessary. Proposal 9: In order for gNB to know that LP-WUS is monitored by a UE correctly, it will be sufficient the implicit indication such as - HARQ-ACK feedback corresponding to scheduled downlink transmission - Scheduled uplink transmission - Other options are not precluded. Proposal 10: It is proposed to study the options for activation/deactivation of LP-WUS monitoring further after the supporting scenarios and options become clear enough since different options can be supported depending on the scenarios and LP-WUS use options. Proposal 11: Regarding the cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable, Option 2 should be considered for further study. - Option 2: all activated serving cells Proposal 12: For LP-WUS transmission, the latest TCI state for CORESET can be applied. - Default TCI state can be also pre-configured in order to prevent application of outdated TCI state. Proposal 13: It should be supported fallback mechanisms which the MR switches to legacy operation in case the channel condition of LP-WUS is not good enough. - Details of fallback mechanisms can be discussed further.

R1-2408656	Samsung

Proposal 1: For activation/deactivation of LP-WUS monitoring, prioritize Option 2 and Option 3, and support explicit confirmation message(s) from the UE to the gNB for handshake in Option 2. Proposal 2: For Option 1-1 in PDCCH monitoring after LP-WUS indication: - The periodicity for LP-WUS monitoring occasions is same as the DRX cycle; - At least one LP-WUS monitoring occasion is configured before the drx-onDurationTimer, subject to the minimum delay requirement; - After receiving LP-WUS indicating wake-up, the UE starts drx-onDurationTimer from the beginning of the next DRX cycle; - The UE monitors PDCCH within the drx-onDurationTimer; - Other UE behaviour (including the other timers and measurement) maintains the same as legacy. Proposal 3: For Option 1-2 in PDCCH monitoring after LP-WUS indication: - RAN1 needs to further clarify which one or both of the following two scenarios is supported: - Scenario 1: Option 1-2 timer start within the drx-onDurationTimer; - Scenario 2: Option 1-2 timer start outside the drx-onDurationTimer. - For Scenario 2, RAN1 needs to further clarify whether RRM/RLM/BFD is performed during gap between the start of Option 1-2 timer and start of the drx-onDurationTimer. Proposal 4: For the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring, support Alt 1 in RAN1#118 agreement: - Support two set of minimum gaps per SCS (i.e., X=2), and the UE can report its capability of one set of minimum gaps. - UE’s reported value shall include the duration for time/frequency synchronization of MR. Proposal 5: The QCL source for LP-WUS can be provided to the UE as part of the configuration for LP-WUS. Proposal 6: RAN1 needs to clarify whether to support the secondary group DRX configuration with LP-WUS.

R1-2408681	LG Electronics

Proposal #1: Specify LP-WUS MOs with a periodicity. - LP-WUS inside monitoring window triggers PDCCH monitoring by starting drx-onDurationTimer (Option 1-1). - LP-WUS outside monitoring window triggers PDCCH monitoring outside legacy DRX Active Time (Option 1-2). Proposal #2: Consider LP-WUS triggers the start of drx-InactivityTimer or a new timer. Proposal #3: Consider candidate starting points of potential DRX Active Time triggered by LP-WUS. Proposal #4: Consider a UE can monitor LP-WUS to trigger the starting of drx-onDurationTimer (i.e., Option 1-1) when the UE is monitoring PDCCH outside legacy DRX Active Time. Proposal #5: Support that a UE reports single value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. Proposal #6: The minimum time gap between LP-WUS reception and MR to start PDCCH monitoring should include the duration required for time/frequency synchronization of MR. Proposal #7: For LP-WUS monitoring in RRC CONNECTED mode, consider followings on TCI-state of LP-WUS - Re-use existing TCI-state framework to indicate TCI state of LP-WUS. - If TCI-state is configured but not indicated, a UE assumes that LP-WUS is QCLed with SSB that the UE recently received for random access or initial access. Proposal #8: Support LP-WUS triggering PDCCH monitoring for the serving cells based on gNB indication/configuration. (i.e., option 1). Proposal #9: Study how SCell of the UE is operated when start of drx-onDurationTimer is triggered by LP-WUS replacing DCP. Proposal #10: Co-existence of Rel-17 PDCCH monitoring adaptation and LP-WUS can be revisited after the LP-WUS procedure about PDCCH monitoring are discussed. Proposal #11: Further discuss how LP-WUS (and LP-SS) can be handled or interacted with cell DTX configurations. Proposal #12: Specify activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. Proposal #13: Specify activation/deactivation of LP-WUS monitoring based on DRX timer and/or UL transmission at least considering followings, - LP-WUS monitoring is deactivated when drx-onDurationTimer and drx-InactivityTimer is running. - LP-WUS monitoring is activated when drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-RetransmissionTimerSL, ra-ContentionResolutionTimer, or msgB-ResponseWindow is running. Proposal #14: Consider the case where LP-WUS monitoring is in active state even during PDCCH monitoring is activated. Proposal #15: To overcome limited maximum number of LP-WUS information bits, consider the combination of time/frequency resources and LP-WUS payload for UE identification. Proposal #16: For supporting more functionality or information bits, consider the transmission of overlaid OFDM sequence over LP-WUS for CONNECTED modes. Proposal #17: Discuss how to compose the payload and overlaid OFDM sequence of LP-WUS for CONNECTED mode at least as follows, - The essential indication for UE behavior is contained in the payload of LP-WUS. - The additional information for UE behavior is contained in the overlaid OFDM sequence of LP-WUS.

R1-2408693	MediaTek Inc.

Observation 1: Option 1-1 is suitable for predictable, periodic traffic like VoIP, while Option 1-2 is designed for unpredictable or misaligned periodic traffic such as XR. Proposal 1: Support both Option 1-1 and Option 1-2 for PDCCH monitoring in RRC CONNECTED mode to accommodate different traffic types. Observation 2: Reporting one value per SCS simplifies the process, reduces complexity, and minimizes reporting overhead. Proposal 2: UE reports one value per SCS for the minimum time gap, simplifying the process and reducing reporting overhead. For the reporting value, consider reusing the minimum time gap values of DCI format 2-6 from 38.213 Table 10.3-1. Observation 3: Supporting LP-WUS for Carrier Aggregation (CA) in RRC CONNECTED mode can save power by waking up only part of the carriers. Proposal 3: Support LP-WUS for CA in RRC CONNECTED mode using a bitmap to wake up specific carriers, enabling flexible scheduling and power savings.

R1-2408796	NTT DOCOMO, INC.

Proposal 1: Support Option 1-1 and Option 1-2 as separate UE capabilities Proposal 2: On whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE, down select from one of the following - Alt1: Two options are specified with separate frameworks/configurations (e.g., separate LP-WUS MOs, separate PDCCH monitoring timers). If both frameworks are configured for a UE, the UE operate both simultaneously - Alt2: Two options are specified with a common framework/configuration (e.g., common LP-WUS MOs, common PDCCH monitoring timer). If the framework including both options is configured for a UE, the UE operate both simultaneously Proposal 3: Support Alt 1, i.e., - For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Proposal 4: Consider TCI framework for CORESET as baseline for LP-WUS: RRC configured K TCI states and MAC-CE activates one of them - FFS value range of K - FFS timeline to apply the activated TCI state Proposal 5: Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), support Option 3: A codepoint value corresponding to one or more UEs - FFS details for extension of option 3 when UE is configured with CA Proposal 6: LP-WUS is supported for the case where UE is configured with CA/DC in RRC CONNECTED - LP-WUS indication is applicable to all activated serving cells in the same DRX group/cell group, if configured - For DC case, LP-WUS monitoring/indication is applied per cell group - For dual DRX groups, further consider following options - Option 1: LP-WUS monitoring/indication is applied per DRX group - Option 2: LP-WUS monitoring on one of DRX group, LP-WUS indication to either or both DRX group(s) Proposal 7: For RRC CONNECTED mode, when UE is configured with LP-WUS by gNB RRC signaling, UE is expected to monitor LP-WUS according to the LP-WUS monitoring configuration Proposal 8: For RRC CONNECTED mode, when UE detects a LP-WUS indicating the UE to wake-up, UE monitors PDCCH, and UE is not required to monitor LP-WUS during the PDCCH monitoring Proposal 9: From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, PDCCH monitoring is resumed when LP-WUS monitoring is deactivated by additional indication/condition (if supported) Observation 1: Prior to discuss coexistence with existing features (PDCCH skipping, SSSG switching, cell DTX), RAN1 needs to discuss procedures to trigger PDCCH monitoring

R1-2408822	Ericsson

Observation 1 OOK-based LP-WUS (with ≤2 slots duration) is less reliable than PDCCH for following cases • PDCCH 1Rx reception for PDCCH aggregation levels 8,16 • PDCCH 2Rx reception for PDCCH aggregation levels 4,8,16 • PDCCH 4Rx reception for PDCCH aggregation levels 2,4,8,16 Observation 2 There is no issue for Option 1-1 on coexistence with existing power saving techniques like PDCCH skipping and SSSG switching. While for Option 1-2, the specification impact should be studied further. Based on the discussion in the previous sections we propose the following: Proposal 1 Considering the need to minimize overhead and latency, LP-WUS duration used for connected mode should be limited to at most 2 slots. Proposal 2 OOK-based LP-WUS in connected mode should only be used for triggering PDCCH monitoring for a limited set of PDCCH CCE aggregation levels. For example, • PDCCH aggregation levels 1,2,4 for PDCCH 1Rx reception • PDCCH aggregation levels 1,2 for PDCCH 2Rx reception • PDCCH aggregation level 1 for PDCCH 4Rx reception Proposal 3 Only duty-cycled LP-WUS operation is supported in Rel-19. Proposal 4 For option 1-2, when the legacy DRX On duration timer is running, the UE should monitor LP-WUS and trigger any associated PDCCH monitoring. Proposal 5 For option 1-2, support configuration of a new timer (e.g., wus-onDurationTimer) that is triggered by LP-WUS. Proposal 6 From RAN1 perspective, when the UE is configured with Option 1-2, active time includes the time when the timer triggered by LP-WUS (i.e. wus-onDurationTimer) is running. Proposal 7 For Option 1-1 and Option 1-2, a time window could be configured which can contain multiple LP-WUS monitoring occasions. Proposal 8 For Option 1-1, the periodicity of LP-WUS monitoring is same as the C-DRX cycle length. Proposal 9 For Option 1-2, the periodicity of LP-WUS monitoring is separately configured (e.g. can be different from the C-DRX cycle length). Proposal 10 Support the following for UE capability reporting of minimum time-gap between LP-WUS and PDCCH monitoring. a. (Alt 1 from RAN1#118 agreement) For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. Proposal 11 For Option 1-1 and Option 1-2, wus-offset is defined as follows: • wus-offset for Option 1-1: an offset indicating a start of a time window where the UE starts monitoring LP-WUS, prior to a slot where the drx-onDurationTimer is to be started. • wus-offset for Option 1-2: an offset indicating a time where the UE starts monitoring PDCCH, after a slot where the UE detects LP-WUS. Proposal 12 LP-WUS monitoring is activated/deactivated via dedicated RRC signaling. Proposal 13 With CA (single DRX group), LP-WUS can be configured on the primary cell of the cell group and the detection of LP-WUS will trigger PDCCH monitoring on all the serving cells. Proposal 14 LP-WUS should share the same QCL source with DM-RS antenna port associated with PDCCH receptions in the CORESET. The QCL source for LP-WUS is same as that given by the TCI state of the CORESET where the PDCCH is monitored after receiving LP-WUS.

R1-2408860	Qualcomm Incorporated

Proposal 1: - For option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration - FFS periodicity and offset in relation to the C-DRX configuration - LP-WUS triggers the start of drx-onDurationTimer during which UE monitors PDCCH - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not Proposal 2: - For option 1-1 of LP-WUS CONNECTED mode operation, confirm legacy PDCCH skipping or SSSG switching can be configured simultaneously for further power saving. Proposal 3: - For option 1-1 of LP-WUS CONNECTED mode operation, discuss and clarify whether a UE is expected or not expected to monitor LP-WUS in DRX Active Time. Proposal 4: - RAN1 assumes following details of Option 1-2 in the RAN1#118 agreement, including the FFSs, to be discussed and concluded in RAN2: - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer Proposal 5: - For option 1-2 of LP-WUS CONNECTED mode operation, support joint use of LP-WUS and legacy PDCCH skipping or SSSG switching for further power saving. Proposal 6: - For option 1-2 of LP-WUS CONNECTED mode operation, discuss and clarify whether a UE is expected or not expected to monitor LP-WUS in DRX Active Time. Proposal 7: - Confirm that a UE can trigger recovery from beam failure or radio link failure even when the UE monitors LP-WUS and does not monitor PDCCH - Once the UE triggers the recovery, the UE autonomously activate PDCCH monitoring to receive the response to the recovery request - Additionally, allow a UE to autonomously fallback from LP-WUS monitoring to C-DRX operation when the channel/beam quality is not satisfactory for successful LP-WUS reception - FFS: whether/how to support UE autonomous fallback from LP-WUS monitoring to Rel-16 DCP based DRX adaptation Observation 1: - For Option 1-1, autonomous fallback from LP-WUS monitoring to PDCCH monitoring in C-DRX active time can be enabled without specific handling - It is beneficial to consider UE feedback information to let network know whether the UE is in LP-WUS coverage or not, and with which LP-WUS configuration the UE can receive LP-WUS in the present status - For Option 1-2, autonomous fallback from LP-WUS monitoring to PDCCH monitoring in C-DRX active time may need specific handling - The handling can be e.g., L1/L2 dynamic report of UE status (e.g., indicating whether the UE monitors LP-WUS or the UE fallback to PDCCH monitoring based on C-DRX configuration) Proposal 8: - Consider MAC-CE for LP-WUS activation/deactivation - A UE can be configured with one or multiple LP-WUS configurations - The MAC-CE activates one (or none) of the LP-WUS configurations dynamically Proposal 9: - Support at least the following: - Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - LP-WUS occasion at time t triggers PDCCH monitoring from time t + , where  is based on RRC configuration and/or certain rule(s) (and is not smaller than the reported value). Proposal 10: - On the serving cell that the UE monitors LP-WUS in connected mode, UE can be configured with LP-WUS for Option 1-1 and Option 1-2 simultaneously - Discuss following two alternatives: - Alt.1: MAC-CE activates only one LP-WUS configuration - Active LP-WUS configuration is for enabling either Option 1-1 or Option 1-2 - Alt.2: MAC-CE activates up to two LP-WUS configurations - One active LP-WUS configuration is for enabling Option 1-1 - Another LP-WUS configuration can additionally activated for enabling Option 1-2 Proposal 11: - RRC configuration for LP-WUS (LP-WUS configuration) includes TCI-State configuration - The UE identifies QCL assumption(s) of the LP-WUS reception in the same way as for CORESET/PDCCH - QCL assumption for the LP-WUS reception: - If a UE has not been provided a configuration of TCI state for the LP-WUS, the UE assumes the LP-WUS is QCLed with the SS/PBCH block that the UE identified during initial access procedure - If a UE has been provided a configuration of TCI state for the LP-WUS, the UE assumes the LP-WUS is QCLed with the one or more DL RS configured by the TCI state - QCL Type(s) for LP-WUS reception: - QCL Type(s) defined for CORESET/PDCCH is re-used for LP-WUS Proposal 12: - As baseline, LP-WUS on a serving cell is configured within active DL BWP of the serving cell - FFS: outside active DL BWP of the serving cell (as optional UE capability) Proposal 13: - For a UE configured with DC, support LP-WUS monitoring on a serving cell per cell-group - For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells - For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group - Opt.1: LP-WUS is configured to be monitored per DRX group - For dual DRX groups, LP-WUS monitoring is configured per DRX group - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

R1-2408921	TCL

Observation 1: When CA is configured for the UE, the LP-WUR may not know the CC which the gNB uses for LP-SS and LP-WUS transmissions. This uncertainty can increase power consumption and complicate the RF filter architecture. Proposal 1: Support Alt 1, i.e., UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. Proposal 2: gNB configures time offset(s) to determine a time window for the UE to monitor LP-WUS before the start of drx-onDurationTimer. Proposal 3: For option 1-2, the existing timer duration can be used for the additional active time triggered by LP-WUS. Proposal 4: The following fallback mechanism should be supported: - Deactivation of LP-WUS monitoring and resume PDCCH monitoring by timers. - Deactivation of LP-WUS monitoring and resume PDCCH monitoring by UL signaling including SR, BSR, PRACH, CG PUSCH. Proposal 5: In RRC connected state, the gNB can explicitly configure the CC for the UE where the gNB transmits the LP-SS and LP-WUS when CA is enabled. Proposal 6: LP-WUS can be configured together with Cell DTX.

RAN1 agreements

RAN1#116

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

    - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded.

Agreement

For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits

- FFS value X, which is no more than [8 or 16]

Agreement

For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following

- LP-WUS processing time

- MR transition time for ramp up

- Time/frequency synchronization of MR

- FFS whether UE can report supported minimum time gap from candidate values

FFS: Whether the minimum time gap values can be more than one

Agreement

For RRC CONNECTED mode, from RAN1 perspective,

- PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling

  - FFS whether to support UE assistance.

- LP-WUS monitoring by UE is known to gNB.

  - FFS whether implicit/explicit indication from UE is necessary

- In case LP-WUS monitoring is enabled, following options are further studied

  - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring

  - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

  - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer.

  - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

RAN1#116bis

Agreement

Update the following agreement in RAN1#116 in red:

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

        - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

          - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

        - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered.

- RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2

- Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

RAN1#117

Agreement

For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

- FFS: exact value(s) of the minimum time gap

- FFS: support of multiple minimum time gaps

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state

- FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS

- FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET

Agreement

LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode.

- FFS whether to define a time window for MOs

- It is at least supported that a UE can monitor MOs with a periodicity.

  - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured

Agreement

- Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode

FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Option 1: one or more serving cells based on gNB indication/configuration

Option 2: all activated serving cells

Note: other options are not precluded

Agreement

For RRC CONNECTED mode, LP-WUS can be configured without following existing features.

Rel-16 DCP

Rel-17 PDCCH skipping

Rel-17 SSSG switching

Rel-18 cell DTX

Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

RAN1#118

Agreement

- For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

UE is configured with legacy C-DRX configurations as Rel-18

UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

    - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

LP-WUS triggers the start of a timer during which UE monitors PDCCH

    - FFS the timer is existing timer and/or new timer

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

    - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

    - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

Working assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Agreement

Select one of the following alternatives in RAN1#118bis

Alt 1

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Alt2

For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Different minimum time gaps correspond to different sleep states

Companies are encouraged to details on how the reported values are to be used by the network

Agreement

LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode

- FFS: DC

Agreement

- For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS

FFS applicable QCL type(s)