3GPP TSG RAN WG1 #119                                                                                 R1-2410689

Orlando, US, November 18th – 22nd, 2024

Source:	Moderator(vivo)

Title:	Moderator summary#1 on NR-RAN measurement in NTN

Agenda Item:	8.1

Document for:	Discussion and Decision

- Introduction

This feature lead summary (FLS) document aims to collect and align on company views on maintenance issues on NR measurement in NTN, which contains a summary of the contribution and draft CR to specifications.

R1-2409706	Discussion on NG-RAN measurement in NTN	vivo

1st round discussion: please provide your feedback before Monday 11:00 am

- [open]1st round Discussion

- Companies’ contributions summary

[1] found several places in TS38.215 where the reference point of NR-RAN measurement for NTN is ambiguous and pointed out that it was also not clear which measurement is supported in NR NTN.

SAN for NR NTN

According to the specification in RAN4, the access node used to provide E-UTRA user plane and control plane protocol terminations towards NTN Satellite capable UE is called SAN, instead of BS. The related functions and requirements for SAN are defined in a separate specification, i.e., TS 38.108, while TS 38.104 addresses the functions and requirements for base stations under TN and ATG case only. The description of SAN types for NR NTN is cited below:

TS 38.108 Satellite Access Node: node providing NR user plane and control plane protocol terminations towards NTN satellite capable UE, and connected via the NG interface to the 5GC. It encompasses a transparent payload on board a NTN platform, with satellite-gateway and gNB functions. ====omitted==== - 4.3.1 SAN type 1-H For SAN type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements. Figure 4.3.1-1: Radiated and conducted reference points for SAN type 1-H Radiated characteristics are defined over the air (OTA), where the radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna. The transceiver unit array is part of the composite transceiver functionality receiving and transmitting modulated signal to ensure radio links with users. The satellite payload is composed by a transceiver unit array and a composite antenna array. The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array, in an implementation specific way. How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement clause. - 4.3.2 SAN type 1-O and SAN type 2-O For SAN type 1-O and SAN type 2-O, the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. Figure 4.3.2-1: Radiated reference points for SAN type 1-O and SAN type 2-O

Supported NR-RAN measurement for R18 NTN: gNB Rx-Tx time difference, UL SRS-RSRP and UL SRS-RSRPP

gNB Rx-Tx time difference is agreed to be supported for Network verified UE location for R18 NR NTN. In addition to the gNB Rx – Tx timing difference, 38.305 specifies that for network verification of UE location, NTN UL SRS-RSRP and UL SRS-RSRPP can be optionally used together with gNB Rx – Tx timing difference. Therefore, UL SRS-RSRP and UL SRS-RSRPP should be supported at least for R18 NTN.

TS 38.305 - 4.3.11 Multi-RTT positioning The Multi-RTT positioning method makes use of the UE Rx-Tx time difference measurements (and optionally DL-PRS-RSRP and/or DL-PRS-RSRPP and/or DL-RSCP) of downlink signals received from multiple TRPs, measured by the UE and the measured gNB Rx-Tx time difference measurements (and optionally UL-SRS-RSRP and/or UL-SRS-RSRPP and/or UL-RSCP) at multiple TRPs of uplink signals transmitted from UE. The UE measures the UE Rx-Tx time difference measurements (and optionally DL-PRS-RSRP and/or DL-PRS-RSRPP and/or DL-RSCP of the received signals) using assistance data received from the positioning server, and the TRPs measure the gNB Rx-Tx time difference measurements (and optionally UL-SRS-RSRP and/or UL-SRS-RSRPP and/or UL-RSCP of the received signals) using assistance data received from the positioning server. The measurements are used to determine the RTT at the positioning server which are used to estimate the location of the UE. For network verification of UE location in NTN, the Multi-RTT positioning method makes use of the UE Rx-Tx time difference measurements (and optionally DL-PRS-RSRP and/or DL-PRS-RSRPP) of downlink signals received from a single TRP at different time instances, measured by the UE and the measured gNB Rx-Tx time difference measurements (and optionally UL-SRS-RSRP and/or UL-SRS-RSRPP) at a single TRP at different time instances of uplink signals transmitted from UE. Together with each UE Rx-Tx time difference measurement, the UE also reports the UE Rx – Tx time difference subframe offset measurement in unit of subframe and the DL timing drift due to Doppler on service link between UE and satellite as defined in TS 38.215 [37]. The operation of the Multi-RTT positioning method is described in clause 8.10.

Issue#1 ambiguity for reference point for gNB Rx-Tx time difference, UL SRS-RSRP and UL SRS-RSRPP in R18 NTN

However, the definition of the reference point of gNB Rx-Tx time difference in TS 38.215 does not reference the proper TS 38.108 when the gNB Rx-Tx time difference measurement is conducted for NTN, only the reference point for TN has been clearly defined, as shown in the yellow-highlighted text below. Similar issues also exist for UL SRS-RSRP and UL SRS-RSRPP.

TS 38.105 - 5.2.3 gNB Rx – Tx time difference Definition; The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector. In NTN, the gNB Rx – Tx time difference at the uplink time synchronization reference point [5] is reported..

Issue#2 ambiguity for reference point for other NR-RAN measurement quantities

First, there is no clear RAN1 conclusion on whether other NR measurement (in addition to the above three) is supported for NTN; Second, if there is, the definition of the reference point of NR measurement in TS 38.215 does not reference TS 38.108 or mention SAN.

[1] summarizes the information of other NR-RAN measurement quantities for TN cases. According to the discussion in Positioning, Timing advance (TADV) is used for ECID positioning, and UL Relative Time of Arrival (TUL-RTOA) is utilized for UL-TDOA, both of which are currently unsupported in R18 NTN. RSCP is introduced in R18 and can be used in Multi-RTT positioning method for TN, but there is no clear description of using UL-RSCP for NTN, thus it appears to be unsupported in R17/18 NTN.

It remains unclear whether UL SRS-RSRP and UL SRS-RSRPP are already supported in R17 NTN

Table 1 [1] supported/unsupported information for each metric

|  | Release introduced for TN | Reference to 38.104 for reference point | Whether supported in Rel-17 NTN | Whether supported in Rel-18 NTN |
| --- | --- | --- | --- | --- |
| SSS transmit power | Rel-15 | No | Should be supported | Should be supported |
| UL Relative Time of Arrival (TUL-RTOA) | Rel-16 | Yes | Not supported | Not supported |
| UL Angle of Arrival (UL AoA) | Rel-16 | No | Not supported | Not supported |
| UL SRS reference signal received power (UL SRS-RSRP) | Rel-16 | Yes | unclear | Supported |
| UL SRS reference signal received path power (UL SRS-RSRPP) | Rel-17 | Yes | unclear | Supported |
| Timing advance (TADV) | Rel-17 | Yes | Not supported | Not supported |
| UL reference signal carrier phase (UL RSCP) | Rel-18 | Yes | / | Not supported |

- Collection of Companies’ views

According to the analysis above, companies are invited to provide their views on the following questions:

Q1: Do you agree that the reference point for gNB Rx-Tx time difference, UL SRS-RSRP and UL SRS-RSRPP in 38.215 is not clearly defined for NR NTN?

Companies are invited to provide their views on the questions in the following Table.

| Company | (Y/N) | comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y |  |
| Ericsson | Y |  |
| DCM | Y |  |
| Nokia | Y with comments | Since this measurement is related to NTN, we should observe the following text from 38.215: “In NTN, the gNB Rx – Tx time difference at the uplink time synchronization reference point [5] is reported.”. This means that the gNB is anyway not really reporting something that is directly associated to a measurement point, and hence it would not be beneficial to define the reference point for the measurements, as the gNB is anyway doing manipulation of the measurement results. Mod1: thanks Nokia for comments, my understanding is that measument point(reference point) is benefical even if gNB is doing manipulation of the measurement results. It ensures a consistent baseline across implementations by different vendors and reduce ambiguity in interoperation |

Q2: If Q1 is yes, do you agree with the changes proposed in Draft CR#1 in section 4.1 for gNB Rx-Tx time difference for R18 NR NTN?

Companies are invited to provide their views on the questions in the following Table.

| Company | (Y/N) | comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | We are fine with draft CR#1. |
| Ericsson | Y |  |
| DCM | Y | Alternatively, we can consider to add general texts to describe the difference between TN and NTN for any measurements, so that we can avoid similar discussion/correction in future. |
| Nokia | N | In our understanding it is not useful to have this potential clacification. |

Q3: If Q1 is yes, do you agree with the changes proposed in Draft CR#2 in section 4.2 or Draft CR#3 in section 4.3 (except the cover sheet and release) for UL SRS-RSRP and UL SRS-RSRPP? Which release do you think is the first release to support UL SRS-RSRP and UL SRS-RSRPP for NR NTN?

(Note Draft CR#2 is considered if both quantities are supported since R17, Draft CR#3 is considered if both are supported since R18)

Companies are invited to provide their views on the questions in the following Table.

| Company | (Y/N) | (R17 or R18?) | comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon |  | R18 | Only draft CR#3 is agreeable. |
| Ericsson | Y |  | R18 might be enough. Discuss further. |
| DCM |  | R18 |  |
| Nokia | N |  | In our understanding it is not useful to have this potential clacification. If this is agreed anyway, the earliest release this should be related to would be Rel-18. |

Q4: Do you think there are any other NR-RAN measurement quantities supported for NR NTN and therefore clarifications on the reference points are also needed?

Companies are invited to provide their views on the questions in the following Table.

| Company | (Y/N) | comments |
| --- | --- | --- |
| Ericsson | maybe | UL AoA was agreed as a means (up to gNB implementation) to resolve the mirror position ambiguity for NW verified UE location in Rel-18. From RAN1#114: Conclusion To resolve the mirror positions ambiguity for multi-RTT positioning, the following methods can be used without RAN1 specification impact from RAN1 perspective: - by gNB or LMF implementation - existing ECID method - UL-AoA |
| Nokia | No |  |

- Proposal for Monday online discussion

For gNB Rx – Tx time difference, UL SRS-RSRP and UL SRS-RSRP

Proposal 1: Adopt draft CR#1 for gNB Rx – Tx time difference in section 4.1 for 38.215

Proposal 2: Adopt draft CR#3 for UL SRS-RSRP and UL SRS-RSRP in section 4.3 for 38.215

For UL AoA

According to Ercisson’s comment on UL AoA, it is observed that there is no description of how UL AoA is measured by SAN for NTN(text for UL AoA from 38.215 is cited below), therefore the following proposal is made:

Proposal 3: Clarify whether to support UL-AoA for R18 NTN and whether to update the 38.215 for how UL-AoA is measured by SAN for R18 NTN.

TS 38.215

| Definition | UL Angle of Arrival (UL AoA) is defined as the estimated azimuth angle (A-AoA) and vertical angle (Z-AoA) of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15]. The UL-AoA is determined at the gNB antenna for an UL channel corresponding to this UE. |
| --- | --- |

Conclusion(RAN1#114)

To resolve the mirror positions ambiguity for multi-RTT positioning, the following methods can be used without RAN1 specification impact from RAN1 perspective:

- by gNB or LMF implementation

- existing ECID method

- UL-AoA

Proposal 4: Send a LS to RAN4 to check if there is any issue about the reference point definitions of NR-RAN measurement in NTN.

- Draft CR

  - Draft CR#1 for gNB Rx – Tx time difference in R18 NTN

3GPP TSG-RAN1 Meeting #119	R1-240XXXX

Orlando, US, November 18th – 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR |  | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to reference point for gNB Rx – Tx time difference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Requirements for SAN providing NR user plane and control plane protocol terminations towards NTN satellite capable UE is defined in 38.108. In gNB Rx – Tx time difference definition in 38.215, the reference point for conducted and radiated requirement for SAN in NTN case is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of reference point for gNB Rx – Tx time difference in NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference point for gNB Rx – Tx time difference in NTN is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** unchanged text omitted ***

2	References

[17]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access"

[18]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN"

[19]	3GPP TS 38.108: " Satellite Access Node radio transmission and reception"

*** unchanged text omitted ***

5.2.3	gNB Rx – Tx time difference

| Definition | The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. - for type 1-O or 2-O Satellite access node TS 38.108 [19]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H Satellite access node TS 38.108 [19]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector. - for type 1-O or 2-O Satellite access node TS 38.108 [19]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H Satellite access node TS 38.108 [19]: the Tx Transceiver Array Boundary connector. In NTN, the gNB Rx – Tx time difference at the uplink time synchronization reference point [5] is reported. |
| --- | --- |

*** unchanged text omitted ***

  - Draft CR#2 for UL SRS-RSRP and UL SRS-RSRP in R17 NTN

3GPP TSG-RAN1 Meeting #119	R1-240XXXX

Orlando, US, November 18th – 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR |  | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to reference point for NG-RAN measurement for UL SRS-RSRP, and UL SRS-RSRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2024-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Requirements for SAN providing NR user plane and control plane protocol terminations towards NTN satellite capable UE is defined in 38.108. For the definition of UL SRS-RSRP, and UL SRS-RSRP in 38.215, the reference point for conducted and radiated requirement for SAN in NTN case is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of reference point for UL SRS-RSRP, and UL SRS-RSRP in NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference point for UL SRS-RSRP, and UL SRS-RSRP in NTN is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.5, 5.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** unchanged text omitted ***

2	References

[17]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access"

[18]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN"

[19]	3GPP TS 38.108: " Satellite Access Node radio transmission and reception"

*** unchanged text omitted ***

5.2.5	UL SRS reference signal received power (UL SRS-RSRP)

| Definition | UL SRS reference signal received power (UL SRS-RSRP) is defined as linear average of the power contributions (in [W]) of the resource elements carrying sounding reference signals (SRS). UL SRS-RSRP shall be measured over the configured resource elements within the considered measurement frequency bandwidth in the configured measurement time occasions. The reference point for UL SRS-RSRP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. - for type 1-O or 2-O Satellite access node TS 38.108 [19]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H Satellite access node TS 38.108 [19]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS-RSRP of any of the individual receiver branches. |
| --- | --- |

*** unchanged text omitted ***

5.2.6	UL SRS reference signal received path power (UL SRS-RSRPP)

| Definition | UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry the received UL SRS signal configured for the measurement, where UL SRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. - for type 1-O or 2-O Satellite access node TS 38.108 [19]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H Satellite access node TS 38.108 [19]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path. |
| --- | --- |

*** unchanged text omitted ***

  - Draft CR#3 for UL SRS-RSRP and UL SRS-RSRP in R18 NTN

3GPP TSG-RAN1 Meeting #119	R1-240XXXX

Orlando, US, November 18th – 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR |  | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to reference point for NG-RAN measurement for UL SRS-RSRP, and UL SRS-RSRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Requirements for SAN providing NR user plane and control plane protocol terminations towards NTN satellite capable UE is defined in 38.108. For the definition of UL SRS-RSRP, and UL SRS-RSRP in 38.215, the reference point for conducted and radiated requirement for SAN in NTN case is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of reference point for UL SRS-RSRP, and UL SRS-RSRP in NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference point for UL SRS-RSRP, and UL SRS-RSRP in NTN is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.5, 5.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** unchanged text omitted ***

2	References

[17]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access"

[18]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN"

[19]	3GPP TS 38.108: " Satellite Access Node radio transmission and reception"

*** unchanged text omitted ***

5.2.5	UL SRS reference signal received power (UL SRS-RSRP)

| Definition | UL SRS reference signal received power (UL SRS-RSRP) is defined as linear average of the power contributions (in [W]) of the resource elements carrying sounding reference signals (SRS). UL SRS-RSRP shall be measured over the configured resource elements within the considered measurement frequency bandwidth in the configured measurement time occasions. The reference point for UL SRS-RSRP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. - for type 1-O or 2-O Satellite access node TS 38.108 [19]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H Satellite access node TS 38.108 [19]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS-RSRP of any of the individual receiver branches. |
| --- | --- |

*** unchanged text omitted ***

5.2.6	UL SRS reference signal received path power (UL SRS-RSRPP)

| Definition | UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry the received UL SRS signal configured for the measurement, where UL SRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. - for type 1-O or 2-O Satellite access node TS 38.108 [19]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H Satellite access node TS 38.108 [19]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path. |
| --- | --- |

*** unchanged text omitted ***

- Reference

[1] R1-2409706	Discussion on NG-RAN measurement in NTN	vivo