3GPP TSG RAN WG1 #119                                                                                R1-2410708

Orlando, US, November 18th – 22nd, 2024

Title:	Reply LS on CBR range

Response to:	R1-2409345(R2-2409258)

Release:	Rel-18

Work Items:	NR_pos_enh2-Core

Source:	RAN1

To:	RAN2

CC:

Contact Person:

#### Name:	Ren Da

E-mail Address:	renda@catt.cn

Attachment:

1. Overall Description:

RAN1 thanks RAN2 for the LS on CBR range, which identified that the maximum number of CBR ranges for dedicated SL PRS resource pool, the maximum number of CBR levels for dedicated SL PRS resource pool and the maximum number of SL PRS transmission Configuration index in sl-PRS-TxConfigIndexList-r18 in RRC spec are not fully aligned with the RAN1’s parameter list. In the LS, RAN2 asks the following question:

- RAN2 would like to ask RAN1 if it is acceptable that CBR range, CBR level and SL-PRS-TxConfigIndexList are not extended as RAN2 agreed.

RAN1 has discussed the above question and reached the following agreement in RAN1#119:

Agreement Provide the following response to RAN2’s LS on CBR: - RAN1 prefers that the maximum values for the following parameters in ASN.1 are specified as proposed by RAN1, i.e., - The maximum number of CBR ranges for SL positioning is set to 8 in RRC IE sl-CBR-RangeDedicatedSL-PRS-RP-List-r18, - The maximum number of CBR levels is set to 16 in RRC IE SL-CBR-LevelsDedicatedSL-PRS-RP, - The maximum number of SL PRS transmission Configuration index in sl-PRS-TxConfigIndexList-r18 is set to 16. - However, although it is not preferred, it is acceptable that the CBR range, CBR level and SL-PRS-TxConfigIndexList parameters are not extended, as RAN2 agreed. - RAN1 respectfully requests RAN2 to identify whether there exists any other ASN.1 issue, if the CBR range, CBR level and SL-PRS-TxConfigIndexList parameters are not extended. - E.g. SL-CBR-LevelsConfig-Dedicated-SL-PRS-RP is used but not defined, The maxNrofSL-PRS-TxConfig-r18 should be 63 instead of 64 - E.g. the index range used in SL-PriorityTxConfigIndexDedicatedSL-PRS-RP should not exceed the range defined in SL-CBR-CommonTxDedicatedSL-PRS-RP-List

2. Actions:

To RAN WG2

ACTION: RAN1 respectfully requests RAN2 to take the above response into account in future work.

3. Date of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #120		         17 – 21 Feb 2025	         			Athens, Greece

TSG RAN WG1 Meeting #120bis	          07 – 11 Apr 2025	         			TBD, China