3GPP TSG-RAN WG1 Meeting #119	R1-2409409

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.11.5

Source:	Huawei, HiSilicon

Title:	Discussion on IoT-NTN TDD mode

Document for:	Discussion and Decision

Introduction

In RAN#105, a new work item on Non-Terrestrial Networks (NTN) was approved to introduce a new NB- IoT TDD mode only for 1616-1626.5 MHz MSS band [1]. In RAN1#118bis, some agreements were achieved about the TDD pattern and the corresponding evaluation assumptions [2]. In this contribution, we continue to discuss the candidate TDD pattern and provide the DL synchronization and NPBCH performance.

Discussion on TDD pattern period and DL/UL configuration

In RAN1#118bis, RAN1 discussed the TDD pattern period and the number of consecutive DL/UL subframe, the following agreements were achieved.

Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes): - N = 9: - D = 8 - D = 20 - D = 30 - N = 8: - D = 20 - Companies to report which subframe indices are included in the set of consecutive downlink subframes. - Other combinations are not precluded to be reported by companies NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase. Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes): - N = 9: - U = 8 - U = 20 - U = 30 - N = 8: - U = 20 - Companies to report which subframe indices are included in the set of consecutive uplink subframes. - Other combinations are not precluded to be reported by companies - For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

For the TDD pattern period, we think that the objective of the Work Item is to adapt the current NB-IoT FDD specifications to the deployment on the existing in-orbit satellites in the 1616-1626.5 MHz band and coexists with their operating system at least in FDM manner. The Iridium TDMA frame structure is shown in Figure 1. It can be observed that the TDD frame structure is defined with the periodicity of 90ms. In order to avoid cross link interference and share RF switcher with Iridium system, the DL transmission and UL reception of new IoT NTN TDD system should aligned with Iridium system. By setting N=9 as new TDD pattern period and using the appropriate timing offset for UL and DL transmission, we can align the new TDD pattern with the Iridium TDMA structure. That is why N=9 as baseline in the WID.

For TDD pattern with N=8, it cannot be used in the targeted spectrum of 1616-1626.5 MHz band in the WID due to the coexisting issue, although the standard impacts on existing NB IoT specification is smaller.  Because of 10ms shifting occurring every 80ms with the Iridium TDMA structure, the collision will occur consecutively for multiple periods for the same radio frame. For example, as shown in Figure 2, the first DL radio frame in 80ms TDD period will collide with UL transmission of Iridium TDMA structure for 3 TDD periods. After that, the first DL radio frame in 80ms TDD period will collide with simplex time slot of Iridium TDMA structure for 3 TDD periods, while the simplex time slot will be used as other purpose which will not be used for NB-IoT. As a result, the first DL radio frame with a periodicity of 80ms will be invalid for 6 periods. It’s impossible to design a system that will work properly with such constraint. Therefore, we propose that RAN1 should focus on N=9 for IoT NTN TDD in Rel-19.

Figure 1 Iridium TDMA structure

Figure 2 Coexistence of existing Iridium system and 3GPP new TDD mode with N=8

Observation 1: For a TDD pattern with N=8, any radio frame in the TDD pattern will eventually have a collision that will result in dropping that radio frame, because of 10 ms shifting occurring every 80 ms with the Iridium frame. It’s impossible to design a system that will work properly with N=8 to coexist with the Iridium satellite system.

Proposal 1：RAN1 should focus on a TDD pattern with N=9 when investigating and designing IoT NTN TDD system in Rel-19.

For the GP subframe configuration, in LEO600 and LEO1200 scenarios assumed in the WID, the maximum RTT time of service link (between satellite and UE) can be as long as 12.89ms and 20.89ms respectively, at elevation of 10deg. The GP reserved between contiguous DL subframes and contiguous UL subframes should be at least 13 subframes for LEO600 and 21 subframes for LEO1200. Furthermore, the value of N and the configuration of the periodic pattern are fixed per band according to the WID. Thus, the gap of a TDD pattern should be at least 21 subframes if the same pattern is to be used in either LEO600 or LEO1200 scenarios. It should be noted that the guard period (i.e., one subframe) for type B half-duplex FDD operation (UE is assumed to operate with type B HD FDD procedure) defined in clause 6.2.5 of TS36.211 is not necessarily covered by the gap between DL and UL as it can be avoided by scheduling reception/transmission of other UE.

Observation 2: The gap from the set of contiguous DL subframes to the set of contiguous UL subframes in a TDD pattern should be at least 21 subframes to accommodate the TX/RX switching at eNB and RTT of service link in LEO600/1200.

For the consecutive DL subframes configuration, in legacy NB IoT system operating in FDD band, some basic DL signals/information will be transmitted periodically, such as NPSS, NSSS, MIB-NB or SIB1-NB. An example is shown in Figure 3. The NPSS is transmitted in subframe 5 of every radio frame. The NSSS is transmitted in subframe 9 in the even radio frames. The MIB-NB is transmitted in subframe 0 of every radio frame at a period of 64 radio frames. The SIB1-NB is transmitted at a period of 256 radio frames with 4, 8 or 16 repetitions. The SIB1-NB repetition is transmitted in subframe 4 in the even radio frames of 16 radio frames. Therefore, the basic signals/information will occupy up to 4 subframes in many radio frames. In addition, some other basic SIB-NB information such as SIB2-NB and SIB31-NB will occupy other subframes. If the number of available DL subframes within one TDD pattern period is 8, the remained DL subframes in a TDD pattern is not enough to schedule NPDCCH and NPDSCH, especially considering that some IoT devices are in an environment with poor coverage and need to transmit with multiple repetitions. In IoT NTN scenario, UE usually combines multiple transmission to get the better performance. If the number of consecutive DL subframes in a TDD pattern is 8, NPSS, NSSS, MIB-NB and SIB1-NB can be transmitted at most once in a TDD pattern period. The synchronization delay and reception delay of broadcast information is prolonged.

Figure 3 one radio frame configuration

Observation 3: Considering the consecutive DL subframes should contain the basic DL signals and information (NPSS, NSSS, MIB-NB and SIB1-NB), D=8 is not enough to schedule and transmit DL data.

One of the reasons from proponent companies proposing to design the TDD pattern with only 8 DL subframes per TDD pattern is to coexist with the iridium system, in which the duration of a DL slot 8.28ms as shown in figure 1. However, we also observed that there are 4 consecutive DL slots (around 32ms) and 4 consecutive UL slots in iridium TDMA frame structure. From our perspective, there will not be coexistence issue as long as the total duration of DL transmission (UL reception) of IoT NTN TDD system does not exceed the total duration of consecutive DL(UL) slots. If the small gaps between consecutive DL(UL) slots cannot be used (although we are not clear why it cannot be used), the OFDM symbol overlap with these gaps can be reserved.

Observation 4: There is no coexistence issue if the total duration of consecutive DL/UL subframes in a IoT NTN TDD pattern does not exceed the total duration of DL/UL slots in a Iridium TDMA frame structure.

Based on the discussion above, we have following proposal.

Proposal 2: At least the IoT NTN TDD pattern with D=30 DL consecutive subframes and U=30 consecutive UL subframes per N=9 radio frames should be supported.

Impact on the downlink synchronization and NPBCH decoding

In RAN1#118bis, RAN1 discussed the channels that need to be evaluated and the corresponding simulation assumptions are agreed, the following agreements were achieved.

Agreement For the study phase, RAN1 assumes as a baseline for evaluations (which includes DL synchronization as per the WID) that downlink PHY channels and signals are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD. - For the purpose of study, the DL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and signals and eNB will not transmit the corresponding instances. - FFS: What is the TDD pattern Agreement For the study phase, RAN1 assumes as a baseline for evaluations that uplink PHY channels are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD. - For the purpose of study, the UL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and eNB will not receive the corresponding instances. - FFS: What is the TDD pattern Agreement RAN1 evaluates via link level simulations the following: - NPSS/NSSS detection - NPBCH decoding Agreement For link level simulations of NPSS / NSSS / NPBCH, RAN1 uses the following assumptions: Parameter; Value. Carrier frequency; 1.6GHz. Channel model; NTN TDL-C rural, 30 degrees. Frequency error / timing drift (including XO error); 34 ppm for NPSS / NSSS (24ppm Doppler + 10ppm XO error) 0.1ppm for NPBCH. Variation of frequency error / variation of timing drift; 0.27ppm/s for NPSS / NSSS 0 for NPBCH. UE velocity; 3km/h. Target performance; For NPSS/NSSS: - 99% detection probability with 0.1% false alarm rate - FFS: How to define correct detection, including successful timing / frequency estimation / cell ID detection. For NPBCH: - 1% BLER For the channels / signals above, acquisition delay shall be reported for the corresponding detection probability. Other assumptions (e.g. combining length, frequency error hypothesis, etc.); To be reported and justified by companies.

In legacy NB IoT system operating in FDD band, the NPSS is transmitted from the 4th OFDM symbol to 14th OFDM symbol in subframe 5 of every radio frame. If the new TDD pattern is introduced, for the case that D = 8, 20 or 30, UE can combine the NPSS with 1, 2 or 3 times transmission in a period of new TDD pattern. So, we simulate the NPSS detection of combinations 1, 2 and 3 with a period of 10ms. In the NPSS detection, we using a 2-step method in [3]: auto-correlation coarse timing detection at a sampling rate of 240kHz, followed by a cross-correlation refined timing detection at a sampling rate of 1.92MHz. The detection is considered as successful if the NPSS detection timing error is within +/- 4 samples at 1.92MHz sampling rate, i.e., +/- 2.08us. The NPSS detection performance is shown in Figure 4.

Figure 4 NPSS detection performance

In RAN1#118bis meeting, RAN1 agreed the CNR for LEO-600 and LEO-1200 as follows.

Agreement The target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz: - For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain) - For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain) - When reporting link level simulation results, companies are encouraged to report the CNR margin with respect to the above operating DL CNRs. - Companies may report values for other scenarios (e.g. UE antenna gain of -5.5 dBi, LEO-800) with the corresponding assumptions.

The required SNR and corresponding coverage margin are listed in Table 1. It can be observed that the coverage margin is enough even if UE just detects 1 NPSS to get the DL synchronization under the simulation assumption agreed in the WI.

Table 1 CNR and Coverage margin of NPSS

| NPSS combination | SNR @ 1% BLER | Coverage Margin |  |
| --- | --- | --- | --- |
|  |  | LEO600, Set-1 | LEO1200, Set-1 |
| 1NPSS | 2.6 | 2.31 | 2.91 |
| 2NPSS | -0.8 | 5.71 | 6.31 |
| 3NPSS | -2.4 | 7.31 | 7.91 |

Observation 5: For Set-1 LEO600 and LEO1200 satellite scenario, the single shot NPSS detection can meet the link budget requirement (assume 0dBi antenna gain). The NPSS combination within the same TDD pattern can improve the coverage margin of NPSS.

For some IoT device with lower antenna gain (e.g. -5.5dBi), UE requires to combine at least 2 NPSS. If there are only 8 DL subframes in a TDD pattern with 90ms periodicity, the timing difference between 2 NPSS with 90ms interval is 3.06us due to the assumed timing drift. It’s hard for UE to obtain the correct timing within +/- 2.08us.

Observation 6: If there are only 8 DL subframes per 90ms TDD pattern, UE cannot meet the required CNR of NPSS detection with single shot detection, assuming -5.5dBi antenna gain. It is difficult to combine NPSS across 90ms TDD pattern because the timing drift between 2 NPSSs is 3.06us, exceeding the requirement of timing error +/-2.08us.

In legacy NB IoT system operating in FDD band, the MIB-NB is transmitted in NPBCH channel. The encoded MIB-NB will be divided into 8 sub-blocks, each sub-block will be repeated by 8 times and spanned in 8 continuous radio frames with each repetition transmitted in the fixed subframe 0 in each radio frame. The 8 sub-blocks of MIB-NB spans in 64 continuous radio frames, i. e. 640ms periodicity. If the new TDD pattern is introduced, many MIB-NB sub-blocks and repetitions of sub-blocks are skipped when they are in the GP subframes or UL subframes. For example, if the number of consecutive downlink subframes is D = 8, only 7 sub-blocks of MIB-NB can be transmitted within 640ms TTI, and each sub-block is transmitted only once. If the number of consecutive downlink subframes is increased to D = 30, each sub-block can be transmitted by three times.

The simulation results are shown in Table 2, Figure 5 and Figure 6. It can be observed the 1% BLER of NPBCH can be achieved by single shot detection (one repetition of one subblock, green row) at the operating CNR of 4.91dB (assuming antenna gain of 0 dBi) for LEO600 set-1satellite. However, the margin is not sufficient for UE with antenna gain of -5.5dBi. If there are only 8 consecutive DL subframes per TDD pattern, UE need to wait at least 180ms to combine 3 subblocks across 3 TDD patterns (yellow row). If there are 30 DL subframes per TDD pattern, the latency of PBCH decoding can be reduced to around 90ms to combine repetitions of 2 subblocks across 2 TDD patterns (orange row).

Table 2 Coverage margin of NPBCH

| Repetition of each subblock | Number of subblock | SNR @ %1 BLER | Coverage Margin |  |
| --- | --- | --- | --- | --- |
|  |  |  | LEO600, Set-1 | LEO1200, Set-1 |
| 1 | 1 | 4.9 | 0.01 | 0.61 |
|  | 3 | -0.6 | 5.51 | 6.11 |
|  | 5 | -2.7 | 7.61 | 8.21 |
|  | 7 | -4 | 8.91 | 9.51 |
| 3 | 1 | 1.4 | 3.51 | 4.11 |
|  | 2 | -2.4 | 7.31 | 7.91 |
|  | 3 | -3.8 | 8.71 | 9.31 |
|  | 5 | -6 | 10.91 | 11.51 |
|  | 7 | -7.1 | 12.01 | 12.61 |

Figure 5 NPBCH 1 repetition decoding performance

Figure 6 NPBCH 3 repetitions decoding performance

Observation 7: For Set-1 LEO600 and LEO1200 satellite scenario, the single shot NPBCH detection can meet the link budget requirement assuming 0dBi antenna gain. If antenna gain of -5.5dBi is assumed, the decoding latency of NPBCH in a TDD pattern with D=30 subframes is only half of that in a TDD pattern with D=8 subframes.

Other impacts due to the introduction of TDD pattern

In RAN1#118bis, RAN1 discussed the impact of the TDD pattern on some other channels, the following agreements were achieved.

Agreement The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.): - SIB1-NB - Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31) - Paging - NPRACH / RAR - NPSS/NSSS/NPBCH - NPDCCH, NPUSCH, NPDSCH - Note: Study for the above channels / signals may continue during the normative phase. - It is up to companies to perform link level evaluations for some of the aspects above

As for the configuration of NPRACH resource, in current FDD mode configuration, the periodicity NperiodNPRACHof NPRACH resource for FDD is 40ms, 80ms, 160ms, 240m, …, 2560ms, and the start time NstartNPRACH within a NPRACH periodicity is 8ms, 16ms, 32ms, …,1024ms. NPRACH transmission can start only NstartNPRACH subframes after the start of a radio frame fulfilling nf mod NperiodNPRACH10=0. UE will continuously transmit NPRACH preambles for NrepNPRACH times except for the 40ms gap after every 64 repetitions of NPRACH format 0/1 or after every 16 repetitions for NPRACH format 2. The repetitions of NPRACH transmission are dropped when it collides with subframes for GP or DL. The coverage of NPRACH will be significantly affected due to the lengthy contiguous GP and DL resources.

As for NB-IoT operating in FDD band, the default paging cycle is among {rf128, rf256, rf512, rf1024}. If the paging cycle is configured as rf128, the paging message is transmitted every 128 radio frames in the radio frame satisfying nf mod 128=0, where nf is the radio frame index. For example, in TDD pattern D = 8, the radio frame that satisfying nf mod 9=0 will be the DL frame. If eNB skips the paging transmission when the corresponding frame is for GP or UL, only the radio frame index 0 can be the paging frame. The same issue will happen to the other paging cycle configurations and other TDD patterns. The paging resources are reduced and the paging cycle configuration cannot work. Therefore, the existing pre-configured resources for paging should be studied.

As for the SIB1-NB transmission of NB-IoT operating in FDD band, SIB1-NB is transmitted every other radio frame within 16 continuous radio frames and the maximum repetition number is 16. Therefore, the periodicity of SIB1-NB is 2560ms. Similarly, if the existing SIB1-NB transmission pattern is not changed but only skip the transmission on the subframes for GP and UL, SIB1-NB cannot be decoded because it is not completely transmitted in each repetition.

Observation 8: The performance of NPRACH, Paging and SIB1-NB will be impacted due to reduced number of available DL and UL subframes in the TDD pattern, compared with the legacy FDD frame structure.

Conclusions

The observations and proposals made in this contribution are summarized below:

Observation 1: For a TDD pattern with N=8, any radio frame in the TDD pattern will eventually have a collision that will result in dropping that radio frame, because of 10 ms shifting occurring every 80 ms with the Iridium frame. It’s impossible to design a system that will work properly with N=8 to coexist with the Iridium satellite system.

Proposal 1：RAN1 should focus on a TDD pattern with N=9 when investigating and designing IoT NTN TDD system in Rel-19.

Observation 2: The gap from the set of contiguous DL subframes to the set of contiguous UL subframes in a TDD pattern should be at least 21 subframes to accommodate the TX/RX switching at eNB and RTT of service link in LEO600/1200.

Observation 3: Considering the consecutive DL subframes should contain the basic DL signals and information (NPSS, NSSS, MIB-NB and SIB1-NB), D=8 is not enough to schedule and transmit DL data.

Observation 4: There is no coexistence issue if the total duration of consecutive DL/UL subframes in a IoT NTN TDD pattern does not exceed the total duration of DL/UL slots in a Iridium TDMA frame structure.

Proposal 2: At least the IoT NTN TDD pattern with D=30 DL consecutive subframes and U=30 consecutive UL subframes per N=9 radio frames should be supported.

Observation 5: For Set-1 LEO600 and LEO1200 satellite scenario, the single shot NPSS detection can meet the link budget requirement (assume 0dBi antenna gain). The NPSS combination within the same TDD pattern can improve the coverage margin of NPSS.

Observation 6: If there are only 8 DL subframes per 90ms TDD pattern, UE cannot meet the required CNR of NPSS detection with single shot detection, assuming -5.5dBi antenna gain. It is difficult to combine NPSS across 90ms TDD pattern because the timing drift between 2 NPSSs is 3.06us, exceeding the requirement of timing error +/-2.08us.

Observation 7: For Set-1 LEO600 and LEO1200 satellite scenario, the single shot NPBCH detection can meet the link budget requirement assuming 0dBi antenna gain. If antenna gain of -5.5dBi is assumed, the decoding latency of NPBCH in a TDD pattern with D=30 subframes is only half of that in a TDD pattern with D=8 subframes.

Observation 8: The performance of NPRACH, Paging and SIB1-NB will be impacted due to reduced number of available DL and UL subframes in the TDD pattern, compared with the legacy FDD frame structure.

- References

[1] RP-242415, New WID on introduction of IoT-NTN TDD mode.

[2] RP-242411, Session notes for 9.11 Non-Terrestrial Networks for NR Phase 3, Internet of Things Phase 3, and IoT-NTN TDD mode. Ad-Hoc Chair (Huawei)

[3] R1-161981, NB-PSS and NB-SSS Design, Qualcomm Incorporated