3GPP TSG-RAN WG1 Meeting #119	R1-2409433

Orlando, FL, USA, November 18th – 22nd, 2024

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on RAN4 LS on SSB relation in NES

Document for:	Discussion, Decision

# 1	Introduction

In [1], RAN4 asked RAN1 to answer the following questions:

Obj. 1 related to on-demand SSB SCell operation
Based on the discussion, RAN4 identified that the relation between always-on SSB and on-demand SSB (i.e. Case#2 as agreed in RAN1) has impacts on UE’s behavior and requirements in SCell. RAN4 would like RAN1 to clarify at least the following aspects:

- Q1: What is the relation in terms of periodicity between always-on SSB and OD-SSB?

- Q2: What is the relation in terms of time location between always-on SSB and OD-SSB?

- Q3: What is the relation in terms of frequency location between the always-on SSB and OD-SSB?

- Q4: What is the spatial relation between the always-on SSB and OD-SSB?

Obj.3 related to SSB adaptation in time domain
For the SSB adaption in the objective of adaptation of common signal/channel transmits, RAN4 would also like RAN1 to clarify the relation for SSB adaptation at least for following aspects.

- Q1: What is the relation in terms of time location before and after SSB adaptation?

- Q2: What is the relation in terms of frequency location before and after SSB adaptation?

- Q3: What is the spatial relation before and after SSB adaptation?

In this contribution, we discuss the above questions and suggest answers.

# 2	Discussion

## 2.1	Objective 1: On-demand SSB

The above questions concerns multiplexing of on-demand SSB and always-on SSB in an SCell, a topic that is currently being discussed in RAN1, as per the following agreement from RAN1#118bis.

Agreement
For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases.

- Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB

- Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

At this point, RAN1 has not agreed on the multiplexing issue. Hence, the answers to RAN4’s questions are pending agreements in RAN1. With that said, in the below, we provide our views and preferred answers.

In our companion contribution [2], frequency multiplexing of on-demand and always-on SSB is discussed. It was observed that:

If always-on SSB is located off sync raster, it is preferred that on-demand SSB and always-on SSB are provided on the same frequency.

If always-on SSB is located on sync raster, it is preferred that on-demand SSB and always-on SSB are provided on different frequencies.

Therefore, we think that on-demand SSB and always-on SSB can be provided on the same frequency or on different frequencies. For further discussion on time multiplexing of on-demand and always-on SSB, and spatial relations between other signals/channels and SSB, see [2].

On Question 1, we think that on-demand SSB can be provided with an SSB periodicity that equals any of existing SSB periodicities, i.e., one of {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, and 160 ms}.

A typical use case is that on-demand SSB is provided with periodicity that is less than periodicity of always-on SSB (e.g., to speed up SCell activation). Indeed, if always-on SSB is already present in the SCell, providing on-demand SSB with longer periodicity provides no apparent advantage.

For SCell activation, on-demand SSB would typically be provided with periodicity that is less than periodicity of always-on SSB.

However, there is no reason to restrict periodicity as in Observation 3. For example, if on-demand SSB and always-on SSB are provided on different frequencies on a carrier, it could be the case that one UE on said carrier is configured with both on-demand SSB and always-on SSB, while another UE on the same carrier is configured with only on-demand SSB. In this case, restricting periodicity of the on-demand SSB based on periodicity of always-on SSB would impact two UE’s behavior, but only of the UEs would see the always-on SSB and, thus, may be adversely affected by such restriction.

Our preferred replies for the questions related to Objective 1 are proposed in the following. We also suggest copying the relevant RAN1 agreements related to on-demand SSB in the reply LS to RAN4.

Reply to RAN4’s Question 1 on Objective 1 that periodicity of on-demand SSB is one of  {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms} and that periodicity of on-demand SSB does not depend on periodicity of always-on SSB. This holds irrespectively of whether always-on SSB is provided in the same SCell or not.

Reply to RAN4’s Question 2 on Objective 1 that on-demand SSB and always-on SSB can be provided at least with same time offset.

Reply to RAN4’s Question 3 on Objective 1 that on-demand SSB and always-on SSB can be provided on same frequency or on different frequencies.

Reply to RAN4’s Question 4 on Objective 1 that at least when on-demand SSB and always-on SSB is provided on the same frequency and with same time offset, a signal/channel that is spatially QCLed with an SSB index in the on-demand SSB burst is spatially QCLed with the same SSB index in the always-on SSB burst.

However, further progress in AI 9.5.1 is required before replying to RAN4.

## 2.2	Objective 3: SSB adaptation

We propose the following replies for the questions related to Objective 3, and also suggest copying the relevant RAN1 agreements related to SSB adaptation in the reply LS to RAN4.

Reply to RAN4’s Question 1 on Objective 3 as follows:

  - At least the case where the time location is same before and after SSB adaptation is supported (i.e., SSB burst periodicity is adapted).

  - Any additional cases are still under RAN1 discussion.

Reply to RAN4’s Question 2 on Objective 3 as follows:

  - At least the case where the frequency location is same before and after SSB adaptation is supported (i.e., SSB burst periodicity is adapted).

  - Any additional cases are still under RAN1 discussion.

Reply to RAN4’s Question 3 on Objective 3 as follows:

  - At least the case where there is no change to spatial relation before and after SSB adaptation is supported (i.e., SSB burst periodicity is adapted).

  - Any additional cases are still under RAN1 discussion.

# Conclusion

In the previous sections we made the following observations:

Observation 1	If always-on SSB is located off sync raster, it is preferred that on-demand SSB and always-on SSB are provided on the same frequency.

Observation 2	If always-on SSB is located on sync raster, it is preferred that on-demand SSB and always-on SSB are provided on different frequencies.

Observation 3	For SCell activation, on-demand SSB would typically be provided with periodicity that is less than periodicity of always-on SSB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reply to RAN4’s Question 1 on Objective 1 that periodicity of on-demand SSB is one of  {5 ms, 10 ms, 20 ms, 40 ms, 80 ms, 160 ms} and that periodicity of on-demand SSB does not depend on periodicity of always-on SSB. This holds irrespectively of whether always-on SSB is provided in the same SCell or not.

Proposal 2	Reply to RAN4’s Question 2 on Objective 1 that on-demand SSB and always-on SSB can be provided at least with same time offset.

Proposal 3	Reply to RAN4’s Question 3 on Objective 1 that on-demand SSB and always-on SSB can be provided on same frequency or on different frequencies.

Proposal 4	Reply to RAN4’s Question 4 on Objective 1 that at least when on-demand SSB and always-on SSB is provided on the same frequency and with same time offset, a signal/channel that is spatially QCLed with an SSB index in the on-demand SSB burst is spatially QCLed with the same SSB index in the always-on SSB burst.

Proposal 5	Reply to RAN4’s Question 1 on Objective 3 as follows:

-	At least the case where the time location is same before and after SSB adaptation is supported (i.e., SSB burst periodicity is adapted).

-	Any additional cases are still under RAN1 discussion.

Proposal 6	Reply to RAN4’s Question 2 on Objective 3 as follows:

-	At least the case where the frequency location is same before and after SSB adaptation is supported (i.e., SSB burst periodicity is adapted).

-	Any additional cases are still under RAN1 discussion.

Proposal 7	Reply to RAN4’s Question 3 on Objective 3 as follows:

-	At least the case where there is no change to spatial relation before and after SSB adaptation is supported (i.e., SSB burst periodicity is adapted).

-	Any additional cases are still under RAN1 discussion.

# References

R1-2409350 (R4-2416913), “LS on SSB relation in On-demand SSB and SSB adaptation on SCell”, RAN1#119, Orlando, FL, November 18—22, 2024.

R1-2419441, “On-demand SSB SCell operation”, Ericsson, RAN1#119, Orlando, FL, November 18—22, 2024.