3GPP TSG RAN WG1 #119                                                                                       R1-2409486

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.4.2.3

Source:	Lenovo

Title:	Discussion on channel/signal aspects for Ambient IoT

Document for:	Discussion

# Introduction

Rel-19 study item of ambient IoT has been finalized in RAN#102 meeting in RP-234058 [1] and further revised in RAN#103 meeting in RP-240826. In addition, RAN1#116 meeting started relevant technology discussion on physical layer signal and procedure design as well as channel structure for ambient IoT and a set of preliminary agreements is reached. Furthermore, RAN1#118bis meeting continues the discussion and reached below agreements:

Agreement

For the clock-acquisition part of the R2D time acquisition signal, following is captured in the TR 38.769:

- Clock-acquisition part is based on OOK without line coding and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration

Agreement

For the PRDCH design details, following is captured in the TR 38.769:

Following design options have been studied for PRDCH:

- Option 1: If there is no L1 R2D control information carried by PRDCH, then PRDCH transmission with only R2D data is considered

  - Note: In this option, R2D control information (if any) maybe carried by higher-layer signaling

- Option 2: If any L1 R2D control information is supported & carried by PRDCH, then for the CRC attachment (appended if there is non-zero length CRC), following two options have been studied

  - Option 2-1: Joint CRC is attached to L1 R2D control information and R2D data

  - Option 2-2: Separate CRC is attached to L1 R2D control information and R2D data, respectively

Agreement

For the start-indicator part of the R2D time acquisition signal, for providing the start of the R2D transmission, following is captured in the TR 38.769:

- Following options have been studied for the start-indicator part of the R2D time acquisition signal:

  - Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including

    - Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations

    - Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration

  - Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation

  - For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions.

- Miss-detection ratio (MDR), false-alarm ratio (FAR) and detection complexity have been considered for the design of the R2D start indicator part by following companies

  - It is observed by 1 source [Huawei] that for an FAR of ~0%, the MDR of less than 1% can be achieved with Alt 2 of option 1 (considering 2 ON-OFF transmissions with different durations) and it is also observed that low-complexity and reduced power consumption can be achieved

  - 1 source [ZTE] evaluated Alt 1 of option 1 (considering same duration for ON and OFF) and Alt 2 of option 1 (considering multiple ON-OFF transmissions with same duration) and observed that for an FAR of ~0%, the MDR of less than 1% can be achieved and Alt 1 of option 1 performs better than Alt 2 of option 1.

  - 1 source [CATT] observed with ON-OFF pattern, that for an FAR of ~0%, the MDR of less than 1% can be achieved with a duration of at least 1 OFDM symbol

  - 1 source [Qualcomm] compares the performance between option 1 and option 2. It shows almost similar coverage range (SNR requirement) for target MDR of 1%. For MDR of 10%, it shows that sequence-based design provides better performance, and it is observed that during the available time, it is feasible for all devices to detect the start-indicator sequence within the power budget. It is further observed that the FAR with sequence-based design can be improved in case of interference scenarios when compared with pattern-based design.

- For both the options, it may be beneficial that the start-indicator part is distinguishable at least from other parts of the R2D transmissions

Agreement

For single PDRCH generation at the device, following options are captured in TR 38.769:

- Option 1: With line coding

  - Note: Location of repetition block depends on the repetition types agreed to be studied under agenda 9.4.2.1

  - Note: Presence of the block “small frequency shift with square wave generation” depends on the agreed options for small frequency shift under agenda 9.4.2.1

  - Note: the figure below is for the case where FEC and repetition are present

- Option 2: Without line coding

  - Note: Location of repetition block depends on the repetition types agreed to be studied under agenda 9.4.2.1

  - Note: the figure below is for the case where FEC and repetition are present

Note: Further updates to PDRCH generation can be considered, depending on the progress

Agreement

For the clock-acquisition part of the R2D time acquisition signal for OOK chip duration determination, following options are studied:

- Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M

- Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant

- FFS: Whether/what restriction on M values for the clock-acquisition part

Note: Other functionalities of clock-acquisition part is a separate discussion

Agreement

For the D2R preamble, binary signal is considered.

Agreement

For the topology 2, following is captured in the TR 38.769:

For resource allocation and/or controlling of intermediate UE in topology 2 for R2D and D2R transmissions via the Uu interface between the BS and intermediate UE, following two options have been studied:

    - Option 1: Higher-layer signaling is used

    - Option 2: L1 and higher-layer signaling are used

From RAN1 perspective, detailed aspects on the two options are left up to work item phase.

Following observations related to the two options have been made by following companies:

    - Four sources [Ericsson, Continental, Vivo, Mediatek] observed that option 1 can save overhead compared to option 2.

    - One source [Huawei] observed that the timing gap between R2D message and D2R message is not extended, and inventory rate is comparable with UHF RFID and furthermore, it is observed that with option 1, the standard impact includes no or less impact on L1 signaling for resource allocation and controlling of intermediate UE

    - Six sources [Ericsson, Qualcomm, Oppo, Futurewei, IIT-K, Cewit] observed that option 2 with L1 signaling provides better resource utilization compared to option 1

For the purpose of the Rel-19 study, the study on resource allocation and/or controlling of intermediate UE in topology 2 in RAN1 is considered completed based on two options above. Additional observations can be captured at RAN1#119.

In this contribution, we focus on physical layer channel design for ambient IoT and elaborate our views on physical layer channel structure, synchronization signal, random access channel, and scheduling issues.

# Discussion

## On R2D channel design

For ambient IoT application, considering the limited size and complexity required by practical applications for battery-less devices with no energy storage capability or devices with limited energy storage that do not need to be replaced or recharged manually, the output power of energy harvester is typically from 1µW to a few hundreds of µW. Existing cellular devices may not work well with energy harvesting due to their peak power consumption of higher than 10mW.

For Rel-19 ambient IoT communication, the study targets ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications and addresses use cases and scenarios that cannot be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power. Considering the ambient IoT tag is expected to be very-low end, the ambient IoT tag should be designed with very low cost even affordable for one-off use. Therefore, oscillator of the ambient IoT tag is quite cheap with low accuracy. A typical sampling frequency offset (SFO) of the ambient IoT tag is about 105 ppm. This leads to significant frequency shift and low synchronization accuracy in time domain and frequency domain for ambient IoT communication.

Since the design target for Rel-19 ambient IoT tag is about 1µW power consumption, ultra-simple communication techniques with ultra-low power consumption should be considered, e.g., using OOK as waveform, ASK/FSK/PSK as modulation scheme, Manchester/Miller/FM0 coding as channel coding scheme, TDMA/FDMA/CDMA as multiple access scheme. Especially, using backscatter transmission for uplink leads to quite different physical layer procedures. As a result, almost all the physical channels need to be redesigned for ambient IoT compared with NR.

As analyzed above, the ambient IoT tag can’t maintain sufficiently accurate time-frequency synchronization timing due to the characteristics of ultra-low complexity, ultra-low power consumption and very-low end, Reader-to-Device (R2D) transmissions can be transmitted in burst type in time domain, wherein each R2D transmission burst comprises a synchronization signal at the beginning of the R2D transmission with consecutive time domain resources in time domain and a Guard period (GP) at the end of the R2D transmission to leave the channel empty so as to avoid the inter-burst interference due to the timing shift error between consecutive bursts in time domain.

As mentioned in Section 1, RAN1 has agreed to study a physical channel (PRDCH) at least for R2D data transmission to ambient IoT devices, which can be also used to transmit system information (if defined). One example on PRDCH channel structure is shown in Figure 1:

- A preamble, as agreed in previous RAN1 meeting, can include a start-indicator part providing the start of the R2D transmission and a clock-acquisition part providing the chip level synchronization for subsequent physical channel transmission. The start-indicator part can be designed as a pre-known ON/OFF pattern, i.e., high/low voltage signal to devices. The clock-acquisition part can be designed as several pre-known chips as RFID. In that sense, the device can determine the start of a R2D transmission and estimate the synchronization timing upon detection of the preamble. Both the start-indicator part and the clock-acquisition part in the preamble can be specified in standards and common to multiple cells, i.e., same preamble can be used in multiple cells. In that sense, the ambient IoT device can necessarily detect the minimum preamble which can not only simplify the device receiver complexity and store a single preamble signal but also save the device power consumption on searching the preamble signal. R2D control part should be designed to include minimum control information which is essential for the associated data part in the PRDCH transmission. The R2D control part is transmitted immediately following the preamble and preceding the data part. The R2D control part includes at least device ID which is used to identify the target device for the R2D transmission, where each device is identified by a device ID assigned by the manufacturer or network. This device ID is stored in the memory of the device and unique within the network. When network intends to transmit downlink data/control or trigger the device to report the sensed data, location or other message, the network shall indicate this device ID in the control part of the R2D transmission to uniquely identify this device.

  - For a given device, it monitors the preamble transmission and determines whether the R2D transmission is targeted to it by detecting the device ID in the control part associated with the R2D transmission and comparing the detected device ID with its own device ID stored in its memory. If the detected device ID is equal to its own device ID, then the device shall further decode the received data or control information carried by the data part of the R2D transmission; otherwise, the device shall drop the data part of the R2D transmission and not further process it so as to avoid unnecessary effort on decoding of R2D data/command or D2R scheduling signaling which is not targeted for the device itself and save the power consumption.

  - On the other hand, it is still open in RAN1 whether the R2D control part is regarded as an independent physical channel or a part of the PRDCH. Considering the R2D control part includes only essential information, it is not necessary to design a new physical channel like PDCCH to carry the R2D control information. However, placing the R2D control part in MAC CE will lead to longer latency and unnecessary power consumption at the device side because the device has to decode all the received data till the end of the R2D transmission then check CRC to determine whether the R2D transmission is aimed at itself. Such unnecessary power consumption should be avoided considering the design target for ambient IoT device is about 1uw and the ambient IoT device has very limited capacitor.

- Hence, the R2D control part and the R2D data part can be included in same PRDCH while have different encoding and CRC processing lines, like UCI multiplexing on PUSCH in NR and LTE. In detail, the R2D control part is encoded via Manchester or PIE coding and protected with independent CRC bits, which is separately from the R2D data part. Data part is used to transmit R2D data/command information or scheduling information for D2R transmission. The payload can have a variable size to adapt the R2D data/command information bits or D2R scheduling information bits so as to simplify the channel structure.

  - Since the coverage for ambient IoT readers in indoor scenarios, e.g., factories, may have overlapping which is especially true for multiple UE-type readers in one cell in case of Topology 2. With scrambling the data part, interference randomization can be achieved. The scrambling sequence for R2D transmission can be generated based on the reader ID.

- Midamble is transmitted in the middle of the data part of the R2D transmission for further providing the R2D transmission timing reference to the device. Considering about 10% transmission timing error for the ambient IoT devices, when PIE encoding is used for encoding R2D data with about 1000 bits of the message size for indoor command, the R2D transmission has so relatively long duration that a midamble is necessary within the transmission duration for the device to further refine the synchronization timing. It is worth noting that a Tari in PIE coding has a relatively large range from 6.25µs to 25µs and tolerance on all parameters specified in units of Tari shall be +/–1%. Such uncertainty for PIE encoding requires timing refining during a long transmission.

- Postamble is transmitted at the end of a burst as a sync word with fixed binary pattern for indicating the end of the R2D transmission. With the postamble, the payload size of the data part is not necessarily indicated to the device. Upon detection of the postamble, the device can know the end of the current burst. Considering the message size can be up to about 1000 bits, i.e., 125 bytes, then about 7 bits are needed to indicate the TBS in granularity of bytes. Based on the postamble, this implicit indication can significantly save the bit overhead for indicating the TBS of a PRDCH. Additional benefit based on implicit indication by postamble, the set of possible TB sizes is not necessarily predefined in standards. Any TB size can be supported for R2D transmission. The device determines the end of PRDCH transmission only based on the detection of the postamble.

- Guard period (GP) consists of a predefined duration and intends to leave the channel empty so as to avoid the inter-burst interference due to the timing shift error between consecutive bursts in time domain. Considering the low accuracy of synchronization timing, the inter-bust interference may be caused when the timing shift between consecutive bursts happens. In that sense, to combat the inter-burst interference, a gap period is needed at the end of the transmission burst.

As analyzed above, we have below proposals:

Proposal 1: Study a channel structure for R2D transmission which comprises a preamble, a control part, a data part, midamble, postamble and a guard period.

Proposal 2: Study a R2D control part immediately following the preamble of the R2D transmission and having separate encoding and CRC from the R2D data part.

Proposal 3: Study scrambling for R2D data part.

Proposal 4: Study a postamble at the end of a R2D transmission for indicating the end of the PRDCH transmission.

Figure 1 Channel structure for R2D transmission

Furthermore, common frame structure for the Ambient IoT device includes a control header to convey the type of message carried in the payload which can be system information to the Ambient IoT, downlink trigger/command, downlink data or uplink data, information regarding the additional preamble position and length within the payload, the ‘M’ value in the OOK-4 modulation i.e., OOK-4, M=2 or M=4. The Frame structure needs to support a sync word or delimiter with fixed binary pattern inserted after the preamble to let the receiver know about the end of the preamble pattern and the start of the control.

Figure 2 Common frame structure design for D2R and R2D communication

The preamble design should take into consideration the timing error or the residual timing error due to cheap local oscillator supported at the Ambient IoT devices. The preamble preceding R2D transmission contain multiple parts and the first part of the preamble at the beginning of the slot can be designed as wider pulse duration to tolerate the large timing errors. The pre-configured/known binary modulated pattern or binary sequence OOK-4, M=1 generates wider pulse duration to aid the envelope detector to detect such wider pulses when the clock drift at the beginning of the slot could be larger. Although the wider pulses provides toleration to the time synchronization errors, the narrower pulse in the second part of the preamble generated using for example, OOK-4, M>1provides much finer synchronization due to presence of multiple OOK chips within an OFDM symbol as shown in the Figure 4.

Figure 3 Preamble design options

Proposal 5: Study preamble design containing wide pulse tolerating large timing errors at the beginning followed by narrower pulse duration achieving finer synchronization needed for the R2D reception.

Proposal 6: The preamble designed using OOK chips preceding R2D contains multiple part:

- The first part of the preamble contains wide pulse duration generated using OOK-4, M=1 at the beginning duration

- The second part of the preamble contains narrow pulse duration using OOK-4, M>1 following the first part of the preamble

- The third part contains sync word as delimiter with narrower pulse duration.

- R2D reception at the device determines the chip duration using the second part of the preamble.

## On D2R channel design

Similar as R2D channel design principle, the ambient IoT tag can’t maintain sufficiently accurate time-frequency synchronization timing due to the characteristics of ultra-low complexity, ultra-low power consumption and very-low end, Device-to-Reader (D2R) transmissions can be transmitted in burst type in time domain, wherein each D2R transmission burst comprises a synchronization signal at the beginning of the D2R transmission with consecutive time domain resources in time domain and a Guard period (GP) at the end of the R2D transmission to leave the channel empty so as to avoid the inter-burst interference due to the timing shift error between consecutive bursts in time domain.

As mentioned in Section 1, RAN1 has agreed to study a physical channel (PDRCH) at least for D2R data transmission from ambient IoT devices, which can be also used to transmit response from device to reader during contention-based access procedure. One example on PDRCH channel structure is shown in Figure 4:

- A preamble, as agreed in previous RAN1 meeting, preceding each PDRCH transmission is studied at least as the D2R transmission timing reference so that the reader can estimate the R2D transmission timing upon detection of the preamble. The preamble is a bit sequence pre-known to the ambient IoT device and reader and can be used for channel estimation of the burst at reader side via coherent detection. The preamble is located in the beginning of the D2R transmission and can be used by the reader to identify the start of a PDRCH transmission. A single preamble can be specified in standards to save the device memory for storing preamble.

- D2R control part is necessary at least for contention-based access and should be designed to include minimum control information which is essential for the associated data part in the PDRCH transmission. The D2R control part is transmitted immediately following the preamble and preceding the data part. The D2R control part includes at least device ID which is used to identify the transmitting device for D2R transmission especially for contention-based access. The device ID of the transmitting device is transmitted associated with the PDRCH transmission so that the reader can easily identify the D2R transmissions from a certain device among multiple devices. This is especially true for indoor inventory case considering there may be significant number of devices within one cell triggered for reporting by one inventory command.

  - It is still open in RAN1 whether the D2R control part is regarded as an independent physical channel or a part of the PDRCH. Considering the D2R control part includes only essential information, it is not necessary to design a new physical channel like PUCCH to carry the D2R control information. However, placing the D2R control part in MAC CE will lead to longer latency and unnecessary power consumption at reader side because the reader has to decode all the received data till the end of the D2R transmission then check CRC to determine whether the D2R transmission is from a certain device. Such unnecessary power consumption should be avoided considering the reader is an intermediate UE in Topology 2.

  - Hence, the D2R control part and the D2R data part can be included in same PDRCH while have different encoding and CRC processing lines, like UCI multiplexing on PUSCH in NR and LTE. In detail, the D2R control part is encoded via Manchester coding and protected with independent CRC bits, separately from the D2R data part.

- Data part is used to transmit D2R data for D2R transmission. The payload can have a variable size to adapt the D2R data information bits so as to simplify the channel structure.

  - Since the coverage for ambient IoT readers in indoor scenarios, e.g., factories, may have overlapping which is especially true for multiple UE-type readers in one cell in case of Topology 2. With scrambling the data part, interference randomization can be achieved. The scrambling sequence for D2R transmission can be generated based on the device ID.

- Midamble is transmitted in the middle of the data part of the D2R transmission for the reader to further perform timing/frequency tracking or channel estimation for the associated PDRCH transmission from the device. Channel estimation via coherent detection can improve the D2R coverage which is quite important for D2R transmission as coverage may be a bottleneck for Ambient IoT due to backscattering-based transmission and ultra-low power consumption. Considering the preamble can provide same function of timing/frequency tracking or channel estimation, it is natural that midamble is not needed in case of a short message from device to reader. Midamble can be used for large message size with relatively long duration for D2R transmission so that a midamble is necessary within the transmission duration for the reader to further refine the synchronization timing. However, the possible message size is up to the discussion outcome of RAN2.

  - Whether to apply midamble in D2R transmission can be determined in different ways. In case the determination is made by reader transmitting R2D triggering the D2R transmission, the reader needs to indicate the device to insert midamble into the D2R transmission. Further, the reader may indicate information facilitating device to determine how to insert midamble into D2R transmission, including the placement, the number, the pattern of midambles. Midamble related information can be indicated by the corresponding R2D channel in the D2R scheduling information, in addition to the agreed information indicated for D2R scheduling.

  - In case the determination is made by device, the criteria on whether/how to insert midamble into D2R transmission needs to be pre-defined.

- Postamble is necessarily transmitted at the end of a D2R transmission for indicating the end of the PDRCH transmission especially in case of contention-based D2R transmission. With the postamble, the payload size of the data part is not necessarily indicated to the reader. Upon detection of the postamble, the reader can know the end of the current burst. The main benefit of this implicit indication based on postamble is to save the bit overhead for indicating TBS in the control part for the associated D2R transmission. Although the possible message size is up to the discussion outcome of RAN2, about 4 or 5 bits for indicating TBS for D2R transmission can be saved by postamble.

  - On the other hand, in case of scheduling-based D2R transmission, the concrete payload size is indicated by the corresponding R2D transmission in the scheduling information, then a postamble for indicating the end of the transmission burst is not necessary for D2R transmission.

- Guard period (GP) consists of a predefined duration and intends to leave the channel empty so as to avoid the inter-burst interference due to the timing shift error between consecutive bursts in time domain. Considering the low accuracy of synchronization timing, the inter-bust interference may be caused when the timing shift between consecutive bursts happens. In that sense, to combat the inter-burst interference, a gap period is needed at the end of the transmission burst.

As analyzed above, we have below proposals:

Proposal 7: Study a channel structure for D2R transmission which comprises a preamble, a control part, a data part, midamble, postamble and a guard period.

Proposal 8: Study a D2R control part immediately following the preamble of the D2R transmission and having separate encoding and CRC from the D2R data part.

Proposal 9: Study scrambling for D2R data part.

Proposal 10: Midamble related information can be indicated:

- Option 1. by the corresponding R2D channel in the D2R scheduling information, in addition to the agreed information indicated for D2R scheduling.

- Option 2. in a pre-defined way.

Proposal 11: Study a postamble at the end of a D2R transmission for indicating the end of the PDRCH transmission for contention-based transmission.

Proposal 12: Payload size of a D2R channel can be indicated by the corresponding R2D channel in the scheduling information for scheduling-based transmission.

Figure 4 Channel structure for D2R transmission

## Resource allocation

For NR using CP-OFDMA, flexible and efficient time-frequency resource allocation is realized in both time domain and frequency domain so as to adapt for multiple services with variable data packet size. NR PDCCH indicates symbol-level TDRA and RB-level FDRA which fully improves the resource utilization efficiency of NR.

As analyzed in Section 2.1, for Rel-19 ambient IoT communication aiming at ultra-low complexity devices with ultra-low cost and ultra-low power consumption for the very-low end IoT applications, ultra-simple communication techniques should be considered, e.g., using OOK as waveform, ASK/FSK/PSK as modulation scheme, Manchester/Miller/FM0 coding as channel coding scheme, TDMA/FDMA/CDMA as multiple access scheme. Thus, the flexible resource allocation is deprioritized for ambient IoT and a simple resource allocation scheme is needed.

For simplicity, the frequency resource for ambient IoT communication can be divided into multiple ambient IoT sub-channels via FDM with each sub-channel operating on different carrier frequency. So the frequency resource for each device can be a sub-channel. For each sub-channel, the time domain resource is divided into multiple time units with each time unit assigned for one ambient IoT device. So the time domain resource for each device can be a time unit. One example is shown in Figure 5, where in-band deployment for ambient IoT is considered.

In that sense, the detailed time domain resource (e.g., a time unit) and frequency domain resource (e.g., a sub-channel) for D2R transmission can be carried in the scheduling information of the corresponding R2D transmission.

As analyzed above, we have below proposals:

Proposal 13: Study simple resource allocation scheme with a sub-channel in frequency domain and a time unit in time domain for ambient IoT indicated by a PRDCH for the corresponding D2R transmission.

Figure 5 Time-frequency resource for ambient IoT for in-band deployment

According to Rel-19 SID for Ambient IoT communication, both Topology 1 and Topology 2 from the existing 4 topologies are considered for study in Rel-19. In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and base station. In this topology, the intermediate node is a UE which is capable of Ambient IoT. The intermediate node of UE transfers Ambient IoT data and/or signaling between BS and the Ambient IoT device.

One problem needs to be resolved is on which resource the UE transmits R2D signals/channels to the device and receives D2R signals/channels from the device considering the spectrum for ambient IoT communication uses FDD licensed spectrum. For licensed spectrum, the radio resources should be controlled by gNB to avoid intra-cell interference. How to allocate the radio resource to the intermediate node of UE for ambient IoT communication should be studied.

Using combination of L1 signaling and high layer signaling can dynamically adjust the resource allocation so as to achieve better spectrum utilization compared with pure high layer signaling mechanism. Based on the learning from Rel-16 NR V2X and subsequent sidelink enhancements, where a DCI format is used to dynamically assign a set of resources for sidelink mode 1 resource allocation, it is reasonable for Ambient IoT communication in Topology 2 to transmit a DCI format from gNB to assign a set of time domain resources for the UE to transmit R2D transmission carrying indoor command or indoor inventory message to devices and receive D2R transmissions carrying corresponding responses from the devices.

Since the UE may have DL or UL traffic in addition to providing ambient IoT service, the DCI indicates the exact starting position and the duration of the allocated ambient IoT resources. The starting position is indicated by a timing offset between the end of the PDCCH carrying the DCI and the start of the allocated ambient IoT resources. The duration of the allocated ambient IoT resources is indicated in a number of consecutive time domain resource units. A PUCCH resource is also indicated by the DCI for the UE to report ACK or NACK to gNB. If NACK is reported, it implies the UE has not successfully finished the indoor command or indoor inventory and requests the resource for finishing the remaining service. If ACK is reported, it implies the UE has successfully finished the indoor command or indoor inventory service. Upon reception of the ACK, gNB shall not assign resources for the UE. Alternatively, the study could focus on additional uplink reporting and methods for indicating or configuring related uplink resources during the inventory procedure. For instance, due to the lack of energy of ambient IoT device(s), the pre-configured communication round for one or more of ambient IoT devices might need to be terminated prematurely. Consequently, it would be beneficial to release any remaining communication resources early, request re-access resources sooner, or indicate a waiting period before re-access, taking into account the charging status of the ambient IoT devices.

Proposal 14: Study radio resource allocation mechanism via indicating starting position and the duration of the allocated ambient IoT resources for intermediate node in Topology 2.

Proposal 15: Study PUCCH resource after the allocated ambient IoT resources for intermediate node in Topology 2 to request resource to finish ongoing ambient IoT service.

Proposal 16: Study uplink reporting to enhance resource allocation in Topology 2.

## Coexistence with NR for in-band deployment

As mentioned in the SID, there are three deployment modes for ambient IoT communication, i.e., in-band, guard band, and standalone. For in-band mode, the ambient IoT communication is operating within the band of NR system. For guard band mode, the ambient IoT communication is operating within the guard band region of two adjacent bands. For standalone mode, the ambient IoT communication is allocated with a dedicated spectrum. Among these three deployment modes, the in-band mode is most promising way to fully utilize the existing NR spectrum and the already deployed NR gNB.

However, when ambient IoT communication is transmitted on one part of NR spectrum, e.g., allocating a narrow bandwidth for ambient IoT within a NR carrier bandwidth, there is mutual interference between two systems due to quite low accuracy of the ambient IoT tag. A typical sampling frequency offset (SFO) of the ambient IoT tag is about 105 ppm. This leads to significant frequency shift in frequency domain for ambient IoT communication. A guard band may be needed between ambient IoT and NR. However, the spectrum utilization efficiency is reduced. How to resolve the mutual interference and ensure the coexistence between two systems should be studied.

As analyzed above, we have below proposals:

Proposal 17: Study coexistence mechanism between ambient IoT and NR for in-band deployment.

## Intermediate node operation

For connectivity topology 2 the intermediate node is used to extend the coverage of the gNB, however there may have multiple UEs which are capable to be served as the intermediate node for the communication between gNB and ambient IoT devices. gNB needs to determine which UE(s) could be served as the intermediate node, and the intermediate node selection may depend on whether there have ambient IoT devices around the intermediate node, e.g., only the UE with ambient IoT devices proximate to it could be determined as the intermediate node. On the determination of intermediate node two types of ambient IoT service flow may be considered:

- Case 1: The initiator of the ambient IoT service is the intermediate node, e.g., the UE’s owner. In this case the UE may transmit request to the gNB, then gNB may allocate resources for the communication between intermediate node and ambient IoT device.

Figure 6 Ambient IoT service is initiated by the intermediate node

- Case 2: The initiator of the ambient IoT service is behind core network. In this case the gNB may need to determine which UE could be served as the intermediate node firstly, then gNB could allocate resources to the intermediate node for the communication between intermediate node and ambient IoT device.

Figure 7 Ambient IoT service is initiated behind core network

For case 1 that the ambient IoT service is initiated by the UE, the UE which transmits the request is determined as the intermediate node. However, for case 2 that the ambient IoT service is initiated behind the core network, the gNB may need to select the suitable UE to be served as the intermediate node. Firstly, the gNB could only select from the UEs which are capable to be served as the intermediate node, so the reporting of the UE capability is necessary. Secondly, only the UE with ambient IoT device around it could be served as the intermediate node, so the proximity detection at the UE is needed. The proximity detection may be based on the measurement of the signal transmitted/backscattered by the ambient IoT devices, e.g., RSSI measurement on the resource for the transmission from ambient IoT devices. With the measurement result the UE could report ‘presence’ or ‘absence’ of ambient IoT devices proximate to it, then gNB could determine which UE could be served as the intermediate node.

Proposal 18: Study the intermediate node selection procedure with proximity detection for topology 2.

# Conclusion

In this contribution, we focus on physical layer design for ambient IoT and have below observations and proposals on frame structure, synchronization, random access, and scheduling timing issues:

Proposal 1: Study a channel structure for R2D transmission which comprises a preamble, a control part, a data part, midamble, postamble and a guard period.

Proposal 2: Study a R2D control part immediately following the preamble of the R2D transmission and having separate encoding and CRC from the R2D data part.

Proposal 3: Study scrambling for R2D data part.

Proposal 4: Study a postamble at the end of a R2D transmission for indicating the end of the PRDCH transmission.

Proposal 5: Study preamble design containing wide pulse tolerating large timing errors at the beginning followed by narrower pulse duration achieving finer synchronization needed for the R2D reception.

Proposal 6: The preamble designed using OOK chips preceding R2D contains multiple part:

- The first part of the preamble contains wide pulse duration generated using OOK-4, M=1 at the beginning duration

- The second part of the preamble contains narrow pulse duration using OOK-4, M>1 following the first part of the preamble

- The third part contains sync word as delimiter with narrower pulse duration.

- R2D reception at the device determines the chip duration using the second part of the preamble.

Proposal 7: Study a channel structure for D2R transmission which comprises a preamble, a control part, a data part, midamble, postamble and a guard period.

Proposal 8: Study a D2R control part immediately following the preamble of the D2R transmission and having separate encoding and CRC from the D2R data part.

Proposal 9: Study scrambling for D2R data part.

Proposal 10: Midamble related information can be indicated:

- Option 1. by the corresponding R2D channel in the D2R scheduling information, in addition to the agreed information indicated for D2R scheduling.

- Option 2. in a pre-defined way.

Proposal 11: Study a postamble at the end of a D2R transmission for indicating the end of the PDRCH transmission for contention-based transmission.

Proposal 12: Payload size of a D2R channel can be indicated by the corresponding R2D channel in the scheduling information for scheduling-based transmission.

Proposal 13: Study simple resource allocation scheme with a sub-channel in frequency domain and a time unit in time domain for ambient IoT indicated by a PRDCH for the corresponding D2R transmission.

Proposal 14: Study radio resource allocation mechanism via indicating starting position and the duration of the allocated ambient IoT resources for intermediate node in Topology 2.

Proposal 15: Study PUCCH resource after the allocated ambient IoT resources for intermediate node in Topology 2 to request resource to finish ongoing ambient IoT service.

Proposal 16: Study uplink reporting to enhance resource allocation in Topology 2.

Proposal 17: Study coexistence mechanism between ambient IoT and NR for in-band deployment.

Proposal 18: Study the intermediate node selection procedure with proximity detection for topology 2.

# References

RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#102, Dec. 2023.

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, Mar. 2024.

TR38.848, Study on Ambient IoT (Internet of Things) in RAN

R1-2409485, Discussion on frame structure and physical layer procedures for ambient IoT, Lenovo