3GPP TSG RAN WG1 #119 	                                           R1-2409511

Orlando, US, November 18th – 22nd, 2024

Source: 	CMCC

Title:	Discussion on evaluation methodology, assumptions and results

Agenda item:	9.4.1.1

Document for:	Discussion & Decision

# Background

During RAN#102 meeting, a new study item on Ambient IoT (Internet of Things) in NR was approved for Rel-19 [1], which shall provide new IoT technology relying on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications.

One of the objectives that the study item focuses on is the evaluation assumptions for Ambient IoT, which is shown as follows,

The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1.

The deployment scenarios, detailed link level assumptions for coverage evaluation and link budget calculation methodology have been discussed during previous RAN1 meetings, this contribution will provide observations on link budget results and LLS results for different deployment scenarios, as well as design targets for maximum distance target. And we also recommend some conclusions about A-IoT evaluation.

- Design target of applicable maximum distance and general assumptions, including deployment scenarios, topology and distributions

- Evaluation assumptions and results

  - Link budget results

  - Link-level simulation results

- Recommended conclusions on A-IoT evaluation

# Discussion

## Remaining design targets

The following is tasked to RAN1 according to the objectives,

- Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs Clause 5.8: 2D distribution of devices

And the RAN design targets of user experienced data rate, maximum message size, moving speed of device, latency, connection/device density were also discussed in the RAN#103 meeting with the following proposal being endorsed.

Proposal 5v2 - RAN design targets for user experienced data rate, maximum message size, and moving speed of device: those can be used as assumptions in coverage evaluations, i.e. the coverage evaluations are done under the conditions that meet those targets. - Evaluations of RAN design targets for latency and connection/device density are allowed by the Rel-19 SID and observations on those evaluations can be captured in the TR38.769 Note: this is as per the SID: “NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically.”

Applicable maximum distance target

It is agreed that applicable maximum distance target value(s) are set separately for device 1, device 2a, device 2b. And separate target value ranges for each device type in different evaluation scenarios are achieved in the last RAN1 meeting, which are listed as below.

RAN1#118bis Agreement For Rel-19 Ambient IoT, the target value(s) for applicable maximum distance are refined as follows, - For device 1 in D1T1: - a value within [10 - 20]m - For device 2a in D1T1: - a value within [20 - 30]m - For device 2b in D1T1: - a value within [30 - 50]m - For device 1 in D2T2: - 10 m - For device 2a in D2T2: - a value within [10 - 20]m - For device 2b in D2T2: - a value within [30 - 50]m The distance target is compared to the results which assumes mandatory assumptions (as agreed).

According to the agreement achieved in the last meeting, it is recommended to refine a single target value within the range for applicable maximum distance respectively. By taking the coverage evaluation results into consideration, for device 1, device 2a and device 2b in D1T1 and D2T2, the median of the value range can be considered as the target of applicable maximum distance. Therefore, it is proposed the target value for applicable maximum distance are refined as follows,

For D1T1

- 15m for device 1

- 25m for device 2a

- 40m for device 2b

For D2T2 with InH-Office scenario

- 10m for device 1

- 15m for device 2a

- 40m for device 2b

2D distribution of devices

- The devices are uniformly distributed in the system as a starting point.

- For simplicity, system level simulation is not required.

Proposal 1:

For Applicable maximum distance target,

- For D1T1

  - 15m for device 1

  - 25m for device 2a

  - 40m for device 2b

- For D2T2 with InH-Office scenario

  - 10m for device 1

  - 15m for device 2a

  - 40m for device 2b

For 2D distribution of devices

- The devices are uniformly distributed in the system as a starting point.

- For simplicity, system level simulation is not required.

## Scenarios prioritization

Considering deployment scenarios for coverage evaluation, consensus has been reached on scenarios D1T1-A1/A2/B/C and D2T2-A1/A2/B/C [3]. Among these, priority cases are identified for further evaluation of coverage.

For D1T1,

- All scenarios except scenario 'C' represent new developments in backscattering for monostatic and bistatic coverage. These are scenarios of interest for future research.

For D2T2-A1,

- The gNB must coordinate with multiple intermediate UEs for transmission and reception. Although this reduces the UE's self-interference cancellation capability requirements, the complex signaling exchange and control protocol could potentially introduce unnecessary deployment challenges.

For D2T2-A2,

- Some UE vendors have expressed difficulty in managing self-interference at the UE level. This may pose a challenge for implementation by UE vendors.

For D2T2-B,

- CW outside topology by another intermediate node (e.g., UE or base station) can be used. Hence the UE self-interference handling is not required. This eliminates the need for UE self-interference handling. It requires the gNB to control only one intermediate UE for receiving backscattered signals, presenting a more appealing and potentially easier to deployment scenarios.

Proposal 2: Prioritize D1T1-A1/A2/B and D2T2-B for further coverage evaluation.

## Assumptions for topology and distribution

The following assumptions on topology and distribution for evaluation have been agreed in RAN1#116bis.

RAN1#116bis Agreement The following layout is used for evaluation purpose, - FFS: CW distribution for D1T1-B and D2T2-B Parameter; Assumptions for D1T1; Assumptions for D2T2. Scenario; InF-DH; InH-office; InF-DL. Hall size; 120x60 m; 120 x50 m; 300x150 m. Room height; 10 m; 3m; 10 m. Sectorization; None. BS deployment / Intermediate UE dropping; 18 BSs on a square lattice with spacing D, located D/2 from the walls. - L=120m x W=60m; D=20m - BS height = 8 m; - L=120m x W=50m; - Intermediate UE height = 1.5 m FFS: Intermediate UE dropping; - L=300m x W=150m; - Intermediate UE height = 1.5 m FFS: Intermediate UE dropping. Device distribution; Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area; Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations; Device Height= 1.5m AIoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations. Device mobility (horizontal plane only); 3 kph; 3 kph; 3 kph.

Couple of remaining issues are required to be solved as follows,

- FFS: CW distribution for D1T1-B and D2T2-B

- FFS: Intermediate UE dropping for D2T2

- FFS: which devices are involved in the evaluations for D2T2

CW distribution for D1T1-B and D2T2-B

In the link budget template discussion, the following assumptions are agreed for CW2D distance,

For scenarios ‘B’

  - D1T1-B:

    - 5m,

    - 10m,

    - 20m

    - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario

  - D2T2-B:

    - 5m,

    - 10m,

  - FFS other values

So it addresses the FFS point for CW distribution for D1T1-B and D2T2-B.

Intermediate UE dropping for D2T2

For InH-Office, the following alternatives can be considered,

Alt 1: Uniformly distributed

- Intermediate UE drop uniformly distributed over the horizontal area

Alt 2: Intermediate UE drop like BS deployment

- 12 intermediate UEs on a square lattice with spacing D, located 15m from the walls.

For InF-DL, the following alternatives can be considered,

Alt 1: uniformly distributed

- 18 Intermediate UE drop uniformly distributed over the horizontal area

Alt 2: Intermediate UE drop like BS deployment

- 18 intermediate UEs on a square lattice with spacing D, located D/2 from the walls.

Which devices are involved in the evaluations for D2T2

Whether the intermediate UEs can cover all the devices like D1T1 is a key question to be addressed. Typically, intermediate UEs can be relocated to a more advantageous position closer to the devices slated for inventory. To be simple, an additional gain can be considered in compensate to the pathloss as UE placement gain.

- all devices are considered to be connected to the nearest (minimum MPL) intermediate UE with X dB additional gain as the UE placement gain. FFS X value.

However, for coverage evaluation, no further discussion is needed for Intermediate UE dropping and which devices are involved in the evaluations for D2T2, it can be up to coexistence evaluation discussion which is carried in RAN4 [4].

Proposal 3: Intermediate UE dropping and which devices are involved in the evaluations for D2T2 are up to coexistence evaluation in RAN4.

# Evaluation results

## Link budget evaluation

### Assumptions for coverage evaluation

To evaluate the coverage performance of Ambient IoT, the link budget calculation can be performed, and it is supported that MPL and distance is used as performance evaluation metric. The complete link budget template and assumptions for each item, calculation formula are reached during previous meetings. Based on the agreed link budget template table, our detailed assumptions for link budget calculation are listed in Annex I. Also, some critical assumptions and calculation methods are listed below.

Table 3.1.1-1. Link budget assumptions

| Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- |
| (0) System configuration |  |  |
| [0C] Center frequency (MHz) | 900MHz | 900MHz |
| [0D] Topology/Pathloss model | - For D1T1: InF-DH NLOS - For D2T2: InH-Office LOS | - For D1T1: InF-DH NLOS - For D2T2: InH-Office LOS |
| (1) Transmitter |  |  |
| [1E] Total Tx Power (dBm) | - For BS in DL spectrum: 33dBm - For UL spectrum: 23dBm | - For device 1/2a: - The Device Tx Power is calculated by CW received power, backscatter loss and/or amplifier gain. - For device 2b (Scenarios ‘C’): -20 dBm |
| [1E1] CW Tx power (dBm) | N/A | For scenario ‘A1’, ‘A2’ and ‘B’ - 33dBm if CW in DL spectrum - 23dBm if CW in UL spectrum Note: only applicable for device 1/2a |
| [1E2] CW Tx antenna gain (dBi) | N/A | - 0dBi UE Tx ant gain, or - 6dBi BS Tx ant gain Note: only applicable for device 1/2a |
| [1E3] CW2D distance (m) | N/A | For scenarios ‘B’ - D1T1-B/D2T2-B: 10m For scenarios ‘A1’ and ‘A2’ - CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss Note: only applicable for device 1/2a |
| [1E5] CW received power (dBm) | N/A | For scenarios ‘A1’ and ‘A2’ - Calculated by assuming CW2D pathloss = D2R pathloss For scenarios ‘B’ - Derived by CW Tx power, CW2D distance and other factors |
| [1G] Tx antenna gain (dBi) | - For BS, 6 dBi - For intermediate UE, 0 dBi | - For A-IoT device, 0dBi |
| [1H] Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | - OOK: 6 dB - PSK: 0 dB Note: It is applicable for device 1 and 2a |
| [1J] Ambient IoT on-object antenna penalty | Not applicable | 0.9dB |
| [1K] Ambient IoT backscatter amplifier gain (dB) | N/A | - 10 dB Note: Only for device 2a |
| [1N] Cable, connector, combiner, body losses, etc. (dB) | - For BS, 0 dB - For intermediate UE, 1 dB Note: For Reader deployed for D1T1 indoor scenario, no cable/connector/combiner/body loss is needed to be considered. | N/A |
| (2) Receiver |  |  |
| [2B] Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth: 20MHz | - 15kHz for 0.8kbps data rate for device 1/2a - 90kHz for 6kbps data rate for device 1/2a - 180kHz for 0.8/6kbps data rate for device 2b |
| [2C] Receiver antenna gain (dBi) | - For A-IoT device, 0dBi | - For BS for indoor, 6 dBi - For intermediate UE, 0 dBi |
| [2X] Cable, connector, combiner, body losses, etc. (dB) | N/A | - For BS, 0 dB - For intermediate UE, 1 dB |
| [2H] Ambient IoT on-object antenna penalty | 0.9dB | Not applicable |
| [2J] Budget-Alt1/ Budget-Alt2 | Budget-Alt1 | Budget-Alt2 |
| [2K] CW cancellation (dB) | N/A | - For D1T1-A2, 140dB for BS - For D2T2-A2, 120dB for intermediate UE Note: - Only applicable for device 1/2a - For Scenario ‘A1’, ‘B’, assuming CW has no impact to the receiver sensitivity loss |
| [2L] Receiver Sensitivity (dBm) | For Budget-Alt1, - For device 1 (RF-ED), -36dBm - For device 2 (RF-ED), -45dBm | Calculated Note: the receiver sensitivity includes the receiver sensitivity loss [2K2], i.e. after CW cancellation at least if ‘A2’ scenario is used |

For R2D, Budget-Alt1 is used for link budget calculation at least for device 1/2a with RF-ED. And for D2R, Budget-Alt2 is used, the required SNRs for receiver sensitivity calculation are acquired by LLS results. For coverage evaluation purpose, the following parameters are assumed in the link-level simulation to obtain the required SNR for each case.

Table 3.1.1-2. LLS assumptions for coverage evaluation

| Parameter | Assumptions |
| --- | --- |
| Carrier frequency | 900MHz |
| Data rate | 0.8 kbps; 6 kbps |
| Receiver bandwidth | 0.8 kbps data rate: DSB 15 kHz for device 1/2a; DSB 180kHz for device 2b 6 kbps data rate: DSB 90 kHz for device 1/2a; DSB 180kHz for device 2b |
| Modulation | BPSK for device 1/2a; OOK for device 2b |
| Channel model | TDL-A 30ns; TDL-D 30ns |
| UE speed | 3 km/h |
| Number of RX antennas | 2 for BS; 1 for intermediate UE |
| TBS | 96 bits |
| Sampling frequency | Device 1: Fe = 10% Device 2a: Fe = 1% Device 2b: Fe = 1%, CFO 100ppm |
| Coding | 1/2 Manchester encoding |
| FEC | 1/3 CC |
| ADC | Ideal quantization, 1.92MHz sampling rate |

According to the deployment scenarios assumption discussed in the above section, the detailed results for different deployment scenarios could refer to the link budget spreadsheets attached with this contribution.

To identify the bottleneck coverage case and evaluate the coverage distance for various deployment scenarios and topology, the tolerate pathloss for each case are summarized in the Section 3.1.2 and Section 3.1.3, and the coverage distance is calculated based on the minimum results between R2D link and D2R link.

### D1T1 scenarios-InF-DH NLOS (TDL-A)

Table 3.1.2-1. D1T1-96bit-10%BLER-0.8kbps-OOK-6dBi BS antenna gain

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D1T1-A1 | 1-1 | 67.1 | 76.44 | 67.10 | 36.59 |
|  | 1-2 | 67.1 | 71.44 | 67.10 | 36.59 |
| D1T1-A2 | 1-1 | 67.1 | 60.32 | 60.32 | 17.02 |
|  | 1-2 | 67.1 | 60.31 | 60.31 | 16.99 |
| D1T1-B | 1-4 | 67.1 | 86.59 | 67.10 | 36.59 |
| Device 2a |  |  |  |  |  |
| D1T1-A1 | 1-1 | 76.1 | 82.35 | 76.10 | 95.51 |
|  | 1-2 | 76.1 | 77.35 | 76.10 | 95.51 |
| D1T1-A2 | 1-1 | 76.1 | 66.23 | 66.23 | 33.30 |
|  | 1-2 | 76.1 | 66.22 | 66.22 | 33.26 |
| D1T1-B | 1-4 | 76.1 | 98.42 | 76.10 | 95.51 |
| Device 2a (optional, Budget-Alt2 for R2D) |  |  |  |  |  |
| D1T1-A1 | 1-1 | 127.8/127.6/122.3/120.9 | 82.35 | 82.35 | 184.66 |
|  | 1-2 | 127.8/127.6/122.3/120.9 | 77.35 | 77.35 | 109.04 |
| D1T1-A2 | 1-1 | 127.8/127.6/122.3/120.9 | 66.23 | 66.23 | 33.30 |
|  | 1-2 | 127.8/127.6/122.3/120.9 | 66.22 | 66.22 | 33.26 |
| Device 2b RF-ED |  |  |  |  |  |
| D1T1-C | N/A | 76.1 | 95.78 | 76.10 | 95.51 |
| Device 2b (optional, Budget-Alt2 for R2D) |  |  |  |  |  |
| D1T1-C | N/A | 127.8/127.6/122.3/120.9 | 95.78 | 95.78 | 757.79 |

Table 3.1.2-2. D1T1-96bit-10%BLER-0.8kbps-BPSK-6dBi BS antenna gain

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D1T1-A1 | 1-1 | 67.1 | 79.44 | 67.10 | 36.59 |
|  | 1-2 | 67.1 | 74.44 | 67.10 | 36.59 |
| D1T1-A2 | 1-1 | 67.1 | 63.32 | 63.32 | 24.11 |
|  | 1-2 | 67.1 | 63.31 | 63.31 | 24.08 |
| D1T1-B | 1-4 | 67.1 | 92.59 | 67.10 | 36.59 |
| Device 2a |  |  |  |  |  |
| D1T1-A1 | 1-1 | 76.1 | 85.35 | 76.10 | 95.51 |
|  | 1-2 | 76.1 | 80.35 | 76.10 | 95.51 |
| D1T1-A2 | 1-1 | 76.1 | 69.23 | 69.23 | 46.05 |
|  | 1-2 | 76.1 | 69.22 | 69.22 | 45.99 |
| D1T1-B | 1-4 | 76.1 | 104.42 | 76.10 | 95.51 |
| Device 2a (optional, Budget-Alt2 for R2D) |  |  |  |  |  |
| D1T1-A1 | 1-1 | 127.8/127.6/122.3/120.9 | 85.35 | 85.35 | 253.22 |
|  | 1-2 | 127.8/127.6/122.3/120.9 | 80.35 | 80.35 | 149.60 |
| D1T1-A2 | 1-1 | 127.8/127.6/122.3/120.9 | 69.23 | 69.23 | 46.05 |
|  | 1-2 | 127.8/127.6/122.3/120.9 | 69.22 | 69.22 | 45.99 |

Table 3.1.2-3 D1T1-96bit-10%BLER-6kbps-OOK-6dBi BS antenna gain

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D1T1-A1 | 1-1 | 67.1 | 72.41 | 67.10 | 36.59 |
|  | 1-2 | 67.1 | 67.41 | 67.10 | 36.59 |
| D1T1-A2 | 1-1 | 67.1 | 60.18 | 60.18 | 16.73 |
|  | 1-2 | 67.1 | 60.11 | 60.11 | 16.59 |
| D1T1-B | 1-4 | 67.1 | 78.54 | 67.10 | 36.59 |
| Device 2a |  |  |  |  |  |
| D1T1-A1 | 1-1 | 76.1 | 78.67 | 76.10 | 95.51 |
|  | 1-2 | 76.1 | 73.67 | 73.67 | 73.89 |
| D1T1-A2 | 1-1 | 76.1 | 66.44 | 66.44 | 34.05 |
|  | 1-2 | 76.1 | 66.37 | 66.37 | 33.79 |
| D1T1-B | 1-4 | 76.1 | 91.06 | 76.10 | 95.51 |
| Device 2a (optional, Budget-Alt2 for R2D) |  |  |  |  |  |
| D1T1-A1 | 1-1 | 127.8/127.6/122.3/120.9 | 78.67 | 78.67 | 125.31 |
|  | 1-2 | 127.8/127.6/122.3/120.9 | 73.67 | 73.67 | 73.89 |
| D1T1-A2 | 1-1 | 127.8/127.6/122.3/120.9 | 66.44 | 66.44 | 34.05 |
|  | 1-2 | 127.8/127.6/122.3/120.9 | 66.37 | 66.37 | 33.79 |
| Device 2b RF-ED |  |  |  |  |  |
| D1T1-C | N/A | 76.1 | 87.71 | 76.10 | 95.51 |
| Device 2b (optional, Budget-Alt2 for R2D) |  |  |  |  |  |
| D1T1-C | N/A | 127.8/127.6/122.3/120.9 | 87.71 | 87.71 | 324.33 |

Table 3.1.2-4 D1T1-96bit-10%BLER-6kbps-BPSK-6dBi BS antenna gain

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D1T1-A1 | 1-1 | 67.1 | 75.41 | 67.10 | 36.59 |
|  | 1-2 | 67.1 | 70.41 | 67.10 | 36.59 |
| D1T1-A2 | 1-1 | 67.1 | 63.18 | 63.18 | 23.73 |
|  | 1-2 | 67.1 | 63.11 | 63.11 | 23.54 |
| D1T1-B | 1-4 | 67.1 | 84.54 | 67.10 | 36.59 |
| Device 2a |  |  |  |  |  |
| D1T1-A1 | 1-1 | 76.1 | 81.67 | 76.10 | 95.51 |
|  | 1-2 | 76.1 | 76.67 | 76.10 | 95.51 |
| D1T1-A2 | 1-1 | 76.1 | 69.44 | 69.44 | 47.07 |
|  | 1-2 | 76.1 | 69.37 | 69.37 | 46.72 |
| D1T1-B | 1-4 | 76.1 | 97.06 | 76.10 | 95.51 |
| Device 2a (optional, Budget-Alt2 for R2D) |  |  |  |  |  |
| D1T1-A1 | 1-1 | 127.8/127.6/122.3/120.9 | 81.67 | 81.67 | 171.89 |
|  | 1-2 | 127.8/127.6/122.3/120.9 | 76.67 | 76.67 | 101.48 |
| D1T1-A2 | 1-1 | 127.8/127.6/122.3/120.9 | 69.44 | 69.44 | 47.07 |
|  | 1-2 | 127.8/127.6/122.3/120.9 | 69.37 | 69.37 | 46.72 |

Deployment scenario 1, Topology 1

For device1/2a/2b in each case of D1T1 scenarios, more than 15m coverage can be satisfied. Specifically,

- For device 1 when data rate in LLS is assumed as 0.8kbps,

  - For D1T1-A1, D1T1-B scenario, the coverage is limited by R2D link. The coverage is about 36.6m with 6dBi BS antenna.

  - For D1T1-A2, the coverage is limited by D2R link due to non-ideal self-interference cancellation capability. It is observed that results of cases when CW in UL spectrum and CW in DL spectrum are quite close. The coverage is about 17m for device 1 with OOK modulation, and about 24.1m for device 1 with BPSK modulation.

- For device 2a when data rate in LLS is assumed as 0.8kbps,

  - For D1T1-A1 and D1T1-B scenarios, the coverage is limited by R2D link. The coverage is about 95.5m.

  - For D1T1-A2, the coverage is limited by D2R link. The coverage is about 33.3m for device 2a with OOK modulation, and about 46m for device 2a with BPSK modulation.

- For device 1 when data rate in LLS is assumed as 6kbps,

  - For D1T1-A1 and D1T1-B scenario, the coverage is limited by R2D link, and about 36.6m coverage can be observed.

  - For D1T1-A2, when the BS antenna gain is assumed as 6dBi, the coverage is limited by D2R link. It is observed that results of cases when CW in UL spectrum and CW in DL spectrum are quite close. The coverage is about 16.6m for device 1 with OOK modulation, and about 23.6m for device 1 with BPSK modulation.

- For device 2a when data rate in LLS is assumed as 6kbps,

  - For D1T1-A1, with OOK modulation, the coverage in CW case 1-1 is limited by R2D link. And the coverage is about 95.5m. While the coverage in CW case 1-2 is limited by D2R link, and coverage is about 73.9m.

  - For D1T1-A1 with BPSK modulation and D1T1-B scenarios, the coverage is limited by R2D link, and about 95.5m coverage can be observed.

  - For D1T1-A2, the coverage is limited by D2R link. The coverage is about 32.5~34m for device 2a with OOK modulation, and about 45~47m for device 2a with BPSK modulation.

- For device 2b,

  - The coverage is limited by R2D link. The coverage is about 95.5m with 6dBi BS antenna.

Observation 1: For device1/2a/2b in D1T1 scenarios, more than 15m coverage distance can be satisfied. For device1 in D1T1 scenarios, coverage can reach to more than 15m, and for device 2a/2b, coverage distance can be more than 30m.

Observation 2: For device1 in D1T1-A1 and D1T1-B scenarios, the coverage is limited by R2D link, and the coverage is about 36.6m with 6dBi BS antenna.

Observation 3: For device1 in D1T1-A2 scenario, the coverage is limited by D2R link due to self-interference issue, and the coverage evaluation results are a little different for different data rate assumptions.

- When data rate in LLS is assumed as 0.8kbps, the coverage is about 17m for device 1 with OOK modulation, and about 24.1m for device 1 with BPSK modulation.

- When data rate in LLS is assumed as 6kbps, the coverage is about 16.6m for device 1 with OOK modulation, and about 23.6m for device 1 with BPSK modulation.

Observation 4: For device2a in D1T1-A1 scenario, the coverage limited by R2D or D2R link depends on the data rate assumptions and modulation methods.

- When data rate in LLS is assumed as 0.8kbps, the coverage is limited by R2D link, and the coverage is about 95.5m.

- When data rate in LLS is assumed as 6kbps, with OOK modulation, the coverage in CW case 1-1 is limited by R2D link with about 95.5m of coverage. While the coverage in CW case 1-2 is limited by D2R link, and coverage is about 73.9m.

- When data rate in LLS is assumed as 6kbps, with BPSK modulation, the coverage is limited by R2D link, and about 95.5m coverage is observed.

Observation 5: For device2a in D1T1-A2 scenario, the coverage is limited by D2R link due to self-interference issue, and the coverage evaluation results are a little different for different modulation methods.

- When data rate in LLS is assumed as 0.8kbps, the coverage is about 33.3m with OOK modulation, and about 46m with BPSK modulation.

- When data rate in LLS is assumed as 6kbps, the coverage is about 32.5~34m with OOK modulation, and about 45~47m with BPSK modulation.

Observation 6: For device2a in D1T1-B scenarios, the coverage is limited by R2D link. It is observed the coverage is about 95.5m.

### D2T2 scenarios-InH-Office LOS (TDL-D)

Table 3.1.3-1. D2T2-96bit-10%BLER-0.8kbps-OOK

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D2T2-A1 | 2-2 | 51.1 | 65.38 | 51.10 | 13.61 |
| D2T2-A2 | 2-2 | 51.1 | 52.26 | 51.10 | 13.61 |
| D2T2-B | 2-3 (6dBi CW antenna gain) | 51.1 | 96.66 | 51.10 | 13.61 |
|  | 2-4 (6dBi CW antenna gain) | 51.1 | 86.66 | 51.10 | 13.61 |
|  | 2-4 (0dBi CW antenna gain) | 51.1 | 81.98 | 51.10 | 13.61 |
| Device 2a |  |  |  |  |  |
| D2T2-A1 | 2-2 | 60.1 | 71.23 | 60.10 | 45.09 |
| D2T2-A2 | 2-2 | 60.1 | 58.11 | 58.11 | 34.60 |
| D2T2-B | 2-3 (6dBi CW antenna gain) | 60.1 | 108.36 | 60.10 | 45.09 |
|  | 2-4 (6dBi CW antenna gain) | 60.1 | 98.36 | 60.10 | 45.09 |
|  | 2-4 (0dBi CW antenna gain) | 60.1 | 93.68 | 60.10 | 45.09 |
| Device 2b RF-ED |  |  |  |  |  |
| D2T2-C | N/A | 60.1 | 90.84 | 60.10 | 45.09 |

Table 3.1.3-2. D2T2-96bit-10%BLER-0.8kbps-BPSK

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D2T2-A1 | 2-2 | 51.1 | 68.38 | 51.10 | 13.61 |
| D2T2-A2 | 2-2 | 51.1 | 55.26 | 51.10 | 13.61 |
| D2T2-B | 2-3 (6dBi CW antenna gain) | 51.1 | 102.66 | 51.10 | 13.61 |
|  | 2-4 (6dBi CW antenna gain) | 51.1 | 92.66 | 51.10 | 13.61 |
|  | 2-4 (0dBi CW antenna gain) | 51.1 | 87.98 | 51.10 | 13.61 |
| Device 2a |  |  |  |  |  |
| D2T2-A1 | 2-2 | 60.1 | 74.23 | 60.10 | 45.09 |
| D2T2-A2 | 2-2 | 60.1 | 61.11 | 60.10 | 45.09 |
| D2T2-B | 2-3 (6dBi CW antenna gain) | 60.1 | 114.36 | 60.10 | 45.09 |
|  | 2-4 (6dBi CW antenna gain) | 60.1 | 104.36 | 60.10 | 45.09 |
|  | 2-4 (0dBi CW antenna gain) | 60.1 | 99.68 | 60.10 | 45.09 |

Table 3.1.3-3. D2T2-96bit-10%BLER-6kbps-OOK

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D2T2-A1 | 2-2 | 51.1 | 61.66 | 51.10 | 13.61 |
| D2T2-A2 | 2-2 | 51.1 | 52.40 | 51.10 | 13.61 |
| D2T2-B | 2-3 (6dBi CW antenna gain) | 51.1 | 89.21 | 51.10 | 13.61 |
|  | 2-4 (6dBi CW antenna gain) | 51.1 | 79.21 | 51.10 | 13.61 |
|  | 2-4 (0dBi CW antenna gain) | 51.1 | 74.53 | 51.10 | 13.61 |
| Device 2a |  |  |  |  |  |
| D2T2-A1 | 2-2 | 60.1 | 67.17 | 60.10 | 45.09 |
| D2T2-A2 | 2-2 | 60.1 | 57.91 | 57.91 | 33.68 |
| D2T2-B | 2-3 (6dBi CW antenna gain) | 60.1 | 100.23 | 60.10 | 45.09 |
|  | 2-4 (6dBi CW antenna gain) | 60.1 | 90.23 | 60.10 | 45.09 |
|  | 2-4 (0dBi CW antenna gain) | 60.1 | 85.55 | 60.10 | 45.09 |
| Device 2b RF-ED |  |  |  |  |  |
| D2T2-C | N/A | 60.1 | 82.60 | 60.10 | 45.09 |

Table 3.1.3-4. D2T2-96bit-10%BLER-6kbps-BPSK

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D2T2-A1 | 2-2 | 51.1 | 64.66 | 51.10 | 13.61 |
| D2T2-A2 | 2-2 | 51.1 | 55.40 | 51.10 | 13.61 |
| D2T2-B | 2-3 (6dBi CW antenna gain) | 51.1 | 95.21 | 51.10 | 13.61 |
|  | 2-4 (6dBi CW antenna gain) | 51.1 | 85.21 | 51.10 | 13.61 |
|  | 2-4 (0dBi CW antenna gain) | 51.1 | 80.53 | 51.10 | 13.61 |
| Device 2a |  |  |  |  |  |
| D2T2-A1 | 2-2 | 60.1 | 70.17 | 60.10 | 45.09 |
| D2T2-A2 | 2-2 | 60.1 | 60.91 | 60.10 | 45.09 |
| D2T2-B | 2-3 (6dBi CW antenna gain) | 60.1 | 106.23 | 60.10 | 45.09 |
|  | 2-4 (6dBi CW antenna gain) | 60.1 | 96.23 | 60.10 | 45.09 |
|  | 2-4 (0dBi CW antenna gain) | 60.1 | 91.55 | 60.10 | 45.09 |

Deployment scenario 2, Topology 2

D2T2-96bit-10% -OOK or BPSK

- For device 1, the coverage is limited by R2D link, and the coverage is about 13.6m in each case.

- For device 2a, the coverage is limited by R2D link in most cases, and the coverage limited by R2D is about 45m. While in D2T2-A2 scenario, the coverage of device 2a with OOK modulation is limited by D2R link. In this case, the coverage is about 34.6m when data rate in LLS is assumed as 0.8kbps, and 33.7m when data rate in LLS is assumed as 6kbps.

- For device 2b, the coverage is limited by R2D link, and the coverage is about 45m in each case.

Observation 7: For device 1 in D2T2 scenarios, the coverage of R2D is the bottleneck due to limited transmit power (23 dBm) from intermediate UE and device activation threshold. It is observed that coverage distance is about 13.6m.

Observation 8: For device 2a in D2T2 scenarios, the coverage of R2D is the bottleneck in most cases except D2T2-A2 scenario. While in D2T2-A2 scenario, the coverage of device 2a with OOK modulation is limited by D2R link. In this case, the coverage is about 34.6m when data rate is assumed as 0.8kbps, and 33.7m when data rate is assumed as 6kbps.

Observation 9: For device 2b in D2T2 scenarios, the coverage of R2D is the bottleneck due to limited transmit power (23 dBm) from intermediate UE. It is observed that coverage distance is about 45m.

## Link performance (LLS)

### Evaluation Methodologies

OOK Manchester decoding

For Ambient-IoT downlink, if the Manchester coding is used, it should be very simple reception schemes for ~1uW device, especially considering digital BB processing capability/complexity is restricted for such devices.

One example of such Manchester decoding is provided in [6], which is called the timing-based Manchester decode. In this approach, the ascending/descending edge is used to trigger an interruption and the time between two edges will be captured for decoding. Suppose that the data rate is 2T (i.e., the Manchester coding rate is T), and a decoding procedure is illustrated in the following figure.

The decoder starts timer and captures the first edge. Then, captures the next edge and check if count value equals 2T. Once the count value equals 2T, the decoder is now synchronized with the data clock. It will read current logical level of the incoming pin and save as current bit value (0 as shown in the figure). In subsequent steps, the decoder repeatly captures next edge and compare the stored count value with T. If the value equals T, set current bit to next bit. Else if the value equals 2T, set the opposite of current bit to the next bit. Otherwise, return error. It should be noted that in practice the value of the timer will not be exactly matched to T or 2T. To allow for this it is necessary to create a window of allowable values around the desired times. A typical window can be as large as ±50% of T.

Proposal 4: Timing based Manchester decoding approach by capturing ascending/descending edges is adopted for link level performance evaluation.

### LLS results

For the link-level simulations, a set of LLS assumptions have been agreed. Our detailed parameters assumed for LLS are listed in annex II. And simulation results for different deployment scenarios could refer to the LLS spreadsheets attached with this contribution.

#### R2D

Table 3.2.2-1. Link-level Simulation assumptions for R2D

| Parameter | Assumptions |
| --- | --- |
| Carrier frequency | 900MHz |
| Chip rate | 14/28/112/168 kcps, M=1,2,8,12 |
| Modulation | OOK-4, M = 1, 2, 8, 12 |
| Row Data rate (chip rate*M/2) | 7kbps, 14kbps, 56kbps, 84kbps |
| Signal Bandwidth | 1RB (180kHz) for M=1,2 , 2RB for M=8,12 |
| Subcarrier spacing | 15kHz |
| Channel model | TDL-A 30ns; TDL-D 30ns |
| Device velocity | 3 km/h |
| Number of RX antennas | 1 |
| Time/frequency synchronization | Ideal |
| RF filter | 3rd @10MHz |
| BB filter | 3rd @60kHz |
| TBS | 20 bits, 6 bit CRC |
| ADC | Ideal quantization, 1.92MHz sampling rate |
| Coding | 1/2 Manchester encoding |
| Receiver mode | Device 2a：RF-ED(Energy detector) with samples averaged for detection |

Figure 3.2.2-1: R2D BLER-CNR for 20bit payload size (device 2)

| [0n]Message size | chip_rate | Raw Data rate | M | Transmission BW | [0q] Fe | TDL-A/D | Device type | CNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20-bit | 14 kcps | 7kbps | 1 | 1 | 10% | TDL-A | Device 2 | 4.73 |
| 20-bit | 28 kcps | 14kbps | 2 | 1 | 10% | TDL-A | Device 2 | 4.963 |
| 20-bit | 112 kcps | 56kbps | 8 | 2 | 10% | TDL-A | Device 2 | 7.22 |
| 20-bit | 168 kcps | 84kbps | 12 | 2 | 10% | TDL-A | Device 2 | 8.65 |

#### D2R

Table 3.2.2-2. Link-level Simulation assumptions for D2R

| Parameter | Assumptions |
| --- | --- |
| Carrier frequency | 900MHz |
| Data rate | 0.8 kbps; 6 kbps |
| Chip rate | 0.8 kbps data rate: BPSK chip rate = 7 kcps; 6 kbps data rate: BPSK chip rate = 42 kcps |
| Block structure | preamble (131 chip)+ {96b, 24b CRC, 1/3 TBCC, Manchester, BPSK, midamble(5 chip)} + postamble(131 chip) |
| Subcarrier spacing | 15kHz |
| Channel model | TDL-A 30ns; TDL-D 30ns |
| Device velocity | 3 km/h |
| Number of RX antennas | 2 for BS; 1 for intermediate UE |
| Message size | 96 bits; 20 bits; 400bits |
| Sampling frequency | Sampling frequency at receiver side is higher than 32X chip rate, Device 1 SFO: Fe = 10% Device 2a/2b SFO: Fe = 1% |
| CFO | 100ppm for device 2b |
| Coding | 1/2 Manchester encoding |
| Receiver bandwidth | 0.8 kbps data rate: DSB 15 kHz for device 1/2a; DSB 105kHz for device 2b 6 kbps data rate: DSB 90 kHz for device 1/2a; DSB 180kHz for device 2b |
| Waveform | Unmodulated single tone |
| Modulation | BPSK/OOK for device 1/2a, OOK for device 2b |
| FEC | 1/3 TBCC |
| D2R receiver | Coherent receiver for device 1/2a, based on preamble, midamble and postamble. Receiver with IQ branches Non-coherent receiver for device 2b. Receiver with envelope detector. |

The required SNRs for D2R are listed as follows,

Table 3.2.2-3. Required SNR for D2R (10% BLER)

| [0n]Message size | [2a2] chip_rate | [0p] BLER target | [0q] Fe | Pathloss | TDL-A/D | Device type | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 96-bit | 7kcps (0.8kbps) | 10% | 10% | InF-DH NLOS | TDL-A | Device 1 | -2.44 |
| 96-bit | 7kcps (0.8kbps) | 10% | 1% | InF-DH NLOS | TDL-A | Device 2a | -4.27 |
| 96-bit | 7kcps (0.8kbps) | 10% | 1% | InF-DH NLOS | TDL-A | Device 2b | -1.23 |
| 96-bit | 42kcps (6kbps) | 10% | 10% | InF-DH NLOS | TDL-A | Device 1 | -2.17 |
| 96-bit | 42kcps (6kbps) | 10% | 1% | InF-DH NLOS | TDL-A | Device 2a | -4.69 |
| 96-bit | 42kcps (6kbps) | 10% | 1% | InF-DH NLOS | TDL-A | Device 2b | 6.84 |
| 96-bit | 7kcps (0.8kbps) | 10% | 10% | InH-Office LOS | TDL-D | Device 1 | -4.33 |
| 96-bit | 7kcps (0.8kbps) | 10% | 1% | InH-Office LOS | TDL-D | Device 2a | -6.03 |
| 96-bit | 7kcps (0.8kbps) | 10% | 1% | InH-Office LOS | TDL-D | Device 2b | -4.29 |
| 96-bit | 42kcps (6kbps) | 10% | 10% | InH-Office LOS | TDL-D | Device 1 | -4.66 |
| 96-bit | 42kcps (6kbps) | 10% | 1% | InH-Office LOS | TDL-D | Device 2a | -5.68 |
| 96-bit | 42kcps (6kbps) | 10% | 1% | InH-Office LOS | TDL-D | Device 2b | 3.95 |

Table 3.2.2-4. Required SNR for D2R (1% BLER)

| [0n]Message size | [2a2] chip_rate | [0p] BLER target | [0q] Fe | Pathloss | TDL-A/D | Device type | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 96-bit | 7kcps (0.8kbps) | 1% | 10% | InF-DH NLOS | TDL-A | Device 1 | 10.91 |
| 96-bit | 7kcps (0.8kbps) | 1% | 1% | InF-DH NLOS | TDL-A | Device 2a | 6.75 |
| 96-bit | 7kcps (0.8kbps) | 1% | 1% | InF-DH NLOS | TDL-A | Device 2b | 2.63 |
| 96-bit | 42kcps (6kbps) | 1% | 10% | InF-DH NLOS | TDL-A | Device 1 | 11.00 |
| 96-bit | 42kcps (6kbps) | 1% | 1% | InF-DH NLOS | TDL-A | Device 2a | 6.57 |
| 96-bit | 42kcps (6kbps) | 1% | 1% | InF-DH NLOS | TDL-A | Device 2b | 10.80 |
| 96-bit | 7kcps (0.8kbps) | 1% | 1% | InH-Office LOS | TDL-D | Device 2a | 7.46 |
| 96-bit | 7kcps (0.8kbps) | 1% | 1% | InH-Office LOS | TDL-D | Device 2b | -2.30 |
| 96-bit | 42kcps (6kbps) | 1% | 10% | InH-Office LOS | TDL-D | Device 1 | 9.60 |
| 96-bit | 42kcps (6kbps) | 1% | 1% | InH-Office LOS | TDL-D | Device 2a | 5.11 |
| 96-bit | 42kcps (6kbps) | 1% | 1% | InH-Office LOS | TDL-D | Device 2b | 6.50 |

# A-IoT conclusions for evaluation results

According to the preliminary observations on the numerical statistics results summarized in the last meeting, it is recommended to consider the following conclusions on the study of A-IoT evaluation:

Proposal 5: Capture the following conclusions for evaluation results in the TR.

RAN1 has identified evaluation scenarios and layout for device 1, device 2a and device 2b in Deployment scenario 1 with topology 1 and Deployment scenario 2 with topology 2. RAN1 has defined detailed assumptions of each evaluation scenario for the study of A-IoT, including CW inside/outside topology, applicable device type, CW spectrum, D2R spectrum and R2D spectrum.

RAN1 has identified three aspects of design targets for the study of A-IoT beyond those of TR 38.848, including applicable maximum distance target value, refined definition of latency and 2D distribution of devices.

RAN1 has identified the performance metrics for the studies of A-IoT including coverage, latency for a single A-IoT device and inventory completion time for multiple A-IoT devices.

RAN1 has developed the coverage evaluation methodologies including optional methods to derive receiver sensitivity for link budget calculation. RAN1 has carried out link level simulations for R2D and D2R links in coverage evaluation with detailed parameter assumptions, and the LLS results including required CNR/CINR of R2D link and required SNR/SINR of D2R link are used for calculating receiver sensitivity if budget-Alt2 is considered.

RAN1 has carried out coverage evaluation to calculate MPL and distance for both D1T1 and D2T2 scenarios using corresponding pathloss models. It can be observed that the coverage range of different device types varies in D1T1 and D2T2 scenarios. And for each device type in different evaluation scenarios, maximum achievable coverage can be diverse based on different assumptions including Tx power, receiver sensitivity, BLER target, data rate, receiver architecture, CW cancellation capability, modulation and coding schemes, etc.

- For Device 1 in D1T1 scenarios, compared with the applicable maximum distance target value of 15m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 33dBm and R2D receiver sensitivity of -36dBm, coverage of Device 1 in D1T1-A1/D1T1-B can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 1 in D1T1-A1 can satisfy the target.

  - For cases with different assumptions on CW cancellation capability,

    - Assuming CW cancellation capability of >=120dB and < 140dB, coverage of Device 1 in D1T1-B can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of >=35dBm and R2D receiver sensitivity of -36dBm, coverage of Device 1 in D1T1-A1/D1T1-B can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming BPSK is used, coverage of Device 1 in D1T1-A1 can satisfy the target.

    - Assuming MSK is used, coverage of Device 1 in D1T1-A1/D1T1-A2/D1T1-B can satisfy the target.

- For Device 2a in D1T1 scenarios, compared with the applicable maximum distance target value of 25m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 33dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2a in D1T1-A1/D1T1-B can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 10%/1% and data rate of ~1kbps, coverage of Device 2a in D1T1-A1/D1T1-B can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 2a in D1T1-A1 can satisfy the target.

  - For cases with different assumptions on CW cancellation capability,

    - Assuming CW cancellation capability of >=120dB and < 140dB, coverage of Device 2a in D1T1-B can satisfy the target.

    - Assuming CW cancellation capability >=150dB, coverage of Device 2a in D1T1-A1/D1T1-A2 can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of >=30dBm and <35dBm and R2D receiver sensitivity of <=-45dBm or -40dBm, coverage of Device 2a in D1T1-A1/D1T1-A2/D1T1-B can satisfy the target.

    - Assuming R2D Tx EIRP of >=35dBm and R2D receiver sensitivity of <=-45dBm or -40dBm, coverage of Device 2a in D1T1-A1/D1T1-B can satisfy the target.

  - For cases with FEC coding or without FEC,

    - Assuming FEC code (1/2, 1/3, 1/4) is used, coverage of Device 2a in D1T1-A1 can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming BPSK/MSK is used, coverage of Device 2a in D1T1-A1/D1T1-A2/D1T1-B can satisfy the target.

- For Device 2b in D1T1 scenarios, compared with the applicable maximum distance target value of 40m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 33dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming R2D Tx power of 23/24dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2b in D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming R2D Tx power of 33dBm and R2D receiver sensitivity of >=-40dBm, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 10% and data rate of ~1kbps, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

    - Assuming D2R BLER of 10% and data rate of 5~7kbps, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

    - Assuming D2R BLER of 1% and data rate of ~1kbps, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming D2R BLER of 1% and data rate of 5~7kbps, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming D2R non-coherent receiver is used, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of no more than 30dBm and R2D receiver sensitivity of <=-45dBm, coverage of Device 2b in D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming R2D Tx EIRP of >=35dBm and R2D receiver sensitivity of <=-45dBm, coverage of Device 2b in D1T1-C(Budget-Alt1) and D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming R2D Tx EIRP of >=35dBm and R2D receiver sensitivity of -40dBm, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For cases with FEC coding or without FEC,

    - Assuming FEC code (1/2, 1/3, 1/4) is used, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming FEC is not used, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming OOK/MSK is used, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

    - Assuming BPSK is used, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

- For Device 1 in D2T2 scenarios, compared with the applicable maximum distance target value of 10m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 23/24dBm and R2D receiver sensitivity of -36dBm, coverage of Device 1 in D2T2-A1/D2T2-B can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 10%/1% and data rate of ~1kbps, coverage of Device 1 in D2T2-A2 can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of no more than 30dBm and R2D receiver sensitivity of -36dBm, coverage of Device 1 in D2T2-A1/D2T2-B can satisfy the target.

- For Device 2a in D2T2 scenarios, compared with the applicable maximum distance target value of 15m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 23/24dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2a in D2T2-A1/D2T2-B can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 10%/1% and data rate of ~1kbps, coverage of Device 2a in D2T2-A2/D2T2-B can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 2a in D2T2-A1/D2T2-A2/D2T2-B can satisfy the target.

  - For cases with different assumptions on CW cancellation capability,

    - Assuming CW cancellation capability of < 120dB, coverage of Device 2a in D2T2-B can satisfy the target.

    - Assuming CW cancellation capability of >=120dB and < 140dB, coverage of Device 2a in D2T2-A1 can satisfy the target.

    - Assuming CW cancellation capability >=140dB, coverage of Device 2a in D2T2-A1/D2T2-A2/D2T2-B can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of no more than 30dBm and R2D receiver sensitivity of <=-45dBm, coverage of Device 2a in D2T2-A1/D2T2-B can satisfy target.

  - For cases with FEC coding or without FEC,

    - Assuming FEC code (1/2, 1/3, 1/4) is used, coverage of Device 2a in D2T2-A2/D2T2-B can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming BPSK/MSK is used, coverage of Device 2a in D2T2-A1/D2T2-A2/D2T2-B can satisfy the target.

- For Device 2b in D2T2 scenarios, compared with the applicable maximum distance target value of 40m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 23/24dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2b in D2T2-C(Budget-Alt1), D2T2-C(Budget-Alt2) can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 1% and data rate of ~1kbps, coverage of Device 2b in D2T2-C(Budget-Alt2) can satisfy the target.

    - Assuming D2R BLER of 1% and data rate of 5~7kbps, coverage of Device 2b in D2T2-C(Budget-Alt2) can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 2b in D2T2-C(Budget-Alt1) and D2T2-C(Budget-Alt2) can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of no more than 30dBm and R2D receiver sensitivity of <=-45dBm, coverage of Device 2b in D2T2-C(Budget-Alt1) and D2T2-C(Budget-Alt2) can satisfy the target.

  - For cases with FEC coding or without FEC,

    - Assuming FEC code (1/2, 1/3, 1/4) is used, coverage of Device 2b in D2T2-C(Budget-Alt2) can satisfy the target.

    - Assuming FEC is not used, coverage of Device 2b in D2T2-C(Budget-Alt1) can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming OOK is used, coverage of Device 2b in D2T2-C(Budget-Alt2) can satisfy the target.

    - Assuming BPSK is used, coverage of Device 2b in D2T2-C(Budget-Alt1) and D2T2-C(Budget-Alt2) can satisfy the target.

    - Assuming MSK is used, coverage of Device 2b in D2T2-C(Budget-Alt1) can satisfy the target.

RAN1 has conducted the evaluation of single-device latency for both “inventory-only” case and “inventory and command” case. It is observed that single-device latency can be within 1s requirement bound for various data rates and message payload sizes.

# Conclusions

In this contribution, we share our views on the evaluation for Ambient IoT, and provide simulation analysis and evaluation results. The observations and proposals are summarized as follows.

Observation 1: For device1/2a/2b in D1T1 scenarios, more than 15m coverage distance can be satisfied. For device1 in D1T1 scenarios, coverage can reach to more than 15m, and for device 2a/2b, coverage distance can be more than 30m.

Observation 2: For device1 in D1T1-A1 and D1T1-B scenarios, the coverage is limited by R2D link, and the coverage is about 36.6m with 6dBi BS antenna.

Observation 3: For device1 in D1T1-A2 scenario, the coverage is limited by D2R link due to self-interference issue, and the coverage evaluation results are a little different for different data rate assumptions.

- When data rate in LLS is assumed as 0.8kbps, the coverage is about 17m for device 1 with OOK modulation, and about 24.1m for device 1 with BPSK modulation.

- When data rate in LLS is assumed as 6kbps, the coverage is about 16.6m for device 1 with OOK modulation, and about 23.6m for device 1 with BPSK modulation.

Observation 4: For device2a in D1T1-A1 scenario, the coverage limited by R2D or D2R link depends on the data rate assumptions and modulation methods.

- When data rate in LLS is assumed as 0.8kbps, the coverage is limited by R2D link, and the coverage is about 95.5m.

- When data rate in LLS is assumed as 6kbps, with OOK modulation, the coverage in CW case 1-1 is limited by R2D link with about 95.5m of coverage. While the coverage in CW case 1-2 is limited by D2R link, and coverage is about 73.9m.

- When data rate in LLS is assumed as 6kbps, with BPSK modulation, the coverage is limited by R2D link, and about 95.5m coverage is observed.

Observation 5: For device2a in D1T1-A2 scenario, the coverage is limited by D2R link due to self-interference issue, and the coverage evaluation results are a little different for different modulation methods.

- When data rate in LLS is assumed as 0.8kbps, the coverage is about 33.3m with OOK modulation, and about 46m with BPSK modulation.

- When data rate in LLS is assumed as 6kbps, the coverage is about 32.5~34m with OOK modulation, and about 45~47m with BPSK modulation.

Observation 6: For device2a in D1T1-B scenarios, the coverage is limited by R2D link. It is observed the coverage is about 95.5m.

Observation 7: For device 1 in D2T2 scenarios, the coverage of R2D is the bottleneck due to limited transmit power (23 dBm) from intermediate UE and device activation threshold. It is observed that coverage distance is about 13.6m.

Observation 8: For device 2a in D2T2 scenarios, the coverage of R2D is the bottleneck in most cases except D2T2-A2 scenario. While in D2T2-A2 scenario, the coverage of device 2a with OOK modulation is limited by D2R link. In this case, the coverage is about 34.6m when data rate is assumed as 0.8kbps, and 33.7m when data rate is assumed as 6kbps.

Observation 9: For device 2b in D2T2 scenarios, the coverage of R2D is the bottleneck due to limited transmit power (23 dBm) from intermediate UE. It is observed that coverage distance is about 45m.

Proposal 1:

For Applicable maximum distance target,

- For D1T1

  - 15m for device 1

  - 25m for device 2a

  - 40m for device 2b

- For D2T2 with InH-Office scenario

  - 10m for device 1

  - 15m for device 2a

  - 40m for device 2b

For 2D distribution of devices

- The devices are uniformly distributed in the system as a starting point.

- For simplicity, system level simulation is not required.

Proposal 2: Prioritize D1T1-A1/A2/B and D2T2-B for further coverage evaluation.

Proposal 3: Intermediate UE dropping and which devices are involved in the evaluations for D2T2 are up to coexistence evaluation in RAN4.

Proposal 4: Timing based Manchester decoding approach by capturing ascending/descending edges is adopted for link level performance evaluation.

Proposal 5: Capture the following conclusions for evaluation results in the TR.

RAN1 has identified evaluation scenarios and layout for device 1, device 2a and device 2b in Deployment scenario 1 with topology 1 and Deployment scenario 2 with topology 2. RAN1 has defined detailed assumptions of each evaluation scenario for the study of A-IoT, including CW inside/outside topology, applicable device type, CW spectrum, D2R spectrum and R2D spectrum.

RAN1 has identified three aspects of design targets for the study of A-IoT beyond those of TR 38.848, including applicable maximum distance target value, refined definition of latency and 2D distribution of devices.

RAN1 has identified the performance metrics for the studies of A-IoT including coverage, latency for a single A-IoT device and inventory completion time for multiple A-IoT devices.

RAN1 has developed the coverage evaluation methodologies including optional methods to derive receiver sensitivity for link budget calculation. RAN1 has carried out link level simulations for R2D and D2R links in coverage evaluation with detailed parameter assumptions, and the LLS results including required CNR/CINR of R2D link and required SNR/SINR of D2R link are used for calculating receiver sensitivity if budget-Alt2 is considered.

RAN1 has carried out coverage evaluation to calculate MPL and distance for both D1T1 and D2T2 scenarios using corresponding pathloss models. It can be observed that the coverage range of different device types varies in D1T1 and D2T2 scenarios. And for each device type in different evaluation scenarios, maximum achievable coverage can be diverse based on different assumptions including Tx power, receiver sensitivity, BLER target, data rate, receiver architecture, CW cancellation capability, modulation and coding schemes, etc.

- For Device 1 in D1T1 scenarios, compared with the applicable maximum distance target value of 15m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 33dBm and R2D receiver sensitivity of -36dBm, coverage of Device 1 in D1T1-A1/D1T1-B can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 1 in D1T1-A1 can satisfy the target.

  - For cases with different assumptions on CW cancellation capability,

    - Assuming CW cancellation capability of >=120dB and < 140dB, coverage of Device 1 in D1T1-B can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of >=35dBm and R2D receiver sensitivity of -36dBm, coverage of Device 1 in D1T1-A1/D1T1-B can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming BPSK is used, coverage of Device 1 in D1T1-A1 can satisfy the target.

    - Assuming MSK is used, coverage of Device 1 in D1T1-A1/D1T1-A2/D1T1-B can satisfy the target.

- For Device 2a in D1T1 scenarios, compared with the applicable maximum distance target value of 25m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 33dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2a in D1T1-A1/D1T1-B can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 10%/1% and data rate of ~1kbps, coverage of Device 2a in D1T1-A1/D1T1-B can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 2a in D1T1-A1 can satisfy the target.

  - For cases with different assumptions on CW cancellation capability,

    - Assuming CW cancellation capability of >=120dB and < 140dB, coverage of Device 2a in D1T1-B can satisfy the target.

    - Assuming CW cancellation capability >=150dB, coverage of Device 2a in D1T1-A1/D1T1-A2 can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of >=30dBm and <35dBm and R2D receiver sensitivity of <=-45dBm or -40dBm, coverage of Device 2a in D1T1-A1/D1T1-A2/D1T1-B can satisfy the target.

    - Assuming R2D Tx EIRP of >=35dBm and R2D receiver sensitivity of <=-45dBm or -40dBm, coverage of Device 2a in D1T1-A1/D1T1-B can satisfy the target.

  - For cases with FEC coding or without FEC,

    - Assuming FEC code (1/2, 1/3, 1/4) is used, coverage of Device 2a in D1T1-A1 can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming BPSK/MSK is used, coverage of Device 2a in D1T1-A1/D1T1-A2/D1T1-B can satisfy the target.

- For Device 2b in D1T1 scenarios, compared with the applicable maximum distance target value of 40m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 33dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming R2D Tx power of 23/24dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2b in D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming R2D Tx power of 33dBm and R2D receiver sensitivity of >=-40dBm, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 10% and data rate of ~1kbps, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

    - Assuming D2R BLER of 10% and data rate of 5~7kbps, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

    - Assuming D2R BLER of 1% and data rate of ~1kbps, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming D2R BLER of 1% and data rate of 5~7kbps, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming D2R non-coherent receiver is used, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of no more than 30dBm and R2D receiver sensitivity of <=-45dBm, coverage of Device 2b in D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming R2D Tx EIRP of >=35dBm and R2D receiver sensitivity of <=-45dBm, coverage of Device 2b in D1T1-C(Budget-Alt1) and D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming R2D Tx EIRP of >=35dBm and R2D receiver sensitivity of -40dBm, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For cases with FEC coding or without FEC,

    - Assuming FEC code (1/2, 1/3, 1/4) is used, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

    - Assuming FEC is not used, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming OOK/MSK is used, coverage of Device 2b in D1T1-C(Budget-Alt1) can satisfy the target.

    - Assuming BPSK is used, coverage of Device 2b in D1T1-C(Budget-Alt1), D1T1-C(Budget-Alt2) can satisfy the target.

- For Device 1 in D2T2 scenarios, compared with the applicable maximum distance target value of 10m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 23/24dBm and R2D receiver sensitivity of -36dBm, coverage of Device 1 in D2T2-A1/D2T2-B can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 10%/1% and data rate of ~1kbps, coverage of Device 1 in D2T2-A2 can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of no more than 30dBm and R2D receiver sensitivity of -36dBm, coverage of Device 1 in D2T2-A1/D2T2-B can satisfy the target.

- For Device 2a in D2T2 scenarios, compared with the applicable maximum distance target value of 15m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 23/24dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2a in D2T2-A1/D2T2-B can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 10%/1% and data rate of ~1kbps, coverage of Device 2a in D2T2-A2/D2T2-B can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 2a in D2T2-A1/D2T2-A2/D2T2-B can satisfy the target.

  - For cases with different assumptions on CW cancellation capability,

    - Assuming CW cancellation capability of < 120dB, coverage of Device 2a in D2T2-B can satisfy the target.

    - Assuming CW cancellation capability of >=120dB and < 140dB, coverage of Device 2a in D2T2-A1 can satisfy the target.

    - Assuming CW cancellation capability >=140dB, coverage of Device 2a in D2T2-A1/D2T2-A2/D2T2-B can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of no more than 30dBm and R2D receiver sensitivity of <=-45dBm, coverage of Device 2a in D2T2-A1/D2T2-B can satisfy target.

  - For cases with FEC coding or without FEC,

    - Assuming FEC code (1/2, 1/3, 1/4) is used, coverage of Device 2a in D2T2-A2/D2T2-B can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming BPSK/MSK is used, coverage of Device 2a in D2T2-A1/D2T2-A2/D2T2-B can satisfy the target.

- For Device 2b in D2T2 scenarios, compared with the applicable maximum distance target value of 40m.

  - For cases with different R2D Tx power ranges and R2D receiver sensitivity,

    - Assuming R2D Tx power of 23/24dBm and R2D receiver sensitivity of <= -45dBm, coverage of Device 2b in D2T2-C(Budget-Alt1), D2T2-C(Budget-Alt2) can satisfy the target.

  - For cases with different D2R BLER target and data rate,

    - Assuming D2R BLER of 1% and data rate of ~1kbps, coverage of Device 2b in D2T2-C(Budget-Alt2) can satisfy the target.

    - Assuming D2R BLER of 1% and data rate of 5~7kbps, coverage of Device 2b in D2T2-C(Budget-Alt2) can satisfy the target.

  - For A-IoT devices with coherent receiver or non-coherent receiver,

    - Assuming D2R coherent receiver is used, coverage of Device 2b in D2T2-C(Budget-Alt1) and D2T2-C(Budget-Alt2) can satisfy the target.

  - For cases with different R2D Tx EIRP ranges and R2D receiver sensitivity,

    - Assuming R2D Tx EIRP of no more than 30dBm and R2D receiver sensitivity of <=-45dBm, coverage of Device 2b in D2T2-C(Budget-Alt1) and D2T2-C(Budget-Alt2) can satisfy the target.

  - For cases with FEC coding or without FEC,

    - Assuming FEC code (1/2, 1/3, 1/4) is used, coverage of Device 2b in D2T2-C(Budget-Alt2) can satisfy the target.

    - Assuming FEC is not used, coverage of Device 2b in D2T2-C(Budget-Alt1) can satisfy the target.

  - For cases with different modulation schemes,

    - Assuming OOK is used, coverage of Device 2b in D2T2-C(Budget-Alt2) can satisfy the target.

    - Assuming BPSK is used, coverage of Device 2b in D2T2-C(Budget-Alt1) and D2T2-C(Budget-Alt2) can satisfy the target.

    - Assuming MSK is used, coverage of Device 2b in D2T2-C(Budget-Alt1) can satisfy the target.

RAN1 has conducted the evaluation of single-device latency for both “inventory-only” case and “inventory and command” case. It is observed that single-device latency can be within 1s requirement bound for various data rates and message payload sizes.

# References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #102.

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #103.

- Chairman's Notes RAN1#118bis, October 14th – 18th, 2024.

- Chairman's Notes RAN1#116bis, April 15th – 19th, 2024.

- R4-2410567, WF on co-existence evaluation for ambient IoT, 20th May 2024 - 24th May 2024.

- Chairman's Notes RAN1#118, August 19th – 23rd, 2024.

- Manchester Coding Basics, Atmel-9164-Manchester-Coding-Basics_Application-Note.pdf (microchip.com).

# Annex

## Annex I: Link budget assumptions

Table 6.1-1 Link budget template

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | D1T1-A1/A2/B/C D2T2-A1/A2/B/C | D1T1-A1/A2/B/C D2T2-A1/A2/B/C |
| [0A1] | CW case | N/A | 1-1/1-2/1-4/2-2/2-3/2-4 |
| [0B] | Device 1/2a/2b | Device 1/2a/2b | Device 1/2a/2b |
| [0C] | Center frequency (MHz) | 900MHz (M) | 900MHz (M) |
| [0D] | Topology/Pathloss model | For D2T2: - [0D]-Alt2: InH-Office LOS For D1T1: - InF-DH NLOS | For D2T2: - [0D]-Alt2: InH-Office LOS For D1T1: - InF-DH NLOS |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: - 2(M) antenna elements for 0.9 GHz For Intermediate UE: - 1(M) | 1 |
| [1E] | Total Tx Power (dBm) | - For BS in DL spectrum for indoor - [1E]-R2D-Alt1: 33dBm(M), - For UL spectrum for indoor, - [1E]-R2D-Alt4:23dBm (M) | - For device 1/2a: (see note 1) - [1E]-D2R-Alt1: (For scenarios ‘B’) - The Device Tx Power is calculated by CW received power which can be derived by at least CW2D distance (m) value and other related factors. - [1E]-D2R-Alt2: (For scenarios ‘A1’ and ‘A2’) - The Device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss. - For device 2b: (For scenarios ‘C’) - [1E]-D2R-Alt3: -20 dBm(M) |
| [1E1] | CW Tx power (dBm) | N/A | For scenario ‘A1’, ‘A2’ and ‘B’ - 33dBm if CW in DL spectrum - 23dBm if CW in UL spectrum. Note: only applicable for device 1/2a |
| [1E2] | CW Tx antenna gain (dBi) | N/A | - Company to report, the value equals to - 0dBi UE Tx ant gain, or - 6dBi BS Tx ant gain Note: only applicable for device 1/2a |
| [1E3] | CW2D distance (m) | N/A | For scenarios ‘B’ - D1T1-B: - 10m, - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario. (see note 1) - D2T2-B: - 10m, For scenarios ‘A1’ and ‘A2’ - Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note: only applicable for device 1/2a Note: companies to report which value(s) are evaluated. |
| [1E4] | CW2D pathloss (dB) | N/A | Calculated (see note1) Note: only applicable for device 1/2a |
| [1E5] | CW received power (dBm) | N/A | Calculated (see note1) Note: only applicable for device 1/2a |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), | Refer to LLS table [1a] |
| [1G] | Tx antenna gain (dBi) | - For BS for indoor, 6 dBi(M) - For intermediate UE, 0 dBi | - For A-IoT device, 0dBi |
| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | - OOK: 6 dB - PSK: 0 dB It is applicable for device 1 and 2a |
| [1J] | Ambient IoT on-object antenna penalty | Not applicable | 0.9dB |
| [1K] | Ambient IoT backscatter amplifier gain (dB) | N/A | - 10 dB (M) Note: Only for device 2a |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | - For BS, 0 dB - For intermediate UE, 1 dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth 20MHz | Refer to LLS table [2a3] [receiver bandwidth?] - 15kHz for 0.8kbps data rate for device 1/2a - 90kHz for 6kbps data rate for device 1/2a - 180kHz for 0.8/6kbps data rate for device 2b |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | For RF-ED receiver - 20dB, Device 2 For IF/ZIF receiver - 15dB, Device 2 | For BS as reader - 5dB For intermediate UE as reader - 7dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2F] | Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0.9dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt1 | Budget-Alt2 |
| [2K] | CW cancellation (dB) | N/A | Companies to report for scenario A2/A1/B for BS and intermediate UE. - For D1T1-A2, 140dB for BS - For D2T2-A2, 120dB for intermediate UE Note: - Only applicable for device 1/2a - The value provided is for the unmodulated single-tone CW. The impact of a multi-tone CW, e.g., assuming an [X] dB difference, is FFS |
| [2K1] | Remaining CW interference (dB) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2K2] | Receiver sensitivity loss(dB) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2L] | Receiver Sensitivity (dBm) | For Budget-Alt1, - For device 1 (RF-ED), -36dBm - For device 2 (RF-ED), -45dBm | Calculated (see Note 1) Note: the receiver sensitivity includes the receiver sensitivity loss [2K2], i.e. after CW cancellation at least if ‘A2’ scenario is used |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB FFS: other values are not precluded Note: only applicable for D1T1 | 0 dB FFS: other values are not precluded Note: only applicable for D1T1 |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

Note1: calculated values in the Table XXXX are derived according to the followings,

[1E3]

- For scenarios where CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss ‘A1’ and ‘A2’, [1E3] is derived by assuming pathloss is [1E4] and use the pathloss formula as agreed.

[1E4]

- For scenarios where CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss

  - [1E4] = 0.5* ( [1E1] + [1E2] - [1N](R2D) + [2C] (R2D) – [2H](R2D) – 2*[3A] – 2*[3B] +[3D](R2D) + [1K] – [1H] + [1G] – [1J] + [2C] – [2X] – [2L] + [3C] + [3D] )

  - [1K] is only for device 2a

- Otherwise

  - [1E4] is derived according to pathloss formula by assume distance is [1E3]

[1E5]

- [1E5] = [1E1] + [1E2] - [1N](R2D) - [1E4] + [2C] (R2D) – [2H](R2D) – [3A] – [3B] + [3D](R2D)

[1E]

- [1E] = [1E5]+ [1K] – [1H]

- [1K] is only for device 2a

[1M]:

- For R2D,

  - [1M] = [1E] + [1G] - [1N]

- For D2R

  - [1M] = [1E] + [1G] - [1J]

[2F]:

- [2F] = [2D] + [2E] +lin2dB([2B])

[2G]

- For the R2D LLS for ED, CINR/CNR is reported, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and interference (if any) power spectral density in the device ED channel bandwidth.

- For R2D ZIF receiver, report the same metrics (i.e., CNR/CINR, signal transmission bandwidth, ED bandwidth) as agreed for RF-ED/IF receiver.

- For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth.

- On/off keying backscatter loss (including DC removal loss) is not taken into account in the LLS and is included in link budget table [1H].

[2J]

- For R2D link in the coverage evaluation, for device 1

  - Budget-Alt1 is used (note: receiver architecture is RF ED)

- For R2D link in the coverage evaluation for device 2,

  - Budget-Alt1 is used if receiver architecture is RF ED

  - Budget-Alt2 is used if receiver architecture is IF/ZIF ED

- Note1a: this does not preclude to have LLS for device 1 and 2 R2D link with RF-ED if needed.

- Note1b: For device 2 R2D link with RF-ED, Budget-Alt1 is mandatory, Budget-Alt2 is optional.

- Note1c: this does not imply all M values are achievable with the sensitivity given by Budget-Alt1 for RF ED

- Note1d: For device 2 with an RF ED-based receiver on the R2D link, if the receiver sensitivity derived from Budget-Alt2, assuming a noise figure of [X dB], exceeds the receiver sensitivity based on Budget-Alt1, then Budget-Alt2 is applied.

[2K1]:

- [2K1] = [1E1] + [1E2] -[1N](R2D) + [2C] - [2X] - [2K]

[2K2]:

- 2K2=lin2dB1+dB2lin([2K1])dB2lin([2F])

[2L]:

- For R2D and Budget-Alt2,

  - [2L] = [2G] - lin2dB([2B] / [1F]) + [2F]

  - Note 1e: the term ‘lin2dB([2B] / [1F])’ is applied due to scaling from CNR/CINR to SNR/SINR.

- For D2R,

  - [2L] = [2G] + [2F] + [2K2], device 1/2a

  - [2L] = [2G] + [2F], device 2b

[4A]

- For R2D,

  - [4A]=[1M]+[2C] -[2H]-[2L]-[3A]-[3B]+[3C]+[3D]

- For D2R,

  - [4A]=[1M]+[2C] -[2X]-[2L]-[3A]-[3B]+[3C]+[3D]

[4B]

- [4B] is derived by assuming pathloss is [4A] and use the pathloss formula as agreed.

Note2: (M) denotes the value is mandatory to be evaluated. (O) denotes the value can be optionally evaluated.

## Annex II: LLS assumptions

|  | Parameters |  | Assumptions | Company result1 |
| --- | --- | --- | --- | --- |
|  | R2D/D2R common parameters |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template | 900 MHz |
| [0b] | SCS |  | 15 kHz as baseline | 15 kHz |
| [0c] | Block structure |  | Blocks as agreed in 9.4.2.3, or other blocks reported by companies | R2D: 20bit+6bit CRC D2R: preamble (131 chip)+ {96b, 24b CRC, 1/3 TBCC, Manchester, BPSK, midamble(5 chip)} + postamble(131 chip) |
| [0d] | Channel model |  | <Editor’s Note: will be updated according to the agreements made for channel model> | For D1T1: TDL-A For D2T2: TDL-D |
| [0e] | Delay spread |  | - An RMS delay spread of 30 ns (M) and [150] ns (O) is considered for TDL-A channel model. - An RMS delay spread of 30 ns is considered for TDL-D channel model. | TDL-A: 30 ns TDL-D: 30 ns |
| [0f] | Device velocity |  | 3 km/h | 3 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 | 1 |
| [0h1] | BS | Number of antenna elements | 2 or 4 | 2 |
| [0h2] |  | Number of TXRUs | 2 or 4 | 2 |
| [0j1] | Intermediate UE | Number of antenna elements | 1 or 2 | 1 |
| [0j2] |  | Number of TXRUs | 1 or 2 | 1 |
| [0m] | Reference data rate |  | 1 kbps (M), 5 ~ 7 kbps (M), 48 ~ 60 kbps (O), 0.1 kbps for message size of 20 or 96 bits (O) - Other data rate can be reported by companies - Note1: companies to report the exact data rate. - Note 2: the exact data rate is close to the values listed above. - Note 3: The exact data rate is calculated by dividing the message size (excluding CRC) by the total transmission time including applicable overheads (e.g., CRC, pre/mid/post-ambles if present). - Note 4: the exact data rate may be related to coding scheme, repetition and etc. - Note 5: all data rates considered are for evaluation purpose only.. | R2D: 5.4kbps; 10.8kbps; 43.1kbps; 64.6kbps D2R:0.8 kbps; 6 kbps |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. - Note 1: companies to report the M value and chip length used for each message size - Note 2: CRC is not included for the message size | 96 bits; 20bits |
| [0p] | BLER target |  | 1%, 10% | 10% |
| [0q] | Sampling frequency |  | Companies to report the Sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): - (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, - (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, - (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, - FFS: Optionally evaluate a fixed value SFO for device 1 and 2 - Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution - Note: Above values are only for sampling purpose. - FFS other values - Note: Above assumptions are only for LLS evaluation purpose only for R2D and [D2R]. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. - Note: Accuracy can be improved after clock calibration for at least device 2. FFS applicable for device 1 - Note: SFO after clock calibration can be applied to Fe. - FFS other models CFO for device 2b. - [(100ppm(M), 200ppm(O), 1000ppm(O, only as initial CFO), with drift rate of [TBD]ppm/s)]” - Note: companies to report whether they assumed the value as initial CFO or after calibration Note: Above assumptions are for LLS evaluation purpose only | Device 1: Fe = 10% Device 2a: Fe = 1% Device 2b: Fe = 1%. CFO 100ppm |
| [0r] | Device 1/2a/2b |  | Options are as follows, - Device 1, RF-ED - Device 2a, RF-ED - Device 2b, RF-ED/IF-ED/ZIF <Editor’s Note: will be updated according to agreements from 9.4.1.2> | Device 1, Device 2a, Device 2b RF-ED |
|  | R2D specific parameters |  |  |  |
| [1a] | Transmission bandwidth |  | 180 kHz as baseline. Other larger values are not precluded. | 180kHz; 360kHz |
| [1b] | FFS: ED bandwidth |  | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. | 20MHz |
| [1c] | FFS: BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at [Y] kHz, half of R2D transmission bandwidth. Companies to report X = {3, 5}. | 3rd @60kHz |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator | OOK-4, M =1, 2, 8, 12 |
| [1e] | Modulation |  | OOK Companies to report, e.g., OOK-1, OOK-4 with M chips per OFDM symbol | OOK |
| [1f] | Line code |  | Companies to report, e.g., Manchester, PIE | 1/2 Manchester encoding |
| [1g] | FEC |  | No FEC as baseline |  |
| [1h] | ADC bit width |  | 1-bit for device 1 4-bit for device 2 | Ideal quantization |
| [1j] | Detection/decoding method for Line code |  | Companies to report |  |
|  | D2R specific parameters |  |  |  |
| [2a1] | Transmission bandwidth (w.r.t. D2R data rate) |  | - [2a1]-Alt1 (M): - DSB - X kHz is considered for D2R transmission bandwidth. - The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz. - [2a1]-Alt2: - SSB - X kHz is considered for D2R transmission bandwidth. - The value is for one sideband, i.e., the total transmission bandwidth for SSB is X kHz. - For device 2b only, FFS for device 2a. X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies | 0.8 kbps data rate: DSB 14 kHz; 6 kbps data rate: DSB 84 kHz |
| [2a2] | [OOK/BPSK/BFSK chip rate] |  | Companies to report | 0.8 kbps data rate: BPSK chip rate = 7 kcps; 6 kbps data rate: BPSK chip rate = 42 kcps |
| [2a3] | Receiver bandwidth |  | D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. - Assume the receiver matches the transmitter's modulation, i.e., to receiver uses SSB when transmitter uses SSB, receiver uses DSB when transmitter uses DSB. Companies to report the value, and further down-selection of the values and DSB/SSB is not precluded. | 0.8 kbps data rate: DSB 15 kHz for device 1/2a; 6 kbps data rate: DSB 90 kHz for device 1/2a 180kHz for device 2b |
| [2b] | Waveform (CW) |  | Companies to report waveform, e.g., unmodulated single tone, multi-tone(multiple unmodulated single tone) | Unmodulated single tone |
| [2d] | Modulation |  | Companies to report modulation, e.g., OOK, BPSK, BFSK | BPSK; OOK |
| [2e] | Line code |  | Companies to report, e.g., Manchester encoding, FM0 encoding, Miller encoding, no line coding | Manchester |
| [2g] | FEC |  | Companies to report, e.g., CC, No FEC | 1/3 CC |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit |  |
| [2j] | D2R receiver |  | Companies to report, e.g., coherent receiver / non-coherent receiver | Coherent receiver for device 1/2a non-coherent receiver for device 2b |
|  | Other assumptions |  |  |  |
| [3a] | Other assumptions |  | To be reported by company |  |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |  |

## Annex III(Informative)

### Link budget template agreed in RAN1#117

Table. XXX. Link budget template

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | D1T1-A1/A2/B/C D2T2-A1/A2/B/C | D1T1-A1/A2/B/C D2T2-A1/A2/B/C |
| [0A1] | CW case | N/A | 1-1/1-2/1-4/2-2/2-3/2-4 |
| [0B] | Device 1/2a/2b | Device 1/2a/2b | Device 1/2a/2b |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Topology/Pathloss model | For D2T2: - [0D]-Alt1: InF-DL NLOS - [0D]-Alt2: InH-Office LOS For D1T1: - InF-DH NLOS | For D2T2: - [0D]-Alt1: InF-DL NLOS - [0D]-Alt2: InH-Office LOS For D1T1: - InF-DH NLOS |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: - 2(M) or 4(O) antenna elements for 0.9 GHz For Intermediate UE: - 1(M) or 2(O) | 1 |
| [1E] | Total Tx Power (dBm) | - For BS in DL spectrum for indoor - [1E]-R2D-Alt1: 33dBm(M), - [1E]-R2D-Alt2: 38dBm(O), - [1E]-R2D-Alt3: 24dBm(M) - Companies to report if PSD constraints are imposed (company to report the condition for applying PSD constraints in Row [5A]: Other notes) - For UL spectrum for indoor, - [1E]-R2D-Alt4:23dBm (M) - [1E]-R2D-Alt5:26dBm(O) | - For device 1/2a: (see note 1) - [1E]-D2R-Alt1: (For scenarios ‘B’) - The Device Tx Power is calculated by CW received power which can be derived by at least CW2D distance (m) value and other related factors. - [1E]-D2R-Alt2: (For scenarios ‘A1’ and ‘A2’) - The Device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss. - For device 2b: (For scenarios ‘C’) - [1E]-D2R-Alt3: -20 dBm(M) - [1E]-D2R-Alt4: -10 dBm(O) |
| [1E1] | CW Tx power (dBm) | N/A | For scenario ‘A1’, ‘A2’ and ‘B’ - Report a value from the candidate values [1E]-R2D-Alt1/[1E]-R2D-Alt2/[1E]-R2D-Alt3 from [1E]-R2D if CW in DL spectrum - Report a value from the candidate values [1E]-R2D-Alt4/[1E]-R2D-Alt5 from [1E]-R2D if CW in UL spectrum. Note: only applicable for device 1/2a |
| [1E2] | CW Tx antenna gain (dBi) | N/A | - Company to report, the value equals to - UE Tx ant gain, or - BS Tx ant gain Note: only applicable for device 1/2a |
| [1E3] | CW2D distance (m) | N/A | For scenarios ‘B’ - D1T1-B: - 5m, - 10m, - 20m - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario. (see note 1) - D2T2-B: - 5m, - 10m, - FFS other values For scenarios ‘A1’ and ‘A2’ - Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note: only applicable for device 1/2a Note: companies to report which value(s) are evaluated. |
| [1E4] | CW2D pathloss (dB) | N/A | Calculated (see note1) Note: only applicable for device 1/2a |
| [1E5] | CW received power (dBm) | N/A | Calculated (see note1) Note: only applicable for device 1/2a |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), 360kHz(O), 1.08MHz(O) | Refer to LLS table [1a] |
| [1G] | Tx antenna gain (dBi) | - For BS for indoor, 6 dBi(M), 2dBi(M) - For intermediate UE, 0 dBi | - For A-IoT device, 0dBi |
| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | - OOK: 6 dB - PSK: 0 dB - FSK: Y dB It is applicable for device 1 and 2a Companies to report and justify their assumptions for Y. Companies to report in row 3D if they assume any additional related loss. |
| [1J] | Ambient IoT on-object antenna penalty | Not applicable | 0.9dB or 4.7dB |
| [1K] | Ambient IoT backscatter amplifier gain (dB) | N/A | - 10 dB (M) - 15 dB (O) Note: Only for device 2a |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | - For BS, X dB, X <=3 to be reported by companies with justification provided in row 5A - For intermediate UE, 1 dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) FFS: any limitation of the EIRP subject to future discussion | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a] [receiver bandwidth?] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | For RF-ED receiver - 20dB, Device 2 - FFS other values For IF/ZIF receiver - 15dB, Device 2 | For BS as reader - 5dB For intermediate UE as reader - 7dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2F] | Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0.9dB or 4.7dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt1/ Budget-Alt2 (see note1) | Budget-Alt2 |
| [2K] | CW cancellation (dB) | N/A | Companies to report for scenario A2/A1/B for BS and intermediate UE. Note: - Only applicable for device 1/2a - The value provided is for the unmodulated single-tone CW. The impact of a multi-tone CW, e.g., assuming an [X] dB difference, is FFS |
| [2K1] | Remaining CW interference (dB) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2K2] | Receiver sensitivity loss(dB) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2L] | Receiver Sensitivity (dBm) | For Budget-Alt1, - For device 1 (RF-ED), for example: - {-30dBm, -36dBm, -40dBm, etc} - For device 2 (RF-ED), for example: - {-40dBm, -45dBm, etc} For Budget-Alt2, - Calculated (see note1) | Calculated (see Note 1) Note: the receiver sensitivity includes the receiver sensitivity loss [2K2], i.e. after CW cancellation at least if ‘A2’ scenario is used |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB FFS: other values are not precluded Note: only applicable for D1T1 | 0 dB FFS: other values are not precluded Note: only applicable for D1T1 |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

Note1: calculated values in the Table XXXX are derived according to the followings,

[1E3]

- For scenarios where CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss ‘A1’ and ‘A2’, [1E3] is derived by assuming pathloss is [1E4] and use the pathloss formula as agreed.

[1E4]

- For scenarios where CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss

  - [1E4] = 0.5* ( [1E1] + [1E2] - [1N](R2D) + [2C] (R2D) – [2H](R2D) – 2*[3A] – 2*[3B] +[3D](R2D) + [1K] – [1H] + [1G] – [1J] + [2C] – [2X] – [2L] + [3C] + [3D] )

  - [1K] is only for device 2a

- Otherwise

  - [1E4] is derived according to pathloss formula by assume distance is [1E3]

[1E5]

- [1E5] = [1E1] + [1E2] - [1N](R2D) - [1E4] + [2C] (R2D) – [2H](R2D) – [3A] – [3B] + [3D](R2D)

[1E]

- [1E] = [1E5]+ [1K] – [1H]

- [1K] is only for device 2a

[1M]:

- For R2D,

  - [1M] = [1E] + [1G] - [1N]

- For D2R

  - [1M] = [1E] + [1G] - [1J]

[2F]:

- [2F] = [2D] + [2E] +lin2dB([2B])

[2G]

- For the R2D LLS for ED, CINR/CNR is reported, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and interference (if any) power spectral density in the device ED channel bandwidth.

- For R2D ZIF receiver, report the same metrics (i.e., CNR/CINR, signal transmission bandwidth, ED bandwidth) as agreed for RF-ED/IF receiver.

- For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth.

- On/off keying backscatter loss (including DC removal loss) is not taken into account in the LLS and is included in link budget table [1H].

[2J]

- For R2D link in the coverage evaluation, for device 1

  - Budget-Alt1 is used (note: receiver architecture is RF ED)

- For R2D link in the coverage evaluation for device 2,

  - Budget-Alt1 is used if receiver architecture is RF ED

  - Budget-Alt2 is used if receiver architecture is IF/ZIF ED

- Note1a: this does not preclude to have LLS for device 1 and 2 R2D link with RF-ED if needed.

- Note1b: For device 2 R2D link with RF-ED, Budget-Alt1 is mandatory, Budget-Alt2 is optional.

- Note1c: this does not imply all M values are achievable with the sensitivity given by Budget-Alt1 for RF ED

- Note1d: For device 2 with an RF ED-based receiver on the R2D link, if the receiver sensitivity derived from Budget-Alt2, assuming a noise figure of [X dB], exceeds the receiver sensitivity based on Budget-Alt1, then Budget-Alt2 is applied.

[2K1]:

- [2K1] = [1E1] + [1E2] -[1N](R2D) + [2C] - [2X] - [2K]

[2K2]:

- 2K2=lin2dB1+dB2lin([2K1])dB2lin([2F])

[2L]:

- For R2D and Budget-Alt2,

  - [2L] = [2G] - lin2dB([2B] / [1F]) + [2F]

  - Note 1e: the term ‘lin2dB([2B] / [1F])’ is applied due to scaling from CNR/CINR to SNR/SINR.

- For D2R,

  - [2L] = [2G] + [2F] + [2K2], device 1/2a

  - [2L] = [2G] + [2F], device 2b

[4A]

- For R2D,

  - [4A]=[1M]+[2C] -[2H]-[2L]-[3A]-[3B]+[3C]+[3D]

- For D2R,

  - [4A]=[1M]+[2C] -[2X]-[2L]-[3A]-[3B]+[3C]+[3D]

[4B]

- [4B] is derived by assuming pathloss is [4A] and use the pathloss formula as agreed.

Note2: (M) denotes the value is mandatory to be evaluated. (O) denotes the value can be optionally evaluated.

### Link budget template agreed in RAN1#117 and RAN1#118

The following table of coverage evaluation assumptions in link level simulation is considered.

|  | Parameters |  | Assumptions | Company result1 | Company result 2 |
| --- | --- | --- | --- | --- | --- |
|  | R2D/D2R common parameters |  |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template |  |  |
| [0b] | SCS |  | 15 kHz as baseline |  |  |
| [0c] | Block structure |  | Blocks as agreed in 9.4.2.3, or other blocks reported by companies |  |  |
| [0d] | Channel model |  | <Editor’s Note: will be updated according to the agreements made for channel model> |  |  |
| [0e] | Delay spread |  | [30, 150] ns - An RMS delay spread of 30 ns (M) and [150] ns (O) is considered for TDL-A channel model. - An RMS delay spread of 30 ns is considered for TDL-D channel model. |  |  |
| [0f] | Device velocity |  | 3 km/h |  |  |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |  |  |
| [0h1] | BS | Number of antenna elements | 2 or 4 |  |  |
| [0h2] |  | Number of TXRUs | 2 or 4 |  |  |
| [0j1] | Intermediate UE | Number of antenna elements | 1 or 2 |  |  |
| [0j2] |  | Number of TXRUs | 1 or 2 |  |  |
| [0m] | Reference data rate |  | 1 kbps (M), 5 ~ 7 kbps (M), 48 ~ 60 kbps (O), 0.1 kbps for message size of 20 or 96 bits (O) - Other data rate can be reported by companies - Note1: companies to report the exact data rate. - Note 2: the exact data rate is close to the values listed above. - Note 3: The exact data rate is calculated by dividing the message size (excluding CRC) by the total transmission time including applicable overheads (e.g., CRC, pre/mid/post-ambles if present). - Note 4: the exact data rate may be related to coding scheme, repetition and etc. - Note 5: all data rates considered are for evaluation purpose only. |  |  |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. - Note 1: companies to report the M value and chip length used for each message size - Note 2: CRC is not included for the message size |  |  |
| [0p] | BLER target |  | 1%, 10% |  |  |
| [0q] | Sampling frequency |  | Agreement Confirm the working assumption on [0q] Sampling frequency with the following modifications: Working assumption on [0q] (sampling frequency) in link level simulation table Companies to report the Sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): - (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, - (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, - (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, - FFS: Optionally evaluate a fixed value SFO for device 1 and 2 - Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution - Note: Above values are only for sampling purpose. - FFS other values - Note: Above assumptions are only for LLS evaluation purpose only for R2D and [D2R]. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. - Note: Accuracy can be improved after clock calibration for at least device 2. FFS applicable for device 1 - Note: SFO after clock calibration can be applied to Fe. - FFS other models CFO for device 2b. - [(100ppm(M), 200ppm(O), 1000ppm(O, only as initial CFO), with drift rate of [TBD]ppm/s)]” - Note: companies to report whether they assumed the value as initial CFO or after calibration Note: Above assumptions are for LLS evaluation purpose only |  |  |
| [0r] | Device 1/2a/2b |  | Options are as follows, - Device 1, RF-ED - Device 2a, RF-ED - Device 2b, RF-ED/IF-ED/ZIF <Editor’s Note: will be updated according to agreements from 9.4.1.2> |  |  |
|  | R2D specific parameters |  |  |  |  |
| [1a] | Transmission bandwidth |  | 180 kHz as baseline. Other larger values are not precluded. |  |  |
| [1b] | FFS: ED bandwidth |  | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. |  |  |
| [1c] | FFS: BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at [Y] kHz, half of R2D transmission bandwidth. Companies to report X = {3, 5}. |  |  |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |  |  |
| [1e] | Modulation |  | OOK Companies to report, e.g., OOK-1, OOK-4 with M chips per OFDM symbol |  |  |
| [1f] | Line code |  | Companies to report, e.g., Manchester, PIE |  |  |
| [1g] | FEC |  | No FEC as baseline |  |  |
| [1h] | ADC bit width |  | 1-bit for device 1 4-bit for device 2 |  |  |
| [1j] | Detection/decoding method for Line code |  | Companies to report |  |  |
|  | D2R specific parameters |  |  |  |  |
| [2a1] | Transmission bandwidth (w.r.t. D2R data rate) |  | - [2a1]-Alt1 (M): - DSB - X kHz (M) and Y kHz (O) is considered for D2R transmission bandwidth. - The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz (M) and Y kHz (O). - [2a1]-Alt2: - SSB - X kHz (M) and Y kHz (O) is considered for D2R transmission bandwidth. - The value is for one sideband, i.e., the total transmission bandwidth for DSBSSB is X kHz (M) and Y kHz (O). - For device 2b only, FFS for device 2a. - The value of X and Y is as follows, to be down-select from alternative 1 and 2 - Alternative 1: - X = {15 (M), 180 (O)} - Y =180 - Alternative 2: - X and Y reported by companies, - the value may be related to, e.g., - Reference data rate - Coding scheme - Repetition - With or without SFS - SSB or DSB - X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies |  |  |
| [2a2] | [OOK/BPSK/BFSK chip rate] |  | Companies to report |  |  |
| [2a3] | Receiver bandwidth |  | D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. - Assume the receiver matches the transmitter's modulation, i.e., to receiver uses SSB when transmitter uses SSB, receiver uses DSB when transmitter uses DSB. Companies to report the value, and further down-selection of the values and DSB/SSB is not precluded. |  |  |
| [2b] | Waveform (CW) |  | Companies to report waveform, e.g., unmodulated single tone, multi-tone(multiple unmodulated single tone) |  |  |
| [2d] | Modulation |  | Companies to report modulation, e.g., OOK, BPSK, BFSK |  |  |
| [2e] | Line code |  | Companies to report, e.g., Manchester encoding, FM0 encoding, Miller encoding, no line coding |  |  |
| [2g] | FEC |  | Companies to report, e.g., CC, No FEC |  |  |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit |  |  |
| [2j] | D2R receiver |  | FFS: Reader receiver, e.g., coherent receiver / non-coherent receiver Companies to report, e.g., coherent receiver / non-coherent receiver |  |  |
|  | Other assumptions |  |  |  |  |
| [3a] | Other assumptions |  | To be reported by company |  |  |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |  |  |
| Note 1: -These values are only for evaluation purpose and any differences among device types (if any) are not intended for harmonized design approach.Any differences among devicestypes (if any) are for evaluation purpose only. Note 2: (M) denotes the value is mandatory to be evaluated. (O) denotes the value can be optionally evaluated. |  |  |  |  |  |

## Annex IV: Evaluation assumptions for inventory completion time

Table 6.4-1: General assumptions

| Parameter |  | Values |  |
| --- | --- | --- | --- |
|  |  | Direction 1 | Direction 2 |
| Number of devices |  | 600 |  |
| Device distribution |  | Uniformly distributed, i.e., charging time is uniformly distributed within [T_charge,min, T_charge,max] |  |
| Device type |  | Device 1, Device 2 | Device 2 |
| Power model | ON | 1 uW (Device 1) 100 uW (Device 2) | 100 uW |
|  | OFF | 0 |  |
|  | SLEEP | N/A | 0.1 uW |
| Device energy | Capacitor | 10 uF |  |
|  | V_max | 1.2 V |  |
|  | V_min | 0.7 V |  |
|  | E_max | 7.2 uJ |  |
|  | E_avl | 4.75 uJ |  |
|  | Sustainable time | 4.75 s (Device 1) 47.5 ms (Device 2) |  |
| Charging | RF incident power | [Near, Far] = [-27 dBm, -30 dBm] |  |
|  | PCE | [Near, Far] = [25%, 20%] |  |
|  | Charging speed | [Near, Far] = [0.5 uJ/s, 0.2 uJ/s] |  |
|  | First charging time | [T_charge,min, T_charge,max] = [14.4 s, 36 s] |  |
|  | Follow-up charging time | [T_charge,min, T_charge,max] = [9.5 s, 23.75 s] |  |
| Inventory | Data rate | Option 1: [R2D, D2R] = [56 kbps, 160 kbps] Option 2: [R2D, D2R] = [14 kbps, 40 kbps] |  |
|  | FDMA resources | 2/4/8 (Data rate option 1) 2/4 (Data rate option 2) |  |
|  | Paging | 40 + 22 bit |  |
|  | Decrement | 4 bit |  |
|  | Msg 1 | 16 bit |  |
|  | Msg 2 | 18 bit |  |
|  | Msg 3 | 96 bit |  |
|  | T4 | 50 us |  |
|  | T1 | 25 us |  |
|  | T2 | 100 us |  |

Table 6.4-2: Assumptions for Direction 1

| Parameters |  | Values |
| --- | --- | --- |
| First paging transmission time |  | 14.4 s |
| Paging period | Device 1 | Option 1: 2 s Option 2: 4 s |
|  | Device 2 | Option 1: 20 ms Option 2: 40 ms |

Table 6.4-3: Assumptions for Direction 2

| Parameters |  | Values |
| --- | --- | --- |
| First paging transmission time |  | 36 s |
| Solution 1 | Start indication period | 5 ms |
|  | Paging period | 2 s |
|  | Duty cycle period | 2 s |
|  | Duty cycle ON duration | 5 ms (Data rate option 1) 10 ms (Data rate option 2) |
|  | Duty cycle ON occasion | 400 (Data rate option 1) 200 (Data rate option 2) |
|  | Decrement command interval | 5 ms (Data rate option 1) 10 ms (Data rate option 1) |
| Solution 2 | Start indication period | 5 ms |
|  | Decrement indication period | 5 ms (Data rate option 1) 10 ms (Data rate option 2) |
|  | Decrement command interval | T_na = 0.5 ms (wo successful access) T_a = 1.5 ms (w successful access, data rate option 1) T_a = 5 ms (w successful access, data rate option 2) |
|  | Slot interval in SLEEP | 0.5*(T_na + T_a) |

127.8/127.6/122.3/120.9 denotes R2D pathloss for M=1,2,8,12 by 1RB, 1RB, 2RB, 2RB