3GPP TSG RAN WG1 #119	        		               R1-2409613

Orlando, USA, November 18th – 22nd, 2024

Agenda item:		9.10.1

Source:			Samsung

Title:			Remaining topics on enabling TX/RX during RRM measurements for XR

Document for:		Discussion and decision

Introduction

This contribution considers outstanding discussion points from RAN1#118bis on enabling Tx/Rx during RRM measurements for XR [1, 2].

Discussion

Whether to restrict the indication for enabling Tx/Rx during an RRM measurement period to be same in all DCIs after the first DCI where such an indication was made was extensively discussed in RAN1#118bis without conclusion.

From a network implementation perspective, it is clear that such a restriction is unnecessary as the network can trivially realize it if so desired.

From a UE implementation perspective, the only relevant aspect is the timeline for applying the DCI indication for whether or not Tx/Rx is enabled during a next RRM measurement period. Whether the DCI indication was received a long time before the timeline is irrelevant, as for any functionality in NR for which a timeline applies. The UE implementation complexity is defined by the requirement to follow the indication provided at exactly the timeline (that is anyway the purpose of having a timeline). For example, the UE needs to support the case that it receives only one DCI at the time defined by the timeline before the next RRM measurement period, regardless of what the DCI indication is. The UE implementation will actually be somewhat more complex if a restriction is placed on what a DCI can indicate for Tx/Rx during an RRM measurement period because the UE will need to remember previous indications and consider as invalid/dismiss later DCIs that do not follow an expected indication.

From an operational perspective, it should be rather obvious that restricting the indication by later DCIs can only be detrimental as the network cannot adapt to the latest situation. For example, the network may indicate in a first DCI for a UE to skip a next RRM measurement period, and the network may subsequently receive an RSRP report with small value indicating a likely mobility event for the UE. If a restriction is placed that the network cannot change the indication to the UE to Tx/Rx during the next RRM measurement period (i.e. skip RRM measurements), the network cannot reach the UE and inform it that it needs to perform the RRM measurements and then the mobility event is likely to occur. For example, the network can indicate in a first DCI to the UE to skip the next RRM measurement period and subsequently realize that the XR packet cannot be delivered, for example due to a failing PDB. It is then preferable for the network to indicate to the UE to perform the RRM measurements for mobility but that would not be possible if there are restrictions on what the network can indicate.

Observation 1: Restricting future indications to a UE for whether or not it should perform Tx/Rx during an RRM measurement period offers no benefit to the network implementation, is somewhat detrimental to the UE implementation, and can be materially detrimental to the operation of the network that deploys that feature.

Observation 2: There is no need to restrict what a DCI can indicate to a UE regarding Tx/Rx during an RRM measurement period.

Whether to support indication by DCI formats 0_3/1_3 for a UE to skip a next RRM measurement period was also discussed in RAN1#118bis without conclusion. Since DCI formats 0_3/1_3 target multi-carrier scheduling, XR can be considered a use-case due to the high data rate requirements. However, neither PDCCH overhead nor PDCCH blocking reduction (the primary benefits from using DCI formats 0_3/1_3) are issues for XR due to the large TB sizes and the small number of scheduled UEs, including non-XR UEs, when scheduling XR traffic.

A field of DCI formats 0_3/1_3 can have one of several types, such as being applicable to all cells, or only to a particular cell such as the PCell, or only to an associated scheduled cell. Further, in Rel-19, DCI formats 0_3/1_3 would support inter-band CA and scheduled cells with mixed SCS. If DCI formats 0_3/1_3 are to support indication for a UE to skip a next RRM measurement period, minor optimizations could be envisioned, such as using a single bit when the set of all possible scheduled cells is in a same band and using more than one bit in case of more than one bands (number of bits is configured). However, for specification simplicity and as the benefit from any optimization would be marginal, it would be simpler to have 1 bit per scheduled cell for the indication regardless of intra-band or inter-band CA. Further, different UE capabilities may need to be defined, and the corresponding UE features, for supporting by DCI formats 0_3/1_3 the indication for a UE to skip a next RRM measurement period.

Overall, the specification impact for supporting the indication by DCI formats 0_3/1_3 for a UE to skip a next RRM measurement period can be small but the same applies for any additional benefit. For completeness of specifications, including support for DCI format 0_3/1_3 to provide indication for a UE to skip a next RRM measurement period would be reasonable and 1 bit can be added for each scheduled cell in the configured set of scheduled cells (i.e. “Type-2” field).

Observation 3: Supporting the indication for a UE to skip a next RRM measurement period by DCI formats 0_3/1_3 is mostly meaningful for completeness of specifications while associated benefits and specification impact would be minor.

Conclusions

This contribution considered outstanding discussion points from RAN1#118bis on enabling Tx/Rx during RRM measurements for XR. The following observations are made.

Observation 1: Restricting future indications to a UE for whether or not it should perform Tx/Rx during an RRM measurement period offers no benefit to the network implementation, is somewhat detrimental to the UE implementation, and can be materially detrimental to the operation of the network that deploys that feature.

Observation 2: There is no need to restrict what a DCI can indicate to a UE regarding Tx/Rx during an RRM measurement period.

Observation 3: Supporting the indication for a UE to skip a next RRM measurement period by DCI formats 0_3/1_3 is mostly meaningful for completeness of specifications while associated benefits and specification impact would be minor.

References

- RAN1#118bis Chairman notes

- R1-2409106, “Moderator summary #3 - Enabling TX/RX for XR during RRM measurements”