3GPP TSG RAN WG1 #119                                                                 R1-2409652

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.11.3

Source:	Spreadtrum, UNISOC, SGITG

Title:	Discussion on NR-NTN uplink capacity and throughput enhancement

Document for:	Discussion and decision

Introduction

The WID on Non-Terrestrial Networks (NTN) for NR Phase 3 is approved in [1].

The objectives of the work item are the following: - Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design - Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3] - Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it. - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In this paper, we share our views on NR-NTN uplink capacity and throughput enhancement.

OCC multiplexing scheme of PUSCH

In RAN1#118b, the following working assumption on OCC schemes for PUSCH had been achieved [2].

Working assumption For the normative phase, - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support OCC length 4 with one of the following OCC techniques - Option 1: Inter-slot with OCC length 4 to multiplex up to 4 UEs. - Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. - Note 1: - At least consider 8 slots, 16 slots, and 20 slots for VoIP with BLER 2% target, with 1 RB, 2 RBs when comparing Option 1, Option 2, and Option 3. Companies can additionally report on 4 slots at least for 2 RBs. - Option 2 assumes TBoMS, FFS Option 3 assumes TBoMS Note 2: as part of the working assumption, it is assumed that there would be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Proposal 1: The above working assumption can be confirmed.

In the above working assumption, the following OCC techniques for supporting OCC length 4 had been included.

- Option 1: Inter-slot with OCC length 4 to multiplex up to 4 UEs.

- Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs.

- Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs.

The following table summarizes the Pros and Cons of inter-slot time-domain OCC and intra-symbol pre-DFT-s OCC.

Table 1: Pros and Cons of inter-slot time-domain OCC and intra-symbol pre-DFT-s OCC

| OCC scheme | Pros | Cons |
| --- | --- | --- |
| Inter-slot time-domain OCC with PUSCH repetition Type A | - Small specification impact - Enables multiplexing gain by spreading codes across repetitions in the time domain, potentially boosting capacity. | - More sensitive to frequency errors and timing drift - Special UCI and RV handlings are required. - OCC spreading should be restricted within a hop, FH with DMRS bundling can be baseline - OCC length restricted by PRACH, SRS and/or measurement gap |
| Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) | - Less sensitive to frequency errors and timing drift - No restriction of frequency hopping | Large specification impact - Requires updates of the rate matching - Transport block size will be scaled with OCC length. - OCC multiplexing with non-OCC UE not possible - Only 1PRB scheduling has benefit. - Impacts on transmission power, low PAPR. |

If Option 2 is supported, OCC length 2 with inter-slot OCC is used to multiplex up to 2 UEs and intra-symbol pre-DFT OCC with OCC length 4 is used to multiplex up to 4 UEs. In this case, support for both inter-slot OCC and intra-symbol pre-DFT OCC will bring large specification impact. The OCC scheme applied to PUSCH transmission need to be indicated by network.

If Option 3 is supported, UE needs to support both inter-slot OCC and intra-symbol pre-DFT OCC simultaneously so that four UEs can be multiplexed to the same time-frequency resources. Second, TBoMS is an option feature, which it not good idea to combine with basic PUSCH capacity enhancement. Since it really restrict the case use for PUSCH capacity in NTN. And if TBoMS applied in Option 3, it is not clear how to apply Inter-slot OCC with OCC length 2 to TBoMS. As a matter of fact, UE implementation complexity and large specification impact will be brought.

In our view, one single OCC technique for UL capacity enhancement is enough. We do not need two different OCC techniques for one purpose. Option 2 and 3 are very redundant.  Considering the trade-off between performance and specification impacts, inter-slot time-domain OCC with OCC length 2 or 4 should be supported.

Proposal 2: Inter-slot with OCC length 4 to multiplex up to 4 UEs  should be supported.

Potential specification aspects on OCC technique

In RAN1#117, the following agreement on the potential specification aspects on OCC techniques had been achieved [3].

Agreement RAN1 to at least further study the potential specification aspects on OCC techniques: - TBS calculation / Rate matching - UCI multiplexing - RV cycling across repetitions - Frequency hopping, e.g. intra /inter slot - OCC indication/configuration - Power control - FFS others aspects

In this section, we share our views on the potential specification aspects on OCC scheme, especially focus on inter-slot time-domain OCC with PUSCH repetition Type A.

For discussion purpose, we give one terminology of PUSCH OCC group for PUSCH repetitions covered by one OCC sequence. The number of PUSCH repetitions in one PUSCH OCC group equals OCC length. As shown in the following Figure, one PUSCH OCC group contains 4 PUSCH repetitions and they are multiplexed with other PUSCHs with OCC4.

Figure 1: Example of PUSCH OCC group for PUSCH repetitions covered by one OCC sequence.

Proposal 3: For discussion purpose, PUSCH OCC group can be defined, which is PUSCH repetitions covered by one OCC sequence.

TBS calculation / Rate matching

For inter-slot time-domain OCC with PUSCH repetition type A, there is no change on TBS calculation or rate matching.  Since the number of REs allocated for PUSCH within a PRB  and the total number of RE allocated for PUSCH  keep same across all the slots of PUSCH repetition type A. The following conclusion had been achieved in last meeting.

Conclusion For TBS calculation and rate matching for OCC with PUSCH, for inter-slot OCC in the working assumption of RAN1#118bis: - for inter-slot OCC for OCC length 2 and for inter-slot OCC for OCC length 4 in option 1 in the working assumption of RAN1#118bis - No change in determination of TBS. - No change for rate matching.

UCI multiplexing

For inter-slot time-domain OCC with PUSCH repetition type A, UCI multiplexing needs some enhancements. The issues of UCI multiplexing include:

- How to do UCI multiplexing if a PUCCH overlaps with PUSCH repetition type A in a slot

- Which slot(s) that PUCCH located can be multiplexed to PUSCH and timeline requirement

- CSI report on PUSCH

Firstly, all types of UCI multiplexing on PUSCH are defined per slot in legacy, e.g. when PUCCH overlapping with PUSCH, the UCI is multiplexed on PUSCH and PUCCH is not transmitted. However, if UCI multiplexing is per-slot basis, it breaks the orthogonality of OCC among the repetitions in PUSCH OCC group. One way is to multiplex UCI on all repetitions of PUSCH OCC group if there is UCI in one slot. Such as an example in Figure 2, PUCCH1 with HARQ-ACK overlaps with PUSCH Rep#1, the HARQ-ACK can be piggyback on PUSCH Rep#1 only in legacy. When time-domain OCC with PUSCH applies, the HARQ-ACK can be multiplexed on all repetitions of one PUSCH OCC group in which PUCCH overlaps with.

Figure 2: Example of a PUCCH overlaps with PUSCH repetition type A in a slot

Another issue for UCI multiplexing is which slot(s) that PUCCH located can be multiplexed to PUSCH and timeline requirement. If PUCCH overlaps with PUSCH in a slot expect the first repetition in a PUSCH OCC group, whether or not the UCI of PUCCH can be multiplexed on all slots. If allowed, timeline needs to discuss, such as if at least one DCI exists among the overlapping PUCCH and PUSCH, or CSI reference resource. As shown in Figure 3, CSI on PUCCH2 overlaps with PUSCH Rep#2. If it can be multiplexed on PUSCH OCC group 0, which slot that CSI reference resource is determined, according to CSI reporting slot n+1 or slot n. In legacy, slot n+1 is the CSI reporting slot, but in this moment, slot n is the actual first CSI reporting slot. On one hand, if legacy slot n+1 reuses, the time for CSI processing is huge problem. On the other hand, if new slot n applies to determine CSI reference resource, dynamic determination is a big challenge for UE implementation and timeline processing. Frankly speaking, it is unbearable. Our proposal is only the PUCCH overlaps with PUSCH in the first repetition of a PUSCH OCC group, the UCI of PUCCH can be multiplexed on all repetitions of the PUSCH OCC group. Otherwise, the UCI on PUCCH is not transmitted/dropped.

Figure 3: Example of which slot(s) that PUCCH located can be multiplexed to PUSCH and timeline requirement.

The third issue is A-CSI triggered by DCI on PUSCH. In the current spec, the A-CSI is only multiplexed on the first repetition. As same as the first issue, the A-CSI can be multiplexed on all repetitions of the first PUSCH OCC group, instead of the first one.

Proposal 4: Only when PUCCH overlaps with PUSCH in the first repetition of a PUSCH OCC group, the UCI of PUCCH can be multiplexed on PUSCH, otherwise the UCI on PUCCH is dropped.

- UCI of PUCCH multiplex on all repetitions of the PUSCH OCC group

Proposal 5: When a DCI triggers A-CSI report on PUSCH, the A-CSI multiplex on all repetitions of the first PUSCH OCC group.

RV cycling across repetitions

In RAN1#118b, the following agreement on RV cycling for OCC with PUSCH had been achieved.

Agreement For RV cycling for OCC with PUSCH - For inter-slot OCC for OCC length 2 and for inter-slot OCC for OCC length 4 in option 1 in the working assumption of RAN1#118bis - Same RV value is used in one OCC group (i.e., OCC length applied to N slots). - FFS: RV cycling can be additionally used across OCC groups.

In legacy, RV of nth PUSCH repetition is determined by RV ID in DCI for dynamic grant, or RV sequence configured for a CG-PUSCH. For inter-slot time-domain OCC with PUSCH repetition Type A, to ensure orthogonality, repetitions in a PUSCH OCC group use same RV and RV cycling can be across PUSCH OCC groups. RV cycling across OCC groups can also provide the additional gain for HARQ combination. It is proposed to also enable RV cycling across OCC groups. Of course, DG-PUSCH and CG-PUSCH RV cycling are different according to current spec, which can be applied separately only across OCC groups, instead of PUSCH occasions.

Actually, this RV determination is similar as RV of TBoMS with N*K, where N is TB over N slots, K is repetition number. The RV of TBoMS remains same in each N slots, and do RV cycling across other N slots. Then, same mechanism applies. The following table defined in 38.214 gives RV ID for dynamic PUSCH. For time-domain OCC with PUSCH repetition type A, ((n-(n mod M))/M) mod 4 can be used to determine the RV ID, where M is OCC length.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission rvid indicated by the DCI scheduling the PUSCH; rvid to be applied to nth transmission occasion (repetition Type A) or TB processing over multiple slots) or nth actual repetition (repetition Type B). ((n-(n mod N))/N) mod 4 = 0; ((n-(n mod N))/N) mod 4 = 1; ((n-(n mod N))/N) mod 4 = 2; ((n-(n mod N))/N) mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3.

Proposal 6: For inter-slot time-domain OCC with PUSCH repetition Type A, RV cycling can be additionally used across OCC groups

- RV in TBoMS can be used as a baseline, such as ((n-(n mod M))/M) mod 4 determines the RV ID, where M is OCC length

Frequency hopping

For inter-slot time-domain OCC with PUSCH repetition Type A, since the phase continuity may not be guaranteed after frequency hopping, it cannot support intra-slot frequency hopping or inter-slot frequency hopping, only inter-slot frequency hopping with enhancement is possible. Such the inter-slot FH with DMRS bundling. The frequency hopping only hops across different intervals. There is no hop among slots in one interval, the following starting RB of two hops are defined in 38.214. It can also used as a baseline.

For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group. Furthermore, instead of additional frequency hopping interval, OCC length can be directly applied.

In case of inter-slot frequency hopping and when pusch-DMRS-Bundling is enabled, and when a PUSCH is not scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot is given by: RBstartnsμ=RBstartRBstart+RBoffsetmodNBWPsizensμNFH mod 2=0nsμNFH mod 2=1 where nsμ is the current slot number within a system radio frame, NFH is the value of the higher layer parameter pusch-FrequencyHopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

Proposal 7: For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group.

- The RB start index in inter-slot frequency hopping with DMRS bundling can be used as baseline.

OCC indication/configuration

For PUSCH transmission with OCC, how UE determines OCC for PUSCH transmission need to be discussed. In the current specification, there are multiple resource scheduling methods for PUSCH transmission, as shown in Figure 4. Due to the current WID indicating that there is no need for any enhancement in the initial access process, we only need to consider the PUSCH resource scheduling methods of DCI dynamic scheduling and configured grant. Msg 3 PUSCH and Msg A PUSCH are out of scope.

Figure 4: PUSCH scheduling in NR

- DCI dynamic scheduling

For PUSCH transmission with DCI dynamic scheduling, the following two OCC determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 8: For PUSCH transmission with DCI dynamic scheduling, the following two OCC determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

- Configured grant

For configured grant type 1, the OCC used for PUSCH transmission can be configured through RRC. For configured grant type 2, the following two OCC determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 9: For configured grant type 1, the OCC used for PUSCH transmission can be configured through RRC.

Proposal 10: For configured grant type 2, the following two OCC determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Dynamic UL grant for UE group

In current specification, one DCI can only grant one UE for UL/DL data transmission. When the PUSCH UL capacity is increased by means of OCC, the DL resource for PDCCH signaling to send UL grant to the UEs also increases. It will have a serious negative impact on DL capacity and throughput. To alleviate this issue, a direct way is to schedule the data transmission of a group of UEs through a DCI. In addition, the time-frequency resources for each UE within the UE group based on OCC transmission is same, which is beneficial for one DCI to schedule a group of UEs for data transmission.

Proposal 11: Dynamic UL grant for UE group can be considered for uplink capacity and throughput enhancement.

UE grouping for OCC based PUSCH transmission

In last meeting, some companies propose to study UE grouping for OCC based PUSCH transmission to prevent near-far problem. The WID only considers OCC based PUSCH transmission in the RRC connected state. In the RRC connected state, UE will perform channel quality measurement and reporting activities. The network can perform UE grouping for OCC based PUSCH transmission based on the channel quality reported by UE. Therefore, UE grouping for OCC based PUSCH transmission in RRC connected state can be implemented based on the network.

In addition, in NTN, there is little difference in signal strength between the center and edge of the cell, as shown in the following Figure [3]

Figure 5: A sketch of near-far effect in different scenarios: (a) Terrestrial Network; (b) NTN

Observation 1: UE grouping for OCC based PUSCH transmission in RRC connected state can be implemented based on the network.

Conclusion

In this contribution, we provided views on Redcap positioning. In summary, we have following observations and proposals:

Observation 1 UE grouping for OCC based PUSCH transmission in RRC connected state can be implemented based on the network.

Proposal 1: The above working assumption can be confirmed.

Proposal 2: Inter-slot with OCC length 4 to multiplex up to 4 UEs  should be supported.

Proposal 3: For discussion purpose, PUSCH OCC group can be defined, which is PUSCH repetitions covered by one OCC sequence.

Proposal 4: Only when PUCCH overlaps with PUSCH in the first repetition of a PUSCH OCC group, the UCI of PUCCH can be multiplexed on PUSCH, otherwise the UCI on PUCCH is dropped.

- UCI of PUCCH multiplex on all repetitions of the PUSCH OCC group

Proposal 5: When a DCI triggers A-CSI report on PUSCH, the A-CSI multiplex on all repetitions of the first PUSCH OCC group.

Proposal 6: For inter-slot time-domain OCC with PUSCH repetition Type A, repetitions in a PUSCH OCC group use same RV.

- RV cycling is across different PUSCH OCC groups

- RV in TBoMS can be used as a baseline, such as ((n-(n mod M))/M) mod 4 determines the RV ID, where M is OCC length

Proposal 7: For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group.

- The RB start index in inter-slot frequency hopping with DMRS bundling can be used as baseline.

Proposal 8: For PUSCH transmission with DCI dynamic scheduling, the following two OCC determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 9: For configured grant type 1, the OCC used for PUSCH transmission can be configured through RRC.

Proposal 10: For configured grant type 2, the following two OCC determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 11: Dynamic UL grant for UE group can be considered for uplink capacity and throughput enhancement.

Reference

- RP-241667  Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

- 3GPP RAN1#118b Chairman’s Notes, Hefei, China, 14th – 18th October, 2024.

- 3GPP RAN1#117 Chairman’s Notes, Japan, May 20th –24th, 2024.