3GPP TSG RAN WG1 #119	                                   	   R1-2409686

Orlando, US, November 18th – 22nd, 2024

Source: 	vivo

Title:	Discussions on on-demand SSB SCell operation

Agenda Item:	9.5.1

Document for:	Discussion and Decision

- Introduction

In RAN#102, it has been approved in the Rel-19 Network Energy Saving WID [1] to specify the procedures and signaling method(s) to support on-demand SSB SCell operation for UEs.

- Specify procedures and signalling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signalling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we provide our views on the on-demand SSB SCell operation, including the scenarios and potential issues, as well as some possible solutions.

- Scenarios of On-demand SSB operation in SCell

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

In the RAN1#116bis meeting, it was agreed that on-demand SSB can be indicated by gNB for Scenario #2 and Scenario #2A. Meanwhile, Scenario #3A and Scenario #3B will be further studied. The intentions to support on-demand SSB operation in Scenario #2A and Scenario #3A are the same, i.e., shorten the delay of SCell activation procedure. As shown in Figure 1, the difference between Scenario #2A and Scenario #3A lies in that the on-demand SSB indication should be transmitted along with the SCell activation command in the same slot on Scenario #2A, while in Scenario #3A, the on-demand SSB indication could be transmitted after the SCell activation command. From the perspective of delay, Scenario #2A could minimize the delay of the SCell activation procedure by the earlier on-demand SSB indication compared to Scenario #3A. However, in our opinion, whether to send on-demand SSB indication in Scenario #2A or Scenario #3A can depend on network implementation, hence Scenario #3A can also be supported.

Figure 1: The Scenario of on-demand SSB operation on SCell

Proposal 1：For on-demand SSB SCell operation, support Scenario #3A and it is up to gNB implementation to indicate on-demand SSB in Scenario #2A or Scenario #3A.

While in Scenario #3B, when an SCell is activated, UE would perform synchronization and AGC periodically on this SCell for data transmission and reception. Hence, periodical SSB transmission needs to be always available and cannot be fully shut down (i.e. case 1) when SCell is active. Hence, it should be guaranteed that there are always-on periodic SSB transmissions in Scenario #3B to satisfy such requirement. Given periodic SSB transmissions are already available in Scenario #3B, there is no clear motivation to support additional on-demand SSB when SCell is activated. Therefore, there is no need to support on-demand SSB Scell operation in scenario #3B.

Proposal 2: For on-demand SSB SCell operation, do not support Scenario #3B, i.e., on-demand SSB should not be indicated by gNB after SCell activation is complete.

In the previous meeting, it was proposed by some companies to consider whether the on-demand SSB could be supported by SSB-less SCell where the reference cell is specified.  If following the current spec, UE obtains timing reference from the configured or default reference cell for the SSB-less Scell. Besides, gNB needs to guarantee the condition of SSB-less Scell as specified by RAN4 is satisfied. For proponent that supports on-demand SSB operation for SSB-less Scell, it is argued that gNB is hard to guarantee the condition especially the RTD condition for a UE that may be moving anywhere in the cell. Thus on-demand SSB is needed to perform measurement of RTD between SSB-less Scell and reference cell. However, whether the argued problem for SSB-less Scell is existing needs RAN4 input and verification, which is out of RAN1 capability. Besides, the definition of RTD measurement based on on-demand SSB seems out of current WID scope.

Observation 1: RAN4 input on the problem of SSB-less Scell is needed to verify the motivation to support on-demand SSB in SSB-less Scell.

Proposal 3: Do not discuss support of on-demand SSB in SSB-less SCell where reference cell is configured until more RAN4 input is available.

- Limitation of on-demand SSB

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

Based on the above agreements, one remaining issue is whether to support of OD-SSB for CD-SSB located on sync-raster. In our opinion, the on-demand SSB should not be CD-SSB located on sync raster. If on-demand SSB is associated with SIB#1, SSB period needs to be indicated in the SIB1, then UE may misunderstand that the detected SSB is periodically transmitted. However, the on-demand SSB is actually a ‘short term’ signal for measurement. This misalignment will cause impact on legacy UEs. For example, when a legacy UE detect an on-demand CD-SSB associated with SIB1, it will regard this as an accessible serving cell where the SSB is periodically transmitted as SIB#1 configures. However, when on-demand SSB terminates its transmission, legacy UE will see a connection failure, which will degrade legacy UE’s UE experience. Therefore, OD-SSB for CD-SSB located on sync raster shouldn’t be supported to avoid such impact to legacy UEs.

Proposal 4: For on-demand SSB on the cell, do not support OD-SSB for CD-SSB located on sync raster.

About the relation between on-demand SSB and always-on SSB in time and frequency domain, there are two Mux-Cases given in the last meeting. In Mux-Case #1, always-on SSB would not overlap with on-demand SSB in time domain. Given the frequency-domain is not mentioned, this case can be divided into two sub-cases shown below:

- Mux-Case #1a: Always-on SSB and on-demand SSB overlap in neither  time-domain nor frequency-domain

- Mux-Case #1b: Always-on SSB and on-demand SSB overlap only in frequency-domain

Similarly, the Mux-Case #2 can be divided into three sub-cases shown below:

- Mux-Case #2a: Always-on SSB and on-demand SSB overlap only in time-domain.

- Mux-Case #2b: Always-on SSB and on-demand SSB overlap only in frequency-domain.

- Mux-Case #2c: Always-on SSB and on-demand SSB overlap in both time and frequency domain.

To decide which Mux-case should be supported, we should first discuss whether the overlap in time-domain or frequency domain should be allowed or not. About time-domain, there is no special reason to limit that always-on SSB and on-demand overlap in time-domain. But it should be noticed that when the periodicity of on-demand SSB is set as 5ms, there would always be an overlap given that the on-demand SSB would be transmitted in every half frame. Thus, there is no need to restrict that whether there is time-domain overlap between always-on SSB and on-demand SSB or not.

While in frequency domain, if always-on SSB and on-demand SSB locate in different frequencies, UE need to retune RF chain to search on-demand SSB after on-demand SSB is activated. Then there would be an interruption impacting all the serving cell. Thus restricting always-on SSB and on-demand SSB locate in a same frequency is needed.

Therefore, Mux-Case #1b, Mux-Case #2b and Mux-Case #2c should be supported, while Mux-Case #1b and Mux-Case #2b are the same one.

Proposal 5: For a cell supporting on-demand SSB SCell operation, always-on SSB and on-demand SSB could overlap only in frequency-domain (i.e. Mux-Case #1b and Mux-Case #2b), or overlap in both time and frequency domain (i.e. Mux-Case #2c).

- Timing of On-demand SSB operation

  - Start timing after on demand SSB indication (Time instance A)

Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

As the RRC signalling has been agreed to indicate the transmission of on-demand SSB and the processing time of RRC signalling is different from that of MAC CE, the time instance A should also be determined for SSB burst(s) indicated by on-demand SSB SCell operation via a RRC message. Otherwise, UE would not know when to start to receive the on-demand SSB and may miss the ‘short-term’ on-demand SSB transmission.

Similarly, the time instance A could also be the beginning of the first slot containing the first actually transmitted SSB index within the first “possible” on-demand SSB burst which is at least T’ slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission. The difference lies in the value of T’, as the processing time of RRC signalling is different from that of MAC CE. Similar to the timeline of MAC CE indicating on-demand SSB transmission, after the network activate the on-demand SSB through RRC reconfiguration message, UE would response an RRCReconfigurationcomplete message. Then network starts to transmit on-demand SSB. Therefore, the value of T’ should at least contains the time gap between the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission to the time instance where the network successfully complete the process of RRCReconfigurationcomplete message. Hence, T’ is not less than T_min’ = TRRC_process+T1+T2, where TRRC_process is the requirement time from the end of the reception of RRCReconfiguration message to the time when the UE shall be ready for the recption of uplink grant for UE, i.e. RRC procedure delay defined in TS38.331, as shown in Figure 2.  T1 is the delay from slot n'+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationcomplete message and slot n' is the last slot overlapping with the PDSCH containing the RRC reconfiguration message. In addition, T2 is the Reconfigurationcomplete message processing delay in network and could be 3ms [3].

Figure 2: Illustration of RRC procedure delay

Proposal 6: Time instance A is the beginning of the first slot containing the first actually transmitted SSB index within the first “possible” on-demand SSB burst which is at least T’ slots after the slot where UE receives a RRCReconfiguration message from gNB to indicate on-demand SSB transmission.

Proposal 7: T’ is not less than T_min’ = TRRC_process+T1+T2, where TRRC_process is the RRC procedure delay. T1 is the delay from slot n'+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationcomplete message and slot n’ is the last slot overlapping with the PDSCH containing the RRC reconfiguration message. In addition, T2 is the RRCReconfigurationcomplete message processing delay in network.

When SCell with on-demand SSB transmission and cell with signalling transmission have different numerologies, the value of T is different in different cells, and it’s not clear in which cell the slot n is determined. One alternative is to determine time instance A with the numerology of the cell with signalling transmission. And slot n is the slot where UE receives signalling from gNB in the cell with signalling transmission. Another alternative is to determine time instance A with the numerology of SCell with on-demand SSB transmission since the on-demand SSB transmission is performed on this cell. Correspondingly, slot n should be the slot in the SCell with on-demand SSB transmission overlapped with the slot where UE receives signalling from gNB in the cell with signalling transmission.

By contrast, it’s more preferred to determine time instance A with the numerology of the cell with signalling transmission, to keep the same with the numerology of the signalling transmission, since the processing time is related to the numerology of the signalling.

Proposal 8: When SCell with on demand SSB transmission and cell with signalling transmission have different numerologies, the value of T is determined with the numerology of the cell with signalling transmission.

  - End timing for on demand SSB transmission

Agreement For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

In the last meeting, some options for on-demand SSB transmission patterns were introduced to deactivate on-demand SSB transmission. To select the supported option(s), the preferred option is discussed separately from perspective of the following sub-cases:

- Sub-case #1: Case 1 and Scenario #2

- Sub-case #2: Case 2 and Scenario #2

- Sub-case #3: Case 1 and Scenario #2A

- Sub-case #4: Case 2 and Scenario #2A

In Sub-case #1, on-demand SSB before SCell activation is mainly to obtain the measurement results for multiple SCells, where network could determine which one is more suitable to be activated. In this sense, several SSB occassions within a short period is enough to meet RAN4 L3 measurement requirement as shown in Figure 3. Besides, the minimum number of SSB occasions for completing L3 measurement is known to both network and UE, which means that the transmission duration can be determined at the network side. Hence, there is no need to transmit a dedicated deactivation signalling as Option 1 or Option 1A to deactivate the on-demand SSB transmission, given that deactivation signalling would bring extra complexity to both network and UE. The network could just explicitly configure or indicate the transmission number N of on-demand SSB bursts or the duration of the on-demand SSB transmission window as Option 2 and Option 3. Then UE would know where the on-demand SSB terminates. By contrast, the overhead of indication about the on-demand SSB burst(s) transmission number in Option 2 could be less than that of a location of time instance in Option 3. In summary, Option 2 is preferred for Sub-case #1.

In Sub-case #2, the motivation of on-demand SSB transmission is similar with Sub-case #1. Since there is always-on SSB in this sub-case, there is even no need to have long-term SSB. Thus  Option 2 is also preferred for Sub-case #2.

Figure 3: Measurement based on on-demand SSB before SCell activation

Figure 4: On-demand SSB shortens the delay of the SCell activation procedure

Observation 2: Option 2 is the best option to be applicable for Sub-case #1 (i.e. the combination of Case #1 and Scenario#2) and Sub-case #2(i.e. the combination of Case #2 and Scenario#2).

In Sub-case #3, on-demand SSB is triggered for SCell activation purpose. Thus, on-demand SSB should be transmitted at least during the completion of SCell activation procedure as shown in Figure 4. After the completion of SCell activation, UE would perform synchronization and AGC periodically on this SCell for data transmission and reception. Thus, there should be periodical SSB transmission to guarantee the synchronization and AGC procedure. As there is no always-on SSB in this Sub-case, on-demand SSB should not terminates before the SCell deactivation. Therefore, Option 4 or Option 4A, i.e. on-demand SSB transmission should be deactivated when SCell is deactivated, should be supported for Sub-case #3.

Observation 3: Option 4 or Option 4A is the best option to be applicable for Sub-case #3 (i.e the combination of Case #1 and Scenario#2A).

In Sub-case #4, there is always-on SSB transmitted on SCell. The synchronization and AGC procedure could be ensured by always-on SSB. Therefore, on-demand SSB is not needed to transmit after the completion of SCell activation and the transmission of on-demand SSB could only be utilized to accelerate the SCell activation procedure. It means the transmission duration could be predictable for the network. Hence, similar to Sub-case #1 and Sub-case #2, Option 2 is enough for Sub-case #4.

Observation 4: Option 2 is the best option to be applicable for Sub-case #4 (i.e. the combination of Case #2 and Scenario#2A).

Proposal 9: Support Option 2 and Option 4/4A to deactivate on-demand SSB transmission from a UE perspective and UE expects indication of Option 4/4A when on-demand SSB indication is transmitted in Scenario #2A and there is no always on SSB (i.e. Case 1).

To maintain a unified design, a specific value of N could indicate the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated, i.e. Option 2. And a nonnumerical indication of N could indicate that on-demand SSB transmission is deactivated along with the deactivation of SCell.

Proposal 10: A nonnumerical indication of N indicates that on-demand SSB transmission is deactivated along with the deactivation of SCell.

- Configuration of On-demand SSB operation

Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not Agreement For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

When there is no always-on SSB on the cell, i.e. Case #1, the on-demand SSB configuration should be provided to UE to determine how to receive on-demand SSB and some parameters have been agreed in the previous meetings.

A remaining issue is how to configure the time domain location of on-demand SSB burst. To determine the time domain location of SSB bursts on a cell, the timing of this cell should be known to UE first. Given the timing offset between the cell transmitting on-demand SSB and the cell transmitting RRC configuration would be indicated by ca-slotoffset parameter in the Servingcellconfig, the timing of this cell is known to UE. Then UE should know the periodicity and the SFN offset of the transmitted SSB relative to the start of the SSB period to determine the located frame of SSB burst. Last, the half frame index indication would help UE to find the SSB accurately. In summary, in addition to the periodicity parameter agreed before, the SFN offset and half frame index should be provided for on-demand SSB configuration by RRC. And it should be noted that when periodicity is configured with only 5ms, on-demand SSB would locate in every half frame, thus the SFN offset and half frame index may not needed to be configured.

In addition, as analysed in Chapter 4.2, the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated should also be configured by RRC to support Option 2. Furthermore, there should be multiple candidate values of N to support different on-demand SSB transmission durations for different scenarios. Similarly, the transmitted on-demand SSB positions in burst could also be configured with multiple candidate types, and the final transmitted pattern is indicated by MAC CE for different scenarios, as the suitable beam may be changed with the mobility of UE.

Proposal 11: For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC for Case #1:

- Frequency of the on-demand SSB (agreed)

- Periodicity of the on-demand SSB (agreed)

- SSB positions within an on-demand SSB burst (agreed)

- Sub-carrier spacing of the on-demand SSB (agreed)

- Physical Cell ID of the on-demand SSB (agreed)

- Downlink transmit power of on-demand SSB (agreed)

- SFN offset

- half frame index

- The number N of on-demand SSB bursts to be transmitted

Proposal 12: For a cell supporting on-demand SSB SCell operation, the on-demand SSB transmission number N and SSB positions in burst can be configured with multiple candidate values by RRC and the applicable value can be indicated by MAC CE for Case #1.

Similarly, to support on-demand SSB SCell operation, the parameters provided by RRC for Case #1 should also be provided for Case #2. But when always-on SSB is periodically transmitted on the cell, i.e. Case #2, there would be overall configurations about always-on SSB in the serving cell configuration. Thus if a parameter of on-demand SSB is the same with always-on SSB, there would be no need to be configured by RRC once again Then, it should be discussed first which parameter of on-demand SSB would be the same with always-on SSB.

As analysed in Chapter 3, always-on SSB and on-demand SSB should be restricted to locate on the same frequency, avoiding the interruption brought by RF retuning between always-on SSB and on-demand SSB. In addition, there is no motivation to support a different SCS and downlink transmit power of on-demand SSB. And the Physical Cell ID is bound to be the same one. Hence, the parameters, e.g. Physical Cell ID, sub-carrier spacing, frequency, and downlink transmit power can be referred to the configuration of always-on SSB.

Proposal 13: For a cell supporting on-demand SSB SCell operation, at least the following parameters for on-demand SSB configuration can be referred to the configuration of always-on SSB for Case #2:

- Frequency of the on-demand SSB

- Sub-carrier spacing of the on-demand SSB

- Physical Cell ID of the on-demand SSB

- Downlink transmit power of on-demand SSB

As on-demand SSB is mainly designed to accelerate the procedure of SCell measurement and SCell activation, the periodicity should be shorted than that of always-on SSB. Meanwhile, the time domain of on-demand SSB is not required to be the same with always-on SSB, thus the SFN offset and half frame index should be separately configured by RRC. Moreover, to further achieve the power saving of the network, the transmitted on-demand SSB positions in burst could only be part of the always-on SSB positions in burst, as some beams of SSB are useless for UE.

Proposal 14: For a cell supporting on-demand SSB SCell operation, support to independently provide at least the following parameters for on-demand SSB configuration by RRC for Case #2:

- Periodicity of the on-demand SSB

- SFN offset

- half frame index

- SSB positions within an on-demand SSB burst

- The number N of on-demand SSB bursts to be transmitted

Similar to Case #1, there should be multiple candidate values of N and SSB positions within an on-demand SSB burst to support different on-demand SSB transmission durations and different on-demand SSB transmission pattern respectively for different scenarios.

Proposal 15: For a cell supporting on-demand SSB SCell operation, the on-demand SSB transmission number N and SSB positions in burst can be configured with multiple candidate values by RRC and the applicable value can be indicated by MAC CE for Case #2.

Whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters should be discussed. If the legacy RRC parameters are reused, there should be an indication to indicate whether the SSB related RRC parameters are used for on-demand SSB or always-on SSB. Thus even if the name of parameters are not changed, UE would know which SSB these parameters apply. However, some parameters of on-demand SSB are different from always-on SSB, e.g. periodicity, thus a new RRC parameters is always needed. Even if in Case #1, there are some new RRC parameters, e.g. SFN offset, half frame index and the number N of on-demand SSB bursts, should be defined. Thus, for simplicity and unified design, the parameters for on-demand SSB configuration should be configured by new parameters.

Proposal 16: The parameters for on-demand SSB configuration are configured by new parameters for both Case #1 and Case #2.

- L1 measurement and report

Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not

  - On-demand SSB for CSI report

When always-on SSB is periodically transmitted on the cell, there would be two kinds of SSB on the cell. Then when UE performs L1 measurement report, it should be determined whether the CSI report configuration is associated with both on-demand SSB and always-on SSB (i.e. Option 1), or only one of always-on SSB and on-demand SSB (i.e. Option 2).

When CSI report configuration is associated with both on-demand SSB and always-on SSB, the associated SSB resources would be uneven given the on-demand SSB is tiggered by network dynamically. This is a new behavior compared to legacy and will increase the complexity of UE. Moreover, as the periodicity of on-demand SSB and always-on SSB is different, if the periodicity of CSI report is the same as the periodicity of on-demand SSB, there might be no measurement resource between the two report resources after on-demand SSB is deactivated as shown in Figure 5, given the periodicity of always-on SSB is much longer than on-demand SSB. In this case, the report resource would be wasted if UE report the measurement result obtained in last CSI report period again or does not report in this report resource.

Figure 5: No SSB before CSI reference resource in some CSI report period

When CSI report configuration is associated with one of on-demand SSB and always-on SSB, the existing CSI report configuration and mechanism can be mostly reused. The only question that needs to be solved is how to distinguish the CSI resource associated with a CSI report configuration is always-on SSB or on-demand SSB. One possible method is to introduce a dedicated CSI report configuration for on-demand SSB, e.g. ODSSB-CSI-reportconfig, then UE could know the associated SSB type directly. Another possible method is to indicate the SSB type in the resource config associated with the CSI report configuration. UE could also determine which SSB is associated with the report configuration.

By contrast, Option 2, i.e. a CSI report configuration is associated with one of always-on SSB and on-demand SSB, is more simple and has less spec impact when always-on SSB is periodically transmitted on the cell.

Proposal 17: For UE to perform L1 measurement based on on-demand SSB when always-on SSB is periodically transmitted on the cell, Option 2, i.e. a CSI report configuration is associated with one of always-on SSB and on-demand SSB, is supported.

In legacy, semi-persistent CSI report could be triggered by MAC CE or DCI, and aperiodic CSI report is triggered by DCI. A associated CSI-reportconfig is indicated in trigger signalling, then UE could determine the resource location and the resource type to perform measurement. Similarly, for a cell configured with on-demand SSB, UE could distinguish whether to measure on-demand SSB or always-on SSB through the associated report configuration. Thus, reusing the triggering/activation mechanisms for semi-persistent and aperiodic CSI report based on on-demand SSB is enough.

Proposal 18: Semi-persistent and aperiodic CSI report configuration can be associated with any kind of on-demand SSB.

However, how to support triggering periodic CSI report based on on-demand SSB should be further studied. In legacy, periodic CSI report should be associated with a periodic measurement resource. But on-demand SSB would be deactivated after a short duration in most scenarios, thus it is more like a semi-persistent resource. Hence, to support periodic CSI report, one possible method is to allow triggering periodic CSI report based on on-demand SSB only when on-demand SSB is activated at the time SCell is configured to UE through RRC signalling and is deactivated along with the deactivation of SCell. In this case, on-demand SSB could be considered a periodic resource.

Proposal 19: Periodic CSI report configuration can only be associated with on-demand SSB that is activated through RRC signalling and will be deactivated along with the deactivation of SCell.

  - On-demand SSB for LTM

LTM is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by using a cell switch command signalled via a MAC CE. The cell switch command indicates an LTM candidate configuration that the gNB previously prepared and provided to the UE through RRC signalling. Then the UE switches to the target configuration according to the cell switch command. The LTM procedure can be used to reduce the mobility latency.

To be specific, a UE can be indicated SSB per candidate cell for the UE to obtain synchronization and measure corresponding L1-RSRPs. In addition, a Candidate Cell TCI States Activation/Deactivation MAC CE can activate TCI states associated with SSB of corresponding candidate cells. This allows the UE to be DL synchronized with those cells, thereby facilitating a faster cell switch to one of those cells. And the UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an SSB indication of the candidate cell for the PRACH transmission. Then UE could perform RACH-less LTM cell switch procedure.

It can be seen that a periodical SSB transmission is necessary for UE to perform the LTM procedure in legacy. If there is only on-demand SSB transmission in a cell, this cell should transfer the SSB transmission pattern after configured as a LTM candidate cell. Then the cell cannot further obtain the network power saving gain. Thus, if on-demand SSB is supported for LTM, the network could still maintain the power saving state even if being configured as a LTM candidate cell. In addition, the on-demand SSB transmission could shorten the delay of the synchronization and measurement procedure to further reduce the mobility latency as the on-demand SSB could adopt a shorter periodicity.

However, the on-demand SSB triggered in Scenario #2 and Scenario #2A would likely be deactivated after a short- term transmission. Then if the SSB resource configured in the LTM candidate cell is the on-demand SSB indicated in Scenario #2 and Scenario #2A, there may be no transmitted SSB for UE to measure during the LTM procedure after the on-demand SSB deactivates. To solve this problem, it is better to allow that on-demand SSB could be triggered by the related LTM signaling, e.g. LTM configuration message, or triggered during the LTM procedure. But obviously, this is an absolutely separate issue from the on-demand SSB SCell operation we discussed in the last few meetings. Hence, we prefer to prioritize the discussion about on-demand SSB SCell operation in this stage and deprioritize the discussion about the support of LTM.

Proposal 20: Deprioritize the discussion on support of LTM based on OD-SSB before OD-SSB for Scell measurement and activation is finalized.

- Collision between on-demand SSB transmission and other transmissions

In legacy, there might be collision between SSB reception and other UL transmissions, e.g. PUSCH, PUCCH or PRACH. UE would not transmit the UL transmission in the slot overlap with SSB reception as shown in below:

TS 38.213 Clause 11.1 If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided dl-OrJointTCI-StateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in - any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

Similarly, the collision between on-demand SSB and other transmission would also exists. Then how to solve the collision in on-demand SSB operation should be studied. One possible method is to perform a prioritization procedure between on-demand SSB transmission and other transmissions. Then, the type of transmission, the time pattern of on-demand SSB, and some other elements may all have an impact on the prioritization procedure.

Proposal 21: To support on-demand SSB operation, further discuss the collision between on-demand SSB transmission and other transmissions.

- Discussios on RAN4 LS

Regarding the following questions of the relation between always-on SSB and OD-SSB, RAN1 provides the following answers:

Q1: What is the relation in terms of periodicity between always-on SSB and OD-SSB?

Discussion 1: As discussed in RAN1, the on-demand SSB is designed to accelerate the measurement procedure before the SCell activatition or achive a faster SCell activation procedure. And the network could maintain an always-on SSB with a relatively long period to achieve the power saving gain. Hence, the periodicity of OD-SSB should be shorter than the periodicity of always-on SSB.

Q2: What is the relation in terms of time location between always-on SSB and OD-SSB?

Discussion 2: From RAN1’s perspective, the time domain locations of both always-on SSB and OD-SSB are flexible. They may locate on the same half frame and may not. This depends on different configurations. In summary, there is no restriction about the relation in terms of time location between always-on SSB and OD-SSB.

Q3: What is the relation in terms of frequency location between the always-on SSB and OD-SSB?

Discussion 3: The frequency location between always-on SSB and OD-SSB should be the same. If always-on SSB and on-demand SSB locate in different frequencies, UE needs to retune RF chain to receive on-demand SSB after on-demand SSB is activated. Then there would be an interruption impacting all the serving cells. Thus restricting always-on SSB and on-demand SSB locate in a same frequency is needed to reduce UE complexity.

Q4: What is the spatial relation between the always-on SSB and OD-SSB?

Discussion 4: When there is an always-on SSB on the cell with a long period, OD-SSB is triggered with a more intensive transmission pattern to replace always-on SSB and accelerate the measurement procedure. Thus the network could achieve the power saving gain. Hence, there should be a QCL relation between always-on SSB and OD-SSB. For example, always-on SSB index #1 is QCLed with OD-SSB index#1. In addition, the transmitted OD-SSB should be at least the subset of the SSB positions in burst of always-on SSB to achieve the QCL relation.

Based on the above discussions, the answers to these questions for OD-SSB is provided in [4].

- Conclusion

This contribution focuses on the discussion of on-demand SSB for SCell with the following observations and proposals:

Observation 1: RAN4 input on the problem of SSB-less Scell is needed to verify the motivation to support on-demand SSB in SSB-less Scell.

Observation 2: Option 2 is the best option to be applicable for Sub-case #1(i.e. the combination of Case #1 and Scenario#2) and Sub-case #2(i.e. the combination of Case #2 and Scenario#2).

Observation 3: Option 4 or Option 4A is the best option to be applicable for Sub-case #3, i.e the combination of Case #1 and Scenario#2A.

Observation 4: Option 2 is the best option to be applicable for Sub-case #4(i.e. the combination of Case #2 and Scenario#2A).

Proposal 1：For on-demand SSB SCell operation, support Scenario #3A and it is up to gNB implementation to indicate on-demand SSB in Scenario #2A or Scenario #3A.

Proposal 2: For on-demand SSB SCell operation, do not support Scenario #3B, i.e., on-demand SSB should not be indicated by gNB after SCell activation is complete.

Proposal 3: Do not discuss support of on-demand SSB in SSB-less SCell where reference cell is configured until more RAN4 input is available.

Proposal 4: For on-demand SSB on the cell, do not support OD-SSB for CD-SSB located on sync raster.

Proposal 5: For a cell supporting on-demand SSB SCell operation, always-on SSB and on-demand SSB could overlap only in frequency-domain (i.e. Mux-Case #1b and Mux-Case #2b), or overlap in both time and frequency domain (i.e. Mux-Case #2c).

Proposal 6: Time instance A is the beginning of the first slot containing the first actually transmitted SSB index within the first “possible” on-demand SSB burst which is at least T’ slots after the slot where UE receives a RRCReconfiguration message from gNB to indicate on-demand SSB transmission.

Proposal 7: T’ is not less than T_min’ = TRRC_process+T1+T2, where TRRC_process is the RRC procedure delay. T1 is the delay from slot n'+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationcomplete message and slot n’ is the last slot overlapping with the PDSCH containing the RRC reconfiguration message. In addition, T2 is the RRCReconfigurationcomplete message processing delay in network.

Proposal 8: When SCell with on demand SSB transmission and cell with signalling transmission have different numerologies, the value of T is determined with the numerology of the cell with signalling transmission.

Proposal 9: Support Option 2 and Option 4/4A to deactivate on-demand SSB transmission from a UE perspective and UE expects Option 4/4A when on-demand SSB indication is transmitted in Scenario #2A.

Proposal 10: A nonnumerical indication of N indicates that on-demand SSB transmission is deactivated along with the deactivation of SCell.

Proposal 11: For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC for Case #1:

- Frequency of the on-demand SSB (agreed)

- Periodicity of the on-demand SSB (agreed)

- SSB positions within an on-demand SSB burst (agreed)

- Sub-carrier spacing of the on-demand SSB (agreed)

- Physical Cell ID of the on-demand SSB (agreed)

- Downlink transmit power of on-demand SSB (agreed)

- SFN offset

- half frame index

- The number N of on-demand SSB bursts to be transmitted

Proposal 12: For a cell supporting on-demand SSB SCell operation, the on-demand SSB transmission number N and SSB positions in burst can be configured with multiple candidate values by RRC and the applicable value can be indicated by MAC CE for Case #1.

Proposal 13: For a cell supporting on-demand SSB SCell operation, at least the following parameters for on-demand SSB configuration can be referred to the configuration of always-on SSB for Case #2:

- Frequency of the on-demand SSB

- Sub-carrier spacing of the on-demand SSB

- Physical Cell ID of the on-demand SSB

- Downlink transmit power of on-demand SSB

Proposal 14: For a cell supporting on-demand SSB SCell operation, support to independently provide at least the following parameters for on-demand SSB configuration by RRC for Case #2:

- Periodicity of the on-demand SSB

- SFN offset

- half frame index

- SSB positions within an on-demand SSB burst

- The number N of on-demand SSB bursts to be transmitted

Proposal 15: For a cell supporting on-demand SSB SCell operation, the on-demand SSB transmission number N and SSB positions in burst can be configured with multiple candidate values by RRC and the applicable value can be indicated by MAC CE for Case #2.

Proposal 16: The parameters for on-demand SSB configuration are configured by new parameters for both Case #1 and Case #2.

Proposal 17: For UE to perform L1 measurement based on on-demand SSB when always-on SSB is periodically transmitted on the cell, Option 2, i.e. a CSI report configuration is associated with one of always-on SSB and on-demand SSB, is supported.

Proposal 18: Semi-persistent and aperiodic CSI report configuration can be associated with any kind of on-demand SSB.

Proposal 19: Periodic CSI report configuration can only be associated with on-demand SSB that is activated through RRC signalling and will be deactivated along with the deactivation of SCell.

Proposal 20: Deprioritize the discussion on support of LTM based on OD-SSB before OD-SSB for Scell measurement and activation is finalized.

Proposal 21: To support on-demand SSB operation, further discuss the collision between on-demand SSB transmission and other transmissions.

# Reference

- 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11th-15th, 2023, New WID: Enhancements of network energy savings for NR, RP-234065

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification.

- 3GPP TR 37.910, Study on self evaluation towards IMT-2020 submission Chapter 5.7.2.1.

- vivo, 3GPP TSG-RAN WG1 Meeting #119, R1-2409664, Draft reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell, Orlando, USA, 18th Nov. – 22nd Nov. 2024