3GPP TSG RAN WG1 #119                                                                                                     R1-2409728

Orlando, US, Nov 18th  – 22nd, 2024

Title 	:  Discussion on UL capacity enhancement for NR NTN

Source 	:  ZTE Corporation, Sanechips

Agenda item	:  9.11.3

Document for	:  Discussion

Introduction

For the WI Non-Terrestrial Networks (NTN) for NR Phase 3[1], in RAN1#118bis, the following agreements have been made, inter-slot OCC is supported for OCC length 2, 3 options are listed for further down-selection for OCC length 4.

Working assumption For the normative phase, - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support OCC length 4 with one of the following OCC techniques - Option 1: Inter-slot with OCC length 4 to multiplex up to 4 UEs. - Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. - Note 1: - At least consider 8 slots, 16 slots, and 20 slots for VoIP with BLER 2% target, with 1 RB, 2 RBs when comparing Option 1, Option 2, and Option 3. Companies can additionally report on 4 slots at least for 2 RBs. - Option 2 assumes TBoMS, FFS Option 3 assumes TBoMS Note 2: as part of the working assumption, it is assumed that there would be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Conclusion For TBS calculation and rate matching for OCC with PUSCH, for inter-slot OCC in the working assumption of RAN1#118bis: - for inter-slot OCC for OCC length 2 and for inter-slot OCC for OCC length 4 in option 1 in the working assumption of RAN1#118bis - No change in determination of TBS - No change for rate matching Agreement For RV cycling for OCC with PUSCH - For inter-slot OCC for OCC length 2 and for inter-slot OCC for OCC length 4 in option 1 in the working assumption of RAN1#118bis - Same RV value is used in one OCC group (i.e., OCC length applied to N slots). - FFS: RV cycling can be additionally used across OCC groups Agreement For OCC sequence for OCC with PUSCH: - For OCC length 2, re-use orthogonal sequence [1 1; 1 -1]

In this contribution, the views on UL capacity enhancement are further elaborated based on the progress.

Discussion on UL capacity enhancement

Performance evaluation for OCC techniques

In this section, the performance of inter-slot time domain OCC and intra-symbol pre-DFT OCC with TBoMS are evaluated. The simulation assumption can be found in Appendix, and MMSE algorithm is applied at receiver side.

As for the combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs, this method is not considered with the following reasons:

- If TBoMS is required, it means that UE has to be bonded with TBoMS capability similar as Option 2, the complexity of the combination method would be higher than Option 2 but the performance of this combination would be worse than Option 2, there is no benefit for Option 3.

- If TBoMS is not required, the performance of Option 3 would be worse than intra-symbol pre-DFT OCC-2, which has been simulated and analyzed in section 2.2.2 in [4], the performance is much worse than inter-slot OCC due to the increased coding rate.

Therefore, Option 3 is not considered with or without TBoMS.

Proposal 1: Option 3, Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs, is not considered.

In the following sections, Option 1 and Option 2 are further simulated and analyzed.

UE grouping

According to the previous study, in addition to the noise (e.g., link budget), the frequency offset is one of important factor on the performance. In general, it is mainly from the crystal oscillator error in the terminal deviating from its nominal frequency and residual synchronization or precompensation error.

- For the crystal oscillator, if external conditions (such as temperature and pressure) remain relatively stable, the frequency error of the crystal oscillator is usually relatively stable. Therefore, the frequency offset caused by the crystal oscillator error will also remain within a relatively constant range over a certain period.

- For others, considering that the NR NTN UL enhancement in this release only focuses on connected state, it’s naturally to assume that base station has already performed frequency offset estimation during initial access stage, which means that before the PUSCH scheduling in connected state, base station already has basic knowledge on the frequency offset (e.g., estimated via PRACH or SRS, etc) of the UE.

Assuming that PUSCH with 1 or 4 repetitions are used for FO estimation from BS side, then a basic FO range can be estimated by BS before scheduling UEs for OCC multiplexing, the FO error range is illustrated in the following figure.

Figure 1. FO estimation error with 1 repetition (left) and 4 repetitions (right)

It can be seen that the FO estimation error is within 60Hz for single UE, and the estimation accuracy can be further improved with increasing number of repetitions. Therefore, it’s reasonable for base station to group the UEs with similar frequency offset for OCC based UE multiplexing.

In the simulation in the next section, UEs are assumed uniformly with frequency offset [-200Hz, 200Hz], but base station may only pair the UEs for OCC multiplexing if the difference between frequency offset is within 200Hz or 100Hz. Then, it can be found that an acceptable performance can be achieved for inter-slot scheme with 4 UEs for both VoIP for different RB number and repetition number.

Observation 1: FO estimation error for PUSCH without repetition is within 60Hz, and the FO estimation accuracy improves with increased repetition number.

Observation 2: It’s reasonable for BS to group UEs with similar FO for UE multiplexing with inter-slot OCC.

Simulation results

For inter-slot time domain OCC-4 and intra-symbol OCC-4 with TBoMS, VoIP scenario is evaluated with different repetition numbers and RB numbers as shown below:

- 2 RBs

Figure 2. VoIP scenario with 4 repetitions (left) and 8 repetitions (right)

Figure 3. VoIP scenario 16 repetitions (left) and 20 repetitions (right)

- 1 RB

Figure 4. VoIP scenario with 8 repetitions (left) and 16 repetitions (right)

Figure 5. VoIP scenario with 20 repetitions

It can be seen from the results that,

- With UE grouping within 200 Hz FO

  - Inter-slot OCC-4 can achieve similar performance with intra-symbol OCC with TBoMS for 1, 2 RBs, 8, 16 and 20 repetitions in VoIP scenario.

  - Inter-slot OCC-4 has 2.5dB performance degradation @2% BLER compared to intra-symbol OCC-4 for 2 RBs and 4 repetitions.

- With UE grouping within 100 Hz FO

  - Inter-slot OCC-4 can achieve similar performance with intra-symbol OCC with TBoMS for 2 RBs and 4 repetitions in VoIP scenario.

- Without UE grouping

  - Inter-slot OCC-4 has less than 1dB performance degradation compared with intra-symbol OCC with TBoMS for 2 RBs and 16 or 20 repetitions.

  - Inter-slot OCC-4 has less than 1.5dB performance degradation compared with intra-symbol OCC with TBoMS for 1 RBs and 16 or 20 repetitions.

Observation 3: Inter-slot time domain OCC with UE grouping can achieve similar performance as intra-symbol OCC with TBoMS for repetition number 4, 8, 16, 20 and RB number 1, 2.

Observation 4: Inter-slot time domain OCC without UE grouping can achieve similar performance as intra-symbol OCC with TBoMS for repetition number 16, 20 and RB number 1, 2.

Proposal 2: Considering the trade-off between performance and specification impact, inter-slot time domain OCC should be supported with OCC length 4.

Analysis on OCC techniques

Resource mapping

In NR specification, the PUSCH transmission of repetitions with repetition type A are configured in slot. Then, for inter-slot time domain OCC, it’s the most straightforward way to associate the OCC sequence with repetitions, i.e., each element of the OCC sequence is mapped to each repetition(s) as shown in Figure 6. In this way, the legacy resource mapping method will be maintained which is also aligned with the processing of repeated transmission with limited efforts to support the de-OCC operation, i.e., OCC combination among repetitions can be done with the knowledge of different OCC sequences for different UEs, then the per UE signal can be extracted from the superimposed signal.

Figure 6. Illustration of inter-slot time domain OCC

Observation 5: Legacy resource mapping can be maintained for inter-slot time domain OCC.

Intra-symbol pre-DFT-s OCC refers to application of OCC across REs, that is to say, different OCC elements can be applied on different REs with the same content, it requires the original data mapping to be further spread in frequency domain, which is different from existing PUSCH resource mapping in current specification. As shown in Figure 7, the Intra-symbol Pre-DFT OCC procedure is between the modulation module and DFT module, assuming that the OCC length is 2, the allocated frequency domain resource is 1 RB, the modulated symbols will be mapped in half of the frequency domain resources, i.e. 6 REs, then frequency domain spreading would be performed to repeat the 6 REs into 1 RB, each 6 REs are multiplied with an element of the OCC sequence.

Figure 7 Illustration of Intra-symbol Pre-DFT OCC

Observation 6: Intra-symbol pre-DFT-s OCC requires significant spec change on the legacy resource mapping for PUSCH.

TBoMS

TBoMS is an optional feature for coverage enhancement introduced in Rel-17, so it’s not necessary to bond TBoMS tightly with OCC technique, which would reduce the scheduling flexibility and increase the complexity.

Regarding combination of TBoMS with inter-slot OCC scheme, after the application of TBoMS, the granularity for performing OCC is increased, i.e. each OCC code has to be applied for more than 1 slots, which brings more difficulty on maintaining the orthogonality among the time domain units for performing OCC, so there is no need to jointly use TBoMS along with inter-slot time domain OCC.

While for intra-symbol pre-DFT OCC, it’s actually a kind of frequency domain spreading, the available RE number is reduced thus the TBoMS may be jointly used to maintain a reasonable coding rate. However, as mentioned above, TBoMS is an optional feature, then TBoMS cannot be assumed as always bonded with OCC technique.

Observation 7: TBoMS is not required to be configured along with inter-slot time domain OCC.

Observation 8: Intra-symbol Pre-DFT OCC requires to be jointly used with the TBoMS to maintain reasonable coding rate in some scenarios.

Redundancy version

In existing dynamic grant based PUSCH transmission, the redundancy version is determined according to clause 6.1.2.1 of TS38.214, different redundancy versions are applied for the adjacent repetitions in a certain order. However, if inter-slot OCC is applied, the repetitions within the OCC length should be kept the same to maintain the orthogonality, thus the existing redundancy determination mechanism has to be revised.

In RAN1#118bis, it has been agreed that the redundancy versions within one OCC group are kept the same, the pending point is whether to assign different RV values across OCC groups. On one hand, RV cycling across OCC groups may only happen when the repetition number is large, but in this case, the link budget seems poor and the de-occ operation may not work well within one OCC group. On the other hand, assuming that a UE is configured with OCC-2 [+1,+1], it’s feasible for BS to schedule the UE to be multiplexed with another UE with OCC-4 [+1 -1 +1 -1] if RV is same for all repetitions, however, since existing RV cycling for DG PUSCH is mandatory, these 2 UEs with different OCC length cannot be multiplexed together due to different RV value. Therefore, the simplest way to resolve the issue is to always assign the same RV to all repetitions, e.g. always 0.

Proposal 3: In NR NTN with inter-slot time domain OCC, the redundancy versions should be kept the same across OCC groups.

Backward compatibility

Considering the large amount of pre-Rel-19 UEs in existing network, it’s very important to consider the backward compatibility. Since the inter-slot time domain OCC does not change the legacy resource mapping, that is to say, for a pre-Rel-19 UE, it can be naturally regarded as been applied with OCC sequence with full 1, e.g.,  [+1 +1] or [+1,+1,+1, +1]. Then, it can be directly multiplexed with an Rel-19 UE with other OCC sequences. In this way, the system throughput can be doubled without allocating any extra time or frequency domain resources. However, this is not applicable for the intra-symbol pre-DFT OCC due to the additional changes on resource mapping.

Observation 9: Inter-slot time domain OCC can support backward compatibility with non-Rel-19 UEs while intra-symbol pre-DFT OCC cannot.

UCI multiplexing

UCI multiplexing is an important feature from previous release, which would improve the latency and robustness for the transmission. For PUSCH transmission with multiple repetitions with OCC, if UCI is multiplexed on one of these repetitions, the repetition with UCI becomes different from other repetitions, the orthogonality among these repetitions will be destroyed, and de-OCC operation cannot be applied at receiver side.

Therefore, it would be difficult to jointly configure UCI multiplexing and OCC, considering that UCI multiplexing has been specified in previous release, OCC in Rel-19 should have lower priority than UCI multiplexing, the simplest way is to avoid configuring OCC on the repetitions with UCI multiplexing, i.e. if UCI is multiplexed on one of repetitions, OCC is not applied on these repetitions. This implies that if gNB schedules a UE with repetitions and UCI multiplexing, gNB will not configure OCC for this UE, and gNB will not schedule another UE to multiplex with this UE in the same time and frequency domain resource.

Proposal 4: If UCI is multiplexed on one of repetitions, OCC is not applied on these repetitions.

Indication of OCC

After determination of OCC scheme, other aspects including how to indicate the OCC sequence to the UE should also be discussed.

For dynamic grant based PUSCH transmission, considering the limited DCI size, the most essential OCC indication is the OCC sequence index, assuming that the sequence pool has already been specified in spec. As for the OCC length, it may be determined by the repetition number, for example, if the repetition number is less than 4, the indicated sequence index can refer to the table for OCC length 2, otherwise, the indicated sequence index can refer to the table for OCC length 4.

For configured grant based PUSCH transmission, since the resource of PUSCH transmission is pre-defined, whether to enable OCC should be explicitly indicated in RRC signalling. Moreover, the OCC sequence index should also be indicated in RRC to select proper OCC sequence from the sequence pool. As for the OCC length, similar as dynamic grant based PUSCH transmission, it can also be determined by the repetition number.

Moreover, with the increased PUSCH capacity via OCC, it was pointed out by some companies that the capacity of PDCCH might be a bottleneck limiting the possibility to grant a high number of UEs for OCC-based PUSCH transmissions. It is true that PDCCH capacity could be the bottleneck in certain scenarios such as multi-UE in broadcast/multicast scenario, multi-cell scenario, and multi-panel scenario. Consequently, RAN1 has specified single DCI scheduling among multiple UEs/cells/panels. Similarly, RAN1 may also need to further study whether and how to support a single DCI scheduling a group of UEs with OCC-based PUSCH transmissions.

Proposal 5: For dynamic grant based PUSCH transmission, OCC sequence index can be indicated in DCI.

Proposal 6: For configured grant based PUSCH transmission, OCC enabler and OCC sequence index can be indicated in RRC signalling.

Proposal 7: For DG and CG based PUSCH transmission, OCC length can be implicitly determined by the repetition number.

Proposal 8: The indication of the OCC sequence index can be per UE or per UE group.

Conclusion

In this contribution, the simulation assumption and OCC design are discussed with following observations and proposals.

Observation 1: FO estimation error for PUSCH without repetition is within 60Hz, and the FO estimation accuracy improves with increased repetition number.

Observation 2: It’s reasonable for BS to group UEs with similar FO for UE multiplexing with inter-slot OCC.

Observation 3: Inter-slot time domain OCC with UE grouping can achieve similar performance as intra-symbol OCC with TBoMS for repetition number 4, 8, 16, 20 and RB number 1, 2.

Observation 4: Inter-slot time domain OCC without UE grouping can achieve similar performance as intra-symbol OCC with TBoMS for repetition number 16, 20 and RB number 1, 2.

Observation 5: Legacy resource mapping can be maintained for inter-slot time domain OCC.

Observation 6: Intra-symbol pre-DFT-s OCC requires significant spec change on the legacy resource mapping for PUSCH.

Observation 7: TBoMS is not required to be configured along with inter-slot time domain OCC.

Observation 8: Intra-symbol Pre-DFT OCC requires to be jointly used with the TBoMS to maintain reasonable coding rate in some scenarios.

Observation 9: Inter-slot time domain OCC can support backward compatibility with non-Rel-19 UEs while intra-symbol pre-DFT OCC cannot.

Proposal 1: Option 3, Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs, is not considered.

Proposal 2: Considering the trade-off between performance and specification impact, inter-slot time domain OCC should be supported with OCC length 4.

Proposal 3: In NR NTN with inter-slot time domain OCC, the redundancy versions should be kept the same across OCC groups.

Proposal 4: If UCI is multiplexed on one of repetitions, OCC is not applied on these repetitions.

Proposal 5: For dynamic grant based PUSCH transmission, OCC sequence index can be indicated in DCI.

Proposal 6: For configured grant based PUSCH transmission, OCC enabler and OCC sequence index can be indicated in RRC signalling.

Proposal 7: For DG and CG based PUSCH transmission, OCC length can be implicitly determined by the repetition number.

Proposal 8: The indication of the OCC sequence index can be per UE or per UE group.

Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- 3GPP TS 38.133, NR, Requirements for support of radio resource management, V18.4.0

- 3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.1.0

- R1-2407935 Discussion on UL capacity enhancement for NR NTN ZTE Corporation, Sanechips, RAN1#118bis

Appendix

| Parameter | Value |
| --- | --- |
| Channel model | NTN-TDL-C Rural, 30° elevation angle |
| Carrier frequency | 2 GHz |
| Subcarrier spacing | 15 kHz |
| UE speed | 3 km/h |
| Frequency hopping | No frequency hopping |
| PUSCH mapping type A with | 14 OS- for OCC across slots including DMRS |
| HARQ configuration | No HARQ |
| Channel coding | LDPC |
| TBS | 184 bits for VoIP |
| DMRS configuration | 1 port per UE Double-symbol DMRS |
| PRBs | 1 and 2 PRBs |
| Repetition number | 4,8,16,20 |
| OCC length | 4 |
| OCC sequence | DFT sequence in Table 6.3.2.6.3-2 in TS38.211 |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| TO | Uniform selection from [-0.94us, 0.94us] |
| FO | Uniform selection from [-0.1 ppm, +0.1 ppm] |
| Receiver algorithm | MMSE |
| Channel estimation | Real channel estimation |