3GPP TSG RAN WG1 #119		R1-2409802

Orlando, USA, November 18th-22nd, 2024

Source: 	Apple Inc.

Title:                     On remaining frame structure and timing aspects for AIoT

Agenda item:	9.4.2.2

Document for:	Discussion and Decision

# 1 Introduction

A new study item on solutions for Ambient IoT (Internet of Things) was approved in [1]. In the RAN1 118bis meeting, the following was agreed for frame structure, random access and timing relation aspects [2]:

Agreement

TP in R1-2409187 is endorsed in principle for section 6.2 and Annex X of TR38.769, subject to possible revisions.

- The contents in the Tables are further checked by companies and targeted for completion in RAN1#119.

Agreement

The start indicator part of the R2D time acquisition signal is not included in TD2R_min.

Agreement

For R2D reception, the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code.

Agreement

- The TR will capture the following options, and companies are encouraged to analyze the tradeoffs among the following D2R amble(s) options:

  - Option 1: D2R preamble only

  - Option 2: D2R preamble + X midamble(s), where X 1

  - Option 3: D2R preamble + postamble

  - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1

- For the above options, companies are encouraged to report at least the following:

- Purpose(s) of the preamble, midamble and postamble

- Whether companies assume multiple options can be supported

Agreement

RAN1 studies following:

- A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s).

- The study includes

  - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency.

  - Size(s) for resource allocation in the time domain

  - Determination of the X time domain resource(s) by the device

  - Addressing timing errors for adjacent time domain resources due to residual SFO of the device

Agreement

Study FDMA and/or TDMA of D2R transmissions for Msg3 from multiple devices in response to a given set of one or multiple Msg2 transmission(s) during access procedure, including following

- How the frequency and time domain resources are allocated for the FDMA and/or TDMA of D2R transmissions for Msg3

In this contribution, we discuss the open issues of frame structure, the necessity of synchronization operation, random access procedure and the timing relation for Ambient IOT devices based on the latest progress.

# 2.  Discussions

## 2.1 Frame structure

For DL transmission, it was agreed to align NR and Ambient IoT at least on a symbol boundary to simplify the gNB imiplementation e.g., single FFT etc. However, it remains FFS on UL transmission for Ambient IOT devices.

Due to low-cost and low-complexity, the initial SFO of an Ambient IoT device is relaxed to be as large as ~105 ppm and the exact value depends on the oscillator implementation. Due to the significant ~105  ppm SFO target and extreme low power consumption target (~1µW) for low-CAT device, it is quite challenging and costly in terms of power consumption to align Uplink timing between NW and Ambient IoT on a symbol or slot boundary.

We therefore propose:

Proposal 1:  For UL transmission, no need of timing alignment on symbol or slot boundary between NR and A-IOT, due to time drift at device

The following was agreed in RAN1 116bis meeting [3]:

Agreement At least the following time domain frame structure is studied for A-IoT R2D and D2R transmission. - For R2D transmission, - A R2D timing acquisition signal (e.g. R2D preamble) is included at least for timing acquisition and for indicating the start of the R2D transmission in time domain. - For D2R transmission, - A D2R timing acquisition signal (e.g. D2R preamble) is included at least for timing acquisition and for indicating the start of the D2R transmission in time domain. - FFS other necessary component(s), e.g. midamble, postamble, periodic sync signal, control fields, guard period

In the RFID spec, the reference time interval (Tari) for DL is implicitly indicated by the preamble preceding a DL communicaiton.  For R2D transmission, a R2D timing acqusition signal is always included and can also provide the chip length for the following PRDCH transmission.

Proposal 2: The R2D timing acquisition part preceding the PRDSCH indicates the chip length used for the following PRDCH transmission.

For the D2R transmissioin, three options were identified for further study during RAN1 116bis meeting [4]:

How to indicate/determine the D2R chip length? - Option 1: by R2D preamble - Option 2: by R2D control information - Option 3: by D2R transmission bandwidth, e.g. chip length =2double sideband transmission bandwidth

Compared to Opt.1, Opt.2 enhances the scheduling flexibility and enables the configuration of varied chip lengths for different device types, even when they share the same R2D preamble seqeunce. For instance, a shorter chip length can be used for advance Ambident IoT devices i.e., Type 2a or 2b to achieve higher data rates. In our view, Opt.2 still opens door for Opt.3, depending on how the control information is defined. If the bandwidth of DSB signal is specified, Opt.2 is equivalent to Opt.3.

Therefore, we propose:

Proposal 3: The chip length of PDRCH is explicitly indicated by R2D control information.

## 2.2 Scheduling and timing relations

In RAN1 116 meeting, the following was agreed [5]:

Agreement For further discussion, the following terminologies are used for A-IoT for studying processing time aspects: - TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it. - TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it. - TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device. - TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device. - The study should consider at least following aspects - Implementation restrictions for the existing BS/UE - [Processing time is common or different for different A-IoT devices] - [Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)] - FFS other timing aspects

The following was agreed in RAN1 117 meeting for further study [2]:

Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

The TR2D_min is primarily determined by the processing capabilities of A-IOT hardware, making it feasible to define different TR2D_min values for various A-IoT device. Opt.1 was used in RFID specification and results in less signaling overhead compared to Opt.2. With Opt.1, A-IoT device must start transmitting the corresponding PDRCH within a winodw of [TR2D_min, TR2D_max]. Since the window timing is defined at the A-IoT device side and timing error due to SFO varies across devices, it is unclear what factors should be considerd to specify the ‘TR2D_max’ value. Additionally, the main concern on Opt.1 is that it can not support multiple TDMed PDRCH transmissons from different A-IOT devices scheduled by a single R2D transmission command due to a single transmission occasion within a window.

Figure 1: PDRCH transmission timing in Opt.2.

Opt.2 is simliar as the current NR design where the timing information for a D2R transmission is provided by control information in the preceding R2D transmission. The advantage ofOpt.2 is its potential to support TDMed PDRCH transmissions from multiple devices using a single R2D command, offering better flexibility and efficient scheduling compared to Opt.1. This also benefits power consumption at both reader and A-IoT device due to single command transmission and reception. For Opt.2, the actual reception timing at reader side can be expressed as: Treception=t0+2Tp+TR2D+∆SFO, where Tp is propagration delay and ∆SFO are timing drift caused by SFO. Therefore, even for Opt.2, there is a window at reader for D2R reception to account for the impact of SFO. Concerns have been raised on the resource overehead of Opt.2 due to substantial SFO. Since timing drift (i.e., ∆SFO) is a cumulative effect of the SFO over time, the overhead and feasibility of Opt.2 depends on several factors, such as a time gap between the R2D command and the last PDRCH in the intended use case, and a sustainable timing accuracy of sampling clock after adjusting it based on the received R2D command. These problems need to be addressed for Opt.2.

Proposal 4: Additional clarifications is needed on the benefit and the factors to determine the value of ‘TR2D_max’ for Opt.1 in order to justify the necessity.

The following was agreed in RAN1 118 meeting for the PRDSCH timing [8]:

Agreement When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices

The purpose of TD2R_max for Msg2 reponse is to establish a corresponding device behavior for contention based access when no reponse is received due to accesss failure. Since the access failure detection discussions are still onging in RAN2, we recommend deferring the decision until RAN2 concludes on how to handle the Msg2 failure case and whether a similar timer will be defined in MAC layer, as in legacy NR system.

Proposal 5: RAN1 defers the discussion on defining a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it

## 2.3 Random access for Inventory procedure

The following was agreed in RAN2 126 meeting [6]:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

The coverage targetted by Ambient device is up to 50m in indoor deplyement scenario and the one way shorter end-to-end maximum latency including query/triggering time targets to 1 second.  As discussed earlier, one drawback of anti-collision procotol used in Query procedure for RFID is the high collision rate in case of dense use case and eventually causes wastage valuable enery and large latency as device needs to wait for the next Query round. To achive the design goal of extended coverage and low latency, it is imporant to enhance the anti-collision protocol for Ambient IoT devices. The collision rate of Query responses can be reduced by supporting multiple frequency points for high-CAT device to transmit Query responses (i.e., PRACHs signal), regardless of backscattering-based or UE-genrated signals. In addition, the fequency-shifted signal can improve the detection performance and link budget of Query reponse due to shifting the backscatter signal away from the incoming carrier wave frequency to mitigate the interfence at the gNB receiver. The frequency-shifted operation will cause extra power consumption. While, considering the benefit of increased Query response capacity and the reduced latency for Query procedure, support of ‘frequency-shifted’ backscattering signal for high-CAT devices is imporant to explorit the signficantly higher peak power i.e., a few hundred µW.

Proposal 6: For Msg1 and Msg3 transmission, support parallel FDMed resources by frequency shifting operation to cope with multiplexing a high number of A-IoT devices

# 3. Conclusion

In this contribution, we have presented our views on the frame structure, synchronization, random access and timing relation aspects. We proposed the following:

Proposal 1:  For UL transmission, no need of timing alignment on symbol or slot boundary between NR and A-IOT, due to time drift at device

Proposal 2: The R2D timing acquisition part preceding the PRDSCH indicates the chip length used for the following PRDCH transmission.

Proposal 3: The chip length of PDRCH is explicitly indicated by R2D control information.

Proposal 4: Additional clarifications is needed on the benefit and the factors to determine the value of ‘TR2D_max’ for Opt.1 in order to justify the necessity.

Proposal 5: RAN1 defers the discussion on defining a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it

Proposal 6: For Msg1 and Msg3 transmission, support parallel FDMed resources by frequency shifting operation to cope with multiplexing a high number of A-IoT devices

# References

RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Huawei (moderator, RAN1 Vice-Chair), RAN#102, December 2023.

RAN1 117 meeting Chairman notes

RAN1 116bis meeting Chairman notes

R1-2403776, “Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT”, Moderator (Vivo), RAN1#116bis, April 2024.

RAN1 116 meeting Chairman notes

RAN2 126 meeting Chairman notes

RAN 103 meeting Chairman notes

RAN1 118 meeting Chairman notes