3GPP TSG RAN WG1 Meeting #119  							          					R1-2409986

Orlando, USA, November 18th – 22nd, 2024

Agenda item:		9.1.2

Source:	Nokia

Title:	AI/ML for Positioning Accuracy Enhancement

Document for:		Discussion and Decision

# Introduction

RAN #102 meeting approved the Rel-19 WI on AI/ML for NR Air Interface [1], based on the AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air [2] and revised in RAN1#105. In this contribution, we discuss the enhancements related to AI/ML for positioning, work item objectives related to AI/ML positioning sub-use cases, and the latest agreements, observations, and conclusions done in previous meetings (See Appendix A).

The WI objectives are captured in the following.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models … text omitted ... - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases. - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. … text omitted ... Study objectives: … text omitted ... - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study ….… text omitted ... NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

We organize this technical document in sections highlighting our main proposals and observations related to only first-priority cases as per the guidelines in [3]. In Section 2, we address the main aspects of performance monitoring. In Section 3 and Section 4, we provide relevant aspects to be considered for ensuring consistency and data collection details, respectively.  Section 5 addresses aspects related to inference operation. Section 6 addresses details of the functionality framework. In addition to the previous content, Section 7 addresses aspects related to the usage of AIML to enable round-trip methods. Finally, in Section 8, we summarize observations and proposals.

# Performance monitoring

In the previous meetings, only few agreements-for-study were achieved between companies without any concrete agreement. Furthermore, the specification of performance monitoring has impact on the inference input. For this reason, RAN1 should prioritize the discussion on monitoring for all first-priority cases (e.g. Case 1, Case 3a) for both label-free and label-based monitoring.

In this section, key aspects on label-free monitoring, label-based monitoring, and their respective signalization are discussed in the following subsections.

## Monitoring approaches and necessary data to derive monitoring metric

In the study item, several methods were studied for the purpose of monitoring. The methods were organized in two very well-defined approaches, one is label-free monitoring, and another is label-based monitoring. In our view, both are beneficial to monitor the performance with its respective specification impact.

RAN1 to standardize both monitoring approaches, label-free based monitoring and label-based monitoring for first priority cases.

we organize the discussion in two well defined approaches, one for label-free monitoring, and another for label-based monitoring.

### Label-free performance monitoring

The label-based monitoring has a dependency on the availability of ground truth, which in some cases may not easily to implement in practical scenarios. However, label-free monitoring based on buffered inference output, drifting monitoring, and confidence level associated with the inference output considered in the study item Rel. 18 demonstrated that are capable to do the performance monitoring.

Confidence level associated with inference output

For direct AI/ML positioning (e.g., Case 1), there are several limitations regarding the availability of ground truth information (e.g., impossibility to run a high-accuracy positioning method, limited PRUs deployed in the region of interest). In the absence of ground truth, one option for monitoring can be based on the confidence level associated with model inference output. Specifically, in Case 1, the AI/ML model can be defined and trained to provide an additional output ‘confidence level’ which indicates the level of reliability associated with the estimated UE location.

Drifting monitoring

Another alternative might rely on using for example input RSRPP measurements corresponding to inference input. Here, measurements used in inference may be compared to the characteristics of the measurements contained in the training dataset to derive the monitoring metric. The main reasoning behind this approach is to check whether the inference input data is similar enough in a statistical manner to use the training input dataset to reach the expected model accuracy. In fact, a big deviation from the training data has a high probability of inducing degraded performance during inference. This is generally referred to as the process of drifting monitoring.

Based on buffered inference output

Another approach for monitoring without ground truth may use historical/buffered inference output when deriving the monitoring metric (e.g., statistics as standard deviation). Considering the random nature of wireless channels in both time and spatial domains, the estimation of UE location may impact the generalization capabilities of positioning inference using AI/ML. Thus, a light approach for monitoring is analyzing the historical/buffered inference output using a monitoring metric (e.g., standard deviation).

For Case 1 and Case 3a, label-free monitoring may consist of: i) confidence level associated with model inference output, e.g., UE location estimate; ii) statistics of measurement(s) compared to the statistics of measurements in the training data set; iii) standard deviation as monitoring metric based on the historical/buffered/successive inference output.

Based on the above, for monitoring in Case 1 without ground truth, the following data would be necessary: statistics of the measurements corresponding to inference input, i.e. RSRPP measurements; historical/buffered inference output, i.e., UE location; characteristics (e.g., statistics) of training data.

For Case 1, necessary data for label-free monitoring consists of i) statistics of the measurements corresponding to inference input, i.e., RSRPP measurements; ii) historical/buffered inference output (UE location); and iii) characteristics (e.g., statistics) of training data.

In RAN1#118bis, several companies, including Nokia indicated their interest in specifying label-free monitoring. However, it was not clear what could be the specification impact. In this regard, in addition to the signalization of monitoring decision to LMF, specific settings for the label-free performance monitoring method may require additional assistance from the LMF.

For AI/ML positioning Case 1 and Case 3a, label-free monitoring methods are supported, where the entity hosting the model for inference executes the performance monitoring without external information on the ground truth label.

For label-free monitoring in Case 1, UE sends at least the monitoring outcome (indication of events, e.g. failure) to LMF and/or monitoring metric output to LMF (LMF does always the final decision). FFS: other specification impact (e.g., assistance required by label-free monitoring, signalling of statistics of model input, statistics of model output).

### Label-based performance monitoring

In some scenarios, ground truth information (or its approximation), e.g., from PRU, might be available to derive the monitoring metric. Specifically, for direct AI/ML positioning (Case 1 and Case 3b), a statistical difference of the target UE inference output using measurements collected from PRUs can be compared with PRUs location, such as in terms of mean square error (MSE).

Performance metric based on ground truth label (or its approximation) is defined as the statistical difference (e.g., MSE) between ground truth label and inference output, where measurements associated with ground truth information can be used as inference input.

Therefore, for direct AI/ML positioning and label-based monitoring, the following data would be necessary: measurements collected from PRU, for example, RSRPP measurements; ground truth (or its approximation) for UE location; and estimated UE location corresponding to the inference output.

Necessary data for label-based performance monitoring (or its approximation) consists of at least: i) ground truth (or its approximation), e.g., collected from PRU; ii) measurements corresponding to inference input, e.g., measurements collected from PRU; and iii) inference output, e.g., UE location estimation.

## Signaling, procedures, and other aspects for Monitoring

### Monitoring aspects for Case 1

For Case 1, the label-based and label-free monitoring may involve PRU, UE, and LMF entities. Table 1 summarizes possible combinations between entities to derive the monitoring metrics and entities doing the decision based on the monitoring outcome. There are multiple options for the decision entity to make the final decision based on the monitoring output. In general, the monitoring metric calculation may be done on the UE-side and/or on the LMF-side.

Table 2 - Combination of scenarios between entities obtaining metric/statistics for Case 1 monitoring and entities determining decision based on performance monitoring outcome.

|  | Entity generating ground truth | Entity generating measurement for performance monitoring | Entity deriving monitoring metric | Entity doing monitoring decision |
| --- | --- | --- | --- | --- |
| LMF-side performance monitoring | PRU/UE | UE/PRU | LMF | LMF |
|  | LMF |  |  |  |
| UE-assisted performance monitoring | PRU/UE | UE/PRU | UE | LMF |
|  | LMF |  |  |  |
| UE-side performance monitoring | PRU/UE | UE/PRU | UE | UE |
|  | LMF |  |  |  |

For the case where the monitoring metric is derived in the LMF-side, the reporting of measurements from the UE to LMF is required and it should be done prioritizing/using new IEs in LPP.

For the cases when UE is the entity deriving the performance monitoring metric, UE must report to LMF (i) the derived monitoring metric to determine the monitoring outcome or (ii) the monitoring outcome. Both approaches may enable a functionality decision based on performance monitoring outcome at the LMF side.

For Case 1 monitoring, LMF may request UE to derive a performance monitoring metric, and UE reports the derived performance metric or reports monitoring outcome to LMF.

A generic mechanism for performance monitoring for Case 1 is shown in Figure 1, where the UE is the node responsible for model inference. Whenever UE detects changes in the environment, it may request LMF to share relevant procedures for functionality performance monitoring. Next, LMF could collect the data from various trusted entities such as PRU and share it with the UE. Here, the model monitoring condition (e.g., geographical area, PRS information) can be indicated by the UE to the LMF so that the LMF can select the suitable monitoring data for the UE.

Figure 1 - Mechanism to performance monitoring for Case 1 direct AI/ML positioning.

In RAN1#116-bis the following study was agreed between companies:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Based on the agreement the performance monitoring metric calculation for Case 1 both option A and B can be used for calculation of performance monitoring metric at the UE and LMF, respectively. While Option A-1, Option A-2 and Option B-1 can reuse legacy procedures and signaling for estimating the ground truth information. In the other side, Option A-3 and Option B-2 offer more reliable approach for monitoring thanks to the involvement of PRU. Details of the advantages of the lates options are indicated in the following bullets:

- Option B-2: In this option, LMF shares with UE only the necessary data for monitoring, consisting of measurements collected from PRUs associated with ground truth, and requests UE to report inference output using the provided measurements as input. In this option, LMF is the entity computing the monitoring metric.

- Option A-3: In this option, LMF can share with UE both: the measurements and associated ground truth labels. In addition, the LMF requests the UE to compute the monitoring metric. UE could subsequently compute and report monitoring metrics to LMF.

For label-based monitoring of Case 1, for UE to derive monitoring metric, support Option A-3: LMF provides the UE necessary data for monitoring over LPP which contains measurements (e.g., measurements (e.g., RSRPP) collected from PRU(s)) and associated ground truth (e.g., PRU location) including quality indicators. In addition, target UE computes the monitoring metric internally and provides it to the LMF if requested to do so.

For label-based monitoring of Case 1, for LMF to derive monitoring metric, support Option B-2: PRU(s) provides measurements (e.g., RSRPP) to LMF. LMF provides measurements collected from PRU(s) to the UE. UE reports inference output, based on measurements collected from PRU(s), to LMF. LMF compares the UE inference output with PRU ground truth labels associated with these measurements to calculate monitoring metric.

Label-free monitoring for Case 1 has similar signalization requirements, the unique difference is that no message is required to deliver information relative to the ground truth.

For label-free monitoring of UE-side models, for UE to derive monitoring metric, LMF may provide UE necessary data and configuration for monitoring.

For label-free monitoring of UE-side models, for LMF to derive monitoring metric, UE may provide LMF necessary data for monitoring, which may contain: (i) statistics of UE measurements (e.g., RSRPP), (ii) standard deviation of UE inference output (e.g., UE location estimation in Case 1).

### Monitoring aspects for Case 3a

For Case 3a, the following agreement was done in RAN1#118.

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

Considering that RAN3 have started the discussion for Case 3a, RAN1 should be limited to support RAN3 in case any clarification is required. So, all decisions on performance monitoring for Case 3a is up to RAN3.

RAN1 should not do any assumption or decision on Case 3a regarding performance monitoring.

### Monitoring aspects for Case 3b

Monitoring details, including entities calculating the monitoring metric for Case 3b must be delayed until RAN3 start to discuss details on performance monitoring in coordination with SA2, including entities involved in the monitoring.

RAN1 to delay the discussion on any aspect related to performance monitoring for Case 3b.

In RAN1#116 the following agreement reached:

Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

Considering that the performance monitoring for LMF-side model is up to implementation, it is not expected that the UE/PRU or gNB may assist the responsible NF (to be determined by SA2 WG) on making any monitoring decision. However, it is expected that UE/PRU or gNB may report measurements to be used by the responsible NF (to be determined by SA2) on calculating the monitoring metric.

RAN1 to consider only the measurement reporting from gNB to LMF for assisting on the performance monitoring for LMF-side model cases.

RAN1 to consider that whether/what assistance information is sent from UE/PRU or gNB to LMF for performance monitoring of LMF-side model, the final decision on using such assistance is up to the responsible NF (to be determined by SA2).

### PRU selection for monitoring

Selection of a PRU for label-based monitoring to use the ground truth (i.e., the location estimate) and other data (i.e., measurements corresponding to model input) for a certain target UE is a challenging problem. Although close to the PRU, the target UE may experience different channel/ propagation/ measurement characteristics (e.g., LOS/NLOS profiles) compared to the PRU. Moreover, a PRU and a target UE may have different measurement capabilities which renders the PRU measurements improper to be used for estimating the location of target UE. As another example, PRU and target UE may experience dissimilar timing-related errors such as timing error group (TEG) profiles. It follows from the above that the relative location of a PRU with respect to the UE itself is insufficient for selecting this PRU for generating ground truth and other data for the target UE.

For the target UE to collect necessary information and assess whether a PRU can generate similar measurements as target UE, the LMF may provide assistance to UEs in selecting suitable PRUs for ground truth and other data generation. In addition, LMF may provide positioning measurements collected from various PRUs, e.g., closest PRUs around the target UE that have similar capabilities as target UE, together with any conditions to assess the similarity between these measurements and target UE’s measurements, such as a threshold for a predetermined similarity score.

For label-based monitoring, LMF may provide to target UE, measurements collected from different PRUs along with a similarity context (e.g., threshold for a predetermined similarity score) to assist selecting a suitable PRU.

# Ensuring consistency between training and inference

One of the objectives of the WI is to specify methods to ensure consistency between training and inference:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - … - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In the SI, RAN1 considered some generalization-related evaluations in Rel-18, which was verified in [2] that those specific evaluations achieved low generalization performance. For instance, the clutter density in one specific indoor scenario has a direct effect in the rate of LoS/NLoS links collected in one specific dataset for training purposes. Thus, it is expected that areas with specific channel characteristics used for data collection/training may be aligned with inference assumptions to reduce any potential performance degradation. In this scope, certain channel characteristics (e.g., LOS/NLOS rate), specific signal distortions, rare events matched with certain areas/sites must be considered in the scope of the solutions to ensure consistency between training and inference regarding NW-side additional conditions.

Based on the current agreements on AI 9.1.3.3 and clause 4.2.3 of [2], the following potential approaches may be considered to ensure consistency between training and inference: consistency based on assistance data (e.g. PRS configuration) , consistency by Associated ID, and consistency assisted by monitoring (UE/LMF performance assessment).

RAN 1 to consider the following approaches to ensure consistency between training and inference regarding NW-side additional conditions: (i) consistency based on Assistance data, (ii) based on Associated ID,  and (iii) consistency assisted by monitoring.

## Consistency based on PRS configuration and Associated ID

To ensure consistency between training and inference, the data collection at the UE may follow specific data categorization based on a unique global identifier, which is composed by:

- the global cell ID (GCI), which is already available in the legacy specification and

- a specific NW-identifier(s) or network-identifier(s) for PRS configurations. This is referred by associated-ID in AI 9.1.3.3

Overall, the network identifiers shall consider most of these aspects, in addition it must be capable of representing set of NW-additional conditions that is applicable in the training phase and refer to the same identifiers in the inference phase (when the corresponding NW assumptions match with the NW-side additional conditions). Here, the network identifier(s) or Associated ID(s) are unique identifiers for specific network vendor, which may be used to implicitly differentiate the same or different sets of NW-additional conditions applicable for AI/ML positioning.

The Associated ID are a set of identifiers exclusive to a network vendor.

During inference stage, the GCI(s) and available NW-identifier(s) (associated ID) for PRS configurations shall be included in the configuration, which are used by the UE to identify the applicable models for inference.

For UE-side models, to ensure consistency between training and inference related to NW-additional conditions, an assistance data (e.g. PRS configuration) used for data collection (training) or inference shall be associated with a global cell identity (GCI) and a network-identifier (associated ID). FFS: details of network identifier (associated ID).

As an alternative solution, RAN1 may consider other methods such as defining “context scenarios” for the inference operation, where context scenarios may provide some high-level information or requirements related to the datasets used for training (in other words, high-level info about NW additional conditions). This sort of information may be exchanged between the LMF and the UE prior to the inference. As UE-sided models are selected by the UE, the LMF may assist indicating the specific context scenario for the inference. Overall, how to map NW-additional conditions to some sort of high-level information that considers a context scenario is required to be defined to enable this sort of scheme. For Case 1, specific context scenarios based on areas with specific channel characteristics or TRP patterns may be mapped to fixed geographical information as TRPs transmitting DL PRS during inference.

For Case 1, RAN1 to study the possibility defining validity area and granularity requirements of this area for consistency purposes.

As per the working assumption in AI 9.1.3.3 (general aspects), an Associated ID aims to assist UE to ensure consistency within a cell. However, there are still no guarantees in 3GPP that the Associated ID(s) can be consistent across cells, gNBs, service areas, and PLMNs. Instead, time information (e.g., via timestamps, timeframes) can be used to check whether PRS configuration and Associated ID(s) are the same or how they differ between the times assumed for training and inference.

For example:

- NW logs that it provided conditions X during the time T1-T2 to UE1 on May 8, where UE1 collected data for training.

- Later, on June 10, UE1 is required to perform inference (e.g., due to a location request), and UE1 indicates to NW that it used data collected during T1-T2 on May 8 to train its model.

- To ensure consistency between training and inference, UE can indicate the timeframe of the dataset it used for training, upon which the NW compares the PRS configuration and Associated ID between the indicated timeframe and the upcoming inference time, and informs UE about any changes in the PRS configuration and Associated ID.

In RAN1#118bis, the following agreement was done for study purposes:

Agreement For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable. Information; UE-based DL-TdoA; UE-based DL-AoD. 1; Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement. 2; Timing relative to the serving (reference) TRP of candidate NR TRPs. 3; DL-PRS configuration of candidate NR TRPs. 4; Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation. 5; SSB information of the TRPs (the time/frequency occupancy of SSBs). 6; Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). 8; Fine Timing relative to the serving (reference) TRP of candidate NR TRPs. 9; PRS-only TP indication. 10; The association information of DL-PRS resources with TRP Tx TEG ID. 11; LOS/NLOS indicators. 12; On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation. 13; Validity Area of the Assistance Data. 14; PRU measurements together with the location information of the PRU. 15; Data facilitating the integrity results determination of the calculated location. 16; TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP). 17; Expected Angle Assistance information. 18; PRS priority list. [1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0 [2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

In our view, the following IEs must be considered as explicit, implicit and never required (other) to enable a new potential method for Case 1.

RAN 1 to consider the following categorization of IEs related to Case 1 for consistency purposes:

|  | Information | Case 1 |
| --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | E |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs | I |
| 3 | DL-PRS configuration of candidate NR TRPs | E |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation | E |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) | E |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | I |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | O |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | I |
| 9 | PRS-only TP indication | O |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID | E |
| 11 | LOS/NLOS indicators | I |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation | E |
| 13 | Validity Area of the Assistance Data | E |
| 14 | PRU measurements together with the location information of the PRU | O |
| 15 | Data facilitating the integrity results determination of the calculated location | O |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) | I |
| 17 | Expected Angle Assistance information | O |
| 18 | PRS priority list | O |

In legacy UE-based positioning, LMF may inform UE about timing errors at TRPs, in order to assist UE in applying timing corrections when calculating its location estimate. Since the impact of timing errors is critical for high-accuracy positioning, the same approach can also be followed for AI/ML-based positioning. Further, AI/ML models may account for timing errors more efficiently than compensating them via simple methods, e.g., by subtracting the error margin from a ToA estimate. Instead, more advanced use of this information could be made, such as utilizing it as input to AI/ML model in combination with other information provided in the assistance data. While how to use this timing error information would be up to implementation, configuring the transmission and/or reception of DL PRS and UL SRS by the UE/gNB within certain timing errors would allow the flexibility to collect preferred measurements for training, monitoring/update, and inference of UE-side, gNB-side, and LMF-side AI/ML models, as well as to ensure consistency between training and inference.

The association information of DL-PRS resources with TRP Tx TEG ID is explicitly indicated to UE as in the legacy assistance data.

LMF may request UE or gNB to transmit UL SRS / DL PRS or provide their measurements with specific Tx/Rx timing error. For ensuring consistency, UE may indicate to LMF timing errors it assumed for training of UE-side model.

## Consistency assisted by monitoring

In [2], there are multiple flavors to enable consistency assisted by monitoring. For example:

Option 1: when the performance assessment is done at the model level, here the selection of the suitable model and the assessment of the NW-side additional condition between inference and training is up to UE implementation.

Option 2: when the performance assessment is done at the functionality level, which means that a set of functionalities (active and inactive functionalities) are assessed by the UE, based on this assessment the UE may indicate applicable functionalities to the NW.

Option 3: when the performance assessment is done at the LMF side with UE-assistance in doing a down-selection of potential functionalities.

To RAN1 further discuss the following options to ensure consistency between training and inference regarding NW-side additional conditions: Option 1 (UE-sided model assessment), Option 2 (UE-sided functionality assessment), and Option 3 (NW-side functionality assessment with UE-assistance).

# Data Collection

Regarding data collection, the technical report disclosed in [2] mentions some enhancements for signalling, configuration, and measurement reporting for data collection, e.g., signalling aspects related to assistance information. However, based on the RAN1 scope, the priority must be the discussion on data content and entities involved in the data collection (including data collection for training). In addition, enhancements related to UE data collection for monitoring and inference may be based on legacy positioning measurement and reporting frameworks.

Further details on the necessary data content for training, data ground truth generation, and reporting overhead are discussed in the following subsections.

## Training data content and quality

With respect to quality indicators to improve the model training, in Section 6.4.2.1 of the TR disclosed in [2], the following observation is mentioned:

… text omitted ... For AI/ML based positioning, the positioning accuracy is affected by the training dataset size for a given UE distribution area (or equivalently, sample density in #samples/m2), when the UE is distributed uniformly in training data collection. There exists a tradeoff between the training dataset size and the achievable positioning accuracy. The larger the training dataset size (i.e., higher sample density), the smaller the positioning error (in meters), until a saturation point is reached where additional training data does not bring further improvement to the positioning accuracy. … text omitted ...

Based on this observation, there are two clear aspects that impact on the positioning accuracy, one is the dataset density, and the other is based on the UEs ground truth/labels distribution following a uniform distribution. These dataset quality indicators for training purposes may be considered in the normative work, in this regard, the dataset size may be quantified with the sample density in #samples/ m2 and another metric may quantify the similarity of distribution of the target dataset with the uniform distribution.

The quality of content of the dataset used for training, RAN1 considers the sample’s density (#samples/m2) and the distribution similarity between the target dataset and the uniform distribution.

### LMF assistance for ensuring ground truth quality

For data collection purposes, obtaining an accurate ground truth (positioning label). In this regard, to reduce the impact of label inaccuracy, the UE may consider proactive action during data collection. Therefore, UE collects samples with high confidence on the obtained ground truth (positioning label). In case of single positioning source for sample labelling, UE may only collects samples with high positioning confidence level. Alternatively, UE may obtain positioning label from multiple sources e.g., non-NR and/or NR RAT-dependent positioning methods. In this case, UE may only collect samples of the position estimations by different consistent sources. To enable the sample evaluation process, a label consistency score can be defined as a label quality indicator. The label consistency score/quality needs to hold the following properties:

- providing higher score value when positioning estimation(s) has/have higher accuracy,

- in case of consistency between estimated positions from multiple positioning sources, the score value increases proportionally to the number of consistent positioning sources.

For ground truth data collection (Part B), to assess the quality of ground truth, LMF may assist UE for obtaining label consistency score/quality of the positioning estimates from one or multiple positioning sources.

Another approach to ensure high quality data collection for ground truth information is by complementing the absolute positioning provided by PRUs with the relative positioning generated by sensor deployed in the UE (e.g., gyroscope). Once the relative and absolute positioning information are linked and signaled with LMF assistance, any regular UE is capable to generate reliable ground truth.

For data collection of high-quality ground truth, LMF assists UEs on linking the PRU location (absolute information) with the relative information generated by sensors (e.g. accelerometers, gyros, magnetometers) deployed in the UE.

In RAN1#118, the following proposal was discussed between companies, but no agreement was achieved.

Proposal 4.1.2-2 For training data collection of AI/ML based positioning Cases 1, 2b, 3b, quality indicator of label is represented using one of the following options: - Option 1 (value in meters): reuse the existing IE. That is, IE NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - Option 2 (value 0 to 1): hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality.

In terms of lower specification impact, Option 1 is the natural direction. However, as AIML positioning in Rel. 19 is based on supervised learning scheme, it is a common understanding between companies that the label quality plays a fundamental role on the inference performance and in some cases for monitoring (e.g. label-based monitoring). So, the indicator of the label quality must be complemented by using additional reference information, For example, PRU (probably the unique ground truth information in AIML Positioning) may be used as reference by LMF to assist new label quality indicator using values in meters or hard/soft information of the quality, as stated in Option 2.

For training data collection of AI/ML based positioning, quality indicator of label is represented by a value between a predefined range (e.g. between 0 and 1 with 1 indicating the highest quality). FFS the predefined range.

### Additional data sample for Part A

In RAN1 #116-bis, the following agreement was achieved between companies:

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

Part A may include additional data to complement the channel measurements. It may consider different granularities for input inference such as:

- Inference input granularity 1: with inputs as radio measurements only

- Inference input granularity 2: with inputs radio measurements + TRP id

- Inference input granularity 3: With inputs radio measurements + TRP id + Other Assistance data

In this context, the data collection may be enhanced with a new flag which enables the categorize the data and enables therefore more flexibility and better adaptation to some methods that aims to ensure consistency between training and inference.

For training data collection of AI/ML based positioning, an additional data (e.g., TRP ID) may be included in Part A along with the data type to indicate if it is mandatory or optional.

### Time stamp information

The time stamp associated with the AI/ML training data (measurement and/or label) is to be captured and reported by the data generating entity to the LMF. This is particularly relevant for example in scenarios where the measurement data (Part A component of the training data) collection entity and label data (Part B component of the training data) generation entities are different.

To capture and report time stamp of the AI/ML training data via legacy means, currently the LPP supports reporting either SFN information or the UTC time information from the UE to the LMF. Further, NRPPa supports optional reporting of time in seconds relative to 00:00:00 on 1 January 1900 (Relative Time 1900) along with mandatory SFN information reporting from the gNB to the LMF.

The SFN information carries NR time stamp which can include not only the SFN number but also the slot number, and hence it can capture the time resolution in milliseconds (ms). However, since the SFN number ranges from 0 to 1023, any data collection and reporting of the collected data beyond 10.24 ms (SFN cycle length) results in SFN ambiguity. Hence, the SFN information alone is not sufficient to capture the time stamp information for the data collection

On the other hand, usage of UTC time can avoid the time information ambiguity. However, the ms level granularity cannot be achieved with the UTC time of format YYMMDDhhmmssZ. Hence, UTC time in milliseconds needs to be adopted if the UTC time information is to be used. Note that, the reporting of UTC time information, in particular at ms-level of precision, increases the signaling overhead. Hence, the always reporting of UTC time information is not desirable. A similar analysis between gNB/TRP and LMF indicates that the same applies to Relative Time 1900 reporting over NRPPa.

To this end, the SFN information associated with the training data can be always reported to capture the time information. And this is complemented with the UTC time information over LPP and Relative Time 1900 over NRPPa only when there is a change in SFN cycle to resolve the SFN ambiguity.

To resolve the SFN time info ambiguity with reduced signaling overhead, the SFN time info for a channel measurement and/or a label can be complemented with the UTC time info only when there is a change in SFN cycle between two consecutive channel measurements and/or labels in the report.

For reporting of UTC time with SFN time info as time stamp of channel measurements and label is considered in RAN1 as configurable.

## Entities generating training data

RAN1 made the below working assumptions in RAN1#116bis as they concern with RAN2 WG:

Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

For the aforementioned working assumption, RAN2 WG input is needed to confirm whether the assumptions are valid and feasible. To this end, an LS can be sent to RAN2 to check on the assumption.

Send an LS to RAN2 to query on the feasibility and validity of the working assumptions made for training data generation of AI/ML based positioning Case 1.

## Positioning method selection for ground truth generation

UE may use a variety of methods to generate ground truth such as based on non-NR, NR RAT-dependent, or NR RAT-independent positioning methods. For RAT-dependent methods, LMF needs to be involved as per the legacy positioning framework. To generate a ground truth label (or its approximation) associated with its inference output (UE location estimation), UE may send a positioning request to LMF to get its location information.

Nevertheless, since Case 1 relies on DL measurements, if LMF utilizes the same DL-based methods to estimate the target UE’s position, which is intended to be used as ground truth generation, then the estimation would not be reliable in case of imperfections, e.g., NLOS links, UE measurement inaccuracy, timing error or estimation errors. Thus, it is important if UE can indicate a suitable method (that is possibly non-DL based method) or LMF determines a suitable method (that is possibly non-DL based method) to generate ground truth for UE-side models.

Similarly, multiple methods can be used to estimate UE’s position, among one of which could be selected according to different criteria, e.g., with respect to confidence of estimation or relative difference between the estimations using different methods (e.g., RAT, non-RAT based etc.). For this, the LMF may indicate UE, positioning method(s) together with necessary criteria to select an estimation to be used as ground truth.

In data collection for ground truth generation by the target UE for Case 1, LMF may indicate UE positioning method(s) (e.g., non-NR and/or NR RAT-dependent and/or NR RAT-independent) with necessary criteria (e.g., required confidence value) for an estimation to be used as ground truth.

## Reducing reporting overhead for data collection

In this section, we will discuss two alternatives for reducing the signaling overhead involved in the data collection. Note that the data collection refers to both training/inference/monitoring. These alternatives are:

- Window based approach.

- Differential quantization approach.

### Window based approach

For LMF-side model (Case 3b) one crucial aspect is how the UE/gNB reports the measurement to the LMF while avoiding overhead. In this regard, due to clustered nature of the dominant taps, the overhead can further be reduced by choosing multiple windows. By doing so, the overhead is reduced significantly compared to the existing schemes. Furthermore, the power information observed over each window may be estimated if needed to determine which cluster has the dominant component. Details of the reporting are disclosed in Section 5.5.1 of [5].

For the data collection of Case 3b (LMF-side), for inference input, depending on the channel observation, UE/gNB can determine/select the required reporting configuration to enable the reporting windowing scheme without compromising the DL/UL measurements content.

### Differential quantization approach

For any data collection purposes, LMF may ask channel measurements to NG-RAN (inference Case 3b, monitoring Case 3a) or UE (monitoring Case 1) entities to report radio measurements. In this case, several options can be considered for reporting the positioning measurement signals. One alternative is the differential quantization, which is detailed in Section 5.5.2 of [5].

For assisted Case 1 monitoring and Inference Case 3b, to reduce the signaling overhead of reporting measurements from UE (case 1) and NG-RAN (Case 3b) to LMF, UE/NG-RAN may use a reporting scheme based on differential quantization indicating at least the number of RSRPP messages for carrying the measurement report.

# Inference Operation

The inference operation discussed in this section follows three important aspects: AI/ML inference input, AI/ML inference output, and signaling and procedures for inference.

## AI/ML inference input

In the previous 3GPP meetings controversial aspects on addressing AI/ML inference input were discussed in several on-line and off-line sessions. One is related to the necessity to specify inference input, channel measurement representation, and path information. These three topics are discussed in the following.

### Need for specifying inference input

For Case 1/Case 3a, the inference input is based on the local UE/NG-RAN measurements, respectively. Both are internally available at UE/NG-RAN. The necessary measurement(s) for the model input are performed by UE using the DL PRS transmitted by TRP(s) for Case 1 and performed by TRP using UL SRS transmitted by UE for Case 3a. Some companies used this argument to avoid any specification impact on them. However, specification of inference input for UE-side and gNB-side models would be necessary since some label-free and label-based monitoring cases are considering LMF assistance for calculating the monitoring metric. For instance, scenarios where the UE or NG-RAN may be assisted by the LMF, which would require reporting the respective measurement.

Further, for LMF-side models, the measurements need to be specified anyways since the UE or NG-RAN need to report measurements to LMF.

The discussion of the definition on sample-based representation should include Case 1, to enable label-based monitoring cases, if required.

### Timing information: sample-based vs path-based

In RAN1#117, the following agreements were reached between companies:

Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

In terms of specification impact, the path-based representation (e.g., legacy RSRPP) may be reused in AI/ML positioning, requiring in some cases only an extension of the number of taps to provide enough information for training and inference of AI/ML model.

For the case of sample-based representation, in terms of specification impact, may include details of Nt, N’t. In addition, the sub-sampling considered in [2] may be considered as a generic version of the path-based representation, where the power and delay indication are not following the grid representation.

RAN1 to consider both: path-based and sample-based representation for AI/ML positioning cases.

In RAN1#117, the following agreement was achieved:

Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec.

Based on this agreement the sample-based measurement is composed of Nt’ samples of the estimated channel response in time domain. These samples can be used for inference and data collection. For example, different scenario (RF channel characteristics) may require specific samples collection methodology, positioning requirements, overhead and UE capabilities.  Therefore, an appropriate positioning measurement configuration (i.e., sample selection rules) can be adapted based on the channel conditions, mobility conditions, positioning requirements, area, TRPs, etc, thereby inducing a fair trade-off between accuracy and signaling overhead (as stated in Rel. 18 SI).

When sample-based representation is used, LMF configures the UE/gNB to determine N’t and choose sample selection strategy/rule-based for a fair trade-off between positioning requirements and signaling overhead.

In RAN1#118 the following agreement was made for the sample-based measurements:

Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation.

RAN1#118bis FL summary proposal (R1-2409286) on the sample-based measurement.

Proposal 2.1.1.4-1 For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, RAN1 study the following options for down-selection for determining the starting time of the list of Nt consecutive samples: - Option A. the starting time of the list of Nt samples is the timing of the first detected path (with timing granularity T). - Option B. the starting time of the list of Nt consecutive samples is the timing of the earliest sample that the first path power is detectable. - Option C. the starting point of the Nt consecutive samples is determined by a fixed offset (with timing granularity T) relative to the reference time. - Proponent companies provide details on the value of the fixed offset. - Option D. the starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time. - Proponent companies provide details on the value range and signaling details of the configured offset.

The number and resolution of time-domain samples obtained from a time-domain channel measurement may impact a granularity of a channel estimation based on the time-domain channel measurement. Thus, the selection of samples to be collected and reported may impact AI/ML inference input and thereby positioning accuracy. The time-domain channel information illustrated in the example shown in the Figure 2.

Figure 2 - Channel response in time domain.

As highlighted in the RAN1#118bis agreement, the sample selection can be based on the strongest signal power and first detected path in time. However, such approach does not adequately capture full channel information e.g., range of samples. One possible approach to above mentioned issue involves using the first detected path as the reference time and collect optimal samples considering statistical parameters (e.g., variance, standard deviation, root mean squared or similar metric) related to distribution characteristics of signal. Specifically, for the Nt consecutive samples the variance around the first detected sample (which could be strongest path) can be utilized to determine the value range of the Nt’. Moreover, the UE only report set of offsets corresponding to the detected peaks, and thus maintain the same inference accuracy and at the same time reduce the reporting overhead.

Based on the FL summary (R1-2409286) from RAN1#118bis, for sample-based measurement four options are listed for determining the starting time of the list of Nt consecutive samples. In our understanding Option D can be further discussed where the starting point of the Nt consecutive samples is determined by a configured offset relative to the reference time. The value range for the samples Nt’ is again based on the detected peaks and corresponding variance.

The starting time of the list of Nt samples can be based on the first detected path. The value range of samples is based on two key factors, the time offset of the strongest samples relative to reference point and the statistical parameters applied across those strongest samples.

For sample-based, LMF configure the UE/gNB for starting time of list of Nt samples and value range of Nt. The optimal value for the time offset and range of Nt is up to UE-implementation.

As a starting point, Option-D can be considered for further investigation while other options are not precluded.

In RAN1#118bis, the following agreement was done between companies.

Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information

The current agreement may be valid for path-based representation. However, as sample-based representation is adding additional details on the generation of the channel measurement in case Sample-based representation is confirmed for Case 1 or Case 3b, RAN1 should consider studying the necessity to define a quality indicator to assess potential ambiguities.

RAN1 to study the definition of a specific quality indicator for Sample-based representation. FFS: the signalization involved on the reporting of the quality indicator.

### Phase information

In RAN1#116 meeting, the following investigation was agreed between companies:

Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

Details of using phase information are described in the following subsections.

#### Phase information of the first path

Carrier phase measurements are sensitive to LOS determination, and they have the challenge of resolving the integer ambiguity. If RAN considers introducing this new measurement, both challenges need to be solved. For the first challenge, carrier phase may be reported for more than one dominant component, while for the later, differential measurements of carrier phase may be considered, or alternatively, assistance to the UE/gNB must be provided to derive the integer ambiguity w/o AI/ML.

#### Channel impulse response

Based on evaluation results from R1-2307242, a comparison of the performance of ML model considering as input: (1) CIR (complex values) versus (2) PDP (real values) for the cases of direct positioning and assisted positioning (e.g.,LoS/NLoS indicator) as well as a variety of scenarios with different setting parameters (depicted in the following table). Here, the number of antennas is 1, the truncation is done at Nt = 128 without subsampling.

Table 1 - Description of scenarios used to compare the performance of CIR and PDP.

| Scenarios | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Clutter density | 40% | 40% | 40% | 60% | 60% | 60% |
| N. of TRPs | 18 | 10 | 3 | 18 | 10 | 3 |

Figure 1 represents the performance comparison between CIR and PDP for direct AI/ML positioning. The result indicates that both provide roughly similar performance with some enhancement using CIR in some specific scenarios (especially for direct positioning case). However, the model with CIR as input has higher overhead compared to PDP, specifically if the number of antennas is increased. In Figure 2, a similar performance is observed in AI/ML assisted positioning for the LOS/NLOS indicator, in some scenarios PDP outperforms a little bit to CIR. However, the same observation remains, the main impact of CIR is the additional overhead to be required for reporting.

Figure 3 - Positioning performance for Direct AI/ML positioning.

Figure 4 - LOS/NLOS indicator performance for AI/ML assisted positioning.

CIR is not supported for inference input when the model running at the LMF-side, gNB-side or UE-side cases. In addition, if first legacy RSCP/RSCPD are considered as inference input, the LoS determination and integer ambiguity needs to be studied.

### Enhancements of path-based representation

As described in TR38.843, most companies used the following in Rel-18 study item evaluations:

"Sampling period = 1/(Nf ×∆f). For FR1, sampling period = 1/(4096×30)=8.14 (ns), where Nf =4096 according to 38.211, and ∆f =30 kHz is the subcarrier spacing."

That is, sampling period = 16´Tc was used for SCS=30 kHz.

The input dimension for the measurements is NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of consecutive time domain samples. If N’t (N’t < Nt) samples with the strongest power are selected as model input, with remaining (Nt ‒ N’t) time domain samples set to zero, it is also assumed that timing info for the N’t samples need to be provided as model input.

For example, when N't time domain samples with the strongest power are selected as model input. For model input Nt = 256 using different values N’t. Reducing N't from 256 to 64 does not appreciably degrade the positioning accuracy while overhead shrinks to ¼ the results are summarized in the following table.

| Nt’ | Minimum error | Maximum error | Overhead reduction compared to Nt=256 |
| --- | --- | --- | --- |
| 128 | 1.02 | 1.07 | – |
| 64 | 1.02 | 1.21 | 14 |
| 32 | 1.14 | 2.03 | 18 |
| 16 | 1.12 | 2.54 | 116 |
| 8-9 | 1.42 | 3.29 | 132 |

There is trade-off between accuracy and signaling overhead when extracting and reporting the number of samples.

## AI/ML model inference output

### Case 3a

The output of the AI/ML model inference in Case 3a is a positioning-related intermediate feature. RAN1 has agreed on the following two major intermediate features: LOS/NLOS indicator and timing information.

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

The intermediate feature is reported by NG-RAN (gNB) to LMF for estimating the UE position. The intermediate feature can be reported to LMF via reusing existing procedures in NRPPa.

With regards to details of the report, in RAN1#118, companies were discussing that LOS/NLOS indicator becomes useless when timing information is generated by an AI/ML model for Case 3a since timing information always assumes a “virtual LOS” path. Whereas, in legacy, LOS/NLOS indicator is always associated with a physical path and related measurement.

Therefore, following options can be supported for reporting in Rel. 19:

- Timing information estimated without AI/ML, and LOS/NLOS indicator associated with the reported physical path, estimated with or without AI/ML

- Timing information estimated with AI/ML, no LOS/NLOS indicator associated with the assumed virtual LOS path

In any case, indicating whether any of the reported intermediate features is generated using AI/ML would depend on the definition of a new method or not to enable Case 3a in the specification, which depends on RAN3 discussion. To illustrate, the scenario in which LMF requests gNB to utilize AI/ML to estimate an intermediate feature and report it must be discussed in RAN3 because one approach may be that NG-RAN is independent of LMF to enable Case 3a, then it would become unnecessary for NG-RAN to indicate in its report that AI/ML was used for estimation. Instead, reporting of LOS/NLOS indicator can be simply omitted if the reported timing information is estimated with AI/ML.

In Case 3a, when timing information is estimated with AI/ML, the legacy reporting framework is reused with no LOS/NLOS indicator associated with this estimation.

The decision of enabling Case 3a by a new method and new reporting framework is up to RAN3. So, RAN1 should wait for potential queries from RAN3 before to discuss on this topic.

In RAN1#118bis the following agreement was done between companies.

Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

The legacy may not be sufficient to assess the quality of the inference outcome provided by Case 3a AIML assisted positioning. On this regard, the assessment of the quality of the timing information should account for additional factors with the existing timing measurement quality (Timing Measurement Quality in TS 38.455) such as AI/ML model confidence of timing information inference.

The existing timing quality information is not sufficient to capture the machine learning aspect for AIML assisted positioning (Case 3a), and it should be enhanced.

During model inference for Case 3a, AIML model confidence is taken into account for the quality of timing information assessment. FFS how model confidence is captured in the definition of the timing quality indicator.

### Case 3b

The output of the AI/ML model inference in Case 3b is the estimation of UE position. As LMF deploys the AI/ML model in Case 3b, the estimated UE position is internally available at LMF without any specification impact.

For Case 3b, No RAN1 specification impact is expected for inference.

## Signaling and procedures for inference

### Inference for Case 1

For the inference operation in Case 1, LMF would request UE to perform inference e.g., after LMF receiving a location services (LCS) location request. In response, UE reports its location estimate inferred by the AI/ML functionality to the LMF. Such signaling can utilize LPP Request/Provide Location Information messages, with necessary enhancements to be determined after sufficient RAN1 progress.

Inference of UE-side models may be requested by LMF via LPP Request Location Information. In response, UE reports its inference output to LMF via LPP Provide Location Information.

### Inference for Case 3a

The inference operation at gNB would be necessary when LMF determines to utilize an UL positioning method upon an LCS location request. Thus, LMF could request one or more gNBs involved in the inference operation of their AI/ML models enabled by utilizing NRPPa messages. In addition, the necessary enhancements to be determined after sufficient RAN3 progress.

To perform the desired inference operation, the target UE needs to be configured with UL SRS transmission. While the transmission of the UE could be configured by its serving gNB, the characteristics of the configuration (e.g., bandwidth, periodicity, etc.) could be determined as it is done in legacy.

### Inference for Case 3b

The positioning measurement, potential preprocessing, measurements reporting by NG-RAN (gNB/TRP) would be necessary when LMF determines to utilize an UL positioning method, respectively, upon an LCS location request. Thus, LMF could request the target UE and one or more gNB/TRPs that were determined to be involved in the positioning inference, by requesting positioning measurements by using the legacy framework.

For Case 3b (LMF side model), for inference LMF may request reporting of UL measurements from gNB via NRPPa.

# Functionality Framework

The aim of the functionality framework (as described in [2] section 4.4) is to cover a general functional architecture addressing functionality-based LCM.

To discuss aspects on functionality-based LCM and its relation to positioning feature and UE’s capabilities, two generic examples of positioning features are listed in Table 3.

Table 3 - Example of positioning features for Case 1 direct AI/ML positioning.

| Positioning feature examples | Description |
| --- | --- |
| Direct AI/ML positioning with UE-side model using RSRPP measurements. (AIML_direct_DL_RSRPP_ UEside) | the fingerprint position is estimated by the UE-side model when the inference input is RSRPP (Case 1 AI/ML). |
| Direct AI/ML positioning with UE-side model using PDP measurements and NW-side monitoring assistance capabilities. (AIML_direct_DL_PDP_ UEside_NWsideMonitoring). | The fingerprint position is estimated by the UE-side model when the input is PDP (Case 1 AI/ML). In addition, the UE has capabilities to assist NW in doing performance monitoring. |
| AI/ML assisted positioning with UE side model using RSRPP measurements. (AIML_assisted_UEside) | UE-assisted/LMF-based positioning with UE-side model where the intermediate feature is estimated at the UE-side when the input is RSRPP (Case 2a AI/ML positioning) |

A positioning feature can be thought as a positioning method defined in legacy NR positioning framework. Then, the reporting of UE conditions for Case 1 may be viewed as UE capability report. Hence, the legacy framework with the LPP protocol specified in [5] may be reused for functionality-based LCM purposes. Furthermore, the legacy framework can be used to enable performance monitoring of functionalities since the framework already offers e.g. position estimate and/or measurement report for different positioning methods from UE to LMF over LPP protocol. However, to exploit LPP protocol, new positioning methods based on AI/ML and new IEs may need to be introduced.

A positioning feature corresponds to a positioning method defined in legacy NR positioning framework. Hence, legacy framework with LPP protocol for e.g., capability reporting, measurement reporting can be exploited for functionality-based LCM purposes.

To support a positioning feature for Case 1, a certain set of background conditions must be supported at the UE. Here, the condition may capture a UE capability in supporting an AI/ML operation to enable the positioning feature.

Note that, as captured in [2], a functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

A functionality in positioning use cases can be viewed as a configuration of a set of UE conditions to enable a positioning feature. Hence, a specific functionality is characterized by a set of specific UE conditions and the parameter values of the conditions. Examples of UE conditions include supported number of consecutive time domain samples of RSRPP for input parameter (Nt), supported number of TRPs (N_TRP), supported set conditions for measured DL PRS. Note that a specific positioning feature listed in Table 3 can be realized by several functionalities where each functionality is configured to utilize certain combination of UE conditions (in-turn UE capabilities). Furthermore, to avoid ambiguity, as reference purposes, Table 4 list a potential example (other approaches are expected) which show relation between features, functionalities, and UE conditions.

Table 4 – Example of specific functionality characterized by a set of specific UE conditions and associated parameter values of the condition that realizes an AI/ML-enabled positioning feature.

Feature: AI/ML_direct_DL_RSRPP_UEside Functionality 1-01 : N’t = 64 only, N_TRP = 12 only Functionality 1-02: Nt = 128 only , N_TRP = 1,…,18 only Functionality 1-03: …. Functionality 1-04: …. Note: The parameter values in UE conditions used to distinguish functionalities are used only for illustrative purposes in the example.

Based on the above discussion, new AI/ML-based positioning methods can be defined in LPP (under Section 6.5 of [6]) along with the legacy positioning methods (i.e., OTDOA positioning, DL-TDOA positioning, NR UL positioning, etc.). Here, each new method can reflect the AI/ML-enabled positioning feature as captured in Table 3.

Furthermore, for a UE, the new positioning method can be configured by the network using LPP. Note that there may be multiple LPP configuration possibilities for the same positioning method. That is, the same AI/ML-based positioning method can be configured differently (e.g., one configuration may set N_TRP as 12 and the other may set N_TRP as 18). Each of such LPP configuration of the new method can then be viewed as a functionality.

RAN 1 to discuss the potential approaches on reusing or enhancing the current LPP to enable the functionality framework for AI/ML positioning.

For cases 1, inference configuration for AI/ML positioning functionality is indicated to UE via the PRS configuration provided in LPP Assistance Data and the measurement configuration provided in LPP Request Location Information.

In RAN1#118bis, the following proposal was discussed offline between companies:

Proposal 7.2.3-1 Rel-19 support functionality-based LCM for AI/ML positioning Case 1. - To prepare for inference operation, the target device reports the applicable functionalities by sending a ProvideCapabilities message to the LMF. LMF can send a RequestCapabilities message to inquire on the capability of the target device.

Further, the RAN2 agreements in this aspect includes the following:

Agreements (RAN2#126) - The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. - wait for RAN1 for associate ID discussion - At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements

Agreements (RAN2#127bis) 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

As discussed above, a functionality captures a configuration that enables an AI/ML positioning feature. This configuration is based on UE capabilities. Therefore, the UE capability directly impacts the configuration (i.e., selection of the configuration parameters) and hence needs to be reported to LMF (cf. Step 2) so that the configuration chosen by the LMF for AI/ML positioning is compatible with the UE capability.

Note that it is expected that the UE may have (or have access to) multiple models which may be associated with different network side (additional-)conditions. That is, depending on the network-side (additional-)conditions, the UE may select one model over the other. Hence, the network-side (additional-)condition information is provided to the UE (cf. Step 3).

Since the models at the UE are linked to the network-side (additional-)condition information, for a given network-side (additional-)condition, the UE may have only a subset of its model(s) available. In other words, only a subset of the models may be applicable for the given network-side (additional-)condition. Consequently, only the subset of the UE-side conditions is applicable for the given network-side (additional-)condition. This needs to be indicated to the LMF so that only the applicable conditions are taken into account while setting up a suitable configuration for the AI/ML operation (cf. Step 4). Here, for a given set of models for a UE, the applicable condition(s) is a subset of the conditions that is associated with the applicable model(s) linked to the given network-side (additional-)condition information. Hence, for a given set of models, while the capability of the UE does not change dynamically and remains static, the applicability of the UE conditions varies only based on the network side (additional-)condition information. For Rel. 19, no other criteria may be to the applicability of the UE conditions.

For a given set of models for the UE, the UE conditions and hence the capability of the UE remains static, but the applicability indication of step 4 in the RAN2 agreement for LCM for UE sided model varies depending on the network-side (additional-)condition(s).

UE may report applicable conditions to LMF via UE capability report, where the applicable conditions are the subset of conditions that can be supported at the UE for the given network-side (additional-)condition.

## Functionality framework assumptions

Some practicalities on managing the set of functionalities defined for one specific UE or UEs may report dynamic/online information to the LMF which may impact on the functionality setting. However, to avoid additional specification impact, no reconfiguration is expected after the functionality was set by the LMF.

For UE-sided models, no reconfiguration is expected on any functionality based on the preferred dynamic/online indications coming from the UE.

In cases where the UE is deriving the monitoring metric, one important assumption should be considered in terms of the Functionality. Here, the UE may not be allowed to change/modify the functionality. Having LMF to take the functionality decision is more reasonable as compared to UE because 1) network has much better knowledge of the entities involved in functionality setting/monitoring, 2) Autonomously change of functionality at the UE may degrade the positioning performance and also leads to waste the UE resources, 3) Network has better understanding of available resources (time/frequency) and spatial distributions of UEs,  4) In the legacy approach network decides the positioning methods, in similar fashion we can leverage existing positioning framework for functionality decision driven by network.

For all use cases (e.g., Cases 1, 3a, 3b) LMF is the only entity to determine functionality decision (e.g., functionality switching, selection, (de)activation, fallback) based on monitoring outcome.

# AI/ML aided positioning round-trip methods

During the study item, it has been observed that the AI/ML positioning model exhibits various degree of robustness w.r.t. the magnitude of the synchronization error. When the synchronization error is variable – the typical case with the UE being synchronized only to its own gNB and not to all TRPs, the AIML model needs to be to dynamically compensate for changes in said error and this may result in more complex models, which need considerably more training data to use. To counteract such effects, legacy positioning introduced time-difference of arrival and round-trip methods. To enable the former method in AI/ML positioning, the LMF needs to coordinate the training data collection between the UE and the gNB, since a single training sample has now multiple sources.

To cope with synchronization errors, legacy round-trip methods may be reused in AI/ML positioning and to that end, new measurements may need to be defined to ensure collection of coherent measurements from the UE and the gNB.

There are variant of applying multi-RTT using AI/ML, however, it may require additional specification effort. Thus, if multi-RTT is to be considered in the specification, it must be limited to the legacy including some additional assistance data reporting between involved entities.

The AI/ML multi-RTT positioning method may be built on top of legacy multi-RTT positioning by including at least new assistance data sent by the LMF to the UE/gNB to coordinate the deployment and usage of AI/ML RTT functionalities.

# Conclusion

In this contribution, we discuss details of AI/ML for positioning enhancement use case and have the following observations and proposals:

## Observations

Observation 1: The Associated ID are a set of identifiers exclusive to a network vendor.

Observation 2: During inference stage, the GCI(s) and available NW-identifier(s) (associated ID) for PRS configurations shall be included in the configuration, which are used by the UE to identify the applicable models for inference.

Observation 3: To resolve the SFN time info ambiguity with reduced signaling overhead, the SFN time info for a channel measurement and/or a label can be complemented with the UTC time info only when there is a change in SFN cycle between two consecutive channel measurements and/or labels in the report.

Observation 4: For assisted Case 1 monitoring and Inference Case 3b, to reduce the signaling overhead of reporting measurements from UE (case 1) and NG-RAN (Case 3b) to LMF, UE/NG-RAN may use a reporting scheme based on differential quantization indicating at least the number of RSRPP messages for carrying the measurement report.

Observation 5: The discussion of the definition on sample-based representation should include Case 1, to enable label-based monitoring cases, if required.

Observation 6: The starting time of the list of Nt samples can be based on the first detected path. The value range of samples is based on two key factors, the time offset of the strongest samples relative to reference point and the statistical parameters applied acros those strongest samples.

Observation 7: There is trade-off between accuracy and signaling overhead when extracting and reporting the number of samples.

Observation 8: The existing timing quality information is not sufficient to capture the machine learning aspect for AIML assisted positioning (Case 3a), and it should be enhanced.

Observation 9: A positioning feature corresponds to a positioning method defined in legacy NR positioning framework. Hence, legacy framework with LPP protocol for e.g., capability reporting, measurement reporting can be exploited for functionality-based LCM purposes.

Observation 10: For a given set of models for the UE, the UE conditions and hence the capability of the UE remains static, but the applicability indication of step 4 in the RAN2 agreement for LCM for UE sided model varies depending on the network-side (additional-)condition(s).

Observation 11: To cope with synchronization errors, legacy round-trip methods may be reused in AI/ML positioning and to that end, new measurements may need to be defined to ensure collection of coherent measurements from the UE and the gNB.

## Proposals

Proposal 1: RAN1 to standardize both monitoring approaches, label-free based monitoring and label-based monitoring for first priority cases.

Proposal 2: For Case 1 and Case 3a, label-free monitoring may consist of: i) confidence level associated with model inference output, e.g., UE location estimate; ii) statistics of measurement(s) compared to the statistics of measurements in the training data set; iii) standard deviation as monitoring metric based on the historical/buffered/successive inference output.

Proposal 3: For Case 1, necessary data for label-free monitoring consists of i) statistics of the measurements corresponding to inference input, i.e., RSRPP measurements; ii) historical/buffered inference output (UE location); and iii) characteristics (e.g., statistics) of training data.

Proposal 4: For AI/ML positioning Case 1 and Case 3a, label-free monitoring methods are supported, where the entity hosting the model for inference execute the performance monitoring without external information on the ground truth label.

Proposal 5: For label-free monitoring in Case 1, UE sends at least the monitoring outcome (indication of events, e.g. failure) to LMF and/or monitoring metric output to LMF (LMF does always the final decision). FFS: other specification impact (e.g., assistance required by label-free monitoring, signalling of statistics of model input, statistics of model output).

Proposal 6: Performance metric based on ground truth label (or its approximation) is defined as the statistical difference (e.g., MSE) between ground truth label and inference output, where measurements associated with ground truth information can be used as inferene input.

Proposal 7: Necessary data for label-based performance monitoring (or its approximation) consists of at least: i) ground truth (or its approximation), e.g., collected from PRU; ii) measurements corresponding to inference input, e.g., measurements collected from PRU; and iii) inference output, e.g., UE location estimation.

Proposal 8: For Case 1 monitoring, LMF may request UE to derive a performance monitoring metric, and UE reports the derived performance metric or reports monitoring outcome to LMF.

Proposal 9: For label-based monitoring of Case 1, for UE to derive monitoring metric, support Option A-3: LMF provides the UE necessary data for monitoring over LPP which contains measurements (e.g., measurements (e.g., RSRPP) collected from PRU(s)) and associated ground truth (e.g., PRU location) including quality indicators. In addition, target UE computes the monitoring metric internally and provides it to the LMF if requested to do so.

Proposal 10: For label-based monitoring of Case 1, for LMF to derive monitoring metric, support Option B-2: PRU(s) provides measurements (e.g., RSRPP) to LMF. LMF provides measurements collected from PRU(s) to the UE. UE reports inference output, based on measurements collected from PRU(s), to LMF. LMF compares the UE inference output with PRU ground truth labels associated with these measurements to calculate monitoring metric.

Proposal 11: For label-free monitoring of UE-side models, for UE to derive monitoring metric, LMF may provide UE necessary data and configuration for monitoring.

Proposal 12: For label-free monitoring of UE-side models, for LMF to derive monitoring metric, UE may provide LMF necessary data for monitoring, which may contain: (i) statistics of UE measurements (e.g., RSRPP), (ii) standard deviation of UE inference output (e.g., UE location estimation in Case 1).

Proposal 13: RAN1 should not do any assumption or decision on Case 3a regarding performance monitoring.

Proposal 14: RAN1 to delay the discussion on any aspect related to performance monitoring for Case 3b.

Proposal 15: RAN1 to consider only the measurement reporting from gNB to LMF for assisting on the performance monitoring for LMF-side model cases.

Proposal 16: RAN1 to consider that whether/what assistance information is sent from UE/PRU or gNB to LMF for performance monitoring of LMF-side model, the final decision on using such assistance is up to the responsible NF (to be determined by SA2).

Proposal 17: For label-based monitoring, LMF may provide to target UE, measurements collected from different PRUs along with a similarity context (e.g., threshold for a predetermined similarity score) to assist selecting a suitable PRU.

Proposal 18: RAN 1 to consider the following approaches to ensure consistency between training and inference regarding NW-side additional conditions: (i) consistency based on Assistance data, (ii) based on Associated ID,  and (iii) consistency assisted by monitoring.

Proposal 19: For UE-side models, to ensure consistency between training and inference related to NW-additional conditions, an assistance data (e.g. PRS configuration) used for data collection (training) or inference shall be associated with a global cell identity (GCI) and a network-identifier (associated ID). FFS: details of network identifier (associated ID).

Proposal 20: For Case 1, RAN1 to study the possibility defining validity area and granularity requirements of this area for consistency purposes.

To ensure consistency between training and inference, UE can indicate the timeframe of the dataset it used for training, upon which the NW compares the PRS configuration and Associated ID between the indicated timeframe and the upcoming inference time, and informs UE about any changes in the PRS configuration and Associated ID.

Proposal 21: RAN 1 to consider the following categorization of IEs related to Case 1 for consistency purposes:

|  | Information | Case 1 |
| --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | E |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs | I |
| 3 | DL-PRS configuration of candidate NR TRPs | E |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation | E |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) | E |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | I |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | O |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | I |
| 9 | PRS-only TP indication | O |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID | E |
| 11 | LOS/NLOS indicators | I |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation | E |
| 13 | Validity Area of the Assistance Data | E |
| 14 | PRU measurements together with the location information of the PRU | O |
| 15 | Data facilitating the integrity results determination of the calculated location | O |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) | I |
| 17 | Expected Angle Assistance information | O |
| 18 | PRS priority list | O |

Proposal 22: The association information of DL-PRS resources with TRP Tx TEG ID is explicitly indicated to UE as in the legacy assistance data.

Proposal 23: LMF may request UE or gNB to transmit UL SRS / DL PRS or provide their measurements with specific Tx/Rx timing error. For ensuring consistency, UE may indicate to LMF timing errors it assumed for training of UE-side model.

Proposal 24: To RAN1 further discuss the following options to ensure consistency between training and inference regarding NW-side additional conditions: Option 1 (UE-sided model assessment), Option 2 (UE-sided functionality assessment), and Option 3 (NW-side functionlity assessment with UE-assistance).

Proposal 25: The quality of content of the dataset used for training, RAN1 considers the sample’s density (#samples/m2) and the distribution similarity between the target dataset and the uniform distribution.

Proposal 26: For ground truth data collection (Part B), to assess the quality of ground truth, LMF may assist UE for obtaining label consistency score/quality of the positioning estimates from one or multiple positioning sources.

Proposal 27: For data collection of high-quality ground truth, LMF assists UEs on linking the PRU location (absolute information) with the relative information generated by sensors (e.g. accelerometers, gyros, magnetometers) deployed in the UE.

Proposal 28: For training data collection of AI/ML based positioning, quality indicator of label is represented by a value between a predefined range (e.g. between 0 and 1 with 1 indicating the highest quality). FFS the predefined range.

Proposal 29: For training data collection of AI/ML based positioning, an additional data (e.g., TRP ID) may be included in Part A along with the data type to indicate if it is mandatory or optional.

Proposal 30: For reporting of UTC time with SFN time info as time stamp of channel measurements and label is considered in RAN1 as configurable.

Proposal 31: Send an LS to RAN2 to query on the feasibility and validity of the working assumptions made for training data generation of AI/ML based positioning Case 1.

Proposal 32: In data collection for ground truth generation by the target UE for Case 1, LMF may indicate UE positioning method(s) (e.g., non-NR and/or NR RAT-dependent and/or NR RAT-independent) with necessary criteria (e.g., required confidence value) for an estimation to be used as ground truth.

Proposal 33: For the data collection of Case 3b (LMF-side), for inference input, depending on the channel observation, UE/gNB can determine/select the required reporting configuration to enable the reporting windowing scheme without compromising the DL/UL measurement content.

Proposal 34: RAN1 to consider both: path-based and sample-based representation for AI/ML positioning cases.

Proposal 35: When sample-based representation is used, LMF configures the UE/gNB to determine N’t and choose sample selection strategy/rule-based for a fair trade-off between positioning requirements and signaling overhead.

Proposal 36: For sample-based, LMF configure the UE/gNB for starting time of list of Nt samples and value range of Nt. The optimal value for the time offset and range of Nt is up to UE-implementation.

Proposal 37: As a starting point, Option-D can be considered for further investigation while other options are not precluded.

Proposal 38: RAN1 to study the definition of a specific quality indicator for Sample-based representation. FFS: the signalization involved on the reporting of the quality indicator.

Proposal 39: CIR is not supported for inference input when the model running at the LMF-side, gNB-side or UE-side cases. In addition, if first legacy RSCP/RSCPD are considered as inference input, the LoS determination and integer ambiguity needs to be studied.

Proposal 40: In Case 3a, when timing information is estimated with AI/ML, the legacy reporting framework is reused with no LOS/NLOS indicator associated with this estimation.

Proposal 41: The decision of enabling Case 3a by a new method and new reporting framework is up to RAN3. So, RAN1 should wait for potential queries from RAN3 before to discuss on this topic.

Proposal 42: During model inference for Case 3a, AIML model confidence is taken into account for the quality of timing information assessment. FFS how model confidence is captured in the definition of the timing quality indicator.

Proposal 43: For Case 3b, No RAN1 specification impact is expected for inference.

Proposal 44: Inference of UE-side models may be requested by LMF via LPP Request Location Information. In response, UE reports its inference output to LMF via LPP Provide Location Information.

Proposal 45: For Case 3b (LMF side model), for inference LMF may request reporting of UL measurements from gNB via NRPPa.

Proposal 46: RAN 1 to discuss the potential approaches on reusing or enhancing the current LPP to enable the functionality framework for AI/ML positioning.

Proposal 47: For cases 1, inference configuration for AI/ML positioning functionality is indicated to UE via the PRS configuration provided in LPP Assistance Data and the measurement configuration provided in LPP Request Location Information.

Proposal 48: UE may report applicable conditions to LMF via UE capability report, where the applicable conditions are the subset of conditions that can be supported at the UE for the given network-side (additional-)condition.

Proposal 49: For UE-sided models, no reconfiguration is expected on any functionality based on the preferred dynamic/online indications coming from the UE.

Proposal 50: For all use cases (e.g., Cases 1, 3a, 3b) LMF is the only entity to determine functionality decision (e.g., functionality switching, selection, (de)activation, fallback) based on monitoring outcome.

Proposal 51: The AI/ML multi-RTT positioning method may be built on top of legacy multi-RTT positioning by including at least new assistance data sent by the LMF to the UE/gNB to coordinate the deployment and usage of AI/ML RTT functionalities.

# References

| [1] | Qualcomm (Moderator), “RP-234039 - New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN Meeting #102, Edinburgh, 2023. |
| --- | --- |
| [2] | Qualcomm (Moderator), “3GPP TR 38.843 - Study on Artificial Intelligence (AI)/Machine Learning (ML) Rel. 18,” 2023. |
| [3] | Qualcomm (Moderator), “RP-242399 - Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) Rel. 18,” 3GPP TSG RAN Meeting #105, Melbourne, 2024. |
| [4] | Nokia, “R1, 2402997 - AI/ML for Positioning Accuracy Enhancement , 3GPP TSG RAN WG1 #116-bis,” Changsha, China, 2024. |
| [5] | 3GPP, “TS 37.355 - LTE Positioning Protocol (LPP),” 2024. |
| [6] | 3GPP, “TS 38.215 - NR Physical layer measurements,” 2024. |
| [7] | 3GPP, “TS 38.214 - NR Physical layer procedures for data,” 3GPP, 2024. |

# Appendix A

The agreements reached on th [5]e agenda item 9.1.2 related to AI/ML for Positioning Accuracy Enhancement are indicated in each 3GPP RAN1 meeting.

Agreements reached in 3GPP RAN1#116 meeting

Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE. Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement - For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information. Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

Agreements reached in 3GPP RAN1#116-bis meeting

Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Conclusion - It is out of RAN1 scope to decide whether/how synthetic data (i.e., not direct physical data) and related entities are used in AI/ML based positioning. In RAN1 discussion, data (e.g., measurement data, label data) refer to physical data, not synthetic data. Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 2a, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A.NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Agreements reached in 3GPP RAN1#117 meeting

Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Agreement Draft LS R1-2405577 is endorsed in principle by adding the latest agreements made in this meeting and adding “agreements” to “Note: the working assumptions above are based on RAN1 understanding for RAN work item (NR_AIML_air).” Agreement Final LS R1-2405578 is endorsed.

Agreements reached in 3GPP RAN1#118 meeting

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB. Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by UE, the Nt’ selected samples are expected to be selected based on a rule. - FFS: the rule - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Agreement For Rel-19 AI/ML based positioning Case 2b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameters values of Nt, Nt', to the UE via LPP, subject to UE capability. - FFS: k

Agreements reached in 3GPP RAN1#118bis meeting

Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information Conclusion For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB. Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Agreement For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable. Information; UE-based DL-TdoA; UE-based DL-AoD. 1; Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement. 2; Timing relative to the serving (reference) TRP of candidate NR TRPs. 3; DL-PRS configuration of candidate NR TRPs. 4; Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation. 5; SSB information of the TRPs (the time/frequency occupancy of SSBs). 6; Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). 8; Fine Timing relative to the serving (reference) TRP of candidate NR TRPs. 9; PRS-only TP indication. 10; The association information of DL-PRS resources with TRP Tx TEG ID. 11; LOS/NLOS indicators. 12; On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation. 13; Validity Area of the Assistance Data. 14; PRU measurements together with the location information of the PRU. 15; Data facilitating the integrity results determination of the calculated location. 16; TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP). 17; Expected Angle Assistance information. 18; PRS priority list. [1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0 [2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0