3GPP TSG RAN WG1 #119			 R1-2410013

Orlando, US, November 18th – 22nd, 2024

Agenda item:	9.7.2

Source:	Moderator (Xiaomi)

Title:	Summary #3 on ISAC channel modelling

Document for:	Discussion/Decision

Introduction

In RAN #105, the updated SID on channel modelling for Integrated Sensing and Communication (ISAC) was agreed. The change is to remove the checkpoint for additional study beyond channel modelling [1].

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

This document summarizes the contributions and discussions on ISAC channel modelling in RAN1 #118bis meeting.

- The proposals in this document are tagged and color coded respectively

  - [H]: High Priority, plan to handle it in offline and online session.

  - [M]: Medium Priority, no plan for offline/online session unless a clear consensus is observed.

  - [L]: Low Priority, no plan for discussion in offline/online session. However, please provide your comments which could be good reference for further study.

- The new/revised proposals in current round for discussion is further tagged with [FL4]. Note: Some proposals without update are still marked with early round number. If any company didn’t provide a comment yet, or prefer to update the comments, feel free to continue comment on them.

The following email thread is assigned for discussion of the study item (agenda 9.7):

[119-R19-ISAC] Email discussion on Rel-19 ISAC channel model – xiaomi (Yingyang)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

Proposed online proposals

Tuesday (11/19)

After Tuesday online session

Agreement

Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction.

[H][FL1] Proposal 4.2-1-rev1

For the components A, B1, B2 of the RCS model a scattering point of a target for monostatic, at least for a small size UAV:

- The component A is expressed in dBsm scale

- The component B1 is expressed in dB scale

- The component B2 is expressed in dB scale

- The log-normal distribution of component B2 is charactered by a Gaussian distribution (μB2, σB22)

  - Note: there is a fixed relation that μB2=-ln1020σB22

  - Note: the mean of linear B2 values is expln1010μB2+ln210200σB22=1

Wednesday (11/20)

After Wednesday online session

Agreement

- To generate indirect paths of NLOS ray + NLOS ray in the target channel

  - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

    - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2,

      - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1

  - Other methods are up to company choice for complexity reduction

  - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration

- The power threshold for path dropping is X=[-25] dB

  - X is relative to the strongest indirect path in the target channel

- FFS: further power normalization of target channel is performed after path dropping,

- Note: power normalization when target channel and background channel are combined can be discussed separately

- FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

Agreement

The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1=0 dB

  - A is mean RCS value

  - B2 is modelled using log-normal distribution

- Model 2

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

- FFS: conditions for using which model

Agreement

The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

Agreement

When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- the recommended five scattering points are located in front, left, back, right and roof side of the vehicle

- the following RCS model is supported for each scattering point

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

[H][FL2] Proposal 6.3-1-rev2

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- Option 1: randomly drop N virtual receives following certain distribution, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario. FFS the distribution for virtual receiver dropping

  - N is determined by validation results for each scenario, FFS value N

  - Note: Validation results are necessary to set the parameters of the N receivers for the scenario

- The same existing TR for TRP-TRP or UE-UE bistatic sensing mode is used to generate the background channel between the TRP or UE as sensing TX and each of the N virtual receivers.

Solution B (Previously option 2)

- drop N virtual targets following certain distribution, generate the target channel for the N virtual targets, the sum of the N target channels is the background channel

  - Alt 1: only direct path is modeled for each virtual target

  - Alt 2: both direct path and indirect paths can be modelled for each virtual target

- FFS the number, the distribution, RCS, XPR, etc. for virtual targets

Solution C (Previous Option 4)

The BS mono-static background is modelled complying with the framework in TR38.901 with updated parameters captured in R1-2409394

- The path loss is modelled as LrdcdB~X(μrdcf,σSF);

- The reference  ϕref,  AOD is modelled as ϕref,  AOD~uniform(0°,360°) ;

- The reference θref,ZOD is modelled as 90° and 155° for reflection/diffraction and scattering, respectively;

- The ZOA = ZOD and AOD = ZOD in the mono-static background;

- The generation steps are modelled as in section 4;

- The updated parameters are modelled as in Table 7

Agreement

EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model.

Agreement

- If blockage/forward scattering between sensing targets is not considered, a propagation path from Tx to Rx interacting with more than one sensing targets is not modelled

- FFS whether/how blockage/forward scattering can be modelled in the target channel.

Agreement

- Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula,

rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft

Where,

    - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point

    - rtx,n,mT  is the spherical unit vector at transmitter for the link from Tx to the scattering point

    - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx

    - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx

  - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

    - ft is only applicable for indirect path

    - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray

    - Support two terms of  2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray

  - Doppler is separately determined for each of the multiple scattering points of a target

  - vspt can include macro-Doppler and/or micro-Doppler motion, vspt=vmacrot+vmicro,pt,

    - Note: the equation above is missing “=” sign after vsp

  - FFS: maximum speed of moving scatterers

  - FFS: ratio of moving scatterers among all scatterers

[H][FL2] Proposal 5.3-2-rev1

- When indirect path in the target channel of a target is modelled by at least stochastic clutter(s), the large-scale scaling factor LTX-ST-RX of the target channel is determined by,

LTX-ST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB

Where,

    - PLd1is pathloss between Tx and target, where d1 is the distance between Tx and target

    - PLd2 is pathloss between Rx and target, where d2 is the distance between target and Rx

    - A is RCS component A of the target (note: A to replace σRCS,A in the equation above)

    - SFdB is the shadow fading of the target channel

      - SFdB=SFdB,1+SFdB,2, where, SFdB,1,SFdB,2 are respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

- The presence or not of EO type-2 in the target channel doesn’t change the determined large-scale scaling factor LTX-ST-RX

- LTX-ST-RX is used to scale the generated channel coefficient of the target channel

  - FFS impact of power normalization if supported between target channel and background channel

Thursday (11/21)

[H][FL4] Proposal 7.1-1-rev2

- The following options are supported to generate the combined ISAC channel for bistatic

  - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by simply summing the target channel(s) and background channel, i.e., power normalization is not considered.

  - Option 2: As an additional model, power normalization is considered when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between

    - Alt 1: Power normalization on both target channel and background channel by a common scaling factor

    - Alt 2: Power normalization on background channel only

    - Alt 3: the target channel of a target will replace one cluster in the background channel

- No additional limitation on the total number of clusters in the ISAC channel model

- FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2

- FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

[H][FL4] Proposal 4.3-1-rev2

To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ

  - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π]

  - FFS correlation between αi,1,αi,2,βi,1,βi,2

  - FFS specular reflection

The following options are considered for further study, down select one option from the following

- Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio

  - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution

- Option 2 (NIST): αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i

- Option 3 (Ericsson): αi,1,αi,2,βi,1,βi,2 are variables generated for path i

  - CPMsp,i defined in LCS

- Option 4 (Huawei): αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i

[H][FL4] Proposal 5.3-2-rev2

- When indirect path in the target channel of a target is modelled by at least stochastic clutter(s), the large-scale scaling factor LTX-ST-RX of the target channel is determined by,

LTX-ST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgA+SFdB

Where,

    - PLd1is pathloss between Tx and target, where d1 is the distance between Tx and target

    - PLd2 is pathloss between Rx and target, where d2 is the distance between target and Rx

    - A is RCS component A of the target

    - SFdB is the shadow fading of the target channel

      - SFdB=SFdB,1+SFdB,2, where, SFdB,1,SFdB,2 are respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

- The presence or not of EO type-2 in the target channel doesn’t change the determined large-scale scaling factor LTX-ST-RX

- LTX-ST-RX is used to scale the generated channel coefficient of the target channel

  - FFS impact of power normalization if supported between target channel and background channel

[H[FL4] Proposal 5.6-1-rev1

- The orientation of EO type-2 should be defined in the evaluation parameters of a sensing scenario if applicable

- The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2

[M][FL4] Proposal 5.2-3-rev2

To avoid extremely large RCS values, component B2 of RCS is upper bounded by kσ dB for the log-normal distribution. FFS k as scaling factor

[H][FL4] Proposal 6.3-1-rev2

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- drop N virtual receives following certain distribution, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario. FFS the distribution for virtual receiver dropping

  - N is determined by validation results for each scenario, FFS value N

Supported by: ZTE, Xiaomi, LGE, Samsung, Lenovo, MTK, BUPT, CATT, vivo, OPPO, NIST

Solution B (Previously option 2)

- drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel

  - Alt 1: only direct path is modeled for each ground clutter

  - Alt 2: both direct path and indirect paths can be modelled for each virtual target

- FFS the number, the distribution, RCS, XPR, etc. for ground clutters

Supported by: Ericsson, Nokia, [QC]

Solution C (Previous Option 4)

the BS mono-static background is modelled complying with the framework in TR38.901 with updated parameters captured in R1-2409394

- The path loss is modelled as LrdcdB~X(μrdcf,σSF);

- The reference  ϕref,  AOD is modelled as ϕref,  AOD~uniform(0°,360°) ;

- The reference θref,ZOD is modelled as 90° and 155° for reflection/diffraction and scattering, respectively;

- The ZOA = ZOD and AOD = ZOD in the mono-static background;

- The generation steps are modelled as in section 4;

- The updated parameters are modelled as in Table 7

Supported by: Huawei,

[M][FL4] Proposal 5.8-1-rev2

When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

- Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

Proposed offline proposals

Monday (11/18)

After Monday offline session

[H][FL1] Proposal 4.1-1-rev1

- Bistatic RCS values/patterns for a scattering point of a target are to be obtained by measurement, or ray-tracing model, then the ray-tracing based results to validate the ISAC channel model

  - The bistatic RCS values/patterns are defined modeled by fixing an incident direction in LCS of target and vary the scattered directions in LCS of target; then change to other incident directions

[H][FL1] Proposal 4.2-1-rev1

To capture the component A, B1, B2 of the RCS model for a scattering point of a target UAV into the TR

- The component A is the mean of the linear RCS values and is expressed in dB scale

  - FFS targets other than UAV

- The component B1 is expressed in dB scale.

  - Note: the mean of linear B1 value equals to 1.

- The log-normal distribution of component B2 is charactered by a Gaussian distribution (μB2, σB22)

  - Note: there is a fixed relation that μB2=-ln1020σB22

  - Note: the mean of linear B2 values is expln1010μB2+ln210200σB22=1

FFS the above definition is extended to other type of targets

[H][FL1] Proposal 4.2-3

[Moderator’s note] The following RCS models are supported for human with single scattering point for monostatic

- A is mean RCS value

  - Different mean RCS values can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1=1

- FFS Model 2

  - B1 have dependency on incident/scattered angles

- B2 is modelled using log-normal distribution for monostatic

- Note: whether exact values of A/B1/B2 for Model 1 and/or model 2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model.

[H][FL1] Proposal 4.2-5

The following RCS models are supported for vehicle with single scattering point for monostatic

- A is mean RCS value

- B2 is modelled using log-normal distribution for monostatic

- Different mean RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- FFS Model 1

  - B1=1

- Model 2

  - B1 have dependency on incident/scattered angles

  - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

  - Alt 2: a piecewise function

  - Alt 3: Lookup table

- Note: whether exact values of A/B1/B2 for Model 1 and/or model 2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model.

[H][FL1] Proposal 4.2-6

For vehicle modelled with multiple scattering points for monostatic, the following RCS model is supported for each scattering point

- A is mean RCS value

- B1 have dependency on incident/scattered angles

  - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

  - Alt 2: a piecewise function

  - Alt 3: Lookup table

- B2 is modelled using log-normal distribution

- Note: whether exact values of A/B1/B2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model.

Tuesday (11/19)

After Tuesday offline session

[H][FL2] Proposal 5.2-2-rev3

- To generate indirect paths of NLOS ray + NLOS ray in the target channel

  - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

Supported by: vivo, Ericsson, OPPO, Samsung, Nokia, InterDigital, Sony

  - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

    - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2,

      - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1

    - FFS need for power normalization

Supported by: Xiaomi, ZTE, CMCC, CATT, LGE, QC, Apple, vivo (normalization after path dropping), InterDigital

  - Other methods are up to company choice for complexity reduction

  - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration

- The power threshold for path dropping is X=[-25] dB

  - X is relative to the strongest direct/indirect path in the target channel

- FFS: further power normalization of target channel is performed after path dropping,

  - Note: power normalization between target channel and background channel can be discussed separately

- FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

Wednesday (11/20)

After Wednesday offline session

[H][FL4] Proposal 4.3-1-rev2

To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ, i.e., CPMsp,i=αi,1expjΦspθθβi,1expjΦspθϕβi,2expjΦspϕθαi,2expjΦspϕϕ

  - The initial random phase Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ is [uniformly distributed within -π,π]

  - FFS correlation between αi,1,αi,2,βi,1,βi,2

  - FFS specular reflection

The following options are considered for further study, down select one option from the following

- Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp-1 is generated for path i, where κsp is XPR ratio

  - κsp is randomly generated by log-normal distribution. FFS mean/variance of the distribution

- Option 2 (NIST): αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i

- Option 3 (Ericsson): αi,1,αi,2,βi,1,βi,2 are variables generated for path i

  - CPMsp,i defined in LCS

- Option 4 (Huawei): αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i

[M][FL4] Proposal 5.2-3-rev2

To avoid extremely large RCS values, component B2 of RCS is upper bounded by kσ dB for the log-normal distribution. FFS k as scaling factor

[H][FL4] Proposal 5.3-2-rev2

- When indirect path in the target channel of a target is modelled by at least stochastic clutter(s), the large-scale scaling factor LTX-ST-RX of the target channel is determined by,

LTX-ST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgA+SFdB

Where,

    - PLd1is pathloss between Tx and target, where d1 is the distance between Tx and target

    - PLd2 is pathloss between Rx and target, where d2 is the distance between target and Rx

    - A is RCS component A of the target

    - SFdB is the shadow fading of the target channel

      - SFdB=SFdB,1+SFdB,2, where, SFdB,1,SFdB,2 are respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

- The presence or not of EO type-2 in the target channel doesn’t change the determined large-scale scaling factor LTX-ST-RX

- LTX-ST-RX is used to scale the generated channel coefficient of the target channel

  - FFS impact of power normalization if supported between target channel and background channel

[H[FL4] Proposal 5.6-1-rev1

- The orientation of EO type-2 should be defined in the evaluation parameters of a sensing scenario if applicable

- The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2

[M][FL4] Proposal 5.8-1-rev1

When the EO type-2 is modelled in the target channel, the LOS condition of the Tx-target link and target-Rx link are determined by Option 2,

- Option 2: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

[H][FL4] Proposal 6.3-1-rev2

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- drop N virtual receives following certain distribution, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario. FFS the distribution for virtual receiver dropping

  - N is determined by validation results for each scenario, FFS value N

Supported by: ZTE, Xiaomi, LGE, Samsung, Lenovo, MTK, BUPT, CATT, vivo, OPPO, NIST

Solution B (Previously option 2)

- drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel

  - Alt 1: only direct path is modeled for each ground clutter

  - Alt 2: both direct path and indirect paths can be modelled for each virtual target

- FFS the number, the distribution, RCS, XPR, etc. for ground clutters

Supported by: Ericsson, Nokia, [QC]

Solution C (Previous Option 4)

the BS mono-static background is modelled complying with the framework in TR38.901 with updated parameters captured in R1-2409394

- The path loss is modelled as LrdcdB~X(μrdcf,σSF);

- The reference  ϕref,  AOD is modelled as ϕref,  AOD~uniform(0°,360°) ;

- The reference θref,ZOD is modelled as 90° and 155° for reflection/diffraction and scattering, respectively;

- The ZOA = ZOD and AOD = ZOD in the mono-static background;

- The generation steps are modelled as in section 4;

- The updated parameters are modelled as in Table 7

Supported by: Huawei,

[H][FL4] Proposal 7.1-1-rev2

- The following options are supported to generate the combined ISAC channel for bistatic

  - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by simply summing the target channel(s) and background channel, i.e., power normalization is not considered.

  - Option 2: As an additional model, power normalization is considered when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between

    - Alt 1: Power normalization on both target channel and background channel by a common scaling factor

    - Alt 2: Power normalization on background channel only

    - Alt 3: the target channel of a target will replace one cluster in the background channel

- No additional limitation on the total number of clusters in the ISAC channel model

- FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2

- FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

Physical object modelling

General

Summary on company views

RCS in time domain

Study how to guarantee correlation over the incident and scattered angles for B2: Ericsson

RCS as a random time-domain process: LGE

The stochastic component B2​ in the RCS calculation can be modelled as a spatially and temporally correlated process.: NIST

Figure 2.1.2-4: Small-Scale RCS (B2) autocorrelation function as a function of aspect angle lag.

Ericsson: The radar cross-section of a pedestrian can be approximated as a time-varying smooth curve with some variations around it

Bistatic RCS

- Derived based on monostatic RCS, RCSBi≈RCSMo∙cosβ2: LG, Huawei

  - Negative due to the accuracy: vivo, ZTE, BUPT, Xiaomi

- By measurement or RT results: ZTE, Ericsson, Xiaomi, Apple, vivo, BUPT, AT&T, EUROCOM

How to collect bistatic RCS

- Option 1: Fix an incident direction and vary the scattered directions; then change to another incident angle

Supported by: ZTE

- Option 2: Fix a bistatic angle and vary the incident directions; then change to another bistatic angle.

Supported by: Xiaomi, NIST, Huawei

Forward RCS

Ericsson: Study whether the channel model should deal with forward scattering and what impact forward scattering has on the scattering model

Figure 1	Gains of the two propagation paths (line-of-sight and scattered) at 3.5 GHz carrier frequency of the two polarizations.

- σf=4πS2λ2: LG

- ϕf=λD: LG

LG

- discusses the impact of forward scattering. The forward scattering appears when the target object is close the line between TX and RX. In such a case, the RCS increases by several orders of magnitude. The reason for that is the forward scattering can be modeled using Babinet's principle and the corresponding RCS is determined solely by the silhouette of the target object as seen at RX. This phenomenon may drastically improve the detection performance.

- Forward scattering RCS is defined as a special case of the bistatic RCS corresponding to bistatic angles larger than 140°. Perform additional study on the properties of RCS in the forward scattering region since it can have a direct impact on many use cases.

Vivo

- In forward-scattering region, the smaller the size of object, the larger the forward scattering RCS, and vice versa.

- The forward-scattering RCS is more pronounced at bistatic angles ranging from 150˚"~18" 0˚.

- RAN1 mainly focuses the study on bistatic region for RCS design.

- RAN1 studies RCS impact on forward scattering region, considering at least the minimum bistatic angle (e.g., 150ᵒ~180ᵒ) and the size of the object.

Bistatic RCS

[Moderator’s note] Based on the contributions, most companies observe that bistatic RCS cannot be accurately derived by monostatic RCS if the bistatic angle is relatively large. Two options can be used to define the bistatic RCS values/patterns

- Option 1: Fix an incident direction and vary the scattered directions; then change to another incident angle

- Option 2: Fix a bistatic angle and vary the incident directions; then change to another bistatic angle.

With Option 1

- The RCS pattern would be angular dependent

- the RCS pattern for different incident directions can be separately defined

  - Applicable to any target type

- As a special case, same RCS pattern may be applicable for different incident angle

With Option 2

- the RCS pattern for each bistatic angle can be isotropic hence modeled by a random distribution.

  - Need to check if such modeling is applicable to any target type

- Different mean/variance of the random distribution for different bistatic angles.

Please provide your views on the following proposal. Specifically, please provide your views on the two options. Let’s see if we can do down selection for progress

[H][FL1] Proposal 4.1-1

- Bistatic RCS values/patterns for a scattering point of a target are to be obtained by measurement or ray-tracing

- Down select between the following two options for bistatic RCS model

  - Option 1: Bistatic RCS values/patterns of each incident direction is defined by varying the scattered directions.

  - Option 2: Bistatic RCS values/patterns of each bistatic angle is defined by varying the incident directions.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | Option 1. One question for option 2, is it still based on mono-static result ? In our view, even the bi-static angle is the same, the RCS is still different for varying incident directions. |
| Huawei, HiSilicon | No | Huawei has been proposing the RCS for vehicle with both mono-static and bi-static and proved up to 3dB MMSE between deriving the bistatic RCS from the mono-static table and the measured bistatic RCS. However, FL never allowed such discussion, how could it be concluded not feasible given no other companies brought such comparison. The second bullet is unclear for intention. Is it talking about how to capture the RCS for bistatic or about how to obtain the RCS by measurement or RT? |
| Intel |  | Option 1 should be prioritized as more general. Option 2 in our understanding may not be accurate for objects with obvious RCS dependency on incident and scattering angles, such as vehicles. |
| NIST |  | During our measurement and analysis, we adopted option 2. However, the difference between option 1 and option 2 is not very clear to us. RCS can be a function of both bistatic angle and aspect angle. |
| LGE |  | In option 1, the bistatic RCS is represented by the function of (incident angle, scattered angle), while in option 2 by the function of (incident angle, bistatic angle). They seem equivalent after all in that one can be converted to the other. We share the view with NIST that the difference between two options is not clear. |
| Samsung |  | Prefer to Option 1 |
| Nokia, NSB |  | These two options need further study. Option 1 should be the starting point. Option 2 might be a special consideration for bistatic with a larger bistatic angle. Not sure it is ready to have a conclusion on either Option 1 or Option 2. |
| Moderator | @Huawei: So your main concern is on the fist bullet. Due to limited TU, we didn’t discussed it in online or offline session, but already discuss it in the summary document for several meeting. Majority view from early inputs is bistatic RCS by measurement or RT. Let’s do the two bullets in parallel to accelerate the discussion. @All: seems Option 1 gets clear support. So, I delete Option 2 to check if Option 1 is OK for others. |  |

[H][FL1] Proposal 4.1-1-rev1

- Bistatic RCS values/patterns for a scattering point of a target are to be obtained by measurement or ray-tracing

  - The bistatic RCS values/patterns is defined by fixing an incident direction and vary the scattered directions; then change to other incident directions

- Down select between the following two options for bistatic RCS model

  - Option 1: Bistatic RCS values/patterns of each incident direction is defined by varying the scattered directions.

  - Option 2: Bistatic RCS values/patterns of each bistatic angle is defined by varying the incident directions.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | As observed by many companies, forward scattering is a new issue specific to bi-static sensing. RAN1 can discuss how to address it, e.g., including it as part of RCS or treating it separately with blockage. The decision is needed before companies report bi-static RCS obtained from measurements or ray-tracing. Option 2 in Proposal 4.1-1 is different from FL explanation above. Maybe Option 2 means Bistatic RCS values/patterns of each bistatic angle is defined by varying regardless of the incident directions Option 1 is preferred over Option 2. |
| ZTE | Yes |  |
| CATT/CICTCI |  | Need to clarify three issues: 1. What’s this proposal’s relationship with previous RCS agreement about A/B1/B2 ? 2. If the value/pattern to be obtained from measurement , is it a lookup table or a curve fitting ? 3 Does the measurement need to be done per sensing object (human/small UAV/large UAV/AGV etc ) ? |
| New H3C | Yes |  |
| Qualcomm |  | It appears from the discussion above that Option 1 wants to say that the bistatic RCS is a function of (incident angle, scattered angle), but it written differently in the above proposal. If that is the intention, then we are generally supportive, but still we prefer a different wording. |
| vivo | Yes | Basically, both options have the same meaning. Nevertheless, we prefer Option 2. |
| CMCC | Yes |  |
| OPPO |  | We understand the formula RCSBi≈RCSMo∙cosβ2 is not universally true. But it is not clear to us whether the proposal means that the ISAC CM TR would need to capture a lot of measurements for bistatic RCS and therefore make the bistatic RCS modeling in a way totally independent and different from mono-static RCS. Our view is: - The bistatic RCS for a target should be an analytical formula accepting contiguously varying parameters, instead of a lookup table with discrete parameter samples. - There should be some connection between mono-static RCS and bi-static RCS, instead of leaving the two in different and independent modeling philosophy. At last, for the Option 1 and Option 2, our understanding is that the incident direction in the modeling like TR38.901 is defined in relative to a global coordination system, not in relative to target orientation. So the RCS in both options may also depend on the target orientation. |
| Spreadtrum | Yes | Although our first preference is to generate the bistatic RCS based on the monostatic RCS, we can adopt the current proposal for progress. As FL explained, for simplicity, same RCS pattern may be applicable for different incident angle for some use cases. |
| Xiaomi |  | The RCS raw data measured by the two options are equivalent. Prefer Option 2 and the bistatic RCS can be obtained based on a function or a table when a bistatic angle is determined based on the topology. If Option 1 is supported, the orientation should be considered. |
| Lenovo |  | RCS shows dependencies both as a function of incident angle (in the LCS of the object) and reflection angle (or equivalently the bistatic angle) Option 1 is prefered |
| AT&T |  | For the first bullet, it is not clear how we can obtain RCS values/patterns from ray tracing. We prefer to reword it to - Bistatic RCS values/patterns for a scattering point of a target are to be obtained by measurement - For option 1 versus option 2, if we were to model angular dependency for bistatic RCS, it seems that we will need both options. |
| Apple | Yes | Essentially, that the RCS is a function of the incident direction, i.e. orientation towards the object (similar to some of the mono-static RCSs measured) and the scattered angle. How do we handle the case where the RCS is incident angle invariant e.g. a spherical type target ? |
| Moderator | @QC, @Lenovo: revise the wording as proposed, but essentially the same proposal |  |

[H][FL1] Proposal 4.1-1-rev2

- Bistatic RCS values/patterns for a scattering point of a target are to be obtained by measurement or ray-tracing

  - The bistatic RCS values/patterns is defined as a function of incident angle and scattered angle by fixing an incident direction and vary the scattered directions; then change to other incident directions

[Moderator’s note] After Monday offline session

Note: I revise a bit on ray-tracing part, my early citing of early agreement doesn’t make a sentence.

[H][FL2] Proposal 4.1-1-rev3

- Bistatic RCS values/patterns for a scattering point of a target are to be obtained by measurement, or ray-tracing model which is validated by experiment results, then the ray-tracing based results to validate the ISAC channel model

  - The bistatic RCS values/patterns are defined modeled by fixing an incident direction in LCS of target and vary the scattered directions in LCS of target; then change to other incident directions

[Moderator’s note] agreement made in Tuesday online session

- Agreement

Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction.

Correlation of RCS in adjacent incident/scattered angles

[Moderator’s note] In reality, there will some correlation if the incident/scattered angles are quite close. However, it is not clear how much correlation is it based on the quite limited inputs. Therefore, the companies are encouraged to bring more results for validation.

- [M][FL1] Proposal 4.1-2

- Companies are encouraged to bring more results validating the correlation between RCS values in adjacent incident/scattered angles at target/scattering point.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | It is hard to the threshold of ‘close’. Based on our observation, even the two angles are 1 degree different, we still see the RCS gap which can be reflected in B2. Hence, we may not need to model this correlation. |
| Huawei, HiSilicon |  | Available results should be observed and shown instead of being disregarded while encouraging more other companies to do so |
| NIST |  | Based on our UAV measurement data, we observed that B2 correlation is a function of the aspect angle difference. |
| Ericsson | Yes | The main importance of modeling such correlation is to ensure continuity as the angles are changing continuously, i.e. due to target movement/rotation or movement by the Tx or Rx. So this is critical if spatial consistency is to be ensured. |
| New H3C | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| Xiaomi |  | Prefer to deprioritize the discussion until more validation results are provided by the companies. |
| Apple | Yes |  |

Forward RCS

[Moderator’s note] Based on the input, forward scattering is something to be considered in the RCS model, however it seems low priority at the moment.

- [L][FL1] Proposal 4.1-3

- RAN1 to study whether/how forward scattering RCS should be considered in ISAC channel modelling.

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE | Yes | Depending on the dropping of Tx, ST and Rx, the following three cases can happen. - Backward scattering (monostatic) - Bistatic - Forward scattering So we need to handle these three scattering cases in the channel modeling anyway. It’s well known that the forward scattering would enhance the bistatic RCS, which requires the further study. Putting FFS on this point to the bistatic RCS discussion is ok. |
| Ericsson | Yes | This issue should be addressed before the discussion on bi-static RCS. Otherwise, it is not clear if the bi-static RCS measurements submitted by companies include forward scattering or not. For some sensing modes and use cases, forward scattering or blocking may be high priority. This includes intruder detection. |
| CATT/CICTIC |  | Changed the proposal to : whether/how forward scattering RCS should be considered in ISAC channel modelling can be studied. |
| New H3C | Yes | Fine with CATT’s modification |
| vivo | Yes | Considering the small region incurring forward scattering RCS, its impact should be very small. Therefore, we suggest that RAN1 does not model forward scattering RCS. |
| CMCC |  | The difference among “forward scattering RCS”, “bistatic RCS value when the bistatic angle is 180° ”, “blockage model” should be illustrated. Otherwise, we can't make a choice. |
| BUPT |  | We agree with the perspective of Proposal 7.2-1 that RAN should avoid duplicating the modeling of both blocking by an object and forward scattering by the same object. More field channel measurements should be conducted to explore the issues of blockage and forward scattering in ISAC channel modeling. |

Details on RCS model

Agreement if RCS related coefficient of a scattering point is included in small scale, the RCS related coefficients are separately determined for different pairs of incident/scattered ray(s) at the scattering point. Agreement RAN1 strives to define a single option per target per monostatic/bistatic sensing mode from the following two options to generate RCS values/patterns for a scattering point of a target. - Option 2: The RCS=A*B of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - A is [mean] RCS value. FFS value(s) A - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B - B is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS - Option 3: The RCS=A*B=A*B1*B2 of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - FFS: A = 1 m2 or [mean] RCS value - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B is further split into B1, B2, i.e., B=B1*B2 - B1 is deterministic based on incident/scattered angles - FFS: B1 is defined by a function or by a table - B2 is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B2 is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS Agreement RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic - B2 of UAV is modelled using log-normal distribution for monostatic - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc. - For UAV of small size (option 2 for UAV size in UAV parameters table) - B1=1 - A is mean RCS value - For UAV of large size (option 1 for UAV size in UAV parameters table) - B1 have dependency on incident/scattered angles - A is mean RCS value

Summary on company views

Human

- RCS component B1

  - How to model B1

    - second order polynomial:

    - lookup table: CATT

    - a piecewise function: BUPT

Human

- RCS component A

  - A=1: Ericsson

  - Mean RCS: Xiaomi, vivo, CATT, BUPT, NIST, EUROCOM, IDC, CT, DOCOMO, AT&T, ZTE

- RCS component B1

  - B1=1: vivo, Xiaomi, Apple, CATT. BUPT, NIST, AT&T, DOCOMO, EUROCOM, IDC, CT, LG, ZTE

  - B1 is angular dependent: OPPO, Ericsson (elevation only)

- RCS component B2

  - Log-normal: ZTE, vivo, Xiaomi, Apple, AT&T, BUPT, Ericsson, EUROCOM, IDC, CMCC, NIST

  - Randomly generated per ray:

Vehicle

- RCS component A

  - 1 m2: Huawei, Ericsson

  - Mean RCS: Xiaomi, CATT, BUPT, QC, EUROCOM, IDC, CT, Tejas, DOCOMO, AT&T, ZTE

- RCS component B1

  - B1=1: AT&T

  - Angular dependent: Huawei, Xiaomi, Intel, CATT, BUPT, Ericsson, AT&T, DOCOMO, IDC, EUROCOM, TOYOTA, CT, Tejas (table), LG, ZTE

  - How to model B1

    - Antenna radiation power pattern: Huawei, Xiaomi, ZTE, Tejas, Intel

    - a piecewise function: BUPT

    - Lookup table: CATT

- RCS component B2

  - Log-normal: Huawei, BUPT, Ericsson, QC, EUROCOM, IDC, CT, CMCC, ZTE

- Multiple scattering points: Huawei(5), ZTE(5)

- Single point in near range, multiple otherwise: QC

AGV

- Same as vehicle: Huawei(4), Tejas

- B1=1: LG

Huawei: When the target is modelled with multiple scattering points, each scattering point is characterized with spherical angles with θ″∈0°, 180° and φ″∈-180°, 180° together with a distinct coordinate

OPPO: A=ijp1,iRCSi,jp2,jBi,j=RCSi,jklp1,kRCSk,lp2,l

Ericsson:

- If general RCS proposals are given based on measurements of monostatic RCS, they should be compatible with bistatic RCS.

- Study RCS of the types of intended and unintended targets, which may be modelled in the future study item and use the list of objects and sizes in Table 1 as a starting point

- RCS of an object is usually provided with a local coordination system (LCS).

- The incident and scattered angles based on GCS should be converted to LCS, and RCS of the converted angles is used for the path

NVIDIA: both A and B should be modeled deterministically and is dependent on incident and scattered directions at target.

Nokia: component A is modelled as Lognormalμ,σ2

Xiaomi: following two methods are considered for B1

- Quantization of measured RCS

- Interpolation of measured RCS

Ericsson: There are different interpretations on mean RCS. In the latter two interpretations, zenith angular range of a target’s RCS is different across sensing modes.

- Mean RCS is calculated over 360 degrees in azimuth and 180 degrees in zenith.

- Mean RCS is calculated over the angular range where RCS is measured.

- Mean RCS is calculated over the angular range of direct/indirect paths in target channel.

Samsung, BUPT, vivo, Ericsson: Frequency dependency

BUPT, vivo: The RCS of the sensing targets is positively correlated with frequency

Vivo: RCS is not dependent on the distance between Tx/Rx and target

BUPT: The horizontal RCS model for vehicles and large UAVs can be extended to a three-dimensional model

ZTE In RAN#118bis meeting, the above agreements about RCS model has been approved, which split RCS value in linear scale into three parts A, B1 and B2 as shown in the above Option 3. According to the agreements, A is the mean value of large scale RCS, and B (including B1 and B2) is the normalized values in small scale. However, there are two following implicit issues to be clarified. - The average operation to obtain A should been done for linear-scaled RCS value rather than dB-scale RCS value. Since the mean value of linear-scaled RCS and the mean value of dB-scale RCS are not the same, which is ERCSlinear≠10ERCSdB10 Hence, it should be clarified that A should be obtained as ERCSlinear from linear-scaled RCS values. - The B2 obeys log-normal distribution, which has mean value 1 (1 is a linear-scaled value), and variance V. However, the linear-scaled mean value of 1 of B2 is not equal to 0 mean value in dB-scale. For example, if B2 obeys a log-normal distribution with parameters μ and σ2 , which is noted as Lognormalμ,σ2. Then 10log10B2 obeys a normal distribution Νμ,σ2. The mean value of 10log10B2 is expected as μ and variance as σ2. Then the linear-scaled mean value of B2 should be derived as EB2=e1n1010μ+ln210200σ2 ( it should be noticed that EB2=e1n1010μ+ln210200σ2is not equal to 10μ10). If we assume that 10log10B2 has a mean value μ=0, then EB2=e1n1010μ+ln210200σ2 is larger than 1. To make E[B2] be 1, μ=-ln1020σ2 should be satisfied. In short, for log-normal distribution, μ = 1 in linear-scaled value is not equal to μ = 0 in dB-scaled value.

- [Moderator’s note]

According to the agreement in last meeting, the only option on RCS for UAV is Option 3. Both variations, i.e., B1 is either fixed to 1 or angular dependent are under Option 3. To have a unified design, as proposed by multiple companies, it would be reasonable to discuss RCS model for other target types based on Option 3.

As ZTE discussed (cited just above the moderator’s note), the relation between mean/variance in dB scale and that in linear scale for a random distribution is NOT just a simple transform, i.e., ERCSlinear≠10ERCSdB10. Specifically, to make mean of linear B2 value as 1, the corresponding mean in dB scale must be a negative value. In summary, for a log-normal distribution characterized by a Gaussian distribution with mean μ and variance σ2, we have the following relation

- The linear mean value of the log-normal distribution is A=expln1010u+ln210200σ2

- B1=1. Alternatively, if B1 is angular dependent, A*B1 may be viewed together as mean in a specific incident/scattered angles

- Mean of B2 in dB scale is μB2=-ln1020σ2

- Variance of B2 in dB scale is still σB22=σ2

Such relation needs to be taking into account when deriving A/B1/B2 values for a RCS model

Detailed proposals for each target type are organized in each respective sub section.

RCS values for UAV

Summary on company views

The following tables captures all inputs with proposed RCS values

| Company | Results on RCS |
| --- | --- |
| Huawei | Table 1: UAV mono-static RCS when modelled as a single scattering point RCS; Small size (0.3m x 0.4m x 0.2m, option 2 for UAV size). A; Mean value. The value is based on measurement data from companies, e.g. -20dBsm.. B1; 0dB. B2; Deviation value. The value is based on measurement data from companies, e.g. 4dB.. Proposal 3: Regarding RCS modelling for UAV: - For UAV of small size, A as the mean RCS value is -20dBsm. - For UAV of large size, A = 1 m2. |
| Samsung | Proposal 19: B1/B2 should be angle-dependent and can be modelled as follows - For zenith angles near 90° (e.g., 90 – X to 90 degree, with X close to 0), determine with a deterministic function that increases as the angle approaches 90°. - For lower zenith angles (e.g., 0 to 90 – X degree), assign fixed values according to a pre-defined table and allow slight variance for B2 in near 90° to capture observed randomness, but set the variance close to zero at lower angles. |
| ZTE | Proposal 3: For UAV of small size (Length x Width x Height: 0.3m x 0.4m x 0.2m) at 4.9GHz - B1=1 - A is mean RCS value of 0.053m2 corresponding -12.74 dbsm - B2 of UAV is modelled using log-normal distribution. Draw B2 linear-scaled value as B2=10X10, Where, X~Nμ,σ is Gaussian distributed with σ=3.6and μ=-ln1020σ2=-1.45. |
| QC | Proposal 8: With regards to RCS modelling for a small UAV, for each scattering point, for each polarization, - A is the mean with dBsm=-6.5 - B2 has σdBsm=6.2 |
| Intel | - Mean monostatic RCS of a drone (quadcopter) across angle directions for HH and VV polarization is around -18 dBsm in 10 GHz carrier frequency - Mean monostatic RCS of a drone (quadcopter) across angle directions for HV and VH polarization is around -45 dBsm in 10 GHz carrier frequency - RCS of a drone is less dependent from the angles, however, still shows peaks in some specific angular directions, - RCS cross-polarization components can be modelled in addition to the same polarization components. |
| Tejas | Proposal 4: At 9 GHz, μRCS=-10dBsm,σRCS=fnwings for x-copter class UAVs, these parameters should be computed for other frequencies as well. Proposal 5: The RCS for Flying Wing type UAVs can be approximated using sinusoid functions with perturbation as a function of shape, incident and scattering angles. Proposal 6: Based on the measurements available, RCS can also be approximated using the model proposed for antenna radiation pattern in TR 38.901. Table 2: RCS models for different categories of UAVs. UAV type; RCS Model. x-copter UAVs (Quadcopter, hexacopter and octacopter UAVs such as Phontam-2/4/6); μRCS=-10dBsm, σRCS=fdtx-target,l,h,w,nwings; where nwings denotes the number on the UAV, f=9GHz.. Flying Wing type UAVs such as Euclid UAV; μRCS=μ0θ,ϑ,θsc,ϑsc,f+αf.cos⁡2ϑ, σRCS=fdtx-target,l,h,w,∅,θ,d; where nwings denotes the number on the UAV, θ=85°.. |
| BUPT | using a piecewise function to model the B1 component. logB1=a1φ-φ12+c1,φϵ-π18,π18a2φ-φ22+c2,φϵ4π9,5π9a3φ-φ32+c3,φϵ17π18,π∪a4φ-φ42+c4,φϵ-5π9,-4π9μ,φϵothers Taking 10GHz as an example, the fitting parameters are shown in Table 3. The fitting effect is shown in Error: Reference source not found. Table 3 The fitting parameter results for the UAV at 10 GHz φ; logA; logB1; logB2. ak; φk; ck. ∪0,π18; -8.3527; -140.96; 0; 3.46; N0,2.362. 4π9,5π9; -1120.75; π2; 13.01. 17π18,19π18; -656.56; π; 11.0. 13π9,14π9; -793.78; 3π2; 14.78. Others; -1.3249. Proposal 1: For large UAVs, theA component can be modeled as the mean RCS value, the B1 component can be modeled as a piecewise function with respect to the incident angle/scattering angle, with parts having larger RCS values modeled by a quadratic function, and the remaining parts modeled as the mean. The B2 component can be modeled as a normal distribution with a mean of 0. Proposal 2: The RCS of humans, vehicles, and UAVs is positively correlated with frequency overall, and the relationship can be expressed as: RCShuman=0.1787*frequence-6.171 RCSvehicle=0.2083*frequence-11.49 RCSUAV=0.1889*frequence+1.63 |
| NIST | Observation 1: The mean RCS (A) value for the UAV in monostatic configuration is observed to be -23.8 dBsm, with a standard deviation of 4.37 dBsm. Proposal 1: The mean RCS of UAV in monostatic configuration can be included in the Table CITATION Mod24 \l 1033 [1] as shown below. polarization; frequency (GHz); mean (dBsm); standard deviation (dBsm); variance (dBsm2). NIST; VV; 28; -23.8; 4.37. Observation 1: No clear pattern or trend is observed for the mean RCS value (A) across different bistatic angles (α). The mean RCS value, calculated from the aggregated data for all bistatic configurations, is -23.69 dBsm. The component B1α,β in σtgα,β is deterministically defined as B1α,β,n=B1'n-kβ=α2,modN, where n=1,2,⋯,N and N denotes the cardinality of the set β={0,⋯,360∘}. The notation B1'n-k,modN represents a circular shift of sequence B1' by k positions and the value of k is determined by the index where β=α2 in β=0,⋯,360∘. The sequence B1' for 0∘≤β≤360∘ is defined using the second order polynomial as follows B1'β=p1β2+p2β+p3,∧0∘≤β≤90∘p1180-β2+p2180-β+p3,90∘<β≤180∘p1β-1802+p2β-180+p3,180∘<β≤270∘p1360-β2+p2360-β+p3,270∘<β≤360∘ where the fitting parameters p1,p2 and p3 are defined as p1=0.005, p2=-0.4638, and p3=5.53, respectively and B1' is expressed in dB. Proposal 3: The single scatter UAV model is summarized in Table 2.1.2.3.1. Table 2.1.2.3.1: UAV target values for single scatter RCS model - Target; - A (m2); - B1 (dB); - B2, µ; - B2, σ. - UAV; - 0.0042; - p1β2+p2β+p3p1=0.005,p2=-0.4638,p3=5.53 β=min⁡modmodβ-α2,360,180,180-modmodβ-α2,360,180; - -0.47; - 1.068. Here, the monostatic case is obtained with α=0. |
| Sony | Proposal 3：For UAV large class, no need to model the RCS correlation dependent on angles, since the correlation angle results are very small according to simulation. Table 4: Summary of RCS statistic of two UAV models with different shapes Model Description / Frequency; Polarization; mean [dBsm]; standard deviation [dBsm]; Correlation angle [°]. UAV Model 1 (x-copter class with size 1.6m x 1.5m x 0.7m) / 3.5GHz; V-V; 0.08; 6.57; 0.41. H-H; 0.14; 6.71; 0.47. V-H; -9.92; 6.61; 0.76. H-V; -14.17; 7.95; 1.57. UAV Model 2 (x-copter class with size 1.6m x 1.5m x 0.7m) / 3.5GHz; V-V; -7.04; 6.71; 1.68. H-H; -6.82; 6.40; 0.86. V-H; -16.95; 6.24; 0.66. H-V; -16.92; 6.34; 1.49. Proposal 4：For modeling of UAV, support B1 as a function of the zenith angle only. No need to consider dependency on the azimuth. |
| LG | Proposal 5: Calculate the monostatic RCS of UAV Option 1 as the multiplication of the component A = 0.3 m2, the component B1 dependent on the incident direction (FFS) and the component B2 generated as the lognormal random variable with parameters (-0.844, 1.299). Proposal 6: Calculate the monostatic RCS of UAV Option 2 as the multiplication of the component A = 0.125 m2, the component B1=1 and the component B2 generated as the lognormal random variable with parameters (-0.247, 0.703). Proposal 7: Calculate the bistatic RCS of both UAV Option 1 and UAV Option 2 by multiplying the corresponding monostatic RCS by the component C2 dependent on the bistatic angle as set by equation Error: Reference source not found. That is, for UAV Option 1 B1= C1(θI, φI)*cos(β/2) and for UAV Option 2 B1= cos(β/2) where β is a bistatic angle. |
| CATT | Table 5 RCS reference value for monostatic Target type; A; B1; B2. UAV; small; -12.4dBsm; 1dBsm; Log-normal distribution with (μ=0 dBsm, σ2=3.1^2 dBsm2). large; 0.9dBsm; Look up table based on experiment data; Log-normal distribution with (0, 6.3^2). human; 3.6dBsm; 1dBsm; Log-normal distribution with (0, 2.4^2). vehicle; 8.3dBsm; Look up table based on experiment data; Normal distribution with (0, 5.7^2). - For human, RCS modeling Option 3 is preferred, where A is a mean RCS value, B1 is set to 1, B2 is modelled using log-normal distribution for monostatic. - For vehicle, RCS modeling Option 3 is preferred, where A is a mean RCS value, B1 is related to angles and obtained by looking up a table, B2 is modelled by normal distribution for monostatic. |

- Issue 1: General definition of A/B1/B2

[Moderator’s note] Base on all the inputs (hope I didn’t miss a result), the reported RCS values for UAV for monostatic is summarized in the following table.

|  | polarization | frequency (GHz) | Measurement results |  | A (dBsm) | B1 (dB) | B2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | mean (dBsm) | standard deviation (dBsm) |  |  | mean (dB) | standard deviation (dB) |
| Huawei |  |  |  |  | -20 | 0 |  | 4 |
| ZTE |  | 4.9 |  |  | -12.74 | 0 | -1.45 | 3.6 |
| QC |  |  |  |  | -6.5 | 0 |  | 6.2 |
| LG |  | 8-10 |  |  | -9 | 0 | -0.247 | 0.703 |
| CATT |  |  |  |  | -12.4 | 0 | 0 | 3.1 |
| Huawei |  |  |  |  | 0 | Antenna radiation pattern |  |  |
| BUPT |  |  |  |  | -8.3527 | a piecewise function | 0 | 2.36 |
| NIST | VV | 28 |  |  | -23.76 | second order polynomial | -0.47 | 1.068 |
| LG |  | 8-10 |  |  | -5.2 | FFS | -0.844 | 1.299 |
| CATT |  |  |  |  | 0.9 | Lookup table | 0 | 6.3 |
| Samsung |  |  |  |  |  |  |  |  |
| Intel | VV | 10 | -18.58 |  |  |  |  |  |
| Tejas | VV, HH | 9 | -10 |  |  |  |  |  |

[Moderator’s note] A general observation is that different companies may have different understanding on A, B1, B2, which results quite diverse proposed values. It is then critical to align our definitions on each parameter. The early agreement RCS=A*B1*B2 assuming A, B1, B2 are all linear values. On the other hand, dB values are commonly used in the definition of many other parameters in TR 38.901. A general observation is

- A is the mean of the linear RCS values

- B1 is deterministic. The mean of linear B1 values equals to 1 (B1 fixed to 1 as special case)

- B2 follows log-normal distribution, which is charactered by a Gaussian distribution (μB2, σB22). Note: there is a fixed relation that μB2=-ln1020σB22

It seems clear that we should use dB value (μB2, σB22) to characterize B2 in the TR, to align with many other parameters in 38.901. However, how to capture A, B1 in the TR is still open.

- Note: no matter how to capture A, A must be the mean of the linear RCS values.

- Note: no matter how to capture B1, the mean of linear B1 value must equal to 1.

To accelerate the discussion, I directly make a possible way forward. Please check if you have concern on such way.

[H][FL1] Proposal 4.2-1

To capture the component A, B1, B2 of the RCS model for UAV into the TR

- The component A is the mean of the linear RCS values and is expressed in dB scale

- The component B1 is expressed in dB scale.

  - Note: the mean of linear B1 value equals to 1.

- The distribution of component B2 is charactered by a Gaussian distribution (μB2, σB22)

  - Note: there is a fixed relation that μB2=-ln1020σB22

FFS the above definition is extended to other type of targets

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | For B2, it is better to say B2 is log-normal distribution (μB2, σB22) to make the mean of linear B2 value equals to 1. |
| Huawei, HiSilicon | No | Is it proposal intended to talk about how to capture A, B1, B2? If that is the case, it is premature to make the decision before seeing more progress of RCS for other sensing targets. The first note implies this proposal for the small size UAV. How about the large size? |
| Intel |  | It is preferred to clarify the A, B1, B2 for all use cases first and then make necessary adjustments for particular types of targets. Current proposal focuses on UAV but the bullets on at least A and B1 are applicable in general in our understanding. So, it is suggested to split the proposal on common part and UAV-specific part instead of introducing the last FFS bullet. |
| NIST | Yes | For B2, we may want to make it clear that the mean and standard derivation are in dB or the fitting parameters from the nature log. |
| LGE |  | We need an additional restriction on B2. Since A is the mean RCS in linear scale, the means of B2 should also equal to 1 in linear scale, same as B1. Given B2 of log-normal distribution (μB2, σB22), the mean of B2 is eμB2+12σB22, which should be 1. - The distribution of component B2 is charactered by a Gaussian distribution (μB2, σB22) - Note: there is a fixed relation that μB2=-ln1020σB22 - Note: the mean of linear B2 value equals to 1. eμB2+12σB22=1 |
| Nokia, NSB | Yes with changes | Component A: agreeable Component B1: okay Component B2: Better wording would be “B2 is a log-normal distribution, or log(B2) is a normal distribution with log(B2)~N(0, σ2” |
| Lenovo | Yes | Perhaps the square meter unit can be captured as part of A |
| Moderator | @ZTE, Nokia: In fact, the wording for B2 is referring to 38.901. “The long-term (log-normal) fading in the logarithmic scale around the mean path loss PL (dB) is characterized by a Gaussian distribution with zero mean and standard deviation.” Direct using log-normal distribution (μB2, σB22) should be OK. However, not sure someone may think μB2, σB22 are statistics of linear RCS value. I add log-normal in the bullet (didn’t write it before since it is agreed in last meeting) @Huawei: Since we already make agreement on A/B1/B2 for UAV, and there seems some misalignment on log-normal distribution (actually, this is a pure mathematics problem). It would be helpful/urgent for clarification. Otherwise, we cannot move forward for UAV and other targets. @Intel: Fine to make it general. One FFS is added for the first sub-bullet since we don’t have agreement whether A is 1 m2 or mean RCS value for other targets other than UAV @NIST: Many other parameters in 38.901 is in dB scale, so the proposal @LG: The existing note μB2=-ln1020σB22 actually indicates that the mean of linear B2 value is 1. I’m fine to add more clarification if it is helpful, but the mean of linear B2 values should be expln1010u+ln210200σ2=1 assuming u,σ2 are for dB RCS values |  |

[H][FL1] Proposal 4.2-1-rev1

To capture the component A, B1, B2 of the RCS model for a scattering point of a target UAV into the TR

- The component A is the mean of the linear RCS values and is expressed in dB scale

  - FFS targets other than UAV

- The component B1 is expressed in dB scale.

  - Note: the mean of linear B1 value equals to 1.

- The log-normal distribution of component B2 is charactered by a Gaussian distribution (μB2, σB22)

  - Note: there is a fixed relation that μB2=-ln1020σB22

  - Note: the mean of linear B2 values is expln1010μB2+ln210200σB22=1

FFS the above definition is extended to other type of targets

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| CATT/CICTCI | Yes |  |
| New H3C | Yes |  |
| Qualcomm | Yes |  |
| vivo |  | We are fine with the proposal but feel that the note is premature and should be removed. |
| CMCC |  | - It’s necessary to clarify the extraction steps to component A, B1, and B2. For example, if we extract the A at the first step, B1 component will shrink and its mean values may not be 1. If we extract the B1 at the first step, A is not the mean of the linear RCS values. - It’s really hard for small and large UAVs to satisfy the requirements that “the mean of linear B1 value equals to 1” , “B2 is charactered by a Gaussian distribution (μB2, σB22)”, “μB2=-ln1020σB22”, simultaneously. - The note μB2=-ln1020σB22 is not necessary. |
| OPPO | N (need clarification first) | We can see the motivation behind the proposal, which intends to formulate the mean of A*B1*B2 (all three parameters are in linear scale) as: Mean(A*B1*B2) = A * Mean(B1) * Mean(B2) with Mean(B1)=1 and Mean(B2)=1. But the question is what sample space is defined to apply the “Mean()” function. In our view, there can be two cases: - Case A: The purpose of RCS modeling based on {A,B1,B2} is to model the universal RCS characteristic of a target without considering to put the target in a Tx-Target-Rx geometry. In this case, the sample space to derive Mean(B1) should take into account the angle-dependent RCS values upon all incident angles e.g., within [0,360], and Mean(B2) should count all statistic random sample space of B2. In this case, the RCS modeling is independent from any small-scale or large-scale components, because it does not bind to any Tx and Rx. - Case B: The purpose of RCS modeling based on {A,B1,B2} is to model the specific RCS effectiveness of a target when the target takes a specific position and holds the specific incident/scattering angles with Tx/Rx. Then the Mean() function, if needed, should only count the sample paths associated with the Tx/Rx, rather than the universal ones. It was agreed in RAN1 #118 that {A, B1, B2} are the terms used in large-scale and small-scale components of target channel, which seems to suggest Case B. Anyhow, for Case A, it seems conflict to the earlier RAN1 agreement that A is a component in large-scale and {B1, B2} are components in small-scale, and the requirements of Mean(B1)=1 and Mean(B2)=1 are not so critical; for Case B, it does not seem to match the direction of the other proposals such as Proposal 4.2-2. Therefore, the current RAN1 discussion steps look confusing in exploring the task given by earlier RAN1 agreement, which is somehow the Case B, by applying the statistics logic which matches Case A. It should be clarified whether the intention of the proposal is for Case A or Case B, because the Mean(B1)=1 in case A does not necessarily mean Mean(B1)=1 in Case B, and vice versa. Same applies to Mean(B2). It is better to clarify first what the group intends to do with the two cases. |
| SONY | Yes |  |
| Spreadtrum | Yes |  |
| Xiaomi | Yes |  |
| Ericsson | No | This proposal applies to mono-static angle-independent RCS. mono-static can be added in the main sentence. In fact there is no need to define A and B1 separately. The only purpose of doing so is to use B1 in path generation and A in the large-scaling scaling factor. However, if the product of A, B1, and B2 is used in the step of path generation, we can avoid the complicate normalization problems for B1, B2, CPM. See our comments to [H][FL1] Proposal 5.3-2. For angle-independent RCS, mean RCS should be calculated over the expected angular range of the direct/indirect paths in the target channels, and mean RCS cannot be predetermined without a simulation. If A and B1 have to be defined, we prefer A=0dBsm. Note that in 38.901 the distribution of the shadow fading is log-normal and only its standard deviation for each scenario is given in Table 7.4.1-1, which means the mean and median values are 0. The same applies to B2, where μB2=0 means the median value of B2 = exp(μB2)=1. Our suggestion is in blue as follows. To capture the component A, B1, B2 of the mono-static RCS model for a scattering point of a target UAV into the TR - The component A is the mean of the linear RCS values and is expressed in dB scale - FFS targets other than UAV - The component B1 is expressed in dB scale. - Note: the mean of linear B1 value equals to 1. - No need to define A and B1 separately - The log-normal distribution of component B2 is charactered by a Gaussian distribution (μB2, σB22), μB2=0 - Note: there is a fixed relation that μB2=-ln1020σB22 - Note: the mean of linear B2 values is expln1010μB2+ln102200σB22=1 FFS the above definition is extended to other type of targets |
| Toyota | Yes |  |
| BUPT |  | We agree with the proposal that A is the mean of the linear RCS values, the mean of linear B1 equals to 1, and B2 follows a normal distribution in the dB domain with μB2=-ln1020σB22 We would like to further clarify whether B1 should be fitted in the linear domain or in the dB domain. |
| Huawei, HiSilicon |  | We are ok with E//’s version. |

[Moderator’s note] version after Tuesday online session.

A further revision is I limit it to only small size UAV, let’s see if we could clarify it which is at least useful for collection RCS results in later meetings

[H][FL2] Proposal 4.2-1-rev2

For the components A, B1, B2 of the RCS model for UAV of small size (option 2 for UAV size in UAV parameters table) for monostatic a scattering point of a target for monostatic, at least for a small size UAV:

- The component A is the mean of the linear RCS values in all incident/scattered angles and is expressed in dBsm scale

- The component B1 is 0dB expressed in dB scale

- The component B2 is expressed in dB scale

- The log-normal distribution of component B2 is charactered by a Gaussian distribution (μB2, σB22)

  - Note: there is a fixed relation that μB2=-ln1020σB22

  - Note: the mean of linear B2 values is expln1010μB2+ln210200σB22=1

[Moderator’s note] further revised taking into account if A=1m2

- [H][FL3] Proposal 4.2-1-rev3

To capture the component A, B1, B2 of the RCS model with B1 fixed to 1 for a scattering point of a target into the TR

- The component A is the mean of the linear RCS values in all incident/scattered angles and is expressed in dBsm scale

- The component B1 is 0 dB.

- The log-normal distribution of component B2 is charactered by a Gaussian distribution (μB2, σB22)

  - Note: there is a fixed relation that μB2=-ln1020σB22

  - Note: the mean of linear B2 values is expln1010μB2+ln210200σB22=1

To capture the component A, B1, B2 of the RCS model with angular dependent B1 for a scattering point of a target into the TR

- The component A is 1 m2, i.e., 0 dBsm

- The component B1 is expressed in dB scale.

- The log-normal distribution of component B2 is charactered by a Gaussian distribution (μB2, σB22) with μB2=0dB

| Company | Yes/No | Comments |
| --- | --- | --- |

- Issue 2: Exact RCS values of UAV

[Moderator’s note] Hopefully Proposal 4.2-1 is agreeable. Please help to revise the reported A, B1, B2 values if a different definition is previously used. Even when there is no change on values, please also add your results in the following tables

- [H][FL1] Proposal 4.2-2

Please help to update reported values of A, B1, B2 in the following tables for UAV. Please avoid add large formula for B1 in Table X2. Please add details on B1 and any comments if necessary in the 3rd table.

Table X1. Values for UAV with same size

| Company | polarization | frequency (GHz) | A (dBsm) | B1 (dB) | B2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | mean (dB) | standard deviation (dB) |
| ZTE | HH | 4.9 | -12.74 | 0 | -1.45 | 3.6 |
| BUPT | VV | 4.9 | -25.2503 | 0 | 0 | 3.1301 |
| BUPT | VV | 15 | -23.3343 | 0 | 0 | 3.2519 |
| BUPT | VV | 28 | -19.5213 | 0 | 0 | 3.9184 |
|  |  |  |  | 0 |  |  |

Table X2. Values for UAV with large size

| Company | polarization | frequency (GHz) | A (dBsm) | B1 (dB) | B2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | mean (dB) | standard deviation (dB) |
| NIST | VV | 28 | -23.76 | 2nd order polynomial | -2.04 | 4.64 |
| BUPT | VV | 10 | -8.3527 | a piecewise function | 0 | 2.3604 |
| BUPT | VV | 15 | -10.1426 | a piecewise function | 0 | 2.4213 |
| BUPT | VV | 20 | -6.0206 | a piecewise function | 0 | 2.1351 |
| BUPT | VV | 28 | -4.8573 | a piecewise function | 0 | 2.3252 |
| BUPT | VV | 36 | -4.5176 | a piecewise function | 0 | 3.443 |
| BUPT | VH | 15 | -19.7811 | a piecewise function | 0 | 2.4617 |
| BUPT | HV | 15 | -19.5583 | a piecewise function | 0 | 2.3249 |
| BUPT | HH | 15 | -10.036 | a piecewise function | 0 | 2.3918 |

| Company | Comments |
| --- | --- |
| NIST | We reported B2 as Lognormal(µ, σ2) in our contribution, and its parameters were based on natural log. Here, we converted those values to dB for Table X2 |
| LGE | Suggest to discuss this proposal after completing/agreeing Proposal 4.2-1. We need to confirm that the mean of B2 in linear scale equal to 1, as we commend in Proposal 4.2-1. |
| Nokia, NSB | This is a good proposal. However, it should be discussed after agreements on A/B1/B2 definitions. |
| Spreadtrum | Similar view as LGE and Nokia. |
| BUPT | Large UAVs exhibit scattering characteristics in the vertical dimension that are similar to those in the horizontal dimension. In the horizontal dimension (θ=π2, φ), the B1 component for UAV can be represented as: logB1θ=π2, φ=a1(φ-φ1)2+c1 , φ∈[35π18,2π)∪[0,π18]a2(φ-φ2)2+c2, φ∈[4π9,5π9]a3(φ-φ3)2+c3, φ∈17π18,19π18a4(φ-φ4)2+c4, φ∈[13π9,14π9]μ, φϵothers φ; logA; logB1θ=π2, φ. ak; φk; ck. [35π18,2π)∪[0,π18]; -8.3527; -140.96; 0; 3.46. [4π9,5π9]; -1120.75; π2; 13.01. 17π18,19π18; -656.56; π; 11.0. [13π9,14π9]; -793.78; 3π2; 14.78. Others; -1.3249. the B1 component in the horizontal dimension can be represented as: logB1θ=π2, φ=a1(φ-φ1)2+c1 , φ∈[35π18,2π)∪[0,π18]a2(φ-φ2)2+c2, φ∈[4π9,5π9]a3(φ-φ3)2+c3, φ∈17π18,19π18a4(φ-φ4)2+c4, φ∈[13π9,14π9]μ, φϵothers wherein， φ; logA; logB1θ=π2, φ; logB2. ak; φk; ck. [35π18,2π)∪[0,π18]; -8.3527; -140.96; 0; 3.46; N(0,2.362). [4π9,5π9]; -1120.75; π2; 13.01. 17π18,19π18; -656.56; π; 11.0. [13π9,14π9]; -793.78; 3π2; 14.78. Others; -1.3249. The B1 modelling approach we have presented can also be transformed into the form of an antenna radiation pattern. Proposal 7: The RCS of humans, vehicles, and UAVs is positively correlated with frequency overall, and the relationship can be expressed as: Ahuman = 0.1787*frequence-6.171 Avehicle =0.1889*frequence + 1.63 AUAV = 0.2083*frequence-11.49 |

- [H][FL1] Proposal 4.2-2A -hold on

- For UAV of small size (option 2 for UAV size in UAV parameters table) with single scattering point for monostatic

  - A in dB scale equals to

  - B1=0dB

  - B2 is characterized by a Gaussian distribution with mean μ and standard deviation σ

    - μ

    - σ

- [M][FL1] Proposal 4.2-2B -hold on

- For UAV of large size (option 1 for UAV size in UAV parameters table) with single scattering point for monostatic

  - A in dB scale equals to

  - B1

  - B2 is characterized by a Gaussian distribution with mean μ and standard deviation σ

    - μ

    - σ

RCS values for human

Summary on company views

| Company | Results on RCS |
| --- | --- |
| Apple | Table 6: Standing Human (Frequency Dependent) Frequency; Mean = A; Std = B2. 6; -2.1921; 4.7104. 12; -2.39304; 3.6671. 24; -2.29847; 3.95312. 36; -1.8057; 3.88219. Table 7: Walking Human (Frequency Dependent) Frequency; Mean = A; Std = B2. 6; -2.14215; 3.88197. 12; -2.24825; 4.0528. 24; -2.00197; 3.94813. 36; -2.0289; 4.3289. Table 8: Frequency Independent Parameters Mode (all frequencies); Mean = A; Std = B2. Standing; -2.17604; 4.07384. Walking; -2.10521; 4.05329. |
| ZTE | Proposal 4: RCS Option 3 is selected to model RCS of pedestrian with single scattering point - For Adult pedestrian (approximately 0.5 meters in length, 0.5 meters in width, and 1.75 meters in height) at 4.9GHz - B1=1 - A is mean RCS value of 0.4m2 - B2 of pedestrian is modelled using log-normal distribution. Draw B2 values as B2=10X10, Where, X~Nμ,σ is Gaussian distributed with σ=4.13and μ=-ln1020σ2=-1.96. |
| Ericsson | Proposal 17 Mono-static RCS of a human is elevation angle dependent. Proposal 18 Monostatic RCS of a human is independent of azimuth angle. |
| vivo | Table 9: The RCS mathematical model with normal distribution for human at various operation frequencies. Frequency; Mean; Standard deviation. 4.9GHz; -5.6785; 2.2575. 10GHz; -4.4679; 2.7437. 20GHz; -3.9933; 2.4178. 28GHz; -1.7976; 2.6104. 36GHz; -0.1655; 2.1442. Consequently, the RCS for human can be modelled as the log-normal distribution with the mean value corresponding to the component A, mathematically formulated as Ahuman=0.0029f2+0.0538f-5.7748 Eq. 1 The standard deviation corresponding to the component B2 can be defined by the average value of measured results, where B2human=2.44 Eq. 2 |
| CATT | Table 10 RCS reference value for monostatic Target type; A; B1; B2. UAV; small; -12.4dBsm; 1dBsm; Log-normal distribution with (μ=0 dBsm, σ2=3.1^2 dBsm2). large; 0.9dBsm; Look up table based on experiment data; Log-normal distribution with (0, 6.3^2). human; 3.6dBsm; 1dBsm; Log-normal distribution with (0, 2.4^2). vehicle; 8.3dBsm; Look up table based on experiment data; Normal distribution with (0, 5.7^2). |
| AT&T | For monostatic sensing, we propose the following RCS modeling for a human target. Proposal 2: For monostatic sensing, to model the RCS of an adult human target with single scattering point - A is mean RCS value given by -6.2 dBsm - B is modelled using a log-normal distribution with mean 1 and variance 10.0 For bistatic sensing, we propose the following RCS modeling for a human target. Proposal 3: For bistatic sensing, to model the RCS of a adult human target with single scattering point - A is mean RCS value given -14.4 dBsm - B is modelled using a log-normal distribution with mean 1 and variance 6.7 |
| Xiaomi | Fig. 1 Human mean RCS with four polarizations. Table 11 Human RCS parameters for specific bistatic angle. Angle; RCS component A σRCS,co; Value σRCS,XσRCS,co-1; Variance of RCS component B2. 0°; 3.93; 1192; 0.80. 10°; 3.84; 1040; 0.79. 20°; 3.61; 737.89; 0.83. 30°; 3.41; 473.71; 0.85. 40°; 3.15; 265.52; 0.89. 50°; 2.92; 146.83; 0.92. 60°; 3.02; 89.63; 0.98. 70°; 3.57; 53.39; 0.81. 80°; 3.52; 31.31; 0.87. 90°; 3.23; 18.65; 0.85. 100°; 4.53; 15.81; 0.74. 110°; 5.24; 13.03; 0.93. 120°; 2.05; 6.39; 1.35. 130°; 2.83; 7.41; 1.37. 140°; 2.44; 5.92; 0.53. 150°; 1.85; 5.91; 0.38. 160°; 0.91; 8.65; 2.20. 170°; 0.24; 26.81; 3.71. |
| BUPT | Proposal 8: For human modelling with single scattering point, Human RCS can also be calculated by RCS=A×B1×B2. Because RCS exhibits isotropic scattering characteristics, the B1 component is set to 1, and the B2 component is set to the variance σ, representing the fluctuation of RCS. - The RCS parameters for human in terms of the frequency, the mean and standard deviation listed in Table 12 can be a reference. Table 12 The RCS mathematical model with normal distribution for human in different measurement configurations. Frequency; Distance; Sensing Mode; Parameters of Normal distribution; Value. 20 GHz; 12 m; Mono-static; μ; -3.9933. σ; 2.4178. 28 GHz; 12 m; Mono-static; μ; -1.7976. σ; 2.6103. 36 GHz; 12 m; Mono-static; μ; -0.1655. Proposal 9: The RCS of humans, vehicles, and UAVs is positively correlated with frequency overall, and the relationship can be expressed as: RCShuman=0.1787*frequence-6.171 RCSvehicle=0.2083*frequence-11.49 RCSUAV=0.1889*frequence+1.63 |
| OPPO | Proposal 2: The human body RCS in mono-static mode is modeled as RCS=fθ+x in unit of dBsm, where - fθ=aθ-θc2+cforHH∧VVbforHV∧VH - θc is the angle direction perpendicular to human body’s front/back sides. - x is a random value in Gaussian distribution of Nμ=0,σ. - The parameters a,b,c and σ may depend on human body size, pose and attire. It is sufficient for ISAC channel model TR to capture a limited number of combinations of these parameters for information only. For HH and VV cases, a=-16π2, c=2, θc=π2,∧0≤θ<3π2,∧π≤θ<2π; For VH and HV cases, a=0, c=-24 |
| NIST | Proposal 5: For monostatic sensing, the RCS of a human target can be modelled asσtg=A×B1×B2, where A, B1, and B2 are defined as follows: Table 2.2.1-1: Monostatic human target parameters for single scatter RCS model - Subject; - A (m2); - B1; - B2, µ; - B2, σ. - A; - 0.15; - 1; - -0.38; - 1.03. - B; - 0.21; - 1; - -0.59; - 1.36. - C; - 0.13; - 1; - -0.32; - 0.99. - All; - 0.16; - 1; - -0.46; - 1.14. Proposal 6: For bi-static sensing, the RCS of a human target can be modelled asσtg=A×B1×B2, where A, B1, and B2 are defined as follows: Table 2.2.2-1: Bistatic human target parameters for single scatter RCS model - Subject; - A (m2); - B1; - B2, µ; - B2, σ. - A; - 0.05; - 1; - -0.62; - 1.36. - B; - 0.01; - 1; - -0.63; - 1.27. - C; - 0.04; - 1; - -0.61; - 1.27. - All; - 0.03; - 1; - -0.83; - 1.49. |
| QC | Proposal 10: With regards to RCS modelling for a pedestrian, for each scattering point, for each polarization, use RCS modelling Option 2: - A is the mean with dBsm=-100for adult and [-5 to -15] for child. - With change in carrier frequency, A doesn’t change much, whereas variance B can vary significantly. Value of B is FFS. - Values for statistical distribution parameters (A and B) to model a pedestrian can also vary based on the shape of pedestrian (e.g., adult or child), viewing angle (esp. elevation), and resolution of sensing device (such as, dictated by bandwidth and antenna dimensions). - Study whether to model B with a Weibull distribution, or a log-normal distribution. - FFS: Those with strong angle dependence can be approximated by Weibull distribution, and those with weak angular dependence with log-normal distribution. |
| LG | Proposal 10: Calculate the monostatic RCS of human as the multiplication of the component A = 0.5 m2 (Adult pedestrian) or A = 0.25 m2 (Child), the component B1=1 and the component B2 generated as the chi-squared random variable with four degrees of freedom and the mean value equal to 1 (the standard deviation equal to 0.707). Proposal 11: Calculate the bistatic RCS of human by multiplying the corresponding monostatic RCS by the component C2 dependent on the bistatic angle as set by equation Error: Reference source not found. That is, B1 = cos(β/2) where β is a bistatic angle. |

- Issue 1: General model

Based on companies’ inputs and RCS model Option 3, the following proposal is made for human. One question from moderator is whether it is OK to delete Model 2 for human?

[H][FL1] Proposal 4.2-3

[Moderator’s note] The following RCS models are supported for human with single scattering point for monostatic

- A is mean RCS value

  - Different mean RCS values can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1=1

- FFS Model 2

  - B1 have dependency on incident/scattered angles

- B2 is modelled using log-normal distribution for monostatic

- Note: whether exact values of A/B1/B2 for Model 1 and/or model 2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon | No | In general, if the RCS is angular dependent, modelling as multiple scattering points should be firstly considered, and A should not be the mean value. The note is unclear about the intension. |
| NIST | Yes |  |
| LGE | Yes | Support with the following clarification. - B2 is modelled using log-normal distribution for monostatic - Note: the mean of linear B2 value equals to 1. |
| Samsung |  | Support if FFS in Model 2 for B1 is maintained. We need time to check whether there is elevation angle dependency |
| Nokia, NSB |  | Bullets on Component A and B2 are agreeable. For B1, the dependency on the incident/scattered angles should be included. |
| EURECOM | Yes |  |
| Ericsson |  | Mono-static RCS of human is elevation-angle dependent and azimuth-angle independent. Consider a human moving closer to or further away from a BS. The change of elevation angles results in a time-varying RCS, which can be useful when analyzing human detection. Therefore, we suggest A=1sq.m and B1 is angle dependent. |
| CATT/CICTCI | Yes | Model 1 |
| New H3C | Yes | Model 1 |
| Qualcomm | No | We believe that RCS of a human needs to have dependency from the incident/scattered angles (specifically the elevation angle dependency), and we prefer not to have 2 different models with respect to this aspect. |
| vivo | Yes |  |
| CMCC | Yes | We prefer model 1. |
| Spreadtrum | Yes | Support the proposal in principle. We also think the note is not clear, is it possible not to capture the values of A/B1/B2 if agreed? |
| Xiaomi | Yes | Model 1 |
| Lenovo |  | Are we assuming a human may always be modeled via a single scattering point? Suggest to discuss this first (i.e., where/if a modeling option via single scattering point is valid, and then define the modeling parameters) Model 2 is preferred. At least dependency to the elevation angle needs to be captured |
| AT&T |  | We agree with the bullet on A and B2. For B1, for azimuth angle at least, we did not see any angle dependency in our measurements |
| BUPT | Yes | We prefer model 1. |
| Apple | Yes | For the mono-static case, the RCS is on average angel independent as shown below: |

[Moderator’s note] After Monday offline session,

[H][FL2] Proposal 4.2-3-rev1

The following RCS models are supported for human with single scattering point for monostatic

- A is mean RCS value

  - Different mean RCS values can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1=1

- FFS Model 2

  - B1 have dependency on incident/scattered angles

- B2 is modelled using log-normal distribution for monostatic

- Note: whether exact values of A/B1/B2 for Model 1 and/or model 2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model.

[Moderator’s note] further revised

[H][FL3] Proposal 4.2-3-rev2

The following RCS models are supported for human with single scattering point for monostatic

- Model 1

  - B1=0 dB

  - A is mean RCS value

  - B2 is modelled using log-normal distribution

  - Different mean RCS values can be supported for human due to different size, shape, frequency, etc.

- Model 2

  - B1 have dependency on incident/scattered angles

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a piecewise function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - Different mean RCS values can be supported for human due to different size, shape, frequency, etc.

  - FFS RCS component A

[Moderator’s note] agreement in Wednesday online session

- Agreement

The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1=0 dB

  - A is mean RCS value

  - B2 is modelled using log-normal distribution

- Model 2

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

- FFS: conditions for using which model

- Issue 2: Exact RCS values of human

[Moderator’s note] Hopefully Proposal 4.2-1 is agreeable. Please help to revise the reported A, B1, B2 values if a different definition is previously used. Even when there is no change on values, please also add your results in the following tables

- [H][FL1] Proposal 4.2-4

Please help to update reported values of A, B1, B2 in the following tables for human. Please avoid add large formula for B1 in Table Y2. Please add details on B1 and any comments if necessary in the 3rd table.

Table Y1. Values for human if B1 fixed 1

| Company | polarization | frequency (GHz) | A (dBsm) | B1 (dB) | B2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | mean (dB) | standard deviation (dB) |
| ZTE | HH | 4.9 | -4 | 0 | -1.96 | 4.13 |
| NIST | VV | 28 | -7.96 | 0 | -2.0 | 4.95 |
| vivo | vv or hh | 4.9GHz | -5.6785 | 0 | 0 | 2.2575 |
| vivo | vv or hh | 10GHz | -4.4679 | 0 | 0 | 2.7437 |
| vivo | vv or hh | 20GHz | -3.9933 | 0 | 0 | 2.4178 |
| vivo | vv or hh | 28GHz | -1.7976 | 0 | 0 | 2.6104 |
| vivo | vv or hh | 36GHz | -0.1655 | 0 | 0 | 2.1442 |
| BUPT | VV | 15 | -2.3721 | 0 | 0 | 1.88826 |
| Xiaomi | VV | 4.9 | 6.04 | 0 | -3.39 | 5.49 |
| Xiaomi | HH | 4.9 | 5.84 | 0 | -3.39 | 5.43 |

Table Y2. Values for human if B1 is angular dependent

| Company | polarization | frequency (GHz) | A (dBsm) | B1 (dB) | B2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | mean (dB) | standard deviation (dB) |

| Company | Comments |
| --- | --- |
| LGE | Suggest to postpone this discussion until we get the agreement on Proposal 4.2-3. |
| Nokia, NSB | Agree with LGE’s view. |
| vivo | Based on the results provided by proponents, it is better to take the average value for A, B2 (mean and sd) as a function of frequency. According to our experiment measurements, we propose that A value: Ahuman=0.0029f2+0.0538f-5.7748 B2 value: Mean = 0, Standard deviation B2human=2.44. |
| BUPT | Proposal 7: The RCS of humans, vehicles, and UAVs is positively correlated with frequency overall, and the relationship can be expressed as: Ahuman = 0.1787*frequence-6.171 Avehicle =0.1889*frequence + 1.63 AUAV = 0.2083*frequence-11.49 |

- [M][FL1] Proposal 4.2-4A -hold on

- For RCS model 1 of human with single scattering point for monostatic

  - A in dB scale equals to

  - B1=0dB

  - B2 is characterized by a Gaussian distribution with mean μ and standard deviation σ

    - μ

    - σ

RCS values for vehicle

Summary on company views

| Company | Results on RCS |
| --- | --- |
| Huawei | Table 13: The B1 (scattering power pattern of a scattering point) for vehicle for mono-static and bi-static channel Parameter; Values. Vertical cut of the scattering power pattern for each scattering point (dB); σ″dBθ″,φ″=0°=-min12θ″-θ0θ3dB,n2,σmax, where σmax=21dB and θ″∈0°, 180°. Horizontal cut of the scattering power pattern for each scattering point (dB); σ″dBθ″=90°,φ″=-min12φ″-φnφ3dB,n2,σmax, where σmax=21dB and φ″-φn∈-180°, 180°. 3D scattering power pattern(dB) for each scattering point; σ″dBθ″,φ″=σn*cosβ2-min-σ″dBθ″,φ″=0°+σ″dBθ″=90°,φ″,σmax β=scatteringangle-incidentangle. The parameters for each scattering point; Scattering points for vehicle; n; φn; φ3dB,n; θ0; θ3dB,n; σn [dB]. Front; 1; 0°; 24.4°; 90°; 28.1°; 11.42. Left; 2; 90°; 15.6°; 90°; 16°; 19.65. Back; 3; 180°; 35.7°; 90°; 45.6°; 13.09. Right; 4; -90°; 15.6°; 90°; 16°; 19.65. top; 5; /; /; 0°; 11.5°; 21.88.. Notes: θ″ represents the incident and scattering angle for each scattering point independently. φ″ represents the incident and scattering angle for each scattering point independently. The σn represents the practical maximum power of the scattering point. For bi-static link, the (θ″,φ″) represents the bisector angle of the bi-static link. At least applicable to the vehicle of type 1 and type 2 size (i.e., passenger vehicle).. |
| CATT | Table 14 RCS reference value for monostatic Target type; A; B1; B2. UAV; small; -12.4dBsm; 1dBsm; Log-normal distribution with (μ=0 dBsm, σ2=3.1^2 dBsm2). large; 0.9dBsm; Look up table based on experiment data; Log-normal distribution with (0, 6.3^2). human; 3.6dBsm; 1dBsm; Log-normal distribution with (0, 2.4^2). vehicle; 8.3dBsm; Look up table based on experiment data; Normal distribution with (0, 5.7^2). |
| ZTE | Proposal 5: For mono-static, the RCS=A*B=A*B1*B2 of a scattering point for a passenger vehicle Type 1/2 can be generated by - A has mean RCS value 11.72 m2 corresponding 10.69 dBsm - B1 is deterministic based on incident/scattered angles B1dB=rcsdBθ,φ-6.61. Where, rcsdBθ,φ=Gmax-min-AVdBθ+AHdBφ,Amax Where, AVdBθ=-min12θ-θcenterθ3dB2,Amax, AHdBφ=-min12φ-φcenterφ3dB2,Amax, Area/n; Side of vehicle; θcenter; φcenter; φ3dB; θ3dB; Amax; Gmax; Applicable range of ZoA/θ; Applicable range of AoA/φ. 1; Left; 78.8; 0°; 47.5°; 99.5°; 5.8; 9.5; [20°, 90°]; [0°, 45°]∪[315°, 360°]. 2; Back; 71.6°; 90°; 39.6°; 43.9°; 2.76; 4.62; [20°, 90°]; [45°,135°]. 3; Right; 78.8°; 180°; 47.5°; 99.5°; 5.8; 9.5; [20°, 90°]; [135°,225°]. 4; Front; 20.0°; 270°; 95.7°; 48.4°; 10.05; 9.49; [20°, 90°]; [225°,315°]. 5; Roof; 0°; -; -; 28.42; 3.63; 10.99; [0°, 20°]; [0°,360°]. - B2 is generated by log-normal distribution - Draw B2 values as B2=10X10, Where, X~Nμ,σ is Gaussian distributed with σ=3.28and μ=-1.24. Proposal 6: For Bi-static, the RCS modeling is RCS_car = rcsdBθi,φi,θs,φs + glint, where glint is Normal distribution, and rcsdBθi,φi,θs,φs=Greflection-min-AVdBθs+AHdBφs,Amax Wherein, AVdBθs=-min12θs-θreflectθiθ3dB2,Amax, AHdBφs=-min12φs-φreflectφiφ3dB2,Amax, Greflection= GmaxRCS*cos((φi-φreflect)/2)*cos((θi-θreflect)/2), Side of vehicle; ZoA/θi; AoA/φi; (θn,φn; φ3dBn; θ3dB; Amax; GmaxRCS. Left; [20°,90°]; [0°, 90°]∪ [270°, 360°]; (78.8, 0); 47.5°; 99.5°; 5.8; 9.5. Back; [20°,90°]; (0°,180°); (71.6, 90); 39.6°; 43.9°; 2.76; 4.62. Right; [20°,90°]; (90°,270°); (78.8, 180); 47.5°; 99.5°; 5.8; 9.5. Front; [20°,90°]; (180°,360°); (20, 270); 95.7°; 48.4°; 10.05; 9.49. Roof; [0°,20°]; (0°,360°); (0, -); -; 28.42; 3.63; 10.99. Note： if θn=0, then Greflection = GmaxRCS*cos(θi). |
| Intel | Table 15. Mean monostatic RCS for a car model at 10 GHz Polarization; VV; VH; HV; HH. Mean RCS, dBsm; -2.09; -31.90; -30.38; -2.08. |
| AT&T | For monostatic sensing, Proposal 5: For monostatic sensing, to model the RCS of a large vehicle with single scattering point - A is mean RCS value given by 7.7 dBsm - B is modelled using a log-normal distribution with mean 1 and variance 8.4 For bistatic sensing, Proposal 6: For bistatic sensing, to model the RCS of a adult human target with single scattering point - A is mean RCS value given -0.1 dBsm - B is modelled using a log-normal distribution with mean 1 and variance 6.1 |
| Tejas | Proposal 7: The RCS scattering model for vehicle can be approximated using the antenna radiation model stated in TR 38.901 based on the RCS measurements available. |
| BUPT | using a piecewise function to model the B1 component. logB1=a1φ-φ12+c1,φϵ-π18,π18a2φ-φ22+c2,φϵ4π9,5π9a3φ-φ32+c3,φϵ17π18,π∪a4φ-φ42+c4,φϵ-5π9,-4π9μ,φϵothers Taking 10 GHz as an example, the fitting parameters are shown in Table 16. The fitting effect is shown in Error: Reference source not found. Table 16 The fitting parameter results for the vehicle at 10 GHz φ; logA; logB1; logB2. ak; φk; ck. ∪0,π12; 5.4453; -200.77; 0; 13.1353; N0,2.132. 5π12,7π12; -18.59; π2; 6.0515. 11π12,13π12; -245.68; π; 13.6096. 17π12,19π12; -76.52; 3π2; 5.1123. Proposal 12: For large vehicles, theA component can be modeled as the mean RCS value, the B1 component can be modeled as a piecewise function with respect to the incident angle/scattering angle, with parts having larger RCS values modeled by a quadratic function, and the remaining parts modeled as the mean. The B2 component can be modeled as a normal distribution with a mean of 0. Proposal 13: The RCS of humans, vehicles, and UAVs is positively correlated with frequency overall, and the relationship can be expressed as: RCShuman=0.1787*frequence-6.171 RCSvehicle=0.2083*frequence-11.49 RCSUAV=0.1889*frequence+1.63 |
| Xiaomi | Table 17 Detailed parameters of the SU7 monostatic RCS with VV polarization. Scattering point; θ0; φ0; θ3dB; φ3dB; RCSmax [dB]; Amax [dB]; θ; φ. Left; 90°; 0°; 9°; 13°; 20.06; 10.61; [30°,90°]; [-45°,45°]. Back; 52°; 90°; 22°; 15°; 20.16; 13.44; [30°,90°]; (45°,135°). Right; 90°; 180°; 9°; 13°; 20.06; 11.45; [30°,90°]; [135°,225°]. Front; 90°; 270°; 12°; 33°; 13.03; 8.32; [30°,90°]; (225°,315°). Ceiling; 0°; ---; 38°; ---; 18.03; 10.09; [0°,30°); [0°,360°]. |
| QC | Proposal 9: With regards to RCS modelling for a vehicle, we propose the following: - Support one of the following options: - Option-1: 5-point scattering model with scattering centres at left, right, front, back, and roof seems with its distinct element pattern. - The mean (A) and variance (B) will vary based on distance. - Option-2: 5-point scattering model at near range, whereas 1-point scattering model at far range. - When to use 5-point and when to use 1-point is FFS. - RCS of each scattering point can be modelled as lognormal. - The value of the log-normal distribution parameters will depend on carrier frequency, incident/reflected angle, sensing Bandwidth & antenna dimensions, and distance. - Mean RCS, A, is [10 to 20] dBsm. - Value of B is FFS. - Mean RCS can be modelled linearly with carrier frequency in logarithmic scale. The proportionality factor can depend on the shape and type of vehicle. |
| LG | Proposal 14: Calculate the monostatic RCS of vehicle Type 1/2 as the multiplication of the component A = 1.6 m2, the component B1 as a function of angular dependent K-factor as set by Error: Reference source not found and the component B2 generated as the non-central chi-squared random variable with two degrees of freedom. The equality of its mean value to one should be provided for the given value of K-factor. |

- [Moderator’s note]

Based on companies’ inputs and RCS model Option 3, the following proposal is made for vehicle with single scattering point. One question from moderator is whether it is OK to delete Model 1 for vehicle?

[H][FL1] Proposal 4.2-5

The following RCS models are supported for vehicle with single scattering point for monostatic

- A is mean RCS value

- B2 is modelled using log-normal distribution for monostatic

- Different mean RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- FFS Model 1

  - B1=1

- Model 2

  - B1 have dependency on incident/scattered angles

  - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

  - Alt 2: a piecewise function

  - Alt 3: Lookup table

- Note: whether exact values of A/B1/B2 for Model 1 and/or model 2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We are open to support multiple options for B1 in the TR. |
| Huawei, HiSilicon | No | We don’t see any views that the RCS of vehicle is angular independent. As long as the RCS is angular dependent, it should be modelled with multiple scattering points because it is what is observed from the detected paths with strong power at specific angles. Hence, B1 is important, A should be 1m or A is useless and not needed at all. Intention of the note should be clarified. |
| Intel |  | Agree with ZTE that multiple RCS models could be captured in the TR. In future when the ISAC channel model is used in a technology study, the appropriate model could be chosen to represent the target vehicles of interest. Further, B1 dependency on angles could be formulated regardless of the vehicle scenario. RAN1 can reuse these or other alternatives for other targets including AGV, large size UAV. |
| LGE |  | We’re fine with deleting Model 1, and defining B1 either as angular dependent or multiple scattering point. In any case, we need the following clarifications. - B2 is modelled using log-normal distribution for monostatic - Note: the mean of linear B2 value equals to 1. - Model 2 - B1 have dependency on incident/scattered angles Note: the mean of linear B1 value equals to 1. |
| Samsung |  | Support the FL’s proposal We are okay to delete Model 1 |
| Nokia, NSB |  | We are okay with Model 2 for B1. These alternatives of B1 can be further discussed. |
| Moderator | @all: I realize both antenna radiation pattern and piecewise function proposed by BUPT are using second order polynomial. NIST has a proposal on second order polynomial for B1 of UAV too. Since antenna radiation pattern is something in current 38.901 and it is more detailed including setting some upper/lower bound for the RCS values, can we agree on using antenna radiation pattern for B1? I made such revision in the current proposal |  |
| EURECOM |  | Support Model 2 |
| CATT/CICTCI | Yes | Model 2 |
| New H3C | Yes | Model 2 |
| Qualcomm | comment | As it has been pointed out in our paper, according to [10], which includes also measurements/RCS-evaluation efforts from other papers, it is pointed out that the RCS of a car varies with distance and we would like to make that clear in the following sentence: - Different mean RCS values can be supported for vehicle due to different distance from the Tx/Rx, size, shape, frequency, etc. [10] Kärnfelt, Camilla, Alain Péden, Ali Bazzi, Ghayath El Haj Shhadé, Mohamad Abbas, and Thierry Chonavel. "77 GHz ACC radar simulation platform." In 2009 9th International Conference on Intelligent Transport Systems Telecommunications,(ITST), pp. 209-214. IEEE, 2009. We are OK with model 2, but prefer to leave the sub-options for further study. |
| vivo | Yes |  |
| Xiaomi | Yes | Support antenna radiation pattern for B1. |
| Ericsson | No | Vehicles have strong directivity of RCS. They are more likely to go through particular angle ranges during the period of simulation, rather than 360 azimuth and 180 zenith. Therefore, the mean RCS (a mean over the whole sphere) is not a good representation of the RCS that the channel will experience. We suggest A=1sq.m and B1 is angle dependent, if they have to be defined separately. Alt3 can be kept at least until when other alternatives can be proven accurate. Alt 1 needs to provide at least 4 (maybe 6) local peaks, corresponding to the four sides (and maybe top and bottom) of the vehicle. So it cannot reuse 38.901 antenna pattern function with only one peak direction. But we can sum together 4-6 such functions. When companies report RCS measurements, the orientation of the vehicle in the local coordinate system should be reported as well. |
| Lenovo |  | It is not clear in our view that a vehicle can always be modeled via a single scattering point (before defining the details of single scattering points, lets agree on when/if is can be applied) |
| Toyota | Yes | We support Model 2. We are OK to delete Model 1. |
| BUPT | Yes | We agree to use antenna radiation pattern for B1 and delete Model 1. Additionally, it should be noted that we provide the expression for B1 component of vehicles in both horizontal and vertical dimensions here. In fact, the B1 modeling approach we have provided can also be transformed into the form of antenna radiation pattern. In the horizontal dimension (θ=π2, φ), the B1 component for a vehicle can be represented as: logB1θ=π2, φ=a1(φ-φ1)2+c1 , φ∈[23π12,2π)∪[0,π12]a2(φ-φ2)2+c2, φ∈[5π12,7π12]a3(φ-φ3)2+c3, φ∈[11π12,13π12]a4(φ-φ4)2+c4, φ∈[17π12,19π12]μ, φϵothers φ; logA; logB1θ=π2, φ; logB2. ak; φk; ck. [23π12,2π)∪[0,π12]; 5.4453; -200.77; 0; 13.1353; N(0,2.132). [5π12,7π12]; -18.59; π2; 6.0515. 11π12,13π12; -245.68; π; 13.6096. [17π12,19π12]; -76.52; 3π2; 5.1123. Others; -2.7564. the B1 component in the horizontal dimension can be represented as: logB1θ=π2, φ=a1(φ-φ1)2+c1 , φ∈[-23π12,2π)∪[0,π12]a2(φ-φ2)2+c2, φ∈[5π12,7π12]a3(φ-φ3)2+c3, φ∈[11π12,13π12]a4(φ-φ4)2+c4, φ∈[17π12,19π12]μ, φϵothers wherein， φ; logA; logB1θ=π2, φ; logB2. ak; φk; ck. [-23π12,2π)∪[0,π12]; 5.4453; -200.77; 0; 13.1353; N(0,2.132). [5π12,7π12]; -18.59; π2; 6.0515. 11π12,13π12; -245.68; π; 13.6096. Proposal 7: The RCS of humans, vehicles, and UAVs is positively correlated with frequency overall, and the relationship can be expressed as: Ahuman = 0.1787*frequence-6.171 Avehicle =0.1889*frequence + 1.63 AUAV = 0.2083*frequence-11.49 |

[Moderator’s note] delete the note similar to Proposal 4.2-3-rev1

[H][FL2] Proposal 4.2-5-rev1

The following RCS models are supported for vehicle with single scattering point for monostatic

- A is mean RCS value

- B2 is modelled using log-normal distribution for monostatic

- Different mean RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- FFS Model 1

  - B1=1

- Model 2

  - B1 have dependency on incident/scattered angles

  - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

  - Alt 2: a piecewise function

  - Alt 3: Lookup table

- Note: whether exact values of A/B1/B2 for Model 1 and/or model 2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model.

[Moderator’s note] further revised

[H][FL3] Proposal 4.2-5-rev2

The following RCS model is supported for vehicle with single scattering point for monostatic

- FFS Model 1

  - B1=0 dB

  - A is mean RCS value

  - B2 is modelled using log-normal distribution for monostatic

  - Different mean RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- Model 2

  - B1 have dependency on incident/scattered angles

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a piecewise function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - Different mean RCS values can be supported for vehicle due to different size, shape, frequency, etc.

  - FFS RCS component A

[Moderator’s note] agreement in Wednesday online session

- Agreement

The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

[Moderator’s note] Almost all contributions discuss the issue of target with multiple scattering points (referring to section 4.4). Vehicle is the most typical target type that can be modeled by multiple scattering points. Therefore, based on companies’ inputs and RCS model Option 3, the following proposal is made for vehicle with multiple scattering points.

[H][FL1] Proposal 4.2-6

For vehicle modelled with multiple scattering points for monostatic, the following RCS model is supported for each scattering point

- A is mean RCS value

- B1 have dependency on incident/scattered angles

  - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

  - Alt 2: a piecewise function

  - Alt 3: Lookup table

- B2 is modelled using log-normal distribution

- Note: whether exact values of A/B1/B2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model.

| Company | Comments |
| --- | --- |
| ZTE | Support |
| Huawei, HiSilicon | Suggest correcting the discussion direction to face and discuss the RCS directly, instead of playing with A, B1, B2, etc. Three Alts for B1 is just the formulation difference, for example, what the lookup table is and perhaps the proponent actually means the same thing as in Alt1. The Note is meaningless, without values captured in the TR, the RCS for vehicle is incomplete so that the SI for the channel modelling is not completed. To FL, different tables are given for UAV and Human for collecting values, why is not such table given for vehicle? Huawei and some other company have been proposing the same table from the very beginning. If no other companies provide input till the end, will FL never start such discussion for this SI? |
| Intel | In our understanding the difference comparing to P 4.2-5 for single scattering point is to directly apply Model 2 for multiple scattering points. We’d like to avoid having similar discussions around the same model in two different proposals. This could be achieved by the following update: - H][FL1] Proposal 4.2-6 For vehicle modelled with multiple scattering points for monostatic, the following RCS model is supported for each scattering point - Each scattering point RCS is modelled following the single-scattering point procedure Model 2 - A is mean RCS value - B1 have dependency on incident/scattered angles - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a piecewise function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - Note: whether exact values of A/B1/B2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model. |
| LGE | For simplicity, we think that a single scattering point is defined for each side of a vehicle – e.g. front, rear, left, right and rooftop side. Then based on the incident/scattered angle, we can define the RCS of each scattering point. For example, we can set RCS value to zero if the corresponding side is not visible from Tx. With this understanding, we need the following operation. - RCS value of a vehicle for a specific incident/scattered angle is the sum of all RCS values of the sides of the vehicle, which are visible from both Tx and Rx. In addition, we need constraint on the mean of B1 and B2, similar to other proposals. Note: the mean of B1 and B2 in linear scale equals to 1. |
| Nokia, NSB | RCS should be defined per target, instead of per single scattering point, for targets with multiple scattering points. Therefore, this proposal is not agreeable. For targets with multiple scattering point, we shall define the overall RCS (or mean RCS for the large scale parameter). Each scattering point shall have its own RCS σk, and the overall kσk=RCS of the target. This proposal is more like to define K number of targets, each of which has one scattering point, at one location, instead of one target with K scattering points. |
| Moderator | @Intel: there may be a difference. In 4.2-6 for single scattering point, the applicable phi/theta value for a beam (e.g., front, back, left, right, roof) is limited to a small area of space. However, in 4.2-7, for each point, the applicable phi/theta are all area of space. So, let’s avoid saying reusing model of single scattering point @all: same revision as proposal 4.2-5 |
| CATT/CICTCI | Generally fine but for B1 we prefer to further study alt1/alt3 |
| Qualcomm | We are supportive of the proposal. It may be useful to clarify the number of points to be used. - We suggest a 5-point scattering model with scattering centres at left, right, front, back, and roof. - The mean (A) and variance (B) may vary based on distance. Also, whether a vehicle is modelled with a single or multiple scattering points, it may depend on the distance. We think an FFS may be needed to connect proposals 4.2-5 and 4.2-6, unless the FL’s plan is to treat that aspect in a general proposal about multiple points scattering.. |
| vivo | Support |
| CMCC | Please clarify what’s the RCS value in the statement “A is the mean RCS value”? Whether the RCS value samples from the whole target or a just scattering point? Because the RCS is defined for a target or a physical object with volume and surface. It is not for a single scattering point. |
| Xiaomi | Open to discuss. |
| Ericsson | For multiple scattering point modelling, we have the same suggestion as single scattering point. A=1sq.m and B1 is angle dependent, if they are defined separately. Better to keep all alternatives for now and some further study is needed. We can start from Alt 3 and compare it with other alternatives to see the approximation accuracy. |
| Lenovo | Yes The note is not needed. |
| Toyota | We are OK with the proposal. We would like to better understand the intention of having two models for a vehicle (Proposals 4.2-5 and 4.2-6) and the relationship of the two models. |
| Huawei, HiSilicon2 | Per companies’ comments, vehicle modelled with multiple scattering points in general seems agreeable. Instead of debating A, B1 and B2, also taking some suggestions from other companies above, can we try the following high-level principle proposal? Proposal xx: When the target is modelled with multiple scattering points, the number and the locations of the scattering points should be representative of the sensing target in terms of the shape and/or size. - As for vehicle, the recommended five scattering points are located in front, left, back, right and roof side of the vehicle, respectively. - each scattering point is characterized with zenith angle θ″∈[0°, 180°], azimuth angle φ″∈-180°, 180° , and a distinct local coordinate. - each scattering point has a scattering power pattern, which can be defined analogous to the antenna radiation power pattern in TR38.901 as follows: Parameter; Values. Vertical cut of the scattering power pattern for each scattering point (dB); σ″dBθ″, φ″=0°=-min12θ″-θ0θ3dB,n2,σmax, where σmax=[21] dB and θ″∈0°, 180°. Horizontal cut of the scattering power pattern for each scattering point (dB); σ″dBθ″=90°, φ″=-min12φ″-φnφ3dB,n2,σmax, where σmax=[21[dB and φ″-φn∈-180°, 180°. 3D scattering power pattern(dB) for each scattering point; σ″dBθ″,φ″=σn-min-σ″dBθ″, φ″=0°+σ″dBθ″=90°, φ″,σmax. The parameters for each scattering point; Scattering points for vehicle; n; φn; φ3dB, n; θ0; θ3dB,n; σn [dB]. Front; 1; 0°; [24.4°]; 90°; [28.1°]; [11.42]. Left; 2; 90°; [15.6°]; 90°; [16°]; [19.65]. Back; 3; 180°; [35.7°]; 90°; [45.6°]; [13.09]. Right; 4; -90°; [15.6°]; 90°; [16°]; [19.65]. Roof; 5; /; /; 0°; [11.5°]; [21.88].. Notes: θ″ represents the incident and scattering angle for each scattering point independently. φ″ represents the incident and scattering angle for each scattering point independently. The σn represents the practical maximum power of the scattering point.. |

[Moderator’s note] Based on the companies’ input, RCS of vehicle is more controversial, which is also highly related to the discussion of multiple scattering points of target. Therefore, the moderator suggests to wait more inputs or certain progress on handling multiple scattering points in the ISAC channel model.

[H][FL1] Proposal 4.2-7

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | How does FL observe the RCS of vehicle is more controversial? It is getting to converge actually. If FL disallows the discussion continually, how can any controversy be resolved? Again, if RCS modelling of vehicle is not resolved or captured in the TR, the SI is not completed. |
| Moderator | @Huawei: Let do it step by step, the high level design first. Please check my update on Porposal 4.2-5/6. We have too many targets, even agreeing on using antenna radiation pattern for B1 is a very good progress. The exact values of each beam can be discussed in later meeting. |

[Moderator’s note] delete the note similar to Proposal 4.2-3-rev1

[H][FL2] Proposal 4.2-6-rev1

For vehicle modelled with multiple scattering points for monostatic, the following RCS model is supported for each scattering point

- A is mean RCS value

- B1 have dependency on incident/scattered angles

  - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

  - Alt 2: a piecewise function

  - Alt 3: Lookup table

- B2 is modelled using log-normal distribution

- Note: whether exact values of A/B1/B2 can be captured in the CR to 38.901 is depending on the validation results on the RCS model.

[Moderator’s note] further revised

[H][FL3] Proposal 4.2-6-rev2

For vehicle modelled with multiple scattering points for monostatic,

- the recommended five scattering points are located in front, left, back, right and roof side of the vehicle, respectively

- the following RCS model is supported for each scattering point

  - B1 have dependency on incident/scattered angles

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a piecewise function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - Different mean RCS values can be supported for vehicle due to different size, shape, frequency, etc.

  - FFS RCS component A

[Moderator’s note] agreement in Wednesday online session

- Agreement

When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- the recommended five scattering points are located in front, left, back, right and roof side of the vehicle

- the following RCS model is supported for each scattering point

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

RCS values for AGV

Summary on company views

| Company | Results on RCS |
| --- | --- |
| Huawei | Table 18: The B1 (scattering power pattern of a scattering point) for AGV for mono-static and bi-static channel Parameter; Values. Vertical cut of the scattering power pattern for each scattering point (dB); σ″dBθ″,φ″=0°=-min12θ″-θ0θ3dB,n2,σmax, where σmax=24dB and θ″∈0°, 180°. Horizontal cut of the scattering power pattern for each scattering point (dB); σ″dBθ″=90°,φ″=-min12φ″-φnφ3dB,n2,σmax, where σmax=24dB and φ″-φn∈-180°, 180°. 3D scattering power pattern(dB) for each scattering point; σ″dBθ″,φ″=σn*cosβ2-min-σ″dBθ″,φ″=0°+σ″dBθ″=90°,φ″,σmax. The parameters for each scattering point; Scattering points for AGV; n; φn; φ3dB,n; θn; θ3dB,n; σn [dB]. Left; 1; 90°; 11.6°; 60°; 20.6°; 9.31. Back; 2; 180°; 11.3°; 90°; 10.1°; 8.89. Right; 3; -90°; 11.6°; 60°; 20.6°; 9.31. Top; 4; /; /; 0°; 6.3°; 14.41.. Notes: θ″ represents the incident and scattering angle for each scattering point independently. φ″ represents the incident and scattering angle for each scattering point independently. The σn represents the practical maximum power of the scattering point. For bi-static link, the (θ″,φ″) represents the bisector angle of the bi-static link.. |
| LG | Proposal 15: Calculate the monostatic RCS of AGV as the multiplication of the component A = 1 m2 (AGV Option 1) or A = 4 m2 (AGV Option 2), the component B1=1 and the component B2 generated as the exponential random variable with the mean value equal to 1 (the standard deviation equal to 1). Proposal 16: Calculate the bistatic RCS of AGV by multiplying the corresponding monostatic RCS by the component C2 dependent on the bistatic angle as set by equation Error: Reference source not found. That is, B1 = cos(β/2) where β is a bistatic angle. |

- [Moderator’s note]

Based on the companies’ input, RCS of vehicle is more controversial, which is also highly related to the discussion of multiple scattering points of target. Therefore, the moderator suggests to wait more inputs or certain progress on handling multiple scattering points in the ISAC channel model.

[H][FL1] Proposal 4.2-8

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Is FL’t note saying vehicle or AGV? Vehicle and AGV are different sensing targets, we don’t see the reason to suspend the discussion of RCS for AGV. Again, if RCS modelling of AGV is not resolved or captured in the TR, the SI is not completed. |
| Moderator | @Huawei: Let’s focus on normal vehicle first. Later we can discuss AGV is single or multiple scattering points and the detailed model. |

Polarization at an object other than EO type-2

- Agreement The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp - FFS whether CPMsp is angular/ray-dependent or independent - FFS whether polarization matrix CPMsp is modelled assuming specular reflection or random coefficient for diffraction or scattering - FFS whether polarization matrix CPMsp is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx

Summary on company views

RCS polarization VV, HH, VH, HV

In RAN1 #118, RAN1 agreed that RCS is modelled as a scalar value. The exact scalar RCS value for each target type needs further discussion. Considering polarization H and V, there are 4 polarization RCS values, i.e., HH, VV, HV, VH, which can be used to derive XPR.

- Xiaomi, ZTE, BUPT: similar/same RCS for HH, VV; same value for RCS HV, VH; RCS VH is much lower than RCS VV

Cross polarization matrix

CPMsp=expjΦspθθκsp-1expjΦspθϕκsp-1expjΦspϕθexpjΦspϕϕ: vivo, CATT, ZTE, QC, Xiaomi, BUPT, Sony

- Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ uniformly distributed within -π,π: BUPT, QC, ZTE

- XPR κsp can be newly determined by the experiment campaign: vivo

  - Log-normal: QC, ZTE

BUPT:

- κhuman=0.0707

- κUAV=0.1102

- κvehicle=0.1068

ZTE

- Proposal 3: For UAV of small size (Length x Width x Height: 0.3m x 0.4m x 0.2m) at 4.9GHz

  - XPR for the UAV is modelled as log-Normal distribution. Draw linear-scaled XPR value as

=10Y10,

Where, Y~NμXPR,σXPR is Gaussian distributed with μXPR24.4and σXPR= 3.05.

- Proposal 4: RCS Option 3 is selected to model RCS of pedestrian with single scattering point

  - XPR for pedestrian is modelled using log-Normal distributed. Draw XPR values as

=10Y10,

Where, Y~NμXPR,σXPR is Gaussian distributed with μXPR25.8 and σXPR= 2.99.

Proposal 5: For mono-static, the RCS=A*B=A*B1*B2 of a scattering point for a passenger vehicle Type 1/2 can be generated by

  - XPR for the Vehicle is modelled using log-Normal distributed. Draw XPR values as

=10Y10,

Where, Y~NμXPR,σXPR is Gaussian distributed with μXPR22.3 and σXPR= 4.71.

NIST: CPMsp=ejθVV10-βi10ejθVH10-γi10ejθHV10αi10ejθHH

  - αi can be modelled as a normal distribution

  - βiand γi follow the same normal distribution, βiand γi may have different values

  - θVV, θVH,θHV,∧θHHcan be independently following a uniform distribution in the range (-π, π)

LGE

| RCS=σbvv4π | (1) |
| --- | --- |
| CPMsp=1σbvhσbvvejδvhσbhvσbvvejδhvσbhhσbvvejδhh | (2) |

Specular reflection

- specular reflection applicability to the target: Ericsson, IDC, Intel

- Random coefficient for diffraction or scattering: E//, Xiaomi, IDC

- CPMsprotated target=R1⋅CPMsp⋅R2:  Ericsson

Ericsson: CPMsp must be specified in a local coordinate system in which the z-axis is parallel to the z-axis in the global coordinate system, zGCS⋅zLCS,target=1

- [Moderator’s note]

Based on the companies’ inputs, it seems CPM of a direct/indirect path of a scattering point can be modelled by XPR and initial random phase. Majority companies prefer to reuse the same principle as section 7.5, 38.901, while two companies propose a more flexible model.

Given we just started discussion on this issue, it would be better to collect more views on the two options. Please provide your preferred option

[H][FL1] Proposal 4.3-1

Down selection between the following two options to model the polarization matrix of a direct/indirect path at a scattering point of a target.

Option 1:

- For a direct/indirect path of a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp is modelled by XPR ratio κsp, and initial random phases Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ, i.e., CPMsp=expjΦspθθκsp-1expjΦspθϕκsp-1expjΦspϕθexpjΦspϕϕ

  - κsp is randomly generated by log-normal distribution. FFS mean/variance of the distribution

  - The initial random phase Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ is uniformly distributed within -π,π

Option 2:

- For a direct/indirect path of a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp is modelled by αsp,βsp,γsp and initial random phases Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ, i.e., CPMsp=ejΦspθθ10-βi10ejΦspθϕ10-γi10ejΦspϕθ10αi10ejΦspϕϕ

- αi can be modelled as a normal distribution

- βiand γi follow the same normal distribution, βiand γi may have different values

- FFS correlation between αsp,βsp,γsp

- The initial random phase Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ is uniformly distributed within -π,π

| Company | Option 1 or 2 | Comments |
| --- | --- | --- |
| ZTE | Option 1 | We are also open to option 2 which may be more precise as long as it can be justified by the measurement/RT results. |
| Huawei, HiSilicon | Neither | Whether XPR should be modelled should depend on how much XPR is observed. For example, in the current TR38.901, the max XPR is 12dB@ RMa, 11dB@inH, 9dB@UMa, 8dB@UMi. If the XPR is as large as ~15/20 or even 30dB, it might be negligible. Again, we should look at the RCS value first for each sensing target and decide whether XPR is worth being modelled then, instead of agreeing it should be modelled in general. Why doesn’t FL make the proposal for EO type-2? |
| Intel |  | If specular reflection can be applicable to targets, we think the random phase approach is not accurate in these cases. It seems the random phase Φ generated for each of the components even for the direct path. Probably to resolve this, both options bullets should be updated as “For a direct/indirect path of a scattering point of an object other than EO type-2 when specular reflection cases…” |
| NIST | Option 2 | We proposed option 2 based on our measurements. Please note our model is for each path/scattering point (R1-2410136). For Option 1, is the observation for a single path or for the whole target consisting of multiple paths? |
| LGE | Option 1 | Option 1 is the simplified version of option 2, assuming that αi=0 and βi=γi. As we agreed to use the form of option 1 for CPMtx,sp and CPMsp,rx , we prefer option 1 for simplicity and commonality. |
| Samsung |  | Support Option 1. We are not sure the reason why option 2 has power imbalance between co-polarization terms. |
| Moderator | @Huawei, @all: just to clarify my logic, section 4 is for object/target, mainly some RCS issues. If it is the polarization of the object/target itself, it is also in section 4. On the other hand, EO type-2 is a bit different. In 7.6.8 in 38.901, it actually gives an model for the full link, i.e., “node 1 – ground – node 2”, instead of ground only. This is the reason I put EO type-2 related design into section 5.6 @Intel: The proposal is using “other than EO type-2”, so this is a full design for non-EOtype-2 object/target. If majority prefer to have multiple options for non-EOtype-2 object/target, I will revise per your suggestion. |  |
| CATT/CICTCI | Option 1 | We can further study the need for option 2 |
| Qualcomm | Option 1 |  |
| vivo | Option 1 | To test the components of HH, VV, HV, and VH, four experiments with the same setting but at different times need to be conducted. Due to the random factors occurring at different times, it is hard to make sure of the temporal consistency when we compare the components of HH and VV, and the components of HV and VH. This may incur the unbalanced XPR factor. If we take Option 2 for CPM, we need to modify the CPM for communication channel as well. |
| CMCC | Option1 |  |
| Spreadtrum | Option 1 | We prefer a simplified model. Randomly generate different values for αsp,βsp and γsp in option 2 seems not needed. |
| Xiaomi | Option 1 | Prefer a simple solution. Based on the measurement, more companies show that the VV RCS and HH RCS exhibit similar value, while VH RCS and HV RCS exhibit similar value. |
| Ericsson | No | Option 1 assumes same HH and VV power and same HV and VH power, which conflicts with our measurements. In option 2, the component of VV can also have a scaling factor, say δ, which follows the same distribution as α but with different values. It is arbitrary to assume the scaling factor of VV=1. With Brewster angle, scaling factor of VV is 0. Both options assume CPM is angle-independent and path-independent. Is i path index or target index? It is not clear how to derive the four values αi，βi ,γi, δ with angle-dependent RCS measurements. In our view, if a target’s RCS is angle-dependent, so is CPM. The proposal doesn’t mention which coordinate system it refers to. Is this in the local coordinate system of the target (which would rotate if the target rotates) or in the global coordinate system (in which CPM_tx,sp and CPM_sp,rx are given)? |
| InterDigital | No | We agree with Intel that specular reflection should be considered in the model as it can be applicable for some targets. As we explained in our contribution, the surface of the obstacle can be a flat surface causing specular reflection. Unifying all reflection characteristics as scatter is a concern for us. As a compromise, we propose the following. Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ can follow the distribution with a mean and adjustable standard deviation. For example, the angles can be uniform distribution with an adjustable mean and standard deviation. Another example is truncated Gaussian distribution with adjustable mean and standard deviation. Adjustable mean and standard deviation of the distribution provides flexibility to model either scatter or specular reflection. |
| Lenovo |  | Direct and indirect paths and specular/non-specular can be separated, at least regarding the initial phase. We have not yet established that the initial phase is completely geometry independent. For indirect path this can be considered similar to 38.901. |
| BUPT | Option1 | For humans, vehicles, and UAVs as sensing targets, The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp, where CPMsp is given by: CPMsp=expjΦθθXPRtarget-1expjΦθϕXPRtarget-1expjΦϕθexpjΦϕϕ Here, the XPR[dB] for humans, UAVs, and vehicles follows a normal distribution: XPRhuman[dB]~N(11.5509,2.1532), XPRUAVdB~N(9.7659, 3.7682), XPRvehicle[dB]~N(9.456,2.517062), and expjΦθθ，expjΦϕθ，expjΦθϕ，expjΦϕϕare uniformly distributed over the interval [−π,π]. |
| Apple | Option 1 |  |
| Moderator | @Intel, Lenovo: can a vehicle in certain incident/scatter angles be modelled as pure specular reflection. I guess not. I mean, even when specular reflection happens, there must be some scattered signals arrive at receiver too. So there is likely to be still a random phase. In summary, the amplitude of the reflection is already modelled by RCS, the random phase just modelled by the polarization matrix. @all: since clear majority view prefer Option 1. So I propose the agree on Option 1. |  |

[H][FL2] Proposal 4.3-1-rev1

To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp is modelled by XPR ratio κsp, and initial random phases Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ, i.e., CPMsp=expjΦspθθκsp-1expjΦspθϕκsp-1expjΦspϕθexpjΦspϕϕ

  - κsp is randomly generated by log-normal distribution. FFS mean/variance of the distribution

  - The initial random phase Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ is uniformly distributed within -π,π

| Company | Option 1 or 2 | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | We haven’t looked at and discussed based on the measurement whether the κsp is large enough to be worth being modelled. |
| NIST |  | Some clarification regarding option 2. We feel RCS already considers the angle dependency, so CPM can be to angle independent, and let RCS handle the angle dependent part. Also, i refers to a path in option 2. If i is target, we can accept option 1. We feel we need to be clear about what we are modeling here. I heard a similar polarization modeling is being discussed in 7-24 GHz study, should we also cross-check their measurements/discussions? |
| InterDigital |  | We don’t support the proposal. Even when we don’t have perfect specular reflection, we don’t expect phase to be uniformly distributed. We propose the following. Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ can follow the distribution with a mean and adjustable standard deviation. For example, the angles can be uniform distribution with an adjustable mean and standard deviation. Another example is truncated Gaussian distribution with adjustable mean and standard deviation. Adjustable mean and standard deviation of the distribution provides flexibility to model either scatter or specular reflection. |
| Moderator | Revised by offline discussion |  |

- [H][FL4] Proposal 4.3-1-rev2

To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ

  - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π]

  - FFS correlation between αi,1,αi,2,βi,1,βi,2

  - FFS specular reflection

The following options are considered for further study, down select one option from the following

- Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio

  - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution

- Option 2 (NIST): αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i

- Option 3 (Ericsson): αi,1,αi,2,βi,1,βi,2 are variables generated for path i

  - CPMsp,i defined in LCS

- Option 4 (Huawei): αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i

Target with multiple scattering points

Summary on company views

In general, for a target modelled with multiple scattering points, many details need further discussions.

target modelled with multiple scattering points is OPTIONAL: CATT

Single vs. multiple scattering points

The number of scattering points for a target can be verified by PDP measurement

- Multiple points for vehicle: Huawei, Xiaomi, BUPT

ZTE: option for multiple scattering points for UAV, human

CAICT

- single point for UAV in low altitude scene, human outdoors, etc.

- multiple point for automotive vehicles, automated guided vehicles, human indoor etc.

Sony

- vehicle and human: multiple scattering points

- UAV: single scattering point model only

Factors to decide single or multiple scattering points for a target?

- the distance between Tx/Rx and the target: Nokia, vivo, Samsung, Apple, Xiaomi, AT&T, CATT, EUROCOM, CAICT, IDC, Tiami

  - NO: ZTE

- target type: Nokia, vivo, Xiaomi, EUROCOM, Sony, AT&T, DOCOMO, Apple, CAICT, Lenovo

- use case and deployment scenario: Nokia, vivo, Xiaomi, AT&T, Apple, MTK

- frequency: vivo, CATT

- target size: Huawei, vivo, Apple, Nokia, Samsung, MTK, DOCOMO, Xiaomi, CATT, EUROCOM, AT&T, TOYOTA, CAICT, Tiami, Lenovo

- target shape: Huawei, vivo, Nokia, Apple, CAICT,

- sensing resolution: MTK, CAICT,

- sensing service: MTK

- incident/scattered angle: Lenovo

Number of scattering point(s)

- Modeling all types of sensing targets as a single scattering point in Rel-19: ZTE

  - As start point: Xiaomi, BUPT (prioritized), AT&T

- N points should limit in the size or shape of the object: Huawei, vivo, ZTE

- The number of scattering points of a target

  - keeps unchanged in the whole simulation: ZTE, Nokia, Apple, CATT, AT&T, EURECOM

  - vary over time in the simulation: Nokia

- The location of scattering points of a target are not fixed: CMCC, Lenovo

- The same number and locations of scattering points apply regardless of sensing Tx/Rx: QC, Xiaomi, ZTE, CATT, Apple, AT&T

Location of the multiple points

- N points are dropped within size/shape of the target: vivo, Xiaomi

- Relative location of the N points not changed: QC, CATT, Apple, Xiaomi, ZTE, MTK

Lenovo: For modelling of a target via one or multiple scattering points, the scattering point locations shall be determined as part of the target channel modelling procedure.

Xiaomi: need to determine the location of the single scattering point of the target

Interaction between the multiple points

- NO rays scattered from one point to another point of the same target: QC, Intel, CATT, CT, Apple, AT&T, MTK, EUROCOM, Xiaomi

RCS of the multiple scattering points

- RCS values are individually determined: Huawei, QC, CATT, vivo, CT, Apple, Tiami, EUROCOM

  - Single/common RCS pattern: DOCOMO

- RCS of entire target equals to sum of RCS of each scattering points: vivo, Nokia, LG

Visibility of the multiple (N) scattering points

- N points always visible: ZTE

TOYOTA: translation motion and rotational motion

DOCOMO

- Model level-1: modelling with single scattering point (SP) at the center of target

- Model level-2: modelling with single scattering point (SP) at the center of target and Nps pseudo-SPs with fixed positions

- Nps = 4, for human and UAV cases

- Nps = 12, for vehicle, AGV, animal cases

- Single SP has an RCS value/pattern (simulation or measurement result)

- Each pseudo-SP does not have RCS value but can have pseudo-RCS value referring to the RCS value of SP by using some functions of differences delay time and angle between the SP and pseudo-SP in spatial-time profile

- [Moderator’s note]

The above listed issues had been discussing using FL summary documents in past 4 RAN1 meetings. The proposal 4.4-1 tries to summarize the majority views. Since they are actually quite basic design principles, it would be quite helpful to agree on the proposal to check further design details.

Regarding the number/location of the multiple scattering points. Two options were discussed and majority companies prefer Option 1, i.e., the simple one. Given TU for the Rel-19 study item is quite limited, it is generally preferred to adopt certain simplification for the modelling. I directly set it as possible working assumption for further confirmation

- Option 1: The number and locations of the scattering points are common to each pair of sensing Tx/Rx

- Option 2: The number and/or locations of the scattering points are dependent on the directions of sensing Tx/Rx

Some companies comment on the self-blockage issue at the target, i.e., some scattering points in the opposite direction is invisible to sensing Tx/Rx, such effects can be modelled by two methods. The first method is directly self-blockage modelling. The target may be considered as a cuboid. However, it is questionable of is accuracy. The other method is to effectively remove the invisible scattering points by the angular dependent RCS, i.e., the invisible scattering point must have a low RCS in the interested incident/scattered direction. Hope the following FFS can address the concern

- FFS A scattering point that is invisible from an incident direction is modelled by angular dependent RCS component B1 or self-blockage at the target

Please comment if you have a strong concern on the proposal.

[H][FL1] Proposal 4.4-1

If a target is modelled with multiple scattering points,

- The number of scattering points of a target is generated in the beginning of the simulation and keeps unchanged in the whole simulation

  - The number of scattering points may be dependent on multiple factors, including type/size of the target, the scenario/use case. FFS dependence on the distance between sensing Tx/Rx and the target

- (Working assumption) The number and locations of the scattering points are common to each pair of sensing Tx/Rx

- FFS A scattering point that is invisible from an incident direction or a scattered direction is modelled by angular dependent RCS component B1 or self-blockage at the target

- The changes of relative locations (i.e., micro motion) of the multiple scattering points are NOT considered in the modelling of delay and angle of a direct/indirect path

  - FFS micro-Doppler

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- RCS values of the multiple scattering points are individually determined

| Company | Comments |
| --- | --- |
| ZTE | OK. For micro-Doppler, we don’t need to model the locations of each of scattering points. The modeling is for Doppler effect. Hence, this proposal should be also applicable for micro-Doppler. The whole FFS part can be deleted. |
| Huawei, HiSilicon | In general, the proposal is not wrong. However, - The first main bullet together with the subbullet is more about how to use the >1 points in the simulation, rather than modeling itself. - The working assumption bullet seems also simulation related but may be good also to modelling. - The second and third last bullets are more about the target channel regarding multiple scattering points rather than multiple scattering points modelling itself. - The last bullet is indeed about multiple scattering points modelling. Suggests FL also take other high level multiple scattering points modelling methodology for discussion. For example, the following proposals can be considered: Proposal 4: When the target is modelled with multiple scattering points, the number and the locations of the scattering points should be representative of the sensing target in terms of the shape and/or size. Proposal 5: When the target is modelled with multiple scattering points, each scattering point is characterized with spherical angles with together with a distinct coordinate. Proposal 6: When the target is modelled with multiple scattering points, each scattering point has a scattering power pattern. |
| Intel | In the 2nd bullet, we don’t need to put WA from the beginning, can discuss it first. We think due to spatial consistency the number of scattering points must be the same for a target on different sensing links. Further, “the number” is already covered by the 1st bullet, so this bullet can focus on the location aspect only. |
| LGE | The blockage of a scattering point happens not only for the incident path but also for the scattered path. Suggest the following modification. - FFS A scattering point that is invisible from an incident direction or a scattered direction is modelled by angular dependent RCS component B1 or self-blockage at the target We also suggest to remove the micro-Doppler FFS point. - The changes of relative locations (i.e., micro motion) of the multiple scattering points are NOT considered in the modelling of delay and angle of a direct/indirect path - FFS micro-Doppler The wording ‘individually’ in the last bullet is unclear to us. Does it mean ‘independently’? If so, we think it depends on how to define the multiple scattering points. For example, if the multiple scattering points are defined within a short distance (e.g. within the same side of a vehicle), we think that some correlation between RCS of the scattering points need to be considered. Otherwise, if a scattering point is defined for each side of an object, they can be modeled independently. |
| Nokia, NSB | The last bullet “RCS values of the multiple scattering points are individually determined” needs further clarification. Propose with this change: RCS values of the multiple scattering points are individually determined, where the total sum of the RCS values of all scattering points equal to the RCS value of the target. |
| Moderator | @Intel: since we already discuss several meetings for the proposal though no time to actually treat it in offline/online session. I put WA to check if some opponent can be acceptable. @LGE: Good comments. I directly add your words in the current proposal @ZTE, LGE: since micro-Doppler is handled in another proposal 5.4-1, I delete the FFS on micro Doppler per your suggestion. @Nokia: Agree you have a point, but better to leave such issue on combined RCS a separate discussion. If we mixed too many things in this very first proposal for multiple scattering points, I’m afraid it become controversial again. To @all, we definitely need everyone to be constructure since we have no TU for endless discussion. Sincerely hope everyone can be OK for majority view for progress. |
| EURECOM | Support the proposal |
| Ericsson | We are generally fine, except for the following bullet. - The changes of relative locations (i.e., micro motion) of the multiple scattering points are NOT considered in the modelling of delay and angle of a direct/indirect path This depends on the location of multiple scattering points. If they are located at the rotors of a UAV or arms/legs of a human, their relative location changes in time. If they are positioned at the doors of a vehicle, their relative locations are constant. With multiple scattering points of a target, it seems natural and in the spirit of the proposal to essentially model the target channel as a sum of independent target channels for each of the scattering points. But the bullet mentioned would make the target channels dependent on each other since the movement of each scattering point is now restricted to be the same as the other scattering points. This might complicate implementation. |
| CATT/CICTCI | Generally OK but We prefer to study the criterion to decide a target can be modelled with multiple scattering points |
| New H3C | OK |
| Qualcomm | OK |
| vivo | Support |
| Spreadtrum | Support the proposal in principle. Regarding the last bullet, we think it’s related with Proposal 5.5-1. We suggest to mark it as FFS for now. |
| Xiaomi | Support in principle. |
| Lenovo | Generally, OK. the relative changes are not limited to the rotational movement (when a human standing up). If relative location change is not considered, this also means the micro doppler is not captured. |
| AT&T | Generally ok |
| Toyota | OK |
| BUPT | Support the proposal. Based on the measurements, the number and distribution of the scattering points on the vehicle are closely related to the current angle between the antenna and the vehicle. About the situation that the scattering point is invisible from an incident direction, prefer modelled by angular dependent RCS component B1. |
| Apple | - Clarification of “The number and locations of the scattering points are common to each pair of sensing Tx/Rx” - What evaluation scenario are we assuming this for ? - Clarification of difference between “micro-motion” and “micro-Doppler”. It seems from the comments that these are assumed to be synonymous. - Is this saying that RAN1 assumes the relative locations of the multiple scattering points of a target are not changed and that translational motion of the target can be modelled while rotational motion of the target should not be considered ? |
| Moderator | It seems the 4th bullet is not stable, so I make it a FFS |

- [H][FL2] Proposal 4.4-1-rev1

If a target is modelled with multiple scattering points,

- The number of scattering points of a target is generated in the beginning of the simulation and keeps unchanged in the whole simulation

  - The number of scattering points may be dependent on multiple factors, including type/size of the target, the scenario/use case. FFS dependence on the distance between sensing Tx/Rx and the target

- (Working assumption) The number and locations of the scattering points are common to each pair of sensing Tx/Rx

- FFS A scattering point that is invisible from an incident direction or a scattered direction is modelled by angular dependent RCS component B1 or self-blockage at the target

- FFS whether/how to model the changes of relative locations (i.e., micro motion) of the multiple scattering points are NOT considered in the modelling of delay and angle of a direct/indirect path

  - FFS micro-Doppler

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- RCS values of the multiple scattering points are individually determined

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | As commented this proposal is more about how to handle these points during simulation. Before agreed on that, we should discuss and agree on some high level modelling itself. As commented to [H][FL1] Proposal 4.2-6 by’ Huawei, HiSilicon2’ |

Target channel modelling

Existing TRs reused as starting point

Agreement In order to generate each of the Tx-target link and target-Rx link in the target channel, the large scale and small scale parameters defined in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, TR 38.858, TR 38.859, TR 38.802, TR 38.854, etc. are used as starting point

Summary on company views

Existing TRs applicable in target channel (especially for UE-target link)

- Ericsson: For Uma, Umi, Rma, study link between outdoor target and UE

- Ericsson: study reusing BS-UE channel model in 38.901 for indoor UE and indoor target

Apple: Angular spread may be different at the Tx and Rx for bi-static sensing but is the same for mono-static sensing.

CAICT: K factor for target channel should be designed in two categories.

- Category 1 is for UAV above background environments, which define LOS power ratio larger than 95%.

- Category 2 is for UAV below background environments and other targets, which is similar to K factor in TR 38.901.

- [Moderator’s note]

Please check Proposal 6.2-2 first, then this proposal.

RAN1 already agree on reusing existing TR as start point for the Tx-target link and target-Rx link. However, the exact mapping between the STX/SRX, the target and the associated existing TRs is still missing.

Assuming Proposal 6.2-2 is agreeable, the proper existing TRs to generate the Tx-target link and the target-Rx link in the target channel can be simply derived. Please check if you have any concern on such mapping.

- [H][FL1] Proposal 5.1-1

- To generate the Tx-target link and the target-Rx link in the target channel, the following mapping between STX/SRX, target and the proper case of existing TRs is adopted

  - UAV is considered as aerial UE

  - Vehicle is considered as vehicle UE

  - Human, AGV and objects creating hazard are considered as normal UE

| STX/SRX | Target | Case of existing TRs as starting point |
| --- | --- | --- |
| TRP | UAV | Case 4: TRP-Aerial UE link of same scenario |
|  | Human | Case 2: TRP-normal UE link of same scenario |
|  | Vehicle | Case 3: TRP-Vehicle UE link of same scenario |
|  | AGV | Case 2: TRP-normal UE link of same scenario |
|  | Object (Hazard) | Case 2: TRP-normal UE link of same scenario |
| Terrestrial UE | UAV | Case 7: normal UE - Aerial UE link of same scenario |
|  | Human | Case 5: normal UE-normal UE link of same scenario |
|  | Vehicle | Case 6: normal UE - Vehicle UE link of same scenario |
|  | AGV | Case 5: normal UE-normal UE link of same scenario |
|  | Object (Hazard) | Case 5: normal UE-normal UE link of same scenario |
| Vehicle UE | Human | Case 6: normal UE - Vehicle UE link of same scenario |
|  | Vehicle | Case 8: Vehicle UE - Vehicle UE link of same scenario |
|  | Object (Hazard) | Case 6: normal UE - Vehicle UE link of same scenario |
| Aerial UE | UAV | Case 9: Aerial UE - Aerial UE link of same scenario |
| AGV UE | AGV | Case 5: normal UE - normal UE link of same scenario |

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon | Should be lower priority | This is high priority from FL. However, we are NOT expecting any offline/online discussion on this one, which can be reviewed when discussing the CR so not deserving the precious discussion time. |
| LGE |  | We’re ok with the proposal in general. We think that UE-type RSU defined in 36.885 and 37.885 as STX/SRT also need to be considered. |
| Samsung | Yes |  |
| Nokia, NSB | Yes | Fine with the proposal. |
| EURECOM | Yes |  |
| CATT/CICTCI | Ok |  |
| New H3C | Yes |  |
| Qualcomm | Low priority | It also appears to us that this makes sense to be lower priority currently. |
| vivo | Yes |  |
| CMCC | Yes |  |
| OPPO |  | We are open to whether to treat AGV as a normal UE or a vehicle. |
| SONY | OK |  |
| Spreadtrum | Yes |  |
| Xiaomi | Yes |  |
| Toyota | Yes |  |
| BUPT | Yes | We agree with the approach of maximizing the reasonable reuse of existing TRs, while it is necessary to perform experimental validation or provide supplementary analysis for the three specific cases: - Case 3: TR 37.885 lacks parameters for BS-UE in Urban Grid and Highway scenarios, which must be provided based on experimental data. - Case 5: For UE-UE targeting human objects in InH scenarios and AGV (Autonomous Guided Vehicle) objects in InF scenarios, the corresponding parameters are missing and need to be supplemented through measurements. Case 6: For UE-UE targeting vehicle objects in UMi, UMa, and RMa scenarios, TR 38.858 recommends directly modifying antenna height. However, the reliability of this approach requires experimental validation. |

Concatenation of Tx-target and target-Rx link

- Agreement For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel, - An indirect path in small scale is modelled by concatenation of path(s) from Tx to target and from target to Rx, i.e., Option 1 in the agreement of RAN1 #117 - AoD/ZoD/AoA/ZoA from Tx to target or from target to Rx are - generated for a LOS ray at least based on the 3D location of Tx/Rx and target in the global coordinate system - stochastically generated for a NLOS ray using section 7, 38.901 as starting point - Delay is sum of delay of LOS/NLOS ray from Tx to target and the LOS/NLOS ray from target to Rx - Doppler is generated by spherical unit vector by AoD/ZoD at Tx and velocity of Tx, by spherical unit vector by AoA/ZoA at Rx and velocity of Rx, and by spherical unit vectors by AoA/ZoA/AoD/ZoD at target and velocity of target - FFS The mobility of stochastic clutter - The power of the indirect path is generated as the product of the power of the LOS/NLOS ray from Tx to target, the power of the LOS/NLOS ray from target to Rx and the effect of RCS - FFS initial phase - FFS how to model effect of RCS at target - FFS whether/how to model polarization at target - FFS How to reduce complexity - FFS applicability of the indirect path generation to each/a single scattering point when the target is modelled as multiple scattering points

Summary on company views

In general, it is commonly agreed that the generated paths in the concatenation of the tx-target and target-rx links should not have a statistics significantly different from that of the full ray-level convolution of the two links.

- Limit maximum number of bounces: EURECOM(2), Tiami, Xiaomi, Apple

- Some NLOS-NLOS indirect paths should be kept in the target channel: CMCC, vivo

Some parameters of the modelled stochastic clusters could be changed

- number of clusters can be reduced: Xiaomi, CAICT, MTK, Nokia

  - reduced

    - Huawei (<10)

    - ZTE: about N/3

  - NO: QC, EUROCOM, Tiami

- number of rays per cluster can be reduced: Xiaomi, MTK, Nokia

  - NO, still 20 rays per cluster: QC, ZTE, Huawei, Tiami, EUROCOM

For path dropping, the metric is the product of power of a cluster in Tx-target link, power of a cluster in target-Rx link and RCS of the pair of clusters

  - Supported by Ericsson

Options for concatenation

The following detailed solutions for concatenation with complexity reduction are proposed based on contribution review

- Option 0: Sony, Ericsson

- Option 0A: Sony

- Option 1: Intel, MTK, Lekha, CT

- Option 1A: Intel, MTK

- Option 2: CMCC, QC, BUPT, CAICT, SPRD, IDC, Lekha

  - Random cluster coupling: CMCC, BUPT

- Option 2A: Apple, QC, CAICT

- Option 3: CMCC, vivo, Xiaomi, CATT, BUPT, EUROCOM, CT

  - Random cluster coupling: CMCC, BUPT, Xiaomi

  - max (the number of clusters in TX-Target link, the number of clusters in Target-RX link): Xiaomi

  - min (the number of clusters in TX-Target link, the number of clusters in Target-RX link): vivo(3-2), BUPT

- Option 3A: ZTE

Note: orange colour means simulation results available. Block colour means analysis without results

Huawei: in 1-by-1 mapping, the two strongest clusters are forcing to 1 by 1 couple. The N-2 weakest clusters are randomly 1 by 1 coupled

Power normalization

- NO: ZTE

- Scaling power of

  - Alt 1: NLOS+NLOS path only: CMCC, vivo, Xiaomi, CATT, BUPT

  - Alt 2: LOS+NLOS, NLOS+LOS, NLOS+NLOS path: CMCC

  - Alt 3: all direct/indirect path:

- Scaling factor is

  - the ratio of number of N+N paths in the Option 0/0A and Option 1/1A/2/2A/3/3A: CATT, QC

  - the ratio of total power of N+N paths in the Option 0/0A and Option 1/1A/2/2A/3/3A: Xiaomi

ZTE: almost all of NLOS + NLOS paths have a power less than 25dB compared with the maximum power of LOS + NLOS / NLOS + LOS paths

Path dropping

Considering impact of pathloss of target channel and background channel: CMCC

|  | drop LOS+NLOS, NLOS+LOS, NLOS+NLOS path | drop NLOS+NLOS path only |
| --- | --- | --- |
| ray-level drop | vivo (-25, -35 not work), Xiaomi (-25 not work) | vivo (-25 not work) |
| Cluster-level drop | vivo (-25 not work) | vivo (-25 not work) |

- EUROCOM: -30dB as threshold

Simulation results

Power

- Separate collection for L+L, L+N, N+L, N+N

  - No power normalization

    - Similar: Samsung (Opt1, 3), Xiaomi

  - Power normalization

    - Large difference: CATT

- Total power

  - No power normalization

    - Clear difference for Option 1/2/3 from Option 0: Samsung, CATT, OPPO

  - Power normalization

    - Same: CATT

- CATT: Neither CDF curve of all path power, nor the 4 CDF curves respective for path power of 1) LOS+LOS, 2) all LOS+NLOS, 3) all NLOS+LOS and 4) all NLOS+NLOS, can be used to distinguish the performance of different Options for other indirect paths of NLOS + NLOS.

Delay spread, angular spread

| Normalization | Propagation condition | Concatenation schemes |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Option 1 | Option 2 | Option 3 | 0A/1A/2A/3A |
| Alt 1 | Case 1 | Xiaomi, vivo, Intel, CATT, QC, BUPT | Vivo, CMCC, QC(with difference), BUPT | Xiaomi, vivo, CMCC, CATT, BUPT |  |
|  |  |  | Xiaomi (DS), CATT (AS InH) |  | Xiaomi (DS, AS), CATT(AS) |
|  | Case 2 | Xiaomi, vivo, Intel, CATT, QC, BUPT | Xiaomi, vivo, QC(with difference), BUPT | Xiaomi, vivo, CATT, BUPT |  |
|  |  |  | CATT(DS InH, AS) |  | Xiaomi (DS), CATT(DS) |
|  | Case 3 | Xiaomi, vivo, Intel, CATT, QC, BUPT | Vivo, QC(with difference), BUPT | Xiaomi, vivo(3-2), CATT, BUPT |  |
|  |  |  | Xiaomi (DS, AS), CATT(DS InH, AS) | Vivo (3-1) | Xiaomi (DS, AS), CATT(DS) |
|  | Case 4 | Xiaomi, vivo, Intel, CATT, QC, BUPT | Vivo, CATT, QC(with difference), BUPT | Xiaomi, vivo, CATT, BUPT | ZTE (3A)， QC(2A with difference) |
|  |  |  | Xiaomi (DS, AS), ZTE(AS) |  |  |
| NO, Alt 3 (vivo) | Case 1 | vivo | vivo | vivo | vivo |
|  |  | Xiaomi (DS/AS: small difference) | Xiaomi (DS, AS, small difference) | Xiaomi (DS, AS, small difference) | Xiaomi (DS, AS, small difference) |
|  | Case 2 |  |  |  |  |
|  |  | Xiaomi (DS: small difference, AS), Vivo (AS) | Xiaomi (DS: small difference, AS), Vivo (AS) | Xiaomi (DS: small difference, AS), Vivo (AS) | Xiaomi (DS: small difference, AS), Vivo (AS) |
|  | Case 3 |  |  |  |  |
|  |  | Xiaomi (DS: small difference, AS), Vivo (AS) | Xiaomi (DS: small difference, AS), Vivo (AS) | Xiaomi (DS: small difference, AS), Vivo (AS) | Xiaomi (DS: small difference, AS), Vivo (AS) |
|  | Case 4 | Xiaomi, vivo | vivo | Xiaomi, vivo |  |
|  |  |  | Xiaomi (DS, AS) |  |  |

Up to company to do complexity reduction: Samsung. Ericsson, OPPO

Ericsson: Option 0 is used as baseline for the discussion of channel model and channel calibration

BUPT: truncating B2 values within the range of (−σ, σ) for the log-normal distribution to avoid low-probability extreme values

QC: Htargetτ,t=i=1Ntargetsj=1Mscatter-pointsiHtar,i,jτ,t

- Issue 1: Reduced number of clusters/rays

[Moderator’s note] Diverse views regarding reduction of number of clusters per link and/or number of rays per cluster are observed. There were comments that such reduction impacts the coexistence of communication and sensing, and also hurts the spatial consistency, if different number of clusters are modeled in the Tx-target link, target-Rx link and the background channel.

Please check if the following proposal is acceptable.

- [H][FL1] Proposal 5.2-1

- The following two parameters in each of Tx-target link or target-Rx link for a scenario are kept unchanged comparing with the same scenario in existing TRs

  - The number of stochastic clusters per link

  - The number of rays per stochastic cluster

- The proposal can be revisited if spatial consistency between target channel and background channel is not modelled.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | We are OK to go for this way for simplicity even though less clusters are observed from our results. |
| Huawei, HiSilicon | No | Reducing the number of clusters is still being discussed in 7-24GHz topic. If it is to be agreed there, it should be naturally applied for each link for sensing. No need to pursue this proposal for now in ISAC CM and can wait and see the situation in 7-24GHz item. |
| Intel |  | Agree |
| LGE |  | Support |
| Samsung |  | Support |
| Nokia, NSB |  | Agree. |
| Qualcomm | Yes | We agree with this proposal, and disagree with Huawei’s reply that the above is relate to the 7-24 GHz discussion. We think these discussions are independent. |
| vivo | Yes |  |
| CMCC |  | The number of rays per stochastic clusters can be reduced. The ray number in TR 38.901 doesn’t mean there are real 20 paths of each cluster. It is used to describe the intra-cluster angle distribution pattern as Laplacian. Because the paths’ powers are equal, thus the path pattern around the center cluster angle can be plot as follows:(αm=1) If we reduce the rays to half and take a value every other sample, the offset table may be (just for example): Ray num; Basis vector of offset angles αm. 1,2; ±0.1413. 3,4; ±0.3715. 5,6; ±0.6797. 7,8; ±1.1481. 9,10; ±2.1551. The intra-cluster angle pattern is: The intra-cluster angle spread doesn’t change. Thus, we can reduce the ray number. |
| OPPO |  | Not quite understand the 2nd bullet. How does the 1st bullet fail if special consistency between target channel and background channel is not modeled? |
| SONY |  | Support the above FL’s proposal |
| Spreadtrum |  | Support |
| Xiaomi |  | Fine with the proposal. |
| Ericsson | Yes | We submit simulation results of Option 0. Please add our name in the summary part like the following for Option 0. Options for concatenation The following detailed solutions for concatenation with complexity reduction are proposed based on contribution review - Option 0: Sony, Ericsson - Option 0A: Sony - Option 1: Intel, MTK, Lekha, CT - Option 1A: Intel, MTK |
| BUPT | Yes | We think the number of stochastic clusters per link and the rays per stochastic cluster should keep unchanged. Reduction will lead to changes in statistical characteristics and affect the evaluation of spatial consistency. As demonstrated by the down-selection simulation results, the statistical characteristics of Option 2 and Option 3, after NLOS-NLOS power normalization, are close to those of Option 0, and both methods have relatively low complexity. |
| Apple |  | Please clarify the second bullet |
| Moderator | @OPPO, Apple: As analysis in Moderator note above the proposal, if we want to model spatial consistency between background channel and target channel, it is good to have same number of clusters/rays as legacy 38.901. On the other hand, if such spatial consistency is not necessary, we can do anything (should be reasonable ) for target channel. |  |

- Issue 2: Options for concatenation

[Moderator’s note] Based on the discussions in last RAN1 meeting, some companies prefer to model full convolution of the NLOS rays in Tx-target link and target-Rx link. While some other companies prefer to drop all indirect path of NLOS ray + NLOS ray for LOS condition Case 1/2/3. One potential compromise is to try a way forward in the middle. Simulation has been done by multiple companies to see if there a possible solution.

Based on the evaluations/analysis, some observations from moderators

- Option 3 achieves the best similarity (almost identical) to Option 0 especially observed by companies who submitted simulation results, with power normalization.

- Option 2 also gets some support. To be honest, some proponents just give the analysis without results.

- Option 1 also gets a support. However, it doesn’t look a compromise since the generated number of indirect paths are still large.

- One company did simulation based on power metric and conclude N+N paths can be dropped in Case 1/2/3, however, DS/AS are not analysed in such cases.

Based on the results, the moderator would suggest to go with Option 3 as the single/compromised solution for concatenation of Tx-target link and target-Rx link. Please also express your opinions on down selection of Alt 1 vs. 2, Alt A vs. B.

[H][FL1] Proposal 5.2-2

- To generate indirect paths of NLOS ray + NLOS ray in the target channel, ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

  - If number of clusters in the two links are different, e.g., N1, N2 respectively for link 1 and link 2, N2>N1, down select from the following options

    - Alt 1: max(N1, N2) clusters for each link, i.e., randomly N2-N1 clusters are repeated in link 1

    - Alt 2: min (N1, N2) clusters for each link, i.e., randomly N2-N1 clusters are dropped in link 2

- Power normalization of the target channel is performed by adjusting the power of the generated indirect paths of NLOS ray + NLOS ray, down select from the following options

  - Alt A: the scaling factor is the ratio of number of N+N paths in Option 0 and 3.

  - Alt B: the scaling factor is the ratio of total power of N+N paths in the Option 0 and 3

    - RCS value is included to calculate power of a N+N path

- The power threshold for path dropping is X<-25 dB, FFS X

- No further power normalization of target channel is not performed after path dropping,

  - Note: power normalization between target channel and background channel can be discussed separately

- FFS the set of remaining indirect paths can be updated during movement of Tx, target or Rx

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | - Comment 1: We have concern to do power normalization for NLOS+NLOS paths. As shown in the following left figure, NLOS+NLOS paths do not impact sensing performance even the number of rays is huge. However, after normalization, the power of the left NLOS+NLOS paths will be increased significantly, e.g. 400 times based on the above Alt A. Then, This is not aligned with the reality, and will negatively impact sensing performance when Rx doing sensing detection. We suggest not to power normalization. Alternatively, we are open to do full convolution and drop the paths based on the power threshold without power normalization. - Comment 2: In Alt B, RCS value is included to calculate power of a N+N paths for both option 0 and option 3, then do power normalization. In such case, the computation complexity has been very high for simulation, then what is meaningful to support option 3 compared with option 0 ? - Comment 3: In Alt1 and Alt 2, we think clusters should be paths for option 3. |
| Huawei, HiSilicon |  | We suppose all the simulations provided for this meeting to address this issue are based on angular independent RCS. We wonder whether the observation is still valid when considering angular dependent RCS. In general, we can be ok with up to company for concatenation simplification. We also hope don’t waste more time on this issue since there are more other more important issues need to be discussed. |
| Intel |  | We are fine discussing the details of the complexity reduction approaches but we also think the full convolution method should be agreed as a reference. Based on our analysis it is still unclear if the channel similarity degradation is acceptable or not. Thus, the agreement should cover both Option 0 as a reference and the Option 3 (or other) as a complexity reduction scheme. For the proposal, in case the strong clusters are generated which have sub-clusters, are the sub-clusters handled separately similar to clusters? In our understanding, those can be handled separately, in this case the number of rays for cluster/subcluster can be 4,6,10 or 20. For such coupling, the proposal also needs to handle the different number of rays per cluster in addition to the different number of clusters. Same Alt. 1 and Alt. 2 can be applied to the different number of rays. Between Alt. A and B, we think Alt. B is more general and accurate and should be prioritized. |
| LGE |  | For the 2nd bullet, we share the view with ZTE that the power normalization only for NLOS-NLOS case does not reflect the realistic channel due to arbitrary boosting of NLOS power compared to LOS power. Any NLOS clusters/rays with power smaller than a threshold should be discarded. For the 3rd bullet, instead of adjusting the path dropping threshold, applying -25dB path dropping threshold for the target channel separately from the background channel is more aligned with the existing 38.901 method. |
| Samsung |  | If it is one of the methods for evaluation, we can support. However, if it is baseline for channel model, it’s difficult to support. We still consider that complexity reduction is optimization method for evaluation, not a way to build a concrete channel model. Before calibration, it should be kept as full coupling since we don’t know whether complexity reduction method with power normalization really can replace it maintaining channel randomness. On the other hand, we have questions for power normalization. 1) We had concluded that it depends on companies’ report in offline discussion in the last meeting. However, if there is no consolidated method how to do it, it will generate other problem in evaluation or calibration step. Could you make another proposal for it? 2) We want to clarify what is the w/ and w/o power normalization. Our understanding is that: [Without normalization] - Pn' is generated based on step 6 in 7.5 of TR38.901 - Normalization is that Pn=Pn'n=1NPn' - Pn is generated for each segment such as PnTx-target and Pntarget-Rx - Based on equation (7.5-28), each path’s amplitude term (in here, path power) will be PnTx-targetM and Pntarget-RxM. - Then, concatenation is conducted and each path’s power will be PnTx-targetMPntarget-RxM. [With normalization after concatenation] - Pn' is generated based on step 6 in 7.5 of TR38.901 - Pn' is generated for each segment such as PnTx-target' and Pntarget-Rx' - Path power is PnTx-target'M and Pntarget-Rx'M, respectively - Concatenation is conducted such as PnTx-target'MPntarget-Rx'M, - Normalization is that PnTemp=PnTx-target'mPntarget-Rx'mn=1Nm=1MPnTx-target'mPntarget-Rx'm, - Then, each path’s power will be PnTemp Is it correct understanding? |
| Nokia, NSB |  | Agree with ZTE/LGE on the power normalization issue with NLOS-NLOS. If no normalization is applied, how about the difference between Option 0 and Option 3? If we cannot reach an agreement, can’t we agree with the full convolution method as the baseline approach? |
| Moderator | @LGE, ZTE, Nokia: just a reply that the power normalization is not arbitrary ☹. In full convolution method, each NLOS ray in the target-Rx link is concatenated with all NLOS rays in Tx-target link. Assuming 20 clusters in each of the TX-target and target-Rx link, the average power will be increased by 400 times compared with single N+N path. So, power normalization captures the average effect of power. On the other hand, I agree with you it has limitation, I means, it cannot reflect instant power cumulation due to phase addition, and it also cannot capture the exact difference of delay or Doppler for the different NLOS rays in the Tx-target link. However, from the evaluation from multiple sources, the difference on DS between Option 0/3 can be marginal. @Samsung: Your formulas are correct if RCS is fixed to 1 and if same number of clusters in Tx-target and target-Rx link. Otherwise, some updates are necessary @all: Considering the concern received for the proposal. No company challenges the correctness of full convolution, while most/all companies may have a concern on complexity of full convolution. One possibility is to support both schemes of full convolution and complexity reduction. Given the simulation results submitted to this meeting, it seems Option 3 can be recommended one if it is not the single choice for complexity reduction. Please check the following updated proposal |  |

[H][FL1] Proposal 5.2-2-rev1

- Option 0 is supported to generate indirect paths of NLOS ray + NLOS ray in the target channel, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

- Complexity reduction is allowed by reducing the number of the generated indirect paths of NLOS ray + NLOS ray. A recommended scheme for complexity reduction is Option 3, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

    - If number of clusters in the two links are different, e.g., N1, N2 respectively for link 1 and link 2, N2>N1, down select from the following options

      - Alt 1: max(N1, N2) clusters for each link, i.e., randomly N2-N1 clusters are repeated in link 1

      - Alt 2: min (N1, N2) clusters for each link, i.e., randomly N2-N1 clusters are dropped in link 2

  - Power normalization of the target channel is performed by adjusting the power of the generated indirect paths of NLOS ray + NLOS ray, down select from the following options

    - Alt A: the scaling factor is the ratio of number of N+N paths in Option 0 and 3.

    - Alt B: the scaling factor is the ratio of total power of N+N paths in the Option 0 and 3

  - RCS value is included to calculate power of a N+N path

  - The power threshold for path dropping is X<-25 dB, FFS X

  - No further power normalization of target channel is not performed after path dropping,

    - Note: power normalization between target channel and background channel can be discussed separately

  - FFS the set of remaining indirect paths can be updated during movement of Tx, target or Rx

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | We have concern on Option 3, especially for angle-dependent RCS. In our simulation with angle-dependent RCS, a large number of NLOS+NLOS paths have low gain, while a small number of them have high gain. Random ray pairing will mean a lot of weak combinations are kept but some significant combinations are dropped. If generated paths are subject to dropping based on power, we need to make sure at least the strong paths are generated. With Option 0 or 1, one or more paths in the strongest cluster combination are generated. But random ray pairing in Option 3 doesn’t make sure one of the generated paths comes from the strongest cluster combination. Option 0 used together with threshold for path dropping, e.g., -20dB in the figure, can remove a lot of weak paths, lowering complexity. Specific options can be up to implementation. Companies can make sure the statistics of the channel remain the same with complexity reduction implemented, just like companies’ simulations for this meeting. Some other comments to the revised proposal: Option 0 + path dropping based on power threshold is an option of complexity reduction. The current wording of [H][FL1] Proposal 5.2-2-rev1 seems to imply that Option 0 is not complexity reduction. For Power threshold for path dropping, what is the baseline path to compare with? We suggest an absolute power threshold. The last three sub-bullets are about dropping some of the generated paths and apply to Option 0 too. They should have a smaller indent. We reuse the format of the agreement in RAN1#118b and have the wording suggestion below. - On other indirect paths of NLOS + NLOS - Option 0 is supported to generate indirect paths of NLOS ray + NLOS ray in the target channel, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Complexity reduction is additionally allowed by reducing the number of the generated indirect paths of NLOS ray + NLOS ray. […] - The power threshold for path dropping is X<-25 dB or Y an absolute threshold (dBm), FFS X - No further power normalization of target channel is not performed after path dropping, - Note: power normalization between target channel and background channel can be discussed separately - FFS the set of remaining indirect paths can be updated during movement of Tx, target or Rx |
| ZTE |  | For FL’s reply on ‘Assuming 20 clusters in each of the TX-target and target-Rx link, the average power will be increased by 400 times compared with single N+N path’, I am confused why average power per path is increased by 400 times for full convolution. Again, in Alt B, RCS value is included to calculate power of a N+N paths for both option 0 and option 3, then do power normalization. In such case, the computation complexity has been very high for simulation, the benefit of opiton 3 will be gone. Whether X <=-25dB should be decided first. For compatibility with communication, we suggest reusing -25. In such case, Case 3A without normalization may be the best case. |
| CATT/CICTCI |  | Generally OK, but there is no need to require option 0 to be mandatary. Companies can choose complexity reduction method in the simulation. |
| Qualcomm | No | We didn’t get a chance to reply in the previous round, as maybe several companies. We still don’t consider Option 0 a baseline; It is shown in multiple papers that there is no difference in the statistics compared to the simplified Options Furthermore, with regards to the details of Option 3, we would like to add an Alt. 3, and also clarify, that all the alternatives need to talk about “rays” and not “clusters” since this is Option 3 (1-to-1 mapping of rays): - If number of rays in the two links are different, e.g., N1, N2 respectively for link 1 and link 2, down select from the following options - Alt 1: max(N1, N2) rays for each link, i.e., randomly N2-N1 rays are repeated in link 1 - Alt 2: min (N1, N2) rays for each link, i.e., randomly N2-N1 rays are dropped in link 2 Alt. 3: If N1<N2 randomly N2-N1 rays are dropped in link 2, otherwise randomly N2-N1 rays are repeated in link 1 |
| vivo | Yes | In the second bullet, “RCS value is included to calculate power of a N+N path” should be modified as RCS component B value is included to calculate power of a N+N path. Another modification (either remove No or not may be needed as: No further power normalization of target channel is not performed after path dropping, |
| CMCC |  | - In the first down selection we prefer Alt 2: min (N1, N2) clusters for each link, i.e., randomly N2-N1 clusters are dropped in link 2. - In the second down selection we prefer Alt B: the scaling factor is the ratio of total power of N+N paths in the Option 0 and 3. But we don’t agree “RCS value is included to calculate power of a N+N path” when conducting the normalization. The concatenated clusters and rays are for each TX-target and RX-target. The power normalization here is also for making the summation of relative power to be 1. The RCS is an addition part. If the RCS is contained in the normalization here, there will not be component A of RCS can be extracted to the large-scale fading. - The statement “No further power normalization of target channel is not performed after path dropping” may change to “No further power normalization of target channel is performed after path dropping”. |
| OPPO | No | First, the power normalization step is a trouble maker in our view. Assume Tx-target link is in LOS condition. Then, each NLOS ray (called ray c) on Target-Rx link is concatenated with the LOS ray (called ray a) and a NLOS ray (called ray b) on Tx-Target link, as shown in figure below. It can be mathematically proved that Powerb+cPowera+c=ρ∙pbM∙KR, where ρ is power normalization coefficient, pb is the cluster power of the cluster containing ray b, M is the number of rays per cluster, KR is KR factor on Tx-Target link. Assume further M=20, KR=μKR-σKR=-1dB in InF scenario and ρ follows Alt-B, the CDF of Powera+cPowerb+c and CDF for concatenated ray powers are given in figures below. Note that the simulation assumes angle-independent RCS=1. # Case 1 - LOS+LOS # Case 2 - LOS+NLOS It can be seen that, due to the proposed power normalization, - Prob{Powera+c < Powerb+c }≈40%, which means a large percentage of NLOS rays on Target-Rx link would observe its NLOS+NLOS ray propagation concatenation being stronger than its LOS+NLOS propagation concatenation. This is somehow against the common sense: the shorter the propagation delay, the stronger the propagation power. - Power normalization shifts the ray-power CDF right-ward by a large amount. Take Alt-A as example, the shift is 10*log10(400) =26dB between Option 3 and Option 0. Similar amount of shift is observed from above figures for Alt-B. So if people consider Option 0 is the true reflection of what it should be in reality, Option 3 has the ray-level power CDF deviating artificially too much. However, if power normalization is not performed, a large amount of ray powers are dropped, which introduces modeling error for both sensing evaluation and communication evaluation. In summary, we see it is problematic for both doing power normalization and not doing power normalization as proposed. So we think Option 3 should not be adopted. |
| SONY |  | We support to conclude the option 0 as the baseline. As for the complexity reduction, we are open for each option to make progress on this issue. |
| Xiaomi |  | We are generally fine with the proposal with some modification. In our simulation result, the delay spread and angle spread of option 3 is nearly same as option 0 when power normalization is considered and can be used for complexity reduction. For Alt 1 and Alt 2, the simulation results for option 3 are similar, and Alt 2 is preferred for more complexity reduction. As mentioned by ZTE, path should be used instead of clusters. For Alt A and Alt B, Alt. B is preferred since it is more accurate. |
| InterDigital |  | For complexity reduction, we prefer Alt. 2. We have a concern with Alt. 1 which artificially inflates the number of clusters. |
| BUPT | Yes | Alt2 ensures good simulation results while maintaining lower complexity. AltB normalizes by the power ratio, which guarantees overall target channel power normalization. On the other hand, AltA uses the quantity ratio, which may lead to a lack of overall power normalization. |
| Apple | No | Need to understand why Option 3 vs Option 2 decided |
| Moderator | @all: based on curves from ZTE and OPPO, it seems we cannot apply power normalization. On the other hand, scheme w/o power normalization results in difference compared to Option 0. Note: above comments assume no path dropping. Then, if path dropping is considered, and if we have the freedom to adjust the power threshold, then it is possible we reduce the difference between Option 0 and other options. Note: my understanding is that proponent of Option 0 would also like to apply path dropping which provide some “limited” complexity reduction, since we only need to work on the remaining paths in steps after path dropping. Note: since it is now assumed no power normalization, I add Option 4 based on one proposal from Huawei, i.e., |  |

[H][FL2] Proposal 5.2-2-rev2

- To generate indirect paths of NLOS ray + NLOS ray in the target channel

  - Option 0 is supported, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Solution(s) that generate a reduced number of indirect paths of NLOS ray + NLOS ray is/are supported. The following option(s) is/are recommended

    - Option 3: ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

    - If number of clusters in the two links are different, e.g., N1, N2 respectively for link 1 and link 2, N2>N1, down select from the following options

      - Alt 1: max(N1, N2) clusters for each link, i.e., randomly N2-N1 clusters are repeated in link 1

      - Alt 2: min (N1, N2) clusters for each link, i.e., randomly N2-N1 clusters are dropped in link 2

      - Alt 3: If N1<N2 randomly N2-N1 rays are dropped in link 2, otherwise randomly N2-N1 rays are dropped repeated in link 1

    - Power normalization of the target channel is NOT performed

- The power threshold for path dropping is X<-25 dB, FFS X

- No further power normalization of target channel is performed after path dropping,

  - Note: power normalization between target channel and background channel can be discussed separately

- FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

[Moderator’s note] After Tuesday offline session

[H][FL2] Proposal 5.2-2-rev3

- To generate indirect paths of NLOS ray + NLOS ray in the target channel

  - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

Supported by: vivo, Ericsson, OPPO, Samsung, Nokia, InterDigital, Sony

  - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

    - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2,

      - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1

    - FFS need for power normalization

Supported by: Xiaomi, ZTE, CMCC, CATT, LGE, QC, Apple, vivo (normalization after path dropping), InterDigital

  - Other methods are up to company choice for complexity reduction

  - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration

- The power threshold for path dropping is X=[-25] dB

  - X is relative to the strongest direct/indirect path in the target channel

- FFS: further power normalization of target channel is performed after path dropping,

  - Note: power normalization between target channel and background channel can be discussed separately

- FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

[Moderator’s note] agreement in Wednesday online session

- Agreement

- To generate indirect paths of NLOS ray + NLOS ray in the target channel

  - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

    - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2,

      - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1

  - Other methods are up to company choice for complexity reduction

  - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration

- The power threshold for path dropping is X=[-25] dB

  - X is relative to the strongest indirect path in the target channel

- FFS: further power normalization of target channel is performed after path dropping,

- Note: power normalization when target channel and background channel are combined can be discussed separately

- FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

- Issue 3: upper/lower bounds of RCS

[Moderator’s note] As discussed in R1-2409992, a generated log-normal B2 value may be occasionally very large. It seems reasonable to set a upper bound for the RCS value. Therefore, please check if the following proposal is agreeable. If the issue is considered valid, please also indicate your preference to Option 1/2 or any variations

[M][FL1] Proposal 5.2-3

To avoid extremely large RCS values, down select from the following options

- Option 1: An upper bound RCSmax for RCS (=A*B1*B2) value is introduced, i.e., the RCS value for a pair of incident/scattered angle at a scattering point of a target is modeled as min(A*B1*B2, RCSmax)

- Option 2: Truncating B2 values within the range of (−σ, σ) for the log-normal distribution

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | Option 2 |
| Huawei, HiSilicon |  | In general, we should look at the RCS of target first and especially the variance that is to be modelled which seems relevant to this proposal. This proposal currently is more like how to handle something specially in evaluation rather than the modelling itself. |
| LGE |  | The max. RCS value should be determined based on the size of the object. In this sense, option 2 is more reasonable. But we’re ok with option 1 if RCSmax is determined based on the object size. |
| Nokia, NSB |  | Option 1. For Option 2, we may select B2 in the range of (-inf, σ |
| Ericsson | Yes |  |
| vivo |  | This may be not needed. |
| CMCC |  | Option2 |
| SONY | Yes |  |
| Xiaomi | Yes | Prefer Option2. |
| InterDigital | Yes | Option 2 |
| BUPT | Yes | Option 1： selects the maximum RCS value as the boundary, which aligns with physical significance. The maximum value needs to be provided by each company based on actual measurements. [R1-2409992] Option 2： on the other hand, derives the boundary based on the statistical probability distribution function, directly extending the values fitted to the existing B2 distribution. The specific value is open for discussion; for a normal distribution, a 3σ truncation value typically covers 99.7% of the probability range. [R1-2409992] |
| Moderator | Seems we could go direction of Option 2 |  |

[M][FL2] Proposal 5.2-3-rev1

To avoid extremely large RCS values, component B2 of RCS is truncated within the range of (−inf, kσ) for the log-normal distribution. FFS k

[Moderator’s note] further revised by offline

- [M][FL4] Proposal 5.2-3-rev2

To avoid extremely large RCS values, component B2 of RCS is upper bounded by kσ dB for the log-normal distribution. FFS k as scaling factor

Delay/power of direct/indirect path

Delay/power in Tx-target or target-Rx link

- Agreement In LOS condition between sensing Tx/Rx and target, the power of LOS ray is generated following power of LOS ray in TR 38.901.

Summary on company views

Scaling delay spread in each Tx-target/target-Rx link

- Factor 1/2 applies to τn1'=12∙-rτ∙DS∙lnXn1: vivo

  - Not needed: Xiaomi

Scaling shadowing term in cluster power calculation in each Tx-target/target-Rx link

- Factor 1/2 applies to shadowing,  zn1'N0,ξ22: vivo

  - Not needed: Xiaomi

Power of LOS/NLOS ray

- Pathloss of Tx-target and target-Rx link follows existing TRs: QC, Xiaomi, Samsung, CATT, vivo

- NLOS cluster power follows Step 6 of 38.901: Nokia, Xiaomi, Lekha

- separate K factor for Tx-target link and target-Rx link: vivo, Xiaomi, Samsung

- [Moderator’s note]

No matter which option is adopted for LOS ray, it seems the power of NLOS clusters can only be modelled relative to the LOS ray by random Ricean factor KR, since NLOS clusters have to be stochastically generated.

For EO type-2, it is straightforward to model its power by factor ‘b’ from agreement in RAN1#118 and the power of associated LOS ray

- [H][FL1] Proposal 5.3-1

- When the stochastic clusters are used to generate the indirect paths in the target channel of a target

  - Step 5 in section 7.5, 38.901 is reused to generate the delay for each NLOS cluster.

  - Step 6 in section 7.5, 38.901 is reused to generate the power for each NLOS cluster.

- The power of NLOS ray generated by EO type-2 in a Tx-target or target-Rx link is modeled by factor ‘b’ from agreement in RAN1#118 and the power of associated LOS ray

- Note: the polarization matrix of EO type-2 may also impact the specular reflected power

- FFS if LOS ray is not present

| Company | Option 1 or 2 | Comments |
| --- | --- | --- |
| ZTE |  | Support the FL proposal |
| Huawei, HiSilcon |  | We have agreed on the following proposal: Agreement In order to generate each of the Tx-target link and target-Rx link in the target channel, the large scale and small scale parameters defined in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, TR 38.858, TR 38.859, TR 38.802, TR 38.854, etc. are used as starting point With that, the only valuable point of this proposal is about the pathloss of the reflected path for EO type-2. If that’s the case, then the proposal should be revised for that particular purpose. The FFS can be addressed that despite the LOS/NLOS condition, the pathloss of the reflected path for EO type-2 should be based on the LOS ray of the link. |
| Intel |  | Suggest scoping the proposal to the discussion on the target channel generation with stochastic clusters. There is another discussion on generation of background channel for monostatic cases which may be different from current 38.901. - [H][FL1] Proposal 5.3-1 When the stochastic clusters are used to generate the indirect paths in the target channel of a target - Step 5 in section 7.5, 38.901 is reused to generate the delay for each NLOS cluster. - Step 6 in section 7.5, 38.901 is reused to generate the power for each NLOS cluster. The power of NLOS ray generated by EO type-2 in a Tx-target or target-Rx link is modeled by factor ‘b’ from agreement in RAN1#118 and the power of associated LOS ray - Note: the polarization matrix of EO type-2 may also impact the specular reflected power - FFS if LOS ray is not present |
| LGE |  | We’re ok with Intel’s modification. |
| Samsung |  | Support |
| Nokia, NSB |  | Okay with the proposal. |
| Moderator | Thanks Intel. Revised as proposed |  |
| EURECOM |  | Support |
| Ericsson |  | Both the target channel and the background channel need to have absolute delays rather than relative delays. At least for monostatic sensing. Hence the optional procedure in section 7.6.9 in TR 38.901 should be included in the proposal. |
| CATT/CICTCI | OK |  |
| Qualcomm |  | OK |
| vivo |  | We disagree with the proposal especially for Step 6. It is the evidence that we clearly show in our simulation results. Without the modification in power, DS could increase significantly (see Table 5 in our contribution R1-2409693), that makes the channel more dispersive. We suggest making the decision by relying on the simulation results. |
| OPPO |  | There is one thing in 3rd bullet (for EO2) that should be considered whether to keep the same logic as in current 38.901: the power normalization. In current 38.901, the ground reflection ray does not participate the cluster/ray power normalization in small-scale, at least the TR reads like it does not. This is one of the reasons that the ground reflection is just a part of “additional modeling features” instead of a core feature of the modeling. This ground reflection power certainly would impact the total power reaching the destination. So we think it should be discussed in RAN1 whether the EO-2 path on each of Tx-target link and Target-Rx link. |
| SONY | OK |  |
| Spreadtrum |  | Support |
| Xiaomi |  | Fine with the proposal. |
| Moderator | @vivo: Xiaomi also has results to show there is no much impact by not having the proposed scaling factor 1/2. Further, as discussed in the offline, the absolute distance of Tx-target link and target-Rx link can be even longer than the Tx-Rx link, so it is not proper to use 1/2 as scattering factor. Since clearly no other company support vivo proposal, it would be high appreciated if you could accept the proposal for progress. Thanks. @OPPO, we are discussing your concerned issue. From the replies, there is no support to additionally model impact of EO type-2 to pathloss of target channel. Whether/how EO type-2 impacts background channel is a separate discussion. Please continue discussions |  |

Concatenated pathloss formula

Summary on company views

Pathloss of the target channel

- PLtarget,dB=PLd1+PLd2+10lgc24πf2-10lgσRCS,A

  - Supported by vivo, Nokia, QC, Xiaomi, EURECOM, CATT, AT&T, CAICT, Apple, MTK, IDC, Lenovo, Tiami, Sony

- If a dielectric blocker to block the LoS path: PL2hop(d1,d2)=PL(d1+d2)+L

    - Supported by QC

Ericsson: Mean RCS should not be used in the equation to calculate LTX-ST-RX

Ericsson: For each of Tx-target link and target-Rx link, a large-scale pathloss is calculated by reusing step 3 in TR 38.901 and applied before the two links are concatenated.

Shadowing

existing shadow fading model in TR 38.901: Apple, DOCOMO

Separate shadow fading for each link, σTx-tar-Rx = σ1+σ2: Ericsson, DOCOMO

σTx-tar-Rx = max⁡σ1,σ2: DOCOMO

- [Moderator’s note]

Based on working assumption that RCS=A*B with component A included in large scale, the concatenated pathloss PLTX-ST-RX for the target channel (Tx-target link + target-Rx link) can be determined. The concatenated pathloss, together with shadowing is then used to scale the channel coefficients of direct/indirect paths to form the final fast fading channel for ISAC, e.g., 
Htarget,u,s=1PLTX-ST-RXSF∙Hu,sτ,t

On the other hand, there is a view that such concatenated pathloss is not necessarily captured in the CR to 38.901 explicitly. Instead, pathloss for Tx-target link and target-Rx link can be respectively used to scale the channel coefficients of LOS ray and NLOS clusters in the two links. Then, the pathloss scaled channel coefficients of the two links are concatenated to form the final fast fading channel for ISAC. Effectively, this option doesn’t introduce concatenated pathloss PLTX-ST-RX. Instead, respective PLd1, PLd2, c24πf2 and RCS component A are used to scale the channel coefficient of the direct/indirect paths. 
Htarget,u,s=1PLd1PLd2∙4πf2c2∙σRCS,ASF∙Hu,sτ,t

The above two methods are equivalent in mathematics. Either way could work from the moderator’s understanding. Given 14 companies proposed or validated the correctness of concatenated pathloss formula, the moderator makes the following proposal.

- Impact of shadow fading are added in the formula, with 3 options to generate SF. Please express your view on the preferred option.

[H][FL1] Proposal 5.3-2

- When indirect path in the target channel of a target is modelled by at least stochastic clutter(s), the large-scale scaling factor LTX-ST-RX of the target channel is determined by,

LTX-ST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB

Where,

    - PLd1is pathloss between Tx and target, where d1 is the distance between Tx and target

    - PLd2 is pathloss between Rx and target, where d2 is the distance between target and Rx

    - σRCS,A is RCS component A of the target

    - SFdB is the shadow fading of the target channel., down select from

      - Option 1: SFdB=SFdB,1+SFdB,2, where, SFdB,1,SFdB,2 are respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

      - Option 2: SFdB=maxSFdB,1,SFdB,2, where, SFdB,1,SFdB,2 are respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

      - Option 3: SFdB is generated for the target channel referring to step 4 in section 7.5, TR 38.901 as starting point

- The presence or not of EO type-2 in the target channel doesn’t change the determined large-scale scaling factor LTX-ST-RX

- LTX-ST-RX is used to scale the generated channel coefficient of the target channel

  - FFS impact of power normalization if supported between target channel and background channel

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon | No | We firstly agreed on that working assumption that RCS consists of A and B in large scale and small scale respectively. The agreeableness was based on such large-scale scaling factor is needed, so without A, the unit of this L will not be dB which is not correct, and then A=1 is proposed based on the fact that there is anyway a case RCS is angular dependent. However, we realized that this L is NOT needed at all in the first place, so we don’t need component A at all either. That working assumption should be reverted, and all the RCS discussion should have happened earlier to directly discuss the RCS itself for each sensing target, instead of playing A, B1, B2, etc…without mentioning the target at all. The discussion direction should be corrected as soon as possible! |
| LGE |  | We’re ok with the proposal. Among the options, we prefer option 1. |
|  |  | Support in general. For shadow fading, we prefer to Option 1. Shadow fading can be +/- value. It will be a natural way. However, for EO type-2, we have concern on no changing the large-scale factor. The existence of EO type-2 can impact to large scale parameters, e.g., pathloss due to LOS condition changes. EO type-2 is too big to simply ignore it. |
| Nokia, NSB |  | Technically we are okay with the equation. However, we would prefer to use original “pathloss” terminology, instead of “large scale scaling factor”. For shadow, we also prefer option 1. |
| Moderator | @Samsung: for EO type-2, it actually follows the principle of 7.6.8, i.e., the modeling of an additional ground reflection doesn’t change the calculated pathloss from Tx to Rx. |  |
| EURECOM | Yes |  |
| Ericsson | No | Same feeling with HW. We are afraid that we are creating unnecessary troubles for us. The purpose of this proposal and the proposals on RCS A, B1 and CPM is to generate the concatenated channel coefficients with power of 1 and then apply the scaling factor after two links are concatenated. This requires us to extract power-related values, e.g. component A, and normalize B1, B2, and CPM. (Note that we don’t agree with [H[FL1] Proposal 5.6-3, power normalization of CPM tx_sp_rx.) All these troubles can be avoided by applying corresponding pathloss and fading to each link before concatenating the two links with RCS and CPM, which is well summarized by FL, and make a clean specification. If RCS measurements follow the log-normal distribution of μ and σ2, FL has clarified how to calculate A and B1, copied below. Companies can calculate A and the proposed new scaling factor by implementation. We do not see the need for standardizing the scaling factor and complicating things, especially given limited time. In summary, for a log-normal distribution characterized by a Gaussian distribution with mean μ and variance σ2, we have the following relation - The linear mean value of the log-normal distribution is A=expln1010u+ln102200σ2 - B1=1. Alternatively, if B1 is angular dependent, A*B1 may be viewed together as mean in a specific incident/scattered angles - Mean of B2 in dB scale is μB2=-ln1020σ2 - Variance of B2 in dB scale is still σB22=σ2. Moreover, for angle-dependent RCS, mean value covers all 360 azimuth and 180 zenith, which is much larger than the angular range of incident rays and scattering rays of a target. A parameter based on such mean value does not represent the real target channel. |
| CATT/CICTCI | OK |  |
| Qualcomm |  | We are generally supportive, but also open to consider the other option that FL is suggesting and seems Huawei is also pointing out in their response. We ll value offline discussion on this. |
| vivo | Yes |  |
| CMCC |  | We need more measurement results to verify these three options of SF. |
| OPPO |  | Firstly, due to RCS component A being used as mean RCS, the small-scale part does not sum to unit power and therefore the large-scale part can only be called “large-scale scaling factor” instead of “large-scale fading”. But with this new terminology, we wonder where in the ISAC channel model could use this new term of “large-scale scaling factor” by itself (i.e., without looking at the small-scale construction). Please clarify. If there is no other places in ISAC channel model would use this term, why should RAN1 bother to model it? Secondly, for shadow fading part, we prefer to option 1. Shadow fading is introduced in TR 38.901 to add statistic variation to the deterministic pathloss formula. It can be considered as a correction to the pathloss for a specific deployment environment. So it is necessary to add shadow fading to associate with each pathloss being generated. |
| SONY |  | We are generally OK with this proposal and support option1 for shadow fading |
| Spreadtrum | Yes |  |
| Xiaomi | Yes |  |
| InterDigital | Yes |  |
| Toyota | Yes | We are OK with the proposal. For shadowing, we prefer Option 1. |
| BUPT |  | The issue of path loss concatenation has been well demonstrated in measurements [R1-2402679] in #116bis, and further measurements are needed to validate the down-selection of SF parameters. |
| Apple | Yes | Option 1 |
| Moderator | @all: Now, only Option 1 for shadow fading is kept. |  |

[H][FL2] Proposal 5.3-2-rev1

- When indirect path in the target channel of a target is modelled by at least stochastic clutter(s), the large-scale scaling factor LTX-ST-RX of the target channel is determined by,

LTX-ST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB

Where,

    - PLd1is pathloss between Tx and target, where d1 is the distance between Tx and target

    - PLd2 is pathloss between Rx and target, where d2 is the distance between target and Rx

    - σRCS,A is RCS component A of the target

    - SFdB is the shadow fading of the target channel, down select from

      - Option 1: SFdB=SFdB,1+SFdB,2, where, SFdB,1,SFdB,2 are respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

      - Option 2: SFdB=maxSFdB,1,SFdB,2, where, SFdB,1,SFdB,2 are respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

      - Option 3: SFdB is generated for the target channel referring to step 4 in section 7.5, TR 38.901 as starting point

- The presence or not of EO type-2 in the target channel doesn’t change the determined large-scale scaling factor LTX-ST-RX

- LTX-ST-RX is used to scale the generated channel coefficient of the target channel

  - FFS impact of power normalization if supported between target channel and background channel

[Moderator’s note] revised in Wednesday online session

- [H][FL4] Proposal 5.3-2-rev2

- When indirect path in the target channel of a target is modelled by at least stochastic clutter(s), the large-scale scaling factor LTX-ST-RX of the target channel is determined by,

LTX-ST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgA+SFdB

Where,

    - PLd1is pathloss between Tx and target, where d1 is the distance between Tx and target

    - PLd2 is pathloss between Rx and target, where d2 is the distance between target and Rx

    - A is RCS component A of the target

    - SFdB is the shadow fading of the target channel

      - SFdB=SFdB,1+SFdB,2, where, SFdB,1,SFdB,2 are respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

- The presence or not of EO type-2 in the target channel doesn’t change the determined large-scale scaling factor LTX-ST-RX

- LTX-ST-RX is used to scale the generated channel coefficient of the target channel

  - FFS impact of power normalization if supported between target channel and background channel

Doppler

Summary on company views

Ericsson, Nokia, SPRD, vivo: Dual mobility in 7.6.10, 38.901 as start point to model Doppler for indirect path

- especially p, should be studied via measurements in selected scenarios: Ericsson

Samsung: RAN1 to clarify the scope of discussion regarding the mobility of stochastic clutters

Micro Doppler

- NO: EURECOM, Sony (lower than macro Dopller)

- QC, IDC propose to model micro-doppler in the ISAC channel model.

- OPPO, vivo, Ericsson, ZTE proposes to add a placeholder for micro-doppler for forward compatibility

- Dual connectivity in 7.6.10: Nokia, ZTE, vivo

Full formula

- 0trrx,n,mTτvrxτ-vST,n,mτdτλ0+0trtx,n,mTτvtxτ-vST,n,mτdτλ0 + 2αn,mDn,mλ0t: ZTE

- 1λrrxTt∙vrxt+rtxTt∙vtxt+1λrrx'Tt+rtx'Tt∙vmatMacro-Doppler+1λrrx'Tt+rtx'Tt∙vmitMicro-Doppler-fD,othert: vivo

Simplified formula: if the simulation duration time is short compared to the moving speed of target, Tx and Rx, i.e. the geometry relationship of target, Tx and Rx do not change significantly, the rrx,n,mτ and  rtx,n,mτ are approximated as rrx,n,mand rtx,n,m,

- rrx,n,mT∙vrx-vST+rtx,n,mT∙vtx-vSTλ0: Huawei

- rrx,n,mT∙vrx-vST+rtx,n,mT∙vtx-vSTλ0 + 2αn,mDn,mλ0: ZTE, Sony, EURECOM

- vDoppler=rα,txT⋅vtx+rα,rxT∙vλ0+rβ,txT∙v+rβ,rxT⋅vrxλ0+ft: SPRD

- expj2πrrx,n',m'Tτvrx-vmacroλ0texpj2πrtx,n,mTτvtx-vmacroλ0texp-j2π0trrx,n',m'Tτvmicroτdτλ0exp-j2π0trtx,n,mTτvmicroτdτλ0: QC

- exp-j4πxt∙rRxλ+ϕexp-j4πxt∙rTxλ+ϕ: IDC

- rtx,n,mT.vtx+rtarget,arrival,n,mT.vtarget+rrx,n',mT.vrx+rtarget,departure,n',mT.vtarget: Xiaomi

Function for micro motion/speed

- Vivo, BUPT: for human arm/leg vibration, fDt=2λωvDvsinωvtcosϕ'cosθ

- OPPO: Rel-19 ISAC channel model may provide informational examples of specific micro-Doppler functions for interested sensing targets

- IDC: as part of R19 ISAC channel modelling

- ZTE: to be discussed in performance evaluation stage

- [Moderator’s notes]

Macro-Doppler is already agreed in the early agreement on direct/indirect path generation. However, it is doubtable whether micro-Doppler is in the scope of the current study item. Multiple companies discuss its advantage using micro-Doppler in the differentiation of target from unintended object. In fact, referring to scenario discussion in 9.7.1, velocity of UAV can be 0, it motivates modelling micro-Doppler. On the other hand, the exact patterns for micro-Doppler may be different for different use cases, scenarios and target types. Based on company proposals, a placeholder for micro-Doppler can be included in the ISAC channel model. The exact function for micro-Doppler can be discussed in remaining time in Rel-19 or inputted in the solution evaluation stage.

[H][FL1] Proposal 5.4-1

- Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula,

rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft

Where,

    - rrx,n',m'T is the spherical unit vector at receiver for the link from the scattering point to Rx

    - rtx,n,mT  is the spherical unit vector at transmitter for the link from Tx to the scattering point

    - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx

    - rp,n,mT is the spherical unit vector at the scattering point for the link from Tx to the scattering point

  - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

    - ft is only applicable for indirect path

    - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray

    - Support two terms of  2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray

  - Doppler is separately determined for each of the multiple scattering points of a target

  - vspt can include macro-Doppler and/or micro-Doppler motion, vsptvmacrot+vmicro,pt, where, vmacrot is the velocity of the target, vmicro,pt is velocity due to micro motion of a scattering point of the target

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | - rrx,n',m'T should be the spherical unit vector at receiver for the link from Rx to the scattering point - We have concern on the wording of ‘due to micro motion of a scattering point of the target’ for now. Some micro-Doppler patterns were agreed in ETSI ISAC ISG, those patterns were per target rather than per shattering point. For example, for human vibration, single micro-Doppler motion is sufficient for all parts of human. Hence, we suggest to delete the red part in the FL summary. The details can be further studied. |
| Huawei, HiSilicon | No | This one should be low priority. We expect this proposal will NOT be discussed again before more progress on RCS for other sensing targets. |
| LGE |  | The role of f(t) is not clear. We suggest to remove that bullet. We’re ok with just making a placeholder for the micro-Doppler, not any more. Suggest to remove the details on the micro motion on the last bullet. We share the view with Huawei/HiSillicon. |
| Nokia, NSB |  | The first part of the equation (the part without f(t)) should be agreeable. |
| Moderator | @ZTE: the wording on rrx,n',m'T should be fine since it is referring to the link, i.e., sp to Rx @LGE: f(t) is for the Doppler effect of movement of clusters, which is referred to 7.6.10 to define f(t). In summary, it may be just referring a component in current 38.901 @ZTE, LGE: deleted red part in last sub-bullet |  |
| Ericsson |  | Moving scatterers have more profound impacts on sensing than on communication, since doppler frequency of an object is an important indicator for detection and tracking. We suggest performing measurements and studying the parameters. The following FFS can be added. FFS: parameters v_scatt and p in dual mobility model in 7.6.10. Also, if dual mobility is enabled in the target channel, it should also be enabled in the background channel for the sake of consistency. |
| ZTE |  | Sorry for the further clarification questions, here is my understanding if we only consider horizontal domain angle along with X axis: rp,n,mT = - rtx,n,mT , where rtx,n,mT is vector from Tx to ST rp,n',m'T = - rrx,n',m'T , where rrx,n',m'T is vector from ST to RX Hence, the FL proposal is equivalent to rrx,n',m'Tt.[ vrxt-vspt]+rtx,n,mTt.[vtxt-vspt]λ0 For mono-static mode, since Tx and Rx have the same location, we have rrx,n',m'T= - rtx,n,mT rtx,n,mTt.[- vrxt+vspt]+rtx,n,mTt.[vtxt-vspt]λ0 When Tx and Rx have zero speed, and only ST is moving, the Doppler-effect become zero. That’s why I think the rrx,n',m'T is vector from RX to ST. |
| Qualcomm |  | Support in general; need to check further the directions of these vectors, as ZTE is pointing out. |
| vivo | Yes |  |
| OPPO |  | If f(t) is intended to cover dual-mobility only, it should not be a time-varying term. We can use fdualmob instead of f(t). |
| SONY |  | Agree with ZTE the rrx,n',m'T refer to the direction spherical unit vector from Rx to target |
| Xiaomi | Yes |  |
| InterDigital |  | Although our preference is to capture a model for vmicro,pt, current wording with the FL’s latest modification is fine as a compromise. |
| Lenovo | Yes |  |
| BUPT | Yes | From the field measurements, we have observed that the Micro-Doppler effect may need to be considered in ISAC channel modeling. The formula fDt=2λωvDvsinωvt can serve as a foundational starting point for modeling the Micro-Doppler effect. |
| Moderator | @all: reflect ZTE’s comments |  |

[H][FL2] Proposal 5.4-1-rev1

- Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula,

rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft

Where,

    - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point to Rx

    - rtx,n,mT  is the spherical unit vector at transmitter for the link from Tx to the scattering point

    - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx

    - rp,n,mT is the spherical unit vector at the scattering point for the link from Tx to the scattering point to Tx

  - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

    - ft is only applicable for indirect path

    - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray

    - Support two terms of  2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray

  - Doppler is separately determined for each of the multiple scattering points of a target

  - vspt can include macro-Doppler and/or micro-Doppler motion, vsptvmacrot+vmicro,pt, where, vmacrot is the velocity of the target, vmicro,pt is velocity due to micro motion of a scattering point of the target

[Moderator’s note] agreement in Wednesday online session

- Agreement

- Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula,

rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft

Where,

    - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point

    - rtx,n,mT  is the spherical unit vector at transmitter for the link from Tx to the scattering point

    - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx

    - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx

  - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

    - ft is only applicable for indirect path

    - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray

    - Support two terms of  2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray

  - Doppler is separately determined for each of the multiple scattering points of a target

  - vspt can include macro-Doppler and/or micro-Doppler motion, vspt=vmacrot+vmicro,pt,

  - FFS: maximum speed of moving scatterers

  - FFS: ratio of moving scatterers among all scatterers

Target with multiple scattering points

Summary on company views

Separate handling each scattering points

- Each scattering point is separately handled as if a target with single scattering point: Huawei, QC, Lenovo?, Samsung, vivo, EURECOM, NVIDIA

  - The direct path generation for the target with single scattering point applies to each scattering point: QC

  - The indirect path generation for the target with single scattering point applies to each scattering point

Lenovo: correction of paths of scattering points follows spatial consistency.

LOS cluster

Samsung: modelled as multiple point scatters with interaction between each other:

Huawei: The indirect path generation can be applied to one of the scattering points or the geometric centroid of the target

- K-factor and stochastic clusters are generated per target: Huawei

DOCOMO: pseudo-scattering points

- Each LOS pseudo-scattering point is modeled with a direct path

- Total number of direct paths (LOS rays) nLOS_s is equal to summation of single SP and LOS pseudo-SPs

- Humans and UAVs cases: nLOS_r = 5

- AGVs and animals, vehicles cases: nLOS_r = 5/8/10 (dependent on positions of Tx, Rx and targets)

IDC: spread of arriving rays associated with the sensing target decreases as we increase Tx-target-Rx distance

- [Moderator’s note]

Based on the contributions, for a target with N>1 scattering points, one simple solution is to treat each scattering point separately as if a target with single scattering point. N sets of NLOS clusters need to be generated. The N sets of NLOS clusters may have certain correlations for spatial consistency. In summary,

- Option 1: Each scattering point is separately handled as if a target with single scattering point, i.e.,

  - one direct path can be generated respectively for each scattering point

  - separate NLOS clusters are modeled for each scattering point

    - FFS whether/how to consider spatial consistency on the NLOS clusters of the multiple scattering points

Option 1 results in N times increased complexity for processing NLOS clusters compared to single scattering point case. One scheme to mitigate the complexity is to generate single set of NLOS clusters which is associated with one of the N scattering points, or a new/virtual scattering point, e.g., the geometric centre of the target.

- Option 2: joint target channel generation for a group of scattering points,

  - One direct path is generated respectively for each scattering point in the group of scattering points

  - Single set of NLOS clusters can be generated for the group of points

    - Alt 1: Assuming one scattering point in the group of scattering points

    - Alt 2: Assuming one new/virtual scattering point, e.g., in the geometric centre of the target

Based on the contributions, Option 1 gets a majority support. However, since we didn’t officially discuss this issue before, let me just ask a general question to confirm the companies’ views.

- [M][FL2] Proposal 5.5-1

For a target with multiple scattering points, Option 1 for target channel generation is agreed

- Option 1: Each scattering point is separately handled as if a target with single scattering point

  - one direct path can be generated respectively for each scattering point, as agreed in RAN1 #118bis

  - separate NLOS clusters are modeled for each scattering point

    - FFS whether/how to consider spatial consistency on the NLOS clusters of the multiple scattering points

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE |  | We prefer above Option 2 (Alt1) for simplicity. |
| Huawei, HiSilicon | Should be high priority issue | We agreed on the following in the last meeting: Agreement A single direct path is modeled for a scattering point of target With that, the first sub-sub-bullet is agreed impliedly - one direct path can be generated respectively for each scattering point Then the point of the proposal should focus more on the indirect path generation, the following sub-bullet can be deleted since the meaning is a bit vague. - Each scattering point is separately handled as if a target with single scattering point In general, the proposal is suggested to be updated as follows: - [H][FL1] Proposal 5.5-1 For a target modelled with multiple scattering points, - The direct path generation as agreed for the target with single scattering point applies to each scattering point. - The indirect path generation as agreed for the target with single scattering point is applied to each of the scattering points - The K-factor is generated per target, i.e. the scattering points associated within a target share the same K-factor. - The stochastic clusters are generated per target, i.e., the scattering points associated within a target share the same stochastic clusters and fast fading parameters |
| LGE |  | We’re fine with FL proposal. Regarding the shared K factors and clusters suggested by HW, if we consider the vehicle detection case, each scattering point representing each side of a vehicle may not experience the same channel condition. |
| Nokia, NSB |  | Okay with Option 1. |
| Moderator | @Huawei: agree that, based on the cited agreement, “A single direct path is modeled for a scattering point of target”, it is implied one direct path for each scattering point. Just add “, as agreed in RAN1 #118bis” in the end. Regarding K factor or clusters, I believe LGE’s comments are reasonable, so, let’s further check it in spatial consistency discussion. Please note I also update the proposal for spatial consistency to add the bullet proposed by LGE |  |
| EURECOM |  | Support this proposal. |
| Ericsson | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| Xiaomi |  | Similar view with ZTE. |
| Lenovo | Yes |  |
| Apple | Yes |  |
| Moderator | Continue discussions |  |

Polarization, EO type-2

Agreement When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to b,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario - FFS which deployment scenario(s) EO type-2 will apply - Agreement To model the effect of polarization for each direct/indirect path: - Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For a LOS ray from Tx to target or from target to Rx, CPM=100-1 for CPMtx,sp or CPMsp,rx - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx is generated by XPR ratio  and initial random phases referring to TR 38.901 as start point - FFS how to normalize on CPMtx,sp,rx - FFS CPMsp of a scattering point of the target - FFS: how to model the effect of polarization when EO type-2 is present

Summary on company views

In RAN1 #118, specular reflection is agreed to model EO type-2 with reference to 7.6.8 in 38.901 as start point. The exact polarization matrix needs to be extended for wall or other TO type-2.

Need to consider the orientation of EO type-2: Huawei, QC, ZTE

Only model specular reflection: vivo, CATT

FFS specular reflection is sufficient: Samsung, NVIDIA

model EO type-2 with multiple scattering points: Samsung

reflection with more than one bounce caused by EO type-2 can be dropped: CATT

Pathloss

PLEO=dEOd3DPLTx-STST-Rx: Huawei (LOS, NLOS)

FFS if NLOS condition: CATT

finite size of the EO type-2

the finite size of the EO type-2 affects identification of specular reflection points: Ericsson, CATT, vivo, Intel, QC

- the scaling factor b may be modified appropriately: Intel, Ericsson

Ericsson: reflection coefficients is dependent on the material of the object, the finite size of the surface and the overlap of the surface with the Fresnel zones.

Ericsson: If the size of the EO is larger than the first Fresnel zone and the reflection point with equal incidence angle and reflection angle is on the EO, Type-2 EO is modelled as a reflective surface with the Fresnel reflection coefficients. Otherwise, it is not modelled.

AT&T: When EO type-2 is modelled, diffuse scattering can be considered to model EO-type-2 in addition to specular reflection, when the conditions for specular reflection are not met

EO type-2 if NLOS condition?

QC, CATT: EO type-2 is only modelled when LOS ray exists in the same Tx-target or target-Rx link

CATT: FFS EO type-2 in NLOS condition

EO type-2 model

Huawei: Pathloss of wall is PLWR=dWRd3DPLLOS

Huawei: Absolute delay of wall is τWR=dWRc=d2D,tx+d2D,rx2+d2D2-d2D,tx- d2D,rx2+htx-hrx2c

Huawei, ZTE: CPM of wall is 
CPMwall=cosγ2sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1sinγ1cosγ1

Ericsson: Model blocking of type 2 environment objects reusing the blockage model B in TR38.901, updating the model to account for the shape and orientation of the object.

Which scenario(s) EO type-2 can be modelled

- Indoor scenario: Nokia, vivo

- Urban grid: Huawei, vivo, CATT

  - NO: Nokia

- Ground for outdoor: Nokia

- Applicability of EO Type-2 in different scenarios can be discussed in the first sub-agenda: QC

Normalization of CPMtx-sp-rx: vivo, Apple, ZTE, Tejas, Apple

- CPMtx,sp,rxnorm=d11d12d21d2212d11+d22=2d11+d22d11d12d21d22: ZTE

- [Moderator’s note]

For EO type-2 with finite size, the specular reflection point in mathematics may not be in the area of EO type-2 for a pair of sensing Tx/Rx. As proposed by some companies, it sounds straightforward that EO type-2 is not modelled in such case.

[H[FL1] Proposal 5.6-1

- The orientation of EO type-2 should be explicitly/implicitly defined in the evaluation parameters of a sensing scenario if applicable

- The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is only modelled in the target channel only if the specular reflection point is in the area of the EO type-2

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | The proposal has been reflected in the procedures in our contribution R1-2410648 |
| Huawei, HiSilicon | Yes with comment | Is the first bullet needed for modelling? It is more about evaluation. The ‘only’ be deleted from the second bullet. |
| Intel | Yes |  |
| LGE | Yes |  |
| Samsung |  | Support with modification. To avoid the additional discussion whether EO type-2 is modelled in target channel or background channel, “In the target channel, EO type-2 is only modelled only in the target channel if the specular reflection point is in the area of the EO type-2” |
| Nokia, NSB | Yes |  |
| Moderator | @Huawei: when we write down the polarization matrix for EO type-2, we need to the orientation of e.g. a wall, right? @Samsung: revised as proposed |  |

| Ericsson | Yes | The first bullet applies to targets also. EO Type-2 is modelled in target channel only. |
| --- | --- | --- |
| CATT/CICTCI | OK |  |
| New H3C | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| SONY | Yes |  |
| Xiaomi | Yes |  |
| Toyota | Yes |  |
| BUPT |  | The orientation should be explicitly defined in the evaluation parameters of a sensing scenario so that the reflection point can be determined. It is only necessary to model EO type-2 when the reflection point can be found in the area of EO type-2. |
| Apple | Yes |  |
| Moderator | Revised by offline |  |

- [H[FL4] Proposal 5.6-1-rev1

- The orientation of EO type-2 should be defined in the evaluation parameters of a sensing scenario if applicable

- The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2

[Moderator’s note] ZTE and Huawei both propose a procedure to derive the polarization matrix for EO type-2, e.g., a wall, which could be a good starting point for the review by other companies.

- [M][FL1] Proposal 5.6-2

If EO type-2 is modelled in an indirect path, polarization of the indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

- For the NLOS ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., the NLOS ray Tx - EO type-2 -target, or target - EO type-2 - Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2, i.e., CPMeo

- To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | The first subbullet is not clear, we suggest to delete it. The procedures between two contributions are almost the same. Since ours have more figures to explain the details, can I suggest using R1-2410648 as starting point for saving effort. Suggested wording: If EO type-2 is modelled, the procedure (step 1-7 ) in section 14 in R1-2410648 is taken as starting point for the channel coefficient for the EO type 2 reflected path. - Companies are encourage to check if anything incorrect and come back next meeting |
| Huawei, HiSilicon | Should be high priority | Huawei provided the TP, which can simplify the discussion. Suggest FL take the TP in R1-2409394 as the starting point to collect views. Modify the proposal as: - [H][FL1] Proposal 5.6-2 When the indirect path is created by EO type-2: - For a NLOS ray generated by a EO type-2 in the Tx-target link or the target-Rx link, CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2, i.e., CPMeo - The TP in R1-2409394 is taken as the starting point to collect views. |
| LGE |  | Considering the agreement that CPMtx,sp,rx = CPMsp,rx * CPMeo * CPMtx,sp, the meaning of the first bullet is not clear. |
| Nokia, NSB |  | Agree with the 1st bullet. |
| Moderator | @ZTE, LGE: we had an agreement CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp but it is only for stochastic cluster, with EO type-2 FFS. That is, we didn’t agree using product of 3 component to model polarization when EO type-2 exists. This is the intention of the first bullet. I add revisions. Hope it is clear now. @Huawei, ZTE: My understanding is both your two tdocs follow the same principle, but of course some difference in details wording/formula organization. It is better to ask companies to review both. |  |
| Ericsson |  | Is the following the intention of the first bullet? - For a NLOS ray generated by a EO type-2 in the Tx-target link or the target-Rx link, CPMtx,sp or CPMsp,rx is replaced with the polarization matrix of EO type-2, i.e., CPMeo |
| ZTE |  | If we check Huawei’s contribution, the procedures for the channel coefficient part for reflection path are basically the same as ZTE’s previous contribution. In this meeting, we add more details, such as how to calculate A and B in step 1 in our contribution. That’s why we think it is good if those procedures can be the starting point for further check. With regards to the three matrix, I am not sure if we can directly agree with this because rotation matrix should be considered additionally. |
| CATT/CICTCI |  | We’d like to see the justification of the changes of polarization matrix of EO type-2 |
| vivo |  | The content in the first bullet is not clear to us. The polarization matrix should always contain three matrices, one is CPMtx,sp or CPMsp,rx, one is CPMsp, and one is 100-1. |
| Xiaomi |  | Fine with the proposal. |

[Moderator’s note] As captured as a FFS point in the agreement, if the absolute values on the main diagonal of CPMtx,sp,rx are much different from value 1, the matrix CPMtx,sp,rx will impact the power of direct/indirect path which is not the intended function of polarization matrix. Therefore, normalization on the values on the main diagonal is proposed by multiple companies.

- [H[FL1] Proposal 5.6-3

- The polarization matrix CPMtx,sp,rxnorm of a direct/indirect path is obtained by normalizing CPMtx,sp,rx of the direct/indirect path, i.e.,

CPMtx,sp,rxnorm=d11d12d21d2212d11+d22=2d11+d22d11d12d21d22,

where, CPMtx,sp,rx=d11d12d21d22 is the product of polarization matrix of Tx-target link, the target, and the target-Rx link.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | It should be noted that this is mainly for regular case rather than EO type 2. |
| Huawei, HiSilicon |  | We wonder whether this normalization is performed per the scattering point or per path of each scattering point? |
| LGE | Yes |  |
| Nokia, NSB |  | Should indicate that this normalization is performed per path. |
| Ericsson | No | There is no reason that CPM_tx,sp.rx should be normalized. It only creates problems, e.g. if the denominator (¦d11¦+¦d22¦) becomes very small. On the contrary, the CPM_tx,sp,rx must not be normalized. Else the impact of polarization mismatch between Tx-target channel, target scattering, and target-Rx channels is not accounted for. One well-known example from physics is a reflection at the Brewster angle which has coefficient 0 for one of the polarizations. Two such reflections can cause a CPM that is all zeros. |
| CATT/CICTCI | Yes |  |
| vivo | Yes |  |
| SONY | Yes |  |
| Xiaomi | Yes |  |
| Lenovo |  | Normalization does not preserve the power if there is any cross polarization or unequal diagonal values |
| Apple | Yes |  |
| Moderator | My understanding is this proposal applies per ray for a scattering point |  |

Interaction between multiple targets

Summary on company views

Interaction between two targets or EO type-1

- Option 1: the interaction between a target and a second target/EO type-1 is modelled

  - Supported by: Samsung (11-15dB low), NVIDIA

- Option 2: the interaction between a target and a second target/EO type-1 is NOT modelled

  - Supported by: Xiaomi, vivo, ZTE (with validation results), QC, Apple, MTK, AT&T, CATT, EURECOM

- Option 3: the interaction between a target, and a second targets/EO type-1 with specular reflection is modelled

  - Supported by: E//

ZTE

Table 1 Interaction power between two targets

| Related types of objects | Receiving signal power | Receiving signal power [dB] |
| --- | --- | --- |
| Direct link for Tx-ST1-Rxor link for Tx-ST2-Rx | 1.887503e-10 | -97.24 |
| Indirect link for Tx-ST1-ST2-Rx or Tx-ST2-ST1-Rx | 1.1370469e-13 | -129.44 |

Samsung

| Paths | EO type 1 simulation results (path gain [dB]) | EO type 2 simulation results (path gain [dB]) |
| --- | --- | --- |
| Tx-Target-Rx | -104.1 dB | -103.7 dB |
| Tx-Target-EO -Rx or Tx-EO-Target-Rx | -114.9 dB | -118.8 dB |

Table 3. Preliminary simulation results of one bounce and two bounces for the target channel

IDC: An unintended target is a blockage factor in the target channel and further study whether Blockage model A or Blockage model B should be implemented

- [Moderator’s note]

Majority companies prefer to not model the interaction between targets. In fact, Since a minimum distance between targets is introduced in the evaluation of sensing solutions, the interaction between two targets is normally weak, which motivate to not model a path from Tx to target 1, target 2, then to the Rx.

On the other hand, it is also proposed by some companies that specular reflection may happen between two targets which cause relative high power of the interaction path. However, the probability for specular reflection could be low.

Please check if you have strong concern on the proposal, Otherwise, I will submit it to online session.

[H][FL1] Proposal 5.7-1

- A propagation path from Tx to Rx interacting with more than one sensing targets is not modelled

- FFS whether/how blockage can be modelled in the target channel.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Intel |  | In our understanding, this question is in the same domain as modeling of EO type 2 / type 1 interaction with the target. All those interactions should be considered together. We prefer to have the interaction modeling possibility to be open especially if the interactions with EO are modelled. At the same time, depending on particular use case and technology study, those interactions could be disabled. |
| LGE | Yes | We prefer to limit the specular reflection to EO type-2, excluding the sensing target. |
| Samsung |  | Same view with Intel. |
| Nokia, NSB | Yes |  |
| CATT/CICTCI | Yes |  |
| EURECOM | Yes |  |
| Qualcomm | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| OPPO | Yes |  |
| SONY | Yes |  |
| Spreadtrum | Yes |  |
| Xiaomi | Yes |  |
| Ericsson |  | For a Tx-target1-target2-Rx path, the scattering off target 1 can be in the angular range of backward scattering or forward scattering. Does the first bullet intend to preclude backward scattering off target 1 and the second bullet leave forward scattering off target 1 for further study? |
| Lenovo | comment | In the deployment scenario agenda item, we have the discussion of some target types, e.g., scooter, etc. which imply multiple target interactions. Moreover, the case of NLOSv in the TR 37.885 also requires capturing the paths interacting with more of one target. As such, the “not modeled” wording is not appropriate, it can be said that a path is dropped, which also defines a modeling step. Hence the following edit is proposed: A propagation path from Tx to Rx interacting with more than one sensing targets is not modelled dropped |
| Toyota | Yes |  |
| Apple | Yes |  |
| Moderator | Adding forward scattering per Ericsson’s comments |  |

[H][FL2] Proposal 5.7-1-rev1

- A propagation path from Tx to Rx interacting with more than one sensing targets is not modelled

- FFS whether/how blockage/forward scattering can be modelled in the target channel.

[Moderator’s note] agreement in Wednesday online session

- Agreement

- If blockage/forward scattering between sensing targets is not considered, a propagation path from Tx to Rx interacting with more than one sensing targets is not modelled

- FFS whether/how blockage/forward scattering can be modelled in the target channel.

LOS condition

- Agreement When the stochastic cluster is used to model indirect path in the target channel - For bistatic, the LOS condition from Tx to target and from target to Rx is determined separately for a target - FFS: The correlation of LOS condition of Tx-target and Rx-target links of a target - For monostatic, a same LOS condition is determined for Tx to target and target to Rx - The LOS condition from Tx to target and/or from target to Rx is determined with the LOS probability - The probability schemes in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, etc. are considered as start point - FFS: How to consider the impacts of target height on LOS probability.

Summary on company views

LOS probability

- To consider impact of height in LOS probability determination: CATT, Xiaomi, SPRD

- Correlation of LOS condition of Tx-target and Rx-target links is reflected in spatial consistency: vivo, Nokia, Xiaomi, CATT, Apple,

CATT, CAICT: The LOS/NLOS probability of target channel (H_target) and background channel (H_background) should be determined respectively.

If EO type-2 is modelled

- Still reuse LOS probability methodology defined in TR 38.901: Ericsson, Samsung, CATT

  - Ericsson: Impact of EO type-2 is modelled using blockage model B after generating LOS condition with a probability

- place EOs based on the LOS/NLOS conditions determined by the distance-based model in TR 38.901: Samsung

- Consider the geometrical locations of EOs in the LOS probability calculation: Samsung

- If type-2 EO block the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition: Huawei, CATT, SPRD, Tiami (applicable to background channel too)

- Deterministically by EO type-2: Lenovo, NVIDIA

[Moderator’s note] the following three options on LOS condition determination are proposed so far, when the EO type-2 is used to model indirect path in the target channel

- Option 1: Still reuse LOS probability methodology defined in TR 38.901

  - Impact of EO type-2 is modelled using blockage model B after generating LOS condition with a probability

- Option 2: If type-2 EO block the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

- Option 3: If type-2 EO block the direct path of one link, the link is determined as NLOS condition, and otherwise it is LOS

From the discussion in last RAN1 meeting, it seems Option 2 gets a majority support. So the moderator makes the following proposal.

[M][FL1] Proposal 5.8-1

When the EO type-2 is modelled in the target channel, the LOS condition of the Tx-target link and target-Rx link are determined by Option 2,

- Option 2: If type-2 EO block the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

| Company | Preference | Comments |
| --- | --- | --- |
| ZTE |  | OK |
| Huawei, HiSilicon | Option2 | Should be high priority issue. |
| LGE | Yes |  |
| Nokia, NSB |  | Okay. |
| EURECOM | Yes |  |
| Ericsson | No | We have already agreed to use TR 38.901 as the starting point. In 38.901 the determination of LOS or NLOS happens without consideration of explicit blockers. After this determination, path loss, clusters, and rays are generated. Then, but not before, the blocker is introduced in the channel and its impact on all the rays are calculated. The same approach should be taken also for blocking by EO type 2. |
| vivo |  | We believe that EO type-2 should be independent of the LOS condition. The EO type-2 path can coexist with LOS path. Whether the EO type-2 path exists is geometrically based on the EO location and environments surround. |
| CMCC | Option2 | If EO type-2 is used, the LOS condition of the Tx-target link and target-Rx link are determined by Option 2. |
| Xiaomi |  | Fine with the proposal. |
| Lenovo | Option 2 | Further, upon NLOs condition due to EO type-2, the Blockage model B can be used to model the effect of the blockage Also, we propose to add a note: “when the LOS status is determined in the absence of the EO type 2, the link channel parameters are taken according to the LOS status. The NLOS condition generated merely based on the EO type-2 blockage which is modeled via the blockage model B ” |
| Apple | Yes |  |
| Moderator | Revised by offline |  |

[M][FL4] Proposal 5.8-1-rev1

When the EO type-2 is modelled in the target channel, the LOS condition of the Tx-target link and target-Rx link are determined by Option 2,

- Option 2: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

[Moderator’s note] further revised after offline session

- [M][FL4] Proposal 5.8-1-rev2

When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

- Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

| Company | Preference | Comments |
| --- | --- | --- |

EO type-1

Summary on company views

EO type-1 is not explicitly modelled, but it can be handled as a target: QC, CATT, Ericsson, Nokia, Samsung, EUROCOM, IDC

[Moderator’s note] Many companies comment EO type-1 can be treated the same as target, so that EO type-1 is not necessarily defined for ISAC channel model, given it is already agreed that ISAC channel model can support multiple sensing targets for a pair of sensing Tx/Rx

[H][FL1] Proposal 5.9-1

- EO type-1 is treated as a sensing target in the ISAC channel model

  - Discussion on RCS, polarization, micro-Doppler (if supported) for EO type-1 is of lower priority than the agreed target types UAV/Human/vehicle/AGC/object creating hazard in roads/highways

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Intel | Yes | We also need to discuss interaction of EO type-1 and targets |
| LGE |  | The meaning of the first bullet may be misleading that EO type-1 is a kind of a sensing target. We suggest the following clarification. - Scattering by EO type-1 is treated in a similar way as a sensing target in the ISAC channel model |
| Nokia, NSB | Yes |  |
| Moderator | @LGE: I think the current wording should be fine. So we don’t need special design for EO type-1 any more, just consider it as a target. |  |
| EURECOM | Yes |  |
| Ericsson | No | We agree to the main bullet. Without unintended targets, the sensing scenario is incomplete. |
| CATT/CICTCI | yes |  |
| Qualcomm | Yes | OK with LGE’s variation also |
| vivo | Yes |  |
| CMCC |  | The key point is whether EO type-1 interacts with the target. If it doesn’t, then EO type-1 is a target. |
| OPPO | No | EO type-1 impacts sensing performance significantly. Without EO type-1, false detection will be too optimistic. So EO type-1 should be modelled and treated as the same priority as target. We suggest to remove the sub-bullet. |
| SONY | Yes |  |
| Spreadtrum | Yes |  |
| Xiaomi | Yes |  |
| InterDigital | No | The sub-bullet is not needed as it does not bring any benefit to the discussion. If EO type 1 is a target, it will inherit any characteristics a target can have. The first bullet is sufficient. |
| Toyota | Yes |  |
| Apple | Yes |  |
| Moderator | To address some companies’ concerns, delete the sub-bullet. Discussion on details of target of EO type-1 will be contribution driven and subject to available time in the study item |  |

[H][FL2] Proposal 5.9-1-rev1

- EO type-1 is treated as a sensing target in the ISAC channel model

  - Discussion on RCS, polarization, micro-Doppler (if supported) for EO type-1 is of lower priority than the agreed target types UAV/Human/vehicle/AGC/object creating hazard in roads/highways

[Moderator’s note] agreement in Wednesday online session

- Agreement

EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model.

Absolute time of arrival

Agreement For radio propagation Case 1, for modelling the target channel of a target with single scattering point, - To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS - FFS initial phase - FFS how to model RCS, polarization of target - FFS number of direct paths - FFS on detailed modelling of indirect path(s) - FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points - Agreement A single direct path is modeled for a scattering point of target - In each of the Tx-target and target RX links, the first NLOS cluster is generated with same delay as the LOS ray (when the absolute delay modelling of Δτ as in section 7.6.9, TR 38.901 is not applied) and with the same direction as the LOS ray. - FFS how to generate NLOS cluster when Δτ is applied

Summary on company views

Absolute delay is only specified for InF in the latest TR and using absolute delay for UMa and UMi is now being discussed in AI 9.8.

QC: supported an optional model for absolute arrival time

Apple: When Δτ is applied, it is not applied to the first NLOS cluster generated as it is effective part of the LOS link

Huawei:

Table 19: Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH | InH | UMi | UMa |
| --- | --- | --- | --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 | -7.9 | -7.5 | -7.4 |
|  | σlgΔτ | 0.4 | 0.4 | 0.3 | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 | 10 | 15 | 50 |

Model a LOS cluster consisting of M LOS rays: OPPO

- [Moderator’s note]

Inputs from more companies are necessary to find the way forward. Companies are encourage to express views on following questions.

[M][FL1] Question 5.10-1

- Q1: Is it agreeable to support absolute delay model Δτ as a feature for ISAC channel model in Rel-19?

- Q2: When absolute delay model Δτ is supported, should Δτ be limited to the stochastic clusters other than the first NLOS cluster?

| Company | Comments |
| --- | --- |
| ZTE | Q1: We definitely need absolute delay for ISAC, we are discussing location detection for sensing, absolute delay is very important. Q2: Δτ should be used for all NLOS clusters including the first one as 38.901 specified. |
| Huawei, HiSilicon | Q1: yes, absolute delay is needed for ISAC. Q2: should be applied to all the NLOS clusters. |
| LGE | We share the view with ZTE and Huawei/HiSilicon. |
| Nokia, NSB | Q1: yes. Q2: Yes, other NLOS clusters should be applied. |
| Ericsson | Q1: It seems necessary to support absolute delay modeling. Note that only parameters of InF are captured in 38.901 and using absolute delay for UMa and UMi is now being discussed in AI 9.8. Q2: No. The target channel and the communication channel should be consistent with each other, so there shouldn’t be a different procedure for the former. |
| vivo | First of all, the modeling of absolute delay model Δτ is necessary. This is because the positioning is one part of feature that should be realized by sensing. If absolute delay model Δτ is supported, all the clusters should be modelled. |
| Xiaomi | Q1: Yes. Q2: All the NLOS clusters. |
| Lenovo | Q1. Yes; absolute ToA is needed to obtain realistic impact of the background channel noise on the target observation. Q2. Yes, since the delay of the first NLOS cluster is intended as part of the LOS path |
| Apple | Q1: Yes Q2: Yes |

- [M][FL2] Proposal 5.10-1

- It is agreed to support absolute delay model Δτ as a feature for ISAC channel model in Rel-19

- When absolute delay model Δτ is supported, FFS Δτ is limited to the stochastic clusters other than the first NLOS cluster?

| Company | Comments |
| --- | --- |

Background channel modelling

Modelling EO or not

Summary on company views

Background channel without EO is prioritized: vivo

EO type-2 in background channel if it is modelled in target channel

- Yes: Huawei, QC, CATT, AT&T, Tiami, Sony, ZTE

- NO: Ericsson

  - Ericsson: the received power of the sensing Rx’s in the ISAC channel model with the absence of sensing targets is different from that in legacy UE-BS communication channel

- Neutral: Xiaomi

Clutter models from radar fields

- Supported by E//, Samsung

- [Moderator’s note]

Regarding EO type-1, please comment using section 5.9-1.

For EO type-2, companies are encouraged to provide views on the following proposal.

- [M][FL1] Proposal 6.1-1

- EO type-2 can be modelled in the background channel when it is modeled in the target channel

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes | Location of EO type-2 should be common to both target and background channel. |
| EURECOM | Yes |  |
| Ericsson | No | Type-2 EO if modelled in background channel would lead to the consequence that the received power of the sensing Rx’s in the ISAC channel model with the absence of sensing targets is different from that in legacy UE-BS communication channel. |
| vivo | Yes |  |
| CMCC |  | We only accept that EO type-2 is modeled to determine the LOS condition of TX-RX at present stage. If EO type-2 is also modeled to determine the channel coefficient of background, we must calibrate the results which are from EO type-2 and random procedure in TR 38.901. |
| Xiaomi |  | EO type-2 can be optionally modelled considering that Explicit ground reflection model is optional for BS-UE communication channel. |
| Lenovo | Yes | EO type-2 modelling should be consistent between the background channel and the target channel. |

Bistatic sensing mode

Agreement On the background channel for TRP-TRP and UE-UE bistatic sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as starting point - Update on values of the LSP/SSP parameters can be discussed based on validation data acquired by measurement or ray-tracing model - FFS The power threshold for removing clusters in step 6, i.e., -25 dB is revised to X<-25 dB. FFS X - FFS whether/how to resolve the inconsistency between TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 when UAV and TRP are set to same height

Summary on company views

Bistatic

E// observes that the channel models for the background TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 are inconsistent with each other.

Table 1	Comparison of K-factor in fast-fading models

| Model | K-factor |
| --- | --- |
| TR 38.858: TRP-TRP UMa | LOS:  = 9 dB,  = 3.5 dB NLOS: N/A |
| TR 36.777: UMa-UAV Alternative 1 | LOS:  = 20 dB,  = 0 dB NLOS:  = 10 dB,  = 0 dB |
| TR 36.777: UMa-UAV Alternative 2 | LOS:  = 4.217⋅log10hUT+5.787 = 11 dB at hUT = 25 m,  = 8.158⋅exp0.0046⋅hUT = 9.2 dB at hUT = 25 m NLOS: N/A |
| TR 36.777: UMa-UAV Alternative 3 | LOS:  = 15 dB,  = 0 dB NLOS: N/A |

Vivo, Intel, CATT: The channel of TRP-TPR link is generated based on TR36.777 in UAV scenario

E// proposes to perform measurements to parameterize the stochastic model of the BS-BS and UE-UE channels. Use the outcome of these studies to decide on whether the ISAC background channel modelling can be based on reusing or modifying the models in TRs 38.858 and 37.885 or if new modelling needs to be developed.

QC

- For a vehicle UE – Vehicle UE: use TR 37.885

- For a non-vehicle UEs, reuse the SL Positioning TR solution:

  - Option 1: BS-to-UE channel model defined in TR 38.901 [11] is revised:

  - Option 2: D2D channel model from 36.843 A.2.1.2 is used.

-25 dB power threshold for removing clusters needs to be changed: Ericsson, Lenovo

- NO: CATT, QC

- Issue 1: How to reflect existing TRs for ISAC channel generation

[Moderator’s note] RAN1 already agree on reusing existing TRs as start point for the Tx-target link, target-Rx link and the background channel in bistatic sensing mode. However, the exact mapping between the Tx-target link, target-Rx link, the background channel, and the associated existing TRs is still missing.

- [M][FL1] Question 6.2-1

Which option is preferred to capture the large scale/small scale parameters to generate channel for Tx-target link, target-Rx link or the background channel in the CR to TR 38.901?

- Option 1: Add a simple reference to proper sections of an existing TRs in the CR. If certain large scale/small scale parameters for channel generation needs further updates, it is also captured in the CR

- Option 2: Copy all large scale/small scale parameters for channel generation from the existing TRs, with possible updates for ISAC, into the CR

  - TR 38.901 will be self-contained for ISAC channel model

| Company | Option 1 or 2 | Comments |
| --- | --- | --- |
| ZTE |  | Option 1 may be better to keep the TR shorter |
| Huawei, HiSilicon | Either | No strong opinion as long as it is clearly stated. Low priority and can be reviewed in the CR stage. |
| LGE |  | It’s a matter of the CR stage. |
| Nokia, NSB |  | We can delay this discussion at a later stage. |
| Ericsson | Option 2 |  |
| vivo | Option 1 | Option 1 is easy to understand, whereby the reader can know which part is modified and/or added. |
| CMCC | Option2 |  |
| Xiaomi |  | Option 1 is preferred. |

- Issue 2: Reference TRs

[Moderator’s note] The possible sensing Tx/Rx includes TRP, terrestrial UE, vehicle UE, aerial UE and AGV UE. Since channel for terrestrial UE and AGV UE are both supported in TR 38.901, which are both referred as normal UE.

In the following table, the proper existing TRs to generate channel between any two nodes from TRP, normal UE, vehicle UE and aerial UE are proposed. There are still multiple options for some combinations, which needs further down-selection later.

NOTE: An existing TR may define a channel model based on the system level channel model in TR 38.901, or based on the CDL/TDL channel model in TR 38.901. Since we are discussing a system level channel model for ISAC, it is straightforward we only refer to an updated system level channel model in the existing TR. An example is in 36.777, “B.1.1 Alternative 1” is based CDL-D, while “B.1.2 Alternative 2” and “B.1.3 Alternative 3” are updating section 7.5 in TR 38.901. Therefore, it is “B.1.2 Alternative 2” and/or “B.1.3 Alternative 3” can act as the reference.

Please check if you have any concern on such mapping. Opinion on how to down select the option for each combination are appreciated.

- [H][FL1] Proposal 6.2-2

- The related sections in the following existing TRs are used as starting point to generate a channel between any two nodes from TRP, normal UE, vehicle UE and aerial UE.

| Case | Node 1 | Node 2 | Existing TRs as starting point |
| --- | --- | --- | --- |
| 1 | TRP | TRP | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - TRP-TRP link of scenario UMi, UMa, InH, and InF in section A.3 of TR 38.858 - FFS reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hUE=35m Highway - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 in FR2 - FFS reuse TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1, e.g., hUE=35m Urban grid - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 for Urban grid HST - FFS reuse TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 |
| 2 | TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: HST in section Annex A2 of TR 38.802 and section 6.2 of TR 38.854 - Option 2: TRP-UE link of scenario RMa in section 7 of TR 38.901 |
| 3 | TRP | vehicle UE | Highway and Urban grid - V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 - Option 2: [only for UMa, RMa] V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 |
| 4 | TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - Annex B.1.2 and Annex B.1.3 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |
| 5 | normal UE | normal UE | UMi, UMa, RMa, InH, and InF, UMi-AV, UMa-AV, and RMa-AV - UE-UE of UMi, UMa, InH, and InF in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: FFS reuse TRP-UE link of scenario InH and InF in section 7 of TR 38.901 - Option 3: UE-UE link in section A.1 of TR 38.859 Highway and Urban grid - P2P channel model in section 6 of TR 37.885 HST - Option 1: P2P channel model in section 6 of TR 37.885 - Option 2: UE-UE channel model for RMa - Option 3: UE-UE link in section A.1 of TR 38.859 |
| 6 | normal UE | vehicle UE | UMi, UMa, RMa - Option 1: UE-UE of UMi and UMa in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: V2P in section 6 of TR 37.885 - option 3: UE-UE link in section A.1 of TR 38.859 Highway and Urban grid - V2P in section 6 of TR 37.885 |
| 7 | normal UE | aerial UE | FFS reuse TRP-aerial UE of UMi-AV in Annex A and B of TR 36.777 for UMi-AV, UMa-AV, and RMa-AV |
| 8 | vehicle UE | vehicle UE | Highway and Urban grid - V2V link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: UE-UE link of scenario UMi and UMa in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: V2V link of scenario Highway and Urban grid in section 6 of TR 37.885 - Option 3: UE-UE link in section A.1 of TR 38.859 |
| 9 | aerial UE | aerial UE | UMi-AV, UMa-AV, RMa-AV - FFS aerial UE-aerial UE link of scenarios UMi-AV, UMa-AV, RMa-AV in section Annex A and B of TR 36.777 |

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Should be lower priority than the target channel | Expect no offline/on discussion at all. Can be reviewed/decided in the CR stage. |
| LGE |  | We also need to consider UE-type RSU in association with 36.885 and 37.885. |
| Samsung |  | Support |
| Ericsson |  | In our view, BS-UE channel in InH and InF can be reused for BS-BS and UE-UE channel in the same scenarios, as it is captured as Option 2 for Case 5. This option can be added for Case 1 as well, like the red text below. Case 1 UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - TRP-TRP link of scenario UMi, UMa, InH, and InF in section A.3 of TR 38.858 - FFS reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hUE=35m - Option 2: FFS reuse TRP-UE link of scenario InH and InF in section 7 of TR 38.901 […] Another comment is about the three alternatives in 36.777. “NOTE: An existing TR may define a channel model based on the system level channel model in TR 38.901, or based on the CDL/TDL channel model in TR 38.901. Since we are discussing a system level channel model for ISAC, it is straightforward we only refer to an updated system level channel model in the existing TR. An example is in 36.777, “B.1.1 Alternative 1” is based CDL-D, while “B.1.2 Alternative 2” and “B.1.3 Alternative 3” are updating section 7.5 in TR 38.901. Therefore, it is “B.1.2 Alternative 2” and/or “B.1.3 Alternative 3” can act as the reference.” This is not correct. B.1.1 Alternative 1 in 36.777 is also a system level channel model. The study item on Study on Enhanced Support for Aerial Vehicles agreed to let companies use either of the three alternatives for system level simulation since consensus could not be reached on down-selection. To be consistent with the communication TRP-UAV UE channel those companies that elected to base their implementation and simulations on Alternative 1 should be allowed to also use Alternative 1 for the TRP-UAV target channel. Therefore, we propose to clarify it and remove the sub-bullet for case 4: - Annex B.1.2 and Annex B.1.3 |
| Qualcomm |  | Low priority at this stage |
| vivo |  | In Case 1, we do not think it is reasonable since the ISD between gNB in highway scenario is 1732m, which is equals to the ISD value defined in RMa. The FFS bullet seems more desirable for the TRP-TRP link in the highway scenario. In Case 6, Option 1 is not acceptable because vehicle blockage model is not introduced as the scenario defined in TR37.885, and the same comment to Option 3, |
| CMCC | Yes |  |
| Xiaomi |  | Fine with the proposal. |
| BUPT |  | For background channels, TRP-UE and UE-TRP cases can directly reuse existing TRs. The direct modification of the antenna height using TR 38.858 for TRP-TRP and UE-UE requires further measured verification. |
| Moderator | @Ericsson: for InH, InF, 38.858 is also based on 38.901 with revisions, not sure what you would updates in “Option 2: FFS reuse TRP-UE link of scenario InH and InF in section 7 of TR 38.901” Regarding UAV & 36.777, if Annex B.1.1 is used, how would you gets the parameter values of the system level ISAC channel model from the link level CDL model? @vivo: For highway in FR2, it is actually ISD 500m referring to 37.885, so at least the listed options in Case 1 should be possible. The second bullet is actually for FR1. So these two bullets are the single whole proposal |  |

[Moderator’s note] Assuming Proposal 6.2-2 is agreeable, the proper existing TRs for background channel in bistatic sensing mode can be simply derived. Please check if you have any concern on such mapping.

- [H][FL1] Proposal 6.2-3

- To generate the background channel of bistatic sensing mode, the following mapping between STX/SRX and existing TRs is adopted

| STX/SRX | SRX/STX | Case of existing TRs as starting point |
| --- | --- | --- |
| TRP | TRP | Case 1: TRP-TRP link of same scenario |
|  | Terrestrial UE | Case 2: TRP-normal UE link of same scenario |
|  | Vehicle UE | Case 3: TRP-Vehicle UE link of same scenario |
|  | Aerial UE | Case 4: TRP-Aerial UE link of same scenario |
|  | AGV UE | Case 1: TRP-normal UE link of same scenario |
| Terrestrial UE | Terrestrial UE | Case 5: normal UE-normal UE link of same scenario |
|  | Vehicle UE | Case 6: normal UE - Vehicle UE link of same scenario |
|  | Aerial UE | Case 7: normal UE - Aerial UE link of same scenario |
|  | AGV UE | Case 5: normal UE -normal UE link of same scenario |
| Vehicle UE | Vehicle UE | Case 8: Vehicle UE - Vehicle UE link of same scenario |
| Aerial UE | Aerial UE | Case 9: Aerial UE - Aerial UE link of same scenario |
| AGV UE | AGV UE | Case 5: normal UE - normal UE link of same scenario |

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Should be lower priority than the target channel | Expect no offline/on discussion at all. Can be reviewed/decided in the CR stage. |
| LGE |  | We also need to consider UE-type RSU in association with 36.885 and 37.885. |
| Ericsson | Yes |  |
| vivo |  | It is the same comment as in case 1 of TRP-TRP link in highway scenario based on the proposal of 6-2-2. |
| CMCC | Yes |  |
| OPPO |  | We are open to whether to treat AGV as a normal UE or a vehicle. |
| Xiaomi |  | Fine with the proposal. |
| BUPT | Yes | We think that the above mapping covers all cases. |

[Moderator’s note] Multiple companies discuss the misalignment between 36.777 and 38.858 for a TRP-TRP channel, and propose to simply follow the model defined in 36.777.

- [M][FL1] Proposal 6.2-4

The TRP-TRP channel is generated following TR36.777 in UAV scenario

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE |  | Considering the possible updates, we can add ‘as a starting point’ at the end of the proposal. |
| Ericsson |  | If BS-aerial UE channel is reused for BS-BS channel, will the following bullet in 38.858 be reused? - The angle of arrival statistics (ASA, ZSA) are set to the same values as the angle of departure statistics (ASD, ZSD) There are three fast-fading alternatives in 36.777 with different small-scale parameters. Which one will be used? |
| vivo | Yes |  |
| CMCC | Yes |  |
| Xiaomi |  | The TRP-TRP channel is generated following TR36.777 in UAV scenario could be used for scenario UMi, UMa, and RMa. Considering that scenario of InH and InF are not contained in TR36.777 in UAV scenario but considered in TR 38.858, the TRP-TRP channel model of scenario of InH and InF can be referred to TR 38.858. |

- Issue 3: on issue of -25dB threshold

[Moderator’s note] There was a discussion in last meeting on the -25dB issue, but no agreement is made. Since a dropped path in background channel by -25dB threshold may have a power much higher than the target channel, some companies propose to reduce the threshold. On the other hand, some other companies think that the major part of background channel is already modeled, so not need to change the threshold.

Let’s have one more round trying the following proposal, to see if a consensus can be achieved.

- [M][FL1] Proposal 6.2-5

The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is revised to X<-25 dB. FFS value X

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Should be lower priority than the target channel | If this threshold is changed, don’t we worry about the inconsistency with communication channel for which the threshold is unchanged then? |
| LGE |  | We also have a concern on the inconsistency with the communication channel. |
| Nokia, NSB |  | We would prefer to keep the existing -25dB (no change). |
| Ericsson | Yes | In sensing, we want to distinguish a target from the background clutter. The detection of this target requires suppression of the energy coming from the clutter. The strong paths in background are LOS between Tx and Rx and ground reflection between Tx and Rx, shown in Figure 1 and illustrated in Figure 2 as the two largest circles. The strong paths in background channel are not strong interference for object detection and can be mitigated due to different angle, delay, and doppler from those of the target. Figure 1 Zoomed in version of the measured power delay profile for the BS-BS background channel, showing the LOS and the ground reflection paths. Weak paths, especially scattered off from mobile scatterers in the background may show similar doppler signature as sensing target and become strong interference to sensing performance. If -25dB threshold is reused, the paths within the black rectangle in Figure 2 will be dropped. However, in Figure 3 the paths with non-zero doppler caused by moving scatterers in the background have power about 40dB lower than the strongest path. They may have similar Doppler signature as sensing target and would be interference to sensing algorithms though with low power. Figure 2 Scatter plot showing the distribution of multipath in azimuth, elevation, power (proportional to the marker size) and delay (proportional to the marker color). Figure 3 Delay-Doppler plot based on a time series of impulse responses between a pair of SMa/RMa base stations in LOS. Paths with non-zero Doppler frequencies have been highlighted. The value X can be determined so that some paths in background channel have similar power as those in target channel. If communication is simulated together with sensing, lowering the threshold to be below -25dB will have no impact on communication channel’s performance, only increase complexity. |
| ZTE |  | We would prefer to keep the existing -25dB (no change). |
| vivo | Yes | It should be mentioned for sensing only, and thus, modified as The power threshold for sensing to removeing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is revised to X<-25 dB. FFS value X |
| Xiaomi |  | It could be kept as -25 dB for simplicity since background channel is usually generated referring the existing TR for TRP-UE communication channel model. |
| Lenovo |  | 38.901 is intended to properly approximate the communication channel parameters, and not the environment noise on the target channel. Hence, the dropping is justified in for the current 901 purpose, but not for the purpose of sensing modeling. Hence, we suggest the following two options: - We can remove the dropping all together, or Keep the dropping only if the cluster parameters of delay, doppler, AoA/ZoA from the sensing receiver perspective is sufficiently distinct from the (direct/indirect) paths of the target channel |

Monostatic sensing mode

- Agreement In order to define the background channel for TRP and UE mono-static sensing mode, - RAN1 to study how to model the background channel - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario - FFS EO is modelled in the background channel - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures - FFS EO is modelled in the background channel - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations. - Other options are not precluded

Summary on company views

Ericsson, Lenovo: A monostatic channel is the sum of two parts: crosstalk and a NLOS channel.

- Crosstalk is difficult to model generally and should be modelled outside of TR38.901 and outside of this study item.

- BS monostatic channel modelling in the SI focuses on the NLOS channel

validation is necessary: AT&T

The proposals for background channel with monostatic sensing mode is quite diverse.

- Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario

  - ZTE, CAICT, Lenovo (multiple virtual Rx), OPPO (if pathloss never exceed range of existing pathloss model), CATT, MTK, EUROCOM

ZTE - For UMa scenario, the virtual RX follows that - Gamma distributions can be used to generate the distance between Tx/Rx and the virtual Rx, and to generate the virtual Rx height - Horizontal angle between Tx/Rx and the virtual Rx can be set as 0. - Pathloss can be derived by TR 38.901 based on Tx/Rx and the virtual Rx. - Delay, AoD and ZoD of clusters can be derived by TR 38.901 based on Tx/Rx and the virtual Rx.

- Option 2: New channel model framework and parameters based on measurement results or ray-tracing model validated by experimental results or radar literatures

  - Ericsson, vivo, Intel

Ericsson - Step 1: Set environment, network layout, and antenna array parameters - Step 2: Generate a grid of ground clutter RCS values (RCS per unit area x area of grid cell) - Step 3: Calculate the path loss between BS and each grid cell - Step 4: Determine the power of the backscattered path by using the radar equation (path loss x2 + RCS) - Step 5: Generate a single ray for each grid cell.

Intel: The following aspects are studied for the new monostatic background channel generation procedure: - No explicit pathloss generation for background channel, - No explicit LOS state generation, - Uniform generation of cluster angels in azimuth domain, non-uniform generation of cluster angles in zenith domain, - New cluster power and delay generation procedure capturing absence of the TX-RX direction

- Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations.

  - CMCC, Apple

- Option 4: Existing channel model framework with parameters adjusted for monostatic based on validation data acquired by measurement or ray-tracing model

  - Huawei, BUPT

- Option 5: Adding a fixed or variable white Gaussian whose power reflects the interference suppression ability of the sensing receiver

  - Apple

CAICT: Large scale fading of background channel for TRP mono-static can be modeled as normal distribution with μ varying along frequency and fixed σ.

- (For 7GHz, large scale fading satisfies N94.63dB,5.632. For 26GHz, large scale fading satisfies N106.46dB,5.572. For 100GHz, large scale fading satisfies N118.36dB,5.62)

- [Moderator’s note]

Still multiple options are proposed by companies. Fortunately, the views start to converge. Option 1 gets a majority support. Option 2 gets some supports too, however, it seems different companies may have different new model in mind. One comment from one proponent of Option 2 is Option 2 can be considered as dropping many virtual receivers.  Given the situation, please check the following revised proposal based on Option 1.

[H][FL1] Proposal 6.3-1

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 as starting point

- Option 1: randomly drop N virtual receives, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario

  - FFS N=1 or N>1

- The same existing TR for TRP-TRP or UE-UE bistatic sensing mode is used to generate the background channel between the TRP or UE as sensing TX and each of the N virtual receivers.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | Support In option 1, it can specifically define the distribution of virtual receivers to make the generated channel match to the background channel from measurement / RT results. The benefit is to make background power especially for large scale power compatible with bi-static sensing mode. Also, it is the simplest way compared with other options. |
| Huawei, HiSilicon | Should be lower priority than the target channel | Expect not reaching on this before more progress on the target channel. |
| Samsung |  | If we follow the option 2, the virtual Rx should be literally virtual thing to generate channel parameter such as large scale and small scale parameters. When it comes to channel coefficient in TR38.901, the angular domain parameters in field pattern need to be aligned between Tx and Rx for monostatic. |
| Ericsson | No | Compared with a single virtual UE, multiple virtual UEs can generate the background channel more similar to BS mono-static BS channel at the cost of complexity. Let’s assume there are 100 candidate positions for virtual UEs, a single virtual UE is selected from the 100 positions. Selecting a set of two virtual UEs requires comparison among 100*99/2 sets. The rationale of Option 2 is to model ground scatterers in background channel deterministically. Each BS-scatter-BS path is modelled reusing direct path of target channel model, if a ground scatterer is assumed as a target. In the figure below, three channel impulse responses in the righthand side correspond to three ground scatterers. 2D distance, d2D, between BS and a ground scatterer can be calculated based on delay of a path. With angles of the path, the position of the scatterer relative to BS can be determined. RCS of the scatterer can also be calculated based on 2*pathloss of d2D and received power of the path. Our understanding of the rationale of Option 1 is that the BS-tree1-BS path in Figure 1(a) resembles BS-tree1-virtual UE path in Figure 1(b), if the two trees are located at the center of two circles, which radii are BS-tree1 distance and BS-tree2 distance respectively. BS mono-static channel is modelled with NLOS paths between BS and virtual UE. Figure 1 Illustration of Option 1 Note that departure and arrival angle of a path in such mono-static background channel are different. Given the maximum BS-UE distance (2D) supported by pathloss model of 38.901 is 5km for UMa and UMi, the maximum distance between BS and target/clutter is smaller than 2.5km. It is challenging to locate the virtual UE without causing large deviation in the following cases. - Environment scatterers and the BS are positioned in a straight line, Figure 2(a) - BS - virtual UE distance is smaller than min. BS - UT distance (2D), Figure 2(b) - BS - virtual UE distance is larger than max. BS - UT distance (2D), Figure 2(c) - There are at least three scatterers in the environment, Figure 2(d) Figure 2 different scenarios In addition to the above, we suggest not using stochastic clusters but rather many stochastically generated (but deterministically located) clutter locations, and simply using the radar equation (ie free space propagation) to compute the contribution to the monostatic background channel from each such clutter. |
| ZTE2 |  | @Ericsson We think both ways are workable. But the current FL proposal is much easier than your suggestion which needs to further define the number, distribution, sizes, RCS, XPR, etc. For those stochastic clusters. Most importantly, it may not be comparable with the agreement for bi-static sensing mode. It is noted that we may need to further do performance evaluation for comparison between mono-static and bi-static sensing mode. |
| CATT/CICTCI | Yes |  |
| Qualcomm | No | We think it is more natural to drop N virtual reflectors/scatterers (similar to what Ericsson is suggesting) and define the distribution of those since this is supposed to be the background channel of monostatic sensing. |
| CMCC |  | Our measurement results have shown that the reflectors in target channel are the EOs in monostatic background channel. If we have to use option 1 as starting point, the virtual RX is at the same place with the target. |
| OPPO | Ok | We just expect the measurements taken by companies can show the proof that the measured pathloss for background channel never exceeds the bistatic pathloss in current TR. |
| Xiaomi |  | Fine with the proposal. |
| Lenovo |  | We suggest to add the following implementation alternative for this option, as following based on spatial consistency procedure: - Selection of a virtual Rx position as the closest location to Tx for which the Tx-Rx channel can be obtained via 38.901 - Generation of the NLOS paths of the Tx-virtual Rx channel - Utilize spatial consistency procedure A of 38.901 to move the virtual Rx towards the true Rx position in step wise manner and update the NLOS Path parameters - The updated path parameters of the Tx-virtual Rx (after arrival to the true Rx position) is used as the NLOS path parameters of the Tx-Rx channel NOTE: this procedure does not provide the LOS path of the monostatic channel, but given the assumed self-interference cancellation of the direct channel, only NLOS paths are of interest for our channel modeling |
| BUPT |  | We think it is important to ensure the modeled mono-static background channel aligns with measured results in terms of PL, delay, angle, and other statistical parameters. There are currently various methods, each with its own strengths. For the method of dropping one virtual RX, if the RX is determined based on the similarity between the mono-static background channel’s PL and the TR 38.901’s PL, the generated multipath delays and angles are likely to deviate from measurements. BUPT [R1-2410659] proposes a method that drops multiple cluster centers, equivalent to multiple virtual RXs, and generates two identical LOS clusters between TX/RX and each virtual RX based on the TR 38.901. This approach produces multipath delays and angles that closely match measured results, though the calculation method for PL still requires further validation. The multipath positions obtained from a single simulation are shown in the figure below. Given the issues with the methods mentioned above, we think integrating and refining these methods would be beneficial. We recommend that this proposal highlights the need to validate specific implementations through measurements. |
| NIST |  | The background channel models are using some type of approximation to generate background clutters. It is important that these models are statistically validated against measurement data. In terms of using ISAC channel model to simulate target detection and evaluate detection algorithms, the target model and the EO models are more important than the background channel, particularly the spatial accuracy of the background clusters. Therefore, a good balance between model efficiency and accuracy is desired. |
| Moderator | Based on some offline, I add a solution B for possible compromise. Not quite sure if it is agreeable. If not, let still use the early proposal as starting point since it is a bit more support. |  |

[H][FL2] Proposal 6.3-1-rev1

Solution A:

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 as starting point

- Option 1: randomly drop N virtual receives, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario

  - FFS N=1 or N>1

- The same existing TR for TRP-TRP or UE-UE bistatic sensing mode is used to generate the background channel between the TRP or UE as sensing TX and each of the N virtual receivers.

Solution B:

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- Option 1: randomly drop N virtual receives, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario

  - For each receiver, AOA/ZOA are enforced to be same as AOD/ZOD of the coupled rays in a cluster in step 9.

  - FFS N=1 or N>1, FFS value N

  - Note: Validation results are necessary to set the parameters of the N receivers for the scenario

- The same existing TR for TRP-TRP or UE-UE bistatic sensing mode is used to generate the background channel between the TRP or UE as sensing TX and each of the N virtual receivers.

[Moderator’s note] Some further updates

To @Ericsson, not sure if the wording on Solution B reflect your preference. Please comment if any update is necessary. In general, suggestions/comments/preference on all options are welcome.

- [H][FL4] Proposal 6.3-1-rev2

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- drop N virtual receives following certain distribution, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario. FFS the distribution for virtual receiver dropping

  - N is determined by validation results for each scenario, FFS value N

Supported by: ZTE, Xiaomi, LGE, Samsung, Lenovo, MTK, BUPT, CATT, vivo, OPPO, NIST

Solution B (Previously option 2)

- drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel

  - Alt 1: only direct path is modeled for each ground clutter

  - Alt 2: both direct path and indirect paths can be modelled for each virtual target

- FFS the number, the distribution, RCS, XPR, etc. for ground clutters

Supported by: Ericsson, Nokia, [QC]

Solution C (Previous Option 4)

the BS mono-static background is modelled complying with the framework in TR38.901 with updated parameters captured in R1-2409394

- The path loss is modelled as LrdcdB~X(μrdcf,σSF);

- The reference  ϕref,  AOD is modelled as ϕref,  AOD~uniform(0°,360°) ;

- The reference θref,ZOD is modelled as 90° and 155° for reflection/diffraction and scattering, respectively;

- The ZOA = ZOD and AOD = ZOD in the mono-static background;

- The generation steps are modelled as in section 4;

- The updated parameters are modelled as in Table 7

Supported by: Huawei,

| Company | Yes/No | Comments |
| --- | --- | --- |

Combined channel

Interactions between target channel and background channel

Summary on company views

Multiple options are proposed to combine the target channel and background channel

- No power normalization: ZTE, Ericsson, Lenovo, Xiaomi, CATT

  - Need to account for blockage due to introduce of target: Lenovo

- Power normalization: vivo, Intel, BUPT AT&T, CAICT, IDC

  - Optional and scenario dependent: QC

Multiple options can be supported: OPPO, QC

Power normalization

Option 1: normalize both target channel and background channel: vivo, OPPO

Option 2: normalize background channel only: BUPT, OPPO

- PLISAC=PLtarget+Oisac∙PLbackground38.90136.77737.885: BUPT, IDC

Blockage issue when combining target and background channel: Ericsson, BUPT

Intel:

- The average power distribution of a combined ISAC channel is equal to the average power distribution of a communication channel when no sensing is modelled,

- The cluster power distribution of a combined ISAC channel is aligned with the cluster power distribution of a communication channel when no sensing is modelled.

Xiaomi: For both target channel and background channel, the propagation time delay corresponding to the LOS ray is considered in the channel coefficient calculation

- [Moderator’s note]

Combining with or without power normalization both get some supports. Further, considering a limitation on total number of clusters or blockage are also proposed, which may provide somewhat similar functions as power normalization. Companies are encouraged to provide views on the following proposal. Please also indicate your preferred alternative if Option 2 is agreeable.

[H][FL1] Proposal 7.1-1

- The following options to generate the combined ISAC channel are considered for further study

  - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by simply summing the target channel(s) and background channel, i.e., power normalization is not considered.

  - Option 2: Optionally, power normalization is considered when summing the target channel(s) and background channel. Down select between

    - Alt 1: Power normalization on both target channel and background channel

    - Alt 2: Power normalization on background channel only

- No additional limitation on the total number of clusters in the ISAC channel model

- FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | Option 1 The target channel power especially for the direct path should be independent from the background channel. If normalization is done between target channel and background channel, this would lead to different direct path power in different scenarios. For example, for a target channel with direct path of a UAV, the receiving power at Rx in UMi scenarios and RMa scenario will be different after normalization because the background channel is different. This would not be aligned with the reality. Vice versa. |
| Huawei, HiSilicon | Should be lower priority than the target channel or the RCS of the target |  |
| Intel |  | Agree to study the options. |
| LGE |  | We’re fine with the proposal. Considering that the target channel is one of the background channel from the communication channel modeling point of view, we think the existing power normalization should be applied. In that sense, we prefer Alt 1 of option 2. |
| Samsung |  | Support |
| Nokia, NSB |  | Agree to have these two options. |
| Moderator | Welcome more inputs in general, we need an agreement otherwise we never have a ISAC channel model |  |
| EURECOM |  | Support. Prefer Alt1 of Option 2 |
| Ericsson | Yes | Support Option 1, provided that the target channel does not contribute with so much power that it changes the communication performance over the Tx-Rx link, when communication and sensing are simulated together. |
| CATT/CICTCI | Yes | Option 1 |
| New H3C | Yes |  |
| Qualcomm | Support |  |
| vivo | Yes | The necessity of power normalization is dependent on how may targets are added on the background channel, and how much impact occurs on the communication channel. To this end, only Alt 1 in Option 2 is a possible solution. |
| CMCC |  | We prefer option 2 and Alt 1. |
| OPPO | Yes | Option 2 is preferred. For Option-1, if Tx-Rx channel is determined as in LOS condition, adding target channel as an additional NLOS path power from Tx to Rx may effectively change the KR factor. |
| Xiaomi | Yes | Fine with the proposal. |
| Lenovo |  | Option 1. Suggest to change FFS to: FFS how Blockage is modelled for the background channel due to sensing target and/or EO type-2 |
| BUPT |  | We prefer Option 2. Based on our measurement results [R1-2408263], when a target is placed within the background channel, it blocks the original clusters/paths of the background channel, leading to a reduction in power and the disappearance of some clusters in background channel. |
| Apple | Yes | Agree this needs to be studied |
| Moderator | It is again observed both options get support. One way forward is to support both options |  |

[H][FL2] Proposal 7.1-1-rev1

- The following options are supported to generate the combined ISAC channel

  - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by simply summing the target channel(s) and background channel, i.e., power normalization is not considered.

  - Option 2: Optionally, power normalization is considered when summing the target channel(s) and background channel. Down select between

    - Alt 1: Power normalization on both target channel and background channel

    - Alt 2: Power normalization on background channel only

- No additional limitation on the total number of clusters in the ISAC channel model

- FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2

[Moderator’s note] revised by offline

- [H][FL4] Proposal 7.1-1-rev2

- The following options are supported to generate the combined ISAC channel for bistatic

  - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by simply summing the target channel(s) and background channel, i.e., power normalization is not considered.

  - Option 2: As an additional model, power normalization is considered when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between

    - Alt 1: Power normalization on both target channel and background channel by a common scaling factor

    - Alt 2: Power normalization on background channel only

    - Alt 3: the target channel of a target will replace one cluster in the background channel

- No additional limitation on the total number of clusters in the ISAC channel model

- FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2

- FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

Blockage vs. forward scattering

Summary on company views

E//, BUPT, QC, Lenovo, IDC discuss the modelling of blockage in ISAC channel modelling. Based on the company inputs, an intention of blockage modelling for sensing is both to maintain consistency of the ISAC channel model for communication, e.g., when a sensing target blocks an effective path for communication, but also, and more importantly, to capture the impact of blockage as a means for obtaining valuable sensing information.

Blockage model B: E// (EO type-2), Lenovo

Eliminate the blocked path and add a ray due to forward scattering: Lenovo

- [Moderator’s note]

Blockage may largely impact the sensing performance hence some companies proposed to model it

Though existing section 7.6.4, TR 38.901 may be used as start point, it is expected that some changes are deemed due to new sensing operation

Forward scattering may be considered when blockage happens, e.g., target/object is on the baseline from sensing Tx to Rx. However, duplicated modeling should be avoided

- [L][FL1] Proposal 7.2-1

- The Blockage model defined in section 7.6, TR 38.901 is considered in the ISAC channel model as start point

  - FFS any enhancement is necessary

- RAN should avoid duplicated modeling of blocking by an object and forward scattering by the same object

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE |  | We’re fine with reusing the existing blockage model as a starting point. But for the sensing target case, we prefer to consider the forward scattering instead of the blockage. |
| Ericsson |  | Does this proposal imply that forward scattering is only modelled with blocking? |
| CMCC | Yes |  |
| Lenovo | Yes |  |
| BUPT | Yes | More field channel measurements should be conducted to explore the issues of blockage and forward scattering in ISAC channel modeling. |

Spatial consistency modelling

- Agreement -RAN1 #117bis - Spatial consistency should be supported for ISAC channel - Spatial consistency should be supported based on movement of sensing Tx, sensing target and/or sensing Rx - FFS EO handling

Summary on company views

Study All 6 sensing modes: vivo

Which links should spatial consistency be considered

- same target, different Tx/Rx: vivo, OPPO, CATT, Xiaomi, Sony

- different targets, same/different Tx/Rx: vivo, OPPO, CATT, Xiaomi, Sony

- between Tx-target link and target-Rx link: OPPO, Samsung, AT&T, QC, vivo, Sony, Xiaomi, ZTE

- between target channel and background channel: OPPO, AT&T, ZTE

  - NO: Sony, Lenovo? Vivo?

Xiaomi: For heterogeneous types of sensing Tx and Rx in bistatic sensing, evaluate and consider the spatial consistency between Tx->Target link and Target->Rx link, i.e., whether and how to model the correlation.

Reuse legacy spatial consistency in 38.901 (site-specific model)

- Spatial consistency in 7.6.3.3, 38.901 as a starting point: Apple

- Spatially consistent procedures in legacy TR 38.901 can be reused for TRP-target or target-TRP link by replacing UTs with targets: SPDR, Xiaomi, AT&T(at least TRP-TRP), EURECOM

- QC: TR38.901 procedures for spatial consistency can be reused for each hop

  - NOT consider new ray being added or old ray being dropping, not change number of bounces

- Reuse existing site specific/all specific model: IDC, ZTE (+ Rx specific grid), Sony

- Spatial consistency of background channel follows existing 38.901: vivo, Lenovo, MTK

Sensing target specific model: vivo, IDC, CATT, Sony

- Model correction of target specific 2D grid by Cholesky decomposition of correlation matrix: vivo

Site and target specific spatial consistency: IDC, Sony

Xiaomi: Uniform grid spaced by corresponding correlation distance:

Procedure A:

- SPRD: When the sensing target speed is taken into account, the change of distance can be expressed as:

rα,txTtk-1⋅vtxtk-1+rα,rxTtk-1∙vtk-1+rβ,txTtk-1∙vtk-1+rβ,rxT⋅vrxtk-1∙∆t

Procedure B: Xiaomi

- NO: SPDR

3D spatial consistency

- Model correction of target specific 3D grid by Cholesky decomposition of correlation matrix: vivo

- 3D grid: ZTE

Deterministic model

Vivo, CATT, MTK, NVIDIA: Naturally meet the spatial consistency, i.e., based on deterministic locations and movement (if any) of the environment objects

Mobility

Samsung, CATT proposes to consider the spatial consistency modelling for the stationary Tx/Rx and moving sensing target first

Temporal consistency

- Spatial and temporal consistency of multiple instances of the same or different sensing modes: Lenovo

- spatial-temporal consistency of RCS B2: NIST, Lenovo

General

- [Moderator’s note]

Based on all inputs, there is a good support to reuse existing spatial consistency for at least TRP-target or target-TRP link. Such backward compatibility is also beneficial when joint sensing and communication are considered. The spatial consistency of other links is also discussed by companies, however no much detailed solutions are provided. It would be better to leave it open for further study.

Further, there is a good support to extend to 3D spatial consistency for at least UAV scenario

- [H][FL1] Proposal 8.1-1

- Spatially consistent procedures in legacy TR 38.901 are reused for TRP-TRP bistatic and TRP monostatic by replacing UTs with targets

  - FFS if procedure A and/or B needs enhancement

- FFS spatial consistency model for other sensing modes

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Intel | Yes |  |
| Nokia, NSB | Yes |  |
| EURECOM | Yes |  |
| Ericsson | Yes |  |
| CATT/CICTCI | Yes |  |
| New H3C | Yes |  |
| Qualcomm | Yes |  |
| vivo |  | We have serious concerns on the proposal in case of “different targets, same Tx/Rx”. We have shown the clear evidence that the correlation in this case cannot be guaranteed. We suggest that prior to agreeing on the proposal, we should discuss which links should be considered with spatial consistency. |
| OPPO |  | We wonder whether the same proposal can apply to other sensing modes like TRP-UE bistatic and UE monostatic. |

[Moderator’s note] Xiaomi raises one issue that the Tx-target link and target-Rx link may refer to existing channel models of different scenarios. As shown in the following figure, assuming a communication scenario of UMa, BS-UE bistatic sensing mode,

- BS-target link straightforwardly follows BS-UE channel model of UMa in 38.901

- Multiple options may be considered to find a reference for Target-UE link

  - Option 1: Treated as UE-UE and referring to 38.858 which is defined based on UMi

  - Option 1: Treated as vehicle-vehicle and referring to 37.885 (V2V)

With any of the two options, the BS-target link and target-UE link are actually generated referring different communication scenarios. In this case, it seems a challenge to model spatial consistency.

- [H][FL1] Proposal 8.1-2

- Spatial consistency between Tx-target and target-Rx link is NOT modeled for the two links if generated referring to existing channel models of different scenarios.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE |  | We suggest not to make this conclusion for now. It depends on the final methodology. |
| LGE |  | Fine with FL proposal. |
| Nokia, NSB | Yes |  |
| EURECOM | Yes |  |
| Ericsson |  | Even if the two links are modelled with different communication scenarios/TRs, it is possible to model the spatial consistency of each link. However, the question is whether the correlation between the spatial consistency on the Tx-target and the target-Rx links should be considered. This needs further study. |
| CATT/CICTCI | Yes |  |
| vivo |  | It depends on the method of spatial consistency. If the target-specific mechanism is used for spatial consistency, this link with a certain correlation can be easily generated. |
| CMCC |  | If the target channel is modeled by EO type-2, this proposal may block some conditions. |
| OPPO |  | If the proposal is taken, then in a multi-static sensing geometry with two Tx’s (say Tx1 and Tx2) and one Rx and one Target, the spatial consistency calculation between Tx1-Target and Tx2-Target faces the similar challenge (if any) as for the one between Tx-Target and Target-Rx. But we see from Proposal 8.1-3 that the spatial consistency calculation between Tx1-Target and Tx2-Target is supported. |

[Moderator’s note] In the sensing operation, possible links are divided into Tx-target link, target-Rx link, and Tx-Rx link (i.e., the background channel). A general observation is that many companies mention about the spatial consistency between Tx-target link and target-Rx link of same/different target, of same different sensing Tx/Rx. On the other hand, it seems some companies want to avoid spatial consistency model between a link in target channel and the Tx-Rx link.

Companies are encouraged to express views on the following proposal, especially whether Case 3 should be supported or not.

- [H][FL1] Proposal 8.1-3

- Spatial consistency is considered for the following cases unless an exception is specified

  - Case 1: For a target, the links to/from multiple sensing Tx/Rx which are located within correlation distance to the target

  - Case 2: For a sensing Tx/Rx, the links to/from multiple targets which are located within correlation distance to the sensing Tx/Rx

  - Case 3: For a first sensing Tx/Rx, the links to/from a target and a second sensing Tx/Rx which are located within correlation distance to the first sensing Tx/Rx

  - Case 4: For a sensing Tx/Rx, the links to/from multiple scattering points of the same target

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Intel |  | Prefer deleting “…which are located within correlation distance to…” parts from the bullets since the spatial consistency mechanisms should be using the correlation distances as a parameter, and it may not be accurate to state the consistency is considered within a distance and not considered outside of the distance. |
| LGE |  | We’re fine with the proposal in general. One comment is the case of the multiple scattering points. If a target is modelled with the multiple scattering points, we also need the spatial consistency between links to/from sensing Tx/Rx and the scattering points. - Case 4: For a sensing Tx/Rx, the links to/from multiple scattering points of the same target |
| Moderator | @Intel: Not sure if any companies prefer to model spatial consistency even when the distance is larger than correlation distance, or prefer to introduce a new mechanism to replace correlation distance. If not, we can keep the condition @LGE: added as proposed, thanks. |  |
| EURECOM | Yes |  |
| Ericsson |  | This is covered by proposals 5.1-1 and 6.2-2 which equates targets with UEs/TRPs, hence the same spatial consistency modeling should apply. |
| vivo | yes |  |
| OPPO | Yes |  |
| InterDigital |  | We suggest to replace “target” with “scattering point” to be consistent with the agreements so far. |

Site/target specific spatial consistency

- [Moderator’s note]

From the companies’ inputs, the spatial consistency can be categorized as

- Site specific spatial consistency

- Target specific spatial consistency

- Site and target specific spatial consistency

However, the inputs are quite limited. To accelerate the discussion, please proponent on each option clarify the exact behaviours. If there is any concern, please add comments too

- [H][FL1] Question 8.2-1

Q1: For proponent of site specific spatial consistency, assuming 2D grid is assigned to TX1, please clarify how to model spatial consistency for

- TX1-ST1 vs. TX1-ST2

- TX1-ST1 vs. ST1-RX1

- TX1-ST1 vs. TX1-RX1

- ST1-RX1 vs. ST2-RX1

- TX1-ST1-RX1 vs. TX1-ST2-RX1

- ST1-RX1 vs. ST1-RX2

- TX1-ST1-RX1 vs. TX1-ST1-RX2

Q2: if TX1 moves, how to reflect it in spatial consistency?

| Company | Comments |
| --- | --- |
| ZTE | Our solution is to have site specific grid and Rx-specific grid: Step 1: a network grid is generated in site-specific or all-correlated manner as legacy for spatial consistency of the links from Tx to ST, and from Tx to Rx. Step 2: a Rx-specific grid is generated for the links from ST to Rx. Then, the spatial consistency between sensing targets is realized for the links from STs to Rx. The Rx-specific grid is generated based on a shift from the network grid where the UE and the STs are moved in the network grid to make the UE location in the origin of the grid. Based on this procedures, above all spatial consistency can be achieved. The details can be found in our contribution R1-2410648. |
| vivo | It should be noticed that - TX1-ST1 vs. TX1-ST2：ST1 and ST2 are taken as RX - TX1-ST1 vs. ST1-RX1: Spatial consistency does not work for ST1-RX1 - TX1-ST1 vs. TX1-RX1: ST1 is taken as RX - ST1-RX1 vs. ST2-RX1: Spatial consistency does not work for ST1-RX1 and ST2-RX2 - TX1-ST1-RX1 vs. TX1-ST2-RX1: Spatial consistency does not work for TX1-ST1-RX1 vs. TX1-ST2-RX1 - ST1-RX1 vs. ST1-RX2: Spatial consistency does not work for ST1-RX1 and ST1-RX2 - TX1-ST1-RX1 vs. TX1-ST1-RX2: Spatial consistency does not work for TX1-ST1-RX1 and TX1-ST1-RX2 |

- [H][FL1] Question 8.2-2

Q1: For proponent of target specific spatial consistency, assuming 2D grid is assigned to ST1 and ST2 respectively, please clarify how to model spatial consistency for (same question as Q1, 8.1-3)

- TX1-ST1 vs. TX1-ST2

- TX1-ST1 vs. ST1-RX1

- TX1-ST1 vs. TX1-RX1

- TX1-ST1-RX1 vs. TX1-ST2-RX1

- TX1-ST1-RX1 vs. TX1-ST1-RX2

Q2: if ST1 and/or ST2 moves, how to reflect it in spatial consistency?

| Company | Comments |
| --- | --- |
|  | For TX1-ST1 vs. TX1-RX1, we are not sure how it works, because we assume that the procedure is taken as follows: - TX1-ST1 vs. TX1-ST2: First, 2D grids assigned to ST1 and ST2 are generated with correlation. Then, generate consistent for TX1-ST1 and TX1-ST2 based on these grids, respectively. - TX1-ST1 vs. ST1-RX1: TX1 is taken as RX. Then, generate consistent for TX1-ST1 and ST1-RX1 based on grids of ST1 respectively. - TX1-ST1 vs. TX1-RX1: First, spatially consistent procedures in legacy TR 38.901 are still used for communication channel TX1-RX1. Then, 2D grids assigned to ST1 and TX1 are generated with correlation - TX1-ST1-RX1 vs. TX1-ST2-RX1: 2D grids assigned to ST1 and ST2 are generated with correlation. Then, generate consistent for TX1-ST1-RX1 and TX1-ST2-RX1 based on these grids, respectively. - TX1-ST1-RX1 vs. TX1-ST1-RX2: Generate consistent for TX1-ST1-RX1 and TX1-ST1-RX2 based on grid of ST1 |

- [H][FL1] Question 8.2-3

Q1: For proponent of site and target specific spatial consistency, please clarify the behavior for the following links

- TX1-ST1 vs. TX1-ST2

- TX1-ST1 vs. ST1-RX1

- TX1-ST1 vs. TX1-RX1

- ST1-RX1 vs. ST2-RX1

- TX1-ST1-RX1 vs. TX1-ST2-RX1

- ST1-RX1 vs. ST1-RX2

- TX1-ST1-RX1 vs. TX1-ST1-RX2

Q2: if a Tx/Rx/target moves, how to reflect it in spatial consistency?

| Company | Yes or No | Comments |
| --- | --- | --- |
| vivo | Yes | Regarding Q1, using site and target specific spatial consistency, all the path can be generated. Regarding Q2, if any Tx/Rx/target moves, we need to update its/their grid followed by the procedure in TR38.901. |

Calibration

Summary on company views

Need to focus on a subset of scenario and sensing mode is used for the calibration: vivo

Needs to calibrate for all target types: Nokia

Only SLS should be considered for calibration: Xiaomi

The target channel and the combined channel should all be calibrated: vivo

Necessary calibration

- large scale, full calibration: Huawei

- large scale, full calibration, spatial consistency: CATT, vivo, Xiaomi

Target attachment

- X pair(s) sensing Tx/Rx for a target: Xiaomi

- Only the best beam pair for a pair of sensing Tx/Rx for a target: Xiaomi

Calibration metric

- Calibration metric in TR 38.901 as start point: Huawei

- EO type-2 needs calibration: Huawei

- Large scale calibration

  - Consider Tx/Rx antenna pattern, pathloss of two links and SF, large scale RCS: Huawei

  - Consider Tx/Rx antenna pattern, pathloss: vivo

Interference

- Interference from other UEs and other sensing targets or unintended objects: vivo

Vivo: Table 20. Calibration metrics for ISAC channel

|  | Calibration metrics |
| --- | --- |
| Phase-1 large-scale calibration | - Coupling loss (dB) - FFS: Geometry SINR (dB) - FFS: Geometry SIR (dB) |
| Phase-2 full calibration | - Coupling loss (dB) - Delay spread (nsec) - ASD (degrees) - ZSD (degrees) - ASA (degrees) - ZSA (degrees) - FFS: Wideband SIR (dB) |
| Phase-3 additional features calibration | For spatial consistency: - Coupling loss (dB) - Xoorr – delay - Xoorr – AoA - Xoorr – LOS - Xoorr – channel response - FFS: Wideband SIR (dB) |

Xiaomi: Performance metrics for calibration should be determined for sensing, and the metrics in TR 38.901 can be considered as the starting point.

- Large-scale calibration: Coupling loss, SINR, and SIR.

- Full calibration: Coupling loss, SIR, CDF of Delay Spread and Angle Spread.

- Spatial consistency calibration: Coupling loss, SINR, Cross-correlation coefficient of delay/AOA/LOS-NLOS status/channel response, CDF of average varying rate of power/delay/AOA.

General

[Moderator’s note] Calibration on the ISAC channel was not ever discussed in RAN1 yet

As done in Rel-15, it is essential to calibrate the ISAC channel assuming different companies, which helps to minimize the potential diverse in performance evaluation of solutions

Target attachment (i.e., proper pair(s) of sensing Tx/Rx and proper beams pair(s) of Tx/Rx) and related calibration metrics for calibration needs to be defined

- [H][FL1] Proposal 9.1-1

- Large scale calibration, full calibration, and spatial consistency calibration are considered

  - for the target channel, and

  - for the combined channel of target channel and background channel

- For calibration purpose, RAN1 to further discuss the way to determine the proper pair(s) of sensing Tx/Rx and proper beams pair(s) of the Tx/Rx

- Calibration metrics in TR 38.901 are considered as the starting point to define performance metrics for calibration for sensing.

  - Large-scale calibration: Coupling loss, Geometry SINR, and Geometry SIR.

  - Full calibration: Coupling loss, Wideband SIR, CDF of Delay Spread and Angle Spread.

  - Spatial consistency calibration: Coupling loss, Wideband SINR, Cross-correlation coefficient of delay/AOA/LOS-NLOS status/channel response, CDF of average varying rate of power/delay/AOA.

  - FFS whether/how to modify the definitions of some or all of the following metrics

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | Given the modelling is much lagged especially the RCS aspect, all calibration discussion is suggested to be handled in AI9.7.1 |
| Samsung |  | Support. For the contents of performance metrics, it seems need to be discussed more for down selection. |
| Nokia, NSB | Yes | Support this. We need some discussions related to calibration. |
| Ericsson |  | It should be a common understanding that sensing has different performance metrics to communication. However, the ISAC channel model should support both sensing and communication. - For communication calibration we propose to simply reuse previous calibration procedures and metrics with limited or no updates. The main purpose of the communication calibration is to ensure that the metrics are essentially unchanged even when the target channel is added to the background channel. For sensing calibration, we propose to focus on the target channel. Since this channel contains several crucial deterministic components the calibration procedures and scenarios can be approached very differently compared to previous calibrations of stochastic communication channels. For instance, calibration evaluation assumptions can use a much smaller set of deterministic Tx/Rx/target locations rather than the large system configuration with stochastic dropping used for previous calibrations. An attractive approach would be to use CDL models to represent the direct and indirect paths in the Tx-target and target-background links, since then all companies naturally use the same clusters and rays without the need to randomization. |
| vivo | Yes |  |
| OPPO |  | The definition of SIR needs clarification. What does ‘S’ in SIR refer to? Similarly, how to calculate ‘I’ in SIR? In addition, how and is it good enough to calibrate the “large scale” if the small-scale power does not sum to unit power due to: - The EO-2 path is added to Tx-target link and/or Target-Rx link without power normalization on each link. - The small-scale power summation is not normalized in and after the concatenation. |
| Xiaomi |  | Fine with the proposal. |
| BUPT | Yes | Large-scale, Full calibration, and spatial consistency of TR 38.901 should be considered as the starting point in SLS. |

Target attachment

- [M][FL1] Proposal 9.1-2

- The best pair of Tx/Rx beams of a pair of sensing Tx/Rx for a target is used in calibration

- For calibration purpose, the best pair of sensing Tx/Rx is selected for a target, according to the received power of the target channel from port 0 using the best pair of Tx/Rx beams

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia, NSB |  | Shall we discuss about calibration criteria/metrics first before we selection Tx/Rx path? |
| Xiaomi |  | Fine with the proposal. |

Definitions on calibration metrics

- [M][FL1] Proposal 9.1-3

For calibration purpose, the following definition on the calibration metrics are agreed

- The metrics for large-scale calibration are defined considering pathloss of two links, shadowing, RCS component A, Tx/Rx antenna pattern, power of LOS ray in the two links, and polarization matrix of LOS ray, i.e., 100-1

- The metrics for full calibration and spatial consistency calibration are defined considering pathloss of two links, shadowing, RCS component A, Tx/Rx antenna pattern, power of NLOS rays in the two links, RCS component B/C, and polarization matrix of Tx-target link, the target and target-Rx link.

- FFS the exact definition of each metric

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia, NSB |  | More discussion might be needed on the list of these metrics. |

Link level channel model

Summary on company views

ZTE, Vivo: stochastic link level channel model for ISAC by adding sensing target cluster based on CDL channel model defined in TR38.901

- Model sensing target as one or more clusters: vivo

- Select one or more clusters for target: ZTE

- Newly add one or more clusters for target: ZTE

[Moderator’s note] Welcome more inputs in future meetings.

- Reference

- RP-242348, “Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz, V18.0.0 (2024-03)”

- R1-2409394	Channel modelling for ISAC	Huawei, HiSilicon

- R1-2409472	Discussion on ISAC channel modeling	EURECOM

- R1-2409524	Discussion on channel modeling methodology for ISAC	CMCC,BUPT,SEU, PML

- R1-2409609	Discussion on ISAC channel modelling	Samsung

- R1-2409647	Discussion on ISAC channel modeling	Spreadtrum, UNISOC

- R1-2409693	Views on Rel-19 ISAC channel modelling	vivo, BUPT

- R1-2409718	Discussion on ISAC channel modelling	TOYOTA InfoTechnology Center

- R1-2409740	Discussion on ISAC channel modeling	Intel Corporation

- R1-2409767	Discussion on ISAC channel modeling	Nokia, Nokia Shanghai Bell

- R1-2409777	Channel modeling for integrated sensing and communication with NR	NVIDIA

- R1-2409818	Discussion on ISAC channel modelling	Apple

- R1-2409837	Discussion on ISAC channel modelling	LG Electronics

- R1-2409847	Discussion on ISAC channel modeling	InterDigital, Inc.

- R1-2409908	Discussion on ISAC channel model	Xiaomi, BJTU, BUPT

- R1-2409953	Discussion on ISAC channel modelling	CATT, CICTCI

- R1-2409977	Discussions on ISAC Channel Modelling	Lekha Wireless Solutions

- R1-2409992	ISAC Channel Modeling and Measurement Validation	BUPT, CMCC, VIVO

- R1-2410007	Discussion on ISAC channel modelling	China Telecom

- R1-2410098	Study on ISAC channel modelling	OPPO

- R1-2410126	Discussion on ISAC Channel Modelling	Ericsson

- R1-2410136	Discussion on ISAC channel modeling	NIST

- R1-2410163	ISAC channel modelling	Tejas Networks Limited

- R1-2410235	Views on Channel Modelling for ISAC	Sony

- R1-2410321	Discussion on Channel Modelling for ISAC	Lenovo

- R1-2410333	Discussions on ISAC Channel Modeling	AT&T

- R1-2410370	Considerations on ISAC channel modelling	CAICT

- R1-2410401	Discussion on ISAC channel modeling	NTT DOCOMO, INC.

- R1-2410448	Discussion on channel modelling for ISAC	ZTE Corporation, Sanechips

- R1-2410490	Discussion on ISAC channel modelling	Qualcomm Incorporated

- R1-2410525	Discussion on ISAC channel modelling	MediaTek Inc.

- ANNEX: All agreements

- RAN1 #116

Agreement

The common framework for ISAC channel model is composed of a component of target channel and a component of background channel,

HISAC=Htarget+Hbackground

- Target channel Htarget includes all [multipath] components impacted by the sensing target(s).

  - FFS details of the target channel

- Background channel Hbackground includes other [multipath] components not belonging to target channel

  - FFS details of the background channel

- FFS whether/how to model environment object(s), i.e., object(s) with known location, other than sensing target(s)

  - FFS whether/how to model propagation path(s) between the target(s) and the environment object(s)

- FFS whether/how to model propagation path(s) between the target(s) and the stochastic clutter(s)

- Note: the notation HISAC can be revised later if needed

- RAN1 #116bis

Agreement

The following cases of radio propagation in the target channel are considered for the study

| Case | Tx-target | Target-Rx |
| --- | --- | --- |
| 1 | LOS condition | LOS condition |
| 2 | LOS condition | NLOS condition |
| 3 | NLOS condition | LOS condition |
| 4 | NLOS condition | NLOS condition |

- Case 1/2/3/4 can be considered for bistatic sensing mode

- At least Case 1/4 can be considered for monostatic sensing mode

- Note: It doesn’t imply the channel response for each link is separately generated then concatenated

- FFS how to determine LOS condition and NLOS condition, e.g., based on LOS probability, or determined based on geometrical locations of environment object (EO).

- In LOS condition, line of sight ray(s) are present between Tx/Rx and target, and there may or may not exist non-line of sight ray(s) between Tx/Rx and target too

- In NLOS condition, there only exist non-line of sight ray(s) between Tx/Rx and target

Agreement

- In the target channel between Tx and Rx, scattering of a sensing target can be modelled as single scattering point or multiple scattering points

- FFS one or multiple incoming/output rays corresponding to a scattering point

- FFS how to select single or multiple scattering points for the target, e.g. depending on the distance between target and Tx/Rx, size/shape of target, etc.

- Note: the sensing target can be assumed in far field of sensing Tx/Rx.

- FFS details to model the single or multiple scattering points

Agreement

RCS of a physical object shows dependency to at least the following factors:

- Type of the object

  - The size of the object

  - The material of the object

  - The shape of the object

- Orientation of the object

- FFS: Distance between Tx/Rx and the object

- The incident angle and scatter angle

- The carrier frequency

- polarization of the transmitter and receiver

- FFS Temporal or spatial consistency

- FFS antenna pattern

- FFS whether/how to model the above factors in the CR, e.g. with an RCS model with a scattering point

Agreement

EO is a non-target object with known location.

- FFS other known parameters of the EO

- FFS details on EO modelling

The following options for EO modelling are considered for further study

  - Option 1: EO is modelled different from a sensing target

    - Applicable at least for an EO having extremely large size (referred as EO type-2 for discussion purpose)

    - FFS modeled similar to section 7.6.8 ground reflection in TR 38.901

    - FFS EO modelling impacts the target channel and/or the background channel

  - Option 2: EO is modeled same/similar as a sensing target

    - Applicable for an EO having comparable physical characteristics as a sensing target, (referred as EO type-1 for discussion purpose)

    - FFS Applicable for EO type-2

    - FFS EO modelling impacts the target channel and/or the background channel

  - Option 3: EO is modeled and its location is determined from a stochastic clutter generated following the cluster generation in TR 38.901

    - FFS details

  - Option 4: EO is not modelled

  - Other options are not precluded

  - Note: it is not precluded that multiple options can be supported in the channel modelling

Agreement

The following options are considered for further study to model the target channel for a target

- Option 1: modelled by concatenation of path(s) from Tx to target and from target to Rx

- Option 2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry

- Option 3: combination of Option 1 and Option 2

Agreement

If a target is modelled with single scattering point, the following options to model RCS of the target are considered for further study.

  - Option 1: Random RCS value generated by a statistical distribution, depending on the factor(s) having impacts on the RCS modelling.

    - FFS the distribution.

    - FFS the factor(s)

  - Option 2: Deterministic RCS value is defined by a function and/or a table, depending on the factor(s) having impacts on the RCS modelling

    - Note: Constant RCS for a target type can be a special case of Option 2

    - FFS the factor(s)

    - FFS details of function and/or table

  - Option 3: combination of Option 1 & 2, e.g., RCS value is generated by combining a deterministic component and a randomly generated component.

  - FFS application of each option to large scale fading and/or small scale fading

  - FFS target with multiple scattering points

Agreement

- Interested companies are encouraged to submit validation results together with their proposal for ISAC channel modelling

- Up to each company to select the way for validation

  - Option 1: Experimental results

  - Option 2: Experimental results to validate a ray-tracing model, then the ray-tracing based results to validate the ISAC channel model

    - Note: the layout of the scenario used for validation is up to company choice

Agreement

ISAC channel model for link level simulation is to be discussed after the system level channel model is sufficiently stable with basic functionalities.

- RAN1 #117

Agreement

- Multiple sensing targets can be modelled in the ISAC channel of a pair of sensing Tx and sensing Rx

  - FFS whether to model a propagation path from Tx to Rx interacting with more than one sensing target

- The same sensing target can be modelled in the ISAC channels of multiple pairs of sensing Tx and Rx

Agreement

- For discussion purpose, the propagation paths in the target channel are classified

  - The direct path, i.e., LOS ray from Tx to target + LOS ray from target to Rx

  - The indirect paths, i.e., any propagation path other than the direct path, including

    - LOS ray from Tx to target + NLOS ray from target to Rx

    - NLOS ray from Tx to target + LOS ray from target to Rx

    - NLOS ray from Tx to target + NLOS ray from target to Rx

- For radio propagation Case 1,

  - For a direct path, the following parameters are [deterministically] generated at least based on the geometry location of Tx, target and Rx

    - AoA/ZoA at Rx

    - AoD/ZoD at Tx

    - AoA/ZoA/AoD/ZoD at target

    - delay

    - FFS initial phase

    - Doppler

    - FFS power/polarization including the impact of RCS

    - FFS the number of direct path(s) for a target

  - FFS on detailed modelling of indirect path(s)

- FFS on details of modelling of indirect paths in  radio propagation Case 2/3/4

- To generate the channel coefficients of direct/indirect path(s) in the target channel, the channel coefficient generation function in step 11 in section 7.5 of TR 38.901 (e.g., formula 7.5-22) is used as the start point

  - Note: modification to step 11 is deemed necessary

  - FFS adding impact of small scale RCS

  - FFS Doppler

Agreement

- Spatial consistency should be supported for ISAC channel

- Spatial consistency should be supported based on movement of sensing Tx, sensing target and/or sensing Rx

  - FFS EO handling

Agreement

When the stochastic cluster is used to generate the indirect paths in the target channel of a target

  - The stochastic cluster generation in section 7, TR 38.901 is used as starting point.

    - FFS a stochastic cluster is generated between Tx and Rx satisfying Tx-target-Rx geometry, or between Tx/Rx and target

      - FFS modification to stochastic cluster generation in section 7, TR 38.901

      - FFS use of sub-cluster to model the indirect paths

Note: RAN1 continues studying using EO to generate the indirect paths in the target channel of a target

Agreement

When the stochastic cluster is used to model indirect path in the target channel

- For bistatic, the LOS condition from Tx to target and from target to Rx is determined separately for a target

  - FFS: The correlation of LOS condition of Tx-target and Rx-target links of a target

- For monostatic, a same LOS condition is determined for Tx to target and target to Rx

- The LOS condition from Tx to target and/or from target to Rx is determined with the LOS probability

  - The probability schemes in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, etc. are considered as start point

  - FFS: How to consider the impacts of target height on LOS probability.

Agreement

When stochastic cluster is used to model indirect path in the target channel, down-select between the following options

- Option 1: modelled by concatenation of path(s) from Tx to target and from target to Rx

  - For each of the Tx-target link and target-Rx link,

    - The parameters delay, power, angle, [initial phase], [Doppler] of NLOS ray(s) in the link Tx-to-Target or Target to RX are generated

      - FFS following cluster generation in section 7, 38.901

    - The target channel is generated by concatenating the parameters of the Tx-target link and target-Rx link.

      - FFS on Convolutional or 1-by-1 coupling or 1-to-many coupling

  - FFS how to combine the clusters in target channel and the clusters in background channel

- Option 2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry of the direct path

  - The parameters delay, power, angle, initial phase of a stochastic (sub-)cluster between Tx and Rx are generated following cluster generation in section 7, 38.901

    - The parameters [delay], [power], [angle], [Doppler] of the (sub-)cluster are updated by the target property

  - FFS how to combine the clusters in target channel and the clusters in background channel

- RAN1 #118

Agreement

if RCS related coefficient of a scattering point is included in small scale, the RCS related coefficients are separately determined for different pairs of incident/scattered ray(s) at the scattering point.

Agreement

For radio propagation Case 1, for modelling the target channel of a target with single scattering point,

- To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated

  - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system

  - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system

  - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c

  - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx

  - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS

  - FFS initial phase

  - FFS how to model RCS, polarization of target

- FFS number of direct paths

- FFS on detailed modelling of indirect path(s)

- FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points

Agreement

For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel,

- An indirect path in small scale is modelled by concatenation of path(s) from Tx to target and from target to Rx, i.e., Option 1 in the agreement of RAN1 #117

  - AoD/ZoD/AoA/ZoA from Tx to target or from target to Rx are

    - generated for a LOS ray at least based on the 3D location of Tx/Rx and target in the global coordinate system

    - stochastically generated for a NLOS ray using section 7, 38.901 as starting point

  - Delay is sum of delay of LOS/NLOS ray from Tx to target and the LOS/NLOS ray from target to Rx

  - Doppler is generated by spherical unit vector by AoD/ZoD at Tx and velocity of Tx, by spherical unit vector by AoA/ZoA at Rx and velocity of Rx, and by spherical unit vectors by AoA/ZoA/AoD/ZoD at target and velocity of target

    - FFS The mobility of stochastic clutter

  - The power of the indirect path is generated as the product of the power of the LOS/NLOS ray from Tx to target, the power of the LOS/NLOS ray from target to Rx and the effect of RCS

  - FFS initial phase

  - FFS how to model effect of RCS at target

  - FFS whether/how to model polarization at target

  - FFS How to reduce complexity

  - FFS applicability of the indirect path generation to each/a single scattering point when the target is modelled as multiple scattering points

Working assumption

The RCS related coefficient of a scattering point can be modelled with two components, i.e., linear value RCS = A*B

- A first RCS component A (m2) is included in large scale

  - FFS the first RCS component is deterministic or stochastic

  - FFS The first component is dependent on incident and scattered directions at target

- A second RCS component B (unit ratio) is included in small scale

  - FFS The second component is dependent on incident and scattered directions at target

  - FFS the second RCS component is deterministic or stochastic or combination

- Note: RCS component A or B can be disabled by setting its linear value to 1

  - Whether to disable a component can be discussed per target type

- FFS how to determine A and B for each target

- FFS whether/how to model polarization impact at target

- FFS whether/how to normalize power accounting for target channel and background channel

Agreement

The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp

  - FFS whether CPMsp is angular/ray-dependent or independent

  - FFS whether polarization matrix CPMsp is modelled assuming specular reflection or random coefficient for diffraction or scattering

  - FFS whether polarization matrix CPMsp is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx

Agreement

For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode,

- The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point

In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode,

- RAN1 to study how to model background channel

  - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results

FFS whether/how to do power normalization between target channel and background channel

Agreement

In order to define the background channel for TRP and UE mono-static sensing mode,

- RAN1 to study how to model the background channel

  - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario

    - FFS EO is modelled in the background channel

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures

    - FFS EO is modelled in the background channel

  - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations.

  - Other options are not precluded

Agreement

When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference

- As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901)

  - FFS any update to b,R∥specular,R⊥specular

  - FFS any update taking EO orientation into account

- FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size

- FFS whether diffraction and scattering can be considered in addition to specular reflection

- EO type-2 is an optional modelling component if supported in a sensing scenario

  - FFS which deployment scenario(s) EO type-2 will apply

- RAN1 #118bis

Agreement

RAN1 strives to define a single option per target per monostatic/bistatic sensing mode from the following two options to generate RCS values/patterns for a scattering point of a target.

- Option 2: The RCS=A*B of a scattering point can be generated by

  - The component A is commonly applied to any incident/scattered angles at the scattering point

    - A is [mean] RCS value. FFS value(s) A

      - Note: Mean RCS value is defined as the mean value of the distribution of RCS

  - The component B

    - B is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2

      - B is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths

  - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS

- Option 3: The RCS=A*B=A*B1*B2 of a scattering point can be generated by

  - The component A is commonly applied to any incident/scattered angles at the scattering point

    - FFS: A = 1 m2 or [mean] RCS value

      - Note: Mean RCS value is defined as the mean value of the distribution of RCS

  - The component B is further split into B1, B2, i.e., B=B1*B2

    - B1 is deterministic based on incident/scattered angles

      - FFS: B1 is defined by a function or by a table

    - B2 is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2

      - B2 is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths

  - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS

Agreement

RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic

  - B2 of UAV is modelled using log-normal distribution for monostatic

  - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc.

  - For UAV of small size (option 2 for UAV size in UAV parameters table)

    - B1=1

    - A is mean RCS value

  - For UAV of large size (option 1 for UAV size in UAV parameters table)

    - B1 have dependency on incident/scattered angles

    - A is mean RCS value

Agreement

To model the effect of polarization for each direct/indirect path:

- Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link

  - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

    - For a LOS ray from Tx to target or from target to Rx, CPM=100-1 for CPMtx,sp or CPMsp,rx

    - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx is generated by XPR ratio  and initial random phases referring to TR 38.901 as start point

    - FFS how to normalize on CPMtx,sp,rx

    - FFS CPMsp of a scattering point of the target

- FFS: how to model the effect of polarization when EO type-2 is present

Agreement

A single direct path is modeled for a scattering point of target

- In each of the Tx-target and target RX links, the first NLOS cluster is generated with same delay as the LOS ray (when the absolute delay modelling of Δτ as in section 7.6.9, TR 38.901 is not applied) and with the same direction as the LOS ray.

  - FFS how to generate NLOS cluster when Δτ is applied

Agreement

In order to generate each of the Tx-target link and target-Rx link in the target channel, the large scale and small scale parameters defined in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, TR 38.858, TR 38.859, TR 38.802, TR 38.854, etc. are used as starting point

Agreement

On the background channel for TRP-TRP and UE-UE bistatic sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as starting point

- Update on values of the LSP/SSP parameters can be discussed based on validation data acquired by measurement or ray-tracing model

  - FFS The power threshold for removing clusters in step 6, i.e., -25 dB is revised to X<-25 dB. FFS X

- FFS whether/how to resolve the inconsistency between TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 when UAV and TRP are set to same height

Agreement

3D spatial consistency needs to be studied for at least UAV scenario

Agreement

In LOS condition between sensing Tx/Rx and target, the power of LOS ray is generated following power of LOS ray in TR 38.901.

Agreement

The following options are to be studied for the concatenation of Tx-target and target-Rx link in the target channel

- Direct path (if present) is always kept

- Indirect paths of LOS+NLOS, NLOS+LOS (if present) are generated

- On other indirect paths of NLOS + NLOS

  - Option 0: ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 0A: ray level full convolution between Tx-target link and target-Rx link only for radio propagation Case 4

  - Option 1: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4

  - Option 1A: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4

  - Option 2: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4

  - Option 2A: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4

  - Option 3: ray level 1-by-1 coupling between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 3A: ray level 1-by-1 coupling between Tx-target link and target-Rx link only for radio propagation Case 4

- Note: reducing the number of rays per cluster and/or reducing the number of clusters can be considered for the options above

- Any indirect path with power metric less than [threshold] is dropped

  - the power metric of a path is the product of power of a ray in Tx-target link, power of a ray in target-Rx link and RCS of the pair of rays

  - FFS power normalization of target channel after path dropping

  - FFS the set of remaining indirect paths can be updated during movement of Tx, target or Rx