3GPP TSG RAN WG1 Meeting #119				      R1-2410205

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.1.2

Source:		Fraunhofer IIS, Fraunhofer HHI

Title:		AI/ML positioning accuracy enhancement

Document for:	Discussion & Decision

# Introduction

The positioning related objectives of the WID in RP-234039 [1] with regards to potential specification impact consists of:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In this contribution, we present our perspectives on issues encountered during previous meetings, including sample-based and path-based measurements; the assistance information required to support training data collection; and the mechanisms of model/functionality monitoring and management.

# Signaling and measurement enhancements

## Path-based and sample-based measurements

The following agreements were reached regarding sample-based and path-based definitions in RAN1#117:

Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

RAN1#118 addressed he determination of Nt' and k for sample-based measurement according to the following agreements:

Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples, - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples. - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation.

Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

Even when the sample-based approach may be viewed as a specific implementation of the path-based approach, sample-based measurements provide critical time-domain granularity, facilitating alignment between the LMF and the reporting entity. On the other hand, path-based measurements ensure consistency of reporting with existing specifications and are favorable for implementations aligned with current releases. Thus, supporting both alternatives allows for a comprehensive framework that enhances measurement accuracy and interoperability.

Proposal 1: 	Support both Alternative (a) sample-based measurement and Alternative (b) path-based measurement report.

Regarding the starting point of the timing window, our preference is for the option using the timing of the first detected path.

Proposal 2:  	The starting time of the list of Nt samples is determined by the first detected path in time in the Nt window

Additionally, relying only on the strongest path might be a suboptimal approach especially in scenario when the identified relevant paths are less than the reported values. The reporting methodology can be enhanced to accommodate various path selection criteria. In most implementations the reporting will benefit from a LOS path as a reliable information among multiple time windows or TRPs.

Proposal 3:  	To minimize discrepancies between training and inference, the desired selection criteria can be signaled:

- First paths

- Strongest paths

- First paths and strongest paths

## Information included in CIR (complex valued correlation function)

The key aspects related to the necessity of complex-valued CIR reporting can be summarized in the following Table:

| Aspect | Advantages | Considerations |
| --- | --- | --- |
| Feasibility, Report generation | The complex valued reporting allows a straight-forward implementation. Data without further processing are forwarded to the LMF. This avoids ambiguity issues and simplifies the definition of the related procedures. | Implementation and specification effort is considered. Definition of measurements without ambiguity may require a detailed specification of the procedures. |
| Future model development | Ensures the availability and suitability of data in case future model development demands different input pre-processing. | Collect as rich data as possible when collecting training data for model development |
| Lossless Channel Information | Performance gains were already demonstrated in SI evaluations. The lossless reporting allows future enhancements. | CIR representation shall enable accurate channel reconstruction of the relevant parts. |
| Data Lifetime | Reporting without ambiguity avoids device specific collection of training data, as well as ambiguity between training and inference. | Optimize utilization of complex-valued CIR data to minimize the need for frequent retraining and data collection. |
| Overhead | CIR reporting requires more data but reduces the need for additional training data from multiple sources. It also provides richer information, beneficial for bandwidth-limited applications. | Potential reduction in overhead common to both CIR and PDP reporting. An optional complex reporting feature allows for omission if not substantial for certain applications |
| Specification Impact | Aligned with agreed delay magnitude reporting. Instead of triplets (delay, magnitude, phase) vectors of I and Q values are reported together with the used sampling period and the time stamp of the first sample. | Scenario specific CIR information or signaling should be down prioritized |

Table 1 - Advantages and Considerations for CIR Reporting

To align with potential enhancements and applicability scenarios, and to prevent the loss of valuable information that could benefit future AI/ML models, it's crucial not to omit phase information in the reports.

Proposal 4: 	Support a complex valued sample-based reporting offering:

- Lossless reporting of the channel impulse response

- Supporting future enhancements of the AI/ML model

# Assistance information needed to support training data collection

## Part A and Part B

The training data can be collected under different configurations for the availability of A and B parts:

- Collecting training data containing only input measurements (part A) (and not GTLs), to develop a channel charting model [3] for example.

- Collecting training data containing more input measurements (part A) than combinations of input measurements and corresponding GTLs (parts A and B), e.g., to utilize for semi-supervised model training.

- Collecting training data that contain parts A and B in all data samples.

Proposal 5: 	Training data collection can be configured to collect any proportion of unlabeled (data samples with only part A) and labeled (data samples with both parts A and B) data.

## Data collection optimization

We distinguish between two phases of training data collection: for initial model development and follow-up model improvement/adaptation (e.g., for fine-tuning).

Since the data collection process is usually the most “expensive” stage of the AI/ML model LCM, it might be beneficial to collect as rich data as possible, when collecting training data for model development.

When collecting training data for a follow-up model improvement/adaptation, we are usually interested in collecting only those data that represent failure cases for the already available model. These data can be collected opportunistically or in an event-based manner:

- Specific input patterns are measured: A specific input pattern (e.g., a specific range of SINR values), for which the model does not perform as expected, is detected.

- Environmental changes are detected: Environmental changes occur, such as a re-arrangement of obstacles in a part of a factory floor that renders a fingerprinting model obsolete in that sub-area. In this case, it makes sense to collect data in the problematic area only, i.e., when the UE or a known PRU is in this sub-area.

- High quality GTLs can be generated: In AI/ML practice, it is not uncommon that the initial AI/ML model has been generated using a plethora of data from different sources, but it needs to be fine-tuned using specific area data to perform adequately in different areas not represented in the training data. Here, a small number of new/area data with high-quality GTLs are required, hence, when the opportunity of creating a high-quality label is presented (e.g., through available PRUs or landmarks), training data collection (for fine-tuning) should be triggered.

- Monitoring events have been reported: an indication of reduced model performance (and possibly switching to a different model or to fallback) is reported. This can imply that under the specific model inputs during inference, the model underperformed, therefore collecting training data under these conditions can be beneficial to ensure model quality through fine-tuning or re-training.

Observation 1: 	Training data collection for initial AI/ML model training and training data collection for follow-up model improvement or adaptation (e.g., for fine-tuning) have different requirements.

Proposal 6: 	Support opportunistic/event-based training data collection for follow-up model improvement/adaptation. Data collection can be triggered for example when:

- A specific input pattern (e.g., a specific range of SINR values), for which the model does not perform as expected, is detected.

- The UE or a known PRU is in a sub-area that has become challenging for the model.

- High quality GTLs can be generated.

- A monitoring event indicating reduced model performance (and possibly switching to a different model or to fallback) has been reported.

# Functionality/Model monitoring and management

To alleviate any confusion on the terminology we use in this section, we refer to the high-level LCM Figure 2 from TR38.843. Here, a functionality/model management component is placed at the center of the LCM, which:

- Takes as inputs relevant information from inference and monitoring sessions

- Provides high-level decisions, such as functionality or model (de-) activation/switching, fallback, model retraining/fine-tuning and model transfer, as outputs

Figure 2 Functional framework for AI/ML for NR Air Interface [2]

At RAN1#118, companies were requested to present their views at RAN1#118-bis on the mechanisms of model/functionality monitoring and management for Cases 1 and 3a. We have distilled our views in the following key topics related to Cases 1 and 3a:

- Which entity is responsible for functionality management in Cases 1/3a?

- Which entity triggers functionality or model performance monitoring in Cases 1/3a?

- Which entity is responsible for the monitoring metric calculation in Cases 1/3a?

- Which entity is responsible for model management in Cases 1/3a?

- Which are the possible decisions regarding functionality/model management in Cases 1/3a?

- How can functionality/model management be more effective in Cases 1/3a?

## Entity responsible for functionality management in Cases 1/3a

RAN2#125-bis reached the following agreement:

Agreements - For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). - “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

In both Case 1 and Case 3a, functionality management (i.e., decision on functionality activation/deactivation/switching or fallback) should be handled by the NW (LMF), due to the extensive information it has on the (radio) conditions in the (AI/ML supported) area. Therefore, we do not see the “UE autonomous, decision reported to the network” option of the RAN2 agreement as viable.

Proposal 7: 	At least for Case 1 and Case 3a, functionality management is provided by the network (LMF).

## Entity that triggers functionality or model performance monitoring in Cases 1/3a

In all cases, the NW-side is responsible for maintaining acceptable positioning accuracy (performance) within each (AI/ML supported) area. One of the mechanisms to achieve this is the ability to switch between positioning functionalities or deactivate the AI/ML feature and switch to classical positioning (fallback).

As different UEs can have different AI/ML model implementations, supported by diverse model management mechanisms in Case 1, the decision to activate a specific AI/ML positioning functionality in an (AI/ML supported) area depends not only on UE capabilities, but also on individual or crowdsourcing data (from several UEs) on the performance of their respective models under different conditions.

For Case 3a, functionality monitoring is still of interest to the NW-side, but we do not see the benefit behind the gNB triggering this monitoring.

Proposal 8: 	At least for Case 1 and Case 3a, functionality monitoring is triggered by the NW (LMF).

When it comes to model monitoring, for Case 1 it can be triggered by the UE (to ensure that the deployed model is still performing as expected). It can also be initiated by the NW (LMF), to monitor a specific functionality (implemented by UE-sided models in Case 1) in a given (AI/ML supported) area. The latter implies that in a NW-initiated UE-sided model monitoring session, the result of the UE model monitoring metric needs to be reported back to the NW.

Proposal 9: 	For Case 1, UE-sided model monitoring can be triggered by the UE or the NW (LMF).

Case 3a is similar – either the gNB or the NW can request a gNB-sided model monitoring session.

Proposal 10: 	For Case 3a, gNB-sided model monitoring can be triggered by the gNB or the NW (LMF).

## Entity responsible for the model monitoring metric calculation in Cases 1/3a

For label-based model monitoring, we have the following agreements for Case 1:

Agreement (RAN1 #116bis) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Conclusion (RAN1 #118) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

For Option A:

- Option A-1: there is a plethora of feasible methods here. Another example that alleviates the signaling and computation burden for the LMF is utilizing landmarks (such as QR codes) for acquiring a reliable position for the UE.

- Option A-2: this option could involve additional signaling from the LMF related to monitoring events, monitoring time intervals, or providing specific assistance data for monitoring purposes.

- Option A-3: in our view, this option can be feasible for model monitoring, even if the UE is not near a PRU. Depending on the distribution of PRUs in an AI/ML supported area, PRU measurements and locations for a subset of these can be sent to the UE. Then, the UE can determine (and/or report to the NW) its (average, worst-case, etc.) model performance in the designated area.

For Option B:

- Option B-1: This is complementary to Option A-1, in cases where the NW (LMF) is not willing to share the exact position calculated in the NW-side.

- Option B-2: In our view, is complementary to Option A-3. It is not necessarily required for the UE to be near a PRU for this monitoring approach. The LMF can provide to the UE, measurements from several PRUs in the AI/ML supported area and the UE can report back the predicted positions. This way, the NW can monitor both the UE-sided model and the activated functionality.

Proposal 11: 	For model performance monitoring of AI/ML positioning, ensure that Rel-19 supports the necessary signaling mechanisms for Options A-1, A-2, A-3, B-1, and B-2, including signaling for ground truth labels, monitoring events, time intervals, and assistance data.

## Entity responsible for model management in Cases 1/3a

For Case 1, Options A-1, A-2, A-3, B-1 and B-2 in the RAN1#116b agreement for model performance monitoring metric calculation are not directly associated with which entity initiates the UE-sided model monitoring session. NW can initiate the monitoring session and request the UE to calculate the model performance monitoring metric (Option A) or the UE can initiate it and request the LMF to support (Option B). Similarly, a different entity than the one calculating the monitoring performance metric can be responsible for the model management.

In Case 1 (Case 3a), the entity responsible for model management is:

- The UE (gNB), when the model has been developed by the UE (gNB) vendor (or vendor collaboration)

- The UE (gNB) or the LMF, when the model has been developed by the NW-side and it is then transferred to the UE (e.g., Z4 model transfer) or the gNB

Proposal 12: 	For Case 1/3a, if the UE/gNB model is proprietary to the NW, model management is performed by the UE/gNB. If the model has been developed by the NW side, model management can be performed either by the LMF or the UE/gNB.

## Possible decisions regarding functionality/model management in Cases 1/3a

The possible decisions regarding functionality/model that follow-up a monitoring session have not been thoroughly discussed, but typical proposals include actions like functionality or model activation/deactivation/switching, deactivation of the AI/ML feature or even model re-training or fine-tuning with new data. It goes without saying that deciding on model re-training and new data collection based on the outcome of a single monitoring performance metric does not make sense; such drastic measures should be the outcome of consistent (both in different locations and time-instances) and multi-faceted monitoring.

As identified in [2], different monitoring metrics can be applied, focusing on input or output data statistics, system performance metric(s) that can be a proxy for the AI/ML model performance or comparison with the true label (e.g., 3D position, ToA/AoA estimation, LOS/NLOS classification, etc.).

The fact is that different monitoring metrics (or analytics) come with different benefits and requirements. Some are easier to apply (e.g., monitor input data) but might not necessarily reflect a problem with the model (for example, for Case 3a, even if input data statistics are “far” from the training data statistics, the model could still reach sufficient accuracy). Other monitoring metrics, like utilizing a ground truth label for monitoring, are far more accurate when it comes to determining an underperforming model, might however necessitate more resources (e.g., the availability of a PRU).

Even when acquiring ground truth labels for monitoring, the label validity, quality, and confidence also depends on the method used for generating the label. For example, a label reliability might differ depending on the source such as a PRU or a landmark (when available) compared to labels generated by other RAT or non-RAT methods.

A reasonable strategy here is to monitor metrics that are easy to infer locally (e.g., input data statistics) and if these indicate a potential problem with the functionality/model, a plurality of monitoring metrics can be further triggered, for determining if a problem with the model is indeed present. Depending on the monitoring quality requirements, more frequent monitoring, with more accurate monitoring metrics (e.g., ground truth labels of the highest quality/confidence) and with more detailed reporting (e.g., F1-score vs accuracy in Case 3a, confidence of monitoring result vs simple problem confirmation, etc.) could be configured.

Proposal 13: 	For both Case 1 and Case 3a consider a two-stage monitoring process: upon identifying of a potential model performance issue through early metrics, activate a broader set of monitoring metrics to confirm the actual presence and impact of the issue.

Once the presence and severity of a model performance issue has been determined, the root cause of the issue needs to be determined, since this is the only way to make informative, long-term decisions regarding the functionality/model suitability, at least in a specific AI/ML supported area. For example:

- If model is applied in an area (or under different reception conditions, such as LOS probability, or hardware assumptions like synchronization or number of TRPs) where it has not been applied before, and training data from the area (or radio environment) were not part of the model training data, a model underperforming might imply that the model is not suitable for the specific area. This could require deactivation of the model (and possibly also the associated functionality) in the specific area, until it is re-trained or fine-tuned with area-specific data.

- If a model is applied in an area (or under a set of conditions, such as a configuration of environment blockers or reflectors) that it is known it should work, but conditions have changed, the model can underperform. The question in this case is whether this is due to a temporal (e.g., a temporary blockage that does not require any model update), a periodic (e.g., an area changes at known or easy-to-determine time periods, which can imply that a dedicated model needs to be trained for these periods) or permanent effect (so the model needs to be updated/re-trained).

Proposal 14: 	The possible actions/decisions of the functionality/model management entity can be:

- More enhanced long-term monitoring for a specific model/functionality:

  - Configuration of event-based or scheduled monitoring in different time intervals, with the flexibility to adjust the monitoring frequency

  - Providing customization for reporting detail, notifying monitoring entity on the level of detail in monitoring reports.

  - Supporting different methods of generating ground truth labels for monitoring purposes, with varying quality of labels

- Functionality or model (de-)activation/switching or fallback (temporary)

- Functionality or model permanent marked as non-applicable in an area, with possible triggering for new data collection

- AI/ML feature permanently deactivated in an area

As these variations are area-specific, it makes sense that the NW (LMF) monitors for such effects and informs the incoming UEs. So, when several functionalities or models (in any positioning case) appear to have performance issues, based on data collected from several UEs, the NW (LMF) can configure a wide-range monitoring campaign, to determine if these problems have temporal, periodic or permanent causes. In any case, the NW can mark areas (and/or time-periods) where a functionality/model is expected to perform well. This information can be made available to UEs entering these areas, while data collection for model update/re-train can focus on areas that are marked as problematic for specific functionalities/models.

Proposal 15: 	Support validity indication for the AI/ML models. The indication shall include at least information about the existence of ML assisted areas.

## How can functionality/model management be more effective in Cases 1/3a

Apart from the UE (Case 1) or gNB (Case 3a) model under-performing, positioning accuracy can also be hindered by an unoptimized model or functionality management component, mainly due to two reasons: i) activation of (or switching to) an unsuitable model or functionality; and ii) high delay when switching between models or functionalities or to fallback.

### Determining suitable models/functionalities for activation or switching

When selecting an initial functionality to be activated, or a functionality to switch to, the expectation is that the specific functionality (through the supported UE/gNB models in Cases 1 and 3a) can ensure a set of QoS KPIs posed by the NW. This means that the UE-/gNB-side AI/ML model that is used in the activated functionality can perform according to certain criteria (e.g., accuracy of predicted position). As the NW (LMF) that decides on the functionality can have limited or no information on the models available at the UE and the NG-RAN node in functionality-based LCM, the decision on which model/functionality combination for every UE/gNB would perform best in each case is not trivial.

Going one step further, the NW (LMF) responsible for functionality management needs to balance the expected performance of applying a specific functionality/model against the potential cost/overhead, especially for the UE-/gNB-side. For example, a UE can support several functionalities in Case 1, but for a given functionality it has a plurality of physical models that have a trade-off between prediction accuracy, (computational and storage) complexity and power consumption. Or it might need to download some models (or parts of models) from the NW or from the User Plane.

Proposal 16: 	In both functionality-based and model-based LCM for Case 1 and Case 3a, UE/gNB provides an expected performance vs cost/overhead information to the LMF that is associated with a functionality or model. The cost/overhead information includes:

      - information on (logical or physical) model properties, such as size, computational complexity or power consumption

      - availability of the physical model on the device

      - latency for activating functionality or model.

Determining when to switch to a functionality (or model) is only part of the story. There is always the case that this switch was not the best option, due to several factors that can only become known if monitoring continues for some time interval after the switch. For example:

- The newly-activated functionality/model underperforms directly or after few inference steps and switching to a different functionality/model or fallback is required;

- There is a constant switching of functionalities/models (ping-pong effect).

- The newly-activated functionality/model has higher complexity/overhead (e.g., model size/complexity, more – and more frequent – measurements required, etc.), but the performance gain is insignificant.

Availability of such “follow-up” performance-related information is critical for assessing the quality of the functionality/model management entity and must be logged and/or reported.

Proposal 17: 	Support monitoring the events resulting from functionality/model management decisions (fallback or (de)activation/switching) for a pre-defined time window.

Proposal 18: 	Support a monitoring report including the outcome (performance, robustness, overhead/cost, etc.) of functionality or model management decisions when required.

### Reducing delays in functionality/models switching

In several occasions it might be crucial for the UE/gNB to make a low latency functionality or model management-related decision to ensure QoS (e.g., fallback when AI/ML model performance is below a threshold), without waiting for explicit confirmation/approval from the NW. To achieve this in a way that the NW (LMF) would maintain decision making responsibility over functionalities, the network configures several functionalities at the UE/gNB (e.g., for switching to different functionality/(logical) model in different areas or radio conditions) and provide a configuration determining when to automatically switch between functionalities or fallback, based on some criteria/threshold values. As the conditions or UE capability change dynamically, the NW (LMF) can update the configuration on demand.

Proposal 19: 	In Cases 1 and 3a, the NW configures multiple functionalities at the UE/gNB and provides a configuration for automatic functionality switching and fallback, depending on specific parameters or threshold values.

# Conclusion

Based on the discussion in the document, the following proposals are presented:

Proposal 1: 	Support both Alternative (a) sample-based measurement and Alternative (b) path-based measurement report.

Proposal 2:  	The starting time of the list of Nt samples is determined by the first detected path in time in the Nt window

Proposal 3:  	To minimize discrepancies between training and inference, the desired selection criteria can be signaled:

- First paths

- Strongest paths

- First paths and strongest paths

Proposal 4: 	Support a complex valued sample-based reporting offering:

- Lossless reporting of the channel impulse response

- Supporting future enhancements of the AI/ML model

Proposal 5: 	Training data collection can be configured to collect any proportion of unlabeled (data samples with only part A) and labeled (data samples with both parts A and B) data.

Proposal 6: 	Support opportunistic/event-based training data collection for follow-up model improvement/adaptation. Data collection can be triggered for example when:

- A specific input pattern (e.g., a specific range of SINR values), for which the model does not perform as expected, is detected.

- The UE or a known PRU is in a sub-area that has become challenging for the model.

- High quality GTLs can be generated.

- A monitoring event indicating reduced model performance (and possibly switching to a different model or to fallback) has been reported.

Proposal 7: 	At least for Case 1 and Case 3a, functionality management is provided by the network (LMF).

Proposal 8: 	At least for Case 1 and Case 3a, functionality monitoring is triggered by the NW (LMF).

Proposal 9: 	For Case 1, UE-sided model monitoring can be triggered by the UE or the NW (LMF).

Proposal 10: 	For Case 3a, gNB-sided model monitoring can be triggered by the gNB or the NW (LMF).

Proposal 11: 	For model performance monitoring of AI/ML positioning, ensure that Rel-19 supports the necessary signaling mechanisms for Options A-1, A-2, A-3, B-1, and B-2, including signaling for ground truth labels, monitoring events, time intervals, and assistance data.

Proposal 12: 	For Case 1/3a, if the UE/gNB model is proprietary to the NW, model management is performed by the UE/gNB. If the model has been developed by the NW side, model management can be performed either by the LMF or the UE/gNB.

Proposal 13: 	For both Case 1 and Case 3a consider a two-stage monitoring process: upon identifying of a potential model performance issue through early metrics, activate a broader set of monitoring metrics to confirm the actual presence and impact of the issue.

Proposal 14: 	The possible actions/decisions of the functionality/model management entity can be:

- More enhanced long-term monitoring for a specific model/functionality:

  - Configuration of event-based or scheduled monitoring in different time intervals, with the flexibility to adjust the monitoring frequency

  - Providing customization for reporting detail, notifying monitoring entity on the level of detail in monitoring reports.

  - Supporting different methods of generating ground truth labels for monitoring purposes, with varying quality of labels

- Functionality or model (de-)activation/switching or fallback (temporary)

- Functionality or model permanent marked as non-applicable in an area, with possible triggering for new data collection

- AI/ML feature permanently deactivated in an area

Proposal 15: 	Support validity indication for the AI/ML models. The indication shall include at least information about the existence of ML assisted areas.

Proposal 16: 	In both functionality-based and model-based LCM for Case 1 and Case 3a, UE/gNB provides an expected performance vs cost/overhead information to the LMF that is associated with a functionality or model. The cost/overhead information includes:

      - information on (logical or physical) model properties, such as size, computational complexity or power consumption

      - availability of the physical model on the device

      - latency for activating functionality or model.

Proposal 17: 	Support monitoring the events resulting from functionality/model management decisions (fallback or (de)activation/switching) for a pre-defined time window.

Proposal 18: 	Support a monitoring report including the outcome (performance, robustness, overhead/cost, etc.) of functionality or model management decisions when required.

# References

- 3GPP RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG-RAN #102, Edinburg, Scotland, Dec 11th – Dec 15th, 2023.

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, V18.0.0 (2023-12).

- Studer, C., Medjkouh, S., Gonultaş, E., Goldstein, T., & Tirkkonen, O. (2018). Channel charting: Locating users within the radio environment using channel state information. IEEE Access, 6, 47682-47698.