3GPP TSG RAN WG1 #119	R1-2410405

Orlando, US, November 18th – 22nd, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on DL coverage enhancement for NR-NTN

Agenda Item:	9.11.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#104 meeting [1], the R19 NR-NTN WID was updated based on the controversial situation in RAN1. For DL coverage enhancement, it was confirmed that SSB periodicity extension is within the scope for system level enhancements. On the other hand, impact to legacy UEs is concerned and further discussion is expected. In this contribution, we share our further views on DL coverage enhancement for NR-NTN.

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

- Discussion

  - System level enhancements

    - SSB periodicity extension vs wider bean footprint for SSB/SIB

At the last meeting, SSB periodicity extension was agreed, and the maximum value is 160 ms. Meanwhile, 320 ms periodicity was proposed by some companies and a possibility to support 320 ms as maximum is still remaining as an FFS point. In our understanding, the main concern not to allow periodicity beyond 160 ms is spec impact including RAN4 spec, which seems to be valid.

Instead of periodicity beyond 160 ms, 160 ms periodicity with wider beam footprint for SSB/SIB can achieve the same. Basically, SSB extension discussion is based on a specific beam footprint size. Alternatively, if wider beam footprint is used, the same is achieved without extending SSB periodicity, and of course with CNR degradation. Although applying wider beam footprint is satellite implementation issue, our understanding of this option is that narrower beam footprint will be used as early as possible, i.e., even during initial access. For this purpose, some signaling enhancement will be necessary.

Fig.1: Wider beam footprint for SSB/SIB.

For details of this approach, there are at least two possible procedures: 1) UE detects the best narrower beam footprint by using location information and transmits only PRACH corresponding to the best one, or 2) UE performs PRACH transmissions for all narrower beam footprints corresponding to the best SSB/SIB wider beam footprint and gNB/satellite responds to the best one via Msg2. Although 1) was proposed by some companies, feasibility to include location information in e.g., SIB may be questionable. The second procedure or any others, if any, should be listed first and all should be discussed carefully.

Proposal 1:

- Conclude 160 ms SSB periodicity as the maximum.

Proposal 2:

- RAN1 assumes that SSB/SIB beam footprint can be wider than beams for PRACH and/or subsequent channels/signals.

  - At least the following two approaches are discussed.

    - UE detects the best narrower beam footprint based on location information and responds only to the best one via PRACH.

    - UE responds to all narrower beams corresponding to the best wider SSB/SIB beam via PRACH transmissions and gNB/satellite responds to the best one via Msg2.

    - Other problems to support dynamic and flexible power sharing or different satellite beam patterns/size

In the current specifications, there are a lot of beam-related mechanisms. However, some of them will not work in case of dynamic and flexible power sharing or different satellite beam patterns/size.

- Cell DTX/DRX parameters

The existing cell DTX/DRX parameter (i.e., cellDTXDRX-Config) seems not workable in NTN. R19 NR NTN discussion has been done under the assumption that cell DTX/DRX timing is different among beams. It is impossible that all satellite beams are active simultaneously, thus this assumption is baseline in any NTN scenario. Meanwhile, the cell DTX/DRX parameter was introduced in R18 NES WI and are defined not per beam but per cell. Motivation of the parameter is to allow gNB to sleep. That is, all beams are expected to be ‘OFF’ during a time duration. This is not aligned with motivation of DTX/DRX of NW side in NTN. For NTN, beam DTX/DRX parameter is necessary.

Proposal 3:

- Configuration of cellDTXDRX-Config is provided per beam (e.g., per SSB index).

- Channel overlapped with SSB in different beam

In the current specification, when PDCCH/PDSCH RX or PUSCH TX is overlapped with SSB, the PDCCH/PDSCH RX or PUSCH TX is dropped and SSB is prioritized, regardless of which beam is used for each. Simultaneous SSB and other TX/RX in different beams are not assumed for TN, so this prioritization is reasonable. However, in R19 NTN discussion, some satellite beams are active simultaneously and it is feasible that for instance, SSB is transmitted in a satellite beam and data TX/RX is performed in another satellite beam. In this situation, unnecessary dropping/skipping behavior will be performed due to the mechanism defined for TN. This issue should be avoided.

It is noted that this behavior exists even in R17/18 NTN, but it is not considered as a problem. In R17/18 NTN discussion, the number of simultaneous active beams and available satellite transmit power are assumed much larger. Flexible scheduling is possible without such an unnecessary dropping behavior. However, this is not the case in R19 NTN. In our understanding, it is necessary to consider a situation where SSB in a satellite beam is overlapped with other TX/RX in another satellite beam, and a UE does not monitor the SSB and would perform the TX/RX, for dynamic and flexible power sharing or different satellite beam patterns/size.

Fig.2. Overlap between SSB and SIB19 in different beams.

Proposal 4:

- Discuss whether it is feasible that SSB and other TX/RX are overlapped in different beams from UE perspective and the TX/RX should be prioritized at a UE.

  - If feasible, discuss what kind of prioritization is introduced.

- SSB transmission power among beams

Now we are considering support of dynamic and flexible power sharing or different satellite beam patterns/size. In this consideration, SSB power may be different among beams depending on communication status in the system. Meanwhile, a note is that changing SSB power may not be feasible due to negative impact to measurement-related behavior.

If SSB power could be different among SSB beams, parameters relevant to SSB beam need to be discussed. In the current specifications, ss-PBCH-BlockPower / powerControlOffsetSS / PowerControlOffset can be provided for appropriate CSI acquisition or PDSCH demodulation. These parameters are defined not per SSB beam but per cell. It is expected that SSB power is constant among beams in TN, and thereby there was no motivation to define them per SSB beam. However, the parameters should be provided per SSB beam to support different power among SSB beams

Proposal 5:

- Discuss whether SSB transmit power can be different among beams.

  - Link level enhancements

    - PDCCH enhancement

At the last meeting, RAN1 agreed that PDCCH enhancement will be introduced in R19 NTN, for all CSS types except for type 3. Three alternatives were raised, and further study is required. Besides, CSS type 3 and USS may also be the enhancement target, may not. Further clarification is necessary.

Regarding enhancement mechanisms, we believe that PDCCH repetition is the most reasonable choice since this has been introduced in R17 MIMO WI for multi-TRP scenario. Meanwhile, it may be difficult to support the completely same approach. In R17 PDCCH repetition, two search space configurations are provided, and they have the same periodicity, monitoring DCI format, etc. Under the conditions, the same search space linking ID is provided for each of the two search space configurations, and with that, PDCCH repetition can be transmitted on these two search spaces.

However, at least for PDCCH to schedule SIB1 PDSCH, only a single search space configuration is available in the current specification, which is provided via MIB/PBCH. There is no sufficient room to add one more search space configuration and also search space linking ID. Therefore, we should find another mechanism to determine two search spaces for PDCCH repetition to schedule SIB1 PDSCH.

Observation 1:

- R17 PDCCH repetition uses two separate search space configurations and also search space linking ID configurations, which cannot be provided via MIB/PBCH for PDCCH to schedule SIB1 PDSCH, due to lack of available bits in MIB/PBCH.

To solve this problem, two linked search spaces should be determined according to a single search space configuration. For example, the search spaces with red color below are provided by the legacy parameter, while a dedicated configuration is not defined for the search spaces with green color; instead of that, e.g., subsequent adjacent symbols of the search space with red color are always used for the search spaces with green color. Only one bit to indicate ‘ON’ of the search spaces with green color and thus PDCCH repetition is what MIB/PBCH needs to provide. For this ‘ON’ indication, a spare bit in MIB may be available while using this valuable bit will be allowed only if the feature is essential and there is no other choice. PDCCH enhancement for NTN will meet these requirements.

For the other PDCCH, the same kind of parameters as in R17 can be provided, e.g., via SIB1. We do not see any problem to take the same approach.

Proposal 6:

- Support PDCCH repetition for all CSS types except for type 3.

Proposal 7:

- For PDCCH repetition to schedule SIB1 PDSCH,

  - Two linked search spaces are determined according to a single configuration (searchSpaceZero), e.g., at adjacent symbols.

  - The linked search spaces are associated with the same CORESET (controlResourceSetZero).

  - A spare bit in MIB indicates enabling of PDCCH repetition for SIB1 PDSCH reception.

Proposal 8:

- For PDCCH repetition for all CSS types except for SIB PDSCH scheduling and type 3,

  - Reuse R17 mechanism.

    - i.e., two search space configurations are provided, and they are linked by search space linking configurations.

For CSS type 3 and USS, at the last meeting, it was argued that the current spec supports PDCCH repetition for CSS type 3 and USS regardless of single-TRP or multi-TRP. We checked the current specifications carefully and we have the same understanding as the above argument.

However, there seems to be a problem; R17 UE capability signaling is not suitable for R19 NTN. Several FGs were defined for R17 PDCCH repetition. In those capabilities, there is no differentiation between single-TRP scenario and multi-TRP scenario. That is, support indication via R17 UE capability signaling means that the UE supports the feature in both single-TRP scenario and multi-TRP scenario, which is unnecessary requirement for NTN scenario. To solve this problem, UE capability signaling must be enhanced so that UE can indicate support of PDCCH repetition only in single-TRP scenario. For example, separate capability signaling only for single-TRP scenario is defined and R19 NTN UE can use this new capability instead of R17 capability signaling.

Table 1: R17 FGs for PDCCH repetition.

| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs including PDCCH repetition for Type 3 CSS 2. Required number of BDs for the two PDCCH candidates 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot |  | mTRP-PDCCH-Repetition-r17 { numBD-twoPDCCH-r17, maxNumOverlaps-r17, } | FeatureSetDownlink-v1700 | n/a | n/a | n/a | Component 2 candidate values: 2 or 3 Component 3 candidate values: {1,2,3, 5, 10, 20, 40} NOTE 1: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot. NOTE 2: for component 3, each unique pair of overlaps is counted as one. NOTE 3: This FG does not include supporting Two QCL-TypeD in time-domain overlapping CORESETs in FR2. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | 23-2-1 | mTRP-PDCCH-individual-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1b | PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | Support of PDCCH repetition for PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | 3-2 23-2-1 | mTRP-PDCCH-anySpan-3Symbols-r17 | MIMO-ParametersPerBand | n/a | FR1 only | n/a | Applicable to 15KHz SCS only | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1c | PDCCH repetition with PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot | Support of PDCCH repetition for PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot | 22-12 23-2-1 | mTRP-PDCCH-singleSpan-r17 | MIMO-ParametersPerBand | No | FR1 only | No | Applicable to 15KHz SCS only | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1d | PDCCH repetition for Case 2 PDCCH monitoring with a span gap | 1. Support of PDCCH repetition for PDCCH monitoring of any occasions with span gap as defined in FG 3-5b. 2. Supported mode of PDCCH repetition 3. X per CC 4. X across all CCs | 3-5b, 23-2-1 | mTRP-PDCCH-Case2-1SpanGap-r17 { scs-15kHz-r17, scs-30kHz-r17, scs-60kHz-r17, scs-120kHz-r17 } | FeatureSetDownlink-v1700 | n/a | n/a | n/a | This capability is necessary for each SCS. Component 2 candidate values: {intra-span, inter-span, both} Component 3 candidate values: {4, 8, 16, 32, 44, 64, no limit} Component 4 candidate values: {4, 8, 16, 32, 44, 64, 128, 256, 512, no limit} NOTE: - Components 3 and 4 are reported only if UE supports inter-span PDCCH repetition. - The limit (X) is associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span, where "received" and "not been received" is wrt the end of the corresponding span of PDCCH candidate. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. - Candidate value "no limit" does not imply BD limit can be exceeded. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1e | PDCCH repetition for Rel-16 PDCCH monitoring | 1. Support of PDCCH repetition with Rel-16 PDCCH monitoring capability as defined in FG 11-2 family. 2. Supported mode of PDCCH repetition 3. X per CC 4. X across all CCs | 11-2, 23-2-1 | mTRP-PDCCH-legacyMonitoring-r17 { scs-15kHz-r17, scs-30kHz-r17 } | FeatureSetDownlink-v1700 | n/a | n/a | n/a | This capability is signalled for SCS 15 kHz and 30 kHz. Component2: {intra-span, inter-span, both} Component3: {4, 8, 16, 32, 44, 64, no limit} Component 4: {4, 8, 16, 32, 44, 64, 128, 256, 512, no limit} NOTE: - Components 3 and 4 are reported only if UE supports inter-span PDCCH repetition. - The limit X is associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span, where "received" and "not been received" is w.r.t. the end of the corresponding span of PDCCH candidate. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. - Candidate value "no limit" does not imply BD limit can be exceeded. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition | 23-2-1 | mTRP-PDCCH-TwoQCL-TypeD-r17 | MIMO-ParametersPerBand | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-4 | Simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP | Support of simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP | 23-2-1, 16-2a | mTRP-PDCCH-multiDCI-multiTRP-r17 | FeatureSetDownlink-v1700 | n/a | n/a | n/a | Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values | Optional with capability signalling |

Observation 2:

- R17 PDCCH repetition can be applied to both single-TRP scenario and multi-TRP scenario.

- UE capability signaling for R17 PDCCH repetition does not have any differentiation between single-TRP scenario and multi-TRP scenario, which means that UE reporting the capability signaling as ‘support’ must support both scenarios.

Proposal 9:

- For CSS type 3 and USS, define UE capability signaling of PDCCH repetition in case of single-TRP scenario.

In addition, in R17 PDCCH repetition, two patterns of PDCCH decoding behavior were assumed.

- A: Two blind decoding

Decode of the first PDCCH + Decode of the second PDCCH

- B: Three blind decoding

Decode of the first PDCCH + Decode of the second PDCCH + Decode of a soft-combined PDCCH

R17 PDCCH repetition has been specified assuming multi-TRP scenario, thereby these behavior considerations would be reasonable. Either may fall into low SNR, e.g., due to blocking in FR2, while the other may keep higher SNR in LOS. In this case, A above works well.

On the other hand, in R19 NTN, PDCCH repetition is used to boost SNR by soft-combining in single-TRP scenario. In this case, what UE should do is only decode of a soft-combined PDCCH and thus decode of either PDCCH may not be necessary. In summary, it may be better to reconsider definition of blind decoding burden at UE for PDCCH repetition.

Observation 3:

- R17 PDCCH repetition is assumed with the following two decoding behavior, which may not be reasonable in R19 NTN.

  - A: Two blind decoding (Decode of the first PDCCH + Decode of the second PDCCH)

  - B: Three blind decoding (A + Decode of a soft-combined PDCCH)

Proposal 10:

- Discuss whether R17 blind decoding burden of PDCCH repetition can be reused for R19 NTN.

    - SIB1/SIB19(/other SIBs) PDSCH enhancement

As discussed and included in the agreements on PDCCH/Msg4 PDSCH, the target CNR is set to -8 dB. Then in consideration of the required SNRs for SIB1/SIB19, which were agreed as observations previously, SIB1/SIB19(/other SIBs) PDSCH should also be enhanced. As the solution, introducing repetition feature would be the straightforward approach.

For details of repetition, RAN1 should care system level problem, for discussion on link level enhancement. The problem is that available slots for each beam footprint are only a few. Just support of conventional repetition-type approach may not be enough. Given that, we suggest discussing the illustrated approach below. In this illustration, both slot w/ SSB and slot w/o SSB are used for SIB transmissions, without unused symbols. That is, A) below is the conventional way and B) below is our considering approach.

A): SIB is transmitted starting from a slot without SSB

B): SIB is transmitted starting from a slot with SSB and continued with different length at the subsequent slots without SSB

Fig.3. More efficient use of slots with/without SSB.

We share evaluation results for repetition with different length per repetition. Parameters for SIB1/SIB19 PDSCH with different length can be found below.

Table 2: Simulation parameters for SIB1/SIB19 PDSCH – normal repetition vs repetition with different length

| Parameter | SIB1 |  | SIB19 |  |
| --- | --- | --- | --- | --- |
|  | Normal rep | Rep w/ different length | Normal rep | Rep w/ different length |
| Target metric | 10% iBLER |  |  |  |
| Center frequency | 2 GHz |  |  |  |
| Frequency bandwidth | 5 MHz |  |  |  |
| SCS | 15 kHz |  |  |  |
| Frequency offset (ppm) | 0.1 |  |  |  |
| Doppler frequency drift (ppm/s) | 0.27 |  |  |  |
| Channel model | NTN-TDL-C |  |  |  |
| UE speed | 3 km/h |  |  |  |
| PRB allocation | 24 |  |  |  |
| Symbol allocation (S, L) | (2, 12) | Slot w/ SSB: (6, 8) Slot w/o SSB: (0, 14) | (2, 12) | Slot w/ SSB: (6, 8) Slot w/o SSB: (0, 14) |
| Slot aggregation | 1, 2, 4 |  | 1, 2 |  |
| PDSCH mapping type | Type A | Slot w/ SSB: Type B Slot w/o SSB: Type A | Type A | Slot w/ SSB: Type B Slot w/o SSB: Type A |
| DM-RS configuration type | Type 1 |  |  |  |
| DM-RS Length | 1 |  |  |  |
| DM-RS symbol | [2, 6, 9] | Slot w/ SSB: [6, 12] Slot w/o SSB: [2, 7, 11] | [2, 6, 9] | Slot w/ SSB: [6, 12] Slot w/o SSB: [2, 7, 11] |
| FDM b/w data and DMRS | No |  |  |  |
| TBS | 1344 |  | 616 |  |

Fig.4. Illustrations – normal repetition vs repetition with different length (2 repetitions case)

Fig.5. Link level simulation results (SIB1/SIB19 PDSCH) – normal repetition vs repetition with different length.

As you can see, although a small performance difference is observed, the difference is marginal. Therefore, we can assume that the mechanism of repetition with different length is beneficial to reduce one slot required for this beam.

Proposal 11:

- Support SIB1/SIB19(/other SIBs) PDSCH repetition.

  - Scheduling DCI indicates repetition factor.

  - Support repetitions w/ different length per repetition.

    - Msg4 PDSCH enhancement

Although Msg4 PDSCH repetition was close to agreement, it was blocked – it was argued that HARQ-based rescheduling should be used instead of repetition. However, we believe that this argument is not reasonable since for coverage enhancement, Msg3 PUSCH rep / Msg4 PUCCH rep were introduced as an efficient/practical way even though rescheduling for these had been supported already. We do not see any reason why repetition of these channels is allowed while repetition of Msg4 PDSCH should be prohibited. In NTN, disadvantage of HARQ-based rescheduling is even larger than in TN due to its much larger RTT.

For details of repetition, discussion/analysis/argument are the same as those for SIB1/SIB19(/other SIBs). Therefore, this proposal is submitted.

Proposal 12:

- Support Msg4 PDSCH repetition.

  - Scheduling DCI indicates repetition factor.

  - Support repetitions w/ different length per repetition.

Simulation evaluation results are shown below for reference, which is copy from our contribution at the last meeting [2].

Fig.6. Link level simulation results (Msg4 PDSCH) – normal repetition vs repetition with different length.

- Conclusion

In this contribution, we discussed DL coverage enhancement for NR-NTN. Observations/Proposals are summarized as following:

Proposal 1:

- Conclude 160 ms SSB periodicity as the maximum.

Proposal 2:

- RAN1 assumes that SSB/SIB beam footprint can be wider than beams for PRACH and/or subsequent channels/signals.

  - At least the following two approaches are discussed.

    - UE detects the best narrower beam footprint based on location information and responds only to the best one via PRACH.

    - UE responds to all narrower beams corresponding to the best wider SSB/SIB beam via PRACH transmissions and gNB/satellite responds to the best one via Msg2.

Proposal 3:

- Configuration of cellDTXDRX-Config is provided per beam (e.g., per SSB index).

Proposal 4:

- Discuss whether it is feasible that SSB and other TX/RX are overlapped in different beams from UE perspective and the TX/RX should be prioritized at a UE.

  - If feasible, discuss what kind of prioritization is introduced.

Proposal 5:

- Discuss whether SSB transmit power can be different among beams.

Observation 1:

- R17 PDCCH repetition uses two separate search space configurations and also search space linking ID configurations, which cannot be provided via MIB/PBCH for PDCCH to schedule SIB1 PDSCH, due to lack of available bits in MIB/PBCH.

Proposal 6:

- Support PDCCH repetition for all CSS types except for type 3.

Proposal 7:

- For PDCCH repetition to schedule SIB1 PDSCH,

  - Two linked search spaces are determined according to a single configuration (searchSpaceZero), e.g., at adjacent symbols.

  - The linked search spaces are associated with the same CORESET (controlResourceSetZero).

  - A spare bit in MIB indicates enabling of PDCCH repetition for SIB1 PDSCH reception.

Proposal 8:

- For PDCCH repetition for all CSS types except for SIB PDSCH scheduling and type 3,

  - Reuse R17 mechanism.

    - i.e., two search space configurations are provided, and they are linked by search space linking configurations.

Observation 2:

- R17 PDCCH repetition can be applied to both single-TRP scenario and multi-TRP scenario.

- UE capability signaling for R17 PDCCH repetition does not have any differentiation between single-TRP scenario and multi-TRP scenario, which means that UE reporting the capability signaling as ‘support’ must support both scenarios.

Proposal 9:

- For CSS type 3 and USS, define UE capability signaling of PDCCH repetition in case of single-TRP scenario.

Observation 3:

- R17 PDCCH repetition is assumed with the following two decoding behavior, which may not be reasonable in R19 NTN.

  - A: Two blind decoding (Decode of the first PDCCH + Decode of the second PDCCH)

  - B: Three blind decoding (A + Decode of a soft-combined PDCCH)

Proposal 10:

- Discuss whether R17 blind decoding burden of PDCCH repetition can be reused for R19 NTN.

Proposal 11:

- Support SIB1/SIB19(/other SIBs) PDSCH repetition.

  - Scheduling DCI indicates repetition factor.

  - Support repetitions w/ different length per repetition.

Proposal 12:

- Support Msg4 PDSCH repetition.

  - Scheduling DCI indicates repetition factor.

  - Support repetitions w/ different length per repetition.

References

- RP-241667		Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3		Thales, CATT

- R1-2409037	Discussion on DL coverage enhancement for NR-NTN		NTT DOCOMO, INC.

Agreements

Agreement

For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1:

| LEO600km Set1-1 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size(Note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 31.24 |
| Aggregated EIRP (Total) (dBW) | 61.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints*** | 1058 |
| Total number of simultaneously active beams ** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set1-2 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 23 |
| Aggregated EIRP (Total) (dBW) | 53* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 16 |
| % simultaneously active beams** | 1.5 % |
| *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set 1-3 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 |
| Payload Total DL power level (dBW) | 23.24 |
| Aggregated EIRP (Total) (dBW) | 53.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band):

| LEO600km Set1-1 FR2 (i.e., Ka-band) |
| --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Agreement

Adopt the following phased array antenna parameters for LEO 600km in FR1:

| Satellite phased array antenna Characteristics | LEO-600 |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | [65] for H [65] for V |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | [400 elements (20 x 20)] |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | [4dB] |

Agreement

RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level:

At least:

- CDF of the received SINR

- The dwell time and revisit time interval for each beam illumination across the coverage

- Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio

Other metrics may be reported such as

- CDF of the cell throughput

- CDF of user perceived throughput (UPT)

- CDF of Latency

- Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889)

For system level study based on analytical evaluation:

- N1 beam footprints are in state “off”

  - These beam footprints are not served by any signal (no satellite service in this area)

- N2 beam footprints are in state “common messages only”

  - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access.

  - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation

- N3 beam footprints are in state “active traffic”

  - These beam footprints have X active (e.g. VoNR) users each.

  - These beam footprints are also served the necessary information for cell discovery and initial access

- N1 + N2 + N3 = “Total number of beam footprints “

- N1, N2, N3, X are to be reported by companies.

- Resource utilization obtained under the assumptions above is to be reported by companies.

- Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Agreement

For NR NTN Rel-19 DL coverage evaluation, UE characteristics for handheld terminals in Table 6.1.1.1-3 in TR 38.821 can be reused, with the following:

- -5.5 dBi antenna gain is assumed

- at least 2Rx are considered at the UE

  - 4Rx can be optionally considered and reported

Note: Redcap device is not considered in the scope of DL coverage study

Agreement

The following traffic models are considered for system level evaluation of DL coverage:

- FTP3: as in Table 6.1.1.1-7 of TR 38.821: 0.5MB as packet size, 200ms as mean inter-arrival time

- FTP3 IM: 0.1MB as packet size, 2s as mean inter-arrival time

- VoIP can be considered in the evaluation.

It is up to company report which traffic model is used among the discussed traffic models in their evaluations.

- Other models may be used as well, and parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported.

| Traffic type | FTP | IM | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in Rel-18 NTN CE. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |

Agreement

For NR NTN Rel-19 DL coverage evaluation, Beam layout defined in Table 6.1.1.1-4 in TR 38.821 can be reused.

- Using other beam layouts is not precluded, and should be reported by companies

Agreement

For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to 0 at least if phase array antenna is assumed.

Values different from 0 can be optionally reported

Agreement

DL coverage is evaluated at link level with the following considerations:

- NGSO at LEO-600 operating in FR1 is considered in priority

- Additional satellite payload parameters defined for system level evaluation are used

- FFS: Antenna gain reduction due to steering loss can be considered

Agreement

For the evaluation of NTN downlink coverage at link level, reuse the target data rate from Rel-18 NTN Coverage enhancements:

- For VoIP: AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval

- For data rate service: both 3 kbps and 1Mbps can be considered

  - Companies can also use the data rates corresponding to the traffic types used for system level evaluations

Agreement

For link-level study, downlink coverage performance in NR NTN is evaluated according to the following steps.

Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821

Step 2: Required SNR of target service is evaluated by LLS

Step 3: The CNR and the required SNR are compared

Agreement

For link-level study, for NR NTN DL coverage enhancement, the following channels/signals can be considered for evaluations:

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDSCH carry SIB, e.g., SIB1, SIB 19

- PDSCH for paging

- PDCCH

- Broadcast PDCCH (e.g. PDCCH of Msg.2, paging)

- SSB

Note: RAN1 will aim to identify necessary link-level enhancements for these channels in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential link-level enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread:

  - Channel model as in Table 6.1.2-4 of TR38.821, NTN-TDL-C (LOS)

- Evaluation scenario:

  - Rural (LOS)

- Channel estimation: Realistic estimation:

  - Companies are encouraged to report channel estimation method.

- SCS:

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: TBD

- Frequency offset: 0.1 ppm

Agreement

For link budget calculation, parameters in the following table are assumed:

| Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz for DL (S-band) |
| Satellite altitude | 600 km |
| Target elevation angle | 30° (LEO) |
| Atmospheric loss | Equation (6.6-8) in [38.811] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [38.811] Ionospheric loss: = 2.2 dB Tropospheric loss: Table 6.6.6.2.1-1 of [38.811] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| UE antenna gain | -5.5dBi |
| Free space path loss | Equation (6.6-2) in [38.811] |
| Polarization loss | 3dB |
| Outcome | CNR |

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 8 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 24 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. 24 PRBs for SIB1 and SIB19 |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.2 | 72 bits |
| Payload size for PDSCH of SIB1 | FFS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Payload size for PDSCH of SIB19 | FFS |
| Other parameters | Reported by companies. |

Agreement

Antenna gain reduction due to steering loss is not considered in the link level evaluation.

Note: This is aligned with the assumptions made in Rel-18 UL coverage enhancement

Observation

The CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively.

Agreement

Confirm the Satellite phased-array antenna parameters for LEO 600km in FR1 defined in RAN1#116.

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 65 for H 65 for V |
| Antenna element spacing | 0.667 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 400 elements (20 x 20) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | 4 dB |

Al least the above model is considered for SLS to ease the alignment between evaluation results. The model below can be optionally considered:

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | TR38.820 section 7.2.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V |
| Antenna element spacing | 0.5 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 676 elements (26 x 26) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi (Note 1) |
| Steering loss at 30° elevation angle | 2.5 dB |

Note 1: The maximum antenna gain is determined by considering an overall array efficiency [of 50%.]

Agreement

For coverage evaluation of PDSCH in NR NTN, the following payload sizes for PDSCH are assumed:

| Payload | value |
| --- | --- |
| Payload size for PDSCH of SIB1 | Option 1: 800 bits Option 2: 1280 bits |
| Payload size for PDSCH of SIB19 | 616 bits |

Note: At least the above values are simulated and reported. Other values can be considered.

Note: the values above are not the TBS.

Agreement

For DL coverage study at system level, it is up to companies to report the following parameters for LEO600km Set1-1 FR2:

Beam size

Satellite EIRP density /beam (dBW/MHz)

Payload Total DL power level (dBW)

Aggregated EIRP (Total) (dBW)

Satellite Tx max Gain

EIRP per Satellite beam (dBW)

Agreement

For coverage performance evaluation of DL channels/signals before the SIB19 acquisition, the maximum Doppler frequency drift is assumed to be equal to 0.27 ppm/s based on TR 38.821.

Observation

Based on LLS results on PDCCH coverage evaluation collected from different sources:

- It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1.

      - The coverage margin is around 4 dB compared to CNR of -1.9 dB.

  - With parameter LEO600km Set1-3 FR1:

    - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average compared to CNR of -9.9 dB.

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDCCH.

Observation

Based on LLS results on PDSCH Msg2 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg2 is in average equal to – 10.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 9 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 12 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 1 dB on average compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg2.

Observation

Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg4:

      - The coverage margin is around 3.3 dB on average compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with Msg4:

    - The coverage gap is around 4.7 dB on average compared to CNR of -9.9 dB

  - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg4.

Observation

Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources:

- For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources)

- For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2FR1:

  - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is around 3.9 dB on average compared to CNR of -1.9 dB

  - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2:

      - The coverage margin is around 1.5 dB on average compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1:

      - The coverage gap is around 4.1 dB on average compared to CNR of -9.9 dB

  - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is 3.4 dB compared to CNR of -9.9 dB

  - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2:

      - The coverage gap is around 6.5 dB on average compared to CNR of -9.9 dB

- Note: some results assumed SIB1 combination (where SIB1 is repeated within 160 ms) and some results assumed no SIB1 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB1.

Observation

Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with SIB19:

      - The coverage margin is around 4.2 dB on average compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 10 sources observed that there is a coverage gap for PDSCH with SIB19:

      - The coverage gap is around 3.5 dB on average compared to CNR of -9.9 dB

- Note: all the results above assumed no SIB19 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB19.

Observation

Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km:

- For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied.

- Note: coverage ratio is N2+N3/ total beam footprints

- Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs.

Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint:

- 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%.

- 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively.

  - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams.

- 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%.

  - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth.

Note: RAN1 will further investigate the impact of SSB periodicity extension

Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary

Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...)

Observation

Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources)

- With parameter LEO600km Set1-1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 9.1 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 2.3 dB on average, compared to CNR of -9.9 dB

  - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH for VoIP.

Observation

Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 9.1 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 1.6 dB on average, compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH 3kbps.

Observation

Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps:

  - The coverage margin is around 2.2 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps:

  - The coverage gap is around 5.5 dB on average, compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH 1Mbps.

Observation

Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints:

- With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used.

- With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used.

- Note: the overhead of SIB19 was included in some of the results

- Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation

Agreement

As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider:

- Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

  - Default value[s] with extended periodicity assumed by NTN UE for initial access can be:

    - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms}

- Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

- Link-level enhancements for the following channels:

  - PDCCH

  - PDSCH with Msg 4

  - PDSCH with SIB1/SIB19.

    - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact.

  - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

Agreement

For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

- The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms.

  - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

Agreement

Support PDCCH CSS Link level enhancement in Rel-19 for all CSS types except type 3.

- The following techniques are for further study:

  - PDCCH repetition, including:

    - Option 1: Intra-slot PDCCH repetition

    - Option 2: Inter-slot PDCCH repetition

  - CORESET length (i.e. number of OFDM symbols) extension

  - DCI format optimization (e.g. size reduction, etc)

- Note: the same technique is intended to apply to all search space types targeted for link level enhancements

- For the above techniques, at least the following aspects should be discussed for the relevant candidate techniques:

  - Configuration

  - Backward compatibility and UE behaviour of legacy UE

  - Linked Search Space

  - Blocking probability

  - DCI format size budget

  - Resource overhead

  - Impact on CORESET0

- Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

- FFS: whether to apply the selected solution to PDCCH CSS type3 and PDCCH USS

Agreement

For PDSCH with Msg4 Link level enhancement:

- Continue studying PDSCH repetition

  - Further discuss the specification impact for at least the following:

    - Procedure and signaling

    - Repetition factor

  - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.