3GPP TSG RAN WG1 #119                               R1-2410544

Orlando, US, November 18th – 22rd, 2024

Agenda Item:	9.3.2

Source: 	ASUSTeK

Title:  	Discussion on SBFD random access operation

Document for:	Discussion and Decision

- Introduction

In this document, we first discuss the collision handling that will need when valid legacy ROs across non-SBFD symbol(s) and SBFD symbol(s) in the meantime. The next is PRACH mask indication for CFRA triggered by PDCCH order for SBFD-aware UEs.

- Discussion

2.1 Collision handling

In previous RAN1 meetings, there are some agreements.

Agreement [1]

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

Agreement [2]

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols.

- Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon).

  - FFS the details

- FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s).

Conclusion [1]

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1:

- The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

- It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

  - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs.

Agreement [1]

A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default.

A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

Agreement [3]

Extend the previous agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode.

Agreement [3]

For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following:

- For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

- For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

It was confirmed that the RO is valid for legacy UE and should also be applicable to SBFD-aware UE. Furthermore, if the non-SBFD symbols are indicated as flexible symbols through the tdd UL-DL-ConfigurationCommon, the legacy RO (i.e., the RO valid for legacy UE) must be in a usable uplink sub-band in accordance with RACH Configuration Option 1 and Alt. 1-1. This ensures that the ROs associated with SSBs between legacy UE and SBFD-aware UE do not result in discrepancies, as illustrated in Figure 1.

Figure 1: The mismatching situation between different UEs in the same cell due to without the restriction which is confirm the legacy ROs in flexible symbol must locate in the UL sub-band range.

Conversely, there are currently no specific rules analogous to those for RACH Configuration Option 1 that apply to RACH Configuration Option 2, which may require similar constraints. The absence of such constraints could lead to complications within the mechanism. For instance, if legacy ROs are in an unusable uplink or downlink sub-band for SBFD-aware UEs, the network typically categorizes these ROs as invalid. Consequently, while these ROs may be valid for legacy UEs, they will be deemed invalid for SBFD-aware UEs. This discrepancy may result in a mismatch in the SSB-RO mapping between legacy UEs and SBFD-aware UEs.

Proposal 1:  RACH Configuration Option 2 need to constraint that, when legacy ROs are configured in symbols overlapping with SBFD symbols indicated as flexible by the tdd-UL-DL-ConfigurationCommon, the network ensures these ROs are within the usable uplink PRBs.

In accordance with Proposals 1, we consider the scenario in which RACH Configuration Option 2 is implemented, a RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default, and there is a legacy RO is configured in symbols that covers SBFD symbols indicated as flexible by the tdd-UL-DL-ConfigurationCommon. The network must ensure that this RO falls within the usable uplink PRBs. However, as illustrated in Figure 2, the RO is across both SBFD and non-SBFD symbols. It is necessary to determine whether this RO should be classified as valid or invalid, as no detailed or appropriate guidelines currently exist to address this issue. Therefore, it is imperative to resolve this conflict.

Figure 2: There is a legacy RO which will be across different type symbols in SBFD-aware UE and make RO validation conflict.

The first handling option involves the network preventing any collision by ensuring that if a legacy RO has already been determined valid through legacy RACH validation, the network will not permit that RO, within the same cell, to be across both SBFD and non-SBFD symbols for SBFD-aware UEs. This approach is the simplest but also the most restrictive. Since RACH configurations are determined by a predefined table, this restriction limits network actions and reduces the flexibility of RO configuration.

The second handling option is to treat any RO that meets the conflicting condition as invalid. The legacy RO will first be validated through the legacy RACH validation process, followed by the mapping of SSB to the corresponding valid ROs. The collision described may still occur at this stage. To address this, an additional step is introduced: a secondary RACH validation, which checks whether the RO is across both non-SBFD and SBFD symbols. If any RO crosses these different types of symbols, it will be re-determined as invalid. This approach does not interfere with the original mapping relationship between legacy ROs and SSBs, ensuring that SBFD-aware UEs retain the same configuration as legacy UEs.

Proposal 2:  For handling the collision of legacy ROs on SBFD-aware UEs:

- Option 1: If there is a legacy RO has already been determined as valid RO by legacy RACH validation, Network will not allow the legacy RO in the same cell be on across SBFD and non-SBFD symbols, e.g., for SBFD aware UE.

- Option 2: If the valid legacy RO determined by legacy RACH validation is across SBFD and non-SBFD symbol at the same time, it will be further re-determined as invalid RO by additional RACH validation.

2.2 PRACH mask index

In previous RAN1 meetings, there is an agreement.

Agreement [4]

For SBFD-aware UEs and PRACH mask indication for CFRA triggered by PDCCH order, discuss whether/how to indicate/determine additional-RO only, legacy-RO only, or both is used.

Regarding the adjustment of the PDCCH order, there are two possible approaches. The first approach involves distinguishing PDCCH orders into two categories (e.g., by PRACH mask index): one for legacy ROs and another for additional ROs. The second approach allows the PDCCH order to indicate both legacy and additional ROs using the same RO index simultaneously. The first approach offers greater flexibility since it can specify the two types of ROs separately. However, it raises concerns about how UEs will determine whether the received PDCCH order corresponds to legacy or additional ROs, and whether different types of PDCCH orders should utilize different formats. The second approach has the limitation of indicating both legacy and additional ROs with the same index concurrently, but its advantage lies in maintaining a consistent PDCCH order, which means that UEs will not need to differentiate between the types, as only one format will be used.

Proposal 3:  There are two approaches to determine the RO for CFRA triggered by PDCCH order for SBFD-aware UEs:

- Option 1: Distinguishing PDCCH orders into at least two categories, one for legacy ROs and another for additional ROs, to indicate different type ROs independently.

- Option 2: PRACH mask index field of PDCCH order indicates both legacy and additional ROs using the same RO index simultaneously.

- Conclusion

In this contribution, we have following proposals for random access operation for SBFD aware UE:

Proposal 1:  RACH Configuration Option 2 need to constraint that, when legacy ROs are configured in symbols overlapping with SBFD symbols indicated as flexible by the tdd-UL-DL-ConfigurationCommon, the network ensures these ROs are within the usable uplink PRBs.

Proposal 2:  For handling the collision of legacy ROs on SBFD-aware UEs:

- Option 1: If there is a legacy RO has already been determined as valid RO by legacy RACH validation, Network will not allow the legacy RO in the same cell be on across SBFD and non-SBFD symbols, e.g., for SBFD aware UE.

- Option 2: If the valid legacy RO determined by legacy RACH validation is across SBFD and non-SBFD symbol at the same time, it will be further re-determined as invalid RO by additional RACH validation.

Proposal 3:  There are two approaches to determine the RO for CFRA triggered by PDCCH order for SBFD-aware UEs:

- Option 1: Distinguishing PDCCH orders into at least two categories, one for legacy ROs and another for additional ROs, to indicate different type ROs independently.

- Option 2: PRACH mask index field of PDCCH order indicates both legacy and additional ROs using the same RO index simultaneously.

- Reference

- RAN1#117 Draft Minutes report

- RAN1#116b Final Minutes report

- RAN1 #118 Draft Minutes report

- RAN1 #118b chairman’s note