3GPP TSG RAN WG1 #120			R1-2501131

Athens, GR, February 17th – 21st, 2025

Source:	Sharp

Title:	Discussion on specification support for AI/ML based positioning accuracy enhancements

Agenda Item:	9.1.2

Document for:	Discussion and Decision

Introduction

In the RAN#105 meeting, objective of AI/ML positioning on WID was revised as follows [1]:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

In this contribution, we discuss specification support for positioning accuracy enhancements.

Discussions

Model input

In the last RAN1 meeting [7], the following agreements were made for model input of AI/ML based positioning Case 3b:

Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

The remaining issues are starting time of the time window for sample-based measurement for UE-side measurement, and whether transmit offset from gNB to LMF for gNB-side measurement.

Starting time of the time window for sample-based measurement for UE-side measurement

In RAN1#119 meeting, for gNB-side measurement, it was agreed that the starting time is a first detected path rounded down with timing granularity T. However, for the UE-side measurement, the UE may perform training and/or performance monitoring using channel measurement generated by different vendor’s UE. Therefore, the starting time should be aligned based on absolute timing. In RAN1#119 meeting, the following proposal was made [9] and we believe that this proposal can be reused for the UE-side measurement.

Proposal 2.1.1.3-1 For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - If LMF configures an offset relative to a reference time, and the gNB follows the configuration, - starting time = reference time + offset - FFS: value range of the offset - Otherwise - starting time = first detected sample (i.e., first detected path with timing granularity T). Note: UE-side measurement is a separate discussion

Proposal 1: For AI/ML-based positioning, for UE-side measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

- If LMF configures an offset relative to a reference time, and the UE follows the configuration,

  - starting time = reference time + offset

    - FFS: value range of the offset

- Otherwise

  - starting time = first detected sample (i.e., first detected path with timing granularity T).

Whether transmit starting time offset of the Nt time window

For Case 3b gNB-side measurement, whether transmit starting time offset of the Nt time window from gNB to LMF is still remaining issue.

From our perspective, reporting of the starting time is helpful for LMF to align the multiple measurement data in time domain. Further, the legacy reporting framework, such as UL-RTOA, can be reused to report the offset value, so specification effort may be limited. The same can be said about UE-side measurement. Therefore, we made a following proposal:

Proposal 2: For the AI/ML based positioning, starting time of the window is reported from gNB to LMF.

Phase information for model input

In RAN1#116 meeting, phase information for model input was discussed and following agreement was made [2].

Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

In our understanding, there are mainly two ways to report phase information of the channel: One is (a) continuous phase information for each sample/path (e.g. including CIR), and the other one is (b) phase information for single sample/path (e.g. the first sample/path phase). The phase reporting is already supported in the Rel-18 spec, so additional specification effort may be not so large. And we should not concern about the signalling overhead for phase information report because whether to report the phase information can be up to LMF decision. For (a), the positioning accuracy enhancement is observed in the TR 38.843, so the CIR report should be supported. However, in our understanding, (b) may not contribute to the positioning accuracy enhancement because the single-phase information may be randomized. Therefore, we make a following proposal:

Proposal 3: For AI/ML based positioning, continuous phase information is supported for model input.

Model output for AI/ML assisted positioning for Case 3a

LOS/NLOS indicator

In RAN1#116 meeting, for AI/ML positioning case 2a and case 3a, at least LOS/NLOS indicator and/or timing information (UL-RTOA and gNB Rx-Tx Time Difference for uplink, and DL-RSTD and UE Rx-Tx Time Difference for downlink) are supported for reporting [2][7].

Agreement (RAN1#116) For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement (RAN1#116) For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Conclusion (RAN1#119) For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements

In AI/ML assisted positioning, it was agreed that at least LOS/NLOS indicator and/or timing information are supported for reporting. Further, according to the conclusion in the previous meeting, RAN1 concludes that LOS/NLOS indicator is not reported independently, that is the LOS/NLOS indicator must be reported with timing information. However, whether to change the meaning of the LOS/NLOS indicator is not determined in a case that the timing information is derived by AI/ML.

In our understanding, the meaning of LOS/NLOS indicators needs to be discussed separately depending on whether the associated timing information is predicted by AI/ML or not. In a case that the associated timing information is not predicted by AI/ML, the meaning of the LOS/NLOS indicator does not need to be changed, regardless of whether the LOS/NLOS indicator is predicted by AI/ML.

Observation 1: In a case that the associated timing information is not predicted by AI/ML, the meaning of the LOS/NLOS indicator does not need to be changed, regardless of whether the LOS/NLOS indicator is predicted by AI/ML.

However, in a case that the associated timing information is predicted by AI/ML, current NRPPa specification (TS38.455) can be interpreted in two ways for AI/ML assisted positioning as follows:
Interpretation 1) the LOS/NLOS indicator provides the confidence of virtual LOS propagation path of the associated timing information of the AI/ML model output.
Interpretation 2) the LOS/NLOS indicator provides the likelihood of a physical LOS propagation path of the channel measurement used for input of the AI/ML model.

Observation 2: In a case that the associated timing information is predicted by AI/ML, current NRPPa specification (TS38.455) can be interpreted in two ways for AI/ML assisted positioning as follows:
Interpretation 1) the LOS/NLOS indicator provides the confidence of virtual LOS propagation path of the associated timing information of the AI/ML model output.
Interpretation 2) the LOS/NLOS indicator provides the likelihood of a physical LOS propagation path of the channel measurement used for input of the AI/ML model.

Therefore, specifications for LOS/NLOS indicator should be modified so that the LOS/NLOS indicator is clearly understood in Interpretation 1 or 2 if the LOS/NLOS indicator is provided. On the other hand, it is possible not to report the LOS/NLOS indicator. Therefore, we made following options:

- Option 1: the LOS/NLOS indicator is provided with specification change in case that the associated timing information is predicted by AI/ML.

  - Option 1-1: the LOS/NLOS indicator provides the confidence of a virtual LOS propagation path of the associated timing information of the AI/ML model output.

  - Option 1-2: the LOS/NLOS indicator provides the likelihood of a physical LOS propagation path of the channel measurement used for input of the AI/ML model.

- Option 2: the LOS/NLOS indicator is not provided in case that the associated timing information is predicted by AI/ML.

Proposal 4: For AI/ML assisted positioning in case 3a, study the following options for down-selection:

- Option 1: the LOS/NLOS indicator is provided with definition change in current specification in case that the associated timing information is predicted by AI/ML.

  - Option 1-1: the LOS/NLOS indicator provides the confidence of a virtual LOS propagation path of the associated timing information of the AI/ML model output.

  - Option 1-2: the LOS/NLOS indicator provides the likelihood of a physical LOS propagation path of the channel measurement used for input of the AI/ML model.

- Option 2: the LOS/NLOS indicator is not provided in case that the associated timing information is predicted by AI/ML.

For Option 1-1, we understand that the virtual LOS propagation path means the 3D-path between a RS transmission point and a reception point in a straight line ignoring blockers. The LOS/NLOS indicator which associated with timing information generated by AI/ML may be better interpreted as “confidence” rather than “likelihood”. Because AI/ML model strives to always train with label data of the timing information that the propagation path is assumed as LOS, “likelihood” of the LOS propagation path may be always LOS. If the LOS/NLOS indicator is interpreted as “confidence” of the virtual LOS propagation path, it will be valid information.

For Option 1-2, the LOS/NLOS indicator is based on the channel measurement used for input of the AI/ML model in inference phase. But in our understanding, there is no strong correlation between the likelihood of a LOS propagation path of channel measurement used for input of AI/ML model and prediction accuracy of timing information. Therefore, LMF may not utilize such LOS/NLOS indicator to select associated timing information.

In case that the Option 1 is not valid for LOS/NLOS indicator for associated timing information predicted by AI/ML, Option 2 can be selected. However, at least we believe Option 1-1 can work in this case. Therefore, we prefer Option 1-1.

Proposal 5: For the case that the associated timing information is predicted by AI/ML, the LOS/NLOS indicator provides the confidence of a virtual LOS propagation path of the associated timing information of the AI/ML model output.

Indicator which provides whether the timing information is made by AI/ML

If we select Option 1 in the above section, since the meaning of LOS/NLOS indicator is changed, an indicator that provides whether the timing information is made by AI/ML is useful. Therefore, such indicator should be reported for AI/ML assisted positioning. Otherwise, even if the Option 2 is selected, the indicator can be helpful to aware LMF that the timing information is made by AI/ML.

Proposal 6: For AI/ML assisted positioning Case 3a, indicator that provides whether the timing information is made by AI/ML is reported.

Training data collection

Association between Part A and Part B

According to the agreement on the RAN1#116bis [3], components of Part A and Part B was agreed as follows:

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

Further, the following proposal was made in RAN1#119 [9], but not agreed:

Proposal 4.4.2-1 For training data collection of AI/ML based positioning, if Part A and Part B are generated by the same entity, the paired Part A and Part B are provided in one report. - If multiple entries of Part A and Part B are reported, the Part A entry and the corresponding Part B entry are grouped in a pair for reporting. - The paired Part A and Part B entry share the same time stamp. Proposal 4.4.2-2 For training data collection of AI/ML based positioning, if Part A and Part B are generated by different entities, the pairing between a Part A entry and a Part B entry is supported via at least: - A UE identifier, and - The time stamp of Part A and the time stamp of Part B. - Note: Different time stamp granularity can be used for Part A and Part B. For example, “Measurement time” in 38.455 and UTCTime in 37.355 have different granularities.

At least for AI/ML positioning Case 1 and 3a, Part A and Part B can be generated by same entity, for example, both of contents can be generated by PRU or UE. Therefore, Proposal 4.4.2-1 can be applied at least for Case 1 and 3a.

For AI/ML positioning Case 3b and also Case 1 and 3a, the Part A and Part B can be generated by different entities. Therefore, Proposal 4.4.2-2 can be applied at least for Case 1, 3a and 3b.

Proposal 7: For training data collection of AI/ML based positioning Case 1 and 3a, if Part A and Part B are generated by the same entity, the paired Part A and Part B are provided in one report.

- If multiple entries of Part A and Part B are reported, the Part A entry and the corresponding Part B entry are grouped in a pair for reporting.

- The paired Part A and Part B entry share the same time stamp.

Proposal 8: For training data collection of AI/ML based positioning Case 1, 3a and 3b, if Part A and Part B are generated by different entities, the pairing between a Part A entry and a Part B entry is supported via at least:

- A UE identifier, and

- The time stamp of Part A and the time stamp of Part B.

  - Note: Different time stamp granularity can be used for Part A and Part B. For example, “Measurement time” in 38.455 and UTCTime in 37.355 have different granularities.

Ensure consistency between training and inference

Assistance information for AI/ML based positioning Case 1

In RAN1#119 meeting, the following agreement was made for AI/ML positioning Case 1 [6]:

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

According to the agreement, for AI/ML based positioning Case 1, almost all assistance information from legacy UE-based DL-TDOA, except for TRP location (info #7), may be provided from LMF to UE. In this meeting, we should discuss whether info #7 should be provided or not.

In legacy UE-based TDOA positioning, LMF needs to provide TRP location so that the UE can estimate its own location. The legacy TRP location has a role of providing the reference position to the target UE. On the other hand, AI/ML based positioning, reference location is provided on the label data in a training phase, thus the AI/ML model of the target-UE is already imprinted the reference position. Therefore, TRP location is not required for AI/ML-based positioning itself during inference phase. However, it will be beneficial for the purpose of to ensure consistency between training and inference. Although, the consistency can be ensured by performance monitoring if the ID or info#7 is not provided, positioning accuracy may temporarily decrease in inference phase until performance monitoring. The motivation to introduce the associated ID instead of the info#7 is to does not disclose geographical coordinates of the TRPs.

Observation 3: Explicit or implicit indication of TRP location is beneficial to ensure consistency between training and inference.

Observation 4: Although, the consistency can be ensured by performance monitoring if the ID or info#7 is not provided, positioning accuracy may temporarily decrease in inference phase until performance monitoring.

In the last meeting, 4 alternatives were agreed for performance monitoring of AI/ML based positioning Case 1. In conclusion, we support Alternative 2, that is the Info #7 and/or associated ID is optionally provided. Thus, we will discuss for associated ID design hereafter.

Regarding the associated ID, at the RAN1#118 meeting, the following agreement was made for AI/ML Beam Management [5].

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

According to the above agreement, the same associated ID implies the “similar” physical properties of a DL TX beam or beam set/list. It means NW side additional condition associated with the same associated ID have some tolerance. The same principle can be applied for the AI/ML positioning. To avoid complicating the specification, we prefer how to define the associated ID and the physical properties should be up to network implementation without specifying.

Proposal 9: For AI/ML based positioning, UE may assume the similar properties (e.g. Geographical coordinates of the TRPs) with the same associated ID
- Note: the how to define the associated ID and the physical properties should be up to network implementation without specifying.

For the implicit indication, an associated ID can be same even if the TRP location is slightly changed (within the tolerance). However, when associated ID during the inference phase is changed from stored associated ID during the training phase, the UE shall determine that the consistency between training and inference is not ensured and shall update/deactivate/fallback its own AI/ML functionality, because UE does not aware how much NW-side additional condition has changed. Further, the associated ID is preferred to be designed that LMF changes associated ID with slight TRP location changes taking into account the UE that has poorly generalized AI/ML models. Considering this, even the UE that has highly generalized AI/ML model shall update/deactivate/fallback its own AI/ML functionality even if the TRP location is slightly changed (beyond the slight tolerance). It should be avoided from the perspective of the UE that has highly generalized AI/ML model.

Observation 5: The implicit indication requires the UE that has highly generalized AI/ML model to performs update/deactivate/fallback.

To avoid this issue, explicit indication of the TRP location is beneficial that UE controls its own models/functionalities. If the UE is provided explicitly indicated TRP location, the UE can update/deactivate/fallback its own model/functionality according to the generalization performance of its model. Therefore, we support Alternative 2.

Proposal 10: UE shall determine that the consistency between training and inference is not ensured and shall update/deactivate/fallback its own AI/ML functionality when the associated ID is changed from stored associated ID during the training phase.

Observation 3: If the UE is provided explicitly indicated TRP location, the UE can update/deactivate/fallback its own model/functionality according to the generalization performance of its model.

Proposal 11: For performance monitoring of Case 1, Alternative 2 is supported:
 •Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF.

Performance monitoring

Performance monitoring for Case 1

In RAN1#116 bis and RAN1#118 meeting, the following agreement and conclusion were made for performance monitoring for AI/ML positioning Case 1 [3][5][7]:

Agreement (RAN1#116bis) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side. - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Conclusion (RAN1#118) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

Agreement (RAN1#119) For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

For performance monitoring of AI/ML positioning Case 1, it was agreed that the Option A, that is the target UE-side performance monitoring, is supported. The content of monitoring outcome is still FFS.

Before we discuss this issue, we should define the performance monitoring metric outcome. In our understanding, there are two interpretations for the monitoring outcome. On one hand, the monitoring outcome indicates the request of switching/update/fallback of the AI/ML functionality/model. This may be reported in a case that UE determines the current AI/ML model is inappropriate. On the other hand, the monitoring outcome indicates such as the difference between predicted location and ground-truth location. In this case, LMF shall determine switching/update/fallback of the AI/ML functionality/model. Therefore, for UE-side performance monitoring, following two options can be considered:
Option 1: UE makes a decision whether its own model is appropriate or not.
Option 2: UE reports performance monitoring metric outcome to LMF and LMF makes a decision whether UE’s model is appropriate or not.

Observation 7: For UE-side performance monitoring, following two options can be considered:
Option 1: UE makes a decision whether its own model is appropriate or not, and UE reports the decision.
Option 2: UE reports performance monitoring metrics to LMF and LMF makes a decision whether UE’s model is appropriate or not.

Figure 1 Performance monitoring for AI/ML positioning Case 1

For the Option 1, the monitoring outcome may be 0/1 binary value. Also, the UE may not have to report monitoring outcome if the UE determines that its own model is appropriate for inference. Therefore, the signalling overhead can be reduced for the Option 1. Also, the LMF can optionally configure the threshold of appropriateness of the model for the UE. For the Option 2, the monitoring outcome may be an error value that reflects the difference between measured data and ground truth label. Therefore, the LMF can make a comprehensive decision. However, if LMF configures the threshold for the error value, the Option 1 can also make a decision based on the LMF. Therefore, we support the Option 1.

Proposal 12: UE makes a decision whether its own model is appropriate or not and UE reports the decision that indicates the model is inappropriate.

Proposal 13: LMF can configure threshold to UE to determine whether its own model is appropriate or not.

Conclusion

In this contribution, we have discussed our views on specification support for positioning accuracy enhancement and have the following observations and proposals.

Proposal 1: For AI/ML-based positioning, for UE-side measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

- If LMF configures an offset relative to a reference time, and the UE follows the configuration,

  - starting time = reference time + offset

    - FFS: value range of the offset

- Otherwise

  - starting time = first detected sample (i.e., first detected path with timing granularity T).

Proposal 2: For the AI/ML based positioning, starting time of the window is reported from gNB to LMF.

Proposal 3: For AI/ML based positioning, continuous phase information is supported for model input.

Observation 1: In a case that the associated timing information is not predicted by AI/ML, the meaning of the LOS/NLOS indicator does not need to be changed, regardless of whether the LOS/NLOS indicator is predicted by AI/ML.

Observation 2: In a case that the associated timing information is predicted by AI/ML, current NRPPa specification (TS38.455) can be interpreted in two ways for AI/ML assisted positioning as follows:
Interpretation 1) the LOS/NLOS indicator provides the confidence of virtual LOS propagation path of the associated timing information of the AI/ML model output.
Interpretation 2) the LOS/NLOS indicator provides the likelihood of a physical LOS propagation path of the channel measurement used for input of the AI/ML model.

Proposal 4: For AI/ML assisted positioning in case 3a, study the following options for down-selection:

- Option 1: the LOS/NLOS indicator is provided with definition change in current specification in case that the associated timing information is predicted by AI/ML.

  - Option 1-1: the LOS/NLOS indicator provides the confidence of a virtual LOS propagation path of the associated timing information of the AI/ML model output.

  - Option 1-2: the LOS/NLOS indicator provides the likelihood of a physical LOS propagation path of the channel measurement used for input of the AI/ML model.

- Option 2: the LOS/NLOS indicator is not provided in case that the associated timing information is predicted by AI/ML.

Proposal 5: For the case that the associated timing information is predicted by AI/ML, the LOS/NLOS indicator provides the confidence of a virtual LOS propagation path of the associated timing information of the AI/ML model output.

Proposal 6: For AI/ML assisted positioning Case 3a, indicator that provides whether the timing information is made by AI/ML is reported.

Proposal 7: For training data collection of AI/ML based positioning Case 1 and 3a, if Part A and Part B are generated by the same entity, the paired Part A and Part B are provided in one report.

- If multiple entries of Part A and Part B are reported, the Part A entry and the corresponding Part B entry are grouped in a pair for reporting.

- The paired Part A and Part B entry share the same time stamp.

Proposal 8: For training data collection of AI/ML based positioning Case 1, 3a and 3b, if Part A and Part B are generated by different entities, the pairing between a Part A entry and a Part B entry is supported via at least:

Observation 3: Explicit or implicit indication of TRP location is beneficial to ensure consistency between training and inference.

Observation 4: Although, the consistency can be ensured by performance monitoring if the ID or info#7 is not provided, positioning accuracy may temporarily decrease in inference phase until performance monitoring.

Proposal 9: For AI/ML based positioning, UE may assume the similar properties (e.g. Geographical coordinates of the TRPs) with the same associated ID
- Note: the how to define the associated ID and the physical properties should be up to network implementation without specifying.

Observation 5: The implicit indication requires the UE that has highly generalized AI/ML model to performs update/deactivate/fallback.

Proposal 10: UE shall determine that the consistency between training and inference is not ensured and shall update/deactivate/fallback its own AI/ML functionality when the associated ID is changed from stored associated ID during the training phase.

Observation 6: If the UE is provided explicitly indicated TRP location, the UE can update/deactivate/fallback its own model/functionality according to the generalization performance of its model.

Proposal 11: For performance monitoring of Case 1, Alternative 2 is supported:
 •Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF.

Observation 7: For UE-side performance monitoring, following two options can be considered:
Option 1: UE makes a decision whether its own model is appropriate or not, and UE reports the decision.
Option 2: UE reports performance monitoring metrics to LMF and LMF makes a decision whether UE’s model is appropriate or not.

Proposal 12: UE makes a decision whether its own model is appropriate or not and UE reports the decision that indicates the model is inappropriate.

Proposal 13: LMF can configure threshold to UE to determine whether its own model is appropriate or not.

References

RP-242399 “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Qualcomm.

“RAN1 Chairman’s Notes,” RAN1 #116, Mar. 2024.

“RAN1 Chairman’s Notes,” RAN1 #116bis, Apr. 2024.

“RAN1 Chairman’s Notes,” RAN1 #117, May. 2024.

“RAN1 Chairman’s Notes,” RAN1 #118, Aug. 2024.

“RAN1 Chairman’s Notes,” RAN1 #118bis, Oct. 2024.

“RAN1 Chairman’s Notes,” RAN1 #119, Nov. 2024.

R1-2409286, Moderator (Ericsson), Final summary of specification support for positioning accuracy enhancement, RAN1 #118bis, Oct. 2024.

R1-2410921, Moderator (Ericsson), Final summary of specification support for positioning accuracy enhancement, RAN1 #119, Nov. 2024.