3GPP TSG RAN WG1 #120			R1-2501366

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.11.5

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Feature lead summary #1 on IoT-NTN TDD mode

Document for:	Discussion and Decision

# Background

In RAN#106, a revised work item on Introduction of IoT-NTN TDD mode was approved [1] with the following objective:

The work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

This work item includes the following objectives:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures,  including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2]

      - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band.

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

## 1.1 Previous agreements:

### 1.1.1 RAN1#118b

Agreement

The target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz:

- For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain).

- For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain).

- When reporting link level simulation results, companies are encouraged to report the CNR margin with respect to the above operating DL CNRs.

- Companies may report values for other scenarios (e.g. UE antenna gain of -5.5 dBi, LEO-800) with the corresponding assumptions.

Agreement

For the study phase, RAN1 assumes as a baseline for evaluations (which includes DL synchronization as per the WID) that downlink PHY channels and signals are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD.

- For the purpose of study, the DL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and signals and eNB will not transmit the corresponding instances.

  - FFS: What is the TDD pattern

Agreement

RAN1 evaluates via link level simulations the following:

- NPSS/NSSS detection

- NPBCH decoding

Agreement

The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.):

- SIB1-NB

- Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31)

- Paging

- NPRACH / RAR

- NPSS/NSSS/NPBCH

- NPDCCH, NPUSCH, NPDSCH

- Note: Study for the above channels / signals may continue during the normative phase.

- It is up to companies to perform link level evaluations for some of the aspects above.

Agreement

For link level simulations of NPSS / NSSS / NPBCH, RAN1 uses the following assumptions:

| Parameter | Value |
| --- | --- |
| Carrier frequency | 1.6GHz |
| Channel model | NTN TDL-C rural, 30 degrees |
| Frequency error / timing drift (including XO error) | 34 ppm for NPSS / NSSS (24ppm Doppler + 10ppm XO error) 0.1ppm for NPBCH |
| Variation of frequency error / variation of timing drift | [0.27ppm/s] for NPSS / NSSS 0 for NPBCH |
| UE velocity | 3km/h |
| Target performance | For NPSS/NSSS: - 99% detection probability with 0.1% false alarm rate - FFS: How to define correct detection, including successful timing / frequency estimation / cell ID detection. For NPBCH: - 1% BLER For the channels / signals above, acquisition delay shall be reported for the corresponding detection probability |
| Other assumptions (e.g. combining length, frequency error hypothesis, etc.) | To be reported and justified by companies |

Agreement

For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes):

- N = 9:

  - D = 8

  - D = 20

  - D = 30

- N = 8:

  - D = 20

- Companies to report which subframe indices are included in the set of consecutive downlink subframes.

- Other combinations are not precluded to be reported by companies

NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

Agreement

For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes):

- N = 9:

  - U = 8

  - U = 20

  - U = 30

- N = 8:

  - U = 20

- Companies to report which subframe indices are included in the set of consecutive uplink subframes.

- Other combinations are not precluded to be reported by companies

- For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U

NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

Agreement

For the study phase, RAN1 assumes as a baseline for evaluations that uplink PHY channels are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD.

- For the purpose of study, the UL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and eNB will not receive the corresponding instances.

  - FFS: What is the TDD pattern

### 1.1.2 RAN1#119

Observation

RAN1 makes the following observations on downlink synchronization:

Case 1: For N=9, D=8:

- For NPSS:

  - For one-shot detection (90ms delay), 9 sources provided simulation results, with a median required SNR of 1.30dB. The link budget margin is (median margin):

    - 4.21 dB for LEO-1200 (0dBi antenna gain)

    - -1.89 dB for LEO-600 (-5.5dBi antenna gain).

  - For two-shot detection (180ms delay), 3 sources provided simulation results, with a median required SNR of -1.20dB. The link budget margin is (median margin):

    - 6.71dB for LEO 1200 (0dBi antenna gain)

    - 0.61dB for LEO-600 (-5.5dBi antenna gain).

  - For four-shot detection (360ms delay), 3 sources provided simulation results, with a median required SNR of -3.80dB. The link budget margin is (median margin):

    - 9.31dB for LEO-1200 (0dBi antenna gain)

    - 3.21dB for LEO-600 (-5.5dBi antenna gain).

  - NOTE: The assumptions for combining for 2-shot and 3-shot detection are different across companies.

- For NSSS:

  - For one-shot detection (180ms delay), 5 sources provided simulation results, with a median required SNR of -2.50dB. The link budget margin is (median margin):

    - 8.01dB for LEO-1200 (0dBi antenna gain)

    - 1.91dB for LEO-600 (-5.5dBi antenna gain)

- For NPBCH:

  - For 90ms combining, 4 sources provided simulation results, with a median required SNR of 3.48dB. The link budget margin is (median margin):

    - 2.03dB for LEO-1200 (0dBi antenna gain)

    - -4.07dB for LEO-600 (-5.5dBi antenna gain)

  - For 640ms combining, 6 sources provided results, with a median required SNR of  -4.25dB. The link budget margin is (median margin):

    - 9.76dB for LEO-1200 (0dBi antenna gain)

    - 3.66dB for LEO-600 (-5.5dBi antenna gain)

Case 2: For N=9, D=20:

- For NPSS:

  - For a combining of 2 consecutive NPSS (80ms delay), 1 source provided simulation results, with a median required SNR of -0.8dB. The link budget margin is (median margin):

    - 6.31dB for LEO-1200 (0dBi antenna gain)

    - 0.21dB for LEO-600 (-5.5dBi antenna gain)

- For NPBCH:

  - For 640ms combining, 1 source provided simulation results, with a median required SNR of -5.10dB. The link budget margin is (median margin):

    - 10.61dB for LEO-1200 (0dBi antenna gain)

    - 4.51dB for LEO-600 (-5.5dBi antenna gain)

Case 3: For N=9, D=30:

- For NPSS:

  - For combining of 3 consecutive NPSS (90ms delay), 1 source provided simulation results, with a median required SNR of -2.4 dB. The link budget margin is (median margin):

    - 7.91dB for LEO-1200 (0dBi antenna gain)

    - 1.81dB for LEO-600 (-5.5dBi antenna gain)

- For NPBCH:

  - For 640ms combining, 1 source provided simulation results, with a median required SNR of
 -7.10dB the link budget margin is (median margin):

    - 12.61dB for LEO-1200 (0dBi antenna gain)

    - 6.51dB for LEO-600 (-5.5dBi antenna gain)

NOTE: Other evaluated scenarios (LEO-600 with 0dBi antenna gain, LEO-1200 with -5.5dBi antenna gain, LEO-800 with 0dBi antenna gain) have margins in between LEO-1200 (0dBi antenna gain) and LEO-600 (-5.5dBi antenna gain)

NOTE: Different companies provided input for different cases. For the companies that evaluated multiple cases, the performance (in terms of acquisition delay) of Case 3 and Case 2 is better than the performance of Case 1.

Observation

Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective:

- Case 1: N=9, D=8

- Case 2: N=9, D=20

- Case 3: N=9, D=30

Observation

In Rel-19, RAN1 concludes that at least N=9 is feasible from the following points of view:

- DL synchronization, based on the link level evaluations submitted at RAN#119.

- Coexistence with the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

Agreement

Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints:

- At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods.

  - The same downlink slot is used in all the 90ms periods.

- At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods.

  - The same uplink slot is used in all the 90ms periods.

- The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4).

- NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

Observation

RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

Conclusion

For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits.

- When considering only NPSS/NSSS/NPBCH/SIB1-NB, and if the number of SIB1-NB repetitions is 16:

  - If additionalTransmissionSIB1 is set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 43.75% in the anchor carrier.

  - If additionalTransmissionSIB1 is not set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 37.5% in the anchor carrier.

  - NOTE: In the above calculations, overhead due to transmission of other SIBs has not been included.

- The value of X may be further studied during the normative phase. Note: X is a TBS value supported by the specifications for SIB1-NB.

Agreement

The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options:

- Option 1: Always the same across H-SFNs.

  - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9.

  - Option 1-2: H-SFN duration is unchanged

- Option 2: Different across some H-SFNs.

  - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB)

  - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Agreement

RAN1 to further discuss how to handle the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPRACH (including NPRACH occasions) with non-U NB-IoT subframes, including studying at least the following:

- New periodicities to align with the TDD structure.

- Postponement of a channel when it overlaps with non-U NB-IoT subframes.

- Restriction of a channel to be fully confined within a single set of U NB-IoT subframes.

- Dropping (full or partial) of channels when they overlap with non-U NB-IoT subframes.

- Impact on segmented pre-compensation.

Agreement

RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following:

- New periodicities to align with the TDD structure.

- Postponement of a channel when it overlaps with non-D NB-IoT subframes.

- Restriction of a channel to be fully confined within a single set of D NB-IoT subframes.

- Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes.

- The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications

Agreement

The downlink subframes within D=8 are to be down-selected among the following:

- Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames)

- Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames)

- Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames)

- Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames)

# TDD frame structure - general

## 2.1 U subframes in the TDD frame structure (& guard period)

The work item states the following:

- Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band.

The following are some inputs to this meeting on the guard period:

- Fixed:

  - [TH], [IRI] propose to have a fixed guard period of 50ms between DL and UL

  - [HW] proposes 48ms fixed guard period between DL and UL

  - [XMI]: the slots allocated to IOT NTN are Iridium slots #3 (UL and DL)

  - [QC]: The offset between DL and UL subframes is fixed in specifications.

  - [LGE]: No adjustable guard period is introduced

- Adjustable / configurable:

  - [ZTE],[OPPO], [DCM] proposes to make the guard period configurable

  - [DCM]: Support adjustable guard period

Several companies (e.g. [HW], [QC]) mention that small adjustments to the guard period can be implemented by setting the common TA parameters appropriately.

Given the majority under “fixed” and the guidance from the work item, the following is proposed.

### [HIGH PRIO] Proposal 2.1-1: The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [50]ms at the ULSRP.

| Company | Comment |
| --- | --- |
| Nordic | We are fine with hard-coded value, but in future this could be more configurable. |
| Spreadtrum | Support. As WID mentioned, the guard period should be a fixed value. Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band.. |
| Xiaomi | We are OK with this direction, i.e. fixed guard period. However, before we set the guard period value itself, maybe the actual slot pair allocated to 3GPP needs to be settled first. The reason is the guard time between Simplex Slot to UL slot is 1.24ms and to DL slot is 1ms according to the public figure below. While the guard time between adjacent UL slots are 0.22ms and adjacent DL slots are 0.1ms. If we take DL slot 3 and UL slot 3 as example, the gap can be calculated as 20.32+8.28*3+1+1.24+0.22*2+0.1=47.94 ms. Following this calculation, the fixed value seems sufficient set as 48ms. As for the reason to consider DL slot 3 and UL slot 3, we provided detailed analysis in our Tdoc regarding how this structure suits the locations of NPSS/NSSS/SIB1-NB ### Proposal 2.1-1: The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [50]ms at the ULSRP. The slots allocated to IoT NTN are Iridium DL slot#3 and UL slot#3 |
| OPPO | We would like to correct the summary on our view. What we suggest is that when the D,U value is larger than 8, the guard period should be configured by the network, and here the guard period is the between the consecutive D subframes or the period between D and U. We agree that the guard period between D and U can be realized without additional signaling, e.g., fixed in number of subframes (assuming the same index between TDMA DL slot index and UL slot index) + corrections from common TA. |
| Lenovo | We are fine with the guard period as fixed value in Rel.19, and the value should be integer of mini seconds to aligned with 3GPP subframe boundary. |
| LGE | We support the proposal. 50msec guard period would be sufficient to cover all the cases including LEO600 and LEO1200 with transparent payload or regenerative payload. |
| TCL | We are fine with the proposal in Rel.19. |
| Thales | Support FL proposal |
| CATT | We prefer the fixed pattern. But it should clarify how many DL/UL slot pairs are supported within 90ms duration. |
| ZTE | OK with fixed gap. Regarding the value of guard gap, it should be larger than the largest interval between potential slot pairs in Iridium’s legacy frame structure, i.e., largest interval among gaps between (DL1&UL1, DL2&UL2, DL3&UL3, and DL4&UL4). Further clarification of guard intervals between slots in Iridium’s legacy frame structure may be needed to justify whether the fixed GP is large enough. |
| Nokia, NSB | We have similar view as some other companies that the guard period between different pair of ULx/DLx may be different. One fixed value may not work and causing some subframe may be not available for Tx/Rx. Before RAN1 decide this value, more information should be provided for e.g. gaps in the frame structure. Additionally, for this period, RAN1 should consider both number of available resource for Tx/Rx and impact to spec/implementation. |
| Iridium | Iridium supports this proposal. Also, we do not oppose the configurable gap; but in the interest of time, configurability best to be left to future releases. To CATT: We can only use one D/U pair, but need to be able to pick which pair to use (1,2,3 or 4). |
| Apple | If only one UL and DL pair is allocated for TDD mode operation, this proposal is not needed. The DL and UL guard period include simplex (22.32ms), three slots (3*8.8ms) and gaps (?ms). so the guard period is at least 48.72ms, and the guard period is different for different UL and DL pair, as the gaps are different between DL slots, between UL and DL, UL slots. Anyway the gap(s) is fixed for a UL and DL pair. If more UL and DL pairs are configured for TDD mode operation, then the guard period needs to be considered considering the round trip delay and gap between the UL and DL pairs. |
| Huawei, HiSilicon | fine with the direction. As for how to capture in the specification, e.g. whether the pattern is defined in terms of downlink/uplink subframes or gap, it can be discussed later. Based on the design constraint agree in last meeting, the gap should be 48-49ms. |

## 2.2 D subframes in the TDD frame structure

In RAN1#119 it was agreed to downselect among the following options

| Option 1: [3 4 5 6 7 8 9 0] | Option 2: 4 5 6 7 8 9 0 1 |
| --- | --- |
| Option 3: [8 9 0 1 2 3 4 5] | Option 4: [9 0 1 2 3 4 5 6] |

The following input has been received to this meeting:

- Option 1: [TH], [OPPO], [IRI], [XMI], [QC], [OPPO]

- Option 2: [CATT]

- Option 3: [HW]

- Not option 2: [CMCC]

- Option 1 or 2: [DCM], [NOR], [LEN]

- ZTE: Configurable

- Samsung: Support only one option

Several companies also mention that most of the patterns should work. Therefore, feature lead propose to go with the majority view

### [HIGH PRIO] Proposal 2.2-1: The downlink subframes within D=8 are fixed to:

- Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames)

| Company | Comment |
| --- | --- |
| Nordic | OK to go majority views, even though essentiality of SIB1 in SF3 depends on minimum target SNR for the system. |
| Xiaomi | Support FL proposal |
| OPPO | OK |
| Lenovo | We are fine with Option 1 to support the additional SIB1 transmission in subframe 3. |
| LGE | OK. |
| TCL | OK |
| Thales | Support FL proposal |
| CATT | OK |
| ZTE | OK with majority view. |
| Nokia, NSB | We think at least the option 1 is the best for now, comparing to other options. |
| Iridium | We support this proposal |
| Apple | OK. |
| Huawei, HiSilicon | We can accept option 1, although our preference is option 3. |

## 2.3 SFN wrap-around

For the issue of alignment of 90ms periodicity with SFN wrap-around, in the previous meeting the following was agreed:

The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options:

- Option 1: Always the same across H-SFNs.

  - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9.

  - Option 1-2: H-SFN duration is unchanged

- Option 2: Different across some H-SFNs.

  - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB)

  - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

The following input was provided to this meeting:

- Option 1-1: [OPPO], [CMCC], [CATT], [HW], [TCL]

    - [TCL]: Aligned with SFN#0, SFN skipped

- Option 2-1: [DCM], [QC], [NOR], [APP], [LGE]

    - [NOR], [LGE]: Option 2-1 implicitly (no spec impact)

    - [APP]: By UE implementation

    - [QC] Option 2-2 can be complementary to 2-1, up to RAN2.

- [XMI]: Either 1-1 or 2-1 (2-1 with implicit offset preferred)

- [ZTE]: Offset is configurable, constant pattern preferred

- [VIVO, SPDR]: Either 1-2 or 2-1

There is a clear majority for 1-1 or 2-1, but no clear majority between these two.

### [HIGH PRIO] Proposal 2.3-1: The offset between the 90ms TDD structure and the 10240ms H-SFN is downselected among the following options:

- Option 1: Always the same across H-SFNs.

    - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9.

- Option 2: Different across some H-SFNs.

    - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB)

| Company | Comment |
| --- | --- |
| Nordic | As explained in our contribution, there is no need for any specification change, it would be actually contra-productive to go with Option 1 |
| Spreadtrum | RAN 1 should discuss which option has less impact on legacy spec. For option 1-1, it would change current H-SFN duration to 1017 radio frames. For option 2-1, UE can obtain the offset by NPSS/NSSS/NPBCH/SIB1-NB which may without spec impact. In our view, option 2-1 is better than option 1-1. |
| Xiaomi | Support FL proposal. |
| OPPO | Support |
| Lenovo | Option 1-1 may have large impact to higher layer H-SFN related parameter/timer determination/counting, and Option 2-1 may need additional indication of the offset in broadcast channel. In order to avoid potential large specification change at this stage, we slightly prefer Option 1-2 as a compromise (not much impact to current standard). |
| Vivo1 | First, we need to clarify whether this is a pure RAN1 issue or should involve RAN2 insights. From our understanding, RAN1 cares about whether UE can acquire the SFN timing and determine the corresponding TDD pattern (i.e., the location of D and U resource set). In that sense, any of the above options can satisfy this purpose. The difference lies in different options is the corresponding RAN2 impact. Therefore, it would be better to analysis the impact/outcome of each option instead of picking up the majority options. Option 1-1 and Option 2-2 have a similar issue that they break the periodicity of broadcast message, which depends on H-SFN or SFN. A lot of RAN2 changes would be required and thus should not be preferred. Option 1-2 and option 2-1 work without significant RAN2 changes. Our first preference is option1-2 since it has the less impact on legacy 3GPP framework. While we can live with option2-1 if no indication is introduced. In our understanding, the frame offset between 1st TDD pattern and a H-SFN will follow a cycling pattern (e.g., [0 2 4 6 8 1 3 5 7]) as long as the frame offset between first subframe of the D resource set of the 1st TDD pattern and H-SFN0 is fixed(e.g., fixed as zero). When a UE accesses a cell, after it obtains H-SFN (e.g., H-SFN#X) from SIB1, it can derive the corresponding offset between the H-SFN#X and 1st TDD pattern location in the H-SFN#X based on acquired timing information, and automatically derive the time offset in the subsequent H-SFN cycles. Therefore, as long as the NW follow the cycling pattern, the frame offset can be derived. ### [HIGH PRIO] Proposal 2.3-1: The offset between the 90ms TDD structure and the 10240ms H-SFN is downselected among the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE derive is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) implicitly If companies insist on keeping option 1-1, the down selection should be down by RAN2 instead of RAN1 |
| LGE | OK. Regarding option 1, it would be carefully necessary to check the issue that the MIB or SIB1-NB contents can be changed within their own period. To be specific, MIB periodicity or SIB1-NB periodicity will not span the new H-SFN duration. So, some MIB period or SIB1-NB period will be overlapping with more than one H-SFNs. It seems contradict with the fact that the MIB or SIB1-NB does not change within its own period. Moreover, this change will make the soft combining of MIB or SIB1-NB within a period impossible. Finally, we may need to go with Option 2-1. |
| TCL | Support |
| Thales | We think Option 1-1 might have some side effects on higher layer procedures, so it may be necessary to consult RAN2 first. We are willing to support Option 2-1 Additionally, it appears that the issue of SFN wrap-around is not well understood by everyone. It might be beneficial to first discuss and agree on an observation that explains the issue. We recognize that the non-constant relationship between the periodic pattern and SFN will lead to increased complexity in scheduling and implementation. It would be best to capture and clarify this in an observation. |
| CATT | We prefer Option 1-1 because it is straight forward. In this moment we can keep two options on the table. |
| ZTE | Regarding option 1-1, modification of H-SFN will lead to significant spec impact, as the periodicity is legacy spec generally assume an 1024 duration when counting H-SFN, e.g., when considering modification period. As a result, RAN2 spec will be significantly impacted. Hence, option 1-2 which does not have such impact should be considered for further downselection instead of option 1-1. |
| Nokia, NSB | We do not support. We think both of these options can not provide resource for periodic (e.g. with period as multiple of 8ms or multiple of 10ms) transmission of e.g. NPBCH/NPSS/NSS based on the available subframe resource, then the period of the transmissions may be increased much because of no available resource in the frame structure, which will reduce the system efficiency very much. More study is needed for the available resource in the frame structure with 90ms and we think adjustment of H-SFN in the 90ms frame structure should be discussed to guarantee no or no large degradation of periodic channels. |
| Iridium | Based on other proposals we have made on truncation and postponement we believe that no spec changes are necessary. Option 2-1 seems to have the least amount of spec impact of the current proposal. |
| Apple | We suggest to add another Option UE implementation to determine the offset. UE can knows out the SNF index and H-SFN index after detecting the NPBCH, NSSS and SIB1-NB. Therefore, UE knows about the offset between 90ms TDD structure and current H-SFN index. even in the next H-SFN index, UE could figure out the new offset easily. |
| Huawei, HiSilicon | Fine with the proposal to further down select between option 1-1 and option 2-1. |

# Handling of downlink channels

## 3.1 General

A majority of companies propose to define non-D subframes as “non NB-IoT DL subframes” either explicitly ([TH], [HW], [IRI], [QC]) or implicitly by specifying postponements for NPDSCH / NPDCCH  ([CATT, CMCC, VIVO, OPPO, NOR]). Some other views are as follows

- [SPDR] proposes that further clarification is needed on UE behavior.

- [SS] proposes to consider available DL subframes to determine NPDCCH/NPDSCH resource

- [ZTE] proposes to not define non-D subframes as “non-NB-IoT DL subframes” due to latency concerns due to postponement and propose to drop the channels instead.

In view of the above, FL proposes the following, with the note that adjustments to the current specifications with respect to treating the NB-IoT DL subframes:

### [HIGH PRIO] Proposal 3.1-1: The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications

- NPDCCH / NPDSCH are postponed as per current specifications.

- FFS: NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes”

| Company | Comment |
| --- | --- |
| Nordic | Support |
| Spreadtrum | Support |
| Xiaomi | OK with the direction but prefer not to have the FFS part. The proposed way forward would be ### [HIGH PRIO] Proposal 3.1-1: The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH are postponed as per current specifications. |
| OPPO | For NPDCCH, we think that from the specification point of view, we should rather consider the search space determination (only considering D NB-IOT suframes, instead of saying NPDCCH being postponed. For NPDSCH, we are fine with the proposal. The FFS point needs further clarification, what does it mean? |
| Lenovo | Firstly, it is better to have non-D NBIoT subframe definition at least in the FLS, which means the non DL subframe in 90ms TDD pattern, including gap and UL subframe? For the main bullet, in current TS36.213 Section 16.4, we only mentioned which subframe is considered as NBIoT DL subframe, the remaining subframes are not NBIoT DL subframe automatically. So it is better to reformulate the proposal similar as Only the DL subfames by the TDD pattern can be considered by the UE as “NB-IoT DL subframes” from the specifications. |
| Vivo1 | fine |
| LGE | OK. Regarding the NPDCCH, we have similar understanding with OPPO. According to the specification, it seems that the NPDCCH candidates are defined over the NB-IoT DL subframes. Simply, we can say that “NPDCCH/NPDSCH are treated as per current specifications”. |
| TCL | Support |
| Thales | In principle the postponement of a NPDCCH search space when it overlaps with non-D NB-IoT subframes, could be considered. However, the following considerations should be taken into account: - The search space candidates may span consecutives DL transmission occasions, as shown in the example below: with Rmax= 8, G=4. Thereby T = 32. The problem is that there might be an overlap between different search periods; the search space candidates will be within to different search periods. - Postponement of NPDCCH Search space starting subframe could not resolve potential overlapping between search space periods - The eNB should configure values that result in a T larger enough resolve the issue (1): To this aim, we propose to introduce a scaling factor F that could be used to expand T. This scaling factor should be equal to 90/8 that’s is F= 11,5 (to have scaling factor of 2n, F could be set to 8 or 16). - eNB shall only configure values that result in T>=4 *F Regarding the PDSCH: we think Downlink transmission gap should be also discussed In case of NB-IoT NTN TDD, with a periodic pattern of 90ms and reduced DL subframes within one Dl transmission occasion (i.e. 8 subframes), the same user could be served for long time even if a large number of repetitions might not be required (with sufficient link budget). Therefore, the downlink transmission gap may need to be configured. However, the downlink transmission gap configuration parameters are not suitable and need to be adapted to the TDD pattern with N= 9 (90ms) and D=8. A scaling factor of 90/8=11.25. Proposal 10 In NB-IoT NTN TDD, the periodicity of a DL transmission gap is configured using a new parameter dl-GapPeriodicity-r19: - dl-GapPeriodicity-r19 = dl-GapPeriodicity-13 * F - FFS: value of F |
| CATT | Support |
| ZTE | If NB-IoT TDD-specific rules are to be defined, it seems better not to define non-D NB-IoT subframes as “non NB-IoT DL subframes”. Otherwise, there will be a mix of postponement caused by legacy spec and NB-IoT NTN TDD operation. |
| Nokia, NSB | We are fine for the main bullet but not support the sub-bullet 1. The availability of the resource will be important for the system efficiency. Postpone is not a good way for the frame structure with more than half resource not available. We think more study is needed before RAN1 agreement on this. |
| Iridium | We support this proposal |
| Apple | Support |
| Huawei, HiSilicon | Fine with the main bullet. For the 1st bullet, we suggest to change “postponed” to “mapped”. For 2nd bullet, what does the TDD-specific rules referring to? |

Also, given the outcome of the study phase + the input to this meeting, it is proposed that the channels for initial sync (including up to SIB1-NB) are dropped in non-D NB-IoT subframes

### [HIGH PRIO] Proposal 3.1-2: NPSS/NSSS/NPBCH/SIB1-NB transmissions are dropped in non-D NB-IoT subframes

| Company | Comment |
| --- | --- |
| Nordic | Support, but I believe we already have agreement like this |
| Spreadtrum | Support |
| Xiaomi | We have some confusion. According to our Tdoc Figure below, if we only allocate Iridium DL slot #3 for IoT NTN carrying PBCH/NPSS/NSSS/SIB1-NB, there is no occasion that the sync. Channels will need to be dropped according to the patterns in red. However, if we are talking about the radio frames/subframes not allocated to Iridium as well, then this proposal may be good. |
| OPPO | can we say: no transmission in non-D NB-IOT subframes in general. So we don’t need to spell out all the channels/signals. |
| Lenovo | Fine with the proposal. |
| Vivo1 | agree |
| LGE | OK. |
| TCL | Ok |
| Thales | Ok |
| CATT | OK |
| ZTE | Agree |
| Nokia, NSB | We think it need more time to discuss as the efficiency of NPSS/NSSS/NPBCH/SIB1-NB transmissions will be significantly reduced if dropped in non-D NB-IoT subframes, as resulting available NPSS/NSSS/NPBCH/SIB1-NB to about 20%. |
| Iridium | We support this proposal |
| Apple | Support |
| Huawei, HiSilicon | support |

## 3.2 Handling of SI

For SI other than SIB1, the following input has been received:

- [CMCC, HW,Iri]: They propose to postpone SI repetitions until the next available DL subframe, and they propose to skip the 2nd SI repetition transmission if it overlaps with the 1st one (after postponing)

- [NOR] proposes to transmit sibX only within D subframes in the corresponding SI window.

- [Iri] proposes to truncate repetitions if they overlap with the next SI window.

Feature lead proposes the following:

### [LOW PRIO] Proposal 3.2-3: For SI other than SIB1, repetitions are postponed as per the current specifications with the following modifications:

- If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlap (after postponing) with a second radio frame according to si-RepetitionPattern, the remaining repetitions corresponding to the first radio frame are dropped.

- SI transmissions are not postponed beyond the end of the corresponding SI-window

| Company | Comment |
| --- | --- |
| Nordic | OK |
| Xiaomi | We would prefer to go with the direction of controlling SI transmissions so that they are not overflowed since dropping will sacrifice coverage/detection performance of the impacted SIB message. Actually, according to our analysis, although for the case when the SI message has TBS value larger than 120, the transmission occasions that need to accommodate the repetition may need up to 8 subframes and may overflow to a later SI window. We can limit the TBS of the SIB message such that the transmission of a repeated SIB message in an earlier SI window will not overflow to a later SIB window For reception of SI message with repetition, to avoid the overflow of the repetitions from an earlier window to a later window, UE is not expected to receive the SI message with TBS value larger than 120. |
| Vivo1 | SI transmission/window handling seems to be a RAN2 issue, we suggest leaving it to RAN2. Postponing periodic SI transmission may lead congestion in the following D subframes thus is not preferred. While from RAN1 perspective, dropping SI in non-D subframes seems to be an easier solution. As the SI periodicity and SI window length do not divide 90ms, there will always be some valid occasions for SI transmission after a few SI periodicity. |
| LGE | From our side, we can discuss RAR and MSG4 transmission together with the other SI transmission. As we know, after applying TDD pattern, the existing RAR window size and the contention resolution window would not be sufficient. |
| Thales | Support |
| CATT | We don’t support this proposal. If the repetition has not been completed within the first radio frame, the rest repetition can be postponed. The repetition pattern can be adapted to the new TDD pattern. |
| ZTE | Our first preference is to directly drop the SI transmission overlapped with non-D subframe to avoid congestion. If the postponement is applied, certain dropping criteria should be introduced to handle the congestion of different SI repetitions. Hence, the proposal is needed. But the wording can be updated, since the inherent rule of dropping is that a second SI transmission start but the first SI transmission does not end, which causes congestion. For example, update the first bullet as: If a repetition of an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlap (after postponing) with is postponed after the start time of a latter SI transmission correspond to a second radio frame according to si-RepetitionPattern, the remaining repetitions corresponding to the first radio frame are dropped. Regarding second bullet, it is preferred to use similar wording as first bullet, e.g.,: If a repetition of an SI transmissions are not is postponed beyond the end of the corresponding SI-window, the remaining repetitions are dropped. |
| Nokia, NSB | We do not agree with the subbullet. Always dropping remaining repetitions of the first radio frame may be problematic and needs further study by RAN1. |
| Iridium | We support this proposal |
| Apple | Support |
| Huawei, HiSilicon | Fine with main bullet and 2nd sub-bullet. For 1st subbullet, the “remaining repetitions” is not clear. If the remaining repetitions corresponding to the first radio frame are dropped, the dropping may happen for each radio frame. If the remaining repetitions corresponding to the second radio frame are dropped. Then the 3rd transmission will not overlap with the 1st transmission, and the 3rd transmission can still be transmitted without dropping. |

## 3.3 Others

[DCM] proposes to add new periodicities to NPDCCH.

[HW] proposes that, for all search spaces, UE should skip the following monitoring occasions of a search space if they overlap with ongoing monitoring occasion of the same search space [HW]

[QC] proposes that the UE may assume that NRS is available in every DL subframe not including sync signals.

Further discussion is needed in these points

# Handling of uplink channels

## 4.1 NPUSCH

On the behavior of NPUSCH colliding with non-U subframes, the following input has been received:

- [SS]: Consider available UL subframes to determine NPUSCH resources

- [Iri, HW]: Non-U subframes are not NB-IoT UL subframes

- [APP, CATT, QC, vivo]: Postpone NPUSCH

- [ZTE, VIVO] Drop NPUSCH

It is FS’s understanding that postponing NPUSCH can be achieved with the current framework of “NB-IoT UL subframes” and, therefore, the following is proposed:

### [HIGH PRIO] Proposal 4.1-1: The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications

| Company | Comment |
| --- | --- |
| Nordic | we could even directly agree on a TP here. |
| Spreadtrum | Support |
| Xiaomi | We think similar approach from NPDSCH handling can be re-used. ### [HIGH PRIO] Proposal 3.1-1: The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications - NUSCH are postponed as per current specifications. |
| Vivo1 | For NPUSCH, we support postpone NPUSCH instead of dropping. We made corrections on the summary part. |
| LGE | OK. |
| Thales | Support |
| CATT | Support |
| ZTE | Similar comment as DL. It seems better to directly specify UE behavior with respect to non-U subframes instead of defining “non-U NB-IoT subframes” as “non NB-IoT UL subframes” if NB-IoT NTN TDD specific behavior is to be defined. |
| Nokia, NSB | Generally we agree on this if no big impact. |
| Iridium | We support this proposal |
| Apple | Support |

## NPRACH

On the behavior of NPRACH colliding with non-U subframes, the following input has been received:

- NPRACH is postponed [TCL, QC, Iri, Nor, OPPO, CATT]

    - Unit of postponement is PRU: majority of companies.

    - Unit of postponement is symbol group: [OPPO]

- NPRACH is dropped [ZTE, VIVO]

Given the clear majority, feature lead proposes the following:

### [HIGH PRIO] Proposal 4.2-1: NPRACH transmissions are postponed in in non-U NB-IoT subframes

- The unit of postponement is:

    - Option 1: A single symbol group

    - Option 2: P symbol groups.

| Company | Comment |
| --- | --- |
| Nordic | We do not support Option 1, it makes no sense to make one NPRACH block discontinuous. |
| Spreadtrum | For option1, one symbol group is postponed if it can’t be done in one UL burst. For option 2, P symbol groups is postponed if it can’t be done in one UL burst, for example, there are 2 repetitions, first repetition can be transmitted in one UL burst and second in next UL burst. Comparing to option 1, option 2 maybe better for decoding performance but has longer delay. |
| Xiaomi | We think this proposal should be considered at the same level of priority of proposal 4.2-2. We prefer the solution in proposal 4.2-2 |
| OPPO | OK |
| Lenovo | P should be larger than 1 in Option 2, right? |
| Vivo1 | Proposal 4.2-1 and Proposal 4.2-2 should be discussed together as they provide different solutions for resolving NPRACH collision. If new PRACH periodicities (multiple of 90ms) are supported, we assume that there is no need for postpone as the PRACH transmission is always shorter than 8 subframes. In addition, if multiple repetitions are configured for PRACH, and if postpone is adopted, it is questionable whether the repetitions located within different U resource sets can be jointly used for TA estimation considering the 90ms gap between them. |
| LGE | We have similar view with vivo. Depending on the NPRACH resource periodicity modification, we may not need to have other approaches. |
| TCL | We think proposal 4.2-1 and proposal 4.2-2 should be iscussed together. We prefer the proposal 4.2-1. |
| Thales | We think the unit of postponement should be NPRACH preamble repetition unit. So support option 2. Option 2 allows to maintain the existing specification reule: “The P symbol groups are transmitted in a time contiguous manner” Option 2 could be modified as: Option 2: P=4 symbol groups. From this perspective: only preamble Format 0 and preamble Format 1 with preamble repetition unit (ms) of 5.6 and 6.4 ms respectevly could be supported in IoT NTN TDD with D and U= 8ms. |
| CATT | Support |
| ZTE | For option 2, it is preferred to use NPRACH repetition directly, since postponement of P symbol groups from different NPRACH repetitions may also break the NPRACH structure as option 1. |
| Nokia, NSB | The availability of the resource will be important for the system efficiency. Postpone is not a good way for the frame structure with more than half resource not available. We think more study is needed before RAN1 agreement on this. |
| Iridium | We support option 2. Option 1 breaks udesignp the P symbol group which could lead to more decoding failures. |
| Apple | Support |
| Huawei, HiSilicon | Agree with Thales, option 2 is preferred as the postponement of NPRACH preamble by repetition unit is aligned with the current specification design. |

Several companies proposed to introduce new periodicities for NPRACH (e.g. [LEN, LGE, DCM, NOR, APP, XMI, OPPO]) that are multiples of 90ms.

### [LOW PRIO] Proposal 4.2-2: New NPRACH periodicities (multiple of 90ms) are introduced for NB-IoT NTN TDD.

| Company | Comment |
| --- | --- |
| Nordic | We are supportive, because it may simplify determination of slots for NPUSCH. And it would make NPRACH occasions more regular. |
| Xiaomi | Support FL proposal |
| OPPO | OK |
| Vivo1 | Support, but should be discussed tougher with Proposal 4.2-1 |
| LGE | OK |
| Thales | We are open to to introduce new NPRACH periodicities (multiple of 90ms). However, existing periodicities could be reused with additional rule as per the following proposal. And nprach-Periodicity (ms) sould be equal to 80ms or higher Proposal 12 For NTN IoT TDD mode, NPRACH transmission can start at the beginning of the next uplink transmission occasion after the start of a radio frame fulfilling |
| CATT | OK |
| ZTE | No need if postponement is applied. |
| Nokia, NSB | We do not support to introduce new periodicity as it may impact much on the specification and implementation. For the periodic Tx/Rx, RAN1 should discuss the efficiency and impact together for the suitable solution. |
| Iridium | We are open to this proposal, and are also open to discussion in conjunction with the proposal 4.2-1 |
| Apple | Support |
| Huawei, HiSilicon | support |

Given the basic unit of NPRACH format 2 is longer than 8ms, [APP, Iri, TH] propose to not support NPUSCH format 2 for NB-IoT NTN TDD

### [LOW PRIO] Proposal 4.2-3: NPRACH format 2 is not supported in NB-IoT NTN TDD

| Company | Comment |
| --- | --- |
| Nordic | We support restriction. No issue to still support, but less testing required. |
| Xiaomi | We think this kind of not supporting should be applied uniformly. A similar consideration on whether to support 3.75kHz proposed in our TDoc, maybe we can discuss them together. |
| OPPO | OK |
| Vivo1 | support |
| LGE | OK |
| Thales | Support |
| ZTE | Fine with the proposal |
| Nokia, NSB | We agree not to use NPRACH format 2 due to the time restriction of 8 UL subframes. |
| Iridium | We support this proposal |
| Apple | Support |

## 4.3 Others

[Iri] proposed that uplink gaps are not needed, since the UE can anyway monitor the downlink in between the uplink transmissions.

### [LOW PRIO] Proposal 4.3-1: Uplink transmission gaps do not apply in NB-IoT NTN TDD.

| Company | Comment |
| --- | --- |
| Nordic | Agree |
| Xiaomi | Some clarification is needed. If the DL/UL slots are allocated and periodically repeated, the gap between UL slots carrying UL transmission should be fixed? |
| OPPO | not support, the uplink gap is used to ensure the UE synchronization. This does not change when it comes to NB-IOT TDD. But the gap duration may need check if the current 40 ms is still valid. |
| Vivo1 | This seems to be a conclusion. As the length of continuous UL transmission is always less than U subframes, UL gaps never apply |
| Thales | We made similar proposal in our contribution: Proposal 13 In NB IoT NTN TDD, there is no need to insert a gap of 40 ms: - After NPUSCH transmissions and/or postponements (or postponements due to NPRACH) of 256ms every 64 NPRACH preamble repetitions. |
| ZTE | Seems not an essential issue. |
| Nokia, NSB | We think the issue mentioned by Oppo should be studied. |
| Iridium | We support this proposal |
| Apple | If only one UL and DL pair is allocated for TDD mode, then the uplink gap is not needed. if we agreed more than one pairs can be used for TDD mode operation, maybe gap is needed. |
| Huawei, HiSilicon | Support |

On segmented pre-compensation, the following input was received:

- [Iri] pre-compensation shall always be done either at the start of the uplink active time or during the uplink active time

- [OPPO] UL segment should only be defined in consecutive U subframes

- [QC] a new pre-compensation segment is started at the beginning of each uplink burst

- [ZTE] Dropped NPUSCH / NPRACH are counted for segment determination

It is FL’s understanding that [OPPO] and [Iri] proposals are equivalent.

### [LOW PRIO] Proposal 4.3-2: For segmented pre-compensation, RAN1 to further discuss at least the following options:

- Option 1: Dropped NPUSCH / NPRACH are counted for segment determination

- Option 2: A new pre-compensation segment is started at the beginning of each uplink burst

- Option 3: UL segment should only be defined in consecutive U subframes

| Company | Comment |
| --- | --- |
| Nordic | Option 4: are not applicable to IoT TDD. |
| Xiaomi | Prefer focus on other issues firstly. |
| OPPO | OK |
| Vivo1 | We think when to pre-compensate can be up to UE implementation, no need to specify one of the options |
| LGE | We also fine with Option4 suggested by Nordic. In case of Option 2 and 3, the UL segment gap would be used unnecessarily. |
| Thales | Ok |
| ZTE | Option 1 is preferred, which is simple and straightforward. For option 2, additional spec impact is needed but the gain is not very clear. While option 3 seems to be achievable by legacy spec if network configure a segment length shorter than TDD pattern length. |
| Nokia, NSB | OK to discuss. We prefer option 2. |
| Iridium | We prefer Opt 2 or 3. Open to discuss the support of other options that UE vendors prefer. |

# Others

## 5.1 Kmac

Two contributions ([TH] and [IRI]) propose to extend the Kmac value due to the operation through ISL as explained in the figure below from [TH]:

### [LOW PRIO] Proposal 5.1-1: Extend the value of Kmac to 1-1023ms for operation in NB-IoT NTN TDD.

| Company | Comment |
| --- | --- |
| Nordic | RAN2 issue, but supportive. |
| Xiaomi | OK with the motivation |
| Thales | Ok |
| Nokia, NSB | Do not agree. Proponents need to further argue why 512 ms is insufficient for a few “ISL hops”. |
| Iridium | We support this proposal |

# Proposals for Tuesday online session:

Offline consensus:

[HIGH PRIO] Proposal 2.1-1: The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [48 - 50]ms at the ULSRP

- NOTE: This corresponds to the largest separation between DL and UL Iridium slot pairs. Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA

[HIGH PRIO] Proposal 2.2-1: The downlink subframes within D=8 are fixed to:

- Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames)

[HIGH PRIO] Proposal 3.1-2 [NOKIA NEEDS TO CHECK]: NPSS/NSSS/NPBCH/SIB1-NB transmissions are dropped in non-D NB-IoT subframes

- NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

NOTE FROM FL: The following has been agreed in previous meetings (emphasis added)

Agreement (RAN1#118b)

For the study phase, RAN1 assumes as a baseline for evaluations (which includes DL synchronization as per the WID) that downlink PHY channels and signals are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD.

- For the purpose of study, the DL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and signals and eNB will not transmit the corresponding instances.

  - FFS: What is the TDD pattern

Observation (RAN1#119)

Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective:

- Case 1: N=9, D=8

- Case 2: N=9, D=20

- Case 3: N=9, D=30

[HIGH PRIO] Proposal 4.1-1: The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications

- NPUSCH is mapped as per current specifications.

- NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure.

[HIGH PRIO] Proposal 3.1-1: The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications

- NPDCCH / NPDSCH are mapped as per current specifications.

- FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes”

- FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor)

- FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD.

- NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

Offline discussion but no consensus yet:

Alternative 1:

[HIGH PRIO] Proposal 2.3-1v1: The offset between the 90ms TDD structure and the 10240ms H-SFN is:

- Option 2: Different across some H-SFNs.

  - It is up to RAN2 whether to specify option 2-2 (Total number of H-SFN is changed to be a multiple of 9)

    - In case Option 2-2 is specified by RAN2, it is possible to specify the 90ms TDD structure in terms of SFN / H-SFN.

    - In case Option 2-2 is not specified by RAN2, the UE needs to derive the 90ms TDD structure based on observations of downlink signals and it is not possible to specify the 90ms TDD structure in terms of SFN / H-SFN.

Alternative 2:

[HIGH PRIO] Proposal 2.3-1v2: The offset between the 90ms TDD structure and the 10240ms H-SFN is downselected among the following options:

- Option 1: Always the same across H-SFNs.

  - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9.

- Option 2: Different across some H-SFNs.

  - Option 2-1: UE derives the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB). No additional parameter is introduced at least for the anchor carrier.

Both options are feasible from RAN1 perspective. Send an LS to RAN2 asking whether Option 1 is feasible from RAN2 perspective and their recommendation of which option to select.

Not discussed in offline:

[New proposal] For NPRACH, at least one of the following options are specified:

- Option 1: New NPRACH periodicities (multiple of 90ms) are introduced for NB-IoT NTN TDD.

- Option 2: NPRACH transmissions are postponed in in non-U NB-IoT subframes

  - The unit of postponement is:

    - Option 1: A single symbol group

    - Option 2: P symbol groups.

      - NOTE: P is the parameter in TS 36.211, Subclause 10.1.6.1

[Note from FL: P is defined in the table below]

[LOW PRIO] Proposal 4.2-3: NPRACH format 2 is not supported in NB-IoT NTN TDD

[LOW PRIO] Proposal 4.3-1: Uplink transmission gaps do not apply in NB-IoT NTN TDD.

[LOW PRIO] Proposal 4.3-2: For segmented pre-compensation, RAN1 to further discuss at least the following options:

- Option 1: Dropped NPUSCH / NPRACH are counted for segment determination

- Option 2: A new pre-compensation segment is started at the beginning of each uplink burst

- Option 3: UL segment should only be defined in consecutive U subframes

- Option 4: Segmented pre-compensation does not apply for NB-IoT NTN TDD.

[LOW PRIO] Proposal 5.1-1: Extend the value of Kmac to 1-1023ms for operation in NB-IoT NTN TDD.

[LOW PRIO] Proposal 3.2-3: For SI other than SIB1,

- Option 1: repetitions are postponed as per the current specifications with the following modifications:

  - If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlap (after postponing) with a second radio frame according to si-RepetitionPattern, the remaining repetitions corresponding to the first radio frame are dropped.

  - SI transmissions are not postponed beyond the end of the corresponding SI-window

- Option 2: Up to RAN2 to handle

Note from FL: The behavior above is defined in TS 36.331

# Proposals from contributions

| R1-2500038 | THALES [TH] | Definition of TDD pattern Proposal 1 The TDD-DL-UL-Pattern provides the following fixed parameters for NB-IoT NTN TDD operation in 1616-1626.5 MHz MSS band: - DL-UL-TransmissionPeriodicity which indicates the periodicity of the DL-UL pattern - DL-UL-TransmissionPeriodicity is equal to 9 radio frames - nrOfDownlinkSubframes which indicates the number of consecutive DL subframes at the beginning of each DL-UL pattern - nrofdownlinksubframes is equal to 8 subframes - nrOfUplinkSubframes which indicates the number of consecutive UL subframes, - nrOfUplinkSubframes is equal to 8 subframes - DownlinkToUplinkGuardPeriod which indicates the downlink to uplink guard period for TDD operation. - DownlinkToUplinkGuardPeriod is equal to 50 subframes Frame structure for NB-IoT NTN TDD mode Proposal 2 In IoT NTN TDD, NB-IoT NTN TDD downlink subframe is a subframe that is within the downlink transmission occasions (D=8) of the TDD pattern. Proposal 3 NB-IoT NTN TDD downlink subframes (i.e the downlink subframes within D=8) are the subframes [3 4 5 6 7 8 9 0] of the frame structure type 1. UE DL synchronization evaluation results Proposal 4 For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for all SIB1-NB TBS values supported by the specifications for SIB1-NB, at least when the configured number of repetitions is 16. Other SI scheduling in IoT NTN TDD mode Observation 8 In NB-IoT NTN TDD mode of operation with N=9 and nrOfDownlinkSubframes=8, a single SIB2-NB may span two consecutive DL transmission period when SIB2-NB is transmitted in 8 consecutive valid subframes. Observation 9 In NB-IoT NTN TDD mode of operation, the actual DL transmission period may not coincide with the first radio frame for SI message transmission in the SI window indicated by si-RadioFrameOffset. Proposal 5 In NB-IoT NTN TDD, SI message can be transmitted a number of times over 2 or 8 consecutive NB IoT NTN TDD downlink subframes that may span two consecutive DL transmission. NPDCCH analysis Proposal 6 The start of the SI window is postponed to the next DL transmission occasion if it coincides with non NB IoT NTN TDD downlink subframe. Proposal 7 NPDCCH/Search space candidate is postponed to the next DL transmission occasion if it coincides with non NB IoT NTN TDD downlink subframe. FFS: Whether a scaling factor F is introduced to expand the search period T as follows: T= Rmax *G*F FFS: value of F NPDSCH transmission Proposal 8 The UE shall not expect NPDSCH in subframe if it is not a NB-IoT NTN TDD downlink subframe. Note: a subframe is assumed as a NB-IoT NTN TDD DL subframe if it is within a DL transmission occasion. Proposal 9 NPDSCH transmissions that could not be completed within a DL transmission occasion, are postponed until the next transmission occasion, except for transmissions of NPDSCH carrying SIB1-NB in subframes 3 and 4 for frame structure type 1 Proposal 10 In NB-IoT NTN TDD, the periodicity of a DL transmission gap is configured using a new parameter dl-GapPeriodicity-r19: - dl-GapPeriodicity-r19 = dl-GapPeriodicity-13 * F - FFS: value of F NPRACH analysis Observation 10 The P symbol groups are transmitted in a time contiguous manner, therefore, only preamble Format 0 and preamble Format 1 with preamble repetition unit (ms) of 5.6 and 6.4 ms respectevly could be supported in IoT NTN TDD with D and U= 8ms. Proposal 11 Only NPRACH preamble format 0 and preamble format 1 defined for frame structure type 1, are supported in IoT NTN TDD Observation 11 For a 1% miss detection rate of the NPRACH, the min required SNR is equal to: - 15.1 dB for single NPRACH repetition unit received - 10.6 dB for 2 NPRACH repetition units received during a 2x9 radio frames period NPRACH decoding margin (dB). Number of Repetitions; SNR (dB); LEO600; LEO1200. 1; 15,1; 2,79; -2,61. 2; 10,6; 7,29; 1,89. Proposal 12 For NTN IoT TDD mode, NPRACH transmission can start at the beginning of the next uplink transmission occasion after the start of a radio frame fulfilling NPUSCH transmission Proposal 13 In NB IoT NTN TDD, there is no need to insert a gap of 40 ms: - After NPUSCH transmissions and/or postponements (or postponements due to NPRACH) of 256ms - Every 64 NPRACH preamble repetitions. Kmac value range extension Proposal 14 Support Kmac_r19 with value range 1 – 1023 ms |
| --- | --- | --- |
| R1-2500086 | Huawei, HiSilicon [HW] | Proposal 1：For the 8 DL subframes location across two consecutive radio frames, option 3: [8 9 0 1 2 3 4 5] should be adopted. Proposal 2：Support option 1-1 to solve the orphan radio frames in a 90ms TDD pattern when mapping TDD patterns into a 10240ms H-SFN. Proposal 3: For IoT NTN TDD, the starting point of a TDD pattern is aligned with starting point of subframe #8, radio frame #0. A subframe is DL subframe if it satisfies the condition of 10∙nf+ns-8 mod 90=0,1,2,3,4,5,6,7. A subframe is UL subframe if it satisfies the condition of 10∙nf+ns-8 mod 90=56,57,58,59,60,61,62,63. Observation 1: According to the public paper, the duration of guard periods between consecutive Iridium DL slots are different from those between consecutive UL slots. Proposal 4：The difference of DL to UL guard periods corresponding to different iridium DL/UL slot pairs can be implicitly compensated by NTACommon through nta-Common. Explicit configuration of DL to UL guard period is not needed. Observation 2: SIB1-NB reception following current specifications is feasible for all existing SIB1-NB TBS values, at least when the configured number of repetitions is 16 and additionalTransmissionSIB1 is set to TRUE in the scenario of LEO600(0dBi) and LEO1200(0dBi). If additionalTransmissionSIB1 is set to TRUE, SIB1-NB reception for 208bit TBS is feasible in the scenario of LEO600(-5.5dBi). Proposal 5：The non-D subframes of IoT NTN TDD should not be assumed as “NB-IoT DL subframes” irrespective of the configuration of downlinkBitmap-r13. Proposal 6: The value range of starting subframe configuration(G) for USS (npdcch-NumRepetitions-RA) and type-2 CSS (npdcch-StartSF-CSS-RA) for NB-IoT TDD in TN should be used in IoT NTN TDD. Proposal 7: For all search spaces, UE should skip the following monitoring occasions of a search space if they overlap with ongoing monitoring occasion of the same search space. Proposal 8: The RAR window configuration and RA-RNTI calculation for NB-IoT TDD in TN should be used for IoT NTN TDD. Proposal 9: For the reception of NPDSCH repetition in a SI-window, UE should skip the following NPDSCH repetition reception if corresponding valid NB-IoT DL subframe overlaps with those for the ongoing reception of NPDSCH repetition. Proposal 10：The non-U subframes of IoT NTN TDD should not be assumed as “NB-IoT UL subframes” irrespective of the configuration of downlinkBitmap-r13. Proposal 11: To reduce the overlap of NPRACH occasions with non-U IoT NTN TDD subframes, new NPRACH periodicities should be introduced to align with the TDD structure. If a NPRACH transmission can’t be finished within a TDD pattern, the remaining repetitions can be postponed to the NB-IoT UL subframes of the following TDD patterns. |
| R1-2500186 | Spreadtrum, UNISOC [SPDR] | - In IoT-NTN TDD mode, the period of NPSS and NSSS will be greatly extended. - For N=9 and D=8, UE may miss one PBCH block, which causes UE can’t acquire entire MIB. - UE can pre-compensate time/frequency in non-U and non-D NB-IoT subframes. - RAN1 to discuss how to ensure UE can acquire entire MIB. - Current specifications for SIB1-NB can be reused. - For DL, option 1 and 4 can be precluded. Option 5 need more clarification for UE behavior. Option 1: New periodicities to align with the TDD structure. Option 4: Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes. Option 5: The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications - For UL, option 1 and 4 can be precluded: Option 1: New periodicities to align with the TDD structure. Option 4: Dropping (full or partial) of channels when they overlap with non-U NB-IoT subframes. - Option 1-2 and 2-1 can be used to align 90ms TDD structure and the 10240ms H-SFN. Option 1-2: H-SFN duration is unchanged Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) |
| R1-2500211 | CATT [CATT] | Proposal 1：NPSS should be transmitted twice with repetition to meet the detection target within one 90ms and NPSS will be transmitted in the earlier subframes in one D slot. Proposal 2：NSSS can be transmitted once for each 90ms-duration to help UE to do fast synchronization. Proposal 3：NPBCH should be transmitted with one subframe at least within 90ms duration and NPBCH periodicity will be extended to 720ms. Proposal 4：When the operating mode is IoT NTN TDD, RAN1 needs to further study the repetition number of SIB1-NB transmission based on link budget and performance simulation. Proposal 5：At least one subframe in one D slot within 90ms duration should be reserved for SIB1 transmission. Proposal 6: At least one subframe within 90ms duration should be reserved for SIB31 to help UE to get ephemeris information timely. Proposal 7：For NPDCCH (such as msg2/4) and NPDSCH that does not carry SIB1-NB, whether repetition transmission is required is up to link budget and performance simulation. Proposal 8：In IoT NTN TDD mode, when NPDCCH/NPDSCH overlaps with non-D NB-IoT subframes, consider postponing it to the next period for transmission. Proposal 9: NPRACH periodicity should be configured larger than 90ms to reserve the UL subframes for NPUSCH transmission since NPRACH length is very long. Proposal 10：In IoT NTN TDD mode, when NPUSCH/NPRACH (including NPRACH occasions) overlaps with non-U NB-IoT subframes, consider postponing it to the next period for transmission. Proposal 11：Considering the downlink synchronization requirements, Option 2 can be prioritized. Proposal 12：For the offset between the 90ms TDD structure and the 10240ms H-SFN, it is recommended to prioritize Option 1-1 to minimize the impact on the specifications. Proposal 13：Following legacy Iridium pattern, the TDD gap within one 90ms duration can be fixed depending on the slot index of DL and UL pair. |
| R1-2500305 | CMCC [CMCC] | Proposal 1: The following set of DL subframes can be considered for the downlink frame structure within a period of 9 radio frames - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) - Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames) - Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames) Proposal 2: Considering the offset between the 90ms TDD structure and the 10240ms H-SFN, Option 1-1 can be supported, - The first TDD pattern can start at the same SFN (e.g., SFN0) within each 10240ms H-SFN, and the last radio frames before the SFN wraps around back to 0 will not be used to carry the TDD pattern. Proposal 3: For IoT-NTN TDD mode, with proper configuration of SI window, the SI repetition can be postponed to the available DL subframes within a TDD pattern if overlap with the non-D NB IoT subframes. Proposal 4: For IoT-NTN TDD mode, if postponement of SI repetition will overlap with the next SI repetition transmission, skip such SI repetition transmission. Proposal 5: For IoT-NTN TDD mode, in case of one SI message transmission across several consecutive valid subframes, if partial SI message transmission mapping to subframes that are non-D NB-IoT subframes, that part of SI message is postponed to later valid downlink subframes within the TDD pattern. Proposal 6: For IoT-NTN TDD mode, if NPRACH (including NPRACH occasions) will overlap with non-U NB-IoT subframes, the following options can be considered, - The ROs overlapping with available UL subframes within TDD pattern can be used for PRACH transmission. - Postponement of PRACH repetition when it overlaps with non-U NB-IoT subframes |
| R1-2500370 | Vivo [VIVO] | Proposal 1: No new TBS value for SIB1-NB is introduced in Rel-19. Proposal 2: Support option 1-2 or option 2-1 if the offset between 1st 90ms TDD pattern and SFN0 of H-SFN0 is predefined (e.g., being fixed as 0) as the solution of the offset between the 90ms TDD structure and the 10240ms H-SFN. Proposal 3: For NPDSCH scheduled by NPDCCH and NPDCCH, support postponement when it overlaps with non-D NB-IoT subframes. Proposal 4: For NPSS/NSSS/NPBCH/SIB1, support dropping (full or partial) when it overlaps with non-D NB-IoT subframes. Proposal 5: For NPUSCH, the postponement can be adopted when it overlaps with non-U NB-IoT subframes. Proposal 6: For NPRACH, if the preamble overlaps with non-U NB-IoT subframes, the overlapped part should be dropped, and the remaining non-overlapped part can still transmit. |
| R1-2500386 | ZTE Corporation, Sanechips [ZTE] | Observation 1: When DL subframe number is fixed as D=8, the DL subframe configuration of TDD pattern can be determined by the DL offset between first DL subframe and start time of corresponding radio frame. Proposal 1: The DL offset, i.e., offset between first DL subframe and start time of corresponding radio frame, can be configured by network in SIB1-NB. Proposal 2: The TDD pattern is preferred to be aligned with radio frame timing for easier resource determination. The radio frame containing the first DL subframe is the first radio frame of the TDD pattern. Proposal 3: The pattern offset, i.e., offset between first TDD pattern and SFN0, can be configured by network in SIB1-NB. Proposal 4: A constant pattern offset is preferred in each hyper frame for easier resource determination of TDD pattern. UE will not set subframe format according to TDD pattern if the remaining radio frame number of a hyper frame is smaller than N. Proposal 5: The guard gap length of TDD pattern, i.e., interval between DL and UL subframes, can be configured by network in SIB1-NB. Observation 2: Introducing new periodicities to align with TDD structure will cause large impact on specification and implementation. Observation 3: Postponement of a channel when overlapping with non-D subframe will significantly increase the delay and cause congestion, since DL duration is smaller than 1/10 of TDD pattern period. Observation 4: Defining non-D NB-IoT subframes not as“NB-IoT DL subframes” will significantly increase the delay and cause congestion, since DL duration is smaller than 1/10 of TDD pattern period. Observation 5: Restriction of a channel to be fully confined within a single set of D NB-IoT subframes will introduce strong restriction on network scheduling and increase implementation complexity. Proposal 6: For NPDCCH/NPDSCH (other than the one carrying SIB1-NB) overlapped with non-D NB-IoT subframes, the transmissions on non-D NB-IoT subframes should be dropped. Proposal 7: The start point and time length of SI window and RAR window should be determined in same way as legacy NB-IoT system. Proposal 8: The PO should be determined in same way as legacy NB-IoT system. For NPDCCH addressing paging message, the NPDCCH repetitions overlapped with non-D NB-IoT subframes can be dropped. Proposal 9: For NPUSCH/NPRACH overlapped with non-U NB-IoT subframes, the transmissions on non-D NB-IoT subframes should be dropped. Proposal 10: The NPRACH occasion should be determined in same way as legacy NB-IoT system. For NPRACH starting from a RO, the NPRACH repetitions overlapped with non-U NB-IoT subframes can be dropped. Proposal 11: The dropped NPUSCH/NPRACH can be counted for segment determination of pre-compensation, which simplifies the implementation. |
| R1-2500447 | OPPO [OPPO] | Proposal 1: If Iridium allows, RAN1 considers multiple DL/UL slots across 90ms period and they share the same slot index. Proposal 2: If multiple DL/UL slots can be used across 90ms period, RAN1 considers D=U=8/20/30. Proposal 3: DL synchronization is performed only in default D subframe. Proposal 4: eNB can configure larger number of subframes for D in SIB1-NB and guard period in D, so that the system can benefit from enhanced throughput. Proposal 5: No guard period is expected in default D subframe, and NPSS/NSSS/NPBCH/SIB1-NB are not expected outside default D subframe. Proposal 6: The definition of uplink time synchronization reference point remains the same, i.e., DL and UL are at least subframe aligned at reference point. Proposal 7: eNB configures an offset to determine the starting U subframe based on the location of the D subframe. Proposal 8: Adopt option 1 as the only option for DL synchronization. Proposal 9: The option 1 is used to determine the default D location during initial DL sync. Proposal 10: RAN1 adopts Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. Proposal 11: Consider to shorten H-SFN length by cutting off 16 radio frames. Proposal 12: RAN1 considers to extend the SIB1-NB transmission period as well as the one SIB1-NB transmission duration by a factor of N=9. Proposal 13: RAN1 considers to align the SI window with period D subframe start, and change the SI window length and period to multiple of 90 ms. Proposal 14: NPRACH period configuration can be modified to add a period of 90ms or multiple of 90ms. Proposal 15: When NPRACH transmission cannot fit into U subframe, postponing NPRACH to next U subframe can be considered with postpone granularity of symbol group. Proposal 16: RAN1 considers search space splitting over multiple D subframes. Proposal 17: RAR window and CR timer starting time postpone is to be considered to ensure the starting time is in D subframe. Proposal 18: Outside D subframe, the CR timer can be paused. Proposal 19: For scheduled NPDSCH/NPUSCH, postponing and splitting can be considered to make the transmission within D/U subframes for 90ms TDD structure. Proposal 20: UL segment should only be defined in consecutive U subframes. Proposal 21: RAN1 considers to extend the paging cycle by a factor of N=9. Proposal 22: RAN1 considers the NRS can be only transmitted in the D NB-IoT DL subframes on top of the current UE assumption. Proposal 23: RAN1 considers to count the transmission gap only in the D NB-IoT DL subframes. |
| R1-2500509 | Nokia, Nokia Shanghai Bell [NOK] | Proposal 1: RAN1 should discuss how to allocate the period of active contiguous subframe for UL to cover all the UE with different differential delay. Proposal 2: For the selection of the UL and DL duration, RAN1 should consider how to guarantee the periodic Tx/Rx are in the available slot. Proposal 3: for down selection of the DL subframe options, the aspect that NPSSS should be before NSSS in the same DL duration should be considered. Proposal 4: For transmission of periodic channels, RAN1 shall consider SFN index should be counted based on only available SFN, instead of all SFN that includes many unavailable SFNs. Proposal 5: RAN1 should consider adjusted SFN index counting, e.g. some SFN duration is ignored in SFN index counting, to guarantee no degradation of periodic channels. Proposal 6: RAN1 to discuss whether HARQ feedback is needed between two active radio frames separated according to the periodicity N. Proposal 7: RAN1 should study how to have accurate DL synchronization and cell search in this IoT NTN TDD mode. Proposal 8: RAN1 to send LS to RAN4 to study both DL synchronization and cell search issue in this IoT NTN TDD mode. Proposal 9: Before make furthere study on the impact to the specification and feasibility of the mapping of the channel/signals, the exact information on the starting/ending time of the subframes in UL and DL duration should be clear. Proposal 10: RAN1 discuss reusing FDD segment precompensation method in IoT NTN TDD. Proposal 11: RAN1 to discuss the impact on the achievable link budget if the number of repetitions and resource unit length are restricted due to the use of every Nth radio frame and a limited number of DL/UL subframes. Proposal 12: RAN1 to evaluate the cell capacity when accounting for at least NPSS, NSSS, NRS, MIB and relevant System Information Blocks (at least SIB1-5 and SIB31). Proposal 13: RAN1 should evaluate the UL capacity for NPRACH and NPUSCH considering the active radio frame per N radio frame. |
| R1-2500524 | Iridium Satellite LLC [IRI] | Proposal 1: When operating in NB-IoT NTN TDD mode, there is no need to update the SIB-1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time. Proposal 2: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time. Proposal 3: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity. Proposal 4: In IoT-NTN TDD mode, SI transmissions shall be truncated, if due to the proposed postponement they overlap with the transmission of the next SI repetition within the SI window. Proposal 5: In IoT-NTN TDD mode, SI transmission shall be truncated if, due to the proposed SI transmission postponement, it overflows the configured SI window length. Proposal 6: In NB-IoT NTN TDD mode, by making modifications to the SI scheduling mechanism, all SI messages can be delivered to the UE. This can be achieved by updating 36.331 §5.2.3a as follows: 1> if the UE is a NB-IoT UE: 2> receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB, and SystemInformationBlockType1-NB, and in the IoT-NTN TDD mode excluding subframes outside of active downlink time. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern; In the IoT-NTN TDD mode, if postponement of SI subframes due to downlink inactive time results in overlap with the subframes of the next SI repetition, or falls outside SI window, SI transmission should be truncated to the last non-overlapping subframe or last subframe within the SI window. Proposal 7: In IoT-NTN TDD mode, the definition of NB-IoT downlink subframe shall be expanded to exclude downlink inactive subframes by updating the text in 36.213 §16.4 as follows: A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows: - If the UE is operating in IoT-NTN TDD mode and the subframe is outside the downlink active period the subframe is not assumed as a NB-IoT DL subframe. - If the UE determines that the subframe contains NPSS/NSSS/NPBCH/SystemInformationBlockType1-NB … Proposal 8: In IoT-NTN TDD mode, the definition of NB-IoT uplink subframe shall be expanded to exclude uplink inactive subframes by updating the text in 36.213 §16.5 as follows: A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows: - If the UE is operating in IoT-NTN TDD mode and the UL subframe is within the UL active period. - If higher layer parameter resourceReservationConfigUL is configured… Proposal 9: In IoT-NTN TDD mode, with NPRACH repetition > 1, whole preamble repetition unit consisting of P symbols groups should be transmitted in consecutive UL active time opportunities. This can be achieved by adding text in 36.211 §10.1.6.1 as follows: In IoT-NTN TDD mode, when the transmission of P symbol groups overlaps with a subframe which is not NB-IoT UL subframe, the start of the P symbol groups will be postponed to the next NB-IoT UL subframe. Proposal 10: In IoT-NTN TDD mode, the uplink timing and frequency pre-compensation shall always be done either at the start of the uplink active time or during the uplink active time. This can be achieved by adding text in 36.213 §16.1.2 as follows: For a NB-IoT UE communicating over NTN, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of Nsegmentprecompensation time units, where the quantity Nsegmentprecompensation is provided by higher layers, as specified in 3GPP TS 36.331 [11]. In IoT-NTN TDD mode, for any uplink segments not starting in a NB-IoT uplink subframe, time and frequency pre-compensation adjustment is postponed to the start of the next NB-IoT uplink subframe. Proposal 11: In IoT-NTN TDD mode, the NPDSCH transmission gap shall always start in the downlink active time. This can be achieved by adding text in 36.211 §10.2.3.4 as follows: The NPDSCH transmission can be configured by higher layers with transmission gaps where the NPDSCH transmission is postponed. There are no gaps in the NPDSCH transmission if Rmax<Ngap, threshold where Ngap, threshold is given by the higher layer parameter dl-GapThreshold and Rmax is given by [4]. The gap starting frame and subframe is given by 10nf+ns2mod Ngap,period=0 where the gap periodicity Ngap,period is given by the higher layer parameter dl-GapPeriodicity. The gap duration in number of subframes is given by Ngap,duration=Ngap,coeffNgap,period, where Ngap,coeff N is given by the higher layer parameter dl-GapDurationCoeff. In IoT-NTN TDD mode if the gap starting frame and subframe falls in a subframe which is not an NB-IoT downlink subframe, the transmission gap shall be postponed until the start of the next NB-IoT downlink subframe. Proposal 12: For the IoT-NTN TDD band spanning 1616.0 to 1626.5 MHz, set the downlink active time to start at subframe #3 when (N=9) and (D=8). This means the downlink active time should include subframes [3, 4, 5, 6, 7, 8, 9, 0]. Proposal 13: For the IoT-NTN TDD 1616-1626.5 MHz band, set the time gap to 50ms from the end of the downlink active time to the start of the next uplink active time. Proposal 14: Increase Kmac, effectively either by increasing defined value from 9 to 10 bits, or by defining another optional parameter “Kmac_extension”, and the two together would allow for larger delay. |
| R1-2500720 | Xiaomi [XMI] | Proposal 1: The iridium DL slots and UL slots allocated to 3GPP IoT NTN are UL#3 and DL#3. Proposal 2: The downlink subframes within D=8 are: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) Proposal 3: If option 1 is pursued, option 1-1 is preferred. Otherwise, option 2-1 with implicit offset determination is preferred. Proposal 4: New periodicities of NPRACH can be considered to align with the TDD structure and handle the overlap of NPRACH with non-U NB-IoT subframes. Proposal 5: 3.75kHz is not supported in IoT-NTN TDD mode operation. Note: There is no specification impact with such proposal |
| R1-2500807 | Apple [APP] | Observation 1: To be compatible with satellite frame structure, the time offset between satellite frame and NB-IoT TDD mode pattern is in the unit of 0.01ms. Proposal 1: 90ms TDD pattern can be indicated by bitmap or via time offset relative to the DL slot. Observation 2: If only one uplink slot and downlink slot pair is configured for IoT NTN TDD mode operation, the natural guard period is enough to protect the UL transmission and DL reception. Proposal 2: To support IoT-NTN TDD mode, extend the NB-IoT DL subframe indication peridocity to 90ms. Proposal 3: Network indicates the valid uplink subframes within 90ms TDD pattern via bitmap - The NPUSCH transmissions are only allowed in valid uplink subframes. - The NPUSCH transmission in the invalid subframe is postponed to the next available valid uplink subframe. Proposal 4: For IoT-NTN TDD mode, define new NPRACH periodicities based on a 90ms interval. Proposal 5: NPRACH format 2 is not supported for IoT-NTN TDD mode. Proposal 6: The offset between the 90ms TDD structure and the 10240ms H-SFN is derived through UE implementation. |
| R1-2500870 | Samsung [SS] | Proposal 1: Support only one option among the following candidates. - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) - Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames) - Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames) - Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames) Proposal 2: Consider available UL subframes to determine NPUSCH resource. Proposal 3: Consider available DL subframes to determine NPDCCH/NPDSCH resource. Observation 1: For offset between 90ms TDD structure and H-SFN - Option 1-1 needs to check feasibility from RAN2. - Option 1-2 needs more clarification. - Option 1-3 requires large specification changes. - Option 1-4 is another variation of option 1-3, and needs to check feasibility from RAN2. |
| R1-2500889 | Lenovo [LEN] | Proposal 1: A time offset between the TDD pattern and the NBIoT frame boundary should be introduced, the time offset can be fixed as 3 or 4 subframes. Proposal 2: The number of consecutive DL subframes in TDD pattern is further indicated by system information. Proposal 3: The number of consecutive UL subframes of 8 in TDD pattern is the baseline and can be further indicated by system information (can also be derived from the number of consecutive DL subframe). Proposal 4: The existing pre-configured resources for SI/paging need to further study. Proposal 5: New NRACH resources configuration need to further study, at least to align with the new TDD pattern. |
| R1-2500982 | TCL [TCL] | Proposal 1: RAN1 should take the impact of frame structure on the formula of NSSS into consideration. Proposal 2: For IoT NTN TDD mode, RAN1 can consider the following options to reduce the delay of MIB-NB reception: - Option 1: Modify the MSB bits of MIB-NB to decrease the repetition of NPBCH block - Option 2: Repetition of NPBCH block on the other available subframe in a radio frame Proposal 3: The reference time of TDD pattern can align with the radio frame e.g. SFN#0. Proposal 4: To implement TDD pattern, the method of parameter configuration should be based on the least signaling overhead. Proposal 5: The orphan radio frames can be skipped directly before SFN wrap around. Proposal 6: Postpone the transmission of NPRACH to the next available uplink resource when it overlaps with non-Uplink subframes. |
| R1-2501044 | LG Electronics [LGE] | Proposal 1: RAN1 does not pursue specific enhancement for NPSS/NSSS/NPBCH transmissions and receptions in Rel-19 IoT-NTN TDD mode WI. Proposal 2: For IoT-NTN TDD mode, TDD pattern always include subframe#3 to ensure additional SIB1 transmission enabled. Proposal 3: RAN1 does not pursue specific enhancement for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band in Rel-19 IoT-NTN TDD mode WI. Proposal 4: RAN1 does not pursue specific enhancement for adjusting the offset between the 90ms TDD structure and the 10240ms H-SFN in Rel-19 IoT-NTN TDD mode WI. Proposal 5: For IoT-NTN TDD mode, one or more of followings can be considered to define UL subframes and their locations for the TDD pattern: - Modifying NPRACH periodicity and start time considering the TDD pattern with N=9. Proposal 6: For IoT-NTN TDD mode, one or more of followings can be considered for initial access procedure: - RAR window starts in DL subframe provided by the TDD pattern subject to the existing required time. - Subframe offset to indicate RAR-to-Msg3 TX timing is prolonged to ensure that Msg3 is mapped on UL subframe provided by the TDD pattern. - UE (re)starts the contention resolution timer at the DL subframe provided by the TDD pattern subject to the existing required time. |
| R1-2501075 | Sharp [SH] | Proposal 1: RAN1 should clarify if the IoT NTN TDD frame structure follows the same starting points and periods as in legacy TDD. Proposal 2: If LTE frame structure is not enforced, the IoT NTN TDD DL and UL timeslots can be defined at the center of legacy DL and UL slots with corresponding offset or shift values. Proposal 3: If LTE frame structure is maintained, the IoT NTN TDD timeslots can be defined with LTE slot level alignment with possible symbol level offset for IoT NTN DL or UL timeslots to provide D=8 and U=8 that can be fully confined in the legacy DL and DL slots. Proposal 4: For an IoT NTN DL timeslot with 8 full subframes, i.e. D=8, the NPBCH, NPSS and NSSS (if present) are mapped to subframe 0, subnframe 4 and subframe. FFS the case with only 7 full subframes. |
| R1-2501177 | Qualcomm Incorporated [QC] | Proposal 1: The downlink subframes within D=8 are: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) Proposal 2: The offset between the DL TDD pattern and the 3GPP radio frame boundary is derived by the UE based on the detection of NPSS. - No additional signaling is required Proposal 3: The offset between the 90ms TDD structure and the 10240ms H-SFN is - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - It is up to RAN2 whether to also adopt Option 2-2 and change the total number of H-SFNs to be a multiple of 9. Proposal 4: The UE determines the set of contiguous uplink subframes as follows: - The set of UL “subframes” is the set of DL subframes minus an offset at the ULSRP - The offset is fixed in the specifications. - NOTE: The eNB can adjust the guard period by common TA (no specification change needed) Proposal 5: DL subframes not contained within the downlink burst are not NB-IoT DL subframes. - NPDCCH and NPDSCH not carrying SIB1 are postponed until the next NB-IoT DL subframe in case of collision with non-NB-IoT DL subframes. Proposal 6: RAN1 specifies a mechanism to enable NPDCCH monitoring in every downlink burst. Proposal 7: The UE may assume that NRS is available in every DL subframe not including sync signals. Proposal 8: Transmissions of NPUSCH / NPRACH that collide with non-U slots are postponed until the next U-slot. - For NPRACH, the unit of postponement is a PRU. - For 3.75kHz SCS, the unit of postponement is 1 slot (2ms) - For 15khz SCS, the unit of postponement is 2 slots (1ms) Proposal 9: For segmented uplink pre-compensation, and for a transmission that spans multiple uplink bursts, a new pre-compensation segment is started at the beginning of each uplink burst. |
| R1-2501219 | NTT DOCOMO, INC. [DCM] | Proposal 1: Select either one of the following for the downlink subframes within D=8: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) - Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames) Proposal 2: Support adjustable guard period for IoT-TDD pattern design. - The number of flexible D/U is configured/indicated if needed. Proposal 3: The offset between the 90ms TDD structure and the 10240ms H-SFN is: - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB). Proposal 4: Regarding the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPRACH (including NPRACH occasions) with non-U NB-IoT subframes, define new periodicities for UL channels/signals to align with the TDD structure. - E.g., new periodicities of 90ms or multiples of 90ms are configured. Proposal 5: Regarding the overlap of NPDCCH/NPDSCH with non-D NB-IoT subframes, define new periodicities for DL channels/signals to align with the TDD structure. - E.g., new periodicities of 90ms or multiples of 90ms are configured. |
| R1-2501295 | Nordic Semiconductor ASA [NOR] | Proposal-1: Support Option 1 (starting SF#3) or Option 2 (starting SF#4). Only a single Option is supported by specification for the target MSS band. Observation-1: With 90ms pattern D=8 and N=9, a R19 TDD UE may at best obtain 14-15 NPDSCH subframes containing SIB-1 during 2.56s period. Proposal-2: Conclude that no specification changes are required for reception of SIB1-NB in R19 TDD NTN IoT. Proposal-3: Adopt the following new rules for SIBx reception: - Rule#1: First DL subframe of SIBx is the first available D subframe after the start of SI window. - Rule#2: SIBx is transmitted only within D subframes within corresponding SI window. Proposal-4: SIB31 is transmitted the same way as in legacy R17 NTN IoT. For SIB2, consider to either reuse the legacy (with additional rules from Proposal-2) or alternatively SIB2 could follow scheduling procedure of SIB1, except using alternative subframes. Proposal-5: To improve UE power consumption in R18 TDD IoT NTN system, consider - additional NSSS located in DL burst starting subframe that is other than SF#9. - consider providing assistance information to a UE regarding D/U configurations of adjacent anchor and non-anchor carrier(s). Proposal-6: Conclude that SFN and H-SFN wrap-around does not require any specification change. Proposal-7: Consider adjustments in how current NPRACH configurations are interpreted. This to match N=9. Such NPRACH periodicity would be strictly periodic and would range from 90ms to 2610ms. Proposal-8: Initial NPRACH transmission is always postponed to the first available U subframe, NPRACH repetitions (when configured) are postponed to subsequent 90ms DL burst(s) and start from the first U subframe of each DL burst. Proposal-9: NPDCCH and NPDSCH channels are postponed to the next valid DL subframe(s). Discuss further whether any specification changes are required. Proposal-10: In R19 TDD NTN NB-IoT system, maximum supported Rmax is [8] and minimum supported G is [v16]. |

# References

[1] RP-243293, New WID on introduction of IoT-NTN TDD mode