3GPP TSG RAN WG1 #120                                                                                                R1- 24xxxxx

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.1.4.1

Source: 	Moderator (Qualcomm)

Title: 		Final summary of Additional study on AI/ML for NR air interface: CSI compression

Document for:	Discussion and Decision

# Introduction

In RAN#102, a new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The WID includes study objectives related to AI/ML for CSI compression using a two-sided model.

…… Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - …… ……

In RAN#105, the WID was revised as follows

… Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. …

This document summarizes the issues regarding agenda item 9.1.4.1 (Additional study on AI/ML for NR air interface: CSI compression) in RAN#120.

## Contact Information

Please provide / update your contact information.

| Company | Name | E-mail |
| --- | --- | --- |
| Moderator (Qualcomm) | Taesang Yoo | taesangy@qti.qualcomm.com |
| Lenovo | Vahid Pourahmadi | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| Indian Institute of Technology Madras | Anil Kumar Yerrapragada | anilkumar@5gtbiitm.in |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| vivo | Peng SUN | sunpeng@vivo.com |
| SK telecom | Yunesung Kim | yunesung.kim@sktelecom.com |
| Mavenir | Ali Fatih Demir | ali.demir@mavenir.com |
| Panasonic | Tetsuya Yamamoto Hidetoshi Suzuki | yamamoto.tetsuya001@jp.panasonic.com suzuki.hidetoshi@ jp.panasonic.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| NTT DOCOMO | Xin Wang Haruhi Echigo | wangx@docomolabs-beijing.com.cn haruhi.echigo.fw@nttdocomo.com |
| ETRI | Anseok Lee | mailto:alee@etri.re.kr |
| Ericsson | Jingya Li Siva Muruganathan Jianwei Zhang | mailto:Jingya.li@ericsson.com Siva.muruganathan@ericsson.com Jianwei.zhang@ericsson.com |
| ZTE | Lun Li Xingguang Wei | li.lun1@zte.com.cn wei.xingguang@zte.com.cn |
| IIT Kanpur | Chethan Ranganatha | chethanr@iitk.ac.in |
| Spreadtrum | Mimi Chen | Mimi.chen@unisoc.com |
| LG | Jaehoon Chung | jhoon.chung@lge.com |
| CATT | Qianrui Li | liqianrui@catt.cn |
| Intel | Victor Sergeev | Victor.sergeev@intel.com |
| Xiaomi | Liuzhengxuan Liumin | liuzhengxuan@xiaomi.com liumin10@xiaomi.com |
| AT&T | Isfar Tariq Salam Akoum | Isfar.tariq@att.com Salam.akoum@att.com |
| CMCC | Yuhua Cao Yi Zheng Dan Song | caoyuhua@chinamobile.com zhengyi@chinamobile.com songdan@chinamobile.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| MediaTek | Pedram Kheirkhah Sangdeh | Pedram.kheirkhah@mediatek.com |
| Futurewei | Baoling S Sheen | bsheen@futurewei.com |
| TCL | Pu Yuan Tianqi Wu | pu.yuan @tcl.com tianqi1.wu@tcl.com |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| CEWiT(TSDSI) | Shiv Shankar Dhivagar Baskaran | shivshankar@cewit.org.in dhivagar.b@cewit.org.in |
| NEC | Zhen He Pravjyot Deogun | he_zhen@nec.cn pravjyot.deogun@EMEA.NEC.COM |
| Qualcomm | Chenxi Hao | chenxih@qti.qualcomm.com |
| Sony | Hiroki Matsuda Sam Atungsiri | hiroki.matsuda@sony.com sam.atungsiri@sony.com |
| Apple | Huaning Niu | Huaning_niu@apple.com |
| Tejas Networks | Pavan Kalyan | pavankalyand@tejasnetworks.com |

# Temporal domain aspects of CSI compression

## Summary of company proposals

### Case 1/2/5

Ericsson

Proposal 18	Deprioritize AI CSI compression use case 2.

ZTE

Proposal 1:  To reduce the efforts for specification impact analysis and standardization, conduct comparison between Case 2 and Case 3 for potential down selection.

Huawei, HiSilicon

Proposal 16: For the additional potential spec impact of temporal domain CSI compression Case 2 on top of Rel-18 SF domain CSI compression, consider methods to handle the misalignment of the accumulated CSI between NW part model and UE part model due to UCI missing, e.g., NW indicating the reset of historical CSI, or indicating CSI retransmission.

CATT

Proposal 1: For Case 2, specification support on dealing with non-ideal UCI feedback (i.e., UCI loss) is not needed.

Tejas Networks Ltd

Proposal 1: For the EVM of temporal domain CSI compression, consider the following assumptions for the CSI generation part and CSI reconstruction part, respectively:

- CSI generation part at t=T:

- Model input: Pre-processed channel matrix is given as a input to the CsiNet encoder.

- First layer of encoder is Convolutional layer with real and imaginary parts of channel as its input.  This layer uses 3×3 kernel size to generate two feature maps. Following the convolutional layer, we reshape the feature maps into a vector and use a fully connected layer to generate the codeword s, which is a real-valued vector of size M.

- Model output: Encoder transforms channel matrix into a vector of size M×1.

- CSI reconstruction part at t=T+∂ (where ∂ is an uplink latency)

- Model input: Codeword of size M×1is given as a input to the CsiNet decoder.

- Model output: First layer of decoder is dense fully connected layer which gives the initial estimate of the channel followed by several RefineNet units gives the CSI.

FFS: Study the effect of quantization

Proposal 3: For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model, for case2:

- Scenario A provides the baseline for comparison, offering the best-case performance under ideal conditions.

- Scenario B highlights the need for NW-side mitigation strategies, but its effectiveness is limited due to UE-side mismatches.

- Scenario C offers the most comprehensive mitigation, may enable significant performance recovery through coordinated actions between NW and UE.

InterDigital

Proposal 1: Study the specification impact of adapting the length of the historical CSI buffer to channel conditions, such as UE speed.

Proposal 4: TSF compression performance should be evaluated under multiple observation window lengths.

Proposal 5: For further performance improvement of AI-TSF compression Case2 with missing UCI, study different missing UCI mitigation solutions e.g., partial buffer reset, or retransmission of the missed UCI.

Proposal 6: For CSI compression Case 2 with beam domain processing, study long-term reporting of the selected beams associated with the BDP, e.g., as a function of the UE speed.

Google

Proposal 1: For case 2 and case 3, support the UE to report W1 and compressed W2 for a configured rank

- The compressed W2 is calculated based on the AI/ML

- NW can configure the UE to report RI and CQI based on precoded CSI-RS resources after receiving the W1 and compressed W2

- Proposal 2: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report.

Proposal 2: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report.

TCL

Proposal 1: RAN1 should study the benefits and potential issues if temporal domain compression Case 2 is adopted, especially in the following aspects:

- The overall performance gain considering both the accuracy loss and the overhead reduction of the temporal domain compression Case 2.

- The performance monitoring metrics of the temporal domain compression Case 2.

- The potential problem of error accumulation and its solutions.

LG Electronics

Proposal #1: Regarding temporal/spatial/frequency (TSF)-domain CSI compression, study methods/mechanisms to manage the similarity/synchronization of accumulated past CSI at UE-side and/or NW-side.

Proposal #2: Regarding TSF-domain CSI compression, discuss the format of past CSI information and how to report it at least for performance monitoring perspective.

NEC

Proposal 9: Reset of historical CSI information should be supported to address UCI loss. And the definition, determination or indication of the reset value (e.g., previous historical CSI information, initial state) need to be further studied.

Proposal 10: For the case of UCI loss is known to both NW and UE, e.g., CSI report is dropped, NW-signaling is not needed for reset of historical CSI information at UE side.

Proposal 11: At least for Case 2, further study effective availability of historical CSI information over time.

Lenovo

Proposal 21: 	Support procedures/signalling enabling CSI-compression models having both Scaler and vector Quantizers for generation of the CSI-feedback bits.

Xiaomi

Proposal 1: To align the missed historical CSI information when UCI loss occurs, the latest output historic CSI of encoder and decoder before missing UCI could be respectively adopted as the input of encoder and decoder in the current inference instance.

Apple

Proposal 2: For case 2, time-frequency-spatial domain CSI compression, the following potential specification impact are proposed:

- Enable semi-persistent CSI reporting

- Enable DCI based reset state

- Considering UCI retransmission in case of large amount of UCI drop or loss, to avoid the state at UE and gNB out of sync.

- For RI, consider longer term RI update across different CSI reports.

Proposal 4: For case 3 of time-frequency-spatial domain CSI compression, option 2a ensures E2E performance. The intermediate KPI or eventual KPI includes both compression and prediction performance.

Samsung

Proposal#1: For the Case 2 of temporal aspects of AI/ML-based CSI compression using two-sided model, consider at least the following two options for the past CSI information

•	Case 2-1: Past CSI information generated by the UE-part and/or network-part of two-sided model

•	Case 2-2: Information on SD/FD basis vectors as past CSI information with angle-delay domain compression.

Proposal#2: For the Case 2 of temporal aspects of AI/ML-based CSI compression using two-sided model:

•	when past CSI information corresponds to SD/FD basis and AI/ML CSI compression is in the angle-delay domain, consider SD/FD basis reporting per N CSI reporting occasions, i.e., N times longer periodicity.

•	FFS on the values of N.

Proposal#7 In Angle-delay (W2)-domain CSI compression, study the impact of the number of SD/FD basis vectors for performance-complexity tradeoff.

ETRI

Proposal 1: For AI/ML-based CSI compression using two-sided model, when UE and/or NW uses past CSI information, reuse the current specification on CSI-RS transmissions as much as possible.

Proposal 2: For AI/ML-based CSI compression using two-sided model, when NW uses past CSI information, study method to detect and mitigate inconsistency of the availability of past CSI information between the UE and the NW.

Nokia

Proposal 2: RAN1 to consider adopting a recurrent quantization approach (Option 2) for temporal domain Case 2.

Mavenir

Proposal1: Mixed dataset should be considered as an effective way to increase AI model generation.

Proposal2: AI model performance should be evaluated on datasets with different scenario and configuration.

### Case 3/4

Ericsson

Proposal 17	Further study methods to reduce the AI computational complexity for AI CSI compression Case 3.

ZTE

Proposal 1:  To reduce the efforts for specification impact analysis and standardization, conduct comparison between Case 2 and Case 3 for potential down selection.

Huawei, HiSilicon

Proposal 17: For the additional potential spec impact of temporal domain CSI compression Case 3 on top of Rel-18 SF domain CSI compression, consider SPC (separate prediction and compression) with higher priority.

- Regarding the type of Target CSI for training data collection, consider Option 1 (based on future predicted CSI) with higher priority, which can ensure consistent type of Target CSI between training and inference.

- Regarding the type of monitoring label for monitoring, consider Option 1 (based on future predicted CSI) with higher priority, which can ensure consistent type of Target CSI between monitoring label and inference input.

CATT

Proposal 3: In CSI compression using two-sided model use case, regarding separately monitoring of CSI prediction and CSI compression for temporal domain aspects Case 3, for NW-side monitoring for CSI compression, the specification impact of monitoring of CSI compression includes:

- For Option 1, configuration and report for target CSI of CSI compression corresponding to the output of CSI prediction;

- For Option 2b, configuration and report of target CSI of CSI compression corresponding to the measured CSI of the future slot(s) and CSI feedback based on the measured CSI of the future slot(s).

Proposal 4: In CSI compression using two-sided model use case, regarding separately monitoring of CSI prediction and CSI compression for temporal domain aspects Case 3, for UE-side monitoring for CSI compression, the specification impact of monitoring of CSI compression includes configuration of CSI-RS for CSI measurement of the future slot(s) for option 2b.

Proposal 5: In CSI compression using two-sided model use case, regarding separately monitoring of CSI prediction and CSI compression for temporal domain aspects Case 3, study whether the selection of monitoring methods for CSI prediction needs to be comprehensively considered in conjunction with the monitoring methods for CSI compression.

Vivo

Proposal 11:	For data collection of temporal domain aspects Case 3, option 2 (The target CSI for training is derived based on the measured CSI of the future slot(s)) is supported

Proposal 12:	For monitoring of temporal domain aspects Case 3, Option 1(the monitoring label is derived based on the predicted CSI of the future slot(s))

Tejas Networks Ltd

Proposal 4:

In the results template for capturing the evaluation of temporal domain aspects Case 3/4 of AI/ML based CSI compression, it is clarified that the upper bound is calculated based on ideal CSI prediction and without CSI compression.

Proposal 5:

For temporal domain aspects Case 3 and 4, study the impact on LCM aspects (e.g., data collection, training, monitoring, and model control) of separate prediction and compression vs. joint prediction and compression.

Google

Proposal 1: For case 2 and case 3, support the UE to report W1 and compressed W2 for a configured rank

- The compressed W2 is calculated based on the AI/ML

- NW can configure the UE to report RI and CQI based on precoded CSI-RS resources after receiving the W1 and compressed W2

- Proposal 2: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report.

Proposal 2: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report.

TCL

Proposal 2: RAN1 should further study the following specification impact on the monitoring of temporal domain compression Case 3:

- the monitoring data report configuration, e.g., new report quantity, indication of data type.

- the format of CSI quality

LG Electronics

Proposal #3: Regarding TSF-domain CSI compression Case 3 and 4, consider performance monitoring method on joint CSI compression and prediction by adapting the operation on the AI/ML model between CSI compression and prediction.

Proposal #4: To improve LCM aspects on TSF-domain CSI compression Case 3 and 4, consider the following:

- Data quality-based sample selection for model updates

  - Example of the quality metrics: Prediction confidence, temporal consistency, CSI reconstruction error etc.

Samsung

Proposal#4: Among the identified six categories for AI/ML-based CSI compression using two-sided model, for Case 3, consider N4≥1 prediction instances (Doppler time intervals).

Option1: AI/ML-based CSI compression in spatial-frequency-time domain

Option 2: The AI/ML-based CSI compression in angle-delay-time domain

Option 3: The AI/ML-based CSI compression in angle-delay-Doppler domain

Option1: AI/ML-based CSI compression in spatial-frequency-time domain

Option 2: The AI/ML-based CSI compression in angle-delay-time domain

Option 3: The AI/ML-based CSI compression in angle-delay-Doppler domain

Mavenir

Proposal1: Mixed dataset should be considered as an effective way to increase AI model generation.

Proposal2: AI model performance should be evaluated on datasets with different scenario and configuration.

NTT DOCOMO

Proposal 3

- Option 2a for performance monitoring of Case 3 is necessary to support both joint and separate prediction and compression schemes.

- The Rel-18 doppler codebook-like report can be used for predicted future CSI for performance monitoring Option 1.

  - FFS the necessary to enhance the Rel-18 doppler codebook.

- For the data collection for monitoring for Option 2a, an enhanced Rel-18 doppler codebook is necessary to support the report of historical CSI on multiple time instances.

## Discussion

### Potential down-selection of temporal domain cases

##### Discussion 21a: Potential down-selection of temporal domain cases

Please indicate your view on the further down-selection of Case 2 and/or Case 3 during the Rel-19 study:

|  | Supporting companies |
| --- | --- |
| Keep both Case 2 and Case 3 | Huawei/HiSilicon, Xiaomi, TCL, CMCC, NEC, LG, Tejas, NTT DOCOMO |
| Keep Case 2, deprioritize Case 3 |  |
| Keep Case 3, deprioritize Case 2 |  |
| Deprioritize both Case 2 and Case 3, and only focus on Case 0 |  |

| Company | Comments |
| --- | --- |
| Futurewei | Given that we only have 2 remaining meetings left after Feb. meeting for Rel-19, we suggest focusing on Case 0 first. Case 2 and Case 3 can follow or extend from Case 0 after good progress is made for Case 0. |
| Huawei, HiSilicon | Temporal domain compression cases are used for performance improvement to justify the objective of SID. Case 2 and Case 3 are applicable to separate scenarios of low mobility and high mobility, respectively, so keeping both and apply them to different scenarios are beneficial for deployment of AI CSI compression. |
| Xiaomi | We share same view with HW. |
| Samsung | We propose to study the inter-vendor collaboration options on the basis of Case 0 whilst keeping both cases (Case 2 and 3) for the remaining relevant aspects of the study. We suggest the following: Focus on Case 0 for the study on the inter-vendor collaboration options/directions. |
| vivo | Same view as Samsung. |
| ZTE | Considering the potential huge spec impacts for AI/ML-based CSI compression using two-sided model, standardizing the two cases in parallel will bring unaffordable spec efforts. Therefore, it’s recommended to conduct further comparisons between these two high-priority cases for potential down selection due to the limited time remaining. |
| LG | Similar view with HW |
| Fujitsu | We prefer to keep two cases considering that both cases show the performance gain/improvement over benchmarks. |
| Ericsson | We think the focus should be put on use case 0. |
| OPPO | We agree that both of Case 2 and Case 3 can provide additional gain compared to Case 0. But considering the limitation of remained meeting time, we may focus on the spec effort on Case 0. We can conclude that most of spec designs on Case 0 can be extended to Case 2 and Case 3. Specific designs related to Case 2 and Case 3 can be treated later. |
| NTT DOCOMO | At least Case 3 should be supported considering its benefit for practical deployments, reducing the UL report occasions. |
| Mod | Based on the majority opinion and observed performance gains, let’s keep Case 2 and Case 3. HW made a good point that Case 2 and Case 3 are applicable to separate scenarios of low mobility and high mobility, respectively, so keeping both and apply them to different scenarios are beneficial for deployment of AI CSI compression. At the same time, given the limited time remaining, we can focus on Case 0, esp. for inter-vendor collaboration options/directions, and try to leverage the spec design of Case 0 toward Case 2 and Case 3. |

### UCI loss for Case 2 (closed)

Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, study the performance impact resulting from non-ideal UCI feedback. - Scenario A: no UCI loss - Note: Corresponds to an upper bound or re-aligning missing historical CSI information - Scenario B: UCI loss, known at NW and unknown at UE, with mitigation at NW - Note: Corresponds to implementation-based mitigation at NW but no signaling to UE. - Scenario C: UCI loss, known at NW and UE, with mitigation at NW and UE - Note: Corresponds to reset of historical CSI information at both UE and NW or any other mitigation approach enabled by signaling. - UCI loss modeling - 10% UCI loss probability on all UCI reports - Other options are not precluded, e.g., No UCI loss for the first UCI report of each observation window, and 10% UCI loss probability for the subsequent reports of each observation window, as a shortcut to simplify the evaluation work. - Other values for UCI loss probability are not precluded. - Note: The same UCI loss modeling shall be applied to the benchmark for fair comparison. - FFS: partial PMI-related UCI loss

Here is the FL’s summary observations of companies’ evaluations from the tdocs (unchanged from RAN1 #119):

For the evaluations of UCI loss,

Performance of Case 2 with UCI loss compared to the benchmarks under the same UCI loss (fair comparison)

- The performance gain of Scenario A over the benchmark(s) is as follows.

  - Payload X

    - 1 source observes 13.71% and 8.90% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

    - 1 source observes 22% and 25% performance gain over the benchmark 2 (Case 0) for layer 1 and layer 2

  - Payload Y

    - 1 source observes 11.71% and 9.07% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

  - Payload Z

    - 1 source observes 7.54% and 6.78% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

    - 1 source observes 36% and 21% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

- The performance gain of Scenario B with 10% UCI loss over the benchmark(s) with the same 10% UCI loss is as follows.

  - Payload X

    - 1 source observes 9.08% and 3.92% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

  - Payload Y

    - 1 source observes 8% and 4.78% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

  - Payload Z

    - 1 source observes 4.07% and 2.57% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

- The performance gain of Scenario C 10% UCI loss over the benchmark(s) with the same 10% UCI loss is as follows.

  - Payload X

    - 1 source observes 10% and 4.8% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

  - Payload Y

    - 1 source observes 8.77% and 5.53% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

  - Payload Z

    - 1 source observes 4.74% and 3.23% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

Performance of Case 2 with UCI loss compared to the benchmarks without UCI loss (unfair comparison)

- The performance gain of Scenario B with 10% UCI loss over the benchmark(s) without UCI loss is as follows.

  - Payload X

    - 1 source observes 19% and 21% performance gain over the benchmark 2 (Case 0) for layer 1 and layer 2.

  - Payload Z

    - 1 source observes 13% and 0.8% performance gain over benchmark 1 (R16 eType 2) and benchmark 2 (Case 0)

- The performance gain of Scenario C 10% UCI loss over the benchmark(s) without UCI loss is as follows.

  - Payload X

    - 1 source observes 21% and 23% performance gain over the benchmark 2 (Case 0) for layer 1 and layer 2.

  - Payload Z

    - 1 source observes 27% and 13.3% performance gain over baseline 1 (R16 eType 2) and baseline 2 (Case 0)

Performance degradation of Case 2 under UCI loss and mitigation

- The loss in Scenario B (with 10% UCI loss) over Scenario A

  - 1 source observes 0.8-1% performance loss for payload X, 0.4-0.7% performance loss for payload Y, 0.1 % performance loss for payload Z

  - 1 source observes ~4% performance loss for payload X, Y, Z

  - 1 source observes 2% and 3% performance loss for layer 1 and layer 2 for Payload X.

  - 1 source observes 7.6% and 1.7% performance loss for all-zeros are fed to reconstruction and resetting to historical values.

  - 1 source observes 7.2% performance loss for Payload X for layer 1.

  - 1 source observes 16.9% performance loss for Payload Z for layer 1.

- The loss in Scenario C (with 10% UCI loss) over Scenario A

  - 1 source observes 3.8%, 3.4% and 3.5% performance loss for payload X, Y, Z

  - 1 source observes 1.2% and 1.8% performance loss for layer 1 and layer 2 for payload X.

  - 1 source observes 0.8% performance loss.

  - 1 source observes no performance loss for Payload X for layer 1.

  - 1 source observes 6% performance loss for Payload Z for layer 1.

The above results can be summarized as below.

|  | Scenario A | Scenario B | Scenario C |
| --- | --- | --- | --- |
| Benchmark 1 (R16 eType 2) | A1 | B1 | C1 |
| Benchmark 2 (Case 0) | A2 | B2 | C2 |
| Case 2 | A A/A1 = 7.5-36% gain (2 sources) A/A2 = 6.8-25% gain (2 sources) Note: there are a lot more results from a lot more sources for A/A1 and A/A2 from the results spreadsheet captured into Case 2 Observations. | B B/A1 = 13% gain (1 sources) B/A2 = 0.8-21% gain (1 sources) B/A = 0.4-16.9% loss (6 sources) B/B1 = 4.1-9.1% (1 source) B/B2 = 2.6-4.8% (1 source) | C C/A1 = 27% gain (1 sources) C/A2 = 13.3-23% gain (1 sources) C/A = 0-6% loss (5 sources) C/C1 = 4.7-10% (1 source) C/C2 = 3.2-5.5% (1 source) |

What we’re eventually looking for is the gain of Case 2 over the benchmarks under UCI loss (B/B1 and B/B2) and with mitigation signaling (C/C1 and C/2), i.e., how the gain of Case 2 over the benchmarks (A/A1 and A/A2) without UCI loss changes under UCI loss (B/B1 and B/B2) and mitigation signaling (C/C1 and C/C2). While there is only one source who directly evaluated B/B1, B/B2, C/C1 and C/C2, we can also infer B/B1, B/B2, C/C1, and C/2 based on other results available.

- One source directly evaluated C/C1 and C/C2 and showed moderate gains of 3.2-10%.

- One source evaluated C/A1 and C/A2 and showed large gain. And given that the benchmarks C1 and C2 will also be affected by the UCI loss (i.e., C1/A1<1 and C2/A2<1), the gain C/C1=(C/A1)/(C1/A1) and C/C2=(C/A2)/(C2/A2) should be even higher than the gain C/A1 and C/A2.

- Several sources evaluated C/A, showing negligible-to-moderate loss. And many sources contributed toward the evaluation of Case 2 over the benchmarks (A/A1 and A/A2) under no UCI loss, which are captured in the results templates and FL observations, wherein the gains A/A1 and A/A2 are much larger than the loss observed in C/A. Combining the two, the gain C/A1=(A/A1)*(C/A) and C/A2=(A/A2)*(C/A) should be decent. And following the above method, the gain C/C1=(C/A1)/(C1/A1) and C/C2=(C/A2)/(C2/A2) should be even higher than the gain C/A1 and C/A2.

Overall, we can draw conclusion that the gain of Case 2 over the benchmarks is expected to be largely preserved under the presence of UCI loss.

##### Proposal 22a: UCI loss (Same as Proposal 21b of RAN1 #119)

Conclude that, under 10% UCI loss,  Case 2 shows small to large performance drop (by Comparing Scenario B and Scenario A). Signaling support to mitigate the UCI loss can improve the performance and helps preserving the gain of Case 2 over Case 0 (by Comparing Scenario C and Scenario A and benchmark schemes).

Consider the following approaches for signaling support for mitigating the impact of UCI loss

- NW-signaling to reset of historical CSI information at UE

- NW-triggered CSI retransmission

| Support / Can accept | Futurewei, Huawei/HiSilicon, Panasonic, Xiaomi, TCL, CMCC, NEC, LG, Tejas |
| --- | --- |
| Object / Have a concern | NTT DOCOMO |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support. From the observations, UCI missing may jeopardize the synchronization of the CSI buffer between NW part model and UE part model. Introducing a mechanism to resolve the mis-alignment is needed. Note that Scenario B may not resolve the problem, as the NW side is not aware of the RI due to UCI missing, so the NW does not know the specific layers to update the CSI buffer. From our view, reset of historical CSI information could be a simple way to re-align the CSI buffer. Moreover, considering there may be other factors impacting the synchronization of both sides, e.g., error propagation, inference failure, etc., having a mechanism, e.g., CSI buffer resetting, is necessary to fix the problem. |
| Xiaomi | Support We prefer to NW-signaling to reset of historical CSI information at UE. |
| Samsung | Ok. |
| OPPO | Not very clear. Can FL add some details on how these two bullets work? |
| NTT DOCOMO | We have concerns about the CSI retransmission. It introduces unnecessary overhead, since the CSI has been aged at the time of the retransmission. |

##### Proposal 22b: UCI loss

Conclude that, under 10% UCI loss,  Case 2 shows small to large performance drop (by Comparing Scenario B and Scenario A). Signaling support to mitigate the UCI loss can improve the performance and helps preserving the gain of Case 2 over Case 0 (by Comparing Scenario C and Scenario A and benchmark schemes).

Consider the following approaches for signaling support for mitigating the impact of UCI loss

- NW-signaling to reset of historical CSI information at UE

- FFS: NW-triggered CSI retransmission

| Company | Comments |
| --- | --- |

##### Conclusion

Under 10% UCI loss,  Case 2 shows small to large performance drop (by Comparing Scenario B and Scenario A). Signaling support to mitigate the UCI loss can improve the performance and helps preserving the gain of Case 2 over Case 0 (by Comparing Scenario C and Scenario A and benchmark schemes).

##### Agreement

Investigate the following approaches for signaling support for mitigating the impact of UCI loss

- NW-signaling to reset of historical CSI information at UE

- NW-triggered CSI retransmission

### LCM aspects and specification impacts of Case 3

In the RAN1#119 meeting, the following Options have been agreed for further study for data collection, and monitoring, and other LCM aspects and specification impacts of Case 3.

Agreement For temporal domain aspects Case 3, study LCM aspects and specification impacts, consider the following options for training data collection - Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s). - Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s). - Note: During inference, the input to the CSI generation part is derived based on the predicted CSI. consider following options for the monitoring labels - Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s). - CSI prediction output is used as input to CSI generation part. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. - Option 2: The monitoring label is derived based on the measured CSI of the future slot(s) - Option 2a: CSI prediction output is used as input to CSI generation part. - Note: This corresponds to end-to-end monitoring of CSI prediction and compression. - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. Study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts.

Based on discussions brought by companies, the following proposals are made. However, the FL thinks that more analysis can be added, so please provide inputs.

##### Discussion 23a: Training data collection options for Case 3

For temporal domain aspects Case 3, regarding training data collection Option 1 and Option 2, the following observations are made:

| - Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s). - Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s). - Note: During inference, the input to the CSI generation part is derived based on the predicted CSI. |  |  |
| --- | --- | --- |
|  | Option 1 | Option 2 |
| Target CSI availability | Target CSI is not available at UE if UE implements a single AI/ML model for joint prediction and compression. | Target CSI is available at UE. |
| Consistency between training and inference | Consistent | Inconsistent. Performance degradation should be studied. |
| Overhead | Separate data collection is needed separate from Case 0. | Data collection from Case 0 can be reused. |

|  | Support / Can accept | Object / Have a concern |
| --- | --- | --- |
| Option 1 | Huawei/HiSilicon, Xiaomi, TCL, NEC |  |
| Option 2 | Vivo, ZTE, Tejas, NTT DOCOMO |  |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | It should be noted that the Observation window property may be different from the Prediction window property, e.g., number of CSI instances, periodicity, etc. Thus, using measured CSI in Observation window may cause mismatched training data property and inference data property. |
| Xiaomi | Option 2 may not accurately monitor model performance. |
| vivo | We prefer the option 2, that is because even if there is error for each data between measured CSI and predicted CSI, but the overall distribution between predicted CSI and measured CSI should be similar, thus taking measured CSIs of the future slot(s) as training dataset is reasonable, otherwise, the accuracy of predicted CSI is questionable. |
| ZTE | Using measured CSI of the future slot(s) in option 2 can help to obtain higher achievable performance for CSI compression and reduce the data collection overhead, while the distorted training label (i.e., predicted CSI of the future slot(s)) in option 1 may limit the end-to-end performance. |
| LG | Partly agree with Huawei’s comment. But, regarding Option 1, it also has some issues, e.g., it may cause error propagation when predicted CSI is not accurate and so on. So, selecting a specific option may not address the problem on target CSI for training for Case 3. |
| Fujitsu | Support Option 2. |
| OPPO | The pros and cons for Option 1 and 2 should be further clarified. |
| NTT DOCOMO | Prefer Option 2 to collect consistent data for a better E2E performance. |
| Mod | Please provide any other pros/cons not listed in the above table. I don’t think we can choose one option at this point, but let’s at least collect pros/cons analysis. In my understanding, also, there is a feasibility issue with Option 1 if UE implements joint prediction and compression. |

##### Discussion 24a: Monitoring label options for Case 3

For temporal domain aspects Case 3, regarding the monitoring labels Option 1, Option 2a, and Option 2b, the following observations are made:

| - Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s). - CSI prediction output is used as input to CSI generation part. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. - Option 2: The monitoring label is derived based on the measured CSI of the future slot(s) - Option 2a: CSI prediction output is used as input to CSI generation part. - Note: This corresponds to end-to-end monitoring of CSI prediction and compression. - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. |  |  |  |
| --- | --- | --- | --- |
|  | Option 1 | Option 2a | Option 2b |
| Label availability | Label is not available at UE if UE implements a single AI/ML model for joint prediction and compression. | Label can be obtained at UE by measurement. | Label can be obtained at UE by measurement and performing inference. |
| Complexity | No additional inference is needed for monitoring label generation. | No additional inference is needed for monitoring label generation. | Dedicated inference is required for monitoring, which adds complexity. |
| Latency | Lowest latency among the three options since the predicted future CSI can be reported to the NW immediately with the CSI feedback. | Monitoring metric cannot be calculated until the measured future CSI is obtained by the UE. | Monitoring metric cannot be calculated until the measured future CSI is obtained by the UE. |

|  | Support / Can accept | Object / Have a concern |
| --- | --- | --- |
| Option 1 | Huawei/HiSilicon, Xiaomi, NEC, vivo, ZTE |  |
| Option 2a | NTT DOCOMO |  |
| Option 2b |  | NTT DOCOMO |

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Option 1 is a straightforward method to monitor the CSI compression model. Option 2a cannot pinpoint the problem of the CSI compression model. Option 2b may have the inference/monitoring data inconsistency issue. |
| ZTE | Since the measured CSI is available at the UE side, the management and monitoring of CSI prediction part can be handled by UE implementation. Then, the core part is to monitoring the performance of the CSI compression part solely, which means that the input of the CSI compression part and the monitoring label should be the same (i.e., Option 1 and Option 2b). Additionally, consider that CSI prediction output is used as input to CSI generation part during model inference. It’s better to adopt Option 1 to accurately monitoring the realistic model inference performance. |
| LG | Similar view as the above |
| Fujitsu | We prefer to monitor CSI compression only, i.e., Option 1 and Option 2b. When CSI prediction does not work well, Option 2b may be more suitable to do the performance monitoring. |
| OPPO | The pros and cons for Option 1 and 2a/2b should be further clarified. |
| ETRI | From our perspective, the options in above (i.e., Option 1, 2a/b) seem based on the separated CSI prediction and compression operation. For example, CSI prediction ouput in Option 1 cannot be defined within the joint CSI prediction and compression operation. In this regards, we suggest to discuss whether to support the joint operation or not, for the progress. |
| NTT DOOCMO | Option 2a is the only approach that monitors the E2E performance. |

##### Discussion 24b: Monitoring label options for Case 3

For temporal domain aspects Case 3, regarding the monitoring labels Option 1, Option 2a, and Option 2b, the following observations are made:

| - Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s). - CSI prediction output is used as input to CSI generation part. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. - Option 2: The monitoring label is derived based on the measured CSI of the future slot(s) - Option 2a: CSI prediction output is used as input to CSI generation part. - Note: This corresponds to end-to-end monitoring of CSI prediction and compression. - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. |  |  |  |
| --- | --- | --- | --- |
|  | Option 1 | Option 2a | Option 2b |
| Label availability | Label is not available at UE if UE implements a single AI/ML model for joint prediction and compression. | Label can be obtained at UE by measurement. | Label can be obtained at UE by measurement and performing inference. |
| Complexity | No additional inference is needed for monitoring label generation. | No additional inference is needed for monitoring label generation. | Dedicated inference is required for monitoring, which adds complexity. |
| Latency | Lowest latency among the three options since the predicted future CSI can be reported to the NW immediately with the CSI feedback. | Monitoring metric cannot be calculated until the measured future CSI is obtained by the UE. | Monitoring metric cannot be calculated until the measured future CSI is obtained by the UE. |
| Consistency between monitoring and inference | Consistent | Consistent | Inconsistent |

| Company | Comments |
| --- | --- |
| Mod | Option 1 is more popular, but as ETRI noted, Option 1 is not applicable if UE implements joint prediction and compression. Please provide any other pros/cons not listed in the above table. |
| ETRI | To our understanding, Option 2a also falls under the separate CSI prediction and compression operation case, as the CSI prediction output serves as an input to the CSI generation part. Moreover, Option 2b seems also with the separate operation case since its objective is to monitor the performance of CSI compression only. Note that in the joint CSI prediction and compression operation case, monitoring of CSI compression performance only is not available. |

### Model structure for Case 2 and Case 3

In RAN1 #119, the group made the following agreement for studying the standardized model structure. For Case 2 and Case 3, various options of extending Case 0 toward Case 2 and 3 have been agreed.

Agreement For studying the standardized model structure, - For temporal domain Case 0, - In case of spatial-frequency domain input, adopt Transformer as the backbone structure. - Further study angular-frequency and angular-delay domain input - For temporal domain Case 2, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation with additional layers - Conv-LSTM - LSTM - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and changing quantization operation and/or feedback payload over different slots - Note: other options are not precluded - Further study angular-frequency and angular-delay domain input - For temporal domain Case 3, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation - Option 2: Latent adaptation - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input. - Take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure. - Note: Processing steps at the UE to derive angular, delay, and/or Doppler domain basis, and corresponding reconstruction steps at the gNB are considered. - Note: scalar/vector quantization and dequantization are considered. Quantization-aware training with jointly updated quantization method/parameters (Case 2-2) is used.

The following proposal is to study their performance, complexity, and associated specification impact.

##### Proposal 25a: Case 2 and Case 3 model structure and spec impact

- Study the performance, complexity, and specification impact of the various model structure Options of Case 2 that were agreed in RAN1 #119, with potential down-selection.

- Study the performance, complexity, and specification impact of the various model structure Options of Case 3 that were agreed in RAN1 #119, with potential down-selection.

| Company | Comments |
| --- | --- |
| vivo | For model structure study of case 2 and case 3, we prefer to have a slow start for now. Current focus for model structure study should mainly be on case 0 for RAN1. Case 2 and case 3 related work can be conducted more efficiently with some progress on the model structure for case 0 or even in the WI phase based on case 0 model structure. |
| Futurewei | We share similar view as vivo to focus on Case 0 first and potential model structures for Case 2 and Case 3 can follow/extend from Case 0. |
| Xiaomi | We should firstly study the model structure of Case 0 and align the Hyperparameter. |
| Samsung | Support vivo’s comment. |
| ZTE | Agree with vivo. Considering the limited time in Rel-19, we need to focus on the model structure study of Case 0 in prior. If good progress has been achieved for Case 0, we can further study the model structure of Case 2 and Case 3 based on the conclusion of Case 0. |
| CATT | Agree with vivo, prefer to focus first on Case 0 |
| Fujitsu | Similar view as vivo. |
| OPPO | Support vivo’s comment. |
| Tejas | Support vivo’s comment |
| NTT DOCOMO | We share the same view with vivo. The model structure for Case 0 is the fundamental of Case 2 and Case 3. However, we should consider the compatibility issue when studying the model structure for Case 0. |
| Mod | Majority view is to focus on Case 0 for model structure study for now. |

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |

# Localized models

## Summary of company proposals

Nvidia

Proposal 1: Site-specific AI/ML models for CSI compression should be considered to improve performance gain.

## Discussion

<empty>

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |

# Inter-vendor training collaboration

## Summary of company proposals

### Inter-vendor collaboration proposals

Futurewei

Proposal 1: To better understand [issue 9] for Direction C regarding the performance impact due to mismatch between the distribution of dataset used for reference model(s) training and data samples used for inference at UE-side, companies are encouraged to share field sample dataset(s) for performance evaluation purpose.

Ericsson

Proposal 1:	RAN1 conclude that the reference model(s) (inter vendor collaboration option 1) and/or reference model structure(s) (inter vendor collaboration option 3a and 3b), if supported, is/are designed and specified using 3GPP channel model based synthetic data.

Proposal 2	Conclude that, if Direction C is supported for inter-vendor training collaboration, the fully specified reference model for Direction C is the same as the specified model for RAN4 performance testing.

Proposal 4	For Direction B, to mitigate inter-vendor collaboration complexity and improve feasibility for UE to use received parameters directly for inference, at least the model parameter quantization and input data pre-processing must be standardized together with the CSI generation model structure. RAN1 should conclude on the required additional information that needs to be standardized to enable option 3b.

Proposal 5	For Direction B, study and conclude on whether it can bring additional performance gain compared with Direction C (inter vendor collaboration option 1), assuming a common encoder across UEs with standardized model structure, standardized model parameter quantization and standardized input data pre-processing.

Proposal 6	For inter-vendor training collaboration, UE-side first training cases for Direction A, where the exchange of parameter/dataset/model originating from the UE-side and ending at the NW-side is not supported as it necessitates multiple models to operate in parallel at the NW-side.

Proposal 7	For option 3a-1, conclude that in addition to CSI generation model parameters, the following additional information are necessary to be shared from the NW-side to the UE-side:

•	{Target CSI} as training/testing dataset

•	Performance requirements (i.e., target values for model performance metric)

•	Pairing information (i.e., information associated with the exchanged model parameters)

Proposal 8	For option 4-1, conclude that in addition to dataset {target CSI, CSI feedback}, the following additional information are necessary to be shared from the NW-side to the UE-side:

•	Performance requirements (i.e., target values for model performance metric)

•	Pairing information (i.e., the information associated with the exchanged dataset)

Proposal 9	For Direction B (inter vendor collaboration option 3b) and inter vendor collaboration option 3a in Direction A, the following additional issues shall be studied:

•	[Issue 11] Whether to consider 3GPP’s statistical channel model or field data for the design of reference model structure(s). In case of the latter, how does RAN1 collect field data and agree on them?

•	[Issue 12] Is there performance impact due to mismatch between the distribution of the dataset used for designing the reference model structure(s), UE-side data distribution, and NW-side data distribution, and if so, how to address it?

Proposal 10	To solve the data distribution mismatch issue between the NW-side dataset and UE-side dataset, for direction A/B/C, standardize the phase normalization of a precoding vector used as input to an encoder for the CSI compression using two-sided model use case.

Proposal 11	To solve the data distribution mismatch issue between the NW-side dataset and UE-side dataset,

•	For Direction C, consider data augmentation by applying different phase normalizations on the 3GPP channel model based synthetic dataset when training the reference model.

•	For Direction A/B, NW-side firstly uses mixed dataset collected from different UE types to train/update its CSI compression model, and then share the corresponding dataset/parameters/model to the UE-side.

Proposal 12	For Direction A, prioritize the RAN1 study on option 4-1.

Proposal 13	For inter vendor collaboration option 3a in Direction A, Direction B (inter vendor collaboration option 3b) and Direction C (inter vendor collaboration option 1), the following additional issue shall be studied:

•	[Issue 13] How to represent a standardized reference model (structure + parameters) and/or a standardized reference model structure in 3GPP specifications?

ZTE

Proposal 4:  For option 3a-1 of Direction A, at least performance target should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing.

Proposal 5:  For option 3a-1 of Direction A, additional dataset for UE-side encoder retraining can be collected by UE implementations, while the sharing of target CSI from NW-side to UE-side is not essential.

Proposal 6:  For option 4-1 of Direction A, at least performance target and model backbone information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing.

Proposal 7:  For the determination of additional information sharing for options 3a-1 and 4-1, the feasibility of addressing UE-side/NW-side data distribution mismatch with respect to NW-side additional conditions should be concluded.

Proposal 8:  For option 4-1 of Direction A, the model backbone or structure information can only be shared to the opposite node if the proprietary information of the NW side can be maintained.

Proposal 9:  For data distribution mismatch with respect to NW-side additional conditions for Direction A, it can be resolved by the indication of associated ID which implicitly abstracts the NW-side additional conditions.

Proposal 10:  For data distribution mismatch with respect to UE-side additional conditions for Direction A, it can be resolved by NW side timely data collection and subsequent update of delivered dataset/parameter to the UE side.

Proposal 15:  For Direction B, the size of the encoder is much smaller than that of the training dataset and the transferring of parameters is not expected to be occurred frequently within a cell, making option 3b an attraction option in comparison to the dataset transfer related options from overhead consumption perspective.

Proposal 16:  For Direction B, it may be infeasible for the NW side to train multiple encoders tailored to different UEs due to the potential UE proprietary information disclosure risks and offline collaboration efforts.

Proposal 17:  For Direction B, the implementation of universal encoder across various UEs may or may not sacrifice the overall performance due to the lack of UE-specific encoder design and optimization.

Proposal 18:  For Direction B, data categorization using associated IDs and continuous monitoring can be considered for addressing the data distribution mismatch issues.

Proposal 19:  Direction B may not carry risks of disclosing proprietary information if the model parameters to be transferred are widely recognized and devoid of device-specific design decisions.

Proposal 20:  For Direction C, synthetic data generated under 3GPP’s statistical channel model can be a starting point for reference model training.

Proposal 21:  For Direction C, whether there is any performance impact arising from the data distribution mismatch depends on the generalization capability of the standardized reference model and thus further evaluations are needed.

Proposal 22:  For Direction C, model retraining based on data collected in real world can be performed to bridge the gap between synthetic data and field data.

Proposal 23:  Issues 8 and 9 are also applicable to Directions A and B due to the standardization requirement of reference model structure.

Proposal 24:  The performance impact due to NW/UE data distribution mismatch with respect to UE side additional condition (i.e., SVD phase normalization) can be addressed by timely NW-side data collection from existing UEs.

Huawei

Proposal 1: For additional information of Direction A (Option 3a-1/4-1), study the performance target including at least the following aspects:

	End-to-end performance metric (e.g., Alt.1 of Option 3a-1/4-1), encoder only performance metric (e.g., Alt.2 of Option 3a-1/4-1), and/or decoder only performance metric (e.g., Alt.1 of Option 4-1).

	The type of performance metric.

	Testing dataset used for metric calculation to test the performance target.

	Note: the performance target is only applicable for the testing dataset shared from NW.

Proposal 2: For additional information of Direction A Option 4-1, model backbone information is not needed since the UE side can autonomously select appropriate model structure based on the guidance of performance target information indicated by NW side.

Proposal 3: For additional information of Direction A Option 3a-1, the dataset of at least Target CSI should be also shared from NW side to UE side to ensure:

	Aligned dataset for training NW part Decoder and UE part Encoder to avoid generalization problem.

	Aligned dataset for metric calculation to test the performance target at UE side.

	Note: it would be more complex from both spec perspective and UE side implementation perspective to align the dataset construction information if assuming UE side to autonomously collect training and/or testing dataset.

Proposal 4: Regarding the data distribution mismatch issue of Direction A/B, the necessity to resolve it may need careful justification, considering:

	The mismatch issue may not exist considering NW side collect sufficiently diverse training dataset which could be generalized to the new UE side data.

	The mismatch issue (if exists) can be alleviated to a large extent with timely dataset update by NW, where the NW side would perform NW side data collection from the new UEs and update the delivered dataset/model/parameters to represent the new UE side data.

Proposal 5: For the dataset sharing for Option 3a-1/4-1 over air-interface, study the solution to relieve the overhead. E.g.,

	NW splits the overall dataset into many subsets each with a limited number of data samples (e.g., with an overhead affordable by the RRC signaling). The subsets can be separately sent to different UEs, and all subsets are associated with a common dataset ID for the UE side re-combination.

Proposal 7: For studying the standardized dataset format of inter-vendor collaboration Option 4, consider at least the following aspects:

	Data sample format related aspects, e.g., type of ground-truth CSI, the dimension of input/output, CSI feedback related information.

	Dataset construction related aspects, e.g., number of data samples, dataset ID, dataset split/segmentation, association between ground-truth CSI and CSI feedback subject to a data sample, observation window/prediction window information for temporal domain cases.

	Scalability information.

	Target performance information.

Nokia

Proposal 3: Further study the impact of phase normalization on dataset mismatch and mitigation methods.

Proposal 4: While the UE-side can supplement the target CSI in Option 3a-1, target CSI should also be provided by the NW-side.

Proposal 5: Deprioritize Direction B due to issues with matching the downloaded model to various UE implementations and concern over disclosure of proprietary information.

Proposal 6: Consider Direction C as lower priority than Direction A, in case its main benefit of low or no level of required inter-vendor training collaboration effort cannot be achieved due to potential performance degradation and its corresponding remedial measures.

Samsung

Proposal#13: For Issue 9 of Direction C, RAN1 to conclude that data distribution mismatch among datasets for reference model training (Dataset#C), UE-side training dataset (Dataset#B-UE) and NW-side training dataset (Dataset#B-NW)

-	causes minor performance degradation for model input in spatial-frequency (W) domain, when Dataset#B-UE and Dataset#B-NW are from the same field (deployment scenario).

-	causes considerable performance degradation for model input in spatial-frequency (W) domain, when Dataset#B-UE and Dataset#B-NW are not from the same field (deployment scenario).

-	Has negligible impact for model input in angular-delay (W2) domain.

FFS: method to align UE-side training dataset (Dataset#B-UE) and NW-side training dataset (Dataset#B-NW) in terms of deployment scenario.

Proposal#14: For Direction C, consider associated ID to align the distribution of UE-side training dataset (Dataset#B-UE) and NW-side training dataset (Dataset#B-NW) with respect to NW-side additional conditions, e.g., deployment scenario and/or NW-part of two-sided model.

-	The associated ID is indicated with configuration for data collection (target CSI) for training and inference (pairing).

Proposal#15: For Issue 10 of Direction C, RAN1 to conclude data distribution mismatch as a result of UE antenna spacing does not impact performance.

-	UE side may train/update its UE-part of two-sided model independently.

Proposal#16: For Direction A, consider associated ID to align the distribution of UE-side training dataset (Dataset#A-UE) and NW-side training dataset (Dataset#B-NW) with respect to NW-side additional conditions, e.g., deployment scenario and/or NW-part of two-sided model.

-	For Option 4-1, the associated ID is indicated with configuration for data collection for training for UE’s measurement/reporting of target CSI and/or dataset transfer {target CSI, CSI feedback} and/or and inference (pairing).

-	For Option 3a-1, the associated ID is indicated with configuration for data collection for training (target CSI measurement/reporting and/or reference model transfer and/or and inference (pairing).

Note: The network’s indication of the associated ID for training data collection, reference model/dataset transfer, and inference ensures consistency on UE’s assumption on NW-side additional conditions, e.g., NW-part of two-sided model, across training and inference.

Proposal#17: For Issue 6 of Direction A, RAN1 to conclude data distribution mismatch as a result of UE antenna spacing does not impact performance.

-	UE side may train/update its UE-part of two-sided model independently.

Proposal#18: For Option 4-1 of Direction A, consider NW-side sharing the encoder backbone assumption associated with the dataset as additional information.

Vivo

Observation 16:	It is not necessary for NW to further exchange target CSIs in option 3a-1 to achieve a good performance.

Apple

Proposal 1: Additional information for 4-1 to assist UE to train nominal decoder including:

⁃	Performance target/loss function for nominal decoder

⁃	Assisted information for UE to categorize dataset for UE side training

⁃	Reference decoder model backbone and structure.

CATT

Proposal 7: Regarding Issue 1, performance target can be considered as additional information shared from NW-side to UE-side.

Proposal 8: For CSI compression, support at least one of the following options:

	Option 1: Standardize quantization method for CSI feedback, e.g., scalar quantization.

	Option 2: Quantization related information is adopted as additional information for direction A.

Lenovo

Proposal 6: 	 If a NW (UE) does not intend to implement directly the fully specified decoder (encoder) model, further study how it can train/fine-tune its own version of decoder (encoder) model based on the fully specified decoder (encoder) model without causing mismatch between the encoder/decoder sides.

Proposal 7: 	In direction C, further evaluate the feasibility and required procedures including the workload to agree and specify multiple fully specified models.

Proposal 8: 	Due to performance limitation and also the required high specification effort, we suggest deprioritizing Direction C for inter-vendor training collaboration.

Proposal 9: 	For option 4-1 and 3a-1 of Direction A, prioritize schemes based on first construction of the “local” decoder and then training of the encode model.

Proposal 10: 	Confirm that the {target CSI} in option 3a-1 should be the {target CSI} from the samples available at the NW-side.

Proposal 11: 	Confirm that the trained “local” decoder model can be considered as a good representation (the proxy) of the actual NW-side decoder model.

Proposal 12: 	Despite potentially lower complexity, direct use of received parameters (Direction B) (instead of Direction A) may result in UE encoder with not acceptable performance due to mismatch between NW side data distribution and UE side inference data distribution. Further study is needed in this regard.

Proposal 13: 	Until further investigation, give higher priority to options based on Direction A over options based on Direction B.

OPPO

Proposal 1: Regarding the relationship between Direction C and RAN4, the performance of reference model in Direction C is much higher than RAN4 test case. Hence RAN1 cannot directly use the model structure and parameters agreed in RAN4 into Direction C.

Proposal 2: Regarding [issue 8] in Direction C, RAN1 can refer to RAN4’s discussion on non-specified public dataset:

•	3GPP’s statistical channel model can be a starting point

Proposal 3: Regarding [issue 9] in Direction C, UE-side/NW-side fine-tuning can be conducted to alleviate the dataset mismatch issue.

Proposal 4: Regarding [issue 10] in Direction C, additional information at least includes:

•	From UE to NW: target label

•	From NW to UE: output of actual CSI reconstruction part and performance target

Proposal 5: Deprioritize Direction C (Option 1) in Rel-19.

Proposal 7: Regarding [issue 1] for Direction A (option 3a-1), additional information can include:

•	Performance target

•	Dataset and dataset related information

Other necessary additional information is not precluded

Proposal 8: Target CSI sharing from NW-side to UE-side is optional for Direction A (option 3a-1) is UE/UE-side is capable of UE-side data collection.

Proposal 9: Regarding [issue 3] for Direction A and Direction B, overhead concern is more critical for over-the-air exchange compared to offline exchange, mainly from two aspects:

•	Parameters exchange: can be alleviated by proper model compression methods

	E.g., knowledge distillation, parameter quantization, pruning

•	Dataset exchange: can be alleviated by target CSI label quantization methods

	E.g., float16, eType II-like codebook

Proposal 10: Regarding [issue 4] for Direction A, [issue 6] for Direction B, performance impacts due to data distribution mismatch can be addressed by:

•	UE reports UE-side data to NW, and NW performs encoder fine-tuning for sub-option 3a-1 in Direction A and Direction B (option 3b)

Proposal 11: Regarding [issue 5] for Direction B, the feasibility should be considered from following two aspects:

•	Performance aspect: feasible to train multiple encoders for different UEs

•	UE capability aspects: feasible if supporting standardizing multiple encoder structures

Proposal 12: Regarding [Issue 1] for Direction A (Option 4-1), additional information can include:

•	NW-side decoder backbone information

•	Performance target

•	Dataset related information

Other necessary additional information is not precluded

Proposal 13: Regarding [Issue 3] for Direction A (Option 4-1), overhead can be alleviated by target CSI label quantization methods, e.g., float16, eType II-like codebook.

Proposal 14: Regarding [issue 4] for Direction A (Option 4-1), performance impacts due to data distribution mismatch can be addressed by:

•	UE reports UE-side data to NW, and NW performs inference, and then exchange the CSI feedback information corresponding to UE-side data to UE for sub-option 4-1

Proposal 15: Regarding different options to resolve the inter-vendor collaborations and processing into normative work

•	Option 4-1 without model structure standardization should be treated in first priority.

•	Option 3a-1, Direction B and Direction C with model structure (and parameters) standardization should be treated in second priority

Xiaomi

Proposal 2: At least the following additional information is shared to enable UE-side encoder training, validation, and testing for Direction A:

-	Associated ID

-	Performance target

Proposal 3: For Option 4-1, Rel-16 eType II codebook or high-resolution eType II codebook could be considered to transfer target CSI for addressing the overhead issue.

Proposal 4: Sharing NW side training dataset to UE is not needed because mismatch data between UE side and NW side does not bring obvious performance loss for Option 3a-1.

Proposal 5：For saving overhead of transferring model parameters, the parameters of partial neural network layers of encoder are transferred from NW-side to UE-side, while the parameters of remained neural network layers are standardized.

Proposal 6：Suggest the collected dataset based on 3GPP’s statistical channel model for reference model training.

Proposal 7: At least the following additional information is shared to enable UE-side encoder training, validation, and testing at UE side for Direction C:

-	Associated ID

-	Performance target

Proposal 8: At least the following format/type could be studied as a starting point:

−	Precoding matrix in the spatial-frequency domain for target CSI

−	Float 32 for CSI feedback and parameter

Proposal 10: In order to make the two-sided model be compatible, the pairing information for model pairing is assigned by gNB with parameter/dataset exchange for Option 3a-1/4-1.

Proposal 11: The supported inter-vendor collaboration options are indicated via UE capability reporting if multiple inter-vendor collaboration options are supported.

Proposal 12: Quantization approach of CSI feedback needs to be specified for Option 3a-1/3b/4-1.

MTK

Proposal 1.	For direction A, prioritize method 3a-1 over 3a-2, i.e., option 3a-1 without Target CSI

Proposal 2.	For Direction B, discuss how to consider UE’s computational constraints when encoder is provided by NW.

Proposal 3.	For Direction B, discuss whether re-engineering on reference AI/ML models are allowed right before their deployment, and if yes, what are the acceptable re-engineering efforts and constraints to do that.

Proposal 4.	For direction B, limit the updates occurrence to the following occasions:

	UE-side detection

o	UE-side’s additional condition changes

o	Abnormal performance detected using UE-side nominal encoder.

o	Significant changes in statistics and/or semantics of measured CSI

	NW-side decision

o	NW-side decides to change encoder/decoder based on abnormal performance, additional condition changes, Output CSI features, etc.

DCM

Proposal 1

	Conclude in RAN1 that the adaptation for a specified model (Option 1) or model structure (Option 3a-1) can be solved by specifying,

	Pre-/Post-processing schemes.

	Adaptation layer structures (for Option 1 or 3a-1) and parameters (for Option 1).

	For Case 2 and Case 3, support the input/output adaption for the adaption to the temporal domain with the following,

	Input the CSI of multiple time instances to the TF and extend the self-attention of the TF to multiple time instances.

Proposal 2

-	Concluded in RAN1 that Directions A and C are needed for the normative work.

	Direction C is used for the following objectives,

	Provide a practical deployment option with minimized complexity.

	Provide a baseline performance of AI/ML with simplified methods (e.g., model/layer switching) to adapt to the field data.

	Alleviate the issues on the overhead/complexity/proprietary of additional information of Direction A (Option 3a-1/4-1).

	Direction A is used to better adapt to the field data for further improvements.

LGE

Proposal #9: On inter-vendor training collaboration via dataset exchange (i.e., Option 4-1), consider assistant information regarding relationship between target CSI and reconstructed target CSI to reduce/alleviate signaling overhead and model alignment issue.

ETRI

Proposal 5: Regarding the direction A and B, to consider the following aspects:

	Support both Direction A and Direction B

	Regarding Option 3a-1, sharing NW-side target CSI from NW to UE for UE-side offline engineering may not always be necessary

	Training the actual encoder may not necessarily require prior development of the nominal decoder, at least for Option 3a-1

	Among options in direction A, Option 3a-1 is preferred over Option 4-1 due to its lower overhead.

Proposal 6: Regarding the direction C, to consider the following aspects:

	Only UE-side offline engineering to the specified decoder (i.e., Option 1-2) is feasible without intervendor training collaboration.

Proposal 7: For AI/ML-based CSI compression using two-sided model, study functionality/model identification procedure for supporting both Direction A and B.

Proposal 8: For AI/ML based CSI compression using two-sided model, model identification process is necessary for inference and LCM procedures, both for Direction A and B.

Spreadtrum

Proposal 1: For additional information of Direction A (Options 3a-1 and 4-1), the performance target including performance metric:

	At least SGCS should be considered as the performance metric.

	Whether the performance metric are applied to the pair of encoder and decoder or only to encoder will depend on how the UE trained its encoder part.

Proposal 2: For the overhead concern of Direction A, it can be considered to alleviate it by transferring a small amount of data each time.

Proposal 3: If the NW-side data does not contain the UE-side data, performance may deteriorate. UE can report UE-side data to alleviate performance degradation caused by data mismatch

Proposal 4: Direction B does have overhead concern and can't be addressed.

Proposal 5: For direction A, 3GPP’s statistical channel model should be used for reference model(s) training.

CMCC

Proposal 1: For Option 3a-1 and Option 4-1 of Direction A, additional information might be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing, the following could be considered:

	Dataset or information related to collecting dataset

	Performance target

Proposal 3: If the target CSI sharing in Option 3a-1 is justified beneficial for the match between the encoder at UE side and the decoder at NW side, the target CSI sharing for Option 3a-1 can be considered.

Proposal 4: If the target CSI sharing for Option 3a-1 is supported, it should be performed via over the air signaling to alleviate the inter-vendor collaboration complexity.

Proposal 5: Some necessary model related information, such as model backbone, can be aligned between NW side and UE side to achieve better performance for Option 4-1.

Proposal 6: In Direction A, Option 3a-1 can be prioritized.

Proposal 7: For Direction C, 3GPP’s statistical channel model can be a starting point for reference model training, adding field data additionally may be beneficial so that the reference model can match real-world environment better.

Proposal 8: For Direction C, if NW-side and/or UE-side will train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder/decoder, additional information should be shared to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing; if the reference encoder/decoder are actually deployed at UE-side/ NW-side, additional information is not needed to be specified.

Proposal 9: For Direction C, if additional information is needed to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing, it includes:

	Dataset or information related to collecting dataset

	Performance target

Tejas Networks

Proposal 6: For inter-vendor collaboration, for direction A, prioritise Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback).

Proposal 7: Consider the following to overcome data distribution mismatch:

•	Option1: NW should timely updates the datasets with samples collected from diverse UE conditions

•	Option2 : Establish a data set distribution drift detection mechanism at the NW to identify when updates are necessary.

•	Option 3: Develop a standardized phase normalization method for CSI input to reduce the dataset distribution mismatch due to UE side additional conditions.

Proposal 8: For inter-vendor collaboration, for Direction A, consider the NW providing target performance SGCS as additional information to the UE.

Proposal 9: For inter-vendor collaboration, in direction B, use a common encoder across all UEs and standardize model parameters and pre-processing.

Proposal 10: For inter-vendor collaboration, consider standardizing a model structure with configurable hyperparameters to adapt effectively to different deployment scenarios.

Proposal 11: For inter-vendor collaboration, for Direction C, consider standardizing both the encoder and decoder (1-3).

Proposal 12: For Standardizing Scalable Model Structure, Consider Training and Specifying Parameters for One or a Few Combinations.

Proposal 13: For a scalable model structure, consider using transfer learning by training a base model on one or more parameter combinations. Add small adaptation layers to the pre-trained model to efficiently handle specific configurations.

Google

Proposal 13: Defer further discussion on 4-1 and 3a-1 for Direction A until receiving RAN2’s response

Proposal 14: For direction B, it should not require a common encoder across UEs.

Proposal 15: For direction C, Option 2 (Latent adaptation) is preferred compared to Option 1 (Input/output adaptation with additional layers)

TCL

Proposal 7: Option 1 and 2 should not be standardized, at least they are out of the scope of R19.

Proposal 8: RAN 1 should down select among option 3, 4 and 5 considering if unified model format or structure is shared between the NW and UE side model respectively.

•	For option 4, there may be no need for offline-engineering.

•	Option 3 and 5 are preferred if model structure or format is exchanged from NW to UE.

NEC

Proposal 1: For Direction A, the following additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing:

-	Performance requirements, e.g., type of performance metric, minimum performance target or range of performance target.

-	Dataset (at least for model testing).

-	Pairing information, e.g., associated/dataset ID (for option 3a-1), model ID (for option 4-1).

Proposal 2: For Direction A, if option 4-1 or option 3a-1 w/ target CSI sharing form NW to UE is supported, the following method can be used to address the NW/UE data distribution mismatch with respect to UE-side additional condition:

-	UE-side encoder training in Direction A, e.g., training nominal decoder at UE-side using NW-side target CSI or {target CSI, CSI feedback} shared by the NW-side, and subsequently training UE-side encoder with UE-side data.

Proposal 3: For Direction A, further study the following options:

-	Option 4-1.

-	Option 3a-1 w/ target CSI sharing form NW to UE.

Proposal 4: For Direction A, option 3a-1 w/ target CSI sharing form NW to UE, support NW-side to adopt the training dataset mix to ensure the generalization ability of the trained encoder.

Proposal 5: RAN1 to prioritize studying Direction A and C over Direction B.

Nvidia

Proposal 3: RAN1 to conclude that it is feasible to resolve issues related to inter-vendor training collaboration for AI/ML-based CSI compression.

Panasonic

Proposal 1: {Target CSI} in Option 3a-1 should be target CSI exchanged from the NW-side.

Proposal 2: Direction C can be used to define some of “minimum performance” in RAN4. Further extension on top of Direction C (e.g., the combination with Direction A) can be considered in RAN1. What aspect of inter-vendor collaboration is managed by what WGs should be discussed.

Proposal 3: For the model structure and parameter specification in Direction C, train and specify model/parameters for one or a few combinations.

Proposal 4: RAN1 consider the combination of Direction A and Direction C in Rel.19.

Fujistu

Proposal 1:

	To address the performance degradation due to the misalignment of SVD phase normalization, two alternatives could be considered in RAN1:

o	Alt.1: specify phase normalization for the input of reference encoder.

o	Alt.2: introduce a phase correction layer for the input of reference encoder.

Proposal 2:

	For the data distribution mismatch due to UE side additional condition, it is not needed to consider antenna configuration.

Proposal 3:

	For data distribution mismatch resulting from UE local data in Direction A, if non-negligible performance loss (i.e., local gain) is identified, study the method to reduce this performance loss and the following two schemes could be considered in RAN1:

o	Generalized encoder + retrained/redeveloped/finetuned (cell/site specific) decoder

o	Retrained/redeveloped/finetuned (UE specific) encoder + generalized decoder

Proposal 4:

	For Option 3a-1 in Direction A, the exchange of ground-truth CSI should be considered by RAN1.

Proposal 5:

	For the retrained/redeveloped encoder in Direction A, to make NW vendor aware of the change of encoder mapping, test dataset and the corresponding performance target could be shared from NW side to UE side. And the following scheme could be considered in RAN1:

o	When test dataset and the corresponding performance target are not transferred, NW vendor allows UE vendor to change the encoder mapping and NW vendor will not change the decoder mapping given by the reference decoder at NW side.

o	When test target and the corresponding performance target are transferred, the retrained/redeveloped encoder should satisfy the performance target on test dataset and NW vendor may or may not change the decoder mapping given by the reference decoder at NW side.

Proposal 6:

- To enable UE-sided offline engineering and alleviate inter-vendor training collaboration, the following information could be considered to share from NW side to UE side:

  - Model input/output related information

    - Input related information, such as pre-processing (normalization/phase correction/domain-transform/scalability)

    - Output related information, such as post-processing (domain-transform/scalability)

  - Quantization related information

    - Quantization type

    - Quantization granularity (e.g., vector length, quantization bit)

    - Quantization table for trainable vector quantization if applied

  - Dataset related information

    - Scenario/configuration information

    - Cell/site information

Proposal 7:

- To enable UE-sided offline engineering, the minimum performance requirement could be shared to UE side for retrained/redeveloped AI/ML models.

  - For sub-option 3a-1/4-1, NMSE and cross entropy loss could be considered as the performance metric.

Proposal 8:

- Considering there is no performance advantage without decoder related information and more specification effort is required for option 3a-1, RAN1 to consider option 4-1 with high priority.

Proposal 9:

- For data distribution mismatch resulting from UE local data in Direction B, if non-negligible performance loss is identified, RAN1 to study the method to reduce this performance loss and the following two schemes could be considered:

  - Generalized encoder + retrained/redeveloped/finetuned (cell/site specific) decoder

  - Retrained/redeveloped/finetuned (UE specific) encoder + generalized decoder

Proposal 10:

- For Direction C, if synthetic data is used to train reference model, RAN1 to consider the standardized decoder with high priority.

Proposal 11:

- For Direction C, RAN1 to consider training reference models with synthetic data.

Proposal 12:

- For Direction C, RAN1 should specify the reference model to achieve the optimal performance in field and not reuse the reference model (at least model parameters) agreed in RAN4.

- Whether the model structure agreed in RAN4 is reused by RAN1 could be further discussed.

Proposal 13:

- For the actual encoder in Direction C, to make NW vendor aware of the change of encoder mapping at UE side, test dataset and the corresponding performance target could be shared from NW side to UE side. And the following scheme could be considered in RAN1:

  - When the test dataset and the corresponding performance target are not transferred, NW vendor allows UE vendor to change the encoder mapping.

  - When the test dataset and the corresponding performance target are transferred, the actual encoder should satisfy the performance target on the test dataset.

CEWiT

Proposal-1: For evaluation of AI/ML based CSI compression for Option 3a-1, assume Alt.2 to be the starting point.

Proposal-2: {Target-CSI} in Option 3a-1, is necessary and should be target CSI exchanged from NW side.

Proposal-3: Consider Option 4-1, as it requires less specification effort compared to Option 3a-1.

Proposal-4: For option 4-1, consider sending dataset related information like (Dataset ID) and NW sided additional condition for ensuring proper inference.

Proposal-5: In case of Option 4-1, consider offline exchange of dataset from NW side to UE side to avoid overhead concerns.

IIT Kanpur

Proposal 1: For option 4-1, consider including performance target metric along with dataset ID from NW side to serve as the decision criterion to fall back to legacy solutions or terminate training early.

Proposal 2: For Option 3a-1, the performance target metric could be shared along with the Target CSI to serve as a decision criterion. This would enable the system to fall back to legacy solutions or terminate training early if the required performance is not met.

Qualcomm Incorporated

Proposal 1: Deprioritize direction B (option 3b) for addressing inter-vendor collaboration complexity in Rel-19 two-sided CSF study.

Proposal 2: Consider Direction C and Direction A as the way forward to address inter-vendor collaboration complexity.

Proposal 3: RAN1 should focus on the following discussion in remaining meetings of R19

•	Further discussion and potential down-selection on sub-options of Direction A considering performance and specification efforts

•	Study the model structure feasibility and scalability.

o	It can be used to prove the feasibility of sub-option 3a-1, leveraged by RAN4 to standardize a model for various testing cases, or considered as additional information as sub-option 4-1.

Proposal 4: RAN1 should send LS to SA2 and SA5 to study other signalling approaches than using UE as relay (a.k.a., over-the-air approach).

Proposal 5: Lack of agreed solution in R19 should not be the gating factor blocking the normative work because (1) solutions can be discussed/developed during the normative work, and (2) proprietary signalling can be assumed as default option in the absence of standardized signalling.

Proposal 6: Conclude that issue 4 of Direction A, i.e., the performance degradation due to NW / UE data distribution mismatch can be resolved by UE side with proper offline-engineering of training nominal decoder using exchanged dataset or target CSI and training UE side encoder using UE’s own data.

Proposal 7: It is not feasible to specify SVD phase alignment algorithm in RAN1.

Proposal 8: For inter-vendor collaboration sub-option 3a-1, conclude that NW side target CSI sharing to the UE side is a necessary additional information.

Proposal 9: Prioritize option 4-1 over 3a-1 with target CSI sharing from NW side unless the necessity of standardizing encoder parameter sharing and model structure is justified for option 3a-1 with target CSI sharing.

Proposal 10: Specification of models or synthetic data as additional information in Direction C is not sufficient, indication of trained model to the other side to facilitate offline engineering is needed.

### Observations on NW / UE data mismatch

Futurewei

Observation 1: For inter-vendor training collaboration Option 3a-1 in Direction A for CSI compression using two-sided model, when evaluating performance impact due to UE-side / NW-side data distribution mismatch, small SGCS performance degradation is observed at least for training approach Alt. 2 compared to the benchmark in which the actual encoder-decoder pair was trained on dataset B.

- Alt. 2 has 1.62% SGCS performance degradation compared to the benchmark (Case 1)

Observation 2: For Case 3/Direction B in inter-vendor training collaboration for CSI compression using two-sided model, when evaluating performance impact due to UE-side / NW-side data distribution mismatch, small SGCS performance degradation of 1.24% is observed compared to the benchmark/Case 1.

Observation 3: For inter-vendor training collaboration Option 4-1 in Direction A for CSI compression using two-sided model, when training the actual encoder at UE-side according to Alt. 2 using the received dataset with different encoder structure from the encoder structure at NW-side that generated the dataset ({Target CSI, CSI feedback}), SGCS performance is worse than Option 3a-1 Alt. 2 in which the encoder structure is assumed to be based on the reference encoder structure specified.

Observation 4: For inter-vendor training collaboration Option 4-1 in Direction A for CSI compression using two-sided model, when evaluating performance impact due to UE-side / NW-side data distribution mismatch, some SGCS performance degradation is observed at least for training approach Alt. 2 compared to the benchmark in which the actual encoder-decoder pair was trained on dataset B.

- Alt. 2 has 3.42% SGCS performance degradation compared to the benchmark (Case 1)

Observation 5: Considering issue 4 and 6 in AI/ML-based CSI compression using two-sided model regarding performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional condition, there may be no need to consider antenna configuration difference.

Observation 6: For performance degradation due to UE-side / NW-side data distribution mismatch caused by internal algorithm at UE-side, using mixed dataset in training may not alleviate the performance degradation issue.

Ericsson

Observation 9	A CSI compression model trained on one phase normalization can suffer large performance loss if inference is using a different phase normalization. AI performance can be significantly worse than baseline.

Observation 10	A CSI compression model trained on mixed dataset of different phase normalizations can achieve good generalization performance, if inference is using a phase normalization that has been seen during the model training phase. AI performance is better than baseline.

Observation 11	A CSI compression model trained on mixed dataset of different phase normalizations can achieve good generalization performance, if training is done on a diverse dataset, also for inference using a phase normalization that has not been seen during the model training phase. However, with some deterioration compared to the case where it has been seen in the training phase. AI performance is better than baseline, or at least on par with baseline.

Observation 12	Option 4-1 (Alt 1) has better performance than Option 3a-1 (Alt 1). Moreover, combined with that the NW-side has a mixed initial dataset, however not containing the UE-side dataset, Option 4-1 (Alt 1) is an inter-vendor collaboration option that is able to provide gains over baseline.

ZTE

Observation 8: For Direction A/B, compared with the case that the UE-side trains an encoder based on Dataset B, there is almost no performance improvement if the UE side trains the encoder based on both Dataset A (e.g., transferred from the NW side) and Dataset B.

Observation 9: For Direction A/B, if the NW side trains the encoder-decoder pair based on dataset mixed from Dataset A and Dataset B, the performance loss caused by data distribution mismatch is greatly alleviated.

Nokia

Observation 3: For the dataset distribution mismatch conditions studied, only phase normalization significantly affected model performance.

Observation 4: Combined dataset training mitigates the performance loss due to dataset distribution mismatch between the UE and gNB.

Samsung

Observation#16: For Option 4-1 of Direction A, performance degradation is observed when the NW-side encoder backbone associated with the dataset is different from the UE-side encoder backbone.

Apple

Observation 2: option 3a-1 and option 4-1 with Alt 2 where UE-side directly develops and trains the actual encoder, cannot handle UE side additional condition.

Observation 3: option 3a-1 and option 4-1 with Alt 1 where UE-side train a nominal decoder first, can handle UE side additional condition, when additional assisted information is transmitted.

Observation 4: Option 3a-1 with target CSI has very similar training steps as option 4-1, however, incur much higher specification impact including standardize model structure and model delivery, on top of dataset delivery.

CATT

Observation 6: Consider dataset distribution mismatch between NW-side and UE-side induced by different SVD phase normalization methods as UE-side additional conditions, regarding Issue 1, for Option 3a-1, additional information of dataset is not needed.

Observation 8: Regarding Issue 4, for Option 3a-1, considering dataset B as target CSI, there is limited performance impact due to mismatch between NW-side and UE-side data distribution induced by different SVD phase normalization methods as UE-side additional conditions.

Observation 9: Regarding Issue 4, for Option 4-1, there is limited performance impact due to mismatch between NW-side and UE-side data distribution induced by different SVD phase normalization methods as UE-side additional conditions.

Observation 18: Regarding issue 4 for Option 3a-1 in direction A, considering distribution mismatch between NW-side data distribution and UE-side inference data distribution caused by different SVD phase normalization methods as UE-side additional conditions, limited performance degeneration (~ -3%) is observed compared to Case 1 with medium payload.

Observation 19: Regarding issue 4 for Option 3a-1 in direction A, considering distribution mismatch between NW-side data distribution and UE-side inference data distribution caused by different SVD phase normalization methods as UE-side additional conditions, the benefit for UE-side encoder retraining based on dataset A is limited. There is potential risk of non-convergence for encoder model when mixing dataset A and dataset B for encoder retraining.

Observation 20: Consider distribution mismatch between NW-side data distribution and UE-side inference data distribution caused by different SVD phase normalization methods as UE-side additional conditions, in Direction A, Option 3a-1 slightly outperforms Option 4-1.

Observation 21: Regarding Issue6, for Option 4-1 in direction A, considering distribution mismatch between NW-side data distribution and UE-side inference data distribution caused by different SVD phase normalization methods as UE-side additional conditions, limited performance degeneration (~ -3%) is observed compared to Case 1 with medium payload.

OPPO

Observation 4: Regarding the performance of Direction A (option 3a-1) and Direction B, UE-side/NW-side data distribution mismatch due to different SVD phase normalizations may cause large performance loss.

Observation 5: The performance loss caused by UE-side/NW-side data distribution mismatch can be alleviated, if NW can timely collect data from UE-side, and update the NW-side encoder and decoder.

Observation 7: The performance of option 4-1 can be ensured with aligned backbone between NW-side decoder and UE-side encoder.

Xiaomi

Observation 8: For Option 3a-1, the training dataset mismatch between UE side and NW side does not bring obvious performance loss.

Observation 9: For Option 4-1, model performance could be further improved if UE measurement dataset is used to train model.

InterDigital

Observation 25:  For Option 3a-1, it is feasible to perform UE-side off-line engineering to develop a UE-side encoder (including a lower complexity encoder) that is compatible with a proprietary NW-side actual decoder. There is a small performance impact (up to 0.6 dB NMSE degradation, and 3.3% SGCS degradation) relative to the reference encoder-actual decoder pair developed at the NW-side.

Observation 26:  For Option 3a-1, the additional information that needs to be exchanged from the NW-side to the UE-side to enable UE-side off-line engineering of an actual encoder includes: information on the exchanged dataset A (containing the target CSI), model backbone information (e.g., transformers, CNN, etc.), as well as minimum KPI (SGCS, NMSE).

Observation 27:  For Issue 4, for the considered configuration, there is a small performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional conditions, specifically of up to 2.95% SGCS degradation and up to 0.54 dB NMSE degradation.

ETRI

Observation 1: 0.5% SGCS loss is observed in inference on UE-side data when the encoder is trained using NW-side data (i.e., Case 3) compared to the upper bound performance (i.e., Case 1). This reflects that the performance of Direction B is almost similar to the upper bound.

Observation 2: Under Case 2, the performance of Options 3a-1 and 4-1 was found to be comparable.

Observation 3: Under Case 2 and Option 3a-1, the performance of Alt. 1a (using NW-side data for UE-side offline engineering) was observed to be similar to the performance of Alt. 1b and Alt. 2 (which do not utilize NW-side data for UE-side offline engineering).

Observation 4: Under Case 2 and Option 3a-1, the performance of both Alt. 1a and Alt. 1b, and Alt. 2 were observed to be similar, indicating that training the actual encoder does not necessarily be trained on the nominal decoder.

Observation 5: Under Case 2 and Option 4-1, the performance of Alt. 1b was observed as 2.53% higher than Alt. 2.

Observation 6: The performance of UE-side offline engineering (Direction A) is comparable to direct inference (Direction B), regardless of the availability of NW-side data of Direction A.

Panasonic

Observation 1: Option 4-1 may be “feasible, but less reliable” without the assumption that the model structure is aligned based on offline inter-vendor collaboration.

Observation 2: For Direction A, at least the following information should be provided to address Issue 1.

	Training dataset or information related to collecting training dataset

	Performance target

	Testing dataset or information related to collecting testing dataset

Observation 3: For Direction A, Issue 3 can be addressed with the combination of Direction C.

Observation 4: For Direction A, Issue 4 can be addressed with the mechanism for identifying the cause of the performance degradation.

Observation 5: Direction A Option 3a-1 can achieve almost the same performance as joint training if data distribution between NW-side offline training and UE-side offline training is aligned.

Observation 6: For Direction A Option 3a-1, in terms of UE-side additional condition of antenna layout/configuration, the performance loss is small if Dataset A for NW-side decoder and nominal encoder training includes Dataset B.

Fujistu

Observation 3:

	For UE-side / NW-side data distribution mismatch with respect to UE side additional condition, there is no significant performance degradation when UE-sided antenna layout and spacing is considered.

IIT Kanpur

Observation 1: SGCS performance degradation of -2.04% and -3.06% is observed for Alt. 1 and Alt. 2 of Option 3a-1, respectively. While Alt. 2 is relatively easier to train compared to Alt. 1, it experiences a slight performance degradation.

Observation 2: Option 4-1 achieves SGCS performance comparable to the benchmark while providing an NMSE gain of 2.65%. Although Option 4-1 facilitates inter-vendor collaboration, it introduces training delays. To address this, NW could establish a performance target for the UE encoder (E2) to exceed. Given that retraining at the UE incurs delays, this target could serve as a decision criterion to either fall back to legacy solutions or alternatives or terminate training early to save time if the UE encoder does not meet the required performance.

Observation 3: The UE may connect to different NW gNBs at various times, resulting in datasets shared from multiple gNBs. These datasets could be utilized to train the CSI generation/reconstruction components with varying capabilities at the NW side, including differences in the backbone or structure. To accommodate this, the UE must first identify the different gNBs. To facilitate this identification, a dataset ID specific to each gNB could be shared with the UE alongside the dataset.

Observation 4: For different datasets, the UE could utilize the corresponding specific performance target to determine whether the performance metric is met or if retraining with the newer dataset is required. Additionally, the performance metric could be used to evaluate the UE-side model on the newer dataset.

Observation 5: For Option 3a-1, a performance metric could be shared for Alt. 1/Alt. 2 to ensure that the trained nominal decoder/actual encoder meets the expected performance. If the metric is not met, the training could be halted, and the system could fall back to legacy solutions.

Qualcomm Incorporated

Observation 5: On-device operation without UE side offline engineering (option 3b/5b) suffers from performance loss if NW side data distribution is mismatched with UE side data distribution.

Observation 6: UE side offline engineering based on NW side dataset sharing (4-1) and target CSI in addition with NW side encoder (Option 3a-1 with NW-side target CSI) resolves the performance degradation due to UE / NW side data distribution mismatch.

Observation 7: UE side offline engineering by developing encoder directly (Alt2) is worse than developing encoder after developing a nominal decoder (Alt1).

Observation 9: UE side offline engineering based on sharing CSI generation part only (Option 3a-1 without NW-side target CSI) is worse than sharing NW-side target CSI in addition (Option 3a-1 with NW-side target CSI). This is because the latter option allows the UE-side to update encoder mapping according to the mismatch between NW and UE side data.

### Observations on field / synthetic data mismatch

Futurewei

Observation 7: For performance impact evaluation due to mismatch between the distribution of datasets used for reference model(s) training and dataset used for inference, there is no significant SGCS performance impact when using different shared datasets from RAN4 WG in model training and model inference.

Ericsson

Observation 2	For Direction C (inter vendor collaboration option 1), the evaluation results provided by different companies indicate that

a.	Directly use the fully specified reference model(s) in Direction C may have similar performance in the field compared to legacy Rel. 16 eType II codebook, but limited performance in the field compared to Direction A.

b.	The performance of fully specified reference models in Direction C can be further improved if one side retrains/finetunes its part of model using the field data with the other side keeping its model fixed.

i.	Retaining/finetuning of decoder at NW side with fixed refence encoder at UE side may provide better performance than retaining/finetuning of encoder at UE side with fixed decoder at NW side.

c.	Independent retraining/finetuning of models at both NW and UE sides without inter-vendor collaboration may or may not result in performance degradation, compared to directly use the fully specified model(s).

ZTE

Observation 10: For Direction C, the data distribution mismatch between synthetic and field data causes limited performance impact, i.e., -3.31% SGCS loss for layer 1 and -4.08% SGCS loss for layer 2.

Observation 11: For Direction C, data distribution mismatch between NW-side field data and UE-side field data with respect to UE-side additional condition (i.e., phase normalization) causes minor SGCS performance loss.

Observation 12: For Direction C, if the NW-side decoder is trained by mixed dataset from NW-side field data and UE-side field data, the performance loss caused by data distribution mismatch is greatly alleviated.

Samsung

Observation#11: For Case-2A of Direction C, when UE and NW independently train/update the UE-part and NW-part of their actual models to be compatible with fully standardized reference two-sided models with training dataset (Dataset#B) collected from the same field, i.e., UE-side training dataset and NW-side training dataset are from the same distribution (deployment scenario):

•	When the model input is precoding vectors in the spatial-frequency (W) domain, 11.18% SGCS degradation is observed as compared to baseline models trained on Dataset#B.

•	When the model input is precoding vectors in the angle-frequency (W2) domain, a minor 1.51% SGCS gain is observed as compared to baseline model trained on Dataset#B.

Note: The training datasets Dataset#S for reference model and Dataset#B for proprietary models are generated with different deployment scenarios (UMa 80% indoor and InH, respectively).

Observation#12: For Case-2A of Direction C, when the UE side and NW side independently train/update the UE-part and NW-part of their actual models to be compatible with fully standardized reference two-sided model with training dataset (Dataset#B-UE and Dataset#B-NW) collected from the different fields i.e., UE-side training dataset and NW-side training dataset are from different distributions (deployment scenarios):

•	When the model input is precoding vectors in the spatial-frequency (W) domain, up to 30.5% SGCS degradation is observed as compared to baseline models trained on Dataset#B-UE or Dataset#B-NW.

•	When the model input is precoding vectors in the angle-frequency (W2) domain, a minor 2.9% SGCS degradation is observed as compared to baseline models trained on Dataset#B-UE or Dataset#B-NW.

Note: The training datasets Dataset#S for reference model and Dataset#B-UE and Dataset#B-NW for actual models are generated with different deployment scenarios.

Observation#13: It is feasible to separately train/fine-tune UE-part and NW-part of actual models to be compatible with fully standardized reference two-sided model with UE-side and network-side training/finetuning datasets, respectively, at least for the case wherein the model input is precoding vectors in the angular-delay (W2) domain.

Observation#14:  For Case-2A of Direction C, when the UE side and NW side independently train/update the UE-part and NW-part of their actual models to be compatible with fully standardized reference two-sided model with training dataset (Dataset#B-UE and Dataset#B-NW) collected from the different fields i.e., training dataset at UE and NW are from different distributions (deployment scenarios), when Dataset#B-UE and Dataset#B-NW include the dataset for reference model training

•	When the model input is precoding vectors in the spatial-frequency (W) domain, up to 3.6% SGCS degradation is observed as compared to baseline models trained on UE/NW-side training datasets.

Note: The training datasets Dataset#S for reference model and Dataset#B-UE and Dataset#B-NW for actual models are generated with different deployment scenarios (UMa 80% indoor, mix of UMa 80% indoor  InH,  mix of UMa 80% indoor  and InH, respectively).

Observation#15: For Case-2A of Direction C, when the UE side and NW side independently train/update the UE-part and NW-part of their proprietary models to be compatible with fully standardized reference two-sided model with training respective datasets, Dataset#B-UE and Dataset#B-NW with different UE antenna spacing assumptions:

•	When the model input is precoding vectors in the spatial-frequency (W) domain, a minor 0.21% SGCS performance gain is observed as compared to baseline model trained on Dataset#B-UE.

•	When the model input is precoding vectors in the angle-frequency (W2) domain, a minor 0.89% SGCS degradation is observed as compared to baseline models trained on Dataset#B-UE.

Note: The training datasets Dataset#S for reference model and Dataset#B-UE and Dataset#B-NW for proprietary models are generated assuming UE side antenna spacing set as  (0.5, 0.5) λ , (0.5, 0.8) λ and (0.5, 0.5) λ, respectively.

CATT

Observation 14: The reference model in direction C is different from RAN4 specified model.

Observation 16: For direction C, the performance is expected to be impacted due to mismatch among the distribution of the dataset used for reference model(s) training, UE-side data, and NW-side data at least in some regions/scenarios.

Lenovo

Observation 10: 	In direction C, the UE-side and/or the NW-side may use their current field data to fine-tune the respective encoder and decoder models.

Observation 11: 	In direction C, if the NW (UE) directly implement the fully specified decoder (encoder) model as its decoder (encoder) model, the fine-tuning of the other side (i.e., encoder (decoder)) can potentially improve the performance of the two-sided model in field data.

Observation 12: 	In direction C, if both sides train/fine-tune their encoder/decoder model based on their own dataset (instead of direct implementation of the fully specified encoder/decoder), the final two-sided model could potentially experience performance loss. This is due to the mismatch between the datasets that both sides are using for training/fine-tuning.

ETRI

Observation 7: It was observed that, during inference on UE-side data (representing field data), the SGCS loss increased by 37.2% when the encoder/decoder pair was trained using NW-side data (representing synthetic data) in Case 2, compared to the upper bound in Case 1. This reflects the performance of Direction C, where the degradation is significantly more severe than in Directions A and B.

Observation 8: Adjusting the decoder model on the NW side may not be practical, as the NW may be unable or unwilling to manage multiple decoder models customized for each UE, UE vendor, or UE type.

Observation 9: Adjusting both the encoder on the UE side and the decoder model on the NW side may not be practical under Direction C, as the encoder-decoder pair cannot achieve proper alignment.

Observation 10: Offline engineering on the UE side improves inference performance by 21.4%, but the performance remains lower compared to Directions A and B.

Panasonic

Observation 14: If both NW and UE sides train their model separately without inter-vendor collaboration, the performance may be worse than training one side (e.g., NW-side) using Dataset B.

Observation 15: For Direction C, in terms of UE-side additional condition of antenna layout/configuration, the performance loss is small if Dataset A for NW-side decoder and nominal encoder training includes Dataset B.

Fujistu

Observation 15:

- For direction C, it could be observed that:

  - The performance degradation due to data distribution mismatch between synthetic data and field data could be mitigated by retraining the encoder or decoder with field data.

  - The performance of retraining UE-sided encoder is better than that of retraining NW-sided decoder.

  - Significant performance degradation could be observed if UE-sided encoder and NW-sided decoder are independently retrained/redeveloped without inter-vendor collaboration.

Qualcomm Incorporated

Observation 12: Fully standardized reference model(s) may give acceptable performance, as long as either NW-side and UE-side, or both sides, further use field data to further train their encoder/decoder.

Observation 13: Independent training at both sides against specified model without coordination may be worse than training at one side using field data. Therefore, if NW-side trains a decoder using field data against the specified encoder, it is desirable for the NW-side to communicate the decoder information (via Option 4, or Option 3a-1 with NW-side target CSI) to the UE-side.

Observation 14: Mixing synthetic (RAN4) dataset with field dataset during one-sided finetuning in Direction C gives decent RAN4 performance while barely sacrificing the field performance that is achieved by training using field dataset.

Observation 15: Various trade-offs between RAN4 performance and field performance depending on what (encoder, decoder, or both) is trained / finetuned and the mixture ratio of the synthetic and field data

### Signalling for parameter / dataset delivery

Futurewei

Proposal 5: If inter-vendor training collaboration option 3b in Direction B is supported, consider adopting over-the-air delivering method(s).

Proposal 6: If inter-vendor training collaboration option 3a-1 and/or 4-1 in Direction A are/is supported, consider adopting standardized signalling in upper layers or other offline method(s) as delivery options.

Nokia

Proposal 1: For further study of Option 4-1, prioritize offline transfer of the dataset from the NW-side to the UE-side.

Proposal 2: For further study of Option 3a-1, prioritize offline transfer of the encoder and/or dataset from the NW-side to the UE-side.

Xiaomi

Proposal 9: The feasibility of exchanged parameter/dataset via over-the-air or other approaches depends on the progress of RAN2 or other WGs.

Qualcomm Incorporated

Proposal 13: Over-the-air signalling for sharing the dataset and parameters to support UE-side model training is not a suitable and scalable solution.

Proposal 14: For sharing the dataset and parameters from the NW side (e.g., 5GC or OAM) to the UE side entity, 3GPP should be able to develop a solution similar to the following. SA WGs can further study this and provide feedback to RAN WGs.

Proposal 15: RAN2 checks with SA2/SA5 to provide feedback.

## Discussion

| Direction A | views |
| --- | --- |
| Additional information | Performance target / requirement (ZTE, Huawei, Ericsson, CATT, Spreadtrum, Tejas, Networks, QC, OPPO, NEC, Fujistu) - Including E2E, decoder only, encoder only, type of perf metric, testing dataset (Huawei) - SGCS, whether applied to E2E or only encoder (spreadtrum) Pairing information: - information associated with exchanged dataset / parameter or dataset categorization: Ericsson, Apple, Fujistu - Associated ID, ZTE, Samsung Xiaomi, NEC - Pairing ID: QC - Model ID: NEC Model backbone information for 4-1: - Needed: ZTE, Samsung, Apple, OPPO, Futurewei, Panasonic - Not needed: Huawei, QC - If needed, also applicable to 3a-1: QC, InterDigital Quantization alignment (Xiaomi, QC, Fujistu) Model input/output related information (Fujistu) |
| Necessity of target CSI sharing for 3a-1 | Needed (11): Ericsson, Huawei (for robust performance, for testing/validation), Nokia, Lenovo, NEC (at least for testing), Panasonic, Fujistu, CEWiT, IIT Kanpur, QC, InterDigital Not needed (7): ZTE, vivo, CATT, Samsung, Xiaomi, OPPO, ETRI |
| Perf impact due to NW / UE data mismatch | SVD phase alignment standardization - Ericsson, Tejas Networks, Fujistu Can be addressed by timely NW-side data collection and sharing the updated dataset/parameters to UE-side - Huawei, ZTE, Ericsson, OPPO, Spreadtrum Can be addressed by UE-side offline engineering w/o necessarily updating NW-side dataset - QC, Apple, Lenovo No impact from UE side antenna configuration (Samsung, Fujistu) Study impact of phase normalization and mitigation methods (Nokia) |
| Other issues | UE-first training not supported (Ericsson) For 3a-1, Perf impact due to dataset mismatched used in model structure specification vs. field dataset (Ericsson) How to represent a model in 3gpp for all 3 directions. (Ericsson) Format of dataset sharing in option 4, e.g., type of ground-truth, dimension, number of samples, dataset ID, scalability, etc (Huawei) Defer 4-1 and 3a-1 discussion until RAN2 response. (Google) |

| Direction C | views |
| --- | --- |
| Use synthetic data for model specification | Ericsson, ZTE, OPPO, QC, Xiaomi, CMCC, Fujistu |
| RAN4 model or RAN1 model | RAN4 model trained using synthetic data (Ericsson, QC) RAN1 cannot directly use RAN4 model (OPPO) RAN1 model (Fujistu, Samsung) |
| Perf impact due to synthetic vs. field data | Can be improved with offline engineering (Ericsson, ZTE, OPPO, QC) Details captured in the other table |
| Perf impact due to UE / NW data mismatch | Consider phase augmentation by applying different phase normalizations (Ericsson) Addressed by NW timely data collection (ZTE) If mismatched by deployment scenario, considerable degradation if spatial-frequency domain; minor degradation if angle-delay domain (Samsung) If mismatched by UE antenna spacing, no degradation (Samsung) Study phase normalization impact and mitigation methods (Nokia, Fujistu) |
| Additional information | Performance target (QC, Xiaomi, CMCC, Fujistu) Dataset or information related to dataset (QC, CMCC) Testing dataset sharing (Huawei, Fujistu) Target label, output of decoder and performance target (OPPO) Associated ID (Xiaomi) |
| General preference | Support (7): Ericsson, Samsung, Panasonic (minimum performance), QC (minimum performance, RAN4 model), OPPO (2nd priority), DCM (minimum performance, alleviate inter-vendor collaboration overhead/complexity), NEC Deprioritized (3): Nokia (performance degradation cannot be improved without coordination), Lenovo (huge spec effort), OPPO |
| Other aspects | Using field data in evaluation (Futurewei) Align associated ID to align NW side additional condition (Samsung) Standardizing pre/post schemes, adaptation layers, extension to TSF (DCM) UE side offline-engineering to update encoder (ETRI) |

### Addressing performance mismatch due to NW / UE data mismatch

|  | Options | Observations |
| --- | --- | --- |
| Futurewei | 3a-1, Alt2 training, | Minor degradation 1.62% |
|  | Dir B, | Minor degradation, 1.62% |
|  | 4-1, Alt2 training | Worse than 3a-1 Alt2, if backbone mismatch, 3.42% loss |
|  | - | No need to consider antenna configuration impact Mixed dataset training may not alleviate issue due to UE internal algorithm |
| Ericsson | 4-1 vs. 3a-1, Alt1 training | 4-1 > 3a-1 if dataset does not contain UE data |
|  | - | Large loss if mismatched by phase alignment Trained by mixed phase alignment or diverse dataset alleviate the impact |
| ZTE | - | Almost no performance improvement of target CSI sharing Mixing dataset training at NW side can alleviate the impact due to mismatch |
| Nokia | - | Only phase normalization impacts the performance Combined dataset mitigate the loss |
| Samsung | - | Performance degradation seen if backbone mismatch |
| Apple | 3a-1, 4-1, Alt2 | Cannot handle the impact due to mismatch |
|  | 3a-1, 4-1, Alt1 | Can alleviate the impact due to mismatch |
| CATT | 3a-1 w/o target CSI, 4-1 | Target CSI not needed if mismatched by phase normalization method, limited performance difference due to NW / UE mismatch 3a-1 slightly outperforms 4-1 |
| OPPO | - | Performance loss is larger if mismatched phase normalization Alleviated by NW timely data collection 4-1 alleviated by backbone alignment |
| Xiaomi | - | 3a-1, no obvious performance loss 4-1, performance improved if further use UE data to train encoder |
| InterDigital | - | Small performance loss due to UE / NW mismatch |
| ETRI | Dir B | 0.5% performance loss |
|  | 3a-1, 4-1 | Comparable performance |
|  | 3a-1, Alt1 vs. Alt2 training | Using target CSI shared from NW side achieves similar performance to using UE side target CSI |
|  | 4-1, Alt1b, Alt2 | Alt1b training is higher than Alt2 by 2.53% |
| Panasonic | 3a-1 w/ target CSI | Almost alleviate the performance impact due to mismatch |
|  | - | Performance impact small if mismatched by antenna configuration |
| Fujistu | - | No significant degradation if mismatched by antenna configuration |
| IIT Kanpur | 3a-1 Alt1 vs. Alt2 | Alt1 is better than Al2, target CSI sharing is beneficial |
| Qualcomm | Dir B | Significant performance loss 20% if NW data not include UE data |
|  | 4-1 vs. 3a-1 Alt2 | 3a-1 w/o target CSI sharing is worse than 3a-1 w/ target CSI and 4-1 3a-1 w/ target CSI is similar to 4-1 |

Almost all companies see at least one of 1) minor performance impact due to NW / UE mismatch, 2) addressed by mixed training, 3) UE offline engineering w/ decoder trained by shared CSI and encoder training by UE data.

- Minor performance impact: Futurewei, CATT, Xiaomi, InterDigital, ETRI, Panasonic, Fujistu

- Mixed training / timely NW data collection: E///, ZTE, Nokia, OPPO

- UE offline engineering: Apple, QC, IIT Kanpur

- Additional benefit of target CSI for 3a-1

  - Yes: Ericsson, Apple, QC, Xiaomi (small), Panasonic, IIT Kanpur

  - No: ZTE, CATT, ETRI

- Degradation due to backbone mismatch: Samsung, Futurewei, OPPO

- No clear consensus on 3a-1 vs. 4-1

##### Proposal 401a: Observations from X7

Capture the following observations into TR:

For the evaluation of performance impact due to UE-side / NW-side data distribution mismatch with respect to UE-side additional condition (issue 4 and 6),

When dataset A include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 1 source [Panasonic] observes minor performance loss (-0.22% ~ -1.09%) relative to case 1 with NW side target CSI sharing.

When dataset A include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing,

- 1 source [ZTE] observes minor performance loss (-0.67% ~ -0.97%) related to case 1 without NW side target CSI sharing.

- 1 source [OPPO] observes moderate performance loss (-3.3% ~ 4%) relative to case 1 without NW side target CSI sharing

When dataset A include dataset B, for case 3 (Direction B),

- 1 source [ZTE] observes minor performance loss (-0.6% ~ -0.9%) relative to case 1.

When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training.

- 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training.

- 1 source [Ericsson] observe moderate performance loss of -8.3% relative to case 1 for Alt1 training.

- 1 source [OPPO] observe significant performance loss (-51% ~ -62.5%) relative to case 1 for Alt1 UE training.

- 2 sources [CATT, Futurewei] observe minor performance loss (-2.78% ~ -3.2%) relative to case 1 for Alt2 UE training

- 1 source [Apple] observe moderate loss of -6.7% relative to case 1 for Alt2 UE training.

When dataset A does not include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing

- 4 sources [CATT, ZTE, Xiaomi, ETRI] observe minor to moderate performance loss (-0.4% ~ -3.9%) relative case 1.

- 2 source [QC, Ericsson] observe moderate performance loss (-6.7% ~ -8.6%) relative to case 1.

- 1 source [OPPO] observes significant performance loss of -62.1% relative to case 1.

When dataset A does not include dataset B, for case 2, option 4-1,

- 4 sources [QC, Apple, Xiaomi, ETRI] observe zero to minor performance loss (-2.4% ~ 0%) relative to case 1 for Alt1 UE training.

- 1 source [Ericsson] observe minor to moderate performance loss (-2.9% ~ -7.9%) relative to case 1 for Alt1 UE training depending on whether dataset A applies augmentation using various phase normalization methods.

- 4 sources [CATT, Xiaomi, Futurewei, ETRI] observe minor performance loss (-1.41% ~ -3.52%) relative to case 1 for Alt2 UE training.

- 1 source [Apple] observes moderate loss of -7.9% relative to case 1 for Alt2 UE training.

When dataset A does not include dataset B, for case 3, Direction B,

- 4 sources [CATT, vivo, ZTE, ETRI] observe minor loss to positive gain (-3.7% ~ 1%) relative to case 1.

- 1 source [QC] observes significant loss (-20%) relative to case 1.

| Company | Comments |
| --- | --- |
| Mod | Regarding sub-option 4-1 with Alt2 training (direct encoder training), some companies [@Xiaomi in U31, @Futurewei in X31, @ETRI in Y31] indicate that dataset B was used in the training. Since the key of Option 4-1 is to share dataset A to the UE-side, it sounds strange not to use dataset A at all for the UE-side training. Please clarify whether/how dataset A was used in your sub-option 4-1 Alt2 training. @Xiaomi, @Futurewei, @ETRI: Please clarify the dataset used in sub-option 4-1 Alt2 training. @All, please comment if you have different understanding as below - For sub-option 3a-1, it is noted that either dataset A or B or mixture of them can be used in Alt1 or Alt2 training. In row 23, 24 and 26, X/Y samples are used in training up to companies’ choice. If X > 0, it will be considered as NW-side target CSI (dataset A) is shared and used in the training. - For sub-option 4-1 Alt1 training, since dataset A is shared by definition, it is expected that at least one row of 28 and 29 utilizes dataset A, e.g., X/Y samples with X > 0. - For sub-option 4-1 Alt2 training, since dataset A is shared by definition, it is expected that row 31 utilizes dataset A, e.g., X/Y samples with X > 0. |
| Futurewei | In Option 4-1, UE side uses the shared {Target CSI, CSI feedback} from dataset A to train its actual encoder. We corrected this typo in the template. For Option 3a-1, our results show that the performance gain is -1.62%. |
| Xiaomi | For Option 4-1, UE side uses the shared dataset A from NW side to train nominal decoder or nominal encoder. This typo in the template has been corrected and updated in red colour. |
| ZTE | It seems our simulation results are missing for Option 3a-1, and the following revision is suggested as: When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing - 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training. - 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training. - 12 sources [Ericsson, ZTE] observe moderate performance loss of -3.8~-8.3% relative to case 1 for Alt1 training. |
| ETRI | For the Option 4-1 Alt. 2 training, the UE-side directly trains CSI generation model using the shared dataset {target CSI, CSI feedback} shared from NW. Therefore, the entity 0/91 should be corrected to 91/0. We have corrected this in the excel sheet. Thank you for the idication and sorry for any confusion. |

##### Proposal 401b: Observations from X7

Capture the following observations into TR:

For the evaluation of performance impact due to UE-side / NW-side data distribution mismatch with respect to UE-side additional condition (issue 4 and 6),

When dataset A include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 1 source [Panasonic] observes minor performance loss (-0.22% ~ -1.09%) relative to case 1 with NW side target CSI sharing.

When dataset A include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing,

- 1 source [ZTE] observes minor performance loss (-0.67% ~ -0.97%) related to case 1 without NW side target CSI sharing.

- 1 source [OPPO] observes moderate performance loss (-3.3% ~ -4%) relative to case 1 without NW side target CSI sharing

When dataset A include dataset B, for case 3 (Direction B),

- 1 source [ZTE] observes minor performance loss (-0.6% ~ -0.9%) relative to case 1.

When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training.

- 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training.

- 1 source [Ericsson] observe moderate performance loss of -8.3% relative to case 1 for Alt1 training.

- 1 source [OPPO] observe significant performance loss (-51% ~ -62.5%) relative to case 1 for Alt1 UE training.

- 2 sources [CATT, Futurewei] observe minor performance loss (-2.78% ~ -3.2%) relative to case 1 for Alt2 UE training

- 1 source [Apple] observe moderate loss of -6.7% relative to case 1 for Alt2 UE training.

When dataset A does not include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing

- 4 sources [CATT, ZTE, Xiaomi, ETRI] observe minor to moderate performance loss (-0.4% ~ -3.9%) relative case 1.

- 2 source [QC, Ericsson] observe moderate performance loss (-6.7% ~ -8.6%) relative to case 1.

- 1 source [OPPO] observes significant performance loss of -62.1% relative to case 1.

When dataset A does not include dataset B, for case 2, option 4-1,

- 4 sources [QC, Apple, Xiaomi, ETRI] observe zero to minor performance loss (-2.4% ~ 0%) relative to case 1 for Alt1 UE training.

- 1 source [Ericsson] observe minor to moderate performance loss (-2.9% ~ -7.9%) relative to case 1 for Alt1 UE training depending on whether dataset A applies augmentation using various phase normalization methods.

- 4 sources [CATT, Xiaomi, Futurewei, ETRI] observe minor performance loss (-1.41% ~ -3.52%) relative to case 1 for Alt2 UE training.

- 1 source [Apple] observes moderate loss of -7.9% relative to case 1 for Alt2 UE training.

When dataset A does not include dataset B, for case 3, Direction B,

- 4 sources [CATT, vivo, ZTE, ETRI] observe minor loss to positive gain (-3.7% ~ 1%) relative to case 1.

- 1 source [QC] observes significant loss (-20%) relative to case 1.

| Company | Comments |
| --- | --- |
| ZTE | It seems our first-round comment is missing for Option 3a-1, and the following revision is suggested as: When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing - 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training. - 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training. - 12 sources [Ericsson, ZTE] observe moderate performance loss of -3.8~-8.3% relative to case 1 for Alt1 training. |

##### Proposal 401c: Observations from X7

Capture the following observations into TR:

For the evaluation of performance impact due to UE-side / NW-side data distribution mismatch with respect to UE-side additional condition (issue 4 and 6),

When dataset A include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 1 source [Panasonic] observes minor performance loss (-0.22% ~ -1.09%) relative to case 1 with NW side target CSI sharing.

When dataset A include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing,

- 1 source [ZTE] observes minor performance loss (-0.67% ~ -0.97%) related to case 1 without NW side target CSI sharing.

- 1 source [OPPO] observes moderate performance loss (-3.3% ~ -4%) relative to case 1 without NW side target CSI sharing

When dataset A include dataset B, for case 3 (Direction B),

- 1 source [ZTE] observes minor performance loss (-0.6% ~ -0.9%) relative to case 1.

When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training.

- 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training.

- 1 2 source [Ericsson, ZTE] observe moderate performance loss of (-3.8 ~ -8.3%) relative to case 1 for Alt1 training.

- 1 source [OPPO] observe significant performance loss (-51% ~ -62.5%) relative to case 1 for Alt1 UE training.

- 2 sources [CATT, Futurewei] observe minor performance loss (-2.78% ~ -3.2%) relative to case 1 for Alt2 UE training

- 1 source [Apple] observe moderate loss of -6.7% relative to case 1 for Alt2 UE training.

When dataset A does not include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing

- 4 sources [CATT, ZTE, Xiaomi, ETRI] observe minor to moderate performance loss (-0.4% ~ -3.9%) relative case 1.

- 2 source [QC, Ericsson] observe moderate performance loss (-6.7% ~ -8.6%) relative to case 1.

- 1 source [OPPO] observes significant performance loss of -62.1% relative to case 1.

When dataset A does not include dataset B, for case 2, option 4-1,

- 4 sources [QC, Apple, Xiaomi, ETRI] observe zero to minor performance loss (-2.4% ~ 0%) relative to case 1 for Alt1 UE training.

- 1 source [Ericsson] observe minor to moderate performance loss (-2.9% ~ -7.9%) relative to case 1 for Alt1 UE training depending on whether dataset A applies augmentation using various phase normalization methods.

- 4 sources [CATT, Xiaomi, Futurewei, ETRI] observe minor performance loss (-1.41% ~ -3.52%) relative to case 1 for Alt2 UE training.

- 1 source [Apple] observes moderate loss of -7.9% relative to case 1 for Alt2 UE training.

When dataset A does not include dataset B, for case 3, Direction B,

- 4 sources [CATT, vivo, ZTE, ETRI] observe minor loss to positive gain (-3.7% ~ 1%) relative to case 1.

- 1 source [QC] observes significant loss (-20%) relative to case 1.

| Company | Comments |
| --- | --- |
| vivo | Sorry to miss some results in x7, Based on the following table 1 and table 7 in R1-2409671 as below. We propose to update our results as following When dataset A include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing - 1 source [Panasonic] observes minor performance loss (-0.22% ~ -1.09%) relative to case 1 with NW side target CSI sharing. When dataset A include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing, - 2 source [ZTE,vivo] observes minor performance loss (-0.006% ~ -0.97%) related to case 1 without NW side target CSI sharing. - 1 source [OPPO] observes moderate performance loss (-3.3% ~ 4%) relative to case 1 without NW side target CSI sharing When dataset A include dataset B, for case 3 (Direction B), - 2 source [ZTE,vivo] observes minor performance loss (+0.01% ~ -0.9%) relative to case 1. When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing - 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training. - 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training. - 1 2 source [Ericsson, ZTE] observe moderate performance loss of (-3.8 ~ -8.3%) relative to case 1 for Alt1 training. - 1 source [OPPO] observe significant performance loss (-51% ~ -62.5%) relative to case 1 for Alt1 UE training. - 2 sources [CATT, Futurewei] observe minor performance loss (-2.78% ~ -3.2%) relative to case 1 for Alt2 UE training - 1 source [Apple] observe moderate loss of -6.7% relative to case 1 for Alt2 UE training. When dataset A does not include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing - 5 sources [CATT, ZTE, Xiaomi, ETRI, vivo] observe minor to moderate performance loss (-0.4% ~ -3.9%) relative case 1. - 2 source [QC, Ericsson] observe moderate performance loss (-6.7% ~ -8.6%) relative to case 1. - 1 source [OPPO] observes significant performance loss of -62.1% relative to case 1. When dataset A does not include dataset B, for case 2, option 4-1, - 4 sources [QC, Apple, Xiaomi, ETRI] observe zero to minor performance loss (-2.4% ~ 0%) relative to case 1 for Alt1 UE training. - 1 source [Ericsson] observe minor to moderate performance loss (-2.9% ~ -7.9%) relative to case 1 for Alt1 UE training depending on whether dataset A applies augmentation using various phase normalization methods. - 4 sources [CATT, Xiaomi, Futurewei, ETRI] observe minor performance loss (-1.41% ~ -3.52%) relative to case 1 for Alt2 UE training. - 1 source [Apple] observes moderate loss of -7.9% relative to case 1 for Alt2 UE training. - 1 source [vivo] observes moderate loss of -7.7% relative to case 1 for 3a-1 for Alt1 UE training when backbone are different When dataset A does not include dataset B, for case 3, Direction B, - 4 sources [CATT, vivo, ZTE, ETRI] observe minor loss to positive gain (-3.7% ~ 1%) relative to case 1. - 1 source [QC] observes significant loss (-20%) relative to case 1. - Initial results for option 3a-1 without target CSI from NW side Model with mixed ratio [0.5, 0.5]; Model with mixed ratio [0.33, 0.67]; Model with mixed ratio [0.17, 0.83]; Model with mixed ratio [0.09, 0.91]. NW side enc-dec performance on UE A data; 0.763; 0.762; 0.764; 0.762. UE side enc (alt. 1) + NW side dec performance on UE A data; 0.760; 0.758; 0.761; 0.761. UE side enc (alt. 2) + NW side dec performance on UE A data; 0.759; 0.757; 0.760; 0.758. Table 7 Experiments on the impact of different phase alignment methods for SVD(dataset A include o or not include dataset B). Model with mixed ratio Note [1.0, 0.0]; Model with mixed ratio [0.99, 0.01]; Model with mixed ratio [0.95, 0.05]; Model with mixed ratio [0.91, 0.09]; Model with mixed ratio [0.77, 0.23]; Model with mixed ratio [0.66, 0.33]; Model with mixed ratio [0.5, 0.5]; Model with mixed ratio [0.0, 1.0]. Test on eigenvectors with normalized phase on the first element; 0.771; 0.771; 0.771; 0.770; 0.770; 0.771; 0.769; 0.737. Test on eigenvectors with normalized phase on the last element; 0.730(without target CSI); 0.753; 0.764; 0.768; 0.772; 0.772; 0.771; 0.771. |
| Futurewei | Our results for 3a-1 Alt.2 with {Target CSI} sharing was not corrected in the observation yet. The changes are in red text. When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing - 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training. - 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training. - 1 2 source [Ericsson, ZTE] observe moderate performance loss of (-3.8 ~ -8.3%) relative to case 1 for Alt1 training. - 1 source [OPPO] observe significant performance loss (-51% ~ -62.5%) relative to case 1 for Alt1 UE training. - 2 sources [CATT, Futurewei] observe minor performance loss (-1.62%-2.78% ~ -3.2%) relative to case 1 for Alt2 UE training - 1 source [Apple] observe moderate loss of -6.7% relative to case 1 for Alt2 UE training. |
| Mod | Vivo and Futurewei corrections have been incorporated in the updated proposal |

##### Proposal 401d: Observations from X7

Capture the following observations into TR:

For the evaluation of performance impact due to UE-side / NW-side data distribution mismatch with respect to UE-side additional condition (issue 4 and 6),

When dataset A include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 1 source [Panasonic] observes minor performance loss (-0.22% ~ -1.09%) relative to case 1 with NW side target CSI sharing.

When dataset A include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing,

- 1 2 source [ZTE,vivo] observes minor performance loss (-0.67 -0.006% ~ -0.97%) related to case 1 without NW side target CSI sharing.

- 1 source [OPPO] observes moderate performance loss (-3.3% ~ -4%) relative to case 1 without NW side target CSI sharing

When dataset A include dataset B, for case 3 (Direction B),

- 1 2 source [ZTE,vivo] observes minor performance loss  (-0.6% ~ -0.9%  -0.9% ~ +0.01%) relative to case 1.

When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training.

- 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training.

- 1 2 source [Ericsson, ZTE] observe moderate performance loss of (-3.8 ~ -8.3%) relative to case 1 for Alt1 training.

- 1 source [OPPO] observe significant performance loss (-51% ~ -62.5%) relative to case 1 for Alt1 UE training.

- 2 sources [CATT, Futurewei] observe minor performance loss (-1.62% -2.78%  ~ -3.2%) relative to case 1 for Alt2 UE training

- 1 source [Apple] observe moderate loss of -6.7% relative to case 1 for Alt2 UE training.

When dataset A does not include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing

- 4 5 sources [CATT, ZTE, Xiaomi, ETRI, vivo] observe minor to moderate performance loss (-0.4% ~ -3.9%) relative case 1.

- 2 source [QC, Ericsson] observe moderate performance loss (-6.7% ~ -8.6%) relative to case 1.

- 1 source [OPPO] observes significant performance loss of -62.1% relative to case 1.

When dataset A does not include dataset B, for case 2, option 4-1,

- 4 sources [QC, Apple, Xiaomi, ETRI] observe zero to minor performance loss (-2.4% ~ 0%) relative to case 1 for Alt1 UE training.

- 1 source [Ericsson] observe minor to moderate performance loss (-2.9% ~ -7.9%) relative to case 1 for Alt1 UE training depending on whether dataset A applies augmentation using various phase normalization methods.

- 4 sources [CATT, Xiaomi, Futurewei, ETRI] observe minor performance loss (-1.41% ~ -3.52%) relative to case 1 for Alt2 UE training.

- 1 source [Apple] observes moderate loss of -7.9% relative to case 1 for Alt2 UE training.

- 1 source [vivo] observes moderate loss of -7.7% relative to case 1 for 3a-1 for Alt1 UE training when backbone are different.

When dataset A does not include dataset B, for case 3, Direction B,

- 4 sources [CATT, vivo, ZTE, ETRI] observe minor loss to positive gain (-3.7% ~ 1%) relative to case 1.

- 1 source [QC] observes significant loss (-20%) relative to case 1.

| Company | Comments |
| --- | --- |

##### Conclusion 402a: addressing NW / UE data mismatch (Continuation from 42b of RAN1 #119)

Conclude that, for direction A, performance impact, if any, due to NW / UE data distribution mismatch with respect to UE side additional condition can be addressed at least by

- Timely NW-side data collection from existing UEs and sharing the updated dataset and/or model parameters to UE-side, followed by UE-side offline engineering to train UE-side encoder.

- UE-side offline engineering to train UE-side encoder, without triggering a new dataset and/or model parameter sharing to UE-side.

| Company | Comments |
| --- | --- |
| Lenovo | Agree |
| Panasonic | We are fine with the conclusion. |
| TCL | Agree |
| ZTE | For the first bullet, we suggest to delete the word ‘updated’. If the original model is trained by sufficiently diverse dataset or mixed dataset from some typical UE-side additional conditions (e.g., phase normalization settings), it can generalize well to other phase normalization settings including the unseen ones. Then, whether to update the dataset and/or model parameters is optional. Not sure the intention of the second bullet. It seems imply that there is no or only minor performance impact due to NW/UE data distribution mismatch with respect to UE side additional condition, and thus no additional operation is needed. |
| LG | Fine |
| CATT | OK |
| Fujitsu | Generally fine with the proposal. |
| Ericsson | Fine with the first bullet. For the second bullet, more clarification is needed. Is this statement appliable to both option 4-1 and option 3a-1? |
| OPPO | Fine with the first bullet. For the second bullet, we don’t think it works from our evaluations. |
| NTT DOCOMO | Agree |
| CEWiT | Agree with the proposal. |
| Mod | @ZTE: The two bullets are approaches to handle NW / UE data distribution mismatch, if any. If the original model is trained by sufficiently diverse dataset and generalizes well, neither of the two bullets are necessary. @Ericsson: Yes, it’s applicable to either 4-1 or 3a-1 both. @OPPO: Most companies’ evaluations show that the second bullet works, so it’s fair to capture it as an approach. |

##### Conclusion

For direction A, performance impact, if any, due to NW / UE data distribution mismatch with respect to UE side additional condition can be addressed.

### Addressing performance mismatch due to field data and synthetic data for Direction C

|  | Setup | observations |
| --- | --- | --- |
| Futurewei | RAN4 datasets | No significant SGCS performance impacts, need to consider real field data |
| Ericsson | - | Synthetic model may have similar perf as eT2 on field data Fine-tuning at either side can further improve the performance Finetuning at NW side > finetuning at UE side Finetuning at two sides may or may not have degradation |
| ZTE | - | 3%~4% loss testing on field data Mixed dataset training containing UE side alleviate NW / UE mismatch |
| Samsung | Finetuning using field data NW and UE have same dataset | Spatial-frequency domain incurs 11.18% loss Angle-delay domain incurs minor 1.51% loss |
|  | Finetuning using field data NW and UE have different dataset | Up to 30.% performance loss if spatial-frequency domain Minor 2.9% loss if angle-delay domain |
|  | Finetuning using field data + synthetic data | Spatial frequency incurs minor 3.6% loss |
|  | Antenna configuration mismatch at UE and NW | Minor 0.21% loss for spatial frequency domain, minor 0.89% loss for angle-delay domain |
| CATT | - | Performance expected to be impacted by the mismatch |
| Lenovo | - | Finetuning can further improve the performance Independent training at two side could potentially incur loss |
| ETRI | No finetune | Synthetic model incurs significant 37.2% loss |
|  | UE side finetune | Improves by 21.4% |
| Fujistu | - | Finetune can improve the performance UE side encoder finetune is better than NW side decoder finetune Two-side independent finetune could be worse than one-side finetune |
| Qualcomm | Finetune using field data | Finetune at one side can improve the performance, but still worse than Dir A Finetune at two side can be worse |
|  | Finetune using field + synthetic data | Improve the performance on synthetic data without scarifying field data performance |

Minor to significant performance loss observed by companies if tested on field data directly

- Minor: Futurewei (on different synthetic dataset), Samsung (if in angle-delay domain), ZTE

- Same as eT2 (Ericsson)

- Moderate to significant loss: ETRI, QC, Samsung (if in spatial-frequency domain)

Finetune at one side using field data can improve the performance: Ericsson, Lenovo, ETRI, Samsung, Fujistu, QC

- Samsung: If spatial-frequency domain, need to mix field data with synthetic data

Finetune at two sides may or may not incur further loss:

- May or may not: Ericsson

- May: Lenovo, Fujistu, QC

It is noted that QC and vivo used actual field data for B, while other companies used synthetic data as a proxy for field data for B.

##### Proposal 403a: Observations from X8

Capture the following observations into TR:

For the evaluation of performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution (Issue 9),

For case 2 (model trained on dataset S but tested on dataset B),

- 2 sources [vivo, QC] observe significant performance loss (-7.2% ~ -17.4%) relative to case 1, where dataset B consists of actual field data.

- 1 source [ETRI] observes significant performance loss of -37.3% relative to case 1, where dataset S and B are different by TxRU mapping.

- 3 resources [ZTE, Panasonic, Ericsson] observe moderate performance loss (-2.12% ~ -4.75%) relative to case 1, where dataset S and B are generated from different scenarios, antenna layout or UE location. E.g., Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

For case 2A (model further finetuned on dataset B),

- If finetune at NW and fix encoder at UE

  - For using actual field data as dataset B

    - 2 sources [vivo, QC] observes minor to moderate loss (-3.5% ~ -7.66%) loss relative to case 1. The improvement compared to case 2 is moderate to significant (2.6% ~ 13.9%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - When dataset A includes or is the same as dataset B, 3 sources [ZTE, Panasonic, Ericsson] observe minor to moderate loss relative to case 1 (-0.97% ~ -4.67%). The improvement compared to case 2 is negative to minor (-1.01% ~ 1.49%). Dataset S and B are Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

- If finetune at UE and fix decoder at NW

  - For using actual field data as dataset B,

    - 2 sources [vivo, QC] observe moderate to significant loss (-6.8% ~ -12%) relative to case 1. The improvement compared to case 2 is minor moderate (0.4% ~ 7.1%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - 2 source [Panasonic, Ericsson] observe minor to moderate loss (-1.56% ~ -3.81%) relative to case 1. The improvement compared to case 2 is marginal (0.03% ~ 0.75%). Dataset S and B are different by antenna layout and indoor/outdoor ratio.

    - 1 source [ETRI] observes significant loss (-23.8%) relative to case 1. The improvement compared to case 2 is significant (13.5%). Dataset S and B are different by TxRU mapping

- If finetune at UE both NW sides

  - For using actual field data as dataset B

    - 2 sources [vivo, QC] observe moderate to significant loss (-7.3% ~ -56.1%) relative to case 1. The improvement compared to case 2 is negative to significant (-39.3% ~ 10.1%). The loss relative to finetune at one side is minor to significant (-1.7 ~ 48.4%).

  - For using synthetic data as dataset B

    - If the input is in spatial-frequency domain, and dataset A does not contain B, 2 sources [Panasonic, Ericsson] observe minor to moderate loss (-1.1% ~ -3.69%) relative case 1 depending on the modelling of synthetic data. The improvement compared to case 2 is minor (0.57% ~ 1.3%). Compared to finetune at one side, the performance improvement is similar to minor (-0.35% ~ 1%).

    - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-3.67% ~ -11.18%) relative to case 1 depending on whether synthetic data is used in the finetune.

    - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.89% ~1.51%).

| Company | Comments |
| --- | --- |
| Samsung | Thank you FL. This approach is fine. However, there are some details missing in the observation. For example, in Case 2A, vivo’s result shows higher losses, i.e., 56.1% when Dataset A and Dataset B are from different cells (possibly different deployment scenarios). Samsung has also shown similar degradation (up to 30.1%) in the contribution (not included in the excel file). On the other hand, when Dataset A and Dataset B follow the same distribution, e.g., from the same deployment scenario, the performance loss is much smaller. It provides some insight, if these cases are clearly indicated as there could be ways to align the distribution of Dataset A and Dataset B in the configurations for Dataset B collection by UE-side. Thus, we suggest to capture these results separately. |
| ZTE | It seems the wrong number is captured for ZTE, since only SGCS improvement is observed compared with case 2 based on the assumption of “When dataset A includes or is the same as dataset B”. In addition, the performance upper-bound should be “1.61%”, not “1.49%”. So, the editorial error should be fixed as follows: For case 2A (model further finetuned on dataset B), - If finetune at NW and fix encoder at UE - For using actual field data as dataset B - 2 sources [vivo, QC] observes minor to moderate loss (-3.5% ~ -7.66%) loss relative to case 1. The improvement compared to case 2 is moderate to significant (2.6% ~ 13.9%) depending on the specific scenario where the field data is collected. - For using synthetic data as dataset B - When dataset A includes or is the same as dataset B, 3 sources [ZTE, Panasonic, Ericsson] observe minor to moderate loss relative to case 1 (-0.97% ~ -4.67%). The improvement compared to case 2 is negative to minor (-1.01% 0.6% ~ 1.49%1.61%). Dataset S and B are Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio. |

##### Proposal 403b: Observations from X8

Capture the following observations into TR:

For the evaluation of performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution (Issue 9),

For case 2 (model trained on dataset S but tested on dataset B),

- 2 sources [vivo, QC] observe significant performance loss (-7.2% ~ -17.4%) relative to case 1, where dataset B consists of actual field data.

- 1 source [ETRI] observes significant performance loss of -37.3% relative to case 1, where dataset S and B are different by TxRU mapping.

- 3 resources [ZTE, Panasonic, Ericsson] observe moderate performance loss (-2.12% ~ -4.75%) relative to case 1, where dataset S and B are generated from different scenarios, antenna layout or UE location. E.g., Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

For case 2A (model further finetuned on dataset B),

- If finetune at NW and fix encoder at UE

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [vivo] observe moderate loss (-4.63% ~ -7.66%) relative to case 1. The improvement compared to case 2 is minor to moderate (2.58% ~ 9.17%).

    - When dataset A / B mismatch is not considered, 2 1 sources [vivo, QC] observes minor to moderate loss of (-3.5% ~ -7.66%) loss relative to case 1. The improvement compared to case 2 is moderate to significant (2.6% ~ 13.9%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [ZTE] observes moderate loss to case 1 (-4.32% ~ -4.75%). The improvement compared to case 2 is negative (-0.68% ~ -1.02%).

    - When dataset A includes or is the same as dataset B, 3 sources [ZTE, Panasonic, Ericsson] observe minor to moderate loss relative to case 1 (-0.97% ~ -4.67%). The improvement compared to case 2 is negative to minor (-0.61.01% ~ 1.6149%). Dataset S and B are Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

- If finetune at UE and fix decoder at NW

  - For using actual field data as dataset B,

    - 2 sources [vivo, QC] observe moderate to significant loss (-6.8% ~ -12%) relative to case 1. The improvement compared to case 2 is minor moderate (0.4% ~ 7.1%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - 2 source [Panasonic, Ericsson] observe minor to moderate loss (-1.56% ~ -3.81%) relative to case 1. The improvement compared to case 2 is marginal (0.03% ~ 0.75%). Dataset S and B are different by antenna layout and indoor/outdoor ratio.

    - 1 source [ETRI] observes significant loss (-23.8%) relative to case 1. The improvement compared to case 2 is significant (13.5%). Dataset S and B are different by TxRU mapping

- If finetune at both UE and both NW sides

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered and dataset A does not contain B, 1 source [vivo] observes significant loss (-8.5% ~ -44.85%) relative to case 1 depending on specific scenarios for data collection. The improvement compared to case 2 is negative (-28% ~ -1.3%). The loss relative to finetune at one side is negative (-1.72% ~ -37.19%).

    - When dataset A / B mismatch is not considered and A is equal to B, 1 2 sources [vivo, QC] observes moderate to significant loss of (-7.3% ~ -56.1%) relative to case 1. The improvement compared to case 2 is negative to significant (-39.3% ~ 10.1%). The loss relative to finetune at one side is minor to significant (-1.7 ~ 48.4%) negative to moderate (-3.82~ 4.67%).

  - For using synthetic data as dataset B

    - When dataset A / B mismatch is considered and dataset A does not contain B, 1 source [Samsung] observes similar performance (-0.89% ~ 0.21%) as case 1.

    - When dataset A includes B or equal to B,

      - If the input is in spatial-frequency domain, and dataset A does not contain B, 2 sources [Panasonic, Ericsson] observe minor to moderate loss (-1.1% ~ -3.69%) relative case 1 depending on the modelling of synthetic data. The improvement compared to case 2 is minor (0.57% ~ 1.3%). Compared to finetune at one side, the performance improvement is similar to minor (-0.35% ~ 1%).

      - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-3.67% ~ -11.18%) relative to case 1 depending on whether synthetic data is used in the finetune.

      - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.89% ~1.51%).

| Company | Comments |
| --- | --- |
| ZTE | Thanks for FL’s modification, but there’s still a typo for the second sub sub-bullet “When dataset A includes or is the same as dataset B”, where the exact value is “0.6%~1.61%”, not “-0.6%~1.61%”. So, we should remove the minus sign before 0.6. For case 2A (model further finetuned on dataset B), - If finetune at NW and fix encoder at UE - For using synthetic data as dataset B - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [ZTE] observes moderate loss to case 1 (-4.32% ~ -4.75%). The improvement compared to case 2 is negative (-0.68% ~ -1.02%). - When dataset A includes or is the same as dataset B, 3 sources [ZTE, Panasonic, Ericsson] observe minor to moderate loss relative to case 1 (-0.97% ~ -4.67%). The improvement compared to case 2 is negative to minor (0.6 1.01% ~ 1.6149%). Dataset S and B are Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio. |

##### Proposal 403c: Observations from X8

Capture the following observations into TR:

For the evaluation of performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution (Issue 9),

For case 2 (model trained on dataset S but tested on dataset B),

- 2 sources [vivo, QC] observe significant performance loss (-7.2% ~ -17.4%) relative to case 1, where dataset B consists of actual field data.

- 1 source [ETRI] observes significant performance loss of -37.3% relative to case 1, where dataset S and B are different by TxRU mapping.

- 3 resources [ZTE, Panasonic, Ericsson] observe moderate performance loss (-2.12% ~ -4.75%) relative to case 1, where dataset S and B are generated from different scenarios, antenna layout or UE location. E.g., Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

For case 2A (model further finetuned on dataset B),

- If finetune at NW and fix encoder at UE

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [vivo] observe moderate loss (-4.63% ~ -7.66%) relative to case 1. The improvement compared to case 2 is minor to moderate (2.58% ~ 9.17%).

    - When dataset A / B mismatch is not considered, 2 1 sources [vivo, QC] observes minor to moderate loss of (-3.5% ~ -7.66%) loss relative to case 1. The improvement compared to case 2 is moderate to significant (2.6% ~ 13.9%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [ZTE] observes moderate loss to case 1 (-4.32% ~ -4.75%). The improvement compared to case 2 is negative (-0.68% ~ -1.02%).

    - When dataset A includes or is the same as dataset B, 3 sources [ZTE, Panasonic, Ericsson] observe minor to moderate loss relative to case 1 (-0.97% ~ -4.67%). The improvement compared to case 2 is negative to minor (-0.61.01% ~ 1.6149%). Dataset S and B are Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

- If finetune at UE and fix decoder at NW

  - For using actual field data as dataset B,

    - 2 sources [vivo, QC] observe moderate to significant loss (-6.8% ~ -12%) relative to case 1. The improvement compared to case 2 is minor moderate (0.4% ~ 7.1%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - 2 source [Panasonic, Ericsson] observe minor to moderate loss (-1.56% ~ -3.81%) relative to case 1. The improvement compared to case 2 is marginal (0.03% ~ 0.75%). Dataset S and B are different by antenna layout and indoor/outdoor ratio.

    - 1 source [ETRI] observes significant loss (-23.8%) relative to case 1. The improvement compared to case 2 is significant (13.5%). Dataset S and B are different by TxRU mapping

- If finetune at both UE and both NW sides

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered and dataset A does not contain B, 1 source [vivo] observes significant loss (-8.5% ~ -44.85%) relative to case 1 depending on specific scenarios for data collection. The improvement compared to case 2 is negative (-28% ~ -1.3%). The loss relative to finetune at one side is negative (-1.72% ~ -37.19%).

    - When dataset A / B mismatch is not considered and A is equal to B, 1 2 sources [vivo, QC] observes moderate to significant loss of (-7.3% ~ -56.1%) relative to case 1. The improvement compared to case 2 is negative to significant (-39.3% ~ 10.1%). The loss relative to finetune at one side is minor to significant (-1.7 ~ 48.4%) negative to moderate (-3.82~ 4.67%).

  - For using synthetic data as dataset B

    - When dataset A / B mismatch is considered and dataset A does not contain B, 1 source [Samsung] observes similar performance (-0.89% ~ 0.21%) as case 1.

    - When dataset A includes B or equal to B,

      - If the input is in spatial-frequency domain, and dataset A does not contain B, 2 sources [Panasonic, Ericsson] observe minor to moderate loss (-1.1% ~ -3.69%) relative case 1 depending on the modelling of synthetic data. The improvement compared to case 2 is minor (0.57% ~ 1.3%). Compared to finetune at one side, the performance improvement is similar to minor (-0.35% ~ 1%).

      - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-3.67% ~ -11.18%) relative to case 1 depending on whether synthetic data is used in the finetune.

      - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.89% ~1.51%).

| Company | Comments |
| --- | --- |
|  | Thanks FL for addressing this. In our results we assumed the mismatch in the dataset distribution level not on the dataset samples level. We also added results for the case Dataset A and Dataset B from our tdoc to the excel file. The added results are summarised below. - For using synthetic data as dataset B - When dataset A and B are from different NW-side distribution (deployment scenario) A / B mismatch is considered and dataset A does not contain B, 1 source [Samsung] observes similar performance (-0.89% ~ 0.21%) as case 1. - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-29.0% ~ -30.5%) relative to case 1 depending on whether synthetic data is used in the finetune. - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.3% ~-1.3%). - When dataset A includes B or equal to B or dataset A and B are from different NW-side distribution (deployment scenario) - If the input is in spatial-frequency domain, and dataset A does not contain B, 2 sources [Panasonic, Ericsson] observe minor to moderate loss (-1.1% ~ -3.69%) relative case 1 depending on the modelling of synthetic data. The improvement compared to case 2 is minor (0.57% ~ 1.3%). Compared to finetune at one side, the performance improvement is similar to minor (-0.35% ~ 1%). - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-3.67% ~ -11.18%) relative to case 1 depending on whether synthetic data is used in the finetune. - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.89% ~1.51%). |

##### Proposal 403d: Observations from X8

Capture the following observations into TR:

For the evaluation of performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution (Issue 9),

For case 2 (model trained on dataset S but tested on dataset B),

- 2 sources [vivo, QC] observe significant performance loss (-7.2% ~ -17.4%) relative to case 1, where dataset B consists of actual field data.

- 1 source [ETRI] observes significant performance loss of -37.3% relative to case 1, where dataset S and B are different by TxRU mapping.

- 3 resources [ZTE, Panasonic, Ericsson] observe moderate performance loss (-2.12% ~ -4.75%) relative to case 1, where dataset S and B are generated from different scenarios, antenna layout or UE location. E.g., Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

For case 2A (model further finetuned on dataset B),

- If finetune at NW and fix encoder at UE

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [vivo] observe moderate loss (-4.63% ~ -7.66%) relative to case 1. The improvement compared to case 2 is minor to moderate (2.58% ~ 9.17%).

    - When dataset A / B mismatch is not considered, 2 1 sources [vivo, QC] observes minor to moderate loss of (-3.5% ~ -7.66%) loss relative to case 1. The improvement compared to case 2 is moderate to significant (2.6% ~ 13.9%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [ZTE] observes moderate loss to case 1 (-4.32% ~ -4.75%). The improvement compared to case 2 is negative (-0.68% ~ -1.02%).

    - When dataset A includes or is the same as dataset B, 3 sources [ZTE, Panasonic, Ericsson] observe minor to moderate loss relative to case 1 (-0.97% ~ -4.67%). The improvement compared to case 2 is negative to minor (-0.61.01% ~ 1.6149%). Dataset S and B are Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

- If finetune at UE and fix decoder at NW

  - For using actual field data as dataset B,

    - 2 sources [vivo, QC] observe moderate to significant loss (-6.8% ~ -12%) relative to case 1. The improvement compared to case 2 is minor moderate (0.4% ~ 7.1%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - 2 source [Panasonic, Ericsson] observe minor to moderate loss (-1.56% ~ -3.81%) relative to case 1. The improvement compared to case 2 is marginal (0.03% ~ 0.75%). Dataset S and B are different by antenna layout and indoor/outdoor ratio.

    - 1 source [ETRI] observes significant loss (-23.8%) relative to case 1. The improvement compared to case 2 is significant (13.5%). Dataset S and B are different by TxRU mapping

- If finetune at both UE and both NW sides

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered and dataset A does not contain B, 1 source [vivo] observes significant loss (-8.5% ~ -44.85%) relative to case 1 depending on specific scenarios for data collection. The improvement compared to case 2 is negative (-28% ~ -1.3%). The loss relative to finetune at one side is negative (-1.72% ~ -37.19%).

    - When dataset A / B mismatch is not considered and A is equal to B, 1 2 sources [vivo, QC] observes moderate to significant loss of (-7.3% ~ -56.1%) relative to case 1. The improvement compared to case 2 is negative to significant (-39.3% ~ 10.1%). The loss relative to finetune at one side is minor to significant (-1.7 ~ 48.4%) negative to moderate (-3.82~ 4.67%).

  - For using synthetic data as dataset B

    - [When dataset A / B mismatch is considered modelled by Rx antenna spacing and dataset A does not contain B and dataset A has same distribution as S, 1 source [Samsung] observes similar performance (-0.89% ~ 0.21%) as case 1.]

    - When dataset A / B mismatch is modelled by different scenarios (i.e., by NW side condition),

      - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-29.0% ~ -30.5%) relative to case 1 depending on whether synthetic data is used in the finetune.

      - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.3% ~-1.3%)

    - When dataset A includes B or is equal to B or has same distribution as B,

      - If the input is in spatial-frequency domain, and dataset A does not contain B, 2 sources [Panasonic, Ericsson] observe minor to moderate loss (-1.1% ~ -3.69%) relative case 1 depending on the modelling of synthetic data. The improvement compared to case 2 is minor (0.57% ~ 1.3%). Compared to finetune at one side, the performance improvement is similar to minor (-0.35% ~ 1%).

      - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-3.67% ~ -11.18%) relative to case 1 depending on whether synthetic data is used in the finetune.

      - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.89% ~1.51%).

| Company | Comments |
| --- | --- |
| Mod | The cyan highlighted is controversial and will potentially be removed. The reason is that the modelled scenario (wherein A has the same distribution as S, and B has a different Rx antenna spacing than A) is essentially NW/UE-side data distribution mismatch due to UE-side additional condition, and there is no aspect modelled related to synthetic-field data distribution mismatch. |
| Samsung | We do not agree with the above assessment for FL. Please consider the notes below in RAN1#118bis agreement. What we are evaluating is the distribution mismatch between Dataset S and Dataset. It is clearly noted here such mismatch can be modelled with respect of UE-side additional conditions. - Note: Dataset S is meant to represent the synthetic data used for fully specified model development, and Dataset B is meant to represent the field data. - Examples for Dataset S and B: - Modeling via TxRU mapping, UMi vs. UMa, UMa vs. InH, etc. - It is acknowledged that the scenario/configuration modeling between S and B may not be complete representative of the difference between synthetic and field data distribution mismatch. - Synthetic vs. actual field data (if available) - Companies are encouraged to provide details on the scenarios/configurations underlying the field data - Note: Case 1 and Case 2 have been studied during Rel-18 generalization study and conclusions have been captured in the TR. - Note: For Case 2A, companies may additionally evaluate UE-side / NW-side data distribution mismatch with respect to UE side additional condition. This can be done, in Step 2, by treating the training Dataset B as representing UE-side data distribution and considering a training Dataset A representing NW-side data distribution. - Note: Evaluations are assumed to perform without common reference model among companies. |
| Mod | @Samsung Basically, Samsung modelled UE/NW-side data distribution mismatch (via different Rx antenna spacing between dataset A and dataset B). However, there is no modeling of synthetic/field data distribution mismatch, as Samsung’s dataset S is the same as dataset A and differs from dataset B only in Rx antenna spacing. We can’t say that Rx antenna spacing is a valid modeling of synthetic/field data distribution mismatch. |

##### Proposal 404a: Direction C performance (Same as 45b of RAN1 #119)

Conclude that

- Fully specified models in Direction C may have limited performance in the field compared to Direction A.

- The performance of fully specified models in Direction C may be improved at least if either side train their part of model using field data, yet still worse than the achievable performance of Direction A.

| Company | Comments |
| --- | --- |
| Lenovo | Agree |
| Huawei, HiSilicon | Support |
| Panasonic | We are fine with the conclusion. |
| TCL | Agree |
| CMCC | Agree |
| Samsung | We may first capture the possible conclusion from the observation in Proposal 403a without comparing it to Direction A. If needed, the two directions can be compared in a more comprehensive manner later. It is to be noted that Case 1 in the above observation represent hypothetical model trained by 1-to-1 joint training. Thus, we suggest the following modifications: - Fully specified reference models in Direction C, when employed directly, may have limited performance in the field compared to a jointly trained model via offline inter-vendor collaboration. Direction A. - The performance of fully specified reference models in Direction C may be improved at least if either side train their part of model using field data, yet still worse than the achievable performance of jointly trained model via offline inter-vendor collaboration Direction A. |
| ZTE | Fine with the first bullet. However, for the second bullet, some preconditions need to be clearly mentioned. During the evaluations for different inter-vendor collaboration options, it’s always assumed that the performance impact due to NW/UE data distribution mismatch with respect to NW side additional condition (e.g., TxRU mappings, antenna height, antenna down tilt) can be handled easily, e.g., by the indication of associated ID abstracting the NW side additional condition to facilitate UE side data collection. However, the feasibility and approaches of addressing the NW-side additional conditions across training and inference has never been discussed, which may impact the field performance of different inter-vendor collaboration options. In this sense, we suggest the following revision for the second bullet. - The performance of fully specified models in Direction C may be improved at least if either side train their part of model using field data, yet still worse than the achievable performance of Direction A if necessary additional information can be shared from NW to UE. |
| Fujitsu | Generally fine with the proposal. |
| Ericsson | Suggest adding the following sub-bullet under the 2nd bullet • Retaining/finetuning of decoder at NW side with fixed refence encoder at UE side may provide better performance than retaining/finetuning of encoder at UE side with fixed decoder at NW side. |
| OPPO | Fine |
| ETRI | Agree |
| NTT DOCOMO | Support |
| CEWiT | Agree with the conclusion. |

##### Proposal 404b: Direction C performance

Conclude that

- Fully specified models in Direction C, when employed directly, may have limited performance in the field compared to Direction A.

- The performance of fully specified models in Direction C may be improved at least if either side train their part of model using field data, yet still worse than the achievable performance of Direction A.

| Company | Comments |
| --- | --- |

### Way forward and discussion on sub-options

##### Proposal 405a: way forward for Directions (Continuation from 49a of RAN1 #119)

- Direction C with standardized model ensures a minimum performance requirement.

  - Standardized model is specified in RAN4

  - RAN1 assumes the NW-side and UE-side may either implement the RAN4 model, a model equivalent to the RAN4 model, or a model finetuned with field data while being compatible to the RAN4 model.

- Direction A with standardized model parameter / dataset exchange from NW to UE side achieves  better performance.

| Company | Comments |
| --- | --- |
| Lenovo | Agree in general. That would be good to say if RAN4 came to conclusion that it is possible to have standardized model, - Direction C, if a Standardized model is specified in RAN4, that model can be used to with standardized model ensures a minimum performance requirement. - Standardized model is specified in RAN4 - RAN1 assumes the NW-side and UE-side may either implement the RAN4 model, a model equivalent to the RAN4 model, or a model finetuned with field data while being compatible to the RAN4 model |
| Futurewei | We are ok with the proposal in general. |
| Huawei, HiSilicon | OK in general except two comments: 1) For the RAN4 specification part, considering the current model studied at RAN4 is CNN which is different from the RAN1 agreed model structure of TF, we should inform RAN4 to follow RAN1 guidance for model specification. 2) RAN1 does not assume the implementation on model which is proprietary. Instead, RAN1 can assume the implemented model at NW/UE side is compatible with the specified encoder/decoder. - Standardized model is specified in RAN4 following RAN1 guidance - RAN1 assumes the NW-side and UE-side may either implement the RAN4 model, a model equivalent to the RAN4 model, or implement a model finetuned with field data while being compatible to the RAN4 specified encoder/decoder model, respectively. |
| Panasonic | We are fine with the proposal in principle. On the first sub-bullet, RAN1 agreed in the last meeting that Transformer is adopted as the backbone structure for studying the standardized model structure. On the other hand, our understanding on current RAN4 model is CNN. Then, although RAN4 model can be used, it would be better to consider how to align RAN1 studied standardized model and RAN4 studied standardized model. |
| TCL | Agree |
| CMCC | Agree in principle. But as Huawei, Panasonic said, RAN4 are using CNN structure, while RAN1 are study Transformer, it is better to have aligned standardized model between RAN1 and RAN4. |
| Samsung | We propose to defer the discussion on which working group specify the model, if it ever has to be discussed here. We may rather capture a simple conclusion that Direction C ensures a minimum performance requirement. |
| ZTE | General fine. We suggest to additionally mention that standardized encoder is preferred from RAN1 perspective. The encoder specification is easier due to its relatively lower complexity. Besides, the network side generally has stronger computation capability to flexibly train an actual decoder compatible to various specified encoders at different UEs and also guarantee the system performance to large extent. |
| Fujitsu | In our view, the reference model specified in RAN1 and RAN4 should be different. For Direction C, the reference model specified in RAN1 should achieve the optimal performance in field rather than a minimum performance. |
| Ericsson | For the first bullet: Although RAN4 has agreed a model structure, this does not mean that there has been any study on how to define the model structure in RAN4. The RAN4 agreement on a CNN model structure was deliberately arbitrary and it was explicitly agreed that the model structure is not seen as being the final one to use. The investigation in RAN4 is whether *given an agreed model structure* it is possible for companies to algin results and agree on a standardized test decoder that would interoperate with multiple encoders, and also whether the option 4 (standardized labelled dataset or reference encoder) can be made to work. If RAN4 study draws positive conclusion, more serious studies shall be done for model specification, by taking the performance, complexity, and scalability aspects into account. These studies can be done in RAN1, and the study outcome can be informed to RAN4 for the model specification. Hence, we suggest rephrasing the first sub-bullets as o Standardized model is specified in RAN4 o The fully specified reference model for Direction C is the same as the specified model for RAN4 performance testing. For the second bullet: The feasibility of standardized signaling for dataset/model parameter exchange is still under study in RAN2. Hence, we proposal to reformulate the second bullet as the following: • Direction A with standardized model parameter / dataset exchange from NW side to UE side, if feasible, may achieves better performance. |
| OPPO | It is not proper to say RAN4 should do the model specification in RAN1. And as far as I know, RAN4 is only focusing on the reference decoder, the reference encoder standardization is not discussed in RAN4 yet. To address this issue, a LS should be sent from RAN1 to RAN4 to conclude this aspect. |
| NTT DOCOMO | Agree with the proposal. |

##### Proposal 405b: way forward for Directions

- Direction C with standardized model ensures a minimum performance requirement.

  - Standardized model is specified in RAN4

  - The fully specified reference model for Direction C is the same as the specified model for RAN4 performance testing.

  - RAN1 assumes the NW-side and UE-side may either implement the RAN4 model, a model equivalent to the RAN4 model, or a model finetuned with field data while being compatible to the RAN4 model.

- Direction A with standardized model parameter / dataset exchange from NW-side to UE-side may achieves better performance.

| Company | Comments |
| --- | --- |

##### Conclusion

- Direction C with standardized model ensures a minimum performance requirement.

  - The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4.

- Direction A with standardized model parameter / dataset exchange from NW-side to UE-side may achieves better performance.

##### Proposal 406a: Necessity of target CSI sharing (Continuation from Proposal 43b in RAN1 #119)

Regarding target CSI sharing for option 3a-1

- NW-side target CSI sharing from NW-side to UE-side can facilitate UE-side encoder performance validation/testing

- NW-side target CSI sharing from NW-side to UE-side for UE-side encoder training can

  - help UE-side address potential NW/UE-side data distribution mismatch due to UE-side additional condition

  - ensure dataset diversity and good generalization ability for the UE-side encoder development / training

| Company | Comments |
| --- | --- |
| Lenovo | Agree. Just we suggest removing the last bullet - NW-side target CSI sharing from NW-side to UE-side can facilitate UE-side encoder performance validation/testing - NW-side target CSI sharing from NW-side to UE-side for UE-side encoder training can - help UE-side address potential NW/UE-side data distribution mismatch due to UE-side additional condition - ensure dataset diversity and good generalization ability for the UE-side encoder development / training |
| vivo | Do not agree. Based on the evaluation results from companies (ZTE, vivo, CATT, Samsung, Xiaomi, OPPO, ETRI), it is not necessary to have target CSI shared from NW to UE. |
| Futurewei | We are ok with the proposal. |
| Huawei, HiSilicon | Agree that Target CSI is needed for 3a-1. Two points are raised: 1) Besides the motivations provided in the FL proposal, we also identify that if the training and/or testing dataset of Target CSI is autonomously collected by UE side, it would be more complex from both spec perspective and UE side implementation perspective to align the dataset construction information, e.g., list of cells for collecting data, number of data samples for per cell, quality of data, methods for data processing, etc. 2) One question to be clarified is, for the 1st bullet, why Target CSI is used to address the UE-side additional condition? In our view, UE side additional condition can be addressed by UE side autonomous data collection, while Target CSI is used to address the NW-side additional condition (e.g., site information) which cannot be collected by UE side data collection. Changes: - NW-side target CSI sharing from NW-side to UE-side can facilitate UE-side encoder performance validation/testing - NW-side target CSI sharing from NW-side to UE-side for UE-side encoder training can - help UE-side address potential NW/UE-side data distribution mismatch due to NWUE-side additional condition - ensure dataset diversity and good generalization ability for the UE-side encoder development / training - Alleviate burden to align the dataset construction information in contrast to UE side autonomous data collection |
| Panasonic | We are fine with Lenovo’s update. |
| CMCC | Support. But we want to note that target CSI sharing is not mandatory coupled with Option 3a-1. It is still possible that UE side can autonomously collect target CSI. |
| Samsung | We agree target CSI collection with the same NW-side assumption is necessary for performance validation/testing. However, it may suffice if the target CSI for validation follows the same distribution as the NW-side training. This does not necessitate the network to share the quantized target CSI for validation purpose, i.e., UE-side can directly collect (measure) the target CSI as long as some indication ensures consistency with NW-side additional conditions assumed in the NW-side training. If the validation target CSI is from the NW-side training, we may run the risk of overfitting. |
| ZTE | The target CSI is needed to facilitate UE-side encoder performance validation/testing. However, it can be achieved by autonomously UE-side data collection with additional information sharing from the NW side. Specifically, if the NW-side additional condition can be aligned between NW and UE (e.g., by the exchange of associated ID abstracting NW-side additional condition), there would be no data distribution mismatch issue between the target CSI collected by the NW and the target CSI collected by the UE. Then, the sharing NW-side target CSI is not necessary, which instead cause non-negligible signaling overhead. Additionally, the performance impact of data distribution mismatch with respect to UE-side additional condition can be addressed by NW timely data collection and thus NW-side target CSI sharing is not needed. |
| CATT | We do not agree. Our simulation results, together with (ZTE, vivo, Samsung, Xiaomi, OPPO, ETRI) have shown that target CSI sharing is not needed. Therefore, we suggest to remove the two bullets and only conclude: - NW-side target CSI sharing from NW-side to UE-side is not mandatory |
| Fujitsu | Generally fine with the proposal. |
| Ericsson | Ok in general |
| OPPO | Target CSI sharing for option 3a-1 should be optional. |
| ETRI | We agree on need of the target CSI sharing from NW for the testing/validation purpose. However, for the training purpose, considering the minor performance degradation from the UE-side additional condition mismatch and the overhead for sharing the target CSI for testing, we do not support the need of sharing the target CSI from NW as a mandatory. |
| NTT DOCOMO | We have concerns about the UE privacy issue regarding the target CSI sharing. The MNO may not be able to share the data collected from one UE to another A solution to address the UE privacy concern should be studied. |
| QC | Agree with the proposal. Without NW-side target CSI in option 3a-1, the only thing the UE-side has is the encoder (denoted as E) shared by the NW-side. In this case, for a new UE that the UE-side is developing that has different input distribution (due to differences in antenna design, RF, and baseband and pre-processing algorithms), there is no way for the UE-side to tell if a given UE-side encoder (denoted as E’) is good or not. The only way to validate E’ is to have a NW-side decoder (denoted as D) or its proxy model (denoted as D’) and run through E’-D to verify if the final SGCS is good or not. With the NW-side target CSI and E, UE-side can train a proxy decoder D’ which the UE-side can utilize for training and verifying E’. |

##### Proposal 407a: Pairing ID

For inter-vendor-collaboration Options 3a-1 and 4-1, a pairing ID is shared as additional information along with the exchanged dataset or the model parameters.

- The same ID can be used for NW to collect data for NW-side training

- The same ID can be used for UE to collect data for UE-side training

- The same ID can be used for applicability inquiry and reporting

- The same ID can be used for inference configuration

- The same ID can be used for monitoring configuration

- Note: the name pairing ID is for discussion purposes.

| Company | Comments |
| --- | --- |
| Lenovo | We agree in general but with the following changes. For inter-vendor-collaboration In Options 3a-1 and 4-1, a pairing ID is shared as additional information along with the exchanged dataset or the model parameters can be associated with an ID. - The same ID can be used for NW to collect data for NW-side training - The same ID can be used for UE to collect data for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for monitoring configuration - Note: the name pairing ID is for discussion purposes. We suggest to remove the first bullet as the dataset/model parameter usually created after the NW-side data collection and after NW-side training. |
| vivo | Agree with Lenovo’s update. |
| Huawei, HiSilicon | Not clear why it is needed for monitoring? Suggest to removing the monitoring bullet. Change on top of Lenovo’s version: For inter-vendor-collaboration In Options 3a-1 and 4-1, a pairing ID is shared as additional information along with the exchanged dataset or the model parameters can be associated with an ID. - The same ID can be used for NW to collect data for NW-side training - The same ID can be used for UE to collect data for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for monitoring configuration - Note: the name pairing ID is for discussion purposes. |
| Panasonic | We are fine with Lenovo’s update. |
| TCL | Agree with Lenovo’s update. |
| CMCC | Agree with Lenovo’s update. |
| Samsung | Ok. Fine with Lenovo’s update too, except for the exclusion of the usage for NW-side data collection. |
| ZTE | There is no-going discussions in agenda 9.1.4.2 and some agreements have been reached in the last meeting. For example, a first/second indication referring to the model structure/parameter for model transfer/delivery Case z4 or ID-X for the dataset for MI-Option2 (i.e., model identification with dataset transfer) can be transmitted from NW to UE along with the exchanged dataset or the model parameters. Considering the controversy about the relationship between the first/second indication, ID-X and pairing ID in this proposal, we suggest to wait for more progresses in framework agenda to avoid duplicate work. |
| CATT | Agree with Lenovo’s update. |
| Ericsson | Ok with Lenovo’s update in general, suggest to add FFS for the bullet related to NW-side data collection. - FFS: The same ID can be used for NW to collect data for NW-side training |
| OPPO | Same view to ZTE, to avoid the duplicated discussion to 9.1.4.2. |
| NTT DOCOMO | Agree with Lenovo’s update. |
| Mod | @ZTE, @OPPO: It was clarified during the 9.1.4.2 agreement that the agreement in 9.1.4.2 may not be applicable to the two-sided CSI compression use case, mainly because there is differing understanding on whether there is model identification or not. In FL’s understanding, there are quite a few companies who thinks that model identification is not relevant. Moreover, the MI-Options in 9.1.4.2 are for online model identification, while Options 3a-1 and 4-1 is regarding training collaboration between NW-side and UE-side, so the FL does not think that the dataset and/or parameter sharing between NW-side and UE-side involves any online model identification. |

##### Proposal 407b: Pairing ID

For inter-vendor-collaboration In Options 3a-1 and 4-1, a pairing ID is shared as additional information along with the exchanged dataset or the model parameters can be associated with an ID.

- FFS: The same ID can be used for NW to collect data for NW-side training

- The same ID can be used for UE to collect data for UE-side training

- The same ID can be used for applicability inquiry and reporting

- The same ID can be used for inference configuration

- FFS: The same ID can be used for monitoring configuration

- Note: to use the name pairing ID is for discussion purposes.

| Company | Comments |
| --- | --- |

##### Agreement

In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then

- The same ID can be used for UE to collect UE-side target CSI for UE-side training

- The same ID can be used for applicability inquiry and reporting

- The same ID can be used for inference configuration

- FFS: whether ID/even same ID is needed for monitoring configuration

- FFS: where the ID is assigned

- Note: whether the purpose for pair will be specified will be discussed separately.

##### Proposal 408a Performance target for Direction A

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm that performance target is shared as additional information along with the exchanged dataset or the model parameters.

- FFS: type of performance metric, e.g., SGCS

- FFS: input data for deriving the performance target

| Company | Comments |
| --- | --- |
| vivo | We prefer to have discussion on what is performance target first and have companies views aligned on the details how such performance target is calculated, rather than directly confirm its necessity. |
| Huawei, HiSilicon | OK with the direction. Two minor comments: 1) NMSE is also used for evaluation and can also be considered as the type. 2) For the 2nd bullet, does “input data” mean the testing dataset? If so, we agree with it. Change: For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm that performance target is shared as additional information along with the exchanged dataset or the model parameters. - FFS: type of performance metric, e.g., SGCS, NMSE - FFS: input data for deriving the performance target |
| Panasonic | We are fine with the proposal. |
| TCL | Agree |
| CMCC | Agree. |
| Samsung | Ok |
| ZTE | OK |
| LG | Fine |
| CATT | OK |
| Fujitsu | Ok. |
| Ericsson | OK |
| OPPO | Ok in general. But what type of performance metric may depend on the input and the label. If only focusing on the encoder performance, it seems we cannot obtain the SGCS directly. |
| ETRI | Agree |
| NTT DOCOMO | In our view, the performance target sharing is beneficial at least for UE quickly verifying the model. How UE verifies the target performance can be further discussed. |
| Mod | @Huawei/HiSilicon: “input data” means the input dataset that UE-side uses to assess the performance. |

##### Proposal 408b: Performance target for Direction A

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm that performance target is shared as additional information along with the exchanged dataset or the model parameters.

- FFS: type of performance metric, e.g., SGCS, NMSE

- FFS: input data for deriving the performance target

| Company | Comments |
| --- | --- |

##### Proposal 408c: Performance target for Direction A

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm that performance target is shared as additional information along with the exchanged dataset or the model parameters.

- FFS: type of performance metric, e.g., SGCS, NMSE, SGCS statistics

- FFS: input data for deriving evaluating the performance target

| Company | Comments |
| --- | --- |

##### Agreement

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, performance target is confirmed as additional information along with the exchanged dataset or the model parameters.

- FFS: type of performance metric

- FFS: input data for evaluating the performance

##### Discussion 409a: Backbone information for Direction A

There are diverging opinions on whether backbone information needs to be shared as additional information for Option 4-1. Some companies also raised that sharing backbone information is also relevant for Option 3a-1.

|  | Backbone information needs to be shared as additional information. | Backbone information doesn’t need to be shared as additional information |
| --- | --- | --- |
| 4-1 | Vivo, Panasonic, Xiaomi, ZTE, CATT, OPPO, ETRI | Huawei/HiSilicon |
| 3a-1 w/ target CSI sharing |  | Huawei/HiSilicon (for decoder), Panasonic, Xiaomi |
| 3a-1 w/o target CSI sharing |  | Vivo, Huawei/HiSilicon (for decoder), Panasonic, ZTE, CATT |

| Company | Comments |
| --- | --- |
| Lenovo | As option 3a-1 has specified model structure, that would nice if FL can clarify the necessity of backbone information. Is it the “decoder backbone”? |
| Huawei, HiSilicon | For 4-1, UE side can autonomously select appropriate model backbone/structure based on the guidance of performance target information delivered by NW side. Thus, delivery of backbone information is not needed. |
| Panasonic | We share the same comment as Lenovo. |
| CMCC | Backbone information sharing at least can help the other side, i.e. UE side, to find the compatible decoder much easier. But this sharing can be not mandatory coupled with Option 4-1 or 3a-1, it can be shared optionally. |
| Samsung | Our results show that the performance of Option 4-1 degrades when the UE-side trains a CNN encoder based on dataset corresponding to a transformer backbone encoder at the NW-side. We want to clarify the backbone above is rather encoder backbone. |
| ZTE | For option 4-1, the actual encoder and decoder are trained separately, misaligned model backbones or structures would definitely result in performance degradation (up to -5.1% performance degradation according to Rel-18 evaluations in TR). Therefore, it is essential to share additional information regarding the model backbone or structure from the NW side to the UE side for option 4-1. For option 3a-1, there is no such issue since both the reference encoder and decoder are trained at the NW side, and the actual encoder would always mimic the exchanges one. |
| OPPO | For option 4-1, sharing NW-side decoder backbone information would be very helpful so that UE can align the backbone between UE-side encoder and NW-side decoder to improve the final performance. Actually, this additional information can be optional. |
| NTT DOCOMO | We have the same question as Lenovo’s. In our view, the model backbone is specified for Option 3a-1. |
| CEWiT | We share the same view as OPPO. |
| Mod | “Backbone information” for 3a-1 means decoder backbone information. |
| Qualcomm | We think, whether backbone information is to be shared or not, applied to both 4-1 and 3a-1. In 3a-1, a reference encoder structure is shared, such encoder structure is used to convey encoder response, it sheds no light to the decoder backbone that is used by the NW side. In other words, NW is not obliged to use the same backbone as the reference encoder specifies. The actual decoder backbone choice is upto NW implementation. |
| Samsung | Re Mod. We considered the below setup to test whether a single decoder can serve encoders from different backbone structures. We noticed the two encoders cannot learn a single latent mapping, i.e., ztf=zcnn, as forcing this would significantly degrade performance. The other option is to let each encoder learn its own latent mapping but the decoder be able to reconstruct the input from both. For Opt 4-1, this approach corresponds to a case with multiple datasets associated to a single decoder, e,g., Dataset#1 {v, ztf} and Dataset#2 {v, zcnn}. The gNB may need to share dataset specific for certain encoder backbone assumption at the network side or provide backbone information for the assumed encoder at the network. |

##### Discussion 410a: views on sub-options of Direction A

As a reference, here is a summary of companies’ input on their views regarding sub-options of direction A from RAN1 #119. This was copied from Discussion 43a from the FL summary of RAN1 #119.

| Option 4-1 (from last meeting) -- DO NOT MODIFY !! |  |
| --- | --- |
| Support / Can accept | QC, Lenovo, Huawei/HiSilicon, Panasonic, Spreadtrum, Fujitsu, Futurewei, OPPO, LG, Apple |
| Okay / neutral | ETRI, NTT DOCOMO |
| Object / Have a concern |  |

| Option 3a-1 w/ target CSI sharing from NW to UE (from last meeting) -- DO NOT MODIFY !! |  |
| --- | --- |
| Support / Can accept | Lenovo, Panasonic, Futurewei, NTT DOCOMO |
| Okay / neutral | QC, Xiaomi (Open), Fujitsu, ETRI |
| Object / Have a concern |  |

| Option 3a-1 w/o target CSI sharing from NW to UE (from last meeting) -- DO NOT MODIFY !! |  |
| --- | --- |
| Support / Can accept | Vivo, Xiaomi, ETRI, ZTE |
| Okay / neutral |  |
| Object / Have a concern | QC, Lenovo, Panasonic, Fujitsu, Futurewei, Apple, NTT DOCOMO |

Please state your views again below. This is only for reference.

| Option 4-1 (from last meeting) |  |
| --- | --- |
| Support / Can accept | Lenovo, Futurewei, Huawei/HiSilicon, Panasonic, NEC, OPPO |
| Okay / neutral | ETRI |
| Object / Have a concern | Vivo, NTT DOCOMO |

| Option 3a-1 w/ target CSI sharing from NW to UE (from last meeting) |  |
| --- | --- |
| Support / Can accept | Lenovo, Futurewei, Panasonic, NTT DOCOMO |
| Okay / neutral |  |
| Object / Have a concern |  |

| Option 3a-1 w/o target CSI sharing from NW to UE (from last meeting) |  |
| --- | --- |
| Support / Can accept | Vivo, ZTE，CATT, ETRI, NTT DOCOMO |
| Okay / neutral |  |
| Object / Have a concern | Lenovo, Futurewei, Huawei/HiSilicon, Panasonic |

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |

# Model structure standardization feasibility and scalability

## Summary of company proposals

Futurewei

Proposal 3: For Option 3a-1 in Direction A for CSI compression using 2-sided model, regarding the feasibility of scalable model structure, consider using FCN-based layer(s) as one of the options for adaptation layer(s), at least for handling different Tx ports and different bandwidths/number of sub-bands.

- Note: other options are not precluded.

Proposal 4: For Option 3a-1 in Direction A for CSI compression using 2-sided model, regarding the feasibility of scalable model structure across multiple CSI encoder output sizes, consider using mask operation as one of the options to control the final CSI encoder output size during the model training phase.

Ericsson

Proposal 3	RAN1 should await conclusions from RAN4 feasibility study before further discussion on model specification.

Observation 33	For studying of model structure scalability over different numbers of Tx ports, bandwidths, and slots, the following different methods shall be evaluated and compared in terms of model performance, complexity and forward compatibility:

a.	Treat one dimension of the input precoding matrix as a token, and the other dimension(s) of the input precoding matrix as sequence,

i.	Design the FCN (embedding) layer to accommodate the largest expected feature dimension of a token

ii.	Design the FCN (embedding) layer to accommodate a selected most common dimension of a token

iii.	Design the FCN (embedding) layer to accommodate the lowest expected feature dimension of a token

b.	Design multiple FCN (embedding) layers for different input precoding matrix dimensions, and only active one of them for an input precoding matrix in model inference.

c.	Define a fixed-size atomic unit of the input precoding matrix as a token and represent an input precoding matrix as a sequence of fixed-size tokens with variable sequency length.

Observation 34	For studying the model structure scalability over different payload sizes, the following different methods shall be evaluated and compared in terms of model performance, complexity and forward compatibility:

a.	Design the FCN (latent) layer to accommodate the largest expected dimension of the latent space and adapt the output to the desired payload.

b.	Design multiple FCN (latent) layers for different payload dimensions, and only active one of them for generating the desired payload.

c.	Use transformer cross attention techniques to associate the latent space size with a output sequence length, and the output sequence length is selected according to the desired payload size.

ZTE

Proposal 11:  For model with Transformer backbone, the following hyper-parameters can be further studied for model structure standardization:

•	Alternative 1: Typical Transformer backbone

•	Number of Transformer blocks

•	Model dimension

•	Hyper-parameters in the multi-head self-attention modules, at least including number of attention heads and the dimension of each attention head

•	Hyper-parameters in the feed-forward modules, at least including dimension of latent space, and the activation function

•	Alternative 2: Swin Transformer backbone

•	Input dimension

•	Number of Swin-Transformer blocks

•	Model dimension

•	Hyper-parameters in the patch partition modules, at least including patch size, and patch resolution

•	Hyper-parameters in the multi-head self-attention modules, at least including number of attention heads, the dimension of each attention head, window size, QKV bias

•	Hyper-parameters in the feed-forward modules, at least including dimension of latent space, and the activation function

Proposal 12:  For model structure specification for Direction A Option 3a-1, Transformer should be adopted as the backbone structure for two-sided model of CSI compression as a starting point.

Proposal 13:  RAN1 and RAN4 can share the same standardized model structure for two-sided model of CSI compression to avoid the duplicated specification efforts, where the detailed specification and testing can be up to RAN4 study.

Proposal 14:  The scalability solution for scaling the input/output dimension of the AI/ML model (e.g., pre/post-processing of truncation/padding, various quantization granularities, and adaptation layer in the AL/ML model) causes minor or no effects on the model structure specification.

Proposal 25:  For studying the scalability of standardized model structure, temporal domain Case 0 with spatial-frequency domain input can be adopted as a primary use case and Transformer is suggested as the basic backbone structure.

Observation 13: For model scalability over 32 Tx ports and 16 Tx ports, compared with generalization Case 1 that AI model is trained and tested on the same configuration of 16 Tx ports, generalization Case 2 (i.e., AI model is trained on 32 Tx ports and tested on a different configuration of 16 Tx ports) shows large performance degradation of -4.85% for layer 1 SGCS and -7.90% for layer 2 SGCS.

Observation 14: For model scalability over 32 Tx ports and 16 Tx ports, compared with generalization Case 1 that AI model is trained and tested on the same configuration of 16 Tx ports, generalization Case 3 shows marginal performance degradation of -0.44% for layer 1 SGCS and -0.52% for layer 2 SGCS for testing on 16 Tx ports. In addition, less performance loss is observed for testing on 32 Tx ports, i.e., -0.26% for layer 1 SGCS and -0.44%  for layer 2 SGCS.

Proposal 26:  For the scalability verification of AI/ML based CSI compression over various numbers of Tx ports, the model scalability can be achieved by training the model with mixed dataset from different numbers of Tx ports.

Observation 15: For model scalability over low, medium, and high CSI payload sizes, compared with generalization 1 case that AI model is trained and tested on the same configuration, AI model trained with mixed dataset of multiple CSI feedback payload sizes can achieve minor performance loss of -0.76% for layer 1 SGCS and -0.93% for layer 2 SGCS under high payload size.

Observation 16: For model scalability over low, medium, and high CSI payload sizes, compared with generalization 1 case that AI model is trained and tested on the same configuration, AI model trained with mixed dataset of multiple CSI feedback payload sizes can achieve over 1.58% SGCS gains for low payload sizes and over 2.46% SGCS gains for medium payload sizes, respectively

Proposal 27:  For the scalability verification of AI/ML based CSI compression over various CSI feedback payload sizes, the model scalability can be achieved by training the model with mixed dataset from different CSI feedback payload sizes.

Huawei

Proposal 6: For studying the standardized model structure of inter-vendor collaboration Option 3, strive to perform down selection to the factors (e.g., rank>1 options, generalization aspects, scalability approaches, quantization/dequantization method, etc.) so that a single model structure along with pre/post processing approach is aligned before starting the evaluation.

Nokia

Proposal 7: RAN1 to develop methods for specifying multi-configuration encoders via datasets (Direction A Option 4-1) or reference models (Direction A Option 3a-1).

Proposal 8: RAN1 to study how to specify support for scalable model structures.

Samsung

Proposal#19: For model structure standardization, focus the discussion on Case 0:

-	Input/output/latent adaptation for Case 2/3.

Proposal#20: For spatial frequency-domain CSI compression Case 0, for the study of standardized model structure, consider the following model hyper-parameters of transformer model

-	Input and output processing

-	Number of attention blocks for encoder (Lenc) and decoder (Ldec)

-	Model dimension (embedding size)

-	Attention type and number of attention heads (h)

-	Activation function for dense block Enc. & Dec.

-	Quantization type and bits

FFS: other details.

Observation#17: For the transformer model in the spatial-frequency-domain compression, increase on both the model dimension and the number of attention blocks tends to have a monotonic but diminishing performance gain at the expense of increased model size and complexity.

Observation#18: For the transformer model in the spatial-frequency-domain compression, for a given model dimension, increasing the number of attention blocks does not always guarantee performance gain.

Proposal#21: For the transformer model in the spatial-frequency-domain compression, consider balanced model hyper-parameters selection based on the tradeoff between model performance and complexity/size.

Vivo

Proposal 1:	Support classic Transformer model structure (a detailed step by step example of classic Transformer processing for each component is illustrated in table 1) as baseline for studying standardized model structure.

Table 1 Step-by-step equation of CSI compression encoder with Transformer encoder

|  |  | Input dimension X | Equation |
| --- | --- | --- | --- |
| 1. Embedding layer |  | X1∈R13×64 | linear Layery1=X1∙W1+β1 W1∈R64×256 β1∈R256 |
| 2. Positional encoding |  | X2=y1∈R13×256 | y2=X2+Poscoding Poscoding∈R13×256 (a trainable matrix with initialized weights to be zeros) |
| 3. 6 sequential Transformer blocks, and each Transform block includes the right parts. | 3-0 Layer Norm | X3-0∈R13×256 X3-0={x1,...,x13} | y3-0=a*x-ExVarx+Δ+β3-0 a∈R256 β3-0∈R256 |
|  | 3-1 MSA | X3-1∈R13×256 | Per head: Projection: linear(X3-1) X3-1∈R13×256->QKV∈R13×32 Attention: y3-1-att=softmaxQKTS*V QKV∈R13×32 Concatenate& linear Layer: linear(cat(y3-1-att,1,…,y3-1-att,8)) y3-1-att∈R13×32->y3-1∈R13×256 |
|  | 3-2 residual connection | X3-2∈R13×256 | y3-2=X3-0+y3-1 |
|  | 3-3Layer Norm | X3-3∈R13×256 X3-3={x1,...,x13} | y3-3=a*x-ExVarx+Δ+β3-3 a∈R256 β3-0∈R256 |
|  | 3-4 MLP | X3-4∈R13×256 | y3-4=ReLUX3-4∙W3-4-1+β3-4-1.W3-4-2+β3-4-2 W3-4-1∈R256×1024 β3-4-1∈R1024 W3-4-2∈R1024×256 β3-4-2∈R256 |
|  | 3-5 residual connection | X3-5∈R13×256 | y3-5=X3-3+y3-4 |
| 4. Dimension reduction |  | X4∈R13×256 | Reshape：X4∈R13×256 -> X4-rep∈R3328 linear：y4=SigmoidX4-rep∙W5+β5 W4∈R3328×30 β4∈R30 |
| output |  | y4∈R30 |  |

Proposal 2:	Towards model structure standardization for CSI compression with two-sided model in RAN1 in R19, primarily consider the most typical Transformer design considering the reference model can be further optimized for implementation and performance upper-bound can be achieved by simply increasing model size. Further optimizations can be considered only if the involved modifications do not require any new operations and/or key hyper-parameters compared with the typical version.

Proposal 3: For CSI generation model with Transformer backbone, the classic Transformer Encoder is aligned as following for further study of model structure (as shown in table 1):

    - Embedding operation: linear Layery1=X1∙W1+β1

    - Positional encoding operation: y2=X2+Poscoding

    - Operation within each Transformer block

      - Layer norm operation y3-0=a*x-ExVarx+Δ+β3-0

      - Residual connection operation as shown in row 3-2 or 3-4 in Table 1

      - Operation within multi-head attention as shown in row 3-1 in Table 1

        - Self-attention operation

        - Concatenation and linear operation

      - MLP operation as shown in row 3-4 in Table 1

        - Linear part

        - Activation function

    - Dimension reduction operation as shown in row 4 in Table 1

Proposal 4: For model with Transformer backbone, the following hyper-parameter needs to be specified for standardized model structure (with value range or specific values):

- Number of Transformer blocks;

- Model dimension;

- Hyper-parameters in each transformer block

  - Number of attention heads

  - Dimension of each attention head

- Hyper-parameters in the feed-forward blocks

  - Dimension of latent space

  - Activation function

- Hyper-parameters of positional embedding

  - Sequence length

Observation 4:	Given the same model dimension, trade-off between number of attention head and dimension of attention head does not affect performance.

Observation 5:	Given the same complexity, very deep (e.g., 64 blocks) or very shallow (e.g., 1 block) models may show some performance degradation. So, it is suitable to consider a moderate value such as 4 blocks in the specified model structure.

Proposal 5:	Towards a better trade-off between performance and complexity, the following combination can be candidate hyper-parameter for standardized model structure:

	“4 Transformer blocks, model dimension 256, dimension of each head 32, 8 attention heads” for a relatively better performance,

	“4 Transformer blocks, model dimension 128, dimension of each head 16, 8 attention heads” for a relatively lower complexity while keeping most gain of the case.

Observation 6:	A typical CSI compression encoder (including embedding layer, Transformer backbone, and dimension reduction layer) could be a scalable model to multiple input/output shapes. The scalability of model structure can be achieved by activating part of the specified structure by encoder parameter exchange if the corresponding layers (e.g., embedding layer and/or dimension reduction layer) are specified according to the largest possible size.

Observation 7:	Scalability (of specified model structure and model parameters) over different Tx antenna ports (e.g., 8/16/32 ports) can be achieved at the cost of minor performance loss (e.g., from -0.8% to -1.6% for 59bits, from -2% to -2.8% for 104bits, from positive gain to -0.2% for 252bits) via input padding for a specific model (including both structure and parameter)

Observation 8:	Scalability (of specified model structure and model parameters) over different number of subbands (e.g., 15/13/11/10 ports) can be achieved at the cost of minor performance loss (e.g., from -1.5% to -1.1% for 59bits, from -2.0% to -0.7% for 104bits, from positive gain to -0.8% for 252bits) via input padding for a specific model (including both structure and parameter).

Observation 9:	Scalability (of specified model structure and model parameters) over different payload sizes (e.g., 59/104/293 bits or 59/104/152/202/252/293 bits) can be achieved at the cost of minor performance (e.g., from -0.26% to -0.1% for 3 payload sizes, from -1.27% to positive gains for 6 payload sizes) via adaption layers for a specific encoder model (including both structure and parameter)

Observation 10:	Scalability (of specified model structure and model parameters) over different Tx antenna ports, number of subbands, and payload sizes is robust to rank configurations when considering per-layer processing with common structure across ranks and layers.

Observation 11:	From initial results, it is seen that CNN based angular-delay domain input has small SGCS loss compared to spatial-frequency domain input. However, CNN in angular-delay domain input is expected to be simple.

Observation 12:	To solve the scalability issue, similar methods for spatial-frequency domain input could be used in angular-delay domain input

	Tx antenna ports: no scalability issue;

	Number of subbands and higher-layer parameter (numberOfPMI-SubbandsPerCQI-Subband): input padding;

	Payload sizes: adaption layers.

Proposal 6:	Towards model structure standardization for temporal domain case3, reuse the model structure of Case 0 with input/output adaptation, i.e., directly extending the range of self-attention operation to multiple slots.

Observation 14:	Model structures mentioned in the agreement require non-trivial additional efforts for specification on top of case0 Transformer backbone structure:

	Study and definition of new layers (e.g., Conv-LSTM/LSTM layers) in option 1

	Study and definition of new operations (e.g., new quantization method on top of conventional SQ/VQ) in option 2

Proposal 7:	Seek a model structure for temporal domain case2 which only requires straightforward extension of Transformer for case0 to reduce the specification efforts and facilitate the SI progress.

Proposal 8:	Towards model structure standardization for temporal domain case2, consider to use Transformer backbone (same backbone as the model structure for Case 0) with the following adaptation:

	Extending the range of self-attention operation to multiple slots in auto regressive way.

	Hyper parameter adjustment

Apple

Observation 1: Due to various options available and limited evaluation, specify model structure and/or model parameter can be challenging.

CATT

Proposal 1: For studying the standardized model structure, prioritize the model input in spatial-frequency domain.

Proposal 2: For studying the standardized model structure, prioritize the standardization on Case 0 model structure.

Proposal 3: For Case 0 encoder model structure standardization, adopt the classic Transformer structure in Figure 1 as the starting point for the standardized structure, with the option to prune unnecessary modules.

Proposal 4: The following parameters should be considered as hyper-parameters for Transformer backbone:

	Number of Transformer blocks (P1)

	Input dimension of multi-head attention (P2)

	Number of multi-head attention heads (P3）

	Dimension of attention blocks (P4)

	Feed forward dimension (P5)

Observation 1: For Case 0 encoder model structure standardization, hyper-parameter combination provided in Table 1 can provide good SGCS performance with relatively low complexity.

Table 1 Hyper-parameter combination

| P1 | 2 |
| --- | --- |
| P2 | 128 |
| P3 | 8 |
| P4 | 16 |
| P5 | 4*128 |

Observation 2: For Case 0 encoder model structure standardization, with fixed hyper-parameter combination provided in Table 1, the choice between ReLU and GeLU as activation function in Transformer backbone has small impact on SGCS performance.

Observation 3: For Case 0 encoder model structure standardization, with fixed hyper-parameter combination provided in Table 1, removing absolute positional encoding in Transformer backbone has small impact on SGCS performance.

Proposal 5: To specifying scalable model structure over different payload sizes and Tx ports, adaption layers are adopted.

Proposal 6: To support different bandwidths, the model can accommodate CSI reporting across different bandwidths without altering its structure, including the following methods:

	Obtaining CSI feedback for larger bandwidths through multiple inferences;

	Adjusting the input bandwidth;

	Cropping the model output.

OPPO

Proposal 6: Regarding the standardization of model structure in Option 3, suggest to prioritize the work on Case 0. Case 2 and Case 3 can be further considered after model structure in Case 0 has been specified.

Xiaomi

Proposal 13: The hyper-parameter of Transformer model structure for Case 0 should be firstly defined.

Proposal 14: For Option 3a-1 or 4-1, the supported input/output model parameters, such as number of antenna ports, payload size, and so on, are reported to NW for indicating model scalability so that NW can flexibly configure parameters.

DCM

Observation 1

	For Case 0, the input/output adaption for a specified model structure can be solved by specifying the following,

	Pre-/Post-processing (e.g., input padding schemes, output truncation schemes)

	The specified TF (full model or model structure) can be designed with the most significant input/output dimensions.

	Input/Output adaption layers, the structure of the adaptive layers [and their parameters] can be specified.

	The specified TF (full model or model structure) can be designed with the dimensions of the input/output adaption layers.

	For Case 2/3, the input/output adaption can be used to extend the TF to capture temporal domain features,

	The CSI of multiple time instances can be input to the TF, and the self-attention of the TF can be extended directly to capture the CSI features from multiple instances.

CMCC

Observation 4: Based on the results in TR 38.843, the scalable model structure specification and model parameters specification over numbers of Tx ports, CSI feedback payload sizes, and bandwidths, number of slots is feasible for Direction C; the feasibility of the scalable model structure specification can be further improved for Direction A Option 3a-1.

Qualcomm Incorporated

Observation 16: Specific linear embedding and zero-padding achieve good scalability performance for antenna port configurations with marginal loss relative to specific model development. Specific linear embedding slightly outperforms zero-padding.

Observation 17: Transformer with proper positional embedding and zero-padding achieve good generalization performance (with marginal loss relative to specific model development) if sufficient compression tasks are considered in the training phase.

Observation 18: Transformer with specific linear compression layer and specific vector quantization at the end of encoder achieve minor loss relative to specific model design for medium payload and achieve marginal loss for low payload regime.

Proposal 11: Consider transformer-based model structure with positional embedding followed by series of transformer blocks

•	The hyper-parameters include number of transformer blocks = 6, dimension of transformer block = 64, number of attention heads = 8, dimension of attention head = 8.

•	The size of the positional embedding is based on max number of subbands (19) and dimension of the transformer (64)

Proposal 12: Consider following methods for model structure scalability

•	To scale with antenna port configuration, employ specific linear embedding layers with zero-padding prior to the positional embedding

•	To scale with subband configuration, employ respective entries of positional embedding and perform zero-padding to the transformer output based on subband indices

•	To scale with payload configuration, employ specific linear compression layer and specific vector quantization after the transformer blocks

## Discussion

| Companies | Views |
| --- | --- |
| Futurewei | Scalability of subband and ports: use FC as adaptation layers Scalability of payload configuration: use mask operation |
| Ericsson | How to describe a model structure in 3gpp Await RAN4 feasibility study. For scalability on ports, bandwidth, slots, eval considers - One dimension of input as token, the other as sequence - FC layer to accommodate, the largest, the lowest, and the most common dimension of a token - Multiple FC for different input dimensions, active one of them in inference - Fixed-size atomic unit of input precoding matrix as token, represent precoder as a sequence of tokens with variable sequence length For scalability on payload, - Common FC for largest dimension and adapt the output - Multiple FC for different payload and activate one in inference - Cross-attention mechanism, token associated to an output length. |
| ZTE | Model backbone: - Alt1: TF - Depth, model dimension, #head, dim of head, feedforward modules (dimension of latent space, activation function) - Alt2: Swin TF - Input dimension, depth, model dimension, patch partition, RAN1 and RAN4 share same standardization model structure Scalability solution causes minor or no effects on model structure specification Focus on case 0 Scalability achieved by mixing the dataset Scalability study of training on 32 ports and testing on 16 ports, generalization case 2 shows moderate degradation, case 3 has marginal loss Scalability to low, medium and high payload achieves marginal loss or even gain. |
| Huawei | Down-selection to variable scalable aspects, including rank > 1, generalization aspects, scalability approaches, quantization / dequantization method. Single model structure is aligned before evaluation. |
| Nokia | Specify multi-configuration encoders with scalable structure |
| Samsung | Focus on case 0 - Hyper parameters include - Input / output processing - Number of attention blocks - Model dimension - Attention type and number of heads - Activation function - Quantization type Consider balanced hyper-parameter selection based on tradeoff between model performance and complexity |
| Vivo | Support classical TF structure Provide model structure description in Table 1 including - embedding operation, positional encoding operation, operation within each TF block (layer norm, redidual connection operation, operation within muti-head attention, MLP operation) and dimension reduction. Hyperparameters - Depth, model dimension, number of head, dim of head, dimension of latent space, activation function, dimension of positional embedding - Propose candidate values - Depth = 4, d_model = 256, nhead=8, dim_head=32 - Depth = 4, d_model = 128, nhead = 8, dim_head=16 Observations: - Scalability over Tx achieved with minor loss by zero-padding - Scalability over subband achieved with marginal loss via zero-padding - Scalability over payload achieved with marginal loss via mask, and gain with adaptation layers - Scalability over ports, subband, payload robust to rank configuration, considering per-layer operation with specific parameter and same structure For TSF case 3, reuse case 0 model by taking self-attention to time domain For TSF case 2, consider extend the range of self-attention to multiple slots in auto-regressive way. |
| Apple | Challenging to specify model structure due to various available options |
| CATT | Prioritize model input in spatial-frequency domain Prioritize case 0 Consider classical TF structure with option to prune unnecessary modules Hyper-parameters - Depth, model dimension, number of heads, dim of head, feedforward dimension - Provide candidate values: depth=2, d_model = 128, number of head = 8, dim head = 16, ff ratio = 4. - Activation function = Relu or Gelu - Removing positional embedding has small impact Scalability over ports and payload, consider adaptation layers Scalability over subbands, consider - Multiple inference - Adjusting input bandwidth - Cropping model output |
| OPPO | Focus on case 0 |
| Xiaomi | Prioritize case 0 Report supported port configurations, payload, as UE capability |
| DCM | Scalability of input/output can be solved by - Padding / truncation - Specified model - Adaptation layers The above can be extended to TSF case 2/3 |
| CMCC | Model structure specification and scalability is feasible based on results in TR 38.843. |
| Qualcomm | Consider classical TF structure with hyper-parameters - Depth = 6, d_model = 64, nhead = 8, dim_head = 16 - Positional embedding with size based on max number of subbands and d_model Scalability over ports, can be achieved by adaptation layers and / or zero-padding Scalability over subbands, can be achieved by positional embedding and zero-padding Scalability over payload, can be achieved by adaptation layers and quantization |

##### Proposal 51a: Model structure study for Case 0

For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure

Encoder description:

| Embedding layer | Number of tokens: Nsubband, up to Nsubband,max Feature dimension of each token: 2*NumPorts Output dimension dmodel |
| --- | --- |
| Positional encoding | Trailable parameters for each token position Number of tokens positions: Nsubband,max Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead= dmodel / Nhead Dimension of latent space inside feedforward module dFF Activation function choice An optional dropout parameter dropout for regularization |
| Output linear layer | Reshape the matrix from (Nsubband x dmodel) to a vector of dimension (Nsubband * dmodel) Output dimension zdim |
| Vector quantization |  |

Decoder description:

The decoder has a mirroring design as the encoder.

| Company | Comments |
| --- | --- |
| Lenovo | We suggest to first come to conclusion that we need to have standardized model as part of 3a-1 intervenor collaboration, and then start discussion of standardized model. Note that we are assuming that for direction C, RAN1 will rely on RAN4 design/agreement. [Mod] My understanding is that the current CNN-based RAN4 structure is only for the purpose of RAN4 feasibility study and not intended for the final specified model. With RAN1’s model structure study, it is reasonable for RAN4 to utilize the model structure RAN1 studied. |
| vivo | Support the FL proposal. To Lenovo, we have already agreed to study Transformer model structure in last meeting. This is to further conduct the model structure study based on what is agreed from last meeting. Moreover, this study is also to provide feasibility study of 3a-1 and based on this study, the inter-vendor collaborations discussion can be further carried out. |
| Huawei, HiSilicon | OK with the discussion. See some clarification questions in below. Questions for encoder part: For Embedding layer: i) Nsubband,max – does it mean the input subband dimension is padded to the max dimension to fit the Embedding layer? [Mod] If padding is used, it’s padded to the max dimension. If positional embedding is used, only N_subband tokens are used out of N_subband_max positions. ii) Output dimension dmodel – does it mean the output dimension of each token? [Mod] Yes For Transformer blocks: i) dropout parameter dropout for regularization – does it apply to training, or inference? If it only impacts training, then it does not impact the model structure. [Mod] It affects only training. I will remove this in the updated proposal. For Output linear layer: i) For the vector of dimension (Nsubband * dmodel), it should further perform segmentation before performing VQ, i.e., VQ CB is used to quantize per segment. [Mod] I will add scalar quantization and more details. Questions for decoder part: do we need to specify the decoder model structure? For Option 3, it seems no need to specify the decoder. If it is intended for Option 1, then from our experience, the complexity of decoder is usually higher than the encoder, so “a mirroring design” is not accurate – at least the values of hyper parameters may be different from encoder. [Mod] My opinion is that, for the purpose of encoder model structure specification, we will have to agree on the decoder structure, although only the encoder is specified in the end. |
| Xiaomi | How to define the quantization method needs to further discussed. We prefer scalar quantization at this stage, because it is simple and help to align the performance evolution results among companies. [Mod] I will add scalar quantization and more details. |
| Fujitsu | We share a similar view with Xiaomi. It is better to give a general description for quantization method at the current stage. |
| Ericsson | The place where Dropout should happen can also be up to implementation. [Mod] Dropout affects only training. I will remove Dropout in the updated proposal. The "reshape" is not a uniquely defined operation. It depends on if you read it "column wise" or "row wise". [Mod] Each company can use their preferred way at this point. No need to align this for the current study purpose. More discussion is needed for quantization. [Mod] I will add scalar quantization and more details. In addition, the proposed model structure puts restrictions on the possible methods for model structure scalability. As an example, the proposed model structure cannot support the following candidate method for model structure scalability study: - define a fixed-size atomic unit of input precoding matrix as token and represent precoder as a sequence of tokens with variable sequence length. - use cross attention techniques to associate the latent space size with an output sequence length, and the output sequence length is selected according to the desired payload size. Before making such restrictions, different candidate model structures shall be studies and compared in terms of performance vs complexity and forward compatibility (e.g., when more Tx ports or larger bandwidth are supported in later releases). [Mod] What is proposed is based on a popular model structure based on Transformer. For companies with a different structure in mind, please make a proposal for alternative structures. |
| OPPO | Same view to Xiaomi. Scalar quantization should also be considered. [Mod] I will add scalar quantization and more details. |
| ETRI | We are generally fine, but we prefer to have an option to use scalar quantization as the quantization method, as we see it to be a simple but effective apporoach. [Mod] I will add scalar quantization and more details. |

##### Proposal 51b: Model structure study for Case 0

FL note: Please find FL replies inline in the above company comments table.

For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure

Encoder description:

| Embedding layer | Number of tokens: Nsubband, up to Nsubband,max Feature dimension of each token: 2*NumPorts Output dimension dmodel per token |
| --- | --- |
| Positional encoding | Trailable parameters for each token position Number of tokens positions: Nsubband,max Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead= dmodel / Nhead Dimension of latent space inside feedforward module dFF Activation function choice An optional dropout parameter dropout for regularization |
| Output linear layer | Reshape the matrix from (Nsubband x dmodel) to a vector of dimension (Nsubband * dmodel) Output dimension zdim |
| Vector quantization | Scalar quantization: 2 bits Vector quantization - segment size = 2 - bits per segment = 4 bits (2 bits per output dimension) Total payload size = 2*zdim |

Decoder description:

The decoder has a mirroring design as the encoder. Details are to be discussed.

| Company | Comments |
| --- | --- |
| ETRI | The text “Vector Quant” in the figure may need to be changed. And for the flexibility, parameterized design for quantization for scalar/quantiazation may be beneficial at this stage. An example is in below: * Scalar quantization - Number of bits per latent dimension : N_bitsperdim - Total payload size = N_bitsperdim * z_dim * Vector quantization - Segment size : N_seg - Bits per segment : N_bitsperseg - Total payload size = z_dim * N_bitsperseg / N_seg |

##### Proposal 51c: Model structure study for Case 0

For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure

Encoder description:

| Embedding layer | Number of tokens: Nsubband, up to Nsubband,max Feature dimension of each token: 2*NumPorts Output dimension dmodel per token |
| --- | --- |
| Positional encoding | Trailable parameters for each token position Number of tokens positions: Nsubband,max Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead= dmodel / Nhead Dimension of latent space inside feedforward module dFF Activation function choice An optional dropout parameter dropout for regularization |
| Output linear layer | Reshape the matrix from (Nsubband x dmodel) to a vector of dimension (Nsubband * dmodel) Output dimension zdim |
| Vector quantization | Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg Total payload size = Nbit * Zdim |

Decoder description:

The decoder has a mirroring design as the encoder. Details are to be discussed.

| Company | Comments |
| --- | --- |

##### Agreement

For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose,

Encoder description:

| Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer) | Number of tokens Feature dimension of each token Output dimension dmodel per token |
| --- | --- |
| Positional encoding | Number of tokens positions Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice |
| Output linear layer | Reshape the matrix to a vector Output dimension zdim |
| Quantization | Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment |

Decoder description:

The decoder has a mirroring design as the encoder. Details are to be discussed.

##### Proposal 51d: Model structure study for Case 0

Below is the updated diagram to replace the diagram in the agreement.

| Company | Comments |
| --- | --- |
| Samsung | Ok |
| Futurewei | In the updated diagram, the number of bits after quantization is no longer Nbit which represents the number of bits per latent dimension based on the agreement. For scalar quantization, number of bits after quantization is Nbit * Zdim. For vector quantization, number of bits after quantization is Number of segments * bits per segment (where bits per segment is the VQ codebook size) In addition, we think the following description for VQ is confusing: - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg Technically speaking the dimension for output of the linear layer (zdim) can be an arbitrary integer, like 10, 16, 27, 32, etc. unless we agree to define a fixed set. If we assume the zdim is 27 and number of segment (Nseg) is 3 (dimension of each segment is 9 in this case), and assume the VQ codebook size is 10 bits, thus, the total payload size after VQ is 3 * 10 = 30 bits. From the above example, bits per segment after VQ is 10 bits while the calculated number of bits per latent dimension after VQ would be total payload size / latent dimension = 30 / 27 = 1.11…bits. Please clarify in case the interpretation is not the intention. |

##### Proposal 51e: Model structure study for Case 0

FL notes: changes include

- Use different value of N_TF for encoder and decoder

- Rename the two “linear layer”s for decoder into “input linear layer” and “output linear layer”

| Company | Comments |
| --- | --- |

##### Proposal 52a: hyper-parameters for model structure study

- Down-selection from following candidate hyper-parameter combinations

  - Alt1: NTF,dmodel,Nhead,dhead,dFF={4,256,8,32,1024}

  - Alt2: NTF,dmodel,Nhead,dhead,dFF={4,128,8,16,512}

  - Alt3: NTF,dmodel,Nhead,dhead,dFF={6,64,8,8,256}

  - Alt4: NTF,dmodel,Nhead,dhead,dFF={2,128,8,16,512}

  - Other alternatives are not precluded

- For activation function, down-selection from ReLU or GeLU

| Company | Comments |
| --- | --- |
| vivo | Support the FL proposal. |
| Huawei, HiSilicon | This proposal can be discussed after we achieve agreement on 51a. |

##### Proposal 53a: model structure scalability

- For scalability over Tx port configuration, down-selection from

  - Alt1: specific linear embedding layer for each Tx port configuration

  - Alt2: a common linear embedding layer with zero-padding

    - FFS how to perform padding

  - Note: Alt1 and Alt2 can be used in combination.

- For scalability over subband configuration,

  - Alt1: positional embedding specific to each subband index

    - Nsubband tokens out of Nsubband,max token positions are used as input.

    - Zero-padding for inactive subbands positions at the output of transformer blocks prior to output linear layer.

  - Alt2: Zero-padding at the input

- For scalability over payload configurations,

  - Alt1: specific output linear layer for each payload configuration

  - Alt2: truncation/masking of the output linear layer output

| Company | Comments |
| --- | --- |
| vivo | We are fine to discuss model structure scalability. On the other hand, Based on companies evaluation results it seems that scalable model structure can be directly confirmed. And it seems no matter which alternative is selected, the model structure scalability itself can be confirmed. We prefer to confirm feasibility of having scalable model structure and having further study on detailed options. - Confirm the feasibility of designing scalable model structure over different Tx port configurations, sub-band configurations, and payload configurations. - For scalability over Tx port configuration, down-selection from - Alt1: specific linear embedding layer for each Tx port configuration - Alt2: a common linear embedding layer with zero-padding - FFS how to perform padding - Note: Alt1 and Alt2 can be used in combination. - For scalability over subband configuration, - Alt1: positional embedding specific to each subband index - Nsubband tokens out of Nsubband,max token positions are used as input. - Zero-padding for inactive subbands positions at the output of transformer blocks prior to output linear layer. - Alt2: Zero-padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output |
| Futurewei | We are ok with the proposal. |
| Huawei, HiSilicon | OK for the discussion. Our preferences are provided in below. For scalability over Tx port configuration, we prefer Alt2: common linear embedding layer with zero-padding. For scalability over subband configuration, we prefer Alt2: Zero-padding at the input. For scalability over payload configurations, we prefer Alt1: specific output linear layer for each payload configuration |
| ZTE | Generally fine. Considering the complexity and spec efforts associated with specific linear layer design for each Tx port, subband, or payload configuration, it’s much easier to specify the model structure based on the largest feature dimension and perform zero-padding for the model input or truncation/masking for the model output to achieve model structure scalability. |
| Fujitsu | Since the feasibility of scalable structure specification has been agreed to study for the number of slots, we suggest adding the following text to the proposal: - For scalability over slot configurations, - Alt1: specific linear layer for each slot configuration - Alt2: Zero-padding at the input |
| Ericsson | Suggest to add Alt 3 as a candidate for the study. - For scalability over Tx port configuration, down-selection from - Alt1: specific linear embedding layer for each Tx port configuration - Alt2: a common linear embedding layer with zero-padding - FFS how to perform padding - Alt 3: define a fixed-size atomic unit of input precoding matrix (e.g., n_Tx_ports*m_Subbands) as token and represent precoder as a sequence of tokens with variable sequence length, e.g., length = (Total_Tx_port*Total_Subbands)/ (n_Tx_ports*m_Subbands). - Note: Alt1 and Alt2 can be used in combination. - For scalability over subband configuration, - Alt1: positional embedding specific to each subband index - Nsubband tokens out of Nsubband,max token positions are used as input. - Zero-padding for inactive subbands positions at the output of transformer blocks prior to output linear layer. - Alt2: Zero-padding at the input - Alt 3: define a fixed-size atomic unit of input precoding matrix (e.g., n_Tx_ports*m_Subbands) as token and represent precoder as a sequence of tokens with variable sequence length, e.g., length = (Total_Tx_port*Total_Subbands)/ (n_Tx_ports*m_Subbands). - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: use cross attention techniques to associate the latent space size with an output sequence length, and the output sequence length is selected according to the desired payload size. |
| OPPO | This should be discussed after proposal 51a. |

##### Proposal 53b: model structure scalability

- For scalability over Tx port configuration, down-selection from

  - Alt1: specific linear embedding layer for each Tx port configuration

  - Alt2: a common linear embedding layer with zero-padding

    - FFS how to perform padding

  - Alt 3: define a fixed-size atomic unit of over Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. The number of tokens varies with number of Tx ports and number of subbands.

  - Note: Alt1 and Alt2 can be used in combination.

- For scalability over subband configuration,

  - Alt1: positional embedding specific to each subband index

    - Nsubband tokens out of Nsubband,max token positions are used as input.

    - Zero-padding for inactive subbands positions at the output of transformer blocks prior to output linear layer.

  - Alt2: Zero-padding at the input

  - Alt 3: define a fixed-size atomic unit of over Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. The number of tokens varies with number of Tx ports and number of subbands.

- For scalability over payload configurations,

  - Alt1: specific output linear layer for each payload configuration

  - Alt2: truncation/masking of the output linear layer output

| Company | Comments |
| --- | --- |
| Samsung | Ok |

##### Proposal 53c: model structure scalability

For model structure scalability study for temporal domain Case 0,

- For the choice of token dimension and feature dimension,

  - Alt 1: Use subband as the token dimension and Tx port as a feature dimension

    - The number of tokens varies with the number of subbands.

  - Alt 2: Use Tx port as the token dimension and subband as a feature dimension

    - The number of tokens varies with the number of Tx ports.

  - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens.

    - The number of tokens varies with the number of Tx ports and the number of subbands.

- For scalability over Tx port configuration the feature dimension,

  - Alt1: specific linear embedding layer for each Tx port configuration feature size

  - Alt2: a common linear embedding layer with zero-padding (e.g., zero-padding or other techniques for padding values)

    - FFS how to perform padding

  - Alt 3: define a fixed-size atomic unit of over Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. The number of tokens varies with number of Tx ports and number of subbands.

  - Note: Alt1 and Alt2 can be used in combination.

- For scalability over subband configuration the token dimension,

  - Alt1: positional embedding specific to each subband token index

    - Nsubband Ntoken,active tokens out of Nsubband,max Ntoken token positions are used as input.

    - Zero-padding for inactive subbands positions at the output of transformer blocks prior to output linear layer.

  - Alt2: Zero-padding at the input

  - Alt 3: define a fixed-size atomic unit of over Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. The number of tokens varies with number of Tx ports and number of subbands.

- For scalability over payload configurations,

  - Alt1: specific output linear layer for each payload configuration

  - Alt2: truncation/masking of the output linear layer output

  - Alt3: by varying quantization parameters

- Notes

  - Other Alternatives are not precluded.

  - Different Alternatives may be used in combination.

  - Same/similar approach is applied at the decoder side.

- Evaluations to consider:

  - Case 1 (scalable structure): Scalable model structure described above

    - Using one common model structure, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth}.

    - Companies to report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports)

  - Case 2 (dedicated structure): Model structure based on the Case 1 model structure, where the input and the output dimensions are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations.

    - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths.

  - For each scalable model structure choice, companies to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}.

| Company | Comments |
| --- | --- |

# Data collection & model pairing

## Summary of company proposals

Futurewei

Proposal 8: In AI/ML-based CSI compression using two-sided model, at least for NW-side ground-truth data collection for model training, consider adopting Rel-16 eType II CB based quantization with new parameters to achieve better performance.

ZTE

Proposal 29:  To enable high-quality data collection from UE to network, at least support

▪	UE reports data quality related information to NW, e.g., SINR, CQI, positioning information

▪	NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

Huawei

Proposal 8: For the NW side data collection, confirm the necessity and feasibility of UE report of the ground-truth CSI.

- For the data type, consider the following:

  - Precoding matrix

  - Channel matrix

- For the data format, prioritize Rel-16 eType II CB based quantization with new parameters for both above data types, and take the following new parameters as candidates for discussion:

  - L= 8, 10, 12; pv = 0.8, 0.9, 0.95; reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits.

Spreadtrum

Proposal 6: For network side data collection, support enhanced Rel-16 eTypeII codebook design to achieve high-resolution CSI for model training.

Proposal 7: For NW to collect data for training, both L1 signaling based reporting and RRC signaling based reporting should be supported for ground-truth reporting.

Proposal 8: For NW to collect data for training, enhancement on the design of CSI-RS is not needed.

CATT

Proposal 11: In CSI compression using two-sided model use case, both L1 signaling based reporting and RRC signaling based reporting are supported for ground-truth CSI for NW-side data collection for model training.

Proposal 13: In CSI compression using two-sided model use case, for L1 signaling based reporting of ground-truth CSI/target CSI for NW-side data collection, legacy CSI feedback framework can be reused for Case 0.

Proposal 14: In CSI compression using two-sided model use case, for L1 signaling based reporting of ground-truth CSI/target CSI for NW-side data collection, multiple time instants CSI can be reported in the same report for Case 3.

Proposal 15: In CSI compression using two-sided model use case, for NW-side data collection for model training, collecting ground-truth data in type of precoding matrix is supported.

Proposal 16: In CSI compression using two-sided model use case, for NW-side data collection for model training, NW determines the number of layers for ground-truth CSI data collection.

CMCC

Proposal 11: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection, the basic codebook structure could be reused, along with the basic concept of spatial domain, frequency domain and Doppler domain basis.

Proposal 12: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection, the exact supported values of codebook parameters can be studied to make sure high resolution data report.

Vivo

Proposal 9: 	For data collection in CSI compression

      - For NW to collect data for training

        - For Data format of NW to collect data for training, support using codebook-based Rel-16 eType2 as starting point.

        - For the FFS part

          - Report additional information regarding the samples, e.g., CQI, RI, timestamp

          - For CSI-RS and SRS configuration, similar to NW side data collection, reuse existing CSI-RS and SRS configuration as starting point

          - For case 2 and case 3 configuration, inform UE to collect multiple continuous data as a sample and number/time occasions of inputs and outputs.

      - For UE to collect data for training,

        - Different to NW side data collection, UE request RS configuration/transmission for data collection can be considered since the NW side didn’t know the UE data requirement (e.g., periodicity of CSI-RS, subbands of CSI-RS).

        - For CSI-RS and SRS configuration, similar to NW side data collection, reuse existing CSI-RS and SRS configuration as starting point.

        - For Configuration of temporal aspects for temporal case 2/3, P/SP CSI-RS can be supported

        - Model ID for option 3a-1 or ID-X for option 4 can be used as configuration ID.

Tejas Networks

Proposal 14: Consider enabling the network to configure the UE to report ground truth CSI via L1 signalling or RRC.

Proposal 15: For UE-side data collection, the network configures CSI-RS resources for the UE. An associated ID can be used to indicate network-side additional conditions.

OPPO

Proposal 16: Regarding the signaling of ground-truth reporting for NW-side data collection for training, high layer signaling (e.g., MDT) is preferred than L1 signaling.

Proposal 17: Regarding the NW-side data collection data format, float32 should be considered if high layer signaling is used for target CSI label reporting.

Proposal 18: Regarding the codebook-based data format in NW-side data collection, suggest to use Rel-18 eType2 or enhanced Rel-18 eType2 codebook for Case 2 and Case 3 with temporal domain correlations between target CSI labels.

Proposal 19: Regarding the CSI-RS configuration for UE-side and NW-side data collection, at least the follow aspects should be considered:

•	Data collection specific CSI-RS or not

•	Cell-specific or UE specific CSI-RS

Google

Proposal 7: For NW-side data collection, support the following enhancement for Rel-16 eType2 codebook and Rel-18 eType2 codebook for PMI prediction

- Additional O1 and O2 value

- Additional codebook parameter combinations at least for rank 3 and 4

- UE reports the measured CSIs instead of predicted CSI for Rel-18 eType2 codebook for PMI prediction

- Additional number of DD basis for Rel-18 eType2 codebook for PMI prediction

Proposal 8: For NW-side data collection, the potential spec impact for the configured number of layers can be based on introducing a constraint for the RI restriction to configure one RI for the CSI report.

Proposal 9: For NW-side data collection, support the UE to report the following additional information

- L1-SINR measured based on the CSI-RS to indicate the measurement quality

- Timing for the transmission occasion(s) of the CSI-RS for the measured CSI

Proposal 10: Support to configure multiple uplink power control parameter sets for SRS

- The SRS can be used for data collection and other functionalities, where different uplink power control parameter sets can be used for different functionalities

- The NW can indicate which one is applied for the SRS transmission dynamically

Proposal 11: Support both NW-configured and UE-requested based CSI-RS for UE-side data collection

- Corresponding number of CPUs should be occupied for the CSI-RS

- Associated ID should be provided

LGE

Proposal #4: To improve LCM aspects on TSF-domain CSI compression Case 3 and 4, consider the following:

-	Data quality-based sample selection for model updates

o	Example of the quality metrics: Prediction confidence, temporal consistency, CSI reconstruction error etc.

Proposal #5: On data collection for training at least for Case 3, consider CSI-RS enhancement from the following aspects:

-	Separate CMR for ground truth CSI: Adjusting power level and RS density

-	Temporal alignment: Linkage between inference and ground truth CSI measurements

NEC

Proposal 13: For defining the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB, down select from the following options:

- Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use.

- Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use.

- Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID.

Lenovo

Proposal 1: 	For transmission of ground-truth CSI samples, prioritize the performance of transmission of more samples, instead of fewer samples with higher resolution per sample (e.g., more samples with current parameter configurations for Rel-16 Type II, instead of less samples with a new parameter configuration for Rel-16 Type II), especially for cases that the overhead is more important, e.g., ground-truth data transfer for model monitoring or model update.

Proposal 2: 	Support procedures/signaling enabling transmission of subset of CSI samples among the set of measured/collected CSI samples from the environment.

Proposal 3: 	Support procedures/signaling enabling transmission of subset of CSI samples based on the experienced distortion level or quality indictor.

Proposal 4: 	Support procedures/signaling for transmission of additional information such as sample-group size, quality indicator, distortion level along the transmission of the sample itself.

Proposal 5: 	Support procedures/signaling enabling UE to report UW-side additional conditions and also the NW conditions/additional conditions under which the data/samples have been collected.

Proposal 14: 	Support definition of pairing information based on the conditions/additional conditions assigned to the samples of the datasets used for training of the model.

Proposal 15: 	Further study model identification/selection procedures during inference time when different models have been developed for different UE-NW vendor pairs.

Xiaomi

Proposal 18: For NW side data collection for training, the following aspects could be considered:

-	Cell-specific CSI-RS resource is configurated by NW-side to collect training dataset.

-	Associated information, such as TDCP, is reported to capture the UE side condition at least for Case 3.

-	Ground truth CSI could be aperiodically/semi-persistently or periodically reported by using L1 or RRC signalling.

Proposal 19: For UE side data collection for training, the associated ID needs to be configured for indicating NW side additional condition.

Panasonic

Observation 21: Data collection for model training and non-real time (slow) monitoring is not required to be real-time and then latency requirement can be relaxed.

Observation 22: Ground-truth CSI reporting could be realized through U-plane at least for data collection for model training and non-real time (slow) monitoring.

Observation 23: Assuming fast monitoring is 100s of ms order, U-plane, RRC or MAC-CE can be sufficient.

Observation 24: On data sample type / format for ground-truth CSI reporting, high resolution codebook-based format e.g., legacy codebook (e.g., eType II codebook) with potential enhancements such as extend more configurations in some parameters, should be studied.

Observation 25: For NW-side data collection, at least time stamps / situation of measurement, cell ID and UE location should be considered as the UE-side additional condition.

Observation 26: For NW-side data collection, the necessity and feasibility of UE reporting Rx filter assumption to network should be studied. Instead of informing actual configuration, UE-side associated ID is necessary.

Observation 27: For UE-side data collection for UE-side training, in order to identify the scenario / configuration, how to share the NW-side additional condition should be studied. Instead of informing actual configuration, some kind of configuration ID and /or change timing of NW-side additional condition is necessary.

Samsung

Proposal#8: For the cases of CSI compression with temporal aspects, consider the following for the network’s ground-truth CSI collection

For cases that require multiple time-domain samples for inference, cases 2/3/4/5, high resolution codebook quantization including temporal aspects, e.g., Rel-18 eType II-like method with new parameters.

For cases with CSI prediction, e.g., cases 3/4, high resolution codebook quantization for explicit channel matrices, e.g., codebook to report the left and right eigenvectors of a channel matrix H=UΛV^H

Specification impact on measurement and reporting for ground-truth CSI

Proposal#9: For NW’s and UE’s Data collection, consider indication on the network-side additional condition.

•	For two-sided models development, NW-part of two-sided model associated with a dataset can be considered as NW-side additional condition.

ETRI

Proposal 9: For dataset delivery for training collaboration type 3, for CSI compression sub-use case using two-sided model, consider the limited number of dataset samples to assess the feasibility of incorporating standardized signaling.

Fujistu

Proposal 15:

- For CSI compression using two-sided models, RAN 1 to further discuss using codebook-like approach to report ground-truth CSI for AI/ML model training and monitoring, e.g. Rel-16 e-type II-like codebook with enhanced parameter values.

Proposal 16:

- For Case 2, RAN1 to study how to address layer disorder issue for training data collection, model inference and performance monitoring.

MTK

Proposal 5.	For NW-side AI/ML model training, NW can rely on UL CSI samples collected from SRS sent by UEs. Use of DL CSI can be limited to finetuning purposes.

Qualcomm Incorporated

Proposal 16:	Consider using pairing ID for inter-vendor collaboration, UE side data collection, applicable condition enquiry and inference configuration.

Proposal 17:	For two-sided CSF use case, confirm the necessity of UE side data collection.

Proposal 18:	For UE side data collection for two-sided CSI compression use case, existing CSI-RS can be used as the data collection RS with additional configuration of pairing ID.

- Further study triggering event of ground-truth reporting based on SGCS generated via SGCS estimator.

CEWiT

Proposal-6: For NW sided data collection, specify the CSI configuration, parameter combinations and periodicity for data collection.

Proposal-7: For NW sided data collection, additional information e.g. SINR (in terms of CQI) on top of ground truth CSI.

Proposal-8: Consider channel parameter as a part of dataset-ID for data collection.

Proposal-9: For UE sided data collection, existing procedures can be reused for data collection.

## Discussion

Companies’ proposals are summarized as follows.

NW side data collection:

- Data format

  - High resolution eT2 or doppler eT2 (9)

    - Futurewei, Huawei, ZTE, spreadtrum, CMCC, OPPO, Google, Panasonic, Samsung, Fujistu

  - Existing eT2: vivo, QC, Lenovo (more data less quality)

  - Float32: OPPO (if higher layersignaling)

- Data sample type:

  - Precoder over channel matrix (Huawei, CATT)

- Configurations

  - Rank / layer configured by NW (Huawei, Google, CATT)

  - Threshold for data quality (ZTE)

  - Additional condition (Lenovo, Samsung)

- Additional information of UE reporting

  - Data quality (ZTE, Google, CEWiT)

  - Associated information, e.g., TDCP, for case 3, as UE condition (Xiaomi)

  - CQI / RI (vivo)

  - Timestamp (vivo, Google, Panasonic)

- Signalling to carry the report

  - L1 or RRC signaling (Spreadtrum, Tejas Networks, Xiaomi, CATT)

  - Higher layer signaling, e.g, MDT (OPPO)

  - Non-real time, U-plane, RRC, MACCE (Panasonic)

- Measurement

  - Reuse CSI framework, CSI-RS based measurement (CATT, vivo)

  - Cell-specific CSI-RS (Xiaomi, OPPO) or UE specific CSI-RS (OPPO)

  - Separate CSI-RS for ground-truth for TSF case 3, linkage between inference and ground-truth CSI measurements (LGE)

UE side data collection

- Mechanism

  - UE request (vivo, QC, Google)

- Configuration

  - Reusing existing CSI-RS (vivo, QC)

  - ID (for NW side additional condition), e.g., model ID, pairing ID, associated ID (vivo, Tejas Networks, Xiaomi, QC, Google, Lenovo, Panasonic, Samsung)

Other aspects

- Whether multiple CSIs in the same report for temporal case 2/3/4 (CATT).

- UE reports measured CSI instead of predicted CSI for TSF case (Google)

- SRS-based data collection and related enhancements (Google, MTK)

- Procedures / signalling enable transmission of subset of CSI samples based on environment, or experienced distortion or quality indicator. (Lenovo)

- How to address layer disorder issue for case 2 (Fujistu)

- Use CPU framework for processing criteria (Google)

- UE side additional condition, e.g., time stamps, location, cell ID, Rx filter (Panasonic)

Pairing information to keep consistency between training and inference

- Downselection among decoder ID, encoder ID, pairing ID (NEC)

- Procedures / signalling to enable UE/NW associate conditions/ additional conditions (Lenovo)

- Pairing based on conditions assigned to the dataset (Lenovo)

- Via pairing ID (QC)

##### Proposal 61a: Data collection details

< Placeholder for FFS items from the last meeting >

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |

# Monitoring

## Summary of company proposals

Futurewei

Proposal 9: In AI/ML-based CSI compression using two-sided model, for NW-side monitoring based on the target CSI reported by the UE, consider adopting Rel-16 eType II CB based quantization with new parameters to achieve better monitoring performance.

Proposal 10: In AI/ML-based CSI compression using two-sided model, for UE-side monitoring, if the actual or reference CSI reconstruction model is available at UE, support at least the following option:

- Based on the output of the CSI reconstruction model at the UE (Case 2-1)

Proposal 11: In AI/ML-based CSI compression using two-sided model, for UE-side monitoring, if the actual or reference CSI reconstruction model is not available at UE, support at least the following option:

- Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook

Proposal 12: In AI/ML-based CSI compression using two-sided model, for UE-side monitoring, further study the monitoring performance of “Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model” before discussing whether to support this option.

Proposal 13: In AI/ML-based CSI compression using two-sided model, for UE-side monitoring, further study at least the LCM complexity associated with using a proxy model first before discussing whether to support the use of proxy model at UE side for UE-side performance monitoring.

Ericsson

Proposal 14	Conclude that it is necessary to specify UE reporting high resolution target CSI to enable NW-side intermediate KPIs based performance monitoring and performance degradation error cause detection for two-sided CSI-compression use case.

Proposal 15	In CSI compression using two-sided model use case, capture in TR that ground-truth CSI report based on enhancements of the eType-II format with new parameters shall be defined to ensure high-accuracy model performance monitoring and error cause detection at the NW-side. Potential specification impact include:

•	Define the target-CSI format (e.g., Rel16 eType II CB with new parameters) for NW-side data collection (can reuse the ground truth defined for model training data collection)

•	Mechanisms (e.g., RRC-message based methods) to support UE reporting the target CSI together with the encoder output for NW-side data collection for performance monitoring.

•	Signaling and configuration for event triggered and periodical data collection at the NW-side.

Observation 24	For CSI compression using two-sided model use case, comparing to the method of UE-side monitoring via direct estimation of intermediate KPI (e.g., SGCS), the method of UE-side monitoring based on estimation of monitoring output (e.g., SGCS range indication), may provide better monitoring accuracy and generalization performance. Further study is needed.

Proposal 16	For CSI compression using two-sided model use case, for any UE-sided performance monitoring method (if its feasibility and performance are justified), to enable the testability of the quality of the UE reported monitoring metrics, at least the following spec impact are identified:

•	The format of the monitoring metrics

•	Singaling and mechanisms for UE reporting monitoring metrics

•	RAN4 testing of the quality of UE reported monitoring metrics

•	Data collection at the NW-side based on UE reporting monitoring data samples including target CSI, encoder output and monitoring metrics (to enable NW-side test the quality of the reported monitoring metric in the field).

ZTE

Proposal 31:  Prioritize to study the specification impacts on at least the following case for model performance monitoring,

•	NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE.

Proposal 32:  In CSI compression using two-sided model use case, deprioritize the study on UE-side monitoring in Rel-19 study phase.

Proposal 33:  In CSI compression using two-sided model use case, further study a set of appropriate monitoring methodology and reasonable monitoring metrics for the given KPI under rank>1.

Huawei

Proposal 9: For NW-side monitoring, consider ground-truth CSI based monitoring in Rel-19.

	eT2-like high-resolution codebook for reporting format with higher priority.

	Further discuss the reporting mode, e.g., per sample reporting and reporting of a number of monitored samples.

Proposal 10: There is no strong motivation for specifying the UE side proxy model (Case 2-1/2-2) for monitoring.

Proposal 11: For UE-side monitoring, consider precoded RS (transmitted from NW based on the output of the CSI reconstruction model) based monitoring in Rel-19.

- Further study the type of monitoring KPI calculated from the precoded RS.

Spreadtrum

Proposal 11: For performance monitoring, support the following monitoring options.

•	NW side monitoring based on the ground-truth CSI reported by UE.

•	UE side monitoring based on the recovery CSI indicated by NW.

CATT

Proposal 12: In CSI compression using two-sided model use case, L1 signaling based reporting of target CSI is supported for NW-side data collection for performance monitoring.

Proposal 17: In CSI compression using two-sided model use case, for NW-side monitoring Case 1, further study the signaling and procedures for reporting target CSI, with the following two options considered:

	Option 1: The target CSI is reported separately from its associated CSI feedback;

	Option 2: The target CSI is reported together with its associated CSI feedback.

Proposal 18: In CSI compression using two-sided model use case, performance monitoring at UE-side based on reference model or proxy model can be deprioritized.

Proposal 19: In CSI compression using two-sided model use case, if eventual KPI is adopted as monitoring metric, how to exclude the impacts of other factors other than AI/ML model performance should be studied.

Proposal 20: In CSI compression using two-sided model use case, support UE-side monitoring based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model.

Proposal 21: In CSI compression using two-sided model use case, regarding separately monitoring of CSI prediction and CSI compression for temporal domain aspects Case 3, for NW-side monitoring for CSI compression, the specification impact of monitoring of CSI compression includes:

	For Option 1, configuration and report for target CSI of CSI compression corresponding to the output of CSI prediction;

	For Option 2b, configuration and report of target CSI of CSI compression corresponding to the measured CSI of the future slot(s) and CSI feedback based on the measured CSI of the future slot(s).

Proposal 22: In CSI compression using two-sided model use case, regarding separately monitoring of CSI prediction and CSI compression for temporal domain aspects Case 3, for UE-side monitoring for CSI compression, the specification impact of monitoring of CSI compression includes configuration of CSI-RS for CSI measurement of the future slot(s) for option 2b.

Proposal 23: In CSI compression using two-sided model use case, regarding separately monitoring of CSI prediction and CSI compression for temporal domain aspects Case 3, study whether the selection of monitoring methods for CSI prediction needs to be comprehensively considered in conjunction with the monitoring methods for CSI compression.

Proposal 24: In CSI compression using two-sided model use case, for Option 1/3/4/5, when the performance is degraded and it has been identified that the root-cause is not UE-side issue, study whether and how to identify if the root-cause of performance degradation is data drift or NW-side issue.

Proposal 25: In CSI compression using two-sided model use case, if NW-side performance monitoring based on intermediate KPI is adopted, based on target CSI reported by the UE, NW-side monitoring for identifying whether performance degradation is caused by UE-side issues can be considered.

Proposal 26: In CSI compression using two-sided model use case, further study whether and how to support UE-side monitoring for identifying if performance degradation is caused by UE-side issues or not.

CMCC

Proposal 10: For performance monitoring, the following two options can be prioritized:

a)	NW-side monitoring based on the ground-truth CSI report.

b)	UE-side monitoring based on the recovery CSI indication.

Vivo

Proposal 10:	UE side monitoring can be supported for CSI compression with two-sided models

      - For UE-side monitoring, consider monitoring based on either direct estimation of SGCS (Case 2-1) or the output of reconstruction model at the UE (Case 2-2);

Proposal 11:	For data collection of temporal domain aspects Case 3, option 2 (The target CSI for training is derived based on the measured CSI of the future slot(s)) is supported

Proposal 12:	For monitoring of temporal domain aspects Case 3, Option 1(the monitoring label is derived based on the predicted CSI of the future slot(s))

Tejas Networks

Proposal 16: For NW side monitoring consider the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) for better monitoring accuracy.

Proposal 17: For UE side monitoring consider the Case 2-1 for better monitoring accuracy

o	 Based on the output of the CSI reconstruction model at the UE (Case 2-1)

	Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side.

OPPO

Observation 1: For the performance of UE-side monitoring Case 2-1, the percentage of KPIDiff<KPIth1increases when more monitoring samples are averaged.

Observation 2: Using the same backbone between UE-side proxy decoder and NW-side actual decoder slightly outperforms UE-side proxy decoder with different backbone from NW-side actual decoder.

Observation 3: UE-side monitoring Case 2-1 with proxy decoder can provide sufficient generalization between UMa and UMi scenarios.

Observation 4: For UE-side monitoring Case 2-1, UE-side proxy decoder with the same or different backbone to NW-side actual decoder can achieve the performance with PFA<0.06, PMD<0.05 and PCM>0.92 for UMa and UMi scenarios.

Proposal 6: UE-side proxy decoder can be trained with the frozen UE-side actual encoder based on the dataset shared from NW-side.

Proposal 7: Regarding the performance of UE-side monitoring Case 2-1 and Case 2-2, use the following monitoring metric and candidate KPIth value:

- False alarm rate: PFA=PKPIActual<KPIth*PKPIGenie≥KPIth∨KPIActual<KPIth

- Mis-detection rate: PMD=PKPIActual≥KPIth*PKPIGenie<KPIth∨KPIActual≥KPIth

where KPIth=0.5 is suggested.

Proposal 8: Support to study and specify UE-side monitoring Case 2-1 and Case 2-2.

InterDigital

Proposal 2:	Study further the following aspects for model monitoring in Rel-19:

- Details of reporting mechanism for the monitoring metrics with both time/event-trigger based

- Appropriate UE-side monitoring metric which reflects AI/ML model performance accurately

- UE-side monitoring based on precoded RS (CSI-RS, DM-RS)

- Reporting contents/structure of UE-side monitoring metric and its associated feedback overhead

- NW-side monitoring with lower signaling overhead

Proposal 3: Mechanisms for identifying the cause of performance degradation include:

- UE assistance information such as UE-side model monitoring metrics and UE-side out-of-distribution metrics

- Reporting the UE-assessed error cause (e.g., data drift, UE-side, or undetermined)

- Mitigation mechanisms, including fallback to legacy CSI reporting, model switching

Google

Proposal 4: For CSI report based performance monitoring, the following spec impact should be considered:

- New report quantity PMI only should be introduced, where UE reports the PMI based on a configured rank

  - Support a further enhancement to report subband L1-SINR in addition to the PMI to facilitate the precoder and MCS selection for PDSCH

- With regard to measurement accuracy in low SINR case, support the UE to report a state of CSI indicating the CSI is invalid for performance monitoring

- With regard to joint ML based CSI compression and prediction, support to configure whether the UE should perform the CSI predication based on ML or non-ML and the CSI quantization based on ML or non-ML for separate performance monitoring for ML based CSI prediction and ML based CSI compression.

Proposal 5: For SRS based performance monitoring, the following spec impact should be considered:

- Support to configure the SRS linked with a CSI-RS report configuration for ML based CSI, where the UE uses the same ports including antenna virtualization scheme to transmit the SRS and to receive the CSI-RS for ML based CSI

- Support burst based SRS with frequency hopping to facilitate the performance monitoring for wideband channel for coverage-limited UE

Proposal 6: For UE-side monitoring, the following spec impact should be considered:

- Introduce the hypothetical BLER as the metric for performance calculation

- Configuration of precoded CSI-RS for hypothetical BLER calculation

NEC

Proposal 6: Support NW-side monitoring based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook.

Proposal 7: For NW-side monitoring, RAN1 to discuss and evaluate whether ground truth reporting from UE can be transmitted over L1 or higher layer signaling (e.g. RRC).

Proposal 8: Support UE-side monitoring, the following can be considered:

- Based on the output of the CSI reconstruction model at the UE. Where the CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side.

- Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI.

Lenovo

Proposal 16: 	In options 3a-1, study mechanisms to ensure the applicability of the information received for the reference encoder model encoder model for the current input statistics.

Proposal 17: 	Study mechanism to determine the main contributor(s) of the lower performance of the model at least for the issues related to a) the “deployed” encoder and/or decoder model, b) the communication link, or c) data-drift of the input data. We note that, in general, the “deployed” encoder/decoder model could have different performance that the “trained” (reference) encoder or decoder model.

Proposal 18: 	 Study mechanism for root-cause determination based on exchange of some test data-set between the NW and the UE.

Proposal 19: 	Confirm that the local decoder model that is trained using data shared in options 4 and 3a-1 with {target-CSI} has a good match with the NW-side decoder model and it can be use for UE-side monitoring and UE-side root-cause determination.

Proposal 20: 	 Study mechanism for root-cause determination based on exchange of information regarding the NW-side trained encoder and/or decoder model.

Nvidia

Proposal 4: RAN1 to study post-deployment performance monitoring mechanisms to detect performance degradation and non-compliance to guarantee satisfactory performance of AI/ML-based CSI compression in the field.

Xiaomi

Proposal 15: NW-side monitoring model performance based on target CSI reported by UE and UE-side monitoring model performance based on precoded RS are studied with high priority.

Proposal 16: Considering the accuracy of monitoring and signalling overhead, identifying the cause of performance degradation based on UE-side performance monitoring is studied with low priority.

Proposal 17:  In order to identify the cause of the performance degradation, both target CSI and CSI feedback measured by UE are reported for NW-side performance monitoring.

Apple

Proposal 6: For CSI compression using two-sided model, for UE side performance, further study RLF/BFD like mechanism for UE initiated report.

Proposal 7: gNB can use UL SRS measurement to detect data drift due to channel statistics change.

Proposal 8: gNB can trigger root cause detection data collection when performance monitoring detects degradation, and data drift due to channel statistics is not observed. The dataset collection for root cause detection can reuse the immediate MDT with logged data, following current RAN2 agreement on data collection for training.

Proposal 9: Further study root cause detection due to data distribution mismatch due to UE side additional condition.

Panasonic

Observation 28: There are at least two purposes for performance monitoring. One is to check new untested model / parameter behavior. The other is to check current model / parameters are suitable to current environment.

Observation 29 If AI/ML models are trained by UE-side and AI/ML model update cycle is non-real time, UE vendor specific monitoring in offline sufficiently work.

Observation 30: If AI/ML models are trained by network side and AI/ML model update cycle is non-real time, A/B test can be used, and data collected for non-real time performance monitoring can also be used for model training.

Observation 31: Further study Direction 1 and Direction 3 with proxy model framework.

- Direction 1: Network-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side.

- Direction 3: UE-side monitoring based on the output of the CSI reconstruction model at the UE-side.

  - The CSI reconstruction part for performance monitoring at the UE is a proxy model, which is different from the actual CSI reconstruction part at the network.

Samsung

Proposal 10: For performance monitoring, consider causes of performance loss that particularly affect the AI/ML based approach.

- Consider KPI for monitoring such as KPImonitoring=1exp⁡1exp⁡SGCSAIML-SGCSbaseline, where SGCSAIML and SGCSbaseline  are the SGCS for AI/ML-based CSI and baseline CSI, e.g., eType-II, respectively.

- To evaluate the monitoring options, apply the evaluation mechanism agreed in Rel-18 by replacing intermediate KPI by KPImonitoring.

- FFS: baseline CSI for a payload size.

Proposal#11: For NW-side monitoring of two-sided models, when the input CSI is the (W2) domain, consider the same SD, FD and/or DD basis vectors for the CSI report for inference and monitoring purposes.

Proposal#12: To assess the accuracy of NW-side monitoring of two-sided models, when the input CSI is in the (W2) domain, KPIGenie  is calculated with ground-truth CSI of Float32 representation of the W2 matrices.

ETRI

Proposal 10: For the performance monitoring of CSI compression sub-use case using two-sided model, conclude that Case 2-1 is feasible with considerations of complexity, latency, and accuracy.

Fujistu

Proposal 22:

- For CSI compression using two-sided AI/ML models, the feasibility, reliability, and generalization capability of the UE-side AI/ML model performance monitoring using proxy model(s) should be evaluated and concluded before any further discussion on the related specification impacts.

Proposal 23:

- For the NW-side AI/ML model performance monitoring for CSI compression, RAN1 to prioritize the study of using the codebook-based quantization method to obtain the ground-truth CSI. Besides, adding new parameter values to legacy codebook for higher resolution ground-truth CSI should be studied.

Proposal 24:

- For CSI compression using two-sided AI/ML models, RAN1 to study the signaling and configuration for NW-side AI/ML model performance monitoring.

Proposal 25:

- For CSI compression using two-sided AI/ML models, RAN1 to study the signaling and configuration for NW-side AI/ML model performance monitoring.

- For CSI compression using two-sided AI/ML models, regarding the NW-side AI/ML model performance monitoring using an existing CSI feedback scheme as a reference, RAN1 to study the potential specification impacts for the following three options:

  - Option-1: UE selects and reports PMI to the NW.

  - Option-2: UE computes and reports the intermediate KPI for the reference scheme, e.g., the SGCS of the recovered CSI from PMI and the ground-truth CSI.

  - Option-3: NW selects the PMI based on the ground-truth CSI reported by a UE.

Proposal 26:

- For CSI compression using two-sided AI/ML models, RAN1 to study the procedures and signalling needed for the follow-up actions after the AI/ML model performance monitoring, including falling back to legacy codebook-based CSI reporting from AI/ML-based methods.

Proposal 27:

- For the performance monitoring of CSI compression using two-sided AI/ML models, RAN1 to study the potential specification impacts on monitoring the performance of an inactive AI/ML model, taking at least the following cases into consideration:

  - Initial activation of an AI/ML model.

  - Re-activation of an AI/ML model.

Nokia

Proposal 9: Add the following methods to the list of options considered for network-side performance monitoring of CSI compression:

- Direct estimation of an intermediate KPI based on encoded CSI messages

- Statistical monitoring based on pseudo-randomly subsampled elements of CSI information

Qualcomm Incorporated

Proposal 19:	Consider existing parameter-combinations of eType II codebook for ground-truth reporting of NW side monitoring.

Proposal 20: RAN1 to perform evaluation study for monitoring. accuracy requirement, the following methodology is proposed for the study:

- Generate a dataset A and develop an encoder-decoder pair E1D1 under this dataset A. Running inference on this dataset is considered to be the normal scenario where no underlying issue occurs.

  - A false-alarm event occurs if AvgSGCSmeasured,A<SGCSthr where AvgSGCSmeasured,A is the average SGCS over N samples on dataset A. The probability of false-alarm event pFA=ProbAvgSGCSmeasured,A<SGCSthr is defined as number of false-alarm events divided by the total number of samples in dataset A.

- Generate a dataset B from a different distribution than dataset A. Running inference on this dataset using E1D1 is considered to be the abnormal scenario where underlying issue (e.g., data drift) occurs.

  - A miss-detection event occurs if AvgSGCSmeasured,B≥SGCSthr where AvgSGCSmeasured,B is the average SGCS over N samples on dataset B using E1D1. The probability of miss-detection event pMD=ProbAvgSGCSmeasured,B≥SGCSthr is defined as number of miss-detections events divided by the total number of samples in dataset B.

- For various values of N, and for a given false-alarm probability requirement pFAreq, companies to report the detection probability pdetN=1-pMDN=ProbAvgSGCSmeasured,B<SGCSthr corresponding to each chosen N, where the SGCS threshold SGCSthrN for deriving the detection probability pdetNis determined as the value that achieves the pFAreq. Companies to also report the total feedback overhead over the N occasions.

- Companies to also report the required value of N that achieves pdetN≥pdetreq.

- Monitoring accuracy requirement can be derived based on the value N and the accuracy of the ground truth report (i.e., eType2, higher-resolution eType2) that provides satisfactory monitoring decision performance.

Proposal 21: Consider two-phase mechanism for performance monitoring and identification of root cause

- Phase 1: identifying whether there is performance degradation

- Phase 2: identifying whether the performance degradation is due to UE side CSI generation part, NW side CSI reconstruction part, or data drift, etc.

Proposal 22: Consider following model performance monitoring mechanisms

- NW side monitoring mechanism with ground-truth reporting for monitoring of performance degradation and monitoring root cause identification

    - Further study method of handling incurred overhead/ latency

- UE side monitoring mechanism

  - Phase 1 monitoring of performance degradation via SGCS estimator

  - Phase 2 monitoring of root cause identification with ground-truth reporting:

DCM

Proposal 3

- Option 2a for performance monitoring of Case 3 is necessary to support both joint and separate prediction and compression schemes.

- The Rel-18 doppler codebook-like report can be used for predicted future CSI for performance monitoring Option 1.

  - FFS the necessary to enhance the Rel-18 doppler codebook.

- For the data collection for monitoring for Option 2a, an enhanced Rel-18 doppler codebook is necessary to support the report of historical CSI on multiple time instances.

CEWiT

Proposal-10: In the case of selection of number of basis components, the knee point can be a good indicator.

Proposal-11: For model monitoring of AI/ML based CSI compression, reuse necessary signalling for transmitting the basis using legacy methods.

Proposal-12: For model monitoring of AI/ML based CSI compression, prioritize transmission of basis components based monitoring technique.

Proposal-13: For model monitoring of AI/ML based CSI compression, prioritize NW sided monitoring.

Proposal-16: Study root cause with respect to Option 3a-1 and Option 4-1.

## Discussion

Companies’ proposals are summarized as below. For NW side monitoring. Most companies propose same proposals as last meeting.

NW side monitoring

- Support of NW side monitoring

  - General supportive w/o explicit proposing new PC (9):

    - ZTE, CMCC, Spreadtrum, Tejas Networks, NEC, CATT, Panasonic, DCM, Xiaomi

  - Need high-resolution eT2 (4)

    - Huawei, Ericsson, Futurewei, Fujistu

  - Existing eT2 is sufficient (2)

    - QC, MTK

  - Study how to address overhead issue (QC, Apple, InterDigital)

- Mechanism / configuration / reporting

  - per-sample report or reporting of a number of samples (Huawei)

  - reporting W2 w/ same FD/SD bases as the CSI feedback (Samsung)

  - report quality of the ground-truth CSI whether it is invalid for monitoring (Google)

  - configure whether UE perform CSI prediction to report ground-truth (Google)

  - new report quantity, PMI only (Google)

  - report target CSI together or separate with the CSI feedback (Ericsson, CATT)

  - Signaling and configuration for event triggered and periodic data collection at NW side (Ericsson)

  - Two-phase monitoring procedure (Apple, QC)

  - L1 or RRC signaling (Ericsson, NEC)

  - L1 signaling (CATT)

- Supporting SRS-based performance monitoring (Apple, Google)

  - SRS used to detect data drift or statistic change (Apple)

  - Association between SNR and a CSI report (Google)

  - Burst SRS with frequency hopping for wideband channel for coverage-limited UE (Google)

UE side monitoring

- Methods

  - Based on Reference decoder output or actual decoder output if available at UE (6 vs. 3)

    - Support: Tejas Network, NEC, Futurewei (if available), Panasonic, ETRI, OPPO

    - Deprioirtize: Huawei, CATT

    - Concern: Further study generalization performance (Fujistu)

  - Based on SGCS estimator or monitoring output estimator (4 vs. 4)

    - Support: QC, NEC, vivo, OPPO

    - Deprioritize: Huawei (LCM complexity), CATT

    - Concern: further LCM complexity (Futurewei), generalization performance (Fujistu)

    - Further study: Ericsson

  - Based on precoded RS (5 vs. 1)

    - Support: Huawei, CATT, InterDigital, Google, Xiaomi (high priority)

    - Concern: Further study performance before discussing whether to support it (Futurewei)

  - Based on indication of reconstructed CSI (3)

    - Support: CMCC, Spreadtrum, Futurewei

  - None of them (1):

    - ZTE

- Enable testability (RAN4 and with monitoring mechanism) of the quality of UE reported monitoring metrics (Ericsson)

  - Via data collection at NW side includes ground-truth, monitoring metrics, CSI feedback

- Mechanism / configuration / reporting

  - Study signaling / mechanisms for UE reporting monitoring metrics (Ericsson)

  - Details of reporting mechanisms with time/event-triggered (InterDigital)

  - Appropriate monitoring metric (Ericsson, InterDigital, Google (hypo BLER))

  - Two-phase monitoring procedure (Apple, QC)

  - Reporting content/structure of UE side monitoring and associated overhead (InterDigital)

  - RLF/BFD like mechanism (Apple)

Root cause detection

- Achieved via NW side monitoring with ground-truth reporting (Ericsson, CATT, Xiaomi, QC, Apple)

  - Ground-truth reporting via MDT following current RAN2 agreement (Apple)

  - Two-phase monitoring procedure (Apple, QC)

    - Detect root cause when monitoring detects degradation (Apple)

    - ground-truth triggering based on SGCS reported by UE in performance monitoring phase (QC)

- Study UE side root cause detection (CATT, InterDigtial)

  - UE assistance information such as monitoring metrics and UE side OOD metrics (InterDigital)

  - Reporting UE-assessed error cause (InterDigital)

  - Mitigation mechanisms, including fallback / switching (InterDigital)

- Study whether and how to identify root cause of either decoder or data drift (CATT)

- Further study root cause identification due to mismatch of UE side additional condition (Apple)

- Study sharing test dataset to identify the root-cause (Lenovo)

- Study mechanism based on exchanging NW side encoder / decoder model (Lenovo)

- Low priority (Xiaomi)

Other aspects

- New monitoring metrics

  - Monitoring accuracy requirement (False alarm rate, miss-detection rate of average SGCS over N samples vs. threshold) (QC, OPPO, MTK)

  - Exponential of SGCS accuracy and float32 is considered for KPI_genie (Samsung)

- Study mechanism to determine the cause of low performance, due to deployed model, communication link, or data-drift of data. (Lenovo)

- Consider methodology and metric for rank > 1 (ZTE)

- For case 3, spec impact includes

  - configuration and report of target CSI corresponding to output of CSI prediction or corresponding to measured CSI of future slots. (CATT)

  - CSI-RS of measured CSI of future slots (CATT)

- For TSF case 3, option 2 with target CSI corresponding to measured CSI of future slots is supported (vivo)

- Follow-up actions, e.g., fallback to codebook-based scheme (Fujistu)

- Monitoring inactive models (Fujistu)

- Direct KPI estimation using encoded CSI messages as new option for NW side monitoring (Nokia)

- Statistical monitoring based on pseudo-randomly subsampled elements of CSI information (Nokia)

### Monitoring options

##### Proposal 71a: monitoring options

FL note: This is down-selection among the options agreed in RAN1 #117.

Further study following monitoring options in Rel-19, including the necessity, feasibility, and performance

- NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1)

- UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model

| Company | Comments |
| --- | --- |
| Lenovo | We suggest to keep the UE sided monitoring using proxy model or “local decoder” model and SGCS estimator. |
| Futurewei | We are ok with the proposal. |
| Huawei, HiSilicon | OK to further study the two options. |
| Panasonic | We share the same view as Lenovo. |
| TCL | Agree |
| CMCC | OK to further study the two options. |
| ZTE | Prefer NW-side monitoring. For UE-side monitoring based on precoded RS, additional storage burden for UE is needed since UE has to buffer the previous CSI. Besides, monitoring results may not adapt to the channel variation/aging/channel estimation errors due to the latency for precoded RS transmission. Moreover, UE-side monitoring based on precoded RS cannot reflect the model performance directly because the calculated metric will also be seriously impacted by the channel status and precoding strategy at the NW side. |
| NEC | We have the same view as Lenovo. |
| CATT | OK with current FL version |
| Fujitsu | Generally fine with the proposal. |
| OPPO | We do not agree. We have the same view to Lenovo that the UE-side monitoring with proxy model (Case 2-1) and SGCS estimater (Case 2-2) should be kept. We have evaluated the performance of Case 2-1 and it is also workable. |
| NTT DOCOMO | Agree |
| CEWiT | We also prefer NW-side monitoring. |
| ETRI | We also agree on Lenovo. |

### Monitoring accuracy requirement

During Rel-18 study, monitoring accuracy performance was evaluated. More specifically, SGCS accuracy is computed by averaging the per-sample difference between the genie SGCS and calculated SGCS based on ground-truth reporting or SGCS estimation, i.e., ESGCSgenie-SGCSmeasured. However, while monitoring accuracy performance was evaluated in the Rel-18 study, there was no study so far on the monitoring accuracy requirement, i.e., how accurate the monitoring metric (i.e., SGCS estimation accuracy) should be. This study is necessary to determine whether the accuracy of legacy eType2 is sufficient or enhanced accuracy is needed for NW-side monitoring based on target CSI reporting.

For example, Mediatek reported results showing that, the KPIactual (=SGCS estimate) based on PC8 of eType II tightly matches with KPIgenie. In other words, the accuracy of PC8 seems good enough for monitoring decisions, considering that the natural SGCS variations are much larger than the inaccuracy in the KPIactual of PC8.

Here is another plot from Mediatek comparing the KPIactual of PC8 and a higher resolution (PC9).

Visually, it appears that the increased resolution (PC9) may not help much in improving the monitoring decision making.

How do we study the accuracy requirement? Given that the main objective of performance monitoring is to identify if there is a problem such as data distribution drift or any faulty operation/issues and take appropriate actions such as model re-training or deactivation, the monitoring accuracy requirement may be derived based on the accuracy of the monitoring decision making such as miss-detection of underlying issues and falsely triggering actions.

##### Proposal 72a: monitoring accuracy requirement (revised from 62a last meeting)

Evaluate the monitoring accuracy requirement for NW-side and UE-side monitoring approaches.

The following evaluation study is suggested:

- Evaluate the monitoring accuracy and the number of required samples based on monitoring decision making performance. Use false alarm probability and missed detection probability in monitoring decision making as KPIs as a starting point.

- KPIFA =Prob(KPIActual < thr), where the AI/ML model#A is trained based on training dataset from Scenario#A/Configuration#A, and the evaluation is performed on the model#A and a dataset from the same Scenario#A/Configuration#A.

- KPIMD =Prob(KPIActual >= thr), where the AI/ML model#A is trained based on training dataset from Scenario#A/Configuration#A, and the evaluation is performed on either a different model than model#A (representing an erroneous model usage) or a different dataset than Scenario#A/Configuration#A (representing a data distribution drift).

- KPIActual is obtained based on N >= 1 samples.

  - In case of SGCS as KPI, KPIActual can be derived by averaging the SGCS estimate over N samples.

- For each choice of monitoring approach and resolution of ground truth reporting, companies to report the value of N and the total overhead that satisfies both the target false alarm rate and the target missed detection rate.

  - Target false-alarm rate, KPIFA < 0.05 as a baseline.

  - Target miss detection probability KPIMD < 0.1 as a baseline.

  - Companies determine thr appropriately to balance false alarm and miss detection.

- To also consider rank>1 with an appropriate metric for the given KPI.

FL note: Not to be confused with the false alarm and missed detection metrics for monitoring accuracy studied in Rel-18.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Regarding the FL note “This study is necessary to determine whether the accuracy of legacy eType2 is sufficient or enhanced accuracy is needed for NW-side monitoring based on target CSI reporting”, let’s consider an extreme example, where we adopt legacy eT2 as the label fed back by UE, and NW compares with both AI (to derive AI accuracy) and non-AI (to derive non-AI accuracy) - AI is adopted if AI accuracy is higher than non-AI accuracy while non-AI is adopted otherwise. However, since the non-AI benchmark is also subject to legacy eT2, which equals to the eT2 CB of the label, from monitoring results at NW, non-AI benchmark accuracy is always 100%, and there is never a need to enable the AI solution, which is unreasonable. Regarding the definition of false alarm and miss detection, they should not be defined as the absolute value of the actual KPI, but rather the bias between actual KPI and genie-aided KPI. E.g., under Option 1, if KPIDiff > KPIth 1, where KPIDiff is positive value, it means miss detection; if KPIDiff < -KPIth 1, where KPIDiff is negative value, it means false alarm. For evaluating the performance of a monitoring method under different generalization cases, we should consider using KPIGenie to assess how large gap the actual KPI biases from the genie-aided KPI. From the proposed formula, however, we can only say the actual KPI is calculated as high value (KPIActual > thr) or low value (KPIActual < thr), but cannot tell whether this value is confident or not. |
| ZTE | To our understanding, there is no need to further make a new definition or formula since the Option 2 for performance monitoring captured in TR can already reflect FA and MD and we can just reuse it. For example, KPIDiff = (KPIActual > KPIth 2, KPIGenie < KPIth 3) can be used to evaluate MD and KPIDiff = (KPIActual < KPIth 2, KPIGenie > KPIth 3) can be applied to FA. In addition, absolute KPI value can also reflect the actual SGCS performance unlike a gap value for Option 1, which cannot be observed whether the current model performance is failed, e.g., SGCS is very low. In addition, the intention of this proposal is to evaluate the performance of a monitoring method under different generalization cases, so we can consider using KPIGenie to assess the metric of FA and MD, at least based on Option 2. KPIDiff = f ( KPIActual , KPIGenie ) can take the following forms: - Option 1 (baseline for calibration): Gap between KPIActual and KPIGenie, i.e. KPIDiff = (KPIActual - KPIGenie); Monitoring accuracy is the percentage of samples for which ¦ KPIDiff¦ < KPIth 1, where KPIth 1 is a threshold of the intermediate KPI gap which can take the following values: 0.02, 0.05 and 0.1. - Option 2 (optional and up to companies to report): Binary state where KPIActual and KPIGenie, have different relationships to their threshold(s), i.e., KPIDiff = (KPIActual > KPIth 2, KPIGenie < KPIth 3) OR (KPIActual < KPIth 2, KPIGenie > KPIth 3), where KPIth 2 is considered to be the same as KPIth 3. Monitoring accuracy is the percentage of samples for which KPIDiff = 0.. |
| OPPO | We agree to discuss the FA and MD monitoring accuracy metric. But as per our understanding, the false alarm and mis-detection for UE-side Case 2-1/2-3 monitoring should be defined as follows: - False alarm: PKPIGenie≥KPIth∨KPIActual<KPIth. This conditional probability means false alarm happens. When UE monitors the performance loss under KPIActual<KPIth, but the performance of NW-side decoder still works well with KPIGenie≥KPIth. Based on the UE-side monitoring result, UE may trigger some unnecessary model LCM events, such as model switching/updating/fallback, which actually lead to the false alarm. Considering the probability of KPIActual<KPIth, the final false alarm rate should be PFA=PKPIActual<KPIth*PKPIGenie≥KPIth∨KPIActual<KPIth - Mis-detection rate: PKPIGenie<KPIth∨KPIActual≥KPIth. This conditional probability means mis-detection happens. When UE does not monitor the performance loss under KPIActual≥KPIth, but the performance of NW-side decoder is not good enough with KPIGenie<KPIth. Based on the UE-side monitoring result, necessary model LCM event may be missed, which actually cause the mis-detection. Considering the probability of KPIActual≥KPIth, the final mis-detection rate should be: PMD=PKPIActual≥KPIth*PKPIGenie<KPIth∨KPIActual≥KPIth |
| Mod | @HW, @ZTE, @OPPO: The false alarm and miss detection you listed (from Rel-18 study) are monitoring accuracy metrics – they just measure how accurate the KPI_actual is (by comparing against KPI_genie) with some thresholding. They have nothing to do with how much accuracy is desired (i.e., monitoring accuracy requirement) On the other hand, what’s being proposed is to study the monitoring accuracy requirement, by evaluating its impact on the monitoring decision performance. The proposed FA and MD metrics are not about accuracy of the KPI_actual w.r.t. KPI_genie but about the accuracy of monitoring decision making. Quite clearly, monitoring decisions can’t rely on KPI_genie, as KPI_genie is not available. |

### Monitoring metrics

##### Proposal 73a: monitoring accuracy KPI for UE side monitoring using precoded RS (proposal 63a from last meeting)

For UE side monitoring based on precoded RS, evaluate monitoring accuracy KPI (i.e., (KPIActual and KPIActual) based on precoding gain, i.e.,KPIActual=HActual⋅v2HActualF2 and KPIGenie=HGenie⋅v2HGenieF2.

- Companies should report the temporal domain relationship between HGenie and HActual, e.g., doppler, and the gap in terms of milliseconds.

- Other KPIs are not precluded.

- Note: KPIGenie is unknown in reality and is only for KPI accuracy evaluation purpose. The actual monitoring decision should be made without relying on KPIGenie.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | This formula only provides the absolute value of the precoding gain, but can not provide guidance on how to make selection over AI vs non-AI CSI compression solutions. It would be simpler if we directly compare the accuracy of the precoder? E.g., KPIactual is derived by calculating the SGCS between Vactual (subject to recovery CSI) with Vgenie (decomposed from Hgenie). We can generate KPIactual for AI and legacy eT2, and compare their accuracy. |
| ZTE | We can first discuss the Proposal 71a and verify the necessity of studying this method. Then, we can further discuss the performance monitoring metric in detail if necessary. |
| NTT DOCOMO | We would like to clarify the following issue. In KPI_Actual, the two H_Actual in the numerator and denominator may be different ones, depending on the detailed transmission timing of the RSs. For example, the H_Actual in the denominator corresponds to the time instance of UE reports, and the H_Actual in the numerator corresponds to the time instance when the RS for monitoring is transmitted. These two channels may be different ones. We firstly would like to clarify the understanding, and then the impacts of such time mismatch should be studied. |
| Samsung | The direction is fine. However, we suggest to explicitly define H_Genie and H_Actual. Moreover, we suggest to put the precoding gain as calculated in the above as just one example of the precoded CSI-RS based monitoring. A more refined metric that takes multiple layers in to account could also be considered. |

##### Proposal 73b: monitoring accuracy KPI for UE side monitoring using precoded RS

For UE side monitoring based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model, evaluate monitoring accuracy KPI, following the methodologies used in Rel-18 monitoring accuracy evaluations as captured in the TR,

Relevant section from the TR (for reference) Step 1: Generate test dataset including K test samples. - Step 2: For each of the K test samples, a bias factor of monitored intermediate KPI (KPIDiff) is calculated as a function of KPIDiff = f ( KPIActual , KPIGenie ), where KPIActual is the actual intermediate KPI, and KPIGenie is the genie-aided intermediate KPI. - KPIDiff is considered for: - Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar, R16 eType II-like method, etc.) or SRS measurements, where - KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format or SRS measurements. - KPIGenie is calculated with output CSI (as for KPIActual) and the ground-truth CSI of Float32 - Note: if Float32 is used for KPIActual, the monitoring accuracy is 100% if KPIActual and KPIGenie are based on the same CSI sample. - Case 2: UE side monitoring of intermediate KPI with a proxy model, where the monitoring accuracy is evaluated for the output of the proxy model at UE: - Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated based on the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI. Note: if the proxy CSI reconstruction model is the same as the actual CSI reconstruction model at the NW, the monitoring accuracy is 100%. - Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual) - KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI. - KPIDiff = f ( KPIActual , KPIGenie ) can take the following forms: - Option 1 (baseline for calibration): Gap between KPIActual and KPIGenie, i.e. KPIDiff = (KPIActual - KPIGenie); Monitoring accuracy is the percentage of samples for which ¦ KPIDiff¦ < KPIth 1, where KPIth 1 is a threshold of the intermediate KPI gap which can take the following values: 0.02, 0.05 and 0.1. - Option 2 (optional and up to companies to report): Binary state where KPIActual and KPIGenie, have different relationships to their threshold(s), i.e., KPIDiff = (KPIActual > KPIth 2, KPIGenie < KPIth 3) OR (KPIActual < KPIth 2, KPIGenie > KPIth 3), where KPIth 2 is considered to be the same as KPIth 3. Monitoring accuracy is the percentage of samples for which KPIDiff = 0. - Step 3: Calculate the statistical result of the KPIDiff over K test samples which represents the monitoring accuracy performance. - Note: KPIGenie is introduced for the evaluation and comparison purpose; it may not be available in the real network. - Note: the complexity, overhead and latency of the monitoring scheme are to be reported.

and applying the following modifications:

KPIDiff is considered for

- Alt 1: based on precoding gain, i.e.,KPIActual=HActual⋅v2 and KPIGenie=HGenie⋅v2, each averaged over subbands.

  - HActual is the channel matrix when precoded RS is transmitted. HGenie is the channel matrix when CSI is measured and compressed.

    - Companies should report the temporal domain relationship between HGenie and HActual, e.g., doppler, and the gap in terms of milliseconds.

      - Baseline values: 3km/h, 5msec

  - v is the target CSI before compressed at the UE side, v is the reconstructed CSI.

  - KPIActual is the effective channel gain measured from the precoded RS. KPIGenie is the precoding gain of the target CSI on its measurement resource.

    - Note: Companies can also consider measurement error of the effective channel of the precoded RS when calculating KPIActual.

  - FFS: extension to rank>1

- Other alternatives are not precluded.

KPIDiff = f ( KPIActual , KPIGenie ) can take the following forms:

- Option 1 (baseline for calibration): ratio between KPIActual and KPIGenie, i.e. KPIDiff = KPIActual / KPIGenie; Monitoring accuracy is the percentage of samples for which 1-KPIth 1 < | KPIDiff| < 1+KPIth 1, where KPIth 1 is a threshold of the KPI ratio which can take the following values: 0.05, 0.1 and 0.2

- Other options are not precluded.

| Company | Comments |
| --- | --- |
| Yuan | HActual=HGenie⋅W =H13*32⋅w32*32⋅X32 =H13*32⋅w32*32⋅v13*322 =H13*32⋅w32*4⋅v13*322 =HGenie⋅W ⋅v =HGenie ⋅v =H13*32⋅v13*32 |
| Samsung | Thanks FL for the clarification. First, the above definition of H_actual and H_genie, it may not be applicable to Case 3, i.e., for prediction cases H_genie is the channel at prediction instance. Thus, we suggest to limit the proposal either to Case 0 or Case 0/2. Moreover, what we need to evaluate here is the accuracy of the monitoring method in terms of how well it captures reconstruction accuracy/error, e.g., whether the KPI scales up with the reconstruction accuracy/error. Based on the above formulation, KPIDiff = KPIActual / KPIGenie , deviates from value of ‘1’ whenever reconstruction error is large or there gap between HGenie and HActual. We suggest to fix the measurement gap between measurement for inference and measurement for monitoring and redefine the two KPIs as below (unisdering UMa indoor : outdoor 0.8:0.2 (3kmphr:30Kmphr). This way we can identify how much impact the change on HGenie to HActual would impact the monitoring accuracy. KPIGenie=HGenie⋅v2-HGenie⋅v2 KPIActual=HGenie⋅v2-HActual⋅v2 |

##### Proposal 73c: monitoring accuracy KPI for UE side monitoring using precoded RS

For UE side monitoring based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model, evaluate monitoring accuracy KPI, following the methodologies used in Rel-18 monitoring accuracy evaluations as captured in the TR,

Relevant section from the TR (for reference) Step 1: Generate test dataset including K test samples. - Step 2: For each of the K test samples, a bias factor of monitored intermediate KPI (KPIDiff) is calculated as a function of KPIDiff = f ( KPIActual , KPIGenie ), where KPIActual is the actual intermediate KPI, and KPIGenie is the genie-aided intermediate KPI. - KPIDiff is considered for: - Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar, R16 eType II-like method, etc.) or SRS measurements, where - KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format or SRS measurements. - KPIGenie is calculated with output CSI (as for KPIActual) and the ground-truth CSI of Float32 - Note: if Float32 is used for KPIActual, the monitoring accuracy is 100% if KPIActual and KPIGenie are based on the same CSI sample. - Case 2: UE side monitoring of intermediate KPI with a proxy model, where the monitoring accuracy is evaluated for the output of the proxy model at UE: - Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated based on the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI. Note: if the proxy CSI reconstruction model is the same as the actual CSI reconstruction model at the NW, the monitoring accuracy is 100%. - Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual) - KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI. - KPIDiff = f ( KPIActual , KPIGenie ) can take the following forms: - Option 1 (baseline for calibration): Gap between KPIActual and KPIGenie, i.e. KPIDiff = (KPIActual - KPIGenie); Monitoring accuracy is the percentage of samples for which ¦ KPIDiff¦ < KPIth 1, where KPIth 1 is a threshold of the intermediate KPI gap which can take the following values: 0.02, 0.05 and 0.1. - Option 2 (optional and up to companies to report): Binary state where KPIActual and KPIGenie, have different relationships to their threshold(s), i.e., KPIDiff = (KPIActual > KPIth 2, KPIGenie < KPIth 3) OR (KPIActual < KPIth 2, KPIGenie > KPIth 3), where KPIth 2 is considered to be the same as KPIth 3. Monitoring accuracy is the percentage of samples for which KPIDiff = 0. - Step 3: Calculate the statistical result of the KPIDiff over K test samples which represents the monitoring accuracy performance. - Note: KPIGenie is introduced for the evaluation and comparison purpose; it may not be available in the real network. - Note: the complexity, overhead and latency of the monitoring scheme are to be reported.

and applying the following modifications:

KPIDiff is considered for

- Alt 1: based on precoding gain, i.e.,KPIActual=HActual⋅v2 and KPIGenie=HGenie⋅v2 KPIGenie=HActual⋅v2, each averaged over subbands.

  - HActual is the channel matrix when precoded RS is transmitted. HGenie is the channel matrix when CSI is measured and compressed.

    - Companies should report the temporal domain relationship between HGenie and HActual, e.g., doppler, and the gap in terms of milliseconds.

      - Baseline values: 3km/h, 5msec

  - v is the target CSI before compressed at the UE side, v is the reconstructed CSI.

  - KPIActual is the effective channel gain measured from the precoded RS. KPIGenie is the precoding gain of the target CSI on its measurement resource.

    - Note: Companies can also consider measurement error of the effective channel of the precoded RS when calculating KPIActual.

  - FFS: extension to rank>1

- Other alternatives are not precluded.

KPIDiff = f ( KPIActual , KPIGenie ) can take the following forms:

- Option 1 (baseline for calibration): ratio between KPIActual and KPIGenie, i.e. KPIDiff = KPIActual / KPIGenie; Monitoring accuracy is the percentage of samples for which 1-KPIth 1 < | KPIDiff| < 1+KPIth 1, where KPIth 1 is a threshold of the KPI ratio which can take the following values: 0.05, 0.1 and 0.2

- Other options are not precluded.

| Company | Comments |
| --- | --- |
| Xiaomi | We are fine that the proposal is used to evaluate the performance monitoring accuracy of precoded CSI-RS based on recovered CSI. HGenie is not needed. v is the target CSI before compressed at the UE side when spatial-frequency domain precoding vector is adopted. - Alt 1: based on precoding gain, i.e.,KPIActual=HActual⋅v2 and KPIGenie=HGenie⋅v2 KPIGenie=HActual⋅v2, each averaged over subbands. - HActual is the channel matrix when precoded RS is transmitted. HGenie is the channel matrix when CSI is measured and compressed. - Companies should report the temporal domain relationship between HGenie and HActual, e.g., doppler, and the gap in terms of milliseconds. - Baseline values: 3km/h, 5msec - v is the target CSI before compressed at the UE side when spatial-frequency domain precoding vector is adopted, v is the reconstructed CSI. In addition, note that the prospoal could not provide guidance on how to make selection as commended by HW. In order to address the issue, we prefer to calculating KPIreference=HActual⋅vcb2, where vcb is the precoder calculated by using legacy codebook, such as eType II codebook. Then, KPIreference and KPIActual are compared to make decision. |
| Mod | Thanks Xiaomi for the suggestion. Your suggestion seems reasonable, but one question I have is what should be the evaluation metric for monitoring accuracy? |

### Root cause detection

##### Proposal 74a: root cause (proposal 64b from RAN1 #119)

For NW-first training, in inter-vendor training collaboration options 3 and 4, conclude that root cause identification can be achieved at least by ground-truth CSI reporting

- e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

| Company | Comments |
| --- | --- |
| Lenovo | We suggest to have an agreement that include possibility of both root cause identification at the UE-side and NW-side. |
| Huawei, HiSilicon | OK |
| CMCC | Support. |
| ZTE | Fine with the proposal. |
| LG | Fine |
| Fujitsu | Fine with the proposal. |
| CEWiT | OK with the proposal. |

##### Proposal 74b: root cause

For NW-first training, in inter-vendor training collaboration options 3 and 4, conclude that root cause identification can be achieved at least by the following ground-truth CSI reporting

- Data distribution mismatch can be identified by the UE side using target CSI or by the NW side using reported target CSI for training.

- If data distribution mismatch is NOT detected by the NW side and the UE side, additional target CSI and CSI feedback pair report can be used, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

| Company | Comments |
| --- | --- |

##### Conclusion 74c: root cause

For NW-first training, in inter-vendor training collaboration options 3 and 4 Direction A and C, conclude that root cause identification can be achieved at least by the following ground-truth CSI reporting

- Data distribution mismatch can be identified by the UE side using target CSI or by the NW side using reported target CSI for training.

- If data distribution mismatch is NOT detected by the NW side and the UE side, Via additional target CSI and CSI feedback pair report can be used, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

| Company | Comments |
| --- | --- |

##### Conclusion

For NW-first training, in inter-vendor training collaboration Direction A and C, identification of root cause for performance issues can be achieved at least by the following

- Data distribution mismatch that is identified by either NW side or UE side

- Via target CSI and CSI feedback pair report, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |

# Inference aspects (CQI / quantization / CSI framework / config)

## Summary of company proposals

## Discussion

The summary of companies proposals are presented below.

# Complexity performance trade-off

## Summary of company proposals

FUTUREWEI

Proposal 2: For CSI compression using 2-sided model, regarding the trade-off between performance and complexity/overhead, consider adopting model structure with less complexity to reduce both space and computational overhead, at least for encoder model at UE-side.

Ericsson

Proposal 17	Further study methods to reduce the AI computational complexity for AI CSI compression Case 3.

Proposal 18	Deprioritize AI CSI compression use case 2.

LGE

Proposal #10: Study on model complexity reduction method, e.g., knowledge distillation, to further reduce the CSI training/signaling complexity and issue on model alignment for Type 3 training collaboration.

Samsung

Proposal#6: Study the impact of input pre-processing (dimensionality reduction) on performance, model complexity and model generalizability.

Observation#7: In Angle-delay (W2)-domain AI/ML CSI compression, for small payload size (paramCombination-r16  1), ‘Heavy’ and ‘light’ models show comparable SGCS gain over Rel-16 eType II CSI.

Observation#8: In Angle-delay (W2)-domain AI/ML CSI compression, for small payload size (paramCombination-r16 =1), increasing the number of basis vectors (increasing the dimension of the W2 matrix), e.g., paramCombination-r16=3, improves performance.

Proposal#7: In Angle-delay (W2)-domain CSI compression, study the impact of the number of SD/FD basis vectors for performance-complexity tradeoff.

Observation#17: For the transformer model in the spatial-frequency-domain compression, increase on both the model dimension and the number of attention blocks tends to have a monotonic but diminishing performance gain at the expense of increased model size and complexity.

Observation#18: For the transformer model in the spatial-frequency-domain compression, for a given model dimension, increasing the number of attention blocks does not always guarantee performance gain.

Proposal#21: For the transformer model in the spatial-frequency-domain compression, consider balanced model hyper-parameters selection based on the tradeoff between model performance and complexity/size.

Qualcomm Incorporated

Proposal 27:	Complexity reduction is possible in the Transformer-based design with spatial-frequency domain input.

Proposal 28:	Complexity reduction in spatial-frequency domain should be prioritized as it has better complexity/performance tradeoff than using angular-delay domain as input.

IIT Kanpur

Observation 6: The lightweight model exhibits a ~21% NMSE loss and a 3-4% SGCS loss compared to the base model. However, it achieves an 87-88% reduction in FLOP count and a 70-80% reduction in parameter count relative to the base model for Case 0 and Case 2, respectively.

Observation 7: The Transformer architecture exploits the similarity/correlation between tokens through its self-attention mechanism, enabling it to capture contextual relationships and dependencies effectively. The spatial-frequency input or the angular-delay domain input could serve as a more suitable input for the Transformer model, as these representations exhibit stronger correlations across antennas and subcarriers. The Transformer model may be better suited to encode and decode these structured relationships efficiently.

## Discussion

Below, the FL captured the SGCS gain vs. complexity (combined FLOPs for CSI generation part and the CSI reconstruction part) for Case 0 based on companies’ submissions in the Results Spreadsheet.

From the figure, we observe that larger NN models do not necessarily provide larger gains. For example, a model of size around 20MFlops seems to achieve most of the gain. This implies that some of the AI/ML models used in the evaluation study may not have been optimized in terms of complexity that that lower complexity models may achieve similar performance.

Also, it is important to note that the actual AI/ML model complexity depends on various platform-dependent choices for modem implementation. The values reported here should be considered as representative values and not as a precise complexity estimate for implementation of AI/ML-based CSI compression.

Still in this meeting, there are not enough results comparing the performance and complexity trade-off of the angular-delay domain and the spatial-frequency domain approaches, because most companies chose either approach but not both. Companies are encouraged to study this aspect and bring results and discussions in the next meeting.

##### Proposal 91a: complexity-performance trade-off (continuation from 81a of RAN1 #119)

Study performance-complexity trade-off by comparing different AI/ML models, by optimizing existing designs, and by comparing different precoder representation in (spatial-frequency and angle-delay) or (spatial-frequency-time and angle-delay-doppler), by considering the following aspects

- Performance comparison between different AI/ML designs and benchmark schemes

- Complexity numbers (FLOP, calculated/projected latency or power consumption if available, measured latency or power consumption if available) of different AI/ML designs and benchmark schemes

FL Note: FL plans to introduce new, dedicated, spreadsheets to collect complexity-performance numbers from companies, at least for Case 0, and the results will be captured into observations and plots in the RAN1 #120-bis, updating the “Performance vs. complexity” plots agreed in RAN1 #119.

| Company | Comments |
| --- | --- |
| Futurewei | We are fine with the proposal. |
| CATT | OK |
| Fujitsu | OK |
| NTT DOCOMO | Agree |

# Other topics (work plan, new use cases)

## Discussion

### Others

Please provide any other comments for this section.

| Company | Comments |
| --- | --- |

# Proposals for online sessions

## Monday

##### Proposal 74a: root cause (proposal 64b from RAN1 #119)

For NW-first training, in inter-vendor training collaboration options 3 and 4, conclude that root cause identification can be achieved at least by ground-truth CSI reporting

- e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

##### Proposal 22b: UCI loss

Conclude that, under 10% UCI loss,  Case 2 shows small to large performance drop (by Comparing Scenario B and Scenario A). Signaling support to mitigate the UCI loss can improve the performance and helps preserving the gain of Case 2 over Case 0 (by Comparing Scenario C and Scenario A and benchmark schemes).

Consider the following approaches for signaling support for mitigating the impact of UCI loss

- NW-signaling to reset of historical CSI information at UE

- FFS: NW-triggered CSI retransmission

##### Proposal 404b: Direction C performance

Conclude that

- Fully specified models in Direction C, when employed directly, may have limited performance in the field compared to Direction A.

- The performance of fully specified models in Direction C may be improved at least if either side train their part of model using field data, yet still worse than the achievable performance of Direction A.

##### Conclusion 402a: addressing NW / UE data mismatch (Continuation from 42b of RAN1 #119)

Conclude that, for direction A, performance impact, if any, due to NW / UE data distribution mismatch with respect to UE side additional condition can be addressed at least by

- Timely NW-side data collection from existing UEs and sharing the updated dataset and/or model parameters to UE-side, followed by UE-side offline engineering to train UE-side encoder.

- UE-side offline engineering to train UE-side encoder, without triggering a new dataset and/or model parameter sharing to UE-side.

##### Proposal 407b: Pairing ID

For inter-vendor-collaboration In Options 3a-1 and 4-1, a pairing ID is shared as additional information along with the exchanged dataset or the model parameters can be associated with an ID.

- FFS: The same ID can be used for NW to collect data for NW-side training

- The same ID can be used for UE to collect data for UE-side training

- The same ID can be used for applicability inquiry and reporting

- The same ID can be used for inference configuration

- FFS: The same ID can be used for monitoring configuration

- Note: to use the name pairing ID is for discussion purposes.

##### Proposal 408b: Performance target for Direction A

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm that performance target is shared as additional information along with the exchanged dataset or the model parameters.

- FFS: type of performance metric, e.g., SGCS, NMSE

- FFS: input data for deriving the performance target

## Tuesday Morning

##### Proposal 408c: Performance target for Direction A

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm that performance target is shared as additional information along with the exchanged dataset or the model parameters.

- FFS: type of performance metric, e.g., SGCS, NMSE, SGCS statistics

- FFS: input data for deriving evaluating the performance target

##### Proposal 405b: way forward for Directions

- Direction C with standardized model ensures a minimum performance requirement.

  - Standardized model is specified in RAN4

  - The fully specified reference model for Direction C is the same as the specified model for RAN4 performance testing.

  - RAN1 assumes the NW-side and UE-side may either implement the RAN4 model, a model equivalent to the RAN4 model, or a model finetuned with field data while being compatible to the RAN4 model.

- Direction A with standardized model parameter / dataset exchange from NW-side to UE-side may achieves better performance.

##### Conclusion 74c: root cause

For NW-first training, in inter-vendor training collaboration options 3 and 4 Direction A and C, conclude that root cause identification can be achieved at least by the following ground-truth CSI reporting

- Data distribution mismatch can be identified by the UE side using target CSI or by the NW side using reported target CSI for training.

- If data distribution mismatch is NOT detected by the NW side and the UE side, Via additional target CSI and CSI feedback pair report can be used, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

##### Proposal 404b: Direction C performance

Conclude that

- Fully specified models in Direction C, when employed directly, may have limited performance in the field compared to Direction A.

- The performance of fully specified models in Direction C may be improved at least if either side train their part of model using field data, yet still worse than the achievable performance of Direction A.

##### Proposal 51b: Model structure study for Case 0

Please find FL replies inline in the above company comments table.

For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure

Encoder description:

| Embedding layer | Number of tokens: Nsubband, up to Nsubband,max Feature dimension of each token: 2*NumPorts Output dimension dmodel per token |
| --- | --- |
| Positional encoding | Trailable parameters for each token position Number of tokens positions: Nsubband,max Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead= dmodel / Nhead Dimension of latent space inside feedforward module dFF Activation function choice An optional dropout parameter dropout for regularization |
| Output linear layer | Reshape the matrix from (Nsubband x dmodel) to a vector of dimension (Nsubband * dmodel) Output dimension zdim |
| Vector quantization | Scalar quantization: 2 bits Vector quantization - segment size = 2 - bits per segment = 4 bits (2 bits per output dimension) Total payload size = 2*zdim |

Decoder description:

The decoder has a mirroring design as the encoder.

## Tuesday afternoon

##### Proposal 51c: Model structure study for Case 0

For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure

Encoder description:

| Embedding layer | Number of tokens: Nsubband, up to Nsubband,max Feature dimension of each token: 2*NumPorts Output dimension dmodel per token |
| --- | --- |
| Positional encoding | Trailable parameters for each token position Number of tokens positions: Nsubband,max Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead= dmodel / Nhead Dimension of latent space inside feedforward module dFF Activation function choice An optional dropout parameter dropout for regularization |
| Output linear layer | Reshape the matrix from (Nsubband x dmodel) to a vector of dimension (Nsubband * dmodel) Output dimension zdim |
| Vector quantization | Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg Total payload size = Nbit * Zdim |

Decoder description:

The decoder has a mirroring design as the encoder. Details are to be discussed.

##### Proposal 52a: hyper-parameters for model structure study

- Down-selection from following candidate hyper-parameter combinations

  - Alt1: NTF,dmodel,Nhead,dhead,dFF={4,256,8,32,1024}

  - Alt2: NTF,dmodel,Nhead,dhead,dFF={4,128,8,16,512}

  - Alt3: NTF,dmodel,Nhead,dhead,dFF={6,64,8,8,256}

  - Alt4: NTF,dmodel,Nhead,dhead,dFF={2,128,8,16,512}

  - Other alternatives are not precluded

- For activation function, down-selection from ReLU or GeLU

##### Proposal 53a: model structure scalability

- For scalability over Tx port configuration, down-selection from

  - Alt1: specific linear embedding layer for each Tx port configuration

  - Alt2: a common linear embedding layer with zero-padding

    - FFS how to perform padding

  - Note: Alt1 and Alt2 can be used in combination.

- For scalability over subband configuration,

  - Alt1: positional embedding specific to each subband index

    - Nsubband tokens out of Nsubband,max token positions are used as input.

    - Zero-padding for inactive subbands positions at the output of transformer blocks prior to output linear layer.

  - Alt2: Zero-padding at the input

- For scalability over payload configurations,

  - Alt1: specific output linear layer for each payload configuration

  - Alt2: truncation/masking of the output linear layer output

## Thursday

##### Proposal 51e: Model structure study for Case 0

FL notes: changes include

- Use different value of N_TF for encoder and decoder

- Rename the two “linear layer”s for decoder into “input linear layer” and “output linear layer”

##### Proposal 53c: model structure scalability

For model structure scalability study for temporal domain Case 0,

- For the choice of token dimension and feature dimension,

  - Alt 1: Use subband as the token dimension and Tx port as a feature dimension

    - The number of tokens varies with the number of subbands.

  - Alt 2: Use Tx port as the token dimension and subband as a feature dimension

    - The number of tokens varies with the number of Tx ports.

  - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens.

    - The number of tokens varies with the number of Tx ports and the number of subbands.

- For scalability over Tx port configuration the feature dimension,

  - Alt1: specific linear embedding layer for each Tx port configuration feature size

  - Alt2: a common linear embedding layer with zero-padding (e.g., zero-padding or other techniques for padding values)

    - FFS how to perform padding

  - Alt 3: define a fixed-size atomic unit of over Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. The number of tokens varies with number of Tx ports and number of subbands.

  - Note: Alt1 and Alt2 can be used in combination.

- For scalability over subband configuration the token dimension,

  - Alt1: positional embedding specific to each subband token index

    - Nsubband Ntoken,active tokens out of Nsubband,max Ntoken token positions are used as input.

    - Zero-padding for inactive subbands positions at the output of transformer blocks prior to output linear layer.

  - Alt2: Zero-padding at the input

  - Alt 3: define a fixed-size atomic unit of over Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. The number of tokens varies with number of Tx ports and number of subbands.

- For scalability over payload configurations,

  - Alt1: specific output linear layer for each payload configuration

  - Alt2: truncation/masking of the output linear layer output

  - Alt3: by varying quantization parameters

- Notes

  - Other Alternatives are not precluded.

  - Different Alternatives may be used in combination.

  - Same/similar approach is applied at the decoder side.

- Evaluations to consider:

  - Case 1 (scalable structure): Scalable model structure described above

    - Using one common model structure, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth}.

    - Companies to report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports)

  - Case 2 (dedicated structure): Model structure based on the Case 1 model structure, where the input and the output dimensions are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations.

    - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths.

  - For each scalable model structure choice, companies to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}.

##### Proposal 401d: Observations from X7

Capture the following observations into TR:

For the evaluation of performance impact due to UE-side / NW-side data distribution mismatch with respect to UE-side additional condition (issue 4 and 6),

When dataset A include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 1 source [Panasonic] observes minor performance loss (-0.22% ~ -1.09%) relative to case 1 with NW side target CSI sharing.

When dataset A include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing,

- 1 2 source [ZTE,vivo] observes minor performance loss (-0.67 -0.006% ~ -0.97%) related to case 1 without NW side target CSI sharing.

- 1 source [OPPO] observes moderate performance loss (-3.3% ~ -4%) relative to case 1 without NW side target CSI sharing

When dataset A include dataset B, for case 3 (Direction B),

- 1 2 source [ZTE,vivo] observes minor performance loss  (-0.6% ~ -0.9%  -0.9% ~ +0.01%) relative to case 1.

When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training.

- 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training.

- 1 2 source [Ericsson, ZTE] observe moderate performance loss of (-3.8 ~ -8.3%) relative to case 1 for Alt1 training.

- 1 source [OPPO] observe significant performance loss (-51% ~ -62.5%) relative to case 1 for Alt1 UE training.

- 2 sources [CATT, Futurewei] observe minor performance loss (-1.62% -2.78%  ~ -3.2%) relative to case 1 for Alt2 UE training

- 1 source [Apple] observe moderate loss of -6.7% relative to case 1 for Alt2 UE training.

When dataset A does not include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing

- 4 5 sources [CATT, ZTE, Xiaomi, ETRI, vivo] observe minor to moderate performance loss (-0.4% ~ -3.9%) relative case 1.

- 2 source [QC, Ericsson] observe moderate performance loss (-6.7% ~ -8.6%) relative to case 1.

- 1 source [OPPO] observes significant performance loss of -62.1% relative to case 1.

When dataset A does not include dataset B, for case 2, option 4-1,

- 4 sources [QC, Apple, Xiaomi, ETRI] observe zero to minor performance loss (-2.4% ~ 0%) relative to case 1 for Alt1 UE training.

- 1 source [Ericsson] observe minor to moderate performance loss (-2.9% ~ -7.9%) relative to case 1 for Alt1 UE training depending on whether dataset A applies augmentation using various phase normalization methods.

- 4 sources [CATT, Xiaomi, Futurewei, ETRI] observe minor performance loss (-1.41% ~ -3.52%) relative to case 1 for Alt2 UE training.

- 1 source [Apple] observes moderate loss of -7.9% relative to case 1 for Alt2 UE training.

- 1 source [vivo] observes moderate loss of -7.7% relative to case 1 for 3a-1 for Alt1 UE training when backbone are different.

When dataset A does not include dataset B, for case 3, Direction B,

- 4 sources [CATT, vivo, ZTE, ETRI] observe minor loss to positive gain (-3.7% ~ 1%) relative to case 1.

- 1 source [QC] observes significant loss (-20%) relative to case 1.

##### Proposal 403d: Observations from X8

Capture the following observations into TR:

For the evaluation of performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution (Issue 9),

For case 2 (model trained on dataset S but tested on dataset B),

- 2 sources [vivo, QC] observe significant performance loss (-7.2% ~ -17.4%) relative to case 1, where dataset B consists of actual field data.

- 1 source [ETRI] observes significant performance loss of -37.3% relative to case 1, where dataset S and B are different by TxRU mapping.

- 3 resources [ZTE, Panasonic, Ericsson] observe moderate performance loss (-2.12% ~ -4.75%) relative to case 1, where dataset S and B are generated from different scenarios, antenna layout or UE location. E.g., Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

For case 2A (model further finetuned on dataset B),

- If finetune at NW and fix encoder at UE

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [vivo] observe moderate loss (-4.63% ~ -7.66%) relative to case 1. The improvement compared to case 2 is minor to moderate (2.58% ~ 9.17%).

    - When dataset A / B mismatch is not considered, 2 1 sources [vivo, QC] observes minor to moderate loss of (-3.5% ~ -7.66%) loss relative to case 1. The improvement compared to case 2 is moderate to significant (2.6% ~ 13.9%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [ZTE] observes moderate loss to case 1 (-4.32% ~ -4.75%). The improvement compared to case 2 is negative (-0.68% ~ -1.02%).

    - When dataset A includes or is the same as dataset B, 3 sources [ZTE, Panasonic, Ericsson] observe minor to moderate loss relative to case 1 (-0.97% ~ -4.67%). The improvement compared to case 2 is negative to minor (-0.61.01% ~ 1.6149%). Dataset S and B are Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

- If finetune at UE and fix decoder at NW

  - For using actual field data as dataset B,

    - 2 sources [vivo, QC] observe moderate to significant loss (-6.8% ~ -12%) relative to case 1. The improvement compared to case 2 is minor moderate (0.4% ~ 7.1%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - 2 source [Panasonic, Ericsson] observe minor to moderate loss (-1.56% ~ -3.81%) relative to case 1. The improvement compared to case 2 is marginal (0.03% ~ 0.75%). Dataset S and B are different by antenna layout and indoor/outdoor ratio.

    - 1 source [ETRI] observes significant loss (-23.8%) relative to case 1. The improvement compared to case 2 is significant (13.5%). Dataset S and B are different by TxRU mapping

- If finetune at both UE and both NW sides

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered and dataset A does not contain B, 1 source [vivo] observes significant loss (-8.5% ~ -44.85%) relative to case 1 depending on specific scenarios for data collection. The improvement compared to case 2 is negative (-28% ~ -1.3%). The loss relative to finetune at one side is negative (-1.72% ~ -37.19%).

    - When dataset A / B mismatch is not considered and A is equal to B, 1 2 sources [vivo, QC] observes moderate to significant loss of (-7.3% ~ -56.1%) relative to case 1. The improvement compared to case 2 is negative to significant (-39.3% ~ 10.1%). The loss relative to finetune at one side is minor to significant (-1.7 ~ 48.4%) negative to moderate (-3.82~ 4.67%).

  - For using synthetic data as dataset B

    - [When dataset A / B mismatch is considered modelled by Rx antenna spacing and dataset A does not contain B and dataset A has same distribution as S, 1 source [Samsung] observes similar performance (-0.89% ~ 0.21%) as case 1.]

    - When dataset A / B mismatch is modelled by different scenarios (i.e., by NW side condition),

      - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-29.0% ~ -30.5%) relative to case 1 depending on whether synthetic data is used in the finetune.

      - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.3% ~-1.3%)

    - When dataset A includes B or is equal to B or has same distribution as B,

      - If the input is in spatial-frequency domain, and dataset A does not contain B, 2 sources [Panasonic, Ericsson] observe minor to moderate loss (-1.1% ~ -3.69%) relative case 1 depending on the modelling of synthetic data. The improvement compared to case 2 is minor (0.57% ~ 1.3%). Compared to finetune at one side, the performance improvement is similar to minor (-0.35% ~ 1%).

      - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-3.67% ~ -11.18%) relative to case 1 depending on whether synthetic data is used in the finetune.

      - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.89% ~1.51%).

##### Proposal 72a: monitoring accuracy requirement (revised from 62a last meeting)

Evaluate the monitoring accuracy requirement for NW-side and UE-side monitoring approaches.

The following evaluation study is suggested:

- Evaluate the monitoring accuracy and the number of required samples based on monitoring decision making performance. Use false alarm probability and missed detection probability in monitoring decision making as KPIs as a starting point.

- KPIFA =Prob(KPIActual < thr), where the AI/ML model#A is trained based on training dataset from Scenario#A/Configuration#A, and the evaluation is performed on the model#A and a dataset from the same Scenario#A/Configuration#A.

- KPIMD =Prob(KPIActual >= thr), where the AI/ML model#A is trained based on training dataset from Scenario#A/Configuration#A, and the evaluation is performed on either a different model than model#A (representing an erroneous model usage) or a different dataset than Scenario#A/Configuration#A (representing a data distribution drift).

- KPIActual is obtained based on N >= 1 samples.

  - In case of SGCS as KPI, KPIActual can be derived by averaging the SGCS estimate over N samples.

- For each choice of monitoring approach and resolution of ground truth reporting, companies to report the value of N and the total overhead that satisfies both the target false alarm rate and the target missed detection rate.

  - Target false-alarm rate, KPIFA < 0.05 as a baseline.

  - Target miss detection probability KPIMD < 0.1 as a baseline.

  - Companies determine thr appropriately to balance false alarm and miss detection.

- To also consider rank>1 with an appropriate metric for the given KPI.

FL note: Not to be confused with the false alarm and missed detection metrics for monitoring accuracy studied in Rel-18.

# FL closing remark

# List of agreements

## Agreements from RAN1 #116

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study:

| Case | Target CSI slot(s) | Whether the UE uses past CSI information | Whether the network uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |
| 5 | Present slot | No | Yes |

Note 1: For the UE, the past CSI information may include past model inputs and/or any information derived from them. For the network, the past CSI information may include past CSI feedback instances and/or any information derived from them.

Note 2: For case 3 and case 4, the UE may perform prediction as a separate step or jointly with compression. Similarly, the network may perform prediction as a separate step or jointly with reconstruction. Companies to report which option is selected, the number of future slots, and whether the prediction is AI/ML-based or not.

Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well.

Note 4: Down-selection is not precluded.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following as baseline options for UE distribution:

- Option 1: 80% indoor, 20% outdoor

- Option 2: 100% outdoor

Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 10 km/h, 20 km/h, 30 km/h, 60 km/h, 120 km/h. Assumption on O2I car penetration loss and spatial consistency follow the R18 AI based CSI prediction.

Working Assumption

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following benchmark scheme for performance comparison:

- For cases without prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI compression study.

- For cases with prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI prediction study, with R18 MIMO eType II codebook for compressing the feedback.

Agreement

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, study the following aspects of the performance/complexity trade-off when comparing the localized model with a benchmark model that is not localized:

- Performance of the localized model that has similar or lower complexity as the benchmark model.

- Model complexity of the localized model that achieves similar or better performance as the benchmark model.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions:

- CSI-RS configuration

  - Periodic: 5 ms periodicity (baseline), 20 ms periodicity(encouraged)

  - Aperiodic (for cases with prediction): Optional, CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II)

- CSI reporting periodicity: {5, 10, 20} ms; other values are not precluded

- For cases with the use of past CSI information, to report observation window, including number/time distance of historic CSI/channel measurements.

- For cases with prediction, to report prediction window, including number/time distance of predicted CSI/channel.

Agreement

To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc.

Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

Agreement

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Note: While modelling the spatial correlation, strive to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. To report methods to generate training and testing dataset.

Agreement

- For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19,

  - adopt the CSI feedback overhead rate as reference, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead across time.

Note: The CSI feedback overhead of a single report is calculated as in R18 CSI compression study.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for cases with prediction of future CSI, in which prediction and compression are separated, to optionally evaluate a scheme with ideal prediction as an additional evaluation case for reference.

Note: The ideal prediction scheme should model realistic channel estimation.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, Case 4 and Case 5, study the performance impact resulting from non-ideal UCI feedback.

Agreement

For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider at least the following aspects when comparing different options:

- Inter-vendor collaboration complexity, e.g., whether bilateral collaboration is required between vendors.

- Performance.

- Interoperability and RAN4 / testing related aspects.

- Feasibility.

## Agreements from RAN1 #116-bis

Agreement

For the results template used to collect evaluation results for temporal domain compression Case 1/2/5, adopt Table 1 used in Rel-18 as starting point with the following additions:

- Temporal domain CSI setting

  - CSI feedback periodicity

  - CSI-RS periodicity

- Description of model input/output and Case

  - Compression case, e.g., Case 1/2/5

  - Usage of historical CSI at UE/NW side (e.g., number / time distance, eigen-vectors / raw channels, etc)

  - Methods to handle UCI loss (if applicable), e.g., CSI buffer reset, CSI retransmission, etc.

  - Methods to handle rank adaptation (if applicable)

- UE distribution (Option 1 or Option 2) and UE speed

- CSI feedback overhead rate: X/Y/Z bits per normalized time unit

  - Normalized time unit = 5ms and adopt same X/Y/Z values as in Table 1 of Rel-18

- Benchmark scheme

  - Rel-16 eT2 and compression Case 0 (i.e., Rel-18 AI/ML based CSI compression)

- Whether/how spatial consistency is modelled

- Whether/how UCI loss is modelled

  - The same UCI loss model shall be applied to the benchmark for fair comparison.

- Whether/how rank adaptation is modelled

- Modelling of channel estimation error

- Whether/how phase discontinuity is modelled (if applicable)

Agreement

For the results template used to collect evaluation results for temporal domain prediction and compression Case 3/4, adopt Table 1 used in Rel-18 as starting point with the following additions:

- Temporal domain CSI setting

  - CSI feedback periodicity

  - CSI-RS periodicity

- Description of model input/output and use case

  - Compression case, e.g., case 3 / 4

  - Observation window (usage of historical CSI at UE/NW side, e.g., number / time distance, eigen-vectors / raw channels, etc)

  - Prediction window (e.g., time distance between 1st prediction instance and last observation instance, number / time distance of predicted CSI)

  - Methods to handle UCI loss (if applicable)

- UE distribution (Option 1 or Option 2) and UE speed

- CSI feedback overhead rate: X/Y/Z bits per normalized time unit

  - Normalized time unit = 5ms and adopt same X/Y/Z values as in Table 1 of Rel-18

- SGCS values before (if applicable) and after compression

- Assumption on the prediction of future CSI

  - Separate step or jointly with compression

  - If separate, description of the AI or non-AI prediction algorithms: ideal prediction, AI-based prediction, non-AI-based prediction (e.g., nearest historical CSI and its location, learning window size / time correlation matrix size for auto-regression based prediction),

    - Note: the same prediction algorithm to be used for the benchmark scheme.

- Benchmark schemes

  - Description of feedback schemes, i.e., Rel-18 doppler eT2

- Whether/how spatial consistency is modelied

- Whether/how UCI loss is modelled

  - The same UCI loss model shall be applied to the benchmark for fair comparison.

- Modelling of channel estimation error

- Whether/how phase discontinuity is modelled (if applicable) Modelling of phase discontinuity

Conclusion

For multi-vendor results table, adopt Rel-18 Table 4 for joint training and Rel-18 Table 5 for separate training as starting point, with the same additions of above 2 agreements.

Conclusion

For model generalization results table, adopt Rel-18 Table 2 and Generalization Case 1 / 2 / 3 as starting point with same additions above. For generalization aspects, adopt the following

- Various UE speed

- UE distribution

- Various CSI-RS periodicity

Conclusion

For model scalability results table, adopt Rel-18 Table 3 and Generalization Case 1 / 2 / 3 as starting point with same additions above. For generalization aspects, adopt the following

- Various numbers of antenna ports

- Various frequency granularity

- Various payload size

Conclusion:

- Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors).

- It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4.

Observation

- Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed

- Option 1 and 2 may require high specification effort from RAN1 perspective.

Conclusion

- Deprioritize Option 2 for inter-vendor training collaboration.

  - Note: This deprioritization shall not affect the ongoing discussion in RAN4 on RAN4-Option3 and RAN4-Option4.

Agreement

- For Option 3, further define the two sub-options:

  - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 5, further define the two sub-options:

  - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 4, it is clarified that:

  - Dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing.

- Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavors (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Agreement

- For Option 3/4/5, focus further discussion on the following assumptions:

  - Option 3a/5a

    - The model(5a)/parameter(3a) exchange originates from the NW-side and ends at the UE-side.

    - Model(5a)/parameters(3a) exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both.

      - Option 3a-1/5a-1: Model/Parameters exchanged from the NW-side to UE-side is CSI generation part.

      - Option 3a-2/5a-2: Model/Parameters exchanged from the NW-side to UE-side is CSI reconstruction part.

      - Option 3a-3/5a-3: Model/Parameters exchanged from the NW-side to UE-side are both CSI generation part and CSI reconstruction part.

      - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance.

        - Performance target

        - Dataset or information related to collecting dataset

    - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc.

  - Option 3b

    - The method of exchanging is over the air-interface via model transfer/delivery Case z4.

    - The parameter exchange is from NW to UE.

    - Parameters exchanged from the NW-side to UE-side is CSI generation part.

  - Option 5b

    - The method of exchanging is over the air-interface via model transfer/delivery Case z4, assuming that the model structure is aligned based on offline inter-vendor collaboration.

    - The model exchange is from NW to UE.

    - Model exchanged from the NW-side to UE-side is CSI generation part.

  - Option 4:

    - The dataset exchange originates from the NW-side and ends at the UE-side.

    - Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI,  CSI feedback).

    - Option 4-2: Dataset exchanged from the NW-side to UE-side consists of (CSI feedback, reconstructed target CSI).

    - Option 4-3: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback, reconstructed target CSI).

    - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance.

      - Performance target

    - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc.

  - Note: For each option/sub-option of interest, companies to bring discussion on how inter-vendor collaboration complexity, interoperability, and feasibility may be addressed. Companies to strive to provide solution(s) that can address all the following aspects: inter-vendor collaboration complexity, performance, interoperability, and feasibility.

  - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavors (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Agreement

- For the results template used to collect evaluation results for AI/ML-based CSI compression using localized models, adopt Table 1 used in Rel-18 as starting point, capturing the generalized model result and the localized model result as separate columns, with the following additions for the localized model:

- Dataset description

  - Local region modelling: e.g., Option 1 or Option 2, and further details

  - Temporal modelling: e.g., how temporal variation is modelled in train and test sets

  - Dataset description for generalized model

Conclusion

In Rel-19 study of temporal domain aspects of AI/ML-based CSI compression using two-sided model, CSI prediction that is performed entirely at NW-side is deprioritized.

Agreement

- For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for the temporal domain prediction and compression Case 3 and Case 4, adopt the following evaluation assumptions as baseline:

  - Observation window (number/distance):

    - For periodic CSI-RS with 5ms periodicity: 12/5ms, 10/5ms, 8/5ms, 5/5ms, 4/5ms, unrestricted observation window

    - For periodic CSI-RS with 20ms periodicity: up to companies (encouraged)

    - For aperiodic CSI-RS: 12/2ms, 8/2ms, 4/2ms

    - Others can be additionally submitted

  - Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance):  4/5ms/5ms

    - Others can be additionally submitted, e.g. 4/1ms/5ms, 8/1ms/5ms, 4/5ms/10ms, 1/-/5ms

Agreement

For the results template used to collect evaluation results for temporal domain prediction and compression Case 4, adopt Table 1 used in Rel-18 as starting point with the following additions:

- Description of model input/output and use case

  - Methods to handle rank adaptation (if applicable)

## Agreements from RAN1 #117

Conclusion

Standardized signalling, if feasible and specified, can be used for parameter / model exchange in option 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity.

- Standardized signalling may be reused for exchanging CSI generation part, CSI reconstruction part, or both, etc, when necessary and feasible.

- Standarized signalling may be over-the-air, or other approaches.

Standardized signalling, if feasible and specified, can be used for dataset exchange in option 4 to alleviate/resolve the inter-vendor training collaboration complexity.

- Standardized signalling may be reused for dataset exchanging, when necessary and feasible.

- Standarized signalling may be over-the-air, or other approaches.

Note: feasibility will be discussed separately.

Agreement

- For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact.

- Exchange

  - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a)

  - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4)

  - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4)

    - Performance target (3a/5a/4)

    - Dataset or information related to collecting dataset (3a/5a)

    - Any other additional information

- Model pairing (3a/3b/4/5a)

- UE capability (3a/3b/4/5a)

- Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling (3a/3b/4/5a)

- Quantization of feedback (3a/3b/4/5a)

- Model structure details (3a/3b)

Note: Option 3a/4/5a and option 3b serve two different deployment time scales, UE capabilities, device-side optimizations, and training methods, and therefore may be complementary to each other, with potential specification of both.

- Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters.

Agreement

For option 1 / 3 / 4 / 5 and their sub-options, study mechanisms (e.g., post-deployment performance monitoring) for identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation to guarantee good performance in the field.

Agreement

For temporal domain aspects Case 3/4, change the small / medium / large payload region definition as follows:

Note: X, Y, Z, A, B, and C are feedback overhead rates in bits per time unit of 5ms. Note: For X, Y, and Z, α=[2] for rank=1/2 and α=[4] for rank=4 Note: For A, B, and C, β=[0.5] for rank=1 and β=[0.75] for rank=2/4

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression (Cases 1-5), in addition to FLOPs, also consider FLOPs per normalized time unit. Use 5msec as the normalized time unit.

Agreement

In the results template for capturing the evaluation of temporal domain aspects Case 3/4 of AI/ML based CSI compression, regarding the “upper bound”, capture both of the following:

- upper bound based on ideal CSI prediction and without CSI compression

- upper bound based on benchmark CSI prediction and without CSI compression

Agreement

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, regarding training,

- The k-th local model is trained on region #B_k (the k-th local region), 1<=k<=N.

- The generalized model is trained on Region #A that may be constructed via any of the following methods that is appropriate for the given generalized/local region modeling approach.

  - Region #A is the same as the union of regions #B_1, …, #B_N.

  - Region #A is a proper superset of the union of regions #B_1, …, #B_N.

  - Region #A is generated separately from regions #B_1, …, #B_N.

  - Note: companies to report which method was used.

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, regarding testing,

- The trained generalized model, local model, and the non-AI/ML benchmark are tested on the regions #B_1, …, #B_N.

- In case N>1, when reporting the results, companies may report the performance of the generalized model, the local models, and the non-AI/ML benchmark, by averaging the performance over the regions #B_1,…,B_N. Companies to report the value of N.

Agreement

For collecting evaluation results for temporal domain aspects of AI/ML-based CSI compression using localized models, use the same results template used to collect evaluation results for AI/ML-based CSI compression using localized models

- Adding the same temporal setting that is used for results template used to collect evaluation results for temporal domain compression Case 1/2/5.

| Temporal setting | Temporal domain aspect Case 1-5 |
| --- | --- |
|  | CSI-RS configuration: periodic or aperiodic For periodic: periodicity For aperiodic: # of resources K in the CSI-RS burst / time internal m in msec |
|  | CSI reporting periodicity |
|  | Usage of historical CSI at UE side: number / time distance |
|  | Usage of historical CSI at NW side: number / time distance |
|  | Prediction window: number / time distance between prediction instances / distance from the last observation instance to the 1st prediction instance (Only applicable to Case 3,4) |

Agreement

Further study following monitoring options in Rel-19, including the necessity and feasibility,

- NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1)

  - SRS-based monitoring

- UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on the output of the CSI reconstruction model at the UE (Case 2-1)

    - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side.

  - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2)

  - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI

  - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model

  - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook

Regarding monitoring metrics:

- Monitoring accuracy also includes generalization considerations, if applicable.

- Complexity also includes LCM complexity, if applicable.

- Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions.

- Testability of UE reported metrics

Discussion may include the following aspects:

- Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration

- Temporal domain aspects of CSI compression

- How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation

Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches.

Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

Agreement

For temporal domain aspects Case 3 and 4, study the impact on LCM aspects of separate prediction and compression, and joint prediction and compression.

Note: Observations of companies results till RAN1#117 are captured in FL summary R1-2405419.

## Agreements from RAN1 #118

Agreement

For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3.

Agreement

For studying the standardization of model structure RAN1, RAN1 assumes at least the following:

- Precoding matrix as an input (as opposed to raw channel matrix)

  - Per-layer processing, with common structure across ranks and layers (corresponding to Option 2-1, 2-2, 3-1, or 3-2 for handling rank ≥ 1)

- For temporal domain aspects Case 2 and Case 3, strive to reuse the model structure of Case 0 where appropriate, with additional layers or operations either at the input/output domain or at the latent domain.

- For Case 0, use precoding matrix, e.g., eigen vector in spatial-frequency domain, and/or angular, delay domain representation such as eType-II W2.

- For Case 2 and Case 3, use precoding matrix, e.g., spatial-frequency domain for each CSI observation instance, and/or angular, delay, and Doppler domain representation such as eType-II W2.

- Consider scalability approaches over numbers of Tx ports, CSI feedback payload sizes, and bandwidths.

Observation

Option 5 alone doesn’t fully address inter-vendor collaboration complexity, but Option 5 has potential performance benefit and flexibility compared to Option 3.

Agreement

Continue the study of following directions - on-device operation and UE side offline engineering

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4)

  - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues

    - [Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing?

    - [Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it?

    - [Issue 3] Is there an overhead concern, and if so, how to address it?

    - [Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it?

- Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues

  - [Issue 3] Is there an overhead concern, and if so, how to address it?

  - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs?

  - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it?

  - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it?

- Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues

  - [Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them?

  - [Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it?

  - [Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing?

  - Note:

    - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder.

    - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder.

    - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder.

- Note: UE-side data and NW-side data in “UE-side data distribution” and “NW-side data distribution” are field data.

- Note: Some issues identified in one direction may/may not be applicable for other Direction.

- Note: potential down selection among the 3 directions is not precluded

- Study of data distribution mismatch to consider the use of synthetic data and/or field data.

Agreement

Adopt all the observations in Section 2 of R1-2407499.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, study the performance impact resulting from non-ideal UCI feedback.

- Scenario A: no UCI loss

  - Note: Corresponds to an upper bound or re-aligning missing historical CSI information

- Scenario B: UCI loss, known at NW and unknown at UE, with mitigation at NW

  - Note: Corresponds to implementation-based mitigation at NW but no signaling to UE.

- Scenario C: UCI loss, known at NW and UE, with mitigation at NW and UE

  - Note: Corresponds to reset of historical CSI information at both UE and NW or any other mitigation approach enabled by signaling.

- UCI loss modeling

  - 10% UCI loss probability on all UCI reports

  - Other options are not precluded, e.g., No UCI loss for the first UCI report of each observation window, and 10% UCI loss probability for the subsequent reports of each observation window, as a shortcut to simplify the evaluation work.

  - Other values for UCI loss probability are not precluded.

- Note: The same UCI loss modeling shall be applied to the benchmark for fair comparison.

- FFS: partial PMI-related UCI loss

Agreement

According to the Rel-19 study objective,

- For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

RAN1 is recommending extending the study of the AI/ML CSI compression based on RAN1 understanding the study on CSI compression is not completed.

## Agreements from RAN1 #118bis

Conclusion

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Answer to Issue 2: From RAN1 perspective, at least,

  - For option 4-1, there is no proprietary information concern of disclosing encoder input/output from NW side to UE side

  - For option 3a-1, there is no proprietary information concern of disclosing encoder parameters from NW side to UE side

  - Note: If some additional information is disclosed from network side to UE side, proprietary concerns for option 4-1 and 3a-1 can not be precluded by default.

Agreement

For issues listed for inter-vendor collaboration direction B, further study the overhead in Direction B based on the size of the encoder (i.e., number of parameters and quantization level), the number of encoders, and how often the parameters need to be transferred.

Agreement

For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression,

- For Direction A:

  - Further study the following options with potential down-selection

    - 4-1: dataset is {target CSI, CSI feedback},

    - 3a-1: encoder parameter exchange, with {target CSI}

    - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Agreement

- For Direction C:

  - It is clarified that, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side.

  - Study whether the fully specified reference models or their compatible models may provide reasonable performance for deployment at NW and UE

    - Including performance evaluation of training/finetuning the models by modeling data distribution mismatch between synthetic and field data, and/or by using actual field data

    - Including performance evaluation of training/finetuning the models by modelling data  distribution mismatch between NW-side field data and UE-side field data, and/or by using actual field data from both NW-side and UE-side

    - Note: Training/finetuning is done at either or both sides without inter-vendor collaboration.

  - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3)

  - FFS whether the reference model is the same as RAN4 specified model

  - FFS whether a single or multiple reference (and how many) models need to be specified

Agreement

For the evaluation studies of Direction A,

- For encoder parameter exchange from NW-side to UE-side of 3a-1,

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder against the exchanged encoder.

    - Step 2: UE-side develops actual encoder against the nominal decoder

  - Alt. 2: UE-side directly develops and trains the actual encoder.

  - Note: Target CSI is available at UE side

- For dataset exchange from NW-side to UE-side {target CSI, CSI feedback} of 4-1,

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder using the exchanged dataset.

    - Step 2: UE-side develops an actual encoder against the nominal decoder.

  - Alt. 2: UE-side directly develops and trains the actual encoder.

Agreement (further amended on Friday as shown in red)

For discussion on performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional condition (issue 4 and 6), consider

- NW-side data (dataset A) and UE-side data (dataset B) are mismatched in terms of UE-side additional condition (e.g., SVD phase normalization method, UE antenna virtualization, antenna imbalance, antenna spacing / layout, etc)

  - Note: dataset A may or may not contain dataset B

  - Note: if dataset A includes dataset B, this means NW timely collect data from the UE

  - Note: Dataset A and dataset B are aligned in terms of NW-side additional conditions.

- Case 1: An encoder-decoder pair is trained on dataset B. This serves as an upper-bound.

- Case 2 (for direction A):

  - For direction A: NW-side trains a decoder on dataset A, and UE-side trains an encoder based on either dataset A, dataset B, or both (where the choice depends on the inter-vendor collaboration Options (4-1 or 3a-1), the training alternatives (Alt 1 or Alt 2) within the Options, and the choice of target CSI for training)

- Case 3 (for direction B): NW-side train an encoder-decoder on dataset A and transfers the encoder parameters to UE.

- Study issue 4 and 6 by evaluating the inference performance for Case 2 / 3 on test dataset B, comparing to the inference performance for case 1 on test dataset B.

- Note: Evaluations are assumed to perform without common reference model structure among companies.

Agreement

- For the evaluation study of offline engineering for Direction C for Issue 9:

  - Case 1: The AI/ML model is trained based on training Dataset B, and then the AI/ML model performs inference/test on a dataset from test Dataset B.

    - Note: This serves as the upper-bound performance.

  - Case 2: The AI/ML model is trained based on training Dataset S, and then the AI/ML model performs inference/test on test Dataset B.

    - Note: This represents the performance of Direction C when not using field data.

  - Case 2A:

    - Step 1: The AI/ML model is trained based on training Dataset S

    - Step 2:

      - NW-side and/or UE-side may separately train new CSI reconstruction part and/or CSI generation part compatible to the AI/ML model from Step 1 based on training Dataset B, as agreed in Note 1-1, 1-2, and 1-3 as agreed in RAN1#118.

      - Dataset from Step 1 may be additionally used.

    - Step3: The CSI generation part and the CSI reconstruction part from Step 2 performs inference/test on test Dataset B

    - Note: This represents the performance of Direction C when the AI/ML models are updated based on field data.

  - Note: Dataset S is meant to represent the synthetic data used for fully specified model development, and Dataset B is meant to represent the field data.

    - Examples for Dataset S and B:

      - Modeling via TxRU mapping, UMi vs. UMa, UMa vs. InH, etc.

        - It is acknowledged that the scenario/configuration modeling between S and B may not be complete representative of the difference between synthetic and field data distribution mismatch.

      - Synthetic vs. actual field data (if available)

        - Companies are encouraged to provide details on the scenarios/configurations underlying the field data

  - Note: Case 1 and Case 2 have been studied during Rel-18 generalization study and conclusions have been captured in the TR.

  - Note: For Case 2A, companies may additionally evaluate UE-side / NW-side data distribution mismatch with respect to UE side additional condition. This can be done, in Step 2, by treating the training Dataset B as representing UE-side data distribution and considering a training Dataset A representing NW-side data distribution.

  - Note: Evaluations are assumed to perform without common reference model among companies.

## Agreements from RAN1 #119

Conclusion

- For Direction A 4-1 and 3a-1, send LS to RAN2 for the study of feasibility of parameter / dataset exchange

Conclusion

Above LS, additionally CC SA2, SA3, SA5

Agreement

- For Direction A Option 3a-1 and Direction C, study the feasibility of scalable model structure specification over numbers of Tx ports, CSI feedback payload sizes, and bandwidths, number of slots.

  - Feasibility of Direction A Option 3a-1 is contingent on the feasibility of the scalable model structure specification.

  - Feasibility of Direction C is contingent on the feasibility of the scalable model structure specification and model parameters specification.

  - Note: Angular/Delay/Doppler domain conversion and quantization are considered to be part of feasibility study

Agreement

For studying the standardized model structure,

- For temporal domain Case 0,

  - In case of spatial-frequency domain input, adopt Transformer as the backbone structure.

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 2,

  - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation with additional layers

      - Conv-LSTM

      - LSTM

    - Option 2: Latent adaptation on top of Case 0 model structure

      - Reusing Case 0 structure and changing quantization operation and/or feedback payload over different slots

    - Note: other options are not precluded

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 3,

  - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation

    - Option 2: Latent adaptation

  - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input.

- Take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure.

  - Note: Processing steps at the UE to derive angular, delay, and/or Doppler domain basis, and corresponding reconstruction steps at the gNB are considered.

  - Note: scalar/vector quantization and dequantization are considered. Quantization-aware training with jointly updated quantization method/parameters (Case 2-2) is used.

Agreement

For NW to collect data for training, study following spec impacts

- Data format: codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction.

  - FFS if enhancement is needed

  - FFS number of samples in the report.

  - FFS whether channel or precoder is needed for temporal Cases 3

- Configuration of rank/layer, number of subbands

- Mechanism for ground-truth reporting

- FFS: Report additional information regarding the samples, e.g., data quality, FFS the definition of data quality and corresponding parameters.

- FFS if enhancements in CSI-RS and SRS configuration is needed.

- FFS: Report associated information that captures UE side additional condition

- FFS: Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI

- FFS: details of CSI measurement

For UE to collect data for training, study following spec impacts

- NW configuration or UE request, e.g., RS configuration/transmission for data collection

- Whether enhancements in CSI-RS configuration is needed.

- Configuration of temporal aspects for temporal case 2/3, e.g., association between input and output CSI

- FFS: Need of configuration of ID, and configuration of ID

Agreement

To capture all observations in section 2 of R1-2410725 to TR38.843.

Agreement

Draft LS R1-2410915 with the following updates:

Revising “The size becomes 600K * (2000 bits) / (8bits/Byte) = 150 MB+ 11.6 MB = 161.6MB if we assume N1=N2.” To “The size becomes N2 * (2000 bits) / (8bits/Byte) + 11.6 MB” , and

Revising

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

to

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

3.

Deleting

“RAN1 respectfully requests RAN2’s feedback on the feasibility of standardized signaling, considering the size of dataset / encoder parameters provided above.”

4.

Deleting

“The size is based on Case 0 of two-sided CSI compression; the size may or may not be larger for Case 2 and Case 3.” from “For Option 4-1: sharing {target CSI, CSI feedback} dataset:”

Agreement

Final LS R1-2410922 is endorsed.

Agreement

For temporal domain aspects Case 3, study LCM aspects and specification impacts,

consider the following options for training data collection

- Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s).

- Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s).

- Note: During inference, the input to the CSI generation part is derived based on the predicted CSI.

consider following options for the monitoring labels

- Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s).

  - CSI prediction output is used as input to CSI generation part.

  - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately.

- Option 2: The monitoring label is derived based on the measured CSI of the future slot(s)

  - Option 2a: CSI prediction output is used as input to CSI generation part.

    - Note: This corresponds to end-to-end monitoring of CSI prediction and compression.

  - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose.

    - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately.

Study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts.

## Agreements from RAN1 #120

Conclusion

Under 10% UCI loss,  Case 2 shows small to large performance drop (by Comparing Scenario B and Scenario A). Signaling support to mitigate the UCI loss can improve the performance and helps preserving the gain of Case 2 over Case 0 (by Comparing Scenario C and Scenario A and benchmark schemes).

Agreement

Investigate the following approaches for signaling support for mitigating the impact of UCI loss

- NW-signaling to reset of historical CSI information at UE

- NW-triggered CSI retransmission

Conclusion

For direction A, performance impact, if any, due to NW / UE data distribution mismatch with respect to UE side additional condition can be addressed.

Agreement

In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then

- The same ID can be used for UE to collect UE-side target CSI for UE-side training

- The same ID can be used for applicability inquiry and reporting

- The same ID can be used for inference configuration

- FFS: whether ID/even same ID is needed for monitoring configuration

- FFS: where the ID is assigned

- Note: whether the purpose for pair will be specified will be discussed separately.

Agreement

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, performance target is confirmed as additional information along with the exchanged dataset or the model parameters.

- FFS: type of performance metric

- FFS: input data for evaluating the performance

Conclusion

- Direction C with standardized model ensures a minimum performance requirement.

  - The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4.

- Direction A with standardized model parameter / dataset exchange from NW-side to UE-side may achieves better performance.

Conclusion

For NW-first training, in inter-vendor training collaboration Direction A and C, identification of root cause for performance issues can be achieved at least by the following

- Data distribution mismatch that is identified by either NW side or UE side

- Via target CSI and CSI feedback pair report, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

Agreement

For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose,

Encoder description:

| Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer) | Number of tokens Feature dimension of each token Output dimension dmodel per token |
| --- | --- |
| Positional encoding | Number of tokens positions Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice |
| Output linear layer | Reshape the matrix to a vector Output dimension zdim |
| Quantization | Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment |

Decoder description:

The decoder has a mirroring design as the encoder. Details are to be discussed.

# References

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” (Release 18), December 2023.

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #102, December 2023.

- R1-2401561, “Revised Final summary for Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #116, Feb. 2024.

- RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #103, March 2024.

- R1-2403505, “Final summary of Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #116-bis, Apr. 2024.

- R1-2405419, “Final summary of Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #117, May 2024

- R1-2407347, “Final summary of Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #118, Aug. 2024

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #105, September 2024.

- R1-2409161, “Final summary of Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #118-bis, Oct. 2024

- R1-2410724, “Final summary of Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), 3GPP TSG RAN WG1 #119, Nov. 2024