3GPP TSG RAN WG1 Meeting #77,	R1-142377

Seoul, Korea, 19th – 23rd May 2014

Source:	Ericsson

Title:	Gray mapping for 256QAM

Agenda Item:	6.2.3.1

Document for:	Discussion and Decision

# Introduction

The modulation mapping for 256QAM was discussed in RAN1#76bis and the proposal in [1] was agreed as working assumption listed below:

Working assumption (Revisit in RAN1#77):

- Adopt binary reflected Gray mapping for 256QAM shown as follows:

1170 {(1−2bi)[8−(1−2bi+2)[4−(1−2bi+4)[2−(1−2bi+6)]]]
                +j(1−2bi+1)[8−(1−2bi+3)[4−(1−2bi+5)[2−(1−2bi+7)]]]}

To conclude the support of 256QAM in Rel-12, we discuss the modulation mapping for 256QAM in this contribution.

# Binary reflected Gray mappings

The mappings of binary bits to symbols in rectangular QAMs for LTE have been based on separate binary reflected Gray mappings of the I and Q values [2]. More specifically,

- For QPSK, the pair of bits (bi, bi+1) is mapped to

x = 12{(1−2bi) + j(1−2bi+1)}.

- For 16QAM, the quadruplet of bits (bi, bi+1, bi+2, bi+3) is mapped to

x = 110{(1−2bi)[2−(1−2bi+2)] + j(1−2bi+1)[2−(1−2bi+3)]}.

- For 64QAM, the sextuplet of bits (bi, bi+1, bi+2, bi+3, bi+4, bi+5) is mapped to

x = 142{(1−2bi) [4−(1−2bi+2)[2−(1−2bi+4)]]
                           + j(1−2bi+1) [4−(1−2bi+3)[2−(1−2bi+5)]]}.

Following the same mapping principle, for Rel-12 256QAM, the octuplet of bits (bi, bi+1, bi+2, bi+3, bi+4, bi+5, bi+6, bi+7) is mapped to

x = 1170{(1−2bi)[8−(1−2bi+2)[4−(1−2bi+4)[2−(1−2bi+6)]]]
                 + j(1−2bi+1)[8−(1−2bi+3)[4−(1−2bi+5)[2−(1−2bi+7)]]]}.

For reference, the I and Q values of the proposed mapping are listed in Table 1.

# Conclusions

In this contribution, we discussed modulation mapping principles for Rel-12 256QAM support. We concluded with the following proposal.

Proposals:

- Confirm the working assumption from RAN1#76bits, i.e., adopt binary reflected Gray mapping for Rel-12 256QAM:

1170 {(1−2bi)[8−(1−2bi+2)[4−(1−2bi+4)[2−(1−2bi+6)]]]
                +j(1−2bi+1)[8−(1−2bi+3)[4−(1−2bi+5)[2−(1−2bi+7)]]]}.

# References

R1-141635, Gray mapping for 256QAM, Ericsson

TS 36.211 Physical channels and modulation

Table 1 256QAM modulation mapping

|  | I× 170 | Q× 170 |  | I× 170 | Q× 170 |  | I× 170 | Q× 170 |  | I× 170 | Q× 170 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00000000 | 5 | 5 | 01000000 | 5 | −5 | 10000000 | −5 | 5 | 11000000 | −5 | −5 |
| 00000001 | 5 | 7 | 01000001 | 5 | −7 | 10000001 | −5 | 7 | 11000001 | −5 | −7 |
| 00000010 | 7 | 5 | 01000010 | 7 | −5 | 10000010 | −7 | 5 | 11000010 | −7 | −5 |
| 00000011 | 7 | 7 | 01000011 | 7 | −7 | 10000011 | −7 | 7 | 11000011 | −7 | −7 |
| 00000100 | 5 | 3 | 01000100 | 5 | −3 | 10000100 | −5 | 3 | 11000100 | −5 | −3 |
| 00000101 | 5 | 1 | 01000101 | 5 | −1 | 10000101 | −5 | 1 | 11000101 | −5 | −1 |
| 00000110 | 7 | 3 | 01000110 | 7 | −3 | 10000110 | −7 | 3 | 11000110 | −7 | −3 |
| 00000111 | 7 | 1 | 01000111 | 7 | −1 | 10000111 | −7 | 1 | 11000111 | −7 | −1 |
| 00001000 | 3 | 5 | 01001000 | 3 | −5 | 10001000 | −3 | 5 | 11001000 | −3 | −5 |
| 00001001 | 3 | 7 | 01001001 | 3 | −7 | 10001001 | −3 | 7 | 11001001 | −3 | −7 |
| 00001010 | 1 | 5 | 01001010 | 1 | −5 | 10001010 | −1 | 5 | 11001010 | −1 | −5 |
| 00001011 | 1 | 7 | 01001011 | 1 | −7 | 10001011 | −1 | 7 | 11001011 | −1 | −7 |
| 00001100 | 3 | 3 | 01001100 | 3 | −3 | 10001100 | −3 | 3 | 11001100 | −3 | −3 |
| 00001101 | 3 | 1 | 01001101 | 3 | −1 | 10001101 | −3 | 1 | 11001101 | −3 | −1 |
| 00001110 | 1 | 3 | 01001110 | 1 | −3 | 10001110 | −1 | 3 | 11001110 | −1 | −3 |
| 00001111 | 1 | 1 | 01001111 | 1 | −1 | 10001111 | −1 | 1 | 11001111 | −1 | −1 |
| 00010000 | 5 | 11 | 01010000 | 5 | −11 | 10010000 | −5 | 11 | 11010000 | −5 | −11 |
| 00010001 | 5 | 9 | 01010001 | 5 | −9 | 10010001 | −5 | 9 | 11010001 | −5 | −9 |
| 00010010 | 7 | 11 | 01010010 | 7 | −11 | 10010010 | −7 | 11 | 11010010 | −7 | −11 |
| 00010011 | 7 | 9 | 01010011 | 7 | −9 | 10010011 | −7 | 9 | 11010011 | −7 | −9 |
| 00010100 | 5 | 13 | 01010100 | 5 | −13 | 10010100 | −5 | 13 | 11010100 | −5 | −13 |
| 00010101 | 5 | 15 | 01010101 | 5 | −15 | 10010101 | −5 | 15 | 11010101 | −5 | −15 |
| 00010110 | 7 | 13 | 01010110 | 7 | −13 | 10010110 | −7 | 13 | 11010110 | −7 | −13 |
| 00010111 | 7 | 15 | 01010111 | 7 | −15 | 10010111 | −7 | 15 | 11010111 | −7 | −15 |
| 00011000 | 3 | 11 | 01011000 | 3 | −11 | 10011000 | −3 | 11 | 11011000 | −3 | −11 |
| 00011001 | 3 | 9 | 01011001 | 3 | −9 | 10011001 | −3 | 9 | 11011001 | −3 | −9 |
| 00011010 | 1 | 11 | 01011010 | 1 | −11 | 10011010 | −1 | 11 | 11011010 | −1 | −11 |
| 00011011 | 1 | 9 | 01011011 | 1 | −9 | 10011011 | −1 | 9 | 11011011 | −1 | −9 |
| 00011100 | 3 | 13 | 01011100 | 3 | −13 | 10011100 | −3 | 13 | 11011100 | −3 | −13 |
| 00011101 | 3 | 15 | 01011101 | 3 | −15 | 10011101 | −3 | 15 | 11011101 | −3 | −15 |
| 00011110 | 1 | 13 | 01011110 | 1 | −13 | 10011110 | −1 | 13 | 11011110 | −1 | −13 |
| 00011111 | 1 | 15 | 01011111 | 1 | −15 | 10011111 | −1 | 15 | 11011111 | −1 | −15 |
| 00100000 | 11 | 5 | 01100000 | 11 | −5 | 10100000 | −11 | 5 | 11100000 | −11 | −5 |
| 00100001 | 11 | 7 | 01100001 | 11 | −7 | 10100001 | −11 | 7 | 11100001 | −11 | −7 |
| 00100010 | 9 | 5 | 01100010 | 9 | −5 | 10100010 | −9 | 5 | 11100010 | −9 | −5 |
| 00100011 | 9 | 7 | 01100011 | 9 | −7 | 10100011 | −9 | 7 | 11100011 | −9 | −7 |
| 00100100 | 11 | 3 | 01100100 | 11 | −3 | 10100100 | −11 | 3 | 11100100 | −11 | −3 |
| 00100101 | 11 | 1 | 01100101 | 11 | −1 | 10100101 | −11 | 1 | 11100101 | −11 | −1 |
| 00100110 | 9 | 3 | 01100110 | 9 | −3 | 10100110 | −9 | 3 | 11100110 | −9 | −3 |
| 00100111 | 9 | 1 | 01100111 | 9 | −1 | 10100111 | −9 | 1 | 11100111 | −9 | −1 |
| 00101000 | 13 | 5 | 01101000 | 13 | −5 | 10101000 | −13 | 5 | 11101000 | −13 | −5 |
| 00101001 | 13 | 7 | 01101001 | 13 | −7 | 10101001 | −13 | 7 | 11101001 | −13 | −7 |
| 00101010 | 15 | 5 | 01101010 | 15 | −5 | 10101010 | −15 | 5 | 11101010 | −15 | −5 |
| 00101011 | 15 | 7 | 01101011 | 15 | −7 | 10101011 | −15 | 7 | 11101011 | −15 | −7 |
| 00101100 | 13 | 3 | 01101100 | 13 | −3 | 10101100 | −13 | 3 | 11101100 | −13 | −3 |
| 00101101 | 13 | 1 | 01101101 | 13 | −1 | 10101101 | −13 | 1 | 11101101 | −13 | −1 |
| 00101110 | 15 | 3 | 01101110 | 15 | −3 | 10101110 | −15 | 3 | 11101110 | −15 | −3 |
| 00101111 | 15 | 1 | 01101111 | 15 | −1 | 10101111 | −15 | 1 | 11101111 | −15 | −1 |
| 00110000 | 11 | 11 | 01110000 | 11 | −11 | 10110000 | −11 | 11 | 11110000 | −11 | −11 |
| 00110001 | 11 | 9 | 01110001 | 11 | −9 | 10110001 | −11 | 9 | 11110001 | −11 | −9 |
| 00110010 | 9 | 11 | 01110010 | 9 | −11 | 10110010 | −9 | 11 | 11110010 | −9 | −11 |
| 00110011 | 9 | 9 | 01110011 | 9 | −9 | 10110011 | −9 | 9 | 11110011 | −9 | −9 |
| 00110100 | 11 | 13 | 01110100 | 11 | −13 | 10110100 | −11 | 13 | 11110100 | −11 | −13 |
| 00110101 | 11 | 15 | 01110101 | 11 | −15 | 10110101 | −11 | 15 | 11110101 | −11 | −15 |
| 00110110 | 9 | 13 | 01110110 | 9 | −13 | 10110110 | −9 | 13 | 11110110 | −9 | −13 |
| 00110111 | 9 | 15 | 01110111 | 9 | −15 | 10110111 | −9 | 15 | 11110111 | −9 | −15 |
| 00111000 | 13 | 11 | 01111000 | 13 | −11 | 10111000 | −13 | 11 | 11111000 | −13 | −11 |
| 00111001 | 13 | 9 | 01111001 | 13 | −9 | 10111001 | −13 | 9 | 11111001 | −13 | −9 |
| 00111010 | 15 | 11 | 01111010 | 15 | −11 | 10111010 | −15 | 11 | 11111010 | −15 | −11 |
| 00111011 | 15 | 9 | 01111011 | 15 | −9 | 10111011 | −15 | 9 | 11111011 | −15 | −9 |
| 00111100 | 13 | 13 | 01111100 | 13 | −13 | 10111100 | −13 | 13 | 11111100 | −13 | −13 |
| 00111101 | 13 | 15 | 01111101 | 13 | −15 | 10111101 | −13 | 15 | 11111101 | −13 | −15 |
| 00111110 | 15 | 13 | 01111110 | 15 | −13 | 10111110 | −15 | 13 | 11111110 | −15 | −13 |
| 00111111 | 15 | 15 | 01111111 | 15 | −15 | 10111111 | −15 | 15 | 11111111 | −15 | −15 |