3GPP TSG RAN WG1 Meeting #78,	R1-143264

Dresden, Germany, 18th – 22nd August 2014

Source:	Kyocera

Title:	        Resource allocation for Type 1 discovery

Agenda Item:	7.2.3.2.1

Document for:	Discussion/Decision

Introduction

During RAN1#77, resource allocation for D2D discovery was discussed and the following working assumption was agreed [1]. In this contribution, we focus on the repetition pattern of Type 1 D2D discovery.

Design considerations of repetition

In this section we describe the time domain allocation of Type 1 discovery. In order to reduce complexity we suggest time contiguous allocation repetitions.  The repetition pattern could include {1, 2, 4 and 8} repetitions and even/odd patterns applied to even and odd discovery resource IDs as shown in Figure 1. Furthermore, the RV index could be pre-defined pattern, (0, 2, 3, 1). In addition, we also support the PUSCH hopping pattern for the above.

Figure 1: D2D Repetition resources

Proposal 1:

We propose time contiguous allocation repetitions. The repetition pattern could include {1, 2, 4 and 8} repetitions and even/odd patterns applied to even and odd discovery resource IDs.

Proposal 2:

The RV index should support pre-defined (0, 2, 3, 1) pattern.

Proposal 3:

PUSCH hopping is reused for Type 1 discovery.

Repetition gain

In this section we discuss the gain achieved by repetitions. We assumed the repetitions resources are contiguously allocated in the time domain. Further detailed simulation assumptions are described in Appendix A. Figure 2 shows the gain achieved as the number of repetitions increases. However, as the data indicates in Table 1 there is a diminishing return in gain as the number of repetitions increases.

Figure 2: Performance improvement of repetition

Table 1: Repetition gain at BLER = 0.01

| Number of transmissions | Gain |
| --- | --- |
| 2 | 6.0dB |
| 3 | 2.6dB |
| 4 | 1.7dB |
| 5 | 1.2dB |
| 6 | 0.8dB |
| 7 | 0.8dB |
| 8 | 0.5dB |

Conclusions

Proposal 1:

We propose time contiguous allocation repetitions. The repetition pattern could include {1, 2, 4 and 8} repetitions and even/odd patterns applied to even and odd discovery resource IDs.

Proposal 2:

The RV index should support pre-defined (0, 2, 3, 1) pattern.

Proposal 3:

PUSCH hopping is reused for Type 1 discovery.

- Appendix A: Simulation Assumptions

Data Symbol

The physical format of D2D Discovery signal is shown in Figure 3. Simulation assumptions are listed in the Table 2.

Gap

DMRS

Figure 3: The physical format of D2D discovery signal

Figure 5: Resource allocation for repetiton

Table 2: Simulation assumptions

| Carrier frequency | 2000 [MHz] |
| --- | --- |
| System Bandwidth | 10 [MHz] |
| Channel model | ITU-R Umi NLOS w/ dual-mobility |
| Moving direction of Tx UE and Rx UE | Uniform Random(-180-180[deg.]), independently |
| Number of Tx Antennas | 1 |
| Number of Rx Antennas | 2 |
| Number of RB | 3 |
| Modulation | QPSK |
| Bit length | 200 bits |
| Channel coding | Turbo coding |
| CRC | 24 bits |
| RV index | 0 -> 2 -> 3 -> 1 |
| UE velocities pairs (Tx, Rx) | (3,3) |
| Channel Estimation | ZF with linear interpolation in time |
| Receiver Type | MRC |

References

[1] Chairman's Notes RAN1#77