3GPP TSG RAN WG1 Meeting #78,	R1-143265

Dresden, Germany, 18th – 22nd August 2014

Source:	Kyocera

Title:	        SA and Data linkage for Mode 1

Agenda Item:	7.2.3.2.2

Document for:	Discussion/Decision

Introduction

During RAN1#77, Mode 1 resource allocation for D2D communication was discussed [1]. In this contribution we consider further details of Mode 1 resource allocation. SA’s detailed design is described in [2]

SA and data linkage

In this section we describe the linkage between the SA and the data allocation. Figure 1 shows the time and frequency domain allocations.

Figure 1: SA and Data linkage for Mode 1

Frequency domain allocation

In order to keep a simple design we propose PUSCH hopping to be reused for D2D communications.

Proposal 1:

PUSCH hopping is reused for frequency allocation for Mode 1 and Mode 2.

Time domain data allocation

For Mode 1 the eNB is responsible for scheduling the D2D UEs; therefore, the eNB should configure the period and offset of Data transmissions. Then the transmitting UE signals the period and offset in the SA.

Proposal 2:

For Mode 1 the eNB should configure the period and offset of Data transmission. The period and offset is then signaled in the SA.

Conclusions

Proposal 1:

PUSCH hopping is reused for frequency allocation for Mode 1 and Mode 2.

Proposal 2:

For Mode 1 the eNB should configure the period and offset of Data transmission. The period and offset is then signaled in the SA.

References

[1] Chairman's Notes RAN1#77

[2] R1-143261, Scheduling assignment design, Kyocera