3GPP TSG RAN WG1 Meeting #78			R1-143341

Dresden, Germany, 18th – 22nd August 2014

Agenda Item:	7.2.6.2

Source: 	CATR

Title: 	Phase 2 calibration results for 3D channel modelling

Document for:	Discussion and Decision

Introduction

In RAN1#75 meeting, most of open issues on fast fading channel modelling were concluded for 3D-UMa and UMi [1] and updated in current version of TR 36.873 [2]. Also in RAN1#76 meeting, some more details of calibration are clarified[3], in this contribution, initial simulation results are given for Phase 2 calibration of 3D channel modeling.

Simulation assumptions

The main simulation assumptions are listed in Table A1, and the current version of TR 36.873 [2] is adopted in channel generation. And the conclusion of RAN1#74bis meeting descripts the more details of phase 2 calibration:

- Phase 2 calibration details

  - BS antenna configuration:

    - Config 1: K=1, M=2, N=2, ULA, 0.5λ H/V  spacing

    - Config 2: K=M=10, N=2, X-pol, 0.5λ H/V spacing with the antenna weights in the working assumption with θtilt = 12 degrees

  - MS antenna configuration: 2 antennas with the same pol as BS

  - System bandwidth: 10 MHz

  - The following metrics for the serving cell are calibrated for each antenna configuration (collected over multiple runs)

    - CDFs of ESD and ESA

    - CDF of average wideband SINR before receiver (i.e., geometry)

    - CDF of largest (1st) singular value in PRBs at t=0

    - CDF of smallest (2nd) singular value in PRBs at t=0

    - CDF of the ratio between the largest singular value and the smallest singular value in PRBs at t=0

- Additional details

  - Dimension of the channel matrix:

    - 2 x (number of BS antenna ports)

  - Singular value calculation

    - Derived with channel matrices where antenna gain is applied but PL and shadowing are not modeled,

    - Singular values are calculated on a per PRB basis by

  - eig(∑HHH)/N , where the summation is across the PRB and N is number of subcarriers in the PRB

For these metrics, the following definitions are adopted.

- Wideband SINR: The wideband SINR equals the radio of RSRP from CRS port 0 of serving cell over the sum of RSRPs from CRS port 0 of all the other cells and noise power.

- ZSD and ZSA: Two kinds of ZSD and ZSA are produced, including “Stat.” and “Step 4”. “Stat.” denotes the angle spreads obtained according to the equation in [4]. And “Step 4” denotes the generated correlated large scale parameters in step 4.

- Max/Min eignvalue: The eignvalues are computed by eig(∑HHH)/N , where the summation is across the PRB and N is number of subcarriers in the PRB [5]. While antenna gain is applied but PL and shadowing are not modelled for channel matrices.

Simulation results

Figure 1~Figure 5 are the simulation results for wideband SINR, ZSD, ZSA, max eignvalue, min eignvalue, and the ratio of max eignvalue over min eignvalue, respectively. From these figures, it can be observed that the wrapping method has visible impact on all metrics of Phase 2 calibration.

From Figure 2 and Figure 3, though the distribution of “Stat.” and the corresponding “Step 4” are much closed to each other, it can be clearly seen that two curves are not aligned; if companies obtain CDF of ZSD and ZSA at different points there is risk of misalignment of simulation results. We think the CDF curves of ZSD and ZSA should be obtained after step 7.

UMa 		                                  UMi

# Figure 1: CDF of wideband SINR

Uma                                          UMi

# Figure 2: CDF of ZSD

UMa                                UMi

# Figure 3: CDF of ZSA

UMa                                               UMi

# Figure 4: CDF of eignvalues

UMa                                              UMi

# Figure 5: CDF of eignvalues ratio

References

- Chairman's Notes RAN1#75.

- R1-136084, Draft TR 36.873 v08.

- Chairman's Notes RAN1#76.

- TR 25.996.

- Chairman's Notes RAN1#74Bis.

Appendix

# Table A1: Simulation Assumptions

|  | Phase-2 calibration |
| --- | --- |
| Scenarios | 3D-UMa, 3D-UMi |
| BS antenna configurations | Config 1: K=1, M=2, N=2, ULA, 0.5λ H/V spacing Config 2: K=M=10, N=2, X-pol (+/-45), 0.5λ H/V, θetilt = 12 degrees |
| MS antenna configurations | Config 1:2 Rx ULA 0.5λ H spacing Config 1: 2Rx X-pol (0/+90) |
| System bandwidth | 10MHz (50RBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Carrier Frequency | 2GHz |
| Number of UEs per cell | 10 |
| UE distribution | Follows 36.873 3D-UMa, 3D-UMi |
| Polarized antenna modeling | R1-136021 (yellow part) |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Wrapping method | 1) Geographical distance based (baseline) 2) Radio distance based |
| Cluster elimination step 6 | scaling factor not changed after cluster elimination |
| Handover margin (for calibration) | 0 dB |