3GPP TSG-RAN WG1 #78 	R1-143644

August 18 – 22, 2014

Dresden, Germany

Agenda item:	7.2

Source: 	LG Electronics

Title: 	Summary of offline discussion on R1-143164

Document for:	Discussion/Decision

We had an offline discussion on this CR (i.e., R1-143164). Even though the majority of companies would like to clearly capture clearly the definition of validity of UL-DL configuration conveyed by L1 signaling in the specification, but some companies still have concern on this change. So, as a compromise way, we think that it needs to capture a clarification in the chairman note about the exact meaning of following sentences in the specification. This is because there is a different understanding among companies on the meaning of following sentences in the specification, and this clarification will be helpful for RAN1 group to have a common understanding on the current sentences.

Proposal:

It is a common understanding that the following sentences are used for eIMTA UE to check the validity of UL/DL configuration indication signalled on the PDCCH with CRC scrambled by eIMTA-RNTI and the consistency of PDCCH with CRC scrambled by eIMTA-RNTI.

- For a serving cell c, if subframe i is indicated as uplink subframe or a special subframe by higher layer parameter dlReferenceConfig-r12, the UE is not expected to receive a PDCCH with CRC scrambled by eIMTA-RNTI containing an UL/DL configuration for serving cell c that would indicate subframe i as a downlink subframe.

- For a serving cell c, if subframe i is indicated as downlink subframe or a special subframe by higher layer parameter subframeAssignment, the UE is not expected to receive a PDCCH with CRC scrambled by eIMTA-RNTI containing an UL/DL configuration for serving cell c that would indicate subframe i as a uplink subframe.