3GPP TSG RAN WG1 Meeting #78bis	R1-144348 (R1-144226)

Ljubljana, Slovenia, 6th – 10th October 2014

Source:	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Coolpad, Ericsson, ETRI, Huawei, HiSilicon, IAESI, InterDigital, LG, Nokia Corporation, Nokia Networks, Qualcomm, NTT Docomo, Panasonic, Samsung, TI, Verizon Wireless, ZTE

Title:	Regulatory Requirements for Unlicensed Spectrum

Agenda Item:	7.3.2.1

Document for:	Discussion and Decision

# Discussion

During RAN#64, RP-140808 was presented. That contribution outlines the different regulations around the world for unlicensed spectrum. This contribution provides an identical description of the requirements. The purpose is to enable 3GPP to identify what functionality LTE needs to support to be able to realize a single global solution capable of operating on unlicensed spectrum.

Proposal

- To incorporate the following content on regulatory requirements as a baseline into the TR for licensed-assisted access.

# Regulatory Requirements

## 2.1	ITU Region 1

## 2.1.1 Europe

The European regulation is determined by the European Commission and the ECC. The relevant regulations for the 5 GHz bands are found in two Commission Decisions [6, 7] and one ECC Decision [8]. These are interpreted by ETSI and used as a basis for harmonized standards, which are used for conformance declaration when products are placed on the European market. Harmonized European standards have a higher regulatory relevance than other product standards, since they are produced based on a mandate from the Commission with reference to an EU directive. They also go through a public enquiry and voting process, and are cited by the Commission. The European requirements on 5 GHz unlicensed deployment are specified in three ETSI harmonized standards [3, 4, and 5]. Figure 2.1.1-1 summarizes the relevant parts of the 5 GHz band set aside for unlicensed spectrum usage. The 5150-5350 MHz and the 5470-5725 MHz bands are referred here as the broadband radio access networks (BRAN) bands where the wireless access systems (WAS) including RLAN equipment are operating in. Moreover, the 5725-5875 MHz band (in the BRAN domain) is used by the fixed wireless access (FWA) networks and finally the intelligent transport systems (ITS) utilize the 5855-5925 MHz band.

The BFWA and the ITS are designated by the ECC for use as parts of the 5 GHz band and the relevant regulations are found in:

- An ECC Recommendation for FBWA [12], and

- A Commission Decision [13], an ECC Decision [14] and an ECC Recommendation for ITS [15].

General purpose SRD devices can also operate in the band 5725-5875 MHz under the provisions of the ETSI harmonised standard EN 300 440 [16], but with reduced max EIRP of 25 mW.

Figure 2.1.1-1: 5 GHz spectrum allocations in Europe.

The European Commission has recently submitted to CEPT a mandate to study the conditions for the extension of the 5 GHz range designated for WAS/RLANs [17] in order to allow the use by WAS/RLANs of the whole 5150-5925 MHz band.

| WAS/RLANs | Commission Mandate to CEPT- extension of the WAS/RLAN range | WAS/RLANs | Commission Mandate to CEPT- extension of the WAS/RLAN range |
| --- | --- | --- | --- |

5.925 GHz

5.725 GHz

5.47 GHz

5.35 GHz

5.15 GHz

Figure 2.1.1-2: Summary of existing and proposed EU regulations for WAS/RLANs in the 5GHz band

In the rest of this section, the specified ETSI requirements for the WAS/RLAN and FWA bands are summarized. Table 2.1.1-1 provides the limits on the transmit power control (TPC), the RF output power and power density given by the mean EIRP and the mean EIRP density at the highest power level. Additionally the requirements on the transmitter out of band emissions are listed in Table 2.1.1-2, Figures 2.1.1-3a, and Figure 2.1.1-3b. Table 2.1.1-4 illustrates the DFS requirements for some of these bands in Europe. Moreover, the 5150-5350 MHz is restricted to indoor deployments.

ETSI mandates the usage of DFS in some bands as shown in Table 2.1.1-4. Furthermore, a Listen-Before-Talk (LBT) mechanism is requested independently of whether the channel is occupied or not, the LBT parameters being given in Table 2.1.1-5a and 2.1.1-5b. Both the requirements on frame-based equipment and the requirements on load-based equipment are summarized in Table 2.1.1-5a and 2.1.1-5b, respectively. Note that no LBT requirement is requested in [4] for the FWA band.

Table 2.1.1-1: TPC, Transmit power and power spectral density requirements in Europe

|  | Freq. range (MHz) | Max Mean EIRP (dBm) | Max Mean EIRP density (dBm/MHz) | Comment |
| --- | --- | --- | --- | --- |
| WAS/RLAN | 5150-5350 | 23 | 10 | 20 MHz and 40 MHz channels |
|  | 5470-5725 | 30 | 17 |  |
| FWA | 5725-5875 | 33 | 23 | 10 MHz channels |
|  | 5725-5875 | 36 | 23 | 20 MHz channels |
| Transmit Power Control (TPC): - TPC ensures an average reduction in the aggregated transmission power by at least 3 dB (5 dB for FWA) compared with the maximum permitted transmission power. - TCP is not required for channels within the band 5150-5250 MHz. - Without TPC, the highest permissible average EIRP (density) are reduced by 3 dB. |  |  |  |  |

Table 2.1.1- 2: Requirements on out of band emissions in Europe

|  | Frequency range | Max. power | Bandwidth |
| --- | --- | --- | --- |
| WAS/RLAN, FWA | 30-47 MHz | -36 dBm | 100 kHz |
|  | 47-74 MHz | -54 dBm | 100 kHz |
|  | 75-87.5 MHz | -36 dBm | 100 kHz |
|  | 87.5-118 MHz | -54 dBm | 100 kHz |
|  | 118-174 MHz | -36 dBm | 100 kHz |
|  | 174-230 MHz | -54 dBm | 100 kHz |
|  | 230-470 MHz | -36 dBm | 100 kHz |
|  | 470-862 MHz | -54 dBm | 100 kHz |
|  | 0.862-1 GHz | -36 dBm | 100 kHz |
|  | 1-5.15 GHz | -30 dBm | 1 MHz |
|  | 5.35-5.5.47 GHz | -30 dBm | 1 MHz |
|  | 5.725-26 GHz | -30 dBm | 1 MHz |

NOTE:	dBc is the spectral density relative to the maximum spectral power density of the transmitted signal.

Figure 2.1.1-3a: Transmit spectral power mask for RLAN equipment operating within the frequency bands 5150-5250 MHz; 5250-5350 MHz or 5470-5725 MHz

NOTE 1: 0 dB Reference Level is the spectral density relative to the maximum spectral power density of the transmitted signal.

NOTE 2: On the Frequency Offset axis, the figures apply to ChS = 20 MHz whereas the figures in parentheses apply to ChS = 10 MHz.

NOTE 3: Emissions that fall outside the lower and upper band frequency limits of 5 725 MHz and 5 875 MHz,

respectively shall instead meet the unwanted emission limits of clause 4.3.1.

Figure 2.2.1-3b: Transmit spectral power mask for FBWA equipment operating within the frequency band 5725-5875 MHz

FWA devices in the 5.8 GHz range are also subject to an additional requirement of EIRP spectral density limit in the elevation plane, see Table 5.

Table 2.1.1-3: EIRP spectral density limits in the elevation plane (5.8 GHz frequency range)

| EIRP spectral density | Elevation angle |
| --- | --- |
| For sectorised (e.g. P-MP Central or Base Station) and Omni-directional deployments: |  |
| −7 dB(W/MHz) | 0° ≤ θ <4° |
| −2.2 - (1.2*θ) dB(W/MHz) | 4° ≤ θ ≤ 15° |
| −18.4 - (0.15*θ) dB(W/MHz) | θ > 15° |
| For P-MP Customer Terminal Station and P-P deployments: |  |
| −7 dB(W/MHz) | for 0° ≤ θ <8° |
| −2.68 -(0.54*θ) dB(W/MHz) | 8° ≤ θ < 32° |
| −20 dB(W/MHz) | 32° ≤ θ ≤50° |
| −10 - (0.2*θ) dB(W/MHz) | θ > 50° |

Table 2.1.1-4: DFS requirements in Europe

| Parameter | Requirement | Comments |
| --- | --- | --- |
| DFS Threshold (dBm) for WAS/RLAN | -62(dBm) + 10(dBm/MHz) - EIRP Spectral density (dBm/MHz) + G(dBi) | *No DFS requirements on 5150 MHz – 5250 MHz *G denotes antenna gain |
| DFS Threshold (dBm) for FWA | -69 (dBm) + 23(dBm/MHz) - EIRP Spectral density (dBm/MHz) + G(dBi) | *No DFS requirements on 5850 MHz – 5875 MHz *G denotes antenna gain |
| Channel Availability check | 60 seconds outside 5600-5650 MHz | Master mode |
|  | 10 minutes inside 5600-5650 MHz |  |
| Channel move time | 10 seconds | Master and slave modes |
| Non-occupancy time | 30 minutes | After radar detection in either channel availability check or in-service monitoring |
| Uniform Spreading is required across the frequency ranges 5150 -5350 MHz and 5470-5725 MHz. Uniform Spreading is not applicable for equipment that only operates in 5150-5250 MHz band. |  |  |

Table 2.1.1-5a: LBT requirements for Frame-Based-Equipment in Europe

| Parameter | Requirement | Comment |
| --- | --- | --- |
| Clear Channel Assessment (CCA) time | Minimum 20μs |  |
| Channel Occupancy time | Minimum 1 ms, maximum10 ms |  |
| Idle period | Minimum 5% of channel occupancy time |  |
| Fixed frame period | Equals to Channel Occupancy time + Idle Period |  |
| Short control signaling transmission time | Maximum duty cycle of 5% within an observation period of 50ms | Part of Channel occupancy time |
| CCA Energy detection threshold | Assuming receive antenna gain G=0dBi: If EIRP=23dBm at transmitter Threshold ≤ -73 dBm/MHz Otherwise (different transmit power levels, PH) Threshold = -73(dBm/MHz) + 23(dBm) – PH(dBm) | For WAS/RLAN |

Table 2.1.1-5b: LBT requirements for Load-Based-Equipment in Europe

| Parameter | Requirement | Comment |
| --- | --- | --- |
| Clear Channel Assessment (CCA) time | Minimum 20μs |  |
| Extended CCA time | Duration of a random factor N multiplied by the CCA observation time. N shall be randomly selected in the range 1..q every time, q=4…32 |  |
| Channel Occupancy time | <= (13/32) × q ms |  |
| Idle period | Extended CCA time |  |
| Short control signaling transmission time | Maximum duty cycle of 5% within an observation period of 50ms | Part of Channel occupancy time |
| CCA Energy detection threshold | Assuming receive antenna gain G=0dBi: If EIRP=23dBm at transmitter Threshold ≤ -73 dBm/MHz Otherwise (different transmit power levels, PH) Threshold = -73(dBm/MHz) + 23(dBm) – PH(dBm) | For WAS/RLAN |

Interference to Weather Radars

Interference to Weather Radars is also a hot topic in the EU. The two last versions of EN 301 893 (1.6.1 and 1.7.1) have included amendments to better protect these radars, like the prohibition to give the user access to the configuration control settings that would allow him to disconnect the DFS functionality.

The use of the band 5.60-5.65 GHz by WLANs is allowed in Europe, and a Market Surveillance campaign on WLANs 5 GHz has been led at EU level by the ADCO R&TTE Group. The report [18] of the campaign has proposed specific recommendations to improve the situation. These recommendations do not require amendments to the last version of EN 301 893.

## 2.1.2 Israel

In Israel the bands 5150-5250 MHz and 5250-5350 MHz are open to RLANs.

## 2.1.3 Russia

In the Russian Federation the bands 5150-5350 MHz, the band 5470-5725 MHz above 5650 MHz and the band 5725-5825 MHz [20] are allowed to RLANs. Use of DFS is not mandated.

## 2.1.4 South Africa

In South Africa the bands 5150-5250 MHz and 5250-5350 MHz are available to RLANs but restricted to indoor use. The band 5470-5725 MHz is also open to RLANs.

## 2.1.5 Turkey

In Turkey the bands 5150-5250 MHz and 5250-5350 MHz are restricted to indoor use. DFS and TPC are mandated in the band 5470-5725 MHz.

## ITU Region 2

## 2.2.1 USA

The use of unlicensed 5 GHz spectrum in USA is governed by FCC part 15 regulations [1]. In Feb 2013, potential new rules were proposed in FCC 13-22 [2]. Figure 2.2.1-1 summarizes the relevant part 15 rules for 5GHz unlicensed spectrum usage:

Figure 2.2.1-1: Summary of existing and proposed new FCC part 15 rules for 5GHz unlicensed spectrum usage

In Figure 2.2.1-1, U-NII-x bands denote frequency bands for Unlicensed National Information Infrastructure devices usage that are governed by §15.407 [1]. As shown in the figure, there is also an overlapping ruling of §15.247 from 5.725 to 5.85 GHz. A device could choose to follow either U-NII rulings or §15.247 rulings when operating within the frequency range.

In the rest of this section, we summarize FCC paragraphs 15.407 and 15.247 rules. In general either frequency hopping or digital modulation techniques are permitted under part 15 rules. Since LTE is not designed as a frequency hopping system, the rest of the document will focus on regulations related to digital modulation.

Paragraph 15.247 rules relevant for LTE point to multi-point communications are summarized in 4 aspects:

- Transmission Bandwidth:

  - The minimum 6 dB bandwidth shall be at least 500 kHz.

- Maximum Transmit Power:

  - Peak conducted output power shall not exceed 1 W. An alternative to peak power measurements is maximum conducted output power, which is the total transmit power over all antennas and antenna elements when the transmitter is operating at its maximum power control level.

  - Note that, the conducted output power limit is based on the use of antennas with directional gains that do not exceed 6 dBi. If transmitting antennas of directional gain greater than 6 dBi are used, the conducted output power from the intentional radiator shall be reduced by the amount in dB that the directional gain of the antenna exceeds 6 dBi.

- Out of Band Emission:

  - In any 100 kHz bandwidth outside the frequency band, the radio frequency power that is produced by the intentional radiator shall be at least 20 dB below that in the 100 kHz bandwidth within the band that contains the highest level of the desired power, based on either an RF conducted or a radiated measurement. If the transmitter complies with the conducted power limits based on the use of RMS averaging over a time interval, the attenuation required under this paragraph shall be 30 dB instead of 20 dB.

- Power Spectrum Density:

  - The power spectral density conducted from the intentional radiator to the antenna shall not be greater than 8 dBm in any 3 kHz band during any time interval of continuous transmission. The same method of determining the conducted output power shall be used to determine the power spectral density.

Paragraph15.407 rules for UNII devices are summarized in following tables. In Table 2.2.1-1, the maximum transmit power, PSD and out of band emission requirements are listed for UNII-1/2/3 bands. In Table 2.2.1-2, the dynamic frequency selection requirements for radar detection are summarized for UNII-2 devices.

Table 2.2.1-1: Transmit power requirements for UNII devices

|  |  | UNII-1 | UNII-2A | UNII-2C | UNII-3 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range (GHz) |  | 5.15 – 5.25 | 5.25-5.35 | 5.47-5.725 | 5.725-5.85 |  |
| Max conducted output power < min(a, b) (dBm) | a | eNB: 30 UE: 24 | 24 | 24 | 30 |  |
|  | b |  | 11+10logB | 11+10logB |  | B is the 26-dB emission bandwidth in MHz |
| Peak PSD (dBm/MHz) |  | eNB: 17 UE: 11 | 11 | 11 | 30dBm in 500kHz |  |
| Assumed Antenna Gain (dBi) |  | 6 | 6 | 6 | 6* | Peak power is reduced by G-6 dB for directional antennas with gain > 6 dBi; * UNII-3 fixed point to point operation power scaling threshold is 23 dBi |
| Out of band emission | Frequency Support (GHz) | Outside 5.15 – 5.35 | Outside 5.15 – 5.35 | Outside 5.47-5.725 | Outside 5.715-5.865 |  |
|  | EIRP (dBm/MHz) | -27 | -27 | -27 | -27 | Resolution bandwidth 1 MHz |
|  | Frequency Support (GHz) |  |  |  | 5.715-5.86 |  |
|  | EIRP (dBm/MHz) |  |  |  | -17 |  |
| Transmit Power Control |  | N/A | TPC to 6 dB below a mean EIRP of 30 dBm. No TPC for mean EIRP < 27 dBm |  | N/A |  |

Table 2.2.1-2: DFS requirements for UNII-2 Devices

|  | Levels | Comments |
| --- | --- | --- |
| Max EIRP (dBm) | 23 to 30 | * DFS power is averaged in 1 micro-second for 0 dBi antenna. * Uniform spread over available channels. |
| DFS Threshold (dBm) | -64 |  |
| Max EIRP (dBm) | <23 |  |
| DFS Threshold (dBm) | -62 |  |
| Channel Availability check | 60 seconds | Master mode |
| Channel move time | 10 seconds | Master and slave modes |
|  | 200 ms normal operation |  |
| Non-occupancy time | 30 minutes | After radar detection in either channel availability check or in-service monitoring |

Interference to Weather Radars

In order to resolve interference to Terminal Doppler Weather Radar (TDWR) the FCC has defined interim plans to approve UNII devices operating in the 5470-5725 MHz band [9]. These interim plans provide specific actions for equipment authorization and installation, as detailed below. The main elements in the interim plan are:

- Master devices shall not transmit on channels overlapping with the range 5600-5650 MHz band used by TDWRs;

- Professional installation of equipment operating within the band 5470-5720 MHz;

- Prohibition to include configuration controls (like country code settings) that would allow to change the frequency of operations to any frequency other than those specified on the grant of certification for US operation.

In parallel FCC is continuing its work to develop long-term equipment authorization test procedures that will ensure that the devices comply with the rules that protect the TDWR operations.

## 2.2.2	Canada

In Canada, the use of RLANs is forbidden in the band 5600-5650 MHz in order to protect the meteorological radars from interference caused by RLANs. The regulations for RLANs in the 5 GHz range in Canada are defined in RSS-210 Annex 9 [19]. Table 2.2.2-1 presents the transmit power requirements for RLAN devices while Table 2.2.2-1 provides the EIRP spectral density limits in the elevation plane required from RLAN devices operating with an EIRP level higher than 200 mW in the frequency range 5250-5350 MHz.

Table 2.2.2-1: Transmit power requirements for RLAN devices in Canada

| Frequency Range (GHz) |  | 5.15 – 5.25 | 5.25-5.35 | 5.47-5.60 and 5.65-5.725 | 5.725-5.825 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Max conducted output power < min(a, b) (dBm) | a |  | 24 | 24 | 30 |  |
|  | b |  | 11+10logB | 11+10logB | 17+10logB | B is the 26-dB emission bandwidth in MHz |
| Peak PSD (dBm/MHz) |  | 4 | 11 | 11 | 17 | Resolution bandwidth 1 MHz |
| Max e.i.r.p. < min(a, b) (dBm) | a | 23 | 30 | 30 | 36 |  |
|  | b | 10+10logB | 17+10logB | 17+10logB | 23+10logB |  |
| Max e.i.r.p density (dBm/MHz) |  | 10 |  |  |  | Resolution bandwidth 1 MHz |
| Out of band emission | Frequency Support (GHz) | Outside 5.15 – 5.25 | Outside 5.25 – 5.35 | Outside 5.47-5.725 | Outside 5.715-5.825 |  |
|  | EIRP (dBm/MHz) | -27 | -27 | -27 | -17 within 5.705-5.715 and 5.825-5.835; -27 outside | Resolution bandwidth 1 MHz |
| Transmit Power Control |  | N/A | TPC to 6 dB below a mean EIRP of 30 dBm. No TPC for mean EIRP < 27 dBm |  |  |  |
| DFS |  | N/A | Required |  |  |  |
| e.i.r.p. elevation mask |  | N/A | If e.i.r.p. > 23 dBm compliance with e.i.r.p. elevation mask required | N/A | N/A |  |

Table 2.2.2-1:  EIRP spectral density limits in the elevation plane for devices with EIRP > 200 mW

(5.25-5.35 GHz frequency range)

| EIRP spectral density | Elevation angle |
| --- | --- |
| −13 dB(W/MHz) | 0° ≤ θ <8° |

## 2.2.3 Brazil

In Brazil, nearly all the 5 GHz spectrum is allocated for Restricted Radiation, which means low-power unlicensed bands (i.e. any low-power-device can use it on a secondary basis). The bands 5250-5350 MHz, 5470-5650 MHz, 5650-5725 MHz and 5725-5850 MHz are allowed to RLANs [20]. Bands 5150-5250 MHz and 5250-5350 MHz are restricted to indoor use, and DFS is mandated in the bands 5250-5350 MHz and 5470-5725 MHz.

The relevant restrictions by band are as follows [21]:

| From (MHz) | To (MHz) | Service | Restriction [insert reference] |
| --- | --- | --- | --- |
| 5150 | 5350 | Restricted radiation | Indoor use only, EIRP limited to 200mW, EIRP spectral power density limited to 10mW/MHz. DFS mandated between 5250-5350MHz. |
| 5350 | 5470 | Unregulated |  |
| 5470 | 5650 | Restricted radiation | DFS mandated. Max transmitter output power limited to 250mW, EIRP limited to 1W, EIRP spectral power density limited to 50mW/MHz |
| 5650 | 5725 | Restricted radiation or amateur radio | DFS mandated. Max transmitter output power limited to 250mW, EIRP limited to 1W, EIRP spectral power density limited to 50mW/MHz |
| 5725 | 5875 | Restricted radiation (ISM Band) | Max transmitter output power limited to 1W, max EIRP EMF density of 50,000 microvolt per meter (measured at 3 meter distance) |

Note that the 5350-5470MHz band is not regulated.

## 2.2.4 Mexico

In Mexico, the bands 5150-5250 MHz, 5250-5350 MHz, 5470-5600 MHz, 5650-5725 MHz and 5725-5875 MHz are open to RLANs [20].

## ITU Region 3

## 2.3.1	China

The 5150-5350 MHz frequency band is open to unlicensed RLANs indoor deployment in China. Furthermore, mandatory DFS / TPC or DFS only with a 3 dB backoff of the max mean EIRP is required for 5250-5350 MHz.

The key regulatory restrictions include:

- EIRP: ≤200mW

- Power Spectrum Density: ≤10dBm/MHz (EIPR)

- Max Emission at edges fo the used frequency:  ≤-80dBm/Hz (EIRP)

- Spurious Emission (correponding to frequency range outside 2.5*carrier bandwidth )

  - 30-1000MHz: -36dBm/100kHz

  - 48.5-72.5MHz, 76-118MHz, 167-223MHz, 470-798MHz: -54dBm/100kHz

  - 2400-2483.5MHz: -40dBm/1MHz

  - 5150-5350MHz: -33dBm/100kHz

  - 5470-5850MHz: -40dBm/1MHz

  - Other frequency in 1-40GHz: -30dBm/1MHz

The 5725-5850 MHz frequency band is assigned as light licensed, shared among operators and traffic control bureau, open for both WAS (wireless access system) and RLAN, for both indoor and outdoor deployment in China. The key regulatory restrictions as below:

- Transmit Power: ≤500mW and ≤27dBm;

- EIRP: ≤2W and  ≤33dBm

- Power Spectrum Density: ≤13dBm/MHz and ≤19dBm/MHz(EIRP)

- Out of Band Emission:  ≤-80dBm/Hz(≤5725MHz and ≥5850MHz)

- Spurious emission

  - 30-1000MHz: ≤-36dBm/100kHz

  - 2400-2483.5MHz: ≤-40dBm/1MHz

  - 3400-3530MHz: ≤-40dBm/1MHz

  - 5725-5850MHz: ≤-33dBm/100kHz

  - correponding to frequency range outside 2.5*carrier bandwidth

  - Other frequency in 1-2483.5MHz: -30dBm/1MHz

The band 5470-5725 MHz  have not yet been open for RLAN(is put on hold).

## 2.3.2	Japan

With regard to the use of 5 GHz spectrum for RLAN in Japan, the following frequency bands are available:

- 	5150-5250 MHz;

- 	5250-5350 MHz;

- 	5470-5725 MHz.

Table 2.3.2-1 summarizes overview of technical regulatory requirements in Japan based on those for IEEE 802.11a/n/ac. As shown in the table, it should be noted that “Max Burst Length” is specified as less than 4 msec for RLAN systems in Japan.

Table 2.3.2-1: Summary of basic regulatory requirements

Note 1:DFS（Dynamic Frequency Selection）, TPC（Transmitter Power Control）

Figure 2.3.2-1 indicates the spectrum and channelling arrangements available for different channel bandwidth. This figure also provides information on incumbent systems to be coexisting with RLAN systems in Japan.

Figure 3.4-2: Spectrum and Channel arrangement

Further detailed regulatory requirements, such as ACLR, SEM and peak data rate for the respective frequency bands are provided in the following sub-sections.

### 2.3.2.1	5150-5250 and 5250-5350 MHz

Table 2.3.2.1-1: Adjacent Channel Leakage Ratio (ACLR) 1

| Occupied bandwidth | ≤ 18MHz | > 18MHz and ≤ 19MHz | > 19MHz and ≤ 38MHz | > 38MHz and ≤ 78MHz |
| --- | --- | --- | --- | --- |
| ACLR 1 | ≥ 25 dB | ≥ 25 dB | ≥ 25 dB | ≥ 25 dB |
| Measurement bandwidth | 18 MHz | 19 MHz | 38 MHz | 78 MHz |
| Adjacent channel centre frequency offset [MHz] | +20 / -20 | +20 / -20 | +40 / -40 | +80 / -80 |

Table 2.3.2.1-2: Adjacent Channel Leakage Ratio (ACLR) 2

| Occupied bandwidth | ≤ 18MHz | > 18MHz and ≤ 19MHz | > 19MHz and ≤ 38MHz | > 38MHz and ≤ 78MHz |
| --- | --- | --- | --- | --- |
| ACLR 2 | ≥ 40 dB | ≥ 40 dB | ≥ 40 dB | - |
| Measurement bandwidth | 18 MHz | 19 MHz | 38 MHz | - |
| Adjacent channel centre frequency offset [MHz] | +40 / -40 | +40 / -40 | +80 / -80 | - |

- Occupied bandwidth ≤ 18 MHz

Table 2.3.2.1-3: SEM: Occupied bandwidth ≤ 18 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5240 | 5140 ≤ f ≤ 5142 | ≥ 98 and ≤ 100 | ≤ 2.5 W |
|  | 5142 < f ≤ 5150 | ≥ 90 and < 98 | ≤ 15 W |
|  | 5250 ≤ f < 5251 | ≥ 10 and < 11 | ≤ 101-(f-9) mW |
|  | 5251 ≤ f < 5260 | ≥ 11 and < 20 | ≤ 10-1-(890)(f-11) mW |
|  | 5260 ≤ f < 5266.7 | ≥ 20 and < 26.7 | ≤ 10-1.8-(650)(f-20) mW |
|  | 5266.7 ≤ f ≤ 5360 | ≥ 26.7 and ≤ 120 | ≤ 2.5 W |
| 5260 | 5140 ≤ f ≤ 5233.3 | ≥ 26.7 and ≤ 120 | ≤ 2.5 W |
|  | 5233.3 < f ≤ 5240 | ≥ 20 and < 26.7 | ≤ 10-1.8-(650)(f-20) mW |
|  | 5240 < f ≤ 5249 | ≥ 11 and < 20 | ≤ 10-1-(890)(f-11) mW |
|  | 5249 < f ≤ 5250 | ≥ 10 and < 11 | ≤ 101-(f-9) mW |
|  | 5350 ≤ f ≤ 5360 | ≥ 90 and ≤ 100 | ≤ 2.5 W |

- 18 MHz < Occupied bandwidth ≤ 19 MHz

Table 2.3.2.1-4: SEM: 18 MHz < Occupied bandwidth ≤ 19 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5180 | 5135 ≤ f ≤ 5142 | ≥ 38 and ≤ 45 | ≤ 2.5 W |
|  | 5142 < f ≤ 5150 | ≥ 30 and < 38 | ≤ 15 W |
| 5240 | 5250 ≤ f < 5251 | ≥ 10 and < 11 | ≤ 101-(f-9) mW |
|  | 5251 ≤ f < 5260 | ≥ 11 and < 20 | ≤ 10-1-(890)(f-11) mW |
|  | 5260 ≤ f < 5266.7 | ≥ 20 and < 26.7 | ≤ 10-1.8-(650)(f-20) mW |
|  | 5266.7 ≤ f ≤ 5365 | ≥ 26.7 and ≤ 125 | ≤ 2.5 μW |
| 5260 | 5135 ≤ f ≤ 5233.3 | ≥ 26.7 and ≤ 125 | ≤ 2.5 μW |
|  | 5233.3 < f ≤ 5240 | ≥ 20 and < 26.7 | ≤ 10-1.8-(650)(f-20) mW |
|  | 5240 < f ≤ 5249 | ≥ 11 and < 20 | ≤ 10-1-(890)(f-11) mW |
|  | 5249 < f ≤ 5250 | ≥ 10 and < 11 | ≤ 101-(f-9) mW |
| 5320 | 5350 ≤ f ≤ 5365 | ≥ 30 and ≤ 45 | ≤ 2.5 W |

- 19 MHz < Occupied bandwidth ≤ 38 MHz

Table 2.3.2.1-5: SEM: 19 MHz < Occupied bandwidth ≤ 38 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5190 | 5100 ≤ f ≤ 5141.6 | ≥ 48.4 and ≤ 90 | ≤ 2.5 W |
|  | 5141.6 < f ≤ 5150 | ≥ 40 and < 48.4 | ≤ 15 W |
| 5230 | 5250 ≤ f < 5251 | ≥ 20 and < 21 | ≤ 10-f-20+log(1/2) mW |
|  | 5251 ≤ f < 5270 | ≥ 21 and < 40 | ≤ 10-8190f-21-1+log(1/2) mW |
|  | 5270 ≤ f < 5278.4 | ≥ 40 and < 48.4 | ≤ 10-350f-40-1.8+log(1/2) mW |
|  | 5278.4 ≤ f ≤ 5400 | ≥ 48.4 and ≤ 170 | ≤ 2.5 W |
| 5270 | 5100 ≤ f ≤ 5210 | ≥ 60 and ≤ 170 | ≤ 2.5 W |
|  | 5210 < f ≤ 5221.6 | ≥ 48.4 and < 60 | ≤ 2.5 W |
|  | 5221.6 < f ≤ 5230 | ≥ 40 and < 48.4 | ≤ 10-350f-40-1.8+log(1/2) mW |
|  | 5230 < f ≤ 5249 | ≥ 21 and < 40 | ≤ 10-8190f-21-1+log(1/2)mW |
|  | 5249 < f ≤ 5250 | ≥ 20 and < 21 | ≤ 10-f-20+log(1/2) mW |
| 5310 | 5350 ≤ f < 5358.4 | ≥ 40 and < 48.4 | ≤ 15 W |
|  | 5358.4 ≤ f ≤ 5400 | ≥ 48.4 and ≤ 90 | ≤ 2.5 W |

- 38 MHz < Occupied bandwidth ≤ 78 MHz

Table 2.3.2.1-6: SEM: 38 MHz < Occupied bandwidth ≤ 78 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5210 | 5020 ≤ f ≤ 5123.2 | ≥ 86.8 and ≤ 190 | ≤ 2.5 W |
|  | 5123.2 < f ≤ 5150 | ≥ 60 and < 86.8 | ≤ 15 W |
|  | 5250 ≤ f < 5251 | ≥ 40 and < 41 | ≤ 10-f-40+log(1/4) mW |
|  | 5251 ≤ f < 5290 | ≥ 41 and < 80 | ≤ 10-8390f-41-1+log(1/4) mW |
|  | 5290 ≤ f < 5296.7 | ≥ 80 and < 86.7 | ≤ 10-3100f-80-1.8+log(1/4) mW |
|  | 5296.7 ≤ f ≤ 5480 | ≥ 86.7 and ≤ 270 | ≤ 2.5 W |
| 5290 | 5020 ≤ f ≤ 5203.3 | ≥ 86.7 and ≤ 270 | ≤ 2.5 W |
|  | 5203.3 < f ≤ 5210 | ≥ 80 and < 86.7 | ≤ 10-3100f-80-1.8+log(1/4) mW |
|  | 5210 < f ≤ 5249 | ≥ 41 and < 80 | ≤ 10-8390f-41-1+log(1/4) mW |
|  | 5249 < f ≤ 5250 | ≥ 40 and < 41 | ≤ 10-f-40+log(1/4) mW |
|  | 5350 ≤ f < 5376.8 | ≥ 60 and < 86.8 | ≤ 15 W |
|  | 5376.8 ≤ f ≤ 5480 | ≥ 86.8 and ≤ 190 | ≤ 2.5 W |

- 78 MHz < Occupied bandwidth ≤ 158 MHz

Table 2.3.2.1-7: SEM: 78 MHz < Occupied bandwidth ≤ 158 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5250 | 4916 ≤ f ≤ 5099.6 | ≥ 150.4 and ≤ 334 | ≤ 2.5 W |
|  | 5099.6 < f ≤ 5150 | ≥ 100 and < 150.4 | ≤ 15 W |
|  | 5350 ≤ f < 5400.4 | ≥ 100 and < 150.4 | ≤ 15 W |
|  | 5400.4 ≤ f ≤ 5584 | ≥ 150.4 and ≤ 334 | ≤ 2.5 W |

Table 2.3.2.1-8: Peak data rate

| Occupied bandwidth(MHz) | Peak data rate (Mbps) |
| --- | --- |
| ≤ 19 | ≥ 20 |
| > 19 and ≤ 38 | ≥ 40 |
| > 38 and ≤ 78 | ≥ 80 |
| > 78 and ≤ 158 | ≥ 160 |

### 2.3.2.2	5470-5725 MHz

Table 2.3.2.2-1: Adjacent Channel Leakage Ratio (ACLR) 1

| Occupied bandwidth | ≤ 19.7MHz | > 19.7MHz and ≤ 38MHz | > 38MHz and ≤ 78MHz |
| --- | --- | --- | --- |
| ACLR 1 | ≥ 25 dB | ≥ 25 dB | ≥ 25 dB |
| Measurement bandwidth | 19 MHz | 38 MHz | 78 MHz |
| Adjacent channel centre frequency offset [MHz] | +20 / -20 | +40 / -40 | +80 / -80 |

Table 2.3.2.2-2: Adjacent Channel Leakage Ratio (ACLR) 2

| Occupied bandwidth | ≤19.7MHz | > 19.7MHz and ≤ 38MHz | > 38MHz and ≤ 78MHz |
| --- | --- | --- | --- |
| ACLR 2 | ≥ 40 dB | ≥ 40 dB | - |
| Measurement bandwidth | 19 MHz | 38 MHz | - |
| Adjacent channel centre frequency offset [MHz] | +40 / -40 | +80 / -80 | - |

- Occupied bandwidth ≤ 19.7 MHz

Table 2.3.2.2-3: SEM: Occupied bandwidth ≤ 19.7 MHz

| Frequency range(f) (MHz) | e.i.r.p/MHz |
| --- | --- |
| 5455 ≤ f ≤ 5460 | ≤ 2.5 W |
| 5460 < f ≤ 5470 | ≤ 12.5 W |
| 5725 ≤ f < 5740 | ≤ 12.5 W |
| 5740 ≤ f ≤ 5745 | ≤ 2.5 W |

- 19.7 MHz < Occupied bandwidth ≤ 38 MHz

Table 2.3.2.2-4: SEM: 19.7 MHz < Occupied bandwidth ≤ 38 MHz

| Frequency range(f) (MHz) | e.i.r.p/MHz |
| --- | --- |
| 5420 ≤ f ≤ 5460 | ≤ 12.5 W |
| 5460 < f ≤ 5470 | ≤ 50 W |
| 5725 ≤ f ≤ 5760 | ≤ 12.5 W |

- 38 MHz < Occupied bandwidth ≤ 78 MHz

Table 2.3.2.2-5: SEM: 38 MHz < Occupied bandwidth ≤ 78 MHz

| Frequency range(f) (MHz) | e.i.r.p/MHz |
| --- | --- |
| 5340 ≤ f ≤ 5460 | ≤ 12.5 W |
| 5460 < f ≤ 5469.5 | ≤ 50 W |
| 5469.5 < f ≤ 5470 | ≤ 51.2 W |
| 5725 ≤ f ≤ 5800 | ≤ 12.5 W |

- 78 MHz < Occupied bandwidth ≤ 158 MHz

Table 2.3.2.2-6: SEM: 78 MHz < Occupied bandwidth ≤ 158 MHz

| Frequency range(f) (MHz) | e.i.r.p/MHz |
| --- | --- |
| 5236 ≤ f ≤ 5419.6 | ≤ 12.5 W |
| 5419.6 < f ≤ 5470 | ≤ 50 W |
| 5725 ≤ f ≤ 5904 | ≤ 12.5 W |

Table 2.3.2.2-7: Peak data rate

| Occupied bandwidth(MHz) | Peak data rate (Mbps) |
| --- | --- |
| ≤ 19.7 | ≥ 20 |
| > 19.7 and ≤ 38 | ≥ 40 |
| > 38 and ≤ 78 | ≥ 80 |
| > 78 and ≤ 158 | ≥ 160 |

### 2.3.2.3	Simultaneous use of 5150-5250 and 5470-5725 MHz / 5250-5350 and 5470-5725 MHz

When simultaneous use of non-contiguous two channels in 5150-5250 and 5470-5725 MHz or that in 5250-5350 and 5470-5725MHz is employed, the following regulatory requirements are also applied. It should be note that total channel bandwidth is less and equal to 160 MHz in these usages.

Table 2.3.2.3-1: SEM: 78 MHz < Occupied bandwidth ≤ 158 MHz

| Case | Combination of simultaneous transmission Centre Frequencies |  |
| --- | --- | --- |
|  | Centre Frequency 1 (MHz) | Centre Frequency 2 (MHz) |
| 1 | 5210 | 5530 |
| 2 |  | 5610 |
| 3 | 5290 | 5530 |
| 4 |  | 5610 |

Side conditions

  - Occupied bandwidth for respective Carrier for Centre frequency is > 38 MHz and ≤ 78 MHz.

  - Maximum Output power ≤ 1.25 mW

  - e.i.r.p.≤ 1.25 mW

Table 2.3.2.3-2: Adjacent Channel Leakage Ratio (ACLR)

| Occupied bandwidth | > 38MHz and ≤ 78MHz |
| --- | --- |
| ACLR | ≥ 25 dB |
| Measurement bandwidth | 78 MHz |
| Adjacent channel centre frequency offset [MHz] | +80 / -80 |
| Note: Applicable to respective carrier |  |

Table 2.3.2.3-3: SEM: Case 1 and 2

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5,210 | 5,020 ≤ f ≤ 5,134.8 | ≥ 75.2 and ≤ 190 | ≤ 2.5 W |
|  | 5,134.8 < f ≤ 5,150 | ≥ 60 and < 75.2 | ≤ 12.5 W |
|  | 5,250 ≤ f < 5,251 | ≥ 40 and < 41 | ≤ 10-f-40+log⁡(1/8) mW |
|  | 5,251 ≤ f < 5,285.2 | ≥ 41 and < 75.2 | ≤ 10-8390f-41-1+log⁡(1/8) mW |
|  | 5,285.2 ≤ f < 5,370 | ≥ 75.2 and < 160 | ≤ 2.5 W |
| 5,530 | 5,370 ≤ f ≤ 5,454.8 | ≥ 75.2 and ≤ 160 | ≤ 2.5 W |
|  | 5,454.8 < f ≤ 5,470 | ≥ 60 and < 75.2 | ≤ 15 W |
| 5,610 | 5,725 ≤ f ≤ 5,800 | ≥ 115 and ≤ 190 | ≤ 15 W |

Table 2.3.2.3-4: SEM: Case 3 and 4

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5,290 | 5,020 ≤ f ≤ 5,214.8 | ≥ 75.2 and ≤ 270 | ≤ 2.5 W |
|  | 5,214.8 < f ≤ 5,249 | ≥ 41 and < 75.2 | ≤ 10-8390f-41-1+log⁡(1/8) mW |
|  | 5,249 < f ≤ 5,250 | ≥ 40 and < 41 | ≤ 10-f-40+log⁡(1/8) mW |
|  | 5,350 ≤ f < 5,365.2 | ≥ 60 and < 75.2 | ≤ 15 W |
|  | 5,365.2 ≤ f < 5,410 | ≥ 75.2 and < 120 | ≤ 2.5 W |
| 5,530 | 5,410 ≤ f ≤ 5,454.8 | ≥ 75.2 and ≤ 120 | ≤ 2.5 W |
|  | 5,454.8 < f ≤ 5,470 | ≥ 60 and < 75.2 | ≤ 15 W |
| 5,610 | 5,725 ≤ f ≤ 5,800 | ≥ 115 and ≤ 190 | ≤ 15 W |

Table 2.3.2.3-5: Peak data rate

Peak data rate (Mbps)

≥ 160

## 2.3.3	Korea

Table 2.3.3-1: Transmit power requirements for 5GHz Devices

| Frequency Range(MHz) | 5100-5250 |  | 5250-5350 |  | 5470-5650 |  | 5725-5825, 2400~2483.5 7) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average PSD requirement (mW/MHz) | BW(MHz) | PSD | BW(MHz) | PSD | BW(MHz) | PSD | BW(MHz) | PSD |
|  | 0.5-20 | ≤2.5 | 0.5-20 | ≤10 | 0.5-20 | ≤10 | 0.5-26 | ≤10 |
|  | 20-40 | ≤1.25 | 20-40 | ≤5 | 20-40 | ≤5 | 26-40 | ≤5 |
|  | 40-80 | ≤0.625 | 40-80 | ≤2.5 | 40-80 | ≤2.5 | 40-80 | ≤2.5 |
|  |  |  |  |  |  |  | 40-60 1) | ≤0.1 1) |
| Assumed antenna gain G (dBi) 4) | 6 |  | 7 |  | 7 |  | 6, 20 2) (PTP) 3) |  |
| Out of band emission, EIRP (dBm/MHz) | -27 |  | -27 |  | -27 |  | For any 100kHz in outside band, less than -30dBm(2400-2483.5MHz) -27dBm/MHz (5725-5825MHz) |  |
| DFS 5) | No |  | Yes |  | Yes |  | No |  |
| TPC 6) | No |  | Yes |  | Yes |  | No |  |
|  | Notes: 1) 2400-2483.5MHz devices 2 Fixed point to point operation power scaling threshold is 20 dBi 3) PTP: Point to Point communication 4) Peak power is reduced by G-THR dB for directional antennas with gain > THR dBi (THR = 6 dBi for 51005250MHz and 5725-5825MHz or 7dBi for 5250-5350MHz and 5470-5650MHz) 5) DFS: Dynamic Frequency Selection 6) TPC: Transmit Power Control 7) 5725-5825MHz, 2400-2483.5MHz are not allowed to be used for point-to-multipoint service for the same information and omni-directional electro-magnetic wave transmission |  |  |  |  |  |  |  |

Table 2.3.3-1: DFS requirements for 5GHz Devices

|  | Levels | Comments |
| --- | --- | --- |
| DFS Threshold (dBm) for interference detection | -62dBm | Average power considering antenna gain: <10mW/MHz |
|  | -64dBm | Average power considering antenna gain: 10mW/MHz ~50mW/MHz |
| Channel availability check time | > 60 seconds |  |
| Channel move time | < 10 seconds |  |
| Non-occupancy time | > 30 minutes | After radar detection in either channel availability check or in-service monitoring |

Korean regulatory requirements are summarized as follows:

- Average PSD requirement (mW/MHz)

  - It covers both power spectral density and Max transmission power

  - The maximum PSD is defined for a given spectrum range

  - Maximum transmission power :

    - 50mW, 200mW, 200mW and 200/260mW for 5150-5250, 5250-5350, 5470-5650, and 5725-5825 MHz band  respectively

- Antenna gain assumed:

  - 6 dBi for 5150-5250 and 5725-5825MHz band

  - 7 dBi for 52505350 and 5470-5650MHz band

- Out of Band Emission (EIRP (dBm/MHz) )

  - <-27 dBm/MHz

- DFS & TPC

  - DFS is defined for 5250-5350MHz and 5470-5650MHz

  - TPC is defined for 5250-5350MHz and 5470-5650MHz

    - TPC ensures wireless devices with average Tx power including antenna gain larger than 25mW/MHz can reduce its average Tx power by at least 3dB (below 12.5mW/MHz)

- Maximum bandwidth occupancy

  - < 80MHz for 5GHz

- Maximum power spectral density for bandwidth aggregation (contiguous or non-contiguous)

  - Among 5150-5250, 5250-5350, 5470-5650, and 5725-5825 MHz bands, multiple of 80MHz bandwidth can be aggregated in contiguous or non-contiguous manner to form 160MHz bandwidth

  - In this case, maximum power spectral density for 5150-5250MHz should be lower than 0.625mW/MHz while it should be lower than 1.25mW/Hz in the other bands

- Modulation scheme

  - Digital modulation for 5GHz

- Difference from FCC regulation

  - Maximum transmission power in case of 5725 to 5825MHz is still 200mW for 20MHz bandwidth (a bit lower compared to 1W power in UNII-3)

  - 5825-5850MHz is not for WAS (Wireless Access System)

## 2.3.4	India

In India the bands 5150-5250 MHz, 5250-5350 MHz and 5725-5875 MHz are open to RLANs [20].

## 2.3.4	Taiwan

In Taiwan the bands 5250-5350 MHz, 5470-5590 MHz, 5650-5725 MHz and 5725-5850 MHz are allowed to RLANs [20].

## 2.3.5	Singapore

In Singapore the bands 5150-5250 MHz, 5250-5350 MHz, 5470-5725MHz and 5725-5850 MHz are open to RLANs.

## 2.3.6	Australia

In Australia the band bands 5150-5250 MHz, 5250-5350 MHz and 5470-5725 MHz, except the sub-band 5600-5650 MHz, are open to RLANs. DFS and TPC are mandatory in the bands 5250-5350 MHz and 5470-5725 MHz.

# References

[1] FCC Part 15 ruling, http://www.ecfr.gov/cgi-bin/text-idx?SID=3c5e2d1533490603e0131fcdc041030d&node=pt47.1.15&rgn=div5

[2] FCC 13-22, “Notice of proposed rulemaking” , Feb 20, 2013.

[3] ETSI EN 301 893, Harmonized European Standard, “Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN”

[4] ETSI EN 302 502, Harmonized European Standard, “Broadband Radio Access Networks (BRAN); 5,8 GHz fixed broadband data transmitting systems”

[5] ETSI EN 302 571, Harmonized European Standard, “Intelligent Transport Systems (ITS); Radio communications equipment operating in the 5 855 MHz to 5 925 MHz frequency band”

[6]	“Commission decision of 11 July 2005 on the harmonised use of radio spectrum in the 5 GHz frequency band for the implementation of wireless access systems including radio local area networks (WAS/RLANs)” (2005/513/EC).

[7]	“Commission decisions of 12 February 2007 amending Decision 2005/513/EC on the harmonised use of radio spectrum in the 5 GHz frequency band for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLANs)” (2007/90/EC).

[8]	ECC/DEC (04)08, “ECC Decision of 09 July 2004 on the harmonised use of the 5 GHz frequency bands for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLANs)”

[9] KDB 443 999, FCC Office of Engineering and Technology – Laboratory Division: “Interim Plans to Approve UNII Devices Operating in the 5470-5725 MHz Band with Radar Detection and DFS Capabilities” (14th October 2010).

[10] FCC 12-148, “Notice of Proposed Rulemaking and Order: amendment of the Commission’s Rules with regard to Commercial Operations in the 3550-3650 MHz Band” (GN Docket No. 12-354), adopted and released December 12, 2012.

[11] FCC 13-154, “Public Notice. Commission Seeks Comment on Licensing Models and Technical Requirements in the 3550-3650 MHz Band”, released November 1, 2013

[12] ECC Recommendation ECC/REC (06)04: “Use of the band 5725- 5875 MHz for Broadband Fixed Wireless Access (BFWA)”

[13] Commission Decision 2008/671/EC of 5th August2008 on the harmonised use of radio spectrum in the 5875-5905 MHz frequency band for safety related application of Intelligent Transport Systems (IOTS)

[14] ECC Decision (08)01: "ECC Decision of 14 March 2008 on the harmonized use of the 5875-5925 frequency band for Intelligent Transport Systems (ITS)”

[15] ECC Recommendation (08)01:"Use of band 5855-5875 MHz for Intelligent Transport Systems

(ITS)".

[16] ETSI EN 300 440-1 v1.6.1: “Electromagnetic compatibility and Radio spectrum Matters (ERM); Short range devices; Radio equipment to be used in the 1 GHz to 40 GHz frequency range; Part 1: Technical characteristics and test methods”

[17] Document RSCOM 13-32rev3: “Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for Wireless Access Systems including Radio Local Area Networks in the bands 5350-5470 MHz and 5725-5925 MHz ('WAS/RLAN extension bands') for the provision of wireless broadband services”

[18] Group of Administrative Co-operation Under the R&TTE Directive (ADCO R&TTE): Report on the 5th joint cross-border R&TTE Market Surveillance Campaign on WLAN 5 GHz (2013)

[19] RSS-210 Issue 8 (December 2010): Licence-exempt Radio Apparatus (All Frequency Bands): Category I Equipment

[20] IEEE 802.11-12/1159r3: Current view/existing channelization for 20/40/80 MHz.

[21] http://legislacao.anatel.gov.br/resolucoes/2008/104-resolucao-506 , Resolução nº 506, de 1º de julho de 2008, Regulamento sobre Equipamentos de Radiocomunicação de Radiação Restrita.