3GPP TSG RAN WG1 Meeting #80bis		R1- 152380

Belgrade, Serbia, 20th – 24th April 2015

Source:	CATT

Title:	Proposals for Rel-13 CA UL enhancements

Agenda Item:	7.2.2.2.2

Document for:	Discussion/Decision

Proposals

For calibration purpose only, the following assumptions are used to generate the UL SINR CDF:

- Deployment scenario: Rel-12 SCE2a

  - PUCCH carrier frequency

    - Case 1: All UEs transmit PUCCH on the macro carrier frequency

    - Case 2: UEs transmit PUCCH either on the macro carrier frequency or on the small cell carrier frequency, based on RSRP/RSRQ

  - Carrier number for small cells: 1

  - Total BS TX power (Ptotal per carrier) for small cell eNB: 30dBm

  - Number of clusters/buildings per macro cell geographical area: 1

  - Number of small cells per cluster: 4

  - Number of macro sites: 19

- PUCCH power control

  - is set such that the received PUCCH power at the UE’s serving eNB is 20 dB above AWGN variance

- Same UE PUCCH power control is applied in each cell.

- Number of interferers  in each neighbor cell is 1

- The interferer in each cell in each subframe is randomly selected.

- eNB noise figure

  - Macro eNB: 5dB

  - Small cell eNB: 7dB

- Outputs

  - Case 1: All UEs transmit PUCCH on the macro carrier frequency

    - UL SINR CDF of all UEs

    - UL SINR CDF of a UE at {90%, 50%, 10%} DL geometry

  - Case 2: UEs transmit PUCCH either on the macro carrier frequency or on the small cell carrier frequency, based on RSRP/RSRQ

    - UL SINR CDF of all UEs transmitting PUCCH on the small cell carrier frequency

    - UL SINR CDF of a UE at {90%, 50%, 10%} DL geometry of all UEs transmitting PUCCH on the small cell carrier frequency

    - UL SINR CDF of all UEs transmitting PUCCH on the macro cell carrier frequency

    - UL SINR CDF of a UE at {90%, 50%, 10%} DL geometry of all UEs transmitting PUCCH on the macro cell carrier frequency

For information

- Step 1: Obtain the link level performance (i.e. BER/BLER vs. SNR) for a PUCCH structure A and a certain payload size S, i.e. BER/BLER at SNR ;

- Step 2: Obtain the UL SINR PDF for a particular UE, i.e. the probability density function at SNR ;

- Step 3: Calculate the average performance by ;

- Step 4: If  is below the HARQ-ACK performance requirement (e.g. 0.1% BER), then it is said that the UE can support the payload size S using the PUCCH structure A given the assumed assumptions; otherwise, it is said that the UE cannot support the payload size S using the PUCCH structure A given the assumed assumptions.