3GPP TSG RAN WG1 Meeting #83                                                                  R1-156631

Anaheim, USA, 15th – 22nd November 2015

Source:	ZTE

Title:	Discussion and Numerical Validation on the ML Receiver for Gray Mapping Scheme of MUST

Agenda Item:	6.2.7.2

Document for: 	Discussion and Decision

# Introduction

Receiver is an important aspect in MUST study. Suitable transmission scheme would improve the tolerance of receiver to interference. Some technical details of ML receiver design for MUST are discussed in this contribution, with link-level numerical validation. The benefit of Gray Mapping scheme on ML receiver is further analyzed.

# Signal Constellations in NAICS and MUST

In NAICS (Network Assisted Interference Cancellation/Suppression), the channel h1 carrying the target signal s1 and the channel h2 carrying the interference signal s2 are different. The power P of the two signal s1 and s2 is the same, as shown by Fig. 3. The signal s received by UE1 is obtained as below:

s=h1s1+h2s2                                                                   (1)

Here h1 and h2 are complex numbers. So the summation of the constellation for s1 and the constellation for s2 includes rotation. The shape of the constellation of s in general is arbitrary, as shown by Fig. 1. In the irregular constellation, the neighboring relations and distances between constellation points are complicated, which tends to increase the bit error rate (BER).

Figure 1 Signal constellations of NAICS

However, in MUST, the target signal s1 and the interference signal s2 go through the same channel h1. The transmitted signal s of MUST is composed of the target signal s1 with power P and the interference signal s2 with power (1-)P. The transmitted signal s is indicated as the formula below:

s=h1μs1+1-μs2                                                        (2)

where μ is a real number. So the summation of the constellation for s1 and the constellation for s2 would not involve any rotation. Compared with the received irregular constellation in NAICS, the received constellation is always rectangular in MUST if the same precoder is used for pair users. It is expected that demodulation based on regular constellation is smaller than that based on irregular constellation.

Figure 2 Signal constellations of MUST

A symbol level superposition scheme with Gray Mapping is applied to the regular constellation in MUST. It is described in [1].

# Gray Mapping Scheme of MUST

An implementation of the gray mapping scheme is shown in Fig. 3 where information bits of UE1 and UE2 are first encoded. The coded bits of near UE (UE1) XOR the coded bits of the far-away UE (UE2). Then, the modified bits of UE1 and the original bits of UE2 go through each of their modulators separately. In the end, amplitude-weighted symbols of UE1 and UE2 are summed up.

Figure 3 Symbol level superposition with Gray mapping by XOR operation before modulation

An example is illustrated in Fig. 4 where QPSK modulation is used for both UEs. The power ratio is 4: 1. The far-away high powered user (UE2) uses the legacy Gray mapping for its constituent modulation. For the near low powered user (UE1), its constellation depends on the coded bits of UE2. In the case that the coded bits of UE2 are “10”, and the coded bits of UE1 are “10”, the result of XOR is “00”. As a result, the superimposed constellation has Gray mapping property.

Figure 4 An example of achieving Gray constellation via XOR operation. Two QPSK signals with power ratio = 4: 1

# The Benefit of Gray Mapping Scheme on ML Receiver

Methodology for ML/R-ML modeling is described in [2] . ML receiver in MUST ML receiver implements symbol-level interference suppression. ML receiver can obtain MIB (the mutual information per transmitted bit).

MIBML=β∙CBICM                                                                        (3)

where MIBML is the received MIB of ML receiver at an RE. CBICM is the BICM (the bit interleaved coded modulation) normalized spectral efficiency. β is a weighting factor and subjected to such constraints

0<β<1                                                                               (4)

The BICM normalized spectral efficiency CBICM,near of near-user is given by the formula below [2]:

CBICM,near=1-1mneari=0mnear-1Et,zlog2starget∈Mpzstargetstarget∈Mneari,tpzstarget                           (5)

The BICM normalized spectral efficiency CBICM,far of far-user is given by the formula below[2]:

CBICM,far=1-1mfari=0mfar-1Et,zlog2starget∈Mpzstargetstarget∈Mfari,tpzstarget                           (6)

where starget is a modulated symbol. mnear and mfar are the number of bits carried by starget for the near-user and far-user, respectively. M is the set composed by all of the constellation points. Mneari,t and Mfari,t are the sets composed by the constellation points whose ith bit is t for the near-user and far-user, respectively. t∈0,1.  pzstarget is a transition probability of starget based on the received signal z. pzstarget is determined by the Euclidean distance of  z and starget. An example is shown as Fig. 5.

- (b)

Figure 5 An example for the computation of the BICM normalized spectral efficiency

Fig. 5 shows an example of the LLR calculation of the third bit. The solid triangle represents received signal z. Figure 5(a) is the sketch map indicating the distances between z and all modulated symbols starget whose binary numerical value on the third bit is 0. Figure 5(b) is the sketch map indicating the distances between z and all modulated symbols starget whose binary numerical value of the third bit is 1. It is obvious that the third bit of the correct symbol corresponding to z should be 0.

Gray Mapping scheme on constellation can help ML receiver to improve the demodulation, as shown by Fig. 6.

- (b)

Figure 6 Benefit of Gray Mapping on demodulation

The pink triangle represents received signal z in Fig. 6. Each composite symbol carries 6 bits. The first two bits are for far UE, while the last four bits are for near UE. It is observed that Gray Mapping scheme has more positive effect on the demodulation for near UE than Non-Gray Mapping. And Gray Mapping scheme and non-Gray Mapping scheme have the same effect on the demodulation for far UE.

As shown in the Fig. 6(a), based on non-Gray Mapping constellation, the nearest constellation points around signal z have different binary value in the second last bit. This causes some interference. In contrast, as shown in Fig. 6(b), based on Gray Mapping constellation, the nearest constellation points around signal z have the same binary numerical value in all the last four bits. This would benefit the detection.

# Link-level Numerical Validation

The BLER vs. SNR curves are shown in Figs. 7, 8 and 9 where solid lines correspond to Gray mapping scheme and dotted lines are for non-Gray mapping scheme. ML receiver is simulated in two ways. One is using L2S model, the so-called virtual detector without actual bits/symbols going through the channel. The other is with the actual ML receiver, the so-called true detector with actual bits/symbols. BLER curves of virtual ML detector are marked with “predicted” in squares. BLER curves of true ML detector are marked with “reference” in circles. Three power radios are simulated: 0.65, 0.75 and 0.85. MCS of far UE is fixed to 2. MCS of near UE is 6, 9 and 12, respectively.

Figure 7 BLER curves of ML receiver for MCS_F = 2, MCS_N = 6

Figure 8 BLER curves of ML receiver for MCS_F = 2, MCS_N = 9

Figure 9 BLER curves of ML receiver for MCS_F = 2, MCS_N = 12

As observed in Figs. 7, 8 and 9, the predicted BLER curves match well with the reference BLER curves. It verifies the accuracy of our L2S model. The benefit of Gray Mapping over Non-Gray Mapping on ML receiver is obvious. The smaller the power ratio of far user is, the more benefit of Gray Mapping is. The smaller MCS of near UE is, the more benefit of Gray Mapping.

# Conclusion

Technical details of ML receiver design for MUST were provided in this contribution. Link-to-system model was validated. The benefit of the Gray Mapping scheme over Non-Gray Mapping scheme was observed for ML receiver.

# References

- 3GPP R1-154056, Symbol superposition based Gray Mapping scheme for MUST, ZTE, August 2015.

- 3GPP R1-154928, Text Proposal for Annex A.3 of TR 36.859, NTT DOCOMO, Intel, August 2015.