3GPP TSG RAN1 Meeting #83								R1-157637

Anaheim, CA, USA, 16th– 20th November, 2015

Source: 		Ericsson

Title:	NB-IoT - Battery lifetime evaluation for inband operation

Agenda Item:	6.2.6.1.3

Document for:	Discussion and decision

# Introduction

At RAN#69 a new work item on narrowband IoT and tasked RAN1 to evaluate two numerology options for both UL and DL:

- DL: 15 kHz sub-carrier spacing (with normal or extended CP) and 3.75 kHz sub-carrier spacing.

- UL: FDMA with GMSK modulation and SC-FDMA (including single-tone transmission as a special case of SC-FDMA)

One of the performance objectives is for the IoT device to meet the battery lifetime requirement of 10 years. Even if a possible down selection of DL and UL numerology could be seen as independent evaluations, in order to estimate the battery lifetime, a system with both DL and UL must be assumed for the evaluation. For this reason, in this contribution we have evaluated a NB-IoT system as described in [3] in the inband operation using the methodology given in TR45.820 [1]. The evaluation in [2] has been updated by aligning with the assumptions used in [1] for evaluations of NB-CIoT.

This is an update of [8] updating the result with the new PUSCH results as found in [9].

# NB-LTE energy consumption evaluation

## Updated Assumptions

This analysis follows the methodology as described in [2] with the following updates:

- The transmission times for PUSCH have been updated as shown in [9].

- Update to latest PBCH design [6]

- In order to align with the evaluation in section 7.3.6.4 in [1], the effect of BLER has only been included for PUSCH and for other channels no retransmissions are assumed.

- In order to align with the evaluation in section 7.3.6.4 in [1] and [5], the UE only wakes up three times during the 20 s ready timer. During the DRX, the UE is in standby mode.

## Results

The overall reception, transmission and idle times per report at different coverage based on the new assumptions are shown in Table 1.

Table 1 Transmission, Reception and Idle time per report (ms)

|  | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| Tx (50 bytes) | 48 | 268 | 1740 |
| Tx (200 bytes) | 81 | 584 | 3566 |
| Rx | 175 | 486 | 2387 |
| Idle | 1271 | 1505 | 3123 |

The estimated lifetime in years are presented in Table 2for two different packet sizes, two reporting intervals and at different coverage.

Table 2 estimated battery lifetime in years

| Packet size, reporting interval | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| 50 bytes, 2 hours | 26.7 | 13.6 | 3.2 |
| 200 bytes, 2 hours | 23.9 | 8.7 | 1.8 |
| 50 bytes, 1 day | 36.7 | 33.1 | 20.0 |
| 200 bytes, 1 day | 36.3 | 29.7 | 14.2 |

We see that with the updated assumptions in section 2.1, the estimated battery life for NB-LTE is further improved fulfilling the 10 year target for battery lifetime and meets the 10 year requirement for the same scenarios as when operating standalone[7].

Observation 1:

- NB-LTE can meet the battery target of 10 years for the same scenarios when operating inband as for standalone operation.

# Conclusions

In this contribution we evaluated battery lifetime of the NB-LTE when operating inband and made the following observation.

Observation:

- A NB-LTE can meet the battery target of 10 years for the same scenarios when operating inband as for standalone operation.

# References

TR 45.820 v13.0.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

R1-156019, “NB LTE – Exception report latency evaluation in inband operation”, Ericsson

R1-156013, “NB-LTE – General L1 concept description”, Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung

R1-156009, “Narrowband LTE – Synchronization Channel Design and Performance”, Ericsson

GP-150137, “NB M2M – Battery life analysis”, Neul

R1-157422, “NB-IoT - M-PBCH design update”, Ericsson

R1-157390, “NB-IoT - Battery lifetime comparison for standalone operation”, Ericsson

R1-157411, “NB-IoT - Battery lifetime evaluation for inband operation”, Ericsson

R1-157634, “NB-IoT - Battery lifetime comparison for standalone operation”, Ericsson