3GPP TSG RAN WG1 Meeting #83                                                                 	                      R1-157687

Anaheim, USA, 15th – 22nd November 2015

Source:	Panasonic

Title: 	Evaluation result on RV cycling

Agenda Item:		6.2.1.5

Document for:	Discussion and Decision

Introduction

We provide our evaluation result on RV cycling for PUSCH.

Evaluation result

The impact of RV cycling (Z=1 or Z=4) and symbol level combining for PUSCH repetition is evaluated in the condition of EPA, fD = 1 Hz, and RFO = 100 Hz. The detailed parameters are listed in the Table 1 simulation result with BLER performance is shown in Fig. 1. When Z=4, symbol level combining with 4 subframes (X=4) and frequency hopping with hopping interval of 4 subframes (Ych=4) are used. When Z=1, no symbol level combining is used and frequency hopping with Ych=4 is used.

Table 1	Simulation parameters

| Parameters | Values |
| --- | --- |
| Number of antennas | 1 × 2 with low correlation |
| Number of repetitions | NRep = 8, 16, 32, 64, 128 |
| Channel model | EPA(Doppler frequency, fD = 1 Hz) |
| Format | PUSCH 1RB |
| MCS | 9 |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz (The frequency difference of frequency hopping is 10 MHz.) |
| Residual frequency offset (RFO) | 100 Hz |
| Channel estimation | Realistic |
| Equalization | MMSE-FDE |

Fig.1	BLER performance of PUSCH with RV cycling (Z=1 or Z=4).

Observation: RV cycling with Z=4 and symbol level combing provides better performance than RV cycling with Z=1 in large repetition case (R>=8).

Conclusion

In this contribution, we provided link performance of PUSCH with RV cycling (Z=1 or Z=4) and symbol level combining. Based on the simulation results, we provide the following observations:

Observation: RV cycling with Z=4 and symbol level combing provides better performance than RV cycling with Z=1 in large repetition case (R>=8).