3GPP TSG RAN1 Meeting #84bis 	R1-162126

# Busan, South Korea, April 11-15, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | xxxx | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of larger max TBS for TDD UL for eMTC in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATR, CATT, CMCC, Potevio, Spreadtrum, Xinwei |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To increase maximum UL TBS support for eMTC in TDD |  |  |  |  |  |  |  |  |
| Summary of change: |  | For a BL/CE UE, the modulation order and TBS is determined according to table 8.6.1-2 for serving cell with frame structure type 1 and serving cell with uplink-downlink configuration 0, 1 and 6, and new table 8.6.1-3 for serving cell with uplink-downlink configuration 2, 3, 4, 5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The peak rate of eMTC UEs in typical TDD UL-DL configurations is very low. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.1, 8.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 36.212, TS 36.306 |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 8.6.1	Modulation order and redundancy version determination

#### < Unchanged parts are omitted >

For a BL/CE UE, the modulation order is determined according to table 8.6.1-2 for serving cell with frame structure type 1 and serving cell with uplink-downlink configuration 0, 1 and 6, and table 8.6.1-3 for serving cell with uplink-downlink configuration 2, 3, 4, 5. A BL/CE UE configured with CEModeB is not expected to receive a DCI format 6-0B indicating .

For BL/CE UEs, the same redundancy version is applied to PUSCH transmitted in a given block of  consecutive subframes. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies . Denote  as the subframe number of the first uplink subframe intended for PUSCH. The PUSCH transmission spans  consecutive subframes including non-BL/CE subframes where the PUSCH transmission is postponed. For the  block of consecutive subframes, the redundancy version (rvidx) for PUSCH is determined according to Table 7.1.7.1-2 using , where  and. The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured in CEModeA,  and  is determined by the ‘Redundancy version’ field in DCI format 6-0A. For a BL/CE UE configured with CEModeB,  for FDD and  for TDD, and .

Table 8.6.1-2: Modulation and TBS index table for PUSCH for serving cell with frame structure type 1 and serving cell with uplink-downlink configuration 0, 1 and 6

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 4 | 10 |
| 12 | 4 | 11 |
| 13 | 4 | 12 |
| 14 | 4 | 13 |
| 15 | 4 | 14 |

Table 8.6.1-3: Modulation and TBS index table for PUSCH for serving cell with uplink-downlink configuration 2, 3, 4, 5

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 4 | 10 |
| 12 | 4 | 11 |
| 13 | 4 | 12 |
| 14 | 4 | 13 |
| 15 | 4 | 14 |
| 16 | 4 | 15 |
| 17 | 4 | 16 |
| 18 | 4 | 17 |
| 19 | 4 | 18 |
| 20 | 4 | 19 |
| 21 | 4 | 19 |
| 22 | 4 | 20 |
| 23 | 4 | 21 |

### 8.6.2	Transport block size determination

#### < Unchanged parts are omitted >

For a BL/CE UE configured with CEModeA, the UE shall first determine the TBS index () usingand Table 8.6.1-2 for serving cell with frame structure type 1 and serving cell with uplink-downlink configuration 0, 1 and 6, and table 8.6.1-3 for serving cell with uplink-downlink configuration 2, 3, 4, 5. For a BL/CE UE the TBS is determined by the procedure in subclause 7.1.7.2.1.

#### < Unchanged parts are omitted >