3GPP TSG RAN WG1 Meeting #84bis	                 R1-162197

April 11th - 15th 2016

Busan, Korea

Agenda Item:	8.1.3

Source:	Qualcomm Incorporated

Title:	Other KPI evaluation assumptions

Document for:	Discussion

# Introduction

With regard to the Rel-14 RAN study, and latest Technical Report, on Scenarios and Requirements for Next Generation Access Technologies [1], this paper provides some evaluation considerations and proposals for the following scenarios: High speed and Long range scenarios.

# Background

This section includes scenarios attributes, as currently defined in the RAN TR [1], mainly for background and reference.

## High Speed

Table 1. Attributes for High Speed scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro NOTE2 only: Around 4GHz (Tbc) Macro NOTE2+ relay nodes: 1) For BS to relay: Around 4 GHz For relay to UE: Around 30 GHz or Around 70 GH or Around 4 GHz 2) For BS to relay: Around 30 GHz For relay to UE: Around 30 GHz or Around 70 GHz or Around 4 GHz |
| Aggregated system bandwidth NOTE3 | Around 4GHz: Up to 200 MHz (DL+UL) Around 30GHz or Around 70GHz: Up to 1GHz (DL+UL) |
| Layout | Option 1: Macro only Option 2: Macro + relay nodes NOTE3 |
| ISD | Macro cell: ISD = 1732m Small cell within carriages: ISD = 25m |
| BS antenna elements NOTE4 | Around 30GHz: Up to 256 Tx and Rx antenna elements Around 4GHz or Around 2GHz: Up to 256 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Relay Tx: Left to RAN1 study Relay Rx: Left to RAN1 study Around 30GHz: Up to 32 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements |
| User distribution and UE speed | 100% of users in train [100] UEs per macro cell (assuming 1000 passengers per high-speed train and at least 10% activity ratio) Maximum mobility speed: 500km/h |
| Service profile | Note: Whether to use full buffer traffic or non-full-buffer traffic is FFS. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 24 GHz – 40 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.  A range of bands from 66 GHz – 86 GHz identified for WRC-19 are currently being considered and around 70 GHz is chosen as a proxy for this range

NOTE2:	For Macro, it is assumed RRH sharing the same cell ID as captured in Section 6.2 of [5].

NOTE3:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

NOTE5:	BS to relay link should be paid more attention than relay to UE link.

## Long Range

Two scenarios are defined in the TR: extreme rural and extreme long range.

Table 2. Attributes for extreme rural

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Below 3 GHz With a priority on bands below 1GHz Around 700 MHz |
| System Bandwidth | 40 MHz (DL+UL) |
| Layout | Single layer: Isolated Macro cells |
| Cell range | 100 km range (Isolated cell) to be evaluated through system level simulations. 100 km is a starting point, and may be varied to meet traffic density and user experienced data rate targets Feasibility of Higher Range shall be evaluated through Link level evaluation (for example in some scenarios ranges up to 150-300km may be required). |
| User density and UE speed | [TBD] users/km² Speed up to 160 km/h |
| Traffic model | [Average data throughput at busy hours/user: 30 kbps Traffic density: [TBD] kbps/km² User Experienced Data Rate: up to 2 Mbps while stationary and 384 kbps while moving] |

Table 3. Attributes for extreme long range

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Below 3 GHz |
| System Bandwidth | [40] MHz (DL+UL) |
| Layout | Single layer: Isolated Macro cells ad hoc / isolated |
| Cell Range | [150 km] for bands above 1GHz [250 km] for between 700 MHz and 1 GHz [400 km] or more for bands below 700 MHz |
| User density and UE speed | [TBD ] users/km² Speed up to [160km/h] |
| Traffic model | [Average data throughput at busy hours/user: [30kbps] Traffic density: [380-500kbps/km²] User Experienced Data Rate: up to [2]Mbps while stationary and [384kbps] while moving] |

# Discussion and Proposals

This section includes some initial considerations and proposals for the evaluation of the above scenarios.

## High Speed

For the High Speed scenario, it is proposed to focus on the following evaluations and KPIs.

a) Macro + Relay: Focus on Link-level simulation of the BS-to-relay link (e.g. to validate max speed of 500km/h).

b) Macro-only: It may be good to perform some system level evaluation looking at e.g. mobility (Idle/Connected), and UE battery performance. Spectral efficiency and capacity related KPIs can be also considered (perhaps as secondary KPIs).

## Long Range

With regard to the Long Range scenarios defined in the TR, link level evaluations (e.g. based on link-budget calculation) may be sufficient to assess the special long distance requirement(s). 
Further investigation and discussion is suggested on the assumptions and need for system level simulations, e.g. some clarification is needed on the propagation model.

# Conclusions

The following proposals are made:

Proposal 1: For High Speed,

a) Macro + Relay: Focus on Link-level simulation of the BS-to-relay link (e.g. to validate max speed of 500km/h).

b) Macro-only: It may be good to perform some system level evaluation looking at e.g. mobility (Idle/Connected), and UE battery performance. Spectral efficiency and capacity related KPIs can be also considered (perhaps as secondary KPIs).

Proposal 2: For Long Range, link level evaluations (e.g. based on link-budget calculation) may be sufficient to assess the special long distance requirement(s). Further investigation is suggested on the assumptions and need for system level simulations.

# References

[1] 3GPP TR 38.913 V0.3.0: SI TR on Scenarios and Requirements for Next Generation Access Technologies