3GPP TSG RAN WG1 Meeting #84bis 	R1-162254

# Busan, Korea 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR |  | rev |  | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on n_HARQ-ACK calculation for PUCCH format 4/5 in 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_enh_b5c |  |  |  |  |  |  | Date: |  | 2016-03-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | One condition for PUCCH format 3 ‘without PUCCH format 4/5’ is missing. , The condition of PUCCH format 4 or 5 configured with fixed codebook in section 10.1 in determination of codesize based on the number of configured carriers is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be transmit power waste for PUCCH format 4 and PUCCH format 5 with fixed codebook. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- 10.1	UE procedure for determining physical uplink control channel assignment

[…]

For FDD or FDD-TDD and primary cell frame structure 1, with two configured serving cells and PUCCH format 1b with channel selection or for FDD with two or more configured serving cells and PUCCH format 3 and without PUCCH format 4/5 configured or for FDD with PUCCH format 4/5 for HARQ-ACK transmission and configured with higher layer parameter codebooksizeDetermination-r13 = 1,  where  is the number of configured cells and  is the number of transport blocks or the SPS release PDCCH/EPDCCH, if any, received in subframe  in serving cell .

For TDD and a UE not configured with the parameter EIMTA-MainConfigServCell-r12 for any serving cell, if a UE is configured with one serving cell, or the UE is configured with more than one serving cell and the UL/DL configurations of all serving cells are the same, then

-	For TDD with two configured serving cells and PUCCH format 1b with channel selection and a subframe n with M = 1, or for TDD UL/DL configuration 0 and PUCCH format 3 and without PUCCH format 4/5 configured, or for PUCCH format 4/5 for HARQ-ACK transmission and configured with higher layer parameter codebooksizeDetermination-r13 = 1, , where is the number of transport blocks or the SPS release PDCCH/EPDCCH, if any, received in subframe  in serving cell , where , and M is the number of elements in K.

-	For TDD UL/DL configurations 1-6 and PUCCH format 3 and without PUCCH format 4/5 configured, or for TDD UL/DL configurations 1-6 and PUCCH format 4/5 for HARQ-ACK transmission and configured with higher layer parameter codebooksizeDetermination-r13 = 1, or for TDD with two configured serving cells and PUCCH format 1b with channel selection and M = 2,  where  is the  in serving cell ,  is the  in serving cell , and  is the number of HARQ-ACK bits corresponding to the configured DL transmission mode on serving cell . In case spatial HARQ-ACK bundling is applied,  and  is the number of PDCCH/EPDCCH or PDSCH without a corresponding PDCCH/EPDCCH received in subframe  and serving cell, where  and M is the number of elements in K. In case spatial HARQ-ACK bundling is not applied, is the number of transport blocks received or the SPS release PDCCH/EPDCCH received in subframe  in serving cell , where  and M is the number of elements in K. =0 if no transport block or SPS release PDCCH/EPDCCH is detected in subframe(s)  in serving cell , where .

-	For TDD with two configured serving cells and PUCCH format 1b with channel selection and M = 3 or 4,  if UE receives PDSCH or PDCCH/EPDCCH indicating downlink SPS release only on one serving cell within subframes ,where ;otherwise .

For TDD if the UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations, or if the UE is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell, or for FDD-TDD and primary cell frame structure 2, then

- For PUCCH format 3 without PUCCH format 4/5 configured, or for PUCCH format 4/5 for HARQ-ACK transmission and configured with higher layer parameter codebooksizeDetermination-r13 = 1, or for two configured serving cells and PUCCH format 1b with channel selection and  (defined in subclause 10.1.3.2.1 for TDD and subclause 10.1.3A for FDD-TDD),  where  is the  in serving cell ,  is the  in serving cell , and  is the number of HARQ-ACK bits corresponding to the configured DL transmission mode on serving cell . In case spatial HARQ-ACK bundling is applied,  and  is the number of PDCCH/EPDCCH or PDSCH without a corresponding PDCCH/EPDCCH received in subframe  and serving cell, where  and (defined in subclause 7.3.2.2 for TDD and subclause 7.3.4 for FDD-TDD). In case spatial HARQ-ACK bundling is not applied, is the number of transport blocks received or the SPS release PDCCH/EPDCCH received in subframe  in serving cell , where  and (defined in subclause 7.3.2.2 for TDD and subclause 7.3.4 for FDD-TDD). =0 if no transport block or SPS release PDCCH/EPDCCH is detected in subframe(s)  in serving cell , where  and (defined in subclause 7.3.2.2 for TDD and subclause 7.3.4 for FDD-TDD). For a serving cell , set if the DL-reference UL/DL configuration (defined in subclause 10.2) for serving cell   is TDD UL/DL configuration 0,

For two configured serving cells and PUCCH format 1b with channel selection and M = 3 or 4 (defined in subclause 10.1.3.2.1 for TDD and subclause 10.1.3A for FDD-TDD),  if UE receives PDSCH or PDCCH/EPDCCH indicating downlink SPS release only on one serving cell within subframes ,where  and (defined in subclause 7.3.2.2 for TDD and subclause 7.3.4 for FDD-TDD); otherwise .

[…]