3GPP TSG RAN WG1 Meeting #84bis		                                                       R1-162303

Busan, Korea, 11th - 15th April 2016

Source:	CATT

Title:	Evaluation scenarios and configuration parameters for new radio interface

Agenda Item:	8.1.3

Document for:	Discussion and Decision

Introduction

In RAN#71, scenarios including usage scenarios and deployment scenarios and requirements for next generation access technologies have been discussed and TR 38.913 was updated correspondingly. Companying with study in ITU-R WP5D, they are basic guidelines for study and evaluation of new radio interface.

In the updated TR 38.913, 10 deployment scenarios are defined. To be specific, it is decided for eMBB deployment scenarios include indoor hotspot, dense urban, rural, urban macro and high speed. Deployment scenarios for mMTC and URLLC need further study [1]. Some of details of attributed in each deployment are left to RAN1 discussion. This contribution is to discuss general evaluation assumption based on above deployment scenarios decided in [1] mainly for evaluation of Fundamental physical layer signal structure for new RAT and some issues left from RAN. Other detailed evaluation assumptions for any technical features, e.g. massive MIMO, should be discussed in the session of each of features.

Discussion

Reviewing the deployment scenarios for eMBB and their attributes, no more general configuration besides those in [1] is necessary. However, in order to facilitate evaluation for Fundamental physical layer signal structure before any conclusion to other features for new RAT and complete issues left by RAN, the following details should be discussed and decided.

  - Layout, ISD and user distribution

In each of deployment scenarios as following, layout illustration(s) is provided according to [1].

Indoor

Following definition in [1], Figure 1 shows layout of open office with 12 TRPs per 120m x 50m, i.e. 20m ISD.

Figure 1. Layout illustration of deployment scenario of “Indoor”

Dense Urban

As defined in [1], there are two layers in “Dense Urban” including Macro layer and Micro layer. Micro layer is subject to random drop and there are a number of Micro TRPs per Macro TRP. 3 is the baseline of the number of Micro TRP and other number of Micro TRPs per Macro TRP (e.g. 6 or 10) is not precluded.

Considering reality, there would be two possible user distributions: active users are tightly clustered (as studied in the case of small cell) or active users are randomly distributed around Macro area. Both cases could require high area traffic capacity indistinctively. When ISD of Macro-Macro is 200m, the latter case normally happens when there is e.g. Pedestrian Street for shopping. In that case, the whole street needs good coverage, UE experience, and high area traffic capacity. Deployment at lower frequency band can provide no-doubtable coverage when crowded users are distributed in whole area. Closed-by Micro TRP can provide higher capacity to the system. However, due to limited spectrum resources, e.g. 200MHz per operator, more numbers of Micro TRP are needed to reach the required area traffic capacity. Table 1 represents the calculation of area traffic capacity vs. the number of Micro TRP of one Macro TRP. Assuming 3 times spectrum efficiency (7.8bps/Hz/Cell) than the requirement of LTE-A (2.6bps/Hz/Cell in UMi), and bandwidth of 200MHz, area traffic capacity can reach 750Gbps/Km2 as NGMN requires [5] when the number of Micro TRP is 10.

When operating frequency is higher, i.e. above 6GHz for Micro TRP, much larger bandwidth is possible. At bandwidth of 1GHz, assuming spectrum efficiency of 5bps/Hz/Cell can be achieved at Micro TRP, area traffic capacity of 720Gbps/Hz/Cell can be achieved by 1 Macro + 3 Micro TRP. It is noted that it does not mean all requirements can be met in this case. For example, user experience data rate of 1Gbps may not be achieved anywhere within Macro area when the UE is not closed by a TRP in reality. Also further study is needed to solve issues caused by the case of high speed.

In summary, for lower carrier frequency (<6GHz) and meeting good UE experience covering whole Macro area while achieving the required area traffic capacity, more numbers of Micro TRPs are required.

Table 1 Number of TRP vs. Area traffic capacity

| Number of Macro TRPs + number of Micro TRPs | Metrics: Single TRP throughput (Gbps/TRP) Area traffic capacity (Gbps/Km2) | Macro only <6GHz assuming bandwidth 200MHz and spectrum efficiency of 7.8bps/Hz/Cell | Macro+Micro(<6GHz) Assuming bandwidth 200MHz and spectrum efficiency of 7.8bps/Hz/Cell at Micro TRP | Macro+Micro( >6GHz) Assuming bandwidth 1GHz and spectrum efficiency of 5bps/Hz/Cell at Micro TRP |
| --- | --- | --- | --- | --- |
| 1+3 | Single TRP throughput | 1.56 | 1.56 | 5 |
|  | Area traffic capacity | - | 271.3 | 720 |
| 1+6 | Single TRP throughput | - | 1.56 | 5 |
|  | Area traffic capacity | - | 474.78 | 1372.17 |
| 1+7 | Single TRP throughput | - | 1.56 | 5 |
|  | Area traffic capacity | - | 542.61 | 1589.57 |
| 1+9 | Single TRP throughput | - | 1.56 | 5 |
|  | Area traffic capacity | - | 678.26 | 2024.35 |
| 1+10 | Single TRP throughput | - | 1.56 | 5 |
|  | Area traffic capacity | - | 746.09 | 2241.74 |

In order to ensure user experience data rate anywhere in the macro area meanwhile keeping sufficient area traffic capacity, radius of UE’s cluster and distance between clusters need to be defined. Layout illustrations of 1 Macro + 3 Micro TRPs and 1 Macro + 9 Micro TRPs are shown in Figure 2 and 3. The minimum distances between two Micro TRPs (two clusters, 1 TRP per cluster) are provided in Table 2, where it is twice of radius of UE clusters. To facilitate evaluation, i.e. TRP dropping, radius of UE cluster can be defined smaller than those in the table 2 or minimum distance can be defined less than twice of radius. But to keep the same UE density in each cluster, the earlier one is suggested.

Additionally, antenna of Micro TRP is omni at least for lower frequency band and FFS for high frequency band.

Figure 2. Layout illustration of deployment scenario of “Dense Urban”: 3 Micro TRPs per Macro TRP

Figure 3. Layout illustration of deployment scenario of “Dense Urban”: 9 Micro TRPs per Macro TRP

Table 2. Number of Micro TPP per Macro TRP vs. minimum distance between TRPs and UE cluster radius

| Number of the micro TRPs per macro TRP | Minimum distance between Micro TRPs (m) | Radius of UE dropping within a cluster: R (m) |
| --- | --- | --- |
| 3 | 57.9 | <28.9 |
| 6/7 | 42.4 | <21.2 |
| 9 | 32 | <16 |
| 10 | 29.6 | <14.8 |

High speed

Layout illustration of “High speed” can refer to RAN4’s study on performance enhancements for high speed scenario in LTE [2]. Antenna of RRU along railway is dual-direction.

Figure 4. Layout illustration of deployment scenario of “High Speed”

Other deployment scenarios

Layout illustration of other deployment scenario can follow the existing layout in previous study in RAN1 [3] [4], except for the cases for long distance scenarios which only need a single TRP for evaluation.

  - BS and UE antenna elements

The maximum numbers of BS and UE antenna elements have been defined as working assumption in [1]. The exact number of antenna elements and ports are left to RAN1 study to meet requirements, e.g. spectrum efficiency. It also depends on technique features whose performance is relying on the number of antenna, e.g. massive MIMO. On the other hand, for most fundamental physical layer signal structure of new RAT, it may not be necessary to involve the maximum number of antenna elements/ports into evaluation to identify their benefits. So it is proposed to have default number of antennas e.g. 2Tx and 2Rx as baseline at BS and 1Tx and 2Rx at UE for such evaluation. For sure, the exact number also depends on discussion of each fundamental feature.

Also to discuss in general for evaluation of fundamental physical layer signal structure, some antenna configurations are also needed, e.g. antenna height, down-tilt, antenna pattern, antenna gain, Tx power and Rx noise figure. These parameters can refer to the details defined in evaluation to LTE-advanced, i.e. M.2135 [3] and 36.814[4]. The summarized Table 3 of proposed general antenna configurations can be found in Annex.

Conclusion

In this contribution, general evaluation assumption based on above deployment scenarios decided in [1] mainly for evaluation of Fundamental physical layer signal structure for new RAT and some issues left from RAN are discussed. The following are proposed:

Proposal 1: The following proposals shall be discussed can added into the chapter for evaluation in TR.

  - Layout illustrations of Indoor, Dense Urban and High Speed in Figure 2-4 can be captured in TR;

  - Minimum distance between Micro TRPs and radius of UE dropping within a cluster for various number of Micro TPRs per Macro TRP (Table 2) shall be defined and captured in TR;

  - General antenna configurations and default number of antenna (2Tx and 2Rx at BS and 1Tx and 2Rx at UE) as baseline for fundamental physical layer signal structure study can be captured in TR.

Proposal 2: To prioritize deployment scenarios including indoor, dense urban, macro urban and rural for evaluation and study of Fundamental physical layer signal structure, meanwhile to ensure the design can support other deployment scenarios. Additional evaluations for feature design/optimization are allowed in other deployment scenarios.

References

  - 3GPP TR 36.913 V0.2.1 (2016-02), “Study on Scenarios and Requirements for
 Next Generation Access Technologies”

  - 3GPP TR 36.878 V13.0.0 (2016-01), “Study on performance enhancements for high speed scenario in LTE”

  - M.2135-1, Guidelines for evaluation of radio interface technologies for IMT-Advanced, Dec. 2009

  - 3GPP TR 36.814 v9.0.0 (2010-03), “Further advancements for E-UTRA physical layer aspects”

  - NGMN 5G white paper V1.0, Feb.2015

Annex

Table 3 General antenna configurations

|  | Indoor hotpot | Dense Urban |  | Urban Macro | Rural | High Speed |
| --- | --- | --- | --- | --- | --- | --- |
| Transmit power [4] | Pico: <6GHz: 24dBm >6GHz:[X]dBm UE: <6GHz: 23dBm >6GHz:[X]dBm | Macro: 46dBm Micro: <6GHz: 30dBm >6GHz:[X]dBm UE: <6GHz: 23dBm >6GHz:[X]dBm |  | Macro: 49dBm UE: 23dBm | Macro: 49dBm UE: 23dBm | Macro: 49dBm Pico: 24dBm Relay: 33dBm UE: 23dBm |
| Antenna Height [3] | Pico: 3m UE:1.5m | Macro: 25m Micro: 10m UE: 1.5m |  | Macro: 25m UE: 1.5m | Macro: 35m UE: 1.5m | Macro: 35m Pico: 3m UE: 1.5m |
| BS antenna downtilt [4] | Pico (ITU Indoor, indoor hotspot scenario (InH)): N/A | Macro (Case 1 3GPP 3D): 15 degree Micro (ITU Microcellular, urban micro-cell scenario (Umi)): 12 degree |  | Macro (ITU Base coverage urban, Urban macro-cell scenario (Uma)): 12 degree | Macro (ITU High speed, Rural macro-cell scenario (Rma)): 6 degree | Macro (ITU High speed, Rural macro-cell scenario (Rma)): 6 degree |
| Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns) [4] | = 70 degrees, Am = 25 dB |  |  |  |  |  |
| Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns) [4] | = 10, SLAv = 20 dB The parameter is the electrical antenna downtilt. |  |  |  |  |  |
| Combining method in 3D antenna pattern [4] |  |  |  |  |  |  |
| Antenna configuration [4] | Vertically polarized antennas 0.5 wavelength separation at UE, 10 wavelength separation at basestation |  |  |  |  |  |
| BS antenna gain [3][4] | <6GHz: 5dBi [4] >6GHz: [2]dBi |  | <6GHz: 5dBi [4] >6GHz: [2]dBi | 5dBi [4] | 17 dBi [3] | 17 dBi [3] |
| BS Receiver Noise figure [3][4] | <6GHz: 5dB >6GHz: TBD |  |  |  |  |  |