3GPP TSG RAN1 Meeting #84bis 	R1-162592

# Busan, South Korea, April 11-15, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | xxxx | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identify MPDCCH order in CE mode B in TS 36.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1 |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | PDCCH order was introduced via DL assignment with a near-zero chance of scheduling restriction. There is a known issue with the MPDCCH order implementation which causes up to 25% of PDSCH resource allocations to be restricted and not allowed to be scheduled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For MPDCCH used for MPDCCH order, two spared bits corresponding to the location of two MSB bits in the MCS field is fixed to 11. Make “Modulation and coding scheme” the first DCI field in DCI format 6-0B and DCI format 6-1B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPDCCH order significantly restricts PDSCH operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.11, 5.3.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

##### 5.3.3.1.11	Format 6-0B

DCI format 6-0B is used for the scheduling of PUSCH in one UL cell.

The following information is transmitted by means of the DCI format 6-0B:

-	Modulation and coding scheme – 4 bits as defined in section 8.6 of [3]

-	Flag for format 6-0B/format 6-1B differentiation – 1 bit, where value 0 indicates format 6-0B and value 1 indicates format 6-1B

-	Resource block assignment – +3 bits for PUSCH as defined in [3]:

-	 MSB bits provide the narrowband index as defined in section 5.2.4 of [2]

-	3 bits provide the resource allocation within the indicated narrowband as specified in section 8.1.3 of [3]

-	Repetition number – 3 bits as defined in section 8.0 of [3]

-	HARQ process number – 1 bit

-	New data indicator – 1 bit

-	DCI subframe repetition number – 2 bits as defined in section 9.1.5 of [3]

If the number of information bits in format 6-0B mapped onto a given search space is less than the payload size of format 6-1B for scheduling the same serving cell and mapped onto the same search space (including any padding bits appended to format 6-1B), zeros shall be appended to format 6-0B until the payload size equals that of format 6-1B.

#### < Unchanged parts are omitted >

##### 5.3.3.1.13	Format 6-1B

DCI format 6-1B is used for the scheduling of one PDSCH codeword in one cell.

The following information is transmitted by means of the DCI format 6-1B:

Format 6-1B is used for random access procedure initiated by a PDCCH order only if format 6-1B CRC is scrambled with C-RNTI and all the remaining fields are set as follows:

- Reserved bits – 2 bits, where 2 bits shall be set to 1

- Resource block assignment – +1 bits, where all bits shall be set to 1

- Preamble Index – 6 bits

- PRACH Mask Index – 4 bits [5]

- Starting CE level – 2 bits provide the PRACH starting CE level as defined in [5]

- All the remaining bits in format 6-1B for compact scheduling assignment of one PDSCH codeword are set to zero

Otherwise,

- Modulation and coding scheme – 4 bits as defined in section 7.1.7 of [3]

- Flag for format 6-0B/format 6-1B differentiation – 1 bit, where value 0 indicates format 6-0B and value 1 indicates format 6-1B

- Resource block assignment – +1 bits for PDSCH as defined in [3]:

-  MSB bits provide the narrowband index as defined in section 6.2.7 of [2]

- 1 bit provides the resource allocation within the indicated narrowband, where value 0 indicates RBs with PRB index {0, 1, 2, 3} and value 1 indicates that all 6 PRBs are used.

- Repetition number – 3 bits as defined in section 7.1.11 of [3]

- HARQ process number – 1 bit

- New data indicator – 1 bit

- HARQ-ACK resource offset – 2 bits as defined in section 10.1 of [3]

- DCI subframe repetition number –2 bits as defined in section 9.1.5 of [3]

When the format 6-1B CRC is scrambled with a RA-RNTI then the following fields among the fields above are reserved:

- HARQ process number

- New data indicator

- HARQ-ACK resource offset

#### < Unchanged parts are omitted >