3GPP TSG-RAN1#84-BIS	R1-162772

11-15 April 2016

Busan, Korea

Source:                    	Ericsson

Title:  	NB-IoT - DCI content

Document for:        	Discussion and decision

Agenda Item:         	7.2.1.1.1

# Introduction

In 3GPP RAN1 the following agreements have been made so far with regard to DCI message carried by physical downlink control channel NB-IoT or NB-PDCCH [1].

RAN1 NB-IoT adhoc agreements: - The same DCI size for DL and UL is targeted for all operation modes and all coverage cases - NB-PDCCH and NB-PDSCH are multiplexed only based on TDM at subframe level at least per UE - It means that only cross subframe scheduling is supported - Different NB-PDCCHs: - TDM at subframe level for extended and extreme coverage. - Can be multiplexed in one subframe for normal coverage - FFS: FDM+TDM - For PDSCH: - Resource mapping: frequency first, then time. - QPSK baseline, 16-QAM FFS - Single process HARQ for PDSCH is realized by adaptive and asynchronous timing transmission - Channel coding: TBCC - FFS: RV for NB-PDSCH is supported - UL multi-tone transmission for the data with 12 tones is supported - UL multi-tone transmission for the data also supports followings of numbers of multiple - {3} with 4 msec resource unit size - {6} with 2 msec resource unit size - For 15kHz subcarrier spacing, OFDM/SC-FDMA symbol boundary is no change from LTE - Transmission of ACK and/or NACK corresponding to NB-PDSCH is supported - Both 3.75 and 15 kHz subcarrier spacing are supported in transmission of ACK and/or NACK - FFS for the piggy back of SR

RAN1#84 agreements: - DL and UL scheduling delays are indicated in DCI - number of delay values that can be signalled for the UL scheduling delay is less than the number of values for the DL - FFS the number of values and the sets of values - CSS is defined for RAR (as well as paging) - 16QAM is not supported for NB-PDSCH - The maximum TBS for NB-PDSCH is 680 bits - Redundancy versions (RVs) for NB-PDSCH are not supported - Note that the number(s) of subframes that a TB can be mapped onto for NB-PDSCH is FFS - Sub-PRB allocations of the NB-PDSCH are not supported - NB-PDSCH for paging is always scheduled by a control channel - Adaptive HARQ is supported for uplink. - The HARQ re-transmissions in the uplink are asynchronous.

RAN1 NB-IoT adhoc#2 agreements: - System information change - Agree DCI format N2 for the flag = 0 case in R1-161561 (Latest 36.212 CR) - FFS: details on flag = 1 case - DCI content: - Number of repetitions of NB-PDCCH: - 2 bits (except for CSS for paging) - 3 bits for CSS for paging - Scheduling delay between end of NB-PDCCH transmission and start of data transmission: - 3 bits for NB-PDSCH (except for CSS for paging) - 0 bits for paging - 2 bits for NB-PUSCH - Values are FFS. - NPUSCH subcarrier allocations - 5 bits in UL grant are used to jointly indicate the subcarrier number and starting subcarrier for NB-PUSCH transmission with 15 kHz subcarrier spacing. The total number of valid NB-PUSCH allocations for {12, 6, 3, 1} tone transmission formats and 15 kHz numerology is the sum of - One allocation of all 12 tones: {0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} - Two non-overlapping allocations of 6 tones: {0, 1, 2, 3, 4, 5} and {6, 7, 8, 9, 10, 11} - Four non-overlapping allocations of 3 tones: {0, 1, 2}, {3, 4, 5}, {6, 7, 8} and {9, 10, 11} - Twelve non-overlapping single-tone allocations: {0}, {1}, {2}, {3}, {4}, {5}, {6}, {7}, {8}, {9}, {10}, {11} and {12} - 6 bits in UL grant are used to indicate the subcarrier index for NB-PUSCH transmission with 3.75 kHz subcarrier spacing (48 non-overlapping single-tone allocations). - The set of options for the max number of repetitions in an NB-PDCCH search space is the same for all search spaces - Rmax is from: {1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048}

In this contribution we discuss the different DCI formats for both uplink and downlink transmissions.

# DCI format

It has been proposed to define three different DCI format for NB-IoT which are DCIs for uplink grant, DL resource assignment and paging/SI indication and most of the fields in each DCI format and the required number of bits for those fields have been already determined. There are several remaining undetermined DCI fields which we will discuss in this section.

Regarding the uplink number of repetitions for NPUSCH in [1] we propos that the number of repetitions that may be used for NPUSCH includes {1, 2, 4, 8, 16, 32} for the 15 kHz numerology, and {1, 2, 4, 8} for the 3.75 kHz numerology this corresponds to 3 and 2 bits in DCI for each numerology respectively, leading to the same total transmission times for the two numerologies. For NPDSCH in [2] we propose that the number of repetitions that may be used for NPDSCH includes {1, 2, 4, 8, 16, 32, 64, 128, 256}. The number of bits required for these values are indicated in the tables below.

Also in case it is agreed to have cyclic shifts for uplink DMRS in NB-IoT, a corresponding field with 2 bits is needed in uplink grant in order to indicate this cyclic shift [4].

According to the discussion in [5] for downlink HARQ-ACK, 2 bits are required to indicate the frequency offset and 2 bits for timing offsets of the corresponding uplink transmission.

The details for the DCI for uplink grant and downlink assignment according to the agreements and remaining parts mentioned above are listed in tables 1 and 2 respectively.

DCI content for UL grant (DCI format N0):

| Indication flag for DL assignment/ UL grant | 1 bit |  |
| --- | --- | --- |
| New Data Indication (NDI) | 1 bit |  |
| PDCCH number of repetitions | 2 bits |  |
| Dynamic scheduling delay between end of PDCCH and starting of PUSCH | 2 bits |  |
| Indication of NPRB | 3 bits |  |
| Indication of ITBS | 4 bits |  |
| Subcarrier scheduling | 5 bits (15 kHz) (jointly indicate the subcarrier number and starting subcarrier) | 6 bits (3.75 kHz) (indicate the subcarrier index) |
| Dynamic indication of number of repetitions for data channel [2] | 3 bits (15 kHz) | 2 bits (3.75 kHz) |
| Redundancy version | 1 bits |  |
| TPC for PUSCH | 2 bits |  |
| Cyclic shift for DM RS [if cyclic shift introduced for uplink DMRS] [4] | 2 bits |  |
| CRC | 16 bits |  |
| Total | <= 42 |  |

Table 1: DCI content for UL grant

DCI content for DL assignment (DCI format N1):

| Indication flag for DL assignment/ UL grant | 1 bit |
| --- | --- |
| New Data Indication (NDI) | 1 bit |
| S_flag indication between DL unicast and SI update/paging indication | 1 bit |
| PDCCH number of repetitions | 2 bits |
| Dynamic scheduling delay between end of PDCCH and starting of PDSCH | 3 bits |
| Indication of NPRB | 3 bits |
| Indication of ITBS | 4 bits |
| Dynamic indication of number of repetitions for data channel [3] | 4 bits |
| HARQ-ACK resource [5] | 2 bits (frequency offset) |
|  | 2 bits (timing offset) |
| CRC | 16 bits |
| Total | = 39 |

Table 2: DCI content for DL assignment

DCI content for paging and direct indication (DCI format N2):

This format is used for paging and SI update indication. A flag bit in the DCI is used in order to separate these two cases.

If flag is 0: DCI message is a direct indication i.e. SI update notification. In this case there will be “X” bits assigned as SI update bitmap. The value of X should be decided by RAN2.

| Flag indication between SI update / paging | 1 bit |
| --- | --- |
| SI change bitmap | x bits |
| CRC | 16 bits |
| Total | x +17 |

Table 3: DCI content for direct indication

If flag is 1: DCI content is resource allocation assignment for the paging message. In this case, values from format N1 will be used for resource allocation according to table 4. It is also proposed that the direct indication bitmap for SI update could be included in the paging message itself in this case.

| Flag indication between SI update / paging | 1 bit |
| --- | --- |
| PDCCH number of repetitions | 2 bits |
| Dynamic scheduling delay between end of PDCCH and starting of PDSCH | 3 bits |
| Indication of NPRB | 3 bits |
| Indication of ITBS | 4 bits |
| Dynamic indication of number of repetitions for data channel | 4 bits |
| CRC | 16 bits |
| Total | = 33 |

Table 4: DCI content for paging

Proposal 1: 	DCI messages can be carried by NB-PDCCH using payload size no more than 27 (+16 CRC) bits for both UL grant/DL assignments.

Proposal 2: 	The content of the DCI message is according to tables 1, 2, 3, and 4 in this contribution for uplink grant, downlink assignment, direct indication and paging indication respectively.

# Conclusions

In this contribution, we have discussed DCI format carried by NB-PDCCH. Based on the discussions, we make the below proposals:

Proposal 1: 	DCI messages can be carried by NB-PDCCH using payload size no more than 27 (+16 CRC) bits for both UL grant/DL assignments.

Proposal 2: 	The content of the DCI message is according to tables 1, 2, 3, and 4 in this contribution for uplink grant, downlink assignment, direct indication and paging indication respectively.

# References

- R1-16xxxx RAN1, “agreements for Rel-13 NB-IoT v2”, by RAN1 Rapporteur

- R1-161987 - NB-IoT – “Remaining open issues for NPUSCH”, source Ericsson, 3GPP TSG RAN WG1 NB-IoT Ad-Hoc Meeting, Sophia Antipolis, France, 22nd - 24th March 2016

- R1-161986 – “NB-IoT - Remaining issues for NPDSCH design”, source Ericsson, 3GPP TSG RAN WG1 NB-IoT Ad-Hoc Meeting, Sophia Antipolis, France, 22nd - 24th March 2016

- R1-162777 – “NB-IoT - UL Reference signals”, source Ericsson, 3GPP TSG RAN WG1 NB-IoT #84 BIS Meeting, Busan, Korea, 11th - 25th April 2016

- R1-162778 – “NB-IoT - Uplink control information”, source Ericsson, 3GPP TSG RAN WG1 NB-IoT #84 BIS Meeting, Busan, Korea, 11th - 25th April 2016