3GPP TSG-RAN WG1 Meeting #84bis	R1-162821

Busan, Korea, 11th – 15th April 2016

Agenda item:		7.3.10.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:					PUCCH design for sTTI

Document for:		Discussion and Decision

# 1. Introduction

Based on the outcome of RAN#67 captured in the SI description in RP-150465, the following items have been identified specifically for RAN1 studies on TTI shortening and reduced processing times:

- Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signaling

- Backwards compatibility shall be preserved (thus allowing normal operation of pre-Rel 13 UEs on the same carrier);

In this document, we discuss the principles of PUCCH design for short TTI. Short TTI related aspects of UL DMRS, UL HARQ, and TDD specific aspects are considered in [1], [2], and [3], respectively.

# 2. PUCCH design for short TTI

PUCCH design is one of the key aspects in facilitating latency reduction. In order to be able to reduce HARQ round-trip time, the UL control channel conveying HARQ-ACK will need to be shortened similarly as the related PDSCH carrying the data transport block(s). Therefore it seems apparent that a shortened PUCCH will need to be specified to carry at least HARQ-ACK. In principle, it should be straight forward to apply the same design for conveying Scheduling Request as well, which in turn may provide a minor decrease in the observed latency.

On the other hand, the need for using shortened PUCCH for CSI is far less clear. In LTE, periodic PUCCH reports are transmitted on PUCCH when there is no simultaneous PUSCH transmission. However, periodic CSI reporting involves inherently relatively large latency depending on the reporting periodicity, and the length of PUCCH – 1 ms or less – does not have a significant impact on the overall latency. Furthermore, it should be possible for the network to trigger Aperiodic CSI report to be transmitted using shortened PUSCH, which further reduces the need for transmitting periodic CSI over shortened PUCCH.

Observation #1: Primary contents for sTTI PUCCH are HARQ-ACK (+SR). The need for supporting periodic CSI transmission over shortened PUCCH is unclear.

When it comes to shortened PUCCH design, a few requirements need to be considered. In LTE, the coverage of the whole system is often limited by the PUCCH. In order to mitigate this, various means have been specified including e.g. PUCCH repetition to enhance HARQ-ACK coverage. Obviously, for shorter TTI operation to make sense, the impact on coverage should be minimized. On the other hand, shorter TTI means that the energy per bit for e.g. HARQ-ACK transmission is inevitable reduced, and hence there will be some penalty in terms of cell coverage. This issue is more pronounced with TTI lengths less than 0.5 ms and may easily end up limiting the coverage of sTTI feature as a whole.

Observation #2: PUCCH coverage may easily become a bottleneck for sTTI, especially if sTTI length of less than 0.5 is considered.

In order to make shorter TTI operation an attractive feature, special attention needs to be put on PUCCH coverage. We see that the as a starting point, one needs to keep the basic design properties of 1-ms PUCCH, namely frequency hopping to achieve frequency diversity, as well as single carrier properties of the UL waveform (SC-FDMA) to guarantee uncompromised operation at maximum TX power.

Proposal #1): sTTI PUCCH should aim at as large coverage as possible, and

  - Retain support for frequency diversity (i.e. hopping across band edges)

  - Maintain Single carrier properties by utilizing SC-FDMA waveform

Another key design target for LTE PUCCH has been low overhead, which has been achieved with high multiplexing capability. Depending on the PUCCH format (and correspondingly payload), more than 10 UEs can share the same PUCCH PRB. We see that the same design target is equally important with shorter TTIs as well, and efficient multiplexing of different sTTI UEs, as well as both sTTI and legacy UEs should be supported within a PRB.

Observation #3: Increase in UL control overhead due to sTTI should be minimized.

Proposal #2: Multiplexing of sTTI PUCCHs should be supported within the same PRB.

Proposal #3: Multiplexing of sTTI PUCCH and legacy PUCCH should be supported within the same PRB.

# 3. Summary

In this contribution we consider aspects impacting PUCCH design for sTTI on a high level. Based on the discussion, we make the following observations and proposals related to PUCCH design:

Observation #1: Primary contents for sTTI PUCCH are HARQ-ACK (+SR). The need for supporting periodic CSI transmission over shortened PUCCH is unclear.

Observation #2: PUCCH coverage may easily become a bottleneck for sTTI, especially if sTTI length of less than 0.5 is considered.

Proposal #1): sTTI PUCCH should aim at as large coverage as possible, and

  - Retain support for frequency diversity (i.e. hopping across band edges)

  - Maintain Single carrier properties by utilizing SC-FDMA waveform

Observation #3: Increase in UL control overhead due to sTTI should be minimized.

Proposal #2: Multiplexing of sTTI PUCCHs should be supported within the same PRB.

Proposal #3: Multiplexing of sTTI PUCCH and legacy PUCCH should be supported within the same PRB.

# 4. References

- R1-160779 “DMRS for PUSCH with short TTI”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN1#84

- R1-160799, “Considerations on UL HARQ for Supporting Latency Reduction”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN1#84

- R1-160780, “Shorter TTI enhancements for TDD”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN1#84