3GPP TSG RAN WG1 Meeting #84bis 	R1-162835

Busan, Korea 11th - 15th April 2016

Source:	Sharp

Title:	On non-anchor PRBs for NB-IoT multi-carrier operation

Agenda Item:	7.2.1.1.2

Document for:	Discussion and Decision

Introduction

In the RAN1 NB-IoT ad-hoc meeting in Sophia Antipolis, the following agreements were made on NB-IoT multi-carrier operation [1].

Agreements: - Any combination, i.e., inband+inband, inband+guardband, and guardband+guardband should be allowed for NB-IoT multi-carrier operation with the constraint that both guard-bands and the in-band are associated with the same LTE donor cell, i.e., the total span cannot exceed 110 PRBs from the same FFT - No support of NB-IoT multi-carrier operation for standalone mode with either guard-band or in-band mode of operation - Standalone+standalone should be allowed for NB-IoT multi-carrier operation with the constraint that the total frequency span cannot exceed 20MHz and both NB-IoT carriers are synchronized, i.e., the time alignment error shall not exceed the minimum requirement for intra-band contiguous carrier aggregation in TS 36.104 - Send an LS to RAN4 to ask if they have any concerns with this - On PRBs different than the NB-IoT carrier on which the UE has received NB-PSS/SSS, NB-PBCH and SIB transmissions, the NB-IoT UE does not rate match around NB-PBCH and NB-PSS/SSS, i.e., the mapping of NB-PDCCH/PDSCH symbols to REs occurs without consideration of NB-PSS/SSS/PBCH

In this contribution, we share our considerations and clarifications on non-anchor PRBs for NB-IoT multi-carrier operation.

Non-anchor PRB determination

Non-anchor PRBs for in-band operation

In the RAN1 NB-IoT ad hoc meeting in Budapest, it was agreed that one transmission of NB-PSS, NB-SSS, NB-PBCH, and NB-PDSCH never overlaps between multiple LTE PRB bandwidths for in-band operation [2]. In other words, the NB-IoT PRB is aligned with legacy LTE PRB mapping for in-band operation. Therefore, in principle, all legacy LTE PRBs can be utilized as candidate non-anchor PRBs for in-band operation.

Proposal 1:

- For in-band operation, all LTE PRBs can be utilized as candidate non-anchor PRBs.

Anchor/Non-anchor PRBs for guard-band operation

In the RAN1#84 meeting, it was agreed that the center frequency of a candidate anchor PRB is at most 7.5 kHz offset from the 100 kHz channel raster for guard-band operation [3]. This means that we can consider the following two options to determine the NB-IoT PRB candidates:

Option 1: a UE assumes NB-IoT PRB position in LTE guard-band is aligned with continuously extended PRB position from LTE PRB

Option 2: a UE assumes arbitrary NB-IoT PRB center frequency satisfying at most 7.5 kHz frequency offset from the 100 kHz channel raster.

Figure 1. NB-IoT PRB position in LTE guard-band

Figure 1 shows each option in 15 MHz LTE system bandwidth case. In this figure, actually the candidates of NB-IoT PRBs are overlapped in option 2, so all the candidates are not represented for easy view ability in option 2. Option 1 is NB-IoT PRB positions are aligned with LTE PRB grid. Therefore, from network perspective, eNB can easily operate total bandwidth as simply extended PRBs to 110 PRBs in 20 MHz LTE case for example. Moreover, there is no gap between LTE PRB and NB-IoT PRB.

Table. 1 shows the actual candidates of center frequency of NB-IoT carrier as the difference between LTE center frequency and NB-IoT center frequency in option 1 and option 2 [4]. For anchor PRBs, because of the limitation of at most 7.5 kHz offset from channel raster, the candidates of NB-IoT anchor PRBs in guard-band are limited for option 1, and even no anchor PRB can be found in 5 MHz case. On the other hand, in option 2, assuming that 15 kHz subcarrier spacing is kept, but NB-IoT carrier is not aligned with extended PRB grid from LTE PRBs. There is some gap less than LTE PRB bandwidth between NB-IoT PRB and LTE PRB. Because all the NB-IoT PRBs in option 2 meets the limitation of at most 7.5 kHz offset from channel raster, all these PRBs can be used as candidates of anchor PRBs. So option 2 has more anchor PRB candidates. If the guard-band can be used for NB-IoT transmission, it will be better to deploy the anchor PRBs in the guard-bands and non-anchor PRBs in in-band (if need) because the impact on legacy LTE will be less comparing to anchor PRBs in in-band (without NB-PSS/SSS/PBCH on non-anchor PRBs). In our views, RAN1 should clarify which solution is assumed and we prefer option 2

Table 1. Possible center frequencies for NB-IoT PRBs in guard-band operation mode

| LTE system bandwidth | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| Option 1 | None | ± 4597.5 | ± 7207.5 | ± 9097.5 |
| Option 2 | ±2392.5 | ± 4597.5/4702.5 /4807.5/4897.5 | ± 6892.5/6997.5/7102.5/7207.5/7297.5/7402.5 | ± 9097.5/9202.5/9307.5/9397.5/9502.5/9607.5/9697.5/9802.5/9907.5 |

Proposal 2:

- RAN1 should clarify whether the anchor PRB configuration is assumed as option 1 or option 2

  - We slightly prefer option 2.

For non-anchor PRBs, there is not the limitation of at most 7.5 kHz offset from channel raster, so all the NB-IoT PRBs in option 1 can be used as non-anchor PRBs. And it is not reasonable to constrain non-anchor PRBs with the same limitation of at most 7.5 kHz offset from channel raster as anchor PRBs for option 2. In addition, for option 2, there are only 3 non-overlapping PRBs can be used as candidate non-anchor PRBs in each guard-band for the system bandwidth of 15MHz which are less than 4 candidate non-anchor PRBs of option 1. So we prefer option 1 for non-anchor PRBs.

Proposal 3:

- Adopt option 1 to determine non-anchor PRBs for guard-band operation.

Non-anchor PRBs for stand-alone operation

In the RAN1 NB-IoT ad hoc meeting in Budapest, it was agreed that the center frequency of a candidate anchor PRB is aligned to the 100 kHz channel raster for stand-alone operation, i.e., the raster offset is 0. And in the RAN1 NB-IoT ad hoc meeting in Sophia Antipolis, it was agreed that standalone+standalone should be allowed for NB-IoT multi-carrier operation with the constraint that the total frequency span cannot exceed 20MHz and both NB-IoT carriers are synchronized. In order to improve the frequency efficiency, the available non-anchor PRBs should be continuously distributed on both sides of the anchor PRB and the maximum bandwidth is 20MHz.

Proposal 4:

- The available non-anchor PRBs should be continuously distributed on both sides of the anchor PRB and the maximum bandwidth is 20MHz.

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

- For in-band operation, all legacy LTE PRBs can be regarded as candidate non-anchor PRBs.

Proposal 2:

- RAN1 should clarify whether the anchor PRB configuration is assumed as option 1 or option 2

  - We prefer option 2.

Proposal 3:

- Adopt option 1 to determine non-anchor PRBs for guard-band operation.

Proposal 4:

- The available non-anchor PRBs should be continuously distributed on both sides of the anchor PRB and the maximum bandwidth is 20MHz.

References

Chairman’s notes of the 3GPP TSG RAN WG1 NB-IoT Ad-Hoc Meeting, Sophia Antipolis, France, 22nd - 24th March 2016.

Chairman’s notes of the 3GPP TSG RAN WG1 NB-IoT Ad-Hoc Meeting, Budapest, Hungary, January 8-20, 2016.

Chairman’s notes of the 3GPP TSG RAN WG1#84 Meeting, St. Julian’s, Malta, February 15 – 19, 2016.

R1-160022, Channel raster design, Huawei, HiSilicon, 3GPP TSG RAN WG1 NB-IoT ad-hoc Meeting, Budapest, Hungary, 18th – 20th January 2016.

R1-162021, WF on PRB indexing in MIB for NB-IoT in-band operation, Ericsson, LG, 3GPP TSG RAN WG1 NB-IoT Ad-Hoc Meeting, Sophia Antipolis, France, 22nd – 24th March 2016.