3GPP TSG RAN WG1 Meeting #84bis	R1-162841

Busan, Korea, April 11th – 15th, 2016

Source:	Sharp

Title:	Multiple transmission power ratios for MUST

Agenda Item:	7.3.4.1

Document for:	Discussion and Decision

- Introduction

In the WID on MUST, the following objectives are identified [1].

The work item is to specify necessary mechanisms to enable LTE to support downlink intra-cell multiuser superposition transmission for PDSCH with assistance information from serving eNB to a UE regarding its experienced intra-cell interference. A MUST UE receiver is assumed to be capable to cancel or suppress intra-cell interference between co-scheduled MUST users for the following cases. Case 1: Superposed PDSCHs are transmitted using the same transmission scheme and the same spatial precoding vector Case 2: Superposed PDSCHs are transmitted using the same transmit diversity scheme. Case 3: Superposed PDSCHs are transmitted using the same transmission scheme, but their spatial precoding vectors are different. The detailed objectives include: - (RAN4) For Case 1, 2 and 3, identify and agree on the parameter combinations that could be blindly detected jointly for MUST based on TR36.859 and RAN1’s recommendation. - (RAN1) For Case 1 and 2 using up to 2 Tx CRS-based transmission schemes, specify downlink multiuser superposition transmission scheme(s) for MUST category 2 with multiple transmission power ratios or MUST category 2 with single transmission power ratio & legacy constellation for co-scheduled MUST users in each constellation combination. - Down-selection should be further discussed in RAN1. - (RAN1) For Case 1 and 2 using up to 2 Tx CRS-based transmission schemes, specify necessary mechanisms to enable efficient MUST operation. - The configuration of downlink multiuser superposition transmission. - Starting from the candidate parameters of assistance information identified in TR 36.859 and based on the RAN4 identified parameter combinations which could be jointly blindly detected, specify the mechanism to provide MUST assistance information to a UE using R-ML receiver, which may include assistance signalling and blind detection. - (RAN1) For all three Cases using up to 4 Tx CRS-based or up to 8 Tx DMRS-based transmission schemes, evaluate the system-level performance based on the evaluation methodology and assumptions in TR36.859. - (RAN1) For all three Cases using up to 4 Tx CRS-based or up to 8 Tx DMRS-based transmission schemes, identify and, if needed, specify necessary enhancements for MUST operation, following the outcomes of objective 1 to 4. - (RAN2) Specify necessary higher-layer signalling to support the objectives listed above. Note: CSI enhancement is not part of this WI.

In this contribution, we compare the gain of MUST category 2 with multiple transmission power ratios or MUST category 2 with single transmission power ratio & legacy constellation.

MUST category 2 with multiple transmission power ratios

According to the agreed WID, MUST category 2 with multiple transmission power ratios became one of the candidates for MUST. It can adjust the power ratio based on path-loss difference between far UE and near UE. On the other hand, scheduler complexity may be higher when the number of power ratios becomes larger. Therefore, the number of power ratios should be limited.

In the SI phase, each company evaluated the system-level MUST gain with arbitrary transmission power set [2-7]. That is, the transmission power sets are different according to the companies. Therefore, in this contribution, we evaluate MUST gain for various multiple power sets in order to decide adequate number of multiple power ratios.

Evaluation results

Table 1 shows the multiple transmission power ratios in this evaluation.

Table 1: Multiple transmission power ratios for evaluations

| # of transmission power ratios | Transmission power ratios of near UE |
| --- | --- |
| 50 | 0.01~0.50, step:0.01 |
| 8 | 0.05, 0.10, 0.15, 0.20, 0.25, 0.30, 0.35, 0.40 |
| 4 | 0.15, 0.20, 0.25, 0.30 |
| 2 | 0.20, 0.25 |

We assume 2, 4, and 8 transmission power ratios for the evaluations. Moreover, we also evaluate 50 power sets to observe near upper bound. In this evaluation, we assume that non-full buffer traffic model with packet size of 100 kB, and wideband scheduling. Furthermore, the modulation scheme of far UE is restricted only to QPSK. And it is assumed that all the UEs know the assigned transmission power ratios. Other evaluation assumptions are listed in Annex.

Table 2: Throughputs and MUST gains

| Throughput (Mbps) | Non-MUST | MUST category 2 with N transmission power ratio candidates |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | N=50 | N=8 | N=4 | N=2 |  |
| Mean UPT | 5.08 | 5.78 (+13.9%) | 5.65 (+11.1%) | 5.67 (+11.6%) | 5.56 (+9.5%) |  |
| 5%ile UPT | 0.65 | 0.76 (+17.4%) | 0.76 (+16.8%) | 0.75 (+15.0%) | 0.74 (+14.2%) |  |
| 50%ile UPT | 3.31 | 3.98 (+20.4%) | 3.81 (+15.2%) | 3.83 (+15.8%) | 3.81 (+15.2%) |  |
| 95%ile UPT | 15.69 | 17.02 (+8.5%) | 17.02 (+8.5%) | 17.39(+10.9%) | 16.67 (+6.3%) |  |
| RU | 84.7% | 82.5% | 82.3% | 81.9% | 83.6% |  |
| Served/Offered | 0.9688 | 0.9795 | 0.9771 | 0.9768 | 0.97550 |  |
| λ / packet size | 10 / 100 KB |  |  |  |  |
| Note | Maximum transfer time = 1.6 sec, R-ML is applied as a near UE receiver |  |  |  |  |

UPT performances are shown in Table 2. MUST gain over non-MUST is more than 10% if more than 2 transmission power ratios are applied in average UPT performance. This is because application of multiple power ratios can jointly adjust the path-loss difference between UEs and combination of their modulation schemes.

Observation:

- MUST gain can be more than 10% in average UPT when more than 2 power ratios are used.

Proposal:

- RAN1 should introduce MUST category 2 with multiple transmission power ratios.

  - We prefer that the number of transmission power ratios is 4 or more.

- Conclusion

In this contribution, we showed the system level simulation results, and we made the following observation and proposal:

Observation:

- MUST gain can be more than 10% in average UPT when more than 2 power ratios are used.

Proposal:

- RAN1 should introduce MUST category 2 with multiple transmission power ratios.

  - We prefer that the number of transmission power ratios is 4 or more.

- References

- RP-160680, “New work item proposal: Downlink Multiuser Superposition Transmission for LTE”, MediaTek Inc., CMCC, 3GPP RAN Plenary Session #71, Gothenburg, Sweden, March 7-10, 2016.

- R1-156719, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, “System Level Evaluation on MUST,” RAN1#83, Anaheim, US, Nov. 2015.

- R1-154456, MediaTek Inc., “System Level Evaluation on MUST,” RAN1#82, Beijing, China, Aug. 2015.

- R1-156886, LG Electronics, “System-level evaluation results for MUST,” RAN1#83, Anaheim, US, Nov. 2015.

- R1-157212, Ericsson, “MuST System Level Performance and Observations,” RAN1#83, Anaheim, US, Nov. 2015.

- R1-157234, Nokia Networks, “System-level performance of MUST,” RAN1#83, Anaheim, US, Nov. 2015.

- R1-157352, NTT DOCOMO, “System-level evaluation results for downlink multiuser superposition schemes,” RAN1#83, Anaheim, US, Nov. 2015.

- Annex

Table 3 shows the evaluation assumptions.

Table 3: system level simulation assumptions

| Parameters | Assumptions / Values |
| --- | --- |
| Scenario | MUST Scenario 1 |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| Traffic model | FTP model 1, 100KB |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2 Tx, cross-polarized (+45 degree / -45 degree) UE: 2 Rx, cross-polarized (0 degree / 90 degree) |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| minimum distance from macro-cell to UEs | 35 m |
| UE receiver | For inter-cell and inter-spatial-layer interference - MMSE-IRC - Non-ideal covariance matrix estimation modelled by Wishart distribution (degree of freedom: 16) |
| Transmission mode | TM4 (Fixed to rank-1) |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Handover margin | 3 dB |
| Overhead | 3 symbols for DL CCHs |
| Receiver impairment modelling for demodulation | Non-ideal CRS based channel estimation |
| CSI estimation | Non-ideal CRS based CSI calculation |
| Feedback | CSI reporting mode 1-1 Feedback periodicity: 5ms Feedback delay: 5ms |
| EVM | Tx: 8 %, Rx: 4 % |