3GPP TSG RAN WG1 Meeting #84bis 	R1-162844

Busan, Korea, 11th - 15th April 2016

Source:	Sharp

Title:	Link level evaluation results of shortened TTI for PUSCH

Agenda Item:	7.3.10.1

Document for:	Discussion and Decision

Introduction

In the RAN1#84 meeting, the following agreements were made on PUSCH DM-RS for the latency reduction study item [1].

Agreements: - Following aspects are further studied in the next RAN1 meeting - Note: But the study is not limited to them. - Design of sPUSCH DM-RS - Alt.1: DM-RS symbol shared by multiple short-TTIs within the same subframe - Alt.2: DM-RS contained in each sPUSCH - HARQ for sPUSCH - Whether/how to realize asynchronous and/or synchronous HARQ - sTTI operation for Pcell and/or SCells by (e)CA in addition to non-(e)CA case

In this contribution, we show the evaluation results of design of sPUSCH DM-RS.

Performance evaluations

sPUSCH design

In [1], it was agreed that at least DM-RS design and HARQ operation for sPUSCH design are further studied. In this contribution we focus on the DM-RS design for sPUSCH. We assume slot TTI, 4-symbol TTI, and 2-symbol TTI as sTTI.

Figure 1 shows an example of slot TTI, 4-symbol TTI, and 2-symbol TTI of sPUSCH design. As shown in this figure, in slot TTI case as shown in Figure 1 (a), the same DM-RS pattern as legacy TTI can be used. In Figure 1 (b), this design is mentioned in [2], the same DM-RS pattern as legacy TTI is used and each DM-RS symbol is shared by two UEs as shown in this figure. In the 2-symbol TTI shown in Figure 1 (c), this has already been mentioned in [3], and this is aligned with Alt. 2.

Figure 1. An example of sPUSCH structure

In the 2-symbol TTI case, DM-RS symbol is contained in each TTI. Therefore, the DM-RS overhead is a concern.

Observation 1:

- In the 2-symbol TTI, if Alt. 2 is assumed, DM-RS overhead is a concern.

Evaluation results

We evaluated the performances of BLER and throughput. Detailed evaluation assumptions are listed in Annex and it is aligned with the link level evaluation assumption for sPUSCH based on the agreed assumptions in [4]. The number of PRBs for sPUSCH is 25 in this evaluation.

Low mobility case

Figure 2 and 3 show BLER and throughput performance for QPSK with code rate 1/3 and 16QAM with code rate 3/4 in low mobility case. In low mobility case, BLER performance is almost same as the legacy TTI case.

Observation 2:

- In low mobility case, the same performances are achieved from BLER perspective.

| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| --- | --- |
| Figure 2. BLER performances in low mobility case |  |
| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| Figure 3. Throughput performances in low mobility case |  |

Table 1 – 4 show the performance results by using the agreed format in [5].

Table 1. SINR at 10% BLER EPA 3 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 2.1 dB |  | 2.1 dB | 2.17 dB | 1.74 dB |
| 16QAM 3/4 |  | 13.72 dB |  | 13.72 dB | 13.69 dB | 13.53 dB |

Table 2. Throughput at 10% BLER EPA 3 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.92 Mbps |  | 1.52 Mbps | 1.79 Mbps | 1.69 Mbps |
| 16QAM 3/4 |  | 4.74 Mbps |  | 7.00 Mbps | 8.32 Mbps | 8.24 Mbps |

Table 3. SINR at 10% BLER ETU 3 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.06 dB |  | 0.11 dB | - 0.12 dB | 0.22 dB |
| 16QAM 3/4 |  | 12.68 dB |  | 12.59 dB | 12.58 dB | 12.56 dB |

Table 4. Throughput at 10% BLER ETU 3 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.66 Mbps |  | 1.11 Mbps | 1.20 Mbps | 2.25 Mbps |
| 16QAM 3/4 |  | 3.60 Mbps |  | 5.47 Mbps | 6.29 Mbps | 6.03 Mbps |

High mobility case

Figure 4 and 5 show BLER and throughput performance for QPSK with code rate 1/3 and 16QAM with code rate 3/4 in high mobility case. As shown in these figures, 4-symbol TTI is slightly worse, because channel variance in time domain due to high Doppler cannot be compensated within TTI because only one DM-RS symbol is contained within 4 SC-FDMA symbols.

Observation 3:

- In high mobility case, slot TTI and 2-symbol TTI sPUSCHs achieve the same BLER performance as the legacy TTI based PUSCH. 4-symbol TTI’s performance is slightly worse (around -1 dB) than the others.

| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| --- | --- |
| Figure 4. BLER performances in high mobility case |  |
| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| Figure 5. Throughput performances in high mobility case |  |

Table 5 – 8 show the performance results by using the agreed format in [5].

Table 5. SINR at 10% BLER EVA 60 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.33 dB |  | 0.39 dB | 0.3 dB | 0.51 dB |
| 16QAM 3/4 |  | 12.87 dB |  | 13.42 dB | 13.19 dB | 12.66 dB |

Table 6. Throughput at 10% BLER EVA 60 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.69 Mbps |  | 1.15 Mbps | 1.27 Mbps | 1.71 Mbps |
| 16QAM 3/4 |  | 4.02 Mbps |  | 6.53 Mbps | 6.84 Mbps | 6.54 Mbps |

Table 7. SINR at 10% BLER ETU 60 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.2 dB |  | 0.17 dB | 0.16 dB | 0.17 dB |
| 16QAM 3/4 |  | 13.03 dB |  | 13.29 dB | 13.01 dB | 12.65 dB |

Table 8. Throughput at 10% BLER ETU 60 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.68 Mbps |  | 1.13 Mbps | 1.24 Mbps | 1.78 Mbps |
| 16QAM 3/4 |  | 3.80 Mbps |  | 5.97 Mbps | 6.43 Mbps | 6.21 Mbps |

From the observations 1 – 3, we have the following proposal:

Observation 4:

- In low mobility case, all of slot TTI, 4-symbol TTI, and 2-symbol TTI sPUSCHs achieve the same BLER performance as the legacy TTI based PUSCH.

Observation 5:

- In high mobility case, slot TTI and 2-symbol TTI sPUSCHs achieve the same BLER performance as the legacy TTI based PUSCH. 4-symbol TTI’s performance is slightly worse (around -1 dB) than the others.

Conclusion

In this contribution, we have the following observations:

Observation 1:

- In the 2-symbol TTI, if Alt. 2 is assumed, DM-RS overhead is a concern.

Observation 2:

- In low mobility case, the same performances are achieved from BLER perspective.

Observation 3:

- In high mobility case, slot TTI and 2-symbol TTI sPUSCHs achieve the same BLER performance as the legacy TTI based PUSCH. 4-symbol TTI’s performance is slightly worse (around -1 dB) than the others.

Observation 4:

- In low mobility case, all of slot TTI, 4-symbol TTI, and 2-symbol TTI sPUSCHs achieve the same BLER performance as the legacy TTI based PUSCH.

Observation 5:

- In high mobility case, slot TTI and 2-symbol TTI sPUSCHs achieve the same BLER performance as the legacy TTI based PUSCH. 4-symbol TTI’s performance is slightly worse (around -1 dB) than the others.

References

3GPP RAN1#84 chairman’s notes, January 2016.

R1-160294, “Short TTI for UL transmissions”, RAN1#84, February 2016.

R1-160940, “Link level evaluation of PUSCH for short TTI”, RAN1#84, February 2016.

R1-157806, “WF on evaluation methodology for latency reduction”, RAN1#83, November 2015.

R1-161422, “Way forward on simulation results and methodology”, RAN1#84, February 2016.

Annex

Evaluation assumptions

Table 9. Evaluation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| System bandwidth | 10MHz |
| TTI length | 2/4/7/14 symbols |
| Allocated bandwidth | 25 PRBs |
| Channel model | EPA 3km/h, ETU 3km/h, EVA 60km/h, ETU 60 km/h |
| Antenna configuration | 1Tx (UE), 2Rx (eNB) |
| CP length | Normal |
| Transmission mode | TM1 |
| Receiver type | MMSE |
| Channel estimation | Practical |
| Link adaptation | Disabled |
| Modulation and code rate | QPSK 1/3, 16QAM 3/4 |
| HARQ retransmission | Disabled |
| Performance metrics | BLER, throughput |