# 3GPP TSG RAN WG1 Meeting #84bis	R1-162914

# Busan, South Korea, 11th – 15th April 2016

# Title: 	CR Discussion for Ending Partial Subframe in LAA

# Source: 	Nokia, Alcatel-Lucent Shanghai Bell

# Agenda Item:	7.1.3

# Document for: 	Discussion and Decision

# Introduction

There was some discussion in RAN1#84 on whether/what corrections are needed for the support of ending partial subframes in Rel-13 LAA DL. The issues and some proposals are summarized in [1]. In this contribution, we further analyze the issues and present our views on how to complete the support of LAA DL ending partial subframe in the specifications.

# Discussion

For the support of the ending partial subframe, the intention is to follow DwPTS operation in frame structure type 2 (FS2) with some exceptions. Based on the issues summarized in [1], we provide a table that lists the behavior of DwPTS in FS2, the behavior defined for ending partial subframe in FS3 in current specifications, and the proposed behavior.

| Specification | Special handling of DwPTS (compared to a normal SF) in FS2 | Behavior of ending partial SF in FS3 defined in the current specification | Proposed behavior of ending partial SF in FS3 |
| --- | --- | --- | --- |
| 36.211 Section 6.7 | Duration of PDCCH is up to 2 symbols for subframe 1 and 6 (where DwPTS falls into) | Duration of PDCCH is up to 3 symbols | No change to the current specification |
| 36.211 Section 6.8A | Determination of the number of EREGs per ECCE (Table 6.8A.1-1) and antenna port to use for localized EPDCCH (Table 6.8A.5-1) in DwPTS | Not explicitly specified | Reuse the behavior of DwPTS in FS2 |
| 36.211 Section 6.10.3 | UE-specific RS for PDSCH in DwPTS | Not explicitly specified | Reuse the behavior of DwPTS in FS2 |
| 36.211 Section 6.10.3A | DMRS for EPDCCH in DwPTS | Not explicitly specified | Reuse the behavior of DwPTS in FS2 |
| 36.211 Section 6.10.5 | CSI-RS pattern in DwPTS | CSI-RS is not transmitted (as agreed) | No change to the current specification |
| 36.213 Section 9.1.4 | EPDCCH candidates in DwPTS | Same as DwPTS in FS2 | No change to the current specification |
| 36.213 Section 7.1.7 | A scaling factor is applied for TBS determination of PDSCH transmitted in DwPTS | Not explicitly specified – fallback to the default of no scaling factor (not consistent with the RAN1 consensus) | Reuse the behavior of DwPTS in FS2 |
| 36.211 Section 6.11.1 | PSS transmitted in the 3rd symbol in DwPTS | PSS is transmitted in the last symbol in slot 0 (same as FS1) only when the ending partial SF >= 12 symbols (as agreed) | No change to the current specification |
| 36.211 Section 6.11.2 | No SSS transmission in DwPTS | SSS is transmitted in the second-to-last symbol in slot 0 in subframes where PSS is transmitted (same as FS1, as agreed) | No change to the current specification |
| 36.211 Section 6.3.5 | PDSCH is not mapped to REs transmitting PSS | PDSCH is not mapped to REs transmitting PSS/SSS | No change to the current specification |
| 36.211 Section 6.8 | No special handling for PDCCH - duration of PDCCH is up to 2 symbols, so it has no overlap with PSS transmission. | No special handling for PDCCH. It does not overlap with PSS/SSS. | No change to the current specification |
| 36.213 Section 9.1.4 | No special handling - EPDCCH is not transmitted in PRB pairs that contain PSS/SSS | No special handling - EPDCCH is not transmitted in PRB pairs that contain PSS/SSS | No change to the current specification |

The table shows that there are four issues in our view that require changes because the behaviors are not defined in current specifications. The corresponding draft CRs for 36.211 and 36.213 are provided in [2] and [3], respectively.

# References

- R1-161328, “Discussion on the support of DwPTS in partial ending subframes,” Huawei, HiSilicon, CATT, Samsung, RAN1#84, Feb. 2016.

- R1-162915, “Draft CR for 36.211 on the support of ending partial subframe in LAA”, Nokia, Alcatel-Lucent Shanghai Bell, RAN1#84bis, Apr. 2016.

- R1-162916, “Draft CR for 36.213 on TBS determination for ending partial subframe in LAA”, Nokia, Alcatel-Lucent Shanghai Bell, RAN1#84bis, Apr. 2016.