3GPP TSG RAN WG1 Meeting #84bits	R1-162947

Busan, Korea 11th - 15th April 2016

Source: MediaTek Inc.

Title: PUSCH Performance with short TTIs

Agenda Item:  7.3.10.1

Document for:  Discussion

- Introduction

In RAN#66, Study on latency reduction techniques for LTE was approved [1]. In this contribution, we evaluate the performance of normal TTI and short TTI. Short TTIs with 2/3/4/7 OFDM symbols are considered in the simulation. The results show that short TTI can achieve similar performance as normal TTI.

- Simulation

In this paper, short TTIs with 3 OFDM symbols and 4 OFDM symbols are considered. The simulation assumptions are shown in Table 1 (see Appendix). The DMRS patterns of short TTI with 2, 3, 4, and 7 OFDM symbols are shown in Figure 2-3 respectively. Practical channel estimation is performed based on the DMRS patterns shown in Figure 1-Figure 4.

Figure 1Short TTI with 2 OFDM symbols

Figure 2 Short TTI with 3 OFDM symbols

Figure 3 Short TTI with 4 OFDM symbols

Figure 4 Short TTI with 7 OFDM symbols

For low mobility UE (EPA 3km/hr), the results for different order are shown in Table 1-Table 3. For medium mobility UE (EVA 60km/hr), the results for different order are shown in Table 4-Table 6. For high mobility UE (ETU 120km/hr), the results for different order are shown in Table 7-Table 9.

| EPA 3km/hr, MCS = 5 (QPSK) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| STTI length (symbols) | 2 | 3 | 4 | 7 | 14 |
| SINR@10% BLER (dB) | 1.37 | 1.36 | 1.36 | 1.37 | 1.35 |
| Throughput@10% BLER (Mbps) | 1.0155 | 1.4495 | 1.6715 | 1.975 | 2.0077 |

Table 1BLER and Throuthput performance for EPA 3km/hr when MCS = 5

| EPA 3km/hr, MCS = 20 (16QAM) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| STTI length (symbols) | 2 | 3 | 4 | 7 | 14 |
| SINR@10% BLER (dB) | 13.6 | 13.7 | 14.1 | 14.3 | 13.9 |
| Throughput@10% BLER (Mbps) | 5.4319 | 7.3509 | 8.3167 | 9.5052 | 9.5693 |

Table 2 BLER and Throuthput performance for EPA 3km/hr when MCS = 20

| EPA 3km/hr, MCS = 28 (64QAM) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| STTI length (symbols) | 2 | 3 | 4 | 7 | 14 |
| SINR@10% BLER (dB) | 21.25 | 21.5 | 21.5 | 21.75 | 21.75 |
| Throughput@10% BLER (Mbps) | 9.4667 | 12.7725 | 14.4308 | 16.49 | 16.5024 |

Table 3 BLER and Throuthput performance for EPA 3km/hr when MCS = 28

Observation 1. For low mobility UE, shot TTI with 2 OFDM symbols has the lowest required SNR for 10% BLER. Legacy TTI has the highest throughput becase it has the smallest overhead. Shot TTI with 2 OFDM symbols has the lowest throughput due to its large overhead.

| EVA 60km/hr, MCS = 5 (QPSK) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| STTI length (symbols) | 2 | 3 | 4 | 7 | 14 |
| SINR@10% BLER (dB) | -0.25 | -0.25 | -0.1 | -0.1 | -0.2 |
| Throughput@10% BLER (Mbps) | 1.0337 | 1.4756 | 1.679 | 1.964 | 2.0055 |

Table 4 BLER and Throuthput performance for EVA 60 km/hr when MCS = 5

| EVA 60km/hr, MCS = 20 (16QAM) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| STTI length (symbols) | 2 | 3 | 4 | 7 | 14 |
| SINR@10% BLER (dB) | 13.3 | 13 | 13.3 | 13.4 | 13.78 |
| Throughput@10% BLER (Mbps) | 5.4775 | 7.3386 | 8.3538 | 9.5584 | 9.596 |

Table 5 BLER and Throuthput performance for EVA 60 km/hr when MCS = 20

| EVA 60km/hr, MCS = 28 (64QAM) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| STTI length (symbols) | 2 | 3 | 4 | 7 | 14 |
| SINR@10% BLER (dB) | 21.1 | 21.2 | 21.5 | 25 | 21.75 |
| Throughput@10% BLER (Mbps) | 9.4667 | 12.7866 | 14.3429 | 16.4716 | 16.5207 |

Table 6 BLER and Throuthput performance for EVA 60 km/hr when MCS = 28

Observation 2. For medium mobility UE, shot TTI with 2 symbols has the lowest required SNR for 10% BLER. Shot TTI with 7 symbols has the worst BLER performance when MCS = 28 (64QAM). Legacy TTI has the highest throughput becase it has the smallest overhead. Shot TTI with 2  OFDM symbols has the lowest throughput due to its large overhead.

| ETU 120km/hr, MCS = 5 (QPSK) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| STTI length (symbols) | 2 | 3 | 4 | 7 | 14 |
| SINR@10% BLER (dB) | 0.4 | 0.35 | 0.4 | 0.5 | 0.4 |
| Throughput@10% BLER (Mbps) | 1.0292 | 1.4756 | 1.6978 | 1.9662 | 2.0143 |

Table 7 BLER and Throuthput performance for ETU 120 km/hr when MCS = 5

| ETU 120km/hr, MCS = 20 (16QAM) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| STTI length (symbols) | 2 | 3 | 4 | 7 | 14 |
| SINR@10% BLER (dB) | 13.5 | 13.5 | 13.5 | 15.1 | 14 |
| Throughput@10% BLER (Mbps) | 5.4502 | 7.3961 | 8.2795 | 9.5851 | 9.6173 |

Table 8 BLER and Throuthput performance for ETU 120 km/hr when MCS = 20

| ETU 120km/hr, MCS = 28 (64QAM) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| STTI length (symbols) | 2 | 3 | 4 | 7 | 14 |
| SINR@10% BLER (dB) | 23.5 | 23.5 | 30 | - | 23.4 |
| Throughput@10% BLER (Mbps) | 9.4667 | 12.5316 | 14.263 | - | 16.4932 |

Table 9 BLER and Throuthput performance for ETU 120 km/hr when MCS = 28

Observation 3. For high mobility UE, shot TTI with 2 symbols has the lowest required SNR for 10% BLER. Short TTI with 7 symbols has the worst BLER performance for all the MCSs. Legacy TTI has the highest throughput becase it has the smallest overhead. Shot TTI with 2  OFDM symbols has the lowest throughput due to its large overhead.

Therefore, we have

Proposal. New RS pattern which has a sparse density in frequency domain can be considered in short TTI desing to reduce the overhead.

- Conclusion

From above discussion, we have

Observation 1. For low mobility UE, shot TTI with 2 OFDM symbols has the lowest required SNR for 10% BLER. Legacy TTI has the highest throughput becase it has the smallest overhead. Shot TTI with 2 OFDM symbols has the lowest throughput due to its large overhead.

Observation 2. For medium mobility UE, shot TTI with 2 symbols has the lowest required SNR for 10% BLER. Shot TTI with 7 symbols has the worst BLER performance when MCS = 28 (64QAM). Legacy TTI has the highest throughput becase it has the smallest overhead. Shot TTI with 2  OFDM symbols has the lowest throughput due to its large overhead.

Observation 3. For high mobility UE, shot TTI with 2 symbols has the lowest required SNR for 10% BLER. Short TTI with 7 symbols has the worst BLER performance for all the MCSs. Legacy TTI has the highest throughput becase it has the smallest overhead. Shot TTI with 2  OFDM symbols has the lowest throughput due to its large overhead.

Proposal. New RS pattern which has a sparse density in frequency domain can be considered in short TTI desing to reduce the overhead.

References

- RP-150465, New SI proposal: Study on Latency reduction techniques for LTE

Appendix

| Parameter | Value |
| --- | --- |
| Number of trials | 10000 |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz |
| TTI length | 2/3/4/7/14 symbols |
| Allocated bandwidth | 25 PRBs |
| Channel model | EPA 3km/h, EVA 60km/h, ETU 120km/h |
| Antenna configuration | 1Tx(UE), 2Rx(eNB) |
| CP length | Normal |
| Transmission mode | TM1 |
| DMRS configuration | See Figure 1- Figure 4 |
| Receiver type | MMSE |
| Channel estimation | Practical |
| Link adaptation | Disabled |
| Modulation and code rate | MCS=5 (QPSK), 20(16QAM), 28(64QAM) |
| TBS determination | Scaling for short TTI to fix the code rate |
| HARQ retransmission | Disabled |
| Performance metrics | BLER/Throughput |

Table 10 LLS simulation assumption