3GPP TSG RAN WG1 Meeting #84bis                              	R1-162967

Busan, Korea 11th - 15th April 2016

Source:	NTT DOCOMO

Title:	Evaluation of Rank 5~8 Codebook for EB/FD-MIMO

Agenda Item:	7.1.4

Document for: 	Discussion and Decision

Introduction

At the RAN1#83 meeting [1], RAN 1 has made a working assumption for rank 5~8 codebook for EB/FD-MIMO, as follows.

Working assumption:

- Rank 5 - 8 codebook which captured in R1-157789 is supported

At the RAN1#84 meeting [2], this issue was further discussed and the following conclusion regarding rank 5-8 codebook evaluation was made:

Conclusions:

- Companies are requested to evaluate rank 5 – 8 codebook design for class A based on R1-161355, R1-160850, and R1-160613 until the next meeting

  - Companies should take into account the correction of R1-161355

- RAN1 will decide a final conclusion related rank 5 – 8 codebook design issues for class A based on evaluation results and technical analysis before or in the next meeting

- RAN1 also will decide a final conclusion related to R1-161204 before or in the next meeting

In this contribution, we provide performance evaluation results and comparison among the three candidate schemes for making final conclusion.

Simulation on Rank 5-8 Codebook Designs and Comparison

Rank 5~8 codebook proposed in R1-161355 suggests W1 only feedback for rank 5-8 [3]. Proposal in R1-160850 suggests reusing legacy 8-Tx codebook by choosing 8 ports out of the configured 16 or 12 ports [4]. Proposal in R1-160613 [5] considers two additional approaches: one is to apply legacy 8-Tx codebook to both of two seperated 8 ports; and another one is based on the proposal in [6]. In this section,we provide simulation results on the three schemes, i.e.,

- Scheme 1 is based on proposal in [3];

- Scheme 2 is based on proposal in [6];

- and Scheme 3 is based on proposal in [4].

Key evaluation parameter values are summarized in Table A. We simulate the CSI-RS port configurations of (N1, N2) = (4, 2) and (2, 4), and the over-sampling factor of (O1, O2) = (4, 4). We adopt this over-sampling factor value for the sake of comparable complexity and CSI feedback overhead. For Scheme 1 and Scheme 2, W1 beam pattern Config. 3 is considered. Simulation results are summarized in Table 1 and 2.

Table 1: Codebook performance with (M, N, P) = (4, 4, 2).

Table 2: Codebook performance with (M, N, P) = (8, 4, 2).

From the system level simulation results, we observe that Scheme 3 causes obvious performance degradation, because this scheme makes use of 8 ports out of the configured 16 ports, which leads to the loss of power efficiency. It is also observed that similar performance can be obtained by Scheme 1 and Scheme 2.

Observation: Rank 5-8 codebooks in Scheme 1 and Scheme 2 have similar performance. Scheme 3 suffers from obvious performance degradation.

As the codebook design in Scheme 1 has similar performance as Scheme 2, and it has already been captured in CR 36.213, based on the support from a majority of companies, it is proposed to confirm the working assumption. Further enhancement of the rank 5~8 codebook can be discussed in the later LTE releases.

Proposal: Confirm the working assumption on rank 5~8 codebook.

Summary

In this contribution we made the system level evaluation of three codebook designs .Based on the simulation results,we give the obervation and proposal below.

Observation: Rank 5-8 codebooks in Scheme 1 and Scheme 2 have similar performance. Scheme 3 suffers from obvious performance degradation.

Proposal: Confirm the working assumption on rank 5~8 codebook.

- References

- 3GPP, “RAN1 Chairman’s Notes”, Nov. 2015.

- 3GPP, “RAN1 Chairman’s Notes”, Feb. 2016.

- 3GPP, R1-161355, “Correction to rank 5-8 FD-MIMO r2”, Feb. 2016.

- 3GPP, R1-160850, ZTE “Codebook Design for Rank 5-8”, Feb. 2016.

- 3GPP, R1-160613, LG Electronics, “Discussion on rank 5-8 codebook for FD-MIMO”, Feb. 2016.

- 3GPP, R1-157461, LG Electronics, “Details of rank 3-8 codebook for Class A CSI reporting”, Nov. 2015.

- Appendix

Table A: Evaluation assumptions

| Parameter | Values |
| --- | --- |
| Scenario / channel model | 3D-UMi (ISD: 200 m) |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz (50 RBs) |
| eNB antenna configurations | (dH, dV) = (0.5 , 0.8 ), etilt = 100 deg. |
| Total BS Tx power | 41 dBm |
| UE antenna configurations | 8 X-pol (0/90 deg.) |
| UE speed | 3 km/h |
| Indoor UE ratio | 80 % |
| MIMO scheme | SU-MIMO |
| UE receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel. 12 [71-12] assumptions |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel. 12 [71-12] assumptions |
| CSI feedback scheme | Subband PMI and CQI |
| CSI-RS transmission interval / CSI feedback interval | 5 ms for RI, PMI and CQI, 200 ms for beam selection |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes with low traffic loads |
| Scheduler | Proportional fairness |
| Control delay | 6 ms |
| HARQ | Chase combining with 8 ms RTD |